Single Channel, High Speed

Optocouplers

Technical  Data

6N135/6

HCNW135/6

HCNW4502/3

HCPL-2502

HCPL-0452/3

HCPL-0500/1

HCPL-4502/3

Features

• 15 kV/s Minimum  Common Mode Transient Immunity at VCM = 1500 V (4503/0453)

• High Speed: 1 Mb/s

• TTL Compatible

• Available in 8-Pin  DIP, SO-8, Widebody Packages

• Open Collector Output

• Guaranteed Performance from  Temperature: 0C

to 70C

• Safety Approval

UL Recognized  –  3750  V rms for 1 minute  (5000 V rms for

1 minute  for HCNW and Option 020 devices)  per UL1577

CSA Approved

IEC/EN/DIN EN 60747-5-2

Approved

Applications

• High Voltage Insulation

• Video Signal  Isolation

• Power Transistor Isolation in Motor  Drives

• Line Receivers

• Feedback Element in Switched Mode Power Supplies

• High Speed Logic Ground Isolation – TTL/TTL, TTL/ CMOS, TTL/LSTTL

• Replaces Pulse Transformers

• Replaces Slow

Phototransistor Isolators

• Analog  Signal  Ground

Isolation

Functional Diagram

Description

These diode-transistor optocoup- lers use an insulating layer between  a LED and an integrated photodetector to provide  elec- trical insulation between  input and output. Separate connections for the photodiode bias and output-transistor collector increase the speed  up to a hundred times that of a conven- tional phototransistor coupler  by reducing the base-collector capacitance.

–VIORM = 630 V peak  for

HCPL-4503#060

–VIORM = 1414  V peak  for

HCNW devices

NC  1

ANODE  2

8  VCC

*

7  VB

| TRUTH TABLE (POSITIVE LOGIC) LED | VO |
| --- | --- |
| ON OFF | LOW HIGH |

• Dual  Channel Version

Available (253X/4534/053X/

CATHODE  3

6  VO

0534)

• MIL-PRF-38534 Hermetic

Version  Available (55XX/

65XX/4N55)

NC  4                                     5  GND

* NOTE: FOR 4502/3, 0452/3, PIN 7 IS NOT CONNECTED.

A 0.1 F bypass  capacitor must be connected between  pins 5 and 8.

CAUTION: It is advised that  normal static precautions be taken in handling and assembly of this  component to prevent damage and/or degradation which may be induced by ESD.

2

These single channel  optocoup- lers are available  in 8-Pin DIP, SO-8 and Widebody package

output current for 1 TTL load and a 5.6 k pull-up resistor. CTR for these  devices is 19% minimum  at

Schematic

ICC

8

VCC

configurations.

The 6N135,  HCPL-0500,  and

IF = 16 mA.

The HCPL-4502,  HCPL-0452,

2     IF

ANODE

+

VF

IO       6

HCNW135 are for use in TTL/

CMOS, TTL/LSTTL or wide bandwidth analog  applications.

and HCNW4502 provide  the

electrical and switching performance of the 6N136,

– CATHODE

3

SHIELD

VO

5

GND

Current transfer ratio (CTR) for

these  devices is 7% minimum  at

IF   = 16 mA.

The 6N136,  HCPL-2502,

HCPL-0501,  and HCNW136 are designed for high speed  TTL/TTL applications. A standard 16 mA TTL sink current through the input LED will provide  enough

Selection Guide

HCPL-0501,  and HCNW136 with

increased ESD protection.

The HCPL-4503,  HCPL-0453,

and HCNW4503 are similar to the HCPL-4502,  HCPL-0452,  and HCNW4502 optocouplers but

have increased common  mode transient immunity  of 15 kV/s minimum  at VCM = 1500  V guaranteed.

HCPL-4503/0453

HCNW4503

I B

7    * VB

* NOTE: FOR HCPL-4502/-3, HCPL-0452/3, HCNW4502/3, PIN 7 IS NOT CONNECTED.

| Minimum CMR |  |  | 8-Pin DIP (300 Mil) |  | Small-Outline SO-8 |  | Widebody (400 Mil) | Hermetic |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dV/dt (V/s) | VCM (V) | Current Transfer Ratio (%) | Single Channel Package | Dual Channel Package* | Single Channel Package | Dual Channel Package* | Single Channel Package | Single and Dual Channel Packages* |
| 1,000 | 10 | 7 | 6N135 | HCPL-2530 | HCPL-0500 | HCPL-0530 | HCNW135 |  |
|  |  | 19 | 6N136 HCPL-4502† | HCPL-2531 | HCPL-0501 HCPL-0452† | HCPL-0531 | HCNW136 HCNW4502† |  |
|  |  | 15 | HCPL-2502 |  |  |  |  |  |
| 15,000 | 1500 | 19 | HCPL-4503† | HCPL-4534 | HCPL-0453† | HCPL-0534 | HCNW4503† |  |
| 1,000 | 10 | 9 |  |  |  |  |  | HCPL-55XX HCPL-65XX 4N55 |

*Technical  data for these  products are on separate Agilent publications.

†Pin 7, transistor base,  is not connected.

Ordering Information

Specify Part Number  followed by Option Number  (if desired).

Example:

HCPL-4503#XXXX

020 = UL 5000  V rms/1  Minute Option*

060 = IEC/EN/DIN EN 60747-5-2 VIORM = 630 V peak  Option**

300 = Gull Wing Surface  Mount Option†

500 = Tape and Reel Packaging Option

XXXE = Lead Free Option

Option data sheets  available. Contact  your Agilent sales representative or authorized distributor for information.

*For 6N135/6  and HCPL-4502/3  only.

**For HCPL-4503  only.  Combination of Option 020 and Option 060 is not available.

†Gull wing surface  mount  option  applies  to through hole parts  only.

Remarks:  The notation “#” is used for existing  products, while (new) products launched since 15th  July

2001  and lead free option  will use “-”

3

Package Outline Drawings

8-Pin  DIP Package (6N135/6, HCPL-4502/3, HCPL-2502)

9.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

TYPE NUMBER

8        7         6        5

OPTION CODE*

6.35 ± 0.25 (0.250 ± 0.010)

A XXXXZ YYWW

1        2         3

DATE CODE

UL

4        RECOGNITION

1.19 (0.047) MAX.

1.78 (0.070) MAX.

3.56 ± 0.13 (0.140 ± 0.005)

4.70 (0.185) MAX.

+ 0.076

5° TYP.                        0.254 - 0.051

+ 0.003) (0.010 - 0.002)

0.51 (0.020) MIN.

2.92 (0.115) MIN.

1.080 ± 0.320 (0.043 ± 0.013)

0.65 (0.025) MAX.

2.54 ± 0.25 (0.100 ± 0.010)

DIMENSIONS IN MILLIMETERS AND (INCHES).

*MARKING CODE LETTER FOR OPTION NUMBERS "L" = OPTION 020

"V" = OPTION 060

OPTION NUMBERS 300 AND 500 NOT MARKED.

NOTE: FLOATING LEAD PROTRUSION IS 0.25 mm (10 mils) MAX.

8-Pin  DIP Package with Gull Wing Surface Mount  Option  300  (6N135/6, HCPL-4502/3)

LAND PATTERN RECOMMENDATION

1.016 (0.040)

± 0.25

± 0.010)

10.9 (0.430)

| 1 2 3 4 9.65 ± 0.25 (0.380 ± 0.010) 8 7 6 5 |  |
| --- | --- |
|  | 6.350 (0.250 |

1.27 (0.050)

2.0 (0.080)

1.19 (0.047) MAX.

1.780 (0.070) MAX.

3.56 ± 0.13 (0.140 ± 0.005)

9.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

+ 0.076

0.254 - 0.051

+ 0.003) (0.010 - 0.002)

1.080 ± 0.320 (0.043 ± 0.013)

2.54 (0.100) BSC

0.635 ± 0.130 (0.025 ± 0.005)

0.635 ± 0.25 (0.025 ± 0.010)

12° NOM.

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES).

NOTE: FLOATING LEAD PROTRUSION IS 0.25 mm (10 mils) MAX.

4

Small  Outline SO-8 Package (HCPL-0500/1, HCPL-0452/3)

LAND PATTERN RECOMMENDATION

8       7       6       5

XXX

5.994 ± 0.203 (0.236 ± 0.008)

3.937 ± 0.127

(0.155 ± 0.005)

PIN ONE    1

0.406 ± 0.076

YWW

2       3       4

TYPE NUMBER (LAST 3 DIGITS)

DATE CODE

7.49 (0.295)

1.9 (0.075)

(0.016 ± 0.003)                                           1.270 BSC (0.050)

0.64 (0.025)

* 5.080 ± 0.127      (0.200 ± 0.005)

3.175 ± 0.127

(0.125 ± 0.005)                                             1.524 (0.060)

7°

0 ~ 7°

45° X  0.432 (0.017)

0.228 ± 0.025 (0.009 ± 0.001)

* TOTAL PACKAGE LENGTH (INCLUSIVE OF MOLD FLASH)

5.207 ± 0.254 (0.205 ± 0.010)

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES) MAX.

0.305 MIN. (0.012)

0.203 ± 0.102 (0.008 ± 0.004)

8-Pin  Widebody DIP Package (HCNW135/6, HCNW4502/3)

11.15 ± 0.15 (0.442 ± 0.006)

11.00 MAX. (0.433)

9.00 ± 0.15

8         7          6         5

A

TYPE NUMBER DATE CODE

(0.354 ± 0.006)

HCNWXXXX

YYWW

1

2

3

4

1.55 (0.061) MAX.

10.16 (0.400) TYP.

7° TYP.

5.10  MAX. (0.201)

+ 0.076

0.254 - 0.0051

+ 0.003) (0.010 - 0.002)

3.10 (0.122)

3.90 (0.154)

0.51 (0.021) MIN.

2.54 (0.100) TYP.

1.78 ± 0.15 (0.070 ± 0.006)

0.40 (0.016)

0.56 (0.022)

DIMENSIONS IN MILLIMETERS (INCHES).

NOTE: FLOATING LEAD PROTRUSION IS 0.25 mm (10 mils) MAX.

5

8-Pin  Widebody DIP Package with Gull Wing Surface Mount  Option  300  (HCNW135/6, HCNW4502/3)

11.15 ± 0.15 (0.442 ± 0.006)

LAND PATTERN RECOMMENDATION

8         7          6         5

9.00 ± 0.15 (0.354 ± 0.006)

13.56 (0.534)

1

2

3

4

1.3 (0.051)

2.29 (0.09)

1.55 (0.061) MAX.

12.30 ± 0.30 (0.484 ± 0.012)

11.00 MAX. (0.433)

4.00  MAX. (0.158)

1.78 ± 0.15 (0.070 ± 0.006)

2.54 (0.100) BSC

0.75 ± 0.25 (0.030 ± 0.010)

1.00 ± 0.15 (0.039 ± 0.006)

+ 0.076

0.254 - 0.0051

+ 0.003) (0.010 - 0.002)

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES).

NOTE: FLOATING LEAD PROTRUSION IS 0.25 mm (10 mils) MAX.

7° NOM.

Solder Reflow Temperature Profile

300

200

PREHEATING RATE 3°C + 1°C/–0.5°C/SEC. REFLOW HEATING RATE 2.5°C ± 0.5°C/SEC.

PEAK TEMP.

245°C

PEAK TEMP.

240°C

PEAK TEMP.

230°C

TEMPERATURE (°C)

100

160°C

150°C

140°C

2.5°C ± 0.5°C/SEC.

3°C + 1°C/–0.5°C PREHEATING TIME

150°C, 90 + 30 SEC.

30

SEC.

30

SEC.

SOLDERING TIME

200°C

50 SEC.

ROOM TEMPERATURE

0

0

50                          100

150

TIGHT TYPICAL LOOSE

200                         250

TIME (SECONDS)

6

Recommended Pb-Free IR Profile

Tp

TL   217 °C

tp

260 +0/-5 °C

RAMP-UP

3 °C/SEC. MAX.

TIME WITHIN 5 °C of ACTUAL PEAK TEMPERATURE

20-40 SEC.

RAMP-DOWN

TEMPERATURE

Tsmax  150 - 200 °C Tsmin

ts

6 °C/SEC. MAX.

tL

PREHEAT

60 to 180 SEC.

60 to 150 SEC.

25

NOTES:

t 25 °C to PEAK

TIME

THE TIME FROM 25 °C to PEAK TEMPERATURE = 8 MINUTES MAX. Tsmax = 200 °C, Tsmin = 150 °C

Regulatory Information The devices contained in this data sheet  have been approved by the following organizations:

UL

Recognized  under  UL 1577, Component Recognition Program, File E55361.

CSA

Approved under  CSA Component

Acceptance Notice #5, File CA

88324.

IEC/EN/DIN EN 60747-5-2

Approved under

IEC 60747-5-2:1997 + A1:2002

EN 60747-5-2:2001 + A1:2002

DIN EN 60747-5-2 (VDE 0884

Teil 2):2003-01

(HCNW and Option 060 only)

Insulation and Safety Related Specifications

| Parameter | Symbol | 8-Pin DIP (300 Mil) Value | SO-8 Value | Widebody (400 Mil) Value | Units | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum External Air Gap (External Clearance) | L(101) | 7.1 | 4.9 | 9.6 | mm | Measured from input terminals to output terminals, shortest distance through air. |
| Minimum External Tracking (External Creepage) | L(102) | 7.4 | 4.8 | 10.0 | mm | Measured from input terminals to output terminals, shortest distance path along body. |
| Minimum Internal Plastic Gap (Internal Clearance) |  | 0.08 | 0.08 | 1.0 | mm | Through insulation distance, conductor to conductor, usually the direct distance between the photoemitter and photodetector inside the optocoupler cavity. |
| Minimum Internal Tracking (Internal Creepage) |  | NA | NA | 4.0 | mm | Measured from input terminals to output terminals, along internal cavity. |
| Tracking Resistance (Comparative Tracking Index) | CTI | 200 | 200 | 200 | Volts | DIN IEC 112/VDE 0303 Part 1 |
| Isolation Group |  | IIIa | IIIa | IIIa |  | Material Group (DIN VDE 0110, 1/89, Table 1) |

Option 300 - surface  mount  classification is Class A in accordance with CECC 00802.

7

IEC/EN/DIN EN 60747-5-2 Insulation Related Characteristics

(HCPL-4503 OPTION 060  ONLY)

| Description | Symbol | Characteristic | Units |
| --- | --- | --- | --- |
| Installation classification per DIN VDE 0110/1.89, Table 1 for rated mains voltage ≤ 300 V rms |  | I-IV |  |
| for rated mains voltage ≤ 450 V rms |  | I-III |  |
| Climatic Classification |  | 55/100/21 |  |
| Pollution Degree (DIN VDE 0110/1.89) |  | 2 |  |
| Maximum Working Insulation Voltage | VIORM | 630 | V peak |
| Input to Output Test Voltage, Method b* VIORM x 1.875 = VPR, 100% Production Test with tm = 1 sec, Partial Discharge < 5 pC | VPR | 1181 | V peak |
| Input to Output Test Voltage, Method a* VIORM x 1.5 = VPR, Type and sample test, tm = 60 sec, Partial Discharge < 5 pC | VPR | 945 | V peak |
| Highest Allowable Overvoltage* (Transient Overvoltage, tini = 10 sec) | VIOTM | 6000 | V peak |
| Safety Limiting Values (Maximum values allowed in the event of a failure, also see Figure 9, Thermal Derating curve.) Case Temperature Input Current Output Power | TS IS,INPUT PS,OUTPUT | 175 230 600 | C mA mW |
| Insulation Resistance at TS, VIO = 500 V | RS | ≥ 109 | Ω |

IEC/EN/DIN EN 60747-5-2 Insulation Related Characteristics (HCNW135/6, HCNW4502/3

ONLY)

| Description | Symbol | Characteristic | Units |
| --- | --- | --- | --- |
| Installation classification per DIN VDE 0110/1.89, Table 1 for rated mains voltage ≤ 600 V rms |  | I-IV |  |
| for rated mains voltage ≤ 1000 V rms |  | I-III |  |
| Climatic Classification |  | 55/85/21 |  |
| Pollution Degree (DIN VDE 0110/1.89) |  | 2 |  |
| Maximum Working Insulation Voltage | VIORM | 1414 | V peak |
| Input to Output Test Voltage, Method b* VIORM x 1.875 = VPR, 100% Production Test with tm = 1 sec, Partial Discharge < 5 pC | VPR | 2652 | V peak |
| Input to Output Test Voltage, Method a* VIORM x 1.5 = VPR, Type and sample test, tm = 60 sec, Partial Discharge < 5 pC | VPR | 2121 | V peak |
| Highest Allowable Overvoltage* (Transient Overvoltage, tini = 10 sec) | VIOTM | 8000 | V peak |
| Safety Limiting Values (Maximum values allowed in the event of a failure, also see Figure 9, Thermal Derating curve.) Case Temperature Input Current Output Power | TS IS,INPUT PS,OUTPUT | 150 400 700 | C mA mW |
| Insulation Resistance at TS, VIO = 500 V | RS | ≥ 109 | Ω |

*Refer to the front of the optocoupler section  of the current catalog,  under  Product Safety Regulations section  IEC/EN/DIN EN

60747-5-2, for a detailed  description.

Note: Isolation  characteristics are guaranteed only within the safety maximum  ratings  which must be ensured by protective circuits  in application.

8

Absolute Maximum Ratings

| Parameter | Symbol | Device | Min. | Max. | Units | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature* | TS |  | -55 | 125 | C |  |
| Operating Temperature* | TA | 8-Pin DIP SO-8 | - 55 | 100 | C |  |
|  |  | Widebody | -55 | 85 |  |  |
| Average Forward Input Current* | IF(AVG) |  |  | 25 | mA | 1 |
| Peak Forward Input Current* (50% duty cycle, 1 ms pulse width) | IF(PEAK) | 8-Pin DIP SO-8 |  | 50 | mA | 2 |
| (50% duty cycle, 1 ms pulse width) |  | Widebody |  | 40 |  |  |
| Peak Transient Input Current* (≤ 1 s pulse width, 300 pps) | IF(TRANS) | 8-Pin DIP SO-8 |  | 1 | A |  |
|  |  | Widebody |  | 0.1 |  |  |
| Reverse LED Input Voltage* (Pin 3-2) | VR | 8-Pin DIP SO-8 |  | 5 | V |  |
|  |  | Widebody |  | 3 |  |  |
| Input Power Dissipation* | PIN | 8-Pin DIP SO-8 |  | 45 | mW | 3 |
|  |  | Widebody |  | 40 |  |  |
| Average Output Current* (Pin 6) | IO(AVG) |  |  | 8 | mA |  |
| Peak Output Current* | IO(PEAK) |  |  | 16 | mA |  |
| Emitter-Base Reverse Voltage* (Pin 5-7, except 4502/3, 0452/3) | VEBR |  |  | 5 | V |  |
| Supply Voltage (Pin 8-5) | VCC |  | -0.5 | 30 | V |  |
| Output Voltage (Pin 6-5) | VO |  | -0.5 | 20 | V |  |
| Supply Voltage* (Pin 8-5) | VCC |  | -0.5 | 15 | V |  |
| Output Voltage* (Pin 6-5) | VO |  | -0.5 | 15 | V |  |
| Base Current* (Pin 7, except 4502/3, 0452/3) | IB |  |  | 5 | mA |  |
| Output Power Dissipation* | PO |  |  | 100 | mW | 4 |
| Lead Solder Temperature* (Through-Hole Parts Only) 1.6 mm below seating plane, 10 seconds up to seating plane, 10 seconds | TLS | 8-Pin DIP |  | 260 | C |  |
|  |  | Widebody |  | 260 | C |  |
| Reflow Temperature Profile | TRP | SO-8 and Option 300 | See Package Outline Drawings section |  |  |  |

*Data has been  registered with JEDEC for the 6N135/6N136.

9

Electrical Specifications (DC)

Over recommended temperature (TA = 0C to 70C) unless  otherwise specified. See note 13.

| Parameter | Symbol | Device | Min. | Typ.** | Max. | Units | Test Conditions |  |  | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Current Transfer Ratio | CTR* | 6N135 HCPL-0500 HCNW135 | 7 | 18 | 50 | % | TA = 25C | VO = 0.4 V | IF = 16 mA, VCC = 4.5 V | 1, 2, 4 | 5, 11 |
|  |  |  | 5 | 19 |  |  |  | VO = 0.5 V |  |  |  |
|  |  | HCPL-2502 | 15 |  | 22 |  | TA = 25C | VO = 0.4 V |  |  |  |
|  |  |  | 15 | 25 |  |  |  | VO = 0.5 V |  |  |  |
|  |  | 6N136 HCPL-4502/3 HCPL-0501 HCPL-0452/3 HCNW136 HCNW4502/3 | 19 | 24 | 50 |  | TA = 25C | VO = 0.4 V |  |  |  |
|  |  |  | 15 | 25 |  |  |  | VO = 0.5 V |  |  |  |
| Logic Low Output Voltage | VOL | 6N135 HCPL-0500 HCNW135 |  | 0.1 | 0.4 | V | TA = 25C | IO = 1.1 mA | IF = 16 mA, VCC = 4.5 V |  |  |
|  |  |  |  | 0.1 | 0.5 |  |  | IO = 0.8 mA |  |  |  |
|  |  | 6N136 HCPL-2502 HCPL-4502/3 HCPL-0501 HCPL-0452/3 HCNW136 HCNW4502/3 |  | 0.1 | 0.4 |  | TA = 25C | IO = 3.0 mA |  |  |  |
|  |  |  |  | 0.1 | 0.5 |  |  | IO = 2.4 mA |  |  |  |
| Logic High Output Current | IOH* |  |  | 0.003 | 0.5 | A | TA = 25C | VO = VCC = 5.5 V | IF = 0 mA | 7 |  |
|  |  |  |  | 0.01 | 1 |  | TA = 25C | VO = VCC = 15 V |  |  |  |
|  |  |  |  |  | 50 |  |  | VO = VCC = 15 V |  |  |  |
| Logic Low Supply Current | ICCL |  |  | 50 | 200 | A | IF = 16 mA, VO = Open, VCC = 15 V |  |  |  | 13 |
| Logic High Supply Current | ICCH* |  |  | 0.02 | 1 | A | TA = 25C | IF = 16 mA, VO = Open, VCC = 15 V |  |  | 13 |
|  |  |  |  |  | 2 |  |  |  |  |  |  |
| Input Forward Voltage | VF* | 8-Pin DIP SO-8 |  | 1.5 | 1.7 1.8 | V | TA = 25C | IF = 16 mA |  | 3 |  |
|  |  | Widebody | 1.45 | 1.68 | 1.85 |  | TA = 25C | IF = 16 mA |  |  |  |
|  |  |  | 1.35 |  | 1.95 |  |  |  |  |  |  |
| Input Reverse Breakdown Voltage | BVR* | 8-Pin DIP SO-8 | 5 |  |  | V | IR = 10 A |  |  |  |  |
|  |  | Widebody | 3 |  |  |  | IR = 100 A |  |  |  |  |
| Temperature Coefficient of Forward Voltage | ∆VF/ ∆TA | 8-Pin DIP SO-8 |  | -1.6 |  | mV/C | IF = 16 mA |  |  |  |  |
|  |  | Widebody |  | -1.9 |  |  |  |  |  |  |  |
| Input Capacitance | CIN | 8-Pin DIP SO-8 |  | 60 |  | pF | f = 1 MHz, VF = 0 V |  |  |  |  |
|  |  | Widebody |  | 90 |  |  |  |  |  |  |  |
| Transistor DC Current Gain | hFE | 8-Pin DIP SO-8 |  | 150 |  |  | VO = 5 V, IO = 3 mA |  |  |  |  |
|  |  |  |  | 130 |  |  | VO = 0.4 V, IB = 20 A |  |  |  |  |
|  |  | Widebody |  | 180 |  |  | VO = 5 V, IO = 3 mA |  |  |  |  |
|  |  |  |  | 160 |  |  | VO = 0.4 V, IB = 20 A |  |  |  |  |

*For JEDEC registered parts.

**All typicals at TA = 25C.

10

Switching Specifications (AC)

Over recommended temperature (TA = 0C to 70C),  VCC = 5 V, IF = 16 mA unless  otherwise specified.

| Parameter | Sym. | Device | Min. | Typ.** | Max. | Units | Test Conditions |  | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation Delay Time to Logic Low at Output | tPHL* | 6N135 HCPL-0500 HCNW135 |  | 0.2 | 1.5 | s | TA = 25C | RL = 4.1 kΩ | 5, 6, 11 | 8, 9 |
|  |  |  |  |  | 2.0 |  |  |  |  |  |
|  |  | 6N136 HCPL-2502 HCPL-4502/3 HCPL-0501 HCPL-0452/3 HCNW136 HCNW4502/3 |  | 0.2 | 0.8 |  | TA = 25C | RL = 1.9 kΩ |  |  |
|  |  |  |  |  | 1.0 |  |  |  |  |  |
| Propagation Delay Time to Logic High at Output | tPLH* | 6N135 HCPL-0500 HCNW135 |  | 1.3 | 1.5 | s | TA = 25C | RL = 4.1 kΩ | 5, 6, 11 | 8, 9 |
|  |  |  |  |  | 2.0 |  |  |  |  |  |
|  |  | 6N136 HCPL-2502 HCPL-4502/3 HCPL-0501 HCPL-0452/3 HCNW136 HCNW4502/3 |  | 0.6 | 0.8 |  | TA = 25C | RL = 1.9 kΩ |  |  |
|  |  |  |  |  | 1.0 |  |  |  |  |  |
| Common Mode Transient Immunity at Logic High Level Output | ¦CMH¦ | 6N135 HCPL-0500 HCNW135 |  | 1 |  | kV/s | RL = 4.1 kΩ | IF = 0 mA, TA = 25C, VCM = 10 Vp-p CL = 15 pF | 12 | 7, 8, 9 |
|  |  | 6N136 HCPL-2502 HCPL-4502 HCPL-0501 HCPL-0452 HCNW4502 |  | 1 |  |  | RL = 1.9 kΩ |  |  |  |
|  |  | HCPL-4503 HCPL-0453 HCNW4503 | 15 | 30 |  |  | RL = 1.9 kΩ | IF = 0 mA, TA = 25C, VCM = 1500 Vp-p, CL = 15 pF |  |  |
| Common Mode Transient Immunity at Logic Low Level Output | ¦CML¦ | 6N135 HCPL-0500 HCNW135 |  | 1 |  | kV/s | RL = 4.1 kΩ | IF = 16 mA, TA = 25C, VCM = 10 Vp-p CL = 15 pF | 12 | 7, 8, 9 |
|  |  | 6N136 HCPL-2502 HCPL-4502 HCPL-0501 HCPL-0452 HCNW4502 |  | 1 |  |  | RL = 1.9 kΩ |  |  |  |
|  |  | HCPL-4503 HCPL-0453 HCNW4503 | 15 | 30 |  |  | RL = 1.9 kΩ | IF = 16 mA, TA = 25C, VCM = 1500 Vp-p, CL = 15 pF |  |  |
| Bandwidth | BW | 6N135/6 HCPL-2502 HCPL-0500/1 |  | 9 |  | MHz | See Test Circuit |  | 8, 10 | 10 |
|  |  | HCNW135/6 |  | 11 |  |  |  |  |  |  |

*For JEDEC registered parts.

**All typicals at TA = 25C.

11

Package Characteristics

Over recommended temperature (TA = 0C to 70C) unless  otherwise specified.

| Parameter | Sym. | Device | Min. | Typ.* | Max. | Units | Test Conditions | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input-Output Momentary Withstand Voltage** | VISO | 8-Pin DIP SO-8 | 3750 |  |  | V rms | RH < 50%, t = 1 min., TA = 25C |  | 6, 14 |
|  |  | Widebody | 5000 |  |  |  |  |  | 6, 15 |
|  |  | 8-Pin DIP (Option 020) | 5000 |  |  |  |  |  | 6, 12, 15 |
|  | II-O | 8-Pin DIP |  |  | 1 | A | 45% RH, t = 5 s, VI-O = 3 kVdc, TA = 25C |  | 6, 16 |
| Input-Output Resistance | RI-O | 8-Pin DIP SO-8 |  | 1012 |  | Ω | VI-O = 500 Vdc |  | 6 |
|  |  | Widebody | 1012 | 1013 |  |  | TA = 25C |  |  |
|  |  |  | 1011 |  |  |  | TA = 100C |  |  |
| Input-Output Capacitance | CI-O | 8-Pin DIP SO-8 |  | 0.6 |  | pF | f = 1 MHz |  | 6 |
|  |  | Widebody |  | 0.5 | 0.6 |  |  |  |  |

*All typicals  at TA = 25C.

**The Input-Output Momentary  Withstand Voltage is a dielectric voltage  rating  that  should  not be interpreted as an input-output

continuous voltage  rating.  For the continuous voltage  rating  refer  to the IEC/EN/DIN EN 60747-5-2 Insulation Related  Characteristics Table (if applicable), your equipment level safety specification or Agilent Application  Note 1074  entitled  “Optocoupler Input-Output Endurance Voltage,” publication number 5963-2203E.

Notes:

1.  Derate  linearly above 70C free-air  temperature at a rate of 0.8 mA/C (8-Pin DIP).

Derate  linearly above 85C free-air  temperature at a rate of 0.5 mA/C (SO-8).

2.  Derate  linearly above 70C free-air  temperature at a rate of 1.6 mA/C (8-Pin DIP).

Derate  linearly above 85C free-air  temperature at a rate of 1.0 mA/C (SO-8).

3.  Derate  linearly above 70C free-air  temperature at a rate of 0.9 mW/C (8-Pin DIP).

Derate  linearly above 85C free-air  temperature at a rate of 1.1 mW/C (SO-8).

4.  Derate  linearly above 70C free-air  temperature at a rate of 2.0 mW/C (8-Pin DIP).

Derate  linearly above 85C free-air  temperature at a rate of 2.3 mW/C (SO-8).

5.  CURRENT TRANSFER RATIO in percent is defined as the ratio of output collector current, IO, to the forward  LED input current, IF, times 100.

6.  Device considered a two-terminal device: Pins 1, 2, 3, and 4 shorted together and Pins 5, 6, 7, and 8 shorted together.

7.  Common  mode transient immunity  in a Logic High level is the maximum  tolerable (positive) dVCM/dt on the leading edge of the common  mode pulse signal, VCM, to assure  that the output will remain  in a Logic High state  (i.e., VO > 2.0 V). Common  mode transient immunity  in a Logic Low level is the maximum  tolerable (negative) dVCM /dt on the trailing  edge of the common  mode pulse signal, VCM, to assure  that the output will remain  in a Logic Low state  (i.e., VO < 0.8 V).

8.  The 1.9 k load represents 1 TTL unit load of 1.6 mA and the 5.6 k pull-up resistor.

9.  The 4.1 k load represents 1 LSTTL unit load of 0.36 mA and 6.1 k pull-up resistor.

10.  The frequency at which the ac output voltage is 3 dB below its mid-frequency value.

11.  The JEDEC registration for the 6N136 specifies  a minimum  CTR of 15%. Agilent guarantees a minimum  CTR of 19%.

12.  See Option 020 data sheet  for more information.

13.  Use of a 0.1 f bypass  capacitor connected between  pins 5 and 8 is recommended.

14.  In accordance with UL 1577,  each optocoupler is proof tested  by applying  an insulation test voltage   4500  V rms for 1 second

(leakage detection current limit, II-O   5 A). This test is performed before  the 100% Production test shown in the IEC/EN/DIN EN

60747-5-2 Insulation Related Characteristics Table if applicable.

15.  In accordance with UL 1577,  each optocoupler is proof tested  by applying  an insulation test voltage   6000  V rms  for 1 second

(leakage detection current limit, II-O   5 A). This test is performed before  the 100% Production test shown in the IEC/EN/DIN EN

60747-5-2 Insulation Related Characteristics Table if applicable.

16.  This rating  is equally validated  by an equivalent ac proof test.

12

TA = 25°C

8 PIN DIP, SO-8

40 mA

16

TA = 25°C

WIDEBODY

IO – OUTPUT CURRENT – mA

10   VCC = 5.0 V

5

0

35 mA

30 mA

25 mA

20 mA

15 mA

10 mA

IF = 5 mA

VCC = 5.0 V

IO – OUTPUT CURRENT – mA

12

8

4

0

40 mA

35 mA

30 mA

25 mA

20 mA

15 mA

10 mA

IF = 5 mA

0                             10                            20

VO – OUTPUT VOLTAGE – V

0                             10                            20

VO – OUTPUT VOLTAGE – V

Figure 1. DC and Pulsed Transfer Characteristics.

NORMALIZED CURRENT TRANSFER RATIO

1.5

1.0

0.5

0.1

8 PIN DIP, SO-8

6N135, HCPL-0500

6N136, HCPL-4502/3,

HCPL-2502

HCPL-0501/0452/0453

NORMALIZED IF = 16 mA

VO = 0.4 V VCC = 5 V TA = 25°C

0                   1                  10                100

IF – INPUT CURRENT – mA

NORMALIZED CURRENT TRANSFER RATIO

1.5

1.0

0.5

0

WIDEBODY

HCNW135/6, HCNW4502/3

NORMALIZED IF = 16 mA

VO = 0.4 V VCC = 4.5 V TA = 25 °C

0                   1                  10                100

IF  – INPUT CURRENT – mA

Figure 2. Current Transfer Ratio  vs. Input  Current.

1000

IF – FORWARD CURRENT – mA

100

8 PIN DIP, SO-8

IF

1000

100

TA = 25° C

WIDEBODY

10               + VF

–

1.0

TA = 25°C

10

1.0

0.1

0.01

0.001

1.1       1.2        1.3       1.4

1.5

1.6

0.1

IF – FORWARD CURRENT – mA

0.01

0.001

1.2     1.3      1.4     1.5

1.6

IF

+ VF

–

1.7

1.8

VF – FORWARD VOLTAGE – VOLTS

VF – FORWARD VOLTAGE – VOLTS

Figure 3. Input  Current vs. Forward Voltage.

13

NORMALIZED CURRENT TRANSFER RATIO

1.1

1.0

0.9

0.8

8 PIN DIP, SO-8

NORMALIZED IF = 16 mA

VO = 0.4 V VCC = 5 V TA = 25°C

6N135, HCPL-0500

1.1

0.9

0.8

0.7

WIDEBODY

0.7                  6N136, HCPL-4502/3,

HCPL-2502

HCPL-0501, 0452, 0453

0.6

0.5

0.4

-60  -40

-20    0

20    40    60

NORMALIZED CURRENT TRANSFER RATIO

80  100

-60  -40

-20     0

20    40

| 60 80 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | HCNW135/6, HCNW4502/3 |  |  |  |
|  |  |  |  | IF = 16 mA VO = 0.4 V VCC = 5 V TA = 25°C |  |  |  |

100

TA –  TEMPERATURE – °C

TA – TEMPERATURE – °C

Figure 4. Current Transfer Ratio  vs. Temperature.

| 2000 | IF = 16 mA, VCC = 5.0 V 6N135, HCPL-0500 (RL = 4. 6N136, HCPL-0501, HCPL- HCPL-4502/3 (RL = 1.9 kΩ) HCPL-0452/3 |  |  |  | 1 kΩ) 2502 |
| --- | --- | --- | --- | --- | --- |
|  |  | tPLH | tPHL |  |  |

8 PIN DIP, SO-8

1000

WIDEBODY

IF = 16 mA, VCC

= 5.0 V

tp – PROPAGATION DELAY – ns

1500

800

HCNW135 (RL = 4.1 k) HCNW136 (RL = 1.9 k) HCNW4502/3

1000

500

600

400

tp – PROPAGATION DELAY – ns

200

tPLH

tPHL

0

-60           -20

20            60           100

0

-60  -40

-20    0

20    40    60

80  100

TA –  TEMPERATURE – °C

TA –  TEMPERATURE – °C

Figure 5. Propagation Delay  vs. Temperature.

8 PIN DIP, SO-8

tP – PROPAGATION DELAY – µs

3.0                     IF  = 10 mA IF  = 16 mA

tP – PROPAGATION DELAY – µs

6.0

WIDEBODY

I   = 10 mA I t = 16 mA

2.0

1.0

0.8

0.6

0.4

0.2

VCC = 5.0 V TA = 25 °C

tPLH

tPHL

4.0

1.0

0.6

0.4

0.2

VCC = 5.0 V TA = 25 °C

t PLH

t PHL

0.1

1

2         3      4

5   6  7 8 9 10

0.1

1       2       4

10               40       100

RL – LOAD RESISTANCE – (k)

RL – LOAD RESISTANCE – (k)

Figure 6. Propagation Delay  Time  vs. Load Resistance.

14

IOH – LOGIC HIGH OUTPUT CURRENT – nA

10+4

10+3

IF = 0

IOH – LOGIC HIGH OUTPUT CURRENT – nA

8 PIN DIP, SO-8

+3

10

IF = 0

WIDEBODY

10+2

VO = VCC = 15 V

10

10+1

100                                                                                                        +1

10

10-1

10-2

-75

-50   -25     0    +25   +50   +75  +100

100

-60

-40

-20    0    20    40

60   80  100

TA – TEMPERATURE – °C

TA – TEMPERATURE – °C

Figure 7. Logic High Output Current vs. Temperature.

0.30

8 PIN DIP, SO-8

TA = 25°C, RL  = 100 , VCC = 5 V

0.50

WIDEBODY

TA = 25°C, RL  = 100 , VCC = 5 V

0.40

0.20

0.30

0.10

 IF  – SMALL SIGNAL CURRENT TRANSFER RATIO

0.20

0.10

0

0         4         8       12      16

20      25

0

0        4        8        12      16

20      25

 IO – SMALL SIGNAL CURRENT TRANSFER RATIO

I F

 IO

IF – QUIESCENT INPUT CURRENT – mA

IF – QUIESCENT INPUT CURRENT – mA

Figure 8. Small-Signal Current Transfer Ratio  vs. Quiescent Input  Current.

OUTPUT POWER – PS, INPUT CURRENT – IS

800

700

600

500

400

300

200

100

| 0 | PS (mW) IS (mA) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

HCPL-4503 OPTION 060

1000

900

800

700

600

500

400

300

200

100

0

HCNW135/6, HCNW4502/3

0    25

OUTPUT POWER – PS, INPUT CURRENT – IS

50    75   100  125  150 175  200

0     25

50     75    100

125

150

| 175 | PS (mW) IS (mA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

TS – CASE TEMPERATURE – °C

TS – CASE TEMPERATURE – °C

Figure 9. Thermal Derating Curve,  Dependence of Safety Limiting Value with Case  Temperature per IEC/EN/DIN EN 60747-5-2.

15

6N135/6, HCPL-0500/1, HCPL-2502

+5

+12 V

6N135/6, HCPL-0500/1, HCPL-2502

0.1 µF

0

-5

1

2.1 K

2

8

15 K

7

9.1 K

1.2 K

Q 3

+12 V

0.1 µF

-10

VFF

47 µF

100                       3

Q 1

6

0.1 µF

Q 2

470 

VO

(1 M 12 pF

NORMALIZED RESPONSE - dB

-15

51 

1 K

4                                            5

R T

TRIM FOR

100 

TEST INPUT)

| -20 |  | TA = 25 °C |
| --- | --- | --- |

0.1                1.0                 10                100

f - FREQUENCY - MHz

1N4150          22 

UNITY GAIN

Q 1 , Q 2 , Q 3 : 2N3904

TYPICAL LINEARITY = ± 3% AT VIN = 1 Vp-p

TYPICAL SNR = 50dB

TYPICAL R T = 375  TYPICAL VO dc = 3.8 V TYPICAL I F = 9 mA

HCNW135/6

HCNW135/6

Figure 10.  Frequency Response.

| IF |  |  |
| --- | --- | --- |
|  | 5 |  |

0

VO                                                                     V

PULSE        IF

GEN.                        1

ZO = 50 

t r = 5 ns                      2

8

7  *                RL

+5 V

1.5 V

1.5 V

VOL

10% DUTY CYCLE

1/f < 100 µS                   3

IF  MONITOR                      4

6

0.1µF

5

VO

CL = 1.5 µF

t PHL

t PLH

RM

* PIN 7 UNCONNECTED IN HCPL-4502/3, HCPL-0452/3, HCNW4502/3

Figure 11.  Switching Test  Circuit.

VCM

0 V

IF

1

B

2

10%      90%  90%     10%                                                                A

3

8

7   *                   RL

6

+5 V

VO

t r                                            t f

VO

SWITCH AT A: I F = 0 mA VO

SWITCH AT B: I F = 16 mA

5 V VOL

VFF

4                                            5

VCM

+          –

0.1 µF

PULSE GEN.

*PIN 7 UNCONNECTED IN HCPL-4502/3, HCPL-0542/3, HCNW4502/3

Figure 12.  Test  Circuit for Transient Immunity and Typical  Waveforms.

www.agilent.com/semiconductors For product information and a complete list of distributors, please go to our web site.

For technical assistance  call: Americas/Canada: +1 (800) 235-0312 or

(916) 788-6763

Europe: +49 (0) 6441 92460

China: 10800 650 0017

Hong Kong: (+65) 6756 2394

India, Australia, New Zealand: (+65) 6755 1939

Japan: (+81 3) 3335-8152 (Domestic/Interna- tional), or 0120-61-1280 (Domestic Only)

Korea: (+65) 6755 1989

Singapore, Malaysia, Vietnam, Thailand, Philippines, Indonesia: (+65) 6755 2044

Taiwan: (+65) 6755 1843

Data subject to change.

Copyright © 2004 Agilent Technologies, Inc. Obsoletes 5989-0305EN

December 20, 2004

5989-2112EN