High CMR Line Receiver

Optocouplers

Technical  Data

HCPL-2602

HCPL-2612

Features

• 1000 V/ s Minimum  Common

Mode Rejection (CMR) at VCM  = 50 V for HCPL-2602 and 3.5  kV/  s Minimum CMR at VCM = 300  V for HCPL-2612

• Line Termination Included – No Extra Circuitry Required

• Accepts a Broad  Range of

Drive  Conditions

• LED Protection Minimizes

LED Efficiency Degradation

• High Speed: 10 MBd (Limited by Transmission Line in Many Applications)

• Guaranteed AC and DC Performance over Temperature: 0 C to 70 C

• External Base  Lead Allows

“LED Peaking” and LED

Applications

• Isolated Line Receiver

• Computer-Peripheral

Interface

• Microprocessor System

Interface

• Digital Isolation for A/D, D/A Conversion

• Current Sensing

• Instrument Input/Output

Isolation

• Ground  Loop Elimination

• Pulse Transformer

Replacement

• Power Transistor Isolation in Motor  Drives

Functional Diagram

Description

The HCPL-2602/12 are optically coupled  line receivers that combine a GaAsP light emitting diode, an input current regulator and an integrated high gain photo detector. The input regulator serves  as a line termination for line receiver  applications. It clamps  the line voltage and regulates the LED current so line reflections do not interfere with circuit  performance.

The regulator allows a typical LED current of 8.5 mA before  it starts  to shunt  excess  current. The output of the detector IC is

TRUTH TABLE

Current Adjustment

• Safety Approval

UL Recognized – 2500 V rms for 1 Minute

CSA Approved

• MIL-STD-1772 Version

Available (HCPL-1930/1)

NC     1

IN+     2

IN–     3

CATHODE     4

SHIELD

| 8 VCC LED | ENABLE | OUTPUT |
| --- | --- | --- |
| ON | H | L |
| OFF | H | H |
| ON | L | H |
| OFF | L | H |
| ON | NC | L |
| OFF | NC | H |

7    VE

6    VO

5    GND

(POSITIVE LOGIC)

A 0.1   F bypass  capacitor must be connected between  pins 5 and 8.

CAUTION: It is advised that  normal static precautions be taken in handling and assembly of this  component to prevent damage and/or degradation which may be induced by ESD.

1-314

5965-3585E

an open collector Schottky clamped transistor. An enable input gates  the detector. The internal detector shield provides  a guaranteed common  mode transient immunity  specification

of 1000  V/  s for the 2602,  and

3500  V/  s for the 2612.

DC specifications are defined similar to TTL logic. The optocoupler ac and dc operational parameters are guaranteed from

0 C to 70 C allowing trouble-free interfacing with digital logic circuits. An input current of 5 mA will sink an eight gate fan-out (TTL) at the output.

The HCPL-2602/12 are useful as line receivers in high noise environments that conventional line receivers cannot  tolerate. The higher  LED threshold voltage provides  improved immunity  to differential noise and the internally shielded  detector provides  orders of magnitude improvement in common  mode rejection with little or no sacrifice  in speed.

Selection Guide

| Minimum CMR | Input On- Current (mA) | Output Enable | 8-Pin DIP (300 Mil) |  | Small-Outline SO-8 |  | Widebody (400 Mil) | Hermetic |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dV/dt VCM (V/ s) (V) |  |  | Single Channel Package | Dual Channel Package | Single Channel Package | Dual Channel Package | Single Channel Package | Single and Dual Channel Packages |
| NA NA | 5 | YES | 6N137 |  | HCPL-0600 |  | HCNW137 |  |
|  |  | NO |  | HCPL-2630 |  | HCPL-0630 |  |  |
| 5,000 50 |  | YES | HCPL-2601 |  | HCPL-0601 |  | HCNW2601 |  |
|  |  | NO |  | HCPL-2631 |  | HCPL-0631 |  |  |
| 10,000 1,000 |  | YES | HCPL-2611 |  | HCPL-0611 |  | HCNW2611 |  |
|  |  | NO |  | HCPL-4661 |  | HCPL-0661 |  |  |
| 1,000 50 |  | YES | HCPL-2602[1] |  |  |  |  |  |
| 3,500 300 |  | YES | HCPL-2612[1] |  |  |  |  |  |
| 1,000 50 | 3 | YES | HCPL-261A |  | HCPL-061A |  |  |  |
|  |  | NO |  | HCPL-263A |  | HCPL-063A |  |  |
| 1,000[2] 1,000 |  | YES | HCPL-261N |  | HCPL-061N |  |  |  |
|  |  | NO |  | HCPL-263N |  | HCPL-063N |  |  |
| 1,000 50 12.5 |  | [3] |  |  |  |  |  | HCPL-193X HCPL-56XX HCPL-66XX |

Notes:

1. HCPL-2602/2612 devices  include  input  current regulator.

2. 15 kV/  s with VCM = 1 kV can be achieved  using HP application circuit.

3. Enable  is available  for single channel  products only, except  for HCPL-193X devices.

1-315

Ordering Information

Specify Part Number  followed by Option Number  (if desired).

Example:

HCPL-2602#XXX

300 = Gull Wing Surface  Mount Option

500 = Tape and Reel Packaging Option

Option data sheets  available.  Contact  your Hewlett-Packard sales representative or authorized distributor for information.

Schematic

+

2    II

VI

IF                                                                   ICC

8 VCC

IO

VO

6

–                                   90

3

4

SHIELD

IE

7

VE

GND

5

USE OF A 0.1 µF BYPASS CAPACITOR CONNECTED BETWEEN PINS 5 AND 8 IS REQUIRED (SEE NOTE 1).

1-316

Package Outline Drawings

8-Pin  DIP Package

.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

8        7         6        5

TYPE NUMBER DATE CODE

.35 ± 0.25 (0.250 ± 0.010)

HP XXXX

YYWW

1        2         3         4

UL RECOGNITION

1.19 (0.047) MAX.

1.78 (0.070) MAX.

4.70 (0.185) MAX.

5° TYP.

+ 0.076

0.254 - 0.051

+ 0.003) (0.010 - 0.002)

0.51 (0.020) MIN.

2.92 (0.115) MIN.

1.080 ± 0.320 (0.043 ± 0.013)

0.65 (0.025) MAX.

.54 ± 0.25 (0.100 ± 0.010)

DIMENSIONS IN MILLIMETERS AND (INCHES).

8-Pin  DIP Package with Gull Wing Surface Mount  Option  300

PAD LOCATION (FOR REFERENCE ONLY)

1.016 (0.040)

1.194 (0.047)

± 0.25

± 0.010)

4.826 TYP. (0.190)

9.398 (0.370)

9.906 (0.390)

| 1 2 3 4 9.65 ± 0.25 (0.380 ± 0.010) 8 7 6 5 |  |
| --- | --- |
|  | 6.350 (0.250 |

1.194 (0.047)

1.778 (0.070)

0.381 (0.015)

0.635 (0.025)

1.19 (0.047) MAX.

1.780 (0.070) MAX.

4.19  MAX. (0.165)

9.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

+ 0.076

0.254 - 0.051

+ 0.003) (0.010 - 0.002)

1.080 ± 0.320 (0.043 ± 0.013)

2.54 (0.100) BSC

0.635 ± 0.130 (0.025 ± 0.005)

0.635 ± 0.25 (0.025 ± 0.010)

12° NOM.

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES).

1-317

Solder Reflow Temperature Profile (Gull  Wing Surface Mount  Option  300  Parts)

260

240

220

200

TEMPERATURE – °C

180

160

140

120

100

80

60

40

20

0

T = 115°C, 0.3°C/SEC

T = 100°C, 1.5°C/SEC

T = 145°C, 1°C/SEC

0        1        2        3         4        5        6        7        8        9        10       11      12

TIME – MINUTES

Note: Use of nonchlorine activated fluxes is highly recommended.

Regulatory Information The HCPL-2602/2612 have been approved by the following organizations:

UL

Recognized  under  UL 1577, Component Recognition Program, File E55361.

CSA

Approved under  CSA Component

Acceptance Notice #5, File CA

88324.

Insulation and Safety Related Specifications

| Parameter | Symbol | Value | Units | Conditions |
| --- | --- | --- | --- | --- |
| Min. External Air Gap (External Clearance) | L(I01) | 7.1 | mm | Measured from input terminals to output terminals, shortest distance through air. |
| Min. External Tracking Path (External Creepage) | L(I02) | 7.4 | mm | Measured from input terminals to output terminals, shortest distance path along body. |
| Min. Internal Plastic Gap (Internal Clearance) |  | 0.08 | mm | Through insulation distance, conductor to conductor, usually the direct distance between the photoemitter and photodetector inside the optocoupler cavity. |
| Tracking Resistance (Comparative Tracking Index) | CTI | 200 | V | DIN IEC 112/VDE 0303 Part 1 |
| Isolation Group |  | IIIa |  | Material Group (DIN VDE 0110, 1/89, Table 1) |

Option 300 - surface  mount  classification is Class A in accordance with CECC 00802.

1-318

Absolute Maximum Ratings (No Derating  Required  up to 85 C)

| Parameter | Symbol | Min. | Max. | Units |
| --- | --- | --- | --- | --- |
| Storage Temperature | TS | -55 | 125 | C |
| Operating Temperature | TA | -40 | 85 | C |
| Forward Input Current | II |  | 60 | mA |
| Reverse Input Current | IIR |  | 60 | mA |
| Input Current, Pin 4 |  | -10 | 10 | mA |
| Supply Voltage (1 Minute Maximum) | VCC |  | 7 | V |
| Enable Input Voltage (Not to Exceed VCC by more than 500 mV) | VE |  | VCC + 0.5 | V |
| Output Collector Current | IO |  | 50 | mA |
| Output Collector Voltage (Selection for Higher Output Voltages up to 20 V is Available.) | VO |  | 7 | V |
| Output Collector Power Dissipation | PO |  | 40 | mW |
| Lead Solder Temperature | TLS | 260 C for 10 sec., 1.6 mm below seating plane |  |  |
| Solder Reflow Temperature Profile |  | See Package Outline Drawings section |  |  |

Recommended Operating Conditions

| Parameter | Symbol | Min. | Max. | Units |
| --- | --- | --- | --- | --- |
| Input Current, Low Level | IIL | 0 | 250 | A |
| Input Current, High Level | IIH | 5* | 60 | mA |
| Supply Voltage, Output | VCC | 4.5 | 5.5 | V |
| High Level Enable Voltage | VEH | 2.0 | VCC | V |
| Low Level Enable Voltage | VEL | 0 | 0.8 | V |
| Fan Out (@ RL = 1 k ) | N |  | 5 | TTL Loads |
| Output Pull-up Resistor | RL | 330 | 4 K |  |
| Operating Temperature | TA | 0 | 70 | C |

*The initial switching  threshold is 5 mA or less. It is recommended that an input current between  6.3 mA and 10 mA be used to obtain  best performance and to provide  at least

20% LED degradation guardband.

1-319

Electrical Characteristics

Over recommended temperature (TA = 0 C to +70 C) unless  otherwise specified. See note 1.

| Parameter | Sym. | Min. | Typ.* | Max. | Units | Test Conditions | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Level Output Current | IOH |  | 5.5 | 100 | A | VCC = 5.5 V, VO = 5.5 V, II = 250 A, VE = 2.0 V | 1 |  |
| Low Level Output Voltage | VOL |  | 0.35 | 0.6 | V | VCC = 5.5 V, II = 5 mA, VE = 2.0 V, IOL (Sinking) = 13 mA | 2, 4, 5, 14 |  |
| High Level Supply Current | ICCH |  | 7.5 | 10 | mA | VCC = 5.5 V, II = 0 mA, VE = 0.5 V |  |  |
| Low Level Supply Current | ICCL |  | 10 | 13 | mA | VCC = 5.5 V, II = 60 mA, VE = 0.5 V |  |  |
| High Level Enable Current | IEH |  | -0.7 | -1.6 | mA | VCC = 5.5 V, VE = 2.0 V |  |  |
| Low Level Enable Current | IEL |  | -0.9 | -1.6 | mA | VCC = 5.5 V, VE = 0.5 V |  |  |
| High Level Enable Voltage | VEH | 2.0 |  |  | V |  |  | 10 |
| Low Level Enable Voltage | VEL |  |  | 0.8 | V |  |  |  |
| Input Voltage | VI |  | 2.0 | 2.4 | V | II = 5 mA | 3 |  |
|  |  |  | 2.3 | 2.7 |  | II = 60 mA |  |  |
| Input Reverse Voltage | VR |  | 0.75 | 0.95 | V | IR = 5 mA |  |  |
| Input Capacitance | CIN |  | 90 |  | pF | VI = 0 V, f = 1 MHz |  |  |

*All typicals at VCC = 5 V, TA = 25 C.

1-320

Switching Specifications

Over recommended temperature (T

A

= 0 C to +70 C), V

CC

= 5 V, I = 7.5 mA, unless  otherwise specified.

I

| Parameter | Symbol | Device | Min. | Typ.* | Max. | Units | Test Conditions |  |  | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation Delay Time to High Output Level | tPLH |  | 20 | 48 | 75 | ns | TA = 25 C |  |  | 6, 7, 8 | 3 |
|  |  |  |  |  | 100 | ns |  |  |  |  |  |
| Propagation Delay Time to Low Output Level | tPHL |  | 25 | 50 | 75 | ns | TA = 25 C |  |  | 6, 7, 8 | 4 |
|  |  |  |  |  | 100 | ns | RL = 350 CL = 15 pF |  |  |  |  |
| Pulse Width Distortion | ¦tPHL-tPLH¦ |  |  | 3.5 | 35 | ns |  |  |  | 9 | 13 |
| Propagation Delay Skew | tPSK |  |  |  | 40 | ns |  |  |  |  | 12, 13 |
| Output Rise Time (10-90%) | tr |  |  | 24 |  | ns |  |  |  | 12 |  |
| Output Fall Time (90-10%) | tf |  |  | 10 |  | ns |  |  |  | 12 |  |
| Propagation Delay Time of Enable from VEH to VEL | tELH |  |  | 30 |  | ns | RL = 350 , CL = 15 pF, VEL = 0 V, VEH = 3 V |  |  | 10, 11 | 5 |
| Propagation Delay Time of Enable from VEL to VEH | tEHL |  |  | 20 |  | ns | RL = 350 , CL = 15 pF, VEL = 0 V, VEH = 3 V |  |  | 10, 11 | 6 |
| Common Mode Transient Immunity at High Output Level | ¦CMH¦ | HCPL-2602 | 1000 | 10,000 |  | V/ s | VCM = 50 V |  | VO(MIN) = 2 V, RL = 350 II = 0 mA, TA = 25 C | 13 | 7, 9, 10 |
|  |  | HCPL-2612 | 3500 | 15,000 |  |  | VCM = 300 V |  |  |  |  |
| Common Mode Transient Immunity at Low Output Level | ¦CML¦ | HCPL-2602 | 1000 | 10,000 |  | V/ s | VCM = 50 V |  | VO(MAX) = 0.8 V, RL = 350 II = 7.5 mA, TA = 25 C | 13 | 8, 9 10 |
|  |  | HCPL-2612 | 3500 | 15,000 |  |  | VCM = 300 V |  |  |  |  |

*All typicals at VCC = 5 V, TA = 25 C.

Package Characteristics

All Typicals at TA = 25 C

| Parameter | Sym. | Min. | Typ. | Max. | Units | Test Conditions | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input-Output Momentary Withstand Voltage* | VISO | 2500 |  |  | V rms | RH 50%, t = 1 min., TA = 25 C |  | 2, 11 |
| Input-Output Resistance | RI-O |  | 1012 |  |  | VI-O = 500 Vdc |  | 2 |
| Input-Output Capacitance | CI-O |  | 0.6 |  | pF | f = 1 MHz |  | 2 |

*The Input-Output Momentary  Withstand Voltage is a dielectric voltage  rating  that  should  not be interpreted as an input-output continuous voltage  rating.  For the continuous voltage  rating  refer  to the VDE 0884  Insulation Characteristics Table (if applicable), your equipment level safety specification or HP Application  Note 1074  entitled  “Optocoupler Input-Output Endurance Voltage.”

1-321

Notes:

1.  Bypassing  of the power supply line is required, with a 0.1   F ceramic  disc capacitor adjacent to each optocoupler as illustrated in

Figure  15. Total lead length  between  both ends of the capacitor and the isolator  pins should not exceed  20 mm.

2.  Device considered a two terminal  device: pins 1, 2, 3, and 4 shorted together, and pins 5, 6, 7, and 8 shorted together.

3.  The tPLH  propagation delay is measured from the 3.75 mA point on the falling edge of the input pulse to the 1.5 V point on the rising edge of the output pulse.

4.  The tPHL  propagation delay is measured from the 3.75 mA point on the rising edge of the input pulse to the 1.5 V point on the falling edge of the output pulse.

5.  The tELH  enable  propagation delay is measured from the 1.5 V point on the falling edge of the enable  input pulse to the 1.5 V point on the rising edge of the output pulse.

6.  The tEHL  enable  propagation delay is measured from the 1.5 V point on the rising edge of the enable  input pulse to the 1.5 V point on the falling edge of the output pulse.

7. CMH is the maximum  tolerable rate of rise of the common  mode voltage to assure  that the output will remain  in a high logic state

(i.e., VOUT > 2.0 V).

8. CML is the maximum  tolerable rate of fall of the common  mode voltage to assure  that the output will remain  in a low logic state  (i.e.,

VOUT < 0.8 V).

9. For sinusoidal voltages,

|dvCM|

––––––             =  fCMVCM (p-p)

dt     max

10. No external pull up is required for a high logic state  on the enable  input.  If the VE pin is not used, tying VE to VCC will result  in improved CMR performance.

11. In accordance with UL 1577,  each optocoupler is proof tested  by applying  an insulation test voltage of     3000  for one second

(leakage detection current limit, Ii-o      5   A).

12. tPSK is equal to the worst case difference in tPHL  and/or  tPLH  that will be seen between  units at any given temperature within the

operating condition range.

13. See application section  titled “Propagation Delay, Pulse-Width  Distortion and Propagation Delay Skew” for more information.

IOH – HIGH LEVEL OUTPUT CURRENT – µA

15

VCC = 5.5 V VO = 5.5 V VE = 2 V

II = 250 µA

10

VOL – LOW LEVEL OUTPUT VOLTAGE – V

| 0.5 |  |  |  |  | VCC = 5.5 V VE = 2 V II = 5 mA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | IO = 12.8 mA |  |  |  |  |  |
|  |  |  |  |  | IO = 16 mA |  |  |
|  |  |  |  |  | IO = 6.4 mA |  |  |
|  |  | IO = 9.6 mA |  |  |  |  |  |

0.4

| 2.6 |  |  |  | 0°C 25°C |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 70°C |  |

2.4

2.2

2.0

0.3

5

0.2

1.8

1.6

1.4

0

-60  -40

-20    0

20    40

60    80  100

0.1

-60  -40

-20    0

20    40

60    80

100

VI – INPUT VOLTAGE – V

1.2

1.0

0

10      20      30      40

50      60

TA – TEMPERATURE – °C

TA – TEMPERATURE – °C

II – INPUT CURRENT – mA

Figure 1. Typical  High Level  Output

Current vs. Temperature.

Figure 2. Typical  Low Level  Output

Voltage vs. Temperature.

Figure 3. Typical  Input  Characteristics.

6

VO – OUTPUT VOLTAGE – V

5

4

RL = 350

3

| 70 VCC = 5 V VE = 2 V VOL = 0.6 V |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | II = 1 | 0-15 | mA |
|  |  |  |  |  | II = 5.0 mA |  |  |

VCC = 5 V

TA = 25 °C

60

50

RL = 1 K

2

IOL – LOW LEVEL OUTPUT CURRENT – mA

RL = 4 K                                    40

1

0

0        1         2        3        4        5        6

20

-60  -40

-20    0

20    40

60   80

100

IF – FORWARD INPUT CURRENT – mA

TA – TEMPERATURE – °C

Figure 4. Typical  Output Voltage vs. Forward Input  Current.

Figure 5. Typical  Low Level  Output

Current vs. Temperature.

1-322

PULSE GEN.     I I             1

ZO = 50

t f = tr = 5 ns

VCC  8

0.1µF

+5 V

2

INPUT MONITORING                  3

NODE

RM           4

7

6

GND  5

BYPASS

*CL

RL

OUTPUT VO MONITORING NODE

100

tP – PROPAGATION DELAY – ns

80

60

VCC = 5 V II = 7.5 mA

tPHL , RL = 350

1 K

4 K

tPLH , RL = 4 K

*CL IS APPROXIMATELY 15 pF WHICH INCLUDES PROBE AND STRAY WIRING CAPACITANCE.

INPUT I I

tPHL                              tPLH

I I  = 7.50 mA I I  = 3.75 mA

40                           tPLH , RL = 1 K

tPLH , RL = 350

20

0

OUTPUT

-60

-40

-20    0

20   40

60    80

100

VO                                                                                             1.5 V

TA – TEMPERATURE – °C

Figure 6. Test  Circuit for tPHL and tPLH.

Figure 7. Typical  Propagation Delay vs. Temperature.

105

90

VCC = 5 V TA = 25°C

| 40 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RL = 4 k |  |  |  |  |  |  |
|  |  |  |  |  | VCC = 5 V II = 7.5 mA |  |  |
|  | RL = 350 k |  |  |  |  |  |  |
|  |  |  | RL = 1 k |  |  |  |  |

tPLH , RL = 4 K                            30

75                                                                                        20

tPLH , RL = 350

PWD – PULSE WIDTH DISTORTION – ns

60                                                                                        10

tPLH , RL = 1 K

45

tPHL , RL = 350

1 K

tP – PROPAGATION DELAY – ns

30                        4 K

0

-10

5           7           9

11        13         15

-60

-40

-20    0

20   40

60   80

100

II – PULSE INPUT CURRENT – mA

TA – TEMPERATURE – °C

Figure 8. Typical  Propagation Delay vs. Pulse Input  Current.

Figure 9. Typical  Pulse Width

Distortion vs. Temperature.

PULSE GEN. ZO = 50

tf = t r = 5 ns

INPUT  VE

MONITORING NODE

1

7.5 mA

VCC   8

0.1 µF

+5 V

R

I I                  2

7            BYPASS

L

OUTPUT VO

120

V

= 5 V

3                                             6

4                                   GND 5

*CL

MONITORING NODE

CC

VEH = 3 V

VEL = 0 V

90   II = 7.5 mA

tELH, RL = 4 k

*CL IS APPROXIMATELY 15 pF WHICH INCLUDES PROBE AND STRAY WIRING CAPACITANCE.

INPUT VE

tEHL                          t ELH

3.0 V

1.5 V

60

tELH, RL = 1 k

30

tELH, RL = 350

tE – ENABLE PROPAGATION DELAY – ns

tEHL, RL = 350     1 k    4 k

0

OUTPUT

-60  -40

-20    0

20    40

60    80  100

VO                                                                                     1.5 V

TA – TEMPERATURE – °C

Figure 10.  Test  Circuit for tEHL and tELH.

Figure 11.  Typical  Enable Propagation

Delay  vs. Temperature.

1-323

300

290

60

40

tr, tf  – RISE, FALL TIME – ns

VCC = 5 V II = 7.5 mA

RL = 4 k

RL = 1 k

I I

tRISE

tFALL                                         B

A

1                                    VCC  8

2                                             7

3                                             6

4                                   GND  5

VCM

+        –

0.1 µF BYPASS

350

+5 V

OUTPUT VO MONITORING NODE

20                     RL = 350

0                     RL = 350     1 k  , 4 k

PULSE GENERATOR

Z O = 50

-60  -40

-20    0

20    40

60    80  100

VCM (PEAK)

TA – TEMPERATURE – °C

Figure 12.  Typical  Rise  and Fall Time vs. Temperature.

VCM

VO

0 V

SWITCH AT A:  II  = 0 mA

5 V

VO (MIN.) SWITCH AT B:  II  = 7.5 mA

VO (MAX.)

CMH

VO 0.5 V

CML

Figure 13.  Test  Circuit for Common Mode Transient Immunity and Typical

Waveforms.

GND BUS (BACK)

5

4

3

RL = 350

2

RL = 1 k

VCC = 5.0 V VO = 0.6 V

VCC BUS (FRONT) NC

NC

0.1µF

ENABLE (IF USED)

OUTPUT 1

1          RL = 4 k                                                                                    NC

0

ITH – INPUT THRESHOLD CURRENT – mA

-60  -40

-20    0

20    40

60    80  100

0.1µF

ENABLE (IF USED)

TA – TEMPERATURE – °C

NC

OUTPUT 2

Figure 14.  Typical  Input  Threshold

Current vs. Temperature.

10 mm MAX. (SEE NOTE 1)

Figure 15.  Recommended Printed Circuit Board  Layout.

1-324

Using  the  HCPL-2602/12

Line Receiver

Optocouplers

The primary  objectives  to fulfill when connecting an optocoupler to a transmission line are to provide  a minimum,  but not excessive, LED current and to properly terminate the line. The internal regulator in the HCPL-

2602/12 simplifies  this task. Excess current from variable  drive conditions such as line length variations, line driver differences, and power supply fluctuations are shunted by the regulator. In fact, with the LED current regulated,

the line current can be increased to improve  the immunity  of the system  to differential-mode-noise and to enhance the data rate capability. The designer must keep in mind the 60 mA input current maximum  rating  of the

HCPL-2602/12 in such cases,  and may need to use series  limiting or shunting to prevent  overstress.

Design of the termination circuit is also simplified;  in most cases the transmission line can simply be connected directly to the input terminals of the HCPL-2602/12 without  the need for additional series  or shunt  resistors. If reversing line drive is used it may be desirable to use two HCPL-

termination,” but they do not interfere with circuit  performance because the regulator clamps  the line voltage.  At longer  line

lengths, t     increases faster  than t      since the switching  threshold is not exactly halfway between asymptotic line conditions. If optimum  data rate is desired, a series  resistor and peaking capacitor can be used to equalize t      and t    . In general, the peaking  capacitance should be as large as possible; however,  if it is

PLH

PHL

PLH                  PHL

too large it may keep the regulator from achieving  turn-off during  the negative  (or zero) excursions of the input signal. A safe rule:

make C    16t where:

C = peaking  capacitance in

picofarads

t = data bit interval in nanoseconds

Polarity Reversing Drive A single HCPL-2602/12 can also be used with polarity  reversing drive (Figure  b). Current reversal is obtained by way of the

substrate isolation  diode

(substrate to collector). Some reduction of data rate occurs, however,  because the substrate diode stores charge, which must be removed  when the current

then connected in ANTI-SERIES; however,  because of the higher steady-state termination voltage, in comparison to the single

HCPL-2602/12 termination, the forward  current in the substrate diode is lower and consequently there  is less junction  charge  to deal with when switching.

Closing switch B with A open is done mainly to enhance common mode rejection, but also reduces propagation delay slightly because line-to-line  capacitance offers a slight peaking  effect. With switches  A and B both CLOSED, the shield acts as a current return path  which prevents either  input substrate diode from becoming reversed biased.  Thus the data rate is

optimized  as shown in Figure  (c).

Improved Noise Rejection Use of additional logic at the output of two HCPL-2602/12s, operated in the split phase termination, will greatly  improve system  noise rejection in addition to balancing propagation delays

as discussed earlier.

A NAND flip-flop offers infinite common  mode rejection (CMR) for NEGATIVELY sloped  common mode transients but requires t

> t

2602/12 or an external Schottky

changes to the forward  direction.

PLH

for proper operation. A

PHL

diode to optimize  data rate.

The effect of this is a longer  t    .

PHL

This effect can be eliminated and

NOR flip-flop has infinite CMR for

POSITIVELY sloped  transients

Polarity Non-Reversing

data rate improved considerably

but requires t

PHL

PLH

< t

for proper

Drive

High data rates  can be obtained with the HCPL-2602/12 with polarity  non-reversing drive. Figure  (a) illustrates how a

74S140 line driver can be used with the HCPL-2602/12 and shielded,  twisted  pair or coax cable without  any additional components. There are some reflections due to the “active

by use of a Schottky  diode on the

input of the HCPL-2602/12.

For optimum  noise rejection as well as balanced delays, a split- phase  termination should be used along with a flip-flop at the

output (Figure  c). The result  of current reversal  in split-phase operation is seen in Figure  (c) with switches  A and B both OPEN. The coupler  inputs  are

operation. An exclusive-OR flip- flop has infinite CMR for common mode transients of EITHER polarity  and operates with either

PHL                PLH             PHL                PLH

t      > t     or t      < t    .

With the line driver and transmission line shown in Figure (c), t     > t    , so NAND gates  are preferred in the R-S flip-flop. A higher  drive amplitude or

PHL             PLH

1-325

Figure a. Polarity Non-Reversing.

Figure b. Polarity Reversing, Single Ended.

< 1                                 < 1

Figure c. Polarity Reversing, Split  Phase.

Figure d. Flip-Flop Configurations.

1-326

different  circuit  configuration

and often determines the

As mentioned earlier,  t

PSK

can

could make t

< t   , in which

maximum  data rate capability  of a

determine the maximum  parallel

PHL

PLH

case NOR gates  would be pre-

ferred.  If it is not known whether

transmission system.  PWD can be

expressed in percent by dividing

data transmission rate.  Figure  17

is the timing diagram  of a typical

PHL

t

> t

PLH

or t

PHL

PLH

< t

, or if the

the PWD (in ns) by the minimum

parallel  data application with both

drive conditions may vary over

the boundary for these  conditions, the exclusive-OR flip-flop of

Figure  (d) should be used.

RS-422  and RS-423

Line drivers  designed for RS-422 and RS-423 generally  provide adequate voltage and current for operating the HCPL-2602/12. Most drivers  also have characteristics allowing the HCPL-2602/12 to be connected directly to the driver terminals. Worst case drive conditions, however,  would require current

shunting to prevent  overstress of the HCPL-2602/12.

Propagation Delay, Pulse- Width  Distortion and Propagation Delay  Skew Propagation delay is a figure of merit which describes how

quickly a logic signal propagates through a system.  The propaga- tion delay from low to high (t    )

PLH

is the amount  of time required for

an input signal to propagate to the output, causing  the output to change  from low to high. Similarly, the propagation delay

pulse width (in ns) being

transmitted. Typically, PWD on the order  of 20-30% of the minimum  pulse width is tolerable; the exact  figure depends on the particular application (RS232, RS422, T-1, etc.).

Propagation delay skew, t    , is an important parameter to consider

PSK

in parallel  data applications where synchronization of signals  on parallel  data lines is a concern. If the parallel  data is being sent through a group  of optocouplers, differences in propagation delays will cause  the data to arrive at the outputs of the optocouplers at different  times. If this difference

in propagation delays is large enough, it will determine the maximum  rate at which parallel data can be sent through the optocouplers.

Propagation delay skew is defined as the difference between  the minimum  and maximum propagation delays, either  t     or

PLH

t    , for any given group  of optocouplers which are operating under  the same conditions (i.e.,

PHL

the clock and the data lines being

sent through optocouplers. The figure shows data and clock signals  at the inputs  and outputs

of the optocouplers. To obtain  the maximum  data transmission rate, both edges  of the clock signal are being used to clock the data; if only one edge were used, the

clock signal would need to be twice as fast.

Propagation delay skew represents the uncertainty of where an edge might be after being sent through an optocoupler. Figure  17 shows that there  will be uncertainty in both the data and the clock lines. It is important that these  two areas of uncertainty not overlap, otherwise the clock signal might arrive before  all of the data outputs have settled, or some of the data outputs may start  to change  before  the clock signal has arrived.  From these considerations, the absolute minimum  pulse width that can be sent through optocouplers in a

parallel  application is twice t    . A

PSK

cautious design  should use a

from high to low (t

) is the

the same drive current, supply

slightly longer  pulse width to

PHL

amount  of time required for the

input signal to propagate to the output, causing  the output to change  from high to low (see Figure  6).

voltage,  output load, and operating temperature). As illustrated in Figure  16, if the inputs  of a group  of optocouplers are switched  either  ON or OFF at

ensure  that any additional uncertainty in the rest of the circuit  does not cause  a problem.

The t     specified  optocouplers

PSK

the same time, t

PSK

is the

offer the advantages of

Pulse-width  distortion (PWD)

difference between  the shortest

guaranteed specifications for

results  when t

PLH

and t

PHL

differ in

propagation delay, either  t     or

PHL

propagation delays, pulse-width

value. PWD is defined as the

t    , and the longest  propagation

PHL

distortion and propagation delay

difference between  t

PLH

and t

PHL

delay, either  t     or t    .

PLH             PHL

skew over the recommended

temperature, input current, and power supply ranges.

1-327

DATA

I I                            50%

VO

1.5 V

INPUTS CLOCK

I I                            50%

VO                                1.5 V

t PSK

DATA

OUTPUTS CLOCK

t PSK

t PSK

Figure 16.  Illustration of Propagation Delay  Skew - tPSK.

Figure 17.  Parallel Data  Transmission Example.

1-328