H

2.0 Amp Output Current IGBT Gate Drive  Optocoupler

Technical  Data

HCPL-3120

Features

• 2.0  A Minimum  Peak  Output

Current

• 15 kV/  s Minimum  Common Mode Rejection (CMR) at VCM = 1500 V

• 0.5  V Maximum Low Level Output Voltage (VOL) Eliminates Need for Negative Gate  Drive

• ICC = 5 mA Maximum Supply

Current

• Under  Voltage Lock-Out Protection (UVLO) with Hysteresis

• Wide Operating VCC Range:

15 to 30 Volts

• 500  ns Maximum Switching

• Industrial Inverters

• Switch Mode  Power Supplies

(SMPS)

Description

The HCPL-3120  consists of a GaAsP LED optically coupled  to an integrated circuit  with a power output stage.  This optocoupler is ideally suited  for driving power IGBTs and MOSFETs used in

Functional Diagram

N/C  1

motor  control  inverter applica- tions.  The high operating voltage range  of the output stage  pro- vides the drive voltages  required by gate controlled devices.  The voltage and current supplied by this optocoupler makes  it ideally suited  for directly  driving IGBTs with ratings  up to 1200  V/100 A. For IGBTs with higher  ratings, the HCPL-3120  can be used to drive a discrete power stage which drives the IGBT gate.

8    VCC

Speeds

• Industrial Temperature

Range: -40 C to 100  C

• Safety Approval

UL Recognized  - 2500  V rms for 1 minute  per UL1577

CSA Approval

VDE 0884  Approved with VIORM = 630 V peak (Option  060 only)

ANODE     2

CATHODE     3

N/C    4

| TRUTH TABLE LED | VCC - VEE “POSITIVE GOING” (i.e., TURN-ON) | VCC - VEE “NEGATIVE GOING” (i.e., TURN-OFF) | VO |
| --- | --- | --- | --- |
| OFF | 0 - 30 V | 0 - 30 V | LOW |
| ON | 0 - 11 V | 0 - 9.5 V | LOW |
| ON | 11 - 13.5 V | 9.5 - 12 V | TRANSITION |
| ON | 13.5 - 30 V | 12 - 30 V | HIGH |

SHIELD

7    VO

6    VO

5    VEE

Applications

• Isolated IGBT/MOSFET Gate  Drive

• AC and Brushless DC Motor

Drives

A 0.1    F bypass capacitor must be connected between pins 5 and 8.

CAUTION:   It is advised that  normal static precautions be taken in handling and assembly of this  component to prevent damage and/or degradation which may be induced by ESD.

1-182                                                                                                                                                                    5965-4779E

Ordering Information

Specify Part  Number  followed by Option Number  (if desired)

Example

HCPL-3120#XXX

No Option = Standard DIP Package, 50 per tube.

060 = VDE 0884  VIORM = 630 V peak  Option,  50 per tube.

300 = Gull Wing Surface  Mount Option,  50 per tube.

500 = Tape and Reel Packaging Option,  1000  per reel.

Option data sheets  available.  Contact  Hewlett-Packard sales representative or authorized distributor.

Package Outline Drawings

Standard DIP Package

9.40 (0.370)

9.90 (0.390)

8        7         6         5

0.20 (0.008)

HP 3120Z YYWW

OPTION CODE* DATE CODE

6.10 (0.240)

6.60 (0.260)

7.36 (0.290)

7.88 (0.310)

0.33 (0.013)

5° TYP.

PIN ONE      1        2         3        4

1.19 (0.047) MAX.

1.78 (0.070) MAX.

PIN ONE

4.70 (0.185) MAX.

DIMENSIONS IN MILLIMETERS AND (INCHES).

*MARKING CODE LETTER FOR OPTION NUMBERS.

0.51 (0.020) MIN.

"V" =

1   VDD1

VDD2   8

2.92 (0.115) MIN.

OPTION 060

OPTION NUMBERS 300 AND 500 NOT MARKED.

0.76 (0.030)

1.40 (0.055)

0.65 (0.025) MAX.

2.28 (0.090)

2.80 (0.110)

2   VIN+

3   VIN–

4   GND1

VOUT+   7

VOUT–   6

GND2   5

Gull Wing Surface Mount  Option  300

9.65 ± 0.25 (0.380 ± 0.010)

PAD LOCATION (FOR REFERENCE ONLY)

1.016 (0.040)

1.194 (0.047)

8         7          6         5

4.826 TYP. (0.190)

HP 3120Z

YYWW

6.350 ± 0.25                                                       (0.250 ± 0.010)

9.398 (0.370)

9.906 (0.390)

1         2          3         4

MOLDED

1.194 (0.047)

1.778 (0.070)

0.381 (0.015)

0.635 (0.025)

1.19 (0.047) MAX.

1.780 (0.070) MAX.

4.19  MAX. (0.165)

9.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

0.20 (0.008)

0.33 (0.013)

1.080 ± 0.320 (0.043 ± 0.013)

2.540 (0.100) BSC

0.635 ± 0.130 (0.025 ± 0.005)

0.635 ± 0.25 (0.025 ± 0.010)

12° NOM.

DIMENSIONS IN MILLIMETERS (INCHES).

TOLERANCES (UNLESS OTHERWISE SPECIFIED): xx.xx = 0.01 xx.xxx = 0.005

LEAD COPLANARITY MAXIMUM: 0.102 (0.004)

1-183

Reflow Temperature Profile                                                  Regulatory Information

The HCPL-3120  has been

260

240

220

200

TEMPERATURE – °C

180

160

140

120

100

80

60

40

20

0

T = 115°C, 0.3°C/SEC

T = 100°C, 1.5°C/SEC

T = 145°C, 1°C/SEC

approved by the following organizations:

UL

Recognized  under  UL 1577, Component Recognition Program, File E55361.

CSA

Approved under  CSA Component

0        1        2        3         4        5        6        7        8        9        10       11      12

TIME – MINUTES

MAXIMUM SOLDER REFLOW THERMAL PROFILE

(NOTE: USE OF NON-CHLORINE ACTIVATED FLUXES IS RECOMMENDED.)

Acceptance Notice #5, File CA

88324.

VDE (Option 060  Only)

Approved under  VDE 0884/06.92 with VIORM = 630 V peak.

VDE 0884 Insulation Characteristics (Option 060  Only)

| Description | Symbol | Characteristic | Unit |
| --- | --- | --- | --- |
| Installation classification per DIN VDE 0110/1.89, Table 1 for rated mains voltage 300 V rms for rated mains voltage 450 V rms |  | I-IV I-III |  |
| Climatic Classification |  | 55/100/21 |  |
| Pollution Degree (DIN VDE 0110/1.89) |  | 2 |  |
| Maximum Working Insulation Voltage | VIORM | 630 | Vpeak |
| Input to Output Test Voltage, Method b* VIORM x 1.875 = VPR, 100% Production Test with tm = 1 sec, Partial discharge <5 pC | VPR | 1181 | Vpeak |
| Input to Output Test Voltage, Method a* VIORM x 1.5 = VPR, Type and Sample Test, tm = 60 sec, Partial discharge <5 pC | VPR | 945 | Vpeak |
| Highest Allowable Overvoltage* (Transient Overvoltage tini = 10 sec) | VIOTM | 6000 | Vpeak |
| Safety Limiting Values–Maximum Values Allowed in the Event of a Failure, Also See Figure 37, Thermal Derating Curve. Case Temperature Input Current Output Power | TS IS, INPUT PS, OUTPUT | 175 230 600 | C mA mW |
| Insulation Resistance at TS, VIO = 500 V | RS | 109 |  |

*Refer to the front of the optocoupler section  of the current catalog,  under  Product Safety Regulations section, (VDE 0884) for a detailed  description of Method a and Method b partial  discharge test profiles.

Note: Isolation  characteristics are guaranteed only within the safety maximum  ratings  which must be ensured by protective circuits  in application.

1-184

Insulation and Safety Related Specifications

| Parameter | Symbol | Value | Units | Conditions |
| --- | --- | --- | --- | --- |
| Minimum External Air Gap (External Clearance) | L(101) | 7.1 | mm | Measured from input terminals to output terminals, shortest distance through air. |
| Minimum External Tracking (External Creepage) | L(102) | 7.4 | mm | Measured from input terminals to output terminals, shortest distance path along body. |
| Minimum Internal Plastic Gap (Internal Clearance) |  | 0.08 | mm | Insulation thickness between emitter and detector; also known as distance through insulation. |
| Tracking Resistance (Comparative Tracking Index) | CTI | 200 | Volts | DIN IEC 112/VDE 0303 Part 1 |
| Isolation Group |  | IIIa |  | Material Group (DIN VDE 0110, 1/89, Table 1) |

Option 300 - surface  mount  classification is Class A in accordance with CECC 00802.

Absolute Maximum Ratings

| Parameter | Symbol | Min. | Max. | Units | Note |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -55. | 125 | C |  |
| Operating Temperature | TA | -40 | 100 | C |  |
| Average Input Current | IF(AVG) |  | 25 | mA | 1 |
| Peak Transient Input Current (<1 s pulse width, 300 pps) | IF(TRAN) |  | 1.0 | A |  |
| Reverse Input Voltage | VR |  | 5 | Volts |  |
| “High” Peak Output Current | IOH(PEAK) |  | 2.5 | A | 2 |
| “Low” Peak Output Current | IOL(PEAK) |  | 2.5 | A | 2 |
| Supply Voltage | (VCC - VEE) | 0 | 35 | Volts |  |
| Output Voltage | VO | 0 | VCC | Volts |  |
| Output Power Dissipation | PO |  | 250 | mW | 3 |
| Total Power Dissipation | PT |  | 295 | mW | 4 |
| Lead Solder Temperature | 260 C for 10 sec., 1.6 mm below seating plane |  |  |  |  |
| Solder Reflow Temperature Profile | See Package Outline Drawings section |  |  |  |  |

Recommended Operating Conditions

| Parameter | Symbol | Min. | Max. | Units |
| --- | --- | --- | --- | --- |
| Power Supply Voltage | (VCC - VEE) | 15 | 30 | Volts |
| Input Current (ON) | IF(ON) | 7 | 16 | mA |
| Input Voltage (OFF) | VF(OFF) | -3.0 | 0.8 | V |
| Operating Temperature | TA | -40 | 100 | C |

1-185

Electrical Specifications (DC)

Over recommended operating conditions (TA = - 40 to 100  C, IF(ON)  = 7 to 16 mA, VF(OFF)  = -3.0 to 0.8 V, VCC = 15 to 30 V, VEE = Ground) unless  otherwise specified.

Parameter           Symbol      Min.        Typ.*      Max.   Units     Test  Conditions       Fig.      Note

High Level                         IOH                  0.5            1.5                         A       VO = (VCC - 4 V)             2, 3,         5

Output  Current

2.0                                           A       VO = (VCC - 15 V)            17           2

Low Level                           IOL                  0.5            2.0                         A       VO = (VEE + 2.5 V)       5, 6,         5

Output  Current

2.0                                           A       VO = (VEE + 15V)            18           2

High Level Output            VOH           (VCC - 4)    (VCC - 3)                    V       IO = -100 mA                 1, 3,       6, 7

Voltage                                                                                                                                                         19

Low Level Output              VOL                                           0.1          0.5         V       IO = 100 mA                  4, 6, Voltage                                                                                                                                                         20

High Level Supply            ICCH                                          2.0          5.0        mA      Output  Open,                   7, 8

Current                                                                                                   IF = 7 to 16 mA

Low Level Supply             ICCL                                          2.0          5.0        mA      Output  Open, Current                                                                                                             VF  = -3.0 to +0.8 V Threshold Input                IFLH                                           2.3          5.0        mA      IO = 0 mA,                    9, 15,

Current Low to High                                                                                      VO > 5 V                           21

Threshold Input

Voltage High to Low       VFHL                0.8                                           V

Input  Forward                  VF                   1.2            1.5          1.8         V       IF = 10 mA                      16

Voltage

Temperature                  VF /  TA                                    -1.6                    mV/ C   IF = 10 mA Coefficient

of Forward Voltage

Input  Reverse                    BVR                   5                                            V       Ir = 10   A Breakdown Voltage

Input  Capacitance           CIN                                            60                        pF       f = 1 MHz, VF = 0 V

UVLO Threshold           VUVLO+           11.0          12.3        13.5        V       VO > 5 V, IF = 10 mA    22, 36

VUVLO–             9.5           10.7        12.0

UVLO Hysteresis          UVLOHYS                                   1.6

* All typical values at TA = 25 C and VCC - VEE = 30 V, unless  otherwise noted.

1-186

Switching Specifications (AC)

Over recommended operating conditions (TA = - 40 to 100  C, IF(ON)  = 7 to 16 mA, VF(OFF)  = -3.0 to 0.8 V, VCC = 15 to 30 V, VEE = Ground) unless  otherwise specified.

| Parameter | Symbol | Min. | Typ.* | Max. | Units | Test Conditions | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation Delay Time to High Output Level | tPLH | 0.10 | 0.30 | 0.50 | s | Rg = 10 , Cg = 10 nF, f = 10 kHz, Duty Cycle = 50% | 10, 11, 12, 13 14, 23 | 14 |
| Propagation Delay Time to Low Output Level | tPHL | 0.10 | 0.27 | 0.50 | s |  |  |  |
| Pulse Width Distortion | PWD |  |  | 0.3 | s |  |  | 15 |
| Propagation Delay Difference Between Any Two Parts | (tPHL - tPLH) PDD | -0.35 |  | 0.35 | s |  | 34,35 | 10 |
| Rise Time | tr |  | 0.1 |  | s |  | 23 |  |
| Fall Time | tf |  | 0.1 |  | s |  |  |  |
| UVLO Turn On Delay | tUVLO ON |  | 0.8 |  | s | VO > 5 V, IF = 10 mA | 22 |  |
| UVLO Turn Off Delay | tUVLO OFF |  | 0.6 |  |  | VO < 5 V, IF = 10 mA |  |  |
| Output High Level Common Mode Transient Immunity | ¦CMH¦ | 15 | 30 |  | kV/ s | TA = 25 C, IF = 10 to 16 mA, VCM = 1500 V, VCC = 30 V | 24 | 11, 12 |
| Output Low Level Common Mode Transient Immunity | ¦CML¦ | 15 | 30 |  | kV/ s | TA = 25 C, VCM = 1500 V, VF = 0 V, VCC = 30 V |  | 11, 13 |

*All typical values at TA = 25 C and VCC - VEE = 30 V, unless  otherwise noted.

Package Characteristics

| Parameter | Symbol | Min. | Typ. | Max. | Units | Test Conditions | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input-Output Momentary Withstand Voltage** | VISO | 2500 |  |  | VRMS | RH < 50%, t = 1 min., TA = 25 C |  | 8, 9 |
| Resistance (Input - Output) | RI-O |  | 1012 |  |  | VI-O = 500 VDC |  | 9 |
| Capacitance (Input - Output) | CI-O |  | 0.6 |  | pF | f = 1 MHz |  |  |
| LED-to-Case Thermal Resistance | LC |  | 467 |  | C/W | Thermocoupler located at center underside of package | 28 |  |
| LED-to-Detector Thermal Resistance | LD |  | 442 |  | C/W |  |  |  |
| Detector-to-Case Thermal Resistance | DC |  | 126 |  | C/W |  |  |  |

**The Input-Output Momentary  Withstand Voltage is a dielectric voltage  rating  that  should  not be interpreted as an input-output continuous voltage  rating.  For the continuous voltage  rating  refer  to your equipment level safety specification or HP Application  Note

1074  entitled  “Optocoupler Input-Output Endurance Voltage.”

1-187

Notes:

1. Derate  linearly above 70 C free-air temperature at a rate  of 0.3 mA/ C.

2. Maximum pulse width = 10    s, maximum  duty cycle = 0.2%. This value is intended to allow for component tolerances for designs with IO peak minimum  = 2.0 A. See

Applications section  for additional

details  on limiting IOH peak.

3. Derate  linearly above 70 C free-air

temperature at a rate  of 4.8 mW/ C.

4. Derate  linearly above 70 C free-air temperature at a rate  of 5.4 mW/ C. The maximum  LED junction  tempera- ture  should  not exceed  125  C.

5. Maximum pulse width = 50    s, maximum  duty cycle = 0.5%.

6. In this test  VOH is measured with a dc load current. When driving capacitive

loads VOH will approach VCC as IOH

approaches zero amps.

7. Maximum pulse width = 1 ms, maximum  duty cycle = 20%.

8. In accordance with UL1577,  each

optocoupler is proof  tested  by applying  an insulation test voltage

3000  Vrms for 1 second  (leakage

detection current limit, II-O       5   A). This test is performed before  the

100% production test for partial discharge (method b) shown in the VDE 0884  Insulation Characteristic Table, if applicable.

9. Device considered a two-terminal device: pins 1, 2, 3, and 4 shorted together and pins 5, 6, 7, and 8 shorted together.

10. The difference between  tPHL  and tPLH between  any two HCPL-3120  parts under  the same test condition.

11. Pins 1 and 4 need to be connected to

LED common.

12. Common  mode transient immunity  in the high state  is the maximum tolerable dVCM /dt of the common

mode pulse,  VCM, to assure  that  the

output will remain  in the high state

(i.e.,  VO > 15.0 V).

13. Common  mode transient immunity  in

a low state  is the maximum  tolerable dVCM/dt of the common  mode pulse, VCM, to assure  that  the output will

remain  in a low state (i.e., VO < 1.0 V).

14. This load condition approximates the

gate load of a 1200V/75A IGBT.

15. Pulse Width Distortion (PWD) is defined  as |tPHL-tPLH| for any given device.

(VOH – VCC ) – HIGH OUTPUT VOLTAGE DROP – V

| 0 | IF = 7 to 16 mA IOUT = -100 mA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VCC = 15 to 30 V |  |  |  |  |  |

-1           VEE = 0 V

-2

| 2.0 | IF = 7 to 16 mA VOUT = (VCC - 4 V) VCC = 15 to 30 V |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VEE = 0 V |  |  |  |  |  |  |

IOH – OUTPUT HIGH CURRENT – A

1.8

1.6

1.4

(VOH – VCC ) – OUTPUT HIGH VOLTAGE DROP – V

-1

100 °C

-2                                       25 °C

-40 °C

-3

-4

-3

1.2

IF = 7 to 16 mA

-5       VCC = 15 to 30 V

VEE = 0 V

-4

-40  -20

0      20     40

60     80

100

1.0

-40  -20

0      20     40

60     80

100

-6

0         0.5

1.0       1.5

2.0

2.5

TA – TEMPERATURE – °C

TA – TEMPERATURE – °C

IOH – OUTPUT HIGH CURRENT – A

Figure 1. VOH vs. Temperature.                    Figure 2. IOH vs. Temperature.                    Figure 3. VOH vs. IOH.

| 0.25 | VF(OFF) = -3.0 to 0.8 V IOUT = 100 mA VCC = 15 to 30 V |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VEE = 0 V |  |  |  |  |  |  |

0.20

0.15

VOL – OUTPUT LOW VOLTAGE – V

0.10

| 4 | VF(OFF) = -3.0 to 0.8 V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VOUT = 2.5 V VCC = 15 to 30 V |  |  |  |  |  |

3           VEE = 0 V

2

VOL – OUTPUT LOW VOLTAGE – V

4

VF(OFF) = -3.0 to 0.8 V VCC = 15 to 30 V

VEE = 0 V

3

2

0.05

0

-40  -20

0      20     40

60     80

IOL  – OUTPUT LOW CURRENT – A

100

1

0

-40  -20

0      20     40

60     80

100

1

0

0         0.5

1.0       1.5

100 °C

25 °C

-40 °C

2.0

2.5

TA – TEMPERATURE – °C

TA – TEMPERATURE – °C

IOL – OUTPUT LOW CURRENT – A

Figure 4. VOL vs. Temperature.                     Figure 5. IOL vs. Temperature.                     Figure 6. VOL vs. IOL.

1-188

3.5

ICC  – SUPPLY CURRENT – mA

3.0

2.5

2.0

1.5

VCC = 30 V VEE = 0 V

IF = 10 mA for ICCH

IF = 0 mA for ICCL

ICCH

ICCL

3.5

ICC  – SUPPLY CURRENT – mA

3.0

2.5

2.0

1.5

IF = 10 mA for ICCH IF = 0 mA for ICCL TA = 25 °C

VEE = 0 V

IFLH  – LOW TO HIGH CURRENT THRESHOLD – mA

5

| VCC = 15 TO 30 V VEE = 0 V | ICCH ICCL |
| --- | --- |

4              OUTPUT = OPEN

3

2

1

0

-40  -20

0      20     40

60     80

100

15                20

25                 30

-40  -20

0      20     40

60     80

100

TA – TEMPERATURE – °C

VCC – SUPPLY VOLTAGE – V

TA – TEMPERATURE – °C

Figure 7. ICC vs. Temperature.                     Figure 8. ICC vs. VCC.                                          Figure 9. IFLH vs. Temperature.

500

400

300

IF = 10 mA TA = 25 °C Rg = 10 W Cg = 10 nF

DUTY CYCLE = 50%

f = 10 kHz

TPLH

TPHL

500

Tp – PROPAGATION DELAY – ns

400

300

Tp – PROPAGATION DELAY – ns

VCC = 30 V, VEE = 0 V Rg = 10   , Cg = 10 nF TA = 25 °C

DUTY CYCLE = 50%

f = 10 kHz

500

400

300

IF = 10 mA

VCC = 30 V, VEE = 0 V

Rg = 10   , Cg = 10 nF

DUTY CYCLE = 50%

f = 10 kHz

Tp – PROPAGATION DELAY – ns

200

100

15                20

25                 30

200

100

6

8          10

TPLH

TPHL

12         14        16

200

100

-40  -20

0      20     40

TPLH

TPHL

60     80

100

VCC – SUPPLY VOLTAGE – V

IF – FORWARD LED CURRENT – mA

TA – TEMPERATURE – °C

Figure 10.  Propagation Delay  vs. VCC.         Figure 11.  Propagation Delay  vs. IF.             Figure 12.  Propagation Delay  vs.

Temperature.

500

400

300

VCC = 30 V, VEE = 0 V TA = 25 °C

IF = 10 mA Cg = 10 nF

DUTY CYCLE = 50%

f = 10 kHz

500

400

300

| 30 |  |  |  |  |
| --- | --- | --- | --- | --- |

VCC = 30 V, VEE = 0 V TA = 25 °C

Tp – PROPAGATION DELAY – ns

IF = 10 mA                                                                  25

Rg = 10

DUTY CYCLE = 50%                                                 20 f = 10 kHz

15

200

100

Tp – PROPAGATION DELAY – ns

0

10         20

TPLH

TPHL

30         40        50

200

100

0        20

TPLH

TPHL

40         60        80

100

10

VO – OUTPUT VOLTAGE – V

5

0

0         1           2

3          4          5

Rg – SERIES LOAD RESISTANCE –

Cg – LOAD CAPACITANCE – nF

IF – FORWARD LED CURRENT – mA

Figure 13.  Propagation Delay  vs. Rg.

Figure 14.  Propagation Delay  vs. Cg.

Figure 15.  Transfer Characteristics.

1-189

1000

TA = 25°C

100

IF

10                    +

V

F

–

1.0

0.1

IF – FORWARD CURRENT – mA

0.01

1

2

IF = 7 to

16 mA

3

8

0.1 µF

7

6

IOH

–   4 V

+

+

–

VCC = 15 to 30 V

0.001

1.10

1.20

1.30      1.40      1.50

4                                                         5

1.60

VF – FORWARD VOLTAGE – VOLTS

Figure 16.  Input  Current vs. Forward

Voltage.

Figure 17.  IOH Test  Circuit.

1                                                         8

0.1 µF

2                                                         7

3                                                         6

IOL

2.5 V   +

–

+   VCC = 15

–     to 30 V

1

2

IF = 7 to

16 mA

3

8

0.1 µF

7

6

VOH

+

–

100 mA

VCC = 15 to 30 V

4                                                         5                                                                                                  4                                                         5

Figure 18.  IOL Test  Circuit.

Figure 19.  VOH Test  Circuit.

1                                                         8

0.1 µF

2                                                         7

100 mA

1                                                         8

0.1 µF

2                                                         7

+   VCC = 15

IF                                                                                                                             +

VCC = 15

3                                                         6

VOL

–     to 30 V

3

VO > 5 V

6

–     to 30 V

4                                                         5                                                                                                  4                                                         5

Figure 20.  VOL Test  Circuit.                                                            Figure 21.  IFLH Test  Circuit.

1                                                         8

0.1 µF

2                                                         7

IF = 10 mA                                                                                                                   +

VCC

VO > 5 V     –

3                                                         6

4                                                         5

Figure 22.  UVLO Test  Circuit.

1-190

1

IF = 7 to 16 mA

2

500

+

8

7

0.1 µF

+

–

VO

IF

VCC = 15

to 30 V

tr                                               tf

10 KHz   –

50% DUTY                           3

CYCLE

4

6                       10

10 nF

5

VOUT

tPLH

tPHL

90%

50%

10%

Figure 23.  tPLH, tPHL, tr, and tf Test  Circuit and Waveforms.

1

IF

A

B         2

5 V   –

+

3

8

0.1 µF

7

VO       +

–

6

VCC = 30 V

VCM

0 V

t

VO

V  = VCM

t         t

VOH

4                                                         5

–

+

VCM = 1500 V

SWITCH AT A: IF = 10 mA VO

SWITCH AT B: IF = 0 mA

VOL

Figure 24.  CMR Test  Circuit and Waveforms.

Applications Information Eliminating Negative IGBT Gate  Drive

To keep the IGBT firmly off, the HCPL-3120  has a very low maximum  VOL specification of

0.5 V. The HCPL-3120 realizes

this very low VOL by using a

DMOS transistor with 1

(typical) on resistance in its pull down circuit.  When the HCPL-

3120  is in the low state,  the IGBT

gate is shorted to the emitter by Rg + 1    . Minimizing Rg and the lead inductance from the HCPL-

3120  to the IGBT gate and

emitter (possibly  by mounting the HCPL-3120  on a small PC board directly  above the IGBT) can eliminate  the need for negative IGBT gate drive in many applica- tions as shown in Figure  25. Care should  be taken  with such a PC board  design  to avoid routing  the

IGBT collector or emitter traces close to the HCPL-3120  input as this can result  in unwanted coupling  of transient signals  into the HCPL-3120  and degrade performance. (If the IGBT drain must be routed  near the HCPL-

3120  input,  then the LED should be reverse-biased when in the off state,  to prevent  the transient signals  coupled  from the IGBT drain from turning  on the

HCPL-3120.)

+5 V

270

HCPL-3120

1

2

8

0.1 µF

7

VCC = 18 V

+

–

+ HVDC

CONTROL

INPUT                                        3

74XXX

OPEN                               4

COLLECTOR

Rg

Q1                                                 3-PHASE

6                                                                                                        AC

5

Q2                                                   - HVDC

Figure 25.  Recommended LED Drive  and Application Circuit.

1-191

Selecting the  Gate  Resistor (Rg) to Minimize IGBT Switching Losses.

Step  1: Calculate Rg Minimum from  the  IOL Peak  Specifica- tion. The IGBT and Rg in Figure

26 can be analyzed  as a simple

RC circuit  with a voltage supplied by the HCPL-3120.

Rg      (V    – V    - V   )

CC         EE        OL

–––––––––––––––

IOLPEAK

(V    – V    - 2 V)

=      CC         EE

––––––––––––––– IOLPEAK

(15 V + 5 V - 2 V)

=

––––––––––––––––––

2.5 A

=  7.2         8

The VOL value of 2 V in the pre- vious equation is a conservative value of VOL at the peak current of 2.5A (see Figure  6). At lower

Rg values the voltage supplied by the HCPL-3120  is not an ideal voltage step.  This results  in lower peak currents (more  margin)

than predicted by this analysis. When negative  gate drive is not used VEE in the previous  equation is equal to zero volts.

Step  2: Check  the  HCPL-3120

Power Dissipation and Increase Rg if Necessary. The HCPL-3120  total power dissipa- tion (PT) is equal to the sum of the emitter power (PE) and the output power (PO):

PT = PE  + PO

PE  = IF   VF   Duty  Cycle

PO = PO(BIAS) + PO (SWITCHING)

= ICC  (VCC - VEE)

+ ESW(RG, QG) f

For the circuit  in Figure  26 with IF (worst  case)  = 16 mA, Rg = 8    , Max Duty Cycle = 80%, Qg = 500 nC, f = 20 kHz and TA max = 85C:

PE  = 16 mA  1.8 V 0.8 = 23 mW PO = 4.25  mA  20 V

+ 5.2    J 20 kHz

= 85 mW + 104  mW

= 189  mW

> 178  mW (PO(MAX) @ 85C

= 250 mW 15C*4.8  mW/C)

+5 V

270

HCPL-3120

1

2

8

0.1 µF

7

VCC = 15 V

+

–

+ HVDC

CONTROL

Rg

Q1                                                  3-PHASE

INPUT                                        3

74XXX

OPEN                               4

COLLECTOR

6

–  VEE

+

5

= -5 V

Q2

AC

| - HVDC PO Parameter | Description |
| --- | --- |
| ICC | Supply Current |
| VCC | Positive Supply Voltage |
| VEE | Negative Supply Voltage |
| ESW(Rg,Qg) | Energy Dissipated in the HCPL-3120 for each IGBT Switching Cycle (See Figure 27) |
| f | Switching Frequency |

Figure 26.  HCPL-3120 Typical  Application Circuit with Negative IGBT Gate  Drive.

| PE Parameter | Description |
| --- | --- |
| IF | LED Current |
| VF | LED On Voltage |
| Duty Cycle | Maximum LED Duty Cycle |

1-192

The value of 4.25 mA for ICC in the previous  equation was obtained by derating the ICC max of 5 mA (which occurs  at -40 C) to ICC max at 85C (see Figure  7).

Since PO for this case is greater

From the thermal mode in Figure

28 the LED and detector IC junction  temperatures can be expressed as:

TJE  = PE    (  LC||(  LD +  DC) +  CA)

shown in Figure  29. The HCPL-

3120  improves  CMR performance by using a detector IC with an optically transparent Faraday shield,  which diverts  the capaci- tively coupled  current away from the sensitive  IC circuitry. How

than PO(MAX), Rg must be increased to reduce  the HCPL-

3120  power dissipation.

+ PD

*

(–––––L–C         DC––––– +

–––––

LC +  DC +  LD

CA) + TA  ever, this shield does not

eliminate  the capacitive coupling between  the LED and optocoup-

PO(SWITCHING MAX)

= PO(MAX) - PO(BIAS)

= 178  mW - 85 mW

= 93 mW

TJD  = PE (–––––L–C–––D–C––––– +   CA)

+     +

LC          DC          LD

+ PD  (  DC||(  LD +  LC) +  CA) + TA

Inserting the values for   LC and

ler pins 5-8 as shown in Figure 30. This capacitive coupling causes  perturbations in the LED current during  common  mode transients and becomes the

ESW(MAX)

P

O(SWITCHINGMAX)

=  –––––––––––––––

f

DC shown in Figure  28 gives:

major source  of CMR failures  for

a shielded  optocoupler. The main design  objective  of a high CMR

93 mW

=                = 4.65    W

–––––––

20 kHz

For Qg = 500 nC, from Figure

27, a value of ESW = 4.65    W

gives a Rg = 10.3     .

Thermal Model

The steady  state  thermal model for the HCPL-3120  is shown in Figure  28. The thermal resistance values given in this model can be used to calculate the tempera- tures  at each node for a given operating condition. As shown by the model, all heat generated flows through   CA which raises

TJE  = PE  (256  C/W +  CA)

+ PD  (57  C/W +  CA) + TA

TJD  = PE  (57  C/W +  CA)

+ PD  (111  C/W +  CA) + TA

For example, given PE  = 45 mW, PO = 250 mW, TA = 70 C and   CA

= 83 C/W:

TJE  = PE  339 C/W + PD  140 C/W + TA

= 45 mW 339  C/W + 250 mW

140  C/W + 70 C = 120  C

TJD = PE  140 C/W + PD  194 C/W + TA

= 45 mW 140C/W + 250 mW

194  C/W + 70 C = 125  C

LED drive circuit  becomes keeping  the LED in the proper state  (on or off) during  common mode transients. For example, the recommended application circuit  (Figure  25),  can achieve

15 kV/  s CMR while minimizing component complexity.

Techniques to keep the LED in the proper state  are discussed in the next two sections.

Esw – ENERGY PER SWITCHING CYCLE – µJ

14

the case temperature TC accordingly. The value of   CA depends on the conditions of the board  design  and is, therefore, determined by the designer. The value of   CA = 83 C/W was obtained from thermal measure- ments  using a 2.5 x 2.5 inch PC board,  with small traces (no ground  plane), a single HCPL-

TJE and TJD should  be limited to

125C based  on the board  layout

and part  placement (  CA) specific to the application.

LED Drive  Circuit Considerations for Ultra High CMR Performance.

Without a detector shield,  the dominant cause  of optocoupler

12

10

8

6

4

2

0

0         10

Qg = 100 nC

Qg = 500 nC Qg = 1000 nC

VCC = 19 V VEE = -9 V

20         30        40        50

3120  soldered into the center  of

the board  and still air. The absolute maximum  power dissipation derating specifications assume  a   CAvalue of 83 C/W.

CMR failure is capacitive coupling  from the input side of the optocoupler, through the package, to the detector IC as

Rg – GATE RESISTANCE  –

Figure 27.  Energy  Dissipated in the HCPL-3120 for Each IGBT Switching Cycle.

1-193

LD = 442 °C/W

TJE                               TJD

LC = 467 °C/W                             DC = 126 °C/W TC

CA = 83 °C/W*

TA

TJE  = LED junction  temperature

TJD = detector IC junction  temperature

TC = case temperature measured at the center  of the package bottom

LC = LED-to-case thermal resistance

LD = LED-to-detector thermal resistance

DC = detector-to-case thermal resistance

CA = case-to-ambient thermal resistance

CA will depend  on the board  design  and the placement of the part.

Figure 28.  Thermal Model.

CMR with the  LED On

(CMRH).

A high CMR LED drive circuit

must keep the LED on during common  mode transients. This is achieved  by overdriving the LED current beyond  the input threshold so that it is not pulled below the threshold during  a transient. A minimum  LED cur- rent  of 10 mA provides  adequate margin  over the maximum  IFLH of

5 mA to achieve  15 kV/  s CMR.

CMR with the  LED Off

(CMRL).

A high CMR LED drive circuit

must keep the LED off (VF VF(OFF)) during  common  mode transients. For example, during  a

-dVcm/dt transient in Figure  31,

the current flowing through CLEDP

also flows through the RSAT and

VSAT of the logic gate.  As long as

the low state  voltage developed

across  the logic gate is less than VF(OFF), the LED will remain  off and no common  mode failure will occur.

The open  collector drive circuit, shown in Figure  32, cannot  keep the LED off during  a +dVcm/dt transient, since all the current flowing through CLEDN must be supplied by the LED, and it is not recommended for applications requiring ultra high CMRL performance. Figure  33 is an alternative drive circuit  which,

like the recommended application circuit  (Figure  25),  does achieve ultra high CMR performance by shunting the LED in the off state.

Under  Voltage Lockout

Feature.

The HCPL-3120  contains an under voltage lockout  (UVLO) feature that is designed to protect the IGBT under  fault conditions which cause  the HCPL-3120 supply voltage (equivalent to the fully-charged IGBT gate voltage) to drop  below a level necessary to keep the IGBT in a low resistance state.  When the HCPL-3120 output is in the high state  and the supply voltage drops  below the HCPL-3120  VUVLO– threshold

(9.5 < VUVLO– < 12.0)  the opto-

coupler  output will go into the low state  with a typical delay, UVLO Turn Off Delay, of 0.6   s.

When the HCPL-3120  output is in the low state  and the supply voltage rises above the HCPL-

3120  VUVLO+ threshold (11.0  < VUVLO+ < 13.5)  the optocoupler output will go into the high state (assumes LED is “ON”) with a typical delay, UVLO Turn On Delay of 0.8    s.

IPM Dead  Time  and Propagation Delay Specifications.

The HCPL-3120  includes  a Propagation Delay Difference (PDD) specification intended to help designers minimize “dead time” in their power inverter designs.  Dead time is the time period  during  which both the high and low side power transistors (Q1 and Q2 in Figure

25) are off. Any overlap  in Q1 and Q2 conduction will result  in large currents flowing through the power devices between  the high and low voltage motor  rails.

1-194

1                                                         8                                                                        1

CLEDO1                                      8

CLEDP

2                                                         7

CLEDP

2                                                         7

3                                                         6

CLEDN

4                                                         5

3

CLEDN

4

SHIELD

CLEDO2

6

5

Figure 29.  Optocoupler Input  to Output

Capacitance Model  for Unshielded Optocouplers.

Figure 30.  Optocoupler Input  to Output

Capacitance Model  for Shielded Optocouplers.

+5 V

+ VSAT

–

1

CLEDP

2

ILEDP

3

CLEDN

8

0.1

µF

7

6

+    VCC = 18 V

–

• • • Rg

+5 V

1                                                         8

CLEDP

2                                                         7

4                                                         5

SHIELD

• • •

3

Q1                         C

6

LEDN

* THE ARROWS INDICATE THE DIRECTION OF CURRENT FLOW DURING –dVCM/dt.

ILEDN

4                                                         5

SHIELD

+ – VCM

Figure 31.  Equivalent Circuit for Figure 25 During

Common Mode  Transient.

Figure 32.  Not  Recommended Open

Collector Drive  Circuit.

+5 V

1

CLEDP

2

3

CLEDN

4

SHIELD

To minimize dead time in a given

8          design,  the turn on of LED2 should  be delayed  (relative  to the

7          turn off of LED1) so that under worst-case conditions, transistor Q1 has just turned off when

6          transistor Q2 turns  on, as shown in Figure  34. The amount  of delay

5          necessary to achieve  this condi- tions is equal to the maximum value of the propagation delay

which is specified  to be 350 ns over the operating temperature range  of -40 C to 100  C.

Delaying the LED signal by the maximum  propagation delay difference ensures that the minimum  dead time is zero, but it does not tell a designer what the maximum  dead time will be. The maximum  dead time is equivalent

Figure 33.  Recommended LED Drive

Circuit for Ultra-High CMR.

difference specification, PDDMAX,

to the difference between  the

1-195

maximum  and minimum  propaga- tion delay difference specifica- tions as shown in Figure  35. The maximum  dead time for the

HCPL-3120  is 700 ns (= 350 ns - (-350 ns))  over an operating temperature range  of -40 C to

100  C.

Note that the propagation delays used to calculate PDD and dead time are taken  at equal tempera- tures  and test conditions since

the optocouplers under  consider- ation are typically mounted in close proximity  to each other  and are switching  identical  IGBTs.

ILED1                                                                                                                                                                                                   14

VOUT1

VOUT2

ILED2

Q1 ON

Q2 OFF

tPHL MAX

tPLH MIN

Q1 OFF Q2 ON

VO – OUTPUT VOLTAGE – V

12

10

8

6

4

2

0

| 0 5 10 15 20 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | (12.3, 10.8) |  |  |
| ( | 10.7, 9.2) |  |  |  |
| (10.7, 0.1) |  |  | (12.3, 0.1) |  |

PDD* MAX = (tPHL- tPLH)MAX = tPHL MAX - tPLH MIN

*PDD = PROPAGATION DELAY DIFFERENCE

NOTE:  FOR PDD CALCULATIONS THE PROPAGATION DELAYS

ARE TAKEN AT THE SAME TEMPERATURE AND TEST CONDITIONS.

(VCC - VEE ) – SUPPLY VOLTAGE – V

Figure 34.  Minimum  LED Skew for Zero Dead  Time.

Figure 36.  Under  Voltage Lock Out.

ILED1

VOUT1

VOUT2

ILED2

Q1 ON

Q2 OFF

tPHL MIN

tPHL MAX

tPLH MIN

Q1 OFF Q2 ON

OUTPUT POWER – PS, INPUT CURRENT – IS

800

700

600

500

400

300

200

100

0

| 0 25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PS (mW) IS (mA) |  |  |  |  |

50    75   100  125 150  175 200

(tPHL-tPLH) MAX

PDD* MAX

tPLH MAX

TS – CASE TEMPERATURE – °C

Figure 37.  Thermal Derating Curve, Dependence of Safety Limiting Value

MAXIMUM DEAD TIME

(DUE TO OPTOCOUPLER)

= (tPHL MAX - tPHL MIN) + (tPLH MAX - tPLH MIN)

= (tPHL MAX - tPLH MIN) – (tPHL MIN - tPLH MAX)

= PDD* MAX – PDD* MIN

*PDD = PROPAGATION DELAY DIFFERENCE

NOTE:  FOR DEAD TIME AND PDD CALCULATIONS ALL PROPAGATION DELAYS ARE TAKEN AT THE SAME TEMPERATURE AND TEST CONDITIONS.

with Case  Temperature per VDE 0884.

Figure 35.  Waveforms for Dead  Time.

1-196