H

High CMR, High Speed

Optocouplers

Technical  Data

HCPL-4504

HCPL-0454

HCNW4504

Features

• Short  Propagation Delays for TTL and IPM Applications

• 15 kV/  s Minimum  Common Mode Transient Immunity at VCM = 1500 V for TTL/Load Drive

• High CTR at TA = 25 C

>25% for HCPL-4504/0454

>23% for HCNW4504

• Electrical Specifications for

Common IPM Applications

• TTL Compatible

• Guaranteed Performance from  0 C to 70  C

• Open Collector Output

• Safety Approval

UL Recognized  - 2500  V rms

for 1 minute  (5000 V rms for

1 minute  for

HCPL-4504#020 and

HCNW4504)per UL1577

CSA Approved

VDE 0884  Approved

-VIORM = 630 V peak  for

HCPL-4504#060

Applications

• Inverter Circuits and Intelligent Power Module (IPM)  interfacing -

High Common  Mode Transient Immunity  (> 10 kV/  s for an IPM load/drive) and (tPLH  - tPHL) Specified  (See Power Inverter Dead Time section)

• Line Receivers -

Short Propagation Delays and

Low Input-Output Capacitance

• High Speed Logic Ground Isolation - TTL/TTL, TTL/ CMOS, TTL/LSTTL

• Replaces Pulse

Transformers -

Save Board Space and Weight

• Analog  Signal  Ground

Isolation -

Integrated Photodetector Provides  Improved  Linearity over Phototransistors

Functional Diagram

Description

These optocouplers are similar to HP’s other  high speed transistor optocouplers but with shorter propagation delays and higher CTR. The HCPL-4504/0454 and HCNW4504 also have a guaran- teed propagation delay difference (tPLH  - tPHL). These features make these  optocouplers an excellent solution  to IPM inverter dead time and other  switching  problems.

-VIORM = 1414  V peak  for

HCNW4504

BSI Certified  (HCNW4504)

NC  1

ANODE  2

8  VCC

7  NC

| TRUTH TABLE LED | VO |
| --- | --- |
| ON OFF | LOW HIGH |

• Available in 8-Pin  DIP, SO-8, Widebody Packages

CATHODE  3

6  VO

NC  4                                     5  GND

A 0.1   F bypass  capacitor between  pins 5 and 8 is recommended.

CAUTION: It is advised that  normal static precautions be taken in handling and assembly of this  component to prevent damage and/or degradation which may be induced by ESD.

5965-3604E

1-33

The HCPL-4504/0454 and HCNW4504 CTR, propagation delay, and CMR are specified  for both TTL and IPM load/drive conditions. Specifications and typical performance plots for both TTL and IPM conditions are provided  for ease of application.

These single channel, diode- transistor optocouplers are available in 8-Pin DIP, SO-8, and Widebody package configura- tions. An insulating layer between a LED and an integrated photodetector provide  electrical insulation between  input and output. Separate connections for

the photodiode bias and output- transistor collector increase the speed  up to a hundred times that of a conventional phototransistor coupler  by reducing the base collector capacitance.

Selection Guide

| Single Channel Packages |  |  |
| --- | --- | --- |
| 8-Pin DIP (300 Mil) | Small Outline SO-8 | Widebody (400 Mil) |
| HCPL-4504 | HCPL-0454 | HCNW4504 |

Ordering Information

Specify Part Number  followed by Option Number  (if desired). Example:

HCPL-4504#XXX

020 = UL 5000  V rms/1  Minute Option*

060 = VDE 0884  VIORM = 630 V peak  Option*

300 = Gull Wing Surface  Mount Option†

500 = Tape and Reel Packaging Option

Option data sheets  available.  Contact  your Hewlett-Packard sales representative or authorized distributor for information.

*For HCPL-4504  only. Combination of Option 020 and Option 060 is not available.

†Gull wing surface  mount  option  applies  to through hole parts  only.

Schematic

ICC

8

VCC

+     IF

ANODE

2

VF

IO       6

– CATHODE

3

SHIELD

VO

5

GND

1-34

Package Outline Drawings

8-Pin  DIP Package (HCPL-4504)

.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

TYPE NUMBER

8        7         6        5

OPTION CODE*

.35 ± 0.25 (0.250 ± 0.010)

HP XXXXZ YYWW

1        2         3         4

DATE CODE

UL RECOGNITION

1.19 (0.047) MAX.

1.78 (0.070) MAX.

4.70 (0.185) MAX.

5° TYP.

+ 0.076

0.254 - 0.051

+ 0.003) (0.010 - 0.002)

0.51 (0.020) MIN.

2.92 (0.115) MIN.

1.080 ± 0.320 (0.043 ± 0.013)

0.65 (0.025) MAX.

.54 ± 0.25 (0.100 ± 0.010)

DIMENSIONS IN MILLIMETERS AND (INCHES).

* MARKING CODE LETTER FOR OPTION NUMBERS. "L" = OPTION 020

"V" = OPTION 060

OPTION NUMBERS 300 AND 500 NOT MARKED.

8-Pin  DIP Package with Gull Wing Surface Mount  Option  300  (HCPL-4504)

PAD LOCATION (FOR REFERENCE ONLY)

1.016 (0.040)

1.194 (0.047)

± 0.25

± 0.010)

4.826 TYP. (0.190)

9.398 (0.370)

9.906 (0.390)

| 1 2 3 4 9.65 ± 0.25 (0.380 ± 0.010) 8 7 6 5 |  |
| --- | --- |
|  | 6.350 (0.250 |

1.194 (0.047)

1.778 (0.070)

0.381 (0.015)

0.635 (0.025)

1.19 (0.047) MAX.

1.780 (0.070) MAX.

4.19  MAX. (0.165)

9.65 ± 0.25 (0.380 ± 0.010)

7.62 ± 0.25 (0.300 ± 0.010)

+ 0.076

0.254 - 0.051

+ 0.003) (0.010 - 0.002)

1.080 ± 0.320 (0.043 ± 0.013)

2.54 (0.100) BSC

0.635 ± 0.130 (0.025 ± 0.005)

0.635 ± 0.25 (0.025 ± 0.010)

12° NOM.

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES).

1-35

Small  Outline SO-8 Package (HCPL-0454)

8       7       6       5

XXX

5.842 ± 0.203 (0.236 ± 0.008)

3.937 ± 0.127

(0.155 ± 0.005)

YWW

1       2       3       4

TYPE NUMBER (LAST 3 DIGITS)

DATE CODE

0.381 ± 0.076

(0.016 ± 0.003)                                           1.270 BSG (0.050)

5.080 ± 0.127      (0.200 ± 0.005)

3.175 ± 0.127

(0.125 ± 0.005)                                             1.524 (0.060)

7°                45° X  0.432 (0.017)

0.228 ± 0.025 (0.009 ± 0.001)

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES).

0.305 MIN. (0.012)

0.152 ± 0.051 (0.006 ± 0.002)

8-Pin  Widebody DIP Package (HCNW4504)

11.15 ± 0.15 (0.442 ± 0.006)

11.00 MAX. (0.433)

9.00 ± 0.15

8         7          6         5

HP

TYPE NUMBER

(0.354 ± 0.006)

HCNWXXXX

YYWW

DATE CODE

1

2

3

4

1.55 (0.061) MAX.

10.16 (0.400) TYP.

7° TYP.

5.10  MAX. (0.201)

+ 0.076

0.254 - 0.0051

+ 0.003) (0.010 - 0.002)

3.10 (0.122)

3.90 (0.154)

0.51 (0.021) MIN.

2.54 (0.100) TYP.

1.78 ± 0.15 (0.070 ± 0.006)

0.40 (0.016)

0.56 (0.022)

DIMENSIONS IN MILLIMETERS (INCHES).

1-36

8-Pin  Widebody DIP Package with Gull Wing Surface Mount  Option  300  (HCNW4504)

11.15 ± 0.15 (0.442 ± 0.006)

PAD LOCATION (FOR REFERENCE ONLY)

8         7          6

5

9.00 ± 0.15 (0.354 ± 0.006)

6.15 (0.242)TYP.

12.30 ± 0.30 (0.484 ± 0.012)

1

2

3

4

1.3 (0.051)

0.9 (0.035)

1.55 (0.061) MAX.

12.30 ± 0.30 (0.484 ± 0.012)

11.00 MAX. (0.433)

4.00  MAX. (0.158)

1.78 ± 0.15 (0.070 ± 0.006)

2.54 (0.100) BSC

0.75 ± 0.25 (0.030 ± 0.010)

1.00 ± 0.15 (0.039 ± 0.006)

+ 0.076

0.254 - 0.0051

+ 0.003) (0.010 - 0.002)

DIMENSIONS IN MILLIMETERS (INCHES).

LEAD COPLANARITY = 0.10 mm (0.004 INCHES).

7° NOM.

Solder Reflow Temperature Profile

(HCPL-0454 and Gull Wing Surface Mount  Option  Parts)

260

240

220

200

TEMPERATURE – °C

180

160

140

120

100

80

60

40

20

0

T = 115°C, 0.3°C/SEC

T = 100°C, 1.5°C/SEC

T = 145°C, 1°C/SEC

0        1        2        3         4        5        6        7        8        9        10       11      12

TIME – MINUTES

Note: Use of nonchlorine activated fluxes is highly recommended.

1-37

Regulatory Information The devices contained in this data sheet  have been approved by the following organizations:

UL

Recognized  under  UL 1577, Component Recognition Program, File E55361.

CSA

Approved under  CSA Component

Acceptance Notice #5, File CA

88324.

VDE

Approved according to VDE

0884/06.92 (HCNW4504  and

HCPL-4504#060 only).

BSI

Certification according to

BS451:1994,

(BS EN60065:1994); BS EN60950:1992 (BS7002:1992) and EN41003:1993 for Class II applications (HCNW4504  only).

Insulation and Safety Related Specifications

| Parameter | Symbol | 8-Pin DIP (300 Mil) Value | SO-8 Value | Widebody (400 Mil) Value | Units | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum External Air Gap (External Clearance) | L(101) | 7.1 | 4.9 | 9.6 | mm | Measured from input terminals to output terminals, shortest distance through air. |
| Minimum External Tracking (External Creepage) | L(102) | 7.4 | 4.8 | 10.0 | mm | Measured from input terminals to output terminals, shortest distance path along body. |
| Minimum Internal Plastic Gap (Internal Clearance) |  | 0.08 | 0.08 | 1.0 | mm | Through insulation distance, conductor to conductor, usually the direct distance between the photoemitter and photodetector inside the optocoupler cavity. |
| Minimum Internal Tracking (Internal Creepage) |  | NA | NA | 4.0 | mm | Measured from input terminals to output terminals, along internal cavity. |
| Tracking Resistance (Comparative Tracking Index) | CTI | 200 | 200 | 200 | Volts | DIN IEC 112/VDE 0303 Part 1 |
| Isolation Group |  | IIIa | IIIa | IIIa |  | Material Group (DIN VDE 0110, 1/89, Table 1) |

Option 300 - surface  mount  classification is Class A in accordance with CECC 00802.

1-38

VDE 0884 Insulation Related Characteristics

(HCPL-4504 OPTION 060  ONLY)

| Description | Symbol | Characteristic | Units |
| --- | --- | --- | --- |
| Installation classification per DIN VDE 0110/1.89, Table 1 for rated mains voltage 300 V rms |  | I-IV |  |
| for rated mains voltage 450 V rms |  | I-III |  |
| Climatic Classification |  | 55/100/21 |  |
| Pollution Degree (DIN VDE 0110/1.89) |  | 2 |  |
| Maximum Working Insulation Voltage | VIORM | 630 | V peak |
| Input to Output Test Voltage, Method b* VIORM x 1.875 = VPR, 100% Production Test with tm = 1 sec, Partial Discharge < 5 pC | VPR | 1181 | V peak |
| Input to Output Test Voltage, Method a* VIORM x 1.5 = VPR, Type and sample test, tm = 60 sec, Partial Discharge < 5 pC | VPR | 945 | V peak |
| Highest Allowable Overvoltage* (Transient Overvoltage, tini = 10 sec) | VIOTM | 6000 | V peak |
| Safety Limiting Values (Maximum values allowed in the event of a failure, also see Figure 15, Thermal Derating curve.) Case Temperature Input Current Output Power | TS IS,INPUT PS,OUTPUT | 175 230 600 | C mA mW |
| Insulation Resistance at TS, VIO = 500 V | RS | 109 |  |

VDE 0884 Insulation Related Characteristics (HCNW4504 ONLY)

| Description | Symbol | Characteristic | Units |
| --- | --- | --- | --- |
| Installation classification per DIN VDE 0110/1.89, Table 1 for rated mains voltage 600 V rms |  | I-IV |  |
| for rated mains voltage 1000 V rms |  | I-III |  |
| Climatic Classification |  | 55/85/21 |  |
| Pollution Degree (DIN VDE 0110/1.89) |  | 2 |  |
| Maximum Working Insulation Voltage | VIORM | 1414 | V peak |
| Input to Output Test Voltage, Method b* VIORM x 1.875 = VPR, 100% Production Test with tm = 1 sec, Partial Discharge < 5 pC | VPR | 2652 | V peak |
| Input to Output Test Voltage, Method a* VIORM x 1.5 = VPR, Type and sample test, tm = 60 sec, Partial Discharge < 5 pC | VPR | 2121 | V peak |
| Highest Allowable Overvoltage* (Transient Overvoltage, tini = 10 sec) | VIOTM | 8000 | V peak |
| Safety Limiting Values (Maximum values allowed in the event of a failure, also see Figure 15, Thermal Derating curve.) Case Temperature Input Current Output Power | TS IS,INPUT PS,OUTPUT | 150 400 700 | C mA mW |
| Insulation Resistance at TS, VIO = 500 V | RS | 109 |  |

*Refer to the front of the optocoupler section  of the current catalog  under  Product Safety Regulations section  (VDE 0884), for a detailed  description.

Note: Isolation  characteristics are guaranteed only within the safety maximum  ratings  which must be ensured by protective circuits  in

application.

1-39

Absolute Maximum Ratings

| Parameter | Symbol | Device | Min. | Max. | Units | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS |  | -55 | 125 | C |  |
| Operating Temperature | TA | HCPL-4504 HCPL-0454 | - 55 | 100 | C |  |
|  |  | HCNW4504 | -55 | 85 |  |  |
| Average Forward Input Current | IF(AVG) |  |  | 25 | mA | 1 |
| Peak Forward Input Current (50% duty cycle, 1 ms pulse width) | IF(PEAK) | HCPL-4504 HCPL-0454 |  | 50 | mA | 2 |
| (50% duty cycle, 1 ms pulse width) |  | HCNW4504 |  | 40 |  |  |
| Peak Transient Input Current ( 1 s pulse width, 300 pps) | IF(TRANS) | HCPL-4504 HCPL-0454 |  | 1 | A |  |
|  |  | HCNW4504 |  | 0.1 |  |  |
| Reverse LED Input Voltage (Pin 3-2) | VR | HCPL-4504 HCPL-0454 |  | 5 | V |  |
|  |  | HCNW4504 |  | 3 |  |  |
| Input Power Dissipation | PIN | HCPL-4504 HCPL-0454 |  | 45 | mW | 3 |
|  |  | HCNW4504 |  | 40 |  |  |
| Average Output Current (Pin 6) | IO(AVG) |  |  | 8 | mA |  |
| Peak Output Current | IO(PEAK) |  |  | 16 | mA |  |
| Supply Voltage (Pin 8-5) | VCC |  | -0.5 | 30 | V |  |
| Output Voltage (Pin 6-5) | VO |  | -0.5 | 20 | V |  |
| Output Power Dissipation | PO |  |  | 100 | mW | 4 |
| Lead Solder Temperature (Through-Hole Parts Only) 1.6 mm below seating plane, 10 seconds up to seating plane, 10 seconds | TLS | HCPL-4504 |  | 260 | C |  |
|  |  | HCNW4504 |  | 260 | C |  |
| Reflow Temperature Profile | TRP | HCPL-0454 and Option 300 | See Package Outline Drawings section |  |  |  |

1-40

Electrical Specifications (DC)

Over recommended temperature (TA = 0 C to 70 C) unless  otherwise specified. See note 12.

| Parameter | Symbol | Device | Min. | Typ.* | Max. | Units | Test Conditions |  |  | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Current Transfer Ratio | CTR | HCPL-4504 HCPL-0454 | 25 | 32 | 60 | % | TA = 25 C | VO = 0.4 V | IF = 16 mA, VCC = 4.5 V | 1, 2, 4 | 5 |
|  |  |  | 21 | 34 |  |  |  | VO = 0.5 V |  |  |  |
|  |  | HCNW4504 | 23 | 29 | 60 |  | TA = 25 C | VO = 0.4 V |  |  |  |
|  |  |  | 19 | 31 | 63 |  |  | VO = 0.5 V |  |  |  |
| Current Transfer Ratio | CTR | HCPL-4504 HCPL-0454 | 26 | 35 | 65 | % | TA = 25 C | VO = 0.4 V | IF = 12 mA, VCC = 4.5 V | 1, 2, 4 | 5 |
|  |  |  | 22 | 37 |  |  |  | VO = 0.5 V |  |  |  |
|  |  | HCNW4504 | 25 | 33 | 65 |  | TA = 25 C | VO = 0.4 V |  |  |  |
|  |  |  | 21 | 35 | 68 |  |  | VO = 0.5 V |  |  |  |
| Logic Low Output Voltage | VOL | HCPL-4504 HCPL-0454 |  | 0.2 | 0.4 | V | TA = 25 C | IO = 4.0 mA | IF = 16 mA, VCC = 4.5 V |  |  |
|  |  |  |  |  | 0.5 |  |  | IO = 3.3 mA |  |  |  |
|  |  | HCNW4504 |  | 0.2 | 0.4 |  | TA = 25 C | IO = 3.6 mA |  |  |  |
|  |  |  |  |  | 0.5 |  |  | IO = 3.0 mA |  |  |  |
| Logic High Output Current | IOH |  |  | 0.003 | 0.5 | A | TA = 25 C | VO = VCC = 5.5 V | IF = 0 mA | 5 |  |
|  |  |  |  | 0.01 | 1 |  | TA = 25 C | VO = VCC = 15 V |  |  |  |
|  |  |  |  |  | 50 |  |  |  |  |  |  |
| Logic Low Supply Current | ICCL |  |  | 50 | 200 | A | IF = 16 mA, VO = Open, VCC = 15 V |  |  |  | 12 |
| Logic High Supply Current | ICCH |  |  | 0.02 | 1 | A | TA = 25 C | IF = 0 mA, VO = Open, |  |  | 12 |
|  |  |  |  |  | 2 |  | VCC = 15 V |  |  |  |  |
| Input Forward Voltage | VF | HCPL-4504 HCPL-0454 |  | 1.5 | 1.7 | V | TA = 25 C | IF = 16 mA |  | 3 |  |
|  |  |  |  |  | 1.8 |  |  |  |  |  |  |
|  |  | HCNW4504 | 1.45 | 1.59 | 1.85 |  | TA = 25 C | IF = 16 mA |  |  |  |
|  |  |  | 1.35 |  | 1.95 |  |  |  |  |  |  |
| Input Reverse Breakdown Voltage | BVR | HCPL-4504 HCPL-0454 | 5 |  |  | V | IR = 10 A |  |  |  |  |
|  |  | HCNW4504 | 3 |  |  |  | IR = 100 A, TA = 25 C |  |  |  |  |
| Temperature Coefficient of Forward Voltage | VF TA | HCPL-4504 HCPL-0454 |  | -1.6 |  | mV/ C | IF = 16 mA |  |  |  |  |
|  |  | HCNW4504 |  | -1.4 |  |  |  |  |  |  |  |
| Input Capacitance | CIN | HCPL-4504 HCPL-0454 |  | 60 |  | pF | f = 1 MHz, VF = 0 V |  |  |  |  |
|  |  | HCNW4504 |  | 70 |  |  |  |  |  |  |  |

*All typicals  at TA = 25 C.

1-41

AC Switching Specifications

Over recommended temperature (TA = 0 C to 70 C) unless  otherwise specified.

| Parameter | Symbol | Min. | Typ. | Max. | Units | Test Conditions |  | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation Delay Time to Logic Low at Output | tPHL |  | 0.2 | 0.3 | s | TA = 25 C | Pulse: f = 20 kHz, Duty Cycle = 10%, IF = 16 mA, VCC = 5.0 V, RL = 1.9 k , CL = 15 pF, VTHHL = 1.5 V | 6, 8, 9 | 9 |
|  |  |  | 0.2 | 0.5 |  |  |  |  |  |
|  |  | 0.2 | 0.5 | 0.7 |  | TA = 25 C | Pulse: f = 10 kHz, Duty Cycle = 50%, IF = 12 mA, VCC = 15.0 V, RL = 20 k , CL = 100 pF, VTHHL = 1.5 V | 6, 10-14 | 10 |
|  |  | 0.1 | 0.5 | 1.0 |  |  |  |  |  |
| Propagation Delay Time to Logic High at Output | tPLH |  | 0.3 | 0.5 | s | TA = 25 C | Pulse: f = 20 kHz, Duty Cycle = 10%, IF = 16 mA, VCC = 5.0 V, RL = 1.9 k , CL = 15 pF, VTHLH = 1.5 V | 6, 8, 9 | 9 |
|  |  |  | 0.3 | 0.7 |  |  |  |  |  |
|  |  | 0.3 | 0.8 | 1.1 |  | TA = 25 C | Pulse: f = 10 kHz, Duty Cycle = 50%, IF = 12 mA, VCC = 15.0 V, RL = 20 k , CL = 100 pF, VTHLH = 2.0 V | 6, 10-14 | 10 |
|  |  | 0.2 | 0.8 | 1.4 |  |  |  |  |  |
| Propagation Delay Difference Between Any 2 Parts | tPLH-tPHL | -0.4 | 0.3 | 0.9 | s | TA = 25 C | Pulse: f = 10 kHz, Duty Cycle = 50%, IF = 12 mA, VCC = 15.0 V, RL = 20 k , CL = 100 pF, VTHHL = 1.5 V, VTHLH = 2.0 V | 6, 10-14 | 15 |
|  |  | -0.7 | 0.3 | 1.3 |  |  |  |  |  |
| Common Mode Transient Immunity at Logic High Level Output | ¦CMH¦ | 15 | 30 |  | kV/ s | TA = 25 C VCM = 1500 VP-P | VCC = 5.0 V, RL = 1.9 k , CL = 15 pF, IF = 0 mA | 7 | 7, 9 |
|  |  | 15 | 30 |  |  |  | VCC = 15.0 V, RL = 20 k , CL = 100 pF, IF = 0 mA | 7 | 8, 10 |
| Common Mode Transient Immunity at Logic Low Level Output | ¦CML¦ | 15 | 30 |  | kV/ s | TA = 25 C VCM = 1500 VP-P | VCC = 5.0 V, RL = 1.9 k , CL = 15 pF, IF = 16 mA | 7 | 7, 9 |
|  |  | 10 | 30 |  |  |  | VCC = 15.0 V, RL = 20 k , CL = 100 pF, IF = 12 mA | 7 | 8, 10 |
|  |  | 15 | 30 |  |  |  | VCC = 15.0 V, RL = 20 k , CL = 100 pF, IF = 16 mA | 7 7 | 8, 10 8, 10 |

*All typicals  at TA = 25 C.

1-42

Package Characteristics

Over recommended temperature (TA = 0 C to 25 C) unless  otherwise specified.

| Parameter | Sym. | Device |  | Min. | Typ.* | Max. | Units | Test Conditions |  | Fig. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input-Output Momentary Withstand Voltage† | VISO | HCPL-4504 HCPL-0454 |  | 2500 |  |  | V rms | RH 50%, t = 1 min., TA = 25 C |  |  | 6, 13 |
|  |  | HCNW4504 |  | 5000 |  |  |  |  |  |  | 6, 14 |
|  |  | H | CPL-4504 Option 020) | 5000 |  |  |  |  |  |  | 6, 11, 14 |
|  |  | ( |  |  |  |  |  |  |  |  |  |
| Input-Output Resistance | RI-O | HCPL-4504 HCPL-0454 |  |  | 1012 |  |  | VI-O = 500 Vdc |  |  | 6 |
|  |  | HCNW4504 |  | 1012 | 1013 |  |  | TA = 25 C |  |  |  |
|  |  |  |  | 1011 |  |  |  | TA = 100 C |  |  |  |
| Input-Output Capacitance | CI-O | HCPL-4504 HCPL-0454 |  |  | 0.6 |  | pF | f = 1 MHz |  |  | 6 |
|  |  | HCNW4504 |  |  | 0.5 | 0.6 |  |  |  |  |  |

*All typicals  at TA = 25 C..

†The Input-Output Momentary  Withstand Voltage is a dielectric voltage  rating  that  should  not be interpreted as an input-output

continuous voltage  rating.  For the continuous voltage  rating  refer  to the VDE 0884  Insulation Related  Characteristics Table (if applicable), your equipment level safety specification or HP Application  Note 1074  entitled  “Optocoupler Input-Output Endurance Voltage.”

Notes:

1.  Derate  linearly above 70 C free-air  temperature at a rate  of 0.8 mA/ C (8-Pin DIP).

Derate  linearly above 85 C free-air  temperature at a rate  of 0.5 mA/ C (SO-8).

2.  Derate  linearly above 70 C free-air  temperature at a rate  of 1.6 mA/ C (8-Pin DIP).

Derate  linearly above 85 C free-air  temperature at a rate  of 1.0 mA/ C (SO-8).

3.  Derate  linearly above 70 C free-air  temperature at a rate  of 0.9 mW/ C (8-Pin DIP).

Derate  linearly above 85 C free-air  temperature at a rate  of 1.1 mW/ C (SO-8).

4.  Derate  linearly above 70 C free-air  temperature at a rate  of 2.0 mW/ C (8-Pin DIP).

Derate  linearly above 85 C free-air  temperature at a rate  of 2.3 mW/ C (SO-8).

5.  CURRENT TRANSFER RATIO in percent is defined  as the ratio  of output collector current, IO, to the forward  LED input  current, IF, times 100.

6.  Device considered a two-terminal device: Pins 1, 2, 3, and 4 shorted together and Pins 5, 6, 7, and 8 shorted together.

7.  Under TTL load and drive conditions: Common  mode transient immunity  in a Logic High level is the maximum  tolerable (positive)

dVCM/dt on the leading  edge of the common  mode pulse,  VCM, to assure  that  the output will remain  in a Logic High state

(i.e., VO > 2.0 V). Common  mode transient immunity  in a Logic Low level is the maximum  tolerable (negative) dVCM/dt on the

trailing  edge of the common  mode pulse signal,  VCM, to assure  that  the output will remain  in a Logic Low state  (i.e., VO < 0.8 V).

8. Under IPM (Intelligent Power Module) load and LED drive conditions: Common  mode transient immunity  in a Logic High level is

the maximum  tolerable dVCM/dt on the leading  edge of the common  mode pulse,  VCM, to assure  that  the output will remain  in a Logic High state  (i.e., VO > 3.0 V). Common  mode transient immunity  in a Logic Low level is the maximum  tolerable dVCM/dt on the trailing  edge of the common  mode pulse signal, VCM, to assure  that  the output will remain  in a Logic Low state

(i.e., VO < 1.0 V).

9. The 1.9 k    load represents 1 TTL unit load of 1.6 mA and the 5.6 k    pull-up resistor.

10. The RL = 20 k   , CL = 100 pF load represents an IPM (Intelligent Power Module) load.

11. See Option 020 data sheet  for more  information.

12. Use of a 0.1    F bypass  capacitor connected between  pins 5 and 8 is recommended.

13. In accordance with UL 1577,  each  optocoupler is proof  tested  by applying  an insulation test voltage     3000  V rms for 1 second

(leakage detection current limit, Ii-o      5   A). This test is performed before  the 100% Production test shown in the VDE 0884

Insulation Related  Characteristics Table, if applicable.

14. In accordance with UL 1577,  each  optocoupler is proof  tested  by applying  an insulation test voltage     6000  V rms for 1 second

(leakage detection current limit, Ii-o      5   A). This test is performed before  the 100% Production test shown in the VDE 0884

Insulation Related  Characteristics Table, if applicable.

15. The difference between  tPLH  and tPHL  between  any two devices  (same  part  number) under  the same test condition. (See Power

Inverter Dead Time and Propagation Delay Specifications section.)

1-43

TA = 25°C

HCPL-4504/0454

40 mA

TA = 25°C

HCNW4504

10  VCC = 5.0 V

IO – OUTPUT CURRENT – mA

5

0

35 mA

30 mA

25 mA

20 mA

15 mA

10 mA

IF = 5 mA

20  VCC = 5.0 V

IO – OUTPUT CURRENT – mA

18

16

14

12

10

8

6

4

2

0

40 mA

35 mA

30 mA

25 mA

20 mA

15 mA

10 mA

IF = 5 mA

0                             10                            20

VO – OUTPUT VOLTAGE – V

0                             10                            20

VO – OUTPUT VOLTAGE – V

Figure 1. DC and Pulsed Transfer Characteristics.

NORMALIZED CURRENT TRANSFER RATIO

1.5

1.0

HCPL-4504/0454

NORMALIZED CURRENT TRANSFER RATIO

2.0

1.6

HCNW4504

1.2

0.5

| IF = 16 mA |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | NORMALIZED |  |  |  |  |
|  |  |  |  |  |  |  |  | VO = 0.4 V VCC= 5.0 V TA = 25°C |  |  |  |  |

0.8

0.4

0.0

0  2  4   6  8 10 12 14 16 18 20 22 24 26

0

| 0 5 10 15 |  |  |  |  |  | NORMALI IF = 16 mA VO = 0.4 V VCC= 5.0 V TA = 25°C |  |  | ED |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

20         25

IF – INPUT CURRENT – mA

IF – INPUT CURRENT – mA

Figure 2. Current Transfer Ratio  vs. Input  Current.

1000

IF – FORWARD CURRENT – mA

100

10

HCPL-4504/0454

IF

TA = 25°C

+

1000

100

10

HCNW4504

IF

+

TA = 25°C

VF

–

1.0

VF

–

1.0

0.1

0.01

IF – FORWARD CURRENT – mA

0.1

0.01

0.001

1.1       1.2        1.3       1.4

1.5

1.6

0.001

1.2       1.3        1.4       1.5

1.6

1.7

VF – FORWARD VOLTAGE – VOLTS

VF – FORWARD VOLTAGE – VOLTS

Figure 3. Input  Current vs. Forward Voltage.

1-44

NORMALIZED CURRENT TRANSFER RATIO

| 1.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NORMALIZED |  |  |  |  |  |  |  |
|  | IF = 16 mA VO = 0.4 V VCC = 5.0 V TA = 25°C |  |  |  |  |  |  |  |

1.0

0.9

0.8

0.7

HCPL-4504/0454

NORMALIZED CURRENT TRANSFER RATIO

| 1.05 |  |  |  |  | NORMALIZ IF = 16 mA VO = 0.4 V VCC = 5.0 V TA = 25°C |  |  |  | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1.0

0.95

0.9

HCNW4504

IOH – LOGIC HIGH OUTPUT CURRENT – nA

| 10 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IF = 0 mA VO = VCC = 5.0 V |  |  |  |  |  |  |  |

10 3

10 2

10 1

10 0

10-1

0.6

-60 -40  -20   0    20   40   60   80  100 120

TA – TEMPERATURE – °C

0.85

-60 -40  -20   0    20   40   60   80  100 120

TA – TEMPERATURE – °C

10-2

-60 -40  -20    0    20  40   60   80 100 120

TA – TEMPERATURE – °C

Figure 4. Current Transfer Ratio  vs. Temperature.                                                             Figure 5. Logic High Output Current vs. Temperature.

IF

0

VO

VTHHL

| CC |  |  |
| --- | --- | --- |
|  | V |  |

VTHLH

PULSE        IF

GEN.                        1

ZO = 50

t r = 5 ns                      2

3

8

7                    RL

6

0.1µF

VCC

VO

t PHL

VOL

t PLH

I F MONITOR                      4                                     5

CL

RM

Figure 6. Switching Test  Circuit.

VCM

0 V   10%

90%     90%

10%

1                                     8

IF

2                                    7                    RL

VCC

t r                                                   t f

VO

SWITCH AT A:  IF = 0 mA

VO

SWITCH AT B:  IF = 12 mA, 16 mA

VCC

VOL

A B

VFF

3                                     6

4                                     5

VCM

+         –

VO

0.1µF

CL

PULSE GEN.

Figure 7. Test  Circuit for Transient Immunity and Typical  Waveforms.

1-45

0.50

tp – PROPAGATION DELAY – µs

0.45

HCPL-4504/0454

VCC = 5.0 V RL = 1.9 k CL= 15 pF

0.50

tp – PROPAGATION DELAY – µs

0.45

VCC = 5.0 V RL = 1.9 k CL= 15 pF

HCNW4504

1.4

tp – PROPAGATION DELAY – µs

1.2

VCC = 5.0 V TA = 25° C CL = 15 pF

0.40

0.35

0.30

VTHHL = VTHLH = 1.5 V

10% DUTY CYCLE

t PHL

tPLH

0.40

0.35

0.30

VTHHL = VTHLH = 1.5 V

10% DUTY CYCLE

t PHL

tPLH

1.0

0.8

0.6

VTHHL = VTHLH = 1.5 V

10% DUTY CYCLE

tPLH

0.25

0.20

0.15

0.10

IF = 10 mA IF = 16 mA

0.25

0.20

0.15

0.10

IF = 10 mA IF = 16 mA

0.4

0.2

0.0

tPHL

IF = 10 mA            IF  = 16 mA

-60  -40  -20   0   20  40   60   80  100 120

TA – TEMPERATURE – °C

-60  -40  -20   0   20  40   60   80  100 120

TA – TEMPERATURE – °C

0    2    4    6     8   10  12  14  16  18  20

RL – LOAD RESISTANCE – k

Figure 8. Propagation Delay  Time  vs. Temperature.

| Figure 9. Propagation Delay Time vs. Load Resistance. VCC = 15.0 V TA = 25° C CL= 100 pF VTHHL = 1.5 V |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTHLH = 2.0 V 50% DUTY CYCLE |  |  |  |  |  |  |  | t PLH |  |
|  |  |  |  |  | t PHL |  |  |  |  |
|  |  |  |  |  | IF = 10 mA IF = 16 mA |  |  |  |  |

2.6

tp – PROPAGATION DELAY – µs

2.4

2.2

2.0

1.8

1.6

1.4

1.2

1.0

0.6

VCC = 5.0 V TA = 25° C CL = 100 pF

VTHHL = 1.5 V

VTHLH = 2.0 V

50% DUTY CYCLE

t

IF = 10 mA

1.1

tp – PROPAGATION DELAY – µs

1.0

0.9

0.8

0.6

VCC = 15.0 V RL = 20 k

CL= 100 pF VTHHL = 1.5 V VTHLH = 2.0 V

50% DUTY CYCLE

t

IF = 10 mA IF = 16 mA

tp – PROPAGATION DELAY – µs

t PLH

0.4

0.2

0.0

0.4

0.3

PHL

RL– LOAD RESISTANCE – k

TA – TEMPERATURE – °C 1.8

1.6

1.4

1.2

1.0

0.8

0.6

0.4

0.2

0.0

RL – LOAD RESISTANCE – k

Figure 10.  Propagation Delay  Time  vs. Load Resistance.

Figure 11.  Propagation Delay  Time  vs. Temperature.

Figure 12.  Propagation Delay  Time  vs. Load Resistance.

tp – PROPAGATION DELAY – µs

3.0

2.5

2.0

1.5

VCC = 15.0 V TA = 25° C RL= 20 k

VTHHL = 1.5 V

VTHLH = 2.0 V

50% DUTY CYCLE

t PLH

t PHL

tp – PROPAGATION DELAY – µs

1.1

1.0

0.9

0.8

0.7

0.6

t PLH

TA = 25° C RL = 20 k

CL = 100 pF VTHHL  = 1.5 V VTHLH = 2.0 V

50% DUTY CYCLE

0.5

IF = 10 mA IF = 16 mA

0.4

0.3

t PHL

IF  = 10 mA

0.0

0.2

IF = 16 mA

0  100 200 300 400 500 600 700 800 9001000

RL – LOAD CAPACITANCE – pF

10  11  12  13  14  15  16  17  18  19  20

VCC – SUPPLY VOLTAGE – V

Figure 13.  Propagation Delay  Time  vs. Load Capacitance.

Figure 14.  Propagation Delay  Time  vs. Supply  Voltage.

1-46

800

700

600

500

400

OUTPUT POWER – PS, INPUT CURRENT – IS

300 (230)

200

100

HCPL-4504 OPTION 060

LED 1

HCPL-4504/0454

HCNW4504                  8

2                                        7

6

3

5

+

OUT 1

BASE/GATE DRIVE CIRCUIT

+HV

Q1

0

| 0 25 | PS (mW) IS (mA) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

50    75   100  125  150 175  200

+ HCPL-4504/0454

OUTPUT POWER – PS, INPUT CURRENT – IS

1000

900

800

700

600

500

400

300

TS – CASE TEMPERATURE – °C

HCNW4504

| S | PS (mW) I (mA) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

LED 2

HCNW4504

2

3

8

7

6

OUT 2

5

BASE/GATE DRIVE CIRCUIT

Q2

–HV

200

100

0

0     25

50     75    100

125

150

175

Figure 16.  Typical  Power Inverter.

TS – CASE TEMPERATURE – °C

Figure 15.  Thermal Derating Curve, Dependence of Safety Limiting Valve with Case  Temperature per VDE 0884.

LED 1

Power Inverter Dead Time  and Propagation Delay  Specifications

The HCPL-4504/0454 and HCNW4504 include a specifica- tion intended to help designers

OUT 1

tPLH min

tPLH max

(tPLH max–tPLH min) TURN-ON DELAY

minimize “dead time” in their power inverter designs.  The new “propagation delay difference” specification (tPLH - tPHL) is useful for determining not only how

much optocoupler switching  delay is needed  to prevent  “shoot- through”  current, but also for determining the best achievable worst-case dead time for a given design.

When inverter power transistors switch (Q1 and Q2 in Figure  17),

LED 2

OUT 2

(tPLH max–tPLH min)

tPHL min

(tPHL max–tPHL min)

tPHL max

MAXIMUM DEAD TIME

it is essential that they never

Figure 17.  LED Delay  and Dead  Time  Diagram.

1-47

conduct at the same time. Extremely large currents will flow if there  is any overlap  in their conduction during  switching transitions, potentially damaging the transistors and even the sur- rounding circuitry. This “shoot- through”  current is eliminated by delaying the turn-on  of one transistor (Q2) long enough  to ensure  that the opposing transistor (Q1) has completely

turned off. This delay introduces a small amount  of “dead time” at

the output of the inverter during which both transistors are off during  switching  transitions. Minimizing this dead time is an important design  goal for an inverter designer.

The amount  of turn-on  delay needed  depends on the propaga- tion delay characteristics of the optocoupler, as well as the characteristics of the transistor base/gate drive circuit.  Consider- ing only the delay characteristics of the optocoupler (the  charac- teristics of the base/gate drive circuit  can be analyzed in the same way), it is important to

know the minimum  and maximum turn-on  (tPHL) and turn-off (tPLH) propagation delay specifications, preferably over the desired operating temperature range.  The importance of these  specifications is illustrated in Figure  17. The waveforms  labeled  “LED1”, “LED2”, “OUT1”, and “OUT2” are the input and output voltages  of the optocoupler circuits  driving

Q1 and Q2 respectively. Most inverters are designed such that the power transistor turns  on when the optocoupler LED turns on; this ensures that both power transistors will be off in the event of a power loss in the control circuit.  Inverters can also be designed such that the power

transistor turns  off when the optocoupler LED turns  on; this type of design,  however,  requires additional fail-safe circuitry to turn off the power transistor if an over-current condition is

detected. The timing illustrated in Figure  17 assumes that the power transistor turns  on when the optocoupler LED turns  on.

The LED signal to turn on Q2 should be delayed enough  so that an optocoupler with the very fastest turn-on  propagation delay (tPHLmin)  will never turn on before an optocoupler with the very slowest turn-off  propagation delay (tPLHmax)  turns  off. To ensure  this, the turn-on  of the optocoupler should be delayed by an amount

no less than (tPLHmax - tPHLmin), which also happens to be the maximum  data sheet  value for the propagation delay difference specification, (tPLH - tPHL). The HCPL-4504/0454 and

HCNW4504 specify a maximum (tPLH - tPHL) of 1.3    s over an operating temperature range

of 0-70 C.

Although (tPLH-tPHL)max tells the designer how much delay is needed to prevent  shoot-through current, it is insufficient to tell the designer how much dead time a design  will have. Assuming that

the optocoupler turn-on  delay is exactly equal to (tPLH - tPHL)max, the minimum  dead time is zero (i.e., there  is zero time between the turn-off of the very slowest optocoupler and the turn-on  of the very fastest  optocoupler).

Calculating the maximum  dead time is slightly more complicated. Assuming that the LED turn-on delay is still exactly equal to

(tPLH - tPHL)max,  it can be seen in

Figure  17 that the maximum  dead

time is the sum of the maximum difference in turn-on delay plus the maximum  difference in turn- off delay,

[(tPLHmax-tPLHmin)+(tPHLmax-tPHLmin)]. This expression can be

rearranged to obtain

[(tPLHmax-tPHLmin)-(tPHLmin-tPHLmax)], and further  rearranged to obtain

[(tPLH-tPHL)max-(tPLH-tPHL)min],

which is the maximum  minus the minimum  data sheet  values of (tPLH-tPHL).  The difference between the maximum  and minimum  values depends directly

on the total spread in propagation delays and sets the limit on how good the worst-case dead time

can be for a given design. Therefore, optocouplers with tight propagation delay specifications (and not just shorter delays or lower pulse-width  distortion) can achieve short  dead times in power inverters. The HCPL-4504/0454 and HCNW4504 specify a minimum (tPLH - tPHL) of -0.7   s over an operating temperature range  of 0-70 C, resulting in a maximum  dead time of 2.0   s

when the LED turn-on  delay is equal to (tPLH-tPHL)max, or 1.3   s.

It is important to maintain accurate LED turn-on  delays because delays shorter than

(tPLH  - tPHL)max  may allow shoot- through currents, while longer delays will increase the worst-case dead time.

1-48