Agilent HFBR-0400, HFBR-14xx and HFBR-24xx Series Low Cost, Miniature Fiber Optic Components with ST®, SMA,  SC and FC Ports

Data Sheet

Description

The HFBR-0400  Series  of components is designed to provide cost  effective,  high performance fiber  optic communication links  for information systems and industrial applications with  link distances of up to 2.7

kilometers. With the  HFBR-24x6, the  125 MHz analog  receiver,

data rates of up to 160 megabaud are  attainable.

Transmitters and  receivers are directly compatible with  popular “industry-standard” connectors: ST®, SMA, SC and  FC. They are completely specified with multiple fiber  sizes; including

50/125 µm, 62.5/125 µm, 100/

140 µm, and  200 µm.

The HFBR-14x4  high power transmitter and  HFBR-24x6  125

MHz receiver pair  up to provide a duplex solution optimized for

100 Base-SX.   100Base-SX  is a

Fast  Ethernet Standard (100

Mbps) at 850 nm on multimode fiber.

Complete evaluation kits  are available for ST product offerings; including transmitter, receiver, connectored cable,  and technical literature. In addition, ST connectored cables  are available for evaluation.

Applications

•  100Base-SX Fast Ethernet on 850 nm

•  Media/fiber conversion, switches, routers, hubs and NICs on

100Base-SX

•  Local Area Networks

•  Computer to Peripheral Links

•  Computer Monitor Links

•  Digital Cross Connect Links

•  Central Office Switch/PBX Links

•  Video Links

•  Modems and Multiplexers

•  Suitable for Tempest Systems

•  Industrial Control Links

Features

•  Meets IEEE 802.3 Ethernet and

802.5 Token Ring Standards

•  Meets TIA/EIA-785 100Base-SX

standard

•  Low Cost Transmitters and

Receivers

•  Choice of ST®, SMA, SC or FC Ports

•  820 nm Wavelength Technology

•  Signal Rates up to 160 MBd

•  Link Distances up to 2.7 km

•  Specified with 50/125 µm, 62.5/

125 µm, 100/140 µm, and 200 µm

HCS® Fiber

•  Repeatable  ST Connections  within

0.2 dB Typical

•  Unique Optical Port Design for

Efficient Coupling

•  Auto-Insertable and Wave

Solderable

•  No Board Mounting Hardware

Required

•  Wide Operating Temperature

Range -40 °C to +85 °C

•  AlGaAs Emitters 100% Burn-In

Ensures High Reliability

•  Conductive Port Option

ST® is a registered trademark of AT&T.

HCS® is a registered trademark of the SpecTran Corporation.

HFBR-0400 Series Part Number Guide

1                Transmitter

2                Receiver

4                820 nm Transmitter and Receiver products

0                SMA, housed

1                ST, housed

2                FC, housed

E                   SC, housed

HFBR-x4xxaa

T                  Threaded  port option

C                  Conductive  port receiver option

M              Metal port option

2                TX, stadnard power

4                TX, high  power

2                RX, 5 MBd,  TTL output

5                TX, high light output power

6                RX, 125 MHz, Analog Output

| Available Options |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| HFBR-1402 | HFBR-1414 | HFBR-1412TM | HFBR-2412TC | HFBR-2412T | HFBR-2416TC |
| HFBR-1404 | HFBR-1414M | HFBR-14E4 | HFBR-2416 | HFBR-2422 |  |
| HFBR-1412 | HFBR-1414T | HFBR-2402 | HFBR-2416M | HFBR-24E6 |  |
| HFBR-1412T | HFBR-1424 | HFBR-2406 | HFBR-2412 | HFBR-2416T |  |
| Link Selection Guide |  |  |  |  |  |
| Data rate (MBd) | Distance (m) | Transmitter | Receiver | Fiber Size (µm) | Evaluation Kit |
| 5 | 1500 | HFBR-14x2 | HFBR-24x2 | 200 HCS | N/A |
| 5 | 2000 | HFBR-14x4 | HFBR-24x2 | 62.5/125 | HFBR-0410 |
| 20 | 2700 | HFBR-14x4 | HFBR-24x6 | 62.5/125 | HFBR-0414 |
| 32 | 2200 | HFBR-14x4 | HFBR-24x6 | 62.5/125 | HFBR-0414 |
| 55 | 1400 | HFBR-14x4 | HFBR-24x6 | 62.5/125 | HFBR-0414 |
| 125 | 700 | HFBR-14x4 | HFBR-24x6 | 62.5/125 | HFBR-0416 |
| 155 | 600 | HFBR-14x4 | HFBR-24x6 | 62.5/125 | HFBR-0416 |
| 160 | 500 | HFBR-14x4 | HFBR-24x6 | 62.5/125 | HFBR-0416 |

For additional information on specific links see the following individual link descriptions. Distances measured over temperature range from 0 to +70 °C.

2

Applications Support Guide

This section gives the  designer information necessary to use  the HFBR-0400  series components

to make  a functional fiber  optic transceiver.

Agilent  offers  a wide  selection of evaluation kits  for hands-on experience with  fiber  optic products as well as a wide  range of application notes complete with  circuit diagrams and  board layouts.

Furthermore, Agilent’s application support group  is always ready to assist with  any design  consideration.

Application Literature

Title                                                          Description

HFBR-0400 Series Reliability Data                     Transmitter  & Receiver Reliability Data

Application Bulletin 78                                   Low Cost Fiber Optic Links for Digital Applications  up to 155 MBd Application Note 1038                                    Complete Fiber Solutions  for IEEE 802.3  FOIRL, 10Base-FB and 10Base-FL Application Note 1065                                    Complete Solutions for IEEE 802.5J Fiberoptic Token Ring

Application Note 1073                                    HFBR-0219 Test Fixture for 1x9 Fiber Optic Transceivers Application Note 1086                                    Optical Fiber Interconnections in Telecommunication  Products Application Note 1121                                    DC to 32 MBd Fiberoptic Solutions

Application Note 1122                                    2 to 70 MBd Fiberoptic Solutions Application Note 1123                                    20 to 160 MBd Fiberoptic Solutions Application Note 1137                                    Generic Printed Circuit Layout Rules

Application Note 1383                                    Cost Effective Fiber and Media Conversion for 100Base-SX

3

HFBR-0400 Series Evaluation Kits

HFBR-0410 ST Evaluation Kit

Contains the  following:

•  One HFBR-1412  transmitter

•  One HFBR-2412  five

megabaud TTL receiver

•  Three  meters of ST

connectored 62.5/125 µm

fiber  optic  cable  with  low cost plastic ferrules.

•  Related literature

HFBR-0414 ST Evaluation Kit Includes additional components to interface to the  transmitter and  receiver as well as the  PCB to reduce design  time.   Contains the  following:

•  One HFBR-1414T  transmitter

•  One HFBR-2416T  receiver

•  Three  meters of ST

connectored 62.5/125 µm fiber  optic  cable

•  Printed circuit board

•  ML-4622 CP Data  Quantizer

•  74ACTllOOON LED Driver

•  LT1016CN8  Comparator

•  4.7 µH Inductor

•  Related literature

HFBR-0400 SMA Evaluation Kit

Contains the  following:

•  One HFBR-1402  transmitter

•  One HFBR-2402  five

megabaud TTL receiver

•  Two meters of SMA connectored 1000 µm plastic optical fiber

•  Related literature

HFBR-0416 Evaluation Kit

Contains the  following:

•  One fully assembled 1x9 transceiver board for 155

MBd evaluation including:

- HFBR-1414  transmitter

- HFBR-2416  receiver

- circuitry

•  Related literature

Package and Handling Information

Package Information

All HFBR-0400  Series transmitters and  receivers are housed in a low-cost,  dual-inline package that is made  of high strength, heat  resistant, chemically resistant, and  UL

94V-O flame  retardant ULTEM® plastic (UL File #E121562). The transmitters are  easily  identified by the light grey color  connector port.  The receivers are  easily identified by the  dark grey color connector port.  (Black  color  for conductive port). The package is designed for auto-insertion and wave soldering so it is ideal  for high  volume  production applications.

Handling and Design Information Each  part comes  with  a protective port cap  or plug covering the  optics. These  caps/ plugs  will vary  by port style. When soldering, it is advisable to leave  the  protective cap  on

the  unit  to keep  the  optics clean. Good system performance requires clean  port optics and cable  ferrules to avoid obstructing the  optical path.

Clean  compressed air  often  is sufficient to remove particles of dirt;  methanol on a cotton swab also works  well.

Recommended Chemicals for

Cleaning/Degreasing HFBR-0400

Products

Alcohols:  methyl, isopropyl, isobutyl.

Aliphatics: hexane, heptane, Other:  soap  solution, naphtha.

Do not  use  partially halogenated hydrocarbons such  as 1,1.1 trichloroethane, ketones such  as MEK, acetone, chloroform, ethyl acetate, methylene dichloride, phenol, methylene chloride, or

N-methylpyrolldone. Also, Agilent does  not  recommend the use  of cleaners that use halogenated hydrocarbons because of their potential environmental harm.

Ultem® is a registered Trademark of the GE corporation.

4

Mechanical Dimensions

SMA Port

Rx/Tx

COUNTRY OF ORIGIN

A YYWW HFBR-X40X

HFBR-x40x

1/4 - 36 UNS 2A THREAD

12.7 (0.50)

12.7 (0.50)

22.2 (0.87)

6.35              (0.25)

6.4

10.2

(0.25) DIA.

3.6

(0.14)

5.1 (0.20)

(0.40)

(0.15)

PINS 1,4,5,8

0.51 X 0.38 (0.020 X 0.015)

1

8

PINS 2,3,6,7 (0.018) DIA.

0.46

2

7

3

6

4

5

2.54 (0.10)

1.27 (0.05)

2.54 (0.10)

PIN NO. 1

2    3

INDICATOR

Mechanical Dimensions

ST Port

HFBR-x41x

Rx/Tx

COUNTRY OF ORIGIN

A YYWW HFBR-X41X

12.7 (0.50)

27.2 (1.07)

8.2 (0.32)

12.7 (0.50)

6.35 (0.25)

7.0  DIA.

3.6

10.2

(0.28)

(0.14)

5.1 (0.20)

(0.40)

3.81 (0.15)

PINS 1,4,5,8

0.51 X 0.38 (0.020 X 0.015)

1

PINS 2,3,6,7 (0.018) DIA.

0.46

4

5

8      7

6

2.54 (0.10)

1.27 (0.05)

2.54 (0.10)

PIN NO. 1

INDICATOR

5

Mechanical Dimensions

Threaded ST Port

HFBR-x41xT

5.1           (0.20)

Rx/Tx

COUNTRY OF ORIGIN

A YYWW HFBR-X41XT

12.7 (0.50)

27.2                             (1.07)

12.7 (0.50)

7.6 (0.30)

8.4 (0.33)

6.35 (0.25)

DIA.

3.6

10.2

| ) (0.14) |  | 7.1 (0.28 |
| --- | --- | --- |

5.1 (0.20)

(0.40)

3.81         (0.15)

PINS 1,4,5,8

0.51 X 0.38 (0.020 X 0.015)

1

8

PINS 2,3,6,7 (0.018) DIA.

0.46

4

5

2

7

3

6

2.54 DIA. (0.10)

3/8 - 32 UNEF - 2A

1.27 (0.05)

2.54 (0.10)

PIN NO. 1

INDICATOR

Mechanical Dimensions

FC Port

HFBR-x42x

M8 x 0.75 6G THREAD (METRIC)

Rx/Tx

COUNTRY OF ORIGIN

A YYWW HFBR-X42X

12.7 (0.50)

19.6 (0.77)

12.7            (0.50)

7.9

3.6

10.2

(0.31)

(0.14)

5.1 (0.20)

(0.40)

3.81 (0.15)

1

2    3

4

8      7            5

6

2.5 (0.10)

2.5 (0.10)

PIN NO. 1

INDICATOR

6

Mechanical Dimensions

SC Port

HFBR-x4Ex

Rx/Tx

COUNTRY OF ORIGIN

A  YYWW HFBR-X4EX

28.65                               (1.128)

12.7            (0.50)

6.35 (0.25)

10.0 (0.394)

3.60 (0.140)

10.38 (0.409)

3.81 (0.15)

2.54

15.95               (0.628)

1.27                                 (0.050)

5.1 (0.200)

(0.10)

2.54 (0.100)

12.7 (0.500)

7

LED OR DETECTOR IC

HOUSING

LENS–SPHERE

(ON TRANSMITTERS ONLY)

LENS–WINDOW

CONNECTOR PORT

HEADER

EPOXY BACKFILL

PORT GROUNDING PATH INSERT

Figure 1. HFBR-0400 ST Series Cross-Sectional View.

Panel Mount Hardware

HFBR-4401: for SMA Ports                                                                         HFBR-4411: for ST Ports

1/4 - 36 UNEF -

2B THREAD

3/8 - 32 UNEF -

2B THREAD

PART NUMBER

Rx/Tx

COUNTRY OF ORIGIN

A YYWW HFBR-X40X

DATE CODE

0.2 IN.

HEX-NUT

7.87 (0.310)

DIA.

1.65 (0.065)

7.87 (0.310)

TYP. DIA.

HEX-NUT

12.70  DIA. (0.50)

1.65 (0.065)

14.27 (0.563)

TYP. DIA.

WALL

NUT

3/8 - 32 UNEF -

2A THREADING

1 THREAD AVAILABLE

6.61

(0.260) DIA.

WASHER

0.14 (0.005)

10.41 (0.410)

WASHER

MAX. DIA.

0.46 (0.018)

WASHER

(Each HFBR-4401 and HFBR-4411 kit consists of 100 nuts and 100 washers).

Port Cap Hardware

HFBR-4402: 500 SMA Port  Caps

HFBR-4120: 500 ST Port  Plugs  (120 psi)

8

Options

In addition to the  various port styles  available for the  HFBR-

0400 series products, there are also  several extra options that can  be ordered. To order an option, simply  place  the corresponding option number at the  end  of the  part number. See page 2 for available options.

Option  T (Threaded Port Option)

•  Allows ST style  port components to be panel mounted.

•  Compatible with  all current makes of ST® multimode connectors

•  Mechanical dimensions are compliant with  MIL-STD-

83522/13

•  Maximum  wall thickness when using  nuts and  washers from  the  HFBR-4411 hardware kit is 2.8 mm (0.11 inch)

•  Available on all ST ports

Option C (Conductive  Port Receiver

Option)

•  Designed to withstand electrostatic discharge (ESD) of 25 kV to the port

•  Significantly reduces effect  of electromagnetic interference (EMI) on receiver sensitivity

•  Allows designer to separate

the  signal  and  conductive port grounds

•  Recommended for use  in

noisy  environments

•  Available on SMA and threaded ST port style receivers only

Option M (Metal Port Option)

•  Nickel  plated aluminum connector receptacle

•  Designed to withstand electrostatic discharge (ESD) of 15 kV to the port

•  Significantly reduces effect  of electromagnetic interference (EMI) on receiver sensitivity

•  Allows designer to separate the  signal  and  metal  port grounds

•  Recommended for use  in very noisy  environments

•  Available on SMA, ST, and threaded ST ports

9

Typical Link Data

HFBR-0400 Series

Description

The following  technical data is taken from  4 popular links  using the  HFBR-0400  series: the  5

MBd link, Ethernet 20 MBd link, Token  Ring 32 MBd link, and  the corresponds to transceiver solutions combining the  HFBR-

0400  series components and various recommended transceiver design  circuits using off-the-shelf electrical components. This data is meant to be regarded as an example of typical link performance for a given design  and  does  not  call out any  link  limitations. Please refer to the  appropriate application note  given for each link to obtain more  information.

5 MBd Link (HFBR-14xx/24x2)

Link Performance  -40 °C to +85 °C unless otherwise specified

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Optical Power Budget with 50/125 µm fiber | OPB50 | 4.2 | 9.6 |  | dB | HFBR-14x4/24x2 NA = 0.2 |  | Note 1 |
| Optical Power Budget with 62.5/125 µm fiber | OPB62.5 | 8.0 | 15 |  | dB | HFBR-14x4/24x2 NA = 0.27 |  | Note 1 |
| Optical Power Budget with 100/140 µm fiber | OPB100 | 8.0 | 15 |  | dB | HFBR-14x2/24x2 NA = 0.30 |  | Note 1 |
| Optical Power Budget with 200 µm fiber | OPB200 | 12 | 20 |  | dB | HFBR-14x2/24x2 NA = 0.37 |  | Note 1 |
| Date Rate Synchronous |  | dc |  | 5 | MBd |  |  | Note 2 |
| Asynchronous |  | dc |  | 2.5 | MBd |  |  | Note 3, |
|  |  |  |  |  |  |  |  | Fig 7 |
| Propagation Delay LOW to HIGH | tPLH |  | 72 |  | ns | TA = +25 °C PR = -21 dBm peak |  | Figs 6, 7, 8 |
| Propagation Delay HIGH to LOW | tPHL |  | 46 |  | ns |  |  |  |

System Pulse Width

Distortion

tPLH - tPHL                                                                    26                                            ns

Fiber cable length = 1 m

Bit Error Rate                             BER                                                                         10-9                                                                           Data rate <5 Bd

PR > -24 dBm peak

Notes:

1.   OPB at TA = -40 to +85 °C, VCC = 5.0 V dc, IF ON = 60 mA. PR = -24 dBm peak.

2.   Synchronous data rate limit is based on these assumptions: a) 50% duty factor modulation, e.g., Manchester I or BiPhase  Manchester II; b)

continuous data; c) PLL Phase Lock Loop demodulation; d) TTL threshold.

3.   Asynchronous data rate limit is based on these assumptions: a) NRZ data; b) arbitrary timing-no duty factor restriction; c) TTL threshold.

10

5 MBd Logic Link Design

If resistor R1 in Figure  2 is 70.4

W, a forward current IF of 48 mA

is applied to the  HFBR-14x4

LED transmitter. With IF = 48 mA the  HFBR-14x4/24x2 logic link is guaranteed to work  with

62.5/125 µm fiber  optic  cable over the  entire range  of 0 to

1750 meters at a data rate  of dc to 5 MBd, with  arbitrary data format and  pulse  width

distortion typically less than

The following  example will illustrate the  technique for selecting the  appropriate value of IF and  R1.

Maximum  distance required =

400 meters. From  Figure  3 the drive  current should be 15 mA. From  the  transmitter data VF =

1.5 V (max.)  at IF = 15 mA as

shown in Figure  9.

R1 = VCC − VF = 5V − 1.5V

The curves in Figures 3, 4, and  5 are  constructed assuming no inline splice  or any additional system loss. Should the  link consists of any in-line  splices, these curves can  still be used  to calculate link  limits  provided they  are  shifted by the

additional system loss  expressed in dB. For example, Figure  3 indicates that with  48 mA of transmitter drive  current, a 1.75

km link distance is achievable

25%. By setting R1 = 115 W, the transmitter can  be driven with IF = 30 mA, if it is desired to economize on power or achieve lower  pulse  distortion.

IF

R1 = 233 Ω

15 mA

with  62.5/125 µm fiber  which has a maximum attenuation of 4 dB/km. With 2 dB of additional system loss, a 1.25 km link distance is still  achievable.

+5 V        SELECT R1  TO SET IF

HFBR-14xx

TRANSMITTER

HFBR-24x2

RECEIVER

TTL DATA  OUT

R              IF

1                                2                                                                                                                                                                                      2

6                                                                                                                                                                                                                        V

1 KΩ                                       7         T

3

DATA IN

R            6

7 & 3

RL                               CC

0.1 µF

NOTE:

½ 75451

TRANSMISSION DISTANCE =

IT IS ESSENTIAL  THAT A BYPASS CAPACITOR (0.01 µF TO 0.1 µF CERAMIC) BE CONNECTED FROM PIN 2 TO PIN 7 OF THE RECEIVER. TOTAL LEAD LENGTH BETWEEN BOTH ENDS  OF THE CAPACITOR AND THE PINS SHOULD NOT EXCEED 20 MM.

Figure 2. Typical Circuit Configuration.

11

I  TRANSMITTER  FORWARD CURRENT (mA)

0                                                                                            60

WORST CASE

50

-1        OVERDRIVE

-2                                          UNDERDRIVE                                   40

-3                                                                                              30

-4                                                        TYPICAL +25 ˚C

-5                                                          UNDERDRIVE                20

-6

-7

-8                                                                                              10

0

-1

-2       OVERDRIVE

-3

-4

-5

-6

-7

-8

| 0 60 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WORST CASE -40˚C, +85˚C UNDERDRIVE |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | TYPICAL 26˚C UNDERDRIVE |  |  |  |
|  |  |  | CAB | LE A | TTEN | UATI | ON |  | dB/ | km |
|  |  |  |  MAX (-40˚C, +85˚C)  MIN (-40˚C, +85˚C)  TYP (-40˚C, +85˚C) |  |  |  |  |  | 4 1 2.8 |  |

60

I  TRANSMITTER  FORWARD  CURRENT (mA)

10 LOG (t/to)  NORMALIZED TRANSMITTER CURRENT (dB)

WORST CASE                                     50                              -1                                                                                           50

-40 ˚C, +85 ˚C                               40

-2                                                                                           40

30

TYPICAL +25 ˚C

UNDERDRIVE                20                             -3                                                                                            30

10

-9

10LOG(I/Io) NORMALIZED TRANSMITTER CURRENT (dB)

-10

-11

0

CABLE ATTENUATION MAX (-40 ˚C, +85 ˚C) MIN (-40 ˚C, +85 ˚C) TYP (+25 ˚C)

2

dB/km

4

1.5              6

2.8

F

4

-9

10LOG(I/Io) NORMALIZED TRANSMITTER CURRENT (dB)

-10

-11

0

CABLE ATTENUATION MAX (-40 ˚C, +85 ˚C) MIN (-40 ˚C, +85 ˚C) TYP (+25 ˚C)

1                  2                   3

dB/km

5.5

1.0              6

3.3

F

4

IF  – TRANSMITTER FORWARD CURRENT – (mA)

-5                                                                                           20

-6

0            0.4          0.8          1.2          1.6            2

LINK LENGTH (km)

LINK LENGTH (km)

LINK LENGTH (km)

Figure 3. HFBR-1414/HFBR-2412 Link Design

Limits with 62.5/125 µm Cable.

Figure 4. HFBR-14x2/HFBR-24x2 Link Design

Limits with 100/140 µm Cable.

Figure 5. HFBR-14x4/HFBR-24x2 Link Design

Limits with 50/125 µm Cable.

| 75 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tPLH (TYP) @ 25˚C |  |  |  |  |  |  |  |
|  |  | tPHL (TYP) @ 25˚C |  |  |  |  |  |  |  |

70

65

60

55

50

45

40

35

30

25

tPLH OR t PHL PROPOGATION  DELAY –ns

20

-22  -21  -20   -19  -18  -17  -16  -15  -14  -13  -12

P R – RECEIVER POWER  – dBm

Figure 6. Propagation Delay through System with One Meter of Cable.

| 55 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

50

tD – NRZ DISTORTION – ns

45

40

35

30

25

20

-22   -21  -20   -19  -18  -17  -16  -15  -14  -13  -12

P R – RECEIVER POWER  – dBm

Figure 7. Typical Distortion of Pseudo Random

Data at 5 Mb/s.

PULSE GEN

+15 V

RS

RESISTOR VALUE AS NEEDED FOR SETTING OPTICAL POWER OUTPUT FROM  RECEIVER END OF TEST CABLE

½ 75451    1N4150

2, 6, 7

INPUT

100 ns             PULSE REPETITION

FREQ = 1 MHz

100 ns

3

10 W

I  10 W     TRANSMITTER FROM 1-METER

F

INPUT (IF)

TIMING

IF         50%

PT     50%

tPHLT

tPHLT

tPHL

PT -

TEST CABLE

2

+5 V

RL        560

OUTPUT

ANALYSIS EQUIPMENT eg. SCOPE

tPHL

MAX

5 V

tPHL

MIN

tPHL

MAX

MIN

HFBR-2412 RECEIVER

6

7 & 3

0.1 µF

+

15 pF      VO

VO

1.5 V

0

Figure 8. System Propagation Delay Test Circuit and Waveform Timing Definitions.

12

Ethernet 20 MBd Link (HFBR-14x4/24x6)

(refer to Application Note 1038 for details)

Typical Link Performance

| Parameter | Symbol | Typ [1, 2] | Units | Conditions |
| --- | --- | --- | --- | --- |
| Receiver Sensitivity |  | -34.4 | dBm average | 20 MBd D2D2 hexadecimal data |
|  |  |  |  | 2 km 62.5/125 µm fiber |
| Link Jitter |  | 7.56 | ns pk-pk | ECL Out Receiver |
|  |  | 7.03 | ns pk-pk | TTL Out Receiver |
| Transmitter Jitter |  | 0.763 | ns pk-pk | 20 MBd D2D2 hexadecimal data |
| Optical Power | PT | -15.2 | dBm average | 20 MBd D2D2 hexadecimal data Peak IF,ON = 60 mA |
| LED Rise Time | tr | 1.30 | ns | 1 MHz square wave input |
| LED Fall Time | tf | 3.08 | ns |  |
| Mean Difference | ¦tr - tf¦ | 1.77 | ns |  |
| Bit Error Rate | BER | 10-10 |  |  |
| Output Eye Opening |  | 36.7 | ns | At AUI receiver output |
| Data Format 50% Duty Factor |  | 20 | MBd |  |

Notes:

1.   Typical data at TA = +25 °C, VCC = 5.0 V dc.

2.   Typical performance of circuits shown in Figure 1 and Figure 3 of AN-1038 (see applications support section).

Token Ring 32 MBd Link (HFBR-14x4/24x6)

(refer to Application Note 1065 for details)

Typical Link Performance

| Parameter | Symbol | Typ [1, 2] | Units | Conditions |
| --- | --- | --- | --- | --- |
| Receiver Sensitivity |  | -34.1 | dBm average | 32 MBd D2D2 hexadecimal data |
|  |  |  |  | 2 km 62.5/125 µm fiber |
| Link Jitter |  | 6.91 | ns pk-pk | ECL Out Receiver |
|  |  | 5.52 | ns pk-pk | TTL Out Receiver |
| Transmitter Jitter |  | 0.823 | ns pk-pk | 32 MBd D2D2 hexadecimal data |
| Optical Power Logic Level "0" | PT ON | -12.2 | dBm peak | Transmitter TTL in IF ON = 60 mA, |
| Optical Power Logic Level "1" | PT OFF | -82.2 |  | IF OFF = 1 mA |
| LED Rise Time | tr | 1.3 | ns | 1 MHz square wave input |
| LED Fall Time | tf | 3.08 | ns |  |
| Mean Difference | ¦tr - tf¦ | 1.77 | ns |  |
| Bit Error Rate | BER | 10-10 |  |  |
| Data Format 50% Duty Factor |  | 32 | MBd |  |

Notes:

1. Typical data at TA = +25 °C, VCC = 5.0 V dc.

2. Typical performance of circuits shown in Figure 1 and Figure 3 of AN-1065 (see applications support section)

13

155 MBd Link (HFBR-14x4/24x6)

(refer to Application Bulletin 78 for details)

Typical Link Performance

| Parameter | Symbol | Min | Typ [1, 2] | Max | Units | Conditions | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Optical Power Budget with 50/125 µm fiber | OPB50 | 7.9 | 13.9 |  | dB | NA = 0.2 | Note 2 |
| Optical Power Budget with 62.5/125 µm fiber | OPB62 | 11.7 | 17.7 |  | dB | NA = 0.27 |  |
| Optical Power Budget with 100/140 µm fiber | OPB100 | 11.7 | 17.7 |  | dB | NA = 0.30 |  |
| Optical Power Budget with 200 µm HCS fiber | OPB200 | 16.0 | 22.0 |  | dB | NA = 0.35 |  |
| Data Format 20% to 80% Duty |  | 1 |  | 175 | MBd |  |  |
| Factor |  |  |  |  |  |  |  |
| System Pulse Width Distortion | ¦tPLH - tPHL¦ |  | 1 |  | ns | PR = -7 dBm peak 1 m 62.5/125 µm fiber |  |

Bit Error Rate                                    BER                                                      10-9                                                                                                                Data rate < 100 MBaud

PR > -31 dBm peak

Note 2

Notes:

1.   Typical data at TA = +25 °C, VCC = 5.0 V dc, PECL serial interface.

2.   Typical OPB was determined at a probability  of error (BER) of 10-9. Lower probabilities of error can be achieved with short fibers that have less optical loss.

14

HFBR-14x2/14x4 Low-Cost High- Speed Transmitters

Description

The HFBR-14xx  fiber  optic

into  50/125 µm fiber  and  -12 dBm into  62.5/125 µm fiber.  The HFBR-14x2  standard

transmitter  typically can  launch

-12 dBm of optical power at 60

Housed Product

| ANODE 2, 6, 7 3 |  |  |
| --- | --- | --- |

CATHODE

AlGaAs emitter capable of efficiently launching optical power into  four  different optical fiber  sizes:  50/125 µm, 62.5/125

µm, 100/140 µm, and  200 µm

It is ideal  for large  size fiber such  as 100/140 µm. The high launched optical power level is useful  for systems where star couplers, taps,  or inline

4  5

3  6

2  7

1  8

BOTTOM VIEW

| NOTES: PIN | FUNCTION |
| --- | --- |
| 11 2 32 41 51 6 72 81 | NC ANODE CATHODE NC NC ANODE ANODE NC |

1. PINS 1, 4, 5 AND 8

ARE ELECTICALLY CONNECTED.

PIN 1 INDICATOR   2. PINS 2, 6 AND 7 ARE ELECTRICALLY CONNECTED TO THE HEADER.

HCS®. This allows  the  designer flexibility in choosing the fiber size. The HFBR-14xx  is designed

to operate with  the  Agilent

| connectors create large fixed losses. PIN | FUNCTION |
| --- | --- |
| 1 2 3 4 | ANODE CATHODE ANODE ANODE |

Consistent coupling efficiency is

Unhoused Product

1       2

HFBR-24xx  fiber  optic  receivers.

assured by the  double-lens                   4       3

optical system (Figure 1). Power

The HFBR-14xx  transmitter’s high coupling efficiency allows the  emitter to be driven at low current levels  resulting in low power consumption and increased reliability of the transmitter. The HFBR-14x4 high power transmitter is optimized for small  size fiber and  typically can  launch -15.8 dBm optical power at 60 mA

coupled into  any of the  three fiber types  varies less than 5 dB from  part to part at a given drive current and  temperature. Consistent coupling efficiency reduces receiver dynamic range requirements which  allows  for longer  link lengths.

BOTTOM VIEW

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -55 | +85 | °C |  |
| Operating Temperature | TA | -40 | +85 | °C |  |
| Lead Soldering Cycle |  |  |  |  |  |
| Temp |  |  | +260 | °C |  |
| Time |  |  | 10 | sec |  |

Forward Input Current

Peak dc

IFPK

IFdc

200                                  mA

100                                  V

Note 1

Reverse Input Voltage      VBR                                                                                         1.8                                   V

15

Electrical/Optical  Specifications  -40 °C to +85 °C unless otherwise specified.

| Parameter | Symbol | Min | Typ2 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Forward Voltage | VF | 1.48 | 1.70 | 2.09 | V | IF = 60 mA dc | Figure 9 |
|  |  |  | 1.84 |  |  | IF = 100 mA dc |  |
| Forward Voltage Temperature Coefficient | DVF/DT |  | -0.22 |  | mV/°C | IF = 60 mA dc | Figure 9 |
|  |  |  | -0.18 |  |  | IF = 100 mA dc |  |
| Reverse Input Voltage | VBR | 1.8 | 3.8 |  | V | IF = 100 µA dc |  |
| Peak Emission Wavelength | lP | 792 | 820 | 865 | nm |  |  |
| Diode Capacitance | CT |  | 55 |  | pF | V = 0, f = 1 MHz |  |
| Optical Power Temperature Coefficient | DPT/DT |  | -0.006 |  | dB/°C | I = 60 mA dc |  |
|  |  |  | -0.010 |  |  | I = 100 mA dc |  |
| Thermal Resistance | qJA |  | 260 |  | °C/W |  | Notes 3, 8 |
| 14x2 Numerical Aperture | NA |  | 0.49 |  |  |  |  |
| 14x4 Numerical Aperture | NA |  | 0.31 |  |  |  |  |
| 14x2 Optical Port Diameter | D |  | 290 |  | µm |  | Note 4 |
| 14x4 Optical Port Diameter | D |  | 150 |  | µm |  | Note 4 |

HFBR-14x2 Output Power Measured Out of 1 Meter of Cable

| Parameter | Symbol | Min | Typ2 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50/125 µm Fiber Cable NA = 0.2 | PT50 | -21.8 -22.8 | -18.8 | -16.8 -15.8 | dBm peak | TA = +25 °C, IF = 60mA dc | Notes 5, 6, 9 |
|  |  | -20.3 -21.9 | -16.8 | -14.4 -13.8 |  | TA = +25 °C, IF = 100 mA dc |  |
| 62.5/125 µm Fiber Cable NA = 0.275 | PT62 | -19.0 -20.0 | -16.0 | -14.0 -13.0 | dBm peak | TA = +25 °C, IF = 60mA dc |  |
|  |  | -17.5 -19.1 | -14.0 | -11.6 -11.0 |  | TA = +25 °C, IF = 100 mA dc |  |
| 100/140 µm Fiber Cable NA = 0.3 | PT100 | -15.0 16.0 | -12.0 | -10.0 -9.0 | dBm peak | TA = +25 °C, IF = 60mA dc |  |
|  |  | -13.5 -15.1 | -10.0 | -7.6 -7.0 |  | TA = +25 °C, IF = 100 mA dc |  |
| 200 µm HCS Fiber Cable NA - 0.37 | PT200 | -10.7 -11.7 | -7.1 | -4.7 -3.7 | dBm peak | TA = +25 °C, IF = 60mA dc |  |
|  |  | -9.2 -10.8 | -5.2 | -2.3 -1.7 |  | TA = +25 °C, IF = 100 mA dc |  |

CAUTION: The small junction sizes inherent to the design of these components increase the components’ susceptibility to damage from electrostatic discharge (ESD). It is advised that normal static precautions be taken in handling and assembly of these components to prevent damage and/or degradation which may be induced by ESD.

16

HFBR-14x4 Output Power Measured out of 1 Meter of Cable

| Parameter | Symbol | Min | Typ2 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50/125 µm Fiber Cable NA = 0.2 | PT50 | -18.8 -19.8 | -15.8 | -13.8 -12.8 | dBm peak | TA = +25 °C, IF = 60mA dc | Notes 5, 6, 9 |
|  |  | -17.3 -18.9 | -13.8 | -11.4 -10.8 |  | TA = +25 °C, IF = 100 mA dc |  |
| 62.5/125 µm Fiber Cable NA = 0.275 | PT62 | -15.0 -16.0 | -12.0 | -10.0 -9.0 | dBm peak | TA = +25 °C, IF = 60mA dc |  |
|  |  | -13.5 -15.1 | -10.0 | -7.6 -7.0 |  | TA = +25 °C, IF = 100 mA dc |  |
| 100/140 µm Fiber Cable NA = 0.3 | PT100 | -9.5 -10.5 | -6.5 | -4.5 -3.5 | dBm peak | TA = +25 °C, IF = 60mA dc |  |
|  |  | -8.0 -9.6 | -4.5 | -2.1 -1.5 |  | TA = +25 °C, IF = 100 mA dc |  |
| 200 µm HCS Fiber Cable NA - 0.37 | PT200 | -5.2 -6.2 | -3.7 | +0.8 +1.8 | dBm peak | TA = +25 °C, IF = 60mA dc |  |
|  |  | -3.7 -5.3 | -1.7 | +3.2 +3.8 |  | TA = +25 °C, IF = 100 mA dc |  |

| HFBR-14x5 Output Power Measured out of 1 Meter of Cable Parameter | Symbol | Min | Typ2 | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| 62.5/125 µm Fiber Cable NA = 0.275 | PT62 | -11.0 -12.0 | -10.0 -10.0 | -8.0 -7.0 | dBm peak | TA = +25 °C, IF = 60mA |

14x2/14x4 Dynamic Characteristics

Reference

| Parameter | Symbol | Min | Typ2 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rise Time, Fall Time (10% to 90%) | tr, tf |  | 4.0 | 6.5 | nsec No pre- | IF = 60 mA Figure 12 | Note 7 |
|  |  |  |  |  | bias |  |  |
| Rise Time, Fall Time (10% to 90%) | tr, tf |  | 3.0 |  | nsec | IF = 10 to 100 mA | Note 7, Figure 11 |
| Pulse Width Distortion | PWD |  | 0.5 |  | nsec |  | Figure 11 |

Notes:

1.   For IFPK > 100 mA, the time duration should not exceed 2 ns.

2.   Typical data at TA = +25 °C.

3.   Thermal resistance is measured with the transmitter coupled to a connector assembly and mounted on a printed circuit board.

4.   D is measured at the plane of the fiber face and defines a diameter where the optical power density is within 10 dB of the maximum.

5.   PT is measured with a large area detector at the end of 1 meter of mode stripped cable, with an ST® precision ceramic ferrule (MILSTD- 83522/13)

for HFBR-1412/1414, and with an SMA 905 precision ceramic ferrule for HFBR-1402/1404.

6.   When changing mW to dBm, the optical power is referenced to 1 mW (1000 mW). Optical Power P (dBm) = 10 log P (mW)/1000 mW.

7.   Pre-bias is recommended if signal rate >10 MBd, see recommended  drive circuit in Figure 11.

8.   Pins 2, 6 and 7 are welded to the anode header connection to minimize the thermal resistance from junction to ambient. To further reduce the thermal resistance, the anode trace should be made as large as is consistent with good  RF circuit  design.

9.   Fiber NA is measured at the end of 2 meters of mode stripped fiber, using the far-field pattern. NA is defined as the sine of the half angle, determined at 5% of the peak intensity point. When using other manufacturer’s fiber cable, results will vary due to differing NA values and specification

methods.

All HFBR-14XX  LED transmitters are classified as IEC 825-1 Accessible Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go in to effect on January 1, 1997.  AEL Class 1 LED devices are considered eye safe. Contact your Agilent sales representative for more information.

CAUTION: The small junction sizes inherent to the design of these components increase the components’ susceptibility to damage from electrostatic discharge (ESD). It is advised that normal static precautions be taken in handling and assembly of these components to prevent damage and/or degradation which may be induced by ESD.

17

Recommended Drive Circuits The circuit used  to supply current to the  LED transmitter can significantly influence the optical switching characteristics of the  LED. The optical rise/fall times  and  propagation delays can be improved by using  the appropriate circuit techniques. The LED drive  circuit shown in

Figure  11 uses  frequency compensation to reduce the typical rise/fall times  of the  LED and  a small  pre-bias voltage  to minimize propagation delay differences that cause pulse- width distortion. The circuit will typically produce rise/fall times of 3 ns, and  a total  jitter including pulse-width distortion of less than 1 ns. This circuit is recommended for  applications requiring low edge jitter or high-

speed data transmission at signal  rates of up to 155 MBd. Component values for this circuit can  be calculated for different LED drive  currents using  the  equations shown below.  For  additional details about LED drive  circuits, the reader is encouraged to read Agilent  Application Bulletin 78 and  Application Note 1038.

RY (VCC − VF) + 3.97(VCC − VF − 1.6V) IF ON (A)

RY = (5 − 1.84) + 3.97(5 − 1.84 − 1.6)

0.100

1 

RX1 =   

RY   



RY =

3.16 + 6.19

= 93.5 Ω

2  3.97 

0.100

         

REQ2 (Ω) = RX1 − 1

RX2 = RX3 = RX4  = 3(REQ2)

RX1 = 1  93.5  = 11.8 Ω

2  3.97 

REQ2  = 11.8 - 1 = 10.8 Ω

C(pF) = 2000 ps

RX1(Ω)

RX2

= RX3

= RX4

= 3 (10.8) = 32.4 Ω

Example for IF ON = 100mA :

VF can be obtained from Figure 9 (= 1.84 V).

C = 2000 ps  = 169 pF

11.8 Ω

18

| 100 | +85 | °C |  |  |
| --- | --- | --- | --- | --- |
|  | +25 °C |  |  |  |
|  |  |  | -40 °C |  |

80

60

40

IF - FORWARD CURRENT - mA

20

2.0

P(I F ) – P(60 mA) – RELATIVE POWER RATIO

1.8

1.6

1.4

1.2

1.0

0.8

0.6

0.4

0.2

0

| 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

10   20

30   40

50   60

70   80

90  100

3.0

P(I F ) – P(60 mA) – RELATIVE POWER  RATIO – dB

2.0

1.4

1.0

0.8

0

-1.0

-2.0

-3.0

-4.0

-5.0

-7.0

10

1.2        1.4         1.6        1.8        2.0        2.2

VI - FORWARD VOLTAGE - V

IF  – FORWARD CURRENT – mA

Figure 9. Forward Voltage and Current

Characteristics.

Figure 10. Normalized Transmitter Output vs. Forward Current.

¼

2 74F3037

3

1

4, 5

+

4.7 µF

15

14

0.1 µF

12, 13

16

RX2

RX1

+5 V Ry

¼ 74F3037

10                9

11

¼ 74F3037

RX3

C

HFBR-14x2/x4

8                5          RX4

7

¼ 74F3037

Figure 11. Recommended Drive Circuit.

HP8082A PULSE GENERATOR

| SILICON AVALANCHE PHOTODIODE |  | 50 Ω TEST HEAD | HIGH SPEED OSCILLOSCOPE |
| --- | --- | --- | --- |

50  LOAD RESISTOR

Figure 12. Test Circuit for Measuring  tr, tf.

19

HFBR-24x2 Low-Cost 5 MBd

Receiver

designed for direct interfacing to popular logic families. The absence of an internal pull-up

Housed Product

2

6

Vcc

Description

The HFBR-24x2  fiber  optic receiver is designed to operate with  the  Agilent  HFBR-14xx fiber  optic  transmitter and  50/

125 µm, 62.5/125 µm, 100/  140

µm, and  200 µm HCS® fiber optic cable.  Consistent coupling into  the  receiver is assured by the  lensed optical system (Figure 1). Response does  not vary  with  fiber  size ≤ 0.100 µm.

resistor allows  the  open-

collector output to be used  with logic families such  as CMOS requiring voltage  excursions much  higher than VCC.

Both  the  open-collector “Data” output Pin 6 and  VCC Pin 2 are referenced to “Com” Pin 3, 7.

The “Data” output allows  busing, strobing and  wired “OR” circuit configurations. The transmitter

4  5

3  6

2  7

1  8

BOTTOM VIEW

| 1 PIN | FUNCTION |
| --- | --- |
| 1 2 32 41 51 6 72 81 | NC V (5 V) CC COMMON NC NC DATA COMMON NC |

NOTES:

DATA

7 & 3 COMMON

PIN 1 INDICATOR

The HFBR-24x2  receiver incorporates an integrated photo IC containing a photodetector and dc amplifier driving an opencollector Schottky output transistor. The HFBR-24x2  is

is designed to operate from  a

single  +5 V supply. It is essential that a bypass capacitor (0.1 mF ceramic) be connected from  Pin

2 (VCC) to Pin 3 (circuit common) of the  receiver.

1. PINS 1, 4, 5 AND 8 ARE ELECTRICALLY CONNECTED

2. PINS 3 AND 7 ARE ELECTRICALLY CONNECTED  TO HEADER

| Unhoused Product PIN | FUNCTION |
| --- | --- |
| 1 2 3 4 | VCC (5 V) COMMON DATA COMMON |

| 1 | 2 |
| --- | --- |
| 4 | 3 |

BOTTOM VIEW

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -55 | +85 | °C |  |
| Operating Temperature | TA | -40 | +85 | °C |  |
| Lead Soldering Cycle |  |  |  |  | Note 1 |
| Temp |  |  | +260 | °C |  |
| Time |  |  | 10 | sec |  |
| Supply Voltage | VCC | -0.5 | 7.0 | V |  |
| Output Current | IO |  | 25 | mA |  |
| Output Voltage | VO | -0.5 | 18.0 | V |  |
| Output Collector Power Dissipation | PO AV |  | 40 | mW |  |
| Fan Out (TTL) | N |  | 5 |  | Note 2 |

20

Electrical/Optical  Characteristics -40 °C to + 85 °C unless otherwise specified

Fiber sizes with core diameter ≤ 100 µm and NA ≤ 0.35, 4.75 V ≤ VCC ≤ 5.25 V

| Parameter | Symbol | Min | Typ3 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High Level Output Current | IOH |  | 5 | 250 | µA | VO = 18 |  |
|  |  |  |  |  |  | PR < -40 dBm |  |
| Low Level Output Voltage | VOL |  | 0.4 | 0.5 | V | IO = 8 mA PR > -24 dBm |  |
| High Level Supply Current | ICCH |  | 3.5 | 6.3 | mA | VCC = 5.25 V PR < -40 dBm |  |
| Low Level Supply Current | ICCL |  | 6.2 | 10 | mA | VCC = 5.25 V PR > -24 dBm |  |
| Equivalent NA | NA |  | 0.50 |  |  |  |  |
| Optical Port Diameter | D |  | 400 |  | µm |  | Note 4 |

Dynamic Characteristics

-40 °C to +85 °C unless otherwise specified; 4.75 V ≤ VCC ≤ 5.25 V; BER ≤ 10-9

| Parameter | Symbol | Min | Typ3 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peak Optical Input Power Logic Level HIGH | PRH |  |  | -40 0.1 | dBm pk µW pk | lP = 820 nm | Note 5 |
| Peak Optical Input Power Logic Level LOW | PRL | -25.4 2.9 |  | -9.2 120 | dBm pk µW pk | TA = +25 °C, IOL = 8mA | Note 5 |
|  |  | -24.0 4.0 |  | -10.0 100 | dBm pk µW pk | IOL = 8mA |  |
| Propagation Delay LOW to HIGH | tPLHR |  | 65 |  | ns | TA = +25 °C, | Note 6 |

PR = -21 dBm,

Propagation  Delay HIGH to LOW                         tPHLR                                                                           49                                      ns

Data Rate =

5 MBd

Notes:

1.   2.0 mm from where leads enter case.

2.   8 mA load  (5 x 1.6 mA), RL = 560 W.

3.   Typical data at TA = +25 °C, VCC = 5.0 Vdc.

4.   D is the effective diameter of the detector image on the plane of the fiber face. The numerical value is the product of the actual detector diameter and the lens magnification.

5.   Measured at the end of 100/140 mm fiber optic cable with large area detector.

6.   Propagation delay through the system is the result of several sequentially-occurring phenomena. Consequently it is a combination of data-rate- limiting effects and of transmission-time  effects. Because of this, the data-rate limit of the system must be described in terms of time differentials between delays imposed on falling and rising edges.

7.   As the cable length is increased, the propagation delays increase at 5 ns per meter of length. Data rate, as limited by pulse width distortion, is not

affected by increasing cable length if the optical power level at the receiver is maintained.

CAUTION: The small junction sizes inherent to the design of these components increase the components’ susceptibility to damage from electrostatic discharge (ESD). It is advised that normal static precautions be taken in handling and assembly of these components to prevent damage and/or degradation which may be induced by ESD.

21

HFBR-24x6 Low-Cost 125 MHz

Receiver

Description

The HFBR-24x6  fiber  optic receiver is designed to operate with  the  Agilent  HFBR-14xx fiber  optic  transmitters and  50/

preamplifier integrated circuit. The HFBR-24x6  receives an optical signal  and  converts it to an analog  voltage.  The output is a buffered emitter follower. Because the  signal  amplitude from  the  HFBR-24x6  receiver is much  larger than from  a simple

receiver from  noisy  host

systems. Refer  to AN 1038, 1065, or AB 78 for details.

Housed Product

6      Vcc

2       ANALOG SIGNAL

125 µm, 62.5/125 µm, 100/140

µm and  200 µm HCS® fiber  optic

PIN photodiode, it is less

susceptible to EMI, especially at

3 & 7

EE

V

cable.  Consistent coupling into the  receiver is assured by the lensed optical system (Figure 1). Response does  not  vary  with fiber size for core  diameters of

100 mm or less.

The receiver output is an analog signal  which  allows  follow-on

high signaling rates. For very noisy  environments, the conductive or metal  port option is recommended. A receiver dynamic range  of 23 dB over temperature is achievable (assuming 10-9 BER).

The frequency response is

4  5

3  6

2  7

1  8

BOTTOM VIEW

| NOTES: PIN | FUNCTION |
| --- | --- |
| 11 2 32 41 51 6 72 81 | NC SIGNAL VEE NC NC VCC VEE NC |

PIN 1 INDICATOR

circuitry to be optimized for a variety of distance/data rate requirements. Low-cost  external components can  be used  to convert the  analog  output to logic compatible signal  levels  for various data formats and  data rates up to 175 MBd. This

typically dc to 125 MHz. Although the  HFBR-24x6  is an analog  receiver, it is compatible with  digital  systems. Please refer to Application Bulletin 78 for simple and  inexpensive circuits that operate at 155 MBd or higher.

1. PINS 1, 4, 5 AND 8 ARE ISOLATED FROM THE INTERNAL

CIRCUITRY,  BUT ARE ELECTRICALLY CONNECTED  TO EACH OTHER.

2. PINS 3 AND 7 ARE ELECTRICALLY CONNECTED  TO HEADER

Unhoused Product

| 1 2 PIN | FUNCTION |
| --- | --- |
| 1 2 3 4 | SIGNAL VEE VCC VEE |

distance/data rate  trade-off results in increased optical

4       3

The recommended ac coupled

power budget at lower  data rates which  can  be used  for additional distance or splices.

The HFBR-24x6  receiver contains a PIN photodiode and low noise  transimpedance

receiver circuit is shown in

Figure  14. It is essential that a

10 ohm  resistor be connected between pin  6 and  the  power supply, and  a 0.1 mF ceramic bypass capacitor be connected between the  power supply and ground. In addition, pin  6 should be filtered to protect the

6

BOTTOM VIEW

BIAS  & FILTER CIRCUITS

VCC

POSITIVE SUPPLY

300 pF

2

VOUT

ANALOG SIGNAL

5.0 mA

Figure 13. Simplified Schematic Diagram.

3, 7

VEE

NEGATIVE SUPPLY

CAUTION: The small junction sizes inherent to the design of these components increase the components’ susceptibility to damage from electrostatic discharge (ESD). It is advised that normal static precautions be taken in handling and assembly of these components to prevent damage and/or degradation which may be induced by ESD.

22

Absolute Maximum Ratings

| Parameter | Symbol | Min | Max | Units | Reference |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -55 | +85 | °C |  |
| Operating Temperature | TA | -40 | +85 | °C |  |
| Lead Soldering Cycle |  |  |  |  | Note 1 |
| Temp |  |  | +260 | °C |  |
| Time |  |  | 10 | sec |  |
| Supply Voltage | VCC | -0.5 | 6.0 | V |  |
| Output Current | IO |  | 25 | mA |  |
| Signal Pin Voltage | VSIG | -0.5 | VCC | V |  |

Electrical/Optical  Characteristics -40 °C to +85 °C; 4.75 V ≤ Supply Voltage ≤ 5.25 V,

RLOAD = 511 W, Fiber sizes with core diameter ≤ 100 mm, and N.A. ≤ -0.35 unless otherwise specified.

| Parameter | Symbol | Min | Typ2 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Responsivity | RP | 5.3 | 7 | 9.6 | mV/µW | TA = +25 °C @ 820 nm, 50 MHz | Note 3, 4 Figure 18 |
|  |  | 4.5 |  | 11.5 | mV/µW | @ 820 nm, 50 MHz |  |
| RMS Output Noise Voltage | VNO |  | 0.40 | 0.59 | mV | Bandwidth filtered @ 75 MHz | Note 5 |
|  |  |  |  | 0.70 | mV | PR = 0 µW Unfiltered | Figure 15 |
|  |  |  |  |  |  | bandwidth |  |
|  |  |  |  |  |  | PR = 0 µW |  |
| Equivalent Input Optical Noise Power | PN |  | -43.0 | -41.4 | dBm | Bandwidth Filtered |  |
| (RMS) |  |  | 0.050 | 0.065 | µW | @ 75MHz |  |
| Optical Input Power (Overdrive) | PR |  |  | -7.6 175 | dBm pk µW pk | TA = +25 °C | Note 6 Figure 16 |
|  |  |  |  | -8.2 | dBm pk |  |  |
|  |  |  |  | 150 | µW pk |  |  |
| Output Impedance | ZO |  | 30 |  | W | Test Frequency = 50 MHz |  |
| dc Output Voltage | VO dc | -4.2 | -3.1 | -2.4 | V | PR = 0 µW |  |
| Power Supply Current | IEE |  | 9 | 15 | mA | RLOAD = 510 W |  |
| Equivalent NA | NA |  | 0.35 |  |  |  |  |
| Equivalent Diameter | D |  | 324 |  | µm |  | Note 7 |

CAUTION: The small junction sizes inherent to the design of these components increase the components’ susceptibility to damage from electrostatic discharge (ESD). It is advised that normal static precautions be taken in handling and assembly of these components to prevent damage and/or degradation which may be induced by ESD.

23

Dynamic Characteristics -40 °C to +85 °C; 4.75 V ≤ Supply Voltage ≤ 5.25 V; RLOAD = 511 W, CLOAD = 5 pF unless otherwise specified

| Parameter | Symbol | Min | Typ2 | Max | Units | Conditions | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rise/Fall Time 10% to 90% | tr, tf |  | 3.3 | 6.3 | ns | PR = 100 µW peak | Figure 17 |
| Pulse Width Distortion | PWD |  | 0.4 | 2.5 | ns | PR = 150 µW peak | Note 8, Figure 16 |
| Overshoot |  |  | 2 |  | % | PR = 5 µW peak, | Note 9 |
|  |  |  |  |  |  | tr = 1.5 ns |  |
| Bandwidth (Electrical) | BW |  | 125 |  | MHz | -3 dB Electrical |  |
| Bandwidth - Rise Time Product |  |  | 0.41 |  | Hz • s | Note 10 |  |
| Notes: |  |  |  |  |  |  |  |

1.   2.0 mm from where leads enter case.

2.   Typical specifications are for operation at TA = +25 °C and VCC = +5 V dc.

3.   For 200 µm HCS fibers, typical responsivity  will be 6 mV/mW. Other parameters will change as well.

4.   Pin #2 should be ac coupled to a load ³ 510 ohm. Load capacitance must be less than 5 pF.

5.   Measured with a 3 pole Bessel filter with a 75 MHz, -3 dB bandwidth. Recommended receiver filters for various bandwidths are provided in

Application Bulletin 78.

6.   Overdrive is defined at PWD = 2.5 ns.

7.   D is the effective diameter of the detector image on the plane of the fiber face. The numerical value is the product of the actual detector diameter and the lens magnification.

8.   Measured with a 10 ns pulse width, 50% duty cycle, at the 50% amplitude point of the waveform.

9.   Percent overshoot is defined as:

 VPK −



V100% 

x 100%

     V100%       

10. The conversion factor for the rise time to bandwidth is 0.41 since the HFBR-24x6 has a second order bandwidth limiting characteristic.

0.1 µF

+5 V

10 

6

30 pF

2

3 & 7

POST AMP

R LOADS

500  MIN.

LOGIC OUTPUT

Figure 14. Recommended ac Coupled Receiver Circuit. (See AB 78 and AN 1038 for more information.)

CAUTION: The small junction sizes inherent to the design of these components increase the components’ susceptibility to damage from electrostatic discharge (ESD). It is advised that normal static precautions be taken in handling and assembly of these components to prevent damage and/or degradation which may be induced by ESD.

24

150

3.0

6.0

SPECTRAL NOISE  DENSITY – nV/ H Z

125

100

75

50

25

PWD – PULSE  WIDTH  DISTORTION – ns

2.5

2.0

1.5

1.0

0.5

5.0

4.0

3.0

tr, tf – RESPONSE   TIME – ns

2.0

0

| 0 50 100 150 200 250 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

300

0

| 0 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

20      30      40     50

60     70     80

1.0

| -60 -40 -20 0 20 40 60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | tf |  |  |  |  |  |  |
|  | tr |  |  |  |  |  |  |

80    100

FREQUENCY – MHZ

P R – INPUT  OPTICAL POWER – µW

TEMPERATURE – ˚C

Figure 15. Typical Spectral Noise Density vs. Frequency.

Figure 16. Typical Pulse Width Distortion vs. Peak Input Power.

Figure 17. Typical Rise and Fall Times vs. Temperature.

1.25

1.00

0.75

0.50

NORMALIZED  RESPONSE

0.25

| 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

400    480    560    640    720     800

 – WAVELENGTH – nm

880

960   1040

Figure 18. Receiver Spectral Response

Normalized to 820 nm.

www.agilent.com/

semiconductors

For product information and a complete list of distributors, please go to our web site.

For technical assistance call: Americas/Canada: +1 (800) 235-0312 or (916) 788-6763

Europe: +49 (0) 6441 92460

China: 10800 650 0017

Hong Kong: (+65) 6756 2394

India, Australia, New Zealand: (+65) 6755 1939

Japan: (+81 3) 3335-8152(Domestic/International), or

0120-61-1280(Domestic Only) Korea: (+65) 6755 1989

Singapore, Malaysia, Vietnam, Thailand, Philippines, Indonesia: (+65) 6755 2044

Taiwan: (+65) 6755 1843

Data subject to change.

Copyright © 2003 Agilent Technologies, Inc. Obsoletes: 5980-1065E

August 11, 2003

5988-3624EN