HFBR-0501 Series

Versatile Link

The Versatile Fiber Optic Connection

Data Sheet

Description

The Versatile Link series is a complete family of fiber optic link components for applications requiring a low cost solution. The HFBR-0501 series includes transmitters, receivers, connectors and cable specified for easy design. This series of components is ideal for solving problems with voltage isolation/insulation, EMI/RFI immunity or data security. The optical link design is  simplified by the logic compatible receivers and complete specifications for each component. The key optical and electrical parameters of links configured with the HFBR-0501 family are fully guaranteed from 0 to

70C.

A wide variety of package configurations and connectors provide the designer with numerous mechanical solutions to meet application requirements. The transmitter and receiver components have been designed for use in high volume/low cost assembly processes such as  auto insertion and wave soldering.

Transmitters incorporate a  660 nm LED. Receivers include a monolithic dc coupled, digital IC receiver with open collector Schottky output transistor. An internal pullup resistor is available for use in the HFBR-25X1/2/4 receivers. A shield has been integrated into the receiver IC to provide additional, localized noise immunity.

Internal optics have been optimized for use with 1 mm diameter plastic optical fiber. Versatile Link specifications incorporate all connector interface losses. Therefore, optical calculations for common link applications are simplified.

Features

•  Low cost fiber optic components

•  Enhanced digital links dc-5 MBd

•  Extended distance links up to 120 m at 40 kBd

•  Low current link: 6 mA peak supply current

•  Horizontal and vertical mounting

•  Interlocking feature

•  High noise immunity

•  Easy connectoring Simplex, Duplex, and Latching connectors

•  Flame retardant

•  Transmitters incorporate a 660 nm Red LED for easy visibility

•  Compatible with standard TTL circuitry

Applications

•  Reduction of lightning/voltage transient susceptibility

•  Motor controller triggering

•  Data communications and local area networks

•  Electromagnetic Compatibility (EMC) for regulated systems: FCC, VDE, CSA, etc.

•  Tempest-secure data processing equipment

•  Isolation in test and measurement instruments

•  Error free signalling for industrial and manufacturing equipment

•  Automotive communications and control networks

•  Noise immune communication in audio and video equipment

HFBR-0501 Series Part Number Guide

HFBR X5XX

1 = Transmitter

2 = Receiver

5 = 600 nm Transmitter and

Receiver Products                                                        1 = 5 MBd High Performance Link

2 = 1 MBd High Performance Link

3 = 40 kBd Low Current/Extended Distance Link

2 = Horizontal Package                                                      4 = 1 MBd Standard Link

3 = Vertical Package                                                           6 = 155 MBd Receiver

7 = 155 MBd Transmitter

8 = 10 MBd High Performance Link

Link Selection Guide

(Links specified from 0 to 70C, for plastic optical fiber unless specified.)

| Signal Rate | Distance (m) 25C | Distance (m) | Transmitter | Receiver |
| --- | --- | --- | --- | --- |
| 40 kBd | 120 | 110 | HFBR-1523 | HFBR-2523 |
| 1 MBd | 20 | 10 | HFBR-1524 | HFBR-2524 |
| 1 MBd | 55 | 45 | HFBR-1522 | HFBR-2522 |
| 5 Mbd | 30 | 20 | HFBR-1521 | HFBR-2521 |

Evaluation Kit

HFBR-0501 1 MBd Versatile Link:

This  kit  contains: HFBR-1524 Tx,  HFBR-2524 Rx, polishing kit, 3 styles of plastic connectors, Bulkhead feedthrough, 5 meters of 1 mm diameter plastic cable, lapping film and grit paper, and HFBR-0501 data sheet.

Application Literature

Application Note 1035 (Versatile Link)

Package and Handling Information

The compact Versatile Link package is made of a flame retardant VALOX® UL 94 V-0 material (UL file # E121562) and uses the same pad layout as a standard, eight pin dual-in-line package. Vertical and horizontal mountable parts are available. These low profile Versatile Link packages are stackable and are enclosed to provide a dust  resistant seal.  Snap action simplex, simplex latching, duplex, and duplex latching connectors are offered with simplex or duplex cables.

Package Orientation

Performance and pinouts for the vertical and horizontal packages are identical. To provide additional attachment support for the vertical Versatile Link housing, the designer has the option of using a self-tapping screw through a printed circuit board into a mounting hole at the bottom of the package. For most applications this is not necessary.

Package Housing Color

Versatile Link components and simplex connectors are color coded to  eliminate confusion when making connections. Receivers are blue and transmitters are gray, except for the HFBR-15X3 transmitter, which is black.

VALOX® is a registered trademark of the General Electric Corporation.

2

Handling

Versatile Link components are auto-insertable. When wave  soldering is  performed with  Versatile Link components, the optical port plug should be left in to prevent contamination of the port. Do not use reflow

Recommended Chemicals for Cleaning/Degreasing Alcohols: methyl, isopropyl, isobutyl. Aliphatics: hexane, heptane, Other: soap solution, naphtha.

solder processes (i.e., infrared reflow or vapor-phase reflow).   Nonhalogenated  water   soluble   fluxes (i.e., 0% chloride), not rosin based fluxes, are recom-

CAUTION This bag contains MOISTURE-SENSITIVE DEVICES

Level

4

mended for use with Versatile Link components.

Versatile Link  components are  moisture sensitive devices and are shipped in a moisture sealed bag. If the components are exposed to air for an extended period of time, they may require a baking step before the soldering process. Refer to the special labeling on the shipping tube for details.

1. Shelf life in sealed bag: 12 months at <  40C and <  90% Relative

Humidity (RH).

2. After this bag is opened, devices that will be subjected to wave soldering, or  equivalent processing (solder temperature <  260C for

10 sec) must be:

a)  Mounted within 72 hours at factory conditions of  30C/60% RH. b) Stored at  20% RH.

3.  Devices require baking, before mounting, if:

a)  Desiccant changes to  PINK. b) If 2a or 2b are not met.

4. If  baking is required, devices may be baked outside of tube for 20 hours at  75C.

Bag Seal Date:

(If  blank, see barcode label)

Note: LEVEL defined by  EIA  JEDEC Standard J-STD-020

Do not use partially halogenated hydrocarbons such as

1,1.1 trichloroethane, ketones such as MEK, acetone,

chloroform, ethyl acetate, methylene dichloride, phenol,

methylene chloride, or N-methylpyrolldone. Also, Avago

does not recommend the use of  cleaners that use

halogenated hydrocarbons because of their potential

environmental harm.

Mechanical Dimensions

Horizontal Modules                                                                                                                     Vertical Modules

2.0 (0.080)

18.8                  (0.740)

6.8 (0.270)

10.2       (0.400)

5.1 (0.200)

7.6 (0.30)

0.64 (0.025)

77.7.61 (0(0.3.3005))

4.2 (0.165)

18.8

0.740

7.62 (0.300)

3.81 (0.150) MAX.

3.56 (0.140) MIN.

0.51

1.27 (0.050)

(0.020)

2.5 (0.100)

0.64 (0.025) DIA.

2.8 (0.109)

1.85 (0.073)

3

Versatile Link Printed Board Layout Dimensions

Horizontal Module                                                                                                                       Vertical Module

2.54                            (0.100)

7.62 (0.300)

1.01 (0.040) DIA.

TOP VIEW

4    3    2    1

7.62 (0.300)

5                  6

1.85   MIN. (0.073)

PCB EDGE

DIMENSIONS IN MILLIMETERS (INCHES).

Interlocked (Stacked) Assemblies (refer to Figure 1) Horizontal packages may be stacked by placing units with pins facing upward. Initially engage the inter- locking mechanism by sliding the L bracket body from

above into the L slot body of the lower package. Use a

straight edge, such as a ruler, to bring all stacked units

into  uniform alignment. This  technique prevents

potential harm that could occur to fingers and hands of

assemblers from the package pins. Stacked horizontal

packages can be disengaged if necessary. Repeated

stacking and unstacking causes no damage to individual

units.

To stack vertical packages, hold one unit in each hand, with the pins facing away and the optical ports on the bottom. Slide the L bracket unit into the L slot unit. The straight edge used for horizontal package alignment is not needed.

Stacking Horizontal Modules

Figure 1. Interlocked (stacked) horizontal or vertical packages

Stacking Vertical Modules

4

5 MBd Link (HFBR-15X1/25X1)

System Performance 0 to 70C unless otherwise specified.

|  |  | Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High | Data Rate |  | dc |  | 5 | MBd | BER ≤10-9, PRBS:27-1 |  |
| Performance 5 MBd |  |  |  |  |  |  |  |  |  |
|  |  | Link Distance (Standard Cable) |  | 19 27 | 48 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25C | Fig. 3 Note 3 |
|  |  | Link Distance (Improved Cable) |  | 22 27 | 53 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25C | Fig. 4 Note 3 |
|  |  | Propagation Delay | tPLH tPHL |  | 80 50 | 140 140 | ns ns | RL = 560 Ω, CL = 30 pF fiber length = 0.5 m -21.6 ≤PR ≤-9.5 dBm | Fig. 5, 8 Notes 1, 2 |
|  |  | Pulse Width Distortion tPLH-tPHL | tD |  | 30 |  | ns | PR = -15 dBm RL = 560 Ω, CL = 30 pF | Fig. 5, 7 |

Notes:

1. The propagation delay for one metre of cable is typically 5 ns.

2. Typical propagation delay is measured at PR = -15 dBm.

3. Estimated typical link life expectancy at 40C exceeds 10 years at 60 mA.

Figure 2. Typical 5 MBd interface circuit

100

100

I F – FORWARD CURRENT (mA)

50

40

OVERDRIVE

30

UNDERDRIVE

20

10

I F – FORWARD CURRENT (mA)

50

40

OVERDRIVE

30

UNDERDRIVE

20

10

0°C–70°C

25°C

5

0            10           20           30           40           50

0°C–70°C

25°C

5

0         10         20         30        40         50        60

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 3. Guaranteed system performance with standard cable (HFBR-15X1/25X1)

Figure 4. Guaranteed system performance with improved cable (HFBR-15X1/25X1)

5

Figure 5. 5 MBd propagation delay test circuit

Figure 6. Propagation delay test waveforms

500

500

tD – PULSE WIDTH DISTORTION – ns

400

300

HFBR-15X2/25X2

HFBR-15X4/25X4

70°C

25°C

0°C

400

300

200

200

100

0

HFBR-15X1/25X1

70°C

25°C

0°C

100

tp – PROPAGATION DELAY – ns

0

-25

-20          -15

-10           -5             0

-25

-20          -15

| -10 -5 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | tp | LH |  |  |
| HFBR-15X2/25X2 HFBR-15X4/25X4 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | tp | LH |  |  |
|  |  | HFB | R-15 | X1/2 | 5X1 |  |  |  |  |
|  |  |  |  |  |  | tp | HL |  |  |

PR – INPUT OPTICAL POWER – dBm

PR – INPUT OPTICAL POWER – dBm

Figure 7. Typical link pulse width distortion vs. optical power

Figure 8. Typical link propagation delay vs. optical power

6

HFBR-15X1 Transmitter

ANODE    1

CATHODE    2

N.C.    3

N.C.    4

8  DO NOT CONNECT

5  DO NOT CONNECT

Pin #               Function

1                       Anode

2                       Cathode

3                       Open

4                       Open

5                       Do not connect

8                       Do not connect

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these pins.

Absolute Maximum Ratings

Parameter                                               Symbol               Min.           Max.          Units          Reference

Storage Temperature                                          TS                            –40               +85               C Operating Temperature                                       TA                              –40               +85               C

Lead Soldering Cycle            Temp.                                                               260               C                 Note 1

Time                                                                  10               sec

Forward Input Current                                       IFPK                                                            1000             mA              Note 2, 3

IFdc                                                          80

Reverse Input Voltage                                         VBR                                                                5                  V

Notes:

1. 1.6 mm below seating plane.

2. Recommended operating range between 10 and 750 mA.

3. 1 s pulse, 20 s period.

All HFBR-15XX  LED transmitters  are classified as IEC 825-1 Accessible  Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go into effect on January 1, 1997. AEL Class 1 LED devices are considered eye safe. Contact your local Avago sales representative for more information.

7

Transmitter Electrical/Optical Characteristics 0C to 70C unless otherwise specified.

| Parameter | Symbol | Min. | Typ.[5] | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Output Optical Power | PT | -16.5 |  | -7.6 | dBm | IFdc = 60 mA | Notes 1, 2 |
|  |  | -14.3 |  | -8.0 | dBm | IFdc = 60 mA, 25C |  |
| Output Optical Power Temperature Coefficient | ∆PT /∆T |  | -0.85 |  | %/C |  |  |
| Peak Emission Wavelength | PK |  | 660 |  | nm |  |  |
| Forward Voltage | VF | 1.45 | 1.67 | 2.02 | V | IFdc = 60 mA |  |
| Forward Voltage Temperature Coefficient | ∆VF /∆T |  | -1.37 |  | mV/C |  | Fig. 9 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |
| Reverse Input Breakdown Voltage | VBR | 5.0 | 11.0 |  | V | IFdc = 10 A, TA = 25C |  |
| Diode Capacitance | CO |  | 86 |  | pF | VF = 0, f = MHz |  |
| Rise Time | tr |  | 80 |  | ns | 10% to 90%, | Note 3 |
| Fall Time | tf |  | 40 |  | ns |  |  |
|  |  |  |  |  |  | IF = 60 mA |  |

Notes:

1. Measured at the end of 0.5 m standard fiber optic cable with large area detector.

2. Optical power, P (dBm) = 10 Log [P(W)/1000  W].

3. Rise and fall times are measured with a voltage pulse driving the transmitter and a series connected 50  load. A wide bandwidth optical to electrical waveform analyzer, terminated to a 50  input of a wide bandwidth oscilloscope, is used for this response time measurement.

1.8                                                                                                                                                                 5

0

VF – FORWARD VOLTAGE – V

1.7

1.6

0°C

25°C

1.5

70°C

-15

1.4

2                             10

100

PT – NORMALIZED OUTPUT POWER – dB

-20

| 2 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

100

IFdc – TRANSMITTER DRIVE CURRENT (mA)

IFdc – TRANSMITTER DRIVE CURRENT (mA)

Figure 9. Typical forward voltage vs. drive current                                                                                      Figure 10. Normalized typical output power vs. drive current

8

HFBR-25X1 Receiver

DO NOT CONNECT      5

DO NOT CONNECT      8

1000 

4    RL

3    VCC

2    GROUND

1    VO

Pin #               Function

1                       VO

2                       Ground

3                       VCC

4                       RL

5                       Do not connect

8                       Do not connect

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these pins.

Absolute Maximum Ratings

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Supply Voltage |  | VCC | –0.5 | 7 | V | Note 2 |
| Output Collector Current |  | IOAV |  | 25 | mA |  |
| Output Collector Power Dissipation |  | POD |  | 40 | mW |  |
| Output Voltage |  | VO | –0.5 | 18 | V |  |
| Pull-up Voltage |  | VP | –5 | VCC | V |  |
| Fan Out (TTL) |  | N |  | 5 |  |  |

Notes:

1. 1.6 mm below seating plane.

2. It is essential that a bypass capacitor 0.01 F be connected from pin 2 to pin 3 of the receiver. Total lead length between both ends of the capacitor and the pins should not exceed 20 mm.

Receiver Electrical/Optical Characteristics

0C to 70C, 4.75 V VCC 5.25 V unless otherwise specified

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input Optical Power Level for Logic “0” | PR(L) | –21.6 |  | –9.5 | dBm | VOL = 0.5 V IOL = 8 mA | Notes 1, 2, 4 |  |
|  |  | –21.6 |  | –8.7 |  | VOL = 0.5 V IOL = 8 mA, 25C |  |  |
| Input Optical Power Level for Logic “1” | PR(H) |  |  | –43 | dBm | VOL = 5.25 V IOH ≤250 A | Note 1 |  |
| High Level Output Current | IOH |  | 5 | 250 | A | VO = 18 V, PR = 0 | Note 3 |  |
| Low Level Output Current | VOL |  | 0.4 | 0.5 | V | IOL = 8 mA, PR = PR(L)MIN | Note 3 |  |
| High Level Supply Current | ICCH |  | 3.5 | 6.3 | mA | VCC = 5.25 V, PR = 0 | Note 3 |  |
| Low Level Supply Current | ICCL |  | 6.2 | 10 | mA | VCC = 5.25 V PR = -12.5 dBm | Note 3 |  |
| Effective Diameter | D |  | 1 |  | mm |  |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |  |
| Internal Pull-up Resistor | RL | 680 | 1000 | 1700 | Ω |  |  |  |

Notes:

1. Optical flux, P (dBm) = 10 Log [P (W)/1000 W].

2. Measured at the end of the fiber optic cable with large area detector.

3. RL is open.

4. Pulsed LED operation at IF > 80 mA will cause increased link tPLH propagation  delay time. This extended tPLH time contributes to increased pulse width distortion of the receiver output signal.

9

1 MBd Link

(High Performance HFBR-15X2/25X2, Standard HFBR-15X4/25X4)

System Performance  Under recommended operating conditions unless otherwise specified.

|  |  | Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High | Data Rate |  | dc |  | 1 | MBd | BER ≤10-9, PRBS:27-1 |  |
| Performance 1 MBd |  | Link Distance (Standard Cable) |  | 39 47 | 70 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25C | Fig. 14 Notes 1, 3, 4 |
|  |  | Link Distance (Improved Cable) |  | 45 56 | 78 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25C | Fig. 15 Notes 1, 3, 4 |
|  |  | Propagation Delay | tPLH tPHL |  | 180 100 | 250 140 | ns ns | RL = 560 Ω, CL = 30 pF I = 0.5 metre PR = -24 dBm | Fig. 16, 18 Notes 2, 4 |
|  |  | Pulse Width Distortion tPLH-tPHL | tD |  | 80 |  | ns | PR = -24 dBm RL = 560 Ω, CL = 30 pF | Fig. 16, 17 Note 4 |

|  | Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standard | Data Rate |  | dc |  | 1 | MBd | BER ≤10-9, PRBS:27-1 |  |
| 1 MBd | Link Distance (Standard Cable) |  | 8 17 | 43 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25C | Fig. 12 Notes 1, 3, 4 |
|  | Link Distance (Improved Cable) |  | 10 19 | 48 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25C | Fig. 13 Notes 1, 3, 4 |
|  | Propagation Delay | tPLH tPHL |  | 180 100 | 250 140 | ns ns | RL = 560 Ω, CL = 30 pF I = 0.5 metre PR = -20 dBm | Fig. 16, 18 Notes 2, 4 |
|  | Pulse Width Distortion tPLH-tPHL | tD |  | 80 |  | ns | PR = -20 dBm RL = 560 Ω, CL = 30 pF | Fig. 16, 17 Note 4 |

Notes:

1. For IFPK > 80 mA, the duty factor must be such as to keep IFdc 80 mA. In addition, for IFPK > 80 mA, the following rules for pulse width apply: IFPK 160 mA: Pulse width 1 ms

IFPK > 160 mA: Pulse width 1 S, period 20 S.

2. The propagation delay for one meter of cable is typically 5 ns.

3. Estimated typical link life expectancy  at 40C exceeds 10 years at 60 mA.

4. Pulsed LED operation at IFPK > 80 mA will cause increased link tPLH propagation  delay time. This extended tPLH time contributes to increased pulse width distortion of the receiver output signal.

10

Figure 11. Required 1 MBd interface circuit

The HFBR-25X2 receiver can not be overdriven when using the required interface circuit shown in Figure 11.

100

90

I F – FORWARD CURRENT (mA)

80

70

60

100

90

I F – FORWARD CURRENT (mA)

80

70

60

50                                                                                                                                                              50

40                                                                                                                                                              40

30                                                                                                                                                              30

HFBR-15X4/25X4

20

| 0 5 10 15 20 25 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | HFBR-15X4/25X4 |  |  |
|  |  | 0°C–70°C 25°C |  |  |

20

0                      10

0°C–70°C

25°C

20                     30

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 12. Guaranteed system performance for the HFBR-15X4/25X4 Link with standard cable

Figure 13. Guaranteed system performance for the HFBR-15X4/25X4 Link with improved cable

100

50

40

30

20

100

50

40

30

20

UNDERDRIVE

UNDERDRIVE

IF – FORWARD CURRENT (mA)

IF – FORWARD CURRENT (mA)

10                                                                                                                                                                10

0°C–70°C

25°C

5

0           10           20           30           40           50

0°C–70°C

25°C

5

0        10         20         30        40         50        60

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 14. Guaranteed system performance for the HFBR-15X2/25X2 Link with standard cable

Figure 15. Guaranteed system performance for the HFBR-15X2/25X2 Link with improved cable

11

Figure 16. 1 MBd propagation delay test circuit

500

500

tD – PULSE WIDTH DISTORTION – ns

400

300

HFBR-15X2/25X2

HFBR-15X4/25X4

70°C

25°C

0°C

tp – PROPAGATION DELAY – ns

400

300

200

200

100

0

HFBR-15X1/25X1

70°C

25°C

0°C

100

0

-25

-20          -15

-10           -5             0

-25

-20          -15

| -10 -5 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | tp | LH |  |  |
| HFBR-15X2/25X2 HFBR-15X4/25X4 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | tp | LH |  |  |
|  |  | HFB | R-15 | X1/2 | 5X1 |  |  |  |  |
|  |  |  |  |  |  | tp | HL |  |  |

PR – INPUT OPTICAL POWER – dBm

PR – INPUT OPTICAL POWER – dBm

Figure 17. Pulse width distortion vs. optical power

Figure 18. Typical link propagation delay vs. optical power

Figure 19. Propagation delay test waveforms

12

HFBR-15X2/15X4 Transmitters

8  DO NOT CONNECT

Pin #               Function

1                       Anode

2                       Cathode

ANODE    1

3                       Open

CATHODE    2

N.C.    3

N.C.    4

5  DO NOT CONNECT

4                       Open

5                       Do not connect

8                       Do not connect

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |  |
|  | Time |  |  | 10 | sec |  |  |
| Forward Input Current |  | IFPK |  | 1000 | mA | Note 2, 3 |  |
|  |  | IFdc |  | 80 |  |  |  |
| Reverse Input Voltage |  | VBR |  | 5 | V |  |  |

Notes:

1. 1.6 mm below seating plane.

2. Recommended operating range between 10 and 750 mA.

3. 1 s pulse, 20 s period.

All HFBR-15XX  LED transmitters  are classified as IEC 825-1 Accessible  Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go into effect on January 1, 1997. AEL Class 1 LED devices are considered eye safe. Contact your Avago sales representative for more information.

Transmitter Electrical/Optical Characteristics  0C to 70C unless otherwise specified. For forward voltage and output power vs. drive current graphs.

| IF = 60 mA Parameter |  | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Output | HFBR-15X2 | PT | –13.6 –11.2 |  | –4.5 –5.1 | dBm | IFdc = 60 mA IFdc = 60 mA, 25C |  |
| Optical Power | HFBR-15X4 | PT | –17.8 –15.5 |  | –4.5 –5.1 | dBm | IFdc = 60 mA IFdc = 60 mA, 25C |  |
| Output Optical Power Temperature Coefficient |  | ∆PT /∆T |  | –0.85 |  | %/C |  |  |
| Peak Emission Wavelength |  | PK |  | 660 |  | nm |  |  |
| Forward Voltage |  | VF | 1.45 | 1.67 | 2.02 | V | IFdc = 60 mA |  |
| Forward Voltage Temperature Coefficient |  | ∆VF /∆T |  | –1.37 |  | mV/C |  | Fig. 11 |
| Effective Diameter |  | DT |  | 1 |  | mm |  |  |
| Numerical Aperture |  | NA |  | 0.5 |  |  |  |  |
| Reverse Input Breakdown Voltage |  | VBR | 5.0 | 11.0 |  | V | IFdc = 10 A, TA = 25C |  |
| Diode Capacitance |  | CO |  | 86 |  | pF | VF = 0, f = 1 MHz |  |
| Rise Time |  | tr |  | 80 |  | ns | 10% to 90%, | Note 1 |
| Fall Time |  | tf |  | 40 |  | ns |  |  |

Note:

1. Rise and fall times are measured with a voltage pulse driving the transmitter and a series connected 50  load. A wide bandwidth optical to electrical waveform analyzer, terminated to a 50  input of a wide bandwidth oscilloscope, is used for this response time measurement.

13

HFBR-25X2/25X4 Receivers

DO NOT CONNECT      5

DO NOT CONNECT      8

1000 

4    RL

3    VCC

2    GROUND

1    VO

Pin #               Function

1                       VO

2                       Ground

3                       VCC

4                       RL

5                       Do not connect

8                       Do not connect

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Supply Voltage |  | VCC | –0.5 | 7 | V | Note 2 |
| Output Collector Current |  | IOAV |  | 25 | mA |  |
| Output Collector Power Dissipation |  | POD |  | 40 | mW |  |
| Output Voltage |  | VO | –0.5 | 18 | V |  |
| Pull-up Voltage |  | VP | –5 | VCC | V |  |
| Fan Out (TTL) |  | N |  | 5 |  |  |

Notes:

1. 1.6 mm below seating plane.

2. It is essential that a bypass capacitor 0.01 F be connected from pin 2 to pin 3 of the receiver. Total lead length between both ends of the capacitor and the pins should not exceed 20 mm.

Receiver Electrical/Optical Characteristics  0C to 70C, 4.75 V VCC 5.25 V unless otherwise specified.

| Parameter |  | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Receiver Optical Input | HFBR-2522 | PR(L) | –24 |  |  | dBm | VOL = 0 V IOL = 8 mA | Notes 1, 2, 3 |
| Power Level Logic 0 | HFBR-2524 |  | –20 |  |  |  |  | Note 4 |
| Optical Input Power Level Logic 1 |  | PR(H) |  |  | -43 | dBm | VOH = 5.25 V IOH = ≤250 A |  |
| High Level Output Current |  | IOH |  | 5 | 250 | A | VO = 18 V, PR = 0 | Note 5 |
| Low Level Output Voltage |  | VOL |  | 0.4 | 0.5 | V | IOL = 8 mA PR = PR(L)MIN | Note 5 |
| High Level Supply Current |  | ICCH |  | 3.5 | 6.3 | mA | VCC = 5.25 V, PR = 0 | Note 5 |
| Low Level Supply Current |  | ICCL |  | 6.2 | 10 | mA | VCC = 5.25 V, PR = -12.5 dBm | Note 5 |
| Effective Diameter |  | D |  | 1 |  | mm |  |  |
| Numerical Aperture |  | NA |  | 0.5 |  |  |  |  |
| Internal Pull-up Resistor |  | RL | 680 | 1000 | 1700 | Ω |  |  |

Notes:

1. Measured at the end of the fiber optic cable with large area detector.

2. Pulsed LED operation at IF > 80 mA will cause increased link tPLH propagation  delay time. This extended tPLH time contributes to increased pulse width distortion of the receiver output signal.

3. The LED drive circuit of Figure 11 is required for 1 MBd operation of the HFBR-25X2/25X4.

4. Optical flux, P (dBm) = 10 Log [P(W)/1000  W].

5. RL is open.

14

40 kBd Link

System Performance  Under recommended operating conditions unless otherwise specified.

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data Rate |  | dc |  | 40 | kBd | BER ≤10-9, PRBS: 27 - 1 |  |
| Link Distance (Standard Cable) |  | 13 94 | 41 138 |  | m m | IFdc = 2 mA IFdc = 60 mA | Fig. 21 Note 1 |
| Link Distance (Improved Cable) |  | 15 111 | 45 154 |  | m m | IFdc = 2 mA IFdc = 60 mA | Fig. 22 Note 1 |
| Propagation Delay | tPLH tPHL |  | 4 2.5 |  | s s | RL = 3.3 kΩ, CL = 30 pF PR = -25 dBm, 1 m fiber | Fig. 22, 25 Note 2 |
| Pulse Width Distortion tPLH-tPHL | tD |  |  | 7 | s | -39 ≤PR≤- 14 dBm RL = 3.3 kΩ, CL = 30 pF | Fig. 23, 24 |

Notes:

1. Estimated typical link life expectancy  at 40C exceeds 10 years at 60 mA.

2. The propagation delay for one metre of cable is typically 5 ns.

Figure 20. Typical 40 kBd interface circuit

120

100

80

60

I F – FORWARD CURRENT (mA)

40

120

100

80

I F – FORWARD CURRENT (mA)

60

40

20

20

10

6                                                                                                                                                               10

HFBR-15X3/25X3

2

0°C–70°C

25°C

1

6

HFBR-15X3/25X3

4

0°C–70°C

25°C

2

0    10   20   30   40   50   60

70   80   90  100

0   10   20  30   40  50

60  70  80   90  100 110

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 21. Guaranteed system performance with standard cable

Figure 22. Guaranteed system performance with improved cable

15

Figure 23. 40 kBd propagation delay test circuit

| 6 8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

t D – PULSE WIDTH DISTORTION – µs

5                                                                                                                                                                 7

6

4

5

t P – PROPAGATION DELAY – µs

3                                                                                                                                                                 4

3

2

2

1

1

0

-40

-34

-28

-22         -16

-10

0

-40

-34

-28

-22         -16

| -10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | tPLH |  |  |  |  |  |  |
|  |  |  | tPHL |  |  |  |  |  |  |

PR – INPUT OPTICAL POWER, dBm

PR – INPUT OPTICAL POWER, dBm

Figure 24. Typical link pulse width distortion vs. optical power

Figure 25. Typical link propagation delay vs. optical power

Figure 26. Propagation delay test waveforms

16

HFBR-15X3 Transmitter

ANODE    1

CATHODE    2

N.C.    3

N.C.    4

8  DO NOT CONNECT

5  DO NOT CONNECT

Pin #               Function

1              Anode

2              Cathode

3              Open

4              Open

5              Do not connect

8              Do not connect

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Forward Input Current |  | IFPK |  | 1000 | mA | Note 2, 3 |
|  |  | IFdc |  | 80 |  |  |
| Reverse Input Voltage |  | VBR |  | 5 | V |  |

Notes:

1. 1.6 mm below seating plane.

2. Recommended operating range between 10 and 750 mA.

3. 1 s pulse, 20 s period.

All HFBR-15XX  LED transmitters  are classified as IEC 825-1 Accessible  Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go into effect on January 1, 1997. AEL Class 1 LED devices are considered eye safe. Contact your Avago sales representative for more information.

Transmitter Electrical/Optical Characteristics  0C to 70C unless otherwise specified. For forward voltage and output power vs. drive current graphs.

| IF = 60 mA Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Output Optical Power | PT | –11.2 –13.6 –35.5 |  | –5.1 –4.5 | dBm | IFdc = 60 mA, 25C IFdc = 60 mA IFdc = 2 mA, 0-70C | Notes 3, 4 Fig. 9, 10 |
| Output Optical Power Temperature Coefficient | ∆PT /∆T |  | –0.85 |  | %/C |  |  |
| Peak Emission Wavelength | PK |  | 660 |  | nm |  |  |
| Forward Voltage | VF | 1.45 | 1.67 | 2.02 | V | IFdc = 60 mA |  |
| Forward Voltage Temperature Coefficient | ∆VF /∆T |  | –1.37 |  | mV/C |  | Fig. 18 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |
| Reverse Input Breakdown Voltage | VBR | 5.0 | 11.0 |  | V | IFdc = 10 A, TA = 25C |  |
| Diode Capacitance | CO |  | 86 |  | pF | VF = 0, f = 1 MHz |  |
| Rise Time | tr |  | 80 |  | ns | 10% to 90%, | Note 1 |
| Fall Time | tf |  | 40 |  |  |  |  |

Note:

1. Rise and fall times are measured with a voltage pulse driving the transmitter and a series connected 50  load. A wide bandwidth optical to electrical waveform analyzer, terminated to a 50  input of a wide bandwidth oscilloscope, is used for this response time measurement.

17

HFBR-25X3 Receiver

DO NOT CONNECT      5

DO NOT CONNECT      8

4    VCC

3    OPEN

2    GROUND

1    VO

Pin #               Function

1                       VO

2                       Ground

3                       Open

4                       VCC

5                       Do not connect

8                       Do not connect

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these pins.

Absolute Maximum Ratings

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Supply Voltage |  | VCC | –0.5 | 7 | V | Note 2 |
| Average Output Collector Current |  | IO | –1 | 5 | mA |  |
| Output Collector Power Dissipation |  | POD |  | 25 | mW |  |
| Output Voltage |  | VO | –0.5 | 7 | V |  |

Notes:

1. 1.6 mm below seating plane.

2. It is essential that a bypass capacitor 0.01 F be connected from pin 2 to pin 3 of the receiver.

Receiver Electrical/Optical Characteristics  0C to 70C, 4.5 V VCC 5.5 V unless otherwise specified.

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input Optical Power | PR(L) | –39 |  | –13.7 | dBm | VO = VOL, IOL = 3.2 mA | Notes 1, 2, 3 |
| Level Logic 0 |  | –39 |  | –13.3 |  | VO = VOL, IOH = 8 mA, 25C |  |
| Input Optical Power Level Logic 1 | PR(H) |  |  | –53 | dBm | VOH = 5.5 V IOH = ≤40 A | Note 3 |
| High Level Output Voltage | VOH | 2.4 |  |  | V | IO = - 40 A, PR = 0 W |  |
| Low Level Output Voltage | VOL |  |  | 0.4 | V | IOL = 3.2 mA PR = PR(L)MIN | Note 4 |
| High Level Supply Current | ICCH |  | 1.2 | 1.9 | mA | VCC = 5.5 V, PR = 0 W |  |
| Low Level Supply Current | ICCL |  | 2.9 | 3.7 | mA | VCC = 5.5 V, PR = PRL (MIN) | Note 4 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |

Notes:

1. Measured at the end of the fiber optic cable with large area detector.

2. Optical flux, P (dBm) = 10 Log P(W)/1000  W.

3. Because of the very high sensitivity of the HFBR-25X3, the digital output may switch in response to ambient light levels when a cable is not occupying the receiver optical port. The designer should take care to filter out signals from this source if they pose a hazard to the system.

4. Including current in 3.3 k pull-up resistor.

18

For product information  and a complete list of distributors,  please go to our website:          www.avagotech.com

Avago, Avago Technologies, and the A logo are trademarks of Avago Technologies Limited in the United States and other countries. Data subject to change. Copyright © 2008 Avago Technologies Limited.  All rights reserved. Obsoletes 5968-1712E

5988-1765EN  January 30, 2008