Versatile Link

The Versatile Fiber  Optic

Connection

Technical  Data

HFBR-0501 Series

Features

• Low Cost  Fiber  Optic

Components

• Enhanced Digital Links dc-5 MBd

• Extended Distance Links  up to 120  m at 40 kBd

• Low Current Link: 6 mA Peak  Supply  Current

• Horizontal and Vertical

Mounting

• Interlocking Feature

• High Noise Immunity

• Easy  Connectoring Simplex, Duplex, and Latching Connectors

• Flame Retardant

• Transmitters Incorporate a

660  nm Red LED for Easy

Visibility

• Compatible with Standard

TTL Circuitry

Applications

• Reduction of Lightning/Volt- age Transient Susceptibility

• Motor  Controller Triggering

• Data  Communications and

Local Area Networks

• Electromagnetic Compatibility (EMC)  for Regulated Systems: FCC, VDE, CSA, etc.

• Tempest-Secure Data

Processing Equipment

• Isolation in Test  and

Measurement Instruments

• Error Free  Signalling for Industrial and Manufactur- ing Equipment

• Automotive Communications and Control Networks

• Noise Immune Communica- tion  in Audio  and Video Equipment

Description

The Versatile Link series  is a complete family of fiber optic link components for applications requiring a low cost solution. The HFBR-0501  series  includes  trans- mitters, receivers, connectors and cable specified  for easy design. This series  of components is ideal for solving problems with voltage isolation/insulation, EMI/RFI immunity  or data security. The optical  link design  is simplified

by the logic compatible receivers and complete specifications for each component. The key optical and electrical parameters of links configured with the HFBR-0501 family are fully guaranteed from

0  to 70 C.

A wide variety of package config- urations and connectors provide the designer with numerous mechanical solutions to meet application requirements. The

transmitter and receiver  compo- nents  have been designed for use in high volume/low cost assembly processes such as auto insertion and wave soldering.

Transmitters incorporate a 660 nm LED. Receivers  include a monolithic dc coupled, digital IC receiver  with open  collector Schottky  output transistor. An internal pullup resistor is avail- able for use in the HFBR-25X1/2/

4 receivers. A shield has been integrated into the receiver  IC to provide  additional, localized noise immunity.

Internal optics  have been optim- ized for use with 1 mm diameter plastic  optical  fiber. Versatile Link specifications incorporate all connector interface losses.

Therefore, optical  calculations for common  link applications are simplified.

2

HFBR-0501 Series Part Number Guide

HFBR X5XX

1 = Transmitter

2 = Receiver

5 = 600 nm Transmitter and

Receiver  Products                                          1 = 5 MBd High Performance Link

2 = 1 MBd High Performance Link

3 = 40 kBd Low Current/Extended Distance  Link

2 = Horizontal  Package                                             4 = 1 MBd Standard Link

3 = Vertical Package                                                  6 = 155 MBd Receiver

7 = 155 MBd Transmitter

8 = 10 MBd High Performance Link

Link Selection Guide

(Links specified  from 0 to 70  C, for plastic  optical  fiber unless  specified.)

| Signal Rate | Distance (m) 25 C | Distance (m) | Transmitter | Receiver |
| --- | --- | --- | --- | --- |
| 40 kBd | 120 | 110 | HFBR-1523 | HFBR-2523 |
| 1 MBd | 20 | 10 | HFBR-1524 | HFBR-2524 |
| 1 MBd | 55 | 45 | HFBR-1522 | HFBR-2522 |
| 5 Mbd | 30 | 20 | HFBR-1521 | HFBR-2521 |

Evaluation Kit

HFBR-0501 1 MBd Versatile Link:

This kit contains: HFBR-1524  Tx, HFBR-2524  Rx, polishing  kit, 3 styles of plastic  connectors, Bulkhead feedthrough, 5 meters  of 1 mm diameter plastic  cable,  lapping  film and grit paper, and HFBR-0501  data sheet.

Application Literature

Application  Note 1035  (Versatile  Link)

Package and Handling

Information

The compact Versatile Link pack- age is made of a flame retardant VALOX® UL 94 V-0 material

(UL file # E121562) and uses the same pad layout as a standard, eight pin dual-in-line  package. Vertical and horizontal mountable parts  are available.  These low profile Versatile Link packages

are stackable and are enclosed to

provide  a dust resistant seal. Snap action  simplex,  simplex latching, duplex,  and duplex latching  connectors are offered with simplex  or duplex  cables.

Package Orientation Performance and pinouts  for the vertical  and horizontal packages

are identical.  To provide  addi-

tional attachment support for the vertical  Versatile Link housing,

the designer has the option  of using a self-tapping screw through a printed circuit  board into a mounting hole at the bottom of the package. For most applications this is not necessary.

Package Housing Color Versatile Link components and simplex  connectors are color

coded  to eliminate  confusion

VALOX® is a registered trademark of the General  Electric  Corporation.

3

when making connections. Receivers  are blue and transmit- ters are gray, except  for the

HFBR-15X3 transmitter, which is black.

Handling

Versatile Link components are auto-insertable. When wave soldering is performed with Versatile Link components, the

Versatile Link components are moisture sensitive  devices and are shipped in a moisture sealed bag. If the components are exposed to air for an extended period  of time, they may require a baking step before  the solder- ing process. Refer to the special labeling  on the shipping tube for details.

Recommended Chemicals for

Cleaning/Degreasing Alcohols:  methyl, isopropyl, isobutyl.  Aliphatics: hexane,

heptane, Other:  soap solution,

naphtha.

Do not use  partially  halogenated hydrocarbons such as 1,1.1 trichloroethane, ketones such as MEK, acetone, chloroform, ethyl

optical  port  plug should be left in to prevent  contamination of the port.  Do not use reflow solder

CAUTION This bag contains MOISTURE-SENSITIVE DEVICES

Level

4

acetate, methylene dichloride, phenol,  methylene chloride, or N- methylpyrolldone. Also, Agilent

processes (i.e., infrared reflow or vapor-phase reflow). Nonhalogenated water soluble fluxes (i.e., 0% chloride), not rosin based  fluxes, are recom- mended  for use with Versatile Link components.

1. Shelf life in sealed bag: 12 months at < 40  C and < 90% Relative

Humidity (RH).

2. After this  bag is opened, devices that will be subjected to wave soldering, or equivalent processing (solder temperature < 260  C for

10 sec)  must be:

a) Mounted within 72 hours at factory conditions of    30  C/60% RH. b) Stored at     20% RH.

3. Devices require baking, before mounting, if:

a) Desiccant changes to PINK. b) If 2a or 2b are  not  met.

4. If baking is required, devices may be baked outside of tube for 20 hours at 75 C.

Bag Seal Date:                                                                                                              (If blank, see barcode label)

Note: LEVEL defined by EIA JEDEC Standard J-STD-020

does not recommend the use of cleaners that use halogenated hydrocarbons because of their potential environmental harm.

Mechanical Dimensions

Horizontal Modules                                                                                      Vertical Modules

2.0 (0.080)

18.8                  (0.740)

6.8 (0.270)

10.2       (0.400)

5.1 (0.200)

7.6 (0.30)

0.64 (0.025)

77.7.61 (0(0.3.3005))

4.2 (0.165)

18.8

0.740

7.62 (0.300)

3.81 (0.150) MAX.

3.56 (0.140) MIN.

0.51

1.27 (0.050)

(0.020)

2.5 (0.100)

0.64 (0.025) DIA.

2.8 (0.109)

1.85 (0.073)

4

Versatile Link Printed Board  Layout  Dimensions

Horizontal Module                                                                                                                              Vertical Module

2.54                            (0.100)

7.62 (0.300)

1.01 (0.040) DIA.

TOP VIEW

4    3    2    1

7.62 (0.300)

5                  6                                        PCB EDGE

1.85   MIN. (0.073)

DIMENSIONS IN MILLIMETERS (INCHES).

Interlocked (Stacked) Assemblies (refer to Figure 1)

Horizontal  packages may be stacked by placing  units with pins facing upward.  Initially engage

the interlocking mechanism by sliding the L bracket body from above into the L slot body of the lower package. Use a straight

edge, such as a ruler,  to bring  all stacked units into uniform alignment. This technique prevents potential harm that could occur  to fingers  and hands of assemblers from the package

pins. Stacked  horizontal packages can be disengaged if necessary. Repeated stacking  and unstack- ing causes  no damage  to individual units.

To stack  vertical  packages, hold one unit in each hand,  with the pins facing away and the optical ports  on the bottom. Slide the L bracket unit into the L slot unit. The straight edge used for horizontal package alignment is not needed.

Stacking Horizontal Modules

Figure 1. Interlocked (Stacked) Horizontal or Vertical Packages.

Stacking Vertical Modules

5

5 MBd Link (HFBR-15X1/25X1)

System Performance 0 to 70 C unless  otherwise specified.

|  | Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Performance 5 MBd | Data Rate |  | dc |  | 5 | MBd | BER 10-9, PRBS:27-1 |  |
|  | Link Distance (Standard Cable) |  | 19 27 | 48 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25 C | Fig. 3 Note 3 |
|  | Link Distance (Improved Cable) |  | 22 27 | 53 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25 C | Fig. 4 Note 3 |
|  | Propagation Delay | tPLH tPHL |  | 80 50 | 140 140 | ns ns | RL = 560 , CL = 30 pF fiber length = 0.5 m -21.6 PR -9.5 dBm | Fig. 5, 8 Notes 1, 2 |
|  | Pulse Width Distortion tPLH-tPHL | tD |  | 30 |  | ns | PR = -15 dBm RL = 560 , CL = 30 pF | Fig. 5, 7 |

Notes:

1. The propagation delay for one metre  of cable is typically 5 ns.

2. Typical propagation delay is measured at PR = -15 dBm.

3. Estimated typical link life expectancy at 40 C exceeds 10 years at 60 mA.

Figure 2. Typical  5 MBd Interface Circuit.

10                                                                                                                                                                 10

I F – FORWARD CURRENT (mA)

I F – FORWARD CURRENT (mA)

50                                                                                                                                                                 50

OVERDRIVE

30

UNDERDRIVE

20

10

OVERDRIVE

30

UNDERDRIVE

20

10

0°C–70°C

25°C

5

0            10           20           30           40           50

0°C–70°C

25°C

5

0         10         20         30        40         50        60

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 3. Guaranteed System Performance with Standard

Cable  (HFBR-15X1/25X1).

Figure 4. Guaranteed System Performance with Improved

Cable  (HFBR-15X1/25X1).

6

Figure 5. 5 MBd Propagation Delay  Test  Circuit.

Figure 6. Propagation Delay  Test  Waveforms.

500

| 500 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | tp | LH |  |  |
| HFBR-15X2/25X2 HFBR-15X4/25X4 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | tp | LH |  |  |
|  |  | HFB | R-15 | X1/2 | 5X1 |  |  |  |  |
|  |  |  |  |  |  | tp | HL |  |  |

tD – PULSE WIDTH DISTORTION – ns

400

300

HFBR-15X2/25X2

HFBR-15X4/25X4

70°C

25°C

0°C

tp – PROPAGATION DELAY – ns

400

300

200

200

100

0

HFBR-15X1/25X1

70°C

25°C

0°C

100

0

-25

-20          -15

-10           -5             0

-25

-20          -15

-10           -5             0

PR – INPUT OPTICAL POWER – dBm

PR – INPUT OPTICAL POWER – dBm

Figure 7. Typical  Link Pulse Width  Distortion vs. Optical Power.

Figure 8. Typical  Link Propagation Delay  vs. Optical Power.

| 7 Pin # | Function |
| --- | --- |
| 1 | Anode |
| 2 | Cathode |
| 3 | Open |
| 4 | Open |
| 5 | Do not connect |
| 8 | Do not connect |

HFBR-15X1 Transmitter

ANODE    1

CATHODE    2

N.C.    3

4

8  DO NOT CONNECT

N.C.

5  DO NOT CONNECT

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these  pins.

Absolute Maximum Ratings

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Forward Input Current |  | IFPK |  | 1000 | mA | Note 2, 3 |
|  |  | IFdc |  | 80 |  |  |
| Reverse Input Voltage |  | VBR |  | 5 | V |  |

Notes:

1. 1.6 mm below seating  plane.

2. Recommended operating range  between  10 and 750 mA.

3. 1   s pulse,  20    s period.

All HFBR-15XX LED transmitters are classified as IEC 825-1 Accessible Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go into effect on January 1, 1997. AEL Class 1 LED devices are considered eye safe. Contact your local Agilent sales representative for more information.

8

Transmitter Electrical/Optical Characteristics 0 C to 70  C unless  otherwise specified.

| Parameter | Symbol | Min. | Typ.[5] | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Output | PT | -16.5 |  | -7.6 | dBm | IFdc = 60 mA | Notes 1, 2 |
| Optical Power |  | -14.3 |  | -8.0 | dBm | IFdc = 60 mA, 25 C |  |
| Output Optical Power Temperature Coefficient | PT / T |  | -0.85 |  | %/ C |  |  |
| Peak Emission Wavelength | PK |  | 660 |  | nm |  |  |
| Forward Voltage | VF | 1.45 | 1.67 | 2.02 | V | IFdc = 60 mA |  |
| Forward Voltage Temperature Coefficient | VF/ T |  | -1.37 |  | mV/ C |  | Fig. 9 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |
| Reverse Input Breakdown Voltage | VBR | 5.0 | 11.0 |  | V | IFdc = 10 A, TA = 25 C |  |
| Diode Capacitance | CO |  | 86 |  | pF | VF = 0, f = MHz |  |
| Rise Time | tr |  | 80 |  | ns | 10% to 90%, | Note 3 |
| Fall Time | tf |  | 40 |  | ns | IF = 60 mA |  |

Notes:

1. Measured  at the end of 0.5 m standard fiber optic cable with large area detector.

2. Optical  power,  P (dBm)   = 10 Log [P(  W)/1000    W].

3. Rise and fall times are measured with a voltage  pulse driving the transmitter and a series  connected 50      load. A wide bandwidth optical  to electrical waveform  analyzer,  terminated to a 50     input  of a wide bandwidth oscilloscope, is used for this response time measurement.

1.8                                                                                                                                                             5

0

VF – FORWARD VOLTAGE – V

1.7

1.6

0°C

25°C

1.5

70°C

-15

1.4

2                             10

100

PT – NORMALIZED OUTPUT POWER – dB

-20

| 2 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

100

IFdc – TRANSMITTER DRIVE CURRENT (mA)

IFdc – TRANSMITTER DRIVE CURRENT (mA)

Figure 9. Typical  Forward Voltage vs. Drive  Current.               Figure 10.  Normalized Typical  Output Power vs. Drive

Current.

| 9 Pin # | Function |
| --- | --- |
| 1 | VO |
| 2 | Ground |
| 3 | VCC |
| 4 | RL |
| 5 | Do not connect |
| 8 | Do not connect |

HFBR-25X1 Receiver

DO NOT CONNECT      5

DO NOT CONNECT      8

1000

4    RL

3    VCC

2    GROUND

1    VO

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these  pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Supply Voltage |  | VCC | –0.5 | 7 | V | Note 2 |
| Output Collector Current |  | IOAV |  | 25 | mA |  |
| Output Collector Power Dissipation |  | POD |  | 40 | mW |  |
| Output Voltage |  | VO | –0.5 | 18 | V |  |
| Pull-up Voltage |  | VP | –5 | VCC | V |  |
| Fan Out (TTL) |  | N |  | 5 |  |  |

Notes:

1. 1.6 mm below seating  plane.

2. It is essential that  a bypass  capacitor 0.01    F be connected from pin 2 to pin 3 of the receiver. Total lead length  between  both ends of the capacitor and the pins should  not exceed  20 mm.

Receiver Electrical/Optical Characteristics

0 C to 70 C, 4.75 V    VCC     5.25 V unless  otherwise specified

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input Optical Power Level for Logic “0” | PR(L) | –21.6 |  | –9.5 | dBm | VOL = 0.5 V IOL = 8 mA | Notes 1, 2, 4 |
|  |  | –21.6 |  | –8.7 |  | VOL = 0.5 V IOL = 8 mA, 25 C |  |
| Input Optical Power Level for Logic “1” | PR(H) |  |  | –43 | dBm | VOL = 5.25 V IOH 250 A | Note 1 |
| High Level Output Current | IOH |  | 5 | 250 | A | VO = 18 V, PR = 0 | Note 3 |
| Low Level Output Current | VOL |  | 0.4 | 0.5 | V | IOL = 8 mA, PR = PR(L)MIN | Note 3 |
| High Level Supply Current | ICCH |  | 3.5 | 6.3 | mA | VCC = 5.25 V, PR = 0 | Note 3 |
| Low Level Supply Current | ICCL |  | 6.2 | 10 | mA | VCC = 5.25 V PR = -12.5 dBm | Note 3 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |
| Internal Pull-up Resistor | RL | 680 | 1000 | 1700 |  |  |  |

Notes:

1. Optical  flux, P (dBm)  = 10 Log [P (  W)/1000    W].

2. Measured  at the end of the fiber optic cable with large area detector.

3. RL is open.

4. Pulsed  LED operation at IF > 80 mA will cause  increased link tPLH  propagation delay time. This extended tPLH  time contributes to

increased pulse width distortion of the receiver  output signal.

10

1 MBd Link

(High Performance  HFBR-15X2/25X2, Standard HFBR-15X4/25X4)

System Performance Under recommended operating conditions unless  otherwise specified.

|  | Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Performance 1 MBd | Data Rate |  | dc |  | 1 | MBd | BER 10-9, PRBS:27-1 |  |
|  | Link Distance (Standard Cable) |  | 39 47 | 70 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25 C | Fig. 14 Notes 1, 3, 4 |
|  | Link Distance (Improved Cable) |  | 45 56 | 78 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25 C | Fig. 15 Notes 1, 3, 4 |
|  | Propagation Delay | tPLH tPHL |  | 180 100 | 250 140 | ns ns | RL = 560 , CL = 30 pF I = 0.5 metre PR = -24 dBm | Fig. 16, 18 Notes 2, 4 |
|  | Pulse Width Distortion tPLH-tPHL | tD |  | 80 |  | ns | PR = -24 dBm RL = 560 , CL = 30 pF | Fig. 16, 17 Note 4 |

|  | Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standard | Data Rate |  | dc |  | 1 | MBd | BER 10-9, PRBS:27-1 |  |
| 1 MBd | Link Distance (Standard Cable) |  | 8 17 | 43 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25 C | Fig. 12 Notes 1, 3, 4 |
|  | Link Distance (Improved Cable) |  | 10 19 | 48 |  | m m | IFdc = 60 mA IFdc = 60 mA, 25 C | Fig. 13 Notes 1, 3, 4 |
|  | Propagation Delay | tPLH tPHL |  | 180 100 | 250 140 | ns ns | RL = 560 , CL = 30 pF I = 0.5 metre PR = -20 dBm | Fig. 16, 18 Notes 2, 4 |
|  | Pulse Width Distortion tPLH-tPHL | tD |  | 80 |  | ns | PR = -20 dBm RL = 560 , CL = 30 pF | Fig. 16, 17 Note 4 |

Notes:

1. For IFPK  > 80 mA, the duty factor  must be such as to keep IFdc      80 mA. In addition, for IFPK  > 80 mA, the following rules for pulse width apply:

IFPK     160 mA: Pulse width     1 ms

IFPK > 160 mA: Pulse width     1    S, period     20    S.

2. The propagation delay for one meter  of cable is typically 5 ns.

3. Estimated typical link life expectancy at 40 C exceeds 10 years at 60 mA.

4. Pulsed  LED operation at IFPK > 80 mA will cause  increased link tPLH propagation delay time. This extended tPLH time contributes to increased pulse width distortion of the receiver  output signal.

11

Figure 11.  Required 1 MBd Interface Circuit.

The HFBR-25X2 receiver can not  be overdriven when using  the required interface circuit shown in Figure 11.

| 10 10 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | HFBR-15X4/25X4 |  |  |
|  |  | 0°C–70°C 25°C |  |  |

90                                                                                                                                                                     90

I F – FORWARD CURRENT (mA)

I F – FORWARD CURRENT (mA)

80                                                                                                                                                                     80

70                                                                                                                                                                     70

60                                                                                                                                                                     60

50                                                                                                                                                                     50

40                                                                                                                                                                     40

30                                                                                                                                                                     30

HFBR-15X4/25X4

20

0            5            10           15

20           25

20

0                      10

0°C–70°C

25°C

20                     30

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 12.  Guaranteed System Performance for the

HFBR-15X4/25X4 Link with Standard Cable.

Figure 13.  Guaranteed System Performance for the

HFBR-15X4/25X4 Link with Improved Cable.

100

50

40

30

20

100

50

40

30

20

UNDERDRIVE

UNDERDRIVE

IF – FORWARD CURRENT (mA)

IF – FORWARD CURRENT (mA)

10                                                                                                                                                                          10

0°C–70°C

25°C

5

0           10           20           30           40           50

0°C–70°C

25°C

5

0        10         20         30        40         50        60

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 14.  Guaranteed System Performance for the

HFBR-15X2/25X2 Link with Standard Cable.

Figure 15.  Guaranteed System Performance for the

HFBR-15X2/25X2 Link with Improved Cable.

12

Figure 16.  1 MBd Propagation Delay  Test  Circuit.

500

500

tD – PULSE WIDTH DISTORTION – ns

400

300

tp – PROPAGATION DELAY – ns

HFBR-15X2/25X2

HFBR-15X4/25X4

70°C

25°C

0°C

400

300

200

200

100

0

HFBR-15X1/25X1

70°C

25°C

0°C

100

| 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | tp | LH |  |  |
| HFBR-15X2/25X2 HFBR-15X4/25X4 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | tp | LH |  |  |
|  |  | HFB | R-15 | X1/2 | 5X1 |  |  |  |  |
|  |  |  |  |  |  | tp | HL |  |  |

-25

-20          -15

-10           -5             0

-25

-20          -15

-10           -5             0

PR – INPUT OPTICAL POWER – dBm

PR – INPUT OPTICAL POWER – dBm

Figure 17.  Pulse Width  Distortion vs. Optical Power.

Figure 18.  Typical  Link Propagation

Delay  vs. Optical Power.

Figure 19.  Propagation Delay  Test

Waveforms.

| 13 Pin # | Function |
| --- | --- |
| 1 | Anode |
| 2 | Cathode |
| 3 | Open |
| 4 | Open |
| 5 | Do not connect |
| 8 | Do not connect |

HFBR-15X2/15X4 Transmitters

ANODE    1

CATHODE    2

N.C.    3

N.C.    4

8  DO NOT CONNECT

5  DO NOT CONNECT

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not

Absolute Maximum Ratings

electrically connect these  pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Forward Input Current |  | IFPK |  | 1000 | mA | Note 2, 3 |
|  |  | IFdc |  | 80 |  |  |
| Reverse Input Voltage |  | VBR |  | 5 | V |  |

Notes:

1. 1.6 mm below seating  plane.

2. Recommended operating range  between  10 and 750 mA.

3. 1   s pulse,  20    s period.

All HFBR-15XX LED transmitters are classified as IEC 825-1 Accessible Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go into effect on January 1, 1997. AEL Class 1 LED devices are considered eye safe. Contact your Agilent sales representative for more information.

Transmitter Electrical/Optical Characteristics 0 C to 70 C unless  otherwise specified. For forward  voltage and output power vs. drive current graphs.

| Parameter |  | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Output | HFBR-15X2 | PT | –13.6 –11.2 |  | –4.5 –5.1 | dBm | IFdc = 60 mA IFdc = 60 mA, 25 C |  |
| Optical Power | HFBR-15X4 | PT | –17.8 –15.5 |  | –4.5 –5.1 | dBm | IFdc = 60 mA IFdc = 60 mA, 25 C |  |
| Output Optical Power Temperature Coefficient |  | PT/ T |  | –0.85 |  | %/ C |  |  |
| Peak Emission Wavelength |  | PK |  | 660 |  | nm |  |  |
| Forward Voltage |  | VF | 1.45 | 1.67 | 2.02 | V | IFdc = 60 mA |  |
| Forward Voltage Temperature Coefficient |  | VF/ T |  | –1.37 |  | mV/ C |  | Fig. 11 |
| Effective Diameter |  | DT |  | 1 |  | mm |  |  |
| Numerical Aperture |  | NA |  | 0.5 |  |  |  |  |
| Reverse Input Breakdown Voltage |  | VBR | 5.0 | 11.0 |  | V | IFdc = 10 A, TA = 25 C |  |
| Diode Capacitance |  | CO |  | 86 |  | pF | VF = 0, f = 1 MHz |  |
| Rise Time |  | tr |  | 80 |  | ns | 10% to 90%, | Note 1 |
| Fall Time |  | tf |  | 40 |  | ns | IF = 60 mA |  |

Note:

1. Rise and fall times are measured with a voltage  pulse driving the transmitter and a series  connected 50      load. A wide bandwidth optical  to electrical waveform  analyzer,  terminated to a 50     input  of a wide bandwidth oscilloscope, is used for this response time measurement.

| 14 Pin # | Function |
| --- | --- |
| 1 | VO |
| 2 | Ground |
| 3 | VCC |
| 4 | RL |
| 5 | Do not connect |
| 8 | Do not connect |

HFBR-25X2/25X4 Receivers

DO NOT CONNECT      5

DO NOT CONNECT      8

1000

4    RL

3    VCC

2    GROUND

1    VO

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these  pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Supply Voltage |  | VCC | –0.5 | 7 | V | Note 2 |
| Output Collector Current |  | IOAV |  | 25 | mA |  |
| Output Collector Power Dissipation |  | POD |  | 40 | mW |  |
| Output Voltage |  | VO | –0.5 | 18 | V |  |
| Pull-up Voltage |  | VP | –5 | VCC | V |  |
| Fan Out (TTL) |  | N |  | 5 |  |  |

Notes:

1. 1.6 mm below seating  plane.

2. It is essential that  a bypass  capacitor 0.01    F be connected from pin 2 to pin 3 of the receiver. Total lead length  between  both ends

of the capacitor and the pins should  not exceed  20 mm.

Receiver Electrical/Optical Characteristics 0 C to 70  C, 4.75 V   VCC      5.25 V unless  otherwise specified.

| Parameter |  | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Receiver Optical Input | HFBR-2522 | PR(L) | –24 |  |  | dBm | VOL = 0 V IOL = 8 mA | Notes 1, 2, 3 |
| Power Level Logic 0 | HFBR-2524 |  | –20 |  |  |  |  | Note 4 |
| Optical Input Power Level Logic 1 |  | PR(H) |  |  | -43 | dBm | VOH = 5.25 V IOH = 250 A |  |
| High Level Output Current |  | IOH |  | 5 | 250 | A | VO = 18 V, PR = 0 | Note 5 |
| Low Level Output Voltage |  | VOL |  | 0.4 | 0.5 | V | IOL = 8 mA PR = PR(L)MIN | Note 5 |
| High Level Supply Current |  | ICCH |  | 3.5 | 6.3 | mA | VCC = 5.25 V, PR = 0 | Note 5 |
| Low Level Supply Current |  | ICCL |  | 6.2 | 10 | mA | VCC = 5.25 V, PR = -12.5 dBm | Note 5 |
| Effective Diameter |  | D |  | 1 |  | mm |  |  |
| Numerical Aperture |  | NA |  | 0.5 |  |  |  |  |
| Internal Pull-up Resistor |  | RL | 680 | 1000 | 1700 |  |  |  |

Notes:

1. Measured  at the end of the fiber optic cable with large area detector.

2. Pulsed  LED operation at IF > 80 mA will cause  increased link tPLH propagation delay time. This extended tPLH time contributes to increased pulse width distortion of the receiver  output signal.

3. The LED drive circuit  of Figure  11 is required for 1 MBd operation of the HFBR-25X2/25X4.

4. Optical  flux, P (dBm)  = 10 Log [P(   W)/1000    W].

5. RL is open.

15

40 kBd Link

System Performance Under recommended operating conditions unless  otherwise specified.

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data Rate |  | dc |  | 40 | kBd | BER 10-9, PRBS: 27 - 1 |  |
| Link Distance (Standard Cable) |  | 13 94 | 41 138 |  | m m | IFdc = 2 mA IFdc = 60 mA | Fig. 21 Note 1 |
| Link Distance (Improved Cable) |  | 15 111 | 45 154 |  | m m | IFdc = 2 mA IFdc = 60 mA | Fig. 22 Note 1 |
| Propagation Delay | tPLH tPHL |  | 4 2.5 |  | s s | RL = 3.3 k , CL = 30 pF PR = -25 dBm, 1 m fiber | Fig. 22, 25 Note 2 |
| Pulse Width Distortion tPLH-tPHL | tD |  |  | 7 | s | -39 PR - 14 dBm RL = 3.3 k , CL = 30 pF | Fig. 23, 24 |

Notes:

1. Estimated typical link life expectancy at 40  C exceeds 10 years at 60 mA.

2. The propagation delay for one metre  of cable is typically 5 ns.

Figure 20.  Typical  40 kBd Interface Circuit.

12                                                                                                                                                           12

10                                                                                                                                                           10

80                                                                                                                                                           80

I F – FORWARD CURRENT (mA)

I F – FORWARD CURRENT (mA)

60                                                                                                                                                           60

40                                                                                                                                                           40

20

20

10

6                                                                                                                                                           10

HFBR-15X3/25X3

2

0°C–70°C

25°C

1

6

4

0°C–70°C

25°C

2

0    10   20   30   40   50   60

70   80   90  100

0   10   20  30   40  50

60  70  80   90  100 110

– CABLE LENGTH – METRES

– CABLE LENGTH – METRES

Figure 21.  Guaranteed System Performance with

Standard Cable.

Figure 22.  Guaranteed System Performance with Improved

Cable.

16

Figure 23.  40 kBd Propagation Delay  Test  Circuit.

| 6 8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

7

t D – PULSE WIDTH DISTORTION – µs

5

6

4

5

t P – PROPAGATION DELAY – µs

3                                                                                                                                                             4

3

2

2

1

1

0

-40

-34

-28

-22         -16

-10

0

-40

-34

-28

-22         -16

| -10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | tPLH |  |  |  |  |  |  |
|  |  |  | tPHL |  |  |  |  |  |  |

PR – INPUT OPTICAL POWER, dBm

PR – INPUT OPTICAL POWER, dBm

Figure 24.  Typical  Link Pulse Width  Distortion vs. Optical Power.

Figure 25.  Typical  Link Propagation Delay  vs. Optical

Power.

Figure 26.  Propagation Delay  Test  Waveforms.

| 17 Pin # | Function |
| --- | --- |
| 1 | Anode |
| 2 | Cathode |
| 3 | Open |
| 4 | Open |
| 5 | Do not connect |
| 8 | Do not connect |

HFBR-15X3 Transmitter

ANODE    1

CATHODE    2

N.C.    3

N.C.    4

8  DO NOT CONNECT

5  DO NOT CONNECT

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these  pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Forward Input Current |  | IFPK |  | 1000 | mA | Note 2, 3 |
|  |  | IFdc |  | 80 |  |  |
| Reverse Input Voltage |  | VBR |  | 5 | V |  |

Notes:

1. 1.6 mm below seating  plane.

2. Recommended operating range  between  10 and 750 mA.

3. 1   s pulse,  20    s period.

All HFBR-15XX LED transmitters are classified as IEC 825-1 Accessible Emission Limit (AEL) Class 1 based upon the current proposed draft scheduled to go into effect on January 1, 1997. AEL Class 1 LED devices are considered eye safe. Contact your Agilent sales representative for more information.

Transmitter Electrical/Optical Characteristics 0 C to 70 C unless  otherwise specified. For forward  voltage and output power vs. drive current graphs.

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Output Optical Power | PT | –11.2 –13.6 –35.5 |  | –5.1 –4.5 | dBm | IFdc = 60 mA, 25 C IFdc = 60 mA IFdc = 2 mA, 0-70 C | Notes 3, 4 Fig. 9, 10 |
| Output Optical Power Temperature Coefficient | PT/ T |  | –0.85 |  | %/ C |  |  |
| Peak Emission Wavelength | PK |  | 660 |  | nm |  |  |
| Forward Voltage | VF | 1.45 | 1.67 | 2.02 | V | IFdc = 60 mA |  |
| Forward Voltage Temperature Coefficient | VF/ T |  | –1.37 |  | mV/ C |  | Fig. 18 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |
| Reverse Input Breakdown Voltage | VBR | 5.0 | 11.0 |  | V | IFdc = 10 A, TA = 25 C |  |
| Diode Capacitance | CO |  | 86 |  | pF | VF = 0, f = 1 MHz |  |
| Rise Time | tr |  | 80 |  | ns | 10% to 90%, | Note 1 |
| Fall Time | tf |  | 40 |  |  | IF = 60 mA |  |

Note:

1. Rise and fall times are measured with a voltage  pulse driving the transmitter and a series  connected 50      load. A wide bandwidth optical  to electrical waveform  analyzer,  terminated to a 50     input  of a wide bandwidth oscilloscope, is used for this response time measurement.

| 18 Pin # | Function |
| --- | --- |
| 1 | VO |
| 2 | Ground |
| 3 | Open |
| 4 | VCC |
| 5 | Do not connect |
| 8 | Do not connect |

HFBR-25X3 Receiver

DO NOT CONNECT      5

DO NOT CONNECT      8

4    VCC

3    OPEN

2    GROUND

1    VO

Absolute Maximum Ratings

Note: Pins 5 and 8 are for mounting and retaining purposes only. Do not electrically connect these  pins.

| Parameter |  | Symbol | Min. | Max. | Units | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Storage Temperature |  | TS | –40 | +85 | C |  |
| Operating Temperature |  | TA | –40 | +85 | C |  |
| Lead Soldering Cycle | Temp. |  |  | 260 | C | Note 1 |
|  | Time |  |  | 10 | sec |  |
| Supply Voltage |  | VCC | –0.5 | 7 | V | Note 2 |
| Average Output Collector Current |  | IO | –1 | 5 | mA |  |
| Output Collector Power Dissipation |  | POD |  | 25 | mW |  |
| Output Voltage |  | VO | –0.5 | 7 | V |  |

Notes:

1. 1.6 mm below seating  plane.

2. It is essential that  a bypass  capacitor 0.01    F be connected from pin 2 to pin 3 of the receiver.

Receiver Electrical/Optical Characteristics 0 C to 70  C, 4.5 V    VCC     5.5 V unless  otherwise specified.

| Parameter | Symbol | Min. | Typ. | Max. | Units | Conditions | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input Optical Power | PR(L) | –39 |  | –13.7 | dBm | VO = VOL, IOL = 3.2 mA | Notes 1, 2, 3 |
| Level Logic 0 |  | –39 |  | –13.3 |  | VO = VOL, IOH = 8 mA, 25 C |  |
| Input Optical Power Level Logic 1 | PR(H) |  |  | –53 | dBm | VOH = 5.5 V IOH = 40 A | Note 3 |
| High Level Output Voltage | VOH | 2.4 |  |  | V | IO = - 40 A, PR = 0 W |  |
| Low Level Output Voltage | VOL |  |  | 0.4 | V | IOL = 3.2 mA PR = PR(L)MIN | Note 4 |
| High Level Supply Current | ICCH |  | 1.2 | 1.9 | mA | VCC = 5.5 V, PR = 0 W |  |
| Low Level Supply Current | ICCL |  | 2.9 | 3.7 | mA | VCC = 5.5 V, PR = PRL (MIN) | Note 4 |
| Effective Diameter | D |  | 1 |  | mm |  |  |
| Numerical Aperture | NA |  | 0.5 |  |  |  |  |

Notes:

1. Measured  at the end of the fiber optic cable with large area detector.

2. Optical  flux, P (dBm)  = 10 Log P(  W)/1000    W.

3. Because  of the very high sensitivity  of the HFBR-25X3, the digital output may switch in response to ambient  light levels when a cable is not occupying the receiver  optical  port.  The designer should  take care to filter out signals  from this source  if they pose  a hazard  to the system.

4. Including  current in 3.3 k pull-up resistor.

www.semiconductor.agilent.com

Data subject  to change.

Copyright  © 2001  Agilent Technologies, Inc. June 12, 2001

Obsoletes 5968-1712E

5988-1765EN