HFBR-RXXYYYZ Series  (POF) HFBR-EXXYYYZ Series  (POF)

Plastic Optical Fiber Cable and Accessories

for Versatile Link

Data Sheet

Cable Description

The HFBR-R/EXXYYYZ series of plastic fiber optic cables  are  constructed of  a  single  step-index fiber sheathed in a black  polyethylene jacket.  The duplex fiber  consists  of  two  simplex  fibers  joined  with  a zipcord  web.

Standard attenuation and  extra  low loss POF cables are identical  except  for attenuation specifications. Polyethylene jackets on all plastic fiber cables comply with UL VW-1 flame retardant specification (UL file # E89328).

Cables are available in unconnectored or connectored options. Refer to the Ordering  Guide for part number information.

| 500 |  |  |  |
| --- | --- | --- | --- |

R – ATTENUATION – dB/km

400

Features

• Compatible with Avago Versatile Link family of connectors and fiber optic components

• 1 mm diameter Plastic Optical Fiber (POF) in two grades:  low cost standard POF with 0.22 dB/m typical attenuation, or high performance extra low loss POF with 0.19 dB/m typical attenuation

Applications

• Industrial data links for factory automation and plant control

• Intra-system links; board-to-board, rack-to-rack

• Telecommunications switching systems

• Computer-to-peripheral data links, PC bus extension

• Proprietary LANs

• Digitized video

• Medical instruments

• Reduction of lightning and voltage transient susceptibility

• High voltage isolation

300

200

100

620

640

660

680

700

 – WAVELENGTH – nm

Figure 1. Typical POF attenuation vs. wavelength.

Plastic Optical Fiber Specifications: HFBR-R/EXXYYYZ Absolute Maximum  Ratings

| Parameter |  | Symbol | Min. | Max. | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Storage and Operating Temperature |  | TS,O | -55 | +85 | C |  |
| Recommended Operating Temperature |  | TO | -40 | +85 | C |  |
| Installation Temperature |  | TI | -20 | +70 | C | 1 |
| Short Term Tensile | Single Channel | FT |  | 50 | N | 2 |
| Force | Dual Channel | FT |  | 100 | N |  |
| Short Term Bend Radius |  | r | 25 |  | mm | 3, 4 |
| Long Term Bend Radius |  | r | 35 |  | mm |  |
| Long Term Tensile Load |  | FT |  | 1 | N |  |
| Flexing |  |  |  | 1000 | Cycles | 4 |

Mechanical/Optical Characteristics, TA = -40 to +85C unless otherwise specified.

Parameter                                         Symbol       Min.     Typ.[5]          Max.     Unit    Condition

Cable                         Standard Cable,             O                  0.15         0.22         0.27       dB/m    Source is HFBR-15XXZ Attenuation          Type "R"                                                                                       (660 mm LED, 0.5 NA)

Extra Low Loss,                        0.15          0.19          0.23                           = 50 meters

Type "E"

Reference            Standard Cable,             R                  0.12         0.19         0.24      dB/m    Source is 650 nm, Attenuation          Type "R"                                                                                       0.5 NA monochrometer,

Extra Low Loss,                        0.12          0.16          0.19                           = 50 meters

Type "E"                                                                                        Note 7, Figure 1

Numerical Aperture                                 NA         0.46         0.47         0.50                      >2 meters

Diameter, Core and Cladding                    DC                  0.94          1.00          1.06         mm

Diameter, Jacket                                      DJ                  2.13         2.20         2.27        mm     Simplex Cable

Propagation Delay Constant                     l/v                         5.0                         ns/m    Note 6

Mass per Unit Length/Channel                                             5.3                          g/m     Without Connectors Cable Leakage Current                              IL                                             12                           nA      50 kV,     = 0.3 meters Refractive Index     Core                                  n                        1.492

Cladding                                              1.417

Notes:

1. Installation  temperature is the range over which the cable can be bent and pulled without  damage. Below -20C the cable becomes brittle and should not be subjected to mechanical stress.

2. Short Term Tensile Force is for less than 30 minutes.

3. Short Term Bend Radius is for less than 1 hour nonoperating.

4. 90 bend on 25 mm radius mandrel. Bend radius is the radius of the mandrel around which the cable is bent.

5. Typical data are at 25C.

6. Propagation delay constant is the reciprocal of the group velocity for propagation delay of optical power. Group velocity is v=c/n where c is the velocity of light in free space (3xl08 m/s) and n is the effective core index of refraction.

7. Note that R rises at the rate of about 0.0067 dB/C, where the thermal rise refers to the LED temperature changes above 25C. Please refer to

Figure 1 which shows the typical plastic optical fiber attenuation versus wavelength at 25C.

2

Plastic Fiber Connector Styles

Connector Description

Four connector styles are available for termination of plastic  optical fiber: simplex,  simplex  latching, duplex  and duplex  latching. All connectors provide  a snap-in action when mated  to Versatile Link components. Simplex connectors are color coded to facilitate  identification of trans- mitter  and receiver  connections. Duplex connectors are keyed so that proper orientation is ensured during  insertion. If the POF cable/ connector will be used at extreme operating temperatures or experience frequent and wide temperature cycling effects,  the cable/connector attachment can

be strengthened with an RTV adhesive  (see Plastic Connector- ing Instructions for more detail). The connectors are made of a flame retardant VALOX UL94 V-0 material  (UL file # E121562).

SIMPLEX CONNECTOR STYLES HFBR-4501Z/4511Z – Simplex

, HFBR-4525Z

The simplex  connector provides  a quick and stable  connection for applications that require a component-to-connector retention force of 8 Newtons (1.8  lb.).

These connectors are available in gray (HFBR-4501Z) or blue (HFBR-4511Z).

HFBR-4503Z/4513Z – Simplex Latching

HFBR-4525Z

The simplex  latching  connector is designed for rugged  applications requiring a greater retention force

— 80 Newtons ( 18 lb.) — than provided  by a simplex nonlatching connector. When inserting the simplex  latching  connector into a module,  the connector latch mechanism should be aligned with the top surface  of the horizontal modules,  or with the tall vertical side of the vertical  modules. Misalignment  of an inserted latching  connector into either module will not result  in a positive latch. The connector is released

by depressing the rear section  of the connector lever, and then pulling the connector assembly away from the module housing.

The simplex  latching  connector is available in gray (HFBR-4503Z) or blue (HFBR-4513Z).

DUPLEX CONNECTOR STYLES HFBR-4506Z – Duplex

HFBR-4526Z

Duplex connectors provide  con- venient  duplex  cable termination and are keyed to prevent  incorrect insertion into duplex  configured modules.  The duplex  connector is compatible with dual combina- tions of horizontal or vertical Versatile Link components (e.g., two horizontal transmitters, two vertical  receivers, a horizontal transmitter with a horizontal receiver, etc.).  The duplex  non-

latching  connector is available in parchment, off-white (HFBR-

4506Z).

HFBR-4516Z – Duplex Latching

HFBR-4526Z

The duplex  latching  connector is designed for rugged  applications requiring greater retention force than the nonlatching duplex  con- nector. When inserting the duplex latching  connector into a module, the connector latch mechanism should be aligned with the top surface  of the dual combination of horizontal or vertical  Versatile Link components. The duplex  latching connector is available in gray (HFBR-4516Z).

,

Feedthrough/Splice

HFBR-4505Z/4515Z Bulkhead Adapter

The HFBR-4505Z/4515Z adapter mates  two simplex  connectors for panel/bulkhead feedthrough of HFBR-4501Z/4511Z terminated plastic  fiber cable. Maximum panel thickness is 4.1 mm (0.16  inch). This adapter can serve as a cable

in-line splice using two simplex connectors. The adapters are available in gray (HFBR-4505Z) and blue (HFBR-4515Z). This

adapter is not compatible with POF duplex,  POF simplex  latching, or HCS connectors.

3

Plastic Optical Fiber Connector Absolute Maximum  Ratings

| HFBR-4505Z/4515Z Adapter Parameter | Symbol | Min. | Max. | Unit | Note |
| --- | --- | --- | --- | --- | --- |
| Storage and Operating Temperature | TS,O | -40 | 85 | C | 1 |
| Recommended Operating Temperature | TO | -40 | 85 | C | 1 |
| Installation Temperature | Tl | 0 | 70 | C | 1 |
| Nut Torque | TN |  | 0.7 | N-m | 2 |
|  |  |  | 100 | OzF-in. |  |

Notes:

1. Storage and Operating Temperatures refer to the ranges over which the connectors can be used when not subjected to mechanical stress. Installation Temperature refers to the ranges over which connectors may be installed onto the fiber and over which connectors can be connected and disconnected from transmitter and receiver modules.

2. Recommended nut torque is 0.57 N-m.

Plastic Optical Fiber Connector Mechanical/Optical Characteristics

TA = -40 to +85C, Unless Otherwise Specified.

Parameter                    Part Number                   Symbol     Min.   Typ.[1]       Max.    Units    Temp. C      Note

Retention Force,                Simplex,                            FR-C                7            8                            N             +25          2

Connector to                   HFBR-4501Z/4511Z                                    3                                                       -40 to +85

Versatile Link Transmitters and Receivers

Simplex Latching,                             47          80                                             +25

HFBR-4503Z/4513Z                       11                                                      -40 to +85

Duplex,                                             7           12                                             +25

HFBR-4506Z                                               4                                                       -40 to +85

Duplex Latching,                               50          80                                             +25

HFBR-4516Z                                              15                                                      -40 to +85

Tensile Force,                     Simplex,                              FT                8.5          22                           N                              3

Connector to Cable             HFBR-4501Z/4511Z

Simplex Latching,                             8.5         22

HFBR-4503Z/4513Z

Duplex, HFBR-4506Z                                 14          35

Duplex Latching,                               14          35

HFBR-4516Z

Adapter Connector        HFBR-4505Z/4515Z with       CC              0.7         1.5          2.8         dB             25             4, 5 to Connector Loss          HFBR-4501Z/4511Z

Retention Force             HFBR-4505Z/4515Z with       FR-B                7            8                           N Connector to Adapter     HFBR-4501Z/4511Z

Insertion Force,                  Simplex,                              FI                                        8             30           N                              6

Connector to Versatile

Link Transmitters and

Receivers

Notes:

1. Typical data are at +25C.

HFBR-4501Z/4511Z

Simplex Latching,                                         16           35

HFBR-4503Z/4513Z

Duplex, HFBR-4506Z                                               13           46

Duplex Latching                                            22           51

HFBR-4516Z

2. No perceivable reduction in retention force was observed after 2000 insertions. Retention force of non-latching connectors is lower at elevated temperatures. Latching connectors are recommended for applications where a high retention force at high temperatures is desired.

3. For applications where frequent temperature cycling over temperature extremes is expected, please contact Avago Technologies for alternate connectoring techniques.

4. Minimum and maximum limit for CC for 0C to +70C temperature range. Typical value of CC is at +25C.

5. Factory polish or field polish per recommended procedure.

6. Destructive insertion force was typically at 178 N (40 lb.).

4

Step-by-Step Plastic Cable

Connectoring Instructions

The following step-by-step guide describes how to terminate plastic fiber optic cable. It is ideal for both field and factory  installation. Connectors can be easily installed on cable ends with wire strippers, cutters and a crimping  tool.

Finishing  the cable is accom- plished  with the Avago

HFBR-4593Z Polishing  Kit, consisting of a Polishing  Fixture,

600 grit abrasive  paper  and 3 m

pink lapping  film (3M Company, OC3-14).  The connector can be used immediately  after polishing.

Materials  needed  for plastic  fiber termination are:

1. Avago Plastic Optical Fiber Cable (Example: HFBR- RUS500Z, HFBR-RUD500Z, HFBR-EUS500Z, or HFBR- EUD500Z)

2. Industrial Razor Blade or Wire

Cutters

3. 16 Gauge Latching  Wire Strippers (Example: Ideal StripmasterTM type 45-092).

4. HFBR-4597Z Crimping  Tool

5. HFBR-4593Z Polishing  Kit

6. One of the following connectors:

a) HFBR-4501Z/4503Z Gray Simplex/Simplex Latching Connector and HFBR-4525Z Simplex Crimp Ring

b) HFBR-4511Z/4513Z Blue Simplex/Simplex Latching Connector and HFBR-4525Z Simplex Crimp Ring

c) HFBR-4506Z Parchment

(off-white)  Duplex

Connector and HFBR-4526Z Duplex Crimp Ring

d) HFBR-4516Z Gray Latching Duplex Connector and HFBR-4526Z Duplex Crimp Ring

Step 1

The zip cord structure of the duplex  cable permits  easy separation of the channels. The channels should be separated a minimum  of 100 mm (4 in) to a maximum  of 150 mm (6 in) back from the ends to permit connectoring and polishing.

After cutting  the cable to the desired  length,  strip off approxi- mately 7 mm (0.3  in.) of the outer jacket with the 16 gauge wire strippers. Excess  webbing  on the duplex  cable may have to be trimmed to allow the simplex  or simplex  latching  connector to

slide over the cable.

When using the duplex  connector and duplex  cable, the separated duplex  cable must be stripped to equal lengths  on each cable. This allows easy and proper seating  of the cable into the duplex connector.

Step 2

Place the crimp ring and connec- tor over the end of the cable; the fiber should protrude about  3 mm (0.12  in.) through the end of the connector. Carefully position  the ring so that it is entirely  on the connector with the rim of the crimp ring flush with the connec- tor, leaving a small space  between the crimp ring and the flange. Then crimp the ring in place with the crimping  tool. One crimp tool is used for all POF connector crimping  requirements.

For applications with extreme temperature operation or frequent temperature cycling, improved connector to cable attachment

can be achieved  with the use of an RTV (GE Company,  RTV-128 or Dow Corning  3145-RTV)

adhesive.  The RTV is placed  into the connector prior  to insertion of the fiber and the fiber is crimped normally.  The connector can be polished  after the RTV has cured and is then ready for use.

Note:  By convention, place the gray connector on the trans- mitter cable  end  and the blue connector on the receiver cable end  to maintain color coding (different color connectors are mechanically identical).

Simplex connector crimp rings cannot be used with duplex con- nectors and duplex connector crimp rings cannot be used

with simplex connectors because of size  differences. The simplex crimp has a dull  luster appear- ance;  the duplex ring is glossy and has a thinner wall.

5

Step 3

Any excess  fiber protruding from the connector end may be cut off; however,  the trimmed fiber

should extend  at least 1.5 mm

(0.06  in) from the connector end.

Insert  the connector fully into the polishing  fixture  with the trimmed fiber protruding from the bottom of the fixture.  This plastic  polish- ing fixture  can be used to polish two simplex connectors or simplex latching  connectors simultane- ously, or one duplex  connector.

Note:  The four  dots  on the bottom of the polishing fixture are  wear indicators. Replace the polishing fixture when any

dot is no longer visible. Typically, the polishing fixture can  be used 10 times; 10 duplex connectors or 20 simplex connectors, two at a time.

Place the 600 grit abrasive  paper on a flat smooth  surface, pressing down on the connector, polish the fiber and the connector using a figure eight pattern of strokes

until the connector is flush with the bottom  of the polishing fixture.  Wipe the connector and fixture  with a clean cloth or tissue.

Step 4

Place the flush connector and polishing  fixture  on the dull side of the 3 m pink lapping  film and continue to polish the fiber and connector for approximately 25 strokes. The fiber end should be flat, smooth  and clean.

This cable is now ready for use.

Note:  Use of the pink lapping film fine  polishing step  results in approximately 2 dB improvement in coupling performance of either a transmitter-receiver link or a bulkhead/splice over  a 600  grit polish alone. This  fine  polish is comparable to the Avago

factory polish. The fine polishing step  may be omitted where an  extra 2 dB of optical power is not essential, as with short  link lengths. Proper polishing of the tip  of the fiber/ connector face  results in a tip diameter between 2.5 mm (0.098 in.) minimum and 3.2 mm (0.126 in.) maximum..

HFBR-4593Z Polishing Kit

6

Ordering Guide for POF Connectors and Accessories

Plastic Optical Fiber Connectors

HFBR-4501Z     Gray Simplex Connector/Crimp Ring

HFBR-4511Z     Blue Simplex Connector/Crimp Ring

HFBR-4503Z     Gray Simplex Latching  Connector with Crimp Ring HFBR-4513Z     Blue Simplex Latching  Connector with Crimp Ring HFBR-4506Z     Parchment Duplex Connector with Crimp Ring HFBR-4516Z     Gray Duplex Latching  Connector with Crimp Ring HFBR-4505Z     Gray Adapter  (Bulkhead/Feedthrough)

HFBR-4515Z     Blue Adapter  (Bulkhead/Feedthrough)

Plastic Optical Fiber Accessories

HFBR-4522Z     500 HFBR-0500Z Products Port Plugs

HFBR-4525Z     1000  Simplex Crimp Rings

HFBR-4526Z     500 Duplex Crimp Rings

HFBR-4593Z     Polishing  Kit (one polishing  tool, two pieces  600 grit abrasive  paper, and two pieces  3 m pink lapping  film)

HFBR-4597Z     Plastic Fiber Crimping  Tool

Ordering Guide for POF Cable

For Example:

HFBR-RUD500Z is a Standard Attenuation, Unconnectored, Duplex,  500 meter  cable.

HFBR-RLS001Z is a Standard Attenuation, Latching  Simplex Connectored, Simplex, 1 meter cable.

HFBR-RMD010Z is a Standard Attenuation, Standard Duplex Connectored, Duplex,  10 meter cable.

HFBR-RMD100Z is a Standard Attenuation, Standard Duplex Connectored, Duplex,  100 meter cable.

Cable Length Tolerances: The plastic  cable length tolerances are: +10%/-0%.

NOTE: By convention, pre- connectored simplex  POF cables have gray and blue colored connectors on the opposite ends of the same fiber; although oppositely colored,  the connectors are mechanically

identical.  Duplex POF cables  with duplex  connectors use color- coded markings on the duplex fiber cable to differentiate between the channel.

| HFBR- Z |  |  |  |  |
| --- | --- | --- | --- | --- |

Cable  Code

R = Standard Attenuation POF E = Extra  Low Loss POF

Connector Code

U = Unconnectored

N = Standard Simplex Connectors L = Latching  Simplex Connectors M = Standard Duplex Connectors T = Latching  Duplex Connectors

RoHS Compliant

Length  Code  (measured from  tip of connector to tip of connector) Maximum 500 meters  (not in 1 meter

increments).

See www.avagotech.com/fiber/cables e.g., 015 = 15 meters

Channel Code

S = Simplex Cable

D = Duplex Zipcord Cable

Note: Not all possible combinations reflect available part numbers. Please contact your local

Avago representative for a list of current available cable part numbers.

7

Connector Applications

Attachment to Avago Versatile Link Fiber Optic Components

Bulkhead Feedthrough or Panel Mounting for HFBR-4501Z/4511Z  Simplex Connectors

Versatile Link Mechanical Dimensions

All dimensions in mm (inches).

All dimensions  0.25 mm unless  otherwise specified.

Fiber Optic Cable Dimensions

1.0 POF (0.039)

0.230

HCS (0.009)

HFBR-4505Z (Gray)/4515Z (Blue) Adapters

Panel Mounting – Bulkhead Feedthrough

Bulkhead Feedthrough with Two HFBR-4501Z/4511Z  Connectors

Versatile Link Mechanical Dimensions, continued

HFBR-4516Z (Parchment) Duplex Latching Connector

HFBR-4506Z (Parchment) Duplex Connector

HFBR-4503Z (Gray)/4513Z  (Blue) Simplex Latching

Connector

10

HFBR-4501Z (Gray)/4511Z  (Blue) Simplex Connector

For product information  and a complete list of distributors,  please go to our website:          www.avagotech.com

Avago, Avago Technologies, and the A logo are trademarks of Avago Technologies Limited in the United States and other countries. Data subject to change. Copyright © 2006 Avago Technologies Limited. All rights reserved.

AV01-0150EN  May 14, 2006