500kHz and 1MHz

High Efficiency

1.5A Switching Regulators

FEATURES 	DESCRIPTIO

■    Faster Switching with Increased Efficiency

■    Uses Small Inductors: 4.7H

■   All Surface Mount Components

■   Only 0.5 Square Inch of Board Space

■   Low Minimum Supply Voltage: 2.7V

■   Quiescent Current: 4mA Typ

■   Current Limited Power Switch: 1.5A

■   Regulates Positive or Negative Outputs

■   Shutdown Supply Current: 12A Typ

■    Easy External Synchronization

■    8-Pin SO or PDIP Packages

APPLICATIO  S

■    Boost Regulators

■    CCFL Backlight  Driver

■   Laptop Computer Supplies

■   Multiple Output Flyback Supplies

■   Inverting Supplies

The  LT ®1372/LT1377  are monolithic  high frequency switching regulators. They can be operated in all standard switching configurations including boost, buck, flyback, forward, inverting and “Cuk.” A 1.5A high efficiency switch is included on the die, along with all oscillator, control and protection circuitry. All functions of the LT1372/LT1377 are integrated into 8-pin SO/PDIP packages.

The LT1372/LT1377  typically consumes only 4mA quies- cent current and has higher efficiency than previous parts. High frequency switching allows for very small inductors to be used. All surface mount components consume less than 0.5 square inch of board space.

New design techniques increase flexibility and maintain ease of use. Switching is easily synchronized to an exter- nal logic level source. A logic low on the shutdown pin reduces supply current to 12A. Unique error amplifier circuitry can regulate positive or negative output voltage while maintaining simple frequency compensation tech- niques. Nonlinear error amplifier transconductance re- duces output overshoot on start-up or overload recovery. Oscillator frequency shifting protects external compo- nents during overload conditions.

, LTC and LT are registered trademarks of Linear Technology Corporation.

TYPICAL APPLICATIO

5V-to-12V  Boost Converter

12V Output Efficiency

OFF

ON   4

S/S

5V

5

VIN

L1*

4.7H

VSW

D1

MBRS120T3

8

R1

53.6k

1%

VOUT†

12V

*FOR LT1372 USE 10H

VIN = 5V

90

80

+	C1**

LT1372/LT1377

2

FB

+    C4**

COILCRAFT DO1608-472 (4.7H) OR

EFFICIENCY (%)

COILCRAFT DT3316-103 (10H) OR	70

SUMIDA CD43-4R7 (4.7H) OR

22F

GND	VC

6, 7	1

C2

R2

6.19k

1%

22F

SUMIDA CD73-100KC (10H) OR

**AVX TPSD226M025R0200	60

†MAX IOUT

0.047F

R3

2k

C3

0.0047F

LT1372 • TA01

| 50 L1 | IOUT (LT1377) | IOUT (LT1372) |
| --- | --- | --- |
| 4.7H 10H | 0.25A 0.35A | NA 0.29A |

0.01

0.1	1

OUTPUT CURRENT (A)

LT1372 • TA02

ABSOLUTE 	AXI	U	RATI  GS

(Note 1)

Supply Voltage .......................................................   30V Switch Voltage

LT1372/LT1377 ..................................................   35V LT1372HV ..........................................................   42V S/S Pin Voltage .......................................................   30V Feedback Pin Voltage (Transient, 10ms)  ..............   10V Feedback Pin Current ...........................................  10mA

Negative Feedback Pin Voltage

(Transient, 10ms) .............................................  10V Operating Junction Temperature Range

Commercial ........................................  0C to 125C*

Industrial ......................................... – 40C to 125C Short Circuit ......................................... 0C to 150C

Storage Temperature Range ................   – 65C to 150C

Lead Temperature (Soldering, 10 sec) ................. 300C

PACKAGE/ORDER  I  FOR	ATIO

| TOP VIEW VC 1 8 VSW FB 2 7 GND NFB 3 6 GND S S/S 4 5 V N8 PACKAGE S8 PACKAGE 8-LEAD PDIP 8-LEAD PLASTIC SO TJMAX = 125C, JA = 100C/ W (N8) TJMAX = 125C, JA = 120C/ W (S8) | ORDER PART NUMBER |
| --- | --- |
|  | LT1372CN8 LT1372HVIN8 LT1372HVCN8 LT1372IS8 LT1372CS8 LT1372HVIS8 LT1372HVCS8 LT1377CS8 LT1372IN8 LT1377IS8 |
|  | S8 PART MARKING |
|  | 1372 1372H 1377 1372I 1372HI 1377I |

Consult factory for parts specified with wider operating temperature ranges.

*Units  shipped prior to Date Code 9552 are rated at 100C maximum operating temperature.

ELECTRICAL CHARACTERISTICS

The ● denotes specifcatons which appy over the full operating temperature range, otherwise specifications are at TA = 25C.

VIN = 5V, VC = 0.6V, VFB = VREF, VSW, S/S and NFB pins open, unless otherwise noted.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| VREF | Reference Voltage | Measured at Feedback Pin VC = 0.8V | ● | 1.230 1.245 1.260 1.225 1.245 1.265 | V V |
| IFB | Feedback Input Current | VFB = VREF | ● | 250 550 900 | nA nA |
|  | Reference Voltage Line Regulation | 2.7V ≤ VIN ≤ 25V, VC = 0.8V | ● | 0.01 0.03 | %/ V |
| VNFB | Negative Feedback Reference Voltage | Measured at Negative Feedback Pin Feedback Pin Open, VC = 0.8V | ● | – 2.540 – 2.490 – 2.440 – 2.570 – 2.490 – 2.410 | V V |
| INFB | Negative Feedback Input Current | VNFB = VNFR | ● | –45 – 30 – 15 | A |
|  | Negative Feedback Reference Voltage Line Regulation | 2.7V ≤ VIN ≤ 25V, VC = 0.8V | ● | 0.01 0.05 | %/V |
| gm | Error Amplifier Transconductance | ∆IC =  25A | ● | 1100 1500 1900 700 2300 | mho mho |
|  | Error Amplifier Source Current | VFB = VREF – 150mV, VC = 1.5V | ● | 120 200 350 | A |
|  | Error Amplifier Sink Current | VFB = VREF + 150mV, VC = 1.5V | ● | 1400 2400 | A |
|  | Error Amplifier Clamp Voltage | High Clamp, VFB = 1V Low Clamp, VFB = 1.5V |  | 1.70 1.95 2.30 0.25 0.40 0.52 | V V |
| AV | Error Amplifier Voltage Gain |  |  | 500 | V/ V |
|  | VC Pin Threshold | Duty Cycle = 0% |  | 0.8 1 1.25 | V |
| f | Switching Frequency | 2.7V ≤ VIN ≤ 25V LT1372 0C ≤ TJ ≤ 125C – 40C ≤ TJ < 0C (I Grade) LT1377 0C ≤ TJ ≤ 125C – 40C ≤ TJ < 0C (I Grade) | ● ● | 450 500 550 430 500 580 400 580 0.90 1 1.10 0.86 1 1.16 0.80 1.16 | kHz kHz kHz MHz MHz MHz |

ELECTRICAL CHARACTERISTICS

The ● denotes specifcatons which appy over the full operating temperature range, otherwise specifications are at TA = 25C.

VIN = 5V, VC = 0.6V, VFB = VREF, VSW, S/S and NFB pins open, unless otherwise noted.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
|  | Maximum Switch Duty Cycle |  | ● | 85 95 | % |
|  | Switch Current Limit Blanking Time |  |  | 130 260 | ns |
| BV | Output Switch Breakdown Voltage | LT1372/LT1377 LT1372HV 0C ≤ TJ ≤ 125C – 40C ≤ TJ < 0C (I Grade) | ● ● | 35 47 42 47 40 | V V V |
| VSAT | Output Switch “On” Resistance | ISW = 1A | ● | 0.5 0.8 | Ω |
| ILIM | Switch Current Limit | Duty Cycle = 50% Duty Cycle = 80% (Note 2) | ● ● | 1.5 1.9 2.7 1.3 1.7 2.5 | A A |
| ∆IIN ∆ISW | Supply Current Increase During Switch On-Time |  |  | 15 25 | mA/A |
|  | Control Voltage to Switch Current Transconductance |  |  | 2 | A/ V |
|  | Minimum Input Voltage |  | ● | 2.4 2.7 | V |
| IQ | Supply Current | 2.7V ≤ VIN ≤ 25V | ● | 4 5.5 | mA |
|  | Shutdown Supply Current | 2.7V ≤ VIN ≤ 25V, VS/S ≤ 0.6V 0C ≤ TJ ≤ 125C – 40C ≤ TJ < 0C (I Grade) | ● | 12 30 50 | A A |
|  | Shutdown Threshold | 2.7V ≤ VIN ≤ 25V | ● | 0.6 1.3 2 | V |
|  | Shutdown Delay |  | ● | 5 12 25 | s |
|  | S/S Pin Input Current | 0V ≤ VS/S ≤ 5V | ● | –10 15 | A |
|  | Synchronization Frequency Range | LT1372 LT1377 | ● ● | 600 800 1.2 1.6 | kHz MHz |

Note 1: Absolute Maximum Ratings are those values beyond which the life of the device may be impaired.

Note 2: For duty cycles (DC) between 50% and 90%, minimum guaranteed switch current is given by ILIM  = 0.667 (2.75 – DC).

TYPICAL PERFOR 	A	CE CHARACTERISTICS

1.0

0.7

0.6

0.5

0.3

0.2

SWITCH SATURATION VOLTAGE (V)

0.1

0

Switch Saturation Voltage vs Switch Current

150C

100C

–55C

25C

3.0

2.5

2.0

1.5

1.0

SWITCH CURRENT LIMIT  (A)

0.5

0

Switch Current Limit vs Duty Cycle

–55C

25C AND

125C

| 3.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

2.8

INPUT VOLTAGE (V)

2.6

2.4

2.2

2.0

1.8

Minimum Input Voltage vs Temperature

0   0.2 0.4

0.6 0.8

1.0 1.2 1.4 1.6 1.8 2.0

0   10  20

30  40

50  60

70  80

90 100

–50

–25    0

25    50

75   100

125

150

SWITCH CURRENT (A)

LT1372 • G01

DUTY CYCLE (%)

LT1372 • G02

TEMPERATURE (C)

LT1372 • G03

TYPICAL PERFOR 	A CE CHARACTERISTICS

Shutdown Delay and Threshold vs Temperature

20

18

SHUTDOWN DELAY (s)

16	SHUTDOWN THRESHOLD

14

12

10

8	SHUTDOWN DELAY

6

4

2

0

2.0

1.8

SHUTDOWN THRESHOLD (V)

1.6

1.4

1.2

1.0

0.8

0.6

0.4

0.2

0

| 3.0 fSYNC = 700kHz (LT1372) fSYNC = 1.4MHz (LT1377) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LT1 | 377 |  |  |  |
|  |  |  | LT1 | 372 |  |  |  |

2.5

2.0

1.5

1.0

0.5

MINIMUM SYNCHRONIZATION VOLTAGE (VP-P)

0

Minimum Synchronization

Voltage vs Temperature

| 400 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 25C |  |  |  |  |  |
|  |  |  |  | –55C |  |  |  |  |  |
|  |  | 125C |  |  |  |  |  |  |  |

300

200

100

0

ERROR AMPLIFIER OUTPUT CURRENT (A)

–100

–200

–300

Error Amplifier Output Current vs Feedback Pin Voltage

–50 –25   0

25   50	75

100  125  150

–50

–25    0

25    50

75   100

125

150

– 0.3	–0.2

–0.1

VREF

0.1

TEMPERATURE (C)

TEMPERATURE (C)

FEEDBACK PIN VOLTAGE (V)

LT1372 • G04

LT1372 • G05

LT1372 • G06

S/S Pin Input Current vs Voltage

| 5 VIN = 5V |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4

3

2

1

0

–1

–2

S/S PIN INPUT CURRENT (A)

–3

–4

–5

| 110 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SWITCHING  FREQUENCY (% OF TYPICAL)

100

90

80

70

60

50

40

30

20

10

Switching Frequency

vs Feedback Pin Voltage

2000

1800

1600

TRANSCONDUCTANCE (mho)

1400

1200

1000

800

600

400

200

0

Error Amplifier Transconductance vs Temperature

g   = I (VC)

m

V (FB)

–1   0    1    2

3    4	5    6    7    8    9

0   0.1 0.2 0.3 0.4 0.5 0.6 0.7 0.8 0.9 1.0

–50 –25   0

25   50	75

100  125  150

S/S PIN VOLTAGE (V)

FEEDBACK PIN VOLTAGE (V)

TEMPERATURE (C)

LT1372 • G07

LT1372 • G08

LT1372 • G09

2.4

2.2

2.0

VC PIN VOLTAGE (V)

1.8

1.6

1.4

1.2

1.0

0.8

0.6

0.4

VC Pin Threshold and High

Clamp Voltage vs Temperature

VC HIGH CLAMP

VC THRESHOLD

| 800 VFB =VREF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

600

500

400

300

FEEDBACK INPUT CURRENT (nA)

100

0

Feedback Input Current vs Temperature

NEGATIVE FEEDBACK INPUT CURRENT (A)

| 0 VNFB =VNFR |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

–10

–20

–30

–40

–50

Negative Feedback Input Current vs Temperature

–50 –25   0

25   50	75

100  125  150

–50

–25    0

25   50

75   100

125

150

–50 –25   0

25   50	75

100  125  150

TEMPERATURE (C)

TEMPERATURE (C)

TEMPERATURE (C)

LT1372 • G10

LT1372 • G11

LT1372 • G12

PI 	FU  CTIO  S

VC (Pin 1): The compensation pin is used for frequency compensation, current limiting and soft start. It is the output of the error amplifier and the input of the current comparator. Loop frequency compensation can be per-

formed with an RC network connected from the VC pin to ground.

FB (Pin 2): The feedback pin is used for positive output voltage sensing and oscillator frequency shifting. It is the inverting input to the error amplifier. The noninverting input of this amplifier is internally tied to a 1.245V reference. Load on the FB pin should not exceed 250A when the NFB pin is used. See Applications Information.

NFB (Pin 3): The negative feedback pin is used for negative output voltage sensing. It is connected to the inverting input of the negative feedback amplifier through  a 100k source resistor.

S/S (Pin 4): Shutdown and Synchronization Pin. The S/S pin is logic level compatible. Shutdown is active low and the shutdown  threshold  is typically 1.3V. For normal operation, pull the S/S pin high, tie it to VIN or leave it floating. To synchronize switching, drive the S/S pin be-

tween 600kHz and 800kHz (LT1372) or 1.2MHz to 1.6MHz

(LT1377).

VIN (Pin 5): Bypass input supply pin with 10F or more. The part goes into undervoltage lockout when VIN drops below

2.5V. Undervoltage lockout stops switching and pulls the

VC pin low.

GND S (Pin 6): The ground sense pin is a “clean” ground. The internal reference, error amplifier and negative feed- back amplifier are referred to the ground sense pin. Con- nect it to ground. Keep the ground path connection to the output resistor divider and the VC compensation network

free of large ground currents.

GND (Pin 7): The ground pin is the emitter connection of the power switch and has large currents flowing through it. It should be connected directly to a good quality ground plane.

VSW (Pin 8): The switch pin is the collector of the power switch and has large currents flowing  through  it. Keep the traces to the switching components as short as possible to minimize radiation and voltage spikes.

BLOCK DIAGRA

VIN 	SW

S/S

SHUTDOWN DELAY AND RESET

LOW DROPOUT

2.3V REG

ANTI-SAT

SYNC

OSC

LOGIC	DRIVER

SWITCH

NFB

100k

50k

+

NFBA

–

5:1 FREQUENCY SHIFT

COMP

FB

EA

1.245V REF

IA VC	AV  6

0.08

GND SENSE

GND    LT1372 • BD

OPERATIO

The LT1372/LT1377 are current  mode switchers.  This means that switch  duty cycle is directly controlled by switch current rather than by output voltage. Referring to the block diagram, the switch is turned “On” at the start of each oscillator cycle. It is turned “Off” when switch current reaches a predetermined level. Control of output voltage is obtained by using the output of a voltage sensing error amplifier to set current  trip level. This technique  has several advantages. First, it has immediate response to input voltage variations, unlike voltage mode switchers which  have notoriously  poor line transient  response. Second, it reduces the 90 phase shift at mid-frequencies in the energy storage inductor. This greatly simplifies closed-loop frequency compensation under widely vary- ing input voltage or output  load conditions.  Finally, it allows simple pulse-by-pulse current limiting to provide maximum switch protection under output overload or short conditions. A low dropout internal regulator pro- vides a 2.3V supply  for all internal circuitry.  This low dropout design allows input voltage to vary from 2.7V to

25V with virtually no change in device performance.  A

500kHz (LT1372) or 1MHz (LT1377) oscillator is the basic clock for all internal timing. It turns “On” the output switch via the logic and driver circuitry. Special adaptive anti-sat circuitry detects onset of saturation in the power switch and adjusts driver current instantaneously to limit switch saturation. This minimizes driver dissipation and provides very rapid turn-off of the switch.

A 1.245V bandgap reference biases the positive input of the error amplifier. The negative input of the amplifier is brought out for positive output voltage sensing. The error amplifier has nonlinear transconductance to reduce out-

put overshoot on start-up or overload recovery. When the feedback voltage exceeds the reference by 40mV, error amplifier transconductance increases ten times, which reduces output overshoot. The feedback input also invokes oscillator frequency shifting, which helps pro- tect components during overload conditions. When the feedback voltage drops below 0.6V, the oscillator fre- quency is reduced 5:1. Lower switching frequency allows full control of switch current limit by reducing minimum switch duty cycle.

Unique error amplifier circuitry allows the LT1372/LT1377 to directly regulate negative output voltages. The negative feedback amplifier’s 100k source resistor is brought out for negative output voltage sensing. The NFB pin regulates at – 2.49V while the amplifier output internally drives the FB pin to 1.245V. This architecture, which uses the same main error amplifier, prevents duplicating functions and maintains ease of use. Consult Linear Technology Market- ing for units that can regulate down to – 1.25V.

The error signal developed at the amplifier output is brought out externally. This pin (VC) has three different functions. It is used for frequency compensation, current limit adjustment and soft starting. During normal regula- tor operation this pin sits at a voltage between 1V (low output current) and 1.9V (high output current). The error

amplifier is a current output (gm) type, so this voltage can be externally clamped for lowering current limit. Like- wise, a capacitor coupled external clamp will provide soft start. Switch duty cycle goes to zero if the VC pin is pulled below the control pin threshold, placing the LT1372/ LT1377 in an idle mode.

APPLICATIO 	S I  FOR	ATIO

Positive Output Voltage Setting

VOUT

OUT

REF 

R2 

The LT1372/LT1377 develops a 1.245V  reference  (VREF)

from the FB pin to ground. Output voltage is set by

R1    V	= V

FB

1 + R1

connecting  the FB  pin to an  output resistor  divider (Figure 1). The FB pin bias current represents a small error and can usually be ignored for values of R2 up to 7k.

The suggested  value for R2 is 6.19k.  The NFB pin is

PIN

VREF

R1 = R2  VOUT  – 1

R2	1.245

	

LT1372 • F01

normally  left open  for positive  output applications.	Figure 1. Positive Output Resistor Divider

APPLICATIO 	S I  FOR	ATIO

Positive fixed voltage versions are available (consult

Linear Technology marketing).

Negative Output Voltage Setting

The LT1372/LT1377  develops a – 2.49V reference (VNFR) from  the NFB pin to ground. Output voltage is set by connecting  the  NFB pin to an output  resistor  divider (Figure 2). The – 30A  NFB pin bias current (INFB) can cause output voltage errors and should not be ignored. This has been accounted for in the formula in Figure 2. The suggested value for R2 is 2.49k. The FB pin is normally left open for negative output application. See Dual Polarity Output Voltage Sensing for limitatins  on FB pin loading when using the NFB pin.

–VOUT

Shutdown and Synchronization

The dual function S/S pin provides easy shutdown and synchronization. It is logic level compatible  and can be pulled high, tied to VIN or left floating for normal operation. A logic low on the S/S pin activates shutdown, reducing the part’s supply current to 12A. Typical synchronization range is from 1.05 to 1.8 times the part’s natural switching frequency, but is only guaranteed between 600kHz and

800kHz (LT1372) or 1.2MHz and 1.6MHz (LT1377). At start-up, the synchronization signal should not be applied until the feedback pin is above the frequency shift voltage of 0.7V. If the NFB pin is used, synchronization should not be applied until the NFB pin is more negative than – 1.4V. A 12s resetable shutdown delay network guarantees the

part will not go into shutdown while receiving a synchro-

NFB

INFB

R1    –VOUT = VNFB 1 +

R1  + INFB (R1)

nization signal.

Caution should be used when synchronizing above 700kHz

PIN

VNFR

R2    R1 =

VOUT– 2.49

	 + 30  10  

(LT1372) or 1.4MHz (LT1377) because at higher sync frequencies the amplitude of the internal slope compensa-

2.49

R2

R2

–6

LT1372 • F02

tion used to prevent subharmonic switching is reduced.

Figure 2. Negative Output Resistor Divider

Dual Polarity Output Voltage Sensing

Certain applications benefit from sensing both positive and negative output voltages. One example is the “Dual Output Flyback Converter with  Overvoltage Protection” circuit shown in the Typical Applications  section. Each output voltage resistor divider is individually set as de- scribed  above. When both the FB and NFB pins are used, the LT1372/LT1377 acts to prevent either output from going beyond its set output voltage. For example in this application, if the positive output were more heavily loaded than the negative, the negative output would be greater and would regulate at the desired set-point voltage. The positive output  would sag slightly  below its set-point voltage. This technique prevents either output from going unregulated high at no load. Please note that the load on the FB pin should  not exceed 250A when the NFB pin is used. This situation occurs when the resistor dividers are used at both FB and NFB. True load on FB is not the full divider current unless the positive output is shorted to ground. See Dual Output Flyback Converter application.

This type of subharmonic switching only occurs when the duty cycle of the switch is above 50%. Higher inductor values will tend to eliminate problems.

Thermal Considerations

Care should be taken to ensure that the worst-case  input voltage and load current conditions do not cause exces- sive die temperatures. The packages are rated at 120C/ W for SO (S8) and 130C/ W for PDIP (N8).

Average supply current (including  driver current) is: IIN = 4mA + DC (ISW/60  + ISW  0.004)

ISW = switch current

DC = switch duty cycle

Switch power dissipation is given by: PSW = (ISW)2   RSW  DC

RSW = output switch “On” resistance

Total power dissipation of the die is the sum of supply current times supply voltage plus switch power:

PD(TOTAL) = (IIN  VIN) + PSW

APPLICATIO 	S I  FOR	ATIO

Choosing the Inductor

For most applications the inductor will fall in the range of

2.2H to 22H. Lower values are chosen to reduce physi- cal size of the inductor. Higher values allow more output

current  because they reduce peak current  seen by the

power switch, which has a 1.5A limit. Higher values also reduce input ripple voltage and reduce core loss.

When choosing an inductor you might have to consider maximum load current, core and copper losses, allowable component height, output voltage ripple, EMI, fault current in the inductor, saturation, and of course, cost. The following procedure is suggested as a way of handling these somewhat complicated and conflicting requirements.

1. Assume that the average inductor current for a boost converter is equal to load current times VOUT / VIN and decide whether or not the inductor must withstand continuous overload conditions. If average inductor current at maximum load current is 0.5A, for instance, a 0.5A inductor  may not survive a continuous  1.5A

overload condition. Also be aware that boost convert- ers  are not short  circuit  protected, and that under output short conditions, inductor current is limited only by the available current of the input supply.

2. Calculate peak inductor current at full load current to ensure that the inductor  will not saturate. Peak current can be significantly higher than output current, espe- cially with smaller inductors and lighter loads, so don’t omit this step. Powdered iron cores are forgiving be- cause they saturate softly, whereas ferrite cores satu- rate abruptly.  Other core  materials  fall in between somewhere. The following formula assumes continu- ous mode operation but it errors only slightly on the high side for discontinuous mode, so it can be used for all conditions.

radiation, or whether it needs a closed core like a toroid to prevent EMI problems.  One would not want an open core next to a magnetic  storage media for instance! This is a tough decision because the rods or barrels are temptingly  cheap and small, and there are no helpful guidelines to calculate when the magnetic field radia- tion will be a problem.

4. Start shopping for an inductor which meets the re- quirements of core shape, peak current  (to avoid saturation), average current (to limit heating) and fault current. If the inductor gets too hot, wire insulation will melt and cause turn-to-turn shorts. Keep in mind that all good things like high efficiency, low profile and high temperature operation will increase cost, sometimes dramatically.

5. After making an initial choice, consider the secondary things like output voltage ripple, second sourcing, etc. Use the experts in the Linear Technology application department if you feel uncertain about the final choice. They have experience with  a wide range of inductor types and can tell you about the latest developments in low profile, surface mounting, etc.

Output Capacitor

The output capacitor is normally chosen by its effective series resistance, (ESR), because this is what determines output ripple voltage. At 500kHz, any polarized capacitor is essentially  resistive.  To get low ESR takes volume, so physically  smaller capacitors have high ESR. The ESR range for typical LT1372 and LT1377 applications is

0.05  to 0.5. A typical output capacitor is an AVX type

TPS, 22F at 25V, with a guaranteed ESR less than 0.2. This is a “D” size surface mount solid tantalum capacitor.

TPS capacitors are specially constructed and tested for low ESR, so they give the lowest ESR for a given volume.

To further reduce ESR, multiple output capacitors can be

IPEAK = IOUT  VOUT + VIN

VIN(VOUT – VIN)

2(f)(L)(VOUT)

used in parallel. The value in microfarads is not particu- larly critical, and values from 22F to greater than 500F

VIN = Minimum Input Voltage

f = 500kHz Switching Frequency (LT1372) or

1MHz Switching Frequency (LT1377)

3. Decide if the design can tolerate an “open” core geom- etry like a rod or barrel, which have high magnetic  field

work well, but you cannot cheat mother nature on ESR. If you find a tiny 22F solid tantalum capacitor, it will have high ESR, and output  ripple voltage will be terrible. Table

1 shows some typical solid tantalum surface mount capacitors.

APPLICATIO 	S I  FOR	ATIO

Table 1. Surface Mount Solid Tantalum Capacitor

| ESR and Ripple Current E CASE SIZE |  | ESR (MAX Ω) | RIPPLE CURRENT (A) |  |
| --- | --- | --- | --- | --- |
| AVX TPS, Sprague 593D AVX TAJ |  | 0.1 to 0.3 0.7 to 0.9 | 0.7 to 1.1 0.4 |  |
| D CASE SIZE |  |  |  |  |
| AVX TPS, Sprague 593D AVX TAJ |  | 0.1 to 0.3 0.9 to 2.0 | 0.7 to 1.1 0.36 to 0.24 |  |
| C CASE SIZE |  |  |  |  |
| AVX TPS AVX TAJ |  | 0.2 (Typ) 1.8 to 3.0 | 0.5 (Typ) 0.22 to 0.17 |  |
| B CASE SIZE |  |  |  |  |
| AVX TAJ |  | 2.5 to 10 | 0.16 to 0.08 |  |

IRIPPLE = 0.3(VIN)(VOUT – VIN) (f)(L)(VOUT)

f = 500kHz Switching frequency (LT1372) or,

1MHz Switching frequency (LT1377)

Many engineers have heard that solid tantalum capacitors are prone to failure if they undergo high surge currents. This  is historically true and  type  TPS capacitors  are specially tested for surge capability, but surge ruggedness is not a critical  issue with the output capacitor. Solid tantalum capacitors fail during very high turn-on surges, which do not occur at the output  of regulators. High discharge surges, such as when the regulator output is dead shorted, do not harm the capacitors.

Single inductor boost regulators have large RMS ripple current in the output capacitor, which must be rated to handle the current. The formula to calculate this is:

Output Capacitor Ripple Current (RMS)

DC

The input capacitor can see a very high surge current when a battery or high capacitance source is connected “live” and solid tantalum capacitors can fail under this condition. Several manufacturers   have developed  a line of solid tantalum capacitors specially tested for surge capability (AVX TPS series, for instance), but even these units may fail if the input voltage approaches the maximum voltage rating of the capacitor. AVX recommends derating capaci- tor voltage by 2:1 for high surge applications. Ceramic and aluminum electrolytic capacitors may also be used and have a high tolerance to turn-on surges.

Ceramic Capacitors

Higher value, lower  cost ceramic capacitors are now becoming available in smaller case sizes. These are tempt- ing for switching  regulator use because of their very low ESR. Unfortunately,  the ESR is so low that it can cause loop stability  problems. Solid tantalum capacitor ESR generates a loop “zero” at 5kHz to 50kHz that is instrumen- tal in giving acceptable loop phase margin. Ceramic ca- pacitors remain capacitive to beyond 300kHz and usually resonate with their ESL before ESR becomes effective. They are appropriate for input bypassing because of their

IRIPPLE (RMS) = IOUT

= IOUT

Input Capacitors

1 – DC

VOUT – VIN

VIN

high ripple current ratings and tolerance of turn-on surges. Linear Technology plans to issue a Design Note on the use of ceramic capacitors in the near future.

Output Diode

The suggested output diode (D1) is a 1N5818 Schottky or

The input capacitor of a boost converter is less critical due

to the fact that the input current waveform is triangular and does not contain large squarewave currents as is found in the output capacitor. Capacitors in the range of 10F to

100F with an ESR of 0.3 or less work well up to full 1.5A

switch  current.  Higher ESR capacitors may be acceptable at low switch currents. Input capacitor ripple current for boost converter is :

its Motorola equivalent, MBR130. It is rated at 1A average forward current and 30V reverse voltage. Typical forward voltage is 0.42V at 1A. The diode conducts current only during switch off time. Peak reverse voltage for boost converters is equal to regulator output voltage. Average forward current in normal operation is equal to output current.

APPLICATIO 	S I  FOR	ATIO

Frequency Compensation

Loop frequency compensation is performed on the output of the error amplifier (VC pin) with a series RC network. The main pole is formed by the series capacitor and the output impedance ( 500k) of the error amplifier. The pole falls in the range of 2Hz to 20Hz. The series resistor creates a “zero”  at 1kHz to 5kHz, which improves loop stability  and transient  response. A second capacitor, typically  one-tenth the size of the main compensation

capacitor, is sometimes  used to reduce the switching frequency ripple on the VC pin. VC pin ripple is caused by output voltage ripple attenuated by the output divider and multiplied by the error  amplifier.  Without  the second

(magnetic) radiation is minimized by keeping output di- ode, switch  pin, and output  bypass capacitor leads as short as possible. E field radiation is kept low by minimiz- ing the length and area of all traces connected to the switch pin. A ground  plane should always be used under the switcher circuitry to prevent interplane coupling.

The high speed switching current path is shown schemati- cally in Figure 3. Minimum  lead length in this path is essential to ensure clean switching  and low EMI. The path including the switch, output diode, and output capacitor is the only one containing nanosecond rise and fall times. Keep this path as short as possible.

SWITCH

capacitor, VC pin ripple is:

VC Pin Ripple = 1.245(VRIPPLE)(gm)(RC) (VOUT)

VRIPPLE = Output ripple (VP–P)

gm = Error amplifier transconductance

(1500mho)

RC = Series resistor on VC pin

VOUT = DC output voltage

VIN

More Help

L1   NODE

HIGH FREQUENCY CIRCULATING PATH

Figure 3

LOAD

VOUT

LT1372 • F03

To prevent irregular switching,  VC pin ripple should be kept below 50mVP–P. Worst-case VC pin ripple occurs at maximum output load current and will also be increased if poor quality (high ESR) output capacitors are used. The addition of a 0.0047F capacitor on the VC pin reduces switching frequency ripple to only a few millivolts. A low value for RC will also reduce VC pin ripple, but loop phase margin may be inadequate.

Switch Node Considerations

For maximum  efficiency, switch  rise and fall time are made as short as possible. To prevent radiation and high frequency resonance problems, proper layout of the com- ponents connected to the switch node is essential. B field

For more  detailed information  on switching  regulator circuits,  please see Application Note 19. Linear Technol- ogy also offers a computer software program, SwitcherCAD, to assist in designing switching converters. In addition, our applications department is always ready to lend  a helping hand.

TYPICAL APPLICATIO 	S

Positive-to-Negative Converter with Direct Feedback

Dual Output Flyback Converter with Overvoltage Protection

+	C1

VIN

2.7V TO 16V

D2	2

T1*

4

R2

1.21k

1%

R1

13k

1%

22F

5

VIN

• P6KE-15A

D3

1N4148   •

+

C4

47F

VIN

2.7V TO 13V

MBRS140T3

T1*

VOUT

OFF

ON   4

S/S

8

VSW

1	3

D1	R2

–VOUT

–5V

†

+

C1

22F

2, 3	5

P6KE-20A •

+		15V C4

LT1372/LT1377

3

NFB

MBRS130LT3

2.49k

1%

R3

OFF

ON   4

2

FB S/S

5

VIN     8

VSW

1N4148

• 4

6, 7	8

47F

+

C5

VC 	GND

1	6, 7

2.49k

1%

LT1372/LT1377

3

NFB

•	47F

1	–VOUT

C2

0.047F

C3

*COILTRONICS CTX10-2 (407) 241-7876

†MAX IOUT

VC 	GND

MBRS140T3

R4

12.1k

–15V

R1

0.0047F	2k

IOUT

0.3A

0.5A

VIN

3V

5V

LT1372 • TA03

1

C2

0.047F

6, 7

1%

R5

2.49k

0.75A  9V

C3

0.0047F

R3

2k

*DALE LPE-4841-100MB (605) 665-9301

1%

LT1372 • TA04

Low Ripple 5V to – 3V “Cuk”†  Converter

90%  Efficient CCFL Supply

VIN

5V

L1*

2	3

VOUT

–3V

250mA

C2

27pF

5mA MAX LAMP

10

D1

1N4148

1 •	• 4

C2	R1

VIN

4.5V

T1

5	4	3	2	1

5     LT1372/LT1377	8

47F	1k

16V	1%

TO 30V   +

10F	C1

0.1F

C1

22F

10V

VIN

4

S/S

7

GND

6

GND S

VSW

3

NFB

1

VC

+

D1**

C3

47F

16V

C6

0.1F

2.7V TO

Q1	Q2

330

L1	D2

C5

0.0047F

R4

2k

C4

0.047F

R2

4.99k

1%

5.5V

+

2.2F

4

5

VIN

1N5818

8

33H

562*

1N4148

10k

*SUMIDA CLS62-100L

**MOTOROLA MBR0520LT3

LT1372 • TA05

ON OFF

S/S

VSW

20k

†PATENTS MAY APPLY

LT1372/LT1377

2

VFB

DIMMING

GND

6, 7

VC

+ 1

2F

22k

1N4148

OPTIONAL REMOTE DIMMING

0.1F

LT1372 • TA06

C1 = WIMA MKP-20

L1 = COILCRAFT DT3316-333

CCFL BACKLIGHT APPLICATION CIRCUITS CONTAINED IN THIS DATA SHEET ARE

Q1, Q2 = ZETEX ZTX849 OR ROHM 2SC5001 COVERED BY U.S. PATENT NUMBER 5408162

T1 = COILTRONICS CTX 110609

* = 1% FILM RESISTOR

DO NOT SUBSTITUTE COMPONENTS

COILTRONICS (407) 241-7876

COILCRAFT (708)  639-6400

AND OTHER PATENTS PENDING

Information  furnished by Linear Technology Corporation  is believed  to be accurate  and  reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no represen- tation that the interconnection of circuits as described herein will not infringe on existing patent rights.

11

TYPICAL APPLICATIO 	S

2 Li-Ion Cell to 5V SEPIC Converter

VIN

4V TO 9V

OFF

ON  4

S/S

5

VIN

8

VSW

L1A*

10H

•

MBRS130LT3

VOUT†

5V

C1 = AVX TPSD 336M020R0200

+	C1

LT1372/LT1377

FB  2

C2

1F

R2

18.7k

1%

+	C3

C2 = TOKIN 1E105ZY5U-C103-F

C3 = AVX TPSD107M010R0100

*SINGLE INDUCTOR WITH TWO WINDINGS

33F

20V

GND	VC

6, 7	1

L1B*

10H

100F

10V

COILTRONICS CTX10-1

†MAX IOUT

R1

2k

C4

0.047F

C5

0.0047F

R3

6.19k

1%

IOUT

0.45A

0.55A

0.65A

0.72A

VIN

4V

5V

7V

9V

LT1372 • TA07

PACKAGE DESCRIPTIO

N8 Package

8-Lead PDIP (Narrow 0.300)

(LTC DWG # 05-08-1510)

Dimensions in inches (millimeters)  unless otherwise noted.

S8 Package

8-Lead Plastic Small Outline (Narrow 0.150)

(LTC DWG # 05-08-1610)

0.400*    	 (10.160)

8	7	6	5

0.189 – 0.197* (4.801 – 5.004)

8	7	6	5

(6.477  0.381)

0.228 – 0.244 (5.791 – 6.197)

0.150 – 0.157** (3.810 – 3.988)

1	2	3	4

0.300 – 0.325 	 (7.620 – 8.255)

0.009 – 0.015

0.065 (1.651) TYP

0.045 – 0.065 (1.143 – 1.651)

0.130  0.005 (3.302  0.127)

0.010 – 0.020  45

(0.254 – 0.508)

0.008 – 0.010 (0.203 – 0.254)

1	2	3	4

0.053 – 0.069 (1.346 – 1.752)

0– 8 TYP

0.004 – 0.010 (0.101 – 0.254)

(0.229 – 0.381)

+0.035

0.325 –0.015

	

0.100

0.125

(3.175) MIN

0.018  0.003

0.020 (0.508)

MIN

0.016 – 0.050

(0.406 – 1.270)

0.014 – 0.019      	 	 (0.355 – 0.483)

TYP

0.050 (1.270) BSC

SO8 1298

+0.889

8.255 –0.381

(2.54) BSC

(0.457  0.076)

N8 1098

*DIMENSION DOES NOT INCLUDE MOLD FLASH. MOLD FLASH SHALL  NOT EXCEED 0.006" (0.152mm) PER SIDE

*THESE DIMENSIONS DO NOT INCLUDE MOLD FLASH OR PROTRUSIONS. MOLD  FLASH OR PROTRUSIONS  SHALL  NOT EXCEED 0.010  INCH (0.254mm)

**DIMENSION DOES NOT INCLUDE INTERLEAD FLASH. INTERLEAD FLASH SHALL  NOT EXCEED 0.010" (0.254mm) PER SIDE

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT1370 | High Efficiency DC/DC Converter | 42V, 6A, 500kHz Switch |
| LT1767 | 1.5A, 1.25MHz Step-Down Switching Regulator | 3V to 25V Input, VREF = 1.2V, Synchronizable Up to 2MHz, MSOP Package |
| LT1374 | High Efficiency Step-Down Switching Regulator | 25V, 4.5A, 500kHz Switch |
| LTC1735-1 | High Efficiency Step-Down Controller with Power Good | Output Fault Protection, 16-Pin SSOP and SO-8 |
| LTC®3402 | Single Cell, High Current (2A), Micropower, Synchronous 3MHz Step-Up DC/DC Converter | VIN = 0.7V to 5V, Up to 95% Efficiency Synchronizable Oscillator from 100kHz to 3MHz |

Linear Technology Corporation

12

1630 McCarthy Blvd., Milpitas, CA 95035-7417

(408)432-1900 ● FAX: (408) 434-0507 ● www.linear-tech.com

sn13727 13727fbs LT/TP 0401 2K REV B • PRINTED IN THE USA

 LINEAR TECHNOLOGY CORPORATION 1995