500mA, Low  Noise, LDO Micropower Regulators

FEATURES

n  Low Noise: 20μVRMS (10Hz to 100kHz)

n  Output Current: 500mA

n  Low Quiescent Current: 30μA

n  Wide Input Voltage Range: 1.8V to 20V

n  Low Dropout Voltage: 300mV

n  Very Low Shutdown Current: < 1μA

n  No Protection Diodes Needed

n  Fixed Output Voltages: 1.5V, 1.8V, 2.5V, 3V, 3.3V, 5V

n  Adjustable Output from 1.22V to 20V

n  Stable with 3.3μF Output Capacitor

n  Stable with Aluminum, Tantalum or Ceramic

Capacitors

n  Reverse Battery Protection

n  No Reverse Current

n  Overcurrent and Overtemperature Protected

n  8-Lead SO and 12-Lead (4mm × 3mm) DFN Packages

APPLICATIONS

n  Cellular Phones

n  Battery-Powered Systems

n  Noise-Sensitive Instrumentation Systems

DESCRIPTION

The LT®1763 series are micropower, low noise, low dropout regulators. The devices are capable of supplying 500mA of output current with a dropout voltage of 300mV. Designed for use in battery-powered systems, the low 30μA quiescent current makes them an ideal choice. Quiescent current is well controlled; it does not rise in dropout as it does with many other regulators.

A key feature of the LT1763 regulators is low output noise. With the addition of an external 0.01μF bypass capacitor, output noise drops to 20μVRMS over a 10Hz to 100kHz bandwidth. The LT1763 regulators  are stable with output capacitors as low as 3.3μF. Small ceramic capacitors can be used without the series resistance required by other regulators.

Internal protection circuitry includes reverse battery protection, current limiting, thermal limiting and reverse current protection. The parts come in ﬁxed output voltages of 1.5V, 1.8V, 2.5V, 3V, 3.3V and 5V, and as an adjustable device with a 1.22V reference voltage. The LT1763 regulators are available in 8-lead SO and 12-lead, low proﬁle (4mm × 3mm × 0.75mm) DFN packages.

, LT, LTC and LTM are registered trademarks of Linear Technology Corporation. All other trademarks are the property of their respective owners. Protected by U.S. Patents, including 6144250, 6118263.

TYPICAL APPLICATION

Dropout Voltage

3.3V Low Noise Regulator

400

VIN

3.7V TO

20V

1μF

IN		OUT SENSE

LT1763-3.3

+

0.01μF

3.3V AT 500mA

20μVRMS  NOISE

10μF

| 350 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

300

250

200

SHDN

GND

BYP

1763 TA01

150

DROPOUT  VOLTAGE (mV)

100

50

0

0	100	200	300

400

500

OUTPUT CURRENT (mA)

1763 TA02

1763fe

ABSOLUTE MAXIMUM RATINGS

(Note 1)

IN Pin Voltage ........................................................ ± 20V OUT Pin Voltage ..................................................... ± 20V Input to Output Differential Voltage........................ ± 20V SENSE Pin Voltage ............................................... ± 20V ADJ Pin Voltage ......................................................  ±7V BYP Pin Voltage .....................................................±0.6V SHDN Pin Voltage  ................................................ ± 20V Output Short-Circuit Duration ........................  Indeﬁnite

Operating Junction Temperature Range (Note 2)

C, I Grade...........................................– 40°C to 125°C MP Grade...........................................– 55°C to 125°C

Storage Temperature Range

S8 Package ........................................– 65°C to 150°C DFN Package......................................– 65°C to 150°C

Lead Temperature (Soldering, 10 sec)

S8 Package ....................................................... 300°C

PIN CONFIGURATION

TOP VIEW

NC   1

OUT     2

NC   4

SENSE/ADJ*  5

BYP   6

12   NC

11   IN

13

9    NC

8    SHDN

7    GND

OUT    1

SENSE/ADJ* 2

GND  3

BYP  4

TOP VIEW

S8 PACKAGE

8    IN

7    GND

6    GND

5    SHDN

DE PACKAGE

12-LEAD (4mm s 3mm) PLASTIC DFN

TJMAX = 125°C,  JA  = 40°C/W, JC  = 5°C/W

EXPOSED PAD (PIN 13) IS GND, MUST BE SOLDERED TO PCB

*PIN 5: SENSE FOR LT1763-1.5/LT1763-1.8/LT1763-2.5/LT1763-3/LT1763-3.3/LT1763-5

ADJ FOR LT1763

SEE THE APPLICATIONS INFORMATION SECTION.

8-LEAD PLASTIC SO

TJMAX = 150°C,  JA  = 70°C/W, JC  = 35°C/W

*PIN 2: SENSE FOR LT1763-1.5/LT1763-1.8/LT1763-2.5/LT1763-3/LT1763-3.3/LT1763-5

ADJ FOR LT1763

SEE THE APPLICATIONS INFORMATION SECTION.

ORDER INFORMATION

| LEAD FREE FINISH | TAPE AND REEL | PART MARKING* | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- |
| LT1763CDE#PBF | LT1763CDE#TRPBF | 1763 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-1.5#PBF | LT1763CDE-1.5#TRPBF | 76315 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-1.8#PBF | LT1763CDE-1.8#TRPBF | 76318 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-2.5#PBF | LT1763CDE-2.5#TRPBF | 76325 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-3#PBF | LT1763CDE-3#TRPBF | 17633 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-3.3#PBF | LT1763CDE-3.3#TRPBF | 76333 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-5#PBF | LT1763CDE-5#TRPBF | 17635 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CS8#PBF | LT1763CS8#TRPBF | 1763 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8#PBF | LT1763IS8#TRPBF | 1763 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763MPS8#PBF | LT1763MPS8#TRPBF | 1763MP | 8-Lead Plastic SO | – 55°C to 125°C |

1763fe

ORDER INFORMATION

| LEAD FREE FINISH | TAPE AND REEL | PART MARKING* | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- |
| LT1763CS8-1.5#PBF | LT1763CS8-1.5#TRPBF | 176315 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-1.5#PBF | LT1763IS8-1.5#TRPBF | 176315 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-1.8#PBF | LT1763CS8-1.8#TRPBF | 176318 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-1.8#PBF | LT1763IS8-1.8#TRPBF | 176318 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-2.5#PBF | LT1763CS8-2.5#TRPBF | 176325 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-2.5#PBF | LT1763IS8-2.5#TRPBF | 176325 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-3#PBF | LT1763CS8-3#TRPBF | 17633 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-3#PBF | LT1763IS8-3#TRPBF | 17633 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-3.3#PBF | LT1763CS8-3.3#TRPBF | 176333 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-3.3#PBF | LT1763IS8-3.3#TRPBF | 176333 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-5#PBF | LT1763CS8-5#TRPBF | 17635 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-5#PBF | LT1763IS8-5#TRPBF | 17635 | 8-Lead Plastic SO | – 40°C to 125°C |
| LEAD BASED FINISH | TAPE AND REEL | PART MARKING* | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| LT1763CDE | LT1763CDE#TR | 1763 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-1.5 | LT1763CDE-1.5#TR | 76315 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-1.8 | LT1763CDE-1.8#TR | 76318 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-2.5 | LT1763CDE-2.5#TR | 76325 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-3 | LT1763CDE-3#TR | 17633 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-3.3 | LT1763CDE-3.3#TR | 76333 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CDE-5 | LT1763CDE-5#TR | 17635 | 12-Lead (4mm × 3mm) Plastic DFN | – 40°C to 125°C |
| LT1763CS8 | LT1763CS8#TR | 1763 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8 | LT1763IS8#TR | 1763 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763MPS8 | LT1763MPS8#TR | 1763MP | 8-Lead Plastic SO | – 55°C to 125°C |
| LT1763CS8-1.5 | LT1763CS8-1.5#TR | 176315 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-1.5 | LT1763IS8-1.5#TR | 176315 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-1.8 | LT1763CS8-1.8#TR | 176318 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-1.8 | LT1763IS8-1.8#TR | 176318 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-2.5 | LT1763CS8-2.5#TR | 176325 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-2.5 | LT1763IS8-2.5#TR | 176325 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-3 | LT1763CS8-3#TR | 17633 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-3 | LT1763IS8-3#TR | 17633 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-3.3 | LT1763CS8-3.3#TR | 176333 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-3.3 | LT1763IS8-3.3#TR | 176333 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763CS8-5 | LT1763CS8-5#TR | 17635 | 8-Lead Plastic SO | – 40°C to 125°C |
| LT1763IS8-5 | LT1763IS8-5#TR | 17635 | 8-Lead Plastic SO | – 40°C to 125°C |

Consult LTC Marketing for parts speciﬁed with wider operating temperature ranges. *The temperature grade is identiﬁed by a label on the shipping container. For more information on lead free part marking, go to: http://www.linear.com/leadfree/

For more information on tape and reel speciﬁcations, go to: http://www.linear.com/tapeandreel/

1763fe

ELECTRICAL CHARACTERISTICS

The l denotes the speciﬁcations  which apply over the full operating

temperature range, otherwise speciﬁcations  are at TA = 25°C.  (Note  2)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Minimum Operating Voltage | C, I Grade: ILOAD = 500mA (Notes 3, 11) MP Grade: ILOAD = 500mA (Notes 3, 11) | l l | 1.8 2.3 1.8 2.35 | V V |
| Regulated Output Voltage (Note 4) | LT1763-1.5 VIN = 2V, ILOAD = 1mA 2.5V < VIN < 20V, 1mA < ILOAD < 500mA | l | 1.485 1.5 1.515 1.462 1.5 1.538 | V V |
|  | LT1763-1.8 VIN = 2.3V, ILOAD = 1mA 2.8V < VIN < 20V, 1mA < ILOAD < 500mA | l | 1.782 1.8 1.818 1.755 1.8 1.845 | V V |
|  | LT1763-2.5 VIN = 3V, ILOAD = 1mA 3.5V < VIN < 20V, 1mA < ILOAD < 500mA | l | 2.475 2.5 2.525 2.435 2.5 2.565 | V V |
|  | LT1763-3 VIN = 3.5V, ILOAD = 1mA 4V < VIN < 20V, 1mA < ILOAD < 500mA | l | 2.970 3 3.030 2.925 3 3.075 | V V |
|  | LT1763-3.3 VIN = 3.8V, ILOAD = 1mA 4.3V < VIN < 20V, 1mA < ILOAD < 500mA | l | 3.267 3.3 3.333 3.220 3.3 3.380 | V V |
|  | LT1763-5 VIN = 5.5V, ILOAD = 1mA 6V < VIN < 20V, 1mA < ILOAD < 500mA | l | 4.950 5 5.050 4.875 5 5.125 | V V |
| ADJ Pin Voltage (Notes 3, 4) | LT1763 VIN = 2.2V, ILOAD = 1mA C, I Grade: 2.3V < VIN < 20V, 1mA < ILOAD < 500mA MP Grade: 2.35V < VIN < 20V, 1mA < ILOAD < 500mA | l l | 1.208 1.220 1.232 1.190 1.220 1.250 1.190 1.220 1.250 | V V V |
| Line Regulation | LT1763-1.5 ΔVIN = 2V to 20V, ILOAD = 1mA LT1763-1.8 ΔVIN = 2.3V to 20V, ILOAD = 1mA LT1763-2.5 ΔVIN = 3V to 20V, ILOAD = 1mA LT1763-3 ΔVIN = 3.5V to 20V, ILOAD = 1mA LT1763-3.3 ΔVIN = 3.8V to 20V, ILOAD = 1mA LT1763-5 ΔVIN = 5.5V to 20V, ILOAD = 1mA LT1763 (Note 3) C, I Grade: ΔVIN = 2V to 20V, ILOAD = 1mA LT1763 (Note 3) MP Grade: ΔVIN = 2.1V to 20V, ILOAD = 1mA | l l l l l l l l | 1 5 1 5 1 5 1 5 1 5 1 5 1 5 1 5 | mV mV mV mV mV mV mV mV |
| Load Regulation | LT1763-1.5 VIN = 2.5V, ΔILOAD = 1mA to 500mA VIN = 2.5V, ΔILOAD = 1mA to 500mA | l | 3 8 15 | mV mV |
|  | LT1763-1.8 VIN = 2.8V, ΔILOAD = 1mA to 500mA VIN = 2.8V, ΔILOAD = 1mA to 500mA | l | 4 9 18 | mV mV |
|  | LT1763-2.5 VIN = 3.5V, ΔILOAD = 1mA to 500mA VIN = 3.5V, ΔILOAD = 1mA to 500mA | l | 5 12 25 | mV mV |
|  | LT1763-3 VIN = 4V, ΔILOAD = 1mA to 500mA VIN = 4V, ΔILOAD = 1mA to 500mA | l | 7 15 30 | mV mV |
|  | LT1763-3.3 VIN = 4.3V, ΔILOAD = 1mA to 500mA VIN = 4.3V, ΔILOAD = 1mA to 500mA | l | 7 17 33 | mV mV |
|  | LT1763-5 VIN = 6V, ΔILOAD= 1mA to 500mA VIN = 6V, ΔILOAD = 1mA to 500mA | l | 12 25 50 | mV mV |
|  | LT1763 (Note 3) VIN = 2.3V, ΔILOAD = 1mA to 500mA C, I Grade: VIN = 2.3V, ΔILOAD = 1mA to 500mA MP Grade: VIN = 2.35V, ΔILOAD = 1mA to 500mA | l l | 2 6 12 12 | mV mV mV |
| Dropout Voltage VIN = VOUT(NOMINAL) (Notes 5, 6, 11) | ILOAD = 10mA ILOAD = 10mA | l | 0.13 0.19 0.25 | V V |
|  | ILOAD = 50mA ILOAD = 50mA | l | 0.17 0.22 0.32 | V V |
|  | ILOAD = 100mA ILOAD = 100mA | l | 0.20 0.24 0.34 | V V |
|  | ILOAD = 500mA ILOAD = 500mA | l | 0.30 0.35 0.45 | V V |

1763fe

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C.  (Note  2)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| GND Pin Current VIN = VOUT(NOMINAL) (Notes 5, 7) | ILOAD = 0mA ILOAD = 1mA ILOAD = 50mA ILOAD = 100mA ILOAD = 250mA ILOAD = 500mA | l l l l l l | 30 75 65 120 1.1 1.6 2 3 5 8 11 16 | μA μA mA mA mA mA |
| Output Voltage Noise | COUT = 10μF, CBYP = 0.01μF, ILOAD = 500mA, BW = 10Hz to 100kHz |  | 20 | μVRMS |
| ADJ Pin Bias Current | (Notes 3, 8) |  | 30 100 | nA |
| Shutdown Threshold | VOUT = Off to On VOUT = On to Off | l l | 0.8 2 0.25 0.65 | V V |
| SHDN Pin Current (Note 9) | VSHDN = 0V VSHDN = 20V |  | 0.1 1 | μA μA |
| Quiescent Current in Shutdown | VIN = 6V, VSHDN = 0V |  | 0.1 1 | μA |
| Ripple Rejection | VIN – VOUT = 1.5V (Avg), VRIPPLE = 0.5VP-P, fRIPPLE = 120Hz, ILOAD = 500mA |  | 50 65 | dB |
| Current Limit | VIN = 7V, VOUT = 0V C, I Grade: VIN = VOUT(NOMINAL) + 1V or 2.3V (Note 12), ΔVOUT = – 0.1V MP Grade: VIN = 2.35V (Note 12), ΔVOUT = – 0.1V | l l | 520 520 | mA mA |
| Input Reverse Leakage Current | VIN = –20V, VOUT = 0V | l | 1 | mA |
| Reverse Output Current (Note 10) | LT1763-1.5 VOUT = 1.5V, VIN < 1.5V LT1763-1.8 VOUT = 1.8V, VIN < 1.8V LT1763-2.5 VOUT = 2.5V, VIN < 2.5V LT1763-3 VOUT = 3V, VIN < 3V LT1763-3.3 VOUT = 3.3V, VIN < 3.3V LT1763-5 VOUT = 5V, VIN < 5V LT1763 (Note 3) VOUT = 1.22V, VIN < 1.22V |  | 10 20 10 20 10 20 10 20 10 20 10 20 5 10 | μA μA μA μA μA μA μA |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime.

Note 2: The LT1763 regulators  are tested and speciﬁed under pulse

load conditions such that TJ ≅ TA. The LT1763 (C grade)  is 100% tested at TA = 25°C; performance  at – 40°C and 125°C is assured by design, characterization and correlation with statistical process controls. The LT1763 (I grade) is guaranteed over the full – 40°C to 125°C operating junction temperature  range. The LT1763 (MP grade) is 100% tested and guaranteed over the – 55°C to 125°C operating junction temperature range.

Note 3: The LT1763 (adjustable version) is tested and speciﬁed for these conditions with the ADJ pin connected to the OUT pin.

Note 4: Operating conditions are limited by maximum junction temperature. The regulated output voltage speciﬁcation will not apply for all possible combinations  of input voltage and output current. When operating at maximum input voltage, the output current range must be limited. When operating at maximum output current, the input voltage range must be limited.

Note 5: To satisfy requirements  for minimum input voltage, the LT1763 (adjustable version) is tested and speciﬁed for these conditions with an external resistor divider (two 250k resistors) for an output voltage of

2.44V. The external resistor  divider will add a 5μA DC load on the output.

Note 6: Dropout voltage is the minimum input to output voltage differential needed to maintain regulation at a speciﬁed output current. In dropout, the output  voltage will be equal to: VIN – VDROPOUT.

Note 7: GND pin current is tested with VIN = VOUT(NOMINAL) or VIN = 2.3V (C, I grade)  or 2.35V (MP grade), whichever is greater, and a current source load. This means the device is tested while operating in its dropout region. This is the worst-case GND pin current. The GND pin current will

decrease slightly at higher input voltages.

Note 8: ADJ pin bias current ﬂows into the ADJ pin.

Note 9: SHDN pin current  ﬂows into the SHDN pin.

Note 10: Reverse output current is tested with the IN pin grounded  and the OUT pin forced  to the rated output voltage. This current ﬂows into the OUT pin and out the GND pin.

Note 11: For the LT1763, LT1763-1.5 and LT1763-1.8 dropout voltage will be limited by the minimum input voltage speciﬁcation under some output voltage/load conditions. See the curve of Minimum Input Voltage in the Typical Performance  Characteristics.

Note 12: To satisfy requirements for minimum input voltage, current limit is tested at VIN = VOUT(NOMINAL) + 1V or 2.3V (C, I grade)  or 2.35V

(MP grade), whichever is greater.

1763fe

TYPICAL PERFORMANCE CHARACTERISTICS

Typical Dropout Voltage	Guaranteed Dropout Voltage	Dropout Voltage

500

450

DROPOUT  VOLTAGE (mV)

400

350

300

250

200

150

100

500

450

400

350

300

250

200

150

100

= TEST POINTS

TJ b 125°C TJ b 25°C

500

450

DROPOUT  VOLTAGE (mV)

400

350

300

250

200

150

100

IL = 100mA

IL = 250mA

IL = 500mA

IL = 1mA

GUARANTEED DROPOUT  VOLTAGE (mV)

| 50 50 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TJ = 125°C |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TJ = 25°C |  |  |  |  |  |  |

0	0

50

IL = 50mA

0

IL = 10mA

0    50  100 150 200 250 300 350 400 450 500

0    50  100 150 200 250 300 350 400 450 500

–50

–25	0

25	50	75

100    125

OUTPUT CURRENT (mA)

OUTPUT CURRENT (mA)

TEMPERATURE (°C)

1763 G01

1763 G02

1763 G03

Quiescent  Current

| 50 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VSHDN = VIN |  |  |  |  |  |
| VIN = 6V RL = d, IL = 0 (LT1763-1.5/-1.8/-2.5/-3/-3.3/-5) RL = 250k, IL = 5μA (LT1763) |  |  |  |  |  |  |  |

45

QUIESCENT CURRENT (μA)

40

OUTPUT VOLTAGE (V)

35

30

25

20

15

10

5

0

| 1.528 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL = 1mA |  |  |  |  |  |  |  |

1.521

1.514

1.507

1.500

1.493

1.486

1.479

1.472

LT1763-1.5

Output Voltage

| 1.84 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL = 1mA |  |  |  |  |  |  |  |

1.83

OUTPUT VOLTAGE (V)

1.82

1.81

1.80

1.79

1.78

1.77

1.76

LT1763-1.8

Output Voltage

–50

–25

0	25

50	75

100

125

–50

–25

0	25	50

75	100

125

–50

–25

0	25	50

75	100

125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

1763 G04

1763 G05

1763 G06

| 2.54 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL = 1mA |  |  |  |  |  |  |  |

2.53

OUTPUT VOLTAGE (V)

2.52

2.51

2.50

2.49

2.48

2.47

LT1763-2.5

Output Voltage

| 3.060 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL = 1mA |  |  |  |  |  |  |  |

3.045

OUTPUT VOLTAGE (V)

3.030

3.015

3.000

2.985

2.970

2.955

LT1763-3

Output Voltage

| 3.360 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL = 1mA |  |  |  |  |  |  |  |

3.345

3.330

3.315

3.300

3.285

3.270

3.255

LT1763-3.3

OUTPUT VOLTAGE (V)

Output Voltage

2.46

–50

–25

0	25

50	75

100

125

2.940

–50

–25

0	25	50

75	100

125

3.240

–50

–25

0	25	50

75	100

125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

1763 G07

1763 G08

1763 G09

1763fe

OUTPUT VOLTAGE (V)

TYPICAL PERFORMANCE CHARACTERISTICS

LT1763

ADJ Pin Voltage

ADJ PIN VOLTAGE (V)

| IL = 1mA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL = 1mA |  |  |  |  |  |  |  |

250

225

QUIESCENT CURRENT (μA)

200

175

150

125

100

75

LT1763-1.5

Quiescent  Current

TJ = 25°C RL = d

–50

–25

0	25	50

| 75 100 | LT1763-5 Output Voltage |  |
| --- | --- | --- |
| 5.100 |  | 1.240 |
| 5.075 |  | 1.235 |
| 5.050 |  | 1.230 |
| 5.025 |  | 1.225 |
| 5.000 |  | 1.220 |
| 4.975 |  | 1.215 |
| 4.950 |  | 1.210 |
| 4.925 |  | 1.205 |
| 4.900 |  | 1.200 |

125

–50

–25

0	25	50

75	100

125

25

0

0     1     2     3

4     5	6     7

8     9    10

TEMPERATURE (°C)

TEMPERATURE (°C)

INPUT VOLTAGE (V)

1763 G10

1763 G11

1763 G12

250

225

QUIESCENT CURRENT (μA)

200

175

150

125

100

75

50

25

0

LT1763-1.8

Quiescent  Current

VSHDN = VIN

VSHDN = 0V

TJ = 25°C RL = d

250

225

QUIESCENT CURRENT (μA)

200

175

150

125

100

75

50

25

0

LT1763-2.5

Quiescent  Current

TJ = 25°C RL = d

VSHDN = VIN

VSHDN = 0V

250

225

QUIESCENT CURRENT (μA)

200

175

150

125

100

75

50

25

0

LT1763-3

Quiescent  Current

TJ = 25°C RL = d

VSHDN = VIN

VSHDN = 0V

0     1     2     3

4     5	6     7

8     9    10

0     1     2     3

4     5	6     7

8     9    10

0     1     2     3

4     5	6     7

8     9    10

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1763 G13

1763 G14

1763 G15

250

225

QUIESCENT CURRENT (μA)

200

175

150

125

100

75

50

0

LT1763-3.3

Quiescent  Current

TJ = 25°C RL = d

VSHDN = VIN

VSHDN = 0V

250

225

QUIESCENT CURRENT (μA)

200

175

150

125

100

75

50

0

LT1763-5

Quiescent  Current

TJ = 25°C RL = d

VSHDN = VIN

VSHDN = 0V

LT1763

Quiescent  Current

QUIESCENT CURRENT (μA)

40

TJ = 25°C

35	RL = 250k

VSHDN = VIN

30

25

20

15

10

VSHDN = 0V

0

0     1     2     3

4     5	6     7

8     9    10

0     1     2     3

4     5	6     7

8     9    10

0     2     4     6

8    10

12   14

16   18   20

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1763 G16

1763 G17

1763 G18

1763fe

TYPICAL PERFORMANCE CHARACTERISTICS

1200

LT1763-1.5

GND Pin Current

1200

LT1763-1.8

GND Pin Current

GND PIN CURRENT (μA)

1200

LT1763-2.5

GND Pin Current

1000

800

600

RL = 30Ω

IL = 50mA*

TJ = 25°C VIN = VSHDN

*FOR VOUT = 1.5V

1000

GND PIN CURRENT (μA)

800

600

RL = 36Ω

IL = 50mA*

TJ = 25°C VIN = VSHDN

*FOR VOUT = 1.8V

1000

800

600

RL = 50Ω

IL = 50mA*

TJ = 25°C VIN = VSHDN

*FOR VOUT = 2.5V

GND PIN CURRENT (μA)

400

200

0

RL = 150Ω IL = 10mA*

RL = 1.5k

IL = 1mA*

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

400

200

0

RL = 180Ω IL = 10mA*

RL = 1.8k

IL = 1mA*

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

400

200

0

RL = 250Ω IL = 10mA*

RL = 2.5k

IL = 1mA*

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

1763 G19

1763 G20

1763 G21

1200

LT1763-3

GND Pin Current

1200

LT1763-3.3

GND Pin Current

1200

LT1763-5

GND Pin Current

1000

800

600

GND PIN CURRENT (μA)

400

200

0

RL = 60Ω

IL = 50mA*

RL = 300Ω IL = 10mA*

RL = 3k

IL = 1mA*

TJ = 25°C VIN = VSHDN

*FOR VOUT = 3V

1000

800

600

GND PIN CURRENT (μA)

400

200

0

RL = 66Ω

IL = 50mA*

RL = 330Ω IL = 10mA*

RL = 3.3k

IL = 1mA*

TJ = 25°C VIN = VSHDN

GND PIN CURRENT (μA)

*FOR VOUT = 3.3V

1000

800

600

400

200

0

RL = 100Ω IL = 50mA*

TJ = 25°C VIN = VSHDN

*FOR VOUT = 5V

RL = 500Ω IL = 10mA*

RL = 5k

IL = 1mA*

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

1763 G22

1763 G23

1763 G24

1200

1000

800

600

GND PIN CURRENT (μA)

400

LT1763

GND Pin Current

RL = 24.4Ω IL = 50mA*

RL = 122Ω IL = 10mA*

TJ = 25°C VIN = VSHDN

*FOR VOUT = 1.22V

LT1763-1.5

GND Pin Current

12

10

RL = 3Ω

GND PIN CURRENT (mA)

8	IL = 500mA*

6

4

TJ = 25°C VIN = VSHDN

*FOR VOUT = 1.5V

RL = 5Ω

IL = 300mA*

RL = 15Ω

LT1763-1.8

GND Pin Current

12

10

RL = 3.6Ω

8	IL = 500mA*

6

GND PIN CURRENT (mA)

4

TJ = 25°C VIN = VSHDN

*FOR VOUT = 1.8V

RL = 6Ω

IL = 300mA*

RL = 18Ω

200

0

RL = 1.22k

IL = 1mA*

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

2	IL = 100mA*

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

2	IL = 100mA*

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

1763 G25

1763 G26

1763 G27

1763fe

TYPICAL PERFORMANCE CHARACTERISTICS

LT1763-2.5

GND Pin Current

12

10

GND PIN CURRENT (mA)

8	RL = 5Ω

TJ = 25°C VIN = VSHDN

GND PIN CURRENT (mA)

*FOR VOUT = 2.5V

LT1763-3

GND Pin Current

12

10

8	RL = 6Ω

TJ = 25°C VIN = VSHDN

*FOR VOUT = 3V

LT1763-3.3

GND Pin Current

12

10

8	RL = 6.6Ω

TJ = 25°C VIN = VSHDN

*FOR VOUT = 3.3V

IL = 500mA*

6

4

2

RL = 8.33Ω IL = 300mA*

RL = 25Ω

IL = 100mA*

IL = 500mA*

6

4

2

RL = 10Ω

IL = 300mA*

RL = 30Ω

IL = 100mA*

IL = 500mA*

6

GND PIN CURRENT (mA)

4

2

RL = 11Ω

IL = 300mA*

RL = 33Ω

IL = 100mA*

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

1763 G28

1763 G29

1763 G30

LT1763-5

GND Pin Current

12

LT1763

GND Pin Current	GND Pin Current vs ILOAD

12	12

TJ = 25°C VIN = VSHDN

10     *FOR VOUT = 5V

8

6

GND PIN CURRENT (mA)

4

2

RL = 10Ω

IL = 500mA*

RL = 16.7Ω IL = 300mA*

RL = 50Ω

IL = 100mA*

10

RL = 2.44Ω

8	IL = 500mA*

6

GND PIN CURRENT (mA)

4

2

TJ = 25°C VIN = VSHDN

*FOR VOUT = 1.22V

RL = 4.07Ω IL = 300mA*

RL = 12.2Ω IL = 100mA*

VIN = VOUT(NOMINAL) + 1V

10

8

6

GND PIN CURRENT (mA)

4

2

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

0

0     1     2     3     4	5     6     7	8     9    10

INPUT VOLTAGE (V)

0

0    50  100 150 200 250 300 350 400 450 500

OUTPUT CURRENT (mA)

1763 G31

1763 G32

1763 G33

| 1.0 | IL = 1mA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

0.9

SHDN PIN THRESHOLD (V)

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

SHDN Pin Threshold

(On-to-Off)

| 1.0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | IL = 500mA |  |  |  |  |  |  |
|  |  |  |  |  | IL = 1mA |  |  |  |  |  |

0.9

SHDN PIN THRESHOLD (V)

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

SHDN Pin Threshold

(Off-to-On)	SHDN Pin Input Current

| 1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SHDN PIN INPUT CURRENT (μA)

1.2

1.0

0.8

0.6

0.4

0.2

0

–50

–25	0

25	50	75

100    125

–50

–25	0

25	50	75

100    125

0     1     2     3     4	5     6     7	8     9    10

TEMPERATURE (°C)

TEMPERATURE (°C)

SHDN PIN VOLTAGE (V)

1763 G34

1763 G35

1763 G36

1763fe

TYPICAL PERFORMANCE CHARACTERISTICS

SHDN Pin Input Current	ADJ Pin Bias  Current	Current Limit

1.6

SHDN PIN INPUT CURRENT (μA)

1.4

1.2

1.0

0.8

0.6

0.4

0.2

0

VSHDN = 20V

140

120

ADJ PIN BIAS CURRENT (nA)

100

80

40

20

| 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.0

0.9

0.8

CURRENT LIMIT (A)

0.7

0.6

0.5

0.4

0.3

0.2

0.1

| 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VOUT = 0V |  |  |  |  |  |  |

–50

–25

0	25

50	75

100

125

–50

–25	0

25	50	75

100    125

0	1	2

3	4	5	6	7

TEMPERATURE (°C)

TEMPERATURE (°C)

INPUT VOLTAGE (V)

1763 G37

1763 G38

1763 G39

Current Limit	Reverse Output Current	Reverse Output Current

1.0

VIN = 7

REVERSE OUTPUT CURRENT (μA)

VOUT = 0V

TJ = 25°C,  VIN = 0V

90    CURRENT FLOWS

80    INTO OUTPUT PIN

LT1763-1.5

VIN = 0V, VOUT = 1.22V (LT1763)

REVERSE OUTPUT CURRENT (μA)

18   VOUT = 1.5V (LT1763-1.5)

VOUT = 1.8V (LT1763-1.8)

16   VOUT = 2.5V (LT1763-2.5)

14   VOUT = 3V (LT1763-3)

CURRENT LIMIT (A)

0.8

70

LT1763

VOUT

= 3.3V (LT1763-3.3)

0.6

0.4

0.2

0

60	LT1763-1.8

50	LT1763-2.5

40	LT1763-3

LT1763-3.3

30

20

10

0

LT1763-5

12   VOUT = 5V (LT1763-5)

10

8	LT1763-1.5/-1.8/

6	-2.5/-3/-3.3/-5

4	LT1763

2

0

–50

–25	0

25	50	75

100    125

0     1     2     3     4	5     6     7	8     9    10

–50

–25	0

25	50	75

100    125

TEMPERATURE (°C)

OUTPUT VOLTAGE (V)

TEMPERATURE (°C)

1763 G40

1763 G41

1763 G42

Input Ripple Rejection	Input Ripple Rejection	Ripple Rejection

80	80

70	70

68

CBYP = 0.01μF	66

RIPPLE REJECTION (dB)

60

50	COUT = 10μF

40

RIPPLE REJECTION (dB)

60

50   C

40

BYP

= 1000pF

64

62

RIPPLE REJECTION (dB)

CBYP = 100pF

60

30

20    IL = 500mA

VIN = VOUT(NOMINAL) +

10    1V + 50mVRMS  RIPPLE CBYP = 0

0

COUT = 4.7μF

30	58

20    IL = 500mA	56

VIN = VOUT(NOMINAL) +

10    1V + 50mVRMS  RIPPLE	54

COUT = 10μF

| 0 52 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VIN = VOUT (NOMINAL) + 1V + 0.5VP-P RIPPLE AT f = 120Hz IL = 500mA |  |  |  |  |  |  |

10	100

1k	10k

100k	1M

10	100

1k	10k

100k	1M

–50

–25

0	25	50

75	100

125

FREQUENCY (Hz)

1763 G43

FREQUENCY (Hz)

1763 G44

TEMPERATURE (°C)

1763 G45

1763fe

TYPICAL PERFORMANCE CHARACTERISTICS

2.50

LT1763

Minimum Input Voltage	Load Regulation

5

LT1763-1.5

2.25

2.00

1.75

1.50

1.25

IL = 500mA

IL = 1mA

0

–5

–10

LT1763

LT1763-3

LT1763-2.5

LT1763-1.8

1.00

0.75

MINIMUM INPUT VOLTAGE (V)

0.50

0.25

0

VOUT = 1.22V

LOAD REGULATION (mV)

–15

–20

–25

LT1763-3.3

LT1763-5

VIN = VOUT(NOMINAL) + 1V

$IL = 1mA TO 500mA

–50

–25	0

25	50	75

100    125

–50

–25

0	25

50	75

100

125

TEMPERATURE (°C)

TEMPERATURE (°C)

1763 G46

1763 G47

Output Noise  Spectral  Density

CBYP = 0	Output Noise  Spectral  Density

OUTPUT NOISE SPECTRAL DENSITY (μV/Hz)

OUTPUT NOISE SPECTRAL DENSITY (μV/Hz)

10	10

LT1763-3

COUT = 10μF

IL = 500mA

LT1763-5

1

LT1763

LT1763-3.3

LT1763-2.5

LT1763-5	CBYP = 1000pF

1		CBYP = 100pF LT1763

0.1

0.01

LT1763-1.8

COUT = 10μF IL = 500mA

LT1763-1.5

0.1

0.01

CBYP = 0.01μF

| 10 | 100 | 1k | 10k | 100k | 10 | 100 | 1k | 10k 100k |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FREQUENCY (Hz) |  |  |  |  | FREQUENCY (Hz) |  |

1763 G48

1763 G49

160

140

OUTPUT NOISE (μVRMS)

120

100

80

60

40

20

0

RMS Output Noise

vs Bypass Capacitor

COUT = 10μF IL = 500mA

f = 10Hz TO 100kHz

LT1763-5

LT1763-3.3

LT1763-3

LT1763-2.5

LT1763

LT1763-1.8

LT1763-1.5

160

140

OUTPUT NOISE (μVRMS)

120

100

80

60

40

20

0

RMS Output Noise

vs Load Current (10Hz to 100kHz)

COUT = 10μF CBYP = 0

CBYP = 0.01μF

LT1763-5

LT1763

LT1763-5

LT1763

10	100	1000	10000

0.01

0.1	1

10	100	1000

CBYP (pF)

LOAD CURRENT (mA)

1763 G50

1763 G51

1763fe

TYPICAL PERFORMANCE CHARACTERISTICS

LT1763-5

10Hz to 100kHz Output Noise

CBYP = 0

LT1763-5

10Hz to 100kHz Output Noise

CBYP = 100pF

VOUT

100μV/DIV

VOUT

100μV/DIV

COUT = 10μF IL = 500mA

1ms/DIV

1763 G52

COUT = 10μF IL = 500mA

1ms/DIV

1763 G53

VOUT

100μV/DIV

LT1763-5

10Hz to 100kHz Output Noise

CBYP = 1000pF

VOUT

100μV/DIV

LT1763-5

10Hz to 100kHz Output Noise

CBYP = 0.01μF

COUT = 10μF IL = 500mA

1ms/DIV

1763 G54

COUT = 10μF IL = 500mA

1ms/DIV

1763 G55

OUTPUT VOLTAGE DEVIATION (V)

0.4

0.2

0

–0.2

–0.4

LT1763-5

Transient Response

CBYP = 0

VIN = 6V CIN = 10μF COUT = 10μF

0.10

0.05

0

OUTPUT VOLTAGE DEVIATION (V)

–0.05

–0.10

LT1763-5

Transient Response

CBYP = 0.01μF

VIN = 6V CIN = 10μF COUT = 10μF

LOAD CURRENT (mA)

600

400

200

0

0	200	400	600	800	1000

600

400

LOAD CURRENT (mA)

200

0

0

10   20   30   40   50

60   70

80   90  100

TIME (μs)

TIME (μs)

1763 G56

1763 G57

1763fe

PIN FUNCTIONS

(DE12/S8)

NC (Pins 1, 4, 9, 12) DE12 Only: No Connect. No connect pins have no connection to any internal circuitry. These pins may be tied to either GND or VIN , or left ﬂoating.

OUT (Pins 2, 3/Pin 1): Output. The output supplies power to the load. A minimum  output capacitor of 3.3μF is re- quired to prevent oscillations. Larger output capacitors will be required for applications with large transient loads to limit peak voltage transients. See the Applications Infor- mation section for more information on output capacitance and reverse output characteristics.

ADJ (Pin 5/Pin 2): Adjust. For the adjustable LT1763, this is the input to the error ampliﬁer. This pin is internally clamped  to ±7V. It has a bias current of 30nA which ﬂows into the pin (see the curve of ADJ Pin Bias Current vs Temperature in the Typical Performance Characteristics section). The ADJ pin voltage is 1.22V referenced to ground and the output voltage range is 1.22V to 20V.

SENSE  (Pin 5/Pin 2): Output Sense. For ﬁxed volt- age versions of the LT1763  (LT1763-1.5/LT1763-1.8/ LT1763-2.5/LT1763-3/LT1763-3.3/LT1763-5), the SENSE pin is the input to the error ampliﬁer. Optimum regulation will be obtained at the point where the SENSE pin is connected to the OUT  pin of the regulator. In critical applications,  small voltage drops are caused by the resistance (RP) of PC traces between the regulator and the load. These may be eliminated by connecting the SENSE pin to the output at the load as shown in Figure 1 (Kelvin Sense Connection).

supply system where the regulator load is returned to a negative supply) and still allow the device to start and operate.

BYP (Pin 6/Pin 4): Bypass. The BYP pin is used  to bypass the reference of the LT1763 regulators to achieve low noise performance from the regulator. The BYP pin is clamped internally to ±0.6V (one  VBE). A small capacitor from the output to this pin will bypass the reference to lower the output voltage noise. A maximum   value  of 0.01μF can be used for reducing output voltage noise to a typical

20μVRMS over a 10Hz to 100kHz bandwidth.  If not used, this pin must be left unconnected.

GND (Pins 7/Pins 3, 6, 7): Ground.

SHDN (Pin 8/Pin 5): Shutdown.  The SHDN pin is used to put the LT1763 regulators into a low power shutdown state. The output  will be off when the SHDN pin is pulled low. The SHDN pin can be driven either by 5V logic or open-collector  logic with a pull-up resistor. The pull-up resistor is required to supply the pull-up current of the open-collector gate, normally several microamperes,  and the SHDN pin current,  typically 1μA. If unused, the SHDN pin must be connected to VIN. The device  will be in the low power shutdown state if the SHDN pin is not connected.

IN (Pin 10, 11/Pin 8): Input. Power is supplied to the device through the IN pin. A bypass capacitor is required on this pin if the device is more than six inches away from the main input ﬁlter capacitor. In general, the output impedance of a battery rises with frequency, so it is advisable to

8

IN

LT1763

1     RP

OUT

include a bypass capacitor in battery-powered circuits. A bypass capacitor in the range of 1μF to 10μF is sufﬁcient. The LT1763 regulators  are designed  to withstand reverse

+

VIN

5

SHDN

2	+

SENSE

GND

3

RP

LOAD

1763 F01

voltages on the IN pin with respect to ground and the OUT pin. In the case of a reverse input, which can happen  if a battery is plugged in backwards, the device will act as if there is a diode in series with its input. There will be no reverse current ﬂow into the regulator and no reverse

Figure 1. Kelvin Sense Connection

Note that the voltage drop across the external PC traces will add to the dropout voltage of the regulator. The SENSE pin bias current is 10μA at the nominal rated output voltage. The SENSE pin can be pulled below ground (as in a dual

voltage  will appear at the load. The device will protect both itself and the load.

Exposed Pad (Pin 13) DE12 Only: Ground. The Exposed Pad must be soldered to the PCB ground for rated thermal performance.

1763fe

APPLICATIONS INFORMATION

The LT1763 series are 500mA low dropout regulators with micropower quiescent current and shutdown. The devices are capable of supplying 500mA at a dropout voltage of

300mV. Output voltage noise can be lowered to 20μVRMS over a 10Hz to 100kHz bandwidth with the addition of a 0.01μF  reference bypass capacitor. Additionally,  the reference bypass capacitor will improve transient response

to 1.22V: VOUT/1.22V. For example, load regulation for an output current change of 1mA to 500mA is –2mV typical at VOUT = 1.22V. At VOUT = 12V, load regulation is:

(12V/1.22V)(–2mV)  = –19.6mV

of the regulator, lowering the settling time for transient load conditions. The low operating quiescent current (30μA)

VIN

IN

LT1763

OUT

VOUT

+

R2

1.22  ⎛1

R2

⎟ + ( 	)(  2)

⎞

drops to less than 1μA in shutdown. In addition to the

ADJ

VOUT =

V   +

⎝	R1⎠

⎜

IADJ  R

low quiescent  current, the LT1763 regulators incorporate several protection features which make them ideal for use in battery-powered systems. The devices are protected against both reverse input and reverse output voltages.

GND

R1

1763 F02

VADJ = 1.22V

IADJ = 30nA AT 25°C

OUTPUT RANGE  = 1.22V TO 20V

In battery backup applications where the output can be held up by a backup battery when the input is pulled to ground, the LT1763-X acts like it has a diode in series  with its output and prevents reverse current ﬂow. Additionally, in dual supply applications where the regulator load is returned to a negative supply, the output can be pulled below ground by as much as 20V and still allow the device to start and operate.

Adjustable Operation

The adjustable version of the LT1763 has an output voltage range of 1.22V to 20V. The output voltage is set by the ratio of two external resistors, as shown in Figure 2. The device servos the output to maintain the ADJ pin voltage at 1.22V referenced to ground. The current in R1 is then equal to 1.22V/R1 and the current in R2 is the current in R1 plus the ADJ pin bias current. The ADJ pin bias current, 30nA at 25°C, ﬂows through R2 into the ADJ pin. The output voltage can be calculated using the formula in Figure 2. The value of R1 should be no greater than 250k to minimize errors in the output voltage caused by the ADJ pin bias current. Note that in shutdown the output is turned off and the divider current  will be zero. Curves of ADJ Pin Voltage vs Temperature and ADJ Pin Bias Current vs Temperature appear in the Typical Performance Characteristics section.

The adjustable device is tested and speciﬁed with the ADJ pin tied to the OUT pin for an output voltage of 1.22V. Speciﬁcations for output voltages greater than 1.22V will be proportional to the ratio of the desired output voltage

Figure 2. Adjustable Operation

Bypass Capacitance and Low Noise Performance

The LT1763 regulators  may be used with the addition of a bypass capacitor from VOUT  to the BYP pin to lower output voltage noise. A good quality low leakage capacitor is recommended.  This capacitor will bypass the reference of the regulator, providing a low frequency noise pole. The noise pole provided by this bypass capacitor  will lower the output voltage noise to as low as 20μVRMS with the addition of a 0.01μF bypass capacitor. Using a bypass capacitor has the added beneﬁt of improving transient response.  With no bypass capacitor and a 10μF output capacitor, a 10mA to 500mA load step will settle to within 1% of its ﬁnal value in less than 100μs. With the addition of a 0.01μF bypass capacitor, the output will settle to within 1% for a 10mA to 500mA load step in less than 10μs, with total output voltage deviation of less than

2.5% (see the LT1763-5 Transient Response  curve in the Typical Performance  Characteristics  section). However, regulator start-up time is proportional to the size of the bypass capacitor, slowing to 15ms with a 0.01μF bypass capacitor and 10μF output capacitor.

1763fe

APPLICATIONS INFORMATION

Output Capacitance and Transient Response

The LT1763 regulators  are designed to be stable with a wide range of output capacitors.  The ESR of the output capacitor affects stability, most notably with small capaci- tors. A minimum  output capacitor of 3.3μF with an ESR of 3Ω, or less, is recommended to prevent oscillations.

4.0

3.5

3.0

ESR (Ω)

2.5

2.0

1.5

CBYP = 0

STABLE REGION

The LT1763-X is a micropower device and output transient response will be a function of output capacitance. Larger values of output capacitance decrease the peak deviations

1.0

0.5

0

CBYP = 100pF

CBYP = 330pF

CBYP r 1000pF

and provide improved transient response for larger load

1	2	3

4     5   6   7  8  9 10

current changes. Bypass capacitors, used to decouple

CHANGE IN VALUE (%)

individual components powered by the LT1763-X,  will increase the effective output capacitor value. With larger capacitors used to bypass the reference (for low noise operation), larger values of output capacitors are needed. For 100pF of bypass capacitance, 4.7μF of output capaci- tor is recommended.  With a 1000pF bypass capacitor or larger, a 6.8μF output capacitor is recommended.

The shaded region of Figure 3 deﬁnes the range over which the LT1763 regulators are stable. The minimum ESR needed is deﬁned by the amount of bypass capacitance used, while the maximum ESR is 3Ω.

Extra consideration  must be given to the use of ceramic

20

0

–20

–40

–60

–80

OUTPUT CAPACITANCE (μF)

Figure 3. Stability

1763 F03

capacitors. Ceramic capacitors are manufactured  with a

–100

| 0 2 4 |  |  | BOTH CAPACITORS ARE 16V, 1210 CASE SIZE, 10μF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | X5R |  |  |  |  |
|  |  |  |  |  | Y5V |  |  |  |  |

6	8	10

12     14	16

variety of dielectrics, each with different behavior across

temperature and applied voltage. The most common

DC BIAS VOLTAGE (V)

1763 F04

dielectrics used are speciﬁed with EIA  temperature characteristic codes of Z5U, Y5V, X5R and X7R. The Z5U and Y5V dielectrics are good for providing high capacitances in a small package, but they tend to have strong voltage and temperature coefﬁcients,  as shown in Figures 4

Figure 4. Ceramic  Capacitor  DC Bias  Characteristics

40

20

and 5. When used with a 5V regulator,  a 16V 10μF Y5V

capacitor can exhibit an effective value as low as 1μF to

2μF for the DC bias voltage  applied  and over the operating temperature range. The X5R and X7R dielectrics  result in more stable characteristics and are more suitable for use

across temperature, while the X5R is less expensive and is available in higher values. Care still must be exercised when

0

CHANGE IN VALUE (%)

–20

–40

–60

–80

–100

Y5V

BOTH CAPACITORS ARE 16V,

1210 CASE SIZE, 10μF

X5R

–50

–25	0

25	50

75	100     125

using X5R and X7R capacitors;  the X5R and X7R codes

only specify operating temperature range and maximum

TEMPERATURE (°C)

1763 F05

capacitance change over temperature. Capacitance change due to DC bias with X5R and X7R capacitors is better than

Figure 5. Ceramic Capacitor Temperature Characteristics

1763fe

APPLICATIONS INFORMATION

Y5V and Z5U capacitors, but can still be signiﬁcant enough to drop capacitor values below appropriate levels. Capacitor DC bias characteristics tend to improve as component case size increases, but expected capacitance at operating voltage should be veriﬁed.

Voltage  and temperature coefﬁcients are not the only sources of problems. Some ceramic capacitors have a piezoelectric response. A piezoelectric  device  generates voltage across its terminals due to mechanical stress, similar to the way a piezoelectric  accelerometer or microphone works. For a ceramic capacitor, the stress can be induced by vibrations in the system or thermal transients. The resulting voltages produced can cause appreciable amounts of noise, especially when a ceramic capacitor is used for noise bypassing. A ceramic capacitor produced Figure 6’s trace in response to light tapping from a pencil. Similar vibration induced behavior can masquerade as increased output voltage noise.

LT1763-5

COUT = 10μF CBYP = 0.01μF ILOAD = 100mA

VOUT

500μV/DIV

1763 F06

100ms/DIV

Figure 6. Noise Resulting from

Tapping on a Ceramic Capacitor

Thermal Considerations

The power handling capability of the device will be limited by the maximum rated junction temperature (125°C). The power dissipated by the device  will be made up of two components:

1. Output current multiplied by the input/output voltage differential:  (IOUT)(VIN – VOUT), and

2. GND  pin current multiplied  by the input voltage: (IGND)(VIN).

The GND pin current can be found by examining the GND Pin Current curves  in the Typical Performance Character- istics section. Power dissipation will be equal to the sum of the two components listed above.

The LT1763 series regulators have internal thermal limiting designed to protect the device during overload conditions. For continuous normal conditions, the maximum junction temperature rating of 125°C must not be exceeded. It is important to give careful consideration to all sources of thermal resistance from junction-to-ambient. Additional heat sources mounted nearby must also be considered.

For surface mount devices, heat sinking is accomplished by using the heat spreading capabilities of the PC board and its copper traces. Copper board stiffeners and plated through-holes can also be used to spread the heat gener- ated by power devices.

The following tables list thermal resistance for several different board sizes and copper areas. All measurements were taken in still air on 3/32" FR-4 board with one ounce copper.

Table 1. DE Package,  12-Lead DFN

| COPPER AREA |  | BOARD AREA | THERMAL RESISTANCE (JUNCTION-TO-AMBIENT) |
| --- | --- | --- | --- |
| TOPSIDE* | BACKSIDE |  |  |
| 2500mm2 | 2500mm2 | 2500mm2 | 40°C/W |
| 1000mm2 | 2500mm2 | 2500mm2 | 45°C/W |
| 225mm2 | 2500mm2 | 2500mm2 | 50°C/W |
| 100mm2 | 2500mm2 | 2500mm2 | 60°C/W |

* Device is mounted on topside

1763fe

APPLICATIONS INFORMATION

Table 2. SO-8 Package,  8-Lead SO

| COPPER AREA |  | BOARD AREA | THERMAL RESISTANCE (JUNCTION-TO-AMBIENT) |
| --- | --- | --- | --- |
| TOPSIDE* | BACKSIDE |  |  |
| 2500mm2 | 2500mm2 | 2500mm2 | 60°C/W |
| 1000mm2 | 2500mm2 | 2500mm2 | 60°C/W |
| 225mm2 | 2500mm2 | 2500mm2 | 68°C/W |
| 100mm2 | 2500mm2 | 2500mm2 | 74°C/W |
| 50mm2 | 2500mm2 | 2500mm2 | 86°C/W |

* Device is mounted on topside

Calculating Junction Temperature

Example: Given an output voltage of 3.3V, an input voltage range of 4V to 6V, an output current range of 0mA to 250mA and a maximum ambient temperature of 50°C, what will the maximum junction temperature be?

The power dissipated by the device will be equal to: IOUT(MAX)(VIN(MAX) – VOUT) + IGND(VIN(MAX))

where,

IOUT(MAX) = 250mA VIN(MAX) = 6V

IGND at (IOUT = 250mA,  VIN = 6V) = 5mA

So,

P = 250mA(6V – 3.3V) + 5mA(6V) = 0.71W

The thermal resistance will be in the range of 60°C/W to

86°C/W, depending on the copper area. So, the junction temperature rise above ambient will  be approximately equal to:

0.71W(75°C/W)  = 53.3°C

The maximum junction temperature will then be equal to the maximum junction temperature rise above ambient plus the maximum ambient temperature, or :

TJMAX = 50°C + 53.3°C = 103.3°C

Protection Features

The LT1763  regulators incorporate several protection features which make them ideal for use in battery-powered circuits. In addition to the normal protection features associated with monolithic regulators, such as current limiting and thermal limiting, the devices are protected against reverse input voltages, reverse output voltages and reverse voltages from output to input.

Current limit protection and thermal overload protection are intended to protect the device against current overload conditions at the output of the device. For normal operation, the junction temperature should not exceed 125°C.

The input of the device will withstand reverse voltages of

20V. Current ﬂow into the device  will be limited to less than 1mA (typically less than 100μA) and no negative voltage will appear  at the output.  The device will protect both itself and the load. This provides protection against batteries which can be plugged in backward.

The output of the LT1763-X can be pulled below ground without damaging the device. If the input is left open-circuit or grounded, the output can be pulled below ground by

20V. For ﬁxed voltage versions, the output  will act like a large resistor, typically 500k or higher, limiting current ﬂow to less than 100μA. For adjustable versions,  the output will act like an open circuit; no current will ﬂow out of the pin. If the input is powered by a voltage source, the output will source the short-circuit current of the device and will protect itself by thermal limiting. In this case, grounding the SHDN pin will turn off the device and stop the output from sourcing the short-circuit current.

The ADJ pin of the adjustable device can be pulled above or below ground by as much as 7V without damaging  the device.  If the input is left open-circuit  or grounded, the ADJ pin will act like an open circuit when pulled below ground and like a large resistor (typically 100k) in series with a diode when pulled above ground.

In situations where the ADJ pin is connected to a resistor divider that would pull the ADJ pin above its 7V clamp voltage if the output is pulled high, the ADJ pin input current must be limited to less than 5mA. For example, a resistor divider is used to provide a regulated 1.5V output from the

1.22V reference when the output is forced to 20V.

1763fe

APPLICATIONS INFORMATION

The top resistor of the resistor divider must be chosen to limit the current into the ADJ pin to less than 5mA when the ADJ pin is at 7V. The 13V difference between output and ADJ pin divided by the 5mA maximum current into the ADJ pin yields a minimum top resistor value of 2.6k.

In circuits where a backup battery is required, several different input/output conditions can occur. The output voltage may be held up while the input is either pulled to ground, pulled to some intermediate voltage or is left open-circuit. Current ﬂow back into the output will follow the curve shown in Figure 7.

When the IN pin of the LT1763-X is forced below the OUT pin, or the OUT pin is pulled above the IN pin, input cur- rent will typically drop to less than 2μA. This can happen if the input of the device is connected to a discharged (low voltage) battery and the output is held up by either a backup battery or a second regulator circuit. The state of the SHDN pin will have no effect on the reverse output current when the output is pulled above the input.

TJ = 25°C

REVERSE OUTPUT CURRENT (μA)

90  VIN = 0V

CURRENT FLOWS

80  INTO OUTPUT PIN

70  VOUT = VADJ (LT1763)

LT1763-1.5

60

LT1763-1.8

50	LT1763-2.5

40	LT1763-3

30

20

10

LT1763

LT1763-5

LT1763-3.3

0     1     2     3     4	5     6     7	8     9    10

OUTPUT VOLTAGE (V)

1763 F07

Figure 7. Reverse Output Current

1763fe

PACKAGE DESCRIPTION

DE/UE Package

12-Lead  Plastic  DFN (4mm × 3mm)

(Reference LTC DWG # 05-08-1695 Rev D)

3.60 p0.05

3.30 p0.05

0.70 p0.05

4.00 p0.10

(2 SIDES)

3.00 p0.10

R = 0.05

TYP

R = 0.115

7	TYP

3.30 p0.10

0.40 p 0.10

12

2.20 p0.05

1.70 p 0.05

0.25 p 0.05

2.50 REF

0.50 BSC

PACKAGE OUTLINE

PIN 1

TOP MARK (NOTE 6)

0.200 REF

(2 SIDES)

0.75 p0.05

1.70 p 0.10

6

0.25 p 0.05

2.50 REF

1

0.50 BSC

PIN 1 NOTCH R = 0.20 OR

0.35 s 45o

CHAMFER

(UE12/DE12)  DFN 0806 REV D

RECOMMENDED SOLDER PAD PITCH AND DIMENSIONS APPLY SOLDER MASK TO AREAS THAT ARE NOT SOLDERED

0.00 – 0.05

BOTTOM VIEW—EXPOSED PAD

NOTE:

1. DRAWING PROPOSED TO BE A VARIATION OF VERSION (WGED) IN JEDEC PACKAGE OUTLINE M0-229

2. DRAWING NOT TO SCALE

3. ALL DIMENSIONS  ARE IN MILLIMETERS

4. DIMENSIONS  OF EXPOSED PAD ON BOTTOM OF PACKAGE DO NOT INCLUDE

MOLD FLASH. MOLD FLASH, IF PRESENT,  SHALL NOT EXCEED 0.15mm ON ANY SIDE

5. EXPOSED PAD SHALL BE SOLDER PLATED

6. SHADED AREA IS ONLY A REFERENCE FOR PIN 1 LOCATION ON THE TOP AND BOTTOM OF PACKAGE

S8 Package

8-Lead Plastic  Small Outline (Narrow .150 Inch)

(Reference LTC DWG # 05-08-1610)

.245

.050 BSC

.045 ±.005

.189 – .197 (4.801 – 5.004) NOTE 3

8	7	6	5

.150 – .157

.053 – .069 	 (1.346 – 1.752)

.004 – .010 (0.101 – 0.254)

MIN	.160 ±.005

.228 – .244 	 (5.791 – 6.197)

(3.810 – 3.988) NOTE 3

.014 – .019 	 (0.355 – 0.483) TYP

.050   (1.270) BSC

.030 ±.005

TYP

RECOMMENDED SOLDER PAD LAYOUT

1	2	3	4

.010 – .020    s 45° (0.254 – 0.508)

.008 – .010 (0.203 – 0.254)

0°– 8° TYP

NOTE:

1. DIMENSIONS IN

INCHES (MILLIMETERS)

3. THESE DIMENSIONS  DO NOT INCLUDE MOLD FLASH OR PROTRUSIONS. MOLD FLASH OR PROTRUSIONS  SHALL NOT EXCEED .006"  (0.15mm)

.016 – .050

2. DRAWING NOT TO SCALE

(0.406 – 1.270)

SO8 0303

Information furnished by Linear Technology Corporation  is believed to be accurate and reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no representa- tion that the interconnection of its circuits as described herein will not infringe on existing patent rights.

1763fe

19

TYPICAL APPLICATION

Paralleling of Regulators for Higher Output Current

VIN > 3.8V

+	C1

10μF

R1

0.1Ω

IN		OUT SENSE

LT1763-3.3

C4

0.01μF

+

C2

10μF

3.3V

1A

SHDN

R2

0.1Ω

R3

2.2k

R4

2.2k

SHDN

GND

IN

LT1763

SHDN

GND

BYP

OUT BYP

ADJ

C5

0.01μF

R6

2k

R7

1.21k

3   +	8

R5

1	10k

1/2 LT1490

2

–	C3

4    0.01μF

1763 TA03

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT1120 | 125mA Low Dropout Regulator with 20μA IQ | Includes 2.5V Reference and Comparator |
| LT1121 | 150mA Micropower Low Dropout Regulator | 30μA IQ, SOT-223 Package |
| LT1129 | 700mA Micropower Low Dropout Regulator | 50μA Quiescent Current |
| LT1175 | 500mA Negative Low Dropout Micropower Regulator | 45μA IQ, 0.26V Dropout Voltage, SOT-223 Package |
| LT1521 | 300mA Low Dropout Micropower Regulator with Shutdown | 15μA IQ, Reverse Battery Protection |
| LT1529 | 3A Low Dropout Regulator with 50μA IQ | 500mV Dropout Voltage |
| LT1613 | 1.4MHz Single-Cell Micropower DC/DC Converter | SOT-23 Package, Internally Compensated |
| LT1761 Series | 100mA, Low Noise, Low Dropout Micropower Regulators in SOT-23 | 20μA Quiescent Current, 20μVRMS Noise, ThinSOT™ |
| LT1762 Series | 150mA, Low Noise, LDO Micropower Regulators | 25μA Quiescent Current, 20μVRMS Noise, MS8 |
| LT1764A | 3A, Fast Transient Response Low Dropout Regulator | 340mV Dropout Voltage, DD, TO220 |
| LT1962 | 300mA, Fast Transient Response Low Dropout Regulator | 270mV Dropout Voltage, 20μVRML, MS8 |
| LT1963A | 1.5A, Fast Transient Response Low Dropout Regulator | 340mV Dropout Voltage, 40μVRML, DD, TO220, S8, SOT-223 |
| LT3010 | 50mA, 80V Low Noise, LDO Micropower Regulator | 300mV Dropout Voltage, MS8E |
| LT3021 | 500mA, Low Voltage, Very Low Dropout Linear Regulator | 160mV Dropout Voltage, DFN-8 and SOIC-8 Packages |

ThinSOT is a trademark of Linear Technology Corporation.

Linear Technology Corporation

20

1630 McCarthy Blvd., Milpitas, CA  95035-7417

1763fe

LT 1008 REV E • PRINTED IN USA

(408) 432-1900

● FAX: (408) 434-0507

●  www.linear.com	© LINEAR TECHNOLOGY CORPORATION 1999