Dual Step-Down

1.5A	LED Driver

FEATURES 	DESCRIPTIO

■   True Color PWMTM Delivers  Constant Color with

3000:1 Dimming Range

■   Wide Input Range: 4V to 36V Operating,  40V Maximum

■   Accurate and Adjustable Control of LED Current from 50mA to 1.5A

■   High Side  Current Sense Allows Grounded Cathode

LED Operation

■   Open  LED (LT3475) and Short Circuit Protection

■   LT3475-1 Drives LED Strings Up to 25V

■   Accurate and Adjustable 200kHz to 2MHz

Switching Frequency

■   Anti-Phase Switching Reduces Ripple

■   Uses Small Inductors and Ceramic Capacitors

■   Available in the Compact 20-Lead TSSOP Thermally

Enhanced Surface Mount Package

APPLICATIO 	S

■   Automotive and Avionic Lighting

■   Architectural Detail Lighting

■   Display Backlighting

■   Constant-Current Sources

The LT®3475/LT3475-1  are dual step-down DC/DC converters designed to operate as a constant-current source. An internal  sense resistor monitors the output current allowing  accurate current regulation  ideal for driving high current LEDs. The high side current sense al- lows grounded cathode LED operation. High output current accuracy is maintained over a wide current range, from

50mA to 1.5A, allowing a wide dimming range. Unique PWM circuitry allows a dimming range of 3000:1, avoid- ing the color shift normally associated  with LED current dimming.

The high switching frequency offers several advantages, permitting the use of small inductors and ceramic capaci- tors. Small inductors combined with the 20 lead TSSOP surface mount package save space and cost versus alternative solutions.  The constant switching frequency combined with low-impedance ceramic capacitors result in low, predictable output ripple.

With its wide input range of 4V to 36V, the LT3475/LT3475-1 regulate a broad array of power sources. A current mode PWM architecture  provides fast transient response and

cycle-by-cycle current limiting. Frequency foldback and

, LT, LTC and LTM are registered trademarks of Linear Technology Corporation.

All other trademarks are the property of their respective owners. Patents Pending.

thermal shutdown provide additional protection.

TYPICAL APPLICATIO

Dual Step-Down 1.5A LED Driver

VIN

5V TO 36V

Efﬁciency

4.7μF

VIN

SHDN	95

10μH

0.22μF

BOOST1	BOOST2

LT3475

SW1	SW2

0.22μF

90

10μH	85

EFFICIENCY (%)

80

2.2μF

*DIMMING CONTROL

0.1μF

1.5A LED CURRENT

OUT1	OUT2

LED1	LED2

PWM1	PWM2

VC1 	VC2

REF	RT

VADJ1	VADJ2

GND

DIMMING* CONTROL

0.1μF

1.5A LED

24.3k	CURRENT

3475 TA01

2.2μF

75

70

65

60

| 55 VIN = 12V |  |  | TWO SERIES CONNECTED WHITE 1.5A LEDS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SINGLE WHITE 1.5A LED |  |  |  |  |

0	0.5	1

LED CURRENT (A)

1.5

*SEE APPLICATIONS SECTION FOR DETAILS

fSW = 600kHz

3475 TA01b

3475fb

ABSOLUTE 	AXI	U 	RATI	GS

(Note 1)

PIN CONFIGURATION

TOP VIEW

VIN Pin .........................................................(-0.3V), 40V

BOOST Pin Voltage ...................................................60V

BOOST Above SW Pin...............................................30V OUT, LED, Pins  (LT3475) ...........................................15V OUT, LED Pins  (LT3475-1).........................................25V PWM Pin ...................................................................15V VADJ Pin ......................................................................6V

VC, RT, REF Pins ..........................................................3V

SHDN Pin ................................................................... VIN

Maximum Junction Temperature (Note 2)............. 125°C

OUT1   1

LED1   2

BOOST1  3

SW1   4

VIN    5

VIN    6

SW2   7

BOOST2  8

LED2   9

OUT2  10

21

FE PACKAGE

20  PWM1

19  VADJ1

18  VC1

17  REF

16  SHDN

15  GND

14  RT

13  VC2

12  VADJ2

11  PWM2

Operating Temperature Range (Note 3)

LT3475E/LT3475E-1 ............................. –40°C to 85°C LT3475I/LT3475I-1............................. –40°C to 125°C Storage Temperature Range................... –65°C to 150°C Lead Temperature Range (Soldering, 10 sec) ....... 300°C

20-LEAD PLASTIC TSSOP

TJMAX = 125°C, θJA = 30°C/W, θJC = 8°C/W EXPOSED PAD (PIN 21) IS GROUND AND MUST BE ELECTRICALLY CONNECTED TO THE PCB.

ORDER INFORMATION

| LEAD FREE FINISH | TAPE AND REEL | PART MARKING* | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- |
| LT3475EFE#PBF | LT3475EFE#TRPBF | LT3475EFE | 20-Lead Plastic TSSOP | –40°C to 85°C |
| LT3475IFE#PBF | LT3475IFE#TRPBF | LT3475IFE | 20-Lead Plastic TSSOP | –40°C to 125°C |
| LT3475EFE-1#PBF | LT3475EFE-1#TRPBF | LT3475FE-1 | 20-Lead Plastic TSSOP | –40°C to 85°C |
| LT3475IFE-1#PBF | LT3475IFE-1#TRPBF | LT3475FE-1 | 20-Lead Plastic TSSOP | –40°C to 125°C |

Consult LTC Marketing for parts speciﬁed with wider operating temperature ranges. *The temperature grade is identiﬁed by a label on the shipping container. Consult LTC Marketing for information on non-standard lead based ﬁnish parts.

For more information on lead free part marking, go to: http://www.linear.com/leadfree/

For more information on tape and reel speciﬁcations, go to: http://www.linear.com/tapeandreel/

ELECTRICAL CHARACTERISTICS

The ● denotes the speciﬁcations  which apply over the full operating

temperature range, otherwise speciﬁcations  are at TA = 25°C.  VIN = 12V, VBOOST = 16V, VOUT = 4V unless otherwise noted (Note 3)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Minimum Input Voltage |  | ● | 3.7 4 | V |
| Input Quiescent Current | Not Switching |  | 6 8 | mA |
| Shutdown Current | SHDN = 0.3V, VBOOST = VOUT = 0V |  | 0.01 2 | μA |

3475fb

ELECTRICAL CHARACTERISTICS

The ● denotes the speciﬁcations  which apply over the full operating

temperature range, otherwise speciﬁcations  are at TA = 25°C.  VIN = 12V, VBOOST = 16V, VOUT = 4V unless otherwise noted (Note 3)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| LED Pin Current | VADJ Tied to VREF • 2/3 VADJ Tied to VREF • 7/30 LT3475E/LT3475E-1 0°C to 85°C | ● ● | 0.97 1.00 1.03 0.94 1.04 0.336 0.350 0.364 0.325 0.375 0.31 0.385 | A A A A A |
| REF Voltage |  | ● | 1.22 1.25 1.27 | V |
| Reference Voltage Line Regulation | 4V < VIN < 40V |  | 0.05 | %/V |
| Reference Voltage Load Regulation | 0 < IREF < 500μA |  | 0.0002 | %/μA |
| VADJ Pin Bias Current (Note 4) |  | ● | 40 400 | nA |
| Switching Frequency | RT = 24.3k | ● | 530 600 640 | kHz |
| Maximum Duty Cycle | RT = 24.3k RT = 4.32k RT = 100k | ● | 90 95 80 98 | % % % |
| Switching Phase | RT = 24.3k |  | 150 180 210 | Deg |
| Foldback Frequency | RT = 24.3k, VOUT = 0V |  | 80 | kHz |
| SHDN Threshold (to Switch) |  |  | 2.5 2.6 2.74 | V |
| SHDN Pin Current (Note 5) | VSHDN = 2.6V |  | 7 9 11 | μA |
| PWM Threshold |  |  | 0.3 0.8 1.2 | V |
| VC Switching Threshold |  |  | 0.8 | V |
| VC Source Current | VC = 1V |  | 50 | μA |
| VC Sink Current | VC = 1V |  | 50 | μA |
| LED to VC Transresistance |  |  | 500 | V/A |
| LED to VC Current Gain |  |  | 1 | mA/μA |
| VC to Switch Current Gain |  |  | 2.6 | A/V |
| VC Clamp Voltage |  |  | 1.8 | V |
| VC Pin Current in PWM Mode | VC = 1V, VPWM = 0.3V | ● | 10 400 | nA |
| OUT Pin Clamp Voltage (LT3475) |  |  | 13.5 14 14.5 | V |
| OUT Pin Current in PWM Mode | VOUT = 4V, VPWM = 0.3V | ● | 25 50 | μA |
| Switch Current Limit (Note 6) |  |  | 2.3 2.7 3.2 | A |
| Switch VCESAT | ISW =1.5A |  | 350 500 | mV |
| BOOST Pin Current | ISW =1.5A |  | 25 40 | mA |
| Switch Leakage Current |  |  | 0.1 10 | μA |
| Minimum Boost Voltage Above SW |  |  | 1.8 2.5 | V |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime.

Note 2: This IC includes overtemperature protection that is intended

to protect the device during momentary overload conditions. Junction temperature will exceed 125°C when overtemperature protection is active. Continuous operation above the speciﬁed maximum operating junction temperature may impair device reliability.

Note 3: The LT3475E and LT3475E-1 are guaranteed to meet performance speciﬁcations from 0°C to 85°C. Speciﬁcations over the –40°C to 85°C operating temperature range are assured by design, characterization and correlation with statistical process controls.  The LT3475I and LT3475I-1 are guaranteed to meet performance speciﬁcations over the –40°C to

125°C operating temperature range. Note 4: Current ﬂows out of pin. Note 5: Current ﬂows into pin.

Note 6: Current limit is guaranteed by design and/or correlation to static test. Slope compensation  reduces current limit at higher duty cycles.

3475fb

| TYPICAL PERFOR LED Current vs VADJ |  | A | CE CHARACTERISTICS LED Current vs Temperature |  |  | Switch On Voltage |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.50 |  |  | 1.2 |  | 600 |  |  |  |  |  |
| 1.25 |  |  | 1.0 |  | 500 |  |  |  |  |  |
| 1.00 |  |  | 0.8 |  | 400 |  |  |  |  |  |
| 0.75 |  |  | 0.6 |  | 300 |  |  |  |  |  |
| 0.50 |  |  | 0.4 |  | 200 |  |  |  |  |  |
| 0.25 |  |  | 0.2 |  | 100 |  |  |  |  |  |
| 0 |  |  | 0 |  | 0 |  |  |  |  |  |
| 0 0.25 0.5 0.75 | 1 |  | 1.25 –50 –25 0 25 50 75 100 | 125 |  | 0 | 0.5 | 1.0 | 1.5 | 2.0 |

LED CURRENT (A)

VADJ (V)

TEMPERATURE (˚C)

SWITCH CURRENT (A)

LED CURRENT (A)

| 3475 G01 TA = 25°C |  |  |  |  |
| --- | --- | --- | --- | --- |

|  |  | VADJ = VREF • 2/3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VADJ = VREF • 7/30 |  |  |  |  |

3475 G02

3475 G03

3.0

2.5

Switch Current Limit vs Duty Cycle

| 3.5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

3.0

Switch Current Limit vs

Temperature

3.0

2.5

SWITCH ON VOLTAGE (mV)

| Current Limit vs Output Voltage TA = 25°C |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

2.0

1.5

CURRENT LIMIT (A)

1.0

0.5

CURRENT LIMIT (A)

2.5

2.0

1.5

1.0

0.5

CURRENT LIMIT (A)

2.0

1.5

1.0

0.5

0

| 0 20 40 60 80 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | TYPICAL |  |  |
|  | MINIMUM |  |  |  |
|  |  |  | TA = 25°C |  |

100

0

–50

–25	0

25	50

75	100     125

0

| 0 0.5 TA = 25°C |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

1.0    1.5

2.0

2.5    3.0    3.5

4.0

DUTY CYCLE (%)

TEMPERATURE (°C)

VOUT (V)

3475 G04

3475 G05

3475 G06

Oscillator Frequency

vs Temperature	Oscillator Frequency Foldback	Oscillator Frequency vs RT

TA = 25°C

OSCILLATOR FREQUENCY (kHz)

650

600

550

500

450

600

500

400

300

| 200 TA = 25°C RT = 24.3kΩ |  |  |  |  |
| --- | --- | --- | --- | --- |

OSCILLATOR FREQUENCY (kHz)

100

OSCILLATOR FREQUENCY (kHz)

1000

400

–50

–25	0

25	50

| 75 100 125 RT = 24.3kΩ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0

0	0.5	1.0	1.5

2.0

10

2.5	1

10	100

TEMPERATURE (˚C)

3475 G07

VOUT (V)

3475 G08

RT (kΩ)

3475 G09

3475fb

TYPICAL PERFOR	A	CE CHARACTERISTICS

Boost Pin Current	Quiescent  Current

Open-Circuit Output Voltage and

Input Current

| 35 7 TA = 25°C |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

30	6

25	5

INPUT CURRENT (mA)

20	4

15	3

50

TA = 25°C

45

40

35

30

25

20

INPUT CURRENT

LT3475-1

14

LT3475-1	12

10

LT3475

8

6

BOOST PIN CURRENT (mA)

10

5

0

0	0.5	1.0

1.5

2.0

2

1

0

| 0 10 20 TA = 25°C |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

OUTPUT VOLTAGE (V)

15

10

5

0

30	40	0

OUTPUT  VOLTAGE

4

LT3475

2

0

10	20	30	40

SWITCH CURRENT (A)

3475 G10

VIN (V)

3475 G11

VIN (V)

3475 G12

| 1.28 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.27

Reference Voltage

Minimum Input Voltage, Single

1.5A White LED

| 6 TA = 25°C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TO START |  |  |  |
| TO RUN |  |  |  |  |  |
|  |  |  |  | LED VOLTAGE |  |

5

Minimum Input Voltage, Two Series

Connected 1.5A White LEDs

10

TA = 25°C

9

1.26	4

VREF (V)

VIN (V)

1.25	3

1.24	2

VIN (V)

8	TO START

7

TO RUN

LED VOLTAGE

1.23	1	6

1.22

–50

–25	0

25	50

0

75	100     125	0

0.5	1

5

1.5	0

0.5	1

1.5

TEMPERATURE (˚C)

LED CURRENT (A)

LED CURRENT (A)

3475 G13

3475 G14

3475 G15

INPUT CURRENT (mA)

PI 	FU	CTIO	S

OUT1, OUT2 (Pins  1, 10): The OUT pin is the input to the current sense resistor. Connect this pin to the inductor and the output capacitor.

LED1, LED2 (Pins 2, 9): The LED pin is the output of the current sense resistor. Connect the anode of the LED here.

VIN (Pins 5, 6): The VIN pins supply current to the internal circuitry and to the internal power switches and must be locally bypassed.

SW1, SW2 (Pins 4, 7): The SW pin is the output of the internal power switch. Connect this pin to the inductor, switching diode and boost capacitor.

BOOST1, BOOST2 (Pins  3, 8): The BOOST pin is used to provide a drive voltage, higher than the input voltage, to the internal bipolar NPN power switch.

GND (Pins 15, Exposed Pad Pin 21): Ground. Tie the GND pin and the exposed pad directly to the ground plane. The exposed pad metal of the package provides both electrical contact to ground and good thermal contact to the printed circuit board. The exposed pad must be soldered to the circuit board for proper operation. Use a large ground plane and thermal vias to optimize thermal performance.

3475fb

PI 	FU	CTIO	S

RT   (Pin 14): The RT  pin is used to set the internal oscillator frequency. Tie a 24.3k resistor from RT to GND for a 600kHz switching frequency.

SHDN (Pin 16): The SHDN pin is used to shut down the switching regulator and the internal bias circuits. The

2.6V switching threshold can function as an accurate undervoltage  lockout. Pull below 0.3V to shut down the LT3475/LT3475-1. Pull above 2.6V to enable the LT3475/ LT3475-1.  Tie to VIN if the SHDN function is unused.

REF (Pin 17):  The REF pin is the buffered output of the internal reference. Either tie the REF pin to the VADJ pin for a 1.5A output current, or use a resistor divider to generate a lower voltage at the VADJ pin. Leave this pin unconnected  if unused.

VC1, VC2 (Pins  18, 13): The  VC pin is the output of the internal error amp. The voltage on this pin controls the peak switch current. Use this pin to compensate the control loop.

VADJ1, VADJ2 (Pins  19, 12): The VADJ pin is the input to the internal voltage-to-current ampliﬁer. Connect the VADJ pin to the REF pin for a 1.5A output current. For lower output currents, program the VADJ pin using the following formula:  ILED = 1.5A • VADJ/1.25V.

PWM1, PWM2 (Pins 20, 11): The PWM pin controls the connection of the VC  pin to the internal circuitry. When the PWM pin is low, the VC pin is disconnected from the internal circuitry and draws minimal current. If the PWM feature is unused, leave this pin unconnected.

BLOCK DIAGRAM

VIN

CIN 	RT

VIN

SHDN

RT 	VIN

INT REG AND UVLO

MASTER OSC

D1	BOOST1	BOOST2	D2

C1

C1	∑

Q      R

Q      S

SLOPE COMP	SLOPE COMP       ∑	C2	C2

R      Q

MOSC 1 	MOSC 2

S      Q

L1	SW1

Q1

DRIVER

SLAVE OSC

SLAVE OSC

Q2

DRIVER

SW2	L2

D3

OUT1

FREQUENCY FOLDBACK

FREQUENCY FOLDBACK

D4

OUT2

COUT1

0.067Ω 	100Ω

100Ω

0.067Ω

COUT2

LED1	LED2

2V	2V

DLED1

gm1 	gm2

1.25V

DLED 2

PWM 1 	PWM2

Q3	Q4

CC1

VC1

1.25k

1.25k

VC2

CC2

VADJ1

REF

VADJ2   EXPOSED PAD

GND

3475 BD

3475fb

OPERATION

The LT3475 is a dual constant frequency, current mode regulator with internal power switches capable of gen- erating constant 1.5A outputs. Operation can be best understood by referring to the Block Diagram.

If the SHDN pin is tied to ground, the LT3475 is shut down and draws minimal current from the input source tied to VIN. If the SHDN pin exceeds  1V, the internal bias circuits turn on, including the internal regulator, reference and oscillator. The switching regulators will only begin to

operate when the SHDN pin exceeds  2.6V.

The switcher is a current mode regulator. Instead of directly modulating the duty cycle of the power switch, the feedback loop controls the peak current in the switch during each cycle. Compared to voltage mode control, current mode control improves loop dynamics and provides cycle-by- cycle current limit.

A pulse from the oscillator sets the RS ﬂip-ﬂop and turns on the internal NPN bipolar power switch. Current in the switch and the external inductor begins to increase. When this current exceeds a level determined by the voltage at VC, current comparator C1 resets the ﬂip-ﬂop, turning off the switch. The current in the inductor ﬂows through the external Schottky diode and begins to decrease. The cycle begins again at the next pulse from the oscillator. In this way, the voltage on the  VC pin controls the current through the inductor to the output. The internal error ampliﬁer  regulates the output current by continually adjusting the VC pin voltage. The threshold for switching on the VC pin is 0.8V, and an active clamp of 1.8V limits the output current.

The voltage on the VADJ pin sets the current through the LED pin. The NPN, Q3, pulls  a current proportional to the voltage  on the VADJ pin through the 100Ω resistor. The gm ampliﬁer servos the VC pin to set the current through the

0.067Ω resistor and the LED pin. When  the voltage drop across the 0.067Ω resistor is equal to the voltage drop across the 100Ω resistor, the servo loop is balanced.

Tying the REF pin to the VADJ pin sets the LED pin current to 1.5A. Tying a resistor divider to the REF pin allows  the

programming of LED pin currents of less than 1.5A. LED pin current can also be programmed by tying the VADJ pin directly to a voltage source.

An  LED can be dimmed with pulse width modulation using the PWM pin and an external   NFET. If the PWM pin is unconnected or is pulled high, the part operates nominally.  If the PWM pin is pulled low, the VC pin is dis- connected from the internal circuitry and draws minimal current from the compensation capacitor. Circuitry draw- ing current from the OUT pin is also disabled. This way, the VC  pin and the output capacitor store the state of the LED pin current until the PWM is pulled high again. This leads to a highly linear relationship between pulse width and output light, allowing for a large and accurate dimming range.

The RT pin allows programming of the switching frequency. For applications requiring the smallest external components possible, a fast switching frequency can be used. If low dropout or very high input voltages are required, a slower switching frequency can be programmed.

During startup VOUT will be at a low voltage. The NPN, Q3, can only operate correctly with sufﬁcient  voltage of ≈1.7V at VOUT, A comparator senses VOUT and forces the VC pin high until VOUT rises above 2V, and Q3 is op- erating correctly.

The switching regulator performs frequency foldback during overload  conditions.  An ampliﬁer senses when VOUT is less than 2V and begins decreasing the oscillator frequency down from full frequency to 15% of the nominal frequency when VOUT = 0V. The OUT pin is less than 2V during startup, short circuit, and overload conditions. Frequency foldback helps limit switch current under these conditions.

The switch driver operates either from VIN  or from the BOOST pin. An external  capacitor  and Schottky diode are used to generate a voltage at the BOOST pin that is higher than the input supply. This allows the driver to saturate the internal  bipolar  NPN power switch for efﬁcient operation.

3475fb

APPLICATIONS INFORMATION

Open Circuit Protection

The LT3475 has internal open-circuit protection. If the LED is absent or is open circuit, the LT3475 clamps the voltage on the LED pin at 14V. The switching regulator then oper- ates at a very low frequency to limit the input current. The LT3475-1 has no internal open circuit protection. With the LT3475-1, be careful not to violate the ABSMAX voltage of th BOOST pin; if VIN > 25V, external open circuit protection

OUT

10k

22V

100k

VC

3475 F01

circuitry (as shown in Figure 1) may be necessary.The output voltage during an open  LED condition is shown in

the Typical Performance  Characteristics  section.

Figure 1. External Overvoltage Protection

Circuitry for the LT3475-1

LT3475

Undervoltage Lockout

Undervoltage lockout (UVLO) is typically used in situations where the input supply is current limited, or has high source resistance. A switching  regulator  draws constant power from the source, so the source current increases as the source voltage drops. This looks like a negative resistance

VIN

R1

C1	R2

VIN

SHDN

2.6V

9μA

GND

VC

3475 F02

load to the source and can cause the source to current limit or latch low under low source voltage conditions. UVLO prevents the regulator from operating at source voltages where these problems might occur.

An  internal comparator will  force the part into shut- down  when  VIN falls below 3.7V. If an adjustable  UVLO threshold is required, the SHDN pin can be used. The threshold voltage of the SHDN pin comparator is 2.6V. An internal resistor  pulls 9μA to ground from the SHDN pin at the UVLO threshold.

Choose resistors according to the following formula:

2.6V

Figure 2. Undervoltage Lockout

Keep the connections from the resistors to the SHDN pin short and make sure the coupling to the SW and BOOST pins is minimized. If high resistance  values are used, the SHDN pin should be bypassed with a 1nF capacitor to prevent coupling problems from switching nodes.

Setting the Switching Frequency

| The LT3475 uses a constant frequency architecture that can be programmed over a 200kHz to 2MHz range with a single external timing resistor from the RT pin to ground. A graph for selecting the value of RT for a given operating SWITCHING FREQUENCY (MHz) | RT (kΩ) |
| --- | --- |
| 2 | 4.32 |
| 1.5 | 6.81 |
| 1.2 | 9.09 |
| 1 | 11.8 |
| 0.8 | 16.9 |
| 0.6 | 24.3 |
| 0.4 | 40.2 |
| 0.3 | 57.6 |
| 0.2 | 100 |

R2 = V

– 2.6V

frequency is shown in the Typical Applications section.

TH 		– 9μA R1

VTH = UVLO Threshold

Example: Switching  should not start until the input is above 8V.

VTH = 8V R1=100k

Table 1. Switching Frequencies

R2 =	2.6V

8V – 2.6V

100k

– 9μA

= 57.6k

3475fb

APPLICATIONS INFORMATION

Table 1 shows suggested RT selections  for a variety of switching frequencies.

Operating Frequency Selection

The maximum operating voltage is determined by the absolute maximum ratings of the VIN  and BOOST pins, and by the minimum duty cycle.

VOUT + VF

The choice of operating frequency is determined by several factors. There is a tradeoff between efﬁciency and

VIN(MAX ) =

DCMIN

– VF + VSW

component size. A higher switching frequency allows the use of smaller inductors at the cost of increased switching losses and decreased efﬁciency.

Another consideration is the maximum duty cycle. In certain applications, the converter needs to operate at a high duty cycle in order to work at the lowest input voltage possible. The LT3475 has a ﬁxed oscillator off time and a variable

with DCMIN = tON(MIN) • f

where tON(MIN) is equal to 140ns and f is the switching frequency.

Example:  f = 750kHz,  VOUT = 3.4V

DCMIN = 140ns • 750kHz = 0.105

3.4V + 0.4V

on time. As a result, the maximum duty cycle increases as the switching frequency is decreased.

VIN(MAX ) =

– 0.4V + 0.4V = 36V

0.105

Input Voltage Range

The minimum operating voltage is determined either by the LT3475’s undervoltage lockout of 4V, or by its maximum duty cycle. The duty cycle is the fraction of time that the internal switch is on and is determined by the input and output voltages:

( VOUT  + VF )

The minimum duty cycle depends on the switching fre- quency. Running at a lower switching frequency might allow a higher maximum operating voltage. Note that this is a restriction on the operating input voltage; the circuit  will tolerate transient inputs up to the Absolute Maximum Ratings of the VIN and BOOST pins.  The input

voltage should be limited to the VIN operating range (36V)

during overload conditions (short circuit or start up).

DC =

(VIN – VSW + VF )

Minimum On Time

where  VF is the forward voltage drop of the catch diode (~0.4V) and VSW is the voltage drop of the internal switch (~0.4V at maximum load). This leads to a minimum input voltage of:

VOUT + VF

The LT3475 will regulate  the output current at input volt- ages greater than VIN(MAX). For example,  an application with an output voltage of 3V and switching frequency of

1.2MHz has a VIN(MAX) of 20V, as shown in Figure 3. Figure

4 shows operation at 35V. Output ripple and peak inductor

VIN(MIN) =

DCMAX

– VF + VSW

with DCMAX = 1–tOFF(MIN)  • f

where t0FF(MIN) is equal to 167ns and f is the switching frequency.

Example:  f = 600kHz, VOUT = 4V

DCMAX = 1− 167ns • 600kHz = 0.90

4V + 0.4V

VOUT

500mV/DIV

(AC COUPLED)

IL

1A/DIV

VSW

20V/DIV

VIN(MIN) =

– 0.4V + 0.4V = 4.9V

0.9

3475 F03

Figure 3. Operation at VIN(MAX) = 20V. VOUT = 3V and fSW = 1.2MHHz

3475fb

APPLICATIONS INFORMATION

current have signiﬁcantly increased. Exceeding VIN(MAX) is safe if the external components have adequate ratings to handle the peak conditions and if the peak inductor current does not exceed  3.2A. A saturating inductor may further reduce performance.

VOUT

500mV/DIV

(AC COUPLED)

IL

1A/DIV

VSW

20V/DIV

3475 F04

Figure 4. Operation above VIN(MAX). Output

Ripple and Peak Inductor Current Increases

Inductor Selection and Maximum Output Current

A good  ﬁrst choice for the inductor value is:

Table 2. Inductors

| PART NUMBER | VALUE (μH) | IRMS (A) | DCR ( ) | HEIGHT (mm) |
| --- | --- | --- | --- | --- |

Sumida

| CR43-3R3 | 3.3 | 1.44 | 0.086 | 3.5 |
| --- | --- | --- | --- | --- |
| CR43-4R7 | 4.7 | 1.15 | 0.109 | 3.5 |
| CDRH4D16-3R3 | 3.3 | 1.10 | 0.063 | 1.8 |
| CDRH4D28-3R3 | 3.3 | 1.57 | 0.049 | 3.0 |
| CDRH4D28-4R7 | 4.7 | 1.32 | 0.072 | 3.0 |
| CDRH6D26-5R0 | 5.0 | 2.20 | 0.032 | 2.8 |
| CDRH6D26-5R6 | 5.6 | 2.0 | 0.036 | 2.8 |
| CDRH5D28-100 | 10 | 1.30 | 0.048 | 3.0 |
| CDRH5D28-150 | 15 | 1.10 | 0.076 | 3.0 |
| CDRH73-100 | 10 | 1.68 | 0.072 | 3.4 |
| CDRH73-150 | 15 | 1.33 | 0.130 | 3.4 |
| CDRH104R-150 | 15 | 3.1 | 0.050 | 4.0 |

| Coilcraft DO1606T-332 | 3.3 | 1.30 | 0.100 | 2.0 |
| --- | --- | --- | --- | --- |
| DO1606T-472 | 4.7 | 1.10 | 0.120 | 2.0 |
| DO1608C-332 | 3.3 | 2.00 | 0.080 | 2.9 |
| DO1608C-472 | 4.7 | 1.50 | 0.090 | 2.9 |
| MOS6020-332 | 3.3 | 1.80 | 0.046 | 2.0 |
| MOS6020-472 | 10 | 1.50 | 0.050 | 2.0 |
| DO3316P-103 | 10 | 3.9 | 0.038 | 5.2 |
| DO3316P-153 | 15 | 3.1 | 0.046 | 5.2 |

L = (V

1.2MHz

+ V ) •

OUT 	F	f

where  VF is the voltage drop of the catch diode (~0.4V), f is the switching frequency and L is in μH. With this value the maximum load current  will be above 1.6A at all duty cycles. The inductor’s RMS current rating must be greater than the maximum load current and its saturation current should be at least 30% higher. For highest efﬁciency, the series resistance  (DCR) should be less than 0.15Ω. Table 2 lists several vendors and types that are suitable. For robust operation at full load and high input voltages (VIN  > 30V), use an inductor with a saturation current higher than 3.2A.

The optimum inductor for a given application may differ from the one indicated by this simple design guide. A larger value inductor provides a higher maximum load current, and reduces the output voltage ripple. If your load is lower than the maximum load current, then you can relax the value of the inductor and operate with higher ripple current. This allows you to use a physically smaller inductor, or one with a lower DCR resulting in higher efﬁciency. In addition, low inductance may result in discontinuous mode operation, which further reduces maximum load current. For details of maximum output current and discontinuous mode operation, see Linear Technology’s Application Note 44. Finally, for duty cycles greater than 50% (VOUT/VIN > 0.5),  a minimum inductance is required to avoid sub-harmonic oscillations:

800kHz

LMIN = (VOUT + VF ) •	f

3475fb

APPLICATIONS INFORMATION

The current in the inductor is a triangle wave with an average value equal to the load current. The peak switch current is equal to the output current plus half the peak-to-peak inductor ripple current. The LT3475 limits its switch cur- rent in order to protect itself and the system from overload faults. Therefore, the maximum output current that the LT3475 will deliver depends  on the switch current limit, the inductor value, and the input and output voltages.

When the switch is off, the potential across the inductor is the output voltage plus the catch diode drop. This gives the peak-to-peak ripple current in the inductor

(1– DC)( VOUT + VF )

Input Capacitor Selection

Bypass the input of the LT3475 circuit with a 4.7μF or higher ceramic capacitor of X7R or X5R type. A lower value or a less expensive  Y5V type will work  if there is additional bypassing provided by bulk electrolytic capaci- tors  or if the input source impedance is low. The following paragraphs describe the input capacitor considerations in more detail.

Step-down regulators draw current from the input supply in pulses with very fast rise and fall times. The input ca- pacitor is required to reduce the resulting voltage ripple at the LT3475 input and to force this switching current into a

ΔIL =

(L• f)

tight local loop, minimizing  EMI. The input capacitor must

where  f is the switching frequency of the LT3475 and L is the value of the inductor. The peak inductor and switch current is

I		= I	= I	+ ΔIL SW (PK )		L(P K )		OUT 		2

To maintain output regulation, this peak current must be less than the LT3475’s switch  current  limit ILIM. ILIM is at

have low impedance at the switching frequency to do this

effectively, and it must have an adequate ripple current rat- ing. With two switchers operating at the same frequency but with different phases and duty cycles, calculating the input capacitor  RMS current is not simple. However, a conservative value is the RMS input current for the channel that is delivering most power  (VOUT • IOUT):

V	(V	–V	)	I

least 2.3A at low duty cycles and decreases linearly to 1.8A

at DC = 0.9. The maximum output current is a function of the chosen inductor value:

CINRM S = IOUT •  	OUT     IN 	OUT    <  OUT

VIN 	2

IOUT(M A X )  = ILIM –

ΔIL

2

and is largest when VIN = 2VOUT (50%  duty cycle).  As the second, lower power channel draws input current, the

= 2.3A• (1–0.25•DC) – ΔIL

2

Choosing an inductor value so that the ripple current is small will allow a maximum output current near the switch current limit.

One approach to choosing the inductor is to start with the simple rule given above, look at the available inductors, and choose one to meet cost or space goals. Then use these equations to check that the LT3475 will be able to deliver the required output current. Note again that these equations assume that the inductor current is continu- ous. Discontinuous operation occurs when  IOUT is less

than ΔIL/2.

input capacitor’s RMS current actually decreases as the

out-of-phase current cancels the current drawn by the higher power channel. Considering that the maximum load current from a single channel is ~1.5A, RMS ripple current  will always be less than 0.75A.

The high frequency of the LT3475 reduces the energy storage requirements of the input capacitor, so that the capacitance required is less than 10μF. The combination of small size and low impedance (low equivalent series resistance or ESR) of ceramic capacitors makes them the preferred choice. The low ESR results in very low voltage ripple. Ceramic capacitors can handle larger magnitudes of ripple current than other capacitor types of the same value. Use X5R and X7R types.

3475fb

APPLICATIONS INFORMATION

An alternative   to a high value ceramic capacitor is a lower value ceramic along with a larger electrolytic capacitor. The electrolytic capacitor likely needs to be greater than 10μF in order to meet the ESR and ripple current requirements. The input capacitor is likely to see high surge currents when the input source is applied. Tanta- lum capacitors can fail due to an over-surge of current. Only use tantalum capacitors with the appropriate surge current rating. The manufacturer may also recommend operation below the rated voltage of the capacitor.

A ﬁnal caution  is in order regarding the use of ceramic capacitors at the input. A  ceramic input capacitor can combine with stray inductance to form a resonant tank circuit. If power is applied quickly (for example by plug- ging the circuit into a live power source) this tank can ring, doubling the input voltage and damaging the LT3475. The solution is to either clamp the input voltage or dampen the tank circuit by adding a lossy capacitor in parallel with the ceramic capacitor. For details, see Application Note 88.

Output Capacitor Selection

For most LEDs, a 2.2μF, 6.3V ceramic capacitor (X5R or X7R) at the output results in very low output voltage ripple and good transient response. Other types and values will also work. The following discusses tradeoffs in output ripple and transient performance.

The output capacitor ﬁlters the inductor current to generate an output with low voltage ripple. It also stores energy in order to satisfy transient loads and stabilizes the LT3475’s control loop. Because the LT3475 operates at a high frequency, minimal output capacitance is necessary. In addition, the control loop operates  well with or without the presence of output capacitor series resistance (ESR). Ceramic capacitors, which achieve very low output ripple and small circuit size, are therefore an option.

You  can estimate output ripple with the following equation:

VRIPPLE = ΔIL / (8 • f • COUT) for ceramic capacitors where ΔIL   is the peak-to-peak  ripple current in the

inductor. The RMS content of this ripple is very low so the

RMS current rating of the output capacitor is usually not of concern. It can be estimated with the formula:

IC(RM S) = ΔIL /	12

The low ESR and small size of ceramic capacitors make them the preferred type for LT3475 applications.  Not all ceramic capacitors are the same, however. Many of the higher value capacitors use poor dielectrics with high temperature and voltage coefﬁcients. In particular  Y5V and Z5U types lose a large fraction of their capacitance with applied voltage and at temperature extremes. Because loop stability and transient response depend on the value of COUT, this loss may be unacceptable. Use X7R and X5R types. Table 3 lists several capacitor vendors.

Table 3. Low ESR Surface  Mount Capacitors.

| VENDOR | TYPE | SERIES |
| --- | --- | --- |
| Taiyo-Yuden | Ceramic | X5R, X7R |
| AVX | Ceramic | X5R, X7R |
| TDK | Ceramic | X5R, X7R |

Diode Selection

The catch diode (D3 from the Block Diagram) conducts current only during switch off time. Average forward cur- rent in normal operation can be calculated from:

ID(AVG) = IOUT (VIN – VOUT)/VIN

The only reason to consider a diode with a larger current rating than necessary for nominal operation is for the worst-case condition of shorted output. The diode cur- rent will then increase  to one half the typical peak switch current limit.

Peak reverse voltage is equal to the regulator input voltage. Use a diode with a reverse voltage rating greater than the input voltage. Table  4 lists several Schottky diodes and their manufacturers.

Diode reverse leakage can discharge the output capacitor during LED off times while PWM dimming. If operating at high ambient temperatures, use a low leakage Schottky for the widest PWM dimming range.

3475fb

APPLICATIONS INFORMATION

Table 4. Schottky Diodes

|  | VR (V) | IAVE(A) (A) | VF at 1A (mV) | VF at 2A (mV) |
| --- | --- | --- | --- | --- |

On Semiconductor

| MBR0540 | 40 | 0.5 | 620 |  |
| --- | --- | --- | --- | --- |
| MBRM120E | 20 | 1 | 530 |  |
| MBRM140 | 40 | 1 | 550 |  |

Diodes Inc

| B120 | 20 | 1 | 500 |  |
| --- | --- | --- | --- | --- |
| B130 | 30 | 1 | 500 |  |
| B140HB | 40 | 1 | 530 |  |
| DFLS140 | 40 | 1.1 | 510 |  |
| B240 | 40 | 2 |  | 500 |

International Rectiﬁer

| 10BQ030 | 30 | 1 | 420 |  |
| --- | --- | --- | --- | --- |

BOOST Pin Considerations

The capacitor and diode tied to the BOOST pin gener- ate a voltage that is higher than the input voltage. In most cases, a 0.22μF capacitor and fast switching diode (such as the CMDSH-3 or MMSD914LT1) will work well. Figure 5 shows three ways to arrange the boost circuit. The BOOST pin must be more than 2.5V above the SW

pin for full efﬁciency. For outputs  of 3.3V and higher, the standard circuit (Figure 5a) is best. For outputs between

2.8V and 3.3V, use a small Schottky diode (such as the BAT-54). For lower output voltages, the boost diode can be tied to the input (Figure 5b). The circuit in Figure 5a is more efﬁcient because the BOOST pin current comes from a lower voltage source. The anode of the boost diode can be tied to another source that is at least 3V. For example,  if you are generating a 3.3V output, and the 3.3V output is on whenever the LED  is  on, the BOOST  pin can be connected to the 3.3V output. For LT3475-1 applications with higher output voltages, an additional Zener diode may be necessary (Figure 5d) to maintain pin voltage below the absolute maximum. In any case, be sure that the maximum voltage at the BOOST pin is both less than

60V and the voltage difference between the BOOST and

SW pins is less than 30V.

The minimum operating voltage of an LT3475 application is limited by the undervoltage lockout (~3.7V) and by the maximum duty cycle. The boost circuit also limits the minimum input voltage for proper start up. If the input voltage ramps slowly, or the LT3475 turns on when the output is already in regulation, the boost capacitor may not be fully charged. Because the boost capacitor charges

D2	D2

VIN

VIN

BOOST	C3

LT3475

SW

VOUT

VIN

VIN

BOOST LT3475

SW

C3

VOUT

GND

GND

VIN2 > 3V

VBOOST – VSW  ≅ VOUT

MAX VBOOST  ≅ VIN  + VOUT

(5a)

D2

VBOOST – VSW  ≅ VIN

MAX VBOOST   ≅ 2VIN

(5b)

D2

VIN

VIN

BOOST LT3475

SW

C3

VOUT

VIN

VIN

BOOST	C3

LT3475

SW

VOUT

GND

GND

VBOOST – VSW  ≅ VIN2

MAX VBOOST  ≅ VIN2  + VIN

MINIMUM  VALUE FOR VIN2 = 3V

(5c)

3475 F05

VBOOST – VSW – VZ

MAX VBOOST  ≅ VIN  + VOUT – VZ

(5d)

3475 F05

Figure 5. Generating the Boost Voltage

3475fb

APPLICATIONS INFORMATION

with the energy stored in the inductor, the circuit will rely on some minimum load current to get the boost circuit running properly. This minimum load will depend on input and output voltages, and on the arrangement of the boost circuit. The minimum load current generally goes to zero once the circuit has started. The typical performance char- acteristics section shows a plot of minimum load to start and to run as a function of input voltage. Even without an output load current, in many cases the discharged output capacitor will present a load to the switcher that will allow it to start. The plots show the worst case, where  VIN is ramping very slowly.

Programming LED Current

The LED current can be set by adjusting the voltage on the VADJ pin. For a 1.5A LED current, either tie VADJ to REF or to a 1.25V source. For lower output currents,  program the VADJ using the following formula: ILED = 1.5A • VADJ/1.25V. Voltages less than 1.25V can be generated with a voltage divider from the REF pin, as shown in Figure 6. In order to have accurate LED  current, precision resistors are preferred (1% or better is recommended).  Note that the VADJ pin sources a small amount of bias current, so use the following formula to choose resistors:

the voltage on the VADJ pin by tying a low on resistance FET  to the resistor divider string. This allows the se- lection of two different  LED currents. For reliable op- eration program an LED current of no less than 50mA. The maximum current dimming ratio (IRATIO)  can be calculated from the maximum LED current (IMAX) and the minimum LED current (IMIN) as follows:

IMAX/IMIN = IRATIO

Another dimming control circuit (Figure 8) uses the PWM pin and an external  NFET tied to the cathode of the LED. An external  PWM signal is applied to the PWM pin and the gate of the NFET (For PWM dimming ratios of 20 to 1 or less, the NFET can be omitted). The average LED current is proportional to the duty cycle of the PWM signal. When the PWM signal goes low, the NFET turns off, turning off the LED and  leaving  the output capacitor charged. The PWM pin is pulled low as well, which disconnects  the VC pin, storing the voltage in the capacitor tied there. Use the C-RC string shown in Figure 8 and Figure 9 tied to the VC pin for proper operation  during startup.  When the PWM pin goes high again, the LED current returns rapidly to its previous on state since the compensation and output capacitors are at the correct voltage. This fast settling time allows the

R2 =	VADJ

1.25V – VADJ  + 50nA

R1

To minimize the error from variations in VADJ pin current, use resistors with a parallel resistance of less than 4k. Use resistor strings with a high enough series resistance so as not to exceed the 500μA current compliance of the REF pin.

DIM

REF R1

VADJ

R2

LT3475

GND

3475 F07

Dimming Control

There are several different  types of dimming control circuits. One dimming control circuit (Figure 7) changes

Figure 7. Dimming with a MOSFET and Resistor Divider

PWM

REF R1

VADJ

R2

LT3475

GND

3475 F06

100Hz  TO

10kHz

PWM

LED

LT3475

GND

VC

3475 F08

3.3nF

10k

0.1μF

Figure 6. Setting  VADJ with a Resistor Divider

Figure 8. Dimming  Using PWM Signal

3475fb

APPLICATIONS INFORMATION

LT3475 to maintain diode current regulation with PWM pulse widths as short as 7.5 switching cycles (12.5μs for fSW = 600kHz). Maximum PWM period is determined by the system and is unlikely to be longer than 12ms. Using PWM periods shorter than 100μs is not recommended. The maximum  PWM dimming  ratio (PWMRATIO) can be calculated from the maximum  PWM period (tMAX) and minimum PWM pulse width (tMIN) as follows:

tMAX/tMIN = PWMRATIO

Total dimming ratio (DIMRATIO) is the product of the PWM

dimming ratio and the current dimming ratio.

Example:

IMAX = 1A, IMIN = 0.1A, tMAX = 9.9ms tMIN = 3.3μs  (fSW = 1.4MHz)

IRATIO = 1A/0.1A =10:1

Layout Hints

As with all switching regulators, careful attention must be paid to the PCB layout and component placement. To maximize efﬁciency, switch rise and fall times are made as short as possible. To prevent electromagnetic interfer- ence (EMI) problems, proper layout of the high frequency switching path is essential. The voltage signal of the SW and BOOST pins have sharp rise and fall edges.  Minimize the area of all traces connected to the BOOST and SW pins and always use a ground plane under the switching regulator to minimize interplane coupling. In addition, the ground connection for frequency setting resistor RT and capacitors at VC1, VC2 pins (refer to the Block Diagram) should be tied directly to the GND pin and not shared with the power ground path, ensuring a clean, noise-free connection.

PWMRATIO = 9.9ms/3.3μs  = 3000:1

DIMRATIO = 10 • 3000 = 30000:1

To achieve  the maximum PWM dimming ratio, use the circuit shown in Figure 9. This allows PWM pulse widths as short as 4.5 switching  cycles (7.5μs for fSW = 600kHz). Note that if you use the circuit in Figure 9, the rising edge of the two PWM signals must align within 100ns.

20

19

18

17

16

15

14

13

12

11

PWM1

SHDN

1

2

3

4

5

6

7

8

9

10

PWM2

PWM1

220pF

1M

RT 	VC

LT3475

RT 	GND

3475 F09

3.3nF

10k

0.1μF

VIN

Figure 9. Extending the PWM Dimming  Range

VIA TO LOCAL GND PLANE

Figure 10. Recommended Component Placement

3475 F10

3475fb

TYPICAL APPLICATIONS

Dual Step-Down 1A LED Driver

VIN

5V TO 36V	C1

4.7μF

D3	        50V

C4

L2	0.22μF

VIN	SHDN BOOST1		BOOST2

LT3475

D4

C3

0.22μF 	L1

10μH

6.3V

SW1

SW2

6.3V

10μH

D1	D2

C5

2.2μF

6.3V

OUT1	OUT2

LED1	LED2

LED 1	LED 2

VC1	VC2

C2

2.2μF

6.3V

C6

0.1μF

R2

1k

R3

2k

REF	RT

VADJ1 	VADJ2

GND

C7

0.1μF

R1

24.3k

C1 TO C5: X5R OR X7R D1, D2: DFLS140

D3, D4: MBR0540

LED CURRENT = 1A

fSW = 600kHz

3475 TA02

Dual Step-Down 1.5A LED Driver with  1200 : 1 True Color PWM Dimming

VIN

6V TO 36V	C1

4.7μF

D3	        50V

C2

L2	0.22μF

VIN	SHDN BOOST1		BOOST2

LT3475

D4

C3

0.22μF 	L1

10μH

C4

2.2μF

6.3V

6.3V

D1

SW1

SW2

6.3V

D2

10μH

C5

2.2μF

6.3V

OUT1	OUT2

1.5A LED CURRENT

LED 1

LED1	LED2

PWM1	PWM2

LED 2

1.5A LED CURRENT

R3

10k

M1

C8

0.1μF

C6

3.3nF

VC1	VC2

REF	RT

VADJ1 	VADJ2

GND

C7

3.3nF

R4

10k

M2

C9

0.1μF

C8	1M M3	220p	R2

R1

24.3k

fSW = 600kHz

3475 TA03

PWM1	PWM2

D1, D2: B260

D3, D4: MBR0540

C1 TO C5: X5R OR X7R M1, M2: Si2302ADS M3: 2n7002L

3475fb

TYPICAL APPLICATIONS

Step-Down  3A LED Driver

VIN

5V TO 36V	C1

4.7μF

D3	        50V

C2

L2	0.22μF

VIN	SHDN BOOST1		BOOST2

LT3475

D4

C3

0.22μF 	L1

10μH

6.3V

D1

SW1

SW2

6.3V

D2

10μH

C5

2.2μF

6.3V

C4

2.2μF

6.3V

OUT1	OUT2

LED1

LED2

C6

0.1μF

VC1 	VC2

REF	RT

VADJ1 	VADJ2

GND

R1

24.3k

C7

0.1μF

LED 1

3A LED CURRENT

D1, D2: B240A

D3, D4: MBR0540

C1 TO C5: X5R OR X7R

fSW = 600kHz

3475 TA04

Dual Step-Down LED Driver with  Series Connected  LEDs

VIN

10V TO 36V	C1

4.7μF

D3	        50V

C2

L2	0.22μF

VIN	SHDN BOOST1		BOOST2

LT3475

D4

C3

0.22μF 	L1

15μH

10V

SW1

SW2

10V

15μH

D1	D2

C4

2.2μF

10V

OUT1

LED1

OUT2

LED2

C5

2.2μF

10V

1.5A  LED CURRENT

LED 1

C6

0.1μF

LED 3

VC1 	VC2

REF	RT

VADJ1 	VADJ2

GND

R1

24.3k

LED 2

C7

0.1μF

LED 4

1.5A LED CURRENT

D1, D2: B240A

D3, D4: MMSD4148T1

C1 TO C5: X5R OR X7R

fSW = 600kHz

3475 TA05

3475fb

TYPICAL APPLICATIONS

Dual Step-Down 1.5A Red LED Driver

VIN

5V TO 28V	C1

4.7μF

35V

L2

D3

C2

0.22μF

VIN	SHDN BOOST1		BOOST2

LT3475

D4

C3

0.22μF 	L1

10μH

35V

SW1

SW2

35V

10μH

D1	D2

C4

2.2μF

6.3V

OUT1

LED1

OUT2

LED2

C5

2.2μF

6.3V

1.5A  LED CURRENT

C6

0.1μF

LED 1

VC1 	VC2

REF	RT

VADJ1 	VADJ2

GND

R1

24.3k

C7

0.1μF

LED 2

1.5A LED CURRENT

D1, D2: B240A

D3, D4: MMSD4148T1

C1 TO C5: X5R OR X7R

fSW = 600kHz

3475 TA06

3475fb

PACKAGE DESCRIPTION

FE Package

20-Lead Plastic  TSSOP (4.4mm) (Reference LTC DWG # 05-08-1663) Exposed Pad Variation CB

3.86 (.152)

6.40 – 6.60* (.252 – .260)

3.86 (.152)

20  1918 17 16 15 14 13 12  11

6.60 ±0.10

4.50 ±0.10

SEE NOTE 4

2.74 (.108)

0.45 ±0.05

2.74 (.108)

6.40  (.252) BSC

1.05 ±0.10

0.65 BSC RECOMMENDED SOLDER PAD LAYOUT

4.30 – 4.50*

1   2   3   4   5   6   7   8   9 10

1.20  (.047)

(.169 – .177)

0.25

REF

0° – 8°

MAX

0.09 – 0.20 	 (.0035 – .0079)

NOTE:

0.50 – 0.75 (.020 – .030)

0.65 (.0256) BSC

0.195 – 0.30 	 (.0077 – .0118) TYP

0.05 – 0.15  (.002 – .006)

FE20 (CB) TSSOP 0204

1. CONTROLLING DIMENSION: MILLIMETERS

2. DIMENSIONS ARE IN  MILLIMETERS

4. RECOMMENDED MINIMUM  PCB METAL SIZE FOR EXPOSED PAD ATTACHMENT

(INCHES)	*DIMENSIONS  DO NOT INCLUDE MOLD FLASH. MOLD FLASH

3. DRAWING NOT TO SCALE

SHALL NOT EXCEED 0.150mm  (.006") PER SIDE

Information furnished by Linear Technology Corporation is believed to be accurate and reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no representation that the interconnection of its circuits as described herein will not infringe on existing patent rights.

3475fb

19

TYPICAL APPLICATION

Dual Step-Down 1.5A LED Driver with  Four Series Connected LED Output

VIN

21V TO 36V

C1

4.7μF

D1	D2

50V

VIN

SHDN

D3	D4

L1

33μH

R1

1k

C2

0.22μF

16V

D5

BOOST1	BOOST2

LT3475-1

SW1	SW2

OUT1	OUT2

C3

0.22μF

16V D6

L2

33μH

R2

1k

LED1

LED2

R4

10k

D7	C4

12V TO 18V LED VOLTAGE	12V TO 18V LED VOLTAGE

VC1 	VC2

R5

10k

C5	D8

2.2μF

25V

Q1	R6

100k

C6

0.1μF

1.5A LED CURRENT*

REF	RT

VADJ1 	VADJ2

GND

R3

24.3k

C7

0.1μF

1.5A LED CURRENT*

2.2μF

25V

R7	Q2

100k

fSW = 600kHz

D1, D4: 7.5V ZENER DIODE D2, D3: MMSD4148

D5, D6: B240A

D7, D8: 22V ZENER DIODE

R1, R2: USE 0.5W RESISTOR  OF TWO 2k 0.25W RESISTORS IN PARALLEL Q1, Q2: MMBT3904

C1 TO C5: X5R or X7R

*DERATE LED CURRENT AT ELEVATED AMBIENT TEMPERATURES  TO MAINTAIN LT3475-1 JUNCTION TEMPERATURE BELOW 125 °C.

3475 TA08

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT1618 | Constant-Current, 1.4MHz, 1.5A Boost Converter | VIN(MIN) = 1.6V, VIN(MAX) = 18V, VOUT(MAX) = 35V, Analog/PWM, ISD < 1μA, MS10 Package |
| LT3466 | Dual Full Function Step-Up LED Driver | Drivers Up to 20 LEDs, VIN: 2.7V to 24V, VOUT(MAX) = 40V, DFN, TSSOP16E Packages |
| LT3474 | 36V, 1A (ILED), 2MHz Step-Down LED Driver | VIN(MIN) = 4V, VIN(MAX) = 36V, 400:1 True Color PWM, ISD < 1μA, TSSOP16E Package |
| LT3477 | 42V, 3A, 3.5MHz Boost, Buck-Boost, Buck LED Driver | VIN(MIN) = 2.5V, VIN(MAX) = 25V, VOUT(MAX) = 40V, Analog/PWM, ISD < 1μA, QFN, TSSOP20E Packages |
| LT3479 | 3A, Full-Featured DC/DC Converter with Soft-Start and Inrush Current Protection | VIN(MIN) = 2.5V, VIN(MAX) = 24V, VOUT(MAX) = 40V, Analog/PWM, ISD < 1μA, DFN, TSSOP Packages |
| LT3846 | Dual 1.3A, 2MHz, LED Driver | VIN: 2.5V to 24V, VOUT(MAX) = 36V, 1000:1 True Color PWMTM Dimmin, DFN, TSSOP16E Packages |

Linear Technology Corporation

20

1630 McCarthy  Blvd.,  Milpitas,  CA 95035-7417

3475fb

LT 1007 REV B • PRINTED IN USA

(408) 432-1900

● FAX: (408) 434-0507

●  www.linear.com	© LINEAR TECHNOLOGY CORPORATION 2006