Low  Noise, High Frequency,

8th Order Butterworth Lowpass Filter

FEATURES

■   8th Order Filter in a 14-Pin Package

■   140kHz Maximum  Corner Frequency

■   No External Components

■   50:1 and 100:1 Clock to Cutoff Frequency Ratio

■   80µVRMS Total Wideband Noise

■    0.03%  THD or Better

■    Operates from ±2.37V to ±8V Power Supplies

APPLICATIO  S

■   Antialiasing Filters

■   Smoothing Filters

■    Tracking High Frequency Lowpass Filters

DESCRIPTIO

The LTC®1064-2  is a  monolithic   8th order  lowpass Butterworth filter, which provides a maximally flat pass- band. The   attenuation  slope is – 48dB/octave and the maximum attenuation is in excess of 80db. An external TTL or CMOS clock programs the filter’s cutoff frequency. The clock to cutoff frequency ratio is 100:1 (Pin 10 at

V –) or 50:1 (Pin 10 at V +). The maximum cutoff frequency

is 140kHz. No external components are needed.

The LTC1064-2  features  low wideband noise and low harmonic distortion  even for input voltages up to 3VRMS. In fact the LTC1064-2 overall performance competes with equivalent multiple op amp RC active realizations.  The LTC1064-2 is available in a 14-pin DIP or 16-pin surface mounted SW package. The LTC1064-2 is fabricated using

LTC’s enhanced analog CMOS Si-gate process.

The LTC1064-2 is pin compatible with the LTC1064-1.

, LTC and LT are registered trademarks of Linear Technology Corporation.

TYPICAL APPLICATIO

8th Order Clock Sweepable

Lowpass Butterworth Filter	Measured Frequency Response

1	14

NC 	OUT C

2	LTC1064-2	13

0

–15

VS = ±7.5V

VIN

3

AGND

4

8V	V+

5

AGND

6

NC

7

RIN A

NC

V–

fCLK

50/100

VOUT

NC

12 –8V

11 CLOCK = 5MHz

10

9

8

1064 TA01a

–30

–45

VOUT/VIN (dB)

–60

–75

–90

–105

0

100    200

300

400

500

600   700

NOTE: THE POWER SUPPLIES SHOULD BE BYPASSED BY A 0.1µF

CAPACITOR CLOSE TO THE PACKAGE. THE NC PINS 1, 6, 8, AND 13 SHOULD BE PREFERABLY GROUNDED.

FREQUENCY (kHz)

1064-2 TA01b

10642fa

ABSOLUTE 	AXI	U	RATI 	GS

(Note 1)

Total Supply Voltage (V + to V –) ............................ 16.5V Power Dissipation .............................................. 400mW Storage Temperature Range .................  – 65°C to 150°C Lead Temperature (Soldering, 10 sec) .................. 300°C

Operating Temperature Range

LTC1064-2M (OBSOLETE) ...............  – 55°C to 125°C LTC1064-2C .......................................  – 40°C to 85°C

PACKAGE/ORDER  I	FOR	ATIO

| TOP VIEW NC 1 14 OUT C VIN 2 13 NC AGND 3 12 V– V+ 4 11 fCLK AGND 5 10 50/100 NC 6 9 VOUT RIN A 7 8 NC N PACKAGE 14-LEAD PDIP TJMAX = 150°C, JA = 110°C/W | ORDER PART NUMBER | TOP VIEW NC 1 16 OUT C VIN 2 15 NC AGND 3 14 V– V+ 4 13 NC AGND 5 12 fCLK NC 6 11 50/100 NC 7 10 NC RIN A 8 9 VOUT SW PACKAGE 16-LEAD PLASTIC (WIDE) SO TJMAX = 150°C, JA = 130°C/W | ORDER PART NUMBER |
| --- | --- | --- | --- |
|  | LTC1064-2CN |  | LTC1064-2CSW |
| J PACKAGE 14-LEAD CERDIP OBSOLETE PACKAGE Consider the N14 Package for Alternate Source | LTC1064-2MJ LTC1064-2CJ |  |  |

Consult LTC Marketing for parts specified with wider operating temperature ranges.

ELECTRICAL CHARACTERISTICS	The ● denotes the specifications which apply over the full operating temperature range, otherwise specifications are at TA = 25°C. VS = ±7.5V,  100:1, fCLK = 2MHz, R1 = 10k, TTL clock input level unless otherwise specified.

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Passband Gain (Note 2) Gain TempCo –3dB Frequency Gain at –3dB Frequency Stopband Attenuation Stopband Attenuation Stopband Attenuation Stopband Attenuation | Referenced to 0dB, 1Hz to 1kHz 100:1 50:1 Referenced to 0dB, fIN = 20kHz At 1.5f – 3dB , 50:1, fIN = 60kHz At 2f– 3dB , 100:1, fIN = 40kHz At 3f– 3dB , 100:1, fIN = 60kHz At 4f– 3dB , 100:1, fIN = 80kHz | ● ● ● ● | – 0.5 0.15 0.0002 20 40 –3 – 2.75 –24 – 27 –44 – 47 – 74 – 90 | dB dB/°C kHz kHz dB dB dB dB dB |
| Input Frequency Range | 100:1 50:1 |  | 0 <fCLK/2 0 <fCLK | kHz kHz |
| Output Voltage Swing and Operating Input Voltage Range | VS = ±2.37V VS = ±5V VS = ±7.5V | ● ● ● | ±1.1 ±3.1 ±5.0 | V V V |
| Total Harmonic Distortion | VS = ±5V, Input = 1VRMS at 1kHz VS = ±7.5V, Input = 3VRMS at 1kHz |  | 0.015 0.03 | % % |
| Wideband Noise | VS = ±5V, Input = GND 1Hz – 1.99MHz VS = ±7.5V, Input = GND 1Hz – 1.99MHz |  | 80 90 | µVRMS µVRMS |

10642fa

ELECTRICAL CHARACTERISTICS	The ● denotes the specifications which apply over the full operating temperature range, otherwise specifications are at TA = 25°C. VS = ±7.5V,  100:1, fCLK = 2MHz, R1 = 10k, TTL clock input level unless otherwise specified.

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Output DC Offset (Note 2) Output DC Offset TempCo | VS = ±7.5V VS = ±5V |  | ±30 ±125 ±90 | mV µV/°C |
| Input Impedance |  |  | 10 20 | kΩ |
| Output Impedance | fOUT = 10kHz |  | 2 | Ω |
| Output Short-Circuit Current | Source/Sink |  | 3/1 | mA |
| Clock Feedthrough |  |  | 200 | µVRMS |
| Maximum Clock Frequency | 50% Duty Cycle, VS = ±5V 50% Duty Cycle, TA = 25°C, VS = ±7.5V |  | 5 7 | MHz MHz |
| Power Supply Current | VS = ±2.37V, f CLK = 1MHz VS = ±5V, f CLK = 1MHz VS = ±7.5V, fCLK = 1MHz | ● ● ● | 11 22 14 23 26 17 28 32 | mA mA mA mA mA |
| Power Supply Voltage Range |  | ● | ± 2.37 ±8 | V |

Note 1: Absolute Maximum Ratings are those values beyond which the life of a device may be impaired.

Note 2: For tighter specifications  please contact LTC Marketing.

TYPICAL PERFOR	A	CE CHARACTERISTICS

Amplitude Response	Phase Response	Group Delay vs Frequency

15

0

–15

VOUT/VIN (dB)

–30

–45

–60

–75

–90

–105

fCLK = 2MHz f–3dB = 40kHz

fCLK = 5MHz f–3dB = 100kHz

VS = ±7.5V; 50:1

TA = 25°C

fCLK = 7MHz f–3dB = 140kHz

45

0

–45

–90

PHASE (DEG)

–135

–180

–225

–270

–315

–360

–405

| –450 | VS = ±7.5V TA = 25°C fCLK = 1MHz 100:1 f–3dB = 10kHz |
| --- | --- |

220

200

180

GROUP DELAY (µs)

160

140

120

100

80

60

40

20

| 0 VS = ±7.5V TA = 25°C fCLK = 1MHz 100:1 f–3dB = 10kHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

10k

100k	1M

fIN (Hz)

0   1    2   3

4    5    6    7

fIN (kHz)

8   9   10  11

0    1    2   3   4    5   6   7    8    9  10  11

fIN (kHz)

1064 G01

1064 G02

1064 G03

10642fa

TYPICAL PERFOR	A	CE CHARACTERISTICS

Phase vs f –3dB Frequency	Phase Matching 	Noise Spectral Density

30

0

–30

–60

–90

fCLK = 500kHz f–3dB = 10kHz

fCLK = 2MHz f–3dB = 40kHz

2.4

2.0

1.6

1600

1400

OUTPUT NOISE (nV/√Hz)

1200 (1060)

1000

fCLK = 500kHz

100:1, f–3dB = 5kHz

PHASE (DEG)

–120

–150

–180

fCLK = 1MHz f–3dB = 20kHz

1.2

PHASE MATCH (±DEG)

0.8

800

600 (530)

400

fCLK = 2MHz

100:1, f–3dB = 20kHz

–210

0.4

(335)

fCLK

= 5MHz

–240

–270

VS = ±7.5V

50:1

| 0 VS = ±7.5V, fCLK = 1MHz, f–3dB = 20kHz, 50:1 50 UNIT SAMPLE (TA = 25°C TO 125°C) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

200

0

100:1, f–3dB = 50kHz

100

1k	10k

FREQUENCY (Hz)

1064 G04

0	4	8	12	16

FREQUENCY (kHz)

20	24

1064-2 G05

0.1k

1k	10k	100k

FREQUENCY (Hz)

1064-2 G06

0.1

Harmonic Distortion vs Frequency	Harmonic Distortion vs Amplitude

1

fCLK = 1MHz, f–3dB = 20kHz, 50:1

HARMONIC DISTORTION (%)

0.01

±5V, 1VRMS INPUT

±7.5V, 3VRMS INPUT

0.1

±2.5V

±5V

±7V

HARMONIC DISTORTION (%)

0.01

0.001

0.001

fCLK = 1MHz, f–3dB = 20kHz

50:1, 1kHz INPUT

1k

FREQUENCY (Hz)

10k	20k

1064-2 G07

0.5	1.0

AMPLITUDE (VRMS)

5.0

1064-2 G08

Power Suppy vs Current

Amplitude Response with Pin 10 at Ground

44

POWER SUPPLY CURRENT (mA)

40

36

32

28

24	TA = –55°C

20	TA = 25°C

16	TA = 125°C

12

8

4

0

0   2   4   6   8  10 12 14 16 18 20 22 24

POWER SUPPLY VOLTAGE (V)

1064 G09

0

–15

VOUT/VIN (dB)

–30

–45

–60

–75

–90

–105

1

PIN 10 AT GROUND

fCLK =1MHz

10	100 fIN (kHz)

1064-2 G10

10642fa

TYPICAL PERFOR	A	CE CHARACTERISTICS

Table 1. Gain/Delay, f –3dB = 1kHz,

LTC1064-2 Typical Response VS = ±5V, TA = 25°C, fCLK = 50kHz, Ratio = Pin 10 at V + (fltr 50:1)

Table 3. Gain/Delay, f –3dB = 1kHz,

| LTC1064-2 Typical Response VS = ±5V, TA = 25°C, fCLK = 100kHz, Ratio = Pin 10 at V – (fltr 100:1) FREQUENCY (kHz) | GAIN (dB) | DELAY (ms) |
| --- | --- | --- |
| 0.200 | – 0.213 | 0.821 |
| 0.300 | – 0.240 | 0.837 |
| 0.400 | – 0.260 | 0.858 |
| 0.500 | – 0.280 | 0.893 |
| 0.600 | – 0.310 | 0.947 |
| 0.700 | – 0.370 | 1.034 |
| 0.800 | – 0.530 | 1.172 |
| 0.900 | – 1.200 | 1.325 |
| 1.000 | – 3.370 | 1.346 |
| 1.100 | – 7.500 | 1.158 |
| 1.200 | – 12.640 | 0.899 |

| FREQUENCY (kHz) | GAIN (dB) | DELAY (ms) |
| --- | --- | --- |
| 0.200 | – 0.247 | 0.857 |
| 0.300 | – 0.270 | 0.872 |
| 0.400 | – 0.290 | 0.893 |
| 0.500 | – 0.300 | 0.929 |
| 0.600 | – 0.320 | 0.983 |
| 0.700 | – 0.370 | 1.071 |
| 0.800 | – 0.520 | 1.210 |
| 0.900 | – 1.200 | 1.364 |
| 1.000 | – 3.380 | 1.381 |
| 1.100 | – 7.530 | 1.192 |
| 1.200 | – 12.670 | 0.935 |

Table 2. Gain, f –3dB = 1kHz,

LTC1064-2 Typical Response VS = ±5V, TA = 25°C, fCLK = 50kHz, Ratio = Pin 10 at V + (fltr 50:1)

Table 4. Gain, f–3dB = 1kHz,

| LTC1064-2 Typical Response VS = ±5V, TA = 25°C, fCLK = 100kHz, Ratio = Pin 10 at V – (fltr 100:1) FREQUENCY (kHz) | GAIN (dB) |
| --- | --- |
| 0.500 | – 0.279 |
| 1.000 | – 3.370 |
| 1.500 | – 27.500 |
| 2.000 | – 47.200 |
| 2.500 | – 62.300 |
| 3.000 | – 75.130 |
| 3.500 | – 86.090 |
| 4.000 | – 95.210 |
| 4.500 | – 103.030 |
| 5.000 | – 108.690 |
| 5.500 | – 114.830 |
| 6.000 | – 120.540 |
| 6.500 | – 114.750 |
| 7.000 | – 116.430 |
| 7.500 | – 120.790 |
| 8.000 | – 121.290 |
| 8.500 | – 119.970 |
| 9.000 | – 120.020 |
| 9.500 | – 125.170 |

| FREQUENCY (kHz) | GAIN (dB) |
| --- | --- |
| 0.500 | – 0.298 |
| 1.000 | – 3.380 |
| 1.500 | – 27.500 |
| 2.000 | – 47.200 |
| 2.500 | – 63.300 |
| 3.000 | – 75.190 |
| 3.500 | – 86.100 |
| 4.000 | – 95.310 |
| 4.500 | – 104.240 |
| 5.000 | – 109.650 |
| 5.500 | – 121.930 |
| 6.000 | – 123.920 |
| 6.500 | – 114.150 |
| 7.000 | – 116.990 |
| 7.500 | – 120.070 |
| 8.000 | – 113.470 |
| 8.500 | – 130.090 |
| 9.000 | – 114.770 |
| 9.500 | – 117.760 |

10642fa

TYPICAL PERFOR	A	CE CHARACTERISTICS

Table 5. Gain, f –3dB = 20kHz,

LTC1064-2 Typical Response VS = ±7.5V,  TA = 25°C, fCLK = 1MHz, Ratio = Pin 10 at V + (fltr 50:1)

Table 6. Gain, f–3dB = 140kHz,

| LTC1064-2 Typical Response VS = ±7.5V, TA = 25°C, fCLK = 7MHz, Ratio = Pin 10 at V+ (fltr 50:1) FREQUENCY (kHz) | GAIN (dB) |
| --- | --- |
| 50.00 | – 0.238 |
| 60.00 | – 0.140 |
| 70.00 | 0.050 |
| 80.00 | 0.350 |
| 90.00 | 0.810 |
| 100.00 | 1.450 |
| 110.00 | 2.110 |
| 120.00 | 1.830 |
| 130.00 | – 0.700 |
| 140.00 | – 4.840 |
| 150.00 | – 9.350 |
| 160.00 | – 13.690 |
| 170.00 | – 17.760 |
| 180.00 | – 21.600 |
| 190.00 | – 25.200 |
| 200.00 | – 28.500 |
| 210.00 | – 31.800 |
| 220.00 | – 34.800 |
| 230.00 | – 37.700 |
| 240.00 | – 40.500 |
| 250.00 | – 43.200 |
| 260.00 | – 45.700 |
| 270.00 | – 48.200 |
| 280.00 | – 50.500 |
| 290.00 | – 52.700 |
| 300.00 | – 54.900 |

| FREQUENCY (kHz) | GAIN (dB) |
| --- | --- |
| 10.000 | – 0.308 |
| 20.000 | – 3.350 |
| 30.000 | – 27.400 |
| 40.000 | – 47.100 |
| 50.000 | – 62.300 |
| 60.000 | – 74.890 |
| 70.000 | – 85.430 |
| 80.000 | – 95.070 |
| 90.000 | – 103.150 |
| 100.000 | – 108.700 |
| 110.000 | – 107.520 |
| 120.000 | – 108.030 |
| 130.000 | – 104.990 |
| 140.000 | – 106.090 |
| 150.000 | – 105.320 |

Table 7. Gain Non-Butterworth Response (Pin 10 to GND), LTC1064-2 Typical Response VS = ±5V, TA = 25°C,

fCLK = 100kHz

| FREQUENCY (kHz) | GAIN (dB) |
| --- | --- |
| 0.500 | – 0.012 |
| 1.000 | 1.240 |
| 1.500 | – 14.690 |
| 2.000 | – 28.600 |
| 2.500 | – 41.100 |
| 3.000 | – 52.500 |
| 3.500 | – 62.800 |
| 4.000 | – 71.500 |
| 4.500 | – 79.370 |
| 5.000 | – 86.730 |
| 5.500 | – 93.340 |
| 6.000 | – 99.350 |
| 6.500 | – 105.270 |
| 7.000 | – 113.270 |
| 7.500 | – 114.600 |
| 8.000 | – 114.010 |
| 8.500 | – 122.810 |
| 9.000 | – 122.980 |
| 9.500 | – 119.450 |

10642fa

PI 	FU  CTIO  S

(Pin Numbers Refer to the 14-Pin Package)

NC (Pins 1, 6, 8 and 13): The “no connection” pins should be preferably  grounded.  These pins  are not internally connected.

VIN, VOUT (Pins 2, 9): The input Pin 2 is connected to an

18k resistor tied to the inverting input of an op amp. Pin

2 is protected  against static  discharge.  The device’s output, Pin 9, is the output of an op amp which can typically source/sink 3mA/1mA. Although the internal op amps are unity gain stable, driving long coax cables is not recommended.

When testing  the device for noise and distortion,  the output, Pin 9, should be buffered. (Figure 1) The op amp power  supply  wire (or trace)  should  be  connected directly to the power source. To eliminate switching transients from filter output, buffer filter output with a third order lowpass (Figure 5).

AGND (Pins 3, 5): For dual supply operation these pins should be connected to a ground plane. For single supply operation both pins should be tied to one half supply, (Figure 3).

V +, V – (Pins 4, 12): Should be bypassed with a 0.1µF capacitor to an adequate AGND. Low noise, nonswitching power supplies are recommended. To avoid latchup when the power  supplies exhibit  high  turn-on  transients,  a

1N5817 Schottky diode should be added from the V + and

V – pins to ground (Figures 1, 2 and 3).

RIN A, OUT C (Pins 7, 14): A very short connection between Pin 7 and Pin 14 is recommended. This connection should be preferably done under the IC package. In a breadboard use a one inch, or less, shielded coaxial cable: the shield should be grounded. In a PC board,  use a one inch trace or

less; surround the trace by a ground plane.

50/100 (Pin 10): The DC level at this pin determines the ratio of clock frequency to the –3dB frequency of the filter. The ratio is 50:1 when Pin 10 is at V + and 100:1 when Pin 10 is at V –. This pin should be bypassed with a 0.1µF capacitor to analog ground when it’s connected to V – or V + (Figure 1). See Tables 1 through 7 for typical gain and delay responses for the two ratios.

fCLK (Pin 11): For ±5V supplies the logic threshold  level is

1.4V. For ±8V and 0V to 5V supplies the logic threshold

levels are 2.2V and 3V respectively. The logic threshold levels vary ±100mV  over  the full military  temperature range. The recommended duty cycle of the input clock is

50% although for clock frequencies below 500kHz the clock “on” time can be as low as 200ns. The maximum clock frequency for ± 5V supplies is 4MHz. For ±7V sup- plies and above, the maximum clock frequency is 7MHz. Do not allow the clock levels to exceed the power supplies. For single supply operation ≥6V use level shifting at Pin 11 with T2L levels (Figure 4).

10642fa

TYPICAL APPLICATIO   S

1	OUT C   14

POWER SOURCE

1	14

NC

2

VIN

3

13

NC

LTC1064-2	12

V+ 	V–

NC

2

VIN

3

OUT C

13

NC

LTC1064-2	12

4

0.1µF  5

6

AGND V+ AGND

NC

V–

11 fCLK

10

50/100

9

VOUT

V+/V–

20k

0.1µF

20k

–

0.1µF

8

V+

1N5817

0.1µF

AGND

4 V+

5

AGND

6

NC

V–

11 fCLK

10

50/100

9

VOUT

1N5817

V–

0.1µF

7 RIN A

8	0.1µF NC

LT1006

+

VOUT

7	8

RIN A 	NC

RECOMMENDED OP AMPS: LT1022, LT318, LT1056

4	750Ω

0.1µF

1064-2 F01

Figure 2. Using Schottky Diodes to Protect the

IC from Transient Supply Reversal.

1064-2 F02

Figure 1. Buffering the Filter Output. The Buffer Op Amp

Should Not Share the LTC1064-2 Power Lines.

1

NC

2

VIN

OUT C  14

NC 13

3	LTC1064-2	12

–

AGND	V

4	11

V+= 15V

0.1µF

5k

V+/2

V+

5

AGND

6

NC

fCLK

50/100 10

9

VOUT

0V TO 10V

0.1µF	5k

7

RIN A

NC  8

1064-2  F03

Figure 3. Single Supply Operation. If Fast Power Up or Down Transients are Expected, Use a 1N5817  Schottky Diode Between Pin 4 and Pin 5. For V+ = 5V, Derive the Mid-Supply Voltage

with a 7.5k Resistor and an LT1004 2.5V Reference.

10642fa

TYPICAL APPLICATIO   S

1

NC

2

VIN

OUT C   14

NC   13	V+

3	LTC1064-2	12

–

AGND	V

2.2k

V+ 	V+

4

5

11 fCLK

10

5k 1µF

T2L LEVEL

5k	AGND

6

NC

50/100

9

VOUT

RATIO

0.1µF	5k

7

RIN A

NC   8

1064-2  F04

Figure 4. Level Shifting the Input T2L Clock for Single Supply Operation ≥ 6V.

1

NC

2

VIN

3

14

OUT C

13

NC

LTC1064-2	12

V– 	100pF

20k

VOUT

V+

0.1µF

AGND

4  V+

5

AGND

6

NC

7

RIN A

V–

fCLK

50/100

VOUT

NC

11

10 V+/GND/V–

9

8

0.1µF

10k	10k

220pF

15V

–

LT1056

+

50Ω

0.027µF

–15V

1064-2 F05

Figure 5. Adding an Output Buffer-Filter  to Eliminate  Any Clock

Feedthrough. Passband ±0.1dB to 50kHz, – 3dB at 94kHz.

10642fa

PACKAGE DESCRIPTIO

J Package

14-Lead CERDIP (Narrow 0.300, Hermetic)

(LTC DWG # 05-08-1110)

.005 (0.127)

MIN	14

13	12

.785 (19.939)

MAX

11	10	9	8

.025 (0.635)

RAD TYP

.220 – .310 (5.588 – 7.874)

.300 BSC (7.62 BSC)

1	2	3	4	5	6	7

.200 (5.080)

MAX

.015 – .060 (0.381 – 1.524)

.008 – .018 (0.203 – 0.457)

0° – 15°

.045 – .065 (1.143 – 1.651)

NOTE: LEAD DIMENSIONS APPLY TO SOLDER DIP/PLATE OR TIN PLATE LEADS

.014 – .026 (0.360 – 0.660)

.100  (2.54) BSC

.125 (3.175)

MIN

J14 0801

OBSOLETE PACKAGE

10642fa

PACKAGE DESCRIPTIO

N Package

14-Lead PDIP (Narrow 0.300)

(LTC DWG # 05-08-1510)

14	13

.770* (19.558) MAX

12	11	10	9	8

.255 ± .015* (6.477 ± 0.381)

1	2	3

4	5	6	7

.300 – .325 (7.620 – 8.255)

.130 ± .005 (3.302 ± 0.127)

.045 – .065 (1.143 – 1.651)

.008 – .015 	 (0.203 – 0.381)

.020 (0.508) MIN

.065 (1.651)

TYP

NOTE:

+.035

.325 –.015

+0.889

8.255 –0.381

(	)

INCHES

.120 (3.048)

MIN

.005 (0.125) MIN

.100 (2.54) BSC

.018 ± .003 (0.457 ± 0.076)

1. DIMENSIONS ARE MILLIMETERS

*THESE DIMENSIONS DO NOT INCLUDE MOLD FLASH OR PROTRUSIONS. MOLD FLASH OR PROTRUSIONS  SHALL NOT EXCEED .010 INCH (0.254mm)

N14 1002

Information furnished by Linear Technology Corporation is believed to be accurate and reliable. However, no responsibility  is assumed for its use. Linear Technology Corporation makes no represen- tation that the interconnection of its circuits as described herein will not infringe on existing patent rights.

10642fa

11

PACKAGE DESCRIPTIO

SW Package

16-Lead Plastic Small Outline (Wide .300 Inch)

(Reference LTC DWG # 05-08-1620)

.030 ±.005

TYP

N

.050 BSC  .045 ±.005

.398 – .413 	 (10.109 – 10.490) NOTE 4

16   15   14   13   12   11   10	9

N

.420

MIN

.325 ±.005

NOTE 3

.394 – .419 	 (10.007 – 10.643)

1	2	3	N/2

N/2

RECOMMENDED SOLDER PAD LAYOUT

1	2	3	4	5	6	7	8

.005 	 (0.127) RAD MIN

.291 – .299 	 (7.391 – 7.595) NOTE 4

.010 – .029    × 45°

(0.254 – 0.737)

0° – 8° TYP

.093 – .104 	 (2.362 – 2.642)

.037 – .045 	 (0.940 – 1.143)

.009 – .013 	 (0.229 – 0.330)

NOTE 3

.016 – .050

.050  (1.270) BSC

.014 – .019

.004 – .012 	 (0.102 – 0.305)

NOTE:

(0.406 – 1.270)

INCHES

(0.356 – 0.482)

TYP

1. DIMENSIONS IN (MILLIMETERS)

2. DRAWING NOT TO SCALE

3. PIN 1 IDENT, NOTCH ON TOP AND CAVITIES ON THE BOTTOM OF PACKAGES ARE THE MANUFACTURING OPTIONS. THE PART MAY BE SUPPLIED WITH OR WITHOUT ANY OF THE OPTIONS

4. THESE DIMENSIONS DO NOT INCLUDE MOLD FLASH OR PROTRUSIONS. MOLD FLASH OR PROTRUSIONS  SHALL NOT EXCEED .006" (0.15mm)

S16 (WIDE) 0502

Linear Technology Corporation

12

1630 McCarthy Blvd., Milpitas, CA 95035-7417

10642fa

LW/TP 1202 1K REV A • PRINTED IN USA

(408) 432-1900

●   FAX: (408)  434-0507

●      www.linear.com	 LINEAR TECHNOLOGY CORPORATION 1990