200mA Buck-Boost Synchronous DC/DC Converters

FEATURES 	DESCRIPTIO

■     Regulated Output with Input Above, Below or Equal to the Output

■     Single Inductor

■     Up to 90% Efﬁciency

■     VIN Range: 1.8V to 5.5V

■     200mA at 3.3VOUT from 3.6V Input

■     125mA at 3VOUT from 2.5V Input

■     Fixed VOUT Versions (TSOT, DFN): 3.3V, 3V

■     Adjustable VOUT Version (DFN): 2V to 5V

■     Burst Mode®  Operation, No External Compensation

■     Ultra Low Quiescent Current: 16μA, Shutdown

Current <1μA

■     Only 3 External Components Required

(Fixed Voltage Versions)

■     Short-Circuit Protection

■     Output Disconnect in Shutdown

■     Available in 6-Pin ThinSOT and 3mm × 3mm DFN Packages

APPLICATIO 	S

■     Handheld Instruments

■     MP3 Players

■     Handheld computers

■     PDA/GPS

The LTC®3531/LTC3531-3.3/LTC3531-3 are synchronous buck-boost  DC/DC converters that operate from input voltages above, below or equal to the output voltage. The topology incorporated in the ICs provides a continuous transfer through all operating modes, making the product ideal for single cell Li-Ion and multicell alkaline or nickel applications. The converters operate in Burst Mode, mini- mizing solution footprint  and component count as well as providing  high conversion  efﬁciency over a wide range of load currents.

The devices include two 0.5Ω N-channel MOSFET switches and two P-channel switches (0.5Ω,  0.8Ω).  Quiescent current is typically 16μA, making the parts ideal for bat- tery power applications. Other features include a <1μA shutdown current, current limiting,  thermal shutdown and output disconnect. The parts are offered in a 6-pin ThinSOTTM package for ﬁxed voltage versions or a 3mm ×

3mm DFN package for ﬁxed and adjustable versions.

, LT, LTC and LTM are registered trademarks of Linear Technology Corporation. Burst Mode is a registered trademark of Linear Technology Corporation.

ThinSOT is a trademark  of Linear Technology Corporation.

All other trademarks are the property of their respective owners. Protected by U.S. Patents including 6166527.

TYPICAL APPLICATIO

100

Efﬁciency vs VIN

10H	95

VIN

3.1V TO

4.2V

SW1	SW2

VIN 	VOUT

LTC3531-3.3

VOUT

3.3V

160mA

90	BOOST MODE

85

EFFICIENCY (%)

80

4SW

BUCK MODE

Li-Ion

2.2F	10F

SHDN	GND

75	MODE

70

ON  OFF

3531 TA01a

65

3.3VOUT AT 100mA

60

1.5

2	2.5	3

3.5	4

4.5	5

5.5

INPUT VOLTAGE (V)

3531 TA01b

3531fb

ABSOLUTE 	AXI	U	RATI 	GS

(Note 1)

VIN, VOUT, SW1, SW2, SHDN Voltage ...........–0.3 to 6V SW1, SW2 Voltage, <100ns Pulse ..................–0.3 to 7V Operating Temperature Range (Notes 2, 3) –40°C to 85°C

| Storage Temperature Range................... –65°C to 125°C Lead Temperature (TS6, Soldering, 10 sec) .......... 300°C |  |  |
| --- | --- | --- |
| 1 | 9 | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

PACKAGE/ORDER  I

FOR	ATIO

| TOP VIEW SW2 1 6 SW1 GND, PGND 2 5 VIN VOUT 3 4 SHDN S6 PACKAGE 6-LEAD PLASTIC TSOT-23 TJMAX = 125°C, θJA = 102°C/W |  | TOP VIEW SW1 PGND VIN SW2 GND VOUT SHDN FB* DD PACKAGE 8-LEAD (3mm  3mm) PLASTIC DFN TJMAX = 125°C, θJA = 43°C/W EXPOSED PAD IS GND (PIN 9), MUST BE SOLDERED TO PCB *NC FOR LTC3531-3.3V, LTC3531-3.0V. |  |
| --- | --- | --- | --- |
| ORDER PART NUMBER | S6 PART MARKING | ORDER PART NUMBER | DD PART MARKING |
| LTC3531ES6-3.3 LTC3531ES6-3 | LTBWM LTCBK | LTC3531EDD LTC3531EDD-3.3 LTC3531EDD-3 | LBVC LBWH LCBV |
| Order Options Tape and Reel: Add #TR Lead Free: Add #PBF Lead Free Tape and Reel: Add #TRPBF Lead Free Part Marking: http://www.linear.com/leadfree/ |  |  |  |

Consult LTC Marketing for parts speciﬁed with wider operating temperature ranges.

ELECTRICAL CHARACTERISTICS The ● denotes the speciﬁcations which apply over the full operating temperature range, otherwise speciﬁcations are at TA = 25°C. VIN = 3.6V VOUT = 3.3V unless otherwise noted.

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |  |
| --- | --- | --- | --- | --- | --- |
| VIN |  |  |  |  |  |
| Minimum Startup Voltage |  | ● | 1.65 1.8 | V |  |
| VOUT Regulation |  |  |  |  |  |
| Output Voltage (3.3V Version) | No Load | ● | 3.25 3.32 3.39 | V |  |
| Output Voltage (3V Version) | No Load | ● | 2.95 3.02 3.09 | V |  |
| FB Voltage (Adj Version) | No Load | ● | 1.20 1.225 1.25 | V |  |
| FB Input Current (Adj Version) | VFB = 1.225V |  | 1 50 | nA |  |
| Operating Current |  |  |  |  |  |
| Quiescent Current in Sleep: VIN VOUT | VIN = 5V, VOUT = 3.6V, FB = 1.3V VOUT = 3.6V |  | 16 30 6 10 | μA μA |  |
| Shutdown Current VIN | SHDN = 0V, VOUT = 0V |  | 1 | μA |  |
| Switch Performance |  |  |  |  |  |
| NMOS Switch Leakage | Switches B and C |  | 0.2 2 | μA |  |
| PMOS Switch Leakage | Switches A and D |  | 0.2 2 | μA |  |
| NMOS B, C RDSON | VIN = 5V |  | 0.5 | Ω |  |
| PMOS A RDSON | VIN = 5V |  | 0.5 | Ω |  |

3531fb

ELECTRICAL CHARACTERISTICS The ● denotes the speciﬁcations which apply over the full operating temperature range, otherwise speciﬁcations are at TA = 25°C. VIN = 3.6V VOUT = 3.3V unless otherwise noted.

| PARAMETER |  | CONDITIONS |  | MIN TYP MAX | UNITS |  |
| --- | --- | --- | --- | --- | --- | --- |
| PMOS D RDSON (3.3V Version or Adjustable Version set to 3.3V) |  | VOUT = 3.1V |  | 0.8 | Ω |  |
| PMOS D RDSON (3V Version) |  | VOUT = 2.8V |  | 0.9 | Ω |  |
| Peak Current Limit |  | L = 10μH, VIN = 5V |  | 295 365 460 | mA |  |
| SHDN |  |  |  |  |  |  |
| SHDN Input Threshold |  |  |  | 0.4 1 1.4 | V |  |
| SHDN Hysteresis |  |  |  | 60 | mV |  |
| SHDN Leakage Current |  | VSHDN |  | 0.01 1 | μA |  |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime.

Note 2: This IC includes overtemperature protection that is intended

to protect the device during momentary overload conditions. Junction temperature will exceed 125°C when overtemperature protection is active.

Continuous operation above the speciﬁed maximum operating junction temperature may result in device degradation or failure.

Note 3: The LTC3531 is guaranteed to meet performance speciﬁcations from 0°C to 70°C. Speciﬁcations over the – 40°C to 85°C operating temperature range are assured by design, characterization and correlation with statistical process controls.

TYPICAL PERFOR

Maximum IOUT vs VIN

A	CE CHARACTERISTICS

TA = 25°C unless otherwise speciﬁed.

(3.3V Version)	IPEAK, IVALLEY, IZERO vs VIN 	Sleep Currents

| 350 L = 10H VOUT = 3.3V |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

MAXIMUM IOUT (mA)

250

200

150

100

50

0

450

350

CURRENT (mA)

300

250

200

100

50

0

L = 10H

IZERO

20

IPEAK	16

14

12

10

CURRENT (A)

IVALLEY	8

6

4

2

0

IVIN

IVOUT

1.5

2	2.5

3	3.5	4

VIN (V)

4.5	5

5.5

1.5	2

2.5

3	3.5	4

VIN (V)

4.5	5

5.5

1.5    2

2.5

3    3.5	4

VIN (V)

4.5	5	5.5

3531 G01

3531 G02

3531 G03

180

160

140

120

IIN (mA)

100

80

60

40

20

0

| IIN Short Circuit vs VIN |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

100

90

80

70

60

50

40

VRIPPLE PEAK-PEAK (mV)

30

20

10

0

VOUT Ripple vs COUT

(3.3V Version)

10F

22F

50mA LOAD

3.40

3.35

VOUT (V)

3.30

3.25

3.20

3.15

Load Regulation vs COUT

| (3.3V Version) |  |  |  |
| --- | --- | --- | --- |
|  | 10F |  |  |
| 22F |  |  |  |
|  | 47F |  |  |
| VIN = 3.6V |  |  |  |

1.5	2

2.5	3	3.5	4	4.5	5

VIN (V)

5.5

1.5    2

2.5

3    3.5	4

VIN (V)

4.5	5	5.5

0.1

1	10	100

LOAD CURRENT (mA)

1000

3531 G04

3531 G05

3521 G06

3531fb

TYPICAL PERFOR	A	CE CHARACTERISTICS

TA = 25°C unless otherwise speciﬁed.

Start-Up into Resistive Load

| L = 10μH |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

30

28

26

24

RMIN ()

22

20

18

1000

100

FREQUENCY (kHz)

10

Burst Frequency vs Load

(3.3V Version)

L = 10H COUT = 10F

10mA

VOUT (V)

5mA

50mA

| 3.350 VIN = 3.6V ILOAD = 10mA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

3.325

3.300

VOUT Regulation  vs Temperature

(3.3V Version)

16

14

12

10

1.5    2

2.5

3    3.5	4

4.5	5	5.5

1

0.1

1.5

1mA

0.5mA

2	2.5	3

3.5    4

4.5    5

5.5

3.275

3.250

–40

–20	0	20	40

60	80    100

VIN (V)

3531 G07

VIN (V)

3531 G08

TEMPERATURE (C)

3531 G09

| 1000 VIN = 5V VOUT = 3.3V |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | D (PMOS) |  |  |
|  |  |  |  | A (PMOS) |  |  |
|  |  |  | B,C (NMOS) |  |  |  |

900

RDSON (m)

800

700

600

500

400

Switch On Resistances 	IPEAK, IVALLEY vs Temperature

| 400 |  |  | IPEAK |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y |  |  |  |
|  |  | IVALLE |  |  |  |  |  |
|  |  |  |  |  |  | 2.5VIN 3.3VOUT 10H |  |
|  |  |  | IZERO |  |  |  |  |

350

300

250

200

CURRENT (mA)

150

100

50

0

| 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OPERATING |  |  |  |
|  | SHUTDOWN |  |  |  |  |  |

0.9

0.8

VOLTAGE (V)

0.7

0.6

0.5

SHDN Pin Threshold and

Hysteresis

–40

–20	0	20	40

60	80    100

–40

–20	0

20	40

60	80    100

–40

–20	0	20	40

60	80    100

TEMPERATURE (C)

TEMPERATURE (C)

TEMPERATURE (C)

3531 G10

3531 G11

3531 G12

3531fb

TYPICAL PERFOR	A	CE CHARACTERISTICS

TA = 25°C unless otherwise speciﬁed.

Buck Mode at 5VIN,

3.3VOUT 200mA

Buck Mode at 5VIN,

3.3VOUT 100mA

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

L = 10μH COUT = 22μF

5μs/DIV

3531 G13

L = 10μH COUT = 22μF

5μs/DIV

3531 G14

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

Buck Mode Waveforms at 5VIN,

3.3VOUT 20mA

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

4 Switch Mode Waveforms at

3.6VIN, 3.3VOUT 200mA

| L = 10μH | 5μs/DIV | 3531 G15 | L = 10μH | 5μs/DIV | 3531 G16 |
| --- | --- | --- | --- | --- | --- |
| COUT = 22μF |  |  | COUT = 22μF |  |  |

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

4 Switch Mode Waveforms at

3.6VIN, 3.3VOUT 100mA

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

4 Switch Mode Waveforms at

3.6VIN, 3.3VOUT 20mA

SW1

SW2

VOUT (AC) IL

L = 10μH COUT = 22μF

5μs/DIV

3531 G17

L = 10μH COUT = 22μF

5μs/DIV

3531 G18

3531fb

TYPICAL PERFOR	A	CE CHARACTERISTICS

TA = 25°C unless otherwise speciﬁed.

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

Boost Mode Waveforms at

2.5VIN, 3.3VOUT 100mA

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

Boost Mode Waveforms at

2.5VIN, 3.3VOUT 20mA

L = 10μH COUT = 22μF

5μs/DIV

3531 G19

L = 10μH COUT = 22μF

5μs/DIV

3531 G20

SW1

5V/DIV

SW2

5V/DIV

VOUT (AC)

50mV/DIV

IL

200mA/DIV

Shorted Output

200mA/DIV

VOUT

1V/DIV

IL

200mA/DIV

Start-Up into 50mA Load at

3.3 VOUT (Shows Start, Buck, Then 4sw Modes)

L = 10μH COUT = 22μF VIN = 5

1μs/DIV

3531 G21

25μs/DIV

3531 G22

3.6VIN, 3.3VOUT Load Step

200mA to 80mA

SW1 and SW2 Close-Up in Four

Switch Mode

VOUT (AC)

20mV/DIV

20mA LOAD

100mA LOAD

20mA LOAD

SW1

2V/DIV

IL

200mA/DIV

SW2

2V/DIV

COUT = 22μF

25μs/DIV

3531 G23

VIN = 3.6V VOUT = 3.3V

200ns/DIV

3531 G24

3531fb

PI 	FU	CTIO	S

ThinSOT/DFN Packages

SW2 (Pin 1/Pin 7): Buck-Boost Switch Pin Where Internal Switches  C and D are Connected. An optional Schottky diode can be connected from SW2 to VOUT for a moderate efﬁciency improvement. Minimize trace length to keep EMI down.

GND (Pin 2/Pin 3): Signal Ground for the IC.

VIN (Pin 5/Pin 2): Input Supply Pin for the Buck-Boost Converter. A minimum 2.2μF Ceramic Capacitor should be placed between VIN and GND.

FB (NA/Pin 5): Feedback Pin for the Adjustable Version. Connect the resistor divider tap here. The output voltage can be adjusted from 2V to 5V.

PGND (Pin 2/Pin 8): Power Ground for the IC. (Shared on ThinSOT version)

⎛

VOUT  1.225 1

 R2 ⎞

R1

VOUT (Pin 3/Pin 6): Output of the Buck-Boost Synchronous Rectiﬁer. A ﬁlter capacitor is placed from VOUT to GND. A ceramic bypass capacitor is recommended as close to the VOUT and GND pins as possible.

SHDN (Pin 4/Pin 4): External Shutdown Pin. An applied voltage of < 0.4V shuts down the converter. A voltage above >1.4V will enable the converter.

SW1 (Pin 6/Pin 1): Buck-Boost Switch Pin Where Internal Switches A and B are Connected. Connect the inductor from SW1 to SW2.

Exposed Pad (Pin 9, DFN): Solder to PCB ground  for optimal thermal performance.

3531fb

BLOCK DIAGRA

SW1	SW2

VIN

SW A

SW D

VOUT

SW B

GATE DRIVERS AND ANTICROSS CONDUCTION

SW C

IZERO/IVALLEY

DETECT

365mA

VIN

PEAK CURRENT LIMIT

STATE MACHINE AND LOGIC

VBEST

BUCK, 4SW, BOOST

VBEST

AND

MODE DETECT

VIN

VOUT

R2

SHDN

1.65V

UVLO

SHUTDOWN

THERMAL SHUTDOWN

VOUT

COMPARATOR

1.225V VREF

R1

FB

NOT BONDED FOR 3.3V AND

3V VERSIONS

PGND

GND

INTERNAL R1 AND R2

3531 BD

COMBINED

FOR TSOT

DISABLED

FOR ADJ VERSION

3531fb

OPERATIO

The LTC3531,  LTC3531-3.3  and  LTC3531-3  synchro- nous buck-boost converters utilize a Burst Mode control technique to achieve high efﬁciency over a wide dynamic range of load currents.  A 2% accurate comparator  is used to monitor the output voltage. If VOUT is above its programmed reference threshold no switching occurs and only quiescent current is drawn from the power source (sleep  mode).  When VOUT drops  below  the reference threshold the IC “wakes up”, switching commences, and the output capacitor is charged. The value of the output capacitor, the load current, and the comparator hysteresis (~1%) determines the number of current pulses required to pump-up the output capacitor before the part returns to sleep.

In order to determine the best operating mode for the converter, the LTC3531 contains a second comparator that monitors the relative voltage difference between VIN and VOUT. Input and output voltages in the various modes as well as typical inductor currents are shown in Figure 1. Regions of the current waveforms where switches A and D are on provide the highest efﬁciency since energy is transferred directly from the input source to the output.

Boost Mode

If VIN is ~400mV below VOUT, the LTC3531 operates in boost or step-up mode. Referring to Figure 1 (left side) when VOUT falls below its regulation voltage, switches A and C are turned on (VIN is applied across the inductor) and current is ramped until IPEAK is detected. When this

occurs, C is turned off, D is turned on and current is deliv- ered to the output capacitor (VIN – VOUT is applied across the inductor). Inductor current falls when D is on, until an IVALLEY is detected. Terminating at IVALLEY, rather than IZERO, results in an increased load current capability for a given peak current. This AC then AD switch sequence is repeated until the output is pumped above its regulation voltage, a ﬁnal IZERO is detected, and the part returns to sleep mode (IVALLEY is ignored and IZERO is used in all modes once VOUT is above its programmed value).

4-Switch Mode

If (VOUT – 400mV) < ~VIN < (VOUT + 800mV), the LTC3531 operates in 4-switch step-up/down mode. Returning to Figure 1 (center) when VOUT falls below its regulation volt- age, switches A and C are turned on and current is ramped until IPEAK is detected. As with Boost Mode operation, C is then turned off, D is turned on and current is delivered to the output. When A and D are on, the inductor current slope is dependant on the relationship between VIN, VOUT, and the RDSON of the switches. In 4-switch mode, a tOFF timer (approximately 3μs) is used to terminate the AD pulse. Once the tOFF timer expires, switch A is turned off, B is turned on and inductor current is ramped down (VOUT is applied across the inductor) until IVALLEY is detected. This sequence is repeated until the output is regulated, BD switches are turned on, and a ﬁnal IZERO is detected. Anticross conduction circuitry in all modes ensures the P-channel MOSFET and N-channel MOSFET switch pairs (A and B or D and C) are never turned on simultaneously.

VIN	VOUT

VIN

VOUT

A	D

VIN

L

SW1	SW2

IMAX

B	C	IPEAK

tOFF

tOFF

tOFF

IVALLEY

IZERO

AC   AD

AC     AD

AC    AD

BD AC      AD

BD AC

AD     BD

AD    BD

AD  BD

BOOST MODE	4SW MODE	BUCK MODE

Figure 1. Voltage and Current Waveforms

3531 F01

3531fb

OPERATIO

Buck Mode

If VIN is ~800mV above VOUT, the LTC3531 operates in buck or step-down mode. The higher offset between VIN and VOUT (800mV) is required to ensure sufﬁcient mag- netizing voltage across the inductor  when the RDSONS are taken into account. At the beginning of a buck mode cycle (Figure 1 right side) switches A and D are turned on (VIN – VOUT is applied across the inductor), current is delivered to the output and ramped up until IPEAK is detected. When this occurs, A is turned off, B is turned on and inductor current falls (–VOUT across the induc- tor) until an IVALLEY is detected. This AD then BD switch sequence is repeated until the output is pumped above its regulation  voltage, a ﬁnal IZERO is detected, and the part returns to sleep mode.

Start-Up Mode

Before VOUT reaches approximately 1.6V, the D switch is disabled and its body diode is used to transfer current to the output capacitor. In start-up mode, the IVALLEY/IZERO sense circuit is disabled and an alternate algorithm is used to control inductor current. When the LTC3531 is brought out of shutdown (assuming VOUT is discharged) switches A and C are turned on until the inductor current reaches IPEAK. The AC switches are then turned off and inductor current ﬂows to the output through the B switch and D body diode. The period for the B switch/D body diode is controlled by the tOFF timer to ~800nS. This sequence of AC switch-on to IPEAK then B switch and D body diode for

~800ns is repeated until VOUT reaches ~1.6V. Once this

threshold is reached, the LTC3531 will transfer through the required modes until VOUT is brought into regulation.

Due to propagation  delays in the sense circuitry,  the magnitudes of the IPEAK, IVALLEY, and IZERO currents may shift depending on VIN, VOUT and operating mode.

OTHER LTC3531 FEATURES

Shutdown: The part is shut down by pulling SHDN below

0.4V, and made active by pulling the pin up to VIN or VOUT. Note that SHDN an be driven above VIN or VOUT, as long as it is limited to less than 6V.

Output Disconnect and Inrush Limiting:  The LTC3531 is designed to allow true output disconnect by opening both P-channel MOSFET rectiﬁers. This allows VOUT to go to zero volts during shutdown, drawing no current from the input source. It also provides inrush current limiting at turn-on, minimizing surge currents seen by the input supply.

Thermal Shutdown: If the die temperature reaches ap- proximately 150°C, the part will go into thermal shut- down and all switches will be turned off. The part will be enabled again when the die temperature has dropped by

10°C (nominal).  To deliver the power that the LTC3531 is capable of, it is imperative that a good thermal path be provided to dissipate the heat generated within the pack- age. It is recommended that multiple vias in the printed circuit board be used to conduct heat away from the IC and into a copper plane with as much area as possible. Soldering the Exposed Pad to the GND plane (DFN version) is recommended to improve thermal performance.

3531fb

APPLICATIO

S I	FOR	ATIO

Component Selection

Only three power components are required to

complete the design of the buck-boost converter, VOUT programming  resistors are needed for the adjustable version. The high operating frequency and low peak currents of the LTC3531 allow the use of low value, low proﬁle inductors and tiny external ceramic capacitors.

Inductor Selection

For best efﬁciency, choose an inductor with high frequency core material, such as ferrite, to reduce core loses. The inductor  should have low DCR (DC resistance)  to reduce

ductor value with a >500mA current rating and <400mΩ DCR is recommended. For applications where radiated noise is a concern, a toroidal or shielded inductor can be used. Table 2 contains a list of inductor manufacturers.

Capacitor Selection

The buck-boost convertor requires two capacitors. Ceramic X5R types will minimize ESL and ESR while maintaining capacitance at rated voltage over temperature. The VIN capacitor  should  be at least 2.2μF. The VOUT capacitor should be between 4.7μF and 22μF. A larger output capaci- tor should be used if lower peak to peak output voltage ripple is desired. A larger output capacitor will also improve

the I2R  losses, and must  be able to handle the peak

load regulation on V

OUT. See Table 3 for a list of capacitor

| inductor current without saturating. A 10μH to 22μH in- Supplier | Series | Phone | Website |
| --- | --- | --- | --- |
| AVX | X5R | (803) 448-9411 | www.avxcorp.com |
| Murata | X5R | USA: (814) 237-1431 (800) 831-9172 | www.murata.com |
| Sanyo | POSCAP | (619) 661-6322 | www.sanyovideo.com |
| Taiyo Yuden | X5R | (408) 573-4150 | www.taiyo-yuden.com |
| TDK | X5R | (847) 803-6100 | www.component.tdk.com |

manufacturers for input and output capacitor selection.

Table 2. Inductor Vendor Information

Table 3. Capacitor Vendor Information

| Supplier | Series | Phone | Website |
| --- | --- | --- | --- |
| COEV | DN4835 | (800) 227-7040 | www.coev.net |
| Coilcraft | MSS4020 LPO3310 DS1608 | (847) 639-6400 | www.coilcraft.com |
| Murata | LQH43CN LQH32CN | USA: (814) 237-1431 (800) 831-9172 | www.murata.com |
| Sumida | CDRH4D18 CDRH3D16/HP | USA: (847) 956-0666 Japan: 81-3-3607-5111 | www.sumida.com |
| Toko | D312C D412C DB320C | (847) 297-0070 | www.tokoam.com |

3531fb

APPLICATIO

S I	FOR	ATIO

SHUTDOWN

VIN

CIN

COUT

VOUT

L

GND

4 – SHDN

5 – VIN

VOUT – 3

GND – 2

GND

6 – SW1	SW2 – 1

SOT PIN-OUT

Recommended Layout (SOT Versions)

3531fb

TYPICAL APPLICATIO

5V/Li-Ion to 3.3V with ThinSOT (3.3V  Version)

VIN

3.1V TO

5V

+

10H

SW1	SW2

VIN 	VOUT

LTC3531-3.3

VOUT

3.3V

160mA

95

4.4VIN

EFFICIENCY (%)

90	5VIN

85

3.6VIN

80

100

POWER LOSS (mW)

10

5V/Li-Ion

2.2F	10F

–

SHDN	GND

3.1VIN

75	1

ON  OFF

3531 TA02a

POWER LOSS AT 3.6VIN

70

65

0.1

0.1

1	10	100	1000

LOAD CURRENT (mA)

3531 TA02b

2 AA Alkaline  to 3V with ThinSOT (3V Version)

VIN

1.8V TO

3.2V	+

–

10H

SW1	SW2

VIN 	VOUT

LTC3531-3

VOUT

3V

80mA

90

85

80   2.5VIN

75

3.2VIN

100

POWER LOSS (mW)

10

2 x AA

ALKALINE  +

–

2.2F			10F SHDN	GND

1.8VIN

EFFICIENCY (%)

70	1

POWER LOSS AT 3.2VIN

ON  OFF

3531 TA03a

65

60

0.1

0.1

1	10	100	1000

LOAD CURRENT (mA)

3531 TA03b

USB to 5V with 3 × 3 DFN (Adjustable Version)

USB

4.35V  TO

5.25V

4.7F

1

2.2F

10H

SW1	SW2

VIN 	VOUT

LTC3531

FB SHDN	GND

R2

1M

R1

324k

VOUT

5V

200mA

10F

95

90	EFFICIENCY

EFFICIENCY (%)

85

80

75

POWER LOSS

100

POWER LOSS (mW)

10

1

ON  OFF

3531 TA04a

70

65	0.1

0.1

1	10	100	1000

LOAD CURRENT (mA)

3531 TA04b

3531fb

PACKAGE DESCRIPTIO

S6 Package

6-Lead Plastic TSOT-23

(Reference LTC DWG # 05-08-1636)

0.62

MAX

0.95

REF

2.90 BSC (NOTE 4)

1.22 REF

3.85 MAX 2.62 REF

1.4 MIN

2.80 BSC

1.50 – 1.75 (NOTE 4)

PIN ONE ID

RECOMMENDED SOLDER PAD LAYOUT PER IPC CALCULATOR

0.95 BSC

0.30 – 0.45

6 PLCS (NOTE 3)

0.80 – 0.90

0.20 BSC

DATUM ‘A’

1.00 MAX

0.01 – 0.10

0.30 – 0.50  REF

0.09 – 0.20

1.90 BSC

NOTE:

1. DIMENSIONS ARE IN MILLIMETERS

2. DRAWING NOT TO SCALE

3. DIMENSIONS  ARE INCLUSIVE OF PLATING

(NOTE 3)	S6 TSOT-23 0302

4. DIMENSIONS  ARE EXCLUSIVE OF MOLD FLASH AND METAL BURR

5. MOLD FLASH SHALL NOT EXCEED 0.254mm

6. JEDEC PACKAGE REFERENCE IS MO-193

3531fb

PACKAGE DESCRIPTIO

DD Package

8-Lead Plastic DFN (3mm × 3mm)

(Reference LTC DWG # 05-08-1698)

0.675 0.05

3.5 0.05

1.65 0.05

2.15 0.05 (2 SIDES)

PACKAGE OUTLINE

0.25  0.05

0.50

BSC

2.38 0.05 (2 SIDES)

RECOMMENDED SOLDER PAD PITCH AND DIMENSIONS

R = 0.115

TYP

5

0.38  0.10

8

PIN 1

TOP MARK (NOTE 6)

3.00 0.10 (4 SIDES)

1.65  0.10 (2 SIDES)

0.200  REF

0.75 0.05

4

0.25  0.05

(DD) DFN 1203

1

0.00 – 0.05

2.38 0.10 (2 SIDES)

0.50 BSC

NOTE:

BOTTOM VIEW—EXPOSED PAD

1. DRAWING TO BE MADE A JEDEC PACKAGE OUTLINE M0-229 VARIATION OF (WEED-1)

2. DRAWING NOT TO SCALE

3. ALL DIMENSIONS ARE IN MILLIMETERS

4. DIMENSIONS  OF EXPOSED PAD ON BOTTOM OF PACKAGE DO NOT INCLUDE

MOLD FLASH. MOLD FLASH, IF PRESENT, SHALL NOT EXCEED 0.15mm  ON ANY SIDE

5. EXPOSED PAD SHALL BE SOLDER PLATED

6. SHADED AREA IS ONLY A REFERENCE FOR PIN 1 LOCATION ON TOP AND BOTTOM OF PACKAGE

Information furnished by Linear Technology Corporation is believed to be accurate and reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no representation that the interconnection of its circuits as described herein will not infringe on existing patent rights.

3531fb

15

TYPICAL APPLICATIO

Complete USB/Li-Ion Powered System with 3.3VOUT and Linear Charger

5V (NOM) FROM USB

IN1

LTC4055

OUT

VOUT: USB OR BATTERY

OTHER DC/DC

CABLE

SUSPEND USB POWER

IN2

HPWR SUSP

CHRG ACPR VNTC

3.1V TO 5.25V

L1

10H

SW1	SW2

VOUT

+

– Li-Ion

BAT

GND

TIMER CLPROG

PROG

100k

C1

22F

VIN 	VOUT

LTC3531-3.3

SHDN

3.3V

160mA

C2

10F

SHDN  WALL  NTC

100k

0.1F

GND

C1, C2: TAIYO YUDEN JMK316BJ106ML

L1: MURATA LQH43CN100K03 (650mA 0.24)

3531 TA05

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT1930/LT1930A | 1A (ISW), 1.2MHz/2.2MHz, High Efﬁciency Step-Up DC/DC Converter | VIN: 2.6V to 16V, VOUT(MAX) = 34V, IQ = 4.2mA/5.5mA, ISD < 1μA, ThinSOT Package |
| LTC3400/LTC3400B | 600mA (ISW), 1.2MHz Synchronous Step-Up DC/DC Converter | VIN: 0.85V to 5V, VOUT(MAX) = 5V, IQ = 19μA/300μA, ISD < 1μA, ThinSOT Package |
| LTC3401/LTC3402 | 1A/2A (ISW), 3MHz Synchronous Step-Up DC/DC Converter | VIN: 0.5V to 5V, VOUT(MAX) = 6V, IQ = 38μA, ISD < 1μA, MS Package |
| LTC3405/LTC3405A | 300mA (IOUT), 1.5MHz Synchronous Step-Down DC/DC Converter | VIN: 2.7V to 6V, VOUT(MIN) = 0.8V, IQ = 20μA, ISD ≤ 1μA, MS10 Package |
| LTC3406/LTC3406B | 600mA (IOUT), 1.5MHz Synchronous Step-Down DC/DC Converter | VIN: 2.5V to 5.5V, VOUT(MIN) = 0.6V, IQ = 20μA, ISD ≤ 1μA, ThinSOT Package |
| LTC3421 | 3A (ISW), 3MHz Synchronous Step-Up DC/DC Converter | VIN: 0.5V to 4.5V, VOUT(MAX) = 5.25V, IQ = 12μA, ISD < 1μA, QFN Package |
| LTC3422 | 1.5A (ISW), 3MHz Synchronous Step-Up DC/DC Converter | VIN: 0.5V to 4.5V, VOUT(MAX) = 5.25V, IQ = 25μA, ISD < 1μA, 3mm × 3mm DFN Package |
| LTC3426 | 2A (ISW), 1.2MHz Step-Up DC/DC Converter in SOT-23 | VIN: 1.6V to 5V, VOUT up to 5.5V |
| LTC3428 | 4A (ISW), 1.2MHz Step-Up DC/DC Converter | VIN: 1.6V to 5V, VOUT up to 5.5V |
| LTC3429 | 600mA (ISW), 500kHz Synchronous Step-Up DC/DC Converter | VIN: 0.5V to 4.4V, VOUT(MIN) = 5V, IQ = 20μA, ISD < 1μA, QFN Package |
| LTC3440 | 600mA (IOUT), 2MHz Synchronous Buck-Boost DC/DC Converter | VIN: 2.5V to 5.5V, VOUT(MIN) = 5.5V, IQ = 25μA, ISD < 1μA, MS, DFN Packages |
| LTC3441 | 600mA (IOUT), 2MHz Synchronous Buck-Boost DC/DC Converter | VIN: 2.5V to 5.5V, VOUT(MIN) = 5.5V, IQ = 25μA, ISD < 1μA, DFN Package |
| LTC3442 | 2MHz Synchronous Buck-Boost with Auto-Burst | VIN: 2.4V to 5.5V, VOUT up to 5.25V |
| LTC3443 | 1.2A (IOUT), 600kHz Synchronous Buck-Boost DC/DC Converter | VIN: 2.4V to 5.5V, VOUT(MIN) = 5.25V, IQ = 28μA, ISD < 1μA, MS Package |
| LTC3458 | 1.4A, 1.5MHz Synchronous Step-Up DC/DC Converter | VIN: 1.5V to 6V, VOUT up to 7.5V |
| LTC3458L | 1.7A, 1.5MHz Synchronous Step-Up DC/DC Converter | VIN: 1.5V to 6V, VOUT up to 6V |
| LTC3459 | 10V Micropower Synchronous Step-Up DC/DC Converter | VIN: 1.5V to 5.5V, VOUT up to 10V |
| LTC3525/LTC3525-3.3/ LTC3525-5 | 400mA (ISW), Synchronous Step-Up DC/DC Converter with Output Disconnect | VIN: 0.5V to 4.5V, IQ = 7μA, ISD < 1μA, 2mm × 2mm SC70 Package |

Linear Technology Corporation

16

1630 McCarthy  Blvd.,  Milpitas,  CA 95035-7417

3531fb

LT 0807 REV B • PRINTED IN USA

(408) 432-1900

● FAX: (408) 434-0507

● www.linear.com	© LINEAR TECHNOLOGY CORPORATION 2006