FEATURES

n  Complete Backup Battery Manager for Li-Ion/ Polymer, Lead Acid, NiMH/NiCd Batteries and Super Capacitors

n  Charge and Discharge Battery with Voltages Above and Below  the Input Supply Voltage

n  “No Heat” Battery Calibration Discharge Using

System Load

n  Automatic Battery Backup with Input Supply

Removal Using PowerPath™ Control

n  Standalone  for Li-Ion/Polymer, SLA, and Supercaps

n  Optional SMBus/I2C Support Allows Battery

Capacity Calibration Operation with Host

n  Over- and Under-Battery Voltage Protection

n  Adjustable Battery Float Voltage

n  Precision  Charge Voltage ±0.5%

n  Programmable  Charge/Calibration Current Up to

3A with ±3% Accuracy

n  Optional Temperature Qualiﬁed Charging

n  Wide Backup Battery Supply Range: 2.7V to 19V

n  Wide Input Supply Range: 4.5V to 19V

n  38-Lead (5mm × 7mm)  QFN Package

APPLICATIONS

n  Backup Battery Systems

n  Server Memory Backup

n  Medical Equipment

n  High Reliability Systems

Battery Backup

System Manager

DESCRIPTION

The LTC®4110 is a complete single chip, high efﬁciency, ﬂyback battery charge and discharge manager with auto- matic switchover between the input supply and the backup battery or super capacitor. The IC provides four modes of operation: battery backup, battery charge, battery calibra- tion and shutdown. Battery backup and battery charge are automatic standalone modes, while the optional calibration mode requires a CPU host to communicate over an SMBus. During calibration the ﬂyback charger is used in reverse to discharge the battery with a programmable constant current into the system load eliminating heat generation. Three status outputs can be individually reconﬁgured over the SMBus to become GPIOs. User programmable  over- discharge protection is provided.  The SHDN pin isolates the battery to support shipping the product with a charged battery installed.

Multiple LTC4110s can be combined to form a redundant battery backup system or increase the number of battery packs to achieve longer backup run times.

The LTC4110 is available in a low proﬁle (0.75mm), 38-pin

5mm  × 7mm  QFN package. The QFN features an exposed metal die mount pad for optimum thermal performance.

L, LT, LTC and LTM are registered trademarks of Linear Technology Corporation. PowerPath is a trademark of Linear Technology Corporation. All other trademarks are the property of their respective owners.

TYPICAL APPLICATION

Battery Backup System Manager	Server Backup System  (In Backup Mode)

SYSTEM LOAD

DCIN

0V

BACKUP LOAD (DCOUT)

CURRENT FLOW

LTC4110

SYSTEM LOAD (DC/DC, ETC.)

OFF

ON	ON

BATTERY       	 BACKUP

SYSTEM

BACKUP LOAD (MEMORY, ETC.)

BATTERY

MANAGER

CURRENT FLOW

HOST CPU	BATTERY

INID	BATID

I2C BUS

UVLO SET POINT

DCDIV

LTC4110

CHGFET

DCHFET

4110 TA01b

4110 F01

4110fb

ABSOLUTE MAXIMUM RATINGS

(Note 1)

DCIN, BAT, DCOUT, DCDIV, SHDN

to GND ....................................................... – 0.3V to 20V Input Voltage (CLP, CLN) ...............– 0.3V to DCIN + 0.3V Input Voltage (CSP, CSN) ................– 0.3V to BAT + 0.3V Input Voltage

(GPIO1, GPIO2, GPIO3, SELC, SELA, TYPE, VCHG, THA, THB, ISENSE, ACPDLY, SDA, SCL) .... – 0.3V to 7V

ACPb

VDIS VCAL VCHG VREF TIMER

TYPE

Input  Voltage  (VCAL, VDIS)....................... – 0.3V to 1.35V Output Voltage

(ACPb, GPIO1, GPIO2, GPIO3) ................ – 0.3V to 7V CLP-CLN, CSP-CSN ..................................................±1V Operating Temperature Range (Note 2)....– 40°C to 85°C Junction Temperature (Note 3) ............................. 105°C Storage Temperature Range

QFN Package......................................– 65°C to 125°C

PIN CONFIGURATION

INID DCOUT NC BATID VDD

CHGFET

DCHFET

TOP VIEW

| DCIN CLN CLP |  | 38 | 37 | 36 | 35 | 34 | 33 | 32 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 39 |  |  |  |  |  |  |  |  | 31 |
| 2 |  |  |  |  |  |  |  |  |  | 30 |
| 3 |  |  |  |  |  |  |  |  |  | 29 |
| 4 |  |  |  |  |  |  |  |  |  | 28 |
| 5 |  |  |  |  |  |  |  |  |  | 27 |
| 6 |  |  |  |  |  |  |  |  |  | 26 |
| 7 |  |  |  |  |  |  |  |  |  | 25 |
| 8 |  |  |  |  |  |  |  |  |  | 24 |
| 9 |  |  |  |  |  |  |  |  |  | 23 |
| 10 |  |  |  |  |  |  |  |  |  | 22 |
| 11 |  |  |  |  |  |  |  |  |  | 21 |
| 12 |  |  |  |  |  |  |  |  |  | 20 |
|  |  | 13 | 14 | 15 | 16 | 17 | 18 | 19 |  |  |

ACPDLY DCDIV SHDN SDA SCL GPI01

GPI02

GPI03

SELA

BAT SELC ISENSE

SGND

CSN CSP ITH

ICHG

ICAL IPCC THB

THA

UHF PACKAGE

38-LEAD (5mm s 7mm) PLASTIC QFN

TJMAX = 100°C, θJA = 34°C/W

EXPOSED PAD (PIN 39) IS GND, MUST BE SOLDERED TO PCB

ORDER INFORMATION

| LEAD FREE FINISH | TAPE AND REEL | PART MARKING | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- |
| LTC4110EUHF#PBF | LTC4110EUHF#TRPBF | 4110 | 38-Lead (5mm × 7mm) Plastic QFN | –40°C to 85°C |
| LEAD BASED FINISH | TAPE AND REEL | PART MARKING | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| LTC4110EUHF | LTC4110EUHF#TR | 4110 | 38-Lead (5mm × 7mm) Plastic QFN | –40°C to 85°C |

Consult LTC Marketing for parts speciﬁed with wider operating temperature ranges.

For more information on lead free part marking, go to: http://www.linear.com/leadfree/

For more information on tape and reel speciﬁcations, go to: http://www.linear.com/tapeandreel/

4110fb

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C. Unless otherwise speciﬁed,  VDCIN = VDCOUT = VDCDIV = 12V, VBAT = 8.4V, GND = SGND = CLP = CLN = SHDN = 0V and RVREF = 49.9k. All currents into device pins are positive and all currents out of device pins are negative.  All voltages  are referenced to GND, unless otherwise speciﬁed.

| SYMBOL | PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |

Power Input

| DCIN | Operating Voltage Range | Charge or Calibration Modes | l | 4.5 19 | V |
| --- | --- | --- | --- | --- | --- |
| DCOUT | Operating Voltage Range | Charge or Calibration Modes | l | 4.5 19 | V |
|  |  | Backup Mode | l | 2.7 19 | V |
| VBAT | Operating Voltage Range | Backup Mode | l | 2.7 19 | V |
| ISPLY | Supply Current (IDCIN + IDCOUT) in Idle Mode (Note 4) |  |  | 2 3 | mA |
| IBIDL | Battery Current in Idle Mode (Notes 4 and 5) |  |  | 30 45 | μA |
| IBBU | Battery Current in Backup Mode (Note 5) | VDCIN = 0 |  | 2 3 | mA |
| IBSD | Battery Current in Shutdown (Note 5) | VSHDN = VBAT, VDCIN = 0 |  | 20 45 | μA |
| VUVI | Undervoltage Lockout Exit Threshold | VDCIN Increasing | l | 3.7 4 4.45 | V |
| VUVD | Undervoltage Lockout Entry Threshold | VDCIN Decreasing | l | 3.4 3.7 4.1 | V |
| VUVH | Undervoltage Lockout Hysteresis |  |  | 400 | mV |

VDD Regulator

| VDD | Output Voltage | No Load | l | 4.5 4.75 5 | V |
| --- | --- | --- | --- | --- | --- |
| VDD(MIN) | Output Voltage | IDD = –10mA | l | 4.25 | V |

Charging Performance

| VFTOL | Charge Float Voltage Accuracy | 4.20V for Li-Ion. 2.35V for Lead Acid (Note 8) VCHG = GND –5°C < TA < 85°C (Note10) –40°C < TA < 85°C | l | –0.5 0.5 –0.8 0.8 –1 1 | % % % |
| --- | --- | --- | --- | --- | --- |
| VFATOL | Charge Float Voltage Adjust Accuracy | 0.3V and –0.3V for Li-Ion Batteries, 0.15V and –0.15V for Lead Acid Batteries (Note 8) | l | –2 2 | % |
| IBTOL | Bulk Charge Current Accuracy (Note 7) | VCSP – VCSN =100mV VBAT ≥ 3.1V –40°C < TA < 85°C | l | –3 3 –5 5 | % % |
| IPTOL | Preconditioning and Wake-Up Current Accuracy (Note 7) | VBAT ≥ 3.3V (Note 8), VCSP – VCSN = 10mV; Li-Ion and NiMH/NiCd Batteries Only |  | –30 30 | % |
|  |  | VBAT ≤ 3.3 (Note 8), VCSP – VCSN = 10mV; Li-Ion and NiMH/NiCd Batteries Only |  | –40 40 | % |
| ISKVA | Voltage Error Ampliﬁer Sink Current at ITH Pin | VITH = 2V |  | 96 | μA |
| ISRCA | Current Error Ampliﬁer Source Current at ITH Pin | VITH = 2V |  | –24 | μA |
| ISKCA | Current Error Ampliﬁer Sink Current at ITH Pin | VITH = 2V |  | 24 | μA |
| IVCHG | VCHG Pin Bias Current | VCHG = 1.25V |  | –100 100 | nA |
| VBC | Bulk Charge Threshold Voltage; VBAT Increasing (Note 8) | Li-Ion, VCHG = GND NiMH/NiCd |  | 2.80 3.00 3.20 0.84 0.90 0.96 | V V |
| VBCH | Bulk Charge Threshold Voltage Hysteresis; VBAT Decreasing (Note 8) | Li-Ion, VCHG = GND NiMH/NiCd |  | 85 40 | mV mV |
| VAR | Auto Recharge Threshold Voltage; VBAT Decreasing | Standard Li-Ion Only; Speciﬁed as Percentage of Float Voltage |  | 93 95 97 | % |
| VARH | Auto Recharge Threshold Hysteresis Voltage; VBAT Increasing | Standard Li-Ion Only; Speciﬁed as Percentage of Float Voltage |  | 2 | % |

4110fb

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C. Unless otherwise speciﬁed,  VDCIN = VDCOUT = VDCDIV = 12V, VBAT = 8.4V, GND = SGND = CLP = CLN = SHDN = 0V and RVREF = 49.9k. All currents into device pins are positive and all currents out of device pins are negative.  All voltages  are referenced to GND, unless otherwise speciﬁed.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| VBOV | Battery Overvoltage Threshold; VBAT Increasing | All Li-Ion, Lead Acid as Percentage of Float Voltage NiMH/NiCd (Note 8) |  | 105 107.5 110 1.80 1.85 1.90 | % V |
| VBOVH | Battery Overvoltage Threshold Hysteresis; VBAT Increasing. | All Li-Ion, Lead Acid as Percentage of Float Voltage NiMH/NiCd (Note 8) |  | 2 40 | % mV |
| VREF | Reference Pin Voltage Range |  | l | 1.208 1.220 1.232 | V |
| FTMR | Programmed Timer Accuracy | CTIMER = 47nF | l | –15 0 15 | % |
| tTIMEOUT | Time Between Receiving Valid ChargingCurrent() and ChargingVoltage() Commands. Wake-Up Timer. |  | l | 140 175 210 | sec |

Calibration Performance

| VCTOL | Calibration Cut-Off Default Voltage Accuracy; VBAT Decreasing | 2.75V for Li-Ion, 1.93V for Lead Acid, VCAL = GND (Note 8), 0.95V for NiMH/NiCd | l | –1.1 1.1 –1.3 1.3 | % % |
| --- | --- | --- | --- | --- | --- |
| VCTOLH | Calibration Cut-Off Default Voltage Hysteresis; VBAT Increasing. (Note 8) | Li-Ion Lead Acid NiMH/NiCd |  | 85 50 40 | mV mV mV |
| VCATOL | Calibration Cut-Off Voltage Adjust Accuracy | ±400mV for Li-Ion, ±300mV for Lead Acid, ±200mV for NiMH/NiCd (Note 8) | l | –1.5 1.5 | % |
| IFTOL | Calibration Current Accuracy (Note 7) | VCSP – VCSN = –100mV | l | –5 5 | % |
| IVCAL | VCAL Pin Leakage Current | VCAL = 1.25V |  | –100 100 | nA |
| IBDT | Back-Drive Current Limit Threshold | VCLP – VCLN Decreasing VCLN = VDCIN | l | 7 10 13 | mV |
| IBDH | Back-Drive Current Limit Threshold Hysteresis | VCLP – VCLN Increasing VCLN = VDCIN |  | 1 | mV |
| VOVP | Calibration Mode Input Overvoltage Comparator DCDIV Pin Threshold | VDCDIV Rising | l | 1.4 1.5 1.6 | V |
| VOVPH | Calibration Mode Input Overvoltage Comparator DCDIV Pin Hysteresis | VDCDIV Falling |  | 100 | mV |

AC Present and Discharge Cut-Off Comparators

| VAC | AC Present Comparator DCDIV Pin Threshold | VDCDIV Falling | l | 1.196 1.22 1.244 | V |
| --- | --- | --- | --- | --- | --- |
| VACH | AC Present Comparator DCDIV Pin Hysteresis | VDCDIV Rising |  | 50 | mV |
| IAC | AC Present Comparator DCDIV Pin Input Bias Current | VDCDIV = 1.25V |  | 100 | nA |
| tAC | ACPb Pin Externally Programmed Falling Delay | CACPDLY = 100nF, RVREF = 49.9k, VDCDIV Stepped From 1.17V to 1.30V |  | 8 10 12 | ms |
| VDTOL | Discharge Cut-Off Default Voltage Accuracy; VBAT Decreasing | 2.75V for Li-Ion, 1.93V for Lead Acid, VDIS = GND, 0.95V for NiMH/NiCd | l | –1.5 1.5 | % |
| VDTOLH | Discharge Cut-Off Default Voltage Hysteresis; VBAT Increasing (Note 8) | Li-Ion Lead acid NiMH/NiCd |  | 85 50 40 | mV mV mV |
| VDATOL | Discharge Cut-Off Voltage Adjust Accuracy | ±400mV for Li-Ion, ±300mV for Lead Acid, ±200mV for NiMH/NiCd | l | 2 2 | % |
| IVDIS | VDIS Pin Bias Current | VDIS = 1.25V |  | –100 100 | nA |

4110fb

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C. Unless otherwise speciﬁed,  VDCIN = VDCOUT = VDCDIV = 12V, VBAT = 8.4V, GND = SGND = CLP = CLN = SHDN = 0V and RVREF = 49.9k. All currents into device pins are positive and all currents out of device pins are negative.  All voltages  are referenced to GND, unless otherwise speciﬁed.

| SYMBOL | PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |

Input and Battery Ideal Diodes  and Switches

| VFR | Forward Regulation Voltage (VDCIN -VDCOUT, VBAT -VDCOUT) | 2.7V ≤ VDCIN ≤ 19V | l | 10 20 32 | mV |
| --- | --- | --- | --- | --- | --- |
| VREV | Reverse Voltage Turn-Off Voltage (VDCIN-VDCOUT, VBAT -VDCOUT) | 2.7V ≤ VDCIN ≤ 19V | l | –30 –18 –8 | mV |
| VGON | “ON” Gate Clamping Voltage (VDCIN -VINID , VBAT -VBATID ) | IINID, IBATID = 1μA |  | 7 8.3 9.7 | V |
| VGOFF | “OFF” Gate Voltage (VDCIN -VINID, VBAT -VBATID) | IINID, IBATID = –10μA VSHDN = 0V and VDCIN (Shutdown) |  | 0.25 | V |
| VFO | BATID Fast-On Voltage Comparator Threshold | IBATID > 500μA |  | 45 100 | mV |
| tIIDON tIIDOFF | INID Pin Delay Times Turn “ON” Turn “OFF” | CINID = 10nF DCIN is Switched Between 12.2V and 11.8V From DCOUT – VGOFF to DCOUT –6V From DCOUT – VGON to DCOUT –1.5V |  | 450 700 8 20 | μs μs |
| tBIDON tBIDOFF | BATID Pin Delay Times Turn “ON” Turn “OFF” | CBATID = 2.5nF BAT is Switched Between 12.2V and 11.8V From DCOUT – VGOFF to DCOUT –6V From DCOUT – VGON to DCOUT –1.5V |  | 15 60 8 20 | μs μs |

PWM Flyback Converter

| VOHF | CHGFET, DCHFET High | ICHGFET, IDCHFET = –1mA |  | 4.5 4.75 5.25 | V |
| --- | --- | --- | --- | --- | --- |
| VOLF | CHGFET, DCHFET Low | ICHGFET, IDCHFET = 1mA |  | 50 | mV |
| VOLFX | CHGFET, DCHFET in Shutdown and Backup Modes | VDCIN = VDCDIV = VDCOUT = 0V (Shutdown Mode), VDCIN = VDCDIV = 0V (Backup Mode) ICHGFET, IDCHFET = 1μA |  | 100 | mV |
| tR tF | CHGFET, DCHFET Transition Times Rise Time Fall Time | CLOAD = 1.6nF, 20% to 80% CLOAD = 1.6nF, 20% to 80% |  | 35 65 15 65 | ns ns |
| FPWM | PWM Oscillator Switching Frequency |  | l | 255 300 340 | kHz |

SafetySignal  Decoder and Thermistor Interface

| SSOR | SafetySignal Decoder SafetySignal Trip (RES_COLD/RES_OR) | RTHA = 1130Ω ±1%, CTH = 1nF (Note 6) RTHB = 54.9k ±1%. Smart Batteries and Li-Ion Only | l | 95 100 105 | k |
| --- | --- | --- | --- | --- | --- |
| SSCLD | SafetySignal Decoder SafetySignal Trip (RES_IDEAL/RES_COLD) | RTHA = 1130Ω ±1%, CTH = 1nF (Note 6) RTHB = 54.9k ±1% Smart Batteries and Li-Ion Only | l | 28.5 30 31.5 | k |
| SSIDL | SafetySignal Decoder SafetySignal Trip (RES_HOT/RES_IDEAL) | RTHA = 1130Ω ±1%, CTH = 1nF (Note 6) RTHB = 54.9k ±1% Smart Batteries and Li-Ion Only | l | 2.85 3 3.15 | k |
| SSHOT | SafetySignal Decoder SafetySignal Trip (RES_UR/RES_HOT) | RTHA = 1130Ω ±1%, CTH = 1nF (Note 6) RTHB = 54.9k ±1% Smart Batteries and Li-Ion Only | l | 425 500 575 | Ω |
| VHOT | THB Pin Hot Threshold Voltage | VTHB Decreasing; Lead Acid Only | l | 0.28 • 0.30 • 0.36 • VTHA VTHA VTHA | V |
| VHOTH | THB Pin Hot Threshold Hysteresis Voltage | VTHB Increasing; Lead Acid Only |  | 50 | mV |
| VREM | THB Pin Battery Removal Threshold Voltage | VTHB Increasing; Lead Acid Only | l | 0.90 • 0.94 • 0.96 • VTHA VTHA VTHA | V |

4110fb

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C. Unless otherwise speciﬁed,  VDCIN = VDCOUT = VDCDIV = 12V, VBAT = 8.4V, GND = SGND = CLP = CLN = SHDN = 0V and RVREF = 49.9k. All currents into device pins are positive and all currents out of device pins are negative.  All voltages  are referenced to GND, unless otherwise speciﬁed.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| VREMH | THB Pin Battery Removal Threshold Hysteresis Voltage | VTHB Decreasing; Lead Acid Only |  | 25 | mV |

Logic and Status  Output Levels

| VILS | SCL/SDA Input Pins Low Voltage |  | l | 0.8 | V |
| --- | --- | --- | --- | --- | --- |
| VIHS | SCL/SDA Input Pins High Voltage |  | l | 2.1 | V |
| VOLS | SDA Output Pin Low Voltage | IPULL-UP = 350μA | l | 0.4 | V |
| VOLG | ACPb, GPIO1,2,3 Output Pins Low Voltage | IACPb, IGPIO1, IGPIO2, IGPIO3 = 10mA |  | 1 | V |
| IOHG | ACPb, GPIO1,2,3 Output Pins Open Leakage Current | Outputs Open, VACPb, VGPIO1,2,3 = 5V |  | –2 2 | μA |
| VILG | GPIO Input Low Voltage |  | l | 1 | V |
| VIHG | GPIO Input High Voltage |  | l | 1.5 | V |
| VILSD | SHDN Input Pin Low Voltage |  |  | 0.5 | V |
| VIHSD | SHDN Input Pin High Voltage |  |  | 2.4 | V |
| IISD | SHDN Input Pin Pull-Up Current | VSHDN = 2.4V |  | –3.5 –2 –1 | μA |
| TLR | Logic Reset Duration After Power-Up From Zero | VDCIN Transition From 0V to 5V in <1ms; VBAT = 0 |  | 1 | s |

SMBus Timing (Note 9)

| tHIGH | SCL Serial Clock High Period | IPULL-UP = 350μA, CLOAD = 250pF, RPU = 9.31k | l | 4 | μs |
| --- | --- | --- | --- | --- | --- |
| tLOW | SCL Serial Clock Low Period | IPULL-UP = 350μA, CLOAD = 250pF, RPU = 9.31k | l | 4.7 | μs |
| tTO | Timeout Period |  | l | 25 | ms |
| tF | SDA/SCL Fall Time | CLOAD = 250pF, RPU = 9.31k | l | 300 | ns |
| tSU-STA | Start Condition Set-Up Time |  | l | 4.7 | μs |
| tHD-STA | Start Condition Hold Time |  | l | 4 | μs |
| tHD-DAT | SDA to SCL Falling-Edge Hold Time, Slave Clocking in Data |  | l | 300 | ns |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime. Speciﬁc functionality or parametric performance of the device beyond the limits expressly given in the Electrical Characteristics table is not implied by these maximum ratings.

Note 2: The LTC4110E is guaranteed to meet performance speciﬁcations from 0°C to 85°C. Speciﬁcations over the –40°C to 85°C operating temperature range are assured by design, characterization and correlation

Note 4: The LTC4110 is idle with no application load. It is not charging or calibrating the battery and is not in backup or shutdown mode. The internal clock is running and the SMBus is functional.

Note 5: Combined current of CSP, CSN and BAT pins set to VBAT with no application load.

Note 6: CTH is deﬁned as the sum of capacitance on THA, THB SafetySignal.

Note 7: Does not include tolerance of current sense or current programming resistors.

with statistical process controls.

Note 8: Given as a per cell voltage referred to the BAT pin (V

/number of

Note 3: This IC includes overtemperature protection that is intended to protect the device during momentary overload conditions.

series cells).

BAT

Overtemperature protection will become  active at a junction temperature greater than the maximum operating junction temperature. Continuous operation above the speciﬁed maximum operation temperature may result in device degradation or failure. Operating junction temperature TJ (in

°C) is calculated from the ambient temperature TA and the average power

dissipation PD (in watts) by the formula TJ = TA + θJA • PD.

Note 9: Refer to System Management Bus Speciﬁcation, Revision 1.1,

section 2.1 for Timing Diagrams and section 8.1, for tLOW and tTIMEOUT

requirements.

Note 10: Speciﬁcations over the –5°C to 85°C operating ambient temperature range are assured by design, characterization and correlation with statistical process controls.

4110fb

TYPICAL PERFORMANCE CHARACTERISTICS

Typical CHGFET and  DCHFET Waveforms

1200

1000

Output Charging Characteristics Showing  Constant Current and Constant Voltage Operation

2.5

2.0

Supply Current vs DCIN Voltage in

Idle Mode

5V/DIV

0V

VIN = 12V

500ns/DIV

4110 G01

800

IBAT (mA)

600

400

200

IDCIN (mA)

1.5

1.0

0.5

VBAT = 12V (NiMH)

0

| 0 2 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CC |  |  |
|  |  |  |  |  |  |  |  | CV |
|  | PRE-CHARGE |  |  |  |  |  |  |  |

6	8	10	12	14

VBAT (V)

4110 G02

0

| 0 5 10 15 20 |  |  |  |
| --- | --- | --- | --- |

DCIN (V)

4110 G03

140

120

100

IBAT (μA)

80

60

40

20

0

Battery Leakage in Idle

| Mode – IBIDL |  |  |  |  |
| --- | --- | --- | --- | --- |

1.8

1.6

1.4

1.2

1.0

I(mA)

BAT

0.8

0.6

0.4

0.2

Battery Current in Backup

Mode – IBBU

Battery Leakage in Shutdown

| Mode vs Battery Voltage |  |  |  |  |
| --- | --- | --- | --- | --- |

40

35

30

IBAT (μA)

25

20

15

10

5

–20

0	5	10	15

20	25

0

| 0 5 10 15 |  |  |  |  |
| --- | --- | --- | --- | --- |

20	25

0

0	5	10	15

20	25

VBAT (V)

4110 G04

VBAT (V)

4110 G05

VBAT (V)

4110 G06

| 100 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFFICIENCY |  |  |  |  |  |  |  |
|  |  |  |  | P | OWER LOSS |  |  |  |  |

90

80

EFFICIENCY (%)

70

Charging Efﬁciency/Power Loss,

12VIN and 12.6VOUT  (Xfmr = BH

510-1019)	Soft-Start Waveform

2.5

2.0

VBACKUP

2V/DIV

VBATTERY

Backup Mode On and Off

Waveform

60	1.5

50

40	1.0

30

20	0.5

10

0	0

0.2A/DIV

POWER LOSS (W)

0A

2ms/DIV

4110 G08

3V/DIV

0V

10ms/DIV NiMH BATTERY (12V)

ILOAD = 3A

4110 G09

0.05 0.1  0.2  0.3  0.4  0.5  0.6  0.7  0.8  0.9 1.00

ILOAD (A)

VIN = 15V FALLING

4110 G07

4110fb

PIN FUNCTIONS

DCIN (Pin 1): External DC Power Sense Input. Provides a control input and supply for the main supply ideal diode function.

CLN  (Pin 2): Current  Limit Sense Negative Input. See

CLP pin.

CLP (Pin 3): Current Limit Sense Positive Input. This pin and the CLN pin form  a differential input that senses volt- age on an external resistor for reverse current entering the power source while in low loss calibration mode. Should the current approach reversal, this function will terminate calibration. An RC ﬁlter may be required to ﬁlter out system load noise. Connect both CLP and CLN pins to GND to disable this function. A differential voltage of >1V between the CLP and CLN pins may damage the device.

ACPDLY  (Pin 4): ACPb Delay Control  Pin. A  capacitor connected from ACPDLY  to GND  and a resistor from VREF to GND programs delay in the ACPb pin high-to-low transition. Open if minimum delay is desired.

DCDIV  (Pin 5): AC  Present Detection  Input. Backup operation is invoked when the system power voltage, divided by an external resistor divider, falls below the threshold of this pin.

SHDN (Pin 6): Active High Shutdown/Reset Control Logic Input. Forces micropower shutdown mode if high  when DCIN supply is removed. Forces all registers to reset if high when  DCIN supply  is present. Normally tied to ground. Internal pin pull-up current.

SDA (Pin 7): SMBus Bidirectional Data Signal. Connect to VDD when not in use.

SCL (Pin 8): SMBus Clock Signal Input From SMBus Host. Connect to VDD when not in use.

GPIO1 (Pin 9): General Purpose I/O or Charge Status Pin. A logic-level I/O bit port that is conﬁgurable as a host-driven input/output port or as a battery charge status output (CHGb) with an open-drain N-MOSFET that is asserted low when any

smart battery or Li-Ion battery is in any phase of charging or when lead acid battery charge current is >C/x where:

x =	C	•5

ICHG

(See C/x Charge Termination section for more details). If the No SMBus option is selected with the SELA pin, the GPIO1 pin defaults as battery charge status. Refer to Table 5a.

GPIO2 (Pin 10): General Purpose I/O Pin. A logic-level I/O bit port that is conﬁgurable as a host-driven input/output port or as a battery undervoltage status output  (BKUP_FLTb) with an open-drain N-MOSFET that is asserted low only while in backup mode if the battery’s average cell voltage drops below voltage programmed  by the VDIS pin. If the No SMBus option is selected with the SELA pin, then  the GPIO2 pin defaults as battery undervoltage status. Refer to Table 5c.

GPIO3 (Pin 11): General Purpose I/O Pin.  A logic-level I/O bit port that is conﬁgurable as a host-driven input/output port or as a calibration complete status output (CAL_COM- PLETEb) with an open-drain N-MOSFET that is asserted low when calibration has been completed. If the SELA pin is programmed for no SMBus use then the status output is charge fault (CHGFLTb) instead of calibration complete. Refer to Table 5e.

SELA (Pin 12): SMBus Address Selection Input. Selects the LTC4110 SMBus address to facilitate redundant backup systems when standard batteries are used. Connect to GND for 12h,  VDD for 28h and the VREF pin for 20h.  When a smart battery is selected by the TYPE pin, the SELA pin must be connected to GND to select address 12h. If the SMBus is not used or to force all GPIOs to status mode upon power-up, connect pin to a typically 0.5 • VREF volt- age from VREF pin resistor divider. The SMBus address, if used, will be 12h.

4110fb

PIN FUNCTIONS

ACPb (Pin 13):  AC Present Status Digital Output. Open- Drain N-MOSFET output is asserted low when the main supply is present as detected by the DCDIV pin and internal DCIN UVLO.

VDIS  (Pin 14): Battery  Discharge  Voltage Limit During Backup Program Input. Battery threshold voltage at which backup mode will terminate  by turning off the isolation P-MOSFET with the BATID pin. Adjustable from external resistor string biased from VREF pin. For default  threshold connect to GND pin.

VCAL   (Pin 15): Battery Voltage  Limit  During Calibra- tion Program Input. Battery threshold voltage at which calibration will terminate. Adjustable from external resistor string biased from VREF pin. For default threshold connect to GND pin.

VCHG (Pin 16): Battery Float Voltage Program  Input. Trims the ﬂoat voltage during charging. Programmed from external resistor string biased from VREF pin. Connect  to GND for default  ﬂoat voltage.

VREF (Pin 17): Voltage Reference  Output and Timing Pro- gramming Input. Provides a typical virtual reference of 1.220V (VREF) for an external resistor divider tied between this pin and GND that programs the VCHG, VCAL and  VDIS pin functions. Total resistance from VREF to GND, along  with the capacitor on the timer pin, programs the charge  time. Voltage refer- ence output remains active in all modes except shutdown. Load current must be between 10μA and 25μA.

TIMER (Pin 18): Charge Timing Input. A capacitor  con- nected between TIMER and GND along  with the resistance connected from VREF to GND programs the charge time intervals.

TYPE (Pin  19):  Refer  to Table 8.

THA  (Pin 20): SafetySignal Force/Sense  Pin to Smart Battery and Force Pin to Lead Acid Battery Thermistor. See description of operation for more detail. The maxi- mum allowed combined capacitance on THA, THB and SafetySignal is 1nF. For lead acid battery applications the maximum capacitance on the THA pin is 50pF.

THB  (Pin 21): SafetySignal Force/Sense  Pin to Smart Battery and Sense Pin to Lead Acid Battery Thermistor. See description of operation for more detail. The maxi- mum allowed combined capacitance on THA, THB and SafetySignal is 1nF.

IPCC  (Pin 22): Battery Preconditioning  Charge Current Program Input. Programs the battery current during preconditioning or wakeup charging. Programmed from external resistor to GND.

ICAL (Pin 23): Battery Discharge Current During Calibration Program Input. Programs the constant discharge current at the battery during calibration. Programmed from external resistor to GND.

ICHG (Pin 24): Battery Current During Charge Program Input. Programs the battery current while constant-current bulk charging. Programmed from external resistor to GND.

ITH (Pin 25): Control Signal of the Current Mode PWM. AC compensates control loop. Higher ITH voltage corresponds to higher charging current.

CSP (Pin 26): Current Sense Positive Input. This pin and the CSN pin measure  voltage across the external current sense resistor to control battery current during charging and calibration.

CSN (Pin 27): Current Sense Negative Input. This pin and the CSP pin measure voltage across the external current sense resistor to control battery current during charging and calibration.

SGND (Pin 28): Signal Ground Reference Input. This pin should be Kelvin connected to the ﬂyback current sense resistor and to the battery return.

ISENSE  (Pin 29): Current Sense Input. Senses current in the ﬂyback transformer by monitoring voltage across the external current sense resistor. This pin should be Kelvin- connected to the resistor.

SELC (Pin  30):  Refer  to Table 8.

4110fb

PIN FUNCTIONS

BAT (Pin 31): Battery Voltage Sense Input. This pin is used to monitor the battery and control charging voltage through an internal resistor divider connected to this pin that is disconnected in shutdown mode. Also provides a control input for battery ideal diode functions. Pin should be Kelvin- connected to battery to avoid voltage drop errors.

DCHFET (Pin 32): Drives the Gate of an External N-MOSFET. Used to drive energy into the battery side of the high ef- ﬁciency switch mode converter during low loss calibration discharge of the battery. Provides synchronous rectiﬁcation during battery charging.

CHGFET  (Pin 33): Drives the Gate of an External  N- MOSFET. Used to drive energy into the supply side of the high efﬁciency switch mode converter during battery charging. Provides synchronous rectiﬁcation during low loss calibration mode.

VDD (Pin 34): Bypass Capacitor Connection for Internal VDD Regulator. Bypass at pin with 100nF low ESR capaci- tor to GND.

BATID (Pin 35): Drives the Gate of the Battery P-MOSFET Ideal Diode. Controls low loss ideal diode between the battery and backup load when in backup mode. When not in backup mode, the P-MOSFET is turned off to prevent battery power from back driving into main power.

NC (Pin 36): No Connect.

DCOUT (Pin 37): System  Power Output Voltage Monitor Input. Provides a control input for supply input ideal diode and battery ideal diode functions. Also supplies power to the IC. Bypass at pin with 100nF low ESR capacitor to GND.

INID (Pin 38): Drives the Gate of the Supply Input P-MOSFET Ideal Diode. Controls low loss ideal diode between the sup- ply input and backup load when not in backup mode.

Exposed Pad (Pin 39): Ground. The Exposed Pad must be soldered to the PCB.

4110fb

BLOCK DIAGRAM

37  DCOUT

INID 38

SUPPLY INPUT BATTERY PowerPath CONTROLLER

36  NC

35  BATID

DCIN  1

31  BAT

GND 39

VDD   34

VDD

REGULATOR

NUMBER OF CELLS

PRECISION VOLTAGE DIVIDER

CA

CURRENT SELECTION

CHG/DCH SWITCH

27  CSN

26  CSP

CLP  3

CLN    2

DCDIV    5

EA

1.220

CURRENT SWITCH

22  IPCC

24  ICHG

23  ICAL

25  ITH

VREF   17

VCHG   16

VCAL   15

VDIS   14

SELC 30

VOLTAGE REFERENCE ANALOG COMPARATORS AND SWITCHES

÷10

PROGRAMMING CURRENT

PWM LOGIC

33  CHGFET

32  DCHFET

29  ISENSE

28  SGND

TYPE   19

OSC

18  TIMER

SHDN  6

SDA  7

SCL  8

SMBus

INTERFACE

AND CONTROL	TIMER/

UVLO

4   ACPDLY

13  ACPb

SELA 12

THA   20

THB   21

THERMISTOR INTERFACE

CONTROLLER

9   GPIO1

10  GPIO2

11  GPIO3

4110 BD

4110fb

OPERATION

OVERVIEW

In the typical application, the LTC4110 is placed in series with main power supply that powers all or part of the system, which must include the device(s) or system that needs battery backup.

The LTC4110 has four modes of operation:

•  Battery Backup Mode

•  Battery Charge Mode

•  Battery Calibration Mode

•  Shutdown Mode

The LTC4110 provides complete PowerPath  control for the battery backed up load switching automatically from the main power supply to the battery when battery backup mode is required. Low loss ideal diode  FET switches are used to connect the main supply or the battery to the backup load which permit multiple LTC4110’s  to work together in a scalable fashion to permit longer backup times, redundancy and/or higher load currents. In battery charge mode, power is drawn from the main supply by a high efﬁciency synchronous ﬂyback charger. The LTC4110 maintains the state of charge (SOC) of the battery at all times so the battery is ready at all times. Use of a ﬂyback converter permits charging of batteries who’s termination voltage can be greater than the main supply voltage, while at the same time providing  high DC isolation to minimize parasitic drain on the battery. Testing, maintenance support and capacity veriﬁcation of the battery is supported through the LTC4110’s calibration mode. In calibration mode, the same synchronous ﬂyback used to charge the battery is

also used in reverse to allow safe controlled discharge of the battery back into the main supply eliminating wasted heat and energy. The product will not need  to provide any additional thermal management to support this mode. Shutdown mode disconnects the battery from the load to preserve capacity and permits shipping the product with an energized battery installed at the factory, eliminating battery installation at the site. The LTC4110  supports optional control and monitoring of all activities by a host including faults over the industry standard SMBus, which is a variation of the I2C bus. However no host is required as the LTC4110 is fully functional in a standalone mode. Combining all these functions into a single IC reduces circuit area compared to presently available solutions.

The LTC4110  is designed to work with both standard battery and smart battery conﬁgurations. Smart batteries are standard batteries with industry standard gas gauge electronics  built in offering accurate  SOC information for the host. Furthermore,  being intimate with all aspects of the battery, it also has the ability to control the charge process. Smart batteries use the SMBus as the com- munication bus for data exchange and charge control. For more information about smart batteries, see www. sbs-forum.org for speciﬁcations or contact Linear Tech- nology Applications.

It is important to know that the LTC4110 uses the TYPE pin to learn what type of battery it will be working with. The TYPE pin setting  globally affects  all of the operating modes, options including GPIO and control ranges. Table 1 and Table 2 give you a complete breakdown of all the battery types supported relative to the TYPE pin settings

Table 1. LTC4110 Battery Pack Charge Mode Capabilities

| BATTERY TYPE | CHEMISTRY |  |  | MAXIMUM CHARGE TIME (SLA EXCLUDED) |
| --- | --- | --- | --- | --- |
|  | Li-Ion/Polymer | Nickel | SLA/Lead Acid |  |
| Standard Battery | Yes | No | Yes | Adj. Up to 12 Hours |
| Smart Battery | Yes | Yes | Yes | Unlimited |

Table 2. LTC4110 Battery Pack Charge Voltage Capabilities

| CHEMISTRY | VCELL FULL CHARGE | VCELL ADJ. RANGE | SERIES CELL COUNT | NOMINAL STACK VOLTAGE (V) |
| --- | --- | --- | --- | --- |
| Lead Acid | 2.35V | ±0.15V | 2, 3, 5 and 6 | 4, 6, 10 and 12 |
| Li-Ion/Polymer | 4.2V | ±0.3V | 1, 2, 3 and 4 | 3.6, 7.2, 10.8 and 14.4 |
| NiMH/NiCd | N/A | N/A | 4, 6, 9 and 10 | 4.8, 7.2, 10.8 and 12 |
| Super Caps | 2.5V, 2.7V or 3V | Yes | 2 to 7 | 5 to 18 |

4110fb

OPERATION

and ranges. It should be noted that even  if the LTC4110

TYPE pin is not set to a smart battery mode, any SMBus commands sent by a host or a smart battery are still acted upon. For SuperCap support, see the Applications Information section.

BATTERY BACKUP MODE

Figure 1 shows the LTC4110 in backup mode and the corresponding PowerPath enabled. The LTC4110 use the DCDIV pin to typically monitor the DCIN voltage  through an external resistor divider. The DCDIV pin sets the backup mode threshold voltage and senses the need to enter backup mode. DCDIV can alternately be driven with other signals such as logic.  When  the  DCDIV pin voltage drops below  the  AC present threshold voltage (see VAC) backup mode is entered. Backup mode is also entered whenever the internal undervoltage lockout, UVLO, senses that DCIN (VUVD) or DCOUT has fallen to excessively low voltages. In backup mode the battery P-MOSFET ideal diode is enabled to backup the load from the battery. The supply input P-MOSFET ideal diode isolates the main supply input from the load and the ﬂyback switcher N-MOSFETs are inhibited from turning on. Also, after the threshold is passed, hysteresis (VACH) is switched in. When the supply is returning and the AC present threshold voltage plus the hysteresis voltage is reached on the DCDIV pin, both of the battery P-MOSFETs are rapidly switched  off (tdDOFF) and the supply input P-MOSFET ideal diode provides  the load current. When forward biased, the ideal diodes regulate their forward  voltage drop to 20mV typical (VFR) when the

MOSFET is sufﬁciently  sized. If the voltage input falls and results in a forward voltage below 20mV, then the ideal diode will begin turning off at a slow rate. Should the ideal diode see a –18mV typical (VREV) or lower reverse voltage, the ideal diode will turn off quickly (tdDOFF).

While in backup, the battery’s average cell voltage is moni- tored to protect the battery from excessive discharge.  If the cell voltage drops below the value programmed by the VDIS pin (Li-Ion default = 2.75V/cell,  NiMH/NiCd default

= 0.95V/cell, lead acid default = 1.93V/cell), the battery

P-MOSFETs are rapidly turned off and the battery is dis- connected from the load. If DCIN is above  UVLO, the load and the LTC4110 will be powered from the supply input. If DCIN is below  UVLO, the LTC4110  enters the micropower shutdown mode (see the Shutdown Mode section for more details). Also, the SMBus accessible BKUP_FLT fault bit is set and maintained as long as sufﬁcient battery voltage is present (VBAT ≥ 2.7V). This fault bit can be read after DCIN returns to a voltage  level exceeding  the internal UVLO threshold (see VUVI) and  DCOUT has regained suf- ﬁcient voltage (see DCOUT) to provide internal power. If the GPIO2 port is programmed as the BKUP_FLTb status output after DCIN returns, it will be forced low to repre- sent an inverted  BKUP_FLT bit. When  DCIN returns, as sensed by the UVLO, the shutdown mode is automatically cancelled and normal operation can resume, however, the BKUP_FLT bit remains set until either the SHDN pin is set high (all registers reset) or register bits POR_RESET or BUFLT_RST are set. See the Shutdown Mode section for details. During backup, the external thermistor network is monitored for battery presence.

SYSTEM LOAD

DCIN

0V

BACKUP LOAD (DCOUT)

CURRENT FLOW

BATTERY CHARGE MODE

Figure 2 shows the charge path to charge a battery. Cur-

OFF

ON	ON

BATTERY

rent is pulled from the supply input to charge the battery. At the same time, the input supply provides power to both the system load and the backup load. The battery is isolated from the load at all times so it cannot affect

INID	BATID

charger terminations algorithms.

UVLO SET POINT

DCDIV

LTC4110

CHGFET

DCHFET

4110 F01

If we ignore battery chemistry  for a moment, as far as the LTC4110 charger  is concerned, there are only two basic charge modes. When the TYPE pin selects a standard bat-

Figure 1. Backup Mode Operation

tery mode, charge termination is controlled by the LTC4110

4110fb

OPERATION

DCIN

SYSTEM LOAD

BACKUP LOAD

ON

OFF	OFF

BATTERY

current sense resistor (RSNS(BAT)) and CSP/CSN pin resis- tors. The constant voltage (ﬂoat voltage) is programmed to one of four values (4.2V, 8.4V, 12.6V, 16.8V) depending on the number of series cells using the SELC pin and can be adjusted ±0.3V/cell with the VCHG pin. If adjusted, the auto recharge threshold and overvoltage threshold will

CURRENT FLOW

INID		BATID CHGFET

track proportionally.

The charge cycle begins when the supply input is present as sensed by the DCDIV pin and DCIN above  UVLO, the

LTC4110

DCHFET

4110 F02

battery cell voltage is below the auto recharge threshold

(95% of the programmed ﬂoat voltage; see VAR), thermis- tor temperature is within ideal limits,  COLD, under  range

Figure 2. Charge Mode Operation

for the battery chemistry selected. Speciﬁcally the TIMER pin becomes active and used to detect faults conditions or terminate the charge cycle itself as needed. Smart battery SMBus charge control commands are still honored  if any are sent at any time.  A smart battery can safely function in a standard battery mode if identical  in chemistry and voltage conﬁguration as the standard battery. When the TYPE pin selects a smart battery mode, this simply disables the TIMER pin and its function in charge termination. The smart battery is able to restart or terminate a charge cycle at any time using charge commands over the SMBus. This mode also enables smart battery wake-up and watchdog functions based on tTIMEOUT  per the smart battery stan- dards. However it is not recommended to use a standard battery with a LTC4110 conﬁgured for smart battery mode operation.  You can shorten  battery life, damage or destroy the battery. In the extreme case this can cause an explosion since no charge termination mechanisms are active.

The following sections explain detailed operation for each charge mode as selected by the TYPE pin.

STANDARD  LI-ION/POLYMER  BATTERY CHARGE MODE

The charger is programmed for standard Li-Ion batteries by connecting the TYPE pin to GND. During Li-Ion charging, the LTC4110 operates  as a high efﬁciency, synchronous, PWM ﬂyback battery charger with constant-current  and constant ﬂoat voltage regions of operation. The constant- charge current is programmed by the combination of a resistor (RCHG) from the ICHG  pin to ground, a battery

(see SafetySignal Decoder section) or is optioned out and the register bit CHARGE_INHIBIT is cleared (see Tables 6 and 7 for register details).

Soft-start ramps the charge current at a rate set by the capacitor on the ITH pin. When charging  begins,  the pro- grammable timer initiates timing and the CHGb (GPIO1 pin) status output is pulled LOW. An external  capacitor on the TIMER pin, along  with the current set by the total series resistance connected to the VREF pin, sets the total charge time.

If the battery voltage is less than the 3.0V/cell bulk charge threshold (VBC), the charger will begin with a precondition- ing trickle charge current. The trickle current is programmed by the resistor  (RPCC) from the IPCC pin to ground. During preconditioning   trickle charging,  if the battery voltage stays below the bulk charge threshold (VBC) 25% of the programmed bulk charge time, the battery may be defective and the charge sequence will be terminated immediately. To indicate  this fault, the CHGb (GPIO1 pin) becomes high impedance, the CHG_STATE_0 and CHG_STATE_1 register bits will be set low and CHG_FLT register bit will be set high. Charge is terminated and the timer reset until the fault is cleared by the RESET_TO_ZERO or POR_RESET SMBus write commands, SHDN pin toggle or the battery removed and replaced. Removing the supply input will not clear the fault if the battery is present.

If  the battery voltage exceeds 107.5% (VBOV)  of the programmed ﬂoat voltage during any stage of charge, the charger pauses until the voltage drops below the hysteresis (VBOVH). The timer is not stopped and no fault is indicated.

4110fb

OPERATION

PWM STOPPED (BATTERY OVP)

10

RESET

ANY CHARGE STATE

7, 12, 13

14	15	1

11 (BATTERY NEEDS RECHARGE)

ANY CHARGE STATE

8

STOP CHARGE (OVERTEMPERATURE)

RESUME CHARGE STATE

9

PRE-CONDITIONING

CHARGE

5 (PRE-CONDITIONING FAULT)

6 (BULK TIME FAULT)

2	BULK	3

CHARGE

STOP

CHARGE

4 (BATTERY FULL)

TOP-OFF

CHARGE

Figure 3. Standard Li-Ion Charge State Diagram (Does  Not Include Calibration)

4110 F03

| # | Logic | Event (T = True, F = False) [Notes] | Notes and/or Actions (T = True, F = False) |
| --- | --- | --- | --- |
| 1 | Or | RES_OR = F & DCDIV pin = T & SHDN pin = F & CHARGE_INHIBITED = F & CHG_FLT = F & VBAT < VBC RES_OR = F & DCDIV pin = T & SHDN pin = F & CHARGE_INHIBITED = F & CHG_FLT=F & ChargingVoltage() ≠ 0 & ChargingCurrent() ≠ 0 | IPPC & Timer/4(PreCond) = Started & CHG = T & ALARM_INHIBITED = F (RES_OR = F = Bat Inserted -> See ChargeStatus() ) (POR_RESET -> See ChargeMode() |
| 2 |  | VBAT > VBC | IPPC = Off & ICHG = On & Timer/4(PreCond) = Stopped & Timer(Bulk) = Started. |
| 3 |  | C/5 = T | Timer(Bulk) = Stopped & Timer/4(Top Off) = Started |
| 4 |  | Timer/4(Top Off = done [Battery is full] | ICHG = Off & CHG = F (Typical Full State) |
| 5 |  | Timer/4(PreCond) = done before VBAT > VBC | IPPC = Off & CHG_FLT = T & CHG = F |
| 6 |  | Timer(Bulk) = done before C/5 = T | ICHG = Off & CHG_FLT = T & CHG = F |
| 7 | Or | RESET_TO_ZERO = T [See ChargeMode()] CHARGE_INHIBIT=T [See ChargeMode()] | ICHG or IPPC = Off & All Timers = Reset & CHG_FLT = F & CHG = F |
| 8 |  | RES_HOT = T & RES_UR = F [See ChargeStatus()] | ICHG or IPPC = Off & CHG_FLT = T, Timers paused. |
| 9 |  | RES_HOT = F [See ChargeStatus()] | ICHG or IPPC = On & CHG_FLT = F, Timers resume. |
| 10 | Or Or Or Or | DCDIV pin = F RES_OR = T [Bat Removed, See ChargeStatus()] SHDN pin = T VUVD = T POR_RESET = T [See ChargeMode()] | ICHG or IPPC = Off & All Timers = Reset & ALARM_INHIBITED = F & CHG_FLT = F & CHG = F & CHARGE_INHIBITED = F |
| 11 | Or | VAR = T [AutoRestart] ChargingVoltage() & ChargingCurrent() ≠ 0 | (The battery needs another charge cycle or Smart Battery has requested to start another cycle.) |
| 12 | Or Or Or | AlarmWarning() command is sent by Smart Battery over SMBus with any of the following bits set to True: OVER_CHARGED_ALARM TERMINATE_CHARGE_ALARM Reserved ALARM OVER_TEMPERATURE_ALARM | ICHG or IPPC = Off & All Timers = Reset & CHG = F & ALARM_INHIBITED = T (ALARM_INHIBITED bit is found in ChargeStatus()) |
| 13 |  | ChargingVoltage() or ChargingCurrent() = 0 sent | ICHG or IPPC = Off & CHG = F |
| 14 |  | VBOV = T [Battery Overvoltage] | PWM stopped. Timers remain running. |
| 15 |  | VBOV = F | PWM restarted. |

Note: For all charge states, VCHG is always active.

4110fb

OPERATION

When the battery voltage exceeds the bulk charge threshold (VBC), the charger begins the bulk charge portion of the charge cycle. As the battery accepts charge, the voltage increases. Constant-current charge continues until the battery approaches the constant voltage. At this time, the charge current will begin to drop, signaling the beginning of the constant-voltage portion of the charge cycle.

The charger will maintain the constant voltage across the bat- tery until either C/x is reached or 100% of the programmed bulk charge time has elapsed during bulk charge. When the current drops to approximately 20% of the full-scale charge current, an internal C/x comparator will initiate the start of the top-off stage. The top-off stage charges for

25% of the total programmed bulk charge time. When the time elapses, charge is terminated and CHGb (GPIO1 pin) is forced to a high impedance state and CHG_STATE_0 and CHG_STATE_1 register bits will be set low. Should the total bulk charge time elapse before C/x is reached, charge is terminated and a CHG_FLT fault is indicated until cleared by the RESET_TO_ZERO  or POR_RESET SMBus  write commands, SHDN pin toggle or the battery removed and replaced. Fault conditions are not cleared when the supply input is removed  if the battery has sufﬁcient voltage.

An optional  external  thermistor network is sampled at regular intervals to monitor battery temperature and to detect battery presence.  If the thermistor temperature is hot (see the SafetySignal Decoder section), the charge timer is paused, charge current is halted, CHG_FLTb (GPIO3 pin) is forced low and the CHG_FLT bit will be set high. CHGb (GPIO1 pin) , CHG_STATE_0 and CHG_STATE_1 register bits will not be affected. When the thermistor value returns to an acceptable value, charging resumes, CHG_FLTb (GPIO3 pin) returns to high impedance and the CHG_FLT bit will be reset low. An open thermistor indicates absence of a battery. To defeat the temperature monitoring function, replace the thermistor with a resistor to indicate ideal battery temperature. When a thermistor is not used, the resistor circuit must be routed through the battery connector if battery presence detection is required.

After a charge cycle has ended without fault, the charge cycle is automatically restarted  if the average battery cell voltage falls below the auto recharge threshold.  At any

time charging can be forced to stop by pulling the SHDN pin high or setting the CHARGE_INHIBIT bit high through the SMBus.

SMART  BATTERY CHARGE MODE

This section explains operation for smart batteries with a SMBus interface. Smart Li-Ion is selected by connecting the TYPE pin to the VDD pin and smart Nickel (NiMH/NiCd) is selected by connecting the TYPE pin to the VREF pin. The LTC4110 only implements  a subset of smart battery charger commands; the actual charging algorithm is determined by LTC4110 through  external resistors even if the battery is “smart.”

The LTC4110 operates  as a high efﬁciency, synchronous, PWM ﬂyback battery charger with constant current and constant ﬂoat voltage regions of operation. The constant- charge current is programmed by the combination of a resistor (RCHG) from the ICHG  pin to ground, a battery current sense resistor (RSNS(BAT))  and CSP/CSN  pin resistors. For Li-Ion the constant voltage (ﬂoat voltage) is programmed to one of four values (4.2V, 8.4V, 12.6V,

16.8V) depending on the number of series cells using the SELC pin and can be adjusted  ±0.3V/cell with the VCHG pin. For nickel batteries the constant-voltage function is not used, however, a non-zero value is still required to be written to the ChargingVoltage() register. The internal auto recharge function is inhibited for smart batteries.

If  the battery voltage exceeds 107.5% (VBOV)  of the programmed ﬂoat voltage during any stage of charge, the charger pauses until the voltage drops below the hysteresis (VBOVH). The timer is not stopped and no fault is indicated.  This function is disabled when nickel based smart batteries are used.

There are four states associated with smart battery charge mode, namely:

•  SMBus Wake-Up Charge State

•  SMBus Preconditioning Charge State

•  SMBus Bulk Charge State

•  SMBus  OFF State

These states are explained in the following four sections.

4110fb

OPERATION

PWM STOPPED (BATTERY OVP)

WAKE UP	1

CHARGE

RESET

10		ANY CHARGE STATE

11	12

2	4, 7, 13, 14

ANY CHARGE STATE

8

OFF

RESUME CURRENT STATE

9

PRE-CONDITIONING	3

CHARGE

BULK

CHARGE

5 (BAD BATTERY)

4 (BATTERY FULL)

OFF

(OVERTEMPERATURE)

6 (BATTERY RECHARGE REQUEST)

4110 F04

Figure 4. Smart Battery Charge State Diagram (Does Not Include Calibration)

| # | Logic | Event (T = True, F = False) [Notes] | Notes and/or Actions (T = True, F = False) |
| --- | --- | --- | --- |
| 1 | Or | RES_OR = F & DCDIV pin = T & SHDN pin = F & CHARGE_INHIBITED = F & CHG_FLT = F & RES_HOT = F RES_OR = F & DCDIV pin = T & SHDN pin = F & CHARGE_INHIBITED = F & CHG_FLT = F & RES_HOT = T & RES_UR = T | IPPC = On (Constant Current only) & TTIMEOUT = Started & CHG = T |
| 2 |  | ChargingVoltage() & ChargingCurrent() ≠ 0 sent | Timer/4(Pre-Charge) = Started & TTIMEOUT disabled & ALARM_ INHIBITED = F |
| 3 |  | VBAT > VBC | IPPC = Off, ICHG = On, Timer/4(Pre-Charge) = Stopped & Timer(SMBus) = Started |
| 4 |  | ChargingVoltage() or ChargingCurrent() = 0 sent | ICHG = Off & All Timers = Reset & CHG = F |
| 5 |  | Timer/4(Pre-Charge) = Done before VBAT > VBC | IPPC = Off & All Timers = Reset & CHG = F |
| 6 |  | ChargingVoltage() & ChargingCurrent() ≠ 0 sent & RES_OR = F & DCDIV pin = T & SHDN pin = F & CHARGE_INHIBITED = F & CHG_FLT = F | IPPC = On & Timer/4(Pre-Charge) = Started & CHG = T & ALARM_INHIBITED = F |
| 7 |  | TTIMEOUT = Done (Dead Battery or Loss of SMBus) | ICHG = Off & All Timers Reset & CHG = F |
| 8 |  | RES_HOT = T & RES_UR = F [See ChargeStatus()] | ICHG or IPPC = Off & CHG_FLT = T, Timer = Paused. |
| 9 |  | RES_HOT = F [See ChargeStatus()] | ICHG or IPPC = On & CHG_FLT = F, Timer = Resume. |
| 10 | Or Or Or Or | DCDIV pin = F RES_OR = T [Bat Removed, See ChargeStatus()] SHDN pin = T VUVD = T POR_RESET = T [See ChargeMode()] | ICHG or IPPC = Off & All Timers = Reset & CHG_FLT = F & CHG = F & ALARM_INHIBITED = F & CHARGE_INHIBITED = F |
| 11 |  | VBOV = T [Battery Overvoltage] | PWM stopped. Timers remain running. |
| 12 |  | VBOV = F | PWM restarted. |
| 13 | Or | RESET_TO_ZERO = T [See ChargeMode()] CHARGE_INHIBIT = T [See ChargeMode()] | ICHG or IPPC = Off & All Timers = Reset & CHG_FLT = F & CHG = F |
| 14 | Or Or Or | AlarmWarning() command is sent by Smart Battery over SMBus with any of the following bits set to True: OVER_CHARGED_ALARM TERMINATE_CHARGE_ALARM Reserved ALARM OVER_TEMPERATURE_ALARM | ICHG or IPPC = Off. & All Timers = Reset & CHG = F & ALARM_INHIBITED = T (ALARM_INHIBITED bit is found in ChargeStatus()) |

Note: VCHG is active in all charge states except for nickel batteries which operate in constant current mode.

4110fb

OPERATION

SMBUS  WAKE-UP CHARGE STATE

The battery will be charged with a ﬁxed “wake-up” current regardless of previous ChargingCurrent() and Charging- Voltage() register values during wake-up charging. The cur- rent is identical to the preconditioning charge current which is programmed with an external resistor through the IPCC pin. The wake-up timer has the same period as tTIMEOUT, typically 175sec (see tTIMEOUT).

The following conditions must be met to allow wake-up charge of the battery:

•	The SafetySignal must be RES_COLD, RES_IDEAL, or

RES_UR.

•	AC must be present. This is qualiﬁed by DCDIV > VAC

+ VACH and DCIN above  UVLO.

•	Wake-up charge initiates  if a battery does not write non-zero values to ChargingCurrent() and Chargin- Voltage() registers when AC power  is applied and a battery is present or when  AC is present and a battery is subsequently connected.

The following conditions will terminate the wake-up charge state and end charge attempts, unless otherwise noted.

•	The tTIMEOUT period  is reached (see tTIMEOUT) when the SafetySignal is RES_COLD or RES_UR. The state machine will go to the SMBus OFF state. The CHG_FLT bit is not set.

•	The SafetySignal is registering  RES_HOT. The state machine  will go to the SMBus  OFF state.

•	The SafetySignal is registering  RES_OR. The state machine  will go to the reset state.

•	The LTC4110 will leave the wake-up charge state and go into the SMBus preconditioning charge state if the ChargingCurrent()   AND ChargingVoltage()  registers have been written to non-zero values.

•	The AC power  is no longer present (DCDIV < VAC  or DCIN below  UVLO). The state machine  will go to the reset state.

•	The ALARM_INHIBITED   becomes set  in  the ChargerStatus() register. The state machine  will go to the SMBus  OFF state.

•	CHARGE_INHIBIT is set in the BBuControl() register.

Charge is stopped, however, the wake-up timer is not paused. Clearing   CHARGE_INHIBIT will  enable the LTC4110 to resume charging.

•	There is insufﬁcient DCIN voltage  to charge the battery as determined by the internal  UVLO. This causes the state machine to enter the reset state and stop all charge activity. The LTC4110 will resume wake-up charging when there is sufﬁcient DCIN voltage  to charge the battery.

•	The CAL_START bit in the BBuControl() register is set.

Charge is stopped  and the LTC4110 enters the calibra- tion state.

•	Writing a zero value to either the ChargingVoltage() or ChargingCurrent() register. The state machine  will go to the SMBus  OFF state.

•	RESET_TO_ZERO is set in the BBuControl() register.

Charge is stopped; the SMBus  OFF State  is entered.

SMBUS  PRECONDITIONING CHARGE STATE

During the SMBus preconditioning charge state, the charger will be operating in the preconditioning charge current limit. The following conditions  must be met in order to allow SMBus preconditioning charge to start:

•  The ChargingVoltage() AND ChargingCurrent() registers must be written to non-zero  values. The LTC4110 will not directly report the status of these registers. The battery needs only write one pair of ChargingVoltage() and ChargingCurrent() registers to stay in this state. The tTIMEOUT timer is not operational in SMBus precondition-

ing charge state.

•  The SafetySignal must be RES_COLD, RES_IDEAL, or

RES_UR.

•  AC must be present and sufﬁcient. This is qualiﬁed by

DCDIV > VAC + VACH and DCIN > UVLO.

The following conditions  will affect the SMBus precondi- tioning charge state as speciﬁed below:

•	The SafetySignal is registering  RES_HOT. Charge  is stopped; the SMBus  OFF state is entered.

4110fb

OPERATION

•	The SafetySignal is registering  RES_OR. Charge is stopped.  The LTC4110 enters the reset state.

•	The AC power  is no longer present (DCDIV < VAC or

DCIN < UVLO). The LTC4110 enters the reset state.

•	ALARM_INHIBITED  is set in the ChargerStatus() register. Charge is stopped. The LTC4110 enters the SMBus  OFF state.

•	CHARGE_INHIBIT is set in the BBuControl() register.

Charge is stopped, however, the T/4 timer is not paused. Clearing  CHARGE_INHIBIT will enable the LTC4110 to resume charge.

•	RESET_TO_ZERO is set in the BBuControl() register.

Charge is stopped. The LTC4110 enters the SMBus

OFF state.

•	Writing a zero value to ChargeVoltage() or ChargeCur- rent() register. Charge is stopped. The LTC4110 enters the SMBus  OFF state.

•	If the battery voltage exceeds the bulk charge threshold, the LTC4110 will enter the SMBus bulk charge state.

•	If the T/4 timeout occurs, charge is stopped and the

LTC4110 enters the SMBus  OFF state.

•	The CAL_START bit in the BBuControl() register is set. Charge is stopped and the LTC4110 enters the calibration mode.

SMBus BULK CHARGE STATE

The charger  will be operating in the bulk charge current limit during the SMBus bulk charge state. The following conditions must be met in order to allow SMBus bulk charge to start:

•  The ChargeVoltage()  AND ChargeCurrent()  registers must be written to non-zero values. The LTC4110 will not directly report the status of these registers.

•  The SafetySignal must be RES_COLD, RES_IDEAL, or

RES_UR.

•  AC must be present and sufﬁcient. This is qualiﬁed by

DCDIV > VAC + VACH and DCIN > UVLO.

The following conditions will affect the SMBus bulk charge state as speciﬁed below:

•  The ChargeCurrent()  AND ChargeVoltage()  registers have not been  written  for tTIMEOUT. Charge  is stopped and the LTC4110 enters the SMBus  OFF state.

•  The SafetySignal is registering  RES_OR. Charge is stopped and the LTC4110 enters the reset state.

•  The SafetySignal  is registering RES_HOT.  Charge is stopped and the LTC4110 enters the SMBus  OFF state.

•  The AC power  is no longer present (DCDIV < VAC  or DCIN < UVLO). Charge  is stopped and the LTC4110 enters the reset state.

•  ALARM_INHIBITED is set in the ChargerStatus() register.

Charge is stopped and the LTC4110 enters  the SMBus

OFF state.

•  CHARGE_INHIBIT is set in the BBuControl() register.

Charge is stopped. Clearing CHARGE_INHIBIT will en- able the LTC4110 to resume  charge. The tTIMEOUT timer does not pause when  CHARGE_INHIBIT is set.

•  RESET_TO_ZERO is set in the BBuControl() register.

The LTC4110 enters the SMBus  OFF state.

•  Writing a zero value to the ChargeVoltage() or to the ChargeCurrent() register. Charge is stopped and the LTC4110 enters the SMBus  OFF state.

•  The CAL_START bit in the BBuControl() register is set.

Charge is stopped  and the LTC4110 enters the calibra- tion mode.

SMBus  OFF STATE

This state is different from the reset state in that all charge is disallowed regardless of the value of the thermistor.  The following conditions  will affect the SMBus  OFF state as speciﬁed below:

•  The ChargeCurrent()   AND ChargeVoltage()   regis- ters have both been written to non-zero values, the battery thermistor is registering RES_COLD, RES_ IDEAL or RES_UR and CHARGE_INHIBT is clear. The LTC4110  enters the SMBus preconditioning  charge state.

4110fb

OPERATION

•  The CAL_START bit in the BBuControl() register is set.

The LTC4110 enters the calibration state.

•  The battery thermistor is registering  RES_OR. The

LTC4110 enters the reset state.

LEAD ACID BATTERY CHARGE MODE

The charger is programmed for lead acid batteries by con- necting the TYPE pin to a voltage derived from the VREF pin resistor divider of nominally 0.5 • VREF. During charge, the LTC4110 operates  as a high efﬁciency, synchronous, PWM ﬂyback battery charger with constant current and constant ﬂoat voltage regions of operation. The constant- charge current is programmed by the combination of a resistor (RCHG) from the ICHG  pin to ground, a battery current sense resistor  (RSNS) and CSP/CSN pin resistors. The ﬂoat voltage is programmed to one of four values (4.7V, 7.05V, 11.75V, 14.1V) depending on the number of series cells (2, 3, 5 or 6) using the SELC pin and can be adjusted ±0.15V/cell with the VCHG pin.

A new charge cycle begins with the charger in the bulk charge current limited state. In this state, the charger is a current source providing a constant charge rate and the CHGb (GPIO1 pin) is forced  low. No time limits are placed upon lead acid battery charge. The charger monitors the battery voltage and as it reaches the ﬂoat voltage the charger begins its ﬂoat charge. While in ﬂoat, the charge current diminishes as the battery accepts charge. Float voltage temperature compensation and temperature fault monitoring, if desired, are accomplished with an external thermistor network.

Charge is active when the supply input is present as sensed by the DCDIV  pin and DCIN  above UVLO,  thermistor temperature is ideal according to the thermistor monitor circuit (see SafetySignal Decoder) and the charge register bit CHARGE_INHIBIT is cleared. Soft-start ramps the charge current at a rate set by the capacitor on the ITH pin. When

charge begins, the CHGb (GPIO1  pin) status output is

forced to GND. At any time charge can be forced to stop by pulling the SHDN pin high or setting the CHARGE_INHIBIT bit high through the SMBus.

If the battery voltage exceeds 107.5% (VBOV) of the pro- grammed ﬂoat voltage during any stage of charge, the

charger pauses until the voltage drops below the hysteresis

(VBOVH). No fault is indicated.

An optional external NTC thermistor network can be used to provide an adjustable negative TC for the ﬂoat voltage, monitor battery temperature and to detect battery pres- ence. If the thermistor value indicates a hot temperature, voltage falling to VHOT on THB pin, charge current is halted, CHG_FLTb (GPIO3 pin) is forced low and the CHG_FLT bit will be set high. CHGb (GPIO1 pin) and CHG_STATE_0 and CHG_STATE_1 register bits will not be affected. When the thermistor value returns to ideal when the voltage exceeds VHOT  +VHOTH  on THB  pin, charge resumes CHG_FLTb (GPIO3 pin) returns  to high impedance and the CHG_FLT bit will be reset low. An open thermistor indicates an over-range which is considered absence of a battery. Low temperature  is not monitored. However, since bat- tery removal detection looks at the thermistor for a high resistance (VREM on THB pin), extremely cold temperatures may result in an indication of battery absence. To defeat the temperature monitoring register, replace the thermis- tor with a resistor to indicate normal battery temperature. When a thermistor is not used the resistor circuit must be routed through the battery connector  if battery presence detection is required.

BATTERY CALIBRATION MODE

Figure 6 shows the LTC4110 in battery calibration mode and the corresponding PowerPath enabled. During calibration, the host CPU can calibrate a gas gauge or verify the battery’s ability to support a load by use of a low heat producing method. Calibration requires a host to communicate over a SMBus. In the low heat method, a synchronous PWM ﬂyback charger is used in reverse to discharge the battery with a programmable constant-current into the system load thereby saving space and eliminating heat generation compared with resistive loads. Protection circuits prevent accidental overdrive back into the power source if the system load is insufﬁcient. The constant-charge  current is programmed by the combination of a resistor (RCAL) from  the ICAL pin to ground, a battery current sense resis- tor (RSNS(BAT)) and CSP/CSN pin resistors.  Calibration is initiated by setting the CAL_START bit in the BBuControl() register. The CAL_ON bit in the BBuStatus() register will

4110fb

OPERATION

PWM STOPPED (BATTERY OVP)

7	8

RESET

1

4		ANY CHARGE STATE

2	9

ANY CHARGE STATE

RESUME CHARGE STATE

CHARGE

3

STOP

4110 F05

5	6

STOP

(OVERTEMPERATURE)

Figure 5. SLA Charge State Diagram (Does Not Include Calibration)

| # | Logic | Event (T = True, F = False) [Notes] | Notes and/or Actions (T = True, F = False) |
| --- | --- | --- | --- |
| 1 |  | RES_OR = F & DCDIV pin = T & SHDN pin = F & CHARGE_INHIBITED = F & CHG_FLT = F | ICHG = On & CHG = T |
| 2 | Or | VAR = T [AutoRestart] ChargingVoltage() & ChargingCurrent() ≠ 0 sent | ALARM_INHIBITED = F |
| 3 | Or Or | ChargingVoltage() or ChargingCurrent() = 0 sent RESET_TO_ZERO = T [See ChargeMode()] CHARGE_INHIBIT = T [See ChargeMode()] | ICHG = Off & CHG = F |
| 4 | Or Or Or Or | DCDIV pin = F RES_OR = T [Bat Removed, See ChargeStatus()] SHDN pin = T VUVD = T POR_RESET = T [See ChargeMode()] | ICHG = Off & CHG = F & CHARGE_INHIBITED = F & ALARM_INHIBITED = F |
| 5 |  | RES_HOT = T & RES_UR = F [See ChargeStatus()] | ICHG = Off & CHG_FLT = T |
| 6 |  | RES_HOT = F [See ChargeStatus()] | ICHG = On & CHG_FLT = F |
| 7 |  | VBOV = T | PWM stopped. Timers remain running. |
| 8 |  | VBOV = F | PWM restarted. |
| 9 | Or Or Or | AlarmWarning() command is sent by Smart Battery over SMBus with any of the following bits set to True: OVER_CHARGED_ALARM TERMINATE_CHARGE_ALARM Reserved ALARM OVER_TEMPERATURE_ALARM | ICHG = Off & CHG = F & ALARM_INHIBITED = T (ALARM_INHIBITED bit is found in ChargeStatus()) |

Note: For all charge states, VCHG is always active

be set to indicate calibration in progress. Soft-start ramps the discharge current at a rate set by the capacitor on the ITH pin (typically 10ms with 0.1μF capacitor).  A limit to how far the battery  cell voltage will be discharged during calibration can be programmed with the VCAL pin (Li-Ion default = 2.75V/cell, lead acid default = 1.93V/cell, Smart NiMH/NiCd default = 0.95V/cell). When the limit is reached calibration is terminated, the CAL_COMPLETE bit in the BBuStatus() register is set, the CAL_ON bit in the BBuS- tatus() register will be cleared and the charge mode is

automatically entered to begin recharging the battery.  If the GPIO3 is conﬁgured as a calibration complete status output (CAL_COMPLETEb), it will be forced low until reset by the CAL_RESET write bit. Calibration is inhibited during backup or shutdown modes. Calibration is also inhibited when a thermistor is sensed absent.

During calibration, user-programmable supply back-drive protections are provided. These protections prevent a reversal of current into the main supply and/or possibly raising the supply voltage to unsafe levels should the

4110fb

OPERATION

system load not be adequate to absorb the current. The primary protection is accomplished with an external current sense resistor (RCL), connected between the CLP and CLN pins, through which the system load current ﬂows. When the voltage across the resistor reaches 10mV (IBDT) or less, representing a low forward current, calibration mode

DCIN

SYSTEM LOAD

BACKUP LOAD

ON

OFF	OFF

BATTERY

is terminated. The current protection can be completely disabled by connecting both CLP and CLN pins to GND. As an alternative where RCL sensing is not an option for

CURRENT FLOW

INID	BATID

the application, a secondary method is accomplished by monitoring the supply voltage through the DCDIV pin. Once  the  DCDIV pin voltage  goes  above  VOVP, calibration mode is terminated. In either case, the CAL_FLT register is

LTC4110

CHGFET

DCHFET

4110 F06

set high and the charge mode is automatically entered to begin recharging the battery. Both of these protections are automatically disabled when not in calibration. However, in calibration,  one or the other of these two protective methods should be used. You can optionally do both. Failure to implement any form of protection can result in destructive voltages being generated in the application.

If the calibration cycle fails due to loss of the main power source a fault condition results that sets the CAL_FLT register bit and backup mode is entered.

An optional  external  thermistor network is sampled at regular intervals to monitor battery temperature and to detect battery presence.  If the thermistor value indicates a temperature outside of ideal limits (hot or over-range) the calibration current is halted and the CAL_FLT bit will be set high. When the thermistor value returns to an ac- ceptable value (under-range, cold or ideal), charge mode is automatically entered to begin recharging the battery. Calibration can be restarted by clearing the CAL_FLT bit and sending another CAL_START command.

An open thermistor (over-range) indicates absence of a battery. To defeat the temperature monitoring function, re- place the thermistor with a resistor to indicate ideal battery temperature. When a thermistor is not used, the resistor circuit must be routed through the battery  connector  if battery presence detection is required. If the battery should be removed  during calibration, calibration will terminate and the CAL_FLT read bit will be set high.

Figure 6. Calibration Mode Operation

The  CAL_FLT bit can be cleared by writing a one to the CAL_RESET or POR_RESET registers, or by forcing the SHDN pin high.  The CAL_FLT bit is not cleared by remov- ing and reapplying the supply input if the battery has maintained sufﬁcient voltage (VBAT ≥ 2.7V).

Calibration  can start only if  the CAL_FLT  bit in the BBuStatus() register is clear. Once the LTC4110 is in cali- bration state, the following events  will stop calibration:

•  BKDRV is sensed. The CAL_FLT bit is set.

•  A HOT thermistor is sensed. The CAL_FLT bit is set.

•  Loss of battery presence is sensed. The CAL_FLT bit is set.

•  The calibration  cutoff threshold has been reached.

The CAL_COMPLETE bit is set. The LTC4110 will start charging based upon the TYPE and SELA pins.

• An OVER_TEMP_ALARM, RESERVED_ALARM, or TERMINATE_DISCHARGE_ALARM  bit in the Alarm- Warning() register is set. The  CAL_FLT bit is set. The LTC4110 will start charging.

•  Loss  of AC presence. The CAL_FLT bit is set.

SHUTDOWN MODE

The LTC4110  can be forced into either a micropower shutdown state or an all logic register reset state with the SHDN pin.

4110fb

OPERATION

RESUME STATE

ANY STATE

4 (CALIBRATION COMPLETED)

CALIBRATION

RUNNING

7		8 (CALIBRATION FAULT)

6, 9	2	3

CALIBRATION NO	MODE?

YES

RESET

CALIBRATION

1	START

5 (BATTERY IS DEAD)

Figure 7. Calibration State Diagram

4110 F07

| # | Logic | Event (T = True, F = False) [Notes] | Notes and/or Actions (T = True, F = False) |
| --- | --- | --- | --- |
| 1 |  | RES_OR = F & DCDIV pin = T & SHDN pin = F & CAL_FLT = F & CAL_START = T | CAL_COMPLETE = F (Calibration started while in Reset {Idle or Cold Power-Up}) |
| 2 |  | RES_OR = F & DCDIV pin = T & SHDN pin = F & CAL_FLT = F & CAL_START = T | ICHG or IPPC = Off & All Timers = Reset & CAL_COMPLETE = F (Calibration was initiated while in any mode other than Reset.) |
| 3 |  | [Calibration Automatically Started] | ICAL = ON & CAL_ON = T |
| 4 |  | VBAT < VCAL [Battery has reached Discharge] | ICAL = Off & CAL_ON = F & CAL_COMPLETE = T (Normal Calibration Cycle) |
| 5 |  | VBAT < VCAL [Battery is Discharged] | CAL_COMPLETE = T (Battery is already discharged. Cancel Calibration.) |
| 6 |  | AlarmWarning() command is sent by Smart Battery over SMBus with any of the following bits set to True: OVER_TEMP_ALARM or Reserved ALARM or TERMINATED_DISCHARGE_ALARM] | ICAL = Off & CAL_ON = F & ALARM_INHIBITED = T (ALARM_INHIBITED bit is found in ChargeStatus()) |
| 7 |  | CAL_RESET = T | ICAL = Off & CAL_ON = F & CAL_COMPLETE = F & CAL_FLT = F |
| 8 | Or Or Or | RES_HOT = T & RES_UR = F [See ChargeStatus()] RES_OR = T [Bat Removed, See ChargeStatus()] VOVP = T [Output Over-Voltage condition sensed)] IBDT = T [Output Back Drive Current condition sensed)] | ICAL = Off & CAL_ON = F & CAL_FLT = T |
| 9 | Or Or Or | DCDIV pin = F SHDN pin = T VUVD = T POR_RESET = T [See ChargeMode()] | ICAL = Off & CAL_ON = F & ALARM_INHIBITED = F & CHARGE_INHIBITED = F |

REGISTER RESET STATE

The SHDN pin will reset all logic registers when taken high, but only if DCIN is present as determined by DCDIV > VAC

+ VACH  and DCIN above UVLO. Micropower   shutdown

state will not be entered, but the LTC4110 will be idle and

not able to enter charge or calibration modes. If SHDN is switched low then normal  operation  will resume.

While in register reset state, charge and calibration modes are inhibited, and all registers including the backup fault bit register are set to their default states and the internal timer is reset. The status pin ACPb is active, but GPIO1, GPIO2

and GPIO3 are reset to their default states. The SMBus is enabled, however, it is not able to communicate with the LTC4110. The DCIN to DCOUT PowerPath controller is functional and the VDD and VREF pin voltages remain.

MICROPOWER SHUTDOWN STATE

If the SHDN pin remains  high when DCIN is removed as detected by the undervoltage lockout UVLO (see VUVD), micropower shutdown is entered, battery backup mode is inhibited and all registers are reset. During this condition, the level of the SHDN pin is ignored and has no effect.

4110fb

OPERATION

The micropower shutdown state will be maintained  if the DCIN supply  is removed and sufﬁcient battery voltage is present (VBAT ≥ 2.7V). When DCIN is reapplied as detected

+ VSNS

+

RCSP1

RSNS(BAT)

RCSP2

+

CSP

CSN

VICHG =

by the UVLO (see VUVI), regardless of the level of the	– 	–

RICHG/(RCSP1 + RCSP2)*VSNS

SHDN pin, the shutdown state is automatically cancelled. Register reset state is cancelled until DCIN is reapplied as

RCSN1

+

RCSN2		INPUT CURRENT

AMPLIFIER

CURRENT

ICHG

RICHG

determined by the DCDIV pin.

5V IISD

SHDN

LTC4110

SHUTDOWN

4110 F08

BAT

+

–

VFB

– VOLTAGE LOOP EA

+

REFERENCE

+	VOLTAGE

–	ADJUSTED  BY

VCHG PIN

ITH

LOOP EA

BANDGAP

+

–

4110 F09

Figure 8. Shutdown  Control Input

In shutdown; charge, calibration and backup modes are inhibited, all registers are set to their default states (with exception of the backup fault bit register), the internal timer is reset and oscillator disabled, the status pins; ACPb, GPIO1, GPIO2 and GPIO3 are a high impedance and the LTC4110 is put into a micropower state. While in shutdown the SMBus is disabled and the SDA and SCL pins are high impedance. In addition, the shutdown state will disconnect loads from the battery to prevent its discharge as follows:

•  The BATID pin is forced to the battery voltage to turn off the battery P-MOSFETs for isolation of the load from the battery

•  The CHGFET and DCHFET pins are forced to GND to turn off the ﬂyback switcher N-MOSFETs

•  Current into the BAT pin is minimized. Also the VDD and

VREF pin voltages  will fall to zero.

While in shutdown, the LTC4110 will draw a small current from battery (IBSD) if the DCIN supply is absent. If the SHDN pin is open an internal weak pull-up current (IISD) pulls the pin voltage up thereby entering the shutdown state.

PWM OPERATION

A conceptual  diagram of the LTC4110 PWM engine is shown in Figure 9.

+

Figure 9. LTC4110 PWM Engine

The voltage across the external current programming resistor RSNS(BAT) is averaged by the RC network  con- nected to the CSP and CSN pins and then ampliﬁed by a ratio of RICHG/(RCSP1 + RCSP2). This ampliﬁed voltage is compared with the bandgap reference through the cur- rent loop error ampliﬁer to adjust the ITH pin which sets the current comparator threshold to maintain a constant charging current. Once the battery voltage rises to close to the programmed ﬂoat voltage, the voltage loop error ampliﬁer gradually pulls the ITH pin low, reduces the charg- ing current and maintain a constant voltage charging.

C/x CHARGE TERMINATION

LTC4110 monitors  the charging current through  the volt- age on the ICHG pin, once the current drops below 20% of the bulk charging current, an internal C/x comparator  is tripped, and the LTC4110 will enter top-off charge stage if standard  Li-Ion battery mode is selected or release the GPI01 pin if no-host SLA battery  mode is selected. The actual x value depends on the programmed charging cur- rent and the C rate of the battery.

x =	C	•5

ICHG

4110fb

OPERATION

Where:

C = C rate of the battery

ICHG = Programmed charging current

For Example, if we charge  a 3Ah battery with 1A current,

RTHA

1.13k

THA

VINT

THA_SELB

MUX

HI_REF

+	TH_HI

–

+	TH_LO

then x = 15.

VINT

REF  LO_REF	–

SAFETYSIGNAL DECODER

CSS

RTHB

54.9k

THB

THB_SELB

SafetySignal

CONTROL

RES_OR RES_COLD

| Table 3. SafetySignal State Ranges (Except SLA) SafetySignal CHARGE RESISTANCE | CHARGE STATUS BITS | DESCRIPTION |
| --- | --- | --- |
| 0Ω to 500Ω | RES_UR, RES_HOT, BATTERY_PRESENT | Under range |
| 500Ω to 3k | RES_HOT, BATTERY_PRESENT | Hot |
| 3kΩ to 30k | BATTERY_PRESENT | Ideal |
| 30k to 100k | RES_COLD, BATTERY_PRESENT | Cold |
| Above 100k | RES_OR, RES_COLD | Overrange |

RSafetySignal

LATCH

RES_HOT RES_UR

Figure 10. Battery Safety Decoder (Except SLA)

4110 F10

Note: The under range detection scheme is a very important feature of the LTC4110. The RTHA/RSafetySignal divider trip point of 0.307 • 4.75V = 1.46V is well above the 0.047 • VDD = 140mV  threshold of a system using a 10k pull-up.  A system using a 10k pull-up would not be able to resolve the important under range to a hot transition point with a modest 100mV of ground offset between battery and SafetySignal detection circuitry. Such offsets are anticipated when charging at normal current levels.

Table 4. SafetySignal for SLA (7.256k Between THA and  THB)

| SafetySignal CHARGE RESISTANCE | CHARGE STATUS BITS | DESCRIPTION |
| --- | --- | --- |
| 0Ω to 3.1k | RES_HOT, BATTERY_PRESENT | Hot |
| 3.1k to 114k | BATTERY_PRESENT | Ideal |
| 114k | RES_COLD, RES_OR | Battery Removal |

This decoder measures the resistance of the SafetySignal and features high noise immunity at critical trip points. The SafetySignal decoder is shown in Figure 10.

The value of RTHA is 1.13k and RTHB is 54.9k.  SafetySignal sensing is accomplished by a state machine that reconﬁgures the switches of Figure 10 using THA_SELB and THB_SELB, a selectable reference generator, and two comparators. The state machine successively samples the SafetySignal value starting with the RES_OR ≥ RES_COLD threshold,

then RES_C0LD ≥ RES_IDEAL threshold, RES_IDEAL ≥ RES_HOT threshold,  and ﬁnally the RES_HOT ≥ RES_UR threshold. Once the SafetySignal range is determined, the lower value thresholds are not sampled. The SafetySignal decoder block uses the previously determined SafetySignal value to provide the appropriate adjustment in threshold to add hysteresis. The RTHB resistor value is used to measure the RES_OR ≥ RES_COLD and RES_COLD ≥ RES_IDEAL thresholds by connecting the THB pin to an internal voltage and measuring the voltage resultant on the THA pin. The RTHA resistor value is used to measure the RES_IDEAL ≥ RES_HOT and RES_HOT ≥ RES_UR thresholds  by con- necting the THA pin to the internal voltage and measuring the resultant voltage on the THB pin. The SafetySignal impedance is interpreted according to Table 3.

When the DCIN supply  is present, a full sampling  of the SafetySignal is performed every 27ms. When the supply is absent, a low power limited sampling of the SafetySignal is performed every 218ms. A full sampling of the thermistor state is performed only if a change of battery presence is detected when the supply is not present.

GPIO AND STATUS FUNCTIONS

All of the GPIO pins are open drain with N-MOSFET drivers capable of sinking current sufﬁcient to drive an LED (see VOL). The pins are not capable of sourcing any current and instead enter a Hi-Z mode  when the output is not low. An external  pull-up  will be required to create any high output logic state.

4110fb

OPERATION

The three I/O outputs, GPIO1, GPIO2 and GPIO3 are digital

I/O pins with two modes of operation.

1) General Purpose  I/O

2) Status Reporting

A host can set the mode of each I/O pin with each I/O pin’s setting independent of the others such that any combination of status reporting or bit I/O can be implemented. Only a UVLO or a SHDN event will change  the GPIO_n_EN bits back to default values. If you enable a GPIO pin to report status output, it overrides the GPIO_n_OUT setting. In addition, the LTC4110 supports a special power up mode of status reporting  on all 3 IO pins for standalone applica- tions where it is assumed  “no host” exists. This power up status mode is enabled  if the SELA pin is set to 0.5 • VREF voltage as developed from VREF pin resistor divider. This mode does not actually disable the SMBus in any way and if a host does exist in this power up mode, the host can reprogram the I/O settings  at any time.

All GPIO pins operate as digital inputs at all times regard- less of the pin settings with pin state reported on the GPIO_n_IN bits in the BBuStatus() register. However to actually read digital input data from an external device, you must disable the GPIO_n_EN bit. Otherwise  the input will simply reﬂect the output state assuming external powered pull-ups exist.

There are a total of 5 status signals possible. CHGb, C/xb, BKUP-FLTb, CHG_FLTb, and CAL_COMPLETEb. Each  of these signals is asserted low on the output when they are true. CHGb is an asserted low signal when either CHG_STATE_0 or CHG_STATE_1 is set to one. C/xb is asserted low signal when C/x state in the charge cycle is reached. This status signal is only available if the TYPE pin is set to SLA mode and replaces the CHGb status output. BKUP_FLTb is asserted low when the BKUP_FLT bit is set to one in the BBuStatus() register. BKUP_FLT is a sticky bit that is designed to be cleared primarily through the setting of the BUFLT_RST bit in the BBuControl() register. The value of this bit does not inhibit charging or calibration functions. CHG_FLTb is asserted low when the CHG_FLT bit is set to one in the BBuStatus() register. CAL_COMPLETEb bit is asserted low when the conditions of successful calibra- tion cycle are met. CAL_COMPLETEb status output can be used as an interrupt to a host for the purpose of help implementing a simple gas gauge function or capacity veriﬁcation function with a standard battery. However, if the LTC4110 is set up in no host mode, CAL_COMPLETEb as a status signal is not considered usable since it is assumed there is no host to enable calibration mode. Therefore the CHG_FLTb signal  is substituted for CAL_COMPLETEb as the status output signal. Table 5 describes the speciﬁc modes and status signal options  of each GPIO pin.

Table 5a. GPIO1 Modes

| HOST PROGRAMMED BIT SETTINGS |  |  | GPIO_1 MODE | DATA | NOTE |
| --- | --- | --- | --- | --- | --- |
| GPIO_1_EN | GPIO_1_OUT | GPIO_1_CHG |  |  |  |
| 0 | 0 | 0 | Digital Input | Input Data | GPIO_1_IN |
| 1 | X | 1 | Status Output | CHGb | With Pull-Up |
| 1 | 0 | 0 | Digital Output | 0 | With Pull-Up |
| 1 | 1 | 0 | Digital Output | 1 | With Pull-Up |

Table 5b. GPIO1 Power  Up Mode  (SELA = 0.5 • VREF)

| FORCED BIT SETTINGS |  |  | TYPE = SLA | GPIO_1 MODE | DATA | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| GPIO_1_EN | GPIO_1_OUT | GPIO_1_CHG |  |  |  |  |
| 1 | X | 1 | 0 | Status Output | CHGb | With Pull-Up |
| 1 | X | 1 | 1 | Status Output | C/xb | With Pull-Up |

4110fb

OPERATION

Table 5c. GPIO2 Modes

| HOST PROGRAMMED BIT SETTINGS |  |  | GPIO_2 MODE | DATA | NOTE |
| --- | --- | --- | --- | --- | --- |
| GPIO_2_EN | GPIO_2_OUT | GPIO_2_BUFLT |  |  |  |
| 0 | 0 | 0 | Digital Input | Input Data | GPIO_2_IN |
| 1 | X | 1 | Status Output | BKUP_FLTb | With Pull-Up |
| 1 | 0 | 0 | Digital Output | 0 | With Pull-Up |
| 1 | 1 | 0 | Digital Output | 1 | With Pull-Up |

Table 5d. GPIO2 Power  Up Mode  (SELA = 0.5 • VREF)

| FORCED BIT SETTINGS |  |  | GPIO_2 MODE | DATA | NOTE |
| --- | --- | --- | --- | --- | --- |
| GPIO_2_EN | GPIO_2_OUT | GPIO_2_ BUFLT |  |  |  |
| 1 | X | 1 | Status Output | BKUP_FLTb | With Pull-Up |

Table 5e. GPIO3 Modes

| HOST PROGRAMMED BIT SETTINGS |  |  | GPIO_3 MODE | DATA | NOTE |
| --- | --- | --- | --- | --- | --- |
| GPIO_3_EN | GPIO_3_OUT | GPIO_3_CAL |  |  |  |
| 0 | 0 | 0 | Digital Input | Input Data | GPIO_3_IN |
| 1 | X | 1 | Status Output | CAL_COMPLETEb | With Pull-Up |
| 1 | 0 | 0 | Digital Output | 0 | With Pull-Up |
| 1 | 1 | 0 | Digital Output | 1 | With Pull-Up |

Table 5f. GPIO3 Power  Up Mode  (SELA = 0.5 • VREF)

| FORCED BIT SETTINGS |  |  | GPIO_3 MODE | DATA | NOTE |
| --- | --- | --- | --- | --- | --- |
| GPIO_3_EN | GPIO_3_OUT | GPIO_3_ CAL |  |  |  |
| 1 | X | 1 | Status Output | CHG_FLTb | With Pull-Up |

SMBUS INTERFACE

All communications over the SMBus are interpreted by the SMBus interface block. The SMBus interface is a SMBus slave device. All internal LTC4110 registers may be updated and accessed through the SMBus interface as required. The SMBus protocol is a derivative of the I2C-BusTM. (For a complete description of the bus protocol requirements, reference “The I2C-Bus and How to Use It, V1.0” by Phil- ips®, and “System Management Bus Speciﬁcation, Version

1.1,” from the SMBus organization).  See Table 6: Register

Command Set Description and Table 7: Summary of Sup- ported SMBus Functions, for complete details.

All data is clocked into the shift register on the rising edge of SCL. All data  is clocked out of the shift register on the falling edge of SCL. Detection of an SMBus Stop condi- tion, or power-on reset will reset the SMBus interface to an initial state at any time. The LTC4110 command  set is interpreted by the SMBus interface and passed onto the charger controller block as control signals or updates to internal registers. Smart battery charge commands are

4110fb

OPERATION

processed to allow compliance with smart battery charge and discharge termination and protection control. How- ever, there is no actual value processing of the voltage or current charge commands. IC will acknowledge all smart battery write commands, but process only a subset of them.  Full SMBus error and reset handling is supported. The SMBus remains functional during backup mode, but not in SHDN mode.

The LTC4110 SMBus  address can be changed when standard batteries are used to facilitate redundant backup systems. Connect  SELA pin to GND for 12h,  VDD for 28h and VREF for 20h.  When  a smart battery is selected by the TYPE pin the SELA pin must be connected to GND to select address 12h. Note: Although there are only 7 address bits for SMBus, the above addresses shown follow the smart battery convention of including the Read/Write bit as part of the address value. The Read/Write bit becomes the LSB of the SMBus address with the Read/Write bit value assumed to be a 0 value.

If multiple LTC4110s with smart batteries are to be used, each LTC4110 must be SMBus isolated from all other LTC4110s so the main bus or host bus can only see one LTC4110 and its corresponding smart battery at a time. Failure to do so will cause multiple LTC4110s and smart batteries responding to a single host query resulting in errors. There are multiple channel SMBus multiplexer ICs such as the LTC4305 and LTC4306 to help implement the required isolation. Furthermore, if a given SMBus is high in SMBus device count or long in length, you may want to consider using SMBus accelerators.  The above ICs listed support that option.

If the SMBus is not used or to force all GPIOs to status mode upon power-up, connect SELA to a typically 0.5 • VREF voltage  from  VREF pin resistor divider. The SMBus address then, if used, will be 12h.

Pull-ups are required on the SDA and SCL pin such that when they are not being used, they are in a default high state that means no bus activity. The pull-up voltage need only be high enough to satisfy the logic high threshold. Tying the pins low is a valid state on the SMBus that means anything but the bus is free. This state will force the LTC4110’s internal SMBus state machine to reset itself because it thinks the SMBus is hung.

The LTC4110 does not support or respond to the following

SMBus V1.1 timing speciﬁcations:

a) TTIMEOUT (This is not to be confused with the

LTC4110’s tTIMEOUT speciﬁcation.)

b) TLOW:SEXT

c) TLOW:MEXT

The above speciﬁcations  have to do with detecting bus hangs or SMBus devices that are taking too long to reply using clock stretching and slowing down the SMBus bandwidth. The LTC4110 is a slave only device that does not do any clock stretching and works all the way up to maximum 100kHz bus speed. It will not hang the bus. The design will always  reset its SMBus interface upon receiving an SMBus Start Bit or a Stop Bit regardless  of the prior state of the bus.

4110fb

OPERATION

Table 6. Register Command Set Descriptions (XxxxXxxx() – Register Byte, XXXXXXXX – Status Bit)

LABEL	DESCRIPTION

ChargerStatus() – Read Only. The SMBus host uses this command to read the LTC4110’s charge status bits.

| AC_PRESENT | Set to 1 when sufﬁcient input voltage (DCDIV > VAC + VACH and DCIN above UVLO) available and switches load from battery to main supply. Zero indicates backup mode engaged. |
| --- | --- |
| BATTERY_PRESENT | BATTERY_PRESENT is set if a battery is present, otherwise it is cleared. The LTC4110 uses the SafetySignal to determine battery presence. If the LTC4110 detects a RES_OR condition, the BATTERY_PRESENT bit is cleared immediately. The LTC4110 will not set the BATTERY_PRESENT bit until it successfully samples the SafetySignal twice and does not detect a RES_OR condition on either sampling. If AC is not present (DCDIV < VAC or DCIN below UVLO), this bit may not be set for up to one-half second after the battery is connected to the SafetySignal. The ChargingCurrent() and ChargingVoltage() register values are immediately cleared whenever this bit is cleared. Charging will never be allowed if this bit is cleared. |
| ALARM_INHIBITED | ALARM_INHIBITED bit is set if a valid AlarmWarning() message has been received and charging is inhibited as a result. This bit is cleared if POR_RESET is set, both ChargingVoltage() and ChargingCurrent() are rewritten to the LTC4110, the power is removed (DCDIV < VAC or DCIN below UVLO), the SHDN pin is set high, or if a battery is removed. |
| RES_UR | Set to 1 when NTC pin is below 500Ω typical. This bit is never set when TYPE pin selects SLA battery.. |
| RES_HOT | The RES_HOT bit is set only when the SafetySignal resistance is less than 3kΩ (3.1kΩ for SLA) typical, which indicates a hot battery. The RES_HOT bit will be set whenever the RES_UR bit is set. |
| RES_COLD | The RES_COLD bit is set only when the SafetySignal resistance value is greater than 30kΩ typical. The SafetySignal indicates a cold battery. The RES_COLD bit will be set whenever the RES_OR bit is set. This bit is the same as RES_OR for SLA. |
| RES_OR | The RES_OR bit is set when the SafetySignal resistance value is above 100kΩ (114kΩ for SLA) typical. The SafetySiganl indicates an open circuit. |
| LEVEL:3/LEVEL:2 | The LTC4110 always reports itself as a Level 2 Smart Battery Charger. |
| CHARGE_INHIBITED | Indicates charge inhibited is enabled when set to a one. This is a duplicate of the CHARGE_INHIBIT bit in the BBuStatus() register. |

ChargingCurrent() – Write Only. The battery, system  host or other master device sends the desired charging current to the LTC4110.

ChargingCurrent()	LTC4110 only monitors  for zero or non-zero values. A value of zero will stop the charger. A non-zero  value here,  and for ChargingVoltage(),  will restart the charger.

ChargingVoltage() – Write Only. The Battery, System  Host or other master device sends the desired charging voltage to the LTC4110.

ChargingVoltage()	LTC4110 only monitors  for zero or non-zero values. A value of zero will stop the charger. A non-zero  value here,  and for ChargingCurrent(), will restart the charger.

AlarmWarning() – Write Only. The Smart Battery, acting as a bus master device, sends the AlarmWarning() message to the LTC4110 to notify it that one or more alarm conditions exist. Alarm indications are encoded as bit ﬁelds in the battery’s status register, which is then sent to the LTC4110 by this function. Only the OVER_CHARGED_ALARM, TERMINATE_CHARGE_ALARM,RESERVED_ALARM, OVER_TEMP_ALARM and TERMINATE_DISCHARGE_ALARM bits are supported by the LTC4110.  The ALARM_INHIBITED bit in the ChargerStatus() register indicates whether a charging process or a calibration process was halted by a write to this register.

| OVER_CHARGED_ALARM | Set to one indicates battery has been overcharged and stops charge. Setting this bit will only stop a charging process (default = zero). |
| --- | --- |
| TERMINATE_CHARGE_ALARM | Set to one indicates battery requesting charge termination. Setting this bit will only stop a charging process (default = zero). |
| RESERVED_ALARM | Set to one for reserved alarm condition. Setting this bit will stop both a calibration process and a charging process (default = zero). |

4110fb

OPERATION

| LABEL | DESCRIPTION |
| --- | --- |
| OVER_TEMP_ALARM | Set to one indicates battery is temperature is out of range. Setting this bit will stop both a calibration process and a charging process (default = zero). |
| TERMINATE_DISCHARGE_ALARM | Set to one indicates battery requesting discharge termination. Smart battery only. Setting this bit will only stop a calibration process (default = zero). |

BBuStatus() – Read Only. The SMBus host uses this command to read the LTC4110’s status bits.

| CAL_ON | Set to one indicates calibration in progress to discharge the battery. |
| --- | --- |
| CAL_COMPLETE | Set to one indicates calibration process is complete. Can be used as a battery capacity indicator. Bit is cleared by CAL_RESET. This bit is available as a status signal output on the GPIO3 pin. |
| BKUP_ON | Set to one veriﬁes backup mode is active |
| GPIO_1_IN | Shows logic state of general purpose I/O Pin #1. This is always enabled. |
| GPIO_2_IN | Shows logic state of general purpose I/O Pin #2. This is always enabled. |
| GPIO_3_IN | Shows logic state of general purpose I/O Pin #3. This is always enabled. |
| CHG_FLT | Set to one indicates battery charge fault. |
| BKUP_FLT | Set to one indicates battery cell voltage < VDIS . This bit state is retained as long as sufﬁcient VBAT is applied. This bit is available as a status signal output on the GPIO2 port. This bit remains until either the SHDN pin is cycled or register bits POR_RESET or BUFLT_RST are set when DCOUT returns. |
| CAL_FLT | Set to one indicates a calibration fault. Calibration terminated early. |
| CHG_STATE_0 | Combined with CHG_STATE_1 indicates phase of charging. 00 = Off, 01 = precharge, 10 = bulk charge, 11 = top off charge |
| CHG_STATE_1 | See CHG_STATE_0 |
| CHARGE_INHIBITED | Indicates charge inhibited is enabled when set to a one. This as a duplicate of CHARGE_INHIBIT bit in the ChargerStatus() register. |

BBuControl() – Write Only. The SMBus host uses this command to control the LTC4110.

| CAL_START | Set to one starts a discharge based calibration of battery (default = self cleared to zero-off) |
| --- | --- |
| CAL_RESET | Set to one clears the CAL_FLT as well as the CAL COMPLETE and CAL_ON status bits. If calibration is in progress, it will also stop the calibration process (default = self cleared to zero-off) |
| GPIO_1_EN | Set to one enables GPIO1 pin as an output (default = set to one if programming SMBus not used by connecting SELA pin to 0.5VREF, otherwise default = set to zero/GPIO1 high-Z ) |
| GPIO_2_EN | Set to one enables GPIO2 pin as an output (default = set to one if programming SMBus not used by connecting SELA pin to 0.5VREF, otherwise default = set to zero/ GPIO2 high-Z) |
| GPIO_3_EN | Set to one enables GPIO3 pin as an output (default = set to one if programming SMBus not used by connecting SELA pin to 0.5VREF, otherwise default = set to zero/ GPIO3 high-Z) |
| GPIO_1_OUT | Programmable logic bit whose state will be reﬂected on the GPIO1 pin if the GPIO_1_CHG bit is cleared (default = set to zero/GPIO1 pulled low) |
| GPIO_2_OUT | Programmable logic bit whose state will be reﬂected on the GPIO2 pin if the GPIO_2_BUFLT bit is cleared (default = set to zero/GPIO2 pulled low). |
| GPIO_3_OUT | Programmable logic bit whose state will be reﬂected on the GPIO3 pin if the GPIO_3_CALCOM bit is cleared (default = set to zero/GPIO3 pulled low) |

4110fb

OPERATION

| LABEL | DESCRIPTION |
| --- | --- |
| GPIO_1_CHG | Set to one sends an inverted CHG_ON (internal register, set to 1 when either CHG_STATE_0 or CHG_STATE_1 is set to 1) status signal out to the GPIO1 pin. If this bit is set, the value of CHG_ON overrides the value of the GPIO_1_ OUT bit state. Pin must be output enabled with GPIO_1_EN bit (default = set to zero/off) |
| GPIO_2_BUFLT | Set to one sends an inverted BKUP_FLT status signal out to the GPIO2 pin. If this bit is set, the value of BKUP_FLT overrides the value of the GPIO_2_OUT bit state. Pin must be output enabled with GPIO_2_EN bit (default = set to zero/off) |
| GPIO_3_CALCOM | Set to one sends an inverted CAL_COMPLETE signal out to the GPIO3 pin. If this bit is set, the value of CAL_ COMPLETE overrides the value of the GPIO_3_OUT bit state. Pin must be output enabled with GPIO_3_EN bit (default = set to zero/off) |
| RESET_TO_ZERO | Set to one resets all faults and timers in charge and forces the ChargingCurrent() and ChargingVoltage() to zero values. Clears Alarm_Warning() register. Does not affect BBuControl() register. Bit clears to zero automatically after the command is executed (default = cleared to zero-no reset) |
| POR_RESET | Resets LTC4110 to power-on default values. Setting the bit to a one will activate POR_RESET. POR_RESET performs a total chip wide reset like the SHDN pin function without the chip actually shutting down. This includes clearing any bits in registers. The bit clears itself automatically after the command is executed (default = cleared/no reset) |
| BUFLT_RST | Resets the BKUP_FLT bit. The bit clears itself automatically after the command is executed (default = cleared). |
| CHARGE_INHIBIT | Disables charging of battery. Set to one halts charge current while holding the charger state and pausing all battery charge timers without changing the ChargingCurrent() and ChargingVoltage() values. Charge may be enabled by clearing this bit. This bit is automatically cleared when power is reapplied or when a battery is re-inserted (default = cleared to zero-off) |

4110fb

OPERATION

Table 7. Summary of Supported SMBus Functions

| Function | Access | SMBus Address | Command Code | Data Type | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ChargerStatus( ) |  | 7’b0001_ 001 | 8'h13 | Status | AC_PRESENT | BATTERY_PRESENT | POWER_FAIL | ALARM_INHIBITED | RES_UR | RES_HOT | RES_COLD | RES_OR | VOLTAGE_OR | CURRENT_OR | LEVEL:3/LEVEL:2 |  | CURRENT_NOTREG | VOLTAGE_NOTRES | POLLING_ENABLED | CHARGE_INHIBITED |
|  | Read |  |  | Return Value | 1/0 | 1/0 | 0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1/0 |
| Charging- Current( ) |  | 7’b0001_ 001 | 8'h14 | Value | Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Write |  |  | Permitted Values | Unsigned Integer Representing Current in mA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Charging- Voltage( ) |  | 7’b0001_ 001 | 8'h15 | Value | Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Write |  |  | Permitted Values | Unsigned Integer Representing Voltage in mV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alarm- Warning( ) |  | 7’b0001_ 001 | 8'h16 | Control | OVER_CHARGED_ALARM | TERMINATE_CHARGE_ALARM | RESERVED_ALARM | OVER_TEMP_ALARM | TERMINATE_DISCHARGE_ALARM | RESERVED | REMAINING_CAPACITY_ALARM | REMAINING_TIME_ALRAM | INITIALIZED | DISCHARGING | FULLY_CHARGED | FULLY_DISCHARGED | ERROR |  |  |  |
|  | Write |  |  | Permitted Values | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| BBuStatus( ) |  | 7’b0001_ 001 | 8'h3D | Status | CAL__ON | CAL_COMPLETE | BKUP_ON | Reserved | Reserved | GPIO_1_IN | GPIO_2_IN | GPIO_3_IN | Reserved | CHG_FLT | BKUP_FLT | CAL_FLT | CHG_STATE_0 | CHG_STATE_1 | Reserved | CHARGE_INHIBITED |
|  | Read |  |  | Return Values | 1/0 | 1/0 | 1/0 | 0 | 0 | 1/0 | 1/0 | 1/0 | 1 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 0 | 1/0 |
| BBuControl() [Charger- Mode()] |  | 7’b0001_ 001 | 8'h12 | Control | CAL_START | CAL_RESET | GPIO_1_EN | GPIO_2_EN | GPIO_3_EN | GPIO_1_OUT | GPIO_2_OUT | GPIO_3_OUT | Reserved | GPIO_1_CHG | GPIO_2_BUFLT | GPIO_3_CALCOM | RESET_TO_ZERO | POR_RESET | BUFLT_RST | CHARGE_INHIBIT |
|  | Write |  |  | Permitted Values | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 |

Note 1: IC only looks for a zero (off) or a non-zero (on) value. Actual charge current is set by the ICHG pin. Note 2: IC only looks for a zero (off)or a non-zero (on) value. Actual charge voltage is set by the VCHG pin.

4110fb

APPLICATIONS INFORMATION

The ﬁrst conﬁguration option to set for the LTC4110 is the type and cell count of the battery you wish to use. Pins TYPE and SELC are use to set this conﬁguration. Please note NiMH and NiCd batteries  are only supported in the smart battery conﬁguration. The three state input pins SELA, SELC and TYPE should NOT be changed while power is applied to the IC unless in shutdown mode. Such action will result in unpredictable behavior from the LTC4110.

SUPERCAPS

Table 8 shows all of the options with the exception of SuperCaps. SuperCaps are supported by using standard Li-ion or SLA modes in combination with the adjusting the charge voltage with the VCHG pin. As far as the LTC4110 is concerned, it is still working with a Li-ion or SLA bat- tery and will follow all the charge states as required for that chemistry. Table 9 shows the required conﬁguration

based on the desired cap voltage and series cell count. Other per cell voltages can be obtained by adjusting the VCHG pin as required.

When the LTC4110 is conﬁgured to charge a super cap, if TYPE pin is tied to 0.5VREF, use the bulk charge current equation (see the Programming Charging/Calibration Cur- rent section for details) to set the charging current. If TYPE pin is tied to GND, then the charging current will equal to preconditioning charge current when the cap voltage is below the bulk charge threshold (as listed in Table 9) and bulk charge current when the voltage is above the threshold. Simply tie the IPCC pin to ICHG pin if these two currents need to be the same.  If the capacitor  is too small (<10mF), the voltage might rise too fast to be regulated by the loop. In that case, the capacitor  will be charged to over voltage pretection threshold (typically 107.5% of the ﬂoat voltage. See VBOV)

Table 8. Battery Type and Number of Series Cell Selection  for Batteries

|  | STANDARD Li-Ion (TYPE = GND) | SLA (TYPE = 0.5VREF) | SMART NiMH/NiCd (TYPE = VREF) | SMART Li-Ion (TYPE = VDD) |
| --- | --- | --- | --- | --- |
| SELC = GND | 1 | 2 | 4 | 1 |
| SELC = 0.5VREF | 2 | 3 | 6 | 2 |
| SELC = VREF | 3 | 5 | 9 | 3 |
| SELC = VDD | 4 | 6 | 10 | 4 |

Note: When smart battery is selected by the TYPE pin, SELA pin must be connected to GND to select address 12h.

Table 9. Battery Type and Number of Series Cell Selection  for Super Caps

| CAP VOLTAGE (V) | SERIES CAP COUNT | STACK CAP VOLTAGE (V) | TYPE | SELC | VCHG | BULK CHARGE THRESHOLD (V/CELL) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.5 | 2 | 5 | 0.5VREF | GND | 0.625VREF | N/A |
| 2.5 | 3 | 7.5 | 0.5VREF | 0.5VREF | 0.625VREF | N/A |
| 2.5 | 5 | 12.5 | 0.5VREF | VREF | 0.625VREF | N/A |
| 2.5 | 6 | 15 | 0.5VREF | VDD | 0.625VREF | N/A |
| 2.5 | 7 | 17.5 | GND | VDD | 0.646VREF | 1.71 |
| 2.7 | 3 | 8.1 | GND | 0.5VREF | 0.375VREF | 2 |
| 2.7 | 5 | 13.5 | GND | VREF | 0.750VREF | 1.8 |
| 2.7 | 6 | 16.2 | GND | VDD | 0.375VREF | 2 |
| 3 | 3 | 9 | GND | 0.5VREF | 0.750VREF | 2 |
| 3 | 4 | 12 | GND | VREF | 0.333VREF | 2.25 |
| 3 | 5 | 15 | 0.5VREF | VDD | 0.625VREF | N/A |
| 3 | 6 | 18 | GND | VDD | 0.750VREF | 2 |

4110fb

APPLICATIONS INFORMATION

SOFT-START

The LTC4110 is soft-started with the 0.1μF capacitor on the ITH pin. On start-up, the ITH pin voltage  will rise quickly to 0.1V, then ramp up at a rate set by the internal 24μA pull-up current and the external capacitor. Battery charging current starts ramping up when ITH voltage reaches  0.7V and full current  is achieved  with ITH at about 2V. With a

0.1μF capacitor, time to reach full charge  current  is about

8ms and it is assumed that input voltage to the charger will reach full value in less than 8ms. The capacitor can be increased up to 1μF if longer input start-up  times are needed.

In any switching regulator, conventional timer-based soft- starting can be defeated  if the input voltage rises much slower than the timeout period. This happens because the switching regulators in the battery charger and the computer power supply are typically supplying  a ﬁxed amount of power to the load. If input voltage comes up slowly compared to the soft-start time, the regulators will try to deliver full power to the load when the input voltage is still well below its ﬁnal value. If the adapter is current limited, it cannot deliver  full power at reduced output voltages and the possibility exists for a quasi “latch” state where the adapter output stays in a current limited state at reduced output voltage. For instance, if maximum charger plus computer load power is 30W, a 15V adapter might be current limited at 2.5A. If adapter  voltage is less than (30W/2.5A = 12V) when full power is drawn, the adapter voltage  will be pulled down by the constant 30W load until it reaches a lower stable state where the switching regulators can no longer supply full load. This situation can be prevented by utilizing the DCDIV resistor divider, set higher than the minimum adapter voltage where full power can be achieved.

CALIBRATION MODE BACK-DRIVE CURRENT PROTECTION

A resistor between CLP and  CLN programs the minimum supply forward current, this feature prevent the LTC4110 from back-driving the supply in calibration mode and pulling the voltage higher when the system load is low. The resistor value is given by

R	=	IBDT

CL 	IFR(MIN)

where

IBDT = back-drive  current  limit threshold, 10mV typical IFR(MIN) = minimum  forward current  in calibration  mode An RC ﬁlter may be required to ﬁlter out system load noise

as shown in Figure 11.

BATTERY AND CHARGER CURRENT SENSE

The LTC4110 uses two sense resistors to monitor and control all charge and calibration currents: RSNS(BAT) and RSNS(FET).

RSNS(BAT)

RSNS(BAT) is used to monitor the DC current going into the battery for charge, and the current going out of the battery for calibration. Before any current programming can be done, the value of RSNS(BAT) must be determined ﬁrst. Highest accuracy is achieved when full-scale current, IMAX is set to develop a 100mV drop across the resistor. Although values greater than 100mV can be used to improve accuracy, this requires larger sense resistors to handle the extra heat and lower efﬁciency.  IMAX must be set to

LTC4110

BACK

CLP

VIN

10mV	TO SYSTEM

DRIVE	+

LOAD

CL1

10mV

CLN

100nF

5k

CIN

+

4110 F11

Figure 11. Back-Drive Protection

4110fb

APPLICATIONS INFORMATION

the highest current ﬂow between charge and calibration modes, whichever is greater.

Recommended starting values for the ﬁlter is: RCSP1 = RCSN1 between  1K and 2K

RSNS(BAT)

100mV

=

IMAX

RCSP1 + RCSP2 = RCSN1 + RCSN2 = about  3K CCSP = CCSN = about  3 • CITH.

See Table 10 for example values.

RSNS(BAT) accuracy is intentionally made very high to permit development of an accurate host software based capacity measurements of standard batteries. Use resistors with

1% accuracy or better or use a 4-terminal  Kelvin sensing resistor. See the PCB Layout section  for a reasonable no cost Kelvin sensing layout that permits the use of less expensive standard two terminal sense resistors. For more electrical information relating to RSNS(BAT) itself, see the Component Selection section.

As designed, any signiﬁcant  AC ripple voltage  seen by CSP and CSN pins can lead to current sensing errors for both current programming and capacity measurements. To prevent  the Flyback’s AC ripple voltage from interfering with DC accuracy, RSNS(BAT) must  have a RC ﬁlter network

Figure 12 shows typical values for CITH = 0.1μF

RSNS(FET)

The LTC4110’s Flyback converter operates in current mode with RSNS(FET) monitoring cycle-by-cycle transformer cur- rent in both Charge and Calibration modes. The LTC4110’s ISENSE pin serves two functions. First is to regulate the primary current as required by the feedback loop. Second is to monitor the secondary current and check for short circuits.  The traditional Flyback primary and secondary currents  look like the following:

ΔI	IPRI

PRIMARY CURRENT

0

installed between the RSNS(BAT) and CSP and CSN pins.

IPRI

N

SECONDARY CURRENT

The CSP and CSN pins have an input bias current of ±10nA typically. A very large  RCSP1  + RCSP2  value will cause a large current mismatch error. The current ﬂowing into the CSP and CSN pins equals VSNS/(RCSP1  + RCSP2) =

100mV/(RCSP1 + RCSP2), a very small RCSP1 + RCSP2 value will result  in a large current.  Typically a value between 3k and 30k gives the best performance.

0	4110 F13

Figure 13. Flyback Primary and Secondary Current

The waveforms in Figure 13 each assume a view of posi- tive current  ﬂow into the load. The value N represents the ratio of the secondary to the primary with the primary set to a value of 1. Unlike a traditional Flyback topology, the LTC4110 Flyback is bi-directional, so the meaning of “primary” is a function of the operating mode. In order

LTC4110

CA

RCSP2

2k

RCSN2

2k

CCSP

330n

CCSN

330n

RCSP1

1k

RCSN1

1k

RSNS(BAT)

100mV

Figure 12. CSP, CSN RC Filter

4110 F12

4110fb

APPLICATIONS INFORMATION

to monitor the primary current in both sides with a single RSNS(FET)  resistor, both transformer  windings must be connected prior to RSNS(FET). Since the secondary phase is always 180 degrees out of phase with the primary, the following current waveform in Figure 14 is the result.

IPRI

PRIMARY CURRENT

in calibration mode input current is regulated, not the output current.

The LTC4110’s ISENSE  pin has a limited usable positive voltage range  for VSNS(FET). The range  must be between

30mV and 150mV peak in both charge and calibration modes when operating at full current. The negative portion of the waveform is also monitored but has a dynamic trip

0	IPRI

N

SECONDARY CURRENT

4110 F14

level that tracks the actual primary current. The trip level

has a gain factor of –3. If the secondary current trips the negative level, the ﬂyback goes into current limit.

Figure 14. RSNS(FET) Current Waveform

In terms of current sensing, the primary current portion of the above waveform is monitored for peak current (DC

+ AC) at any time in any mode. It does not monitor the batteries’   DC current. The LTC4110 uses leading edge blanking to mask out noise to make the application of this part simple to use. The secondary portion of the above waveform is monitored for negative peak current to sense for short circuit.

The value of ripple current, ΔI, is a direct function of the transformer inductance. See transformer section for more information about transformer ripple current.

You must calculate the IPRI for both charge current mode and calibration current mode. The equation for calculating the IPRI for charge mode is as follows:

These limits have the following implications:

•  The ratio of peak current between IPRI(CHG) and IPRI(CAL)

cannot be greater than 5-to-1 as seen by RSNS(FET).

•  The transformer turns ratio will approximately  reduce

the maximum  available DC current  ratio between  ICHG to ICAL by a factor of 1/N. The additional variables being ripple current and efﬁciency.

•  You cannot  use a transformer with a turns ratio greater than 3.

•  Because efﬁciency is always less than 100%, you never have to worry about peak secondary current causing a false short circuit trip within the turns ratio limit of 3 or less.

As a design starting point, use the lowest value between IPRI(CHG) and IPRI(CAL) for IPRI, let VSNS(FET) be set to 50mV for good efﬁciency and solve for RSNS(FET).

IPRI(CHG) =

ICHG

E

•

VBAT

VDCIN

+ N +

(1)

RSNS(FET) =

VSNS(FET)

IPRI

VBAT • VDCIN

2• f • LPRI • (VBAT + N• VDCIN )

IPRI for the Calibration mode is as follows:

With an initial  value of RSNS(FET)  identiﬁed,  solve for VSNS(FET) using the highest value between  IPRI(CHG) or IPRI(CAL) and see if the calculated  value of VSNS(FET) falls

IPRI(CAL) = ICAL •

VBAT

+ 1 +

below the upper limits. If it is too high, you may have to drop the value of RSNS(FET). If you cannot meet the VSNS(FET)

N• VDCIN

upper or lower limits and/or ratio limits, you may have to

VBAT • VDCIN

(2)

back off on one of the ICHG and ICAL DC current parameters to compensate.

2• f • (N2 • LPRI)•   VDCIN + VBAT

N	Once within all the limits, optimize RSNS(FET) for maximum

efﬁciency by using very low value of RSNS(FET) and/or ﬁnd

The value of E is the ﬂyback efﬁciency. Use 80% (0.8) as

the value since the ﬂyback uses synchronous rectiﬁca- tion. E is not used for the calibration equation because

a popular RSNS(FET) value. The tradeoff of using lower values of RSNS(FET) is increased  waveform jitter due to higher switching noise sensitivity issues.

4110fb

APPLICATIONS INFORMATION

PROGRAMMING  CHARGE VOLTAGE

Depending on the battery chemistry chosen by the TYPE pin, a charge termination voltage or a ﬂoat voltage will be required. The difference between the two is time.  A ﬂoat voltage is applied to a battery forever. The VCHG pin is used to set any of these voltages and the equations remain the same. For this document, we will use the term ﬂoat voltage generically. If nickel chemistry  is chosen, the VCHG pin is disabled placing the charger in constant current mode. If

β = exponential temperature  coefﬁcient of resistance

The LTC4110 is designed to work best with a 5% 10k NTC thermistor with a β near 3750, such as the Siemens/EPCOS B57620C103J062. In this case, R1 = 7256Ω.

R1

THA	+

| VBGR | LOGIC |  |
| --- | --- | --- |
| TH_LO |  | RES_HOT |

you are using a smart battery, wake-up charge is subject to the VCHG pin setting when active.

RNTC

THB

HI_REF

+	TH_HI

–

RES_OR

Connecting the VCHG pin to GND will set the default per cell ﬂoat voltage (4.2V for Li-ion, 2.35V for SLA). If a dif- ferent ﬂoat voltage is needed, tie the VCHG pin to a voltage between 0.25 VBGR and 0.75 VBGR using  a resistor divider on the VREF pin. Unlike VREF, VBGR is an internal reference

+

REF   LO_REF

–

Figure 15. Lead Acid Thermistor

4110 F15

voltage of the same voltage as VREF but with a much tighter

(±0.5%) tolerance than VREF.

LEAD ACID BATTERY TEMPERATURE  COMPENSATION

To program the temperature compensation for SLA charg-

VFLOAT =  2•

where

VCHG

VBGR

1  • 0.6

ing, an external circuit is needed as shown in Figure 16. The values are given by:

R1= R0 • β− 2• T0

β + 2• T0

ΔVFLOAT = Adjusted Float Voltage – Default Float Voltage

VCHG = VCHG Pin Voltage, VBGR = 1.220V

The resistor divider connected to VREF pin will affect timer

k1=	R0

R0 + R1

TCk1= –	β

• R1• R0

2	2

(see the Programming Charge Time with TIMER and VREF

Pins section for more details).

THERMISTOR FOR LEAD ACID BATTERIES

(R1+ R0)

k2 = TCVFL OA T

1.2 • TCk1

• T0

When the TYPE pin is programmed for Lead  Acid, THA pin will be force to VBGR , THB will be used to sense the NTC resistance.  The value of R1 is given by:

k3 = 0.5 + ΔVFL OA T  / 1.2 − k1• k2

1− k2

where:

R1= R0 • β− 2• T0

β + 2• T0

TCV

FLOAT

= temperature  coefﬁcient of the ﬂoat voltage

where:

R0 = thermistor resistance (Ω) at T0

T0 = thermistor reference temperature  (°K)

(Range: –2mV/°C – –6mV/°C)

ΔVFLOAT  = ﬂoat voltage at 25°C – default ﬂoat voltage

2.35V (Range: –0.15V – 0.15V)

For example,  if a 10k NTC with β = 3750 is used, desired

4110fb

APPLICATIONS INFORMATION

VREF

(1 – k3) • RVREF

k3 • RVREF

R2

RSNS (BAT)  = resistor between ﬂyback transformer and battery

RCSP = RCSP1 + RCSP2

RCSN = RCSN1 + RCSN2

RICHG = resistor connected between ICHG pin and GND RIPCC = resistor connected  between IPCC pin and GND RICAL = resistor connected between ICAL pin and GND.

k2 =

R2

R2 + R3

R3

R1

RNTC

10k

R0

VCHG

THA THB

If any programming resistor value on any of the three

pins exceeds 100k, see Flyback Compensation  section for more information.

Pins can be tied together to save components  if any of the currents have the same value.  If two pins share a common programming resistor greater than 100k, only

k1 =

R0 + R1

4110 F16

Figure 16. Lead Acid Temperature Compensation

ﬂoat voltage = 2.5V at 25°C with a temperature coefﬁcient of

–2mV/°C, then R1 = 7256,  k1 = 0.580,  TCk1 = –10.3m/°C,

ΔVFLOAT = 2.5 – 2.35 = 0.15V, k2 = 0.162, k3 = 0.634.

PROGRAMMING CURRENT

Charge/calibration currents are programmed using the following equations:

one compensation circuit is required.

If the TYPE pin is set for SLA/LEAD ACID, then the IPCC

pin is not used. You can leave the IPCC pin open.

PROGRAMMING BACKUP MODE ENTRY THRESHOLD AND CALIBRATION MODE BACK-DRIVE VOLTAGE DETECTION THRESHOLD

A resistor divider connected to supply input sets both the backup mode entry threshold and the calibration mode back-drive voltage detection threshold.

ICHG = R

VBGR

SNS(BAT)

• RCSP

RICHG

R2

BACKUP

V	=

R1

+ 1  • VBGR

VOVP

VBACKDRIVE =

• VBACKUP

IPCC

ICAL

=	VBGR

RSNS(BAT)

=	VBGR

RSNS(BAT)

• RCSP

RIPCC

• RCSN

RICAL

R  2 =

R1

where:

VBGR

VBACKUP    1

VBGR

where:

ICHG = bulk charge  current

IPCC = preconditioning  charge current

ICAL = calibration current

VBGR = 1.220V

VBACKUP = supply voltage when backup starts, it should not be programmed to less than 4.5V

VBACKDRIVE = supply voltage when calibration is terminated, it should not be programmed to more than 20V

VOVP = DCDIV pin back-drive detect threshold in calibration mode, typically 1.5V (see VOVP)

4110fb

APPLICATIONS INFORMATION

R1 = resistor connected between DCDIV and GND

R2 = resistor connected between supply input and

DCDIV

VBGR = reference voltage 1.220V

For example, if supply input = 12V and backup starts when it drops to 11V, then VBACKUP = 11V, VBACKDRIVE = 13.5V, R2/R1 = 8.02, choose R1 = 10k, then R2 = 80.6k.

If a higher  ratio than  VOVP/VBGR = 1.23 is desired between VBACKDRIVE and VBACKUP, a third resistor can be used as shown in Figure 17.

SUPPLY INPUT

OPTIONAL RESISTOR

For example, if supply input = 12V and backup starts when it drops to 8V, calibration terminates when it rises to 16V, and  VDC = VDD = 4.75V, then R2/R1 = 21.87, R3/R1 = 3.88, choose R1 = 10k then R2 = 221k and R3 = 39.2k.

If the noise on supply input is a problem, a capacitor can be connected between DCDIV and GND.

PROGRAMMING  CALIBRATION/BACKUP CUT-OFF THRESHOLD

The pins VCAL  and VDIS  are used to calculate custom discharge cut-off voltages for their respective operating modes. The equations shown below are generic for both. There is no implied relationship between VCAL  and VDIS for they are independent of each other.

TO INCREASE THE

1.23 TO 1 RATIO

R3

VDC

R2

DCDIV

R1

VBGR

CMP

CMP

BACKUP

BOOST

The equations  are most helpful  if you pick the VCUTOFF voltage you want, within the range limits offered, and then solve  for VCAL or VDIS. With the voltage  value  of VCAL or VDIS calculated,  determine  the necessary voltage divider network  from VREF required to get the calculated voltage

1.23 • VBGR

Figure 17. Backup and Boost Detect Comparators

R2	VBACKDRIVE – VBACKUP

=	−

R1	0.23 • VBGR

VBACKDRIVE – 1.23 • VBACKUP  − 1

0.23 • VDC

4110 F17

on these pins respectively. It is recommended that one single series resistor divider network from VREF to ground be used to obtain all of the pin voltages you need. It should be noted that custom values of VCHG would  also  affect the divider network complexity. See Programming Charge Voltage section  for more information.

Connect the VCAL or VDIS pin to GND will set the default calibration/backup cut-off threshold (2.75V for Li-Ion,

1.93V for SLA, 0.95V for NiMH/NiCd). These  threshold voltages can be adjusted (±400mV for Li-Ion, ±300mV

R3 =  VDC

R1	VBGR

•	VBACKDRIVE – VBACKUP	−

VBACKDRIVE – 1.23 • VBACKUP

for SLA, ±200mV for NiMH/NiCd) by tying the pin to ap- propriate voltage on the VREF pin resistor divider according to the following equations:

0.23 • VDC 	− 1

VBACKDRIVE – 1.23 • VBACKUP

where:

VCUTOFF

V

VCAL / VDIS

=	• 4.2

VBGR

VCAL  / VDIS

=	• 2.35

(Li −Ion)

(SLA)

VDC = Any regulated  DC voltage  available in the system

such as SMBus pull up, LED supply  or LTC4110’s VDD

voltage, must be higher than 1.7V. R3 = resistor connected

CUTOFF

VBGR

VCAL / VDIS

between VDC and DCDIV.

VCUTOFF  =	• 2

BGR

V

(NiMH / NiCD)

4110fb

APPLICATIONS INFORMATION

where

VCUTOFF = adjusted cutoff threshold voltage VCAL/VDIS = voltage  on VCAL or VDIS pin VBGR =1.220V

The resistor divider connected to VREF pin will affect  timer. See the Programming  Charge Time with TIMER and VREF Pins section for more details.

PROGRAMMING  CHARGE TIME WITH TIMER AND VREF PINS

Charge time limits for Li-Ion batteries can be programmed by selection of capacitance on the TIMER  pin, but is dependent upon resistance on the VREF pin. Typical pro- grammed bulk charge times range from 2 to 12 hours and is set as follows:

T(Hrs)

If the TYPE pin is set for SLA/LEAD ACID or any nickel based smart battery, the TIMER pin is not used. You can ground the TIMER pin. Furthermore,  if there is no need of any timer function and there is no need of any voltage divider  from VREF to ground, you must still keep a load on the VREF pin between 10μA and 25μA. It is recommended you place a 49.9k load resistor from VREF to ground.

CHARGING BATTERIES OVER 12 HOURS

In situations where required bulk charge time cycle will exceed the 12 hour time limit imposed by the charge TIMER pin, you have two options.  You can have an SMBus host clear  the CHG_FLT bit and force start another charge cycle or you can switch to a smart version of the same battery. If you chose the former, reduce the TIMER pin time to about 2/3 of the actual time required. This will result in faster termination in the second cycle and with autorestart cycles  when  VAR is tripped. If you choose the smart bat-

CTIM E R (F) =

(944 • R

VRE F

(Ω))

tery option, the smart battery itself safely controls charge termination.  Bulk charge can last as long as necessary

As an example if RVREF = 113k and the desired bulk charge time limit is ﬁve hours then CTIMER  = 47nF. See FTMR which directly affects the 944 constant in the Electrical Characteristics  Table for the tolerance.

Avoid capacitors with high leakage currents. The VREF pin load resistor range is 49k to 125k or 10μA to 25μA of load current.  At 125k the maximum capacitance on VREF is limited to a maximum of 50pF to maintain sufﬁcient AC stability for the internal ampliﬁer. At 49k the maximum is

125pF. The maximum capacitance is inversely proportional to the resistance.

The voltage  (VREF) on the VREF  pin can be used as a precision voltage for other uses with some limitations. The total VREF pin current must not exceed 25μA and the capacitance must be limited as discussed above. Load current ﬂuctuations will modulate the programmed charge time. In shutdown mode VREF will drop  to 0V.

In some applications a divided down VREF voltage is needed to program the SELA, SELC, TYPE, VCHG , VCAL and VDIS pins. This is easily implemented by use of a resistor divider connected from VREF to GND that sets the VREF pin current instead of a single resistor.

to charge the battery to 100%. No host is required to do anything, as the battery  will maintain  its full charge state using its SMBus charge commands.

PROGRAMMING  AC PRESENT  INDICATION DELAY TIME WITH ACPDLY AND VREF PINS

When the main supply, DCIN, returns after a power failure the ACPb pin is driven low to indicate presence of main power. This transition can be delayed to allow time for the system to stabilize before actions are taken by the system based on this pin status. The high to low transition only delay on the ACPb pin can be programmed by selection of capacitance on the ACPDLY pin, but is dependent upon resistance on the VREF pin. Typical programmed delay times

range from 10ms to 200ms and is set as follows:

C	(F) =	T(s)

ACPDL Y	2• R	(Ω)

VRE F

As an example  if RVREF = 113k and the desired delay time is 105ms then CTIMER = 470nF. See tAC  in the Electrical Characteristics  Table for the tolerance.

4110fb

APPLICATIONS INFORMATION

Avoid  capacitors with high leakage currents. See the Programming  Charge Time with TIMER and VREF  Pins section for details concerning the VREF pin. For minimum delay open the ACPDLY pin.

BAT PIN CURRENT IN IDLE MODE

When LTC4110 is in IDLE mode (i.e., not in charge, calibra- tion or backup mode), there  will be a typical 30μA current pulled from the battery through the BAT pin,  if this  current is of concern, a diode in series with a resistor can be con- nected between DCIN and battery to compensate it.

SHOW  BATTERY FULL WITH ACPB AND CHGB

Tie the source of an N-MOSFET to ACPb, gate to CHGb and drain in series with R to an LED to show battery full. In that case if CHG or ACP status LED is not needed, replace it with a short but keep the pull-up resistor.

+5V

This current ramp starts at zero right after the primary side MOSFET (CHGFET in charge mode, DCHFET in calibration mode) is turned on. The current rises linearly towards a peak of VSEC/400k (where  VSEC = BAT in charge mode, VSEC = DCIN in calibration mode), shutting off once the primary side MOSFET is turned off. A series resistor (RSL) connecting the ISENSE  pin to the current sense resistor (RSNS(FET)) thus develops a ramping voltage drop. From the perspective of the ISENSE  pin, this ramping voltage adds to the voltage across the sense resistor, effectively reducing the current comparator threshold in proportion to duty cycle. This stabilizes the control loop against subharmonic oscillation. The amount of reduction in the current comparator threshold (ΔVISENSE) can be calculated using the following equation:

ΔV 	= DUTY CYCLE • VSE C  • R

IS E NS E	400k	SL

To program m = m2,

1	400k • RSNS,FET

RSL = N •

F • Lm

FULL	ACP

CHG

CHGb

ACPb

where

N = transformer  turns ratio NBAT/NDCIN

RSNS(FET) = sense resistor connected  between  MOSFET

and GND

f = switching  frequency

Figure 18. Display Battery Full

FLYBACK COMPENSATION

4110 F18

Lm = magnetizing inductance  of the transformer

Designs not needing slope compensation may replace

RSL with a short.

The values given for the ITH pin in the application schematics have been found to compensate both the voltage loop and current loop quite well. However, if the resistor connected to ICHG, ICAL or IPCC is larger than 100k, a 37k resistor in series with a 100nF capacitor should also be connected between that pin and GND to compensate the loop.

SLOPE COMPENSATION

The LTC4110 injects a ramping current through its ISENSE

pin into an external slope compensation resistor (RSL).

CALCULATING IC POWER  DISSIPATION

The power dissipation  of the LTC4110 is dependent upon the gate charge of the two MOSFETs (QG1 and QG2). The gate charge is determined from the manufacturer’s data sheet and is dependent upon both the gate voltage swing and the drain voltage swing of the MOSFET. Use 5V for the gate voltage swing and VDCIN  for the drain voltage swing.

PD = VDCIN • (fOSC (QG1 + QG2) + IQ)

4110fb

APPLICATIONS INFORMATION

Example:

VDCIN   = 12V,  fOSC  = 300kHz,  QG1  = QG2  = 15nC, IQ = 3mA

PD = 144mW

SNUBBER DESIGN

The values given in the applications schematics have been found to work quite well for this 12V-1A application.  Care should be taken in selecting other values for your applica- tion since efﬁciency may be impacted by a poor choice. For a detailed look at snubber design, Application Note

19 is very helpful.

COMPONENT SELECTION Current Sense Resistors

The LTC4110 uses up to three sense resistors—one of them optional. In general, current sense resistors should have a low temperature  coefﬁcient and sufﬁcient power dissipation capability  to avoid self-heating.  Tolerance depends on system accuracy requirements.

RSNS(FET): The power rating of RSNS(FET) is deﬁned by the highest value between  ICHG or ICAL and the transformer turns ratio. Use one the following equations to calculate IRSNS(FET) depending  on which value, ICHG or ICAL which- ever is higher.

IR(SNSFETCHG) =

RCL: RCL power rating is a function of the maximum forward current the system load draws. See Figure 11.

PR(CL)  = IMAX2 • RCL

Find a sense resistor who’s power rating is greater than

PR(CL)

RSNS(BAT): RSNS(BAT)  power rating is a function of the highest current value between  ICHG or ICAL  with which the battery will work. Plug in the higher of the two into IBAT(MAX) and solve:

PR(SNSBAT)  = IMAX2 • RSNS(BAT)

Use a sense resistor with a power rating greater than

PSNS(BAT)

FLYBACK MOSFET SELECTION

The LTC4110  uses two low side N-channel switching MOSFETs in its ﬂyback converter. These MOSFETs have dual roles. An any given time, only one MOSFET is the primary switch while the other acts as a synchronous recti- ﬁer on the secondary to improve efﬁciency. The individual MOSFETs’ roles depend on whether the battery is being charged or calibrated. Each MOSFET speciﬁcation must account for both roles.

The MOSFET voltage ratings in a ﬂyback design must deal with other factors beyond VIN. During switch “on” time, a current is established in the primary leakage inductance (LL) equal to peak primary current (IPRI). When the switch turns off, the energy stored in LL, (Energy = IPRI2 • LL/2)

ICHG •

1+

VBAT

N • VBAT

+ 1

causes the switch voltage to ﬂy up, starting  from the input voltage on up to the breakdown of the MOSFET if the volt-

N • VDCIN    E2 • VDCIN

age is not clamped. Thus, the snubber design is critical in dealing with this voltage spike and can inﬂuence the

IR(SNSFETCAL) =

MOSFET voltage selection value. From a MOSFET point of view, the minimum voltage must be greater than the

ICAL •

1+

VBAT

N • E2 • VBAT

+ 1

snubber clamp  voltage  VSNUB. If VSNUB itself is too low,

N • VDCIN

VDCIN

zener clamp dissipation rises rapidly thus encouraging higher  MOSFET voltages.  The maximum  DC voltage that

Plug in the higher value of the above two results as

IR(SNSFET) and solve for power: PR(SNSFET) = IR(SNSFET)2  • RSNS(FET)

the N-channel MOSFETs sees is:

V	= V	+  VBAT CHG(FET)		DCIN 		N

VCAL(FET) = VBAT + N • VDCIN

4110fb

APPLICATIONS INFORMATION

The VDS ratings  of the MOSFETs need to be higher than these values.

The MOSFET current ratings for the primary side must be higher than IPRI, which is IPRI(CHG) or IPRI(CAL) for charge and Calibration mode respectively. See Equations 1 and 2. MOSFET current ratings for the secondary side must be higher than IPRI/N. Since both MOSFETs must perform both roles, the minimum current rating of the MOSFETs should be greater than the higher of these values.

MOSFET power dissipation is a function of the RMS cur- rent ﬂowing through the MOSFET.

Charge Mode:

IPRI(FETCHG) =

The MOSFET should be speciﬁed for fast or PWM switching. The MOSFET that meets all the above speciﬁcations but has the lowest QG and/or  QGD is often the best choice.

PowerPath MOSFET SELECTION

Important parameters for the selection of PowerPath MOSFETS are the maximum drain-source voltage VDS(MAX), threshold voltage VGS(VT),  on-resistance RDS(ON)  and QGATE.

The maximum allowable drain-source  voltage, VDS(MAX), must be high enough to withstand the maximum drain- source voltage seen in the application.

The gates of these MOSFETs are driven  by the INID (Input

Ideal Diode) and BATID (Battery  Ideal  Diode)  pins. The

ICHG •

E

VBAT • (VBAT + N • VDCIN )

VDCIN

gate turn-on voltage,  VGS, is set by the smaller of the PowerPath supply voltage or the internal clamping volt- age VGON. For the MOSFET driven  from  the INID pin its

ISEC(FETCHG)

= ICHG •

VBAT + N • VDCIN

N • VDCIN

PowerPath supply voltage is the higher of the DCIN pin or DCOUT pin voltage. For the MOSFETs driven from the BATID pin, their PowerPath supply voltage is the higher

Calibration Mode:

of the DCOUT pin or BAT pin voltage. Logic-level VGS(VT)

MOSFET is commonly used, but if a low supply voltage

IPRI(FETCAL)

= ICAL •

VBAT + N • VDCIN

N • VDCIN

limits the gate voltage a sub-logic-level threshold MOSFET

should be considered.

As a general rule, select a MOSFET with  a low enough

ISEC(FETCAL) =

V

• (V

+ N • V	)

RDS(ON) to obtain the desired  VDS while operating  at full current load and an achievable VGS. The MOSFET normally

ICAL • E •  	BAT 	BAT 	DCIN

VDCIN

Where  IPRI(FETCHG) is the same FET as ISEC(FETCAL)  and

IPRI(FETCAL) is the same FET as ISEC(FETCHG).

Using the equation below, plug in the higher current from above into IFET to ﬁnd each FET’s power dissipation for the given mode.

PFET = IFET2 • RDS(ON)

The RDS(ON) value of the MOSFET depends on VGS. Conser- vatively you can use the RDS(ON) value with a VGS rating of

4.5V. If you are using a dual-MOSFET package, determine whether charge mode or calibration mode results is the highest overall power dissipation and use that as the rating for the dual MOSFET.

operates in the linear region and acts like a voltage con- trolled resistor. If the MOSFET is grossly undersized then it can enter the saturation region and a large VDS may result. However, the drain-source diode of the MOSFET, if forward biased will limit VDS. A large VDS combined  with the load current could result in excessively high MOSFET power dissipation.  Keep in mind that the LTC4110  will regulate the forward voltage drop across the MOSFETs at 20mV (VFR) if RDS(ON)  is low enough. The required RDS(ON) can be calculated by dividing 0.02V by the load current in amps. Achieving forward regulation will minimize power loss and heat dissipation, but it is not a necessity. If a forward volt- age drop of more than 20mV is acceptable then a smaller MOSFET can be used, but must be sized compatible with the higher power dissipation. Care should be taken to ensure

4110fb

APPLICATIONS INFORMATION

that the power dissipated is never allowed to rise above the manufacturer’s recommended maximum level.

Switching transition time is another consideration. When the LTC4110  senses a need to switch any PowerPath MOSFETs on or off time delays are encountered. MOSFETs with higher  QGATE will require more bulk capacitance on DCOUT to hold up all the system’s power supply function during the transition. The transition time of a MOSFET to an on or off state is directly proportional to the MOSFET gate charge. Switching times are given in the Electrical Characteristics  Table (see tdDON , tdDOFF).

Regardless of which way you go, we offer the following thoughts.

Turns ratio affects the duty factor of the power converter which impacts current and voltage stress on the power MOSFETs, input and output capacitor RMS currents and transformer utilization  (size vs power). Using a 50% duty factor under nominal operating conditions usually gives reasonable results. For a 50% duty factor, the turns ratio is:

N =  VBAT

TRANSFORMER

There are two ways to design a transformer.

1. Design it yourself.

2. Work with a transformer vendor to identify an off- the-shelf transformer.

Even if you choose  to design it yourself, you still have to ﬁnd a transformer manufacturer to make it for you.

We recommend  contacting a transformer manufacturer directly since they often have online tools that can help you quickly ﬁnd and select the right transformer. There are many off the shelf transformers that can be successfully be used with the LTC4110. Table 10 shows some suggested off the shelf transformers.

If you want to design a custom transformer optimized for your design, Application Note 19 has an example of how to design a Flyback transformer in the “Transformer” section.

VDCIN

VBAT is the nominal battery voltage. N should be calculated for the design operating in charging mode and in calibration mode. The ﬁnal turns ratio should be chosen so that it is approximately equal to the average of the two calculated values for N. In addition, choose a turns ratio which can be made from the ratio of small integers. This allows biﬁlar windings to be used in the transformer, which can reduce the leakage inductance and the need for aggressive snubber design, thus improving efﬁciency.

Avoid transformer saturation under all operating conditions and combinations (usually the biggest problems occur at high output currents and extreme duty cycles). Choose the magnetizing inductance so that the current ripple is about  20% of DC current.

Finally, in low voltage applications, select a transformer with low winding resistance.  This will improve efﬁciency at heavier loads.

Table 10. Recommended Components Values for 12V Input Supply Li-Ion Battery Backup System  Manager

| Cell | MAX (ICHG, ICAL) (A) | RSNS(BAT)(mΩ) | RSNS(FET)(mΩ) | TRANSFORMER INDUCTANCE (μH) | TRANSFORMER VENDOR AND PART NUMBER |
| --- | --- | --- | --- | --- | --- |
| 3 | 1 | 100 | 50 | 24 | BH 510-1019 TDK PCA14.5/6ER-U03S002 |
| 3 | 2 | 50 | 25 | 12 | COILTRONICS VP4-0140-R |
| 3 | 3 | 33 | 15 | 9 | TDK PCA20EFD-U04S002 |
| 4 | 1 | 100 | 50 | 24 | COILTRONICS VPH4-0140-R |
| 4 | 2 | 50 | 25 | 12 | COILTRONICS VPH4-0075-R |
| 4 | 3 | 33 | 15 | 9 | COILTRONICS VP5-0155-R |

Note: 1:1 turns ratio for all the transformers listed in the table..

4110fb

APPLICATIONS INFORMATION

INPUT AND OUTPUT CAPACITORS

The LTC4110 uses a synchronous ﬂyback regulator to provide high battery charging current. A  chip ceramic capacitor is recommended for both the input and output capacitors  because it provides low ESR and ESL and can handle the high RMS ripple currents. However,  some Hi-Q capacitors may produce high transients due to self resonance under some start-up conditions, such as con- necting the charger input to a hot power source. For more information, refer to Application Note 88.

For charge mode, the ripple current can be calculated as follows:

Similar techniques may also be applied to minimize  EMI

from the input leads.

Diodes

Schottky diodes should be placed in parallel with the drain and source of the Flyback MOSFETs. This prevents body diode turn-on and improves efﬁciency by eliminating loss from reverse recovery in these diodes. It also reduces conduction loss during the dead time of the MOSFETs.

PROTECTING SMBUS PINS

The SMBus inputs,  SCL and SDA, are exposed to uncon- trolled transient signals whenever a battery is connected

IRMSDCINCAP

and

ICHG

=	•

E

N • VBAT

VDCIN

to the system.  If the battery contains a static charge, the SMBus inputs are subjected to transients that can cause damage after repeated exposure. Also, if the battery’s posi- tive terminal makes contact to the connector before the negative terminal, the SMBus inputs can be forced below

IRMSBATCAP = ICHG •

VBAT

N • VDCIN

ground with the full battery potential, causing a potential for latch-up in any of the devices connected to the SMBus

For calibration mode, the ripple current can be calculated as follows:

inputs. Therefore, it is good design practice to protect the

SMBus inputs as shown in Figure 19.

I	= I

• E •

N • VBAT

VDD

RMSDCINCAP 	CAL

and

VDCIN

V

CONNECTOR

TO BATTERY	TO SYSTEM

4110 F19

IRMSBATCAP = ICAL •

BAT

N • VDCIN

Figure 19. SMBus Protection

EMI considerations usually make it desirable to minimize ripple current in the battery leads, and beads or induc- tors may be added to increase battery impedance at the

300kHz switching frequency. Switching ripple current splits between the battery and the output capacitor depending on the ESR of the output capacitor and the battery imped- ance. If the ESR of the output capacitor is 0.1Ω and the battery impedance is raised to 2v with a bead or induc- tor, only 5% of the ripple current  will ﬂow in the battery.

START-UP DELAYS

When exiting shutdown mode, internal supplies must ramp up and settle. 500μs-1ms should be adequate after shutdown is exited or when power is quickly (<100μs) ﬁrst applied to the IC. For slow power ramp-up  (>1ms) internal supplies will be in regulation after power input reaches 4.5V. Until internal supplies settle, status outputs may be invalid.

4110fb

APPLICATIONS INFORMATION

OPERATION WITH DUAL BACKUP SYSTEMS

If  a dual backup system consisting of two LTC4110s each with its own backup battery is needed and a SMBus is used, each LTC4110 should be programmed by the SELA pin to have different addresses. If smart batteries with SMBus are used, a SMBus mux may be required to selectively address each battery. This mux may also be used to address the LTC4110. See SMBus Interface section for more information.

BACKUP OPERATION WITH EXTERNAL BACKUP SUPPLY REGULATOR

If a dedicated  DC regulator  with enable  inputs  is used in place of an actual battery to supply backup power, the PowerPath MOSFETs connected to the BATID pin may  not be required. It depends on the regulator’s ability to accept being back driven by a voltage on the DCOUT pin coming from some other power source such as DCIN. The ACPb pin can control the regulator  such that it is turned on when DCIN goes away. However for fastest transient response, keeping the regulator on may prove to work better. The output voltage of the regulator should be less than DCOUT under normal operating conditions so that DCIN is pro- viding the power to the load. The voltage provided by the regulator must not be allowed to go below the lower limit of the DCOUT pin or erratic operation may result.

BACKUP OPERATION WITH A DOWNSTREAM REGULATOR

Since the backup voltage supplied to the load is not regu- lated, often some form of a regulator is needed between the LTC4110 and the actual load. The characteristics  of

DCIN TO BATTERY TRANSITION  CHATTER REMOVAL

The LTC4110 is designed to automatically switch the bat- tery to the output load when DCIN is lost. Under certain conditions, a rapid loss of DCIN can cause the input and battery ideal diode circuits to chatter. The result is the transition time between the DCIN  FET turning fully off and the battery  FET turning  fully on can last in excess of 200ms with each switching on and off multiple times.

This problem is likely  to occur under the following conditions:

1. Large system load causing the INID pin to be more than 3V below DCIN.

2. The DCIN and battery voltages are approximately the same.

3. The DCIN pin goes high impedance very rapidly (less than 10μs)

Q1 and R1 shown in Figure 20 increase the effective hys- teresis of the DCDIV pin by using the ACPb pin to drive Q1. The threshold  of Q1 must be less than the VSUPPLY to assure the drain of Q1 pulls down to ground when ACPb is high. R1 sets the amount of increase in negative

hysteresis you need relative to the values chosen for the DCDIV resistor divider. A 100k  is suggested as a starting point. You will also need to place a capacitor CACPDLY on the ACPDLY pin. This capacitor in conjunction with resistor RVREF should  be set for a delay of 10ms, which is more than sufﬁcient to eliminate all the chatter.

DCDIV R1

100k

this regulator should offer high efﬁciency when running from the battery in backup mode to maximize backup time. Some regulators may need advance warning when to enter into this mode, which can be accomplished by using the LTC4110’s ACPb pin.

Q1

2N7002

Figure 20.

ACPb

4110 F20

4110fb

APPLICATIONS INFORMATION

PCB LAYOUT CONSIDERATIONS

For maximum  efﬁciency, the switch node rise and fall times should be minimized.  To prevent magnetic and electrical ﬁeld (EMI) radiation and high frequency resonant problems, proper layout of the components connected to the IC is essential.

Flyback Layout

Lowest EMI and maximum efﬁciency are obtained when the high frequency switching current loop area is minimized. It is best to make direct connections, avoiding the use of other circuit board copper planes, i.e. no vias, in making the following connections for this prevents current based noise injection into the copper planes below.

1. Input/output capacitors positive terminals need to be placed as close as possible between the ﬂyback transformer “top” or positive supply rail connections and RSNS(FET) ground connection.

2. Place ﬂyback MOSFETs drain connections right next

to the ﬂyback transformers “bottom” connections.

3. Place the RSNS(FET) current sense resistor right next to the N-MOSFET source connections completing the connection back to the input/output capacitors’ negative terminals.

4. Place the snubber connections as close as possible to the circuit after the above layout connections are completed as required. Again, avoid using vias.

5. The layer below the ﬂyback layout should be ground.

Other Recommendations

6. Optionally use vias to connect power supply sources positive and negative (ground) connections from other copper layers to the ﬂyback layout. Place multiple vias in a tight cluster such that they act as one large via. Recommended 1 via for each 0.5A of current

7. The current sense feedback traces must be routed together as a single pair on the same layer at any given time with smallest trace spacing possible. Locate any ﬁlter component on these traces next to the IC and not at the sense resistor location.

8. The control  IC must be close to the switching FET’s gate terminals. Keep the gate drive signals short for a clean  FET drive. This includes  IC supply pins that

connect to the switching FET source pins. The IC can be placed on the opposite side of the PCB relative to ﬂyback layout above.

9. Figure 21 shows an inexpensive way to achieve Kelvin like sensing using standard current sense resistors.

DIRECTION OF CHARGING CURRENT

RSNS(BAT)

4110 F21

TO CSP AND CSN

Figure 21. Kelvin Sensing  of Battery Current

4110fb

TYPICAL APPLICATIONS

Battery Backup System Manager Controlling a Six-Series  Cell

SLA Battery with Temperature Compensation

SUPPLY INPUT (12V)

8.66k

RCL

0.02Ω

1W

0.1μF LOW ESR

INPUT IDEAL DIODE

Q2

0.1μF LOW ESR

TO SYSTEM LOAD

TO BACKUP LOAD

Q3

1.21k

DCIN CLN

CLP

INID

DCOUT NC BATID

CHGFET

20μF VERY LOW ESR

1nF

33Ω

0.5W

5%

T1

1nF

33Ω

0.5W

5%

20μF

BATTERY

IDEAL DIODE

24.3k

7.32k

25.5k

DCDIV THA THB

VREF

DCHFET ISENSE

CSP

CSN

RSL

3.32k

2k	1k

Q1B Q1A

RSNS(FET)

0.05Ω

0.5W

330nF

VERY LOW ESR

RSNS(BAT)

0.1Ω

0.25W

VDD

+

VCHG

BAT

2k	1k

330nF

FLOAT VOLTAGE = 2.35V/CELL AT 25°C  TC = –2mV/°C

12V

–

VDD

16.2k

36.5k

VCAL VDIS ICHG ICAL IPCC

LTC4110

ITH

ACPDLY TIMER

3.01k	0.1μF

+

+

+

+

+

+

10k

+	84.5k

–

VDD

I2C TO HOST

SELA SELC

SDA SCL TYPE

VDD

ACPb

GP101

GP102

GP103

SHDN GND	SGND

0.1μF LOW ESR

NTC

ß = 3750

4110 TA04

NO TIMER

HIGH CURRENT BACKUP LOAD DESIGN

0.5A BACKDRIVE CURRENT CUTOFF (CALIBRATION)

1A CHARGE AND CALIBRATION CURRENT

ALL RESISTORS  ARE 1% UNLESS NOTED OTHERWISE

Q1: Si7216DN Q2: Si7445DP Q3: Si7983DP T1: BH510-1019

4110fb

TYPICAL APPLICATIONS

Battery Backup System Manager Controlling a Nine-Series Cell NiMH Battery with

Calibration Managed by Host Processor

SUPPLY INPUT (12V)

8.66k

RCL

0.02Ω

1W

0.1μF LOW ESR

INPUT IDEAL DIODE

Q2

0.1μF LOW ESR

TO SYSTEM LOAD

TO BACKUP LOAD

Q3

1.21k

DCIN CLN

CLP

INID

DCOUT NC BATID

CHGFET

20μF VERY LOW ESR

1nF

33Ω

0.5W

5%

T1

1nF

33Ω

0.5W

5%

BATTERY

IDEAL DIODE

49.9k

RTHA 1.13k

RTHB 54.9k

DCDIV THA

THB VREF

VCHG

DCHFET ISENSE

CSP CSN

BAT

RSL

3.32k

2k	1k

2k	1k

Q1B Q1A

RSNS(FET)

0.05Ω

0.5W

330nF

330nF

20μF VERY LOW ESR

RSNS(BAT)

0.1Ω

0.25W

+

+

10.8V (9 CELL)

36.5k

187k

37.4k

0.1μF

I2C TO HOST

VCAL

VDIS ICHG ICAL IPCC

SELA SELC

SDA

SCL

LTC4110

ITH

ACPDLY TIMER

VDD ACPb GP101

GP102

GP103

3.01k	0.1μF

0.1μF LOW ESR

I2C TO HOST

+

+

+

+

+	10k

NTC

+

+

TYPE

SHDN GND	SGND

4110 TA05

NO TIMER

HIGH CURRENT BACKUP LOAD DESIGN

0.5A BACKDRIVE CURRENT CUTOFF (CALIBRATION)

1A CHARGE AND CALIBRATION CURRENT

0.2A WAKE-UP/PRECONDITIONING CURRENT HOST PROVIDES SMBus PULL-UP RESISTORS

ALL RESISTORS  ARE 1% UNLESS NOTED OTHERWISE

Q1: Si7216DN Q2: Si7445DN Q3: Si7983DP T1: BH510-1019

4110fb

Dual Battery Backup System Managers Controlling a Two Three-Series  Cell Li-Ion, Gas Gauge Smart

Batteries with Calibration Managed by Host Processor  and SMBus Multiplexer

SUPPLY INPUT (12V)

8.66k

RCL

0.02Ω

1W

INPUT IDEAL DIODE

Q3

INPUT

TO SYSTEM LOAD

1.21k

IDEAL DIODE	TO BACKUP

0.1μF LOW

ESR

0.1μF LOW

ESR

0.1μF Q5	LOW

ESR

Q4

0.1μF

LOW

ESR

LOAD

Q6

DCIN CLN

CLP

INID

DCOUT NC BATID

CHGFET

20μF VERY LOW ESR

1nF

33Ω

0.5W

5%

T1

1nF

33Ω

0.5W

5%

BATTERY

IDEAL DIODE

DCIN CLN

CLP

INID

DCOUT NC BATID

CHGFET

20μF VERY LOW ESR

1nF

33Ω

0.5W

5%

T2

1nF

33Ω

0.5W

5%

BATTERY

IDEAL DIODE

113k

RTHA 1.13k

RTHB 54.9k

DCDIV THA

THB VREF

VCHG

DCHFET ISENSE

CSP CSN

BAT

RSL

3.32k

2k

2k

Q1B Q1A

RSNS(FET)

0.05Ω

0.5W

330nF

1k

1k       330nF

20μF VERY LOW ESR

RSNS(BAT)

0.1Ω

0.25W

+

+

BATTERY 1 (12.6V)

10k

113k

RTHA 1.13k

RTHB 54.9k

DCDIV THA

THB VREF

VCHG

DCHFET ISENSE

CSP CSN

BAT

RSL

3.32k

2k

2k

Q2B Q2A

RSNS(FET)

0.05Ω

0.5W

330nF

1k

1k       330nF

20μF VERY LOW ESR

RSNS(BAT)

0.1Ω

0.25W

+

+

BATTERY 2 (12.6V)

10k

36.5k

VCAL VDIS ICHG

LTC4110

ITH

ACPDLY

3.01k	0.1μF

SMB1

NTC

+

36.5k

VCAL VDIS ICHG

LTC4110

ITH

ACPDLY

3.01k	0.1μF

SMB2

NTC

+

187k

37.4k

0.1μF

ICAL

IPCC

SELA SELC

TIMER

VDD

ACPb

GP101

68nF

0.1μF LOW

ESR

10.8V

3 CELL

187k

37.4k

0.1μF

ICAL

IPCC

SELA SELC

TIMER

VDD

ACPb

GP101

68nF

0.1μF LOW

ESR

10.8V

3 CELL

SMB1

SDA SCL TYPE

GP102

GP103

SHDN GND         SGND

SMB2

SDA SCL TYPE

GP102

GP103

SHDN GND         SGND

4110 TA06

7HR CHARGE TIME

HIGH CURRENT BACKUP LOAD DESIGN

0.5A BACKDRIVE CURRENT CUTOFF

HOST         	SMBUS

2

SMB1

LTC4110

TYPICAL APPLICATIONS

4110fb

50

1A CHARGE AND CALIBRATION CURRENT

0.2A WAKE-UP/PRECONDITIONING CURRENT

ALL RESISTORS  ARE 1% UNLESS NOTED OTHERWISE SEE LTC4305 DATA SHEET FOR PULL-UP  INFORMATION

Q1, Q2: Si7216DN Q3, Q4: Si7445DP Q5, Q6: Si7983DP T1, T2: BH510-1019

I  C/SMBus

MULTIPLEXOR

LTC4305

SMB2

PACKAGE DESCRIPTION

UHF Package

38-Lead Plastic QFN (5mm × 7mm)

(Reference LTC DWG # 05-08-1701)

0.70 p 0.05

5.50 p 0.05	5.15 ± 0.05

4.10 p 0.05

3.00 REF	3.15 ± 0.05

0.25 p 0.05

0.50 BSC

5.5 REF

6.10 p 0.05

7.50 p 0.05

RECOMMENDED SOLDER PAD LAYOUT

APPLY SOLDER MASK TO AREAS THAT ARE NOT SOLDERED

PACKAGE OUTLINE

5.00 p 0.10

0.75 p 0.05

0.00 – 0.05

3.00 REF

37	38

PIN 1 NOTCH

R = 0.30 TYP OR

0.35 s 45o CHAMFER

PIN 1

TOP MARK (SEE NOTE 6)

0.40 p0.10

1

2

7.00 p 0.10

5.50 REF

5.15 ± 0.10

3.15 ± 0.10

0.200 REF

0.25 p 0.05

0.50 BSC

R = 0.125

TYP

(UH) QFN REF C 1107

R = 0.10

TYP

BOTTOM VIEW—EXPOSED PAD

NOTE:

1. DRAWING CONFORMS TO JEDEC PACKAGE OUTLINE M0-220 VARIATION WHKD

2. DRAWING NOT TO SCALE

3. ALL DIMENSIONS  ARE IN MILLIMETERS

4. DIMENSIONS  OF EXPOSED PAD ON BOTTOM OF PACKAGE DO NOT INCLUDE

MOLD FLASH. MOLD FLASH, IF PRESENT,  SHALL NOT EXCEED 0.20mm ON ANY SIDE

5. EXPOSED PAD SHALL BE SOLDER PLATED

6. SHADED AREA IS ONLY A REFERENCE FOR PIN 1 LOCATION ON THE TOP AND BOTTOM OF PACKAGE

4110fb

51

Information furnished by Linear Technology Corporation  is believed to be accurate and reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no representa- tion that the interconnection of its circuits as described herein will not infringe on existing patent rights.

TYPICAL APPLICATION

Battery Backup System Manager Controlling a Three-Series  Cell Li-Ion, Gas

Gauge Smart Battery with Calibration Managed by Host Processor

SUPPLY INPUT (12V)

8.66k

RCL

0.033Ω

0.5W

INPUT IDEAL DIODE

Q2

0.1μF LOW ESR

22μF VERY LOW ESR

0.1μF LOW ESR

TO SYSTEM LOAD

TO BACKUP LOAD

Q3

15ms ACPDLY

7HR TIMER

LOW CURRENT BACKUP DESIGN

2.8V  CUTOFF VOLTAGE FOR VCAL AND VDIS

1A CHARGE CURRENT

0.2A CALIBRATION AND PRECONDITIONING CURRENT

0.3A  BACKDRIVE CURRENT CUTOFF

HOST PROVIDES SMBus PULL-UP RESISTORS

ALL RESISTORS  ARE 1% UNLESS NOTED OTHERWISE

1.21k

25.5k

RTHA 1.13k

RTHB 54.9k

DCIN CLN CLP DCDIV THA

THB VREF

VCHG

VCAL

INID

DCOUT NC BATID

CHGFET DCHFET

ISENSE

CSP CSN

BAT

RSL

3.32k

2k

2k

1nF

33Ω

0.5W

5% Q1A

1k

1k

T1

330nF

330nF

1nF

33Ω

0.5W

5% Q1B

RSNS(FET)

0.05Ω

0.5W

22μF VERY LOW ESR

RSNS(BAT)

0.1Ω

0.25W

+

+

BATTERY IDEAL DIODE

(12.6V)

Q1: Si7216DN Q2: Si7445DP Q3: SiA911DJ T1: BH510-1019

51.1k

30.1k

187k

37.4k

0.1μF

VDIS

ICHG ICAL IPCC

SELA SELC

LTC4110

ITH

ACPDLY TIMER

VDD

ACPb

GP101

3.01k	0.1μF

100nF

0.1μF LOW ESR

0.1μF

I2C TO HOST

+

10.8V

3 CELL

10k

NTC

I2C TO HOST

SDA SCL TYPE

GP102

GP103

SHDN GND	SGND

4110 TA02

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LTC1760 | Smart Battery System Manager | Autonomous Power Management and Battery Charging for Two Smart Batteries, SMBus Rev 1.1 Compliant |
| LTC1960 | Dual Battery Charger/Selector with SPI Interface | Simultaneous Charge or Discharge of Two Batteries, DAC Programmable Current and Voltage, Input Current Limiting Maximizes Charge Current |
| LTC4412/ LTC4412HV | PowerPath Controllers in ThinSOT™ | More Efﬁcient than Diode ORing, Automatic Switching Between DC Sources, Simpliﬁed Load Sharing, 3V ≤ VIN ≤ 28V, (3V ≤ VIN ≤ 36V for HV) ThinSOT Package |
| LTC4414 | 36V, Low Loss PowerPath Controller for Large PFETs | Drives Large QG PFETs, Very Low Loss Replacement for Power Supply ORing Diodes, 3.5V to 36V AC/DC Adapter Voltage Range, MSOP-8 Package |
| LTC4416/ LTC4416-1 | Dual, Low Loss PowerPath Controllers | Drives Large PFETs, Low Loss Replacement for Power Supply ORing Diodes, Operation to 36V, Programmable Autonomous Switching |

ThinSot is a Trademark of Linear Technology Corporation.

4110fb

Linear Technology Corporation

52

1630 McCarthy  Blvd.,  Milpitas,  CA 95035-7417

LT 0409 REV B • PRINTED IN USA

(408) 432-1900

● FAX: (408) 434-0507

●  www.linear.com	© LINEAR TECHNOLOGY CORPORATION 2008