FEATURES

n  25.5W IEEE 802.3at  Compliant (Type 2) PD

n  10V to 57V Auxiliary Power Input

IEEE 802.3at PD with Synchronous No-Opto Flyback Controller and 12V  Aux  Support

DESCRIPTION

The LTC®4278  is an integrated Powered Device (PD)

controller and switching regulator intended for high power

n  Shutdown Pin for Flexible  Auxiliary Power Support

n  Integrated State-of-the-Art No-Opto Synchronous

Flyback Controller

– Isolated Power Supply Efﬁciency >92%

– 88% Efﬁciency Including Diode Bridge and

Hot Swap™ FET

n  Superior EMI Performance

n  Robust 100V 0.7Ω  (Typ) Integrated Hot Swap MOSFET

n  IEEE 802.3at  High Power Available Indicator

n  Integrated Signature Resistor and Programmable

Class Current

n  Undervoltage, Overvoltage and Thermal Protection

n  Short-Circuit Protection with Auto-Restart

n  Programmable Soft-Start and Switching Frequency

n  Complementary Power Good Indicators

n  Thermally Enhanced 7mm × 4mm DFN Package

APPLICATIONS

n  VoIP Phones  with Advanced Display Options

n  Dual-Radio Wireless Access Points

n  PTZ Security Cameras

n  RFID Readers

n  Industrial Controls

IEEE 802.3at and 802.3af applications. With a wide input voltage range, the LTC4278 is speciﬁcally designed to sup- port PD applications that include a low-voltage auxiliary power input such as a 12V wall adaptor.  The inclusion of a shutdown pin provides simple implementation of both PoE and auxiliary dominate applications.  In addition, the LTC4278 supports both 1-event and 2-event classiﬁcations as deﬁned by the IEEE, thereby allowing the use in a wide range of product conﬁgurations.

The LTC4278 synchronous, current mode, ﬂyback control- ler generates multiple supply rails in a single conversion step providing for the highest system efﬁciency while main- taining tight regulation across all outputs. The LTC4278 includes Linear Technology’s patented No-Opto feedback topology to provide  full IEEE 802.3 isolation without the need of an opto-isolator circuit. A true soft-start function allows graceful ramp-up of all output voltages.

The LTC4278 is available in a space saving  32-pin  DFN

package.

L, LT, LTC, LTM, Linear Technology and the Linear logo are registered trademarks of Linear

Technology Corporation. Hot Swap is a trademark of Linear Technology Corporation.

All other trademarks are the property of their respective owners. Protected by U.S. Patents, including 5841643.

TYPICAL APPLICATION

+

25W PD Solution with 12V Auxiliary

2.2μH

EPC3472G-LF

•

0.18μH

5V

5A

47μF   +

AUXILIARY SUPPLY

(10V TO 57V)

–

54V FROM	~ +

100k

680k

PDS5100H

BSS63LT1

+

10μF

294k

41.2k

10μF

TO MICRO CONTROLLER

21.5k

3.01k

1μF

•    •

FDMS2572

HAT2169

100μF

DATA PAIR 	~ –

54V FROM	~ +

SPARE PAIR 	~ –

0.1μF

24k

30.9Ω

UVLO  PWRGD VPORTP

SHDN RCLASS

VPORTN

T2P

FB	VCC

LTC4278

PG     SENSE+

SENSE–

SG VCMP

12mΩ

1μF

2.2nF

VNEG

SYNC GND OSC

PGDLY

tON

ENDLY

RCMP

CCMP

•    •

33pF

10k

100k

38.3k

1.8k

0.1μF

4278 TA01a

4278f

ABSOLUTE MAXIMUM RATINGS

(Notes 1, 2)

Pins with Respect to VPORTN

PIN CONFIGURATION

TOP VIEW

VPORTP Voltage......................................... –0.3V to 100V

VNEG Voltage ......................................... –0.3V to VPORTP

VNEG Pull-Up  Current  ..................................................1A

SHDN ....................................................... –0.3V to 100V

RCLASS, Voltage............................................ –0.3V to 7V RCLASS Source Current...........................................50mA PWRGD Voltage (Note 3)

Low Impedance Source ...... VNEG –0.3V  to VNEG +11V Sink Current.........................................................5mA PWRGD, T2P Voltage ............................... –0.3V to 100V PWRGD, T2P Sink Current .....................................10mA

Pins with Respect to GND

VCC Voltage ................................................ –0.3V to 22V SENSE–, SENSE+ Voltage ........................ –0.5V to +0.5V UVLO, SYNC Voltage...................................–0.3V to VCC FB Current..............................................................±2mA

SHDN T2P RCLASS

NC

VPORTN VPORTN NC

NC SG VCC

tON

ENDLY SYNC SFST OSC FB

1	32

2	31

3	30

4	29

5	28

6	27

7	26

8	33	25

9	24

10	23

11	22

12	21

13	20

14	19

15	18

16	17

DKD32 PACKAGE

VPORTP

NC PWRGD PWRGD NC

VNEG VNEG NC

PG PGDLY RCMP

CCMP

SENSE+

SENSE – UVLO VCMP

VCMP Current .........................................................±1mA

Operating Ambient Temperature Range

LTC4278C-1 ................................................. 0°C to 70°C LTC4278I-1 .............................................. –40°C to 85°C

32-LEAD (7mm × 4mm) PLASTIC DFN

TJMAX = 125°C,  JA  = 34°C/W, JC  = 2°C/W

GND, EXPOSED PAD (PIN 33) MUST BE SOLDERED TO A

HEAT SINKING PLANE THAT IS CONNECTED TO VNEG

ORDER INFORMATION

| LEAD FREE FINISH | TAPE AND REEL | PART MARKING* | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- |
| LTC4278CDKD#PBF LTC4278IDKD#PBF | LTC4278CDKD#TRPBF LTC4278IDKD#TRPBF | 4278 4278 | 32-Lead (7mm × 4mm) Plastic DFN 32-Lead (7mm × 4mm) Plastic DFN | 0°C to 70°C –40°C to 85°C |

Consult LTC Marketing for parts speciﬁed with wider operating temperature ranges. *The temperature grade is identiﬁed by a label on the shipping container.

For more information on lead free part marking, go to: http://www.linear.com/leadfree/

For more information on tape and reel speciﬁcations, go to: http://www.linear.com/tapeandreel/

4278f

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C.

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |

Interface Controller (Note 4)

| Operating Input Voltage Signature Range Classiﬁcation Range ON Voltage OFF Voltage Overvoltage Lockout | At VPORTP (Note 5) | l l l l | 60 1.5 9.8 12.5 21 37.2 30.0 71 | V V V V V V |
| --- | --- | --- | --- | --- |
| ON/OFF Hysteresis Window |  | l | 4.1 | V |
| Signature/Class Hysteresis Window |  | l | 1.4 | V |
| Reset Threshold | State Machine Reset for 2-Event Classiﬁcation | l | 2.57 5.40 | V |

Supply Current

| Supply Current at 57V | Measured at VPORTP Pin | l | 1.35 | mA |
| --- | --- | --- | --- | --- |
| Class 0 Current | VPORTP = 17.5V, No RCLASS Resistor | l | 0.40 | mA |

Signature

| Signature Resistance | 1.5V ≤ VPORTP ≤ 9.8V (Note 6) | l | 23.25 26 | kΩ |
| --- | --- | --- | --- | --- |
| Invalid Signature Resistance, SHDN Invoked | 1.5V ≤ VPORTP ≤ 9.8V, VSHDN = 3V (Note 6) | l | 11 | kΩ |
| Invalid Signature Resistance During Mark Event | (Notes 6, 7) | l | 11 | kΩ |

Classiﬁcation

| Class Accuracy | 10mA < ICLASS < 40mA, 12.5V < VPORTP < 21V (Notes 8, 9) | l | ±3.5 | % |
| --- | --- | --- | --- | --- |
| Classiﬁcation Stability Time | VPORTP Pin Step to 17.5V, RCLASS = 30.9, ICLASS Within 3.5% of Ideal Value (Notes 8, 9) | l | 1 | ms |

Normal Operation

| Inrush Current | VPORTP = 54V, VNEG = 3V | l | 60 100 180 | mA |
| --- | --- | --- | --- | --- |
| Power FET On-Resistance | Tested at 600mA into VNEG, VPORTP = 54V | l | 0.7 1.0 | Ω |
| Power FET Leakage Current at VNEG | VPORTP = SHDN = VNEG = 57V | l | 1 | μA |

Digital Interface

| SHDN Input High Level Voltage |  | l | 3 | V |
| --- | --- | --- | --- | --- |
| SHDN Input Low Level Voltage |  | l | 0.45 | V |
| SHDN Input Resistance | VPORTP = 9.8V, SHDN = 9.65V | l | 100 | kΩ |
| PWRGD, T2P Output Low Voltage | Tested at 1mA, VPORTP = 54V. For T2P, Must Complete 2-Event Classiﬁcation to See Active Low | l | 0.15 | V |
| PWRGD, T2P Leakage Current | Pin Voltage Pulled 57V, VPORTP = VPORTN = 0V | l | 1 | μA |
| PWRGD Output Low Voltage | Tested at 0.5mA, VPORTP = 52V, VNEG = 48V, Output Voltage Is With Respect to VNEG | l | 0.4 | V |
| PWRGD Clamp Voltage | Tested at 2mA, VNEG = 0V, Voltage With Respect to VNEG | l | 12 16.5 | V |
| PWRGD Leakage Current | VPWRGD = 11V, VNEG = 0V, Voltage With Respect to VNEG | l | 1 | μA |

4278f

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C.

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |

PWM Controller (Note 10)

Power Supply

| VCC Operating Range |  | ● | 4.5 20 | V |
| --- | --- | --- | --- | --- |
| VCC Supply Current (ICC) | VCMP = Open (Note 11) | ● | 4 6.4 10 | mA |
| VCC Shutdown Current | VCMP = Open, VUVLO = 0V | ● | 50 150 | μA |

Feedback Ampliﬁer

| Feedback Regulation Voltage (VFB) |  | ● | 1.220 1.237 1.251 | V |
| --- | --- | --- | --- | --- |
| Feedback Pin Input Bias Current | RCMP Open |  | 200 | nA |
| Feedback Ampliﬁer Transconductance | ΔIC = ±10μA | ● | 700 1000 1400 | μmho |
| Feedback Ampliﬁer Source or Sink Current |  | ● | 25 55 90 | μA |
| Feedback Ampliﬁer Clamp Voltage | VFB = 0.9V VFB = 1.4V |  | 2.56 0.84 | V V |
| Reference Voltage Line Regulation | 12V ≤ VCC ≤ 18V | ● | 0.005 0.05 | %/ V |
| Feedback Ampliﬁer Voltage Gain | VCMP = 1.2V to 1.7V |  | 1400 | V/ V |
| Soft-Start Charging Current | VSFST = 1.5V |  | 16 20 25 | μA |
| Soft-Start Discharge Current | VSFST = 1.5V, VUVLO = 0V |  | 0.7 1.3 | mA |
| Control Pin Threshold (VCMP) | Duty Cycle = Min |  | 1 | V |

Gate Outputs

| PG, SG Output High Level |  | ● | 6.6 7.4 8 | V |
| --- | --- | --- | --- | --- |
| PG, SG Output Low Level |  | ● | 0.01 0.05 | V |
| PG, SG Output Shutdown Strength | VUVLO = 0V; IPG, ISG = 20mA | ● | 1.6 2.3 | V |
| PG Rise Time | CPG = 1nF |  | 11 | ns |
| SG Rise Time | CSG = 1nF |  | 15 | ns |
| PG, SG Fall Time | CPG, CSG = 1nF |  | 10 | ns |

Current Ampliﬁer

| VSENSE Switch Current Limit at Maximum VCMP | + | ● | 88 98 110 | mV |
| --- | --- | --- | --- | --- |
| ΔVSENSE /ΔVCMP |  |  | 0.07 | V/ V |
| Sense Voltage Overcurrent Fault Voltage | + | ● | 206 230 | mV |

Timing

VSENSE  , VSFST < 1V

| Switching Frequency (fOSC) | COSC = 100pF | ● | 84 100 110 | kHz |
| --- | --- | --- | --- | --- |
| Oscillator Capacitor Value (COSC) | (Note 12) |  | 33 200 | pF |
| Minimum Switch On Time (tON(MIN)) |  |  | 200 | ns |
| Flyback Enable Delay Time (tENDLY) |  |  | 265 | ns |
| PG Turn-On Delay Time (tPGDLY) |  |  | 200 | ns |
| Maximum Switch Duty Cycle |  | ● | 85 88 | % |
| SYNC Pin Threshold |  | ● | 1.53 2.1 | V |
| SYNC Pin Input Resistance |  |  | 40 | kΩ |

4278f

ELECTRICAL CHARACTERISTICS 	The l denotes the speciﬁcations  which apply over the full operating temperature range, otherwise speciﬁcations  are at TA = 25°C.

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |

Load Compensation

VRCMP with VSENSE

| = 0V Load Compensation to VSENSE Offset Voltage | + |  | 0.8 | mV |
| --- | --- | --- | --- | --- |
| Feedback Pin Load Compensation Current | + |  | 20 | μA |

UVLO Function

VSENSE

= 20mV, VFB = 1.230V

| UVLO Pin Threshold (VUVLO) |  | ● | 1.215 1.240 1.265 | V |
| --- | --- | --- | --- | --- |
| UVLO Pin Bias Current | VUVLO = 1.2V VUVLO = 1.3V |  | –0.25 0.1 0.25 –4.50 –3.4 –2.50 | μA μA |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime.

Note 2: Pins with 100V absolute maximum guaranteed  for T ≥ 0°C,

Note 7: An invalid signature  after the 1st classiﬁcation event is mandated by the IEEE802.3at standard.  See the Applications Information section. Note 8: Class accuracy is with respect to the ideal current deﬁned as

1.237/RCLASS and does not include variations in RCLASS resistance.

Note 9: This parameter is assured by design and wafer level testing.

–

otherwise 90V.

Note 10: VCC = 14V; PG, SG Open; VCMP = 1.4V, VSENSE

= 0V, RCMP = 1k,

Note 3: Active high PWRGD internal clamp self-regulates to 14V with

respect to VNEG.

Note  4: All voltages are with respect to VPORTN pin unless otherwise noted. Note 5: Input voltage speciﬁcations  are deﬁned with respect to LTC4278 pins and meet  IEEE 802.3af/at speciﬁcations when the input diode bridge

is included.

Note 6: Signature resistance is measured via the ΔV/ΔI method with the minimum ΔV of 1V. The LTC4278 signature resistance accounts for the additional series resistance in the input diode bridge.

RtON = 90k, RPGDLY = 27.4k,  RENDLY = 90k, unless otherwise  speciﬁed.  All voltages are with respect to GND.

Note 11: Supply current does not include gate charge current to the

MOSFETs. See the Applications Information section.

Note 12: Component value range guaranteed by design.

4278f

TYPICAL PERFORMANCE CHARACTERISTICS

Input Current vs Input Voltage

25k Detection Range	Input Current vs Input Voltage	Input Current vs Input Voltage

| 0.5 TA = 25°C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

50

TA = 25°C

11.0

0.4

CLASS 4

40

0.3

VPORTP CURRENT (mA)

30	CLASS 3

10.5

VPORTP CURRENT (mA)

0.2

0.1

20	CLASS 2

CLASS 1

10

VPORTP CURRENT (mA)

10.0

0

0	2	4	6	8	10

CLASS 0

0

0	10	20	30	40

50	60

9.5

| 12 CL | ASS 1 OPERATIO |  |  | N |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 85°C |  |  |  |
|  |  |  | –40°C |  |  |  |

14	16

18	20	22

VPORTP  VOLTAGE (V)

4278 G01

VPORTP  VOLTAGE (V) (RISING)

4278 G02

VPORTP  VOLTAGE (V)

4278 G03

Signature Resistance vs Input Voltage

RESISTANCE = V = V2 – V1

VPORTP

| Class Operation vs Time On-Resistance vs Temperature |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

TA = 25°C

TA = 25°C

26

25

24

I 	I2 – I1

IEEE UPPER  LIMIT LTC4278 + 2 DIODES

10V/DIV

CLASS CURRENT

1.0

0.8

0.6

LTC4278  ONLY

SIGNATURE RESISTANCE (kΩ)

23

IEEE LOWER LIMIT

10mA/DIV

0.4

22

V1:   1	3	5	7	9

TIME (10μs/DIV)	4278 G05

0.2

–50

–25	0	25	50

RESISTANCE (Ω)

75	100

V2:  2	4

6	8	10

JUNCTION TEMPERATURE (°C)

VPORTP  VOLTAGE (V)

4278 G04

| 4278 G06 |  |  |  |
| --- | --- | --- | --- |

| 0.8 TA = 25°C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

PWRGD, T2P Output Low Voltage vs Current

1.0

Active High PWRGD

Output Low Voltage vs Current	Inrush Current vs Input Voltage

115

VPWRGD – VPORTN (V) VT2P – VPORTN (V)

0.6

0.4

0.2

0.8

PWRGD (V)

0.6

0.4

0.2

110

CURRENT (mA)

105

100

95

90

0

0	2	4	6	8	10

CURRENT (mA)

0

| 0 0.5 1 TA = 25°C VPORTP – VNEG = 4 |  | V |  |  |
| --- | --- | --- | --- | --- |

CURRENT (mA)

1.5	2

85

40	45	50

VPORTP  VOLTAGE (V)

55	60

4278 G07

4278 G08

4278 G09

4278f

TYPICAL PERFORMANCE CHARACTERISTICS

VCC Shutdown Current

vs Temperature	VCC Current vs Temperature	SENSE Voltage vs Temperature

80	9

70

8

60

50	7

IVCC (mA)

| 40 6 |  |  |  |  |  |  | VUVLO = 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC = 14V |  |  |  |  |  |

VCC CURRENT (μA)

30

5

20

4

10

0	3

DYNAMIC CURRENT CPG = 1nF, CSG = 1nF, fOSC = 100kHz

STATIC PART CURRENT

VCC = 14V

108

106

SENSE  VOLTAGE (mV)

104

102

100

98

96

94

92

90

FB = 1.1V

SENSE = VSENSE+

WITH V	– = 0V

SENSE

–50

–25	0

25	50

75	100     125

–50

–25	0

25	50

75	100     125

–50

–25	0

25	50	75

100    125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

4278 G02

4278 G12

4278 G13

| 220 SENSE = VSENSE+ WITH VSENSE– = 0V |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

215

SENSE  VOLTAGE (mV)

210

205

200

195

190

185

180

SENSE Fault Voltage vs Temperature

110

108

106

104

102

100

98

96

94

92

| 90 COSC = 100pF |  |
| --- | --- |

Oscillator Frequency

fOSC (kHz)

vs Temperature	VFB vs Temperature

1.240

1.239

1.238

1.237

VFB (V)

1.236

1.235

1.234

1.233

1.232

1.231

1.230

–50

–25	0

25	50

75     100    125

–50

–25	0

25	50	75

100    125

–50

–25	0

25	50	75

100    125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

4278 G14

4278 G15

4278 G16

Feedback  Pin Input Bias

vs Temperature	VFB Reset vs Temperature

Feedback Ampliﬁer Output

Current vs VFB

FEEDBACK PIN INPUT BIAS (nA)

250

200

150

100

50

1.03

1.02

1.01

VFB RESET (V)

1.00

0.99

0.98

0.97

50

30

(μA)

10

I

VCMP

–10

–30

–50

125°C

25°C

–40°C

| 0 RCMP OPEN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

–50

–25	0

25	50

75	100     125

0.96

–50

–25	0

25	50

| 75 100 125 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

–70

0.9

1	1.1

1.2	1.3

1.4

1.5

TEMPERATURE (°C)

TEMPERATURE (°C)

VFB (V)

4278 G17

4278 G18

4278 G19

4278f

TYPICAL PERFORMANCE CHARACTERISTICS

Feedback Ampliﬁer Source and

Sink Current vs Temperature

| 70 |  |  | SOURCE CURRENT VFB = 1.1V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINK CURRENT VFB = 1.4V |  |  |

65

60

IVCMP (μA)

55

50

45

40

| 1100 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1050

gm (μmho)

1000

950

900

Feedback Ampliﬁer gm

vs Temperature

| 1700 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1650

1600

1550

1500

AV (V/V)

1450

1400

1350

1300

1250

1200

1150

1100

Feedback Ampliﬁer Voltage Gain vs Temperature

–50

–25	0

25	50

75	100    125

–50

–25	0	25	50

75     100    125

–50

–25	0

25	50

75     100    125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

4278 G20

4278 G21

4278 G22

| 1.250 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

UVLO vs Temperature	IUVLO Hysteresis vs Temperature

| 3.7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.245

3.6

1.240

UVLO (V)

1.235

1.230

1.225

3.5

3.4

IUVLO (μA)

3.3

3.2

3.1

1.220

–50

–25	0

25	50

75	100     125

3.0

–50

–25	0

25	50

75	100     125

TEMPERATURE (°C)

TEMPERATURE (°C)

4278 G23

4278 G24

Soft-Start Charge Current vs Temperature

| 23 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

22

21

20

19

18

SFST CHARGE CURRENT (μA)

17

16

PG, SG Rise and Fall Times vs Load Capacitance

| 80 TA = 25°C |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FALL TIME |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | RISE TIME |  |  |  |  |

70

60

TIME (ns)

50

40

30

20

10

15

–50

–25	0

25	50

75     100    125

0

0     1     2     3

4     5     6     7

8     9    10

TEMPERATURE (°C)

CAPACITANCE (nF)

4278 G25

4278 G26

4278f

TYPICAL PERFORMANCE CHARACTERISTICS

| 340 RtON(MIN) = 158k |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

330

320

tON(MIN) (ns)

310

Minimum  PG On-Time

vs Temperature	PG Delay Time vs Temperature

| 300 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RPGDLY = 27.4k |  |  |  |  |
|  |  |  | RPGDLY = 16.9k |  |  |  |  |

250

200

| 325 RENDLY = 90k |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

tENDLY (ns)

305

285

Enable Delay Time vs Temperature

300

290

280

270

tPGDLY (ns)

150

100

50

265

245

225

260

–50

–25	0

25	50

75     100    125

0

–50

–25	0

25	50	75

100    125

205

–50

–25	0

25	50

75	100     125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

4278 G28

4278 G29

4278 G30

4278f

PIN FUNCTIONS

SHDN (Pin 1): Shutdown Input. Use this pin for auxiliary power application.  Drive SHDN high to disable LTC4278 operation and corrupt the signature resistance. If unused, tie SHDN to VPORTN.

T2P (Pin 2): Type 2 PSE Indicator, Open-Drain. Low imped- ance indicates the presence of a Type 2 PSE.

RCLASS  (Pin 3): Class Select Input. Connect a resistor between  RCLASS and VPORTN to set the classiﬁcation load current (see Table 2).

NC (Pins  4, 7, 8, 25, 28, 31): No Connect.

VPORTN (Pins 5, 6): Input Voltage, Negative Rail. Pin 5 and

Pin 6 must be electrically tied together at the package.

SG (Pin 9): Synchronous Gate Drive Output.  This pin provides an output signal for a secondary-side synchro- nous rectiﬁer. Large dynamic currents may ﬂow during voltage transitions. See the Applications Information section for details.

VCC  (Pin 10): Supply Voltage  Pin. Bypass this pin to GND with a low ESR ceramic capacitor. See the Applica- tions Information section for details.

tON  (Pin 11): Pin for external programming resistor to set the minimum time that the primary switch is on for each cycle. Minimum turn-on facilitates the isolated feed- back method. See the Applications Information section for details.

ENDLY (Pin 12): Pin for external programming resistor to set enable delay time. The enable delay time disables the feedback ampliﬁer for a ﬁxed time after the turn-off of the primary-side MOSFET. This allows the leakage inductance voltage spike to be ignored for ﬂyback voltage sensing. See the Applications Information section for details.

SYNC (Pin 13): External  Sync  Input.  This pin is used to synchronize the internal oscillator with an external clock. The positive edge of the clock causes the oscillator to discharge causing PG to go low (off) and SG high (on). The sync threshold is typically 1.5V. Tie to ground  if unused. See the Applications Information section for details.

SFST (Pin 14): Soft-Start.  This pin, in conjunction  with a capacitor (CSFST) to GND, controls  the ramp-up of peak primary current through the sense resistor. It is also used to control converter inrush at start-up.  The SFST clamps the VCMP voltage and thus limits peak current until soft- start is complete. The ramp time is approximately 70ms per μF of capacitance.  Leave SFST open if not using the soft-start function.

OSC (Pin 15): Oscillator. This pin, in conjunction with an external capacitor (COSC) to GND, deﬁnes  the controller oscillator frequency. The frequency is approximately

100kHz • 100/COSC (pF).

FB (Pin 16): Feedback Ampliﬁer Input. Feedback is usually sensed via a third winding and enabled during the ﬂyback period. This pin also sinks additional current to compensate for load current variation as set by the RCMP pin. Keep the Thevenin equivalent resistance  of the feedback divider at roughly 3k.

VCMP (Pin 17): Frequency Compensation  Control. VCMP is used for frequency compensation of the switcher con- trol loop. It is the output of the feedback ampliﬁer and the input to the current comparator. Switcher frequency compensation components  are placed on this pin to GND. The voltage on this pin is proportional to the peak primary switch current. The feedback ampliﬁer output is enabled during the synchronous switch on time.

UVLO (Pin 18): Undervoltage Lockout. A resistive divider from VPORTP to this pin sets an undervoltage lockout based upon  VPORTP level (not VCC). When the UVLO pin is below its threshold, the gate drives are disabled, but the part draws its normal quiescent current from VCC.

The bias current on this pin has hysteresis such that the bias current is sourced when UVLO threshold is exceeded. This introduces a hysteresis at the pin equivalent to the bias current change times the impedance of the upper divider resistor. The user can control the amount of hysteresis by adjusting the impedance of the divider. Tie the UVLO pin to VCC if not using this function. See the Applications

4278f

PIN FUNCTIONS

Information section for details. This pin is used for the UVLO function of the switching regulator. The PD interface section has an internal UVLO.

SENSE –, SENSE+ (Pins  19, 20): Current Sense Inputs. These pins are used to measure primary-side switch cur- rent through an external sense resistor. Peak primary-side current is used in the converter control loop. Make Kelvin connections to the sense resistor RSENSE to reduce noise

PGDLY (Pin 23): Primary Gate Delay Control. Connect an external programming resistor (RPGDLY) to set delay from synchronous gate turn-off to primary gate turn-on. See the Applications Information section for details.

PG (Pin 24): Primary  Gate Drive. PG is the gate drive pin for the primary-side MOSFET switch. Large dynamic cur- rents ﬂow during voltage transitions. See the Applications Information section for details.

problems. SENSE– connects to the GND side. At maximum current  (VCMP at its maximum  voltage) SENSE pins have

VNEG

(Pins 26, 27): System Negative Rail. Connects VNEG

100mV threshold. The signal is blanked (ignored) during the minimum turn-on time.

CCMP (Pin  21): Load Compensation  Capacitive Control. Connect a capacitor  from CCMP to GND in order  to reduce the effects of parasitic resistances in the feedback sensing path. A 0.1μF ceramic capacitor sufﬁces for most applica- tions. Short this pin to GND when load compensation is not needed.

RCMP  (Pin 22): Load Compensation  Resistive Control. Connect a resistor from RCMP to GND in order to com- pensate for parasitic resistances in the feedback sensing path. In less demanding applications, this resistor is not needed and this pin can be left open. See the Applications Information section for details.

to VPORTN through an internal power MOSFET. Pin 26 and

Pin 27 must be electrically tied together at the package.

PWRGD (Pin  29): Power Good Output, Open-Collector. High impedence  signals power-up completion. PWRGD is referenced to VNEG and features a 14V clamp.

PWRGD (Pin 30): Complementary Power Good Output, Open-Drain. Low impedance signals power-up completion. PWRGD is referenced to VPORTN.

VPORTP (Pin  32): Positive Power Input. Tie to the input port power through the input diode bridge.

Exposed Pad (Pin 33): Ground. This is the negative rail connection for both signal ground and gate driver grounds of the ﬂyback controller. This pin should be connected to VNEG.

4278f

BLOCK DIAGRAM

SHDN

1

T2P

2

RCLASS

3

CLASSIFICATION CURRENT LOAD

1.237V 	+

–

VPORTP

32

16k      25k

NC

31

CONTROL CIRCUITS

PWRGD

30

4   NC

PWRGD

29

VPORTN

5

14V

VPORTN

VNEG

6 	27

7    NC

8    NC VCC

CLAMPS

BOLD LINE INDICATES HIGH CURRENT PATH

VNEG

26

10

0.8V	+

–

1.237V REFERENCE (VFB)

DISABLE

+

–

INTERNAL REGULATOR

0.7

1.3

3V

–	ERROR AMP

+

S    Q

R    Q     COLLAPSE DETECT

FB

16

VCMP

17

UVLO

18

UVLO

1V	SFST

14

IUVLO

TSD

CURRENT COMPARATOR

OVERCURRENT FAULT

CURRENT SENSE AMP

SENSE–

19

CURRENT TRIP

OSC

15

SYNC

13

OSCILLATOR

SLOPE COMPENSATION SET	ENABLE

RCMPF

50k

+	–

LOAD

SENSE+

20

CCMP

21

tON

11

LOGIC BLOCK

COMPENSATION

RCMP

PGDLY

23

TO FB	22

ENDLY

12

PGATE SGATE

GATE DRIVE 	PG

24

25  NC	3V

28  NC

GATE DRIVE

SG

9

GND

33 (EXPOSED PAD)

4278  BD

4278f

APPLICATIONS INFORMATION

OVERVIEW

VPORTP (V)

Power over Ethernet (PoE) continues to gain popularity as more products are taking advantage of having DC power and high speed data available from a single RJ45 connector. As PoE continues to grow in the marketplace, powered device (PD) equipment vendors are running into the 12.95W power limit established by the IEEE 802.3af standard.

50

40

30 	ON

20

10 		CLASSIFICATION DETECTION V2

DETECTION V1

50

dV =  INRUSH

OFF

TIME

The IEE802.3at  standard establishes a higher power allocation  for Power over Ethernet while maintaining backwards  compatibility with the existing  IEEE 802.3af systems. Power sourcing equipment (PSE) and powered devices are distinguished as Type 1 complying with the IEEE 802.3af/IEEE 802.3at  power levels, or Type 2 com-

40 	dt 	C1

30 	OFF	ON

20

VPORTP – VNEG (V)

10

OFF

 = RLOAD C1

TIME TIME

plying with the IEEE 802.3at power levels. The maximum available power of a Type 2 PD is 25.5W.

The IEEE 802.3at standard also establishes a new method of acquiring power classiﬁcation from a PD and communi- cating the presence of a Type 2 PSE.  A Type  2 PSE has the option of acquiring PD power classiﬁcation by performing

2-event classiﬁcation (layer 1) or by communicating with the PD over the data line (layer 2). In turn, a Type 2 PD must be able to recognize both layers of communications and identify a Type 2 PSE.

The LTC4278 is speciﬁcally designed to support the front end of a PD that must operate under the IEEE 802.3at standard. In particular, the LTC4278  provides the T2P

–10

–20

PWRGD – VNEG (V)

–30

VPORTP – PWRGD (V)

–40

–50

20

10

POWER

BAD

PWRGD TRACKS VPORTP

POWER BAD

PD CURRENT

INRUSH

POWER GOOD

POWER GOOD

POWER

BAD

PWRGD TRACKS VPORTP

PWRGD TRACKS VPORTN

POWER BAD

IN DETECTION

RANGE TIME

LOAD, ILOAD

indicator bit which recognizes 2-event classiﬁcation. This indicator bit may be used to alert the LTC4278 output load that a Type 2 PSE is present. With an internal signature resistor, classiﬁcation circuitry, inrush control, and ther-

CLASSIFICATION

DETECTION I2

DETECTION I1

TIME

mal shutdown, the LTC4278 is a complete  PD Interface

I1 =

V1 – 2 DIODE DROPS

25kΩ

I2 =

V2 – 2 DIODE DROPS

25kΩ

solution capable of supporting in the next generation PD

applications.

MODES OF OPERATION

ICLASS DEPENDENT ON RCLASS  SELECTION INRUSH = 100mA

ILOAD = VPORTP

RLOAD

LTC4278

IIN	RLOAD

The LTC4278 has several modes of operation depending on the input voltage applied between  the VPORTP and VPORTN pins. Figure 1 presents an illustration of voltage and current waveforms the LTC4278 may encounter  with the various

PSE

RCLASS

RCLASS

VPORTN

VPORTP

PWRGD PWRGD VNEG

C1

4278 F01

modes of operation summarized  in Table 1.

Figure 1. VNEG, PWRGD, PWRGD and PD Current as a Function of Input Voltage

4278f

APPLICATIONS INFORMATION

Table 1. LTC4278 Modes  of Operation as a Function of Input Voltage

| VPORTP–VPORTN (V) | LTC4278 MODES OF OPERATION |
| --- | --- |
| 0V to 1.4V | Inactive (Reset After 1st Classiﬁcation Event) |
| 1.5V to 9.8V (5.4V to 9.8V) | 25k Signature Resistor Detection Before 1st Classiﬁcation Event (Mark, 11k Signature Corrupt After 1st Classiﬁcation Event) |
| 12.5V to ON/OFF* | Classiﬁcation Load Current Active |
| ON/OFF* to 60V | Inrush and Power Applied To PD Load |
| >71V | Overvoltage Lockout, Classiﬁcation and Hot Swap Are Disabled |

*ON/OFF includes hysteresis. Rising input threshold, 37.2V Max. Falling input threshold,  30V Min.

These modes satisfy the requirements deﬁned in the

IEEE 802.3af/IEEE  802.3at speciﬁcation.

INPUT DIODE BRIDGE

In the IEEE 802.3af/IEEE 802.3at  standard,  the modes of operation reference the input voltage at the PD’s RJ45 connector. Since the PD must handle power received in either polarity from either the data or the spare pair, input diode bridges BR1 and BR2 are connected between the RJ45 connector and the LTC4278 (Figure 2).

The input diode bridge introduces a voltage drop that affects the range for each mode of operation. The LTC4278 compensates for these voltage drops so that a PD built with the LTC4278 meets the IEEE 802.3af/IEEE

802.3at-established  voltage ranges. Note the Electrical Characteristics are referenced with respect to the LTC4278 package pins.

DETECTION

During detection, the PSE looks for a 25k signature resis- tor which identiﬁes the device as a PD. The PSE will apply two voltages in the range of 2.8V to 10V and measures the corresponding currents. Figure 1 shows the detection voltages V1 and V2 and the corresponding PD current. The PSE calculates the signature resistance using the V/I measurement technique.

The LTC4278 presents its precision, temperature-com- pensated 25k resistor between the VPORTP and VPORTN pins, alerting the PSE that a PD is present and requests power to be applied. The LTC4278 signature  resistor  also compensates for the additional series resistance intro-

duced by the input diode bridge. Thus a PD built with the LTC4278 conforms  to the IEEE 802.3af/IEEE  802.3at speciﬁcations.

RJ45

1

TX+ 	T1

TX–

BR1

POWERED DEVICE (PD) INPUT

RX+

3

2

RX–

6

SPARE+

4

5

7

8

TO PHY

BR2

0.1μF

100V  D3

VPORTP

LTC4278

VPORTN

4278 F02

SPARE–

Figure 2. PD Front End Using Diode Bridges on Main and Spare Inputs

4278f

APPLICATIONS INFORMATION

SIGNATURE CORRUPT OPTION

In some designs that include an auxiliary power option, it is necessary to prevent a PD from being detected by a PSE. The LTC4278 signature resistance can be corrupted with the SHDN pin (Figure 3). Taking the SHDN pin high will reduce  the signature resistor below 11k which is an invalid signature  per the IEEE 802.3af/IEEE 802.3at speci- ﬁcation, and alerts the PSE not to apply power. Invoking the SHDN pin also ceases operation for classiﬁcation and disconnects the LTC4278 load from the PD input.  If this feature is not used, connect  SHDN to VPORTN.

Table 2. Summary of Power Classiﬁcations  and LTC4278

RCLASS Resistor Selection

| CLASS | USAGE | MAXIMUM POWER LEVELS AT INPUT OF PD (W) | NOMINAL CLASSIFICATION LOAD CURRENT (mA) | LTC4278 RCLASS RESISTOR (Ω, 1%) |
| --- | --- | --- | --- | --- |
| 0 | Type 1 | 0.44 to 12.95 | < 0.4 | Open |
| 1 | Type 1 | 0.44 to 3.84 | 10.5 | 124 |
| 2 | Type 1 | 3.84 to 6.49 | 18.5 | 69.8 |
| 3 | Type 1 | 6.49 to 12.95 | 28 | 45.3 |
| 4 | Type 2 | 12.95 to 25.5 | 40 | 30.9 |

2-EVENT CLASSIFICATION AND THE T2P PIN

A Type 2 PSE may declare the availability of high power by

TO PSE

LTC4278

SHDN

VPORTN

16k

VPORTP

25k SIGNATURE RESISTOR

SIGNATURE DISABLE

4278 F03

performing a 2-event classiﬁcation (layer 1) or by com- municating over the high speed data line (layer 2). A Type

2 PD must recognize both layers of communication. Since layer 2 communication takes place directly between the PSE and the LTC4278 load, the LTC4278 concerns  itself only with recognizing 2-event classiﬁcation.

In 2-event classiﬁcation, a Type 2 PSE probes for power classiﬁcation  twice. Figure 4 presents an example of a

Figure 3. 25k Signature Resistor with Disable

CLASSIFICATION

Classiﬁcation provides a method for more efﬁcient power allocation by allowing the PSE to identify a PD power clas- siﬁcation. Class 0 is included in the IEEE speciﬁcation for PDs that do not support classiﬁcation. Class 1-3 partitions PDs into three distinct power ranges. Class 4 includes the new power range under IEEE802.3at (see Table 2).

During classiﬁcation probing, the PSE presents a ﬁxed voltage between 15.5V and 20.5V to the PD (Figure 1). The LTC4278 asserts a load current representing the PD power classiﬁcation. The classiﬁcation load current is programmed with a resistor  RCLASS that is chosen from Table 2.

2-event classiﬁcation. The 1st classiﬁcation event occurs when the PSE presents an input voltage between 15.5V to 20.5V and the LTC4278 presents a class 4 load cur- rent. The PSE then drops the input voltage into the mark voltage range of 7V to 10V, signaling the 1st mark event. The PD in the mark voltage range presents  a load current between 0.25mA to 4mA.

The PSE repeats this sequence, signaling the 2nd Clas- siﬁcation and 2nd mark event occurrence. This alerts the LTC4278 that a Type 2 PSE is present. The Type 2 PSE then applies power to the PD and the LTC4278 charges up the reservoir capacitor C1 with a controlled inrush cur- rent. When C1 is fully charged,  and the LTC4278 declares power good, the T2P pin presents  an active low signal, or low impedance output with respect to VPORTN. The T2P output becomes inactive when the LTC4278 input voltage falls below undervoltage lockout threshold.

4278f

APPLICATIONS INFORMATION

VPORTP (V)

50

40

1st CLASS

SIGNATURE CORRUPT DURING MARK

As a member of the IEEE 802.3at working group, Linear

30	2nd CLASS	ON

20

10

OFF

Technology noted that it is possible for a Type 2 PD to

receive a false indication of a 2-event classiﬁcation if a PSE port is pre-charged to a voltage above the detection voltage range before the ﬁrst detection cycle. The IEEE working group modiﬁed the standard to prevent this possibility by

DETECTION V1

DETECTION V2

PD CURRENT

40mA

DETECTION V1

DETECTION V2

1st MARK  2nd MARK

INRUSH

1st CLASS

2nd CLASS

1st MARK  2nd MARK

LOAD, ILOAD

TIME

requiring a Type 2 PD to corrupt the signature resistance during the mark event, alerting the PSE not to apply power. The LTC4278 conforms to this standard by corrupting the signature resistance. This also discharges the port before the PSE begins the next detection cycle.

PD STABILITY DURING CLASSIFICATION

Classiﬁcation presents a challenging stability problem due

50

40

30

20

VPORTP – VNEG (V)

10

VPORTP – T2P (V)

–10

–20

–30

–40

–50

dV =

dt

INRUSH  C1

OFF

ON	OFF

 = RLOAD C1

TIME TIME

TRACKS VPORTN

to the wide range of possible classiﬁcation load current. The onset of the classiﬁcation load current introduces a voltage drop across the cable and increases the forward voltage of the input diode bridge. This may cause the PD to oscillate between detection and classiﬁcation with the onset and removal of the classiﬁcation load current.

The LTC4278 prevents this oscillation by introducing a voltage hysteresis window between the detection and clas- siﬁcation ranges. The hysteresis window accommodates the voltage changes a PD encounters  at the onset of the classiﬁcation load current, thus providing a trouble-free transition between detection and classiﬁcation modes.

The LTC4278 also maintains a positive I-V slope throughout

INRUSH = 100mA	RCLASS = 30.9Ω ILOAD = VPORTN

RLOAD

LTC4278

the classiﬁcation range up to the on-voltage. In the event a PSE overshoots beyond the classiﬁcation voltage range, the available load current aids in returning the PD back into the classiﬁcation voltage range. (The PD input may

IIN

RCLASS VPORTP

RLOAD

otherwise be “trapped” by a reverse-biased diode bridge

PSE

RCLASS	C1

T2P

and the voltage held by the 0.1μF capacitor).

VPORTN

VNEG

4278 F04

INRUSH CURRENT

Once the PSE detects and optionally classiﬁes the PD,

Figure 4. VNEG, T2P and PD Current as a Result of 2-Event Classiﬁcation

the PSE then applies powers on the PD. When the LTC4278 input voltage rises above the on-voltage threshold, LTC4278 connects VNEG  to VPORTN  through the internal power MOSFET.

4278f

APPLICATIONS INFORMATION

To control the power-on surge currents  in the system, the LTC4278 provides a ﬁxed inrush current, allowing C1 to ramp up to the line voltage in a controlled manner.

The LTC4278 keeps the PD inrush current below the PSE current  limit to provide a well controlled  power-up  charac- teristic that is independent of the PSE behavior. This ensures a PD using the LTC4278 interoperability with any PSE.

TURN-ON/ TURN-OFF THRESHOLD

The IEEE 802.3af/at  speciﬁcation  for the PD dictates a maximum turn-on  voltage of 42V and a minimum turn-off voltage of 30V. This speciﬁcation  provides  an adequate voltage to begin PD operation, and to discontinue PD op- eration when the input voltage is too low. In addition, this speciﬁcation  allows PD designs  to incorporate  an ON/OFF hysteresis window to prevent start-up oscillations.

The LTC4278 features  an ON/OFF hysteresis window (see Figure 5) that conforms with the IEEE 802.3af/at speciﬁca- tion and accommodates the voltage drop in the cable and input diode bridge at the onset of the inrush current.

Once C1 is fully charged,  the LTC4278 turns on is internal

does not fall below the OFF threshold. When the LTC4278 input voltage falls below the OFF threshold, the PD load is disconnected, and classiﬁcation mode resumes. C1 discharges through the LTC4278 circuitry.

COMPLEMENTARY POWER GOOD

When LTC4278 fully charges the load capacitor (C1), power good is declared and the LTC4278 load can safely begin operation.  The LTC4278 provides complementary power good signals that remain active during normal operation and are de-asserted when the input voltage falls below the OFF threshold, when the input voltage exceeds the overvoltage lockout (OVLO) threshold, or in the event of a thermal shutdown (see Figure 6).

The PWRGD pin features  an open collector output refer- enced to VNEG which can interface directly with the UVLO pin. When power good is declared and active, the PWRGD pin is high impedance with respect to VNEG. An internal 14V clamp protects the UVLO pin from  an excessive voltage.

LTC4278

MOSFET and passes power to the PD load. The LTC4278

continues to power the PD load as long as the input voltage

OVLO ON/OFF TSD

CONTROL CIRCUIT

30   PWRGD

29   PWRGD

LTC4278

VPORTP

+	C1

5μF

MIN

PD LOAD

VPORTN    5

27   VNEG

TO PSE

ON/OFF AND OVERVOLTAGE LOCKOUT CIRCUIT

| VPORTN VNEG VPORTP – VPORTN VOLTAGE | LTC4278 POWER MOSFET |
| --- | --- |
| 0V TO ON* | OFF |
| >ON* | ON |
| <OFF* | OFF |
| >OVLO | OFF |

4278 F05

CURRENT-LIMITED TURN ON

VPORTN    6

BOLD LINE INDICATES HIGH CURRENT PATH INRUSH COMPLETE

ON < VPORTP < OVLO

AND NOT IN THERMAL SHUTDOWN

26   VNEG

POWER NOT GOOD

POWER GOOD

*INCLUDES ON/OFF HYSTERESIS ON THRESHOLD ≅ 36.1V

OFF THRESHOLD  ≅ 30.7V OVLO THRESHOLD  ≅ 71.0V

VPORTP < OFF VPORTP > OVLO

OR THERMAL SHUTDOWN

4278 F06

Figure 5. LTC4278 ON/OFF and Overvoltage Lockout

Figure 6. LTC4278 Power Good Functional  and State Diagram

4278f

APPLICATIONS INFORMATION

The active low PWRGD pin connects to an internal, open- drain MOSFET referenced to VPORTN and may be used as an indicator bit when power good is declared and active. The PWRGD pin is low impedance with respect to VPORTN.

PWRGD PIN WHEN SHDN IS INVOKED

In PD applications where an auxiliary power supply invokes the SHDN feature,  the PWRGD pin becomes high imped- ance. This prevents the PWRGD pin that is connected to the UVLO pin from interfering with the DC/DC converter operations when powered by an auxiliary power supply.

OVERVOLTAGE LOCKOUT

The LTC4278  includes an overvoltage  lockout  (OVLO) feature (Figure 6) which protects the LTC4278 and its load from an overvoltage event. If the input voltage exceeds the OVLO threshold, the LTC4278 discontinues PD operation. Normal operations resume when the input voltage falls below the OVLO threshold and when C1 is charged up.

THERMAL PROTECTION

The IEEE 802.3af/at speciﬁcation requires a PD to withstand any applied voltage from 0V to 57V indeﬁnitely. However, there are several possible scenarios where a PD may encounter excessive heating.

During classiﬁcation, excessive heating may occur if the PSE exceeds the 75ms probing time limit. At turn-on, when the load capacitor begins to charge, the instantaneous power dissipated by the PD interface can be large before it reaches the line voltage.  And if the PD experiences  a fast input positive voltage step in its operational mode (for example, from 37V to 57V), the instantaneous power dissipated by the PD Interface can be large.

The LTC4278 includes a thermal protection feature which protects the LTC4278  from excessive heating. If  the LTC4278 junction temperature exceeds the over-tempera- ture threshold, the LTC4278 discontinues  PD operations and power good becomes inactive.  Normal  operation resumes when the junction temperature  falls below the overtemperature threshold and when C1 is charged up.

EXTERNAL INTERFACE AND COMPONENT SELECTION Transformer

Nodes on an Ethernet network commonly interface to the outside world via an isolation transformer.  For PDs, the isolation transformer must also include a center tap on the RJ45 connector side (see Figure 7).

The increased current levels in a Type 2 PD over a Type

1 increase the current imbalance in the magnetics which can interfere with data transmission. In addition, proper termination is also required around the transformer to provide correct impedance matching and to avoid radiated and conducted emissions. Transformer vendors such as Bel Fuse, Coilcraft, Halo, Pulse, and Tyco (Table 4) can assist in selecting an appropriate isolation transformer and proper termination methods.

Table 4. Power over Ethernet Transformer Vendors

| VENDOR | CONTACT INFORMATION |
| --- | --- |
| Bel Fuse Inc. | 206 Van Vorst Street Jersey City, NJ 07302 Tel: 201-432-0463 www.belfuse.com |
| Coilcraft Inc. | 1102 Silver Lake Road Gary, IL 60013 Tel: 847-639-6400 www.coilcraft.com |
| Halo Electronics | 1861 Landings Drive Mountain View, CA 94043 Tel: 650-903-3800 www.haloelectronics.com |
| Pulse Engineering | 12220 World Trade Drive San Diego, CA 92128 Tel: 858-674-8100 www.pulseeng.com |
| Tyco Electronics | 308 Constitution Drive Menlo Park, CA 94025-1164 Tel: 800-227-7040 www.circuitprotection.com |

Input Diode Bridge

Figure 2 shows how two diode bridges are typically con- nected in a PD application. One bridge is dedicated to the data pair while the other bridge is dedicated to the spare pair. The LTC4278 supports the use of either silicon or Schottky input diode bridges. However, there are tradeoffs in the choice of diode bridges.

4278f

APPLICATIONS INFORMATION

An input diode bridge must be rated above the maximum current the PD application  will encounter at the tempera- ture the PD will operate.  Diode bridge vendors  typically call out the operating current at room temperature, but derate the maximum current with increasing temperature. Consult the diode bridge vendors for the operating current derating curve.

A silicon diode bridge can consume over 4% of the available power in some PD applications. Using Schottky diodes can help reduce the power loss with a lower forward voltage.

A Schottky   bridge may not be suitable for some high temperature PD application. The leakage current has a voltage dependency that can reduce the perceived signature resistance. In addition, the IEEE 802.3af/at speciﬁcation mandates the leakage back-feeding through the unused bridge cannot generate more than 2.8V across a 100k resistor when a PD is powered with 57V.

Sharing Input Diode Bridges

At higher temperatures,  a PD design may be forced to consider larger bridges in a bigger package because the maximum operating current for the input diode bridge is drastically derated. The larger package may not be accept- able in some space-limited environments.

One solution to consider is to reconnect the diode bridges so that only one of the four diodes conducts current in each package. This conﬁguration extends the maximum operating current while maintaining  a smaller package proﬁle. Figure 7 shows how to reconnect the two diode bridges. Consult the diode bridge vendors for the derating curve when only one of four diodes is in operation.

Input Capacitor

The IEEE  802.3af/at standard includes an impedance requirement in order to implement the AC  disconnect function.  A 0.1μF capacitor  (C14 in Figure 7) is used to meet this AC impedance requirement.

Transient Voltage Suppressor

The LTC4278 speciﬁes  an absolute maximum voltage of

100V and is designed to tolerate brief overvoltage events. However, the pins that interface to the outside world can routinely see excessive peak voltages. To  protect the LTC4278, install a transient voltage suppressor (D3) be- tween the input diode bridge and the LTC4278 as shown in Figure 7.

RJ45

1

2

3

6

TX+

TX–

RX+

RX–

14   T1  1

12 	3

13 	2

10 	5

11 	4

9 	6

TO PHY

BR1

HD01

SPARE+

4

5

7

SPARE–

8

COILCRAFT

ETHI - 230LD

BR2

HD01

C14

0.1μF

100V

D3

SMAJ58A TVS

VPORTP

LTC4278	C1

VPORTN    VNEG

4278 F07

Figure 7. PD Front-End with Isolation  Transformer, Diode Bridges, Capacitors, and a Transient Voltage Suppressor (TVS).

4278f

APPLICATIONS INFORMATION

Classiﬁcation Resistor (RCLASS)

The RCLASS resistor sets the classiﬁcation load current, corresponding to the PD power classiﬁcation. Select the value of RCLASS  from Table 2 and connect the resistor between the RCLASS and VPORTN pins as shown in Figure

4, or ﬂoat the RCLASS  pin if the classiﬁcation load cur- rent is not required. The resistor tolerance must be 1%

TO PSE

–54V

V+

VPORTP

RP

LTC4278

VPORTN 	T2P

TO PD LOAD

or better to avoid degrading the overall accuracy of the classiﬁcation circuit.

Load Capacitor

OPTION 1: SERIES  CONFIGURATION FOR ACTIVE LOW/LOW  IMPEDANCE OUTPUT

V+

VPORTP

The IEEE 802.3af/at  speciﬁcation   requires that the PD

maintains a minimum load capacitance of 5μF and does not specify a maximum load capacitor.  However,  if the

TO PSE

LTC4278

T2P

RP

TO PD LOAD

load capacitor is too large, there may be a problem with

inadvertent power shutdown by the PSE.

–54V

VPORTN 	VNEG

4278 F08

This occurs when the PSE voltage drops quickly. The input diode bridge reverses bias, and the PD load momentarily powers off the load capacitor.  If the PD does not draw power within the PSE’s 300ms disconnection delay, the PSE may remove power from the PD. Thus, it is necessary to evaluate the load current and capacitance to ensure that an inadvertent shutdown cannot occur.

The load capacitor can store signiﬁcant energy when fully charged. The PD design must ensure that this energy is not inadvertently dissipated in the LTC4278. For example, if the VPORTP  pin shorts to VPORTN while  the capacitor is charged, current will ﬂow through the parasitic body diode of the internal  MOSFET and may cause permanent damage to the LTC4278.

T2P Interface

When a 2-event classiﬁcation  sequence successfully completes, the LTC4278 recognizes  this sequence, and provides an indicator bit, declaring the presence of a Type 2 PSE. The open-drain output provides the option to use this signal to communicate to the LTC4278 load, or to leave the pin unconnected.

Figure  8 shows two interface options using the T2P pin and the opto-isolator.  The T2P pin is active low and connects to an opto-isolator to communicate across the

OPTION 2: SHUNT CONFIGURATION FOR ACTIVE HIGH/OPEN COLLECTOR OUTPUT

Figure 8. T2P Interface Examples

DC/DC converter isolation barrier. The pull-up resistor RP is sized according to the requirements of the opto-isolator operating current, the pull-down capability of the T2P pin, and the choice of V+. V+ for example  can come from the PoE supply rail (which the LTC4278 VPORTP is tied to), or from the voltage source that supplies power to the DC/DC converter. Option 1 has the advantage of not drawing power unless T2P is declared active.

Shutdown Interface

To corrupt the signature resistance,  the SHDN pin can be driven high with respect to VPORTN.  If unused, connect SHDN directly to VPORTN.

Auxiliary Power Source

In some applications, it is desirable to power the PD from an auxiliary power source such as a wall adapter.

Auxiliary power  can be injected into an LTC4278-based PD at the input of the LTC4278 VPORTN , at VNEG, or even the power supply output. In addition, some PD applications may desire auxiliary supply dominance or may be conﬁgured

4278f

APPLICATIONS INFORMATION

for PoE dominance. Furthermore, PD applications may also opt for a seamless transition — that is, without power disruption — between PoE and auxiliary power.

The most common auxiliary power option injects power at VNEG. Figure 9 presents an example of this application.In this example, the auxiliary port injects 48V onto the line via diode D1. The components surrounding the SHDN pin are selected so that the LTC4278 does not disconnect power to the output until the auxiliary supply exceeds 36V.

This conﬁguration is an auxiliary-dominant conﬁguration. That is, the auxiliary power source supplies the power even if PoE power  is already present. This conﬁguration also provides a seamless transition from PoE to auxiliary power when auxiliary power is applied, however, the removal of auxiliary power to PoE power is not seamless.

Contact Linear Technology applications support for detail information on implementing a custom auxiliary power supply.

IEEE 802.3at SYSTEM POWER-UP REQUIREMENT

Under the IEEE  802.3at standard, a PD must operate under 12.95W in accordance with IEEE 802.3at standard until it recognizes a Type 2 PSE. Initializing PD operation in 12.95W mode eliminates interoperability issue in case a Type 2 PD connects to a Type 1 PSE. Once the PD rec-

ognizes a Type 2 PSE, the IEEE 802.3at standard requires the PD to wait 80ms in 12.95W operation before 25.5W operation can commence.

MAINTAIN POWER SIGNATURE

In an IEEE 802.3af/at system, the PSE uses the maintain power signature (MPS) to determine  if a PD continues to require power. The MPS requires the PD to periodically draw at least 10mA and also have an AC impedance less than

26.25k in parallel with 0.05μF. If one of these conditions is not met, the PSE may disconnect power to the PD.

SWITCHING REGULATOR OVERVIEW

The LTC4278 includes a current mode converter designed speciﬁcally for use in an isolated ﬂyback topology employing synchronous rectiﬁcation. The LTC4278 operation is similar to traditional current mode switchers. The major difference is that output voltage feedback is derived via sensing the output voltage through the transformer.  This precludes the need of an opto-isolator in isolated designs, thus greatly improving dynamic response and reliability. The LTC4278 has a unique feedback ampliﬁer that samples a transformer winding voltage during the ﬂyback period and uses that voltage to control output voltage. The internal blocks are similar to many current mode controllers. The differences lie in the feedback ampliﬁer and load

RJ45

1

2

3

6

4

TX+ 	T1

TX–

RX+

RX–

SPARE+

TO PHY

+

BR1

–

+

TVS

100k

0.1μF	C1

100V

36V

VPORTP

LTC4278

5

7

SPARE–

8

ISOLATED	+

WALL

BR2

–

10k

10k

SHDN VPORTN

GND

VNEG

D1

TRANSFORMER   –

4278 F09

Figure 9. Auxiliary Power Dominant PD Interface Example

4278f

APPLICATIONS INFORMATION

compensation circuitry.  The logic block also contains circuitry to control the special dynamic requirements of ﬂyback control. For more information on the basics of current mode switcher/controllers and isolated ﬂyback

Combining this with the previous VFLBK expression yields an expression for VOUT in terms of the internal reference, programming resistors and secondary resistances:

converters see Application Note 19.

R1+ R2

OUT

V	=

FB 	SF   	SEC     (	DS(ON) )

R2

• V	• N	   I	• ESR + R

Feedback Ampliﬁer—Pseudo DC Theory

For the following  discussion, refer to the simpliﬁed

Switching Regulator Feedback Ampliﬁer diagram (Figure

10A). When the primary-side MOSFET switch MP turns off, its drain voltage rises above the VPORTP rail. Flyback occurs when the primary  MOSFET is off and the synchronous secondary MOSFET is on. During ﬂyback the voltage on nondriven transformer pins is determined by the secondary voltage. The amplitude of this ﬂyback pulse, as seen on

the third winding, is given as:

VOUT + ISEC • (ESR + RDS(ON) )

The effect of nonzero secondary output impedance is

discussed in further detail (see Load Compensation Theory). The practical aspects of applying this equation for VOUT are found in subsequent sections of the Applications Information.

Feedback Ampliﬁer Dynamic Theory

So far, this has been a pseudo-DC treatment of ﬂyback feedback ampliﬁer operation. But the ﬂyback signal is a pulse, not a DC level. Provision  is made to turn on the ﬂyback ampliﬁer only when the ﬂyback pulse is present,

VFLBK =

NSF

using the enable signal as shown in the timing diagram

(Figure 10b).

RDS(ON) = on-resistance  of the synchronous MOSFET MS

ISEC = transformer  secondary  current

ESR = impedance of secondary circuit capacitor, winding and traces

NSF = transformer effective secondary-to-ﬂyback winding turns ratio (i.e., NS/NFLBK)

The ﬂyback voltage is scaled by an external resistive divider R1/R2 and presented at the FB pin. The feedback ampliﬁer compares the voltage to the internal bandgap reference. The feedback amp is actually a transconductance ampliﬁer whose output is connected to VCMP only during a period in the ﬂyback time. An external  capacitor  on the VCMP pin integrates the net feedback amp current to provide the control voltage to set the current mode trip point. The regulation voltage at the FB pin is nearly equal to the bandgap reference VFB because of the high gain in the overall loop. The relationship between VFLBK and VFB is expressed as:

Minimum Output Switch On Time (tON(MIN))

The LTC4278 affects output voltage regulation  via ﬂyback pulse action.  If the output switch is not turned on, there is no ﬂyback pulse and output voltage information is not available. This causes irregular loop response and start-up/latchup problems. The solution is to require the primary switch to be on for an absolute minimum time per each oscillator cycle. To accomplish this the current limit feedback is blanked each cycle for tON(MIN). If the output load is less than that developed under these conditions, forced continuous operation normally occurs. See subsequent discussions in the Applications Information section for

further details.

Enable Delay Time (ENDLY)

The ﬂyback pulse appears when the primary-side switch shuts off. However, it takes a ﬁnite time until the transformer primary-side  voltage waveform  represents the output

VFLBK

R1+ R2

=	R2	•

VFB

voltage. This is partly due to rise time on the primary- side MOSFET drain  node, but, more importantly, is due

4278f

APPLICATIONS INFORMATION

VFLBK 	T1

R1	LTC4278 FEEDBACK AMP

16  FB	–

1V

VCMP

17

FLYBACK

•

R2	VFB

+

1.237V

COLLAPSE DETECT

VIN

CVCMP

PRIMARY

•

MP

SECONDARY

•

MS

COUT

ISOLATED OUTPUT

R

ENABLE	S    Q

4278 F10a

Figure 10a. LTC4278 Switching  Regulator Feedback  Ampliﬁer

PRIMARY-SIDE MOSFET DRAIN VOLTAGE

PG VOLTAGE

SG VOLTAGE

VIN

VFLBK

0.8 • VFLBK

tON(MIN)

ENABLE DELAY

MIN ENABLE FEEDBACK

AMPLIFIER ENABLED

PG DELAY

4278 F10b

Figure 10b. LTC4278 Switching  Regulator Timing Diagram

4278f

APPLICATIONS INFORMATION

to transformer leakage inductance. The latter causes a voltage spike on the primary side, not directly related to output voltage. Some time is also required for internal settling of the feedback  ampliﬁer circuitry. In order to maintain immunity to these phenomena, a ﬁxed delay is introduced between the switch turn-off command and the enabling of the feedback ampliﬁer. This is termed “enable delay.” In certain cases where the leakage spike is not sufﬁciently settled by the end of the enable delay period, regulation error may result. See the subsequent sections for further details.

the synchronous MOSFET RDS(ON)  and real life nonzero impedances of the transformer secondary and output capacitor. This was represented previously  by the expression, ISEC • (ESR + RDS(ON)). However, it is generally more useful to convert this expression to effective output impedance. Because the secondary current only ﬂows during the off portion of the duty cycle (DC), the effective output impedance equals the lumped secondary impedance divided by off time DC.

Since the off-time duty cycle is equal to 1 – DC, then:

ESR + RDS(ON)

Collapse Detect

Once the feedback ampliﬁer is enabled, some mechanism is then required to disable it. This is accomplished by a

RS(OUT) =

where:

1− DC

collapse detect comparator, which compares the ﬂyback voltage (FB) to a ﬁxed reference, nominally 80% of VFB. When the ﬂyback waveform drops below this level, the feedback ampliﬁer is disabled.

Minimum Enable Time

The feedback ampliﬁer, once enabled, stays on for a ﬁxed minimum time period, termed “minimum enable time.” This prevents lockup, especially when the output voltage is abnormally low, e.g., during start-up. The minimum enable time period ensures that the VCMP node is able to “pump up” and increase the current mode trip point to the level where the collapse detect system  exhibits proper operation. This time is set internally.

Effects of Variable Enable Period

The feedback ampliﬁer is enabled during only a portion of the cycle time. This can vary from the ﬁxed minimum enable time described to a maximum of roughly the off switch time minus the enable delay time. Certain parameters of feedback amp behavior are directly affected by the variable enable period. These include effective transconductance and VCMP node slew rate.

Load Compensation Theory

The LTC4278 uses the ﬂyback pulse to obtain information about the isolated output voltage.  An error source is caused by transformer secondary current ﬂow through

RS(OUT) = effective supply output impedance

DC = duty cycle

RDS(ON) and ESR are as deﬁned previously

This impedance error may be judged acceptable in less critical applications, or if the output  load current  remains relatively constant. In these cases, the external FB resistive divider is adjusted to compensate for nominal expected error. In more demanding applications, output impedance error is minimized by the use of the load compensation function. Figure 11 shows the block diagram of the load compensation function. Switch current is converted to a voltage by the external sense resistor, averaged and lowpass ﬁltered by the internal 50k resistor  RCMPF and the external capacitor on CCMP. This voltage is impressed across the external RCMP resistor by op amp A1 and transistor Q3 producing a current at the collector of Q3 that is subtracted from the FB node. This effectively increases  the voltage required at the top of the R1/R2 feedback divider to achieve equilibrium.

The average primary-side switch current increases to maintain output voltage regulation as output loading increases. The increase in average current increases RCMP resistor current which affects a corresponding increase in sensed output voltage, compensating for the IR drops.

4278f

APPLICATIONS INFORMATION

Assuming  relatively ﬁxed power supply efﬁciency,  Eff, power balance gives:

Nominal output impedance cancellation is obtained by equating this expression with RS(OUT):

POUT = Eff • PIN

VOUT • IOUT = Eff • VIN • IIN

Average primary-side  current is expressed in terms of output current as follows:

K1•    SENSE • R1• N RCMP	SF

R

Solving for RCMP gives:

ESR + RDS(ON)

=

1− DC

IIN = K1•IOUT

RCMP = K1•

RSENSE

• (1− DC)

• R1• NSF

where:

V

K1=      OUT

VIN • Eff

So, the effective change in VOUT target  is:

RSENSE

ESR + RDS(ON)

The practical aspects of applying this equation to determine an appropriate value for the RCMP resistor are discussed subsequently in the Applications Information section.

Transformer Design

VOUT = K1•

thus :

RCMP

• R1• NSF • IOUT

Transformer design/speciﬁcation is the most critical part of a successful application of the LTC4278. The following sections provide basic information about designing the

VOUT  = K1• RSENSE • R1• N

SF

transformer  and potential tradeoffs. If you need help, the

IOUT

where:

RCMP

LTC Applications group is available to assist in the choice

and/or design of the transformer.

Turns Ratios

K1  = dimensionless  variable related to VIN,  VOUT  and

efﬁciency, as previously explained

RSENSE = external sense resistor

VFLBK

The design of the transformer starts with determining duty cycle (DC). DC impacts the current and voltage stress on the power switches, input and output capacitor RMS currents and transformer  utilization (size vs power). The ideal turns ratio is:

R1	•

N	= VOUT

• 1− DC

FB

16	Q1 Q2

VFB

VIN

IDEAL

VIN 	DC

R2    LOAD COMP I

Q3	A1

•

•

MP

RCMPF SENSE+

50k

Avoid extreme  duty cycles, as they generally increase cur-

rent stresses. A reasonable target for duty cycle is 50%

at nominal input voltage.

For instance,  if we wanted a 48V to 5V converter  at 50%

20	DC then:

5	1− 0.5	1

22  RCMP

21  CCMP 	RSENSE

NIDEAL= 48 •

0.5

= 9.6

Figure 11. Load Compensation Diagram

4278 F11

In general, better performance is obtained with a lower turns ratio. A DC of 45.5%  yields a 1:8 ratio.

4278f

APPLICATIONS INFORMATION

Note the use of the external feedback resistive divider ratio to set output voltage provides the user additional freedom in selecting a suitable transformer turns ratio. Turns ratios that are the simple ratios of small integers; e.g., 1:1, 2:1, 3:2 help facilitate transformer construction and improve performance.

When building a supply with multiple outputs derived through a multiple winding transformer, lower duty cycle can improve cross regulation by keeping the synchronous rectiﬁer on longer, and thus, keep secondary windings coupled longer. For a multiple output transformer, the turns ratio between output windings is critical and affects the accuracy of the voltages. The ratio between two output voltages is set with the formula VOUT2 = VOUT1 • N21 where N21 is the turns ratio between the two windings. Also keep the secondary MOSFET RDS(ON)  small to improve cross regulation.

The feedback winding usually provides both the feedback voltage and power for the LTC4278. Set the turns ratio between the output and feedback winding to provide a rectiﬁed voltage that under worst-case conditions is greater than the the preregulator maximum supply voltage. For example  if the preregulator maximum output were 7V:

N		>  VOUT 	 SF		7 + VF

where :

VF = Diode Forward Vo tage

the voltage extends  the ﬂyback pulse width. If the ﬂyback pulse extends beyond the enable delay time, output voltage regulation is affected. The feedback system has a deliberately limited input range, roughly ±50mV referred to the FB node. This rejects higher voltage leakage spikes because once a leakage spike is several volts in amplitude, a further increase in amplitude has little effect on the feedback system. Therefore, it is advisable to arrange the clamp circuit to clamp at as high a voltage as possible, observing MOSFET breakdown, such that leakage spike duration is as short as possible. Application Note 19 provides a good reference on clamp design.

As a rough guide, leakage inductance of several percent (of mutual inductance) or less may require a clamp, but exhibit little to no regulation error due to leakage spike behavior. Inductances from several percent up to, perhaps, ten percent, cause increasing regulation error.

Avoid double digit percentage leakage inductances.  There is a potential for abrupt loss of control at high load current. This curious condition potentially occurs when the leakage spike becomes such a large portion of the ﬂyback waveform that the processing circuitry is fooled into thinking that the leakage spike itself is the real ﬂyback signal!

It then reverts to a potentially stable state whereby the top of the leakage spike is the control point, and the trailing edge of the leakage spike triggers the collapse detect circuitry. This typically reduces the output voltage abruptly to a fraction, roughly one-third to two-thirds of

For our example: NSF >

We will choose 1

3

Leakage Inductance

5

7 + 0.7

1

=

1.56

its correct value.

Once load current is reduced sufﬁciently, the system snaps back to normal operation. When using transformers with considerable leakage inductance, exercise this worst-case check for potential bistability:

1. Operate the prototype supply at maximum expected

Transformer leakage inductance (on either the primary or secondary) causes a spike after the primary-side switch turn-off. This is increasingly prominent at higher load currents, where more stored energy is dissipated. Higher ﬂyback voltage may break down the MOSFET switch  if it has too low a BVDSS rating.

One solution to reducing this spike is to use a clamp circuit to suppress the voltage excursion. However, suppressing

load current.

2. Temporarily short-circuit the output.

3. Observe that normal operation is restored.

If the output  voltage is found to hang up at an abnormally low value, the system has a problem. This is usually evident by simultaneously viewing the primary-side MOSFET drain voltage to observe ﬁrsthand the leakage spike behavior.

4278f

APPLICATIONS INFORMATION

A ﬁnal note—the susceptibility of the system to bistable behavior is somewhat a function of the load current/ voltage characteristics. A load with resistive—i.e., I = V/R

Ripple current and percentage ripple is largest at minimum duty cycle; in other words, at the highest input voltage. LP is calculated from the following equation.

2

2

behavior—is the most apt to be bistable. Capacitive loads

that exhibit I = V2/R behavior  are less susceptible.

Secondary Leakage Inductance

Leakage inductance on the secondary forms an inductive

LP =

where:

( VIN(MAX) • DCMIN )

fOSC • XMAX • PIN

=

( VIN(MAX) • DCMIN )  • Eff fOSC • XMAX • POUT

divider on the transformer secondary, reducing the size of the ﬂyback pulse. This increases the output voltage target by a similar percentage.  Note that unlike leakage spike behavior, this phenomenon is independent of load. Since the secondary leakage inductance is a constant percentage of mutual inductance (within manufacturing variations), the solution is to adjust the feedback resistive divider ratio to compensate.

fOSC is the oscillator frequency

DCMIN is the DC at maximum input voltage

XMAX is ripple current ratio at maximum input voltage

Using common high power PoE values, a 48V (41V < VIN

< 57V) to 5V/5.3A converter with 90% efﬁciency, POUT=

26.5W and PIN = 29.5W.  Using X = 0.4 N = 1/8 and fOSC

= 200kHz:

Winding Resistance Effects

Primary or secondary winding resistance acts to reduce overall efﬁciency (POUT/PIN). Secondary winding resistance increases effective output impedance, degrading load regu-

DCMIN =

1

1+ N• VIN(MAX )

2

VOUT

1

=

1+ 1 • 57

8	5

= 41.2%

lation. Load compensation can mitigate this to some extent but a good design keeps parasitic resistances low.

Biﬁlar Winding

A biﬁlar,  or similar winding, is a good way to minimize troublesome  leakage inductances.  Biﬁlar windings also improve coupling coefﬁcients, and thus improve cross regulation in multiple winding transformers. However, tight coupling usually increases primary-to-secondary capacitance and limits the  primary-to-secondary breakdown voltage, so is not always practical.

Primary Inductance

The transformer primary inductance, LP, is selected based on the peak-to-peak ripple current ratio (X) in the transformer relative to its maximum value. As a general rule, keep X in the range of 20% to 40% (i.e.,  X = 0.2 to

0.4). Higher values of ripple will increase conduction losses, while lower values will require  larger cores.

L  =  	(57V • 0.412) 	= 260H

P	200kHz • 0.4 • 26.5W

Optimization might show that a more efﬁcient solution is obtained at higher peak current but lower inductance and the associated winding series resistance. A simple spreadsheet program is useful for looking at tradeoffs.

Transformer Core Selection

Once LP is known, the type of transformer is selected. High efﬁciency converters use ferrite cores to minimize core loss. Actual core loss is independent of core size for a ﬁxed inductance, but decreases as inductance increases. Since increased inductance is accomplished through more turns of wire, copper losses increase. Thus, transformer design balances core and copper losses. Remember that increased winding  resistance will  degrade cross regulation  and increase the amount of load compensation required.

The main design goals for core selection are reducing copper losses and preventing saturation. Ferrite core material saturates hard, rapidly reducing inductance

4278f

APPLICATIONS INFORMATION

when the peak design current is exceeded. This results in an abrupt increase in inductor ripple current and, consequently, output voltage ripple. Do not allow the core to saturate! The maximum peak primary current occurs

Continuing the example,  if ESR + RDS(ON) = 8m, R2 =

3.32k, then:

R1= 3.32k  5 + 5.3 • 0.008     = 37.28k

1

at minimum VIN:

PIN

XMIN

choose 37.4k.

1.237 • 1/ 3

IPK =

IN(MIN)

V

now :

• DCMAX

1

•  1+

2

1

It is recommended that the Thevenin impedance of the resistive divider (R1||R2) is roughly 3k for bias current cancellation and other reasons.

DCMAX =

N• VIN(MIN)

+

1

VOUT

=

1+ 1 •

8

2

41 = 49.4%

5

Current Sense Resistor Considerations

The external current sense resistor is used to control peak

( VIN(MIN) • DCMAX )

(41• 49.4%)

primary switch current, which controls a number of key

converter characteristics including maximum power and

XMIN =

fOSC

• LP

• PIN

=

200kHz • 260μH • 29.5W

2

external component ratings. Use a noninductive current

= 0.267

Using the example numbers leads to:

sense resistor (no wire-wound resistors). Mounting the resistor directly above an unbroken ground plane connected with wide and short traces keeps stray resistance and

29.5W

0.267

= 1.65A

inductance low.

IPK = 41• 0.494 •  1+	2

Multiple Outputs

One advantage that the ﬂyback topology offers is that additional output voltages can be obtained simply by adding windings. Designing a transformer for such a situation is beyond the scope of this document. For multiple windings, realize that the ﬂyback winding signal is a combination of activity on all the secondary windings. Thus load regulation is affected by each winding’s load. Take care to minimize cross regulation effects.

Setting Feedback Resistive Divider

The expression for VOUT developed in the Operation section is rearranged to yield the following expression for the feedback resistors:

The dual sense pins allow for a full Kelvin connection. Make

sure that SENSE+ and SENSE– are isolated and connect close to the sense resistor.

Peak current occurs at 100mV of sense voltage VSENSE. So the nominal sense resistor is VSENSE/IPK. For example,  a peak switch current of 10A requires a nominal sense resistor of 0.010 Note that the instantaneous peak power in the sense resistor is 1W, and that it is rated accordingly. The use of parallel resistors can help achieve low resistance, low parasitic inductance and increased power capability.

Size RSENSE using worst-case conditions, minimum LP, VSENSE and maximum VIN. Continuing the example, let us assume that our worst-case conditions yield an IPK of 40% above nominal, so IPK = 2.3A. If there is a 10% tolerance on RSENSE and minimum  VSENSE = 88mV, then RSENSE •

110% = 88mV/2.3A and nominal RSENSE = 35m. Round to the nearest available lower value, 33m.

VOUT +ISEC • (ESR + RDS(ON) )

R1= R2

VFB • NSF

1

4278f

APPLICATIONS INFORMATION

Selecting  the Load Compensation Resistor

The expression for RCMP  was derived in the Operation

4. Compute:

RSENSE

section as:

R

• (1− DC)

RCMP = K1• R

S(OUT)

• R1• NSF

RCMP = K1•

SENSE

ESR + RDS(ON)

• R1• NSF

5. Verify this result by connecting a resistor of this value from the RCMP pin to ground.

Continuing the example:

6. Disconnect the ground short to CCMP and connect a 0.1μF

ﬁlter capacitor to ground. Measure the output imped-

K1=

VOUT 	5

ance R

= V

/I

with the new compensation

=	= 0.116

S(OUT)

OUT

OUT

VIN • Eff

DC= 	1

48 • 90%

1

=

= 45.5%

in place. RS(OUT) should have decreased  signiﬁcantly.

Fine tuning is accomplished experimentally by slightly

N•VIN(NOM)

1+ 1 • 48

1+

altering  RCMP. A revised estimate for RCMP is:

VOUT 	8	5

RS(OUT)CMP

If ESR + RDS(ON) = 8m

33m   • (1   0.455)	1

R CMP = RCMP •  1+

RS(OUT)

RCMP = 0.116 •

= 3.25k

• 37.4k  •

8m   	3

where RCʹ MP is the new value for the load compensation resistor. RS(OUT)CMP is the output impedance with RCMP in place and RS(OUT) is the output impedance with no

This value for RCMP is a good starting point, but empirical methods are required for producing the best results. This is because several of the required input variables are difﬁcult to estimate precisely. For instance, the ESR term above includes that of the transformer secondary, but its effective ESR value depends on high frequency behavior, not simply DC winding resistance. Similarly, K1 appears as a simple ratio of VIN to VOUT times efﬁciency, but theoretically estimating  efﬁciency is not a simple calculation.

The suggested empirical method is as follows:

1. Build a prototype of the desired supply including the actual secondary components.

2. Temporarily ground the CCMP pin to disable the load compensation function. Measure output voltage while sweeping output current over the expected range. Approximate the voltage variation as a straight line.

VOUT/IOUT = RS(OUT) .

load compensation (from step 2).

Setting Frequency

The switching frequency of the LTC4278  is set by an external capacitor connected between the OSC pin and ground. Recommended values are between 200pF and

33pF, yielding switching frequencies between 50kHz and

250kHz. Figure 12 shows the nominal relationship between external capacitance and switching frequency. Place the capacitor as close as possible to the IC and minimize OSC

300

fOSC (kHz)

200

100

3. Calculate  a value for the K1 constant based on VIN, VOUT

and the measured efﬁciency.

50

30	100

COSC (pF)

200

4278 F12

Figure 12. fOSC vs OSC Capacitor  Values

4278f

APPLICATIONS INFORMATION

trace length and area to minimize stray capacitance and

The tON(MIN) resistor  is set with the following equation

potential noise pick-up.

tON(MIN)

(ns) − 104

You can synchronize the oscillator frequency to an external frequency. This is done with a signal on the SYNC pin.

RtON(MIN) (k) =

1.063

Set the LTC4278 frequency 10% slower than the desired external frequency using the OSC  pin capacitor, then use a pulse on the SYNC pin of amplitude greater than

2V and with the desired frequency. The rising edge of the SYNC  signal initiates an OSC  capacitor discharge forcing  primary  MOSFET off (PG voltage goes low).  If the oscillator frequency is much different from the sync frequency, problems may occur with slope compensation and system stability.  Also, keep the sync pulse width greater than 500ns.

Selecting Timing Resistors

There are three internal “one-shot” times that are programmed by external application resistors: minimum on-time, enable delay time and primary MOSFET turn-on delay. These are all part of the isolated ﬂyback control technique, and their functions are previously outlined in the Theory of Operation section. The following information should help in selecting and/or optimizing these timing values.

Minimum Output Switch On-Time (tON(MIN))

Minimum on-time is the programmable period during which current limit is blanked (ignored) after the turn-on of the primary-side switch. This improves regulator performance by eliminating false tripping on the leading edge spike in the switch, especially at light loads. This spike is due to both the gate/source charging current and the discharge

Keep RtON(MIN) greater than 70k. A good starting value is 160k.

Enable Delay Time (ENDLY)

Enable delay time provides a programmable delay between turn-off of the primary gate drive node and the subsequent enabling of the feedback ampliﬁer. As discussed earlier, this delay allows the feedback ampliﬁer to ignore the leakage inductance  voltage spike on the primary side. The worst-case leakage spike pulse width is at maximum load conditions. So, set the enable delay time at these conditions.

While the typical applications  for this part use forced continuous operation, it is conceivable that a secondary- side controller might cause discontinuous operation at light loads. Under such conditions, the amount of energy stored in the transformer is small. The ﬂyback waveform becomes “lazy” and some time elapses before it indicates the actual secondary output voltage. The enable delay time should be made long enough to ignore the “irrelevant” portion of the ﬂyback waveform at light loads.

Even though the LTC4278 has a robust  gate drive, the gate transition time slows with very large MOSFETs. Increase delay time as required when using such MOSFETs.

The enable delay resistor is set with the following equation:

of drain capacitance. The isolated ﬂyback sensing requires

RENDLY (k) =

tENDLY

(ns) − 30

a pulse to sense the output. Minimum on-time ensures

that the output switch is always on a minimum time and that there is always a signal to close the loop.

The LTC4278 does not employ cycle skipping at light loads. Therefore, minimum  on-time along with synchronous rectiﬁcation sets the switch over to forced continuous mode operation.

2.616

Keep RENDLY greater than 40k. A good starting point is

56k.

4278f

APPLICATIONS INFORMATION

Primary Gate Delay Time (PGDLY)

Primary gate delay is the programmable time from the turn-off of the synchronous MOSFET to the turn-on of the primary-side MOSFET. Correct setting eliminates overlap between the primary-side switch and secondary-side syn- chronous switch(es) and the subsequent current spike in the transformer. This spike will cause additional component stress and a loss in regulator efﬁciency.

The primary gate delay resistor is set with the following equation:

tPGDLY (ns) + 47

Switcher’s UVLO Pin Function

The UVLO pin provides a user programming undervoltage lockout. This is typically used to provide undervoltage lockout based on VIN. The gate drivers are disabled when UVLO is below the 1.24V  UVLO threshold.  An external resistive divider between the input supply and ground is used to set the turn-on voltage.

The bias current on this pin depends on the pin volt- age and UVLO state. The change provides the user with adjustable UVLO hysteresis. When the pin rises above the UVLO threshold a small current is sourced out of the pin, increasing the voltage on the pin. As the pin voltage

RPGDLY (k) =

9.01

drops below this threshold, the current is stopped, further dropping the voltage on UVLO. In this manner, hysteresis

A good  starting  point is 15k.

Soft-Start Function

The LTC4278 contains  an optional soft-start function that is enabled by connecting an external capacitor between the SFST pin and ground. Internal circuitry prevents the control voltage at the VCMP pin from exceeding  that on the SFST

is produced.

Referring to Figure 13, the voltage hysteresis at VIN  is equal to the change in bias current  times RA. The design procedure is to select the desired VIN  referred voltage hysteresis, VUVHYS. Then:

VUVHYS

pin. There is an initial pull-up circuit to quickly bring the SFST voltage to approximately 0.8V. From there it charges to approximately 2.8V with a 20μA current source.

R A =

where:

IUVLO

The SFST node is discharged to 0.8V when a fault occurs. A fault occurs when the current sense voltage is greater than 200mV or the IC’s thermal (overtemperature) shut- down is tripped. When SFST discharges, the VCMP node voltage is also pulled low to below the minimum current voltage. Once discharged and the fault removed, the

IUVLO = IUVLOL – IUVLOH is approximately 3.4μA

RB is then selected with the desired turn-on voltage:

R =	RA

B	VIN(ON)

– 1

SFST charges up again. In this manner, switch currents are reduced and the stresses in the converter are reduced during fault conditions.

The time it takes to fully charge soft-start is:

CSFST • 1.4V

VIN

RA

VUVLO

IUVLO

VIN

RA

IUVLO

VIN

RA1

RA2

tss =

= 70k • CSFST (F)



20 A

UVLO

RB	LTC4278

UVLO

RB	LTC4278

CUVLO

UVLO RB

(13a) UV Turning On

4278 F13

(13b) UV Turning  Off	(13c) UV Filtering

Figure 13. UVLO Pin Function and Recommended Filtering

4278f

APPLICATIONS INFORMATION

If we wanted a VIN-referred trip point of 36V, with 1.8V (5%) of hysteresis (on at 36V, off at 34.2V):

RA  =  1.8V   = 529k, use 523k

3.4A

RB =  	523k 	= 18.5k, use 18.7k

⎛ 36V  – 1⎟

to turn off QPR.  If the two voltage ranges overlap, the only disadvantage is that a small degradation in efﬁciency may occur. It is also necessary to verify that the worst-case maximum winding voltage is not high enough to damage the B-E junction of QPR.

VIN

⎝1.23V	⎠	•

•

Even with good board layout, board noise may cause problems with UVLO. You can ﬁlter the divider but keep large capacitance off the UVLO node because it will slow the hysteresis produced from the change in bias current. Figure 13c shows an alternate method of ﬁltering by split- ting the RA resistor with the capacitor. The split should put

QPR

CVCC

VCC 	PG

more of the resistance on the UVLO side.

Converter Start-Up

The standard topology for the LTC4278 uses a third trans- former winding on the primary side that provides both the

LTC4278

GND

FB

4278 F14

feedback information and local VCC power for the LTC4278 (Figure 14). This power bootstrapping improves converter efﬁciency but is not inherently self-starting. Start-Up is affected with an external preregulator circuit that conditions the input line voltage for the LTC4278 during start-up.

Upon application of power,  CVCC is charged via the pre- regulator, thereby providing an appropriate supply voltage at the VCC  pin for the LTC4278.  This supply voltage is typically in the range  7V and is used  during start-up. After converter startup, the third transformer winding becomes energized and is designed to generate a higher voltage than the preregulator.  The higher voltage of the third winding turns off QPR and provides an efﬁcient method to power the LTC4278.

Design of the VCC power  circuitry  involves  selecting  ap- propriate voltage ranges for both the preregulator and the third transformer winding. The preregulator voltage

Figure 14. Typical Power Bootstrapping

Control Loop Compensation

Loop frequency compensation is performed by connect- ing a capacitor network from the output of the feedback ampliﬁer  (VCMP pin) to ground as shown in Figure 15. Because of the sampling behavior of the feedback ampliﬁer, compensation is different from traditional current mode controllers. Normally only CVCMP is required.  RVCMP can be used to add a zero, but the phase margin improvement traditionally offered by this extra resistor is usually already accomplished by the nonzero secondary circuit impedance. CVCMP2 can be used to add an additional high frequency

pole and is usually sized at 0.1 times CVCMP.

VCMP

17

is set as low as possible while ensuring it’s worst-case minimum voltage is high enough to drive the switching FETs gates during the startup period. The third winding output voltage is selected to ensure that it’s worst-case

CVCMP2

4278 F15

RVCMP

CVCMP

minimum voltage exceeds the preregulator voltage in order

Figure 15. VCMP Compensation Network

4278f

APPLICATIONS INFORMATION

In further contrast to traditional current mode switchers, VCMP pin ripple is generally not an issue with the LTC4269-1. The dynamic nature of the clamped feedback ampliﬁer forms an effective track/hold type response,  whereby the VCMP voltage changes during the ﬂyback pulse, but is then held during the subsequent switch-on portion of the next cycle. This action naturally holds the VCMP voltage stable during the current comparator sense action (current mode switching).

Application Note 19 provides a method for empirically tweaking frequency compensation.  Basically, it involves introducing  a load current step and monitoring  the response.

Slope Compensation

The LTC4278 incorporates  current slope compensation. Slope compensation is required to ensure current loop stability when the DC is greater than 50%. In some switching regulators, slope compensation reduces the maximum peak current at higher duty cycles. The LTC4278 eliminates this problem by having circuitry that compensates for the slope compensation so that maximum current sense voltage is constant across all duty cycles.

Minimum Load Considerations

In normal use, the peak switch current increases while FB is below the internal reference. This continues until VCMP reaches its 2.56V clamp. At clamp, the primary-side MOSFET will turn off at the rated 100mV VSENSE level. This repeats on the next cycle.

It is possible for the peak primary switch currents as referred across RSENSE to exceed the max 100mV rating because of the minimum switch on time blanking. If the voltage on VSENSE  exceeds 205mV after the minimum turn-on time, the SFST capacitor is discharged, causing the discharge of the VCMP capacitor.  This then reduces the peak current on the next cycle and will reduce overall stress in the primary switch.

Short-Circuit Conditions

Loss of current limit is possible under certain conditions such as an output short-circuit. If the duty cycle exhibited by the minimum on-time is greater than the ratio of secondary winding voltage (referred-to-primary) divided by input voltage, then peak current is not controlled at the nominal value. It ratchets up cycle-by-cycle to some higher level. Expressed mathematically, the requirement to maintain short-circuit control is:

ISC • (RSEC + RDS(ON) )

At light loads, the LTC4278 derived regulator  goes into

DCMIN = tON(MIN)

• fOSC <

VIN • NSP

forced continuous conduction mode. The primary-side

switch always turns on for a short time as set by the tON(MIN)  resistor. If this produces more power than the load requires, power will ﬂow back into the primary dur- ing the off period when the synchronization switch is on. This does not produce any inherently adverse problems, although light load efﬁciency is reduced.

Maximum Load Considerations

The current mode control uses the VCMP  node voltage and ampliﬁed sense resistor voltage as inputs to the current comparator.  When the ampliﬁed sense voltage exceeds the VCMP node voltage, the primary-side switch is turned off.

where:

tON(MIN) is the primary-side switch minimum on-time

ISC is the short-circuit output current

NSP is the secondary-to-primary turns ratio (NSEC/NPRI) (other variables as previously deﬁned)

Trouble is typically encountered  only in applications with a relatively high product of input voltage times secondary to primary turns ratio and/or a relatively long minimum switch on time. Additionally, several real world effects such as transformer leakage inductance, AC winding losses and output switch voltage drop combine to make this simple theoretical calculation a conservative estimate. Prudent design evaluates the switcher for short-circuit protection and adds any additional circuitry to prevent destruction.

4278f

APPLICATIONS INFORMATION

Output Voltage Error Sources

The LTC4278’s  feedback sensing introduces additional

For each secondary-side power MOSFET, the peak cur- rent is:

I	X

minor sources of errors. The following is a summary list:

•  The internal bandgap voltage reference sets the reference voltage for the feedback ampliﬁer. The speciﬁcations detail its variation.

•  The external feedback resistive divider ratio directly

IPK(SEC) =  	OUT 	•  1+    MIN

1  DCMAX 	2

Select a primary-side  power MOSFET  with a BVDSS

greater than:

affects regulated voltage. Use 1% components.

BV 	≥ I	LKG + V

L

VOUT(MAX)

+

•  Leakage  inductance on the transformer secondary reduces the effective secondary-to-feedback  winding

DSS	PK

CP 	IN(MAX)

NSP

turns ratio (NS/NF) from its ideal value. This increases the output voltage target by a similar percentage. Since secondary leakage inductance is constant from part to part (within a tolerance) adjust the feedback resistor ratio to compensate.

•  The transformer secondary current ﬂows through the impedances of the winding resistance, synchronous MOSFET RDS(ON)  and output capacitor ESR. The DC equivalent current for these errors is higher than the load current because conduction occurs only during the converter’s off-time. So, divide the load current by (1 – DC).

If the output load current is relatively constant, the feedback resistive divider is used to compensate for these losses. Otherwise, use the LTC4278 load compensation circuitry (see Load Compensation). If multiple output windings are used, the ﬂyback winding will have a signal that represents

where NSP reﬂects the turns ratio of that secondary-to

primary winding. LLKG is the primary-side leakage induc- tance and CP is the primary-side capacitance (mostly from the drain capacitance (COSS) of the primary-side power MOSFET). A clamp  may be added to reduce the leakage inductance as discussed.

For each secondary-side power MOSFET, the BVDSS should be greater than:

BVDSS ≥ VOUT + VIN(MAX) • NSP

Choose the primary-side MOSFET RDS(ON)  at the nominal gate drive voltage (7.5V). The secondary-side MOSFET gate drive voltage depends on the gate drive method.

Primary-side power MOSFET RMS current is given by:

I	=		PIN RMS(PRI)		 V		DC

an amalgamation of all these windings impedances.  Take

IN(MIN)

MAX

care that you examine worst-case loading conditions when tweaking the voltages.

Power  MOSFET Selection

For each secondary-side power MOSFET RMS current is given by:

I

The power MOSFETs are selected primarily on the criteria of

IRMS(SEC) =

OUT

1− DC

on-resistance RDS(ON), input capacitance, drain-to-source breakdown voltage (BVDSS), maximum  gate voltage (VGS) and maximum drain current (ID(MAX)).

For the primary-side power MOSFET, the peak current is:

MAX

Calculate  MOSFET power dissipation next. Because the primary-side  power MOSFET  operates at high VDS,  a transition power loss term is included for accuracy. CMILLER is the most critical parameter in determining the transition

PIN

XMIN

loss, but is not directly speciﬁed on the data sheets.

IPK(PRI) =

IN(MIN)

V

• DCMAX

•  1+

2

where XMIN  is peak-to-peak current ratio as deﬁned earlier.

4278f

APPLICATIONS INFORMATION

CMILLER is calculated from the gate charge curve included on most MOSFET data sheets (Figure 16).

MILLER EFFECT VGS

a	b

The secondary-side power MOSFETs  typically operate at substantially  lower VDS, so you can neglect transition losses. The dissipation is calculated using:

PDIS(SEC) = IRMS(SEC)2 • RDS(ON)(1 + )

With power dissipation known, the MOSFETs’ junction temperatures are obtained from the equation:

QA 	QB

4278 F16

GATE CHARGE (QG)

Figure 16. Gate Charge Curve

The ﬂat portion  of the curve is the result of the Miller (gate to-drain) capacitance as the drain voltage drops. The Miller capacitance is computed as:

TJ = TA + PDIS  • JA

where TA is the ambient temperature and JA is the MOSFET

junction to ambient thermal resistance.

Once you have TJ iterate your calculations recomputing

 and power dissipations until convergence.

Gate Drive Node Consideration

Q  − Q

CMILLER =   B 	A

VDS

The curve is done for a given VDS. The Miller capacitance for different VDS voltages are estimated by multiplying the computed CMILLER by the ratio of the application  VDS to the curve speciﬁed VDS.

With CMILLER determined, calculate the primary-side power

MOSFET power dissipation:

2

The PG and SG gate drivers are strong drives to minimize gate drive rise and fall times. This improves efﬁciency, but the high frequency components of these signals can cause problems. Keep the traces short and wide to reduce parasitic inductance.

The parasitic inductance creates an LC  tank with the MOSFET gate capacitance. In less than ideal layouts, a series resistance of 5Ω or more may help to dampen the ringing at the expense of slightly slower rise and fall times

PD(PRI) = IRMS(PRI)

V	• PIN(MAX) IN(MAX)		 DCMIN

where:

• RDS(ON) (1+  ) +

• R	•  	CMILLER 	• f

DR  VGATE(MAX) − VTH 	OSC

and poorer efﬁciency.

The LTC4278 gate drives will clamp the max gate voltage to roughly  7.5V, so you can safely use MOSFETs  with maximum  VGS of 10V and larger.

Synchronous Gate Drive

RDR is the gate driver resistance (≈10Ω)

VTH is the MOSFET gate threshold voltage fOSC is the operating frequency

VGATE(MAX) = 7.5V for this part

(1 + ) is generally given for a MOSFET in the form of a normalized  RDS(ON) vs temperature curve. If you don’t have a curve, use  = 0.005/°C • T for low  voltage MOSFETs.

There are several different ways to drive the synchronous gate MOSFET. Full converter isolation requires the synchro- nous gate drive to be isolated. This is usually accomplished by way of a pulse transformer.  Usually the pulse driver is used to drive a buffer on the secondary, as shown in the application on the front page of this data sheet.

However, other schemes are possible. There are gate drivers and secondary-side  synchronous controllers available that provide the buffer function as well as additional features.

4278f

APPLICATIONS INFORMATION

Capacitor Selection

In a ﬂyback converter, the input and output current ﬂows in pulses, placing severe demands on the input and output ﬁlter capacitors. The input and output ﬁlter capacitors are selected based on RMS current ratings and ripple voltage.

Select an input capacitor with a ripple current rating greater than:

PRIMARY CURRENT

SECONDARY CURRENT

OUTPUT VOLTAGE RIPPLE WAVEFORM

IPRI

ΔVCOUT

ΔVESR

IPRI

N

RINGING DUE TO ESL

4278 F17

IRMS(PRI) =

PIN

VIN(MIN)

1− DCMAX

DCMAX

Figure 17. Typical Flyback Converter Waveforms

Continuing the example:

simplicity, we will choose 2% for the maximum output

IRMS(PRI) =

29.5W

41V

1− 49.4%

49.4%

= 0.728A

ripple, divided equally between the ESR step and the charging/discharging V. This percentage ripple changes, depending on the requirements of the application. You can

Keep input capacitor series resistance (ESR) and inductance (ESL) small, as they affect electromagnetic interference suppression. In some instances, high ESR can also produce stability problems because ﬂyback converters

modify the following equations.

For a 1% contribution to the total ripple voltage, the ESR

of the output capacitor is determined by:

exhibit a negative input resistance characteristic. Refer to Application Note 19 for more information.

ESR

COUT

≤ 1%•

VOUT • (1− DCMAX )

IOUT

The output capacitor is sized to handle the ripple current and to ensure acceptable output voltage ripple. The output

The other 1% is due to the bulk C component,  so use:

capacitor should have an RMS current rating greater than:

COUT

≥	IOUT

1% • VOUT • fOSC

I	= I	MAX

DC

In many applications, the output capacitor is created from

RMS(SEC)

OUT

1− DCMAX

multiple  capacitors to achieve desired voltage ripple,

Continuing the example:

49.4%

reliability and cost goals. For example, a low ESR ceramic capacitor can minimize the ESR step, while an electrolytic

IRMS(SEC) = 5.3A

= 5.24A

1− 49.4%

capacitor satisﬁes the required bulk C.

Continuing our example, the output capacitor needs:

This is calculated for each output in a multiple winding

application.

ESR and ESL along with bulk capacitance directly affect the

ESR

COUT

5V • 1− 49.4%

(	)

≤ 1% •	= 4m

5.3A

5.3A

output voltage ripple. The waveforms for a typical ﬂyback

converter are illustrated in Figure 17.

COUT ≥

1% • 5 • 200kHz

= 600F

The maximum acceptable ripple voltage (expressed  as a percentage of the output voltage) is used to establish a starting point for the capacitor values. For the purpose of

These electrical characteristics  require paralleling several low ESR capacitors possibly of mixed type.

4278f

APPLICATIONS INFORMATION

One way to reduce cost and improve output ripple is to use a simple  LC ﬁlter. Figure 18 shows an example of the ﬁlter.

ISOLATION

The 802.3 standard requires Ethernet ports to be electrically isolated from all other conductors that are user accessible.

L1, 0.1μH

VOUT

This includes the metal chassis, other connectors and

FROM	+

SECONDARY

C1	+

47μF

COUT

COUT2

RLOAD

any auxiliary power connection.  For PDs, there are two common methods to meet the isolation requirement.  If

WINDING

470μF

×3

Figure 18.

1μF

4278 F18

there will be any user accessible connection to the PD, then an isolated  DC/DC converter  is necessary to meet the isolation requirements.  If user connections can be

The design of the ﬁlter is beyond the scope of this data sheet. However, as a starting point, use these general guidelines. Start with a COUT 1/4 the size of the nonﬁlter solution. Make C1 1/4 of COUT to make the second ﬁlter pole independent of COUT. C1 may be best implemented with multiple ceramic capacitors.  Make L1 smaller than the output inductance of the transformer. In general, a

0.1μH ﬁlter inductor is sufﬁcient.  Add a small ceramic capacitor  (COUT2) for high frequency  noise on VOUT. For those interested in more details refer to “Second-Stage LC Filter Design,” Ridley, Switching Power Magazine, July

2000 p8-10.

Circuit simulation is a way to optimize output capacitance and ﬁlters, just make sure to include the component parasitic. LTC SwitcherCAD®     is a terriﬁc free circuit simulation tool that is available at www.linear.com.  Final optimization of output ripple must be done on a dedicated PC board. Parasitic inductance due to poor layout can signiﬁcantly impact ripple. Refer to the PC Board Layout section for more details.

ELECTRO STATIC DISCHARGE AND SURGE PROTECTION

The LTC4278  is speciﬁed to operate with an absolute maximum voltage of –100V and is designed to tolerate brief overvoltage events. However, the pins that interface to the outside world (primarily VPORTN and VPORTP) can routinely see peak voltages in excess of 10kV. To protect the LTC4278, it is highly recommended that the SMAJ58A unidirectional

58V transient voltage suppressor  be installed between the diode bridge and the LTC4278 (D3 in Figure 2).

avoided, then it is possible to meet the safety requirement by completely enclosing the PD in an insulated housing. In all PD applications, there should be no user accessible electrical connections to the LTC4278 or support  circuitry other than the RJ-45 port.

LAYOUT CONSIDERATIONS  FOR THE LTC4278

The LTC4278’s PD front end is relatively immune to layout problems. Excessive parasitic capacitance on the RCLASS pin should be avoided. Include a PCB heat sink to which the exposed pad on the bottom of the package can be soldered. This heat sink should be electrically connected to GND. For optimum thermal performance, make the heat sink as large as possible. Voltages in a PD can be as large as 57V for PoE applications, so high voltage layout

techniques should be employed.  The SHDN pin should

be separated from other high voltage pins, like VPORTP, VNEG, to avoid the possibility of leakage currents shutting down the LTC4278.  If not used, tie SHDN to VPORTN. The load capacitor connected between VPORTP and VNEG of the LTC4278 can store signiﬁcant  energy when fully charged. The design of a PD must ensure that this energy is not inadvertently dissipated in the LTC4278.  The polarity- protection diodes prevent an accidental short on the cable from causing damage. However  if, VPORTN  is shorted to VPORTP inside  the PD while capacitor  C1 is charged, current  will ﬂow through  the parasitic body diode of the internal  MOSFET and may cause permanent damage to the LTC4278.

In order to minimize switching noise and improve output load regulation, connect the GND pin of the LTC4278 directly

SwitcherCAD is a registered trademark of Linear Technology Corporation.

4278f

APPLICATIONS INFORMATION

to the ground terminal of the VCC decoupling  capacitor, the bottom terminal of the current sense resistor and the ground terminal of the input capacitor, using a ground plane with multiple vias. Place the VCC  capacitor immediately adjacent to the VCC and GND pins  on the IC package. This capacitor carries high di/dt MOSFET gate drive currents. Use a low ESR ceramic capacitor.

Take care in PCB layout to keep the traces that conduct high switching currents short, wide and with minimal overall loop area. These are typically the traces associated with the switches. This reduces the parasitic inductance and also minimizes magnetic ﬁeld radiation. Figure 19 outlines the critical paths.

Keep electric ﬁeld radiation low by minimizing the length and area of traces (keep stray capacitances low). The drain of the primary-side MOSFET is the worst offender in this category. Always use a ground plane under the switcher circuitry to prevent coupling between  PCB planes.

Check that the maximum  BVDSS ratings  of the MOSFETs are not exceeded due to inductive ringing. This is done by

viewing the MOSFET node voltages with an oscilloscope. If it is breaking down, either choose a higher voltage device, add a snubber or specify an avalanche-rated MOSFET.

Place the small-signal components away from high frequen- cy switching nodes. This allows the use of a pseudo-Kelvin connection for the signal ground, where high di/dt gate driver currents ﬂow out of the IC ground pin in one direction (to the bottom plate of the VCC decoupling capacitor) and small-signal currents ﬂow in the other direction.

Keep the trace from the feedback divider tap to the FB pin short to preclude inadvertent pick-up.

For applications with multiple switching power converters connected to the same input supply, make sure that the input ﬁlter capacitor for the LTC4278 is not shared with other converters. AC input current from another converter could cause substantial input voltage ripple which could interfere with the LTC4278 operation.  A few inches of PC trace or wire (L ≅ 100nH) between  the CIN of the LTC4278 and the actual source VIN, is sufﬁcient to prevent current sharing problems.

T1

VCC

VCC

CVCC

VCC

PG

GATE TURN-ON

GATE TURN-OFF

T2

MP

RSENSE

VIN

CVIN

•

•

•

CR	Q4

GATE TURN-ON

OUT COUT

VCC 	•    •

SG

MS

Q3		GATE TURN-OFF

4278 F19

Figure 19. Layout Critical High Current Paths

4278f

PACKAGE DESCRIPTION

DKD Package

32-Lead  Plastic  DFN (7mm × 4mm)

(Reference LTC DWG # 05-08-1734 Rev A)

0.70 ± 0.05

4.50 ± 0.05

6.43 ±0.05

3.10 ± 0.05

2.65 ±0.05

0.40 BSC

6.00 REF

0.20 ± 0.05

PACKAGE OUTLINE

RECOMMENDED SOLDER PAD LAYOUT

APPLY SOLDER MASK TO AREAS THAT ARE NOT SOLDERED

7.00 ±0.10

17

R = 0.05

TYP

R = 0.115

TYP

32

0.40 ± 0.10

6.43 ±0.10

PIN 1

TOP MARK (SEE NOTE 6)

4.00 ±0.10

2.65 ±0.10

PIN 1 NOTCH R = 0.30 TYP OR

0.35 × 45° CHAMFER

0.200 REF

0.75 ±0.05

0.00 – 0.05

16	0.40 BSC

6.00  REF

BOTTOM VIEW—EXPOSED PAD

0.20 ± 0.05  1

(DKD32)  QFN 0707 REV A

NOTE:

1. DRAWING PROPOSED TO BE MADE VARIATION  OF VERSION (WXXX) IN JEDEC PACKAGE OUTLINE M0-229

2. DRAWING NOT TO SCALE

3. ALL DIMENSIONS ARE IN MILLIMETERS

4. DIMENSIONS  OF EXPOSED PAD ON BOTTOM OF PACKAGE DO NOT INCLUDE

MOLD FLASH. MOLD FLASH, IF PRESENT, SHALL NOT EXCEED 0.20mm  ON ANY SIDE

5. EXPOSED PAD SHALL BE SOLDER PLATED

6. SHADED AREA IS ONLY A REFERENCE FOR PIN 1 LOCATION ON THE TOP AND BOTTOM OF PACKAGE

Information furnished by Linear Technology Corporation  is believed to be accurate and reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no representa- tion that the interconnection of its circuits as described herein will not infringe on existing patent rights.

4278f

39

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT®1952 | Single Switch Synchronous Forward Counter | Synchronous Controller, Programmable Volt-Sec Clamp, Low Start Current |
| LTC3803-3 | Current Mode Flyback DC/DC Controller in ThinSOTTM | 300kHz Constant-Frequency, Adjustable Slope Compensation, Optimized for High Input Voltage Applications |
| LTC3805 | Adjustable Frequency Current Mode Flyback Controller | Slope Comp, Overcurrent Protect, Internal/External Clock |
| LTC3825 | Isolated No-Opto Synchronous Flyback Controller with Wide Input Supply Range | Adjustable Switching Frequency, Programmable Undervoltage Lockout, Accurate Regulation without Trim, Synchronous for High Efﬁciency |
| LTC4257-1 | IEEE 802.3af PD Interface Controller | 100V 400mA Internal Switch, Programmable Classiﬁcation, Dual Current Limit |
| LTC4258 | Quad IEEE 802.3af Power over Ethernet Controller | DC Disconnect Only, IEEE-Compliant PD Detection and Classiﬁcation, Autonomous Operation or I2C Control |
| LTC4259A-1 | Quad IEEE 802.3af Power over Ethernet Controller | AC or DC Disconnect, IEEE-Compliant PD Detection and Classiﬁcation, Autonomous Operation |
| LTC4263 | Single IEEE 802.3af Power over Ethernet Controller | AC or DC Disconnect, IEEE-Compliant PD Detection and Classiﬁcation, Autonomous Operation or I2C Control |
| LTC4263-1 | High Power Single PSE Controller | Internal Switch, Autonomous Operation, 30W |
| LTC4264 | High Power PD Interface Controller with 750mA Current Limit | 750mA Internal Switch, Programmable Classiﬁcation Current to 75mA, Precision Dual Current Limit with Disable. |
| LTC4265 | IEEE 802.3at High Power PD Interface Controller with 2-Event Classiﬁcation | 2-Event Classiﬁcation Recognition, 100mA Inrush Current, Single-Class Programming Resistor, Full Compliance to 802.3at |
| LTC4266 | IEEE 802.3at Quad PSE Controller | Supports IEEE 802.3at Type 1 and Type 2 PDs, 0.34Ω Channel Resistance, Advanced Power Management, High Reliability 4-Point PD Detection, Legacy Capacitance Detect |
| LTC4267-1 | IEEE 802.3af PD Interface with an Integrated Switching Regulator | 100V 400mA Internal Switch, Programmable Classiﬁcation, 200kHz Constant-Frequency PWM, Optimized for IEEE-Compliant PD System |
| LTC4267-3 | IEEE 802.3af PD Interface with an Integrated Switching Regulator | 100V 400mA Internal Switch, Programmable Classiﬁcation, 300kHz Constant-Frequency PWM, Optimized for IEEE-Compliant PD System |
| LTC4268-1 | High Power PD with Synchronous No-Opto Flyback Controller | IEEE 802.3af Compliant, 750mA Hot Swap FET, 92% Power Supply Efﬁciency, Flexible Aux Support, Superior EMI |
| LTC4269-1 | IEEE 802.3af/IEEE 802.3at PD with Synchronous No-Opto Flyback Controller | 2-Event Classiﬁcation Recognition, 92% Power Supply Efﬁciency, Flexible Aux Support, Superior EMI |
| LTC4269-2 | IEEE 802.3af/IEEE 802.3at PD with Synchronous Forward Controller | 2-Event Classiﬁcation Recognition, 94% Power Supply Efﬁciency, Flexible Aux Support, Superior EMI |

ThinSOT is a trademark of Linear Technology Corporation.

Linear Technology Corporation

40

1630 McCarthy  Blvd.,  Milpitas,  CA 95035-7417

4278f

LT 0609 • PRINTED IN USA

(408) 432-1900

● FAX: (408) 434-0507

●  www.linear.com	© LINEAR TECHNOLOGY CORPORATION 2009