® 	SP6660

200mA Charge Pump Inverter or Doubler

■ Inverts or Doubles Input Supply Voltage

■ 93% Power Efficiency at 3.6V

■ 10kHz/80kHz Selectable Oscillator

■ External Oscillator up to 700KHz

■ 5	Output Resistance at 3.6V

■ Low Voltage Battery Operation

■ Ideal for 3.6V Lithium Ion Battery

■ High Output Current – 200mA

■ Pin-Compatible High-Current Upgrade of the ICL7660 and 660 Industry Standard

■ Smallest Package Available for the 660

Industry Standard – 8pin   SOIC

DESCRIPTION 		 The SP6660 is a CMOS DC-DC Monolithic Voltage Converter that can be implemented as a Voltage Inverter or a Positive Voltage Doubler. As a Voltage Inverter, a -1.5V to -4.25V output can be converted from a +1.5V to +4.25V input. As a Voltage Doubler, the SP6660 can provide a +8.0V output at 100mA from a +4.25V input. The SP6660 is ideal for both battery-powered and board level voltage conversion applications with a typical operating current of 400  A and a high efficiency (>90%) over most of its load-current range. Typical end products for this device are operational amplifier and interface power supplies, medical instruments, and hand- held and laptop computers. The SP6660 is available in 8-pin DIP, SOIC, and	SOIC packages.

TYPICAL CIRCUIT: VOLTAGE INVERTER

+VIN

+1.5V to +4.25V

FC	1

CAP+

2

+V

8

SP6660 	7

+VIN

+1.5V to +4.25V

OSC

TYPICAL CIRCUIT: VOLTAGE DOUBLER

+V FC	8

1

DOUBLE VOLTAGE OUTPUT

GND	LV

3	6

C1

CAP+

2

GND

SP6660	7

OSC

C2

1  F to 150  F

1 F to 150 F

CAP-  4

5     OUT

NEGATIVE VOLTAGE

C1

1  F to 150  F

3	LV

6

OUTPUT

CAP-   4

5      OUT

C2

1 F to 150 F

ABSOLUTE MAXIMUM RATINGS

These are stress ratings only and functional operation of the device at these ratings or any other above those indicated in the operation sections of the specifications below is not implied. Exposure to absolute maximum rating conditions for extended periods of time may affect reliability.

Power Supply Voltage

(V+ to GND or GND to OUT).........................................+4.5V LV Input Voltages........................(OUT - 0.3V) to (V+ + 0.3V) FC and OSC Input Voltages..................The least negative of (OUT - 0.3V) or (V+ - 4.5V) to (V+ + 0.3V)

SPECIFICATIONS

OUT and V+ Continuous Output Current.....................250mA Output Short-Circuit Duration to GND.................................1s Operating Temperature Ranges

SP6660C_........................................0˚C to +70˚C SP6660E_.....................................-40˚C to +85˚C

Continuous Power Dissipation (TAMB = 70˚C)

PDIP (derate 9.09mW/˚C above +70˚C)..................727mW

NSOIC (derate 5.88mW/˚C above +70˚C)...............471mW SOIC (derate 4.10mW/˚C above +70˚C)................330mW Operating Temperature...................................-40˚C to +85˚C Storage Temperature....................................-65˚C to +150˚C

Lead Temperature (soldering 10s)..............................+300˚C

| PARAMETER | MIN. | TYP. | MAX. | UNITS | CONDITIONS |
| --- | --- | --- | --- | --- | --- |
| Inverter Circuit at Low Frequency with 150 F Capacitors |  |  |  |  |  |
| V+ = 3.6V, C1 = C2 = 150µF, FC = open, T = T to T ; refer to Figure 1 test circuit. Note 2 AMB MIN MAX |  |  |  |  |  |
| Supply Voltage | 1.5 | 0.93 | 4.25 | V | R = 500 , Note 4 L |
| Supply Current |  | 0.4 | 0.8 | mA | No Load |
| Output Current | 200 |  |  | mA |  |
| Oscillator Input Current |  | ±1 |  | µA |  |
| Oscillator Frequency | 5 | 10 | 20 | kHz |  |
| Output Resistance |  | 5.2 | 10 |  | I = 100mA, Note 3 L |
| Voltage Conversion Efficiency | 99.00 | 99.96 |  | % | No Load |
| Power Efficiency | 88 80 63 | 94 85 70 |  | % | R = 500 L I = 100mA L I = 200mA L |
| Doubler Circuit at Low Frequency with 150 F Capacitors |  |  |  |  |  |
| V+ = 3.6V, C1 = C2 = 150µF, FC = open, T = T to T ; refer to Figure 2 test circuit. Note 2 AMB MIN MAX |  |  |  |  |  |
| Supply Voltage | 2.5 | 1.5 | 4.25 | V | R = 1k , Note 4 L |
| Supply Current |  | 0.4 | 0.8 | mA | No Load |
| Output Current | 200 |  |  | mA |  |
| Oscillator Input Current |  | ±1 |  | µA |  |
| Oscillator Frequency | 5 | 10 | 20 | kHz |  |
| Output Resistance |  | 5.2 | 10 |  | I = 100mA, Note 3 L |
| Voltage Conversion Efficiency | 99.00 | 99.96 |  | % | No Load |
| Power Efficiency | 91 89 79 | 96 93 85 |  | % | R = 1K L I = 100mA L I = 200mA L |

SPECIFICATIONS (continued)

| PARAMETER | MIN. | TYP. | MAX. | UNITS | CONDITIONS |
| --- | --- | --- | --- | --- | --- |
| Inverter Circuit at High Frequency with 22 F Capacitors |  |  |  |  |  |
| V+ = 3.6V, C1 = C2 = 22µF, FC = V+, T = T to T ; refer to Figure 1 test circuit. Note 2 AMB MIN MAX |  |  |  |  |  |
| Supply Voltage | 1.5 | 0.97 | 4.25 | V | R = 500 , Note 4 L |
| Supply Current |  | 0.6 | 1.5 | mA | No Load |
| Output Current | 200 |  |  | mA |  |
| Oscillator Input Current |  | ±8 |  | µA |  |
| Oscillator Frequency | 40 | 80 | 160 | kHz |  |
| Output Resistance |  | 5.0 | 10 |  | I = 100mA, Note 3 L |
| Voltage Conversion Efficiency | 99.00 | 99.96 |  | % | No Load |
| Power Efficiency | 86 80 63 | 92 86 71 |  | % | R = 500 L I = 100mA L I = 200mA L |
| Doubler Circuit at HIgh Frequency with 22 F Capacitors |  |  |  |  |  |
| V+ = 3.6V, C1 = C2 = 22µF, FC = V+, T = T to T ; refer to Figure 2 test circuit. Note 2 AMB MIN MAX |  |  |  |  |  |
| Supply Voltage | 2.5 | 1.6 | 4.25 | V | R = 1k , Note 4 L |
| Supply Current |  | 0.6 | 1.5 | mA | No Load |
| Output Current | 200 |  |  | mA |  |
| Oscillator Input Current |  | ±8 |  | µA |  |
| Oscillator Frequency | 40 | 80 | 160 | kHz |  |
| Output Resistance |  | 5.0 | 10 |  | I = 100mA, Note 3 L |
| Voltage Conversion Efficiency | 99.00 | 99.96 |  | % | No Load |
| Power Efficiency | 90 89 79 | 94 93 85 |  | % | R = 1K L I = 100mA L I = 200mA L |

NOTE 1:  Specified output resistance is a combination of internal switch resistance and capacitor ESR. NOTE 2:  In the test circuit capacitors C1 and C2 are 150  F, 0.2 maximum ESR, tantalum or 22  F, 0.2 maximum ESR, tantalum. Capacitors with higher ESR may reduce output voltage and efficiency.

Refer to Capacitor Selection section.

NOTE 3:  Specified output resistance is a combination of internal switch resistance and capacitor ESR. Refer to Capacitor Selection section.

NOTE 4:  Typical value indicates start-up voltage.

PINOUT

FC CAP+

GND

V+ OSC

LV

| CAP- OUT | SP6660 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

PIN ASSIGNMENTS

Pin  1—  FC  —  Frequency  Control  for  the

Pin 5 — OUT — (Voltage Inverter Circuit)

internal oscillator. FC = open,f

OS C

= 10KHz

Negative voltage output pin.

typical; FC = V+, f

OSC

= 80KHz typical

Pin 5 — OUT — (Positive Voltage Doubler

Pin  2  —  CAP+  —  Connect  to  the  positive terminal of the charge pump capacitor.

Pin 3 — GND — (Voltage Inverter Circuit) Ground.

Pin 3 — GND — (Positive Voltage Doubler

Circuit) Positive supply voltage input.

Pin 4 — CAP- — Connect to the negative terminal of the charge pump capacitor.

Circuit) Ground pin for power supply.

Pin 6 — LV

Low-voltage operation input pin in 660 circuits. In SP6660 circuits can be connected to GND, OUT or left open as desired with no effect.

Pin 7 — OSC — Control pin for the oscillator.

Internally  connected  to  15pf  capacitor. An external capacitor can be added to slow the oscillator. Be careful to minimize stray capitance. An external oscillator can be connected to overdrive the OSC pin.

Pin 8 — V+ — (Voltage Inverter Circuit) Positive voltage input pin for the power supply.

Pin  8  —  V+  —  (Positive  Voltage  Doubler

Circuit) Positive voltage output.

DESCRIPTION

The SP6660 Charge Pump DC-DC Voltage Converter either inverts or doubles the input voltage. As a negative voltage inverter, as shown in Figure 1, a +1.5V to +4.25V input can be converted to a -1.5V to -4.25V output. Figure 2, as a positive voltage doubler, a +2.5V to +4.25V input can be converted to a +5.0V to +8.5V output.

Typical performance curves in Figures 3 to 20 are generated using the test circuits found in Figure 1 and Figure 2. Four operating modes are shown in the curves: Voltage inverter in low and high frequency modes and voltage doubler in low and high frequency modes.

TEST CIRCUIT: VOLTAGE INVERTER

IS

FC 	V+

1

CAP+

2

SP6660

8

7 	OSC

+VIN

C1 	GND 	LV

3 	6

CAP-   4

5 	OUT

VOUT

IL

RL

C2

Figure 1. SP6660 Test Circuit for the Voltage Inverter

+VIN

IS 	TEST CIRCUIT:  VOLTAGE DOUBLER

FC

1

CAP+

2

GND

SP6660

+V

8

7 	OSC

C2 	RL

VOUT

3 		LV C1 	6

CAP-    4

5 	OUT

Figure 2. Test Circuit for the Positive Voltage Doubler

TYPICAL PERFORMANCE CHARACTERISTICS

= +3.6V, T

V

IN

AMB

= 25oC unless otherwise noted. LF = Low Frequency, FC = Open, C1 = C2 = 150  F.

HF = High Frequency, FC = V+, C1 = C2 = 22  F. Inverter Circuit use Figure 1. Doubler Circuit use Figure 2.

3A: Doubler

3B: Inverter

0.6

Supply  Current (mA)

0.5

0.4

0.3

0.2

0.1

0

0.8

Supply  Current (mA)

0.6

0.4

0.2

0

HF LF

1.5	2	2.5	3	3.5	4	4.5

Supply Voltage (V)

HF LF

1	1.5	2	2.5	3	3.5	4	4.5

Supply Voltage (V)

Figure 3A and 3B  Supply Current vs. Supply Voltage

4

3

2

Inverter

Doubler

Supply  Current (mA)

1

0

1	10	100	1000

Oscillator Frequency (kHz)

Figure 4. Supply Current vs. Oscillator Frequency

TYPICAL PERFORMANCE CHARACTERISTICS

VIN  = +3.6V, TAMB  = 25 C unless otherwise noted. LF = Low Frequency, FC = Open, C1 = C2 = 150  F.

o

HF = High Frequency, FC = V+, C1 = C2 = 22  F. Inverter Circuit use Figure 1. Doubler Circuit use Figure 2.

1.8

1.6

Voltage Drop (V)

1.4

1.2

1.0

0.8

0.6

0.4

0.2

0.0

V+ = 3.6V V+ = 2.5V

V+ = 1.5V

0	50	100	150	200	250

Load Current (mA)

Figure 5. Output Voltage Drop vs. Load Current – Inverter LF

100

Power Efficiency (%)

95

90

85

80

75

70

65

60

V+ = 3.6V V+ = 2.5V V+ = 1.5V

0	50	100	150	200	250

Load Current (mA)

Figure 6. Power Efficiency vs. Load Current – Inverter LF

-4

-3

-2

Output Voltage (V)

-1	Inverter IL = 10mA

0	Inverter IL = 100mA

1	Inverter IL = 200mA

2	Doubler IL = 10mA

3	Doubler IL = 100mA

4	Doubler IL = 200mA

5

6

7

8

1	10	100	1000

Oscillator Frequency (kHz)

Figure 7. Output Voltage vs. Oscillator Frequency

TYPICAL PERFORMANCE CHARACTERISTICS

VIN  = +3.6V, TAMB  = 25 C unless otherwise noted. LF = Low Frequency, FC = Open, C1 = C2 = 150  F.

o

HF = High Frequency, FC = V+, C1 = C2 = 22  F. Inverter Circuit use Figure 1. Doubler Circuit use Figure 2.

100

Power Efficiency (%)

90

80

Inverter IL = 10mA Inverter IL = 100mA Inverter IL = 200mA Doubler IL = 10mA Doubler IL = 100mA

70

60

1	10	100	1000

Oscillator Frequency (kHz)

Figure 8. Power Efficiency vs. Oscillator Frequency

60

Oscillator

Frequency (kHz)

50

40

30

20

10

0

1	1.5	2	2.5	3	3.5	4	4.5

Supply Voltage (V)

Figure 9. Oscillator Frequency vs. Supply Voltage – HF

8

6

Oscillator

Frequency (kHz)

4

2

0

1	1.5	2	2.5	3	3.5	4	4.5

Supply Voltage (V)

Figure 10.  Oscillator Frequency vs. Supply Voltage – LF

TYPICAL PERFORMANCE CHARACTERISTICS

VIN  = +3.6V, TAMB  = 25 C unless otherwise noted. LF = Low Frequency, FC = Open, C1 = C2 = 150  F.

o

HF = High Frequency, FC = V+, C1 = C2 = 22  F. Inverter Circuit use Figure 1. Doubler Circuit use Figure 2.

Oscillator Frequency (KHz)

100

10

1

0.1

0.01

LF HF

1	10	100	1000	10000

Capacitance (pF)

Figure 11.  Oscillator Frequency vs. External Capacitance

Oscillator Frequency (KHz)

60

40

20

0

-50	-25	0	25	50	75	100

Temperature (C)

Figure 12.  Oscillator Frequency vs. Temperature where FC=V+

Oscillator Frequency (KHz)

7

6

5

4

3

2

1

0

-50	-25	0	25	50	75	100

Temperature (C)

Figure 13.  Oscillator Frequency vs. Temperature where FC=open

TYPICAL PERFORMANCE CHARACTERISTICS

VIN  = +3.6V, TAMB  = 25 C unless otherwise noted. LF = Low Frequency, FC = Open, C1 = C2 = 150  F.

o

HF = High Frequency, FC = V+, C1 = C2 = 22  F. Inverter Circuit use Figure 1. Doubler Circuit use Figure 2.

16.0

Output Source

Resistance (Ohms)

12.0

8.0

4.0

0.0

LF HF

1.0	1.5	2.0	2.5	3.0	3.5	4.0	4.5

Supply Voltage (V)

Figure 14.  Output Source Resistance vs. Supply Voltage

7

Output

Resistance (ohms)

6

5

4

3

2

1

0

-50	-25	0	25	50	75	100

Temperature (C)

Figure 15.  Output Source Resistance vs. Temperature Inverter LF

7

Output

Resistance (ohms)

6

5

4

3

2

1

0

-50	-25	0	25	50	75	100

Temperature (C)

Figure 16.  Output Source Resistance vs. Temperature where Inverter HF

TYPICAL PERFORMANCE CHARACTERISTICS

VIN  = +3.6V, TAMB  = 25 C unless otherwise noted. LF = Low Frequency, FC = Open, C1 = C2 = 150  F.

o

HF = High Frequency, FC = V+, C1 = C2 = 22  F. Inverter Circuit use Figure 1. Doubler Circuit use Figure 2.

= 3.6V

V

IN

V	= 6.66V

= 3.6V

V	= -3.06V

V

IN

I

OUT

I

= 100mA

L

OUT

= 100mA

L

Figure 17.  Output Noise and Ripple - Doubler LF	Figure 18.  Output Noise and Ripple - Inverter LF

= 3.6V

V

IN

V	= 6.66V

= 3.6V

V	= -3.06V

V

IN

OUT

I

= 100mA

L

L

OUT

= 100mA

I

Figure 19.  Output Noise and Ripple - Doubler HF

Figure 20.  Output Noise and Ripple - Inverter HF

THEORY OF OPERATION

Negative Voltage Inverter

This is the most common application of the

SP6660 where a +1.5V to +4.25V input is converted to a -1.5V to -4.25V output. In the

inverting mode, the SP6660 is typically operated

with LV connected to GND.  Since the LV may be left open, the substitution of the SP6660 for

the ICL7660 industry standard is simplified.

The circuit for the voltage inverter mode can be found in Figure 21. This operating circuit uses only two external capacitors, C1 and C2, for the internal charge pump. This allows designers to avoid any EMI concerns with the costly, space-consuming inductors typically used with switching regulators.

The SP6660 is insensitive to load current changes. Output Source Resistance vs. Supply Voltage and Temperature curves are shown in Figures 14 to 16. A typical output source resistance of 5.2	allows an output voltage of

-4.25V under light load with an input of +4.25V. This output voltage decreases to only -4.0V with a load current draw of 100mA.

The peak-to-peak output ripple voltage is calculated as follows:

VRIPPLE = 	IOUT 	+ I 	(ESRC2)

2(fPUMP)(C2) 	OUT

TYPICAL CIRCUIT: VOLTAGE INVERTER

FC

C1

1 F to 150 F

1

CAP+

2

GND

3

CAP-  4

SP6660

+V

8

7

LV

6

5	OUT

+VIN

+1.5V to +4.25V

OSC

NEGATIVE VOLTAGE OUTPUT

C2

1 F to 150 F

Figure 21.  Typical Operating Circuit for the Voltage Inverter

For a nominal f

PUMP

of 5kHz (where f

OSC

=10kHz)

Positive Voltage Doubler

and C2=150  F with an ESR of 0.2   , the ripple

is approximately 90mV with a 100mA load current. If C2 is raised to 390  F, the ripple drops

to 45mV. The output ripple voltage is calculated by noting that capacitor C2 supplies the output

current during one-half of the charge pump cycle.

OSC is internally connected to a 15pF capacitor. An external capacitor can be added to slow the oscillator. Designers should take care to minimize stray capacitance. An external oscillator may also be connected to overdrive OSC. Refer to the Oscillator Control section for further details.

The SP6660 can double the output voltage of an

input power supply or battery. From a +4.25V

input, the circuit in Figure 22 can provide 100mA

with +8.0V at V+. The no-load voltage output at

V+ is 2(V   ).

INL

LV may be tied to OUT pin for all input voltages in the positive voltage doubler mode. Connect the power-supply positive voltage input to GND pin. Connect the power-supply ground input to OUT pin. V+ is the positive voltage output in this mode.

Designers may overdrive OSC in the positive voltage doubler mode. Refer to the Oscillator Control section for further details.

+VIN

+1.5V to +4.25V

TYPICAL CIRCUIT: VOLTAGE DOUBLER

FC	1

CAP+

2

SP6660

+V

8

7	OSC

DOUBLE VOLTAGE OUTPUT

C2

C1

1  F to 150 F

GND

3	LV

6

1  F to 150 F

CAP-   4

5	OUT

Figure 22.  Typical Operating Circuit for the Positive Voltage Doubler

Optimizing Loss Conditions

Losses in SP6660 applications can be anticipated from the following:

1.  Output Resistance:

LOSS

V

= I

LOAD

x R

OUT

where V

LOSS

is the voltage drop due to the

OUT

SP6660  output  resistance,  I	is  the  load

LOAD

| Figure 23. Four control modes for the SP6660 FC | OSC | Oscillator Frequency |
| --- | --- | --- |
| open | open | 10kHz typical |
| V+ | open | 80kHz typical |
| open or V+ | external capacitor | refer to Figure 11 |
| open | external clock | external clock frequency |

current, and R

is the SP6660 output resistance.

Oscillator Frequency

2.  Charge Pump Capacitor ESR:

Oscillator Control

LOSSC1

C1

V

x I

4 x ESR

LOAD

C1

Refer to Figure 23 for a table of the four control

where V

LOSSC1

is the voltage drop due to the

modes  of  the  SP6660  internal  oscillator

charge pump capacitor, C1, ESR

is the ESR of

frequencies. In the first mode, FC and OSC are

C1, and I

LOAD

is the load current. The loss in C1

open (unconnected) and the internal oscillator

typically runs at 10kHz. OSC is internally connected to a 15pF capacitor.

In the second mode, FC is connected to V+. The charge and discharge current at OSC changes from 1.0  A to 8.0  A, increasing the oscillator frequency eight times to 80kHz.

is larger than the loss in the reservoir capacitor,

C2, because it handles a current almost four times larger than the load current during charge-

pump operation. As a result of this, a change in the capacitor ESR has a much greater impact on

the performance of the SP6660 for C1 than for C2.

3.  Reservoir Capacitor ESR:

V

In the third mode, the oscillator frequency is lowered by connecting a capacitor between OSC

LOSSC2

C2

x I

= ESR

LOAD

C2

and GND.  FC can still multiply the frequency

where V

LOSSC2

is the voltage drop due to the

by eight times in this mode, but for a lower range

reservoir capacitor C2, ESR

is the ESR of C2,

of frequencies. Refer to Figure 11 for these ranges.

and I

LOAD

is the load current.   Increasing the

In the fourth mode, any standard CMOS logic output can be used to drive OSC. OSC may be overdriven by an external oscillator that swings

capacitance  of  C2  and/or  reducing  its  ESR

can  reduce  the  output  ripple  that  may  be caused by the charge pump. A designer can

filter  high-frequency  noise  at  the  output

between V

IN

and GND. When OSC is overdriven,

by implementing a low ESR capacitor at C2.

FC has no effect.

Unlike the 7660 and 660 industry standards, designers may overdrive the oscillator of the SP6660 in both the inverting and the Voltage Doubling Mode.

Generally, capacitors with larger capacitance values  and  higher  voltage  ratings  tend  to

reduce ESR.

Optimizing Capacitor Selection

Refer to Figure 24 for the total output resistance for various capacitance values and oscillator frequencies. The reservoir and charge pump capacitor  values  are  equal.  The  capacitance

values required to maintain comparable ripple and output resistance typically diminish proportionately as the pump frequency of the SP6660 increases.

The test conditions for the curves of Figure 24 are the same as for Figures 2 to 20 for the circuits in Figures 1 and 2; additional conditions are as follows:

C1 = C2 = 0.2	ESR capacitors

Designing  a  Multiple  of  the  SP6660

Negative Inverted Output Voltage

The SP6660 can be cascaded to allow a designer to provide a multiple of the negative inverted output voltage of a single SP6660 device. The approximate total output resistance, R	,of the

TOT

cascaded SP6660 devices is equal to the sum of the individual SP6660 output resistance values, R	. The output voltage, V	, is a multiple of the number of cascaded SP6660 devices and the output voltage of an individual SP6660 device, V	 . Refer to Figure 25 for the circuit cascading SP6660 devices. Note that the capacitance value of C1 for the charge pump and C2 at V	is multiplied respectively to the number of cascaded

OUT

OUT

OUT 	TOT

OUT

R

= 4.2

SP6660 devices.

The flat portion of the curves shown at a 5.2 effective output resistance is a result of the SP6660's 5.25	output resistance where

Connecting the SP6660 in Parallel SP6660 devices can be connected in parallel to  reduce  the  total  output  resistance.  The

approximate total output resistance, R

TOT

, of the

5.2	= R

OUT(SP6660)

+ (4 x ESR

) + ESR  .

multiple devices connected in parallel is equal to the output resistance of an individual SP6660

Instead of the typical 5.2   , R

OUT

C1

C2

= 4.2   is used

device divided by the total number of devices

because the typical specification includes the

effect of the ESRs of the capacitors used in the test circuit in Figures 1 and 2.

Refer to Figures 17, 18, 19 and 20 for the output currents  using  0.33  F  to  220  F  capacitors. Output currents are plotted for 3.0V and 4.5V inputs taking into consideration a 10% to 20% loss in the input voltage. The SP6660 5.2 series resistance limits increases in output current vs. capacitance for values much higher than

47  F. Larger values may still be useful to reduce ripple.

connected. Refer to Figure 26 for the circuit

connecting multiple SP6660 devices in parallel.

Note that only the charge pump capacitor value of C1 is multiplied respectively by the number

of  SP6660  connected  in  parallel.  A  single

capacitor C2 at the output voltage V	of the

OUT

"nth" device connected in parallel serves all devices connected.

+VIN

FC         1 	8

FC         1

+VIN

8

FC         1

+VIN

8

CAP+

2

GND

C1 	3

CAP-   4

SP6660

“1”

7

LV

6

5       OUT

OSC

C1 x 2

CAP+

2

GND

3

CAP-   4

SP6660

“2”

7

LV

6

5       OUT

OSC

C1 x n

CAP+

2

GND

3

CAP-   4

SP6660

“n”

7

LV

6

5       OUT

OSC

V

OUT

C2

VOUT  = -n x VIN

C2 _ 2

C2 _ n

where   VOUT  = output voltage, VIN = input voltage, and

n = the total number of SP6660 devices connected.

Figure 25.  SP6660 Devices Cascaded to Provide a Multiple of a Negative Inverted Output Voltage

+VIN

FC       1	8

FC       1

+VIN

8

FC       1

+VIN

8

RTOT

CAP+

2

GND

C1

3

SP6660

7	OSC

6	C1 _ 2

LV

CAP+

2

GND

3

SP6660

7	OSC

6       LV         C1 _ n

CAP+

2

GND

3

SP6660

7		OSC LV

6

“1” 	“2”

“n”

CAP-  4

5     OUT

CAP-  4

5     OUT

CAP-  4

5     OUT

RTOT =

ROUT

n

where   RTOT = total resistance of the SP6660 devices connected in parallel, 	C2

ROUT = the output resistance of a single SP6660 device, and

n = the total number of SP6660 devices connected in parallel.

Figure 26. SP6660 Devices Connected in Parallel to Reduce Output Resistance

C3

+VIN

FC	1

CAP+

2

GND

C1 	3

SP6660

V+

8

7 		OSC LV

6

D1 	D2

VOUT1

C4

CAP-    4

5 	OUT

VOUT2

C2

VOUT1 = (2 x VIN) - VFD1  - VFD2

VOUT2 = -VIN

where 	VOUT1 = positive doubled output voltage, VIN  = input voltage,

VFD1  = forward bias voltage across D1,

VFD2  = forward bias voltage across D2, and

VOUT2 =  inverted output voltage.

Figure 27. The SP6660 Connected for Negative Voltage Conversion with Positive Supply Multiplication

OUT2

Circuit for Negative Voltage Conversion with Positive Supply Multiplication

A designer can use the circuit in Figure 27 to

provide both an inverted output voltage at V

generate the inverted output voltage at V	. Capacitor C3 is for the charge pump and capacitor C4 is for the reservoir function to generate the multiplied  positive  output  voltage  at  V	.

OUT1

and  a  positive  multiple  of  VIN    at  VOUT2

(subtracting the forward biased voltages of D1

and D2).  Capacitor C1 is for the charge pump and capacitor C2 is for the reservoir function to

OUT1

Designers should pay special attention to the

possibility of higher source impedances at the

generated supplies due to the finite impedance of the common charge pump driver.

VIN

+    C3

DOUBLER

D1

C2

VOUT

1 VIN

LP2985

VOUT  5

+5

C4

4.7  F Cer.

150  F

1 FC

SP6660

V+ 8

150  F

2 GND

GND

GND

CAP+

Tant.

+

C1

150  F Tant.

2 CAP+

3 GND

4 CAP-

OSC 7

LV 6

OUT 5

+  Tant.

GND

FC

3 ON/OFF_N   BYPASS 4

C5

10nF Cer.

Figure 28. The SP6660 and a LDO Regulator Connected as a 3V Input to Regulated 5V Output Converter.

APPLICATIONS

The SP6660 Evaluation Board provides a 3V to

5V  160mA  DC  to  DC  Converter  using  the

SP6660  Doubler  Circuit  and  a  5V  LDO Regulator.

SP6660 Ripple

100

90

= 3.2V

V

IN

V	6660 = 5.53V

IL = 150mA

80

OUT

V

OUT

I

LDO = 4.95V

Power Efficiency (%)

= 150mA	70

LOAD

5VLDO Ripple

60

2.8 	3.0 	3.2 	3.4 	3.6 	3.8 	4.0 	4.2

Input Voltage  (V)

Figure 29.  Ripple and Noise output of the SP6660 and a

LDO Regulator with ILOAD  = 150mA

Figure 30.  Power Efficiency vs Input Voltage - SP6660

Doubler with 5V LDO

100

Power Efficiency (%)

90

80

Vin = 3.0V Vin = 3.3V Vin = 3.6V

200

150

Ripple Voltage (mV)

100

6660  Ripple

IL = 150mA

LDO Ripple

IL = 150mA

70	50

60

1	1 0	100	1000

Load Current (mA)

0

2.8     3.0 	3.2     3.4 	3.6     3.8 	4.0     4.2

Input Voltage (V)

Figure 31. Power Efficiency vs Load Current - SP6660

Doubler with 5V LDO

Figure 32. Ripple Voltage vs Input Voltage - SP6660 Doubler with 5V LDO

PACKAGE: 	PLASTIC DUAL–IN–LINE (NARROW)

E1  E

D1 = 0.005" min. (0.127 min.)

D

A1 = 0.015" min. (0.381min.)

A = 0.210" max. (5.334 max).

A2		C L	Ø

B1

e = 0.100 BSC (2.540 BSC)

B

ALTERNATE END PINS

eA = 0.300 BSC (7.620 BSC)

(BOTH ENDS)

DIMENSIONS (Inches) Minimum/Maximum (mm)

8–PIN 	14–PIN 	16–PIN

18–PIN

20–PIN

22–PIN

A2 		0.115/0.195 (2.921/4.953)

B 		0.014/0.022 (0.356/0.559)

B1 		0.045/0.070 (1.143/1.778)

C 		0.008/0.014 (0.203/0.356)

D 	0.355/0.400

0.115/0.195 (2.921/4.953)

0.014/0.022 (0.356/0.559)

0.045/0.070 (1.143/1.778)

0.008/0.014 (0.203/0.356)

0.735/0.775

0.115/0.195 (2.921/4.953)

0.014/0.022 (0.356/0.559)

0.045/0.070 (1.143/1.778)

0.008/0.014 (0.203/0.356)

0.780/0.800

0.115/0.195 (2.921/4.953)

0.014/0.022 (0.356/0.559)

0.045/0.070 (1.143/1.778)

0.008/0.014 (0.203/0.356)

0.880/0.920

0.115/0.195 (2.921/4.953)

0.014/0.022 (0.356/0.559)

0.045/0.070 (1.143/1.778)

0.008/0.014 (0.203/0.356)

0.980/1.060

0.115/0.195 (2.921/4.953)

0.014/0.022 (0.356/0.559)

0.045/0.070 (1.143/1.778)

0.008/0.014 (0.203/0.356)

1.145/1.155

(9.017/10.160)  (18.669/19.685) (19.812/20.320)  (22.352/23.368)  (24.892/26.924)  (29.083/29.337)

E 		0.300/0.325 (7.620/8.255)

E1 		0.240/0.280 (6.096/7.112)

L 		0.115/0.150 (2.921/3.810)

Ø 		0°/ 15° (0°/15°)

0.300/0.325 (7.620/8.255)

0.240/0.280 (6.096/7.112)

0.115/0.150 (2.921/3.810)

0°/ 15° (0°/15°)

0.300/0.325 (7.620/8.255)

0.240/0.280 (6.096/7.112)

0.115/0.150 (2.921/3.810)

0°/ 15° (0°/15°)

0.300/0.325 (7.620/8.255)

0.240/0.280 (6.096/7.112)

0.115/0.150 (2.921/3.810)

0°/ 15° (0°/15°)

0.300/0.325 (7.620/8.255)

0.240/0.280 (6.096/7.112)

0.115/0.150 (2.921/3.810)

0°/ 15° (0°/15°)

0.300/0.325 (7.620/8.255)

0.240/0.280 (6.096/7.112)

0.115/0.150 (2.921/3.810)

0°/ 15° (0°/15°)

PACKAGE: 	PLASTIC

SMALL OUTLINE (SOIC) (NARROW)

E    H

h x 45° D

A

Ø

e	B	A1	L

DIMENSIONS (Inches) Minimum/Maximum (mm)

A A1

B D E e H h L Ø

8–PIN

0.053/0.069 (1.346/1.748)

0.004/0.010 (0.102/0.249

0.014/0.019 (0.35/0.49)

0.189/0.197 (4.80/5.00)

0.150/0.157 (3.802/3.988)

0.050 BSC (1.270 BSC)

0.228/0.244 (5.801/6.198)

0.010/0.020 (0.254/0.498)

0.016/0.050 (0.406/1.270)

0°/8° (0°/8°)

14–PIN

0.053/0.069 (1.346/1.748)

0.004/0.010 (0.102/0.249)

0.013/0.020 (0.330/0.508)

0.337/0.344 (8.552/8.748)

0.150/0.157 (3.802/3.988)

0.050 BSC (1.270 BSC)

0.228/0.244 (5.801/6.198)

0.010/0.020 (0.254/0.498)

0.016/0.050 (0.406/1.270)

0°/8° (0°/8°)

16–PIN

0.053/0.069 (1.346/1.748)

0.004/0.010 (0.102/0.249)

0.013/0.020 (0.330/0.508)

0.386/0.394 (9.802/10.000)

0.150/0.157 (3.802/3.988)

0.050 BSC (1.270 BSC)

0.228/0.244 (5.801/6.198)

0.010/0.020 (0.254/0.498)

0.016/0.050 (0.406/1.270)

0°/8° (0°/8°)

PACKAGE: 	PLASTIC MICRO SMALL OUTLINE (  SOIC)

0.0965

0.003

0.0256

BSC

0.118

0.012

0.003

R .003

12.0˚

4˚

0.008

0˚ - 6˚

0.006

0.006

0.006

0.006

0.004

0.16

0.003

0.020

0.020

1	2

12.0˚

04.˚01

0.0215

0.006

0.037

Ref

3.0˚

3˚

0.116

0.004

0.013

0.005

0.118

0.004

0.034

0.004

0.040

0.003

0.004

0.002

0.116

0.004

0.118

0.004

All package dimensions in inches

50 	SOIC devices per tube

ORDERING INFORMATION

Model	Temperature Range	Package Type SP6660CP . ............................................. 0˚C to +70˚C .............................................. 8-Pin PDIP SP6660EP . ............................................ -40˚C to +85˚C ............................................ 8-Pin PDIP

SP6660CN . ............................................. 0˚C to +70˚C ........................................... 8-Pin NSOIC SP6660EN . ............................................ -40˚C to +85˚C ......................................... 8-Pin NSOIC

SP6660CU . ............................................. 0˚C to +70˚C ........................................... 8-Pin   SOIC SP6660EU . ............................................ -40˚C to +85˚C ......................................... 8-Pin   SOIC

SP6660EB .......................................................................................................... Evaluation Board

Please consult the factory for pricing and availability on a Tape-On-Reel option.

Corporation

SIGNAL PROCESSING EXCELLENCE

Sipex Corporation

Headquarters and

Sales Office

22 Linnell Circle

Billerica, MA  01821

TEL: (978) 667-8700

FAX: (978) 670-9001

e-mail: sales@sipex.com

Sales Office

233 South Hillview Drive

Milpitas, CA 95035

TEL: (408) 934-7500

FAX: (408) 935-7600

Sipex Corporation reserves the right to make changes to any products described herein. Sipex does not assume any liability arising out of the application or use of any product or circuit described herein; neither does it convey any license under its patent rights nor the rights of others.