® 	SPX29300/01/02/03

3A High Current, Low Dropout Voltage Regulator

Adjustable & Fixed Output, Fast Response Time

FEATURES

■  Adjustable Output Down To 1.25V

■  1% Output Accuracy

■  Output Current of 3A

■  Low Dropout Voltage of  450mV @ 3A

■  Extremely Tight Load and Line

SPX29300

3 Pin TO-263

1	2	3

Regulation

VIN

GND

VOUT

■  Extremely Fast Transient Response

■  Reverse-Battery Protection

■  Zero Current Shutdown (5 pin version)

■  Error Flag Signal Output for Out of

Regulation State (5 pin version)

■  Standard TO-220 and TO-263

Packages

Now Available in Lead Free Packaging

APPLICATIONS

■  Powering VGA & Sound Card

■  Power PC™ Supplies

■  SMPS Post Regulator

■  High Efficiency “Green” Computer

Systems

■  High Efficiency Linear Power Supplies

■  Constant Current Regulators

■  Adjustable Power Supplies

■  Battery Charger

DESCRIPTION The SPX29300/01/02/03 are 3A, highly accurate voltage regulators with a low drop out voltage of 450mV dropout (typical) @ 3A. These regulators are specifically designed for low

voltage applications that require a low dropout voltage and a fast transient response. They

are fully fault protected against over-current, reverse battery, and positive and negative voltage transients. On-Chip trimming adjusts the reference voltage to 1% initial accuracy. Other features in the 5 pin versions include Enable, and Error Flag.

The SPX29300/01/02/03 are offered in 3 & 5-pin TO-220 & TO-263 packages. For a 1.5A

version, refer to the SPX29150 data sheet.

TYPICAL APPLICATIONS CIRCUITS

VIN

1   SPX29300  3

VOUT	IN

V

1  SPX29302  4

2

V

OUT

+   10ﾵF

+ 	2 	+

+

6.8ﾵF

3	5	R1

6.8ﾵF

10ﾵF

GND	ADJ

R2

Figure 1. Fixed Output Linear Regulator	Figure 2. Adjustable Output Linear Regulator

ABSOLUTE MAXIMUM RATINGS

Lead Temperature (soldering, 5 seconds)    ................260ﾰC Storage Temperature Range........................-65ﾰC to +150ﾰC Operating Junction Temperature Range......-40ﾰC to +125ﾰC

Input Voltage (Note 1) .................................................... 16V

ELECTRICAL CHARACTERISTICS (Note 1) at VIN=VOUT + 1V and IOUT = 10mA, CIN = 6.8ﾵF, COUT = 10ﾵF, TA = 25ﾰC, unless otherwise specified. The Boldface applies over the junction temperature range.

| SPX29300/01 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PARAMETER | CONDITIONS | TYP | MIN | MAX | UNITS |
| Fixed Voltage Options |  |  |  |  |  |
| 1.8V Version |  |  |  |  |  |
| Output Voltage | IOUT = 10mA 10mA ≤ IOUT ≤ 3A, 2.8V ≤ VIN ≤ 16V | 1.800 1.800 | 1.782 1.764 | 1.818 1.836 | V |
| 2.5V Version |  |  |  |  |  |
| Output Voltage | IOUT = 10mA 10mA ≤ IOUT ≤ 3A, 3.5V ≤ VIN ≤ 16V | 2.500 2.500 | 2.475 2.450 | 2.525 2.550 | V |
| 3.3V Version |  |  |  |  |  |
| Output Voltage | IOUT = 10mA 10mA ≤ IOUT ≤ 3A, 4.3V ≤ VIN ≤ 16V | 3.300 3.300 | 3.267 3.234 | 3.333 3.366 | V |
| 5.0V Version |  |  |  |  |  |
| Output Voltage | IOUT = 10mA 10mA ≤ IOUT ≤ 3A, 6.0V ≤ VIN ≤ 16V | 5.000 5.000 | 4.950 4.900 | 5.050 5.100 | V |
| All Voltage Options SPX29300/01/02/03 |  |  |  |  |  |
| Line Regulation | IOUT=10mA,(VOUT+1V) ≤ VIN ≤ 16V | 0.06 |  | 0.5 | % |
| Load Regulation | VIN=VOUT+1V, 10mA≤ IOUT ≤ IFL (Note 2) | 0.2 |  | 1 | % |
| ∆V/∆T | VOUT Temp Coefficient (Note 6) | 20 |  | 100 | ppm/ﾰC |
| Dropout Voltage, except 1.8V, (Note 3) | IOUT=100mA IOUT=1.5A IOUT=3.0A | 50 150 450 |  | 175 700 | mV |
| Ground Current (Note 5) | IOUT=1.5A IOUT=3.0A | 10 37 |  | 35 | mA |
| Ground Pin Current at Dropout | VIN=0.5V less than specified VOUT, IOUT=10mA | 0.9 |  |  | mA |
| Current Limit | VOUT=0V (Note 4) | 4.5 | 3.0 |  | A |
| Output Noise Voltage (10Hz to 100kHz) IL=100mA | CL=10ﾵF CL=33ﾵF | 400 260 |  |  | ﾵVRMS |
| Reference Voltage | Adjustable version only | 1.240 | 1.228 1.215 | 1.252 1.265 | V |
| Reference Voltage | Adjustable version only (Note 8) |  | 1.203 | 1.277 | V |
| Adjust Pin Bias Current |  | 40 |  | 80 120 | nA |
| Reference Voltage Temp. Coeff. | (Note 7) | 20 |  |  | ppm/ﾰC |
| Adjust Pin Bias Current Temp. Coeff. |  | 0.1 |  |  | nA/ﾰC |

ELECTRICAL CHARACTERISTICS at VIN=VOUT + 1V and IOUT = 10mA, CIN = 6.8ﾵF, COUT = 10ﾵF, TA = 25ﾰC, unless otherwise specified. The Boldface applies over the junction temperature range. Adjustable versions are set to 5.0V.

| PARAMETER | CONDITIONS | TYP | MIN | MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| FLAG OUTPUT (ERROR COMPARATOR) SPX29301/03 |  |  |  |  |  |
| Output Leakage Current | VOH=16V | 0.01 |  | 1 2 | ﾵA |
| Output Low Voltage | Device set for 5V,VIN=4.5V, IOL=250ﾵA | 220 |  | 300 400 | mV |
| Upper Threshold Voltage | Device set for 5V, (Note 9) | 60 | 40 25 |  | mV |
| Lower Threshold Voltage | Device set for 5V, (Note 9) | 75 |  | 95 140 | mV |
| Hysteresis | Device set for 5V, (Note 9) | 15 |  |  | mV |
| ENABLE Input SPX29301/02 |  |  |  |  |  |
| Input Logic Voltage Low (OFF) High (ON) | VIN<10V |  | 2.4 | 0.8 | V |
| ENABLE Input Pin | VEN=16V VEN=0.8V | 100 |  | 600 750 1 2 | ﾵA ﾵA |
| Regulator Output Current in Shutdown | (Note 10) | 10 |  | 500 | ﾵA |
| Thermal Resistance | TO-200 Junction to Case, at Tab TO-220 Junction to Ambient TO-263 Junction to Case, at Tab TO-263 Junction to Ambient | 3 60 3 60 |  |  | ﾰC/W |

NOTES:

Note 1: Maximum positive supply voltage of 20V must be of limited duration (<100ms) and duty cycle of less than 1%. The maximum continuous supply voltage is 16V.

Note 2: Full load current (IFL) is defined as 3.0A.

Note 3: Dropout voltage is defined as the input to output differential when the output voltage drops to 99% of its nominal value.

Note 4: VIN = VOUT (NOMINAL) +1V.  For example, use VIN = 4.3V for a 3.3V regulator.  Employ pulse-testing procedures to minimize temperature rise. Note 5: Ground pin current is the regulator quiescent current.  The total current drawn from the source is the sum of the load current to the ground current. Note 6: Output voltage temperature coefficient is defined as the worst case voltage change divided by the total temperature range

Note 7: Thermal regulation is defined as the change in output voltage at time T after a change in power dissipation is applied, excluding load / line regulation effects.

Specifications for a 200mA load pulse as VIN = 20V (a 4W pulse) for t = 10ms. Note 8: VREF VOUT  (VIN-1), 2.3VVIN  16V, 10mA  IL  IFL, Tj < Tjmax.

Note 9: Comparator threshold is expressed in terms of a voltage differential at the Adjust terminal below the nominal reference voltage measured 6V input.  To express these thresholds in terms of output voltage change, multiply the error amplifier gain = VOUT/VREF = (R1 + R2)/R2.  For example, at a programmable output voltage of 5V, the Error output is guaranteed to go low when the output drops by 95mVx 5V/ 1.240V = 38mV.  Threshold remain constant as a percent of VOUT as VOUT is varied, with the dropout warning occurring at typically 5% below nominal, 7.7% guaranteed.

Note 10:  VEN  0.8V and VIN  16V, VOUT = 0.

BLOCK DIAGRAM

IN	OUT

FLAG

1.180V	1.240V

+	Reference	+

O.V ILIMIT

28V	R1*

-	-

ADJ

EN

Thermal

Shutdown

R2*

GND

TYPICAL PERFORMANCE CHARACTERISTICS

1200

1000

800

600

Vdropout (mV)

400

200

0

85ﾰC

25ﾰC

CL = 10ﾵF Vin = 3.2V

2.540

2.530

2.520

Vout (V)

2.510

2.500

2.490

2.480

2.470

2.460

2.5V Device IL = 10mA CL = 10ﾵF

4 	6 	8 	10 	12 	14 	16

0 	1.0 	2.0 	3.0

IL (A)

Vin (V)

Figure 3. Dropout Voltage vs Load Current	Figure 4. Line Regulation

80.0

70.0

60.0

50.0

85ﾰC	25ﾰC

| 3.310 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Vin = 4.3V CL = 10ﾵF |  |  |

3.305

Vout (V )

3.300

40.0

Ignd (mA)

30.0

20.0

10.0

0.0

3.3V Device Vin = 4.3V CL = 10ﾵF

0 	1.0 	2.0 	3.0

IL (A)

3.295

3.285

3.280

0 	1.0 	2.0 	3.0

IL (A)

Figure 5. Ground Current vs Load Current	Figure 6. Load Regulation

1.9

1.8

1.7

Vth (V)

1.6

1.5

3.3V Device

1.4

1.3

1.2

-40 	-20 	0 	20 	40 	60 	80 	100 	120

Temperature ( o C)

Figure 7. Enable Threshold vs Temperature

APPLICATION INFORMATION

The SPX29300/01/02/03 incorporates protec- tion against over-current faults, reversed load insertion, over temperature operation, and positive and negative transient voltage.

Thermal Considerations

Although the SPX29300/01/02/03 offers limiting circuitry for overload conditions, it is still necessary to insure that the maximum junction temperature is not exceeded  in the application. Heat will flow through the lowest resistance path, the junction-to-case path.  In order to insure the best thermal flow of the component, proper mounting is required.

TO-220 Design Example:

response is needed. If the power source has a high AC impedance, a 0.1ﾵF ceramic capaci- tor between input & ground is recommended.

Minimum Load Current

To ensure a proper behavior of the regulator under light load, a minimum load of 5mA for SPX29300/01/02/03 is required.

Adjustable Regulator Design

The SPX29302/03 are adjustable regulators that can be programmed to any value between

1.25V and 16V using 2 resistors, R1 and R2. The relationship between the resistors is:

R1 = R2(Vout/1.240-1).

Assume that V

OUT

IN

= 10V, V

= 5V, I

OUT

=

Error Flag

1.5A, T

A

= 50ﾰC, 

HA= 1ﾰC/W, 

CH= 2ﾰC/W,

The SPX29301/03 features an error flag that

and JC= 3Cﾰ/W, where:

T

= ambient temperature,

A

HA= heatsink to ambient thermal resistance

CH= case to heatsink thermal resistance

JC  = junction to case thermal resistance

The power calculated under these conditions is:

indicates either an over current or under

voltage condition.  The flag output goes low, sinking 10mA when either condition occurs.

Enable Input

The SPX29301/02 have an Enable function that switches the regulator on and off.  Their thresholds are TTL compatible.  When the regulator is active, approximately 20 uA flows through the Enable pin.

= (V

P

D

IN

OUT

– V

OUT

) * I

= 7.5W.

Typical Application Circuits

And the junction temperature is calculated as

Figure 1 represents at typical fixed output

T = T  + P

* (	+ 

+   ) or

J 	A 	D

HA 	CH 	JC

regulator.  Figure 2 represents an adjustable

TJ = 50 + 7.5 * (1+2+3) = 95ﾰC

output regulator.  The values of R1 and R2 set the output voltage value as follows:

Reliable operation is insured.

OUT

V

REF

=V

* [1 + (R1/R2)].

Capacitor Requirements

The output capacitor is needed to insure stability and minimize the output noise.  The value of the capacitor varies with the load. However, a minimum value of 10ﾵF alumi- num capacitor will guarantee stability over all load conditions.  A tantalum capacitor is recommended if a faster load transient

For best results, the total series resistance

should be small enough to pass a minimum regulator load current of 5 mA. A minimum value of 10kohms is recommended for R2

with a range between 10kohms and 47 kohms.

PACKAGE: 3 PIN TO-220

E	A

E/2

E2

Q 	øP

B

SEATING A      	 PLANE

A1

H1

D

D1

L1 	 	CHAMFER OPTIONAL

1

C    C 	D     D

L

b 	A

| e Dimensions in (mm) | 3 PIN TO-220 JEDEC TO-220 (AB) Variation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MIN |  | NOM |  | MAX |
| A | .140 |  | - |  | .190 |
| A1 | .020 |  | - |  | .055 |
| A2 | .080 |  | - |  | .115 |
| b | .015 |  | .027 |  | .040 |
| b2 | .045 |  | - |  | .070 |
| c | .014 |  | - |  | .024 |
| D | .560 |  | - |  | .650 |
| D1 | .330 |  | - |  | .355 |
| D2 | .480 |  | - |  | .507 |
| E | .380 |  | - |  | .420 |
| E1 | .270 |  | - |  | .350 |
| E2 | - |  | - |  | .030 |
| e | .100 BSC |  |  |  |  |
| e1 | .200 BSC |  |  |  |  |
| H1 | .230 | - |  | .270 |  |
| L1 | - | - |  | .250 |  |
| L2 | - | - |  | - |  |
| ∆P | .139 | - |  | .161 |  |
| Q | .100 | - |  | .135 |  |

e1  		C

A2

b

WITH PLATING

c

BASE METAL

CONTACT AREA

3 PIN TO-220

PACKAGE: 5 PIN TO-220

E

A

E/2 	B

Q 	øP H1

SEATING A      	 PLANE

F

D

D1 	D

L1

CHAMFER OPTIONAL

C   C 	D    D

L

A

| e b 5 PIN TO-220 | Dimensions in (inches) |  |  |
| --- | --- | --- | --- |
|  | MIN | NOM | MAX |
| A | .160 | - | .190 |
| b | .015 | - | .040 |
| C1 | .014 | - | .022 |
| D | .560 | - | .590 |
| E | .385 | - | .415 |
| e | .385 | - | .415 |
| e1 | .062 | - | .072 |
| F | .045 | - | .055 |
| H1 | .234 | - | .258 |
| J1 | .090 | - | .115 |
| L | .540 | - | .560 |
| L1 |  | - | .250 |
| ∆P | .146 | - | .156 |
| Q | .103 | - | .113 |
| U | - | .30 | - |
| V | - | .24 | - |

C1

e1

J1

5 PIN TO-220

PACKAGE: 3 PIN TO-263

E

E/2

L1

SEATING PLANE A

C2

D 	H

1 	2 	3

B	B

c

e

See View C

| b b2 3-PIN TO-263 JEDEC TO-263 (AA) Variation | Dimensions in inches |  |  |
| --- | --- | --- | --- |
|  | MIN | NOM | MAX |
| A | .160 | - | .190 |
| A1 | .000 | - | .010 |
| b | .020 | - | .039 |
| c D | .015 .330 | - - | .029 .380 |
| D1 | .270 | - | - |
| E | .380 | - | .420 |
| E1 | .245 | - | - |
| e | .100 BSC |  |  |
| H | .575 | - | .625 |
| L | .070 | - | .110 |
| L1 | - | - | .066 |
| L2 | - | - | .070 |
| L3 | .010 BSC |  |  |

GAUGE PLANE

0º-8º

SEATING PLANE

L	A1

View C

b

WITH PLATING

c

BASE METAL

SECTION B-B

3 PIN TO-263

PACKAGE: 5 PIN TO-263

(DATUM A)

SEATING PLANE A

C2

H

E

1	2	3	4	5

L2

c

(E)

E

L1

D1

| 4X e 5 PIN TO-263 JEDEC TO-263 (BB) Variation | Dimensions in (mm) |  |  |
| --- | --- | --- | --- |
|  | MIN | NOM | MAX |
| A | .160 | - | .190 |
| A1 | 0 | - | .010 |
| b | .020 | - | .039 |
| c c2 | .015 .045 | - - | .029 .023 |
| D1 | .270 | - | - |
| E | .380 | - | .420 |
| E1 | .245 | - | - |
| e | .067 BSC |  |  |
| H | .575 | - | .625 |
| L | .070 | - | .110 |
| L1 | - | - | .066 |
| L2 | - | - | .070 |
| L3 | .010 BSC |  |  |

5X b

E1

0º-8º

L3	L

WITH PLATING

(b)

c

BASE METAL

CONTACT AREA

5 PIN TO-263

PACKAGE PINOUTS

TO-263-3 Package  (T)

TO-263-5 Package (T5)

SPX29301 	SPX29302 	SPX29303

1	2	3

1    2    3    4   5

1)ENABLE

2)INPUT

3)GND

4)OUTPUT

5)FLAG

1) ENABLE

2) INPUT

3) GND

4) OUTPUT

5) ADJUST

1)FLAG

2)INPUT

3)GND

4)OUTPUT

5)ADJUST

V IN  GND VOUT Front View

Top View

TO-220-3 Package (U)

TO-220-5 Package (U5)

SPX29301 	SPX29302 	SPX29303

1      2     3

GND

1 2 3 4 5

1)ENABLE

2)INPUT

3)GND

4)OUTPUT

5)FLAG

1) ENABLE

2) INPUT

3) GND

4) OUTPUT

5) ADJUST

1)FLAG

2)INPUT

3)GND

4)OUTPUT

5)ADJUST

VIN 	V OUT

Front View

Top View

*Tab is internally connected to GND

ORDERING INFORMATION

PART NUMBER ACCURACY OUTPUT VOLTAGE PACKAGE

SPX29300U-1.8 1.0% 1.8V 3 lead TO-220

SPX29300U-2.5 1.0% 2.5V 3 lead TO-220

SPX29300U-3.3 1.0% 3.3V 3 lead TO-220

SPX29300U-5.0 1.0% 5.0V 3 lead TO-220

SPX29300T-1.8 1.0% 1.8V 3 lead TO-263

SPX29300T-1.8/TR 1.0% 1.8V 3 lead TO-263

SPX29300T-2.5 1.0% 2.5V 3 lead TO-263

SPX29300T-2.5/TR 1.0% 2.5V 3 lead TO-263

SPX29300T-3.3 1.0% 3.3V 3 lead TO-263

SPX29300T-3.3/TR 1.0% 3.3V 3 lead TO-263

SPX29300T-5.0 1.0% 5.0V 3 lead TO-263

SPX29300T-5.0/TR 1.0% 5.0V 3 lead TO-263

Available in lead free packaging.  To order add   "-L" suffix to part number. Example: SPX29300T-5.0/TR = standard; SPX29300T-L-5.0/TR = lead free

/TR = Tape and Reel

Pack quantity is 500 for TO-263.

Sipex Corporation

Headquarters

233 South Hillview Drive

Milpitas, CA 95035

TEL: (408) 934-7500

FAX: (408) 935-7600m

ORDERING INFORMATION

PART NUMBER ACCURACY OUTPUT VOLTAGE PACKAGE

SPX29301U5-1.8 1.0% 1.8V 5 lead TO-220

SPX29301U5-2.5 1.0% 2.5V 5 lead TO-220

SPX29301U5-3.3 1.0% 3.3V 5 lead TO-220

SPX29301U5-5.0 1.0% 5.0V 5 lead TO-220

SPX29301T5-1.8 1.0% 1.8V 5 lead TO-263

SPX29301T5-1.8/TR 1.0% 1.8V 5 lead TO-263

SPX29301T5-2.5 1.0% 2.5V 5 lead TO-263

SPX29301T5-2.5/TR 1.0% 2.5V 5 lead TO-263

SPX29301T5-3.3 1.0% 3.3V 5 lead TO-263

SPX29301T5-3.3/TR 1.0% 3.3V 5 lead TO-263

SPX29301T5-5.0 1.0% 5.0V 5 lead TO-263

SPX29301T5-5.0/TR 1.0% 5.0V 5 lead TO-263

SPX29302T5 1.0% Adj 5 lead TO-263

SPX29302T5/TR 1.0% Adj 5 lead TO-263

SPX29302U5 1.0% Adj 5 lead TO-220

SPX29303T5 1.0% Adj 5 lead TO-263

SPX29303T5/TR 1.0% Adj 5 lead TO-263

SPX29303U5 1.0% Adj 5 lead TO-220

Available in lead free packaging.  To order add   "-L" suffix to part number. Example: SPX29302T5/TR = standard; SPX29302T5-L-5.0/TR = lead free

/TR = Tape and Reel

Pack quantity is 500 for TO-263.

Corporation

ANALOG EXCELLENCE

Sipex Corporation

Headquarters

233 South Hillview Drive

Milpitas, CA 95035

TEL: (408) 934-7500

FAX: (408) 935-7600m

Sipex Corporation reserves the right to make changes to any products described herein. Sipex does not assume any liability arising out of the application or use of any product or circuit described herein; neither does it convey any license under its patent rights nor the rights of others.