MPMB62D-68KX3

256MB PC-2700 DDR DIMM

MPMB62D-68KX3   PC-2700 CL2.5 184pin DDR DIMM

32Mx64 DDR DIMM based on 16Mx8 DDR SDRAMs with SPD

DESCRIPTION

The MPMB62D-68KX3 is 32M bit x 64 Double Data Rate Synchronous Dynamic RAM

high density memory module based on 128Mb DDR SDRAM respectively.

The MPMB62D-68KX3 consists of sixteen CMOS 16M      8 bit with 4 banks Double Data Rate Synchronous DRAMs in TinyBGA package and a 2K EEPROM in 8-Pin TSSOP package mounted on a 184pin glass-epoxy substrate. Two 0.1µF decoupling capacitors are mounted on the printed circuit board in parallel for each DDR SDRAM.

The MPMB62D-68KX3 is a Dual In- line Memory Module and is intended for mounting into 184pin edge connector sockets.

Synchronous design allows precise cycle control with the use of system clock. Data I/O transactions are possible on both edges of every clock cycle. Range of operating frequencies, programmable latencies and burst lengths allows the same device to be useful for a variety of high bandwidth, high performance memory system applications.

FEATURES

Performance range - 166MHz ( DDR333, CL2.5 )

Double-data-rate architecture; two data transfers per clock cycle

Bi-directional data strobe (DQS) Differential clock inputs (CK and /CK)

DLL aligns DQ and DQS transition with CK transition Auto & self refresh capability (4096 Cycles / 64ms) Single 2.5V ±0.2V power supply

Programmable Read latency 2, 2.5 (clock) Programmable Burst length (2, 4, 8)

Programmable Burst type (Sequential & Interleave) Edge aligned data output, center aligned data input Serial presence detect with EEPROM

PCB : Height (1,181 mil), double sided component

Kingmax - Memory Module                            1                        March 2002  Rev: 1.1

MPMB62D-68KX3

| 256MB PC-2700 DDR DIMM Pin Name | Function |
| --- | --- |
| A0 ~ A11 | Address input (Multiplexed) |
| BA0 , BA1 | Select bank Address |
| DQ0 ~ DQ63 | Data input/output |
| DQS0 ~ DQS7 | Data Strobe input/output |
| CK0 ~ CK2 | Clock input |
| /CK0 ~ /CK2 | Clock input |
| CKE0 , CKE1 | Clock enable input |
| /CS0 , /CS1 | Chip select input |
| /RAS | Row address strobe |
| /CAS | Column address strobe |
| /WE | Write enable |
| DM0 ~ DM7 | Data-in mask |
| VDD | Power supply (2.5V) |
| VDDQ | Power supply for DQS (2.5V) |
| VSS | Ground |
| VREF | Power supply for reference |
| VDDSPD | Serial EEPROM power supply (2.5V~6V) |
| SDA | Serial data I/O |
| SCL | Serial clo ck |
| SA0 ~ SA2 | Address in EEPROM |
| VDDID | VDD identification flag |
| NC | No connection |

PIN CONFIGURATIONS (Front side/Back side)           PIN NAME

| Pin | Front | Pin | Front | Pin | Back | Pin | Back |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VREF | 47 | *DQS8 | 93 | VSS | 139 | VSS |
| 2 | DQ0 | 48 | A0 | 94 | DQ4 | 140 | *DM8 |
| 3 | VSS | 49 | *CB2 | 95 | DQ5 | 141 | A10 |
| 4 | DQ1 | 50 | VSS | 96 | VDDQ | 142 | *CB6 |
| 5 | DQS0 | 51 | *CB3 | 97 | DM0 | 143 | VDDQ |
| 6 | DQ2 | 52 | BA1 | 98 | DQ6 | 144 | *CB7 |
| 7 | VDD | 53 | DQ32 | 99 | DQ7 | 145 | VSS |
| 8 | DQ3 | 54 | VDDQ | 100 | VSS | 146 | DQ36 |
| 9 | NC | 55 | DQ33 | 101 | NC | 147 | DQ37 |
| 10 | NC | 56 | DQS4 | 102 | NC | 148 | VDD |
| 11 | VSS | 57 | DQ34 | 103 | *A13 | 149 | DM4 |
| 12 | DQ8 | 58 | VSS | 104 | VDDQ | 150 | DQ38 |
| 13 | DQ9 | 59 | BA0 | 105 | DQ12 | 151 | DQ39 |
| 14 | DQS1 | 60 | DQ35 | 106 | DQ13 | 152 | VSS |
| 15 | VDDQ | 61 | DQ40 | 107 | DM1 | 153 | DQ44 |
| 16 | CK1 | 62 | VDDQ | 108 | VDD | 154 | /RAS |
| 17 | /CK1 | 63 | /ME | 109 | DQ14 | 155 | DQ45 |
| 18 | VSS | 64 | DQ41 | 110 | DQ15 | 156 | CDDQ |
| 19 | DQ10 | 65 | /CAS | 111 | CKE1 | 157 | /CS0 |
| 20 | DQ11 | 66 | VSS | 112 | CDDQ | 158 | /CS1 |
| 21 | CKE0 | 67 | DQS5 | 113 | *BA2 | 159 | DM5 |
| 22 | VDDQ | 68 | DQ42 | 114 | DQ20 | 160 | VSS |
| 23 | DQ16 | 69 | DQ43 | 115 | *A12 | 161 | DQ46 |
| 24 | DQ17 | 70 | VDD | 116 | VSS | 162 | DQ47 |
| 25 | DQS2 | 71 | NC | 117 | DQ21 | 163 | NC |
| 26 | VSS | 72 | DQ48 | 118 | A11 | 164 | VDDQ |
| 27 | A9 | 73 | DQ49 | 119 | DM2 | 165 | DQ52 |
| 28 | DQ18 | 74 | VSS | 120 | VDD | 166 | DQ53 |
| 29 | A7 | 75 | /CK2 | 121 | DQ22 | 167 | NC |
| 30 | VDDQ | 76 | CK2 | 122 | A8 | 168 | VDD |
| 31 | DQ19 | 77 | VDDQ | 123 | DQ23 | 169 | DM6 |
| 32 | A5 | 78 | DQS6 | 124 | VSS | 170 | DQ54 |
| 33 | DQ24 | 79 | DQ50 | 125 | A6 | 171 | DQ55 |
| 34 | VSS | 80 | DQ51 | 126 | DQ28 | 172 | VDDQ |
| 35 | DQ25 | 81 | VSS | 127 | DQ29 | 173 | NC |
| 36 | DQS3 | 82 | VDDID | 128 | VDDQ | 174 | DQ60 |
| 37 | A4 | 83 | DQ56 | 129 | DM3 | 175 | DQ61 |
| 38 | VDD | 84 | DQ57 | 130 | A3 | 176 | VSS |
| 39 | DQ26 | 85 | VDD | 131 | DQ30 | 177 | DM7 |
| 40 | DQ27 | 86 | DQS7 | 132 | VSS | 178 | DQ62 |
| 41 | A2 | 87 | DQ58 | 133 | DQ31 | 179 | DQ63 |
| 42 | VSS | 88 | DQ59 | 134 | *CB4 | 180 | VDDQ |
| 43 | A1 | 89 | VSS | 135 | *CB5 | 181 | SA0 |
| 44 | *CB0 | 90 | WP | 136 | VDDQ | 182 | SA1 |
| 45 | *CB1 | 91 | SDA | 137 | CK0 | 183 | SA2 |
| 46 | VDD | 92 | SCL | 138 | /CK0 | 184 | VDDSPD |

* These pins are not used in this module.

Kingmax - Memory Module                            2                        March 2002  Rev: 1.1

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

FUNCTIONAL BLOCK DIAGRAM

CLK1

/CLK1

/CS1

/CS0

DQS0

22 x 10

/CLK /CS CLK DQS

120 

/CLK /CS CLK DQS

CLK2

/CLK2

DQS5

22 x 10

/CLK /CS CLK DQS

120 

/CLK /CS CLK DQS

DM0                                DM                                  DM

DM5                                DM                                  DM

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2     U1

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2     U9

DQ3

DQ4

DQ5

DQ6

DQ7

DQ40

DQ41

DQ42

DQ43

DQ44

DQ45

DQ46

DQ47

DQ0

DQ1

DQ2     U6

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U14

DQS1

22 x 10

/CLK /CS CLK DQS

/CLK /CS CLK DQS

DQS6

22 x 10

/CLK /CS CLK DQS

/CLK /CS CLK DQS

DM1                                DM                                  DM

DM6                                DM                                  DM

DQ8

DQ9

DQ10

DQ11

DQ12

DQ13

DQ14

DQ15

DQ0

DQ1

DQ2     U2

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U10

DQ48

DQ49

DQ50

DQ51

DQ52

DQ53

DQ54

DQ55

DQ0

DQ1

DQ2     U7

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U15

DQS2

22 x 10

/CLK /CS CLK DQS

/CLK /CS CLK DQS

DQS7

22 x 10

/CLK /CS CLK DQS

/CLK /CS CLK DQS

DM2                                DM                                  DM

DM7                                DM                                  DM

DQ16

DQ17

DQ18

DQ19

DQ20

DQ21

DQ22

DQ23

DQ0

DQ1

DQ2     U3

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U11

DQ56

DQ57

DQ58

DQ59

DQ60

DQ61

DQ62

DQ63

DQ0

DQ1

DQ2     U8

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U16

CLK0

/CLK0

DQS3

22 x 10

/CLK /CS CLK DQS

/CLK /CS CLK DQS

DQS4

22 x 10

/CLK /CS CLK DQS

120 

/CLK /CS CLK DQS

DM3                                DM                                  DM

DM4                                DM                                  DM

DQ24

DQ25

DQ26

DQ27

DQ28

DQ29

DQ30

DQ31

DQ0

DQ1

DQ2     U4

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U12

DQ32

DQ33

DQ34

DQ35

DQ36

DQ37

DQ38

DQ39

DQ0

DQ1

DQ2     U5

DQ3

DQ4

DQ5

DQ6

DQ7

DQ0

DQ1

DQ2

DQ3

DQ4

DQ5

DQ6

DQ7

U13

A0 ~ A11,BA0,BA1                                SDRAM U1 ~ U16

VDD

VSS

One 0.1uF capacitor per each SDRAM

To SDRAM U1 ~ U16

/RAS, /CAS, /WE CKE0

CKE1

VDDSPD

Serial PD

SDRAM  U1 ~ U16

SDRAM  U1 ~ U8

SDRAM  U9 ~ U16

VREF

One 0.1uF capacitor

To SDRAM U1 ~ U8

VDD WP

SCL SDA

SCL SDA

VSS

per each SDRAM

A0   A1   A2

SA0 SA1 SA2

Kingmax - Memory Module

3                        March 2002   Rev: 1.1

ABSOLUTE MAXIMUM RATIINGS

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Supply voltage relative to VSS | VDD | -1.0 ~ 3.6 | V |
| I/O pins voltage relative to VSS | VIN, VOUT | -0.5 ~ 3.6 | V |
| Operating temperature | TA | 0 ~ +70 | C |
| Storage temperature | TSTG | -55 ~ +125 | C |
| Power dissipation | PD | 16 | W |

Note:  Permanent  device  damage  may  occur  if    “ABSOLUTE   MAXIMUM  RANTINGS”   are exceeded. Functional operation should be restricted to recommended operating condition. Exposure  to higher than recommended  voltage  for extended  periods  of time affect device reliability.

DC OPERATING CONDITIONS AND CHARACTERISTIICS (SSTL_2 In/Out)

Recommended operating conditions (TA  = 0 to 70 C, VDD  = +2.5V ±0.2V)

| Parameter | Symbol | Min | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage | VDD | 2.3 | 2.7 | V |  |
| I/O Reference voltage | VREF | 0.49×VDD | 0.51×VDD | V | 1 |
| I/O Termination voltage (system) | VTT | VREF-0.04 | VREF+0.04 | V | 2 |
| Input logic high voltage | VIH(DC) | VREF+0.15 | VDD+0.3 | V | 3 |
| Input logic low voltage | VIL(DC) | -0.3 | VREF-0.15 | V | 3 |
| Input voltage level, CK and /CK inputs | VIN(DC) | -0.3 | VDD+0.3 | V |  |
| Input differential voltage, CK and /CK inputs | VID(DC) | 0.3 | VDD+0.6 | V | 4 |
| Output high current (VOUT = 1.95V) | IOH | 16.8 | - | mA |  |
| Output low current (VOUT = 0.35V) | IOL | -16.8 | - | mA |  |
| Input leakage current | II | -16 | 16 | µA | 5 |
| Output leakage current | IOZ | -40 | 40 | µA | 6 |

Note  : 1.  VREF   is expected  to  be  equal  to  0.5×VDD   of the  transmitting  devices,  and  must  track variations in the DC level of the same. Peak-to-peak noise on VREF, may not exceed 2% of the DC value.

2.  VTT   is not applied  directly  to the device.  VTT   is a system supply for signal termination resistors, is expected to be set equal to VREF, and must track variations in the DC level of VREF.

3. These parameters should be tested at the pin on actual components. The AC and DC input specifications are relative to a VREF envelop that has been bandwidth limited to 200MHz.

4. VID  is the magnitude of the difference between the input level on CK and the input on /CK.

5. Any input 0V    VIN       VDD, VREF pin 0V    VIN       1.35V, All other pins not under test = 0 V

6. 0V    VOUT       VDD, DQs are disabled.

Kingmax - Memory Module                            4                       March 2002  Rev: 1.1

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

IDD CONDITIONS AND SPECIFICATIONS

Recommended operating conditions unless otherwise noted, TA  = 0 to 70 C, VDD  = +2.5V ±0.2V

| Parameter / Condition | Symbol | Max | Unit | Note |
| --- | --- | --- | --- | --- |
| Operating current - One bank; Active -Precharge; tRC = tRC(Min); tCK = tCK(Min); DQ, DM and DQS inputs changing twice per clock cycle; Address and control inputs changing once per clock cycle | IDD0 | 1080 | mA |  |
| Operating current - One bank; Active -Read-Precharge; Burst = 2; tRC = tRC(Min); tCK = tCK(Min); IOUT = 0 mA; Address and control inputs changing once per clock cycle | IDD1 | 1520 | mA |  |
| Precharge power-down standby current; All banks idle; Power-down Mode; tCK = tCK(Min); CKE = LOW | IDD2P | 520 | mA |  |
| Idle standby current; /CS = HIGH; All banks idle; tCK = tCK(Min); CKE = HIGH; Address and control inputs changing once per clock cycle; VIN = VREF for DQ, DQS and DM | IDD2F | 680 | mA |  |
| Active power-down standby current; One banks active; Power-down Mode; tCK = tCK(Min); CKE = LOW | IDD3P | 560 | mA |  |
| Active standby current; /CS = HIGH; CKE = HIGH; One bank; Active-Precharge; tRC = tRAS(Max); tCK = tCK(Min) ; DQ, DM and DQS inputs changing twice per clock cycle; Address and other control inputs changing once pre clock cycles | IDD3N | 680 | mA |  |
| Operating current; Burst = 2; Reads; Continuous burst; One bank active; Address and control inputs changing once per clock cycle; tCK = tCK(Min); IOUT = 0 mA | IDD4R | 1640 | mA |  |
| Operating current; Burst = 2; Writes; Continuous burst; One bank active; Address and control inputs changing once per clock cycle; tCK = tCK(Min); DQ, DM and DQS inputs changing twice per clock cycle | IDD4W | 1520 | mA |  |
| Auto refresh current; tRC = tRC(Min); distributed refresh | IDD5 | 1960 | mA |  |
| Self refresh current; CKE 0.2V; External clock should be on | IDD6 | 80 | mA |  |
| Operating current; Four bank interleaving READs with BL = 4; Auto Precharge; tRC = tRC(min); tCK = tCK(min); Address and control inputs change only Active READ or WRITE commands | IDD7 | 2560 | mA |  |

Kingmax - Memory Module                            5                        March 2002  Rev: 1.1

MPMB62D-68KX3

256MB PC-2700 DDR  DIMM

AC INPUT OPERATING CONDITIONS

Recommended operating conditions (TA  = 0 to 70 C, VDD  = +2.5V ±0.2V)

| Parameter / Condition | Symbol | Min | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- |
| Input logic high voltage | VIH(AC) | VREF+0.31 | - | V | 1 |
| Input logic low voltage | VIL(AC) | - | VREF-0.31 | V | 1 |
| Input differential voltage, CK and /CK inputs | VID(DC) | 0.7 | VDD+0.6 | V | 2 |
| Input crossing point voltage, CK and /CK inputs | VIX(DC) | 0.5×VDD-0.2 | 0.5×VDD+0.2 | V | 3 |

Note : 1. These parameters should be tested at the pin on actual components. The AC and DC input specific ations are relative to a VREF envelop that has been bandwidth limited to 200MHz.

2. VID  is the magnitude of the difference between the input level on CK and the input on /CK.

3. The value of VIX is expected to be equal 0.5×VDD  of the transmitting device and must track variations in the DC level of the same.

AC OPERATING TEST CONDITIONS (TA  = 0 to 70 C, VDD  = +2.5V ±0.2V)

| Parameter | Value | Unit |
| --- | --- | --- |
| Input reference voltage for Clock | 0.5×VDD | V |
| Input signal maximum peak swing | 1.5 | V |
| Input signal minimum slew rate | 1.0 | V |
| Input Levels(VIH/VIL) | VREF+0.31 / VREF-0.31 | V |
| Input timing measurement reference level | VREF | V |
| Output timing measurement reference level | Vtt | V |
| Output load condition | See Load Circuit |  |

Output Load Circuit (SSTL_2)

Kingmax - Memory Module                            6                        March 2002  Rev: 1.1

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

AC TIMMING PARAMETERS AND SPECIFICATIONS

(These AC characteristics were tested on the component)

| Parameter |  | Symbol | Min | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Row cycle time |  | tRC | 60 | - | ns |  |
| Refresh row cycle time |  | tRFC | 72 | - | ns |  |
| Row active time |  | tRAS | 42 | 70K | ns |  |
| /RAS to /CAS delay |  | tRCD | 18 |  | ns |  |
| Row precharge time |  | tRP | 18 |  | ns | 5 |
| Row active to Row active delay |  | tRRD | 12 |  | ns | 5 |
| Write recovery time |  | tWR | 2.5 |  | tCK |  |
| Last data in to Read command |  | tCDLR | 1 |  | tCK |  |
| Col. address to Col. address delay |  | tCCD | 1 |  | tCK |  |
| Clock cycle time | CL = 2.0 | tCK | 7.5 | 12 | ns | 5 |
|  | CL = 2.5 |  | 6.0 | 12 |  |  |
| Clock high level width |  | tCH | 0.45 | 0.55 | tCK |  |
| Clock low level width |  | tCL | 0.45 | 0.55 | tCK |  |
| DQS-out access time from CK, /CK |  | tDQSCK | -0.6 | +0.6 | ns |  |
| Output data access time from CK, /CK |  | tAC | -0.7 | +0.7 | ns |  |
| Data strobe edge to output data edge |  | tDQSQ | - | +0.35 | ns |  |
| Read Preamble |  | tRPRE | 0.9 | 1.1 | tCK |  |
| Read Post amble |  | tRPST | 0.4 | 0.6 | tCK |  |
| CK to valid DQS-in |  | tDQSS | 0.75 | 1.25 | tCK |  |
| DQS-in setup time |  | tWPRES | 0 |  | ns | 2 |
| DQS-in hold time |  | tWPREH | 0.25 |  | tCK |  |
| DQS falling edge to CK rising-setup time |  | tDSS | 0.2 |  | tCK |  |
| DQS falling edge from CK rising-hold time |  | tDSH | 0.2 |  | tCK |  |
| DQS-in high level width |  | tDQSH | 0.35 |  | tCK |  |
| DQS-in low level width |  | tDQSL | 0.35 |  | tCK |  |
| DQS-in cycle time |  | tDSC | 0.9 | 1.1 | tCK |  |
| Address and Control Input setup time |  | tIS | 0.75 |  | ns | 6 |
| Address and Control Input hold time |  | tIH | 0.75 |  | ns | 6 |
| Data -out high impedance time from CK, /CK |  | tHZ | tACmin - 400ps | tACmax - 400ps | ps |  |
| Data -out low impedance time from CK, /CK |  | tLZ | tACmin - 400ps | tACmax - 400ps | ps |  |
| Input Slew Rate (for input only pins) |  | tSL(I) | 0.5 |  | V/ns | 6 |
| Input Slew Rate (for I/O pins) |  | tSL(IO) | 0.5 |  | V/ns | 7 |
| Output Slew Rate (x8) |  | tSL(O) | 1.0 | 4.5 | V/ns | 10 |
| Output Slew Rate Matching Ratio (rise to fall) |  | tSLMR | 0.67 | 1.5 |  |  |

Kingmax - Memory Module                            7                       March 2002  Rev: 1.1

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

| Parameter | Symbol | Min | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- |
| Mode register set cycle time | tMRD | 12 |  | ns |  |
| DQ & DM setup time to DQS | tDS | 0.45 |  | ns | 7,8,9 |
| DQ & DM hold time to DQS | tDH | 0.45 |  | ns | 7,8,9 |
| DQ & DM input pulse width | tDIPW | 1.75 |  | ns |  |
| Power down exit time | tPDEX | 10 |  | ns |  |
| Exit self refresh to write command | tXSW | 95 |  | ns |  |
| Exit self refresh to bank active command | tXSA | 75 |  | ns | 4 |
| Exit self refresh to read command | tXSR | 200 |  | Cycle |  |
| Refresh interval time | tREF | 15.6 |  | us | 1 |
| Output DQS valid window | tQH | tHPmin - tQHS |  | ns | 5 |
| Clock half period | tHP | tCLmin or tCHmin |  | ns |  |
| Data hold skew factor | tQHS |  | 0.5 | ns |  |
| DQS write post amble time | tWPST | 0.4 | 0.6 | tCK | 3 |

Note : 1. Maximum burst refresh of 8

2. The specific requirement is that DQS be valid (High or Low) on or before this CK edge. The case shown (DQS going from High_Z to logic Low) applies when no writes were previously in progress on the bus. If a previous write was in progress, DQS could be High at this time, depending on tDQSS.

3. The maximum limit for this parameter is not a device limit. The device will operate with a great value for this parameter, but system performance (bus turnaround) will degrade accordingly.

4. A write command can be applie d with tRCD satisfied after this command.

5. For registered DINNs, tCL and tCH are    45% of the period including both the half period jitter (tJIT(HP)) of the PLL and the half period jitter due to cross talk (tJIT(crosstalk)) on the DIMM.

6. Input Setup/Hold Slew Rate Derating

| Input Setup/Hold Slew Rate (V/ns) | tIS (ps) | tIH (ps) |
| --- | --- | --- |
| 0.5 | 0 | 0 |
| 0.4 | +50 | +50 |
| 0.3 | +100 | +100 |

This derating table is used to increase tIS /tIH in the case where the input slew rate is below

0.5V/ns. Input setup/hold slew rate based on the lesser of AC-AC slew rate and DC-DC

slew rate.

Kingmax - Memory Module                            8                       March 2002  Rev: 1.1

7. I/O Setup/Hold Slew Rate Derating

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

| I/O Setup/Hold Slew Rate (V/ns) | tIS (ps) | Tih (ps) |
| --- | --- | --- |
| 0.5 | 0 | 0 |
| 0.4 | +75 | +75 |
| 0.3 | +150 | +150 |

This derating table is used to increase tDS /tDH in the case where the I/O slew rate is below

0.5V/ns. I/O setup/hold slew rate based on the lesser of AC-AC slew rate and DC-DC slew rate.

8. I/O Setup/Hold Plateau Derating

| I/O Input Level (mV) | tIS (ps) | tIH (ps) |
| --- | --- | --- |
| ±280 | +50 | +50 |

This derating table is used to increase tDS /tDH in the case where the input level is flat below VREF ± 310mV for a duration of up to 2ns.

9. I/O Delta Rise/Fall Rate(1/slew-rate) Derating

| Delta Rise/Fall Rate (ns/V) | tIS (ps) | tIH (ps) |
| --- | --- | --- |
| 0 | 0 | 0 |
| ±0.25 | +50 | +50 |
| ±0.5 | +100 | +100 |

This derating table is used to increase  tDS/tDH in the case where the DQ and DQS slew rates differ. The Delta Rise/Fall Rate is collated as 1/SlewRate1-1/SlewRate2. For example, if slew rate 1 = 5V/ns and slew rate 2 =.4V/ns then the Delta Rise/Fall Rate =-0/5ns/V. Input S/H slew rate based on larger of AC-AC delta rise/fall rate and DC-DC delta rise/fall rate.

10. This parameter is fir system simulation purpose. It is guaranteed by design.

Input/Output CAPACITANCE

(VDD  = 2.5V, TA  = 25 C, f = 1MHz, VREF = 1.25V   100mV)

| Pin | Symbol | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Input Capacitance: Command and Address Input Capacitance: /CS0, /CS1 Input Capacitance: CK0, /CK0, CK1, /CK1 Input Capacitance: CKE0, CKE1 Input/Output Capacitance: DQs, DQS | CIN1 CIN2 CIN3 CIN4 CI/O | 65 42 42 27 10 | 81 50 50 34 13 | pF pF pF pF pF |

Kingmax - Memory Module                             9                       March 2002  Rev: 1.1

COMMAND TRUTH TABLE

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

| Command |  |  | CKEn-1 | CKEn | /CS | /RAS | /CAS | /WE | BA0,1 | A10/AP | A11,A9~A0 | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Register | Extended MRS |  | H | X | L | L | L | L | OP code |  |  | 1, 2 |
| Register | Mode Register Set |  | H | X | L | L | L | L | OP code |  |  | 1, 2 |
| Refresh | Auto Refresh |  | H | H | L | L | L | H | X |  |  | 3 |
|  | Self Refresh | Entry |  | L |  |  |  |  |  |  |  | 3 |
|  |  | Exit | L | H | L | H | H | H | X |  |  | 3 |
|  |  |  |  |  | H | X | X | X |  |  |  | 3 |
| Bank Active & Row Address |  |  | H | X | L | L | H | H | V | Row address |  |  |
| Read & Column Address | Auto Precharge Disable |  | H | X | L | H | L | H | V | L | Column address (A0~A9) | 4 |
|  | Auto Precharge Enable |  |  |  |  |  |  |  |  | H |  | 4 |
| Write & Column Address | Auto Precharge Disable |  | H | X | L | H | L | L | V | L | Column address (A0~A9) | 4 |
|  | Auto Precharge Enable |  |  |  |  |  |  |  |  | H |  | 4, 6 |
| Burst Stop |  |  | H | X | L | H | H | L | X |  |  | 7 |
| Precharge | Bank Selection |  | H | X | L | L | H | L | V | L | X |  |
|  | All Banks |  |  |  |  |  |  |  | X | H |  | 5 |
| Active Power Down |  | Entry | H | L | H | X | X | X | X |  |  |  |
|  |  |  |  |  | L | V | V | V |  |  |  |  |
|  |  | Exit | L | H | X | X | X | X |  |  |  |  |
| Precharge Power Down Mode |  | Entry | H | L | H | X | X | X | X |  |  |  |
|  |  |  |  |  | L | H | H | H |  |  |  |  |
|  |  | Exit | L | H | H | X | X | X |  |  |  |  |
|  |  |  |  |  | L | V | V | V |  |  |  |  |
| DM |  |  | H | X |  |  |  |  | X |  |  | 8 |
| No operation (NOP) : Not defined |  |  | H | X | H | X | X | X | X |  |  | 9 |
|  |  |  |  |  | L | H | H | H |  |  |  | 9 |

(V = Valid, X = Don’t Care, H = Logic High, L = Logic Low) Note : 1. OP Code : Operand Code.  A0 ~ A11 & BA0 ~ BA1 : Program keys. (@EMRS/MRS)

2. EMRS/ MRS can be issued only at all banks precharge state.

A new command can be issued 2 clock cycles after EMRS or MRS.

3. Auto refresh functions are same as the CBR refresh of DRAM.

The automatically precharge without row precharge command is meant by "Auto". Auto/self refresh can be issued only at all banks precharge state.

4. BA0 ~ BA1 : Bank select addresses.

If both BA0 and BA1 are "Low" at read, write, row active and precharge, bank A is selected.

If both BA0 is "High" and BA1 is "Low" at read, write, row active and precharge, bank B is selected. If both BA0 is "Low" and BA1 is "High" at read, write, row active and precharge, bank C is selected. If both BA0 and BA1 are "High" at read, write, row active and precharge, bank D is selected.

5. If A10/AP is "High" at row precharge, BA0 and BA1 are ignored and all banks are selected.

6. During burst write with auto precharge, new read/write command can not be issued. Another bank read/write command can be issued after the end of burst.

New row active of the associated bank can be issued at tRP after the end of burst.

7. Burst stop command is valid at every burst length.

8. DM sampled at the rising and falling edges of the DQS and Data-in are masked at the both edges

(Write DM latency is 0).

9. This combination is not defined for any function, which means "No Operation (NOP)" in DDR SDRAM.

Kingmax - Memory Module                           10                       March 2002  Rev: 1.1

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

PACKAGE DIMENSIONS

Units: Millimeter

1.00

2.20

3.00

133.35

131.35

128.95

127.35

B

1.00

2.20

3.00

30.00

17.80

10.00                1

U5           U6           U7           U8

53                                                         92

4.0 ±0.1

4.0 ±0.1

2.30

| 6.35 64.77 U1 | U2 | U3 | U4 |
| --- | --- | --- | --- |

77.47

1.27 ±0.05

A

49.53

B'

6.35

73.295                                                                60.055

Ø2.5 ±0.1

| 93 U9 | U10 | U11 | U12 |
| --- | --- | --- | --- |

R2.0

| 145 U13 | U14 | U15 | U16 |
| --- | --- | --- | --- |

184

Ø2.5 ±0.1

R2.0

R2.0

R2.0

3.81 Max.

6.35

2.175

R 0.9

1.8 ±0.1       1.0 ±0.05

2.90

2.50

0.20

4.0 Min.                 4.0 Min.

Detail A

1.27 ±0.1

Section B-B'

Tolerance : ±0.15 unless otherwise specified

The used device is 16Mx8 SDRAM TinyBGA

Kingmax - Memory Module

11                      March 2002   Rev: 1.1

SERIAL PRESENCE DETECT INFORMATION

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

| Byte # | Function described | Function Supported | Hex value |
| --- | --- | --- | --- |
| 0 | # of bytes written into serial memory at module manufacturer | 128 bytes | 80h |
| 1 | Total # of bytes of SPD memory device | 256 bytes (2K-bit) | 08h |
| 2 | Fundamental memory type | SDRAM DDR | 07h |
| 3 | # of row address on this assembly | 12 | 0Ch |
| 4 | # of column address on this assembly | 10 | 0Ah |
| 5 | # of module Rows on this assembly | 2 Rows | 02h |
| 6 | Data width of this assembly | 64 bits | 40h |
| 7 | Data width of this assembly | - | 00h |
| 8 | VDDQ and interface standard of this assembly | SSTL 2.5V | 04h |
| 9 | DDR SDRAM cycle time from clock @CAS latency of 2.5 | 6.0ns | 60h |
| 10 | DDR SDRAM access time from clock @CAS latency of 2.5 | ± 0.7ns | 70h |
| 11 | DIMM configuration type (Non-parity , Parity , ECC ) | Non-parity , ECC | 00h |
| 12 | Refresh rate & type | 15.6us, support self refresh | 80h |
| 13 | Primary DDR SDRAM width | 8 | 08h |
| 14 | Error checking DDR SDRAM data width | N/A | 00h |
| 15 | Minimum clock delay for back-to-back random column | tCCD=1CLK | 01h |
| 16 | DDR SDRAM device attributes: Burst lengths supported | 2 , 4 , 8 | 0Eh |
| 17 | DDR SDRAM device attributes: # of banks on each DDR | 4 banks | 04h |
| 18 | DDR SDRAM device attributes: CAS Latency supported | 2 &2.5 | 0Ch |
| 19 | DDR SDRAM device attributes: CS Latency | 0 CLK | 01h |
| 20 | DDR SDRAM device attributes: WE Latency | 1 CLK | 02h |
| 21 | DDR SDRAM module attributes | Registered address & Control inputs and On-card DLL | 20h |
| 22 | DDR SDRAM device attributes: General | +/- 0.2 voltage tolerance | 00h |
| 23 | DDR SDRAM cycle time @CAS latency of 2 | 7.5ns | 75h |
| 24 | DDR SDRAM access time @CAS latency of 2 | ± 0.7ns | 70h |
| 25 | SDRAM cycle time @CAS latency of 1.5 | - | 00h |
| 26 | SDRAM access time @CAS latency of 1.5 | - | 00h |
| 27 | Minimum row precharge time (=tRP) | 18ns | 48h |
| 28 | Minimum row active to row active delay (tRRD) | 12ns | 30h |
| 29 | Minimum RAS to CAS delay (=tRCD) | 18ns | 48h |
| 30 | Minimum activate precharge time (=tRAS) | 42ns | 2Ah |
| 31 | Module Row density | 128MB | 20h |
| 32 | Command and Address signal input setup time | 0.75ns | 75h |
| 33 | Command and Address signal input hold time | 0.75ns | 75h |
| 34 | Data signal input setup time | 0.45ns | 45h |
| 35 | Data signal input hold time | 0.45ns | 45h |
| 36~61 | Superset information (maybe used in future) | - | 00h |
| 62 | SPD data revision code | Initial release | 00h |
| 63 | Checksum for bytes 0 ~ 62 |  | E8h |

Kingmax - Memory Module                           12                       March 2002   Rev: 1.1

SERIAL PRESENCE DETECT INFORMATION

MPMB62D-68KX3

256MB PC-2700 DDR DIMM

| 64 | Manufacturer JEDEC ID code | kingmax | 7Fh |
| --- | --- | --- | --- |
| 65 | Manufacturer JEDEC ID code | kingmax | 7Fh |
| 66 | Manufacturer JEDEC ID code | kingmax | 7Fh |
| 67 | Manufacturer JEDEC ID code | kingmax | 25h |
| 68~71 | Manufacturer JEDEC ID code | - | 00h |
| 72 | Manufacturing location | Hsin Chu (A) | 41h |
| 73 | Manufacturer part # (Memory module) | M | 4Dh |
| 74 | Manufacturer part # (184 pin DIMM) | P | 50h |
| 75 | Manufacturer part # (DDR 333) | M | 4Dh |
| 76 | Manufacturer part # (Module density: 256MB) | B | 42h |
| 77 | Manufacturer part # (Module density: 256MB) | 6 | 36h |
| 78 | Manufacturer part # (Vdd = 2.5V) | 2 | 32h |
| 79 | Manufacturer part # (DDR SDRAM) | D | 44h |
| 80 | Manufacturer part # | “-“ | 2Dh |
| 81 | Manufacturer part # (DDR SDRAM type: 16Mb 8) | 6 | 36h |
| 82 | Manufacturer part # (DDR SDRAM type: 16Mb 8) | 8 | 38h |
| 83 | Manufacturer part # (Kingmax logo) | K | 4Bh |
| 84 | Manufacturer part # (Kingmax logo) | X | 58h |
| 85 | Manufacturer part # (CL=2.5) | 3 | 33h |
| 86 | Manufacturer part # | “-“ | 2Dh |
| 87 | Manufacturer part # (option for die source) | * | * |
| 88 | Manufacturer part # (option for module version) | * | * |
| 89 | Manufacturer part # (option for die version) | * | * |
| 90 | Manufacturer part # (reserve) | - | 00h |
| 91 | Manufacturer revision code (reserve) | - | 00h |
| 92 | Manufacturer revision code (reserve) | - | 00h |
| 93 | Manufacturing date (Work Year) - BCD | 2002 | 02h |
| 94 | Manufacturing date (Work Week) - BCD (reserve) | - | 00h |
| 95~98 | Assemble serial # -BCD (reserve) | - | 00h |
| 99~125 | Manufacturer specific data (may be used in future) | - | 00h |
| 126 | Intel specification for frequency | Undefined | 00h |
| 127 | Intel specification details for 100MHz support | Undefined | 00h |
| 128+ | Unused storage locations | - | FFh |

Kingmax - Memory Module                           13                       March 2002  Rev: 1.1