PRELIMINARY

SMART 3 ADVANCED BOOT BLOCK BYTE-WIDE

E

8-MBIT (1024K x 8), 16-MBIT (2056K x 8) FLASH MEMORY FAMILY

28F008B3, 28F016B3

n  Flexible SmartVoltage Technology

2.7V–3.6V Program/Erase

2.7V–3.6V Read Operation

12V VPP Fast Production

Programming

n  2.7V or 1.8V I/O Option

Reduces Overall System Power

n  Optimized Block Sizes

Eight 8-Kbyte Blocks for Data, Top or Bottom Locations

Up to Thirty-One 64-Kbyte Blocks

for Code

n  High Performance

2.7V–3.6V: 120 ns Max Access Time

n  Block Locking

VCC-Level Control through WP#

n  Low Power Consumption

20 mA Maximum Read Current

n  Absolute Hardware-Protection

VPP = GND Option

VCC Lockout Voltage

n  Extended Temperature Operation

–40°C to +85°C

n  Supports Code plus Data Storage Optimized for FDI, Flash Data Integrator Software

Fast Program Suspend Capability

Fast Erase Suspend Capability

n  Extended Cycling Capability

10,000 Block Erase Cycles

n  Automated Byte Program and Block

Erase

Command User Interface

Status Registers

n  SRAM-Compatible Write Interface n  Automatic Power Savings Feature n  Reset/Deep Power-Down

1 µA ICCTypical

Spurious Write Lockout

n  Standard Surface Mount Packaging

48-Ball   BGA* Package

40-Lead TSOP Package

n  Footprint Upgradeable

Upgradeable from 2-, 4- and 8-Mbit

Boot Block

n  ETOX™ V (0.4     Flash Technology

n  x8-Only Input/Output Architecture For Space-Constrained 8-bit Applications

The new Smart 3 Advanced Boot Block, manufactured on Intel’s latest 0.4µ technology, represents a feature- rich solution at overall lower system cost. Smart 3 flash memory devices incorporate low voltage capability (2.7V read, program and erase) with high-speed, low-power operation. Several new features have been added, including the ability to drive the I/O at 1.8V, which significantly reduces system active power and interfaces to 1.8V controllers. A new blocking scheme enables code and data storage within a single device. Add to this the Intel-developed Flash Data Integrator (FDI) software and you have the most cost-effective, monolithic code plus data storage solution on the market today. Smart 3 Advanced Boot Block Byte-Wide products will be available in 40-lead TSOP and 48-ball µBGA* packages. Additional information on this product family can be obtained by accessing Intel’s WWW page: http://www.intel.com/design/flcomp

May 1997                                                                                                                                                                     Order Number: 290605-001

Information in this document is provided in connection with Intel products. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel’s Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications.

Intel may make changes to specifications and product descriptions at any time, without notice.

The 28F008B3 and 28F016B3 may contain design defects or errors known as errata which may cause the product to deviate from published specifications. Current characterized errata are available on request.

*Third-party brands and names are the property of their respective owners.

Contact your local Intel sales office or your distributor to obtain the latest specifications and before placing your product order. Copies of documents which have an ordering number and are referenced in this document, or other Intel literature, may be

obtained from:

Intel Corporation

P.O. Box 7641

Mt. Prospect, IL 60056-7641

or call 1-800-879-4683

or visit Intel’s website at http:\\www.intel.com

COPYRIGHT © INTEL CORPORATION 1996, 1997                                                                                                                                    CG-041493

*Third-party brands and names are the property of their respective owners.

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

CONTENTS

PAGE                                                                                  PAGE

3.5 Power Consumption ...................................26

1.0 INTRODUCTION .............................................5

1.1 Smart 3 Advanced Boot Block Flash

Memory Enhancements ..............................5

1.2 Product Overview.........................................6

2.0 PRODUCT DESCRIPTION ..............................6

2.1 Package Pinouts ..........................................7

2.2 Block Organization ..................................... 11

2.2.1 Parameter Blocks ................................ 11

2.2.2 Main Blocks ......................................... 11

3.0 PRINCIPLES OF OPERATION ..................... 14

3.1 Bus Operation ............................................ 14

3.1.1 Read.................................................... 15

3.1.2 Output Disable..................................... 15

3.1.3 Standby ............................................... 15

3.1.4 Deep Power-Down/Reset .................... 15

3.1.5 Write.................................................... 15

3.2 Modes of Operation.................................... 15

3.2.1 Read Array .......................................... 16

3.2.2 Read Intelligent Identifier ..................... 17

3.2.3 Read Status Register .......................... 17

3.2.4 Program Mode..................................... 18

3.2.5 Erase Mode ......................................... 19

3.3 Block Locking ............................................. 26

3.3.1 VPP = VIL for Complete Protection ....... 26

3.3.2 WP# = VIL for Block Locking ................ 26

3.3.3 WP# = VIH for Block Unlocking ............ 26

3.4 VPP Program and Erase Voltages .............. 26

3.5.1 Active Power .......................................26

3.5.2 Automatic Power Savings (APS) .........27

3.5.3 Standby Power ....................................27

3.5.4 Deep Power-Down Mode.....................27

3.6 Power-Up/Down Operation.........................27

3.6.1 RP# Connected to System Reset ........27

3.6.2 VCC, VPP and RP# Transitions .............27

3.7 Power Supply Decoupling ..........................28

3.7.1 VPP Trace on Printed Circuit Boards ....28

4.0 ABSOLUTE MAXIMUM RATINGS ................29

5.0 OPERATING CONDITIONS

(VCCQ = 2.7V–3.6V).......................................29

5.1 DC Characteristics: VCCQ = 2.7V–3.6V.......30

6.0 OPERATING CONDITIONS

(VCCQ = 1.8V–2.2V).......................................34

6.1 DC Characteristics: VCCQ = 1.8V–2.2V.......34

7.0 AC CHARACTERISTICS ...............................39

7.1 Reset Operations .......................................43

APPENDIX A: Ordering Information .................45

APPENDIX B: Write State Machine Current/

Next States ..................................................46

APPENDIX C: Access Time vs.

Capacitive Load...........................................47

APPENDIX D: Architecture Block Diagram ......48

APPENDIX E: Additional Information ...............49

PRELIMINARY                                                                                       3

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

REVISION HISTORY

| Number | Description |
| --- | --- |
| -001 | Original version |

4                                                                                                                                   PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

1.0     INTRODUCTION

This preliminary datasheet contains the specifications for the Advanced Boot Block flash memory family, which is optimized for low power, portable systems. This family of products features

1.8V–2.2V or 2.V–3.6V I/Os and a low VCC/VPP operating range of 2.7V–3.6V for read and program/erase operations. In addition this family is capable of fast programming at 12V. Throughout

this document, the term “2.7V” refers to the full voltage range 2.7V–3.6V (except where noted otherwise) and “VPP  = 12V” refers to 12V ±5%. Section 1 and 2 provides an overview of the flash memory family including applications, pinouts and pin descriptions. Section 3 describes the memory organization and operation for these products. Finally,  Sections  4,  5,  6  and  7  contain  the operating specifications.

1.1       Smart 3 Advanced Boot Block

Flash Memory Enhancements

The new 8-Mbit and 16-Mbit Smart 3 Advanced Boot Block flash memory provides a convenient upgrade from and/or compatibility to previous 4- Mbit and 8-Mbit Boot Block products. The Smart 3 product functions are similar to lower density products in both command sets and operation, providing similar pinouts to ease density upgrades.

The Smart 3 Advanced Boot Block flash memory features

Enhanced blocking for easy segmentation of code and data or additional design flexibility

Program Suspend command which permits program suspend to read

WP# pin to lock and unlock the upper two (or lower two, depending on location) 8-Kbyte blocks

VCCQ input for 1.8V–2.2V on all I/Os. See Figures 1–3 for pinout diagrams and VCCQ location

Maximum program time specification for improved data storage.

Table 1. Smart 3 Advanced Boot Block Feature Summary

| Feature | 28F016B3/28F008B3/28F004B3 | Reference |
| --- | --- | --- |
| VCC Read Voltage | 2.7V– 3.6V | Table 9, Table 12 |
| VCCQ I/O Voltage | 1.8V–2.2V or 2.7V– 3.6V | Table 9, Table 12 |
| VPP Program/Erase Voltage | 2.7V– 3.6V or 11.4V– 12.6V | Table 9, Table 12 |
| Bus Width | 8 bits | Table 2 |
| Speed | 120 ns | Table 15 |
| Memory Arrangement | 1 Mbit x 8 (8 Mbit), 2 Mbit x 8 (16 Mbit) |  |
| Blocking (top or bottom) | Eight 8-Kbyte parameter blocks (8/16 Mbit) & Fifteen 64-Kbyte blocks (8 Mbit) Thirty-one 64-Kbyte main blocks (16 Mbit) | Section 2.2 Figures 4 and 5 |
| Locking | WP# locks/unlocks parameter blocks All other blocks protected using VPP switch | Section 3.3 Table 8 |
| Operating Temperature | Extended: –40 C to +85 C | Table 9, Table 12 |
| Program/Erase Cycling | 10,000 cycles | Table 9, Table 12 |
| Packages | 40-Lead TSOP, 48-Ball BGA* CSP | Figures 1, 2, and 3 |

PRELIMINARY                                                                                       5

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

1.2       Product Overview

Intel provides the most flexible voltage solution in the flash industry, providing three discrete voltage supply pins: VCC for read operation, VCCQ for output swing, and VPP  for program and erase operation. Discrete supply pins allow system designers to use the optimal voltage levels for their design. All Smart

3 Advanced Boot Block flash memory products provide  program/erase  capability  at  2.7V  or  12V and read with VCC  at 2.7V. Since many designs read from the flash memory a large percentage of the  time,  2.7V  VCC  operation  can  provide substantial power savings. The 12V VPP option maximizes program and erase performance during production programming.

The Smart 3 Advanced Boot Block flash memory products are high-performance devices with low power  operation.  The  available  densities  for  the byte-wide devices (x8) are

a.   8-Mbit     (8,388,608-bit)     flash     memory organized as 1 Mbyte of 8 bits each

b.   16-Mbit    (16,777,216-bit)    flash    memory organized as 2 Mbytes of 8 bits each.

For  word-wide  devices  (x16)  see  the  Smart  3

Advanced  Boot  Block  Word-Wide  Flash  Memory

Family datasheet.

The parameter blocks are located at either the top (denoted by -T suffix) or the bottom (-B suffix) of the address map in order to accommodate different microprocessor protocols for kernel code location. The upper two (or lower two) parameter blocks can be locked to provide complete code security for system initialization code. Locking and unlocking is controlled by WP# (see Section 3.3 for details).

The Command User Interface (CUI) serves as the interface between the microprocessor or microcontroller  and  the  internal  operation  of  the flash memory. The internal Write State Machine (WSM) automatically executes the algorithms and timings  necessary  for  program  and  erase operations, including verification, thereby un- burdening  the  microprocessor  or  microcontroller. The status register indicates the status of the WSM by   signifying   block   erase   or   byte   program completion and status.

Program and erase automation allows program and erase operations to be executed using an industry- standard two-write command sequence to the CUI. Data writes are performed in byte increments. Each byte in the flash memory can be programmed independently of other memory locations; every erase operation erases all locations within a block simultaneously. Program suspend allows system software to suspend the program command in order to read from any other block. Erase suspend allows system software to suspend the block erase command in order to read from or program data to any other block.

The Smart 3 Advanced Boot Block flash memory is also designed with an Automatic Power Savings (APS)  feature  which  minimizes  system  current drain,  allowing  for  very  low  power  designs.  This mode is entered immediately following the completion of a read cycle.

When the CE# and RP# pins are at VCC, the ICC CMOS standby mode is enabled. A deep power- down  mode  is  enabled  when  the  RP#  pin  is  at GND, minimizing power consumption and providing write protection. ICC current in deep power-down is

1 µA typical (2.7V VCC). A minimum reset time of tPHQV   is  required  from  RP#  switching  high  until

outputs  are  valid  to  read  attempts.  With  RP#  at

GND, the WSM is reset and Status Register is cleared. Section 3.5 contains additional information on using the deep power-down feature, along with other power consumption issues.

The RP# pin provides additional protection against unwanted command writes that may occur during system reset and power-up/down sequences due to invalid system bus conditions (see Section 3.6).

Refer to the DC Characteristics Table, Sections 5.1 and 6.1, for complete current and voltage specifications.  Refer  to  the  AC  Characteristics Table, Section 7.0, for read, program and erase performance specifications.

2.0     PRODUCT DESCRIPTION

This section explains device pin description and package pinouts.

6                                                                                                                                   PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

2.1       Package Pinouts

The Smart 3 Advanced Boot Block flash memory is available in 40-lead TSOP (see Figure 1) and 48- ball    BGA  packages  (see  Figures  2  and  3).  In Figure 1, pin changes from one density to the next are circled. Both packages, 40-lead TSOP and 48- ball   BGA package, are 8-bits wide and fully upgradeable across product densities (from 8 Mb to

16 Mb).

28F016

28F008

28F008    28F016

A16

A15

A14

A16                                       1

A15                                       2

A14                                       3

40                       A17

39                       GND

38                       NC

A17

GND A20

A13

A13                                       4

37                       A19                   A19

A12

A11

A9

A8

WE# RP# VPP WP#

A18

A7

A6

A12                                       5

A11                                       6

A9                                         7

A8                                         8

WE#                       9

RP#                     10

VPP                                    11

WP#                     12

A18                                    13

A7                                      14

A6                                      15

Advanced Boot Block

40-Lead TSOP

10 mm x  20 mm

TOP VIEW

36                       A10

35                       DQ7

34                       DQ6

33                       DQ5

32                       DQ4

31                       VCCQ

30                       VCC

29                       NC

28                       DQ3

27                       DQ2

26                       DQ1

A10

DQ7

DQ6

DQ5

DQ4

VCCQ

VCC

NC DQ3

DQ2

DQ1

A5                     A5                                      16

25                       DQ0                DQ0

A4                     A4                                      17

A3                     A3                                      18

A2                     A2                                      19

A1                     A1                                      20

24                       OE#

23                       GND

22                       CE#

21                       A0

OE#

GND CE#

A0

0605-01

Figure 1. 40-Lead TSOP Package

PRELIMINARY                                                                                       7

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

1            2            3          4             5           6            7             8

A           A14                    A12                     A8                     VPP                  WP#            NC              A7                      A4

B           A                A               WE#           RP#            A                A                 A                A

15                        10                                                                                 19                        18                         5                          2

C           A16                    A13                     A9                                                                               A6                      A3                      A1

D           A17                    NC              D5                     NC              D2                     NC            CE#             A0

E          V                 A                 D               NC              D               NC              D              GND

CCQ                      11                         6                                                      3                                                      0

F          GND            D7                     NC              D4                    VCC                    NC              D1                    OE#

NOTE:

0605-03

Dotted connections indicate placeholders where there is no solder ball. These connections are reserved for future upgrades. Routing is not recommended in this area.

Figure 2. 8-Mbit 48-Ball   BGA* Chip Size Package

8                                                                                                                                   PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

1            2            3          4             5           6            7             8

A           A14                    A12                     A8                     VPP                  WP#            A20                     A7                      A4

B           A                A               WE#           RP#            A                A                 A                A

15                        10                                                                                 19                        18                         5                          2

C           A16                    A13                     A9                                                                               A6                      A3                      A1

D           A17                    NC              D5                     NC              D2                     NC            CE#             A0

E          V                 A                 D               NC              D               NC              D              GND

CCQ                      11                         6                                                      3                                                      0

F          GND            D7                     NC              D4                    VCC                    NC              D1                    OE#

NOTE:

0605-02

Dotted connections indicate placeholders where there is no solder ball. These connections are reserved for future upgrades. Routing is not recommended in this area.

Figure 3. 16-Mbit 48-Ball   BGA* Chip Size Package

PRELIMINARY                                                                                       9

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

The pin descriptions table details the usage of each device pin.

Table 2. 16-Mbit Smart 3 Advanced Boot Block Pin Descriptions

| Symbol | Type | Name and Function |
| --- | --- | --- |
| A0–A20 | INPUT | ADDRESS INPUTS for memory addresses. Addresses are internally latched during a program or erase cycle. 28F008B3: A[0-19], 28F016B3: A[0-20] |
| DQ0–DQ7 | INPUT/OUTPUT | DATA INPUTS/OUTPUTS: Inputs array data on the second CE# and WE# cycle during a Program command. Inputs commands to the Command User Interface when CE# and WE# are active. Data is internally latched. Outputs array, Intelligent Identifier and Status Register data. The data pins float to tri-state when the chip is de-selected or the outputs are disabled. |
| CE# | INPUT | CHIP ENABLE: Activates the internal control logic, input buffers, decoders and sense amplifiers. CE# is active low. CE# high de-selects the memory device and reduces power consumption to standby levels. If CE# and RP# are high, but not at a CMOS high level, the standby current will increase due to current flow through the CE# and RP# inputs. |
| OE# | INPUT | OUTPUT ENABLE: Enables the device’s outputs through the data buffers during an array or status register read. OE# is active low. |
| WE# | INPUT | WRITE ENABLE: Controls writes to the Command Register and memory array. WE# is active low. Addresses and data are latched on the rising edge of the second WE# pulse. |
| RP# | INPUT | RESET/DEEP POWER-DOWN: Uses two voltage levels (VIL, VIH) to control reset/deep power-down mode. When RP# is at logic low, the device is in reset/deep power-down mode, which drives the outputs to High-Z, resets the Write State Machine, and draws minimum current. When RP# is at logic high, the device is in standard operation. When RP# transitions from logic-low to logic-high, the device defaults to the read array mode. |
| WP# | INPUT | WRITE PROTECT: Provides a method for locking and unlocking the two lockable parameter blocks. When WP# is at logic low, the lockable blocks are locked, preventing program and erase operations to those blocks. If a program or erase operation is attempted on a locked block, SR.1 and either SR.4 [program] or SR.5 [erase] will be set to indicate the operation failed. When WP# is at logic high, the lockable blocks are unlocked and can be programmed or erased. See Section 3.3 for details on write protection. |

10                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 2. 16-Mbit Smart 3 Advanced Boot Block Pin Descriptions (Continued)

| Symbol | Type | Name and Function |
| --- | --- | --- |
| VCCQ | INPUT | OUTPUT VCC: Enables all outputs to be driven to 2.0V ±10% while the VCC is at 2.7V. When this mode is used, the V CC should be regulated to 2.7V–2.85V to achieve lowest power operation (see Section 6.1: DC Characteristics: VCCQ = 1.8V–2.2V). This input may be tied directly to VCC (2.7V–3.6V). See the DC Characteristics for further details. |
| VCC |  | DEVICE POWER SUPPLY: 2.7V–3.6V |
| VPP |  | PROGRAM/ERASE POWER SUPPLY: For erasing memory array blocks or programming data in each block, a voltage of either 2.7V–3.6V or 12V 5% must be applied to this pin. When VPP < VPPLK all blocks are locked and protected against Program and Erase commands. Applying 11.4V–12.6V to VPP can only be done for a maximum of 1000 cycles on the main blocks and 2500 cycles on the parameter blocks. VPP may be connected to 12V for a total of 80 hours maximum (see Section 3.4 for details). |
| GND |  | GROUND: For all internal circuitry. All ground inputs must be connected. |
| NC |  | NO CONNECT: Pin may be driven or left floating. |

2.2       Block Organization

The Smart 3 Advanced Boot Block is an asymmetrically-blocked architecture that enables system integration of code and data within a single flash device. Each block can be erased independently of the others up to 10,000 times. For the  address  locations  of  each  block,  see  the memory maps in Figure 4 (top boot blocking) and Figure 5 (bottom boot blocking).

2.2.1         PARAMETER BLOCKS

The Smart 3 Advanced Boot Block flash memory architecture includes parameter blocks to facilitate storage  of  frequently  updated  small  parameters (e.g., data that would normally be stored in an EEPROM. By using software techniques, the byte- rewrite functionality of EEPROMs can be emulated. Each 8-/16-Mbit device contains eight parameter blocks of 8 Kbytes (8,192-bytes) each.

2.2.2         MAIN BLOCKS

After the parameter blocks, the remainder of the array is divided into equal size main blocks for data or  code  storage.  Each  16-Mbit  device  contains thirty-one   64-Kbyte   (65,536-byte)   blocks.   Each

8-Mbit device contains fifteen 64-Kbyte blocks.

PRELIMINARY                                                                                     11

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

1FFFFF

1FE000

1FDFFF

1FC000

1FBFFF

1FA000

1F9FFF

1F8000

1F7FFF

1F6000

1F5FFF

1F4000

1F3FFF

1F2000

1F1FFF

1F0000

1EFFFF

1E0000

1DFFFF

1D0000

1CFFFF

1C0000

1BFFFF

1B0000

1AFFFF

1A0000

19FFFF

190000

18FFFF

180000

17FFFF

170000

16FFFF

160000

15FFFF

150000

14FFFF

140000

13FFFF

130000

12FFFF

120000

11FFFF

110000

10FFFF

100000

0FFFFF

0F0000

0EFFFF

0E0000

0DFFFF

0D0000

0CFFFF

0C0000

0BFFFF

0B0000

0AFFFF

0A0000

09FFFF

090000

08FFFF

080000

07FFFF

070000

06FFFF

060000

05FFFF

050000

04FFFF

040000

03FFFF

030000

02FFFF

020000

01FFFF

010000

00FFFF

000000

16-Mbit Advanced Boot

Block

31

30

28

26

15 8-Kbyte Block 38

8-Kbyte Block 37

8-Kbyte Block 36

8-Kbyte Block 35

8-Kbyte Block 34

8-Kbyte Block 33

8-Kbyte Block 32

8-Kbyte Block

64-Kbyte Block

64-Kbyte Block 29

64-Kbyte Block

64-Kbyte Block 27

64-Kbyte Block

64-Kbyte Block 25

64-Kbyte Block 24

64-Kbyte Block 23

64-Kbyte Block 22

64-Kbyte Block 21

64-Kbyte Block 20

64-Kbyte Block 19

64-Kbyte Block 18

64-Kbyte Block 17

64-Kbyte Block 16

64-Kbyte Block

64-Kbyte Block 14

64-Kbyte Block 13

64-Kbyte Block 12

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block 5

64-Kbyte Block

64-Kbyte Block 3

64-Kbyte Block 2

64-Kbyte Block 0

64-Kbyte Block 0

11

10

9

8

7

6

4

FFFFF FE000

FDFFF

FC000

FBFFF

FA000

F9FFF

F8000

F7FFF

F6000

F5FFF

F4000

F3FFF

F2000

F1FFF

F0000

EFFFF

E0000

DFFFF

D0000

CFFFF

C0000

BFFFF

B0000

AFFFF

A0000

9FFFF

90000

8FFFF

80000

7FFFF

70000

6FFFF

60000

5FFFF

50000

4FFFF

40000

3FFFF

30000

2FFFF

20000

1FFFF

10000

0FFFF

00000

8-Mbit Advanced Boot

Block

15

14

12

10 8-Kbyte Block 22

8-Kbyte Block 21

8-Kbyte Block 20

8-Kbyte Block 19

8-Kbyte Block 18

8-Kbyte Block 17

8-Kbyte Block 16

8-Kbyte Block

64-Kbyte Block

64-Kbyte Block 13

64-Kbyte Block

64-Kbyte Block 11

64-Kbyte Block

64-Kbyte Block 9

64-Kbyte Block

64-Kbyte Block 7

64-Kbyte Block

64-Kbyte Block 5

64-Kbyte Block 4

64-Kbyte Block 3

64-Kbyte Block 2

64-Kbyte Block 0

64-Kbyte Block 0

8

6

0605-05

Figure 4. 8-/16-Mbit Advanced Boot Block Byte-Wide Top Boot Memory Maps

12                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

1FFFFF

1F0000

1EFFFF

1E0000

1DFFFF

1D0000

1CFFFF

1C0000

1BFFFF

1B0000

1AFFFF

1A0000

19FFFF

190000

18FFFF

180000

17FFFF

170000

16FFFF

160000

15FFFF

150000

14FFFF

140000

13FFFF

130000

12FFFF

120000

11FFFF

110000

10FFFF

100000

0FFFFF

0F0000

0EFFFF

0E0000

0DFFFF

0D0000

0CFFFF

0C0000

0BFFFF

0B0000

0AFFFF

0A0000

09FFFF

090000

08FFFF

080000

07FFFF

070000

06FFFF

060000

05FFFF

050000

04FFFF

040000

03FFFF

030000

02FFFF

020000

01FFFF

010000

00FFFF

00E000

00DFFF

00C000

00BFFF

00A000

009FFF

008000

007FFF

006000

005FFF

004000

003FFF

002000

001FFF

000000

16-Mbit Advanced Boot

Block

31

30

27

26

21

20

19 64-Kbyte Block 38

64-Kbyte Block 37

64-Kbyte Block 36

64-Kbyte Block 35

64-Kbyte Block 34

64-Kbyte Block 33

64-Kbyte Block 32

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block 29

64-Kbyte Block 28

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block 25

64-Kbyte Block 24

64-Kbyte Block 23

64-Kbyte Block 22

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block 17

64-Kbyte Block

64-Kbyte Block 15

64-Kbyte Block 14

64-Kbyte Block 13

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block

8-Kbyte Block

8-Kbyte Block 6

8-Kbyte Block

8-Kbyte Block

8-Kbyte Block

8-Kbyte Block

8-Kbyte Block

8-Kbyte Block 0

18

16

12

11

10

9

8

7

5

4

3

2

01

FFFFF F0000

EFFFF

E0000

DFFFF

D0000

CFFFF

C0000

BFFFF

B0000

AFFFF

A0000

9FFFF

90000

8FFFF

80000

7FFFF

70000

6FFFF

60000

5FFFF

50000

4FFFF

40000

3FFFF

30000

2FFFF

20000

1FFFF

10000

0FFFF

0E000

0DFFF

0C000

0BFFF

0A000

09FFF

08000

07FFF

06000

05FFF

04000

03FFF

02000

01FFF

00000

8-Mbit Advanced Boot

Block

15

14

8 64-Kbyte Block 22

64-Kbyte Block 21

64-Kbyte Block 20

64-Kbyte Block 19

64-Kbyte Block 18

64-Kbyte Block 17

64-Kbyte Block 16

64-Kbyte Block

64-Kbyte Block

64-Kbyte Block 13

64-Kbyte Block 12

64-Kbyte Block 11

64-Kbyte Block 10

64-Kbyte Block 9

64-Kbyte Block

8-Kbyte Block 7

8-Kbyte Block

8-Kbyte Block 5

8-Kbyte Block 4

8-Kbyte Block 3

8-Kbyte Block 2

8-Kbyte Block 0

8-Kbyte Block 0

6

0605-06

Figure 5. 8-/16-Mbit Advanced Boot Block Byte-Wide Bottom Boot Memory Maps

PRELIMINARY                                                                                     13

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

3.0     PRINCIPLES OF OPERATION

Flash  memory  combines  EEPROM  functionality with  in-circuit  electrical  program  and  erase capability. The Smart 3 Advanced Boot Block flash memory family utilizes a Command User Interface (CUI) and automated algorithms to simplify program and erase operations. The CUI allows for 100% CMOS-level control inputs, fixed power supplies during erasure and programming, and maximum EEPROM compatibility.

When VPP < VPPLK, the device will only execute the following   commands   successfully:   Read   Array, Read Status Register, Clear Status Register and Read Intelligent Identifier. The device provides standard   EEPROM   read,   standby   and   output disable operations. Manufacturer identification and device identification data can be accessed through the CUI. In addition, 2.7V or 12V on VPP  allows program  and  erase  of  the  device.  All  functions

associated with altering memory contents, namely program and erase, are accessible via the CUI.

The internal Write State Machine (WSM) completely automates program and erase operations while the CUI signals the start of an operation and the status register reports status. The CUI handles the WE# interface to the data and address latches, as well as system status requests during WSM operation.

3.1       Bus Operation

Smart   3   Advanced   Boot   Block   flash   memory devices read, program and erase in-system via the local CPU or microcontroller. All bus cycles to or from the flash memory conform to standard microcontroller bus cycles. Four control pins dictate the data flow in and out of the flash component: CE#, OE#, WE# and RP#. These bus operations are summarized in Table 3.

Table 3. Bus Operations for Byte-Wide Mode

| Mode | Notes | RP# | CE# | OE# | WE# | WP# | A0 | VPP | DQ0–7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Read | 1,2,3 | VIH | VIL | VIL | VIH | X | X | X | DOUT |
| Output Disable | 2 | VIH | VIL | VIH | VIH | X | X | X | High Z |
| Standby | 2 | VIH | VIH | X | X | X | X | X | High Z |
| Deep Power-Down | 2,9 | VIL | X | X | X | X | X | X | High Z |
| Intelligent Identifier (Mfr.) | 2,4 | VIH | VIL | VIL | VIH | X | VIL | X | 89 H |
| Intelligent Identifier (Dvc.) | 2,4,5 | VIH | VIL | VIL | VIH | X | VIH | X | See Table 5 |
| Write | 2,6,7, 8 | VIH | VIL | VIH | VIL | X | X | VPPH | DIN |

NOTES:

1.    Refer to DC Characteristics.

2.    X must be VIL, VIH for control pins and addresses, VPPLK , VPPH1 or VPPH2 for VPP.

3.    See DC Characteristics for VPPLK, VPPH1, VPPH2 voltages.

4.    Manufacturer and device codes may also be accessed via a CUI write sequence, A1–A20 = X

5.    See Table 5 for device IDs.

6.    Refer to Table 6 for valid DIN during a write operation.

7.    Command writes for block erase or byte program are only executed when VPP = VPPH1 or VPPH2.

8.    To program or erase the lockable blocks, hold WP# at VIH. See Section 3.3.

9.    RP# must be at GND   0.2V to meet the maximum deep power-down current specified.

14                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

3.1.1         READ

The flash memory has three read modes available: read array, read identifier, and read status. These modes are accessible independent of the VPP voltage. The appropriate read mode command must be issued to the CUI to enter the corresponding mode.  Upon  initial  device  power-up  or  after  exit from deep power-down mode, the device automatically defaults to read array mode.

CE# and OE# must be driven active to obtain data at the outputs. CE# is the device selection control; when active it enables the flash memory device. OE# is the data output (DQ0–DQ7) control and it drives the selected memory data onto the I/O bus. For all read modes, WE# and RP# must be at VIH.

Figure 14 illustrates a read cycle.

3.1.2         OUTPUT DISABLE

With OE# at a logic-high level (VIH), the device outputs are disabled. Output pins DQ0–DQ7 are placed in a high-impedance state.

3.1.3         STANDBY

Deselecting the device by bringing CE# to a logic- high level (VIH) places the device in standby mode, which substantially reduces device power consumption. In standby, outputs DQ0–DQ7 are placed in a high-impedance state independent of

OE#. If deselected during program or erase operation, the device continues to consume active power until the program or erase operation is complete.

3.1.4         DEEP POWER-DOWN/RESET

RP# at VIL  initiates the deep power-down  mode, sometimes referred to as reset mode.

From  read  mode,  RP#  going  low  for  time  tPLPH

accomplishes the following:

1.     deselects the memory

2.     places  output  drivers  in  a  high-impedance state

After return from power-down, a time tPHQV is required until the initial memory access outputs are

valid.  A  delay  (tPHWL   or  tPHEL)  is  required  after return from power-down before a write sequence can be initiated. After this wake-up interval, normal operation is restored. The CUI resets to read array mode, and the status register is set to 80H (ready).

If RP# is taken low for time tPLPH during a program or erase operation, the operation  will  be  aborted and the memory contents at the aborted location are no longer valid. After returning from an aborted operation, time tPHQV  or tPHWL/tPHEL  must be met before a read or write operation is initiated respectively.

3.1.5         WRITE

A write is any command that alters the contents of the memory array. There are two write commands: Program (40H) and Erase (20H). Writing either of these commands to the internal Command User Interface (CUI) initiates  a sequence of  internally- timed functions that culminate in the completion of the requested task (unless that operation is aborted by either RP# being driven to VIL for of tPLRH or an appropriate suspend command).

The Command User Interface does not occupy an addressable memory location. Instead, commands are written into the CUI using standard microprocessor write timings when WE# and CE# are low, OE# = VIH, and the proper address and data (command) are presented. The command is latched on the rising edge of the first WE# or CE# pulse, whichever occurs first. Figure 15 illustrates a write operation.

Device operations are selected by writing specific commands into the CUI. Table 4 defines the available commands. Appendix B provides detailed information on moving between the different modes of operation.

3.2       Modes of Operation

The flash memory has three read modes and two write modes. The read modes are read array, read identifier, and read status. The write modes are program and block erase. Three additional mode (erase suspend to program, erase suspend to read and program suspend to read) are available only during  suspended  operations.  These  modes  are

PRELIMINARY                                                                                     15

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

reached using the commands summarized in Table

4. A comprehensive chart showing the state transitions is in Appendix B.

3.2.1         READ ARRAY

When RP# transitions from VIL  (reset) to VIH, the device will be in the read array mode and will respond to the read control inputs (CE#, address inputs, and OE#) without any commands being written to the CUI.

When the device is in the read array mode, four control signals must be controlled to obtain data at the outputs.

WE# must be logic high (VIH) CE# must be logic low (VIL) OE# must be logic low (VIL) RP# must be logic high (VIH)

In addition, the address of the desired location must be applied to the address pins.

If the device is not in read array mode, as would be the case after a program or erase operation, the Read Array command (FFH) must be written to the CUI before array reads can take place.

Table 4. Command Codes and Descriptions

| Code | Device Mode | Description |
| --- | --- | --- |
| 00 | Invalid/ Reserved | Unassigned commands that should not be used. Intel reserves the right to redefine these codes for future functions. |
| FF | Read Array | Places the device in read array mode, such that array data will be output on the data pins. |
| 40 | Program Set-Up | This is a two-cycle command. The first cycle prepares the CUI for a program operation. The second cycle latches addresses and data information and initiates the WSM to execute the Program algorithm. The flash outputs status register data when CE# or OE# is toggled. A Read Array command is required after programming to read array data. See Section 3.2.4. |
| 10 | Alternate Program Set-Up | (See 40H/Program Set-Up) |
| 20 | Erase Set-Up | Prepares the CUI for the Erase Confirm command. If the next command is not an Erase Confirm command, then the CUI will (a) set both SR.4 and SR.5 of the status register to a “1,” (b) place the device into the read status register mode, and (c) wait for another command. See Section 3.2.5. |
| D0 | Program Resume Erase Resume/ Erase Confirm | If the previous command was an Erase Set-Up command, then the CUI will close the address and data latches, and begin erasing the block indicated on the address pins. If a program or erase operation was previously suspended, this command will resume that operation. During program/erase, the device will respond only to the Read Status Register, Program Suspend/Erase Suspend commands and will output status register data when CE# or OE# is toggled. |

16                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 4.  Command Codes and Descriptions (Continued)

| Code | Device Mode | Description |
| --- | --- | --- |
| B0 | Program Suspend Erase Suspend | Issuing this command will begin to suspend the currently executing program/erase operation. The status register will indicate when the operation has been successfully suspended by setting either the program suspend (SR.2) or erase suspend (SR.6) and the WSM Status bit (SR.7) to a “1” (ready). The WSM will continue to idle in the SUSPEND state, regardless of the state of all input control pins except RP#, which will immediately shut down the WSM and the remainder of the chip if it is driven to VIL. See Sections 3.2.4.1 and 3.2.5.1. |
| 70 | Read Status Register | This command places the device into read status register mode. Reading the device will output the contents of the status register, regardless of the address presented to the device. The device automatically enters this mode after a program or erase operation has been initiated. See Section 3.2.3. |
| 50 | Clear Status Register | The WSM can set the Block Lock Status (SR.1) , V PP Status (SR.3), Program Status (SR.4), and Erase Status (SR.5) bits in the status register to “1,” but it cannot clear them to “0.” Issuing this command clears those bits to “0.” |
| 90 | Intelligent Identifier | Puts the device into the intelligent identifier read mode, so that reading the device will output the manufacturer and device codes (A0 = 0 for manufacturer, A0 = 1 for device, all other address inputs are ignored). See Section 3.2.2. |

NOTE:

See Appendix B for mode transition information.

3.2.2         READ INTELLIGENT IDENTIFIER

| To read the manufacturer and device codes, the device must be in read intelligent identifier mode, which can be reached by writing the Intelligent Identifier command (90H). Once in intelligent identifier mode, A0 = 0 outputs the manufacturer’s identification code and A0 = 1 outputs the device code. See Table 5 for product signatures. To return to read array mode, write the Read Array command (FFH). Size | Mfr. ID | Device ID |  |
| --- | --- | --- | --- |
|  |  | -T (Top Boot) | -B (Bottom Boot) |
| 8-Mbit | 89H | D2H | D3H |
| 16-Mbit | 89H | D0H | D1H |

Table 5. Intelligent Identifier Table

3.2.3         READ STATUS REGISTER

The   device   status   register   indicates   when   a program or erase operation is complete, and the success or failure of that operation. To read the status  register  issue  the  Read  Status  Register (70H) command to the CUI. This causes all subsequent read operations to output data from the status register until another command is written to the CUI. To return to reading from the array, issue the Read Array (FFH) command.

The status register bits are output on DQ0–DQ7.

The contents of the status register are latched on the falling edge of OE# or CE#. This prevents possible bus errors which might occur if status register contents change while being read. CE# or OE# must be toggled with each subsequent status read, or the status register will not indicate completion of a program or erase operation.

PRELIMINARY                                                                                     17

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

When the WSM is active, bit 7 (SR.7) of the status register will  indicate the status  of  the  WSM;  the remaining   bits   in   the   status   register   indicate whether or not the WSM was successful in performing the desired operation (see Table 7).

3.2.3.1           Clearing the Status Register

The WSM sets status bits 1 through 7 to “1,” and clears bits 2, 6 and 7 to “0,” but cannot clear status bits 1 or 3 through 5 to “0.” Because bits 1, 3, 4, and 5 indicate various error conditions, these bits can only be cleared by the controlling CPU through the   use   of   the   Clear   Status   Register   (50H) command.  By  allowing  the  system  software  to control   the   resetting   of   these   bits,   several operations may be performed (such as cumulatively programming several addresses or erasing multiple blocks in sequence) before reading the status register to determine if an error occurred during that series. Clear the Status Register before beginning another command or sequence. Note, again, that the Read Array command must be issued before data can be read from the memory array.

3.2.4         PROGRAM MODE

Programming is executed using a two-write sequence. The Program Setup command (40H) is written to the CUI followed by a second write which specifies the address and data to be programmed. The WSM will execute the following sequence of internally timed events:

1.   Program  the  desired  bits  of  the  addressed memory.

2.   Verify  that  the  desired  bits  are  sufficiently programmed.

Programming of the memory results in specific bits within an address location being changed to a “0.” If the user attempts to program “1”s, there will be no change of the memory cell contents and no error occurs.

The status register indicates programming status: while the program sequence is executing, bit 7 of the status register is a “0.” The status register can be polled by toggling either CE# or OE#. While programming, the only valid commands are Read Status Register, Program Suspend, and Program Resume.

When   programming   is   complete,   the   Program Status bits should be checked. If the programming operation was unsuccessful, bit SR.4 of the status register is set to indicate a program failure. If SR.3 is set then VPP was not within acceptable limits, and the WSM did not execute the program command. If SR.1 is set, a program operation was attempted to a locked block and the operation was aborted.

The status register should be cleared before attempting the next operation. Any CUI instruction can   follow   after   programming   is   completed; however,  to  prevent  inadvertent  status  register reads, be sure to reset the CUI to read array mode.

3.2.4.1           Suspending and Resuming

Program

The Program Suspend command allows program suspension in order to read data in other locations of memory. Once the programming process starts, writing the Program Suspend command to the CUI requests that the WSM suspend the program sequence (at predetermined points in the program algorithm). The device continues to output status register data after the Program Suspend command is written. Polling status register bits SR.7 and SR.2 will  determine  when  the  program  operation  has been suspended (both will be set to “1”). tWHRH1/tEHRH1 specify the program suspend latency.

A Read Array command can now be written to the CUI to read data from blocks other than that which is  suspended.  The  only  other  valid  commands, while program is suspended, are Read Status Register and Program Resume. After the Program Resume command is written to the flash memory, the WSM will continue with the program process and status register bits SR.2 and SR.7 will automatically   be   cleared.   After   the   Program Resume command is written, the device automatically  outputs  status  register  data  when read   (see   Figure 7,   Program   Suspend/Resume Flowchart). VPP must remain at the same VPP level

used for program while in program suspend mode.

RP# must also remain at VIH.

3.2.4.2           VPP Supply Voltage during

Program

VPP  supply voltage considerations are outlined in

Section 3.4.

18                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

3.2.5         ERASE MODE

To erase a block, write the Erase Set-up and Erase Confirm commands to the CUI, along with an address identifying the block to be erased. This address   is   latched   internally   when   the   Erase Confirm command is issued. Block erasure results in all bits within the block being set to “1.” Only one block can be erased at a time.

The WSM will execute the following sequence of internally timed events to:

1.   Program all bits within the block to “0.”

2.   Verify   that   all   bits   within   the   block   are sufficiently programmed to “0.”

3.   Erase all bits within the block to “1.”

4.   Verify   that   all   bits   within   the   block   are sufficiently erased.

While the erase sequence is executing, bit 7 of the status register is a “0.”

When the status register indicates that erasure is complete, check the Erase Status bit to verify that the erase operation was successful. If the Erase operation was unsuccessful, SR.5 of the status register  will  be  set  to  a  “1,”  indicating  an  erase error. If VPP  was not within acceptable limits after

the Erase Confirm command was issued, the WSM

will not execute the erase sequence; instead, SR.5 of the status register is set to indicate an erase

error, and SR.3 is set to a “1” to identify that VPP

supply voltage was not within acceptable limits.

After an erase operation, clear the Status Register (50H)  before  attempting  the  next  operation.  Any CUI   instruction   can   follow   after   erasure   is completed; however, to prevent inadvertent status register reads, it is advisable to reset the flash to read array after the erase is complete.

3.2.5.1           Suspending and Resuming Erase

Since an erase operation requires on the order of seconds to complete, an Erase Suspend command is provided to allow erase-sequence interruption in order to read data from or program data to another block in memory. Once the erase sequence is started, writing the Erase Suspend command to the CUI requests that the WSM pause the erase sequence at a predetermined point in the erase algorithm. The status register will indicate if/when the erase operation has been suspended.

A  Read  Array/Program  command  can  now  be written to the CUI in order to read/write data from/to blocks other than that which is suspended. The Program  command  can  subsequently  be suspended to read yet another array location. The only valid commands while erase is suspended are Erase Resume, Program, Program Resume, Read Array, or Read Status Register.

During  erase  suspend  mode,  the  chip  can  be placed in a pseudo-standby mode by taking CE# to VIH. This reduces active current consumption.

Erase Resume continues the erase sequence when CE# = VIL. As with the end of a standard erase operation, the status register must be read and cleared before the next instruction is issued.

3.2.5.2           VPP Supply Voltage during Erase

VPP  supply voltage considerations are outlined in

Section 3.4.

PRELIMINARY                                                                                     19

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

Table 6. Command Bus Definitions

| Command | Notes | First Bus Cycle |  |  | Second Bus Cycle |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Oper | Addr | Data | Oper | Addr | Data |
| Read Array | 5 | Write | X | FFH |  |  |  |
| Intelligent Identifier | 2,3,5 | Write | X | 90H | Read | IA | ID |
| Read Status Register | 5 | Write | X | 70H | Read | X | SRD |
| Clear Status Register | 5 | Write | X | 50H |  |  |  |
| Write (Program) | 4,5 | Write | X | 40H | Write | PA | PD |
| Alternate Write (Program) | 4,5 | Write | X | 10H | Write | PA | PD |
| Block Erase/Confirm | 5 | Write | X | 20H | Write | BA | D0H |
| Program/Erase Suspend | 5 | Write | X | B0H |  |  |  |
| Program/Erase Resume | 5 | Write | X | D0H |  |  |  |

ADDRESS                                                                      DATA

BA = Block Address                                                       SRD = Status Register Data

IA = Identifier Address                                                    ID = Identifier Data PA = Program Address                                                  PD = Program Data X = Don’t Care

NOTES:

1.    Bus operations are defined in Table 3.

2.    A0 = 0 for manufacturer code, A0 = 1 for device code.

3.    Following the Intelligent Identifier command, two read operations access manufacturer and device codes.

4.    Either 40H or 10H command is valid.

20                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 7. Status Register Bit Definition

| WSMS | ESS | ES | PS | VPPS | PSS | BLS | R |
| --- | --- | --- | --- | --- | --- | --- | --- |

7                   6                   5                   4                    3                   2                   1                   0

NOTES:

SR.7 WRITE STATE MACHINE STATUS

1 = Ready                                (WSMS)

0 = Busy

SR.6 = ERASE-SUSPEND STATUS (ESS)

1 = Erase Suspended

0 = Erase In Progress/Completed

SR.5 = ERASE STATUS (ES)

1 = Error In Block Erasure

0 = Successful Block Erase

SR.4 = PROGRAM STATUS (PS)

1 = Error in Byte Program

0 = Successful Byte Program

SR.3 = VPP STATUS (VPPS)

1 = VPP Low Detect, Operation Abort

0 = VPP OK

SR.2 = PROGRAM SUSPEND STATUS (PSS)

1 = Program Suspended

0 = Program in Progress/Completed

SR.1 = Block Lock Status

1 = Program/Erase attempted on locked block; Operation aborted

0 = No operation to locked blocks

SR.0 = RESERVED FOR FUTURE ENHANCEMENTS (R)

Check Write State Machine bit first to determine Byte Program or Block Erase completion, before checking Program or Erase Status bits.

When Erase Suspend is issued, WSM halts execution and sets both WSMS and ESS bits to “1.” ESS bit remains set to “1” until an Erase Resume command is issued.

When this bit is set to “1,” WSM has applied the max. number of erase pulses to the block and is still unable to verify successful block erasure.

When this bit is set to “1,” WSM has attempted but failed to program a byte.

The VPP Status bit does not provide continuous indication of VPP level. The WSM interrogates VPP level only after the Program or Erase command sequences have been entered, and informs the system if VPP has not been switched on. The VPP is also checked before the operation is verified by the WSM. The VPP Status bit is not guaranteed to report accurate feedback between VPPLK and VPPH.

When Program Suspend is issued, WSM halts execution and sets both WSMS and PSS bits to “1.” PSS bit remains set to “1” until a Program Resume command is issued.

If a program or erase operation is attempted to one of the locked blocks, this bit is set by the WSM. The operation specified is aborted and the device is returned to read status mode.

These bits are reserved for future use and should be masked out when polling the Status Register.

PRELIMINARY                                                                                     21

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

Start

Write 40H

Program Address/Data

Read Status Register

| No Bus Operation | Command | Comments |
| --- | --- | --- |
| Write | Program Setup | Data = 40H |
| Write | Program | Data = Data to Program Addr = Location to Program |
| Read |  | Status Register Data Toggle CE# or OE# to Update Status Register Data |
| Standby |  | Check SR.7 1 = WSM Ready 0 = WSM Busy |
| Repeat for subsequent programming operations. SR Full Status Check can be done after each program or after a sequence of program operations. Write FFH after the last program operation to reset device to read array mode. |  |  |

SR.7 = 1?

Yes

Full Status

Check if Desired

Program Complete

FULL STATUS CHECK PROCEDURE

Read Status Register

Data (See Above)

1

SR.3 =

VPP Range Error

0

SR.4 =

1

Programming Error

0

1

SR.1 =

0

Program Successful

Attempted  Program to

Locked Block - Aborted

| 0605-07 Bus Operation | Command | Comments |
| --- | --- | --- |
| Standby |  | Check SR.3 1 = VPP Low Detect |
| Standby |  | Check SR.4 1 = VPP Program Error |
| Standby |  | Check SR.1 1 = Attempted Program to Locked Block - Program Aborted |
| SR.3 MUST be cleared, if set during a program attempt, before further attempts are allowed by the Write State Machine. SR.1, SR.3 and SR.4 are only cleared by the Clear Staus Register Command, in cases where multiple bytes are programmed before full status is checked. If an error is detected, clear the status register before attempting retry or other error recovery. |  |  |

Figure 6. Automated Byte Programming Flowchart

22                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Start

Write B0H

Read Status Register

SR.7 =

1

SR.2 =

0

0

| Program Completed Bus Operation | Command | Comments |
| --- | --- | --- |
| Write | Program Suspend | Data = B0H Addr = X |
| Read |  | Status Register Data Toggle CE# or OE# to Update Status Register Data Addr = X |
| Standby |  | Check SR.7 1 = WSM Ready 0 = WSM Busy |
| Standby |  | Check SR.2 1 = Program Suspended 0 = Program Completed |
| Write | Read Array | Data = FFH Addr = X |
| Read |  | Read array data from block other than the one being programmed. |
| Write | Program Resume | Data = D0H Addr = X |

1

Write FFH

Read Array Data

Done                 No

Reading

Yes

Write D0H

Write FFH

Program Resumed                    Read Array Data

0605-08

Figure 7. Program Suspend/Resume Flowchart

PRELIMINARY                                                                                     23

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

Start

Write 20H

Write D0H and

Block Address

Read Status Register

No

0

Suspend

Erase Loop

| Yes Bus Operation | Command | Comments |
| --- | --- | --- |
| Write | Erase Setup | Data = 20H Addr = Within Block to Be Erased |
| Write | Erase Confirm | Data = D0H Addr = Within Block to Be Erased |
| Read |  | Status Register Data Toggle CE# or OE# to Update Status Register Data |
| Standby |  | Check SR.7 1 = WSM Ready 0 = WSM Busy |
| Repeat for subsequent block erasures. Full Status Check can be done after each block erase or after a sequence of block erasures. Write FFH after the last write operation to reset device to read array mode. |  |  |

SR.7 =

1

Suspend Erase

Full Status

Check if Desired

Block Erase Complete

FULL STATUS CHECK PROCEDURE

Read Status Register

Data (See Above)

1

SR.3 =

0

1

SR.4,5 =

0

1

SR.5 =

VPP Range Error

Command Sequence

Error

Block Erase Error

0

SR.1 =

0

Block Erase

Successful

1

Attempted Erase of

| Locked Block - Aborted Bus Operation | Command | Comments |
| --- | --- | --- |
| Standby |  | Check SR.3 1 = VPP Low Detect |
| Standby |  | Check SR.4,5 Both 1 = Command Sequence Error |
| Standby |  | Check SR.5 1 = Block Erase Error |
| Standby |  | Check SR.1 1 = Attempted Erase of Locked Block - Erase Aborted |
| SR. 1 and 3 MUST be cleared, if set during an erase attempt, before further attempts are allowed by the Write State Machine. SR.1, 3, 4, 5 are only cleared by the Clear Staus Register Command, in cases where multiple bytes are erased before full status is checked. If an error is detected, clear the status register before attempting retry or other error recovery. |  |  |

0605-09

Figure 8. Automated Block Erase Flowchart

24                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Start

Write B0H

Read Status Register

SR.7 =

1

SR.6 =

0

0

| Erase Completed Bus Operation | Command | Comments |
| --- | --- | --- |
| Write | Erase Suspend | Data = B0H Addr = X |
| Read |  | Status Register Data Toggle CE# or OE# to Update Status Register Data Addr = X |
| Standby |  | Check SR.7 1 = WSM Ready 0 = WSM Busy |
| Standby |  | Check SR.6 1 = Erase Suspended 0 = Erase Completed |
| Write | Read Array | Data = FFH Addr = X |
| Read |  | Read array data from block other than the one being erased. |
| Program |  | Program data to block other than the one being erased. |
| Write | Erase Resume | Data = D0H Addr = X |

1

Write FFH/40H

Read Array Data/ Program Array

Done                 No

Reading and/or

Programming

Yes

Write D0H

Write FFH

Erase Resumed                       Read Array Data

0605-010

Figure 9. Erase Suspend/Resume Flowchart

PRELIMINARY                                                                                     25

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

3.3       Block Locking

The Smart 3 Advanced Boot Block flash memory architecture features two hardware-lockable parameter blocks so that the kernel code for the system can be kept secure while other parameter blocks are programmed or erased as necessary.

3.3.1         VPP = VIL FOR COMPLETE PROTECTION

The VPP programming voltage can be held low for complete write protection of all blocks in the flash device. When VPP is below VPPLK, any program or erase operation will result in a error, prompting the corresponding Status Register bit (SR.3) to be set.

3.3.2         WP# = VIL FOR BLOCK LOCKING

The lockable blocks are locked when WP# = VIL; any program or erase operation to a locked block will result in an error, which will be reflected in the status register. For top configuration, the top two parameter  blocks  (blocks  #37  and  #38  for  the

16-Mbit, and blocks #21 and #22 for the 8-Mbit) are lockable. For the bottom configuration, the bottom two parameter blocks (blocks #0 and #1 for 8-/16- Mbit) are lockable. Unlocked blocks can be programmed  or  erased  normally  (unless  VPP   is below VPPLK).

3.3.3         WP# = VIH FOR BLOCK UNLOCKING

WP# = VIH unlocks all lockable blocks.

These blocks can now be programmed or erased. Note that RP# does not override WP# locking as in

previous Boot Block devices. WP# controls all block

locking   and   VPP     provides   protection   against spurious writes. Table 8 defines the write protection methods.

3.4       VPP Program and Erase

Voltages

Intel’s Smart 3 products provide in-system programming and erase at 2.7V–3.6V VPP. For customers requiring fast programming in their manufacturing  environment,  Smart   3  Advanced Boot Block includes an additional low-cost, backward-compatible 12V programming feature.

The 12V VPP mode enhances programming performance during the short period of time typically found in manufacturing processes;  however,  it  is not intended for extended use. 12V may be applied to VPP  during program and erase operations for a maximum of 1000 cycles on the main blocks and

2500 cycles on the parameter blocks. VPP may be connected to 12V for a total of 80 hours maximum. Stressing the device beyond these limits may cause permanent damage.

Table 8. Write Protection Truth Table for

Advanced Boot Block Flash Memory Family

| VPP | WP# | RP# | Write Protection Provided |
| --- | --- | --- | --- |
| X | X | VIL | All Blocks Locked |
| VIL | X | VIH | All Blocks Locked |
| VPPLK | VIL | VIH | Lockable Blocks Locked |
| VPPLK | VIH | VIH | All Blocks Unlocked |

3.5       Power Consumption

While  in  operation,  the  flash  device  consumes active power. However, Intel flash devices have a three-tiered approach to power savings that can significantly reduce overall system power consumption. The Automatic Power Savings (APS) feature   reduces   power   consumption   when   the device is idle. If the CE# is deasserted, the flash enters  its  standby  mode,  where  current consumption is even lower. If RP# = VIL  the flash enters a deep power-down mode, where current is at a minimum. The combination of these features can minimize overall memory power consumption, and therefore, overall system power consumption.

3.5.1         ACTIVE POWER

With CE# at a logic-low level and RP# at a logic- high level, the device is in the active mode. Refer to the DC Characteristics tables for ICC current values. Active power is the largest contributor to overall system power consumption. Minimizing the active current could have a profound effect on system power consumption, especially for battery-operated devices.

26                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

3.5.2         AUTOMATIC POWER SAVINGS (APS)

Automatic Power Savings provides low-power operation during active mode. Power Reduction Control (PRC) circuitry allows the flash to put itself into a low current state when not being accessed. After  data  is  read  from  the  memory  array,  PRC logic controls the device’s power consumption by entering the APS mode where typical ICC current is comparable to ICCS. The flash stays in this static state with outputs valid until a new location is read.

APS  reduces  active  current  to  standby  current levels for 2.7V–3.6V CMOS input levels.

3.5.3         STANDBY POWER

With CE# at a logic-high level (VIH) and the CUI in read mode, the flash memory is in standby mode, which disables much of the device’s circuitry and substantially reduces power consumption. Outputs (DQ0–DQ7) are placed in a high-impedance state independent of the status of the OE# signal. If CE# transitions to a logic-high level during erase or program operations, the device will continue to perform the operation and consume corresponding active power until the operation is completed.

System engineers should analyze the breakdown of standby time versus active time and quantify the respective  power  consumption  in  each  mode  for their specific application. This will provide a more accurate measure of application-specific power and energy requirements.

3.5.4         DEEP POWER-DOWN MODE

The  deep  power-down  mode  of   the  Smart   3

Advanced Boot Block products switches the device into a low power savings mode, which is especially important for battery-based devices. This mode is activated when RP# = VIL. (GND    0.2V).

During read modes, RP# going low de-selects the memory  and places  the  output  drivers  in  a  high impedance state. Recovery from the deep power- down state, requires a minimum time equal to tPHQV (see AC Characteristics table).

During program or erase modes, RP# transitioning low  will  abort  the  operation,  but   the  memory contents of the address being programmed or the block being erased are no longer valid as the data integrity  has  been  compromised  by  the  abort.

During deep power-down,  all  internal  circuits  are switched to a low power savings mode (RP# transitioning to VIL or turning off power to the device clears the status register).

3.6       Power-Up/Down Operation

The device is protected against accidental block erasure or programming during power transitions. Power supply sequencing is not required, since the device is indifferent as to which power supply, VPP or VCC, powers-up first.

3.6.1         RP# CONNECTED TO SYSTEM RESET

The use of RP# during system reset is important with automated program/erase devices since the system  expects  to  read  from  the  flash  memory when it comes out of reset. If a CPU reset occurs without a flash memory reset, proper CPU initialization   will   not   occur   because   the   flash memory   may   be   providing   status   information instead of array data. Intel recommends connecting RP# to the system CPU RESET# signal to allow proper   CPU/flash   initialization   following   system reset.

System  designers  must  guard  against  spurious writes when VCC voltages are above VLKO and VPP is active. Since both WE# and CE# must be low for a command write, driving either signal to VIH  will inhibit writes to the device. The CUI architecture provides additional protection since alteration of memory contents can only occur after successful completion of the two-step command sequences. The device is also disabled until RP# is brought to VIH, regardless of the state of its control inputs. By holding  the  device  in  reset  (RP#  connected  to system PowerGood) during power-up/down, invalid bus conditions during power-up can be masked, providing yet another level of memory protection.

3.6.2         VCC, VPP AND RP# TRANSITIONS

The CUI latches commands as issued by system software   and   is   not   altered   by   VPP    or   CE# transitions or WSM actions. Its default state upon power-up, after exit from deep power-down mode or after VCC transitions above VLKO (Lockout voltage), is read array mode.

PRELIMINARY                                                                                     27

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

After any program or block erase operation is complete   (even   after   VPP    transitions   down   to VPPLK), the CUI must be reset to read array mode via the Read Array command if access to the flash memory array is desired.

Refer to AP-617 Additional Flash Data Protection Using VPP, RP#, and WP# for a circuit-level description of how to implement the protection schemes discussed in Section 3.5.

3.7       Power Supply Decoupling

Flash memory’s power switching characteristics require  careful  device  decoupling.  System designers should consider three supply current issues:

1.   Standby current levels (ICCS)

2.   Active current levels (ICCR)

3.   Transient peaks produced by falling and rising edges of CE#.

Transient current magnitudes depend on the device outputs’ capacitive and inductive loading. Two-line control and proper decoupling capacitor selection will suppress these transient voltage peaks. Each flash   device   should   have   a   0.1   µF   ceramic capacitor connected between each VCC  and GND, and between its VPP  and GND. These high- frequency, inherently low-inductance capacitors should  be  placed  as  close  as  possible  to  the package leads.

3.7.1         VPP TRACE ON PRINTED CIRCUIT BOARDS

Designing for in-system writes to the flash memory requires special consideration of the VPP power supply trace by the printed circuit board designer. The VPP pin supplies the flash memory cells current for programming and erasing. VPP trace widths and layout should be similar to that of VCC. Adequate VPP    supply   traces,   and   decoupling   capacitors placed adjacent to the component, will decrease spikes and overshoots.

28                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

4.0     ABSOLUTE MAXIMUM RATINGS*

Extended Operating Temperature

During Read ............................ –40°C to +85°C During Block Erase

and Program............................ –40°C to +85°C

Temperature Under Bias ......... –40°C to +85°C Storage Temperature................... –65°C to +125°C Voltage on Any Pin

(except VCC, VCCQ and VPP)

with Respect to GND ............... –0.5V to +5.0V1

VPP Voltage (for Block

Erase and Program)

with Respect to GND .........–0.5V to +13.5V1,2,4

VCC and VCCQ Supply Voltage

with Respect to GND ............... –0.2V to +5.0V1

Output Short Circuit Current...................... 100 mA3

NOTICE: This datasheet contains preliminary information on products in production. The specifications are  subject to change without notice. Verify with your  local  Intel  Sales office that you have the latest datasheet before finalizing a design.

* WARNING: Stressing the device beyond the "Absolute Maximum Ratings" may cause permanent damage. These are stress ratings only. Operation beyond the "Operating Conditions" is not recommended and extended exposure beyond the "Operating Conditions" may effect device reliability.

NOTES:

1.    Minimum DC voltage is –0.5V on input/output pins.

During transitions, this level may undershoot to –2.0V for periods < 20 ns. Maximum DC voltage on input/output pins is VCC + 0.5V which, during transitions, may overshoot to VCC + 2.0V for periods <

20 ns.

2.    Maximum DC voltage on VPP may overshoot to +14.0V

for periods < 20 ns.

3.    Output shorted for no more than one second. No more than one output shorted at a time.

4.    VPP Program voltage is normally 2.7V–3.6V.

Connection to supply of 11.4V–12.6V can only be done

for 1000 cycles on the main blocks and 2500 cycles on the parameter blocks during program/erase. VPP may be connected to 12V for a total of 80 hours maximum. See Section 3.4 for details.

5.0     OPERATING CONDITIONS (VCCQ = 2.7V–3.6V)

Table 9. Temperature and Voltage Operating Conditions4

| Symbol | Parameter | Notes | Min | Max | Units |
| --- | --- | --- | --- | --- | --- |
| TA | Operating Temperature |  | –40 | +85 | °C |
| VCC | 2.7V–3.6V VCC Supply Voltage | 1,4 | 2.7 | 3.6 | Volts |
| VCCQ | 2.7V–3.6V I/O Supply Voltage | 1,2,4 | 2.7 | 3.6 | Volts |
| VPP1 | Program and Erase Voltage | 4 | 2.7 | 3.6 | Volts |
| VPP2 |  | 3 | 11.4 | 12.6 | Volts |
| Cycling | Block Erase Cycling | 5 | 10,000 |  | Cycles |

NOTES:

1.    See DC Characteristics tables for voltage range-specific specifications.

2.    The voltage swing on the inputs, VIN is required to match VCCQ.

3.    Applying VPP = 11.4V–12.6V during a program/erase can only be done for a maximum of 1000 cycles on the main blocks and 2500 cycles on the parameter blocks. VPP may be connected to 12V for a total of 80 hours maximum. See section 3.4 for details.

4.    VCC, VCCQ and VPP1 must share the same supply when all three are between 2.7V and 3.6V.

5.    For operating temperatures of –25 C– +85 C the device is projected to have a minimum block erase cycling of 10,000 to

30,000 cycles.

PRELIMINARY                                                                                     29

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

5.1       DC Characteristics: VCCQ = 2.7V–3.6V

Table 10. DC Characteristics

| Sym | Parameter | Notes | VCC = 2.7V–3.6V |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Typ | Max |  |  |
| ILI | Input Load Current | 1 |  | ± 1.0 | µA | VCC = VCCMax = VCCQMax VIN = VCCQ or GND |
| ILO | Output Leakage Current | 1 |  | ± 10 | µA | VCC = VCCMax = VCCQMax VIN = VCCQ or GND |
| ICCS | VCC Standby Current | 1,7 | 20 | 50 | µA | CMOS INPUTS VCC = VCCMax = VCCQMax CE# = RP# = VCCQ |
| ICCD | VCC Deep Power-Down Current | 1,7 | 1 | 10 | µA | CMOS INPUTS VCC = VCCMax = VCCQMax VIN = VCCQ or GND RP# = GND ± 0.2V |
| ICCR | VCC Read Current | 1,5,7 | 10 | 20 | mA | CMOS INPUTS VCC = VCCMax = VCCQMax OE# = VIH , CE# =VIL f = 5 MHz, IOUT = 0 mA Inputs = VIL or VIH |
| ICCW | VCC Program Current | 1,4,7 | 8 | 20 | mA | VPP = VPPH1 (3V) Program in Progress |
|  |  |  | 8 | 20 | mA | VPP = VPPH2 (12V) Program in Progress |
| ICCE | VCC Erase Current | 1,4,7 | 8 | 20 | mA | VPP = VPPH1 (3V) Erase in Progress |
|  |  |  | 8 | 20 | mA | VPP = VPPH2 (12V) Erase in Progress |
| ICCES | VCC Erase Suspend Current | 1,2,4,7 | 20 | 50 | µA | CE# = VIH Erase Suspend in Progress |
| ICCWS | VCC Program Suspend Current | 1,2,4,7 | 20 | 50 | µA | CE# = VIH Program Suspend in Progress |
| IPPD | VPP Deep Power-Down Current | 1 | 0.2 | 5 | µA | RP# = GND ± 0.2V |
| IPPR | VPP Read Current | 1 | 2 | ±50 | µA | VPP VCC |

30                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 10. DC Characteristics (Continued)

| Sym | Parameter | Notes | VCC = 2.7V–3.6V |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Typ | Max |  |  |
| IPPW | VPP Program Current | 1,4 | 15 | 40 | mA | VPP = VPPH1 (3V) Program in Progress |
|  |  |  | 10 | 25 | mA | VPP = VPPH2 (12V) Program in Progress |
| IPPE | VPP Erase Current | 1,4 | 13 | 25 | mA | VPP = VPPH1 (3V) Erase in Progress |
|  |  |  | 8 | 25 | mA | VPP = VPPH2 (12V) Erase in Progress |
| IPPES | VPP Erase Suspend Current | 1,4 | 50 | 200 | µA | VPP = VPPH1 or VPPH2 Erase Suspend in Progress |
| IPPWS | VPP Program Suspend Current | 1,4 | 50 | 200 | µA | VPP = VPPH1 or VPPH2 Program Suspend in Progress |

PRELIMINARY                                                                                     31

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

Table 10. DC Characteristics (Continued)

| Sym | Parameter | Notes | VCC = 2.7V–3.6V |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |  |
| VIL | Input Low Voltage |  | –0.4 | 0.4 | V |  |
| VIH | Input High Voltage |  | VCCQ – 0.4V |  | V |  |
| VOL | Output Low Voltage |  |  | 0.10 | V | VCC = VCCMin = VCCQMin IOL = 100 A |
| VOH | Output High Voltage |  | VCCQ – 0.1V |  | V | VCC = VCCMin = VCCQMin IOH = –100 A |
| VPPLK | VPP Lock-Out Voltage | 3 | 1.5 |  | V | Complete Write Protection |
| VPPH1 | VPP during Prog/Erase Operations | 3 | 2.7 | 3.6 | V |  |
| VPPH2 |  | 3,6 | 11.4 | 12.6 | V |  |
| VLKO | VCC Program/Erase Lock Voltage |  | 1.5 |  | V |  |
| VLKO2 | VCCQ Program/Erase Lock Voltage |  | 1.2 |  | V |  |

NOTES:

1.    All currents are in RMS unless otherwise noted. Typical values at nominal VCC, TA = +25°C.

2.    ICCES and ICCWS are specified with device de-selected. If device is read while in erase suspend, current draw is ICCR. If the device is read while in program suspend, current draw is ICCR.

3.    Erase and Program are inhibited when VPP < VPPLK and not guaranteed outside the valid VPP ranges of VPPH1 and VPPH2.

4.    Sampled, not 100% tested.

5.    Automatic Power Savings (APS) reduces ICCR to approximately standby levels in static operation (CMOS inputs).

6.    Applying VPP = 11.4V–12.6V during program/erase can only be done for a maximum of 1000 cycles on the main blocks

and 2500 cycles on the parameter blocks. VPP may be connected to 12V for a total of 80 hours maximum. See Section 3.4 for details.

7.    Includes the sum of VCC and VCCQ current.

Table 11. Capacitance (TA = 25°C, f = 1 MHz)

| Sym | Parameter | Notes | Typ | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| CIN | Input Capacitance | 1 | 6 | 8 | pF | VIN = 0V |
| COUT | Output Capacitance | 1 | 10 | 12 | pF | VOUT = 0V |

NOTE:

1.    Sampled, not 100% tested.

32                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

NOTE:

VCCQ

0.0

INPUT

VCCQ

2

TEST POINTS

VCCQ

2

OUTPUT

0605-011

AC test inputs are driven at VCCQ for a logic “1” and 0.0V for a logic “0.” Input timing begins, and output timing ends, at VCCQ/2. Input rise and fall times (10%–90%) <10 ns. Worst case speed conditions are when VCCQ=2.7V.

Figure 10. 2.7V–3.6V Input Range and Measurement Points

VCCQ

| Test Configuration Component Values for Worst Case Speed Conditions Test Configuration | CL (pF) | R1 ( ) | R2 ( ) |
| --- | --- | --- | --- |
| 2.7V Standard Test | 50 | 25K | 25K |

R 1

Device under Test

CL

R 2

Out

NOTE:

See table for component values.

Figure 11. Test Configuration

0605-012

PRELIMINARY                                                                                     33

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

6.0     OPERATING CONDITIONS (VCCQ = 1.8V–2.2V)

Table 12. Temperature and VCC Operating Conditions

| Symbol | Parameter | Notes | Min | Max | Units |
| --- | --- | --- | --- | --- | --- |
| TA | Operating Temperature |  | –40 | +85 | °C |
| VCC1 | 2.7V–2.85V VCC Supply Voltage | 1 | 2.7 | 2.85 | Volts |
| VCC2 | 2.7V–3.3V VCC Supply Voltage | 1 | 2.7 | 3.3 | Volts |
| VCCQ | 1.8V–2.2V I/O Supply Voltage | 1,4 | 1.8 | 2.2 | Volts |
| VPP1 | Program and Erase Voltage | 1 | 2.7 | 2.85 | Volts |
| VPP2 |  | 1 | 2.7 | 3.3 | Volts |
| VPP3 |  | 1,2 | 11.4 | 12.6 | Volts |
| Cycling | Block Erase Cycling | 3 | 10,000 |  | Cycles |

NOTES:

1.    See DC Characteristics tables for voltage range-specific specifications.

2.    Applying VPP = 11.4V–12.6V during program/erase can only be done for a maximum of 1000 cycles on the main blocks and 2500 cycles on the parameter. VPP may be connected to 12V for a total of 80 hours maximum. See Section 3.4 for details.

3.    For operating temperatures of –25 C– +85 C the device is projected to have a minimum block erase cycling of 10,000 to

30,000 cycles.

4.    The voltage swing on the inputs, VIN is required to match VCCQ.

6.1       DC Characteristics: VCCQ = 1.8V–2.2V

These tables are valid for the following power supply combinations only:

1.   VCC1 and VCCQ and (VPP1 or VPP3)

2.   VCC2 and VCCQ and (VPP2 or VPP3)

Wherever the input voltage VIN is mentioned, it is required that VIN matches the chosen VCCQ.

34                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 13. DC Characteristics: VCCQ = 1.8V–2.2V

| Sym | Parameter | Notes | VCC1: 2.7V–2.85V VCC2: 2.7V–3.3V |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Typ | Max |  |  |
| ILI | Input Load Current | 1 |  | ± 1.0 | µA | VCC = VCCMax VCCQ = VCCQMax VIN = VCCQ or GND |
| ILO | Output Leakage Current | 1 |  | ± 10 | µA | VCC = VCC Max VCCQ = VCCQMax VIN = VCCQ or GND |
| ICCS | VCC Standby Current | 1,7 | 20 | 50 | µA | CMOS INPUTS VCC = VCC1 Max (2.7V–2.85V) VCCQ = VCCQMax CE# = RP# = VCCQ |
|  |  |  | 150 | 250 | A | CMOS INPUTS VCC = VCC2 Max (2.7V–3.3V) VCCQ = VCCQMax CE# = RP# = VCCQ |
| ICCD | VCC Deep Power-Down Current | 1,7 | 1 | 10 | µA | CMOS INPUTS VCC = VCCMax (VCC1 or VCC2) VCCQ = VCCQMax VIN = VCCQ or GND RP# = GND ± 0.2V |
| ICCR | VCC Read Current | 1,5,7 | 8 | 18 | mA | CMOS INPUTS VCC = VCC1Max (2.7V–2.85V) VCCQ = VCCQMax OE# = VIH , CE# = VIL f = 5 MHz, IOUT = 0 mA Inputs = VIL or VIH |
|  |  |  | 12 | 23 | mA | CMOS INPUTS VCC = VCC2Max (2.7V–3.3V) VCCQ = VCCQMax OE# = VIH , CE# = VIL f = 5 MHz, IOUT = 0 mA Inputs = GND ± 0.2V or VCCQ |

PRELIMINARY                                                                                     35

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

Table 13. DC Characteristics: VCCQ = 1.8V–2.2V (Continued)

| Sym | Parameter | Notes | VCC1: 2.7V–2.85V VCC2: 2.7V–3.3V |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Typ | Max |  |  |
| ICCW | VCC Program Current | 1,4,7 | 8 | 20 | mA | VPP = VPPH1 or VPPH2 Program in Progress |
|  |  |  | 8 | 20 | mA | VPP = VPPH3 (12V) Program in Progress |
| ICCE | VCC Erase Current | 1,4,7 | 8 | 20 | mA | VPP = VPPH1 or VPPH2 Erase in Progress |
|  |  |  | 8 | 20 | mA | VPP = VPPH3 (12V) Erase in Progress |
| ICCES | VCC Erase Suspend Current | 1,2,4,7 | 20 | 50 | µA | CE# = VIH Erase Suspend in Progress |
| ICCWS | VCC Program Suspend Current | 1,2,4,7 | 20 | 50 | µA | CE# = VIH Program Suspend in Progress |
| IPPD | VPP Deep Power-Down Current | 1 | 0.2 | 5 | µA | RP# = GND ± 0.2V |
| IPPR | VPP Read and Standby Current | 1 | 2 | ±50 | µA | VPP VCC |
| IPPW | VPP Program Current | 1,4 | 15 | 40 | mA | VPP = VPPH1 or VPPH2 Program in Progress |
|  |  |  | 10 | 25 | mA | VPP = VPPH3 (12V) Program in Progress |
| IPPE | VPP Erase Current | 1,4 | 13 | 25 | mA | VPP = VPPH1 or VPPH2 Erase in Progress |
|  |  |  | 8 | 25 | mA | VPP = VPPH3 (12V) Erase in Progress |
| IPPES | VPP Erase Suspend Current | 1 | 50 | 200 | µA | VPP = VPPH1 , VPPH2 , or VPPH3 Erase Suspend in Progress |
| IPPWS | VPP Program Suspend Current | 1 | 50 | 200 | µA | VPP = VPPH1 , VPPH2 , or VPPH3 Program Suspend in Progress |

36                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 13. DC Characteristics: VCCQ = 1.8V–2.2V (Continued)

| Sym | Parameter | Notes | VCC1: 2.7V–2.85V VCC2: 2.7V–3.3V |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Typ | Max |  |  |
| VIL | Input Low Voltage |  | –0.2 | 0.2 | V |  |
| VIH | Input High Voltage |  | VCCQ – 0.2V |  | V |  |
| VOL | Output Low Voltage |  | –0.10 | 0.10 | V | VCC = VCCMin VCCQ = VCCQMin IOL = 100 A |
| VOH | Output High Voltage |  | VCCQ – 0.1V |  | V | VCC = VCCMin VCCQ = VCCQMin IOH = –100 A |
| VPPLK | VPP Lock-Out Voltage | 3 | 1.5 |  | V | Complete Write Protection |
| VPPH1 | VPP during Prog./Erase Operations | 3 | 2.7 | 2.85 | V |  |
| VPPH2 |  | 3 | 2.7 | 3.3 | V |  |
| VPPH3 |  | 3,6 | 11.4 | 12.6 | V |  |
| VLKO1 | VCC Program/Erase Lock Voltage |  | 1.5 |  | V |  |
| VLKO2 | VCCQ Program/Erase Lock Voltage |  | 1.2 |  | V |  |

NOTES:

1.    All currents are in RMS unless otherwise noted. Typical values at nominal VCC, TA = +25°C.

2.    ICCES and ICCWS are specified with device de-selected. If device is read while in erase suspend, current draw is ICCR. If the device is read while in program suspend, current draw is ICCR.

3.    Erases and Writes inhibited when VPP < VPPLK, and not guaranteed outside the valid VPP ranges of VPPH1,VPPH2. or VPPH3.

4.    Sampled, not 100% tested.

5.    Automatic Power Savings (APS) reduces ICCR to approximately standby levels in static operation (CMOS inputs).

6.    Applying VPP = 11.4V–12.6V during program/erase can only be done for a maximum of 1000 cycles on the main blocks

and 2500 cycles on the parameter blocks. VPP may be connected to 12V for a total of 80 hours maximum. See Section 3.4 for details.

7     Includes the sum of VCC and VCCQ current

Table 14. Capacitance (TA = 25°C, f = 1 MHz)

| Sym | Parameter | Notes | Typ | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| CIN | Input Capacitance | 1 | 6 | 8 | pF | VIN = 0V |
| COUT | Output Capacitance | 1 | 10 | 12 | pF | VOUT = 0V |

NOTE:

1.    Sampled, not 100% tested.

PRELIMINARY                                                                                     37

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

NOTE:

VCCQ

0.0

INPUT

VCCQ

2

TEST POINTS

VCCQ

2

OUTPUT

0605-011

AC test inputs are driven at VCCQ for a logic “1” and 0.0V for a logic “0.” Input timing begins, and output timing ends, at VCCQ/2. Input rise and fall times (10%–90%) <10 ns. For worst case speed conditions VCCQ=1.8V.

Figure 12. 1.8V—2.2V Input Range and Measurement Points

VCCQ

| Test Configuration Component Values for Worst Case Speed Conditions Test Configuration | CL (pF) | R1 ( ) | R2 ( ) |
| --- | --- | --- | --- |
| 1.8V Standard Test | 50 | 16.7K | 16.7K |

R 1

Device under Test

CL

R 2

Out

NOTE:

CL includes jig capacitance.

NOTE:

See table for component values.

Figure 13. Test Configuration

0605-012

38                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

7.0     AC CHARACTERISTICS

AC Characteristics are applicable to both VCCQ ranges.

Table 15. AC Characteristics: Read Operations (Extended Temperature)

| # | Symbol | Parameter | Load | CL = 50 pF |  |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC | 2.7V–3.6V4 |  |  |  |  |
|  |  |  | Prod | 120 ns |  | 150 ns |  |  |
|  |  |  | Notes | Min | Max | Min | Max |  |
| R1 | tAVAV | Read Cycle Time |  | 120 |  | 150 |  | ns |
| R2 | tAVQV | Address to Output Delay |  |  | 120 |  | 150 | ns |
| R3 | tELQV | CE# to Output Delay | 2 |  | 120 |  | 150 | ns |
| R4 | tGLQV | OE# to Output Delay | 2 |  | 65 |  | 65 | ns |
| R5 | tPHQV | RP# to Output Delay |  |  | 600 |  | 600 | ns |
| R6 | tELQX | CE# to Output in Low Z | 3 | 0 |  | 0 |  | ns |
| R7 | tGLQX | OE# to Output in Low Z | 3 | 0 |  | 0 |  | ns |
| R8 | tEHQZ | CE# to Output in High Z | 3 |  | 40 |  | 40 | ns |
| R9 | tGHQZ | OE# to Output in High Z | 3 |  | 40 |  | 40 | ns |
| R10 | tOH | Output Hold from Address, CE#, or OE# Change, Whichever Occurs First | 3 | 0 |  | 0 |  | ns |

NOTES:

1.    See AC Input/Output Reference Waveform for timing measurements.

2.    OE# may be delayed up to tELQV–tGLQV after the falling edge of CE# without impact on tELQV.

3.    Sampled, but not 100% tested.

4.    See Test Configuration (Figures 11 and 13), 2.7V–3.6V and 1.8V–2.2V Standard Test component values.

PRELIMINARY                                                                                     39

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

VIH

ADDRESSES (A)

VIL

VIH

Device and

Address Selection

Address Stable

Data

Valid                         Standby

R1

CE# (E)

OE# (G)

VIL

R8

VIH

VIL

R9

VIH

WE# (W)

VIL

VOH DATA (D/Q) VOL

VIH

High Z

R7        R4

R3

R6

Valid Output

R2

R10

High Z

RP#(P)

R5

VIL

0605-015

Figure 14. AC Waveform: Read Operations

40                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

Table 16. AC Characteristics: Write Operations (Extended Temperature)1

| # | Symbol | Parameter | Load | 50 pF |  |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC | 2.7V–3.6V5 |  | 2.7V-3.6V5 |  |  |
|  |  |  | Prod | 120 ns |  | 150 ns |  |  |
|  |  |  | Notes | Min | Max | Min | Max |  |
| W1 | tPHWL tPHEL | RP# High Recovery to WE# (CE#) Going Low |  | 600 |  | 600 |  | ns |
| W2 | tELWL tWLEL | CE# (WE#) Setup to WE# (CE#) Going Low |  | 0 |  | 0 |  | ns |
| W3 | tWLWH tELEH | WE# (CE#) Pulse Width |  | 90 |  | 90 |  | ns |
| W4 | tDVWH tDVEH | Data Setup to WE# (CE#) Going High | 3 | 70 |  | 70 |  | ns |
| W5 | tAVWH tAVEH | Address Setup to WE# (CE#) Going High | 2 | 90 |  | 90 |  | ns |
| W6 | tWHEH tEHWH | CE# (WE#) Hold Time from WE# (CE#) High |  | 0 |  | 0 |  | ns |
| W7 | tWHDX tEHDX | Data Hold Time from WE# (CE#) High | 3 | 0 |  | 0 |  | ns |
| W8 | tWHAX tEHAX | Address Hold Time from WE# (CE#) High | 2 | 0 |  | 0 |  | ns |
| W9 | tWHWL tEHEL | WE# (CE#) Pulse Width High |  | 30 |  | 30 |  | ns |
| W10 | tVPWH tVPEH | VPP Setup to WE# (CE#) Going High | 4 | 200 |  | 200 |  | ns |
| W11 | tQVVL | VPP Hold from Valid SRD | 4 | 0 |  | 0 |  | ns |
|  | tLOCK | Block Unlock / Lock Delay | 4, 6 |  | 200 |  | 200 | ns |

NOTES:

1.    Read timing characteristics during program suspend and erase suspend are the same as during read-only operations.

Refer to AC Characteristics during read mode.

2.    Refer to command definition table for valid AIN (Table6).

3.    Refer to command definition table for valid DIN (Table 6).

4.    Sampled, but not 100% tested.

5.    See Test Configuration (Figure 11 and 13), 2.7V–3.6V and 1.8V–2.2V Standard Test component values.

6.    Time tLOCK is required for successful locking and unlocking of all lockable blocks.

PRELIMINARY                                                                                     41

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

VIH

A              B                   C                   D                   E                   F

ADDRESSES [A]

CE#(WE#) [E(W)] OE# [G]

WE#(CE#) [W(E)]

VIL

VIH

VIL

VIH    W2

VIL

VIH

AIN

AIN

W5             W8

W6

W9

(Note 1)

(Note 1)

DATA [D/Q]

VIL

VIH

VIL

High Z

W1

DIN

W3

W4

W7

DIN

Valid

SRD

DIN

NOTES:

RP# [P]

WP#

V    [V]

PP

VIH VIL VIH VIL

VPPH 2

VPPH1

VPPLK

VIL

W10                                         W11

0605-016

1.    CE# must be toggled low when reading Status Register Data. WE# must be inactive (high) when reading Status Register

Data.

A.   VCC Power-Up and Standby.

B.   Write Program or Erase Setup Command.

C.   Write Valid Address and Data (for Program) or Erase Confirm Command. D.   Automated Program or Erase Delay.

E.   Read Status Register Data (SRD): reflects completed program/erase operation.

F.    Write Read Array Command.

Figure 15. AC Waveform: Program and Erase Operations

42                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

7.1       Reset Operations

RP# (P)

VIH

VIL

t PLPH

tPHQV tPHWL tPHEL

(A) Reset during Read Mode

VIH

RP# (P)

VIL

Abort

Complete

t PLRH

t PLPH

t PHQV t PHWL t PHEL

(B) Reset during Program or Block Erase, t PLPH   < t PLRH

Abort

Deep

VIH

RP# (P)

VIL

Complete  Power- Down

t PLRH

t PLPH

t PHQV t PHWL t PHEL

(C) Reset Program or Block Erase, t PLPH  > t PLRH

0605-17

Figure 16.  AC Waveform: Deep Power-Down/Reset Operation

Reset Specifications

| Symbol | Parameter | Notes | VCC = 2.7–3.6V |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| tPLPH | RP# Low to Reset during Read (If RP# is tied to VCC, this specification is not applicable) | 1,3 | 100 |  | ns |
| tPLRH | RP# Low to Reset during Block Erase or Program | 2,3 |  | 22 | µs |

NOTES:

1.    If tPLPH is < 100 ns the device may still RESET but this is not guaranteed.

2.    If RP# is asserted while a block erase or byte program operation is not executing, the reset will complete within 100 ns.

3.    Sampled but not 100% tested.

PRELIMINARY                                                                                     43

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

Table 17. Erase and Program Timings

| Sym | Parameter | Notes | VPP = 2.7V |  | VPP = 12V |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Typ1 | Max3 | Typ1 | Max3 |  |
| tBWPB | Block Program Time (Parameter) | 2 | .16 | .48 | .08 | .24 | sec |
| tBWMB | Block Program Time (Main) | 2 | 1.23 | 3.69 | .58 | 1.74 | sec |
| tWHQV1 tEHQV1 | Program Time | 2 | 17 | 165 | 8 | 185 | µs |
| tWHQV2 tEHQV2 | Block Erase Time (Parameter) | 2 | 1 | 5.0 | 0.8 | 4.8 | sec |
| tWHQV3 tEHQV3 | Block Erase Time (Main) | 2 | 1.8 | 8.0 | 1.1 | 7.0 | sec |
| tWHRH1 tEHRH1 | Program Suspend Latency | 3 | 5 | 10 | 5 | 10 | µs |
| tWHRH2 tEHRH2 | Erase Suspend Latency | 3 | 5 | 20 | 6 | 12 | µs |

NOTES:

1.    Typical values measured at TA = +25°C and nominal voltages.

2.    Excludes external system-level overhead.

3.    Sampled but not 100% tested.

44                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

APPENDIX A ORDERING INFORMATION

T E 2 8 F 1 6 0 B 3 T 1 2 0

Package

TE = 40-Lead  TSOP

GT = 48-Ball µBGA* CSP

Product line designator

for all Intel Flash products

Access Speed (ns)

(120, 150)

T = Top Blocking

B = Bottom Blocking

Device Density

016 = x8 (16-Mbit)

008 = x8 (8-Mbit)

Product Family

B3 = Smart 3 Advanced  Boot Block

VCC  = 2.7V - 3.6V

VPP  = 2.7V - 3.6V or 11.4V - 12.6V

VALID COMBINATIONS

40-Lead TSOP                        48-Ball   BGA* CSP

Extended                16M         TE28F016B3T120                   GT28F016B3T120

TE28F016B3B120                  GT28F016B3B120

TE28F016B3T150                   GT28F016B3T150

TE28F016B3B150                  GT28F016B3B150

Extended                8M           TE28F008B3T120                   GT28F008B3T120

TE28F008B3B120                  GT28F008B3B120

TE28F008B3T150                   GT28F008B3T150

TE28F008B3B150                  GT28F008B3B150

PRELIMINARY                                                                                     45

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

APPENDIX B

WRITE STATE MACHINE CURRENT/NEXT STATES

|  |  |  | Command Input (and Next State) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Current State | SR.7 | Data When Read | Read Array (FFH) | Program Setup (40/10H) | Erase Setup (20H) | Erase Confirm (D0H) | Program / Erase Susp. (B0H) | Program / Erase Resume (D0) | Read Status (70H) | Clear Status (50H) | Read ID (90H) |
| Read Array | “1” | Array | Read Array | Program Setup | Erase Setup | Read Array |  |  | Read Status | Read Array | Read Identifier |
| Program Setup | “1” | Status | 1 Pgm. | Program (Command input = Data to be programmed) |  |  |  |  |  |  |  |
| Program (Not Comp.) | “0” | Status | Program |  |  |  | Pgm Susp. to Status | Program |  |  |  |
| Program (Complete) | “1” | Status | Read Array | Program Setup | Erase Setup | Read Array |  |  | Read Status | Read Array | Read Identifier |
| Program Suspend to Status | “1” | Status | Prog. Susp. to Array | Program Suspend to Array |  | Program | Program Susp. to Array | Program | Prog. Susp. to Status | Program Suspend to Array |  |
| Program Suspend to Array | “1” | Array | Prog. Susp. to Array | Program Suspend to Array |  | Program | Program Susp. to Array | Program | Prog. Susp. to Status | Prog. Susp. to Array | Prog. Susp. to Array |
| Erase Setup | “1” | Status | Erase Command Error |  |  | Erase | Erase Cmd. Err. | Erase | Erase Command Error |  |  |
| Erase Cmd. Error | “1” | Status | Read Array | Program Setup | Erase Setup | Read Array |  |  | Read Status | Read Array | Read Identifier |
| Erase (Not Comp) | “0” | Status | Erase |  |  |  | Ers. Susp. to Status | Erase |  |  |  |
| Erase (Complete) | “1” | Status | Read Array | Program Setup | Erase Setup | Read Array |  |  | Read Status | Read Array | Read Identifier |
| Erase Suspend to Status | “1” | Status | Erase Susp. to Array | Program Setup | Erase Susp. to Array | Erase | Erase Susp. to Array | Erase | Erase Susp. to Status | Erase Suspend to Array |  |
| Erase. Susp. to Array | “1” | Array | Erase Susp. to Array | Program Setup | Erase Susp. to Array | Erase | Erase Susp. to Array | Erase | Erase Susp. to Status | Erase Suspend to Array |  |
| Read Status | “1” | Status | Read Array | Program Setup | Erase Setup | Read Array |  |  | Read Status | Read Array | Read Identifier |
| Read Identifier | “1” | ID | Read Array | Program Setup | Erase Setup | Read Array |  |  | Read Status | Read Array | Read Identifier |

1.    You cannot program “1”s to the flash. Writing FFH following the Program Setup will initiate the internal program algorithm

of the WSM. Although the algorithm will execute, array data is not changed. The WSM returns to read status mode without reporting any error. Assuming VPP > VPPLK writing a second FFH while in read status mode will return the flash to read array mode.

46                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

APPENDIX C

ACCESS TIME VS. CAPACITIVE LOAD (tAVQV vs. CL)

124

123

Access Time(ns)

122

121

120

119

118

117

116

115

Access Time vs. Load Capacitance

Derating Curve

30                          50                         70                         100

Load Capacitance (pF)

Smart 3 Advanced Boot

Block

NOTE:

VCCQ = 2.7V

This chart shows a derating curve for device access time with respect to capacitive load. The value in the

DC characteristics section of the specification corresponds to C L = 50 pF.

NOTE:

1.    Sampled, but not 100% tested

PRELIMINARY                                                                                     47

SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE                                                 E

APPENDIX D ARCHITECTURE BLOCK DIAGRAM

DQ0-DQ7

VCCQ

Output Buffer

Input Buffer

Output

Multiplexer

Identifier

Data

Register

Register

Power

Reduction

Status

Register

Data

Command User Interface

I/O Logic

CE# WE# OE#

A0-A20

Input Buffer

Control

Y-Decoder

Comparator

8-Kbyte

Parameter Block

64-Kbyte

Main Block

Y-Gating/Sensing                                        Write State

64-Kbyte

Main Block

Machine

RP#

WP# Program/Erase

Voltage Switch

VPP

Address

Latch

8-Kbyte

Parameter Block

X-Decoder

VCC

Address

Counter

GND

48                                                                                                                                 PRELIMINARY

E       SMART 3 ADVANCED BOOT BLOCK–BYTE-WIDE

APPENDIX E ADDITIONAL INFORMATION(1,2)

| Order Number | Document/Tool |
| --- | --- |
| 210830 | 1997 Flash Memory Databook |
| 290580 | Smart 3 Advanced Boot Block Word-Wide 4-Mbit (256K x 16), 8-Mbit (512K x16), 16-Mbit (1024K x16) Flash Memory Family Datasheet |
| 292172 | AP-617 Additional Flash Data Protection Using VPP, RP# and WP# |

NOTE:

1.    Please call the Intel Literature Center at (800) 548-4725 to request Intel documentation. International customers should contact their local Intel or distribution sales office.

2.    Visit Intel’s World Wide Web home page at http://www.Intel.com for technical documentation and tools.

PRELIMINARY                                                                                     49