IS61C1024AL IS64C1024AL

128K x 8	HIGH-SPEED CMOS STATIC RAM

ISSI®

JANUARY 2005

FEATURES

•  High-speed access time: 12, 15 ns

•  Low active power: 160 mW (typical)

•  Low standby power: 1000 µW (typical) CMOS

standby

•  Output Enable (OE) and two Chip Enable

(CE1 and CE2) inputs for ease in applications

•  Fully static operation: no clock or refresh required

•  TTL compatible inputs and outputs

•  Single 5V (±10%) power supply

•  Commercial, industrial, and automotive tempera- ture ranges available

•  Lead free available

DESCRIPTION

The ISSI IS61C1024AL/IS64C1024AL is a very high-speed, low power, 131,072-word by 8-bit CMOS static RAMs. They are fabricated using ISSI's high-performance CMOS technology. This highly reliable process coupled with innovative circuit design techniques, yields higher performance and low power consumption devices.

When CE1 is HIGH or CE2 is LOW (deselected), the device assumes a standby mode at which the power dissipation can be reduced by using CMOS input levels.

Easy memory expansion is provided by using two Chip Enable inputs, CE1 and CE2. The active LOW Write Enable (WE) controls both writing and reading of the memory.

The IS61C1024AL/IS64C1024AL is available in 32-pin 300- mil SOJ, 32-pin 400-mil SOJ, 32-pin TSOP (Type I, 8x20), and 32-pin sTSOP (Type I, 8 x 13.4) packages.

FUNCTIONAL BLOCK DIAGRAM

A0-A16

DECODER

128K x 8

MEMORY ARRAY

VDD

GND

I/O0-I/O7

I/O DATA CIRCUIT

COLUMN I/O

CE1

CE2

OE WE

CONTROL CIRCUIT

Copyright © 2005 Integrated Silicon Solution, Inc.  All rights reserved.  ISSI reserves the right to make changes to this specification and its products at any time without notice.   ISSI assumes no liability arising out of the application or use of any information, products or services described herein. Customers are advised to obtain the latest version of this device specification before relying on any published information and before placing orders for products.

®

PIN CONFIGURATION

| 32-Pin SOJ NC | 1 | 32 | VDD |
| --- | --- | --- | --- |
| A16 | 2 | 31 | A15 |
| A14 | 3 | 30 | CE2 |
| A12 | 4 | 29 | WE |
| A7 | 5 | 28 | A13 |
| A6 | 6 | 27 | A8 |
| A5 | 7 | 26 | A9 |
| A4 | 8 | 25 | A11 |
| A3 | 9 | 24 | OE |
| A2 | 10 | 23 | A10 |
| A1 | 11 | 22 | CE1 |
| A0 | 12 | 21 | I/O7 |
| I/O0 | 13 | 20 | I/O6 |
| I/O1 | 14 | 19 | I/O5 |
| I/O2 | 15 | 18 | I/O4 |
| GND | 16 | 17 | I/O3 |

PIN CONFIGURATION

32-Pin TSOP (Type 1) (T) and sTSOP (Type 1) (H)

| A11 | 1 | 32 | OE |
| --- | --- | --- | --- |
| A9 | 2 | 31 | A10 |
| A8 | 3 | 30 | CE1 |
| A13 | 4 | 29 | I/O7 |
| WE | 5 | 28 | I/O6 |
| CE2 | 6 | 27 | I/O5 |
| A15 | 7 | 26 | I/O4 |
| VDD | 8 | 25 | I/O3 |
| NC | 9 | 24 | GND |
| A16 | 10 | 23 | I/O2 |
| A14 | 11 | 22 | I/O1 |
| A12 | 12 | 21 | I/O0 |
| A7 | 13 | 20 | A0 |
| A6 | 14 | 19 | A1 |

A5	15

A4	16

18	A2

17	A3

PIN DESCRIPTIONS

A0-A16	Address Inputs

CE1 	 Chip Enable 1 Input CE2	 Chip Enable 2 Input OE	 Output Enable Input WE	 Write Enable Input I/O0-I/O7	Input/Output

VDD	Power

GND	Ground

OPERATING RANGE (IS61C1024AL)

Range	Ambient Temperature	VDD Commercial	0°C to +70°C	5V ± 10% Industrial	-40°C to +85°C	5V ± 10%

OPERATING RANGE (IS64C1024AL)

| Range | Ambient Temperature | VDD |
| --- | --- | --- |
| Automotive | -40°C to +125°C | 5V ± 10% |

| TRUTH TABLE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mode | WE | CE1 | CE2 | OE | I/O Operation | VDD Current |
| Not Selected | X | H | X | X | High-Z | ISB1, ISB2 |
| (Power-down) | X | X | L | X | High-Z | ISB1, ISB2 |
| Output Disabled | H | L | H | H | High-Z | ICC1, ICC2 |
| Read | H | L | H | L | DOUT | ICC1, ICC2 |
| Write | L | L | H | X | DIN | ICC1, ICC2 |

®

ABSOLUTE MAXIMUM RATINGS(1)

| Notes: Symbol | Parameter | Value | Unit |
| --- | --- | --- | --- |
| VTERM | Terminal Voltage with Respect to GND | –0.5 to +7.0 | V |
| TSTG | Storage Temperature | –65 to +150 | °C |
| PT | Power Dissipation | 1.5 | W |
| IOUT | DC Output Current (LOW) | 20 | mA |

1. Stress greater than those listed under ABSOLUTE MAXIMUM RATINGS may cause

permanent damage to the device. This is a stress rating only and functional operation of the device at these or any other conditions above those indicated in the opera- tional sections of this specification is not implied. Exposure to absolute maximum rating conditions for extended periods may affect reliability.

CAPACITANCE(1,2)

| Symbol | Parameter | Conditions | Max. | Unit |
| --- | --- | --- | --- | --- |
| CIN | Input Capacitance | VIN = 0V | 5 | pF |
| COUT | Output Capacitance | VOUT = 0V | 7 | pF |

Notes:

1. Tested initially and after any design or process changes that may affect these parameters.

2. Test conditions: TA = 25°C, f = 1 MHz, VDD = 5.0V.

DC ELECTRICAL CHARACTERISTICS (Over Operating Range)

| Symbol | Parameter | Test Conditions |  | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VOH | Output HIGH Voltage | VDD = Min., IOH = –4.0 mA |  | 2.4 | — | V |
| VOL | Output LOW Voltage | VDD = Min., IOL = 8.0 mA |  | — | 0.4 | V |
| VIH | Input HIGH Voltage |  |  | 2.2 | VDD + 0.5 | V |
| VIL | Input LOW Voltage(1) |  |  | –0.3 | 0.8 | V |
| ILI | Input Leakage | GND  VIN  VDD | Com. | –1 | 1 | µA |
|  |  |  | Ind. | –2 | 2 |  |
|  |  |  | Auto. | –5 | 5 |  |
| ILO | Output Leakage | GND  VOUT  VDD | Com. | –1 | 1 | µA |
|  |  | Outputs Disabled | Ind. | –2 | 2 |  |
|  |  |  | Auto. | –5 | 5 |  |

Note:

1. VIL = –3.0V for pulse width less than 10 ns.

®

IS61C1024AL/IS64C1024AL POWER SUPPLY CHARACTERISTICS(1) (Over Operating Range)

-12 ns	-15 ns

| Symbol | Parameter | Test Conditions |  | Min. | Max. | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ICC1 | VDD Operating | VDD = VDD MAX., CE1 = VIL | Com. | — | 35 |  |  | mA |
|  | Supply Current | IOUT = 0 mA, f = 0 | Ind. | — | 40 |  |  |  |
|  |  |  | Auto. |  |  | — | 45 |  |
| ICC2 | VDD Dynamic Operating | VDD = VDD MAX., CE1 = VIL | Com. | — | 45 |  |  | mA |
|  | Supply Current | IOUT = 0 mA, f = fMAX | Ind. | — | 50 |  |  |  |
|  |  |  | Auto. |  |  | — | 55 |  |
|  |  |  | typ.(2) | — | 32 |  |  |  |
| ISB1 | TTL Standby Current | VDD = VDD MAX., | Com. | — | 1 |  |  | mA |
|  | (TTL Inputs) | VIN = VIH or VIL | Ind. | — | 1.5 |  |  |  |
|  |  | CE1  VIH, f = 0 or | Auto. |  |  | — | 2 |  |
|  |  | CE2  VIL, f = 0 |  |  |  |  |  |  |
| ISB2 | CMOS Standby | VDD = VDD MAX., | Com. | — | 400 |  |  | µA |
|  | Current (CMOS Inputs) | CE1  VDD – 0.2V, | Ind. | — | 450 |  |  |  |
|  |  | CE2  0.2V | Auto. |  |  | — | 500 |  |
|  |  | VIN  VDD – 0.2V, or | typ.(2) | — | 200 |  |  |  |
|  |  | VIN  0.2V, f = 0 |  |  |  |  |  |  |

Note:

1. At f = fMAX, address and data inputs are cycling at the maximum frequency, f = 0 means no input lines change.

2.  Typical Values are measured at VDD = 5V, TA = 25oC and not 100% tested.

®

READ CYCLE SWITCHING CHARACTERISTICS(1) (Over Operating Range)

-12	-15

| Symbol | Parameter | Min. | Max. | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| tRC | Read Cycle Time | 12 | — | 15 | — | ns |
| tAA | Address Access Time | — | 12 | — | 15 | ns |
| tOHA | Output Hold Time | 3 | — | 3 | — | ns |
| tACE1 | CE1 Access Time | — | 12 | — | 15 | ns |
| tACE2 | CE2 Access Time | — | 12 | — | 15 | ns |
| tDOE | OE Access Time | — | 6 | — | 7 | ns |
| tLZOE(2) | OE to Low-Z Output | 0 | — | 0 | — | ns |
| tHZOE(2) | OE to High-Z Output | 0 | 6 | 0 | 6 | ns |
| tLZCE1(2) | CE1 to Low-Z Output | 2 | — | 2 | — | ns |
| tLZCE2(2) | CE2 to Low-Z Output | 2 | — | 2 | — | ns |
| tHZCE(2) | CE1 or CE2 to High-Z Output | 0 | 7 | 0 | 8 | ns |
| tPU(3) | CE1 or CE2 to Power-Up | 0 | — | 0 | — | ns |
| tPD(3) | CE1 or CE2 to Power-Down | — | 12 | — | 12 | ns |

Notes:

1. Test conditions assume signal transition times of 3 ns or less, timing reference levels of 1.5V, input pulse

levels of 0 to 3.0V and output loading specified in Figure 1.

2. Tested with the load in Figure 2.  Transition is measured ±500 mV from steady-state voltage. Not 100% tested.

3. Not 100% tested.

AC TEST CONDITIONS

Parameter		 Unit Input Pulse Level	0V to 3.0V Input Rise and Fall Times		 3 ns Input and Output Timing		1.5V

and Reference Level

Output Load	See Figures 1 and 2

AC TEST LOADS

480 

5V

480 

5V

OUTPUT

OUTPUT

30 pF Including jig and scope

255 

5 pF Including jig and scope

255 

Figure 1	Figure 2

®

AC WAVEFORMS

READ CYCLE NO. 1(1,2)

t RC

ADDRESS

DOUT

PREVIOUS DATA VALID

t AA

t OHA 	t OHA

DATA VALID

READ1.eps

READ CYCLE NO. 2(1,3)

t RC

ADDRESS

OE CE1

t AA

t DOE

t LZOE

t OHA

t HZOE

CE2

DOUT

t LZCE1

t LZCE2

HIGH-Z

t ACE1

t ACE2

DATA VALID

t HZCE1

t HZCE2

CE2_RD2.eps

Notes:

1. WE is HIGH for a Read Cycle.

2. The device is continuously selected. OE, CE1 = VIL, CE2 = VIH.

3. Address is valid prior to or coincident with CE1 LOW and CE2 HIGH transitions.

®

WRITE CYCLE SWITCHING CHARACTERISTICS(1,2) (Over Operating Range, Standard and Low Power)

-12 ns	-15 ns

| Symbol | Parameter | Min. | Max. | Min. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| tWC | Write Cycle Time | 12 | — | 15 | — | ns |
| tSCE1 | CE1 to Write End | 10 | — | 12 | — | ns |
| tSCE2 | CE2 to Write End | 10 | — | 12 | — | ns |
| tAW | Address Setup Time to Write End | 10 | — | 12 | — | ns |
| tHA | Address Hold from Write End | 0 | — | 0 | — | ns |
| tSA | Address Setup Time | 0 | — | 0 | — | ns |
| tPWE(3) | WE Pulse Width | 10 | — | 12 | — | ns |
| tSD | Data Setup to Write End | 7 | — | 10 | — | ns |
| tHD | Data Hold from Write End | 0 | — | 0 | — | ns |
| tHZWE(4) | WE LOW to High-Z Output | — | 7 | — | 7 | ns |
| tLZWE(4) | WE HIGH to Low-Z Output | 2 | — | 2 | — | ns |

Notes:

1. Test conditions assume signal transition times of 5 ns or less, timing reference levels of 1.5V, input pulse levels of 0 to 3.0V

and output loading specified in Figure 1.

2. The internal write time is defined by the overlap of CE1 LOW, CE2 HIGH and WE LOW.  All signals must be in valid states to

initiate a Write, but any one can go inactive to terminate the Write.  The Data Input Setup and Hold timing are referenced to the rising or falling edge of the signal that terminates the Write.

3. Tested with OE HIGH.

4. Tested with the load in Figure 2.  Transition is measured ±500 mV from steady-state voltage. Not 100% tested.

®

AC WAVEFORMS

WRITE CYCLE NO. 1 (CE1 Controlled, OE is HIGH or LOW) (1 )

t WC

ADDRESS

CE1

t SA

VALID ADDRESS

t SCE1

t SCE2

t HA

CE2

WE

DOUT

DATA UNDEFINED

t AW

t PWE1

t PWE2

t HZWE

HIGH-Z

t LZWE

t SD

t HD

DIN	 DATAIN VALID

CE2_WR1.eps

WRITE CYCLE NO. 2 (OE is HIGH During Write Cycle) (1,2)

t WC

ADDRESS

OE

VALID ADDRESS

t HA

CE1

LOW

CE2

WE

HIGH

t AW

t PWE1

DOUT

t SA 	t HZWE

DATA UNDEFINED

HIGH-Z

t LZWE

t SD

t HD

DIN

DATAIN VALID

Notes:

CE2_WR2.eps

1. The internal write time is defined by the overlap of CE1 LOW, CE2 HIGH and WE LOW.  All signals must be in valid states to initiate a Write, but any one can go inactive to terminate the Write.  The Data Input Setup and Hold timing are referenced to the rising or falling edge of the signal that terminates the Write.

2. I/O will assume the High-Z state if OE = VIH.

®

WRITE CYCLE NO. 3 (OE is LOW During Write Cycle) (1)

t WC

ADDRESS

VALID ADDRESS

OE CE1

LOW

LOW

t HA

CE2

WE

DOUT

HIGH

t AW

t SA 	t HZWE

DATA UNDEFINED

t PWE2

HIGH-Z

t LZWE

t SD

t HD

DIN

DATAIN VALID

CE2_WR3.eps

®

DATA RETENTION SWITCHING CHARACTERISTICS

Symbol	Parameter	Test Condition	Min.	Typ.(1)	Max.	Unit

| Note: VDR | VDD forDataRetention | SeeDataRetentionWaveform |  | 2.0 |  | 5.5 | V |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IDR | DataRetentionCurrent | VDD = 2.0V, CE1  VDD – 0.2V | Com. | — | 200 | 400 | µA |
|  |  | or CE2  0.2V | Ind. | — | — | 450 |  |
|  |  | VIN  VDD – 0.2V, or VIN  VSS + 0.2V | Auto. | — | — | 500 |  |
| tSDR | DataRetentionSetupTime | SeeDataRetentionWaveform |  | 0 |  | — | ns |
| tRDR | Recovery Time | SeeDataRetentionWaveform |  | tRC |  | — | ns |

1. Typical Values are measured at VDD = 5V, TA = 25oC and not 100% tested.

DATA RETENTION WAVEFORM (CE1 Controlled)

tSDR

Data  Retention Mode

tRDR

4.5V

VDD

2.2V

VDR

CE1

CE1  VDD - 0.2V

GND

DATA RETENTION WAVEFORM (CE2 Controlled)

Data Retention Mode

4.5V

2.2V

VDD

CE2

VDR

0.4V

tSDR 	tRDR

CE2   0.2V

GND

®

ORDERING INFORMATION: IS61C1024AL Commercial Range: 0°C to +70°C

| Speed (ns) | Order Part No. | Package |
| --- | --- | --- |
| 12 | IS61C1024AL-12J IS61C1024AL-12T | 300-mil Plastic SOJ TSOP (Type I) |

ORDERING INFORMATION: IS61C1024AL Industrial Range: –40°C to +85°C

| Speed (ns) | Order Part No. | Package |
| --- | --- | --- |
| 12 | IS61C1024AL-12JI | 300-mil Plastic SOJ |
|  | IS61C1024AL-12JLI | 300-mil Plastic SOJ, Lead-free |
|  | IS61C1024AL-12KI | 400-mil Plastic SOJ |
|  | IS61C1024AL-12KLI | 400-mil Plastic SOJ, Lead-free |
|  | IS61C1024AL-12HI | sTSOP (Type I) |
|  | IS61C1024AL-12TI | TSOP (Type I) |
|  | IS61C1024AL-12TLI | TSOP (Type I), Lead-free |

ORDERING INFORMATION: IS64C1024AL Automotive Range: –40°C to +125°C

| Speed (ns) | Order Part No. | Package |
| --- | --- | --- |
| 15 | IS64C1024AL-15KA3 IS64C1024AL-15TA3 | 400-mil Plastic SOJ TSOP (Type I) |

PACKAGING INFORMATION 	ISSI

®

Plastic STSOP - 32 pins

Package Code: H (Type I)

A2

A

A1

1	N

E	b

e

D1	S

SEATING PLANE

D

 	C

L

Plastic STSOP (H - Type I) Millimeters 	Inches

Symbol   Min 	Max 	Min 	Max

Ref. Std.

N	32

A	—	1.25 	—	0.049

A1	 0.05 		 —	 0.002 		  — A2 	 0.95 		1.05 	 0.037 		0.041 b	0.17 	0.23 	0.007 	0.009

C	0.14 	0.16 	0.0055 	0.0063

D	13.20   13.60 	0.520 	0.535

D1 	11.70   11.90 	0.461 	0.469

E	7.90 	8.10 	0.311 	0.319 e	  0.50 BSC		0.020 BSC

L	0.30 	0.70 	0.012 	0.028

S	0.28 Typ.	0.011 Typ.

Notes:

1.  Controlling dimension: millimeters, unless otherwise specified.

2.  BSC = Basic lead spacing between centers.

3.  Dimensions D1 and E do not include mold flash protru- sions and should be measured from the bottom of the package.

4.  Formed leads shall be planar with respect to one another within 0.004 inches at the seating plane.

|  | 0° | 5° | 0° | 5° |
| --- | --- | --- | --- | --- |

Integrated Silicon Solution, Inc.

PK13197H32 Rev. B 04/21/03

®

400-mil Plastic SOJ Package Code: K

N 	N/2+1

E1

1 	N/2

D

Notes:

1. Controlling dimension:

millimeters.

2. BSC = Basic lead spacing between centers.

3. Dimensions D and E1 do not

include mold flash protrusions

E	and should be measured from

the bottom of the package.

4. Reference document: JEDEC MS-027.

SEATING PLANE

A

b

B

C

A2

e

A1	E2

Millimeters 			Inches 	 Millimeters 			Inches 	 Millimeters 			Inches Symbol 	Min 	Max 		Min 	Max 	Min 	Max 		Min 	Max 	Min 	Max 		Min 	Max No. Leads (N)		28 						32 						36

A	3.25 	3.75 	0.128    0.148 	3.25 	3.75 	0.128   0.148 	3.25 	3.75 	0.128   0.148

A1	0.64 	 —	0.025 	—	0.64 	—	0.025 	—	0.64 	 —	 0.025 	— A2		2.08 		—	 0.082 	 —	 2.08 	 —		0.082 	 —	 2.08 	  —		 0.082 	 — B	 0.38 	 0.51 	 0.015    0.020 	 0.38 	 0.51 	 0.015   0.020 	 0.38 	 0.51 		0.015   0.020 b	0.66 	0.81 	0.026    0.032 	0.66 	 0.81 	0.026   0.032 	0.66 	0.81 	 0.026   0.032

C	0.18 	0.33 	0.007    0.013 	0.18 	0.33 	0.007   0.013 	0.18 	0.33 	0.007   0.013

| D 18.29 18.54 0.720 0.730 20.82 21.08 0.820 |  |  |  |  | 0.830 | 23.37 | 23.62 |  | 0.920 | 0.930 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E 11.05 11.30 0.435 0.445 11.05 11.30 0.435 |  |  |  |  | 0.445 | 11.05 | 11.30 |  | 0.435 | 0.445 |
| E1 10.03 10.29 0.395 0.405 10.03 10.29 0.395 |  |  |  |  | 0.405 | 10.03 | 10.29 |  | 0.395 | 0.405 |
| E2 | 9.40 BSC | 0.370 BSC | 9.40 BSC | 0.370 BSC |  | 9.40 BSC |  |  | 0.370 BSC |  |
| e | 1.27 BSC | 0.050 BSC | 1.27 BSC | 0.050 BSC |  | 1.27 BSC |  |  | 0.050 BSC |  |

Copyright © 2003 Integrated Silicon Solution, Inc.  All rights reserved.  ISSI reserves the right to make changes to this specification and its products at any time without notice.   ISSI assumes no liability arising out of the application or use of any information, products or services described herein. Customers are advised to obtain the latest version of this device specification before relying on any published information and before placing orders for products.

PACKAGING INFORMATION	ISSI

®

Millimeters 	Inches 	Millimeters 	Inches 	Millimeters 	Inches

Symbol 	Min 	Max 	Min 	Max 	Min 	Max 	Min 	Max 	Min 	Max 	Min 	Max

| No. Leads (N) 40 42 44 D 25.91 26.16 1.020 1.030 27.18 27.43 1.070 |  |  |  |  | 1.080 | 28.45 | 28.70 |  | 1.120 | 1.130 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E 11.05 11.30 0.435 0.445 11.05 11.30 0.435 |  |  |  |  | 0.445 | 11.05 | 11.30 |  | 0.435 | 0.445 |
| E1 10.03 10.29 0.395 0.405 10.03 10.29 0.395 |  |  |  |  | 0.405 | 10.03 | 10.29 |  | 0.395 | 0.405 |
| E2 | 9.40 BSC | 0.370 BSC | 9.40 BSC | 0.370 BSC |  | 9.40 BSC |  |  | 0.370 BSC |  |
| e | 1.27 BSC | 0.050 BSC | 1.27 BSC | 0.050 BSC |  | 1.27 BSC |  |  | 0.050 BSC |  |

| A | 3.25 | 3.75 | 0.128 | 0.148 | 3.25 | 3.75 | 0.128 | 0.148 | 3.25 | 3.75 | 0.128 | 0.148 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 0.64 | — | 0.025 | — | 0.64 | — | 0.025 | — | 0.64 | — | 0.025 | — |
| A2 | 2.08 | — | 0.082 | — | 2.08 | — | 0.082 | — | 2.08 | — | 0.082 | — |
| B | 0.38 | 0.51 | 0.015 | 0.020 | 0.38 | 0.51 | 0.015 | 0.020 | 0.38 | 0.51 | 0.015 | 0.020 |
| b | 0.66 | 0.81 | 0.026 | 0.032 | 0.66 | 0.81 | 0.026 | 0.032 | 0.66 | 0.81 | 0.026 | 0.032 |
| C | 0.18 | 0.33 | 0.007 | 0.013 | 0.18 | 0.33 | 0.007 | 0.013 | 0.18 | 0.33 | 0.007 | 0.013 |

Copyright © 2003 Integrated Silicon Solution, Inc.  All rights reserved.  ISSI reserves the right to make changes to this specification and its products at any time without notice.   ISSI assumes no liability arising out of the application or use of any information, products or services described herein. Customers are advised to obtain the latest version of this device specification before relying on any published information and before placing orders for products.

2	Integrated Silicon Solution, Inc. — www.issi.com — 1-800-379-4774

Rev. F

®

300-mil Plastic SOJ Package Code: J

N

E1 	E

1

D 	SEATING PLANE

A

B 	A2

C

b 	A1 	E2

e

MILLIMETERS	INCHES

Sym.	Min.  Typ.  Max.	Min.  Typ. Max.

N0.

Leads	24/26

A	—	—	3.56	—	—   0.140

A1		0.64	—	  —	0.025	 —	— A2	2.41	—	2.67	0.095	—   0.105 b	0.41	—	0.51	0.016	—   0.020

B	0.66	—	0.81	0.026	—   0.032

C	0.20	—	0.25	0.008	—   0.010

D	17.02	—	17.27	0.670	—   0.680

E	8.26	—	8.76	0.325	—   0.345

E1	7.49	—	7.75	0.295   —   0.305

E2	6.27	—	7.29	0.247	—   0.287 e		1.27 BSC		0.050 BSC

Notes:

1.  Controlling dimension: inches, unless otherwise specified.

2.  BSC = Basic lead spacing between centers.

3.  Dimensions D and E1 do not include mold flash protrusions and should be measured from the bottom of thepackage.

4.  Formed leads shall be planar with respect to one another within 0.004 inches at the seating plane.

Copyright © 2003 Integrated Silicon Solution, Inc.  All rights reserved.  ISSI reserves the right to make changes to this specification and its products at any time without notice.   ISSI assumes no liability arising out of the application or use of any information, products or services described herein. Customers are advised to obtain the latest version of this device specification before relying on any published information and before placing orders for products.

PACKAGING INFORMATION 	ISSI

®

300-mil Plastic SOJ Package Code: J

MILLIMETERS	INCHES

MILLIMETERS	INCHES

Sym.	Min.  Typ.  Max.	Min.  Typ.  Max.

N0.

Leads	28

A	—	—	3.56	—	—   0.140

A1	 0.64	—		—	0.025	 —		— A2	2.41	—	2.67	0.095	—	0.105 b	0.41	—	0.51	0.016	—	0.020

B	0.66	—	0.81	0.026	—	0.032

C	0.20	—	0.25	0.008	—	0.010

D	18.29	—	18.54	0.720	—	0.730

E	8.26	—	8.76	0.325	—	0.345

E1	7.49	—	7.75	0.295   —	0.305

E2	6.27	—	7.29	0.247	—	0.287 e		1.27 BSC		0.050 BSC

Sym.	Min.  Typ.  Max.	Min.  Typ.  Max.

N0.

Leads	32

A	—	—	3.56	—	—   0.140

A1	 0.64	—	  —	0.025		—		— A2	2.41	—	2.67	0.095	—	0.105 b	0.41	—	0.51	0.016	—	0.020

B	0.66	—	0.81	0.026	—	0.032

C	0.20	—	0.25	0.008	—	0.010

D	20.83	—	21.08	0.820	—	0.830

E	8.26	—	8.76	0.325	—	0.345

E1	7.49	—	7.75	0.295   —	0.305

E2	6.27	—	7.29	0.247	—	0.287 e		1.27 BSC		0.050 BSC

2	Integrated Silicon Solution, Inc. — www.issi.com — 1-800-379-4774

Rev. D

®

Plastic TSOP-Type  I Package  Code: T (32-pin)

1

E	H

N D

SEATING PLANE

S	A

e	B	A1	α 	C

L

MILLIMETERS 	INCHES

Notes:

1.  Controlling dimension: millimeters, unless otherwise specified.

2.  BSC = Basic lead spacing between centers.

3.  Dimensions D and E do not include mold

flash protrusions and should be measured from the bottom of the package.

4.  Formed leads shall be planar with respect

to one another within 0.004 inches at the seating plane.

| S 0.25 REF 0.010 REF Symbol | Min. | Max. |  | Min. | Max. |
| --- | --- | --- | --- | --- | --- |
| No. Leads |  |  | 32 |  |  |
| A | — | 1.20 |  | — | 0.047 |
| A1 | 0.05 | 0.25 |  | 0.002 | 0.010 |
| B | 0.17 | 0.23 |  | 0.007 | 0.009 |
| C | 0.12 | 0.17 |  | 0.005 | 0.007 |
| D | 7.90 | 8.10 |  | 0.311 | 0.319 |
| E | 18.30 | 18.50 |  | 0.720 | 0.728 |
| H | 19.80 | 20.20 |  | 0.780 | 0.795 |
| e 0.50 BSC 0.020 BSC |  |  |  |  |  |
| L | 0.40 | 0.60 |  | 0.016 | 0.024 |
|  | 0° | 8° |  | 0° | 8° |

Copyright © 2003 Integrated Silicon Solution, Inc.  All rights reserved.  ISSI reserves the right to make changes to this specification and its products at any time without notice.   ISSI assumes no liability arising out of the application or use of any information, products or services described herein. Customers are advised to obtain the latest version of this device specification before relying on any published information and before placing orders for products.