10A High Efficiency

DC/DC µModule

FEATURES

n  Complete Switch Mode Power Supply

n  Wide Input Voltage Range: 4.5V to 20V

n  10A DC, 14A Peak Output Current

n  Parallel Two μModule™  DC/DC Converters for 20A Output Current

n  0.6V to 5V Output Voltage

n  1.5%  Output Voltage Regulation

n  Ultrafast Transient Response

n  Current Mode Control

n  Pb-Free (e4) RoHS Compliant Package  with Gold- Pad Finish

n  Up to 92% Efﬁciency

n  Programmable Soft-Start

n  Output Overvoltage Protection

n  Optional Short-Circuit Shutdown Timer

n  Small Footprint, Low Proﬁle (15mm × 15mm ×

2.8mm) Surface Mount LGA Package

APPLICATIONS

n  Telecom and Networking Equipment

n  Servers

n  Industrial Equipment

n  Point of Load Regulation

L, LT, LTC and LTM are registered trademarks of Linear Technology Corporation.

μModule is a trademark of Linear Technology Corporation. All other trademarks are the property of their respective owners. Protected by U.S. Patents including 5481178, 6100678, 6580258,

5847554, 6304066.

DESCRIPTION

The LTM®4600 is a complete 10A, DC/DC step down power

supply. Included in the package are the switching control- ler, power FETs, inductor,  and all support components. Operating over an input voltage range of 4.5V to 20V, the LTM4600 supports an output voltage range of 0.6V to 5V, set by a single resistor. This high efﬁciency design delivers

10A continuous current (14A peak), needing no heat sinks or airﬂow to meet power speciﬁcations.  Only bulk input and output capacitors are needed to ﬁnish the design.

The low proﬁle package (2.8mm) enables utilization of unused  space on the bottom of PC boards for high density point of load regulation. High switching frequency and an adaptive on-time current mode architecture enables a very fast transient response to line and load changes without sacriﬁcing stability. Fault protection features include integrated overvoltage and short circuit protection with a defeatable shutdown timer. A built-in soft-start timer is adjustable with a small capacitor.

The LTM4600 is packaged in a thermally enhanced, compact (15mm × 15mm) and low proﬁle (2.8mm) over-molded Land Grid Array (LGA) package  suitable  for automated assembly by standard surface mount equipment. The LTM4600 is Pb-free and RoHS compliant.

TYPICAL APPLICATION

10A μModule Power Supply with 4.5V to 20V Input

100

90

Efﬁciency vs Load Current with 12VIN (FCB = 0)

VIN

4.5V TO 20V

CIN

VIN

VOUT

LTM4600

VOSET

COUT

| VOUT 80 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | .2VOUT 1.5VOUT 2.5VOUT 3.3VOUT |

EFFICIENCY (%)

1.5V

10A	70

60

PGND  SGND

66.5k

4600 TA01a

50

40

30

20

0 	2 	4

6 	8 	10

LOAD CURRENT (A)

4600 TA01b

4600fc

ABSOLUTE MAXIMUM RATINGS

(Note 1)

FCB, EXTVCC, PGOOD, RUN/SS, VOUT .......... –0.3V to 6V VIN, SVIN, fADJ ............................................ –0.3V to 20V VOSET, COMP ............................................. –0.3V to 2.7V Operating Temperature Range (Note 2).... –40°C to 85°C Junction Temperature ........................................... 125°C Storage Temperature Range................... –55°C to 125°C

PIN CONFIGURATION

fADJ SVIN EXTVCC VOSET

TOP VIEW

PGND

VOUT

COMP SGND

FCB PGOOD

LGA PACKAGE

104-LEAD (15mm × 15mm × 2.8mm)

TJMAX = 125°C, θJA = 15°C/W, θJC = 6°C/W,

θJA DERIVED FROM 95mm × 76mm PCB WITH 4 LAYERS

WEIGHT = 1.7g

ORDER INFORMATION

| LEAD FREE FINISH | PART MARKING | PACKAGE DESCRIPTION | TEMPERATURE RANGE |
| --- | --- | --- | --- |
| LTM4600EV#PBF | LTM4600EV | 104-Lead (15mm × 15mm × 2.8mm) LGA | –40°C to 85°C |
| LTM4600IV#PBF | LTM4600IV | 104-Lead (15mm × 15mm × 2.8mm) LGA | –40°C to 85°C |

Consult LTC Marketing for parts speciﬁed with wider operating temperature ranges. Consult LTC Marketing for information on non-standard lead based ﬁnish parts.

For more information on lead free part marking, go to: http://www.linear.com/leadfree/

This product is only offered in trays. For more information go to: http://www.linear.com/packaging/

ELECTRICAL CHARACTERISTICS

The l denotes the speciﬁcations  which apply over the –40°C to 85°C

temperature range, otherwise speciﬁcations  are at TA = 25°C,  VIN = 12V. External  CIN = 120μF, COUT = 200μF/Ceramic per typical application (front page) conﬁguration.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| VIN(DC) | Input DC Voltage |  | l | 4.5 20 | V |
| VOUT(DC) | Output Voltage | FCB = 0V VIN = 5V or 12V, VOUT = 1.5V, IOUT = 0A | l | 1.478 1.50 1.522 1.470 1.50 1.530 | V V |

Input Speciﬁcations

| VIN(UVLO) | Under Voltage Lockout Threshold | IOUT = 0A |  | 3.4 4 | V |
| --- | --- | --- | --- | --- | --- |
| IINRUSH(VIN) | Input Inrush Current at Startup | IOUT = 0A. VOUT = 1.5V, FCB = 0 VIN = 5V VIN = 12V |  | 0.6 0.7 | A A |
| IQ(VIN) | Input Supply Bias Current | IOUT = 0A, EXTVCC Open VIN = 12V, VOUT = 1.5V, FCB = 5V VIN = 12V, VOUT = 1.5V, FCB = 0V VIN = 5V, VOUT = 1.5V, FCB = 5V VIN = 5V, VOUT = 1.5V, FCB = 0V Shutdown, RUN = 0.8V, VIN = 12V |  | 1.2 42 1.0 52 35 75 | mA mA mA mA μA |

4600fc

ELECTRICAL CHARACTERISTICS

The l denotes the speciﬁcations  which apply over the –40°C to 85°C

temperature range, otherwise speciﬁcations  are at TA = 25°C,  VIN = 12V. Per typical application (front page) conﬁguration.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| IS(VIN) | Input Supply Current | VIN = 12V, VOUT = 1.5V, IOUT = 10A VIN = 12V, VOUT = 3.3V, IOUT = 10A VIN = 5V, VOUT = 1.5V, IOUT = 10A |  | 1.52 3.13 3.64 | A A A |

Output Speciﬁcations

| IOUTDC | Output Continuous Current Range (See Output Current Derating Curves for Different VIN, VOUT and TA) | VIN = 12V, VOUT = 1.5V |  | 0 10 | A |
| --- | --- | --- | --- | --- | --- |
| ΔVOUT(LINE) VOUT | Line Regulation Accuracy | VOUT = 1.5V, IOUT = 0A, FCB = 0V, VIN = 4.5V to 20V | l | 0.15 0.3 | % |
| ΔVOUT(LOAD) VOUT | Load Regulation Accuracy | VOUT = 1.5V, IOUT = 0A to 10A, FCB = 0V VIN = 5V VIN = 12V (Notes 3, 4) | l | ±1 ±1.5 | % % |
| VOUT(AC) | Output Ripple Voltage | VIN = 12V, VOUT = 1.5V, IOUT = 0A, FCB = 0V |  | 10 15 | mVP-P |
| fs | Output Ripple Voltage Frequency | VOUT = 1.5V, IOUT = 5A, FCB = 0V |  | 850 | kHz |
| tSTART | Turn-On Time | VOUT = 1.5V, IOUT = 1A VIN = 12V VIN = 5V |  | 0.5 0.7 | ms ms |
| ΔVOUTLS | Voltage Drop for Dynamic Load Step | VOUT = 1.5V, Load Step: 0A/μs to 5A/μs COUT = 3 • 22μF 6.3V, 470μF 4V POSCAP, See Table 2 |  | 36 | mV |
| tSETTLE | Settling Time for Dynamic Load Step | Load: 10% to 90% to 10% of Full Load |  | 25 | μs |
| IOUTPK | Output Current Limit | Output Voltage in Foldback VIN = 12V, VOUT = 1.5V VIN = 5V, VOUT = 1.5V |  | 14 14 | A A |

Control Stage

| VOSET | Voltage at VOSET Pin | IOUT = 0A, VOUT = 1.5V | l | 0.591 0.6 0.609 0.594 0.6 0.606 | V V |
| --- | --- | --- | --- | --- | --- |
| VRUN/SS | RUN ON/OFF Threshold |  |  | 0.8 1.5 2 | V |
| IRUN(C)/SS | Soft-Start Charging Current | VRUN/SS = 0V |  | –0.5 –1.2 –3 | μA |
| IRUN(D)/SS | Soft-Start Discharging Current | VRUN/SS = 4V |  | 0.8 1.8 3 | μA |
| VIN – SVIN |  | EXTVCC = 0V, FCB = 0V |  | 100 | mV |
| IEXTVCC | Current into EXTVCC Pin | EXTVCC = 5V, FCB = 0V, VOUT = 1.5V, IOUT = 0A |  | 16 | mA |
| RFBHI | Resistor Between VOUT and VOSET Pins |  |  | 100 | kΩ |
| VFCB | Forced Continuous Threshold |  |  | 0.57 0.6 0.63 | V |
| IFCB | Forced Continuous Pin Current | VFCB = 0.6V |  | –1 –2 | μA |

PGOOD Output

| ΔVOSETH | PGOOD Upper Threshold | VOSET Rising |  | 7.5 10 12.5 | % |
| --- | --- | --- | --- | --- | --- |
| ΔVOSETL | PGOOD Lower Threshold | VOSET Falling |  | –7.5 –10 –12.5 | % |
| ΔVOSET(HYS) | PGOOD Hysteresis | VOSET Returning |  | 2 | % |
| VPGL | PGOOD Low Voltage | IPGOOD = 5mA |  | 0.15 0.4 | V |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime.

Note 2: The LTM4600E is guaranteed to meet performance speciﬁcations from 0°C to 85°C. Speciﬁcations over the –40°C to 85°C operating

temperature range are assured by design, characterization and correlation with statistical process controls.  The LTM4600I is guaranteed over the

–40°C to 85°C temperature range.

Note 3: Test assumes current derating versus temperature.

Note 4: Guaranteed by correlation.

4600fc

TYPICAL PERFORMANCE CHARACTERISTICS

(See Figure 18 for all curves)

100

90

80

70

Efﬁciency vs Load Current with 5VIN (FCB = 0)

100

90

EFFICIENCY (%)

80

70

Efﬁciency vs Load Current with 12VIN (FCB = 0)

100

90

EFFICIENCY (%)

80

70

Efﬁciency vs Load Current with 18VIN (FCB = 0)

EFFICIENCY (%)

60

50

40

30

0	2	4

0.6VOUT

1.2VOUT

1.5VOUT

2.5VOUT

6	8	10

60

50

40

30

0	2	4

0.6VOUT

1.2VOUT

1.5VOUT

2.5VOUT

3.3VOUT

6	8	10

60

50

40

| 30 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1.5VOUT 1.8VOUT 2.5VOUT 3.3VOUT |

0	2	4

6	8	10

LOAD CURRENT (A)

LOAD CURRENT (A)

LOAD CURRENT (A)

4600 G01

4600 G02

4600 G03

Efﬁciency vs Load Current with Different  FCB Settings

90

80	FCB  > 0.7V

70

EFFICIENCY (%)

60

FCB = GND

50

40

1.2V Transient Response	1.5V Transient Response

VOUT = 50mV/DIV

IOUT = 5A/DIV

30

20

0.1

1

LOAD CURRENT (A)

VIN = 12V VOUT = 1.5V

10

25μs/DIV	4600 G05

1.2V AT 5A/μs  LOAD STEP

COUT = 3 • 22μF 6.3V CERAMICS

470μF  4V SANYO POSCAP

C3 = 100pF

25μs/DIV	4600 G06

1.5V AT 5A/μs  LOAD STEP

COUT = 3 • 22μF 6.3V CERAMICS

470μF  4V SANYO POSCAP

C3 = 100pF

4600 G04

1.8V Transient Response	2.5V Transient Response	3.3V Transient Response

25μs/DIV	4600 G07

1.8V AT 5A/μs  LOAD STEP

COUT = 3 • 22μF 6.3V CERAMICS

470μF  4V SANYO POSCAP

C3 = 100pF

25μs/DIV	4600 G08

2.5V AT 5A/μs  LOAD STEP

COUT = 3 • 22μF 6.3V CERAMICS

470μF  4V SANYO POSCAP

C3 = 100pF

25μs/DIV	4600 G09

3.3V AT 5A/μs  LOAD STEP

COUT = 3 • 22μF 6.3V CERAMICS

470μF  4V SANYO POSCAP

C3 = 100pF

4600fc

TYPICAL PERFORMANCE CHARACTERISTICS

(See Figure 18 for all curves)

Start-Up, IOUT = 0A

Start-Up, IOUT = 10A (Resistive Load)

Short-Circuit Protection, IOUT = 0A

VOUT

(0.5V/DIV)

IIN

(0.5A/DIV)

VOUT

(0.5V/DIV)

IIN

(0.5A/DIV)

VOUT

(0.5V/DIV)

IIN

(0.2A/DIV)

VIN = 12V VOUT = 1.5V

200μs/DIV	4600 G10

VIN = 12V VOUT = 1.5V

200μs/DIV	4600 G11

VIN = 12V VOUT = 1.5V

20μs/DIV	4600 G12

COUT = 200μF

NO EXTERNAL SOFT-START CAPACITOR

COUT = 200μF

NO EXTERNAL SOFT-START CAPACITOR

COUT = 2× 200μF/X5R

NO EXTERNAL SOFT-START CAPACITOR

Short-Circuit Protection, IOUT = 10A

VOUT

(0.5V/DIV)

IIN

(0.5A/DIV)

20μs/DIV	4600 G13

5.5

5.0

4.5

4.0

3.5

3.0

VOUT (V)

2.5

2.0

VIN to VOUT Step-Down Ratio

fADJ = OPEN

5V

3.3V

2.5V

1.8V

| 0.00 |  |
| --- | --- |
|  | 25°C |
|  | 100°C |
|  | –45°C |

–0.05

LOAD REGULATION %

–0.10

–0.15

–0.20

–0.25

–0.30

12V Input Load Regulation  vs

Temperature

VIN = 12V VOUT = 1.5V

COUT = 2× 200μF/X5R

NO EXTERNAL SOFT-START CAPACITOR

1.5

1.0

0.5

0

0.6V

1.2V

1.5V

–0.35

–0.40

–0.45

0	5	10	15	20

VIN (V)

0	5	10

LOAD CURRENT

SEE FREQUENCY ADJUSTMENT DISCUSSION FOR 12VIN TO 5VOUT AND 5VIN TO 3.3VOUT CONVERSION

4600 G15

4600 G14

4600fc

PIN FUNCTIONS

(See Package Description for Pin Assignment)

VIN (Bank  1): Power Input Pins. Apply input voltage be- tween these pins and PGND pins. Recommend placing input decoupling capacitance directly between VIN  pins and PGND pins.

fADJ (Pin A15): A 110k resistor  from VIN to this pin sets the one-shot timer current, thereby setting the switching frequency. The LTM4600 switching frequency is typically

850kHz. An external resistor to ground can be selected to reduce the one-shot timer current, thus lower the switching frequency to accommodate a higher duty cycle step down requirement. See the applications section.

SVIN (Pin A17): Supply Pin for Internal PWM Controller. Leave this pin open or add additional decoupling capacitance.

EXTVCC (Pin A19): External 5V supply pin for controller. If left open or grounded, the internal 5V linear regulator  will power the controller and MOSFET drivers. For high input voltage applications, connecting this pin to an external

5V will reduce  the power loss in the power module. The

EXTVCC voltage should never be higher than VIN.

VOSET (Pin A21): The Negative Input of The Error Ampliﬁer. Internally, this pin is connected to VOUT with a 100k precision resistor. Different output voltages can be programmed with additional resistors between the VOSET and SGND pins.

COMP (Pin B23): Current Control Threshold and Error Ampliﬁer Compensation  Point. The current comparator threshold increases with this control voltage. The voltage ranges from 0V to 2.4V with 0.8V corresponding  to zero sense voltage (zero current).

SGND  (Pin D23): Signal Ground Pin. All  small-signal components should connect to this ground, which in turn connects to PGND at one point.

RUN/SS (Pin F23): Run and Soft-Start Control. Forcing this pin below  0.8V will shut down the power supply. Inside the power module, there is a 1000pF capacitor which provides approximately 0.7ms soft-start time with

200μF output capacitance.  Additional soft-start  time can be achieved by adding additional capacitance between the RUN/SS and SGND pins. The internal short-circuit latchoff can be disabled by adding a resistor between this pin and the VIN pin. This pullup resistor must supply a minimum

5μA pull up current.

FCB (Pin G23): Forced Continuous Input. Grounding this pin enables forced continuous mode operation regardless of load conditions.  Tying this pin above 0.63V enables discontinuous conduction mode to achieve high efﬁciency operation at light loads. There is an internal 4.75K resistor between the FCB and SGND pins.

PGOOD (Pin J23): Output Voltage Power Good Indicator. When the output voltage is within 10% of the nominal voltage,  the PGOOD is open drain output. Otherwise, this pin is pulled to ground.

PGND (Bank 2): Power ground pins for both input and output returns.

VOUT (Bank  3): Power Output Pins. Apply output load between  these  pins and PGND pins. Recommend placing High Frequency output decoupling capacitance  directly between these pins and PGND pins.

fADJ SVIN EXTVCC

VOSET

TOP VIEW

VIN

BANK 1

PGND BANK 2

VOUT

BANK 3

A

B    COMP

C

D   SGND

E

F    RUN/SS

G       FCB

H

J        PGOOD

K

L M N

P R T

1         3	5

2	4

7 	9        11        13

6 	8        10        12

15        17

14         16

19        21        23

18        20         22

4600 PN01

4600fc

SIMPLIFIED BLOCK DIAGRAM

SVIN

RUN/SS	LTM4600

1000pF

VIN 4.5V TO 20V

PGOOD COMP FCB

INT COMP

1.5μF 	CIN

Q1

VOUT 1.5V/10A  MAX

COUT

fADJ

SGND

4.75k

10Ω

CONTROLLER

Q2

15μF

6.3V

PGND

R6

66.5k

EXTVCC

VOSET

100k

0.5%

4600 F01

Figure 1. Simpliﬁed  LTM4600 Block Diagram

DECOUPLING REQUIREMENTS

TA = 25°C,  VIN = 12V. Use Figure 1 conﬁguration.

| SYMBOL | PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| CIN | External Input Capacitor Requirement (VIN = 4.5V to 20V, VOUT = 1.5V) | IOUT = 10A |  | 20 | μF |
| COUT | External Output Capacitor Requirement (VIN = 4.5V to 20V, VOUT = 1.5V) | IOUT = 10A, Refer to Table 2 in the Applications Information Section |  | 100 200 | μF |

4600fc

OPERATION

μModule Description

The LTM4600 is a standalone non-isolated synchronous switching DC/DC power supply. It can deliver up to 10A of DC output current with only bulk external input and output capacitors. This module provides a precisely regulated output voltage programmable via one external resistor from

0.6VDC to 5.0VDC, not to exceed 80% of the input voltage. The input voltage range is 4.5V to 20V. A simpliﬁed block diagram is shown in Figure 1 and the typical application schematic is shown in Figure 18.

The LTM4600 contains an integrated LTC constant on-time current-mode  regulator, ultra-low RDS(ON) FETs with fast switching speed and integrated Schottky diode. The typical switching frequency is 850kHz at full load. With current mode control and internal feedback loop compensation, the LTM4600 module has sufﬁcient stability margins and good transient performance under a wide range of operat- ing conditions and with a wide range of output capacitors, even all ceramic output capacitors  (X5R or X7R).

Current mode control provides cycle-by-cycle fast current limit. In addition, foldback current limiting is provided in an over-current  condition while VOSET  drops. Also, the LTM4600 has defeatable short circuit latch off. Internal overvoltage and undervoltage comparators pull the open- drain PGOOD output low if the output feedback voltage exits

a ±10% window around the regulation point. Furthermore, in an overvoltage condition, internal top FET Q1 is turned off and bottom FET Q2 is turned on and held on until the overvoltage condition clears.

Pulling the RUN/SS pin low forces the controller into its shutdown state, turning off both Q1 and Q2. Releasing the pin allows an internal 1.2μA current source to charge up the softstart capacitor. When this voltage reaches 1.5V, the controller turns on and begins switching.

At low load current  the module works in continuous cur- rent mode by default to achieve minimum output voltage ripple. It can be programmed to operate in discontinuous current mode for improved light load efﬁciency when the FCB pin is pulled up above 0.8V and no higher than 6V. The FCB pin has a 4.75k resistor to ground, so a resistor to VIN can set the voltage on the FCB pin.

When  EXTVCC pin is grounded or open, an integrated  5V linear regulator powers the controller and MOSFET gate drivers.  If a minimum 4.7V external bias supply is ap- plied on the EXTVCC pin,  the internal regulator is turned off, and an internal switch connects EXTVCC  to the gate driver voltage. This eliminates the linear regulator power loss with high input voltage, reducing the thermal stress on the controller. The maximum voltage on EXTVCC pin is

6V. The EXTVCC voltage should never be higher than the

VIN voltage.  Also EXTVCC must be sequenced after VIN.

4600fc

APPLICATIONS INFORMATION

The typical LTM4600 application circuit is shown in Figure

18. External component  selection is primarily determined by the maximum load current and output voltage.

Output Voltage Programming and Margining

The PWM controller  of the LTM4600  has an internal

0.6V±1% reference  voltage. As shown in the block dia- gram, a 100k/0.5% internal feedback resistor connects

down when QDOWN  is on and QUP is off. If the output voltage  VO needs to be margined up/down by ±M%, the resistor values of RUP and RDOWN can be calculated from the following equations:

(RSET  RUP )• VO • (1+ M%) = 0.6V (RSET  RUP ) + 100k

VOUT and VOSET pins.  Adding a resistor RSET from VOSET

pin to SGND pin programs the output voltage:

100k + RSET

RSET • VO • (1– M%)

RSET + (100k   RDOWN )

= 0.6V

VO = 0.6V •

RSET

Input Capacitors

The LTM4600 μModule  should be connected to a low

Table 1 shows the standard values of 1% RSET resistor

for typical output voltages:

Table 1.

| RSET (kΩ) | Open | 100 | 66.5 | 49.9 | 43.2 | 31.6 | 22.1 | 13.7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VO (V) | 0.6 | 1.2 | 1.5 | 1.8 | 2 | 2.5 | 3.3 | 5 |

Voltage margining is the dynamic adjustment of the output voltage to its worst case operating range in production testing to stress the load circuitry, verify control/protec- tion functionality of the board and improve the system reliability. Figure 2 shows how to implement margining function with the LTM4600. In addition to the feedback

ac-impedance DC source. High frequency, low ESR input capacitors are required to be placed adjacent to the mod- ule. In Figure 18, the bulk input capacitor  CIN is selected for its ability to handle the large RMS current into the converter. For a buck converter, the switching duty-cycle can be estimated as:

D = VO

VIN

Without considering the inductor current ripple, the RMS

current of the input capacitor can be estimated as:

IO(MAX)

resistor RSET, several external components are added. Turn off both transistor QUP  and QDOWN  to disable the

ICIN(RMS) =

•	D • (1  D)

%

margining. When QUP is on and QDOWN is off, the output voltage is margined up. The output voltage is margined

VOUT

LTM4600

RDOWN

100k

QDOWN

In the above equation, η% is the estimated efﬁciency of

the power module. C1 can be a switcher-rated electrolytic

aluminum  capacitor,  OS-CON capacitor  or high volume ceramic capacitors. Note the capacitor ripple current ratings are often based on only 2000 hours of life. This makes it advisable to properly derate the input capacitor, or choose a capacitor rated at a higher temperature than required. Always contact the capacitor manufacturer for

PGND	SGND

VOSET

2N7002

RSET	RUP

QUP

2N7002

4600 F02

derating requirements.

In Figure 18, the input capacitors are used as high fre- quency input decoupling capacitors. In a typical 10A output application, 1-2 pieces of very low ESR X5R or X7R, 10μF ceramic  capacitors are recommended. This decoupling capacitor should be placed directly adjacent

Figure 2. LTM4600 Margining Implementation

4600fc

APPLICATIONS INFORMATION

the module input pins in the PCB layout to minimize the trace inductance and high frequency AC noise.

Output Capacitors

The LTM4600 is designed for low output voltage ripple. The bulk output capacitors COUT is chosen with low enough effective series resistance (ESR) to meet the output voltage ripple and transient requirements. COUT can be low ESR tantalum capacitor, low ESR polymer capacitor or ceramic capacitor (X5R or X7R). The typical capacitance  is 200μF if all ceramic  output capacitors are used. The internally

Soft-Start and Latchoff with the RUN/SS pin

The RUN/SS pin provides a means to shut down the LTM4600 as well as a timer for soft-start and over-cur- rent latchoff. Pulling the RUN/SS pin below 0.8V puts the LTM4600 into a low quiescent current shutdown (IQ

≤ 75μA). Releasing the pin allows an internal 1.2μA cur-

rent source to charge up the timing capacitor CSS. Inside LTM4600, there is an internal 1000pF capacitor from RUN/ SS pin to ground. If RUN/SS pin has an external  capacitor CSS_EXT to ground, the delay before starting is about:

1.5V

optimized loop compensation provides sufﬁcient stability

margin for all ceramic capacitors applications. Additional output ﬁltering may be required by the system designer,

tDELAY =

1.2μA

• (CSS _ EXT + 1000pF)

if further reduction of output ripple or dynamic transient spike is required. Refer to Table 2 for an output capaci- tance matrix for each output voltage Droop, peak to peak deviation and recovery time during a 5A/μs transient with a speciﬁc output capacitance.

Fault Conditions:  Current Limit and Over current

Foldback

The LTM4600 has a current mode controller, which inher- ently limits the cycle-by-cycle inductor current not only in steady state operation, but also in transient.

To further limit current in the event of an over load condi- tion, the LTM4600 provides foldback current limiting. If the output voltage falls by more than 50%, then the maximum output current is progressively lowered to about one sixth of its full current limit value.

When the voltage on RUN/SS pin reaches 1.5V, the LTM4600

internal switches are operating with a clamping of the maximum output inductor current limited by the RUN/SS pin total soft-start capacitance. As the RUN/SS pin voltage rises  to 3V, the soft-start clamping of the inductor current is released.

VIN to VOUT Step-Down Ratios

There are restrictions in the maximum  VIN to VOUT  step down ratio that can be achieved for a given input voltage. These contraints are shown in the Typical Performance Characteristics curves labeled  “VIN to VOUT  Step-Down Ratio”. Note that additional thermal derating may apply. See the Thermal Considerations and Output Current Derating sections of this data sheet.

After the controller has been started and given adequate

4600fc

APPLICATIONS INFORMATION

Table 2. Output Voltage Response Versus Component Matrix (Refer to Figure 18)

TYPICAL MEASURED VALUES

| COUT1 VENDORS | PART NUMBER | COUT2 VENDORS | PART NUMBER |
| --- | --- | --- | --- |
| TDK | C4532X5R0J107MZ (100μF,6.3V) | SANYO POSCAP | 6TPE330MIL (330μF, 6.3V) |
| TAIYO YUDEN | JMK432BJ107MU-T ( 100μF, 6.3V) | SANYO POSCAP | 2R5TPE470M9 (470μF, 2.5V) |
| TAIYO YUDEN | JMK316BJ226ML-T501 ( 22μF, 6.3V) | SANYO POSCAP | 4TPE470MCL (470μF, 4V) |

| VOUT (V) | CIN (CERAMIC) | CIN (BULK) | COUT1 (CERAMIC) | COUT2 (BULK) | CCOMP | C3 | VIN (V) | DROOP (mV) | PEAK TO PEAK (mV) | RECOVERY TIME (μs) | LOAD STEP (A/μs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 5 | 35 | 68 | 25 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 2.5V | NONE | 100pF | 5 | 35 | 70 | 20 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 5 | 40 | 80 | 20 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 5 | 49 | 98 | 20 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 35 | 68 | 25 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 2.5V | NONE | 100pF | 12 | 35 | 70 | 20 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 12 | 40 | 80 | 20 | 5 |
| 1.2 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 12 | 49 | 98 | 20 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 5 | 36 | 75 | 25 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 2.5V | NONE | 100pF | 5 | 37 | 79 | 20 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 5 | 44 | 84 | 20 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 5 | 61 | 118 | 20 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 36 | 75 | 25 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 2.5V | NONE | 100pF | 12 | 37 | 79 | 20 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 12 | 44 | 89 | 20 | 5 |
| 1.5 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 12 | 54 | 108 | 20 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 5 | 40 | 81 | 30 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 2.5V | NONE | 100pF | 5 | 44 | 88 | 20 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 5 | 46 | 91 | 20 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 5 | 62 | 128 | 20 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 40 | 81 | 30 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 2.5V | NONE | 100pF | 12 | 44 | 85 | 20 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 12 | 44 | 91 | 20 | 5 |
| 1.8 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 12 | 62 | 125 | 20 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 4V | NONE | 100pF | 5 | 48 | 103 | 30 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 5 | 56 | 113 | 30 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 5 | 57 | 116 | 30 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 5 | 60 | 115 | 25 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 48 | 103 | 30 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 51 | 102 | 30 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 12 | 56 | 113 | 30 | 5 |
| 2.5 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 12 | 70 | 159 | 25 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 7 | 64 | 126 | 30 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 4V | NONE | 100pF | 7 | 66 | 132 | 30 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 7 | 82 | 166 | 35 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 7 | 100 | 200 | 25 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 1 × 100μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 52 | 106 | 30 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 3 × 22μF 6.3V | 470μF 4V | NONE | 100pF | 12 | 64 | 129 | 35 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 2 × 100μF 6.3V | 330μF 6.3V | NONE | 100pF | 12 | 64 | 126 | 30 | 5 |
| 3.3 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 12 | 76 | 144 | 25 | 5 |
| 5 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 15 | 188 | 375 | 25 | 5 |
| 5 | 2 × 10μF 25V | 150μF 35V | 4 × 100μF 6.3V | NONE | NONE | 100pF | 20 | 159 | 320 | 25 | 5 |

4600fc

APPLICATIONS INFORMATION

time to charge up the output capacitor, CSS is used as a short-circuit timer. After the RUN/SS pin charges above 4V, if the output voltage falls below 75% of its regulated value, then a short-circuit fault is assumed.  A 1.8μA current then begins discharging CSS. If the fault condition persists until

4V

3V

1.5V

VRUN/SS

3.5V

the RUN/SS pin drops to 3.5V, then the controller turns

off both power  MOSFETs, shutting down the converter permanently. The RUN/SS pin must be actively pulled

SOFT-START

SHORT-CIRCUIT

LATCH ARMED

OUTPUT

t

SHORT-CIRCUIT

down to ground in order to restart operation.

The over-current protection timer requires the soft-start timing capacitor CSS be made large enough to guarantee that the output is in regulation by the time CSS has reached the 4V threshold. In general, this will depend upon the size of the output capacitance, output voltage and load current characteristic. A minimum   external  soft-start capacitor

CLAMPING

OF IL RELEASED

SWITCHING STARTS

OVERLOAD HAPPENS

VO

LATCHOFF

75%VO

can be estimated from:

CSS   EXT + 1000pF > COUT • VOUT (10–3[F / VS ])

Figure 3. RUN/SS Pin Voltage During Startup and

Short-Circuit Protection

Generally 0.1μF is more than sufﬁcient.

VIN

RRUN/SS

VIN

LTM4600

Since the load current is already limited by the current mode control and current foldback circuitry  during a shortcircuit, over-current latchoff operation is NOT always needed or desired, especially   if the output has a large

RUN/SS

PGND  SGND

4600 F04

RECOMMENDED VALUES FOR RRUN/SS

amount of capacitance or the load draws huge currents

during start up. The latchoff feature can be overridden by a pull-up current greater than 5μA but less than 80μA to the RUN/SS pin. The additional currents  prevents the

VIN

4.5V TO 5.5V

10.8V TO 13.8V

16V TO 20V

RRUN/SS

50k

150k

330k

discharge of CSS during a fault and also shortens the soft- start period. Using a resistor from RUN/SS pin to VIN is a simple solution to defeat latchoff. Any pull-up network must be able to maintain RUN/SS above 4V maximum latchoff threshold and overcome the 4μA maximum dis- charge current. Figure 3 shows a conceptual drawing of VRUN during  startup and short circuit.

Figure 4. Defeat Short-Circuit Latchoff with a Pull-Up

Resistor to VIN

Enable

The RUN/SS pin can be driven from logic as shown in Figure 5. This function allows the LTM4600 to be turned on or off remotely. The ON signal can also control the sequence of the output voltage.

RUN/SS

ON	LTM4600

PGND

SGND

2N7002

4600 F05

Figure 5. Enable Circuit with External Logic

4600fc

APPLICATIONS INFORMATION

Output Voltage Tracking

For the applications that require output voltage tracking, several LTM4600 modules can be programmed by the power supply tracking controller such as the LTC2923. Figure 6 shows a typical schematic  with LTC2923. Coin- cident, ratiometric and offset tracking for VO rising  and falling can be implemented with different sets of resistor values. See the LTC2923 data sheet for more details.

1. EXTVCC grounded. Internal 5V LDO is always powered from the internal 5V regulator.

2. EXTVCC connected to an external supply. Internal LDO is shut off. A high efﬁciency supply compatible  with the MOSFET gate drive requirements  (typically 5V) can im- prove overall efﬁciency. With this connection, it is always required that the EXTVCC voltage can not be higher than VIN pin voltage.

VIN

5V

DC/DC

RONB

VCC

Q1

GATE

RAMP

3.3V

VIN

VIN

LTM4600

Discontinuous Operation and FCB Pin

The FCB  pin determines whether the internal bottom MOSFET remains on when the inductor current reverses. There is an internal 4.75k pull-down resistor connecting this pin to ground. The default light load operation mode

RONA

ON

LTC2923

FB1

VOSET

49.9k

VOUT

1.8V

is forced continuous  (PWM) current mode. This mode provides minimum output voltage ripple.

RTB1

RTA1

RTB2

RAMPBUF TRACK1

STATUS SDO

VIN

VIN

LTM4600

In the application where the light load efﬁciency is im- portant, tying the FCB pin above 0.6V threshold enables discontinuous operation where the bottom MOSFET turns

RTA2

TRACK2

GND

FB2

VOSET

66.5k

4600 F06

VOUT

1.5V

off when inductor current reverses. Therefore, the conduc- tion loss is minimized and light load efﬁciency is improved. The penalty is that the controller may skip cycle and the

Figure 6. Output Voltage Tracking with the LTC2923 Controller

EXTVCC Connection

An internal low dropout regulator produces an internal 5V supply that powers the control circuitry and FET drivers. Therefore,  if the system does not have a 5V power rail, the LTM4600 can be directly powered by VIN. The gate driver current through LDO is about 18mA. The internal LDO power dissipation can be calculated as:

PLDO_LOSS = 18mA • (VIN – 5V)

The LTM4600 also provides an external gate driver volt-

output voltage ripple increases at light load.

Paralleling Operation with Load Sharing

Two or more LTM4600 modules can be paralleled to provide higher than 10A output current. Figure 7 shows the neces- sary interconnection between two paralleled modules. The OPTI-LOOP® current  mode control ensures good current sharing among modules to balance the thermal stress. The new feedback equation for two or more LTM4600s in parallel is:

100k + R

SET

age pin EXTVCC. If there is a 5V rail in the system, it is recommended to connect EXTVCC pin to the external 5V rail. Whenever the EXTVCC pin is above 4.7V, the internal

5V LDO is shut off and an internal 50mA P-channel switch

VOUT = 0.6V •   N 	 RSET

where N is the number of LTM4600s in parallel.

connects the EXTVCC to internal 5V. Internal 5V is supplied

from  EXTVCC until this pin drops below 4.5V. Do not apply more than 6V to the EXTVCC pin and ensure that EXTVCC

< VIN. The following  list summaries the possible connec- tions for EXTVCC:

OPTI-LOOP is a registered trademark of Linear Technology Corporation.

4600fc

APPLICATIONS INFORMATION

temperature measurements at the bench, and thermal

VIN

VIN 	VOUT

LTM4600

VOUT

(20AMAX)

modeling analysis. Application Note 103 provides a detailed explanation of the analysis for the thermal models, and the

PGND COMP VOSET

SGND

derating curves. Tables 3 and 4 provide a summary of the

RSET

equivalent θJA for the noted conditions. These equivalent

θJA parameters are correlated to the measured values, and

VIN

PGND

COMP VOSET

LTM4600

SGND VOUT

4600 F07

improve with air-ﬂow. The case temperature is maintained

at 100°C or below for the derating curves. This allows for

4W maximum power dissipation in the total module with top and bottom heatsinking, and 2W power dissipation

Figure 7. Parallel  Two μModules with Load Sharing

Thermal Considerations and Output Current Derating

The power loss curves in Figures 8 and 13 can be used in coordination with the load current derating curves in Figures 9 to 12, and Figures 14 to 15 for calculating an approximate  θJA for the module with various heatsink- ing methods. Thermal models are derived from several

through the top of the module with an approximate θJC

between 6°C/W to 9°C/W. This equates to a total of 124°C

at the junction of the device.

Safety Considerations

The LTM4600 modules do not provide isolation from VIN to VOUT. There is no internal fuse. If required, a slow blow fuse with a rating twice the maximum input current should be provided to protect each unit from catastrophic failure.

Table 3. 1.5V Output

| DERATING CURVE | VIN (V) | POWER LOSS CURVE | AIR FLOW (LFM) | HEATSINK | θJA (°C/W) |
| --- | --- | --- | --- | --- | --- |
| Figures 9, 11 | 5, 12 | Figure 8 | 0 | None | 15.2 |
| Figures 9, 11 | 5, 12 | Figure 8 | 200 | None | 14 |
| Figures 9, 11 | 5, 12 | Figure 8 | 400 | None | 12 |
| Figures 10, 12 | 5, 12 | Figure 8 | 0 | BGA Heatsink | 13.9 |
| Figures 10, 12 | 5, 12 | Figure 8 | 200 | BGA Heatsink | 11.3 |
| Figures 10, 12 | 5, 12 | Figure 8 | 400 | BGA Heatsink | 10.25 |

Table 4. 3.3V Output

| DERATING CURVE | VIN (V) | POWER LOSS CURVE | AIR FLOW (LFM) | HEATSINK | θJA (°C/W) |
| --- | --- | --- | --- | --- | --- |
| Figure 14 | 12 | Figure 13 | 0 | None | 15.2 |
| Figure 14 | 12 | Figure 13 | 200 | None | 14.6 |
| Figure 14 | 12 | Figure 13 | 400 | None | 13.4 |
| Figure 15 | 12 | Figure 13 | 0 | BGA Heatsink | 13.9 |
| Figure 15 | 12 | Figure 13 | 200 | BGA Heatsink | 11.1 |
| Figure 15 | 12 | Figure 13 | 400 | BGA Heatsink | 10.5 |

4600fc

APPLICATIONS INFORMATION

4.5

4.0

3.5

3.0

2.5

2.0

POWER LOSS (W)

1.5

1.0

0.5

0

10	10

VOUT = 1.5V

MAXIMUM LOAD CURRENT (A)

MAXIMUM LOAD CURRENT (A)

| 9 9 |  |  |  |  | VIN = 5V VO = 1.5V |
| --- | --- | --- | --- | --- | --- |
|  | 200 LFM |  |  |  |  |

8

8

7

12V LOSS	5V LOSS	6

| 4 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | VIN = 5V VO = 1.5V |  |
| 200 | LFM |  |  |  |  |

0	2	4

6	8	10

50	60

70	80	90

50	60	70	80

90	100

OUTPUT CURRENT (A)

AMBIENT TEMPERATURE (°C)

AMBIENT TEMPERATURE (°C)

4600 F08

4600 F09

4600 G10

Figure 8. Power Loss vs Load Current	Figure 9. No Heatsink	Figure 10. BGA Heatsink

10

MAXIMUM LOAD CURRENT (A)

9

8

7

6

5

| 4 |  |  |  |  |  | VIN = 12V VO = 1.5V |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 LFM 200 LFM 400 LFM |  |  |  |  |  |  |  |

50     55

60     65

70     75     80     85     90

10

9

8

7

MAXIMUM LOAD CURRENT (A)

6

5

| 50 60 70 80 |  |  |  | VIN = 12V VO = 1.5V |  |
| --- | --- | --- | --- | --- | --- |
| 0LF 200 400 | M LFM LFM |  |  |  |  |

90	100

5.0

4.5

4.0

POWER LOSS (W)

3.5

3.0

2.5

2.0

1.5

1.0

0.5

0

| 0 VIN = 12V VOUT = 3.3V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

2	4	6

8	10

AMBIENT TEMPERATURE (°C)

AMBIENT TEMPERATURE (°C)

OUTPUT CURRENT (A)

4600 F11

4600 G12

4600 F13

Figure 11. No Heatsink	Figure 12. BGA Heatsink	Figure 13. Power Loss vs Load Current

10

9

8

7

6

5

4

3

MAXIMUM LOAD CURRENT (A)

2

1

| 0 |  |  |  |  | VIN = 12V VO = 3.3V |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 LFM 200 LFM 400 LFM |  |  |  |  |  |  |

40	50	60

70	80	90

10

9

8

7

MAXIMUM LOAD CURRENT (A)

6

5

40	60	80

| 100 |  |  | VIN = 12V VO = 3.3V |
| --- | --- | --- | --- |
| 0LFM 200LFM 400LFM |  |  |  |

AMBIENT TEMPERATURE (°C)

AMBIENT TEMPERATURE (°C)

4600 F14

Figure 14. No Heatsink	Figure 15. BGA Heatsink

4600 G15

4600fc

APPLICATIONS INFORMATION

Layout Checklist/Example

The high integration of the LTM4600 makes the PCB board layout very simple and easy. However, to optimize its electri- cal and thermal performance, some layout considerations are still necessary.

•  Use large PCB copper  areas for high current path, in- cluding  VIN, PGND and VOUT. It helps to minimize the PCB conduction  loss and thermal stress

•  Place high frequency ceramic input and output capaci- tors next to the VIN, PGND and VOUT pins  to minimize high frequency noise

•  Place a dedicated power ground layer underneath the unit

•  To minimize the via conduction loss and reduce module thermal stress, use multiple vias for interconnection between top layer and other power layers

•  Do not put a via directly on pad unless it is capped

•  Use a separated SGND ground copper area for com- ponents connected to signal pins. Connect the SGND to PGND underneath the unit

Figure 16 gives a good example of the recommended layout.

LTM4600 Frequency Adjustment

The LTM4600 is designed to typically operate at 850kHz across most input and output conditions. The control ar- chitecture is constant on time valley mode current control. The fADJ pin is typically left open or decoupled with an optional 1000pF capacitor. The switching frequency has been optimized to maintain constant output ripple over the operating conditions. The equations for setting the operat- ing frequency are set around a programmable constant on time. This on time is developed by a programmable current into an on board 10pF capacitor that establishes a ramp that is compared to a voltage threshold equal to the output voltage up to a 2.4V clamp. This ION current  is equal to: ION = (VIN – 0.7V)/110k, with the 110k onboard resistor from VIN to fADJ. The on time is equal to tON = (VOUT/ION)

• 10pF and tOFF = ts – tON. The frequency is equal to: Freq.

= DC/tON. The ION current  is proportional to VIN, and the

regulator duty cycle is inversely proportional to VIN, there-

fore the step-down regulator will remain relatively constant

frequency as the duty cycle adjustment takes place with lowering  VIN. The on time is proportional to VOUT up to a

2.4V clamp.  This will hold frequency relatively constant

with different output voltages up to 2.4V. The regulator switching period is comprised  of the on time and off time as depicted in Figure 17.

VIN

CIN

PGND

LOAD

VOUT

TOP LAYER

4600 F16

Figure 16. Recommended PCB Layout

4600fc

APPLICATIONS INFORMATION

(DC) DUTY CYCLE = tON

ts

tON = 0.41 • 1μs ≅ 410ns

tON

VOUT

tOFF

PERIOD ts

tON

DC =  ts    =  VIN

FREQ = DC

tON

4600 F21

tOFF = 1μs – 410ns ≅ 590ns

tON and tOFF are above the minimums with adequate guard band.

Using the frequency = (ION/[2.4V • 10pF]) • DC, solve for

Figure 17. LTM4600 Switching Period

The LTM4600 has a minimum (tON) on time of 100 nanosec- onds and a minimum  (tOFF) off time of 400 nanoseconds. The 2.4V clamp on the ramp threshold as a function of VOUT will cause the switching frequency to increase by the ratio of VOUT/2.4V for 3.3V and 5V outputs. This is due to the fact the on time will not increase as VOUT  increases past 2.4V. Therefore,  if the nominal switching frequency is 850kHz, then the switching  frequency  will increase to ~1.2MHz for 3.3V, and ~1.7MHz for 5V outputs due to Frequency = (DC/tON) When the switching frequency increases to 1.2MHz, then the time period tS is reduced to ~833 nanoseconds and at 1.7MHz the switching period reduces to ~588 nanoseconds. When higher duty cycle conversions  like 5V to 3.3V and 12V to 5V need to be accommodated, then the switching frequency  can be lowered to alleviate the violation of the 400ns minimum off time. Since the total switching period is tS = tON + tOFF , tOFF will be below the 400ns minimum off time. A resistor from the fADJ pin to ground can shunt current away from the on time generator, thus allowing for a longer on time and a lower switching  frequency.  12V to 5V and 5V to

3.3V derivations are explained in the data sheet to lower switching frequency and accommodate these step-down conversions.

Equations for setting frequency for 12V to 5V: ION = (VIN – 0.7V)/110k;  ION = 103μA

frequency = (ION/[2.4V  • 10pF]) • DC  = 1.79MHz;

DC = duty cycle, duty cycle is (VOUT/VIN)

tS = tON  + tOFF, tON  = on-time,  tOFF = off-time of the switching period; tS = 1/frequency

tOFF must be greater than 400ns, or tS – tON > 400ns. tON = DC • tS

1MHz frequency  or 1μs period is chosen for 12V to 5V.

ION = (1MHz • 2.4V • 10pF) • (1/0.41) ≅ 58μA. ION current

calculated from 12V input was 103μA, so a resistor from

fADJ  to ground = (0.7V/15k) = 46μA. 103μA – 46μA =

57μA, sets the adequate ION current for proper frequency

range for the higher duty cycle conversion of 12V to

5V. Input voltage range is limited to 9V to 16V. Higher input voltages can be used without the 15k on fADJ. The inductor ripple current gets too high above 16V, and the

400ns minimum off-time is limited below 9V. Equations for setting frequency for 5V to 3.3V:

ION = (VIN – 0.7V)/110k;  ION = 39μA

frequency  = (ION/[2.4V  • 10pF]) • DC  = 1.07MHz; DC = duty cycle, duty cycle is (VOUT/VIN)

tS = tON  + tOFF, tON  = DC • tS, tOFF  = off-time of the switching period; tS = 1/frequency

tOFF must be greater than 400ns, or tS – tON > 400ns. The ~450kHz frequency or 2.22μs period is chosen for

5V to 3.3V. Frequency  range is about 450kHz to 650kHz

from 4.5V to 7V input.

tON = 0.66 • 2.22μs ≅ 1.46μs tOFF = 2.22μs  – 1.46μs ≅ 760ns

tON and tOFF are above the minimums with adequate guard band.

Using the frequency = (ION/[2.4V • 10pF]) • DC, solve for ION = (450kHz • 2.4V • 10pF) • (1/0.66) ≅ 16μA. ION current calculated from 5V input was 39μA, so a resistor from fADJ to ground = (0.7V/30.1k) = 23μA. 39μA – 23μA = 16μA,

sets the adequate  ION current  for proper frequency range for the higher duty cycle conversion of 5V to 3.3V. Input voltage range is limited to 4.5V to 7V. Higher input voltages can be used without the 30.1k on fADJ. The inductor  ripple current gets too high above 7V, and the 400ns minimum off-time is limited below 4.5V.

4600fc

APPLICATIONS INFORMATION

5V to 3.3V at 8A

4.5V TO 7V C3      	C1

R1

30.1k

C5

10μF

25V

10μF

25V

VIN

fADJ

100pF

3.3V AT 8A

EFFICIENCY = 93%

RUN/SOFT-START

EXTVCC

FCB

RUN/SS

LTM4600

VOUT

VOSET

SVIN

R2

22.1k

1%

C2      +

22μF

C4

330μF

6.3V

COMP

SGND

PGOOD PGND

OPEN DRAIN

4600 F18

5V TO 3.3V AT 8A WITH fADJ  = 30.1k	C1, C3: TDK C3216X5R1E106MT

C2: TAIYO YUDEN, JMK316BJ226ML

C4: SANYO POS CAP, 6TPE330MIL

12V to 5V at 8A

9V TO 16V C3      	C1

R1

15k

C5

10μF

25V

10μF

25V

VIN

fADJ

100pF

5V AT 8A	EFFICIENCY = 94%

RUN/SOFT-START

EXTVCC

FCB

RUN/SS

LTM4600

VOUT

VOSET

SVIN

R2

13.7k

1%

C2      +

22μF

C4

330μF

6.3V

COMP

SGND

PGOOD PGND

OPEN DRAIN

4600 F19

12V TO 5V AT 8A WITH fADJ  = 15k	C1, C3: TDK C3216X5R1E106MT

C2: TAIYO YUDEN, JMK316BJ226ML

C4: SANYO POSCAP, 6TPE330MIL

VIN to VOUT Step-Down Ratio for

12VIN to 5VOUT and 5VIN to 3.3VOUT

4.5

4.0

3.5

VOUT (V)

3.0

2.5

2.0

1.5

1.0

0.5

0

3.3V: fADJ = 30.1k

5V: fADJ = 15k

3.3V AT 8A

5V AT 8A

1	3	5	7	9	11    13    15    17

VIN (V)

4600 F20

4600fc

TYPICAL APPLICATION

VIN

5V TO 20V GND

+	CIN (BULK)

150μF

CIN (CER)

10μF

2x

EXTVCC

VIN

(MULTIPLE PINS)

VOUT

VOUT

VOUT

C3

100pF

SVIN

fADJ

VOSET

(MULTIPLE PINS)

LTM4600

COUT1     +

22μF

6.3V

×3

COUT2

470μF

REFER TO

R1

66.5k REFER TO TABLE 1

C4

OPT

COMP FCB PGOOD

SGND

PGND (MULTIPLE PINS)

RUN/SS

REFER TO TABLE 2

TABLE 2

0.6V TO 5V

REFER TO STEP-DOWN RATIO GRAPH

GND

4600 F17

Figure 18. Typical Application,  5V to 20V Input, 0.6V to 5V Output, 10A Max

4600fc

TYPICAL APPLICATION

Parallel Operation and Load Sharing

C8

10μF

25V

C7

10μF

25V

4.5V TO 20V

EXTVCC

FCB

VIN

fADJ

VOUT

VOSET

VOUT = 0.6V • ([100k/N] + RSET)/RSET

WHERE N = 2

2.5V

C9	+

22μF

C10

470μF

RUN COMP

LTM4600

SVIN

PGOOD

R4

15.8k

1%

x3	4V

SGND

PGND

RUN/SOFT-START

2.5V AT 20A

C3

10μF

25V

C1

10μF

25V

EXTVCC

FCB

VIN

fADJ

VOUT

VOSET

C4

220pF

2.5V

C2	+

22μF

C5

470μF

RUN COMP

LTM4600

SVIN

PGOOD

R1

100k

x3	4V

SGND

PGND

4600 TA02

C1, C3, C7, C8: TDK C3216X5R1E106MT

C2, C9: TAIYO YUDEN, JMK316BJ226ML-T501

C5, C10: SANYO POSCAP, 4TPE470MCL

Current Sharing Between Two

LTM4600 Modules

12VIN

2.5VOUT

10   20AMAX

INDIVIDUAL SHARE

8

IOUT2

IOUT1

2

0

0	5	10	15	20

TOTAL LOAD

4600 TA03

4600fc

LGA Package

104-Lead (15mm × 15mm)

(Reference LTM DWG # 05-05-1800)

6.5475

6.9888

5.2775

6.9421

5.7158

4.4458

3.1758

2 	3 	4

1

5.7650

8

1.9058

0.6358

0.0000

1.9042

3.1742

5 	6 	7 	16         17

0.6342

9 	10         11

4.4442

5.7142

6.9865

18         19

20

21

4.0075 	12

4.4950

13         14         15

22

2.72 – 2.92

aaa  Z	 	   15     	      X

BSC	Y

1.4675

0.0000

2.7375

0.3175

0.3175

25

2.3600

32

1.0900

39         40

23

26         27         28         29         30         31

24

33         34         35         36         37         38

41         42         43         44         45         46         47         48         49

PAD 1

4       CORNER

1.2700

4.4450

2.5400

50         51

61         62

72         73

52         53         54

63         64         65

74         75         76

55         56         57         58         59         60

66         67         68         69         70         71

77         78         79         80         81         82

CAP

SUBSTRATE

15

BSC

5.7150

6.3500

5.0800

3.8100

2.5400

1.2700

83         84         85         86         87

0.3175

0.0000  0.3175

1.2700

2.5400

3.8100

5.0800

6.3500

88         89         90         91         92         93

0.27 – 0.37

6.9850

94         95         96         97         98

99        100       101       102       103       104

2.45 – 2.55

bbb  Z

Z

DETAIL B

0.11 – 0.27

SUGGESTED  SOLDER  PAD LAYOUT TOP VIEW

12.70  	 BSC

PADS

SEE NOTES

NOTES:

DETAIL B

TOP VIEW

aaa  Z

94         95         96         97         98         99        100       101       102       103       104 	T      3

83         84         85         86         87         88         89         90         91         92         93 	R

1. DIMENSIONING AND TOLERANCING PER ASME Y14.5M-1994

2. ALL DIMENSIONS  ARE IN MILLIMETERS

72         73

61         62

50         51

74         75         76         77         78         79         80         81         82 	P

63         64         65         66         67         68         69         70         71 	N

52         53         54         55         56         57         58         59         60 	M

3     LAND DESIGNATION PER JESD MO-222, SPP-010

4      DETAILS OF PAD #1 IDENTIFIER ARE OPTIONAL,

BUT MUST BE LOCATED WITHIN THE ZONE INDICATED. THE PAD #1 IDENTIFIER IS A MARKED FEATURE OR A NOTCHED BEVELED PAD

13.97

BSC	39

32

25

12

8

40         41

42         43         44         45         46         47         48

33         34         35         36         37         38

26         27         28         29         30         31

13         14         15

49 	L

K

24         J

H

23         G

22 		F E

21 	D

5. PRIMARY DATUM -Z- IS SEATING PLANE

| 6. THE TOTAL NUMBER OF PADS: 104 SYMBOL | TOLERANCE |
| --- | --- |
| aaa | 0.15 |
| bbb | 0.10 |
| eee | 0.15 |

C(0.30)

1

2 	3 	4

9 	10         11

5 	6 	7 	16         17

18         19

C

| 20 B A |  | eee | M | X | Y |
| --- | --- | --- | --- | --- | --- |

PAD 1

1 	3 	5

7 	9        11        13

15       17

19        21       23

2 	4 	6 	8

10        12

14         16

18        20         22 	LGA104 0206

LTM4600

PACKAGE DESCRIPTION

4600fc

21

13.93  	 BSC

BOTTOM VIEW

PACKAGE DESCRIPTION

Pin Assignment Tables

(Arranged by Pin Number)

| PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 - | B1 VIN | C1 - | D1 VIN | E1 - | F1 VIN | G1 PGND | H1 - |
| A2 - | B2 - | C2 - | D2 - | E2 - | F2 - | G2 - | H2 - |
| A3 VIN | B3 - | C3 - | D3 - | E3 - | F3 - | G3 - | H3 - |
| A4 - | B4 - | C4 - | D4 - | E4 - | F4 - | G4 - | H4 - |
| A5 VIN | B5 - | C5 - | D5 - | E5 - | F5 - | G5 - | H5 - |
| A6 - | B6 - | C6 - | D6 - | E6 - | F6 - | G6 - | H6 - |
| A7 VIN | B7 - | C7 - | D7 - | E7 - | F7 - | G7 - | H7 PGND |
| A8 - | B8 - | C8 - | D8 - | E8 - | F8 - | G8 - | H8 - |
| A9 VIN | B9 - | C9 - | D9 - | E9 - | F9 - | G9 - | H9 PGND |
| A10 - | B10 - | C10 VIN | D10 - | E10 VIN | F10 - | G10 - | H10 - |
| A11 VIN | B11 - | C11 - | D11 - | E11 - | F11 - | G11 - | H11 PGND |
| A12 - | B12 - | C12 VIN | D12 - | E12 VIN | F12 - | G12 - | H12 - |
| A13 VIN | B13 - | C13 - | D13 - | E13 - | F13 - | G13 - | H13 PGND |
| A14 - | B14 - | C14 VIN | D14 - | E14 VIN | F14 - | G14 - | H14 - |
| A15 fADJ | B15 - | C15 - | D15 - | E15 - | F15 - | G15 - | H15 PGND |
| A16 - | B16 - | C16 - | D16 - | E16 - | F16 - | G16 - | H16 - |
| A17 SVIN | B17 - | C17 - | D17 - | E17 - | F17 - | G17 - | H17 PGND |
| A18 - | B18 - | C18 - | D18 - | E18 - | F18 - | G18 - | H18 - |
| A19 EXTVCC | B19 - | C19 - | D19 - | E19 - | F19 - | G19 - | H19 - |
| A20 - | B20 - | C20 - | D20 - | E20 - | F20 - | G20 - | H20 - |
| A21 VOSET | B21 - | C21 - | D21 - | E21 - | F21 - | G21 - | H21 - |
| A22 - | B22 - | C22 - | D22 - | E22 - | F22 - | G22 - | H22 - |
| A23 - | B23 COMP | C23 - | D23 SGND | E23 - | F23 RUN/SS | G23 FCB | H23 - |

| PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME | PIN NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J1 PGND | K1 - | L1 - | M1 - | N1 - | P1 - | R1 - | T1 - |
| J2 - | K2 - | L2 PGND | M2 PGND | N2 PGND | P2 VOUT | R2 VOUT | T2 VOUT |
| J3 - | K3 - | L3 - | M3 - | N3 - | P3 - | R3 - | T3 - |
| J4 - | K4 - | L4 PGND | M4 PGND | N4 PGND | P4 VOUT | R4 VOUT | T4 VOUT |
| J5 - | K5 - | L5 - | M5 - | N5 - | P5 - | R5 - | T5 - |
| J6 - | K6 - | L6 PGND | M6 PGND | N6 PGND | P6 VOUT | R6 VOUT | T6 VOUT |
| J7 - | K7 PGND | L7 - | M7 - | N7 - | P7 - | R7 - | T7 - |
| J8 - | K8 | L8 PGND | M8 PGND | N8 PGND | P8 VOUT | R8 VOUT | T8 VOUT |
| J9 - | K9 PGND | L9 - | M9 - | N9 - | P9 - | R9 - | T9 - |
| J10 - | K10 | L10 PGND | M10 PGND | N10 PGND | P10 VOUT | R10 VOUT | T10 VOUT |
| J11 - | K11 PGND | L11 - | M11 - | N11 - | P11 - | R11 - | T11 - |
| J12 - | K12 - | L12 PGND | M12 PGND | N12 PGND | P12 VOUT | R12 VOUT | T12 VOUT |
| J13 - | K13 PGND | L13 - | M13 - | N13 - | P13 - | R13 - | T13 - |
| J14 - | K14 - | L14 PGND | M14 PGND | N14 PGND | P14 VOUT | R14 VOUT | T14 VOUT |
| J15 - | K15 PGND | L15 - | M15 - | N15 - | P15 - | R15 - | T15 - |
| J16 - | K16 - | L16 PGND | M16 PGND | N16 PGND | P16 VOUT | R16 VOUT | T16 VOUT |
| J17 - | K17 PGND | L17 - | M17 - | N17 - | P17 - | R17 - | T17 - |
| J18 - | K18 - | L18 PGND | M18 PGND | N18 PGND | P18 VOUT | R18 VOUT | T18 VOUT |
| J19 - | K19 - | L19 - | M19 - | N19 - | P19 - | R19 - | T19 - |
| J20 - | K20 - | L20 PGND | M20 PGND | N20 PGND | P20 VOUT | R20 VOUT | T20 VOUT |
| J21 - | K21 - | L21 - | M21 - | N21 - | P21 - | R21 - | T21 - |
| J22 - | K22 - | L22 PGND | M22 PGND | N22 PGND | P22 VOUT | R22 VOUT | T22 VOUT |
| J23 PGOOD | K23 - | L23 - | M23 - | N23 - | P23 - | R23 - | T23 - |

4600fc

PACKAGE DESCRIPTION

Pin Assignment Tables

(Arranged by Pin Number)

LTM4600

PIN NAME

| PIN NAME P2 P4 P6 P8 P10 P12 P14 P16 P18 P20 P22 | VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT |
| --- | --- |
| R2 R4 R6 R8 R10 R12 R14 R16 R18 R20 R22 | VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT |
| T2 T4 T6 T8 T10 T12 T14 T16 T18 T20 T22 | VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT VOUT |

| PIN NAME A3 A5 A7 A9 A11 A13 | VIN VIN VIN VIN VIN VIN |
| --- | --- |
| B1 | VIN |
| C10 C12 C14 | VIN VIN VIN |
| D1 | VIN |
| E10 E12 E14 | VIN VIN VIN |
| F1 | VIN |

| PIN NAME A15 | fADJ |
| --- | --- |
| A17 | SVIN |
| A19 | EXTVCC |
| A21 | VOSET |
| B23 | COMP |
| D23 | SGND |
| F23 | RUN/SS |
| G23 | FCB |
| J23 | PGOOD |

| G1 | PGND |
| --- | --- |
| H7 H9 H11 H13 H15 H17 | PGND PGND PGND PGND PGND PGND |
| J1 | PGND |
| K7 K9 K11 K13 K15 K17 | PGND PGND PGND PGND PGND PGND |
| L2 L4 L6 L8 L10 L12 L14 L16 L18 L20 L22 | PGND PGND PGND PGND PGND PGND PGND PGND PGND PGND PGND |
| M2 M4 M6 M8 M10 M12 M14 M16 M18 M20 M22 | PGND PGND PGND PGND PGND PGND PGND PGND PGND PGND PGND |
| N2 N4 N6 N8 N10 N12 N14 N16 N18 N20 N22 | PGND PGND PGND PGND PGND PGND PGND PGND PGND PGND PGND |

Information furnished by Linear Technology Corporation is believed to be accurate and reliable. However, no responsibility is assumed for its use. Linear Technology Corporation makes no representation that the interconnection of its circuits as described herein will not infringe on existing patent rights.

4600fc

23

TYPICAL APPLICATION

1.8V, 10A Regulator

C2

10μF

25V

4.5V AT 20V

C1

10μF

25V

EXTVCC

FCB

VIN

fADJ

VOUT

VOSET

C5

100pF

1.8V AT 10A

C3	+

22μF

C4

470μF

RUN

LTM4600

SVIN

R1	x3	4V

100k

COMP

SGND

PGOOD PGND

4600 TA04

R2

49.9k

1%

PGOOD

C1, C2: TDK C3216X5R1E106MT

C3: TAIYO YUDEN, JMK316BJ226ML-T501

C4: SANYO POSCAP, 4TPE470MCL

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LTC2900 | Quad Supply Monitor with Adjustable Reset Timer | Monitors Four Supplies; Adjustable Reset Timer |
| LTC2923 | Power Supply Tracking Controller | Tracks Both Up and Down; Power Supply Sequencing |
| LT3825/LT3837 | Synchronous Isolated Flyback Controllers | No Optocoupler Required; 3.3V, 12A Output; Simple Design |
| LTM4601 | 12A DC/DC μModule with PLL, Output Tracking/ Margining and Remote Sensing | Synchronizable, PolyPhase Operation, LTM4601-1 Version has no Remote Sensing |
| LTM4602 | 6A DC/DC μModule | Pin Compatible with the LTM4600 |
| LTM4603 | 6A DC/DC μModule with PLL and Outpupt Tracking/ Margining and Remote Sensing | Synchronizable, PolyPhase Operation, LTM4603-1 Version has no Remote Sensing, Pin Compatible with the LTM4601 |

This product contains technology licensed from Silicon Semiconductor Corporation.

®

4600fc

Linear Technology Corporation

24

1630 McCarthy  Blvd.,  Milpitas,  CA 95035-7417

(408) 432-1900 ● FAX: (408) 434-0507  ●  www.linear.com

LT 0807 REV C • PRINTED IN USA

© LINEAR TECHNOLOGY CORPORATION 2006