DS1904

RTC iButton

www.iButton.com

SPECIAL FEATURES

	Real-Time Clock/calendar in binary format

	Uses the same binary time/date representation

as the DS1994 but with 1 second resolution

	Clock accuracy is better than ± 2 minutes per

month at 25°C

	Operating temperature range from -40°C to

+70°C

	Over 10 years of operation

	Presence detector acknowledges when reader first applies voltage

	Meets UL#913 (4th Edit.); Intrinsically Safe

Apparatus, Approved under Entity Concept for use in Class I, Division 1, Group A, B, C and D Locations (application pending)

F5 MicroCan™

COMMON iButton FEATURES

	Unique, factory–lasered and tested 64-bit

registration number (8-bit family code + 48- bit serial number + 8-bit CRC tester) assures absolute traceability because no two parts are

5.89

0.51

40

®

®     24

16.25

alike

	Multidrop controller for MicroLAN

	Digital identification and information by

momentary contact

	Chip-based data carrier compactly stores in-

formation

IO 	GND

000000FBC52B

1-Wire®

17.35

	Data can be accessed while affixed to object

	Economically communicates to host with a

single digital signal at 16.3k bits per second

	Standard 16 mm diameter and 1-Wire®

protocol ensure compatibility with iButton®

Device family

	Button shape is self-aligning with cup-shaped

probes

	Durable stainless steel case engraved with

registration number withstands harsh envi- ronments

	Easily affixed with self-stick adhesive back-

ing, latched by its flange, or locked with a ring pressed onto its rim

iButton DESCRIPTION

All dimensions are shown in millimeters

ORDERING INFORMATION

DS1904L-F5	F5 MicroCan

EXAMPLES OF ACCESSORIES DS9096P	Self-Stick Adhesive Pad DS9101	Multi-Purpose Clip DS9093RA	Mounting Lock Ring DS9093A	Snap-In Fob

DS9092	iButton Probe

1-Wire and iButton are registered trademarks of Dallas

Semiconductor.

The  DS1904  RTC  iButton  is  a  rugged  real-time  clock  module  that  can  be  accessed  with  minimal hardware. Data is transferred serially via the 1-Wire protocol, which requires only a single data lead and a ground return. The DS1904 contains a unique 64-bit factory-lasered ROM and a real-time clock/calendar implemented as a binary counter. The durable MicroCan package is highly resistant to environmental hazards such as dirt, moisture, and shock. Accessories permit the DS1904 to be mounted on almost any

surface  including  printed  circuit  boards  and  plastic  key  fobs.  The  DS1904  adds  functions  such  as calendar,  time  and  date  stamp,  stopwatch,  hour  meter,  interval  timer,  and  logbook  to  any  type  of electronic device or embedded application that uses a microcontroller.

OVERVIEW

The DS1904 has two main data components: 1) 64-bit lasered ROM, and 2) real-time clock counter (Figure 1). The real-time clock utilizes an on-chip oscillator that is connected to a 32.768 kHz crystal. The hierarchical structure of the 1-Wire protocol is shown in Figure 2. The bus master must first provide one of four ROM function commands: 1) Read ROM, 2) Match ROM, 3) Search ROM, 4) Skip ROM. The protocol for these ROM functions is described in Figure 7. After a ROM function command is successfully executed, the real-time clock functions become accessible and the master may then provide one of the real-time clock function commands. The protocol for these commands is described in Figure 5. All data is read and written least significant bit first.

BLOCK DIAGRAM Figure 1

ROM

64-BIT

LID

CONTACT

DATA

FUNCTION CONTROL

LASERED ROM

CLOCK FUNCTION CONTROL

READ/WRITE BUFFER

3V LITHIUM

RTC COUNTER (32-BIT)

1 Hz

OSCILLATOR CONTROL

32.768 kHz

OSCILLATOR

DIVIDER

64-BIT LASERED ROM

Each DS1904 contains a unique ROM code that is 64 bits long. The first eight bits are a 1-Wire family code. The next 48 bits are a unique serial number. The last eight bits are a CRC of the first 56 bits. (See Figure 3.) The 1-Wire CRC is generated using a polynomial generator consisting of a shift register and XOR gates as shown in Figure 4. The polynomial is X8 + X5 + X4 + 1. Additional information about the Dallas Semiconductor 1-Wire Cyclic Redundancy Check is available in the Book of DS19xx iButton Standards. The shift register bits are initialized to zero. Then starting with the least significant bit of the family code, one bit at a time is shifted in. After the 8th bit of the family code has been entered, then the serial number is entered. After the 48th bit of the serial number has been entered, the shift register contains the CRC value. Shifting in the eight bits of CRC should return the shift register to all zeros. The

64-bit ROM and ROM Function Control section allow the DS1904 to operate as a 1-Wire device and follow the 1-Wire protocol detailed in the section "1-Wire Bus System".

HIERARCHICAL STRUCTURE FOR 1-WIRE PROTOCOL Figure 2

Bus

Master

1-Wire Bus	Other

Devices

Command

Level

DS1904

Available

Commands

Data Fields

Affected

1-Wire ROM Function

Commands (see Figure 7)

Read ROM Match ROM Search ROM Skip ROM

64-bit ROM

64-bit ROM

64-bit ROM

N/A

DS1904 specific Function Commands (see Figure 5)

Write Clock

Read Clock

RTC Counter, Device Control

RTC Counter, Device Control

64-BIT LASERED ROM Figure 3

MSB	LSB

8-Bit CRC Code	48-Bit Serial Number	8-Bit Family Code (24h)

MSB	LSB	MSB	LSB	MSB	LSB

1-WIRE CRC GENERATOR Figure 4

Polynomial = X8 + X5 + X4 + 1

1ST STAGE

2ND STAGE

3RD STAGE

4TH STAGE

5TH STAGE

6TH STAGE

7TH STAGE

8TH STAGE

X0	X1	X2	X3

X4	X5	X6	X7	X8

INPUT DATA

TIMEKEEPING

A 32.768 kHz crystal oscillator is used as the time base for the real-time clock counter. The oscillator can be turned on or off under software control. The oscillator must be on for the real time clock to function. The real-time clock counter is double buffered. This allows the master to read time without the data changing while it is being read. To accomplish this, a snapshot of the counter data is transferred to a read/write buffer, which the user accesses.

DEVICE CONTROL BYTE

The on/off control of the 32.768 kHz crystal oscillator is done through the device control byte. This byte can be read and written through the Clock Function commands.

Device Control Byte

7	6	5	4	3	2	1	0

| U4 | U3 | U2 | U1 | OSC | OSC | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Bit 0 - 1	0	No function

Bits 0 and 1 are hard-wired to read all 0’s.

Bit 2 - 3	OSC	Oscillator Enable/Disable

These bits control/report whether the 32.768 kHz crystal oscillator is running. If the oscillator is running, both OSC bits will read 1. If the oscillator is turned off these bits will all read 0. When writing the device control byte both occurrences of the OSC bit should have identical data. Otherwise the value in bit ad- dress 3 (bold) takes precedence.

Bit 4 - 7	Un	General-purpose user flags

These non-volatile bits have no particular function within the chip. They can be read and written under the control of the application software.

REAL-TIME CLOCK

The real-time clock is a 32-bit binary counter. It is incremented once per second. The real-time clock can accumulate 136 years of seconds before rolling over. Time/date is represented by the number of seconds since a reference point, which is determined by the user. For example, 12:00 a.m., January 1, 1970 could be a reference point.

CLOCK FUNCTION COMMANDS

The “Clock Function Flow Chart” (Figure 5) describes the protocols necessary for accessing the real-time clock. With only four bytes of real-time clock and one control byte the DS1904 does not provide random access. Reading and writing always starts with the device control byte followed by the least significant byte of the time data.

CLOCK FUNCTION COMMAND FLOW CHART Figure 5

Master TX Control

Function Command

66H 	N Read Clock

?

Y

99H

Write Clock 	N

?

Y

DS1904 copies RTC Counter to R/W Buffer

Bus Master TX Device Control Byte

Bus Master RX Device Control Byte

Bus Master TX LS Byte (7:0)

Bus Master RX LS Byte (7:0)

Bus Master TX

next Byte (15:8)

Bus Master RX

next Byte (15:8)

Bus Master TX

next Byte (23:16)

Bus Master RX

next Byte (23:16)

Bus Master TX MS Byte (31:24)

Bus Master RX MS Byte (31:24)

N	Bus Master

TX Reset

?

Y

N 	Bus Master

TX Reset

?

Y

N 	Bus Master

TX Reset

?

Y

DS1904 copies

R/W Buffer

to RTC Counter

DS1904 TX Presence Pulse

READ CLOCK [66h]

The read clock command is used to read the device control byte and the contents of the real-time clock counter. After having received the most significant bit of the command code the device copies the actual contents of the real-time clock counter to the read/write buffer. Now the bus master reads data beginning with the device control byte followed by the least significant byte through the most significant byte of the real-time clock. After this the bus master may continue reading from the DS1904. The data received will be the same as in the first pass through the command flow. The read clock command can be ended at any point by issuing a Reset Pulse.

WRITE CLOCK [99h]

The write clock command is used to set the real-time clock counter and to write the device control byte. After issuing the command, the bus master writes first the device control byte, which becomes immedi- ately effective. After this the bus master sends the least significant byte through the most significant byte to be written to the real-time clock counter. The new time data is copied from the read/write buffer to the real-time clock counter and becomes effective as the bus master generates a reset pulse. If the oscillator is intentionally stopped, the real-time clock counter behaves as a four-byte non-volatile memory.

1-WIRE BUS SYSTEM

The 1-Wire bus is a system, which has a single bus master and one or more slaves. In all instances the DS1904 is a slave device. The bus master is typically a microcontroller. The discussion of this bus system is broken down into three topics: hardware configuration, transaction sequence, and 1-Wire signaling (signal types and timing). A 1-Wire protocol defines bus transactions in terms of the bus state during specified time slots that are initiated on the falling edge of sync pulses from the bus master. For a more detailed protocol description, refer to Chapter 4 of the Book of DS19xx iButton Standards.

HARDWARE CONFIGURATION Figure 6

BUS MASTER	VPUP

DS1904 1-WIRE PORT

Open Drain

Port Pin

RX

TX

5 k Ω

Typ.

RX = RECEIVE TX = TRANSMIT

5 µA Typ.

DATA	RX

TX

100 Ω

MOSFET

Hardware Configuration

The 1-Wire bus has only a single line by definition; it is important that each device on the bus be able to drive it at the appropriate time. To facilitate this, each device attached to the 1-Wire bus must have open drain or 3-state outputs. The 1-Wire input of the DS1904 is open drain with an internal circuit equivalent to that shown in Figure 6. A multidrop bus consists of a 1-Wire bus with multiple slaves attached. The 1- Wire bus has a maximum data rate of 16.3k bits per second and requires a pull-up resistor of approxi-

mately 5kΩ.

The idle state for the 1-Wire bus is high. If for any reason a transaction needs to be suspended, the bus MUST be left in the idle state if the transaction is to resume. If this does not occur and the bus is left low for more than 120 μs, one or more of the devices on the bus may be reset. Since the DS1904 gets all its energy for operation through its VDD pin it will NOT perform a power-on reset if the 1-Wire bus is low for an extended time period.

Transaction Sequence

The protocol for accessing the DS1904 via the 1-Wire port is as follows:

•	Initialization

•	ROM Function Command

•	Clock Function Command

INITIALIZATION

All transactions on the 1-Wire bus begin with an initialization sequence. The initialization sequence con- sists of a reset pulse transmitted by the bus master followed by presence pulse(s) transmitted by the slave(s). The presence pulse lets the bus master know that the DS1904 is on the bus and is ready to operate. For more details, see the “1-Wire Signaling” section.

ROM FUNCTION COMMANDS

Once the bus master has detected a presence, it can issue one of the four ROM function commands that the DS1904 supports. All ROM function commands are eight bits long. A list of these commands follows (refer to flowchart in Figure 7):

Read ROM [33h]

This command allows the bus master to read the DS1904’s 8-bit family code, unique 48-bit serial num- ber, and 8-bit CRC. This command should only be used if there is a single slave on the bus. If more than one slave is present on the bus, a data collision will occur when all slaves try to transmit at the same time (open drain will produce a wired-AND result). The resultant family code and 48-bit serial number read by the master will be invalid.

Match ROM [55h]

The match ROM command, followed by a 64-bit ROM sequence, allows the bus master to address a spe- cific DS1904 on a multidrop bus. Only the DS1904 that exactly matches the 64-bit ROM sequence will respond to the following clock function command. All slaves that do not match the 64-bit ROM sequence will wait for a reset pulse. This command can be used with a single or multiple devices on the bus.

SEARCH ROM [F0h]

When a system is initially brought up, the bus master might not know the number of devices on the 1- Wire bus or their 64-bit ROM codes. The search ROM command allows the bus master to use a process of elimination to identify the 64-bit ROM codes of all slave devices on the bus. The search ROM process is the repetition of a simple 3-step routine: read a bit, read the complement of the bit, then write the de- sired value of that bit. The bus master performs this 3-step routine on each bit of the ROM. After one complete pass, the bus master knows the 64-bit ROM code of one device. Additional passes will identify the ROM codes of the remaining devices. See Chapter 5 of the Book of DS19xx iButton Standards for a comprehensive discussion of a search ROM, including an actual example.

ROM FUNCTIONS FLOW CHART Figure 7

Master TX Reset Pulse

DS1904 TX Presence Pulse

Master TX ROM Function Command

N

33H	N Read ROM Command

?

Y

55H Match ROM Command

?

Y

F0H Search ROM Command

?

| N CCH N Skip ROM Command | Y |
| --- | --- |
| DS1904 TX Bit 0 |  |
| DS1904 TX Bit 0 |  |
| Master TX Bit 0 |  |

?

| Y | Y |
| --- | --- |
| DS1904 TX Bit 63 |  |
| DS1904 TX Bit 63 |  |
| Master TX Bit 63 |  |

DS1904 TX Family Code

1 Byte

Master  TX Bit 0

| Bit 0 N N Match ? | Y |
| --- | --- |
| DS1904 TX Bit 1 |  |
| DS1904 TX Bit 1 |  |
| Master TX Bit 1 |  |

|  | Y |
| --- | --- |
| Master TX Bit 1 |  |

Bit 0

Match ?

DS1904 TX Serial Number

6 Bytes

Bit 1	N	N Match ?

Y

Bit 1

Match ?

DS1904 TX CRC Byte

Master  TX Bit 63

Bit 63	N	N Match ?

Bit 63

Match ?

Y 	Y

Master TX Control

Function Command

(SEE FIGURE 5)

Skip ROM [CCh]

This command can save time in a single drop bus system by allowing the bus master to access the clock functions without providing the 64-bit ROM code. If more than one slave is present on the bus and, for example, a read command is issued following the Skip ROM command, data collision will occur on the bus as multiple slaves transmit simultaneously (open drain pull-downs will produce a wired-AND result).

1–WIRE SIGNALING

The DS1904 requires strict protocols to ensure data integrity. The protocol consists of four types of sig- naling on one line: Reset Sequence with Reset Pulse and Presence Pulse, Write 0, Write 1 and Read Data. Except for the presence pulse the bus master initiates all these signals.

The initialization sequence required to begin any communication with the DS1904 is shown in Figure 8. A reset pulse followed by a presence pulse indicates the DS1904 is ready to send or receive data. The bus master transmits (TX) a reset pulse (tRSTL, minimum 480 μs). The bus master then releases the line and goes into receive mode (RX). The 1-Wire bus is pulled to a high state via the pull-up resistor. After detecting the rising edge on the data line, the DS1904 waits (tPDH, 15-60 μs) and then transmits the presence pulse (tPDL, 60-240 μs).

INITIALIZATION PROCEDURE “RESET AND PRESENCE PULSES” Figure 8

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

MASTER TX "RESET PULSE"

tRSTL

MASTER RX "PRESENCE PULSE" tRSTH

tR	tPDH	tPDL

RESISTOR MASTER DS1904

480 µs ≤ tRSTL < ∞*

480 µs ≤ tRSTH < ∞**

15 µs ≤ tPDH < 60 µs

60 ≤ tPDL < 240 µs

*	In order not to mask interrupt signaling by other devices on the 1-Wire bus tRSTL + tR should al- ways be less than 960 µs.

**	Includes recovery time

READ/WRITE TIME SLOTS

The definitions of write and read time slots are illustrated in Figure 9. The master initiates all time slots by driving the data line low. The falling edge of the data line synchronizes the DS1904 to the master by triggering an internal delay circuit. During write time slots, the delay circuit determines when the DS1904

will sample the data line. For a read data time slot, if a “0” is to be transmitted, the delay circuit deter-

mines how long the DS1904 will hold the data line low. If the data bit is a “1”, the DS1904 will not hold the data line low at all.

READ/WRITE TIMING DIAGRAM Figure 9

Write-one Time Slot

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

tLOW1

15µs

tSLOT 	tREC

DS1904	 	 Sampling Window

RESISTOR MASTER

60µs

60 µs ≤ tSLOT < 120 µs

1 µs ≤ tLOW1 < 15 µs

1 µs ≤ tREC < ∞

Write-zero Time Slot

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

tSLOT

DS1904	 	 Sampling Window

15µs

60µs

t LOW0

tREC

RESISTOR MASTER

60 µs ≤ tLOW0 < tSLOT < 120 µs

1 µs ≤ tREC < ∞

Read-data Time Slot

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

tSU

tLOWR

tSLOT 	tREC

Master

Sampling Window

tRELEASE

tRDV

RESISTOR

60 µs ≤ tSLOT < 120 µs

1 µs ≤ t

REC < ∞

MASTER

DS1904

1 µs ≤ tLOWR < 15 µs

0 ≤ tRELEASE  < 45 µs

tRDV = 15 µs tSU < 1 µs

PHYSICAL SPECIFICATION

Size	See mechanical drawing

Weight	3.3 grams

Expected Service Life	10 years at 25°C

Safety	Meets UL#913 (4th Edit.); Intrinsically Safe Apparatus, Approval under Entity Concept for use in Class I, Division 1, Group A, B, C and D Locations (application pending)

ABSOLUTE MAXIMUM RATINGS*

Voltage on 1-Wire to Ground	-0.5V to +7.0V Operating Temperature	-40°C to +70°C Storage Temperature	-40°C to +70°C

*  This is a stress rating only and functional operation of the device at these or any other conditions above those indicated in the operation sections of this specification is not implied. Exposure to abso- lute maximum rating conditions for extended periods of time may affect reliability.

| DC ELECTRICAL CHARACTERISTICS (VPUP=2.8V to 6.0V; -40°C to +70°C) PARAMETER | SYMBOL | MIN | TYP | MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Logic 1 | VIH | 2.2 |  | 6.0 | V | 1 |
| Logic 0 | VIL | -0.3 |  | 0.8 | V | 1,5 |
| Output Logic Low @ 4 mA | VOL |  |  | 0.4 | V | 1 |
| Output Logic High | VOH |  |  | VPUP | V | 1,2 |
| Input Load Current | IL |  | 5 |  | µA | 3 |

| CAPACITANCE (tA = 25°C) PARAMETER | SYMBOL | MIN | TYP | MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Capacitance 1-Wire | CIN |  |  | 50 | pF | 8 |

| AC ELECTRICAL CHARACTERISTICS (VPUP=2.8V to 6.0V; -40°C to +70°C) PARAMETER | SYMBOL | MIN | TYP | MAX | UNITS | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Time Slot | tSLOT | 60 |  | 120 | µs |  |
| Write 1 Low Time | tLOW1 | 1 |  | 15 | µs |  |
| Write 0 Low Time | tLOW0 | 60 |  | 120 | µs |  |
| Read Low Time | tLOWR | 1 |  | 15 | µs |  |
| Read Data Valid | tRDV | exactly 15 |  |  | µs | 7 |
| Release Time | tRELEASE | 0 | 15 | 45 | µs |  |
| Read Data Setup | tSU |  |  | 1 | µs | 4 |
| Recovery Time | tREC | 1 |  |  | µs |  |
| Reset High Time | tRSTH | 480 |  |  | µs |  |
| Reset Low Time | tRSTL | 480 |  | 960 | µs | 6 |
| Presence Detect High | tPDH | 15 |  | 60 | µs |  |
| Presence Detect Low | tPDL | 60 |  | 240 | µs |  |
| RTC Accuracy @ 25°C |  | -2 |  | +2 | min/mth | 9 |

NOTES:

1.   All voltages are referenced to ground.

2.   VPUP = external pull-up voltage.

3.   Input load is to ground.

4.   Read data setup time refers to the time the host must pull the 1-Wire bus low to read a bit. Data is guaranteed to be valid within 1 µs of this falling edge.

5.   Under certain low voltage conditions VIL1MAX may have to be reduced to as much as 0.5V to always guarantee a presence pulse.

6.   The reset low time (tRSTL) should be restricted to a maximum of 960 µs, to allow interrupt signaling, otherwise, it could mask or conceal interrupt pulses.

7.   The master must read while the data is valid.

8.   Guaranteed by design; not production tested.

9.   This specification applies if the 1-Wire is idle (high or low). Communication on the 1-Wire may adversely affect the accuracy of the device. For highest accuracy, connect the DS1904 to a separate 1- Wire port and limit the access to the minimum acceptable by the application.