RELIABILITY REPORT FOR

DS1963S Dallas Semiconductor

4401 South Beltwood Parkway

Dallas, TX 75244-3292

Prepared by:

03/11/2003

Ken Wendel

Reliability Engineering Manager

Dallas Semiconductor

4401 South Beltwood Pkwy. Dallas, TX 75244-3292

Email : ken.wendel@dalsemi.com ph:  972-371-3726

fax: 972-371-6016 mbl: 214-435-6610

Conclusion:

The following qualification successfully meets the quality and reliability standards required of all Dallas

Semiconductor products and processes:

DS1963S

In addition, Dallas Semiconductor's continuous reliability monitor program ensures that all outgoing product will continue to meet Maxim's quality and reliability standards. The current status of the reliability monitor program can be viewed at http://www.maxim-ic.com/TechSupport /dsreliability.html.*

Module Description:

A description of this Module can be found in the product data sheet. You can find the product data sheet at http://dbserv.maxim-ic.com/l_datasheet3.cfm.*

Reliability Derating:

A module device consists of one or more IC's in a single, upward integrated, package. This package is assembled to include batteries, crystals, and other piece parts that make up the configuration of the Module.  Because of either the complexity of the package or the included piece parts, standard high temperature reliability testing is not possible.  Therefore, in order to determine the reliability of module products, the reliability of each of the piece parts is individually determined, then summed to determine the reliability of the integrated module product. If there are "n" significant components in the module then:

Fr (module) = Fr (1) + Fr (2) + Fr (3) + ..... + Fr (n) Fr (module) = Failure rate of module

Fr(n) = Failure rate of the nth component

Failure Rates are reported in FITs (Failures in Time) or MTTF (Mean Time To Failure). The FIT rate is related to MTTF by:

MTTF = 1/Fr

NOTE:  MTTF is frequently used interchangeably with MTBF.

The calculated failure rate for this module/assembly is:

Module Device:	Quantity:	MTTF (Yrs):	FITs:

DS2421	1

Totals:

20404

20404

5.6

6

The parameters used to calculate the module failure rate are as follows

Cf:  60%	Ea:  0.7	B:  0	Tu:  25	°C

Vu:  5.5

Volts

The reliability data follows. A the start of this data is the module assembly information. This is a description of the module. The next section is the detailed reliability data for each stress found in the qualification / monitor. If there are additional processes or assemblies used as part of this report, a description of each will follow which includes the respective reliability data for that process/

assembly.  The reliability data section includes the latest data available.

* Some proprietary products may be excepted from this requirement.

Assembly Information:

Qualification Vehicle:	DS1963S Assembly Site:	Dallas

Pin Count:	2

Package Type:	iButton F50 w/Bump

Body Size:	0

Mold Compound:	FP4323, Dexter Hysol

Lead Frame:	Printed Crt Brd; FR4

Lead Finsh:

Die Attach:	Underfill FP4527, Dexter Hysol

Bond Wire / Size:	Al / 1.25 mil

Flammability:	UL 94-V0

Moisture Sensitivity

(JEDEC J-STD20A)

Date Code Range:

9939	to

9939

MECHANICAL LIFE

DESCRIPTION

DATE CODE CONDITION

READPOINT

QUANTITY	FAILS

MECHANICAL SHOCK

9939

200G, 1/2 SINE, 6 MS

30	CYS

50	0

VIBRATION, VARIABLE F

9939

10g or 0.06", 5Hz-2KHz, X Y Z axis

9	HRS

Total:

50	0

0

STORAGE LIFE

DESCRIPTION

DATE CODE CONDITION

READPOINT

QUANTITY	FAILS

STORAGE LIFE

9939

25 C

1144

HRS

22	0

STORAGE LIFE

9939

85 C

1000

HRS

Total:

77	0

0

TEMPERATURE CYCLE

DESCRIPTION

DATE CODE CONDITION

READPOINT

QUANTITY	FAILS

TEMP CYCLE

9939

-40 TO 85C

1000

CYS

Total:

77	3

3

UNBIASED MOISTURE RESISTANCE

DESCRIPTION

DATE CODE CONDITION

READPOINT

QUANTITY	FAILS

MOISTURE SOAK

9939

85 C/85% R.H.

959

HRS

Total:

77	0

0