DS1992/DS1993

1Kb/4Kb Memory iButton®

SPECIAL FEATURES

ƒ	4096 bits of Read/Write Nonvolatile

Memory (DS1993)

ƒ	1024 bits of Read/Write Nonvolatile

Memory (DS1992)

ƒ	256-bit Scratchpad Ensures Integrity of Data

Transfer

ƒ	Memory Partitioned into 256-bit Pages for

Packetizing Data

ƒ	Data Integrity Assured with Strict

Read/Write Protocols

ƒ	Operating Temperature Range from -40°C to

+70°C

ƒ	Over 10 years of data retention

ORDERING INFORMATION DS1992L-F5	F5 MicroCan DS1993L-F5	F5 MicroCan

EXAMPLES OF ACCESSORIES

DS9096P Self-Stick Adhesive Pad DS9101 Multipurpose Clip DS9093RA Mounting Lock Ring DS9093F Snap-In Fob

DS9092 iButton Probe

F5 MicroCan

COMMON iButton FEATURES

ƒ	Unique,  Factory-Lasered  and  Tested  64-bit

Registration Number (8-bit Family Code +

48-bit Serial Number + 8-bit CRC Tester) Assures Absolute  Traceability  Because  No Two Parts are Alike

ƒ	Multidrop Controller for MicroLAN

ƒ	Digital  Identification  and  Information  by

Momentary Contact

ƒ	Chip-Based Data Carrier Compactly Stores

Information

ƒ	Data  Can  be  Accessed  While  Affixed  to

Object

ƒ	Economically Communicates to Bus Master

with a Single Digital Signal at 16.3kbps

ƒ	Standard 16mm Diameter and 1-Wire®

Protocol Ensure Compatibility with iButton®

Family

ƒ	Button Shape is Self-Aligning with Cup-

Shaped Probes

ƒ	Durable Stainless Steel Case Engraved with

Registration Number Withstands Harsh

Environments

ƒ	Easily Affixed with Self-Stick Adhesive

Backing, Latched by its Flange, or Locked with a Ring Pressed onto its Rim

ƒ	Presence Detector Acknowledges When

Reader First Applies Voltage

ƒ	Meets UL#913 (4th Edit.); Intrinsically Safe

Apparatus, Approved under Entity Concept for use in Class I, Division 1, Group A, B, C and D Locations

1-Wire and iButton are registered trademarks of Maxim Integrated Products, Inc.

iButton DESCRIPTION

The DS1992/DS1993 memory iButtons (hereafter referred to as DS199_) are rugged read/write data carriers  that  act  as  a  localized  database,  easily  accessible  with  minimal  hardware.  The  nonvolatile memory and optional timekeeping capability offer a simple solution to storing and retrieving vital information pertaining to the object to which the iButton is attached. Data is transferred serially through the 1-Wire protocol that requires only a single data lead and a ground return.

The scratchpad is an additional page that acts as a buffer when writing to memory. Data is first written to the scratchpad where it can be read back. After the data has been verified, a copy scratchpad command transfers the data to memory. This process ensures data integrity when modifying the memory. A 48-bit serial number is factory lasered into each DS199_ to provide a guaranteed unique identity that allows for absolute traceability. The durable MicroCan package is highly resistant to environmental hazards such as dirt, moisture, and shock. Its compact coin-shaped profile is self-aligning with mating receptacles, allowing the DS199_ to be easily used by human operators. Accessories permit the DS199_ to be mounted on almost any surface including plastic key fobs, photo-ID badges, and PC boards.

Applications include access control, work-in-progress tracking, electronic travelers, storage of calibration constants, and debit tokens.

OPERATION

The DS199_ have three main data components: 1) 64-bit lasered ROM, 2) 256-bit scratchpad, and 3)

1024-bit (DS1992) or 4096-bit (DS1993) SRAM. All data is read and written least significant bit first.

The memory functions are not available until the ROM function protocol has been established. This protocol is described in the ROM functions flow chart (Figure 9). The master must first provide one of four ROM function commands: 1) read ROM, 2) match ROM, 3) search ROM, or 4) skip ROM. After a ROM function sequence has been successfully executed, the memory functions are accessible and the master can then provide any one of the four memory function commands (Figure 6).

PARASITE POWER

The block diagram (Figure 1) shows the parasite-powered circuitry. This circuitry steals power whenever the data input is high. The data line provides sufficient power as long as the specified timing and voltage requirements are met. The advantages of parasite power are two-fold: 1) by parasiting off this input, battery power is not consumed for 1-Wire ROM function commands, and 2) if the battery is exhausted for any reason, the ROM may still be read normally. The remaining circuitry of the DS1992 and DS1993 is solely operated by battery energy.

64-BIT LASERED ROM

Each DS199_ contain a unique ROM code that is 64 bits long. The first 8 bits are a 1-Wire family code. The next 48 bits are a unique serial number. The last 8 bits are a CRC of the first 56 bits. (See Figure 2.) The 1-Wire CRC is generated using a polynomial generator consisting of a shift register and XOR gates as shown in Figure 3. The polynomial is X8 + X5 + X4 + 1. Additional information about the Dallas 1-Wire Cyclic Redundancy Check is available in the Book of DS19xx iButton Standards. The shift register bits

are initialized to zero. Then starting with the least significant bit of the family code, 1 bit at a time is shifted in. After the 8th bit of the family code has been entered, then the serial number is entered. After the 48th bit of the serial number has been entered, the shift register contains the CRC value. Shifting in the 8 bits of CRC should return the shift register to all zeros.

Figure 1. DS199_ BLOCK DIAGRAM

1-WIRE PORT

1-W

ROM FUNCTION CONTROL

64-BIT LASERED ROM

PARASITE- POWERED CIRCUITRY

MEMORY FUNCTION CONTROL

256-BIT SCRATCHPAD

3V LITHIUM

SRAM

16 PAGES of 256- BITs (1993)

4 PAGES of 256- BITs (1992)

Figure 2. 64-BIT LASERED ROM

MSB	LSB

8-Bit CRC Code	48-Bit Serial Number

8-Bit Family Code (06h)1993 (08h)1992

MSB	LSB	MSB	LSB	MSB	LSB

Figure 3. 1-WIRE CRC CODE

Polynomial = X8 + X5 + X4 + 1

1st

STAGE

2nd

STAGE

3rd

STAGE

4th

STAGE

5th

STAGE

6th

STAGE

7th

STAGE

8th

STAGE

X0	X1	X2	X3	X4	X5	X6	X7	X8

INPUT DATA

Figure 4a. DS1993 MEMORY MAP

SCRATCHPAD

PAGE

NOTE: Each page is 32 bytes (256 bits). The hex values represent the starting address for each page or register.

PAGE 0

PAGE 1

PAGE 2

PAGE 3

PAGE 4

PAGE 5

PAGE 6

PAGE 7

0000h

0020h

0040h

0060h

0080h

00A0h

00C0h

00E0h

MEMORY

PAGE 8

PAGE 9

PAGE 10

PAGE 11

PAGE 12

PAGE 13

PAGE 14

PAGE 15

0100h

0120h

0140h

0160h

0180h

01A0h

01C0h

01E0h

Figure 4b. DS1992 MEMORY MAP

SCRATCHPAD

PAGE

NOTE: Each page is 32 bytes (256 bits). The hex values represent the starting address for each page or register.

PAGE 0

0000h

MEMORY

PAGE 1

PAGE 2

PAGE 3

0020h

0040h

0060h

MEMORY

The memory map in Figure 4 shows a 32-Byte page called the scratchpad, and additional 32-Byte pages called memory. The DS1992 contains pages 0 though 3 that make up the 1024-bit SRAM. The DS1993 contain pages 0 through 15 that make up the 4096-bit SRAM.

The scratchpad is an additional page that acts as a buffer when writing to memory. Data is first written to the scratchpad where it can be read back. After the data has been verified, a copy scratchpad command transfers the data to memory. This process ensures data integrity when modifying the memory.

MEMORY FUNCTION COMMANDS

The Memory Function Flow Chart (Figure 6) describes the protocols necessary for accessing the memory. An example follows the flow chart. Three address registers are provided as shown in Figure 5. The first two registers represent a 16-bit target address (TA1, TA2). The third register is the ending offset/data status byte (E/S).

The target address points to a unique Byte location in memory. The first 5 bits of the target address (T4:T0) represent the Byte offset within a page. This Byte offset points to one of 32 possible Byte locations within a given page. For instance, 00000b points to the first Byte of a page where as 11111b would point to the last Byte of a page.

The third register (E/S) is a read only register. The first 5 bits (E4: E0) of this register are called the ending offset. The ending offset is a Byte offset within a page (1 of 32 Bytes). Bit 5 (PF) is the partial Byte flag. Bit 6 (OF) is the overflow flag. Bit 7 (AA) is the authorization accepted flag.

Figure 5. ADDRESS REGISTERS

TARGET ADDRESS (TA1)

7	6	5	4	3	2	1	0

TARGET ADDRESS (TA2)

| ENDING ADDRESS WITH DATA STATUS (E/S) (READ ONLY) T7 | T6 | T5 | T4 | T3 | T2 | T1 | T0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T15 | T14 | T13 | T12 | T11 | T10 | T9 | T8 |
| AA | OF | PF | E4 | E3 | E2 | E1 | E0 |

Write Scratchpad Command [0Fh]

After issuing the write scratchpad command, the user must first provide the 2-Byte target address, followed by the data to be written to the scratchpad. The data is written to the scratchpad starting at the

byte offset (T4:T0). The ending offset (E4:E0) is the Byte offset at which the host stops writing data. The maximum ending offset is 11111b (31d). If the host attempts to write data past this maximum offset, the overflow flag (OF) is set and the remaining data is ignored. If the user writes an incomplete Byte and an overflow has not occurred, the partial Byte flag (PF) is set.

Read Scratchpad Command [AAh]

This command can be used to verify scratchpad data and target address. After issuing the read scratchpad command, the user can begin reading. The first two Bytes are the target address. The next Byte is the ending offset/data status Byte (E/S) followed by the scratchpad data beginning at the Byte offset (T4: T0). The user can read data until the end of the scratchpad, after which the data read is all logic 1’s.

Copy Scratchpad [55h]

This command is used to copy data from the scratchpad to memory. After issuing the copy scratchpad command, the user must provide a 3-byte authorization pattern. This pattern must exactly match the data contained in the three address registers (TA1, TA2, E/S, in that order). If the pattern matches, the AA (Authorization Accepted) flag is set and the copy begins. A logic 0 is transmitted after the data has been copied until the user issues a reset pulse. Any attempt to reset the part is ignored while the copy is in

progress. Copy typically takes 30μs.

The  data  to  be  copied  is  determined  by  the  three  address  registers.  The  scratchpad  data  from  the beginning offset through the ending offset is copied to memory, starting at the target address. Anywhere from 1 to 32 Bytes can be copied to memory with this command. Whole Bytes are copied even if only partially written. The AA flag is cleared only by executing a write scratchpad command.

Read Memory [F0h]

The read memory command can be used to read the entire memory. After issuing the command, the user must provide the 2-Byte target address. After the two Bytes, the user reads data beginning from the target address and may continue until the end of memory, at which point logic 1’s are read. It is important to realize that the target address registers contains the address provided. The ending offset/data status Byte is unaffected.

The hardware of the DS1992/DS1993 provides a means to accomplish error-free writing to the memory section. To safeguard reading data in the 1-Wire environment and to simultaneously speed up data transfers, it is recommended to packetize data into data packets of the size of one memory page each. Such a packet would typically store a 16-bit CRC with each page of data to ensure rapid, error-free data transfers that eliminate having to read a page multiple times to determine if the received data is correct or not. (See Application Note 114 for the recommended file structure to be used with the 1-Wire environment.)

Figure 6. MEMORY FUNCTIONS FLOW CHART

Master TX Memory

Function Command

0FH

Write	N

Scratchpad

?

Y

Bus Master TX TA1 (T7:T0)

AAH Read Scratchpad

?

Y

Bus Master RX TA1 (T7:T0)

To Figure 6

Second Part

Bus Master TX TA2 (T15:T8)

Bus Master RX TA2 (T15:T8)

DS199X sets Scratchpad Offset = (T4:T0) and Clears (PF, OF, AA)

Master RX Ending Offset with Data Status (E/S)

Master TX Data Byte

To Scratchpad Offset

DS199X sets (E4:E0)

= Scratchpad Offset

Bus Master	Y TX Reset

DS199X Sets Scratchpad Offset=(T4:T0)

Master RX Data Byte From Scratchpad Offset

Y

DS199X Increments	? Scratchpad Offset		N

DS199X Increments

Bus Master

TX Reset

?

N		Scratch- pad Offset =

Scratchpad Offset	N

11111b ?

Y

Y	Bus Master

TX Data

Partial	Y Byte W ritten

?

N

PF = 1

Scratch-

pad Offset =

N

11111b ?

Y Bus Master

RX "1"s

?

OF = 1	N

N	Bus Master

TX Reset

?

Y

From Figure 6

Second Part

DS199X TX

Presence Pulse

(See Figure 9)

Figure 6. MEMORY FUNCTIONS FLOW CHART (Continued)

From Figure 6

First Part

55H Copy Scratchpad

F0H	Y Read Memory

?	Y	N	?

Bus Master TX TA1 (T7:T0)

N

Bus Master TX TA2 (T15:T8)

Bus Master TX E/S Byte

Auth-

rization	 N

Code Match

?

Bus Master TX TA1 (T7:T0)

Bus Master TX TA2 (T15:T8)

DS199X sets Memory

Address = (T15:T0)

Y AA = 1

DS199X TX "1"s

DS199X Increments

Master RX Data

Byte From

Memory Address

DS199X Copies

Scratchpad Data

To Memory

Address Counter

Y

Bus Master

TX Reset

?

DS199X TX "0"s

N	Bus Master

TX Reset

?

Y

N Bus Master

TX Reset

?

N

N	Memory

Address

= 21Dh ?

Y

Bus Master

RX "1"s

Y

To Figure 6

First Part

MEMORY FUNCTION EXAMPLES

Example: Write two data Bytes to memory locations 0026h and 0027h (the seventh and eighth Bytes of page 1). Read entire memory.

| MASTER MODE | DATA (LSB FIRST) | COMMENTS |
| --- | --- | --- |
| TX | Reset | Reset pulse (480μs to 960μs) |
| RX | Presence | Presence pulse |
| TX | CCh | Issue skip ROM command |
| TX | 0Fh | Issue write scratchpad command |
| TX | 26h | TA1, beginning offset = 6 |
| TX | 00h | TA2, address = 0026h |
| TX | <2 data Bytes> | Write 2 Bytes of data to scratchpad |
| TX | Reset | Reset pulse |
| RX | Presence | Presence pulse |
| TX | CCh | Issue skip ROM command |
| TX | Aah | Issue read scratchpad command |
| RX | 26h | Read TA1, beginning offset = 6 |
| RX | 00h | Read TA2, address = 0026h |
| RX | 07h | Read E/S, ending offset = 7, flags = 0 |
| RX | <2 data Bytes> | Read scratchpad data and verify |
| TX | Reset | Reset pulse |
| RX | Presence | Presence pulse |
| TX | CCh | Issue skip ROM command |
| TX | 55h | Issue copy scratchpad command |
| TX | 26h | TA1 TA2 AUTHORIZATION CODE E/S |
| TX | 00h |  |
| TX | 07h |  |
| TX | Reset | Reset pulse |
| RX | Presence | Presence pulse |
| TX | CCh | Issue skip ROM command |
| TX | F0h | Issue read memory command |
| TX | 00h | TA1, beginning offset = 6 |
| TX | 00h | TA2, address = 0000h |
| RX | <128 Bytes (DS1992)> <512 Bytes (DS1993)> | Read entire memory |
| TX | Reset | Reset pulse |
| RX | Presence | Presence pulse, done |

1-WIRE BUS SYSTEM

The 1-Wire bus is a system that has a single bus master and one or more slaves. In all instances the DS199_ is a slave device. The bus master is typically a microcontroller or PC. For small configurations the 1-Wire communication signals can be generated under software control using a single port pin. For multisensor networks, the DS2480B 1-Wire line driver chip or serial port adapters based on this chip (DS9097U series) are recommended. This simplifies the hardware design and frees the microprocessor from responding in real-time.

The discussion of this bus system is broken down into three topics: hardware configuration, transaction sequence, and 1-Wire signaling (signal types and timing). The 1-Wire protocol defines bus transactions in terms of the bus state during specific time slots that are initiated on the falling edge of sync pulses from the bus master. For a more detailed protocol description, refer to Chapter 4 of the Book of DS19xx iButton Standards.

HARDWARE CONFIGURATION

The 1-Wire bus has only a single line by definition; it is important that each device on the bus be able to drive it at the appropriate time. To facilitate this, each device attached to the 1-wire bus must have open- drain or three-state outputs. The 1-Wire port of the DS199_ is open  drain with an internal circuit equivalent to that shown in Figure 8. A multidrop bus consists of a 1-Wire bus with multiple slaves attached.  The  1-Wire  bus  has  a  maximum  data  rate  of  16.3kbps  and  requires  a  pullup  resistor  of

approximately 5kΩ. The idle state for the 1-Wire bus is high. If for any reason a transaction needs to be suspended, the bus must be left in the idle state if the transaction is to resume. If this does not occur and the bus is left low for more than 120μs, one or more of the devices on the bus may be reset.

Figure 8. HARDWARE CONFIGURATION

BUS MASTER

VPUP

DS199X 1-Wire PORT

RPU

RX

DATA	RX

TX	RX = RECEIVE

5 µA	TX Typ.

Open Drain

Port Pin

TX = TRANSMIT	100 Ω

MOSFET

TRANSACTION SEQUENCE

The protocol for accessing the DS199_ through the 1-Wire port is as follows:

ƒ	Initialization

ƒ	ROM Function Command

ƒ	Memory Function Command

ƒ	Transaction/Data

INITIALIZATION

All transactions on the 1-wire bus begin with an initialization sequence. The initialization sequence consists of a reset pulse transmitted by the bus master followed by presence pulse(s) transmitted by the

slave(s). The presence pulse lets the bus master know that the DS199_ is on the bus and is ready to operate. For more details, see the 1-Wire Signaling section.

ROM FUNCTION COMMANDS

Once the bus master has detected a presence, it can issue one of the four ROM function commands. All

ROM function commands are 8 bits long. A list of these commands follows (see the flow chart in Figure

9).

Read ROM [33h]

This command allows the bus master to read the DS199_’s 8-bit family code, unique 48-bit serial number, and 8-bit CRC. This command should only be used if there is a single DS199_ on the bus. If more than one slave is present on the bus, a data collision occurs when all slaves try to transmit at the same time (open drain produces a wired-AND result). The resultant family code and 48-bit serial number usually result in a mismatch of the CRC.

Match ROM [55h]

The match ROM command, followed by a 64-bit ROM sequence, allows the bus master to address a specific DS199_ on a multidrop bus. Only the DS199_ that exactly matches the 64-bit ROM sequence will respond to the following memory function command. All slaves that do not match the 64-bit ROM sequence wait for a reset pulse. This command can be used with single or multiple devices on the bus.

Skip ROM [CCh]

This command can save time in a single drop bus system by allowing the bus master to access the memory functions without providing the 64-bit ROM code. If more than one slave is present on the bus and, for example, a read command is issued following the Skip ROM command, data collision will occur

on the bus as multiple slaves transmit simultaneously (open-drain pulldowns produce a wired-AND

result).

Search ROM [F0h]

When a system is initially brought up, the bus master may not know the number of devices on the 1-Wire bus or their 64-bit ROM codes. The search ROM command allows the bus master to use a process of elimination to identify the 64-bit ROM codes of all slave devices on the bus. The search ROM process is the repetition of a simple 3-step routine: read a bit, read the complement of the bit, then write the desired value of that bit. The bus master performs this simple, 3-step routine on each bit of the ROM. After one complete pass, the bus master knows the 64-bit ROM code of one device. Additional passes will identify the ROM codes of the remaining devices. See Chapter 5 of the Book of DS19xx iButton Standards for a comprehensive discussion of a search ROM, including an actual example.

1-WIRE SIGNALING

The DS199_ require strict protocols to ensure data integrity. The protocol consists of four types of signaling on one line: reset sequence with reset pulse and presence pulse, write 0, write 1, and read data. The bus master initiates all these signals except presence pulse. The initialization sequence required to begin any communication with the DS199_ is shown in Figure 10. A reset pulse followed by a presence pulse indicates the DS199_ is ready to send or receive data given the correct ROM command and

memory function command. The bus master transmits (Tx) a reset pulse (tRSTL, minimum 480μs). The bus master then releases the line and goes into receive mode (Rx). The 1-Wire bus is pulled to a high state through the pullup resistor. After detecting the rising edge on the data line, the DS199_ waits (tPDH, 15μs to 60μs) and then transmits the presence pulse (tPDL, 60μs to 240μs).

Figure 9. ROM FUNCTIONS FLOW CHART

Ma s te r T X

Re s e t Pu ls e

D S 1 99X T X

Pre s e n c e P u ls e

Ma s te r T X R O M

F unctio n  C o m m and

N

33 H	N R ead R O M

C om m and

?

Y

55H

Ma tc h R O M C o mma n d

?

Y

F0 H	N Se a rc h RO M

C o mma n d

?

Y

CC H	N Sk ip RO M

C om m and

?

Y

D S 19 9X T X F a m ily C o d e

1 B y te

| M a s te r T X B it 0 D S 19 9X T X B it 0 |  |
| --- | --- |
| D S 19 9X T X B it 0 |  |
| M a s te r T X B it 0 |  |

D S 19 9X T X

S erial N u m b er

6 B y te s

Bit 0	N	N Ma tc h ?

Y

M a s te r  T X B it 1

Bit 1	N	N Ma tc h ?

Y

Bit 0

Ma tc h ?

Y

D S 19 9X T X B it 1

D S 19 9X T X B it 1

M a s te r  T X B it 1

Bit 1

Ma tc h ?

Y

D S 19 9X T X CRC By te

M a s te r  T X B it 6 3

Bit 6 3	N	N Ma tc h ?

D S 199 X  T X  B it 63

D S 199 X  T X  B it 63

M a s te r  T X B it 6 3

Bit 6 3

Ma tc h ?

Y	Y

Ma s te r T X Me m o ry

F unctio n  C o m m and

Figure 10. INITIALIZATION PROCEDURE RESET AND PRESENCE PULSE

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

MASTER TX "RESET PULSE"

tRSTL

MASTER RX "PRESENCE PULSE" tRSTH

tR	tPDH	tPDL

RESISTOR MASTER DS199X

480 µs ≤ tRSTL < ∞*

480 µs ≤ tRSTH < ∞**

15 µs ≤ tPDH < 60 µs

60 ≤ tPDL < 240 µs

* In order not to mask interrup signaling

by other devices on the 10Wire bus tRSTL

+ tR should always be less than 960 us

** Includes recovery time

READ/WRITE TIME SLOTS

The definitions of write and read time slots are illustrated in Figure 11. The master driving the data line low initiates all time slots. The falling edge of the data line synchronizes the DS199_ to the master by triggering a delay circuit in the DS199_. During write time slots, the delay circuit determines when the

DS199_ samples the data line. For a read data time slot, if a 0 is to be transmitted, the delay circuit

determines how long the DS199_ holds the data line low overriding the 1 generated by the master. If the data bit is a 1, the iButton leaves the read data time slot unchanged.

Figure 11. READ/WRITE TIMING DIAGRAM Write-One Time Slot

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

tLOW1

15µs

tSLOT	tREC

DS199X Sampling Window

RESISTOR MASTER

60µs

60 µs ≤ tSLOT < 120 µs

1 µs ≤ tLOW1 < 15 µs

1 µs ≤ tREC < ∞

Figure 11. READ/WRITE TIMING DIAGRAM (continued)

Write-Zero Time Slot

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

15µs

tSLOT

DS199X Sampling Window

60µs

t LOW0

tREC

RESISTOR

MASTER

60 µs ≤ tLOW0 < tSLOT < 120 µs

1 µs ≤ tREC < ∞

Read-Data Time Slot

VPULLUP VPULLUP MIN VIH MIN

VIL MAX

0V

tSU

tLOWR

tSLOT	tREC

Master

Sampling Window

tRELEASE

tRDV

RESISTOR

60 µs ≤ tSLOT < 120 µs

1 µs ≤ t

REC < ∞

MASTER

DS199X

1 µs ≤ tLOWR < 15 µs

0 ≤ tRELEASE < 45 µs

tRDV = 15 µs tSU < 1 µs

PHYSICAL SPECIFICATIONS

Size	See mechanical drawing

Weight	3.3 grams (F5 package) Expected Service Life	10 years at +25°C

Safety	Meets UL#913 (4th Edit.); Intrinsically Safe Apparatus,

Approved under Entity Concept for use in Class I, Division 1, Group A, B, C and D Locations

ABSOLUTE MAXIMUM RATINGS*

Voltage on any Pin Relative to Ground	-0.5V to +7.0V

Operating Temperature Range	-40°C to +70°C Storage Temperature Range	-40°C to +70°C

* This is a stress rating only and functional operation of the device at these or any other conditions above those indicated in the operation sections of this specification is not implied. Exposure to absolute maximum rating conditions for extended periods of time may affect reliability.

DC ELECTRICAL CHARACTERISTICS	(-40°C to +70°C.)

| PARAMETER | SYMBOL | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| 1-Wire Pullup Voltage (Notes 1, 3) | VPUP | 2.8 6.0 | V |
| Logic 1 (Notes 1, 2) | VIH | 2.2 | V |
| Logic 0 (Note 1) | VIL | -0.3 +0.3 | V |
| Output Logic Low at 4mA (Note 1) | VOL | 0.4 | V |
| Input Load Current (Note 4) | IL | 5 | μA |

| CAPACITANCE (TA = +25°C) PARAMETER | SYMBOL | MIN | TYP | MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| I/O (1-Wire) (Notes 5, 6) | CIN/OUT |  | 100 | 800 | pF |

| AC ELECTRICAL CHARACTERISTICS (VPUP = 2.8V to 6.0V; -40°C to +70°C.) PARAMETER | SYMBOL | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Time Slot | tSLOT | 60 120 | μs |
| Write 1 Low Time | tLOW1 | 1 15 | μs |
| Write 0 Low Time | tLOW0 | 60 120 | μs |
| Read Data Valid | tRDV | exactly 15 | μs |
| Release Time | tRELEASE | 0 15 45 | μs |
| Read Data Setup (Note 7) | tSU | 1 | μs |
| Recovery Time | tREC | 1 | μs |
| Reset Time High (Note 8) | tRSTH | 480 | μs |
| Reset Time Low (Note 9) | tRSTL | 480 960 | μs |
| Presence Detect High | tPDH | 15 60 | μs |
| Presence Detect Low | tPDL | 60 240 | μs |

Note 1: All voltages are referenced to ground.

Note 2: VIH is a function of the external pullup resistor and the VCC power supply.

Note 3: VPUP = external pullup voltage.

Note 4: Input load is to ground.

Note 5: Capacitance on the data line could be 800pF when power is first applied. If a 5kΩ resistor is used to pull up the data line to VPUP, 5μs after power has been applied, the parasite capacitance does not affect normal communications.

Note 6: Guaranteed by design, not production tested.

Note 7: Read data setup time refers to the time the host must pull the 1-Wire bus low to read a bit. Data is guaranteed to be valid within 1μs of this falling edge, and remains valid for 14μs minimum (15μs total from falling edge on 1-Wire bus).

Note 8: An additional reset or communication sequence cannot begin until the reset high time has expired.

Note 9: The reset low time (tRSTL) should be restricted to a maximum of 960μs, to allow interrupt signaling; otherwise, it could mask or conceal interrupt pulses.

REVISION HISTORY

| REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- |
| 071508 | Updated the F5 MicroCan face brand with the latest per PCN H020201. | 1 |
| 101408 | Change the last sentence of the Parasite Power section to “The advantages of parasite power are two-fold: 1) by parasiting off this input, battery power is not consumed for 1-Wire ROM function commands, and 2) if the battery is exhausted for any reason, the ROM may still be read normally. The remaining circuitry of the DS1992 and DS1993 is solely operated by battery energy.” | 2 |
|  | In the DC Electrical Characteristics section, relocated VPUP from the header to the EC table, changed VILMAX from 0.8V to 0.3V, and removed the VOH parameter for the 1-Wire pin. | 15 |

17 of 17

Maxim/Dallas Semiconductor cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim/Dallas Semiconductor product. No circuit patent licenses are implied. Maxim/Dallas Semiconductor reserves the right to change the circuitry and specifications without notice at any time.

M a x i m  In tegra t ed  P r oduc ts ,  1 2 0  S a n  Ga brie l  D r iv e ,  Sun ny vale ,  CA  94 086  40 8 - 7 3 7 - 76 00

© 2008 Maxim Integrated Products

The Maxim logo is a registered trademark of Maxim Integrated Products, Inc. The Dallas logo is a registered trademark of Dallas Semiconductor Corporation.