Rev 0; 4/06

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

General Description

The DS3908 contains two nonvolatile digital  poten- tiometers with programmable-gain  amplifiers buffering the wiper outputs. The potentiometer  position and amplifier gain are controlled through an I2C*-compati- ble serial bus. The DS3908 operates in both 3.3V and

5V systems and features a write-protect  pin that locks the position of the potentiometers and gain registers. Up to eight DS3908s can be placed on a single I2C bus.

DS3908

Features

♦ Two 64-Position Linear Taper Potentiometers

♦ Integral Wiper Buffering Amplifiers with

Selectable Gains of 1V/V, 2V/V, or 4V/V

♦ 100kΩ Potentiometer End-to-End Resistance

♦ Low Potentiometer Temperature Coefficient

♦ Nonvolatile Wiper and Gain Storage

♦ I2C-Compatible Interface

♦ Write-Protect Pin Prevents Accidental Field

Reprogramming

♦ 3V to 5.5V Supply Voltage Range

♦ -40°C to +85°C Operating Temperature Range

♦ 14-Pin TDFN Package

Applications

Pin-Diode Biasing

Power-Supply Calibration Cell Phones and PDAs Portable Electronics

Ordering Information

PART                   TEMP RANGE          PIN-PACKAGE

DS3908N+                      -40°C to +85°C          14 TDFN

+Denotes  lead-free package.

Typical Operating Circuit

Pin  Configuration

TOP VIEW

VCC

POT0                           H0

DS3908

V0

PGA0

VOLTAGE REFERENCE

NV ADJUSTABLE REFERENCE

+

SDA    1

SCL     2

14    VCC

13    H1

SDA

SCL

VOLTAGE L0

A0     3                                                              12    V1

A0                 I2C

A1          INTERFACE A2

POT1

H1

V1                  NV ADJUSTABLE

A1     4

A2     5

DS3908

11    L1

10    H0

WP                                                              PGA1

L1

REFERENCE

VOLTAGE

WP

GND

6                                                              9     V0

7                                                             8     L0

TDFN (3mm x 3mm)

I2C is a trademark of Philips Corp. Purchase of I2C components from Maxim Integrated Products, Inc., or one of its sublicensed Associated Companies, conveys a license under the Philips I2C Patent Rights to use these components in an I2C system, provided that the system conforms to the I2C Standard Specification as defined by Philips.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

ABSOLUTE MAXIMUM RATINGS

Voltage on VCC, SDA, and SCL Relative to GND .....-0.5V to +6.0V Voltage on A0, A1, A2, L0, L1, H0, H1, and WP Relative

to GND................-0.5V to (VCC + 0.5V) (not to exceed +6.0V) Operating  Temperature  Range ...........................-40°C to +85°C

Programming  Temperature  Range .........................0°C to +70°C Storage Temperature  Range .............................-55°C to +125°C Soldering Temperature ............Refer to J-STD-020 Specification

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

RECOMMENDED OPERATING CONDITIONS

(TA = -40°C to +85°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Voltage | VCC | (Note 1) | +3.0 +5.5 | V |
| Input Logic 1 (SCL, SDA, A0, A1, A2, WP) | VIH |  | 0.7 x VCC + VCC 0.3 | V |
| Input Logic 0 (SCL, SDA, A0, A1, A2, WP) | VIL |  | -0.3 0.3 x VCC | V |
| Potentiometer Voltage (L0, L1, H0, H1) |  | VCC = +3.0V to +5.5V | -0.3 VCC + 0.3V | V |

DC ELECTRICAL CHARACTERISTICS

(VCC = +3.0V to +5.5V, TA = -40°C to +85°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Input Leakage | IL |  | -1 +1 | µA |
| Standby Supply Current | ISTBY | VCC = 5.5V (Note 2) | 2 | mA |
| Low-Level Output Voltage (SDA) | VOL1 | 3mA sink current | 0 0.4 | V |
|  | VOL2 | 6mA sink current | 0 0.6 |  |
| I/O Capacitance | CI/O |  | 10 | pF |
| WP Internal Pullup Resistance | RWP |  | 40 65 100 | kΩ |

ANALOG POTENTIOMETER CHARACTERISTICS

(VCC = +3.0V to +5.5V, TA = -40°C to +85°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| End-to-End Resistance |  | +25°C | 79 100 121 | kΩ |
| Absolute Linearity | INL | (Notes 3, 4) | -0.6 +0.6 | LSB |
| Relative Linearity | DNL | (Notes 4, 5) | -0.25 +0.25 | LSB |
| End-to-End Temperature Coefficient |  |  | 50 | ppm/°C |

2

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

PROGRAMMABLE-GAIN AMPLIFIER CHARACTERISTICS

(VCC = +3.0V to +5.5V, TA = -40°C to +85°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Common-Mode Input Voltage | CMVIN |  | 0 VCC - 1.5 | V |
| Gain | G | RL ≥ 2kΩ, G = 1V/V | 0.975 1 1.025 | V/V |
|  |  | RL ≥ 2kΩ, G = 2V/V | 1.925 2 2.05 |  |
|  |  | RL ≥ 2kΩ, G = 4V/V | 3.850 4 4.10 |  |
| Output Voltage Range | VOUT | RL = 2kΩ, -1mA < IOUT < 1mA | 0.3 VCC - 0.3 | V |
| Power-Supply Rejection Ratio | PSRR |  | 60 90 | dB |
| Output Source Current | IOUT:SOURCE | VOUT = 0V, Hx = Lx = 1V | -15 | mA |
| Output Sink Current | IOUT:SINK | VOUT = 1V, Hx = Lx = 0V | 15 | mA |
| Unity-Gain Frequency | fT | Gain = 1V/V, position 3Fh | 3.5 | MHz |
| Amplifier Capacitive Loading | CL |  | 100 | pF |
| Input Offset Voltage | VOS |  | -9 +9 | mV |
| Load Regulation |  | -1mA < IOUT < 1mA | 800 2200 | µV/mA |
| Output-Voltage Slew Rate |  | RL = 10kΩ, CL = 10pF | 270 840 | V/ms |

AC ELECTRICAL CHARACTERISTICS

(VCC = +3.0V to +5.5V, TA = -40°C to +85°C.) (See Figure 2.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| SCL Clock Frequency | fSCL | (Note 6) | 400 | kHz |
| Bus Free Time between STOP and START Conditions | tBUF |  | 1.3 | µs |
| Hold Time (Repeated) START Condition | tHD:STA |  | 0.6 | µs |
| Low Period of SCL | tLOW |  | 1.3 | µs |
| High Period of SCL | tHIGH |  | 0.6 | µs |
| Data Hold Time | tHD:DAT |  | 0 0.9 | µs |
| Data Setup Time | tSU:DAT |  | 100 | ns |
| Start Setup Time | tSU:STA |  | 0.6 | µs |
| SDA and SCL Rise Time | tR | (Note 7) | 20 + 0.1CB 300 | ns |
| SDA and SCL Fall Time | tF | (Note 7) | 20 + 0.1CB 300 | ns |
| STOP Setup Time | tSU:STO |  | 0.6 | µs |
| SDA and SCL Capacitance | CB | (Note 7) | 400 | pF |
| EEPROM Write Time | tW | (Note 8) | 10 17 | ms |
| Startup Time | tST | VCC = 3.0V | 40 | µs |

3

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

NONVOLATILE MEMORY CHARACTERISTICS

(VCC = +3.0V to +5.5V.)

| PARAMETER | SYMBOL | CONDITIONS | MIN MAX | UNITS |
| --- | --- | --- | --- | --- |
| EEPROM Write Cycles |  | At +70°C | 50,000 |  |

Note 1:   All voltages are referenced to ground.

Note 2:   ISTBY specified assuming  control pins are connected as follows: WP must be disconnected or connected high. H terminal connected to VCC, L terminal connected to GND, potentiometer  position  1Dh, PGA is at 2V/V, A0 to A2 connected to VCC, SDA and SCL connected to VCC, with no load.

Note 3:   Absolute linearity is used to measure expected wiper voltage as determined by wiper position in a voltage-divider configuration.

Note 4:   This specification only refers to the potentiometers, and does not include the gain and offset error due to the PGA.

Note 5:   Relative linearity  is used  to determine  the change  of wiper  voltage  between  two adjacent  wiper  positions  in a voltage- divider configuration.

Note 6:   I2C interface  timing  shown is for fast-mode  (400kHz)  operation.  This device  is also backward-compatible with I2C stan-

dard-mode timing.

Note 7:   CB—total capacitance of one bus line in picofarads, timing referenced to 0.9 x VCC and 0.1 x VCC.

Note 8:   EEPROM write begins after a stop condition  occurs.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

2.0

1.8

1.6

1.4

ISTBY (mA)

1.2

1.0

DS3908 toc01

STANDBY SUPPLY CURRENT

vs. SUPPLY VOLTAGE

+85°C    +25°C

2.0

1.8

1.6

1.4

ISTBY (mA)

1.2

1.0

STANDBY SUPPLY CURRENT

vs. TEMPERATURE

DS3908 toc02

| 1.16400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1.16000

ISTBY (mA)

1.15800

DS3908 toc03

STANDBY SUPPLY CURRENT

vs. SCL FREQUENCY

0.8

0.6

0.4

0.2

0

Hx = VCC, Lx = GND A0 TO A2 = VCC SDA, SCL = VCC POT AT 1Dh

GAIN = 2V/V NO LOAD

-40°C

0.8

0.6

0.4

0.2

0

1.15400

1.15200

1.15000

3.0

3.5

4.0

4.5        5.0        5.5

| -40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

-15         10

DS3908 toc05

35         60         85

1                   100

10,000         1,000,000

1.10000

1.00000

VCC

DS3908 toc04

| OUTPUT VOLTAGE vs. POT SETTING GAIN = 1V/V Hx = 1V Lx = 0.3V |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

10

TEMPERATURE (°C)

VOL vs. IOUT:SINK

SCL FREQUENCY (Hz)

DS3908 toc06

VCC - VOH vs. IOUT:SOURCE

10

1.90000                                                                                        1                                                                                               1

1.80000

1.70000

1.60000

OUTPUT VOLTAGE AT Vx (mA)

1.50000

1.40000

1.30000

VOL (V)

0.1

0.01

0.001

+85°C   +25°C

-40°C

VCC - VOH (V)

0.1

0.01

0.001

+85°C   +25°C

-40°C

0        10       20      30      40      50      60

POT SETTING (DEC)

0.001     0.01        0.1         1          10        100

IOUT:SINK (mA)

0.001     0.01        0.1         1          10        100

IOUT:SOURCE (mA)

4

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

0.8

0.6

0.4

0.2

0

-0.2

-0.4

-0.6

-0.8

NORMALIZED POT END-END RESISTANCE (Ω/Ω)

-1.0

DS3908 toc07

NORMALIZED POT END-END RESISTANCE

vs. TEMPERATURE

0.20

0.15

0.10

0.05

0

POT INL (LSB)

-0.05

-0.10

-0.15

-0.20

-1.25

POT INL vs. SETTING

GAIN = 1V/V

0.20

0.15

DS3908 toc08

0.10

0.05

0

POT DNL (LSB)

-0.05

-0.10

-0.15

-0.20

-1.25

POT DNL vs. SETTING

DS3908 toc09

GAIN = 1V/V

-40

-15         10

| 35 60 85 VCC = 5V |  |  |  |  |
| --- | --- | --- | --- | --- |

0        10       20      30      40      50      60

0        10       20      30      40      50      60

TEMPERATURE (°C)

POT0 SETTING (DEC)

POT0 SETTING (DEC)

0.6

0.4

0.3

0.2

0.1

TYPICAL  PGA OFFSET

DS3908 toc10

vs. COMMON-MODE  INPUT VOLTAGE

+25°C

0.25

0.15

0.1

0.05

DS3908 toc11

TYPICAL PGA OFFSET

vs. TEMPERATURE

CMVIN = 0.3V

-0.1

TYPICAL PGA OFFSET (mV)

-0.2

-0.3

-0.4

GAIN = 1V/V

-40°C

-0.05

TYPICAL PGA OFFSET (mV)

-0.1

-0.15

CMVIN = 2.0V

CMVIN = 3.5V

-0.5

DATA OFFSET TO SHOW 0 AT 2V CMVIN

-0.2

0.3

0.7   1.1

1.5   1.9

2.3   2.7   3.1   3.5

-40

-15         10

35         60         85

COMMON-MODE INPUT VOLTAGE (V)

TEMPERATURE (°C)

Pin  Description

| TDFN PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | SDA | I2C Serial Data. Input/output for I2C data. |
| 2 | SCL | I2C Serial Clock. Input for I2C clock. |
| 3, 4, 5 | A0, A1, A2 | Address-Select Inputs. Determines I2C address. Device address is 1010A2A1A0. (See the I2C Slave Address and Address Pins section for more details.) |
| 6 | WP | Write-Protect Input. Must be grounded to write to the registers. An internal pullup will lock the register values if this pin is not connected. |
| 7 | GND | Ground Terminal |
| 8, 11 | L0, L1 | Potentiometer Low Terminals. Voltages on these pins should remain between GND and VCC. |
| 9, 12 | V0, V1 | Amplifier Outputs |
| 10, 13 | H0, H1 | Potentiometer High Terminals. Voltages on these pins should remain between GND and VCC. |
| 14 | VCC | Supply Voltage Terminal |

5

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

Functional Diagram

VCC

SDA SCL A0

A1

A2

WP GND

VCC

I2C INTERFACE

VCC

RWP

EEPROM F8h

F9h

FAh

FBh

DS3908

POT0 REGISTER

POT1 REGISTER

POT0/1  REGISTER

G1                        G0

1x, 2x, 4x GAIN

1x, 2x, 4x GAIN

POT0

POT1

H0

V0

PGA0

L0

H1

V1

PGA1

L1

Detailed Description

The DS3908 contains two nonvolatile digital  poten- tiometers with programmable-gain  amplifiers buffering the wiper outputs.

The potentiometers have 63 equally weighted (linear- taper) resistive elements, for a total of 64 taps. The resistive elements are built using a low-temperature- drift material, and have a typical  100kΩ end-to-end resistance. This produces  an output that is highly lin- ear, with the highest and lowest taps connected to high (Hx) and low (Lx) terminals, respectively.  The poten- tiometers are independently  controlled  using an I2C- compatible  interface. Three address  pins allow one of eight slave addresses  to be selected. The eight slave addresses allow the DS3908 address to be customized for applications with multiple I2C devices, and allow up to eight DS3908s to be placed  on the same I2C bus. The potentiometer positions are saved in EEPROM, and are recalled  during  each power-up  to provide  non- volatile position settings. Once the settings are written, the write-protect  pin prevents accidental  writes to the potentiometers. The write-protection function is ideal for

analog factory calibration  because  it prevents errant transactions on the I2C bus from corrupting the settings of the device. The WP pin contains an internal pullup resistor that must be pulled low to write to the device.

The programmable-gain  amplifiers can be indepen- dently set to one of three different gains—1V/V, 2V/V, or

4V/V. The amplifiers’ common-mode input range is from ground to 1.5V below VCC, and the output is rail-to-rail and capable  of driving 1mA loads, 300mV from each supply rail. The outputs are stable driving 100pF loads for applications that require output filtering.

The addition of the amplifier to buffer the potentiometer wiper offers distinct advantages  over standard digital potentiometers. The buffer provides a high-impedance load for the potentiometer and a low-impedance  volt- age output. This improves the linearity of the output voltage for systems that load the potentiometer by elim- inating the changes in current through both the poten- tiometer and the wiper impedance.  It also allows voltage gain from the potentiometer input to the output. Because the amplifiers are integrated into the DS3908, this is done without increasing  the footprint of the design or the complexity of the PC board.

6

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

I2C Slave Address and Address Pins The DS3908’s I2C slave address is determined by the state of the A0, A1, and A2 address pins as shown in the pin configuration (see Figure 1). Address pins con- nected to GND result in a ‘0’ in the corresponding  bit position in the slave address. Conversely, address pins connected to VCC result in a ‘1’ in the corresponding bit

positions. I2C communication  is described  in detail in

the I2C Serial Interface Description section.

Potentiometer Control

The potentiometers of the DS3908 have 64 taps with 63

| MSB 1 | 0 | 1 | 0 | A2 | A1 | A0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |

SLAVE ADDRESS*

DS3908

LSB

READ/WRITE BIT

resistive elements separating them. Thus, the most and least significant wiper positions connect the amplifier to the voltages at the high and low terminals of the poten- tiometer, respectively.

The potentiometers of the DS3908 are controlled  by communicating with the following registers:

*THE SLAVE ADDRESS IS DETERMINED BY ADDRESS PINS A0, A1, AND A2.

Figure 1. DS3908 Slave Address  Byte

Table 1. Potentiometer Registers

| ADDRESS | POTENTIOMETER | I2C FUNCTIONS | NUMBER OF POSITIONS* | DEFAULTS |
| --- | --- | --- | --- | --- |
| F8h | Pot 0 | Read/Write | 64 (00h to 3Fh) | 1Fh |
| F9h | Pot 1 | Read/Write | 64 (00h to 3Fh) | 1Fh |
| FAh | Pot 0 and Pot 1 | Write Only | 64 (00h to 3Fh) | — |

*The two most significant  bits  of each  potentiometer  position  register  are ignored.  Writing  values  greater  than 3Fh to any of the potentiometer  registers  will result  in a valid  6-bit  position,  without  regard  to the value of the most  significant  two bits.  Example: Register values C2h, 82h, 42h, and 02h are all potentiometer  position 2.

7

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

When writing to the DS3908, the potentiometer  will adjust to the new setting once it has acknowledged  the new data that is being written, and the EEPROM (used to make the setting nonvolatile) will be written following the stop condition at the end of the write command. To change the setting without changing  the EEPROM, ter- minate the write with a repeated start condition before the next stop condition occurs. Using a repeated start

Table 2. Programmable Amplifier Register

condition prevents the 20ms (maximum) delay required for the EEPROM write cycle to finish.

Programmable Amplifier Control The gain of both DS3908 amplifiers is controlled  by writing to register address  FBh. The most significant nibble of the FBh address controls the PGA1 gain, and the least significant nibble controls the PGA0 gain. The format of each nibble is shown in the tables below:

| ADDRESS | REGISTER FORMAT (BINARY) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PGA1 |  |  |  | PGA0 |  |  |  |
| FBh | R* | G12 | G11 | G10 | R* | G02 | G01 | G00 |
|  | bit7 bit0 |  |  |  |  |  |  |  |

Default value = 11h.

*Reserved for future use, write to zeros.

Table 3. Programmable Amplifier Gain Codes

| Gx2Gx1Gx0 | AMPLIFIER GAIN (V/V) |
| --- | --- |
| 00X | 1 |
| 01X | 2 |
| 1XX | 4 |

X = Don’t care.

Writes to this register are similar to writes to the poten- tiometer register. A stop condition must follow the write to ensure that the EEPROM is modified.  A repeated start condition before a stop condition following a write operation will prevent the settings from being stored in EEPROM. (See the I2C Communication section for more details.)

Write Protection The write-protect pin has an internal pullup resistor. To adjust the potentiometers’ position, this pin must be grounded. This pin can be left floating or connected to VCC to write protect the EEPROM memory. All registers can be read when the device is write protected.

8

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

I2C Serial Interface Description

I2C Definitions The following terminology is commonly used to describe I2C data transfers:

Master Device: The master device controls the slave devices on the bus. The master device generates SCL clock pulses, and start and stop conditions.

Slave Devices: Slave devices send and receive data at the master’s request.

Bus Idle or not Busy: Time between stop and start conditions when both SDA and SCL are inactive and in their logic-high states. When the bus is idle it often initi- ates a low-power mode for slave devices.

Start Condition: A start condition is generated by the master to initiate a new data transfer with a slave. Transitioning SDA from high to low while SCL remains high generates a start condition.  See the timing dia- gram for applicable timing.

Stop Condition: A stop condition is generated by the master to end a data transfer with a slave. Transitioning SDA from low to high while SCL remains high generates a stop condition. See the timing diagram for applicable timing.

Repeated Start Condition: The master can use a repeated start condition at the end of one data transfer to indicate that it will immediately initiate a new data trans- fer following the current one. Repeated starts are com- monly used during read operations to identify a specific memory address  to begin a data transfer. A repeated start condition  is issued identically  to a normal start condition. See the timing diagram for applicable timing.

DS3908

Bit Write: Transitions of SDA must occur during the low state of SCL. The data on SDA must remain valid and unchanged during the entire high pulse of SCL plus the setup and hold-time requirements (see Figure 2). Data is shifted into the device during the rising edge of the SCL.

Bit Read: At the end of a write operation, the master must release the SDA bus line for the proper amount of setup time (see Figure 2) before the next rising edge of SCL during a bit read. The device shifts out each bit of data on SDA at the falling edge  of the previous SCL pulse and the data bit is valid at the rising edge of the current SCL pulse. Remember that the master gener- ates all SCL clock pulses including  when it is reading bits from the slave.

Acknowledgement (ACK and NACK): An Acknowledge- ment (ACK) or Not Acknowledge  (NACK) is always the

9th bit transmitted during  a byte transfer. The device receiving data (the master during a read or the slave dur- ing a write operation) performs an ACK by transmitting a zero during the 9th bit. A device performs a NACK by transmitting a one during the 9th bit. Timing (Figure 2) for the ACK and NACK is identical to all other bit writes. An ACK is the acknowledgment that the device is prop- erly receiving data. A NACK is used to terminate a read sequence  or as an indication  that the device  is not receiving data.

Byte Write: A byte write consists of 8 bits of information transferred from the master to the slave (most signifi- cant bit first) plus a 1-bit acknowledgement  from the slave to the master. The 8 bits transmitted by the mas- ter are done according to the bit write definition and the acknowledgement is read using the bit read definition.

SDA

tBUF

tLOW

tR                           tF

tHD:STA

tSP

SCL

STOP          START

tHD:STA

tHD:DAT

tHIGH

tSU:DAT                     REPEATED START

tSU:STA

tSU:STO

NOTE: TIMING  IS REFERENCE TO VIL(MAX)   AND VIH(MIN).

Figure 2. I2C Timing Diagram

9

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

DS3908

Byte Read: A byte read is an 8-bit information transfer from the slave to the master plus a 1-bit ACK or NACK from the master to the slave. The 8 bits of information that are transferred (most significant  bit first) from the slave to the master are read by the master using the bit read definition above, and the master transmits an ACK using the bit write definition to receive additional data bytes. The master must NACK the last byte read to ter- minated communication so the slave will return control of SDA to the master.

Slave Address Byte: Each slave on the I2C bus responds to a slave address byte sent immediately fol- lowing a start condition.  The slave address  byte con- tains the slave address  in the most significant  7 bits and the R/W bit in the least significant bit.

The DS3908’s slave address is determined by the state of the A0, A1, and A2 address pins as shown in Figure 1. Address  pins connected  to GND result in a ‘0’ in the corresponding  bit  position  in the slave address. Conversely, address pins connected to VCC result in a

‘1’ in the corresponding bit positions.

When the R/W bit is 0 (such as in A0h), the master is indicating it will write data to the slave. If R/W = 1, (A1h in this case), the master is indicating  it wants to read from the slave.

If an incorrect slave address is written, the DS3908 will assume the master is communicating  with another I2C device  and ignore the communication  until the next start condition is sent.

Memory Address: During an I2C write operation to the DS3908, the master must transmit a memory address to identify the memory location where the slave is to store the data. The memory address  is always the second byte transmitted during  a write operation following the slave address byte.

I2C Communication Writing a Single Byte to a Slave: The master must generate a start condition, write the slave address byte (R/W = 0), write the memory address, write the byte of data, and generate a stop condition. The master must read the slave’s acknowledgement  during all byte write operations.

When writing to the DS3908, the potentiometer  will adjust to the new setting once it has acknowledged  the new data that is being written, and the EEPROM (used to make the setting nonvolatile) will be written following the stop condition at the end of the write command. To change the setting without changing  the EEPROM, ter- minate the write with a repeated start condition before the next stop condition occurs. Using a repeated start

condition prevents the 20ms (maximum) delay required for the EEPROM write cycle to finish.

If the master continues to write data to the DS3908, without generating a stop condition, then the same reg- ister will be overwritten.

Acknowledge Polling: Any time an EEPROM byte is written, the DS3908 requires the EEPROM write time (tW) after the stop condition to write the contents of the byte to EEPROM. During the EEPROM write time, the device will not acknowledge its slave address because it is busy. It is possible to take advantage of this phe- nomenon by repeatedly addressing the DS3908, which allows communication  to continue as soon as the DS3908 is ready.  The alternative to acknowledge polling is to wait for a maximum period of tW to elapse before attempting to access the device.

EEPROM Write Cycles: The DS3908’s EEPROM write cycles  are specified  in the Nonvolatile  Memory Characteristics table. The specification  shown is at the worst-case temperature. It is capable of handling many additional writes at room temperature.

Reading a Single Byte from a Slave: Unlike the write operation that uses the specified memory address byte to define where the data is to be written, the read oper- ation occurs  at the present  value of the memory address pointer. To read a single byte from the slave, the master generates a start condition, writes the slave address byte with R/W = 1, reads the data byte with a NACK to indicate the end of the transfer, and generates a stop condition.

Manipulating the Address Pointer for Reads: A dummy write cycle  can be used to force the address pointer to a particular value. To do this, the master gen- erates a start condition, writes the slave address  byte (R/W = 0), writes the memory address where it desires to read, generates a repeated start condition, writes the slave address byte (R/W = 1), reads data with ACK or NACK as applicable, and generates a stop condition.

See Figure 3 for a read example using the repeated start condition to specify the memory location.

Applications Information

Power-Supply Decoupling To achieve the best results when using the DS3908, decouple  the power supply  with a 0.01µF or 0.1µF capacitor.  Use a high-quality,  ceramic,  surface-mount capacitor  if possible. Surface-mount components mini- mize lead inductance,  which improves performance, and ceramic  capacitors  tend to have adequate  high- frequency response for decoupling applications.

10

Dual, 64-Position Nonvolatile Digital

Potentiometer with Buffered Outputs

Total Error The total error in a reading  from the DS3908 can be calculated using the following formula:

PotVoltage = (PotCode / 63) x (VH – VL) + VL ErrorPOT = (INLERR / 63) x (VH – VL) ErrorOFFSET = Gain x VOFF

ErrorGAIN = PotVoltage x GainERR

Total Output Error = ErrorPOT + ErrorOFFSET + ErrorGAIN

where:

PotCode = Potentiometer Setting (dec)

GainERR = Amplifier Gain Deviation from Desired (V/V)

DS3908

VOFF = PGA Input Voltage Offset Voltage (V)

INLERR = Potentiometer Integral Non-Linearity (LSB)

For example, the worst-case error for VH = 2V, VL = 0.5V, PGA Gain = 2V/V, PotCode = 31d (1Fh), is given by:

PotVoltage = 31 / 63 x (2.0V - 0.5V) + 0.5V = 1.238V ErrorPOT = (0.6 / 63) x (2.0V - 0.5V) = 0.014V ErrorOFFSET = 2.0V/V x 9mV = 0.018V

ErrorGAIN = PotVoltage x GainERR = 0.0929V

Total Output Error = ErrorPOT + ErrorOFFSET + ErrorGAIN

= 0.014V + 0.018V + 0.0929V = 0.125V

TYPICAL I2C WRITE TRANSACTION

MSB                                                                      LSB                    MSB                                                                       LSB                   MSB                                                                      LSB

| START | 1 | 0 | 1 | 0 | A2 | A1 | A0 | R/W | SLAVE ACK | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | SLAVE ACK | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 | SLAVE ACK | STOP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SLAVE ADDRESS*

READ/ WRITE

REGISTER/MEMORY ADDRESS

DATA

*THE SLAVE ADDRESS IS DETERMINED BY ADDRESS PINS A0, A1, AND A2.

EXAMPLE I2C TRANSACTIONS (WHEN A0, A1, AND A2 ARE CONNECTED TO GND)

A0h                                        F9h

A)  SINGLE BYTE NONVOLATILE WRITE

-WRITE POTENTIOMETER 1

TO 00h

START

1 0 1 0 0 0 0 0    SLAVE ACK

1 1 1 1 1 0 0 1    SLAVE ACK

0 0 0 0 0 0 0 0    SLAVE ACK

STOP

A)  SINGLE BYTE

A0h                                        F9h

SLAVE

SLAVE

SLAVE

REPEATED

VOLATILE WRITE

-WRITE POTENTIOMETER 1

TO 00h

START

1 0 1 0 0 0 0 0

ACK

1 1 1 1 1  0 0 1

ACK

00000000

ACK

START

STOP

A0h                                         F8h                                                                   A1h

DATA

B)  SINGLE BYTE READ

-READ POTENTIOMETER 0

START   1 0 1 0 0 0  0 0

SLAVE ACK

1 1 1  1 1 0 0 0

SLAVE ACK

REPEATED START

1 0 1 0  0 0 0 1

SLAVE ACK

POT 0

MASTER NACK

STOP

Figure 3. I2C Communication Examples

Chip Topology

TRANSISTOR COUNT: 9950

Package Information

For the latest package outline information, go to

www.maxim-ic.com/DallasPackInfo.

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600                                11

©  2006 Maxim Integrated  Products           Printed USA                              is a registered  trademark  of Maxim Integrated  Products, Inc. is a registered  trademark  of Dallas Semiconductor Corporation.