19-1459; Rev 2; 3/02

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

General Description

The MAX104 PECL-compatible, 1Gsps, 8-bit analog-to- digital converter (ADC) allows accurate  digitizing  of analog signals with bandwidths  to 2.2GHz. Fabricated on Maxim’s proprietary  advanced  GST-2 bipolar process, the MAX104 integrates a high-performance track/hold (T/H) amplifier and a quantizer on a single monolithic die.

The innovative design of the internal T/H, which has an exceptionally wide 2.2GHz full-power input bandwidth, results in high performance (greater than 7.5 effective bits) at the Nyquist frequency. A fully differential com- parator design and decoding  circuitry reduce out-of- sequence code errors (thermometer bubbles or sparkle codes) and provide excellent metastable performance of one error per 1016  clock cycles. Unlike other ADCs that can have errors resulting in false full- or zero-scale outputs, the MAX104 limits the error magnitude  to

1LSB.

The analog input is designed  for either differential or single-ended  use with a ±250mV input voltage range. Dual, differential, PECL-compatible output data paths ensure easy interfacing and include an 8:16 demulti- plexer feature that reduces output data rates to one-half

MAX104

Features

♦ 1Gsps Conversion Rate

♦ 2.2GHz Full-Power Analog Input Bandwidth

♦ >7.5 Effective Bits at fIN = 500MHz (Nyquist

Frequency)

♦ ±0.25LSB INL and DNL

♦ 50 Differential Analog Inputs

♦ ±250mV Input Signal Range

♦ On-Chip, +2.5V Precision Bandgap Voltage

Reference

♦ Latched, Differential PECL Digital Outputs

♦ Low Error Rate: 10-16 Metastable States at 1Gsps

♦ Selectable 8:16 Demultiplexer

♦ Internal Demux Reset Input with Reset Output

♦ 192-Contact ESBGA Package

Ordering Information

the sampling clock  rate. The PECL outputs can be

PART

TEMP RANGE          PIN-PACKAGE

operated from any supply between +3V to +5V for com- patibility with +3.3V or +5V referenced systems. Control

MAX104CHC             0°C to +70°C

192 ESBGA

inputs are provided for interleaving additional MAX104 devices to increase the effective system sampling rate.

The MAX104 is packaged in a 25mm x 25mm, 192-con- tact Enhanced Super-Ball Grid Array (ESBGA™) and is specified over the commercial (0°C to +70°C) tempera- ture range.

Applications

Digital RF/IF Signal Processing Direct RF Downconversion High-Speed Data Acquisition Digital Oscilloscopes

High-Energy Physics

Radar/Sonar/ECM Systems

ATE Systems

192-Contact  ESBGA

Ball Assignment Matrix

TOP VIEW

MAX104

Typical Operating Circuit appears at end of data sheet.

ESBGA is a trademark of Amkor/Anam.

ESBGA

Maxim Integrated Products    1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

ABSOLUTE MAXIMUM RATINGS

VCCA to GNDA .........................................................-0.3V to +6V VCCD to GNDD.........................................................-0.3V to +6V VCCI to GNDI ............................................................-0.3V to +6V VCCO to GNDD ........................................-0.3V to (VCCD + 0.3V) AUXEN1, AUXEN2 to GND .....................-0.3V to (VCCD + 0.3V) VEE to GNDI..............................................................-6V to +0.3V Between GNDs ......................................................-0.3V to +0.3V VCCA to VCCD .......................................................-0.3V to +0.3V VCCA to VCCI .........................................................-0.3V to +0.3V PECL Digital Output Current ...............................................50mA REFIN to GNDR ........................................-0.3V to (VCCI + 0.3V) REFOUT Current ................................................+100µA to -5mA ICONST, IPTAT to GNDI .......................................-0.3V to +1.0V TTL/CMOS Control Inputs (DEMUXEN,

DIVSELECT)..........................................-0.3V to (VCCD + 0.3V)

RSTIN+, RSTIN- ......................................-0.3V to (VCCO + 0.3V) VOSADJ Adjust Input ................................-0.3V to (VCCI + 0.3V) CLK+ to CLK- Voltage Difference..........................................±3V CLK+, CLK-.....................................(VEE - 0.3V) to (GNDD + 1V) CLKCOM .........................................(VEE - 0.3V) to (GNDD + 1V) VIN+ to VIN- Voltage Difference ............................................±2V VIN+, VIN- to GNDI ................................................................±2V Continuous Power Dissipation (TA = +70°C)

192-Contact ESBGA (derate 61mW/°C above +70°C) ......4.88W (with heatsink and 200 LFM airflow,

derate 106mW/°C above +70°C)  ........................................8.48W Operating Temperature Range

MAX104CHC........................................................0°C to +70°C Operating Junction Temperature.....................................+150°C Storage Temperature Range .............................-65°C to +150°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DC ELECTRICAL CHARACTERISTICS

(VCCA = VCCI = VCCD = +5.0V ±5%, VEE = -5.0V ±5%, VCCO = +3.0V to VCCD, REFIN connected  to REFOUT, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| ACCURACY |  |  |  |  |
| Resolution | RES |  | 8 | Bits |
| Integral Nonlinearity (Note 1) | INL | TA = +25°C | -0.5 ±0.25 0.5 | LSB |
| Differential Nonlinearity (Note 1) | DNL | TA = +25°C | -0.5 ±0.25 0.5 | LSB |
| Missing Codes |  | No missing codes guaranteed | None | Codes |
| ANALOG INPUTS |  |  |  |  |
| Full-Scale Input Range | VFSR | Note 1 | 475 500 525 | mVp-p |
| Common-Mode Input Range | VCM | Signal + offset w.r.t. GNDI | ±0.8 | V |
| Input Resistance | RIN | VIN+ and VIN- to GNDI, TA = +25°C | 49 50 51 | Ω |
| Input Resistance Temperature Coefficient | TCR |  | 150 | ppm/°C |
| VOS ADJUST CONTROL INPUT |  |  |  |  |
| Input Resistance (Note 2) | RVOS |  | 14 25 | kΩ |
| Input VOS Adjust Range |  | VOSADJ = 0 to 2.5V | ±4 ±5.5 | LSB |
| REFERENCE INPUT AND OUTP | UT |  |  |  |
| Reference Output Voltage | REFOUT | Driving REFIN input only | 2.475 2.50 2.525 | V |
| Reference Output Load Regulation | ∆REFOUT | 0 < ISOURCE < 2.5mA | 5 | mV |
| Reference Input Resistance | RREF | Referenced to GNDR | 4 5 | kΩ |

2

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

DC ELECTRICAL CHARACTERISTICS  (continued)

(VCCA = VCCI = VCCD = +5.0V ±5%, VEE = -5.0V ±5%, VCCO = +3.0V to VCCD, REFIN connected  to REFOUT, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| CLOCK INPUTS (Note 3) |  |  |  |  |
| Clock Input Resistance | RCLK | CLK+ and CLK- to CLKCOM, TA = +25°C | 48 50 52 | Ω |
| Input Resistance Temperature Coefficient | TCR |  | 150 | ppm/°C |
| TTL/CMOS CONTROL INPUTS ( | DEMUXEN, | DIVSELECT) |  |  |
| High-Level Input Voltage | VIH |  | 2.0 | V |
| Low-Level Input Voltage | VIL |  | 0.8 | V |
| High-Level Input Current | IIH | VIH = 2.4V | 50 | µA |
| Low-Level Input Current | IIL | VIL = 0 | -1 1 | µA |
| DEMUX RESET INPUT (Note 4) |  |  |  |  |
| Digital Input High Voltage | VIH |  | -1.165 | V |
| Digital Input Low Voltage | VIL |  | -1.475 | V |
| PECL DIGITAL OUTPUTS (Note | 5) |  |  |  |
| Digital Output High Voltage | VOH |  | -1.025 -0.880 | V |
| Digital Output Low Voltage | VOL |  | -1.810 -1.620 | V |
| POWER REQUIREMENTS |  |  |  |  |
| Positive Analog Supply Current | ICCA |  | 480 780 | mA |
| Positive Input Supply Current | ICCI |  | 108 150 | mA |
| Negative Input Supply Current | IEE |  | -290 -210 | mA |
| Digital Supply Current | ICCD |  | 205 340 | mA |
| Output Supply Current (Note 6) | ICCO |  | 75 115 | mA |
| Power Dissipation (Note 6) | PDISS |  | 5.25 | W |
| Common-Mode Rejection Ratio (Note 7) | CMRR | VIN+ = VIN- = ±0.1V | 40 68 | dB |
| Positive Power-Supply Rejection Ratio (Note 8) | PSRR+ | (Note 9) | 40 73 | dB |
| Negative Power-Supply Rejection Ratio (Note 8) | PSRR- | (Note 10) | 40 68 | dB |

3

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

AC ELECTRICAL CHARACTERISTICS

(VCCA = VCCI = VCCD = 5.0V, VEE = -5.0V, VCCO = 3.3V, REFIN connected  to REFOUT, fS = 1Gsps, fIN at -1dBFS, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| ANALOG INPUT |  |  |  |  |  |
| Analog Input Full-Power Bandwidth | BW-3dB |  |  | 2.2 | GHz |
| Analog Input VSWR | VSWR | fIN = 500MHz |  | 1.1:1 | V/V |
| Transfer Curve Offset | VOS | VOSADJ control input open |  | -2 0 +2 | LSB |
| DYNAMIC SPECIFICATIONS |  |  |  |  |  |
| Effective Number of Bits (Note 11) | ENOB1000 | fIN = 1000MHz | Differential | 7.52 | Bits |
|  |  |  | Single-ended | 7.40 |  |
|  | ENOB500 | fIN = 500MHz | Differential | 7.2 7.55 |  |
|  |  |  | Single-ended | 7.49 |  |
|  | ENOB125 | fIN = 125MHz | Differential | 7.4 7.74 |  |
|  |  |  | Single-ended | 7.73 |  |
| Signal-to-Noise Ratio (No Harmonics) | SNR1000 | fIN = 1000MHz | Differential | 46.4 | dB |
|  |  |  | Single-ended | 46.4 |  |
|  | SNR500 | fIN = 500MHz | Differential | 44.2 47.0 |  |
|  |  |  | Single-ended | 47.1 |  |
|  | SNR125 | fIN = 125MHz | Differential | 45.4 47.4 |  |
|  |  |  | Single-ended | 47.4 |  |
| Total Harmonic Distortion (Note 12) | THD1000 | fIN = 1000MHz | Differential | -52.6 | dB |
|  |  |  | Single-ended | -49.6 |  |
|  | THD500 | fIN = 500MHz | Differential | -52.5 -50 |  |
|  |  |  | Single-ended | -51.3 |  |
|  | THD125 | fIN = 125MHz | Differential | -66.2 -61 |  |
|  |  |  | Single-ended | -67.4 |  |
| Spurious-Free Dynamic Range | SFDR1000 | fIN = 1000MHz | Differential | 52.8 | dB |
|  |  |  | Single-ended | 52.5 |  |
|  | SFDR500 | fIN = 500MHz | Differential | 50 52.3 |  |
|  |  |  | Single-ended | 52.3 |  |
|  | SFDR125 | fIN = 125MHz | Differential | 62 68.9 |  |
|  |  |  | Single-ended | 69.5 |  |
| Signal-to-Noise Ratio and Distortion | SINAD1000 | fIN = 1000MHz | Differential | 46.0 | dB |
|  |  |  | Single-ended | 46.3 |  |
|  | SINAD500 | fIN = 500MHz | Differential | 44.1 46.2 |  |
|  |  |  | Single-ended | 45.9 |  |
|  | SINAD125 | fIN = 125MHz | Differential | 45.3 47.3 |  |
|  |  |  | Single-ended | 47.3 |  |
| Two-Tone Intermodulation | IMD | fIN1 = 124MHz, fIN2 = 126MHz, at -7dB below full-scale |  | -57.7 | dB |

4

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

AC ELECTRICAL CHARACTERISTICS  (continued)

(VCCA = VCCI = VCCD = +5.0V, VEE = -5.0V, VCCO = +3.3V, REFIN connected  to REFOUT, fS = 1Gsps, fIN at -1dBFS, TA = +25°C, unless otherwise noted.)

| PDP PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| TIMING CHARACTERISTICS |  |  |  |  |  |
| Maximum Sample Rate | fMAX |  |  | 1 | Gsps |
| Clock Pulse Width Low | tPWL | Figure 17 |  | 0.45 | ns |
| Clock Pulse Width High | tPWH | Figure 17 |  | 0.45 5 | ns |
| Aperture Delay | tAD | Figure 4 |  | 100 | ps |
| Aperture Jitter | tAJ | Figure 4 |  | <0.5 | ps |
| Reset Input Data Setup Time (Note 13) | tSU | Figure 15 |  | 0 | ps |
| Reset Input Data Hold Time (Note 13) | tHD | Figure 15 |  | 0 | ps |
| CLK to DREADY Propagation Delay | tPD1 | Figure 17 |  | 2.2 | ns |
| DREADY to DATA Propagation Delay (Note 14) | tPD2 | Figure 17 |  | -50 150 350 | ps |
| DATA Rise Time | tRDATA | 20% to 80%, CL = 3pF |  | 420 | ps |
| DATA Fall Time | tFDATA | 20% to 80%, CL = 3pF |  | 360 | ps |
| DREADY Rise Time | tRDREADY | 20% to 80%, CL = 3pF |  | 220 | ps |
| DREADY Fall Time | tFDREADY | 20% to 80%, CL = 3pF |  | 180 | ps |
| Primary Port Pipeline Delay | t | Figures 6, 7, 8 | DIV1, DIV2 modes | 7.5 | Clock Cycles |
|  |  |  | DIV4 mode | 7.5 |  |
| Auxiliary Port Pipeline Delay | tPDA | Figures 6, 7, 8 | DIV1, DIV2 modes | 8.5 | Clock Cycles |
|  |  |  | DIV4 mode | 9.5 |  |

Note 1:   Static linearity parameters are computed from a “best-fit”  straight line through the code transition points. The full-scale range (FSR) is defined as 256 x slope of the line.

Note 2:   The offset control input is a self-biased  voltage divider from the internal +2.5V reference voltage. The nominal open-circuit

voltage is +1.25V. It may be driven from an external potentiometer  connected  between REFOUT and GNDI.

Note 3:   The clock input’s termination voltage can be operated  between -2.0V and GNDI. Observe the absolute maximum ratings on the CLK+ and CLK- inputs.

Note 4:   Input logic levels are measured with respect to the VCCO power-supply voltage.

Note 5:   All PECL digital outputs are loaded with 50 to VCCO - 2.0V. Measurements are made with respect to the VCCO power-

supply voltage.

Note 6:   The current in the VCCO power supply does not include the current in the digital output’s emitter followers, which is a func- tion of the load resistance and the VTT termination voltage.

Note 7:   Common-Mode  Rejection Ratio is defined as the ratio of the change in the transfer-curve  offset voltage to the change in

the common-mode voltage, expressed  in dB.

Note 8:   Power-Supply Rejection Ratio is defined as the ratio of the change in the transfer-curve  offset voltage to the change in power-supply voltage, expressed  in dB.

Note 9:   Measured with the positive supplies tied to the same potential; VCCA = VCCD = VCCI. VCC varies from +4.75V to +5.25V.

Note 10: VEE varies from -5.25V to -4.75V.

5

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Note 11: Effective Number of Bits (ENOB) are computed from a curve fit referenced  to the theoretical full-scale range.

Note 12: Total Harmonic Distortion (THD) is computed from the first five harmonics.

Note 13: Guaranteed by design with a reset pulse one clock period long or greater.

Note 14: Guaranteed by design. The DREADY to DATA propagation delay is measured from the 50% point on the rising edge of the DREADY signal (when the output data changes) to the 50% point on a data output bit. This places the falling edge of the DREADY signal in the middle of the data output valid window, within the differences  between the DREADY and DATA rise and fall times, which gives maximum setup and hold time for latching external data latches.

Typical Operating Characteristics

(VCCA = VCCI = VCCD = +5V, VEE = -5V, VCCO = +3.3V, REFIN connected  to REFOUT, fS = 1Gsps, TA = +25°C, unless otherwise noted.)

EFFECTIVE NUMBER OF BITS vs. ANALOG INPUT FREQUENCY (SINGLE-ENDED ANALOG INPUT DRIVE)

EFFECTIVE NUMBER OF BITS vs. ANALOG INPUT FREQUENCY (DIFFERENTIAL ANALOG INPUT DRIVE)

SIGNAL-TO-NOISE + DISTORTION

vs. ANALOG INPUT FREQUENCY

7.50

ENOB (Bits)

7.25

MAX104toc01

-12dB FS

-6dB FS

-1dB FS

ENOB (Bits)

7.50

7.25

MAX104toc02

-12dB FS

-6dB FS

-1dB FS

50

-1dB FS

-6dB FS

SINAD (dB)

40

MAX104 toc03

SINGLE-ENDED ANALOG INPUT DRIVE

7.00

6.75

7.00

6.75

-12dB FS

35

6.50

10                               100                              1000 1250

6.50

30

10                               100                              1000  1250                      10

100

1000

10,000

ANALOG INPUT FREQUENCY (MHz)

ANALOG INPUT FREQUENCY (MHz)

ANALOG INPUT FREQUENCY (MHz)

SIGNAL-TO-NOISE + DISTORTION

vs. ANALOG INPUT FREQUENCY

55

MAX104 toc04

DIFFERENTIAL  ANALOG INPUT DRIVE

50

-1dB FS

SINAD (dB)

45

-6dB FS

40

-12dB FS

35

SIGNAL-TO-NOISE RATIO

MAX104toc05

vs. ANALOG INPUT FREQUENCY (SINGLE-ENDED ANALOG INPUT DRIVE)

50

-1dB FS

46

-6dB FS

SNR (dB)

42

38                                        -12dB FS

34

SIGNAL-TO-NOISE RATIO

vs. ANALOG INPUT FREQUENCY (DIFFERENTIAL ANALOG INPUT DRIVE)

50

MAX104toc06

-1dB FS

46

-6dB FS

SNR (dB)

42

38                                        -12dB FS

34

30

10                      100

1000

10,000

30

10                               100                              1000  1250

30

10                               100                             1000  1250

ANALOG INPUT FREQUENCY (MHz)

ANALOG INPUT FREQUENCY (MHz)

ANALOG INPUT FREQUENCY (MHz)

6

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Typical Operating Characteristics (continued)

(VCCA = VCCI = VCCD = +5V, VEE = -5V, VCCO = +3.3V, REFIN connected  to REFOUT, fS = 1Gsps, TA = +25°C, unless otherwise noted.)

SPURIOUS-FREE DYNAMIC  RANGE vs. ANALOG INPUT FREQUENCY (SINGLE-ENDED ANALOG INPUT DRIVE)

80

-1dB FS

70

-6dB FS

SFDR (dB)

60      -12dB FS

50

40

SPURIOUS-FREE DYNAMIC  RANGE vs. ANALOG INPUT FREQUENCY (DIFFERENTIAL ANALOG INPUT DRIVE)

MAX104toc09

80

MAX104toc10

-1dB FS

70

-6dB FS

SFDR (dB)

60     -12dB FS

50

40

8.00

7.75

7.50

ENOB (Bits)

7.25

7.00

6.75

EFFECTIVE NUMBER OF BITS

MAX104toc11

vs. CLOCK FREQUENCY (fIN = 125MHz, -1dB FS)

30

10                               100                             1000

ANALOG INPUT FREQUENCY (MHz)

1250

30

| 10 100 1000 1250 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ANALOG INPUT FREQUENCY (MHz)

6.50

| 100 1000 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

CLOCK FREQUENCY (MHz)

8.00

7.75

EFFECTIVE NUMBER OF BITS

vs. CLOCK POWER (fIN = 125MHz, -1dB FS)

MAX104toc12

DIFFERENTIAL CLOCK DRIVE

8.00

7.75

EFFECTIVE NUMBER OF BITS

MAX104-13

vs. VCCI = VCCA = VCCD (fIN = 125MHz, -1dB FS)

7.50

SINGLE-ENDED CLOCK DRIVE

ENOB (Bits)

7.50

7.25

ENOB (Bits)

7.00

6.75

7.25

7.00

6.75

6.50

-12 -10 -8   -6

-4  -2   0

2     4     6

8    10

6.50

4.50

4.70

4.90

5.10         5.30         5.50

8.00

7.75

ENOB (Bits)

7.50

7.25

7.00

6.75

6.50

CLOCK POWER (dBm)  PER SIDE

MAX104-14

| EFFECTIVE NUMBER OF BITS vs. VEE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(fIN = 125MHz, -1dB FS)

VCC (V)

SPURIOUS-FREE DYNAMIC  RANGE

MAX104toc15

vs. CLOCK POWER (fIN = 125MHz, -1dB FS)

75

73

71

69

SFDR (dB)

67

65

63

61

59

| 57 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SINGLE-ENDED CLOCK DRIVE |  |  |  |  |  |  |  |  |
|  | DIFFERENTIAL CLOCK DRIVE |  |  |  |  |  |  |  |  |  |

55

-5.50

-5.30

-5.10

-4.90     -4.70     -4.50

-12 -10 -8  -6  -4   -2    0     2    4

6    8    10

VEE (V)

CLOCK POWER (dBm)  PER SIDE

7

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Typical Operating Characteristics (continued)

(VCCA = VCCI = VCCD = +5V, VEE = -5V, VCCO = +3.3V, REFIN connected  to REFOUT, fS = 1Gsps, TA = +25°C, unless otherwise noted.)

SPURIOUS-FREE DYNAMIC  RANGE

MAX104-16

vs. VCCI = VCCA = VCCD (fIN = 125MHz, -1dB FS)

| 75 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

74

MAX104-17

SPURIOUS-FREE DYNAMIC  RANGE vs. VEE

(fIN = 125MHz, -1dB FS)

75

74

FFT PLOT

(fIN = 125.8545MHz, RECORD LENGTH 8192)

0

MAX104toc18

ENOB = 7.75 BITS SNR = 47.4dB

73

72

SFDR (dB)

71

70

69

68

67

66

65

4.50         4.70         4.90         5.10         5.30         5.50

73

72

SFDR (dB)

71

70

69

68

| 67 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

66

65

-5.50     -5.30     -5.10     -4.90     -4.70     -4.50

-25.6

-51.2

AMPLITUDE (dB)

-76.8

-102.4

-128.0

0

100

200

THD = -66.2dB SFDR = 70.3dB

300          400          500

VCC (V)

FFT PLOT

(fIN = 494.5068MHz, RECORD LENGTH 8192)

0

MAX104toc19

ENOB = 7.51 BITS SNR = 46.8dB

VEE (V)

FFT PLOT

(fIN = 1005.0049MHz, RECORD LENGTH 8192)

0

MAX104toc20

ENOB = 7.51 BITS SNR = 46.3dB

ANALOG INPUT FREQUENCY (MHz)

MAX104toc21

TWO-TONE INTERMODULATION

FFT PLOT (fIN1  = 124MHz,  fIN2 = 126MHz,

7dB  BELOW FULL-SCALE,  RECORD LENGTH 8192)

0

-25.6

-51.2

AMPLITUDE (dB)

-76.8

THD = -51.9dB

SFDR = 52.1dB

-25.6

-51.2

AMPLITUDE (dB)

-76.8

THD = -52.8dB SFDR = 53.7dB

-25.6

-51.2

AMPLITUDE (dB)

-76.8

fIN1

fIN2

-102.4

-102.4

-102.4

-128.0

0

100

200

300          400          500

-128.0

0

100

200

300          400          500

-128.0

0

100

200

300          400          500

ANALOG INPUT FREQUENCY (MHz)

ANALOG INPUT FREQUENCY (MHz)

ANALOG INPUT FREQUENCY (MHz)

| -5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| SMALL-SIGNAL BANDWIDTH = 2.4GHz |  |  |  |  |

-6

-7

AMPLITUDE (dB)

-8

-9

-10

ANALOG INPUT BANDWIDTH

MAX104toc22

-6dB BELOW FULL-SCALE

500               1500    2500

ANALOG INPUT FREQUENCY (MHz)

MAX104toc23

ANALOG INPUT BANDWIDTH FULL-POWER

| 0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| FULL-POWER BANDWIDTH = 2.2GHz |  |  |  |  |

-1

-2

AMPLITUDE (dB)

-3

-4

-5

500               1500    2500

ANALOG INPUT FREQUENCY (MHz)

0.5

0.4

0.3

0.2

INL (LSB)

0.1

0

-0.1

-0.2

-0.3

-0.4

-0.5

INTEGRAL NONLINEARITY

vs. OUTPUT CODE

MAX104toc24

(LOW-FREQUENCY  SERVO LOOP DATA)

0       32     64     96     128   160    192   224    256

OUTPUT CODE

8

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Typical Operating Characteristics (continued)

(VCCA = VCCI = VCCD = +5V, VEE = -5V, VCCO = +3.3V, REFIN connected  to REFOUT, fS = 1Gsps, TA = +25°C, unless otherwise noted.)

0.5

0.4

0.3

0.2

DNL (LSB)

0.1

0

-0.1

-0.2

-0.3

-0.4

-0.5

DIFFERENTIAL NONLINEARITY

vs. OUTPUT CODE

MAX104toc25

(LOW-FREQUENCY  SERVO LOOP DATA)

DREADY (200mV/div)

DATA (200mV/div)

MAX104toc26

DREADY RISE/FALL TIME, DATA OUTPUT RISE/FALL TIME

1.5

1.4

VSWR

1.3

1.2

1.1

1.0

MAX104toc27

| VSWR vs. ANALOG INPUT FREQUENCY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

0       32     64     96     128   160    192   224    256

500ps/div

0            500

1000

1500        2000        2500

-60

-61

-62

-63

THD (dB)

-64

-65

-66

-67

-68

-69

-70

OUTPUT CODE

MAX104-28

TOTAL HARMONIC DISTORTION vs. VEE

(fIN = 125MHz, -1dB FS)

-5.50     -5.30     -5.10     -4.90     -4.70     -4.50

VEE (V)

-60

-61

-62

-63

THD (dB)

-64

-65

-66

-67

-68

-69

-70

ANALOG INPUT FREQUENCY (MHz)

TOTAL HARMONIC DISTORTION

MAX104-29

vs. VCCI = VCCA = VCCD (fIN = 125MHz, -1dB FS)

4.50         4.70         4.90         5.10         5.30         5.50

VCC (V)

Pin  Description

| CONTACT | NAME | FUNCTION |
| --- | --- | --- |
| A1–A4, A6, A7, B1, B2, C1, C2, D1, D2, D3, G1, H1, J2, J3, K1–K3, L2, L3, M1, N1, T2, T3, U1, V1, V2, W1–W4 | GNDI | Analog Ground. For T/H amplifier, clock distribution, bandgap reference, and reference amplifier. |
| A5, B5, C5, H2, H3, M2, M3, U5, V5, W5 | VCCI | Analog Supply Voltage, +5V. Supplies T/H amplifier, clock distri- bution, bandgap reference, and reference amplifier. |
| A8, B8, C8, U6, V6, W6 | GNDA | Analog Ground. For comparator array. |
| A9, B9, C9, U7, V7, W7 | VCCA | Analog Supply Voltage, +5V. Supplies analog comparator array. |
| A10, E17, F2, P3, R17, R18 | TESTPOINT (T.P.) | Test Point. Do not connect. |
| A11, B11, B16, B17, C11, C16, U9, U17, V9, V17, V18, W9 | GNDD | Digital Ground |
| A12–A19, B19, C19, D19, E19, F19, G19, H19, J19, K19, L19, M19, N19, P19, T19, U19, V19, W10-W19 | VCCO | PECL Supply Voltage, +3V to +5V |

9

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Pin  Description (continued)

| CONTACT | NAME | FUNCTION |
| --- | --- | --- |
| B3, B4, C3, C4, E3, F3, G2, G3, N2, N3, U2, U3, U4, V3, V4 | VEE | Analog Supply Voltage, -5V. Supplies T/H amplifier, clock distribu- tion, bandgap reference, and reference amplifier. |
| B6, B7 | GNDR | Reference Ground. Must be connected to GNDI. |
| B10, B18, C10, C17, C18, T17, T18, U8, U18, V8, W8 | VCCD | Digital Supply Voltage, +5V |
| B12 | P0+ | Primary Output Data Bit 0 (LSB) |
| B13 | A0+ | Auxiliary Output Data Bit 0 (LSB) |
| B14 | P1+ | Primary Output Data Bit 1 |
| B15 | A1+ | Auxiliary Output Data Bit 1 |
| C6 | REFIN | Reference Input |
| C7 | REFOUT | Reference Output |
| C12 | P0- | Complementary Primary Output Data Bit 0 (LSB) |
| C13 | A0- | Complementary Auxiliary Output Data Bit 0 (LSB) |
| C14 | P1- | Complementary Primary Output Data Bit 1 |
| C15 | A1- | Complementary Auxiliary Output Data Bit 1 |
| D17 | DIVSELECT | TTL/CMOS Demux Divide Selection Input 1: Decimation DIV4 mode 0: Demultiplexed DIV2 mode |
| D18 | AUXEN2 | Tie to VCCO to power the auxiliary port. Tie to GNDD to power down. |
| E1 | ICONST | Die Temperature Measurement Test Point. See Die Temperature Measurement section. |
| E2 | IPTAT | Die Temperature Measurement Test Point. See Die Temperature Measurement section. |
| E18 | DEMUXEN | TTL/CMOS Demux Enable Control 1: Enable Demux 0: Disable Demux |
| F1 | VOSADJ | Offset Adjust Input |
| F17 | P2- | Complementary Primary Output Data Bit 2 |
| F18 | P2+ | Primary Output Data Bit 2 |
| G17 | A2- | Complementary Auxiliary Output Data Bit 2 |
| G18 | A2+ | Auxiliary Output Data Bit 2 |
| H17 | P3- | Complementary Primary Output Data Bit 3 |
| H18 | P3+ | Primary Output Data Bit 3 |

10

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Pin  Description (continued)

| CONTACT | NAME | FUNCTION |
| --- | --- | --- |
| J1 | VIN- | Differential Input Voltage (-) |
| J17 | A3- | Complementary Auxiliary Output Data Bit 3 |
| J18 | A3+ | Auxiliary Output Data Bit 3 |
| K17 | DREADY- | Complementary Data-Ready Clock |
| K18 | DREADY+ | Data-Ready Clock |
| L1 | VIN+ | Differential Input Voltage (+) |
| L17 | P4- | Complementary Primary Output Data Bit 4 |
| L18 | P4+ | Primary Output Data Bit 4 |
| M17 | A4- | Complementary Auxiliary Output Data Bit 4 |
| M18 | A4+ | Auxiliary Output Data Bit 4 |
| N17 | P5- | Complementary Primary Output Data Bit 5 |
| N18 | P5+ | Primary Output Data Bit 5 |
| P1 | CLK- | Complementary Sampling-Clock Input |
| P2 | TESTPOINT (T.P.) | This contact must be connected to GNDI. |
| P17 | A5- | Complementary Auxiliary Output Data Bit 5 |
| P18 | A5+ | Auxiliary Output Data Bit 5 |
| R1, R2, R3 | CLKCOM | 50Ω Clock Termination Return |
| R19 | AUXEN1 | Tie to VCCO to power the auxiliary port. Tie to GNDD to power down. |
| T1 | CLK+ | Sampling Clock Input |
| U10 | RSTIN- | Complementary PECL Demux Reset Input |
| U11 | RSTOUT- | Complementary PECL Reset Output |
| U12 | OR- | Complementary PECL Overrange Bit |
| U13 | A7- | Complementary Auxiliary Output Data Bit 7 (MSB) |
| U14 | P7- | Complementary Primary Output Data Bit 7 (MSB) |
| U15 | A6- | Complementary Auxiliary Output Data Bit 6 |
| U16 | P6- | Complementary Primary Output Data Bit 6 |
| V10 | RSTIN+ | PECL Demux Reset Input |
| V11 | RSTOUT+ | PECL Reset Output |
| V12 | OR+ | PECL Overrange Bit |
| V13 | A7+ | Auxiliary Output Data Bit 7 (MSB) |
| V14 | P7+ | Primary Output Data Bit 7 (MSB) |
| V15 | A6+ | Auxiliary Output Data Bit 6 |
| V16 | P6+ | Primary Output Data Bit 6 |

11

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

BANDGAP REFERENCE

REF OUT

+2.5V

REF IN

REFERENCE AMPLIFIER

MAX104

VOSADJ

GNDR

MAX104

DIFFERENTIAL PECL OUTPUTS

50

GNDI

BIAS CURRENTS                                                                                                                                                 OVERRANGE

OR

BIT                  2

VIN+

VIN-

50

T/H AMPLIFIER

8-BIT                                  2

FLASH ADC

16

AUXILIARY

DATA PORT            16

PRIMARY

DATA PORT            16

A0–A7

P0–P7

CLK+ CLKCOM CLK-

RSTIN+ RSTIN-

GNDI

50

50

T/H CLOCK DRIVER

RESET INPUT DUAL LATCH

ADC CLOCK DRIVER

LOGIC CLOCK DRIVER

RESET PIPELINE

DELAYED RESET

DEMUX CLOCK GENERATOR

DEMUX CLOCK DRIVER

READY CLOCK          2

DEMUX

RESET OUTPUT          2

RSTOUT

DEMUXEN  DIVSELECT

Figure 1. Simplified Functional Diagram

Detailed Description

The MAX104 is an 8-bit, 1Gsps flash analog-to-digital converter (ADC) with on-chip track/hold (T/H) amplifier and differential PECL-compatible outputs. The ADC (Figure 1) employs a fully differential 8-bit quantizer and a unique encoding  scheme to limit metastable states to typically one error per 1016 clock cycles, with no error exceeding 1LSB max.

An integrated 8:16 output demultiplexer simplifies inter- facing to the part by reducing  the output data rate to one-half the sampling clock  rate. This demultiplexer has  internal  reset  capability  that  allows  multiple

MAX104s to be time-interleaved to achieve higher effective sampling rates.

When clocked  at 1Gsps, the MAX104 provides a typi- cal effective number of bits (ENOB) of >7.5 bits at an analog input frequency of 500MHz. The analog input of the MAX104 is designed for differential or single-ended use with a ±250mV full-scale input range. In addition, this ADC features an on-chip +2.5V precision bandgap reference. If desired, an external reference can also be used.

12

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

DIGITAL OUTPUT

Principle of Operation The MAX104’s flash or parallel architecture provides the fastest multibit conversion of all common integrated ADC designs. The key to this high-speed  flash archi- tecture is the use of an innovative, high-performance comparator  design. The flash converter and down- stream logic translate the comparator outputs into a parallel 8-bit output code and pass this binary code on to the optional 8:16 demultiplexer, where primary and auxiliary ports output PECL-compatible data at up to

500Msps per port (depending  on how the demultiplex- er section is set on the MAX104).

The ideal transfer function appears in Figure 2.

On-Chip Track/Hold Amplifier As with all ADCs, if the input waveform is changing rapidly during  conversion, effective number of bits (ENOB) and signal-to-noise ratio (SNR) specifications will degrade.  The MAX104’s on-chip, wide-bandwidth (2.2GHz) T/H amplifier reduces this effect and increases the ENOB performance significantly, allowing precise

OVERRANGE + 255

255

254

129

128

127

126

3

2

1

(-FS + 1LSB)

0

Figure 2. Transfer Function

ALL INPUTS ARE ESD PROTECTED (NOT SHOWN IN THIS

(+FS - 1LSB)

+FS

0

MAX104

ANALOG INPUT

capture of fast analog data at high conversion rates.

The T/H amplifier buffers the input signal and allows a full-scale signal input range of ±250mV. The T/H ampli- fier’s differential 50 input termination simplifies inter- facing to the MAX104 with controlled impedance lines. Figure 3 shows a simplified diagram of the T/H amplifier stage internal to the MAX104.

Aperture width, delay, and jitter (or uncertainty) are parameters that affect the dynamic  performance  of

SIMPLIFIED DRAWING).

VIN+ VIN-

50

GNDI CLK+

CLK-

50

INPUT

AMPLIFIER

CLOCK SPLITTER

SAMPLING

BRIDGE

BUFFER

AMPLIFIER

CHOLD

GNDI

TO

TO COMPARATORS

high-speed  converters. Aperture jitter, in particular, directly influences SNR and limits the maximum slew rate (dV/dt) that can be digitized  without contributing significant errors. The MAX104’s innovative T/H amplifier

CLKCOM

50

50

COMPARATORS

design limits aperture jitter typically to less than 0.5ps.

Aperture Width Aperture width (tAW) is the time the T/H circuit requires (Figure 4) to disconnect  the hold capacitor  from the input circuit  (for instance, to turn off the sampling bridge and put the T/H unit in hold mode).

Aperture Jitter Aperture jitter (tAJ) is the sample-to-sample variation (Figure 4) in the time between the samples.

Aperture Delay Aperture delay (tAD) is the time defined between the rising edge of the sampling clock and the instant when

Figure 3. Internal Structure of the 2.2GHz T/H Amplifier

CLK CLK

tAW

ANALOG INPUT

tAD

tAJ

SAMPLED DATA (T/H)

an actual sample is taken (Figure 4).

Internal Reference

The MAX104 features an on-chip  +2.5V precision

T/H

TRACK

HOLD

APERTURE DELAY (tAD) APERTURE WIDTH (tAW) APERTURE JITTER (tAJ)

TRACK

bandgap reference, which can be used by connecting

Figure 4. T/H Aperture Timing

13

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

REFOUT to REFIN. This connects the reference output to the positive input of the reference buffer. The buffer’s negative input is internally connected to GNDR. GNDR must be connected  to GNDI on the user’s application board.  REFOUT can source up to 2.5mA to supply external devices if required.

An adjustable external reference can be used to adjust the ADC’s full-scale range. To use an external refer- ence supply, simply connect a high-precision reference to the REFIN pin and leave the REFOUT pin floating. In this configuration, REFOUT must not be simultaneously connected  to avoid conflicts between the two refer- ences. REFIN has a typical input resistance of 5k and accepts  input voltages of +2.5V ±200mV. Using the MAX104’s internal reference is recommended for best performance.

Digital Outputs The MAX104 provides data in offset binary format to dif- ferential PECL outputs. A simplified circuit schematic of the PECL output cell is shown in Figure 5. All PECL out- puts are powered from VCCO, which may be operated

500                  500

DIFF. PAIR

1.8mA

GNDD                                                             GNDD

Figure 5. Simplified PECL Output Structure

GNDD

VCCO

A_+/P_+

A_-/P_-

from any voltage between +3.0V to VCCD for flexible interfacing with either +3.3V or +5V systems. The nomi- nal VCCO supply voltage is +3.3V.

All PECL outputs on the MAX104 are open-emitter types and must be terminated at the far end of each transmission line with 50 to VCCO - 2V. Table 1 lists all MAX104 PECL outputs and their functions.

Table 1. PECL Output Functions

Demultiplexer Operation The MAX104 features an internal data demultiplexer that provides for three different modes of operation (see the following sections on Demultiplexed DIV2 Mode, Non-Demultiplexed DIV1 Mode, and Decimation DIV4

Mode) controlled by two TTL/CMOS-compatible inputs: DEMUXEN and DIVSELECT.

DEMUXEN enables or disables operation of the internal

1:2 demultiplexer. A logic high on DEMUXEN activates the internal demultiplexer, and a logic low deactivates it. With the internal demultiplexer enabled, DIVSELECT controls the selection of the operational mode. DIVSE- LECT low selects demultiplexed DIV2 mode, and DIV- SELECT high selects decimation DIV4 mode (Table 2).

| PECL OUTPUT SIGNALS | FUNCTIONAL DESCRIPTION |
| --- | --- |
| P0+, P0-, P1+, P1-, P2+, P2-, P3+, P3-, P4+, P4-, P5+, P5-, P6+, P6-, P7+, P7- | Primary-Port Differential Outputs from LSB to MSB. A “+” indicates the true outputs; a “-” denotes the complementary outputs. |
| A0+, A0-, A1+, A1-, A2+, A2-, A3+, A3-, A4+, A4-, A5+, A5-, A6+, A6-, A7+, A7- | Auxiliary-Port Differential Outputs from LSB to MSB. A “+” indicates the true outputs; a “-” denotes the complementary outputs. |
| DREADY+, DREADY- | Data-Ready Clock True and Complementary Outputs. These signal lines are used to latch the output data from the primary to the auxiliary output ports. Data changes on the rising edge of the DREADY clock. |
| OR+, OR- | Overrange True and Complementary Outputs |
| RSTOUT+, RSTOUT- | Reset Output True and Complementary Outputs |

14

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

Non-Demultiplexed DIV1 Mode The MAX104 may be operated at up to 500Msps in non- demultiplexed DIV1 mode (Table 2). In this mode, the internal demultiplexer is disabled, and sampled data is presented to the primary port only, with the data repeat- ed at the auxiliary port but delayed by one clock cycle (Figure 6). Since the auxiliary output port contains the same data stream as the primary output port, the auxil- iary port can be shut down to save power by connecting AUXEN1 and AUXEN2 to digital ground (GNDD). This powers down the internal bias cells and causes both outputs (true and complementary) of the auxiliary port to pull up to a logic-high  level. To save additional power, the external 50 termination resistors connected to the

MAX104

PECL termination power supply (VCCO - 2V) may be removed from all auxiliary output ports.

Demultiplexed DIV2 Mode The MAX104 features an internally selectable DIV2 mode (Table 2) that reduces the output data rate to one-half of the sample clock rate. The demultiplexed outputs are presented in dual 8-bit format with two con- secutive samples appearing  in the primary and auxil- iary output ports on the rising edge of the data-ready clock (Figure 7). The auxiliary data port contains the previous sample, and the primary output contains the most recent data sample. AUXEN1 and AUXEN2 must be connected  to VCCO to power-up the auxiliary port PECL output drivers.

ADC SAMPLE NUMBER                           ADC SAMPLES  ON THE RISING EDGE OF CLK+

CLK- CLK

CLK+ DREADY+

DREADY

DREADY-

AUXILIARY DATA PORT

n              n+1             n+2             n+3           n+4             n+5

n+6             n+7            n+8             n+9           n+10          n+11           n+12          n+13

n              n+1             n+2             n+3           n+4

PRIMARY DATA PORT

n+1             n+2             n+3           n+4

n+5

NOTE:   THE AUXILIARY PORT DATA IS DELAYED ONE ADDITIONAL CLOCK CYCLE FROM THE PRIMARY PORT DATA.

GROUNDING AUXEN1 AND AUXEN2 WILL POWER DOWN THE AUXILIARY PORT TO SAVE POWER.

Figure 6. Non-Demuxed,  DIV1-Mode Timing Diagram

ADC SAMPLE NUMBER                           ADC SAMPLES  ON THE RISING EDGE OF CLK+

CLK- CLK

CLK+ DREADY+

DREADY

DREADY-

n              n+1             n+2             n+3           n+4             n+5

n+6             n+7            n+8             n+9           n+10          n+11           n+12          n+13

AUXILIARY DATA PORT

n-1

n+1

n+3

PRIMARY DATA PORT

n                                  n+2                               n+4

NOTE:  THE LATENCY TO THE PRIMARY PORT IS 7.5 CLOCK CYCLES, AND THE LATENCY TO THE AUXILIARY PORT IS 8.5 CLOCK CYCLES.

BOTH THE PRIMARY  AND AUXILIARY  DATA PORTS ARE UPDATED ON THE RISING EDGE OF THE DREADY+ CLOCK.

Figure 7. Demuxed DIV2-Mode Timing Diagram

15

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Decimation DIV4 Mode The MAX104 also offers a special decimated, demulti- plexed output (Figure 8) that discards every other input sample and outputs data at one-quarter the input sam- pling rate for system debugging  at slower output data rates. With an input clock of 1GHz, the effective output data rate will be reduced to 250MHz per output port in the DIV4 mode (Table 2). Since every other sample is discarded, the effective sampling rate is 500Msps.

Overrange Operation A single differential PECL overrange output bit (OR+, OR-) is provided for both primary and auxiliary demulti- plexed outputs. The operation of the overrange bit depends on the status of the internal demultiplexer. In demultiplexed DIV2 mode and decimation DIV4 mode,

the OR bit will flag an overrange condition if either the primary or auxiliary port contains an overranged sam- ple (Table 2). In non-demultiplexed DIV1 mode, the OR port will flag an overrange condition only when the pri- mary output port contains an overranged sample.

Applications Information

Single-Ended Analog Inputs The MAX104 T/H amplifier is designed  to work at full speed for both single-ended  and differential analog inputs (Figure 9). Inputs VIN+ and VIN- feature on-chip, laser-trimmed 50 termination resistors to provide excellent voltage standing wave ratio (VSWR) perfor- mance.

ADC SAMPLE NUMBER                           ADC SAMPLES  ON THE RISING EDGE OF CLK+

CLK- CLK

CLK+ DREADY+

DREADY

DREADY-

AUXILIARY DATA PORT

PRIMARY DATA PORT

n              n+1             n+2             n+3           n+4             n+5

n+6             n+7            n+8             n+9           n+10          n+11           n+12          n+13

n-2                                            n+2 n                                                           n+4

NOTE:   THE LATENCY TO THE PRIMARY PORT REMAINS 7.5 CLOCK CYCLES, WHILE THE LATENCY OF THE AUXILIARY PORT INCREASES TO 9.5 CLOCK CYCLES.

THIS EFFECTIVELY DISCARDS  EVERY OTHER SAMPLE  AND REDUCES THE OUTPUT DATA RATE TO 1/4  THE SAMPLE CLOCK RATE.

Figure 8. Decimation DIV4-Mode Timing Diagram

Table 2. Demultiplexer Operation

| DEMUXEN | DIVSELECT | DEMUX MODE | OVERRANGE BIT OPERATION |
| --- | --- | --- | --- |
| Low | X | DIV1 500Msps (max) | Flags overrange data appearing in primary port only. |
| High | Low | DIV2 500Msps/port | Flags overrange data appearing in either the primary or auxiliary port. |
| High | High | DIV4 250Msps/port |  |

X = Don’t care

16

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

ANALOG INPUTS ARE ESD PROTECTED

(NOT SHOWN IN THIS SIMPLIFIED DRAWING).

VIN+

50

GNDI

+2.8V

+250mV

500mVp-p FS ANALOG INPUT RANGE

500mV

VIN+

MAX104

OV VIN-

50

-250mV                                                                                               t

VIN = ±250mV

VIN-

Figure 10a. Single-Ended  Analog Input Signals

VEE

Figure 9. Simplified Analog Input Structure (Single-Ended/ Differential)

In a typical  single-ended  configuration,  the analog input signal (Figure 10a) enters the T/H amplifier stage at the in-phase input (VIN+), while the inverted phase input (VIN-) is reverse-terminated to GNDI with an

+125mV

±250mV FS ANALOG INPUT RANGE

VIN+

250mV                         -250mV

VIN-

OV

external 50 resistor. Single-ended operation allows for an input amplitude of ±250mV. Table 3 shows a selec- tion of input voltages and their corresponding  output codes for single-ended operation.

Differential Analog Inputs To obtain a full-scale digital output with differential input drive (Figure 10b), 250mVp-p must be applied between VIN+ and VIN- (VIN+ = +125mV and VIN- = -125mV). Midscale digital output codes (01111111 or 10000000) occur when there is no voltage difference between VIN+ and VIN-. For a zero-scale digital output code, the

-125mV                                                                                              t

Figure 10b. Differential Analog Input Signals

in-phase (VIN+) input must see -125mV and the invert- ed input (VIN-) must see +125mV. A differential input drive is recommended  for best performance. Table 4 represents a selection of differential input voltages and their corresponding output codes.

Table 3. Ideal Input Voltage and Output Code Results for Single-Ended Operation

| VIN+ | VIN- | OVERRANGE BIT | OUTPUT CODE |
| --- | --- | --- | --- |
| +250mV | 0V | 1 | 11111111 (full scale) |
| +250mV - 1LSB | 0V | 0 | 11111111 |
| 0V | 0V | 0 | 01111111 toggles 10000000 |
| -250mV + 1LSB | 0V | 0 | 0000001 |
| -250mV | 0V | 0 | 00000000 (zero scale) |

17

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Table 4. Ideal Input Voltage and Output Code Results for Differential Operation

| VIN+ | VIN- | OVERRANGE BIT | OUTPUT CODE |
| --- | --- | --- | --- |
| +125mV | -125mV | 1 | 11111111 (full scale) |
| +125mV - 0.5LSB | -125mV + 0.5LSB | 0 | 11111111 |
| 0V | 0V | 0 | 01111111 toggles 10000000 |
| -125mV + 0.5LSB | +125mV - 0.5LSB | 0 | 00000001 |
| -125mV | +125mV | 0 | 00000000 (zero scale) |

Offset Adjust The MAX104 provides a control input (VOSADJ) to com- pensate for system offsets. The offset adjust input is a self-biased voltage divider from the internal +2.5V preci- sion reference. The nominal open-circuit voltage is one- half the reference voltage. With an input resistance of typically 25k, this pin may be driven by an external

10k potentiometer (Figure 11) connected  between REFOUT and GNDI to correct for offset errors. This con- trol provides a typical ±5.5LSB offset adjustment range.

Clock Operation The MAX104 clock inputs are designed for either single- ended or differential operation (Figure 12) with flexible input drive requirements. Each clock input is terminated

REFOUT

POT

10k

GNDI

MAX104

VOSADJ

with an on-chip, laser-trimmed 50 resistor to CLKCOM (clock-termination return). The CLKCOM termination voltage can be connected  anywhere between ground and -2V for compatibility with standard ECL drive levels.

The clock inputs are internally buffered with a preampli- fier to ensure proper operation of the data converter with even small-amplitude sine-wave sources. The MAX104 was designed  for single-ended,  low-phase- noise sine-wave clock signals with as little as 100mV amplitude (-10dBm). This eliminates the need for an external ECL clock buffer and its added jitter.

Single-Ended Clock Inputs (Sine-Wave Drive) Excellent performance is obtained by AC- or DC-cou- pling a low-phase-noise sine-wave source into a single clock input (Figure 13a, Table 5). For proper DC bal- ance, the undriven clock  input should be externally

50 reverse-terminated to GNDI.

The dynamic  performance  of the data converter is essentially unaffected by clock-drive power levels from

Figure 11. Offset Adjust with External 10k Potentiometer

CLK+

50                                                                                +0.8V CLKCOM

50                                                  GNDI CLK-

CLOCK INPUTS ARE ESD PROTECTED

(NOT SHOWN IN THIS SIMPLIFIED DRAWING).

VEE

Figure 12. Simplified Clock Input Structure (Single-Ended/ Differential)

18

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

-10dBm (100mV clock signal amplitude) to +10dBm (1V clock signal amplitude). The MAX104 dynamic per- formance specifications  are determined by a single- ended  clock  drive of +4dBm  (500mV clock  signal amplitude). To avoid saturation of the input amplifier stage, limit the clock  power level to a maximum of

+10dBm.

Differential Clock Inputs (Sine-Wave Drive) The advantages of differential clock drive (Figure 13b, Table 5) can be obtained  by using an appropriate balun or transformer to convert single-ended sine-wave sources into differential drives. The precision on-chip, laser-trimmed 50 clock-termination resistors ensure excellent amplitude matching. Refer to Single-Ended Clock Inputs (Sine-Wave Drive) for proper input ampli- tude requirements.

MAX104

Single-Ended Clock Inputs (ECL Drive) Configure the MAX104 for single-ended ECL clock drive by connecting the clock inputs as shown in Figure 13c (Table 5). A well bypassed VBB supply (-1.3V) is essen- tial to avoid coupling noise into the undriven clock input, which would degrade dynamic performance.

Differential Clock Inputs (ECL Drive) The MAX104 may be driven from a standard differential (Figure 13d, Table 5) ECL clock source by setting the clock termination voltage at CLKCOM to -2V. Bypass the clock-termination return (CLKCOM) as close as possible to the ADC with a 0.01µF capacitor connected to GNDI.

AC-Coupling Clock Inputs The clock inputs CLK+ and CLK- can also be driven with positive referenced ECL (PECL) logic if the clock

+0.5V

CLK+

CLK- = 0V

+0.5V

CLK+

CLK-

-0.5V

NOTE: CLKCOM = 0V

t                                                                -0.5V                                                                                              t

NOTE: CLKCOM = 0V

Figure 13a. Single-Ended  Clock Input Signals

Figure 13b. Differential Clock Input Signals

-0.8V

CLK+

CLK- = -1.3V

-0.8V

CLK+

CLK-

-1.8V                                                                                               t

-1.8V                                                                                              t

NOTE: CLKCOM = -2V

NOTE: CLKCOM = -2V

Figure 13c. Single-Ended  ECL Clock Drive

Figure 13d. Differential ECL Clock Drive

19

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Table 5. DC-Coupled Clock Drive Options

| CLOCK DRIVE | CLK+ | CLK- | CLKCOM | REFERENCE |
| --- | --- | --- | --- | --- |
| Single-Ended Sine Wave | -10dBm to +4dBm | External 50Ω to GNDI | GNDI | Figure 13a |
| Differential Sine Wave | -10dBm to +4dBm | -10dBm to +4dBm | GNDI | Figure 13b |
| Single-Ended ECL | ECL Drive | -1.3V | -2V | Figure 13c |
| Differential ECL | ECL Drive | ECL Drive | -2V | Figure 13d |

inputs are AC-coupled.  Under this condition, connect CLKCOM to GNDI. Single-ended ECL/PECL/sine-wave drive is also possible if the undriven clock input is reverse-terminated to GNDI through a 50 resistor in series with a capacitor  whose value is identical to that used to couple the driven input.

Demux Reset Operation The MAX104 features an internal 1:2 demultiplexer that reduces the data rate of the output digital data to one- half the sample clock rate. Demux reset is necessary when interleaving multiple MAX104s and/or synchroniz- ing external demultiplexers. The simplified block diagram of Figure 1 shows that the demux reset signal path con- sists of four main circuit blocks. From input to output, they are the reset input dual latch, the reset pipeline, the demux clock generator, and the reset output. The sig- nals associated with the demux reset operation and the control of this section are listed in Table 6.

Reset Input Dual Latch

RSTIN+

RSTIN-

50k

50k

20A

VCCO

The reset input dual-latch circuit block accepts  differ- ential PECL reset inputs referenced to the same VCCO power supply that powers the MAX104 PECL outputs. For applications  that do not require a synchronizing

RESET INPUTS  ARE

ESD PROTECTED

(NOT SHOWN IN THIS SIMPLIFIED DRAWING).

GNDD

reset, the reset inputs can be left open. In this case, they will self-bias to a proper level with internal 50k resistors and 20µA current source. This combination creates a -1V difference between RSTIN+ and RSTIN- to disable the internal reset circuitry. When driven with PECL logic levels terminated with 50 to (VCCO - 2V), the internal biasing network can easily be overdriven. Figure 14 shows a simplified schematic of the reset input structure.

To properly latch the reset input data, the setup time (tSU) and the data hold time (tHD) must be met with respect to the rising edge of the sample clock. The tim- ing diagram of Figure 15 shows the timing relationship of the reset input and sampling clock.

Figure 14. Simplified Reset Input Structure

RSTIN+

50%                      50% RSTIN-

tSU                                tHD

CLK+

50%

CLK-

Figure 15. Reset Input Timing Definitions

20

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Table 6. Demux Operating and Reset Control Signal

| SIGNAL NAME | TYPE | FUNCTION |
| --- | --- | --- |
| CLK+, CLK- | Sampling clock inputs | Master ADC timing signal. The ADC samples on the rising edge of CLK+. |
| DREADY+, DREADY- | Differential PECL outputs | Data-Ready PECL Output. Output data changes on the rising edge of DREADY+. |
| RSTIN+, RSTIN- | Differential PECL inputs | Demux reset input signals. Resets the internal demux when asserted. |
| RSTOUT+, RSTOUT- | Differential PECL outputs | Reset outputs for resetting additional external demux devices |

Reset Pipeline The next section in the reset signal path is the reset pipeline.  The purpose  of this block is to add clock cycles of latency to the reset signal, to match the laten- cy of the converted analog data through the ADC. In this way, when reset data arrives at the RSTOUT+/ RSTOUT- PECL output, it will be time-aligned with the analog data present in the primary and auxiliary ports at the time the reset input was deasserted at RSTIN+/ RSTIN-.

Demux Clock Generator The demux clock generator creates the DIV1, DIV2, or DIV4 clocks required for the different modes of demux and non-demux operation. The TTL/CMOS control inputs DEMUXEN and DIVSELECT control the demuxed mode selection, as described  in Table 2. The timing diagrams in Figure 16 and Figure 17 show the output timing and data alignment in DIV1, DIV2, and DIV4 modes, respectively.

The phase relationship between the sampling clock at the CLK+/CLK- inputs and the data-ready clock at the DREADY+/DREADY- outputs will be random at device power-up. As with all divide-by-two circuits, two possi- ble phase relationships exist between these clocks. The difference between the phases is simply the inver- sion of the DIV2-DREADY clock. The timing diagram in Figure 16 shows this relationship.

Reset all MAX104 devices to a known DREADY phase after initial power-up for applications such as interleav- ing, where two or more MAX104 devices are used to achieve higher effective sampling rates. This synchro- nization is necessary to set the order of output samples between the devices. Resetting the converters accom- plishes this synchronization. The reset signal is used to force the internal counter in the demux clock-generator block to a known phase state.

CLK+

tPWH                                                             tPWL

CLK+

DREADY-

tPD1

50%

CLK-

CLK-

tPD1

DREADY +

"PHASE 1" DREADY+

DREADY +

50%

tFDREADY

tRDREADY

AUXILIARY PORT DATA

tPD2

DREADY -

"PHASE 2" DREADY -

80%                                            80%

20%            20%

PRIMARY PORT DATA

Figure 16. CLK and DREADY Timing in Demuxed DIV2 Mode

Showing Two Possible DREADY Phases

Figure 17. Output Timing for All Modes (DIV1, DIV2, DIV4)

21

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Reset Output Finally, the reset signal is presented in differential PECL format to the last block  of the reset signal path. RSTOUT+/RSTOUT- output the time-aligned reset sig- nal used for resetting additional external demuxes in applications  that need further output data rate reduc- tion. Many demux devices require that their reset signal be asserted for several clock  cycles  while they are clocked.  To accomplish  this, the MAX104 DREADY clock will continue to toggle while RSTOUT is asserted.

When a single MAX104 device is used, no synchroniz- ing reset is required, because the order of the samples in the output ports is unchanged  regardless of the phase of the DREADY clock. In DIV2 mode, the data in the auxiliary port is delayed by 8.5 clock cycles, while the data in the primary port is delayed by 7.5 clock cycles. The older data is always in the auxiliary port, regardless of the phase of the DREADY clock.

The reset output signal, RSTOUT, is delayed by one less clock cycle (6.5 clock cycles) than the primary

port. The reduced  latency of RSTOUT serves to mark the start of synchronized data in the primary and auxil- iary ports. When the RSTOUT signal returns to a zero, the DREADY clock phase is reset.

Since there are two possible phases of the DREADY clock with respect to the input clock, there are two pos- sible timing diagrams to consider. The first timing dia- gram (Figure 18) shows the RSTOUT timing and data alignment of the auxiliary and primary output ports when the DREADY clock phase is already reset. For this example, the RSTIN pulse is two clock cycles long. Under this condition, the DREADY clock  continues uninterrupted, as does the data stream in the auxiliary and primary ports.

The second  timing diagram  (Figure 19) shows the results when the DREADY phase is opposite from the reset phase. In this case, the DREADY clock “swallows” a clock cycle of the sample clock, resynchronizing to the reset phase. Note that the data stream in the auxil- iary and primary ports has reversed. Before reset was

CLK- CLK

ADC SAMPLE NUMBER                           ADC SAMPLES  ON THE RISING EDGE OF CLK+

n              n+1             n+2             n+3           n+4             n+5            n+6             n+7            n+8             n+9           n+10          n+11           n+12          n+13

CLK+                          tSU                                                       tHD

RSTIN-

RESET

INPUT DREADY

AUXILIARY DATA PORT

RSTIN+ DREADY-

DREADY+

n-1

n+1

n+3

PRIMARY DATA PORT

RESET OUT DATA PORT

RSTOUT- RSTOUT+

n                                  n+2                               n+4

NOTE:   THE LATENCY  TO THE RESET OUTPUT IS 6.5 CLOCK  CYCLES. THE LATENCY  TO THE PRIMARY  PORT IS 7.5 CLOCK CYCLES, AND

THE LATENCY TO THE AUXILIARY  PORT IS 8.5 CLOCK CYCLES. ALL DATA PORTS ARE UPDATED ON THE RISING EDGE OF THE DREADY+ CLOCK.

Figure 18. Reset Output Timing in Demuxed DIV2 Mode (DREADY Aligned)

22

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

CLK- CLK

ADC SAMPLE NUMBER                           ADC SAMPLES  ON THE RISING EDGE OF CLK+

MAX104

n              n+1             n+2             n+3           n+4             n+5            n+6             n+7            n+8             n+9           n+10          n+11           n+12          n+13

CLK+                          tSU                                                       tHD

RSTIN-

RESET

INPUT DREADY

AUXILIARY DATA PORT

RSTIN+ DREADY+

DREADY-

n-2

CLOCK  PULSE “SWALLOWED”

OUT-OF-SEQUENCE SAMPLE

n                                          n+2

PRIMARY DATA PORT

RESET OUT DATA PORT

RSTOUT- RSTOUT+

n-1                                n+1

n+4

NOTE:   DREADY PHASE WAS ADJUSTED  TO MATCH  THE RESET PHASE BY “SWALLOWING” ONE INPUT CLOCK CYCLE.

THE AUXILIARY PORT CONTAINS AN OUT-OF-SEQUENCE SAMPLE AS A RESULT OF THE DELAY.

Figure 19. Reset Output Timing in Demuxed DIV2 Mode (DREADY Realigned)

asserted, the auxiliary port contained “even” samples while the primary port contained “odd”  samples. After the RSTOUT is deasserted (which marks the start of the DREADY clock’s reset phase), note that the order of the samples in the ports has been reversed. The auxiliary port also contains an out-of-sequence sample. This is a consequence  of the “swallowed”  clock cycle that was needed to resynchronize DREADY to the reset phase. Also note that the older sample data is always in the aux- iliary port, regardless of the DREADY phase.

These examples illustrate the combinations that result with a reset input signal of two clock cycles. It is also possible to reset the internal MAX104 demux success- fully with a reset pulse of only one clock cycle, provided that the setup-time and hold-time requirements are met with respect to the sample clock. However, this is not recommended when additional external demuxes are used.

Note that many external demuxes require that their reset signals be asserted while they are clocked,  and may require more than one clock cycle of reset. More importantly, if the phase of the DREADY clock is such that a clock pulse will be “swallowed” to resynchronize,

then no reset output will occur at all. In effect, the RSTOUT signal will be “swallowed”  with the clock pulse. The best method to ensure complete system reset is to assert RSTIN for the appropriate number of DREADY clock cycles required to complete reset of the external demuxes.

Die  Temperature Measurement For applications that require monitoring of the die tem- perature, it is possible to determine the die temperature of the MAX104 under normal operating conditions by observing the currents ICONST and IPTAT, at contacts ICONST and IPTAT. ICONST and IPTAT are two 100µA (nominal) currents that are designed  to be equal at

+27°C. These currents are derived from the MAX104’s internal precision +2.5V bandgap  reference. ICONST is designed to be temperature independent, while IPTAT is directly proportional to the absolute temperature. These currents are derived from pnp current sources refer- enced from VCCI and driven into two series diodes con- nected to GNDI. The contacts ICONST and IPTAT may be left open, because internal catch diodes prevent saturation of the current sources. The simplest method

23

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

of determining the die temperature is to measure each current with an ammeter (which shuts off the internal catch diodes) referenced to GNDI. The die temperature in °C is then calculated by the expression:

T                        IPTAT  

To calibrate the circuit, first connect pins 2-3 on JU1 to zero the input of the PTAT path. With the MAX104 pow- ered up, adjust potentiometer R3 until the voltage at the VTEMP output is -2.728V. Connecting pins 1-2 on JU1 restores normal operation to the circuit after the calibra- tion is complete. The voltage at the VTEMP node will

DIE

300

          

                

ICONST 

273

then be proportional to the actual MAX104 die tempera- ture according to the equation:

Another method of determining the die temperature

uses the operational amplifier circuit shown in Figure 20. The circuit produces a voltage that is proportional to the die temperature. A possible application for this signal is speed control for a cooling fan, to maintain constant MAX104 die temperature. The circuit operates by con- verting the ICONST and IPTAT currents to voltages VCON- ST and VPTAT, with appropriate  scaling to account for their equal values at +27°C. This voltage difference is then amplified by two amplifiers in an instrumentation- amplifier configuration with adjustable gain. The nominal value of the circuit gain is 4.5092V/V. The gain of the instrumentation amplifier is given by the expression:

TDIE (°C) = 100  x VTEMP

The overall accuracy  of the die temperature measure- ment using the operational-amplifier scaling circuitry is limited mainly by the accuracy  and matching of the resistors in the circuit.

Thermal Management Depending  on the application  environment for the ESBGA-packaged MAX104, the customer may have to apply an external heatsink to the package  after board assembly. Existing open-tooled heatsinks are available from standard  heatsink suppliers  (see Heatsink Manufacturers). The heatsinks are available with pre- applied adhesive for easy package mounting.

AV   

VTEMP

VCONST  

VPTAT

AV     1 

R1   2  R1

R2           R3

3.32k            5k

10-TURN

IPTAT

6.65k

6.65k

1/4 MAX479

12.1k

12.1k

JU1

2

1

3  VPTAT

R1

7.5k

R2

15k

1/4 MAX479

R2

15k

R1

7.5k

ICONST

6.05k

1/4 MAX479

VCONST

1/4 MAX479

VTEMP

Figure 20. Die Temperature Acquisition  Circuit with the MAX479

24

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

Table 7. Thermal Performance for

MAX104 With or Without Heatsink

MAX104

THERMAL RESISTANCE vs. AIRFLOW

18

| AIRFLOW 16 | MAX104 JA (°C/W) |  |
| --- | --- | --- |
| (linear ft./min.) | WITHOUT HEATSINK | WITH HEATSINK |
| 0 | 16.5 | 12.5 |
| 200 | 14.3 | 9.4 |
| 400 | 13 | 8.3 |
| 800 | 12.5 | 7.4 |

JA (°C/W)

14

12

10

8

Thermal Performance

6

| 0 100 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | WITHOUT HEATSINK |  |  |  |
|  |  |  |  | WITH HEATSINK |  |  |  |  |

200

300    400    500    600    700    800

The MAX104 has been modeled to determine the ther- mal resistance from junction to ambient. Table 7 lists the ADC’s thermal performance parameters:

Ambient Temperature:              TA = +70°C

Heatsink Dimensions:               25mm x 25mm x 10mm

PC Board Size and Layout:      4" x 4"

2 Signal Layers

2 Power Layers

Heatsink Manufacturers Aavid Engineering and IERC provide open-tool, low- profile heatsinks, fitting the 25mm x 25mm ESBGA package.

Aavid Engineering, Inc. Phone: 714-556-2665

Heatsink Catalog #: 335224B00032

Heatsink Dimensions: 25mm x 25mm x 10mm

International Electronic Research Corporation (IERC) Phone: 818-842-7277

Heatsink Catalog #: BDN09-3CB/A01

Heatsink Dimensions: 23.1mm x 23.1mm x 9mm

Bypassing/Layout/Power Supply Grounding and power-supply decoupling strongly influ- ence the MAX104’s performance. At 1GHz clock fre- quency and 8-bit resolution, unwanted digital crosstalk may couple through the input, reference, power supply, and ground connections and adversely influence the dynamic performance of the ADC. Therefore, closely follow the grounding  and power-supply  decoupling guidelines (Figure 22).

AIRFLOW (linear ft./min.)

Figure 21. MAX104 Thermal Performance

Maxim strongly recommends using a multilayer printed circuit board (PCB) with separate ground and power- supply planes. Since the MAX104 has separate analog and digital ground connections (GNDA, GNDI, GNDR, and GNDD, respectively), the PCB should feature sep- arate analog and digital ground sections connected at only one point (star ground at the power supply). Digital signals should run above the digital ground plane, and analog signals should run above the analog ground plane. Keep digital signals far away from the sensitive analog inputs, reference inputs, and clock inputs. High- speed signals, including  clocks, analog inputs, and digital outputs, should be routed on 50 microstrip lines, such as those employed on the MAX104EVKIT.

The MAX104 has separate analog and digital power- supply inputs: VEE (-5V analog and substrate supply) and VCCI (+5V) to power the T/H amplifier, clock distri- bution, bandgap  reference, and reference amplifier; VCCA (+5V) to supply  the ADC’s comparator array; VCCO (+3V to VCCD) to establish power for all PECL- based circuit sections; and VCCD (+5V) to supply all logic circuits of the data converter.

The MAX104 VEE supply  contacts  must not be left open while the part is being powered up. To avoid this condition, add a high-speed Schottky diode (such as a Motorola 1N5817) between VEE and GNDI. This diode prevents the device  substrate from forward biasing, which could cause latchup.

25

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

All supplies should be decoupled with large tantalum or electrolytic capacitors at the point where they enter the PCB. For best performance, bypass all power supplies to the appropriate ground with a 10µF tantalum capaci- tor, to filter power-supply noise, in parallel with a 0.01µF capacitor and a high-quality 47pF ceramic chip capaci- tor located very close to the MAX104 device, to filter very high-frequency noise.

Static Parameter Definitions

Integral Nonlinearity (INL) Integral nonlinearity is the deviation of the values on an actual transfer function from a straight line. This straight

line can be either a best-straight-line fit or a line drawn between the endpoints of the transfer function, once offset and gain errors have been nullified. The static lin- earity parameters for the MAX104 are measured using the best-straight-line fit method.

Differential Nonlinearity (DNL) Differential nonlinearity is the difference between an actual step width and the ideal value of 1LSB. A DNL error specification  of less than 1LSB guarantees no missing codes and a monotonic transfer function.

VCCO

GNDD

10F

10nF          10nF          47pF          47pF          47pF          47pF

NOTE:

LOCATE ALL 47pF CAPACITORS AS CLOSE AS POSSIBLE TO THE MAX104  DEVICE.

VCCI

GNDI

10F          10nF          10nF          47pF          47pF         47pF          47pF

VEE

VCCA

GNDA

10F          10nF          10nF          47pF          47pF

1N5817

GNDI

10F

10nF          10nF          47pF          47pF          47pF          47pF

VCCD

VCCA = +4.75V TO +5.25V VCCD = +4.75V TO +5.25V VCCI = +4.75V TO +5.25V VCCO = +3.0V TO VCCD VEE = -4.75V  TO -5.25V

GNDD

10F          10nF          10nF          47pF          47pF          47pF          47pF

Figure 22. MAX104 Bypassing  and Grounding

26

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

Bit Error Rates (BER) Errors resulting from metastable states may occur when the analog input voltage (at the time the sample is taken) falls close to the decision point of any one of the input comparators. Here, the magnitude of the error depends on the location of the comparator in the comparator net- work. If it is the comparator for the MSB, the error will reach full scale. The MAX104’s unique encoding scheme solves this problem by limiting the magnitude of these errors to 1LSB; it reduces the probability of these errors occurring to typically one in every 1016 clock cycles.

Dynamic Parameter Definitions

Signal-to-Noise Ratio (SNR) For a waveform perfectly  reconstructed  from digital samples, the theoretical maximum SNR is the ratio of the full-scale analog input (RMS value) to the RMS

MAX104

Effective Number of Bits (ENOB) ENOB indicates the global accuracy  of an ADC at a specific  input frequency and sampling rate. An ideal ADC’s error consists of quantization noise only. ENOB is calculated from a curve fit referenced to the theoreti- cal full-scale range.

Signal-to-Noise Plus Distortion (SINAD) SINAD is calculated  from the effective number of bits (ENOB) as follows:

SINAD = (6.02 x ENOB) + 1.76

Total Harmonic Distortion (THD) THD is the ratio of the RMS sum of the first four harmon- ics of the input signal to the fundamental itself. This is expressed as:

quantization error (residual error). The ideal, theoretical

       2

2         2         2        

minimum analog-to-digital noise is caused by quantiza-

THD

  20  log 



 V2

 V3

 V4

 V5

 / V1



tion error only and results directly from the ADC’s reso-

lution (N bits):

SNR(MAX) = (6.02 x N + 1.76)dB

In reality, there are other noise sources besides quanti- zation noise: thermal noise, reference noise, clock jitter, etc. SNR is calculated  by taking the ratio of the RMS signal to the RMS noise, which includes all spectral components minus the fundamental, the first five har- monics, and the DC offset.

                                                  

where V1 is the fundamental amplitude, and V2 through V5 are the amplitudes of the 2nd- through 5th-order harmonics.

Spurious-Free Dynamic Range (SFDR) SFDR is the ratio, expressed in decibels,  of the RMS amplitude of the fundamental (maximum signal compo- nent) to the RMS value of the next largest spurious component, excluding DC offset.

Intermodulation Distortion (IMD) The two-tone IMD is the ratio, expressed in decibels, of either input tone to the worst 3rd-order (or higher) inter- modulation products. The input tone levels are at -7dB full scale.

Chip Information

TRANSISTOR COUNT: 20,486

SUBSTRATE CONNECTED TO VEE

27

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Typical Operating Circuit

+5V ANALOG

+3.3V DIGITAL

Z0 = 50

ALL PECL OUTPUTS MUST BE TERMINATED

50

-5V ANALOG

+5V DIGITAL

LIKE THIS.                       VCCO - 2V

VEE        VCCA     VCCI      VCCD DIVSELECT

AUXEN1   VCCO   AUXEN2

2

OR+/OR-

+5V DIGITAL

DEMUXEN

2   P7+/P7-

VOSADJ

2

P6+/P6-

DIFFERENTIAL ANALOG INPUT

500mVp-p  FS

SAMPLE CLOCK

1GHz

+4dBm

Z0 = 50

Z0 = 50

Z0 = 50

50

GNDI

GNDI

VIN+ VIN-

CLK+

CLK-

CLKCOM

MAX104

PRIMARY PECL OUTPUTS

AUXILARY PECL OUTPUTS

2    P5+/P5-

2

P4+/P4-

2    P3+/P3-

2

TO MEMORY OR DIGITAL SIGNAL PROCESSOR

P2+/P2-

2    P1+/P1-

2

P0+/P0-

2    A7+/A7-

2

A6+/A6-

2    A5+/A5-

2

A4+/A4-

2    A3+/A3-

2

A2+/A2-

RSTIN+ RSTIN-

DREADY+/DREADY- RSTOUT+/RSTOUT-

2    A1+/A1-

2

A0+/A0-

2

2

GNDA   GNDR   GNDI      GNDD     REFOUT   REFIN

28

±5V, 1Gsps, 8-Bit ADC with

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

192-Contact ESBGA PCB Land Pattern

TOP  VIEW

MAX104

29

±5V, 1Gsps, 8-Bit ADC with

SUPER BGA.EPS

On-Chip 2.2GHz Track/Hold Amplifier

MAX104

Package Information (The package  drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package  outline information, go to www.maxim-ic.com/packages.)

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

30                                         Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2002 Maxim Integrated  Products               Printed USA                                  is a registered  trademark of Maxim Integrated  Products.