19-1413; Rev 0; 12/98

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

General Description

The MAX1200 16-bit, monolithic, analog-to-digital  con- verter (ADC) is capable  of conversion rates up to

1Msps. This CMOS integrated circuit uses a fully differ- ential, pipelined architecture with digital error correction and a short self-calibration to ensure 16-bit linearity at full sample rates. An on-chip track/hold (T/H) maintains superb dynamic performance up to the Nyquist frequen- cy. The MAX1200 operates from a single +5V supply.

The fully differential inputs allow an input swing of

±VREF. The reference is also differential with the posi- tive reference (RFPF) typically connected  to +4.096V and the negative reference (RFNF) connected  to ana- log ground. Additional sensing pins (RFPS, RFNS) are provided to compensate for any resistive divider action that may occur. A single-ended  input is also possible using two operational amplifiers.

Power dissipation is typically only 273mW at +5V, at a sampling rate of 1Msps. The device employs a CMOS- compatible,  16-bit parallel, two’s complement output data format. For a higher sampling  speed  (up to

2.2Msps) but lower resolution (14-bit), select the

MAX1201, a pin-compatible version of the MAX1200.

The MAX1200 is available in an MQFP package  and operates over the commercial  (0°C to +70°C) and extended-industrial  (-40°C to +85°C) temperature ranges.

MAX1200

Features

♦ Monolithic 16-Bit, 1Msps A/D Converter

♦ Single +5V Supply

♦ ±VREF Differential Input Voltage Range

♦ 87dB SNR for fIN = 100kHz

♦ 91dB SFDR for fIN = 100kHz

♦ 273mW Low-Power Dissipation

♦ ±0.5LSB Differential Nonlinearity Error

♦ Three-State, Two’s Complement Output Data

♦ On-Demand Self-Calibration

♦ Pin-Compatible 14-Bit Versions Available

(1Msps MAX1205, 2.2Msps MAX1201)

Ordering Information

DNL PART TEMP. RANGE PIN-PACKAGE (LSB)

MAX1200ACMH 0°C to +70°C 44 MQFP ±0.5

MAX1200BCMH 0°C to +70°C 44 MQFP —

MAX1200AEMH -40°C to +85°C 44 MQFP ±0.5

MAX1200BEMH -40°C to +85°C 44 MQFP —

Pin  Configuration

44

43

42

41

40

39

38

37

36

35

34

END_CAL INN

N.C. N.C. INP RFNS RFNF RFPS RFPF CM

TEST0

TOP VIEW

Applications

High-Resolution Imaging Communications Scanners

Data Acquisition

Instrumentation

ST_CAL       1

AGND       2

AVDD        3

AGND       4

AGND       5

AVDD        6

DOR       7

D15        8

D14        9

D13      10

D11

D10

D9

D8

DRVDD DGND D7

D6

D5

D4

D3

12

13

14

15

16

17

18

19

20

21

22

D12      11

MAX1200

33       OE

32       DAV

31      CLK

30      DVDD

29       DGND

28       DGND

27      DVDD

26       TEST1

25      D0

24      D1

23      D2

MQFP

Maxim Integrated Products   1

For free samples & the latest literature: http://www.maxim-ic.com, or phone 1-800-998-8800. For small orders, phone 1-800-835-8769.

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

ABSOLUTE MAXIMUM RATINGS

AVDD to AGND, DGND ..........................................................+7V DVDD to DGND, AGND..........................................................+7V DRVDD to DGND, AGND .......................................................+7V INP, INN, RFPF, RFPS,

RFNF, RFNS, CLK, CM..........(AGND  - 0.3V) to (AVDD + 0.3V) Digital Inputs to DGND ............................-0.3V  to (DVDD + 0.3V) Digital Output (DAV) to DGND ..............-0.3V to (DRVDD + 0.3V) Other Digital Outputs to DGND .............-0.3V to (DRVDD + 0.3V)

Continuous Power Dissipation (TA = +70°C)

44-Pin MQFP (derate 11.11mW/°C above +70°C).......889mW

Operating Temperature Ranges (TA)

MAX1200_CMH ..................................................0°C to +70°C

MAX1200_EMH................................................-40°C to +85°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10sec) .............................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(AVDD = +5V ±5%, DVDD = DRVDD = +3.3V, VRFPS = +4.096V, VRFNS = AGND, VCM = +2.048V, VIN = -0.5dBFS, fCLK = 2.048MHz;

digital output load  20pF; TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| ANALOG INPUT |  |  |  |  |
| Input Voltage Range (Note 2) | VIN | Single-ended | 4.096 | V |
|  |  | Differential | ±4.096 |  |
| Input Resistance (Note 3) | RI |  | 55 | kΩ |
| Input Capacitance | CI | Per side in track mode | 21 | pF |
| EXTERNAL REFERENCE |  |  |  |  |
| Reference Voltage (Note 4) | VREF |  | 4.096 4.5 | V |
| Reference Input Resistance | RREF |  | 700 1000 | Ω |
| TRANSFER CHARACTERISTICS |  |  |  |  |
| Resolution (No missing codes; Note 5) | RES | After calibration, guaranteed for MAX1200A only | 16 | Bits |
| Integral Nonlinearity | INL |  | ±3.5 | LSB |
| Differential Nonlinearity | DNL | MAX1200A | -1 ±0.5 +1 | LSB |
|  |  | MAX1200B | ±0.6 |  |
| Offset Error |  |  | -0.2 ±0.003 +0.2 | %FSR |
| Gain Error |  |  | -5 -3 5 | %FSR |
| Input-Referred Noise |  |  | 75 | µVRMS |
| DYNAMIC SPECIFICATIONS (No | te 6) |  |  |  |
| Maximum Sampling Rate | fSAMPLE | fSAMPLE = fCLK / 2 | 1.024 | Msps |
| Conversion Time (Pipeline Delay/Latency) |  |  | 4 | fSAMPLE Cycles |
| Acquisition Time | tACQ | To full-scale step (0.006%) | 125 | ns |
| Overvoltage Recovery Time | tOVR |  | 450 | ns |
| Aperture Delay | tAD |  | 3 | ns |
| Aperture Jitter | tAJ |  | 5 | psRMS |
| Full-Power Bandwidth |  |  | 3.3 | MHz |
| Small-Signal Bandwidth |  |  | 78 | MHz |

2

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

ELECTRICAL CHARACTERISTICS  (continued)

MAX1200

(AVDD = +5V ±5%, DVDD = DRVDD = +3.3V, VRFPS = +4.096V, VRFNS = AGND, VCM = +2.048V, VIN = -0.5dBFS, fCLK = 2.048MHz;

digital output load  20pF; TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
|  | SNR | VRFPS = 4.096V, VRFNS = AGND | fIN = 99.5kHz | 83 87 |  |
| Signal-to-Noise Ratio (Note 5) |  |  | fIN = 300.5kHz | 84 |  |
|  |  |  | fIN = 504.5kHz | 83 |  |
|  |  | VRFPS = 3.5V, VRFNS = 1.5V | fIN = 99.5kHz | 78 83 | dB |
|  |  |  | fIN = 300.5kHz | 81 |  |
|  |  |  | fIN = 504.5kHz | 80 |  |
| Spurious-Free Dynamic Range (Note 5) | SFDR | VRFPS = 4.096V, VRFNS = AGND | fIN = 99.5kHz | 84 91 | dB |
|  |  |  | fIN = 300.5kHz | 89 |  |
|  |  |  | fIN = 504.5kHz | 88 |  |
|  |  | VRFPS = 3.5V, VRFNS = 1.5V | fIN = 99.5kHz | 85 92 |  |
|  |  |  | fIN = 300.5kHz | 91 |  |
|  |  |  | fIN = 504.5kHz | 90 |  |
| Total Harmonic Distortion (Note 5) | THD | VRFPS = 4.096V, VRFNS = AGND | fIN = 99.5kHz | -87 -82 | dB |
|  |  |  | fIN = 300.5kHz | -86 |  |
|  |  |  | fIN = 504.5kHz | -85 |  |
|  |  | VRFPS = 3.5V, VRFNS = 1.5V | fIN = 99.5kHz | -90 -84 |  |
|  |  |  | fIN = 300.5kHz | -89 |  |
|  |  |  | fIN = 504.5kHz | -88 |  |
| Signal-to-Noise Ratio plus Distortion (Note 5) | SINAD | VRFPS = 4.096V, VRFNS = AGND | fIN = 99.5kHz | 80 84 | dB |
|  |  |  | fIN = 300.5kHz | 82 |  |
|  |  |  | fIN = 504.5kHz | 81 |  |
|  |  | VRFPS = 3.5V, VRFNS = 1.5V | fIN = 99.5kHz | 77 82 |  |
|  |  |  | fIN = 300.5kHz | 80.5 |  |
|  |  |  | fIN = 504.5kHz | 79.5 |  |
| POWER REQUIREMENTS |  |  |  |  |  |
| Analog Supply Voltage | AVDD |  |  | 4.75 5 5.25 | V |
| Analog Supply Current | I(AVDD) |  |  | 51 70 | mA |
| Digital Supply Voltage | DVDD |  |  | 3 5.25 | V |
| Digital Supply Current | I(DVDD) |  |  | 0.4 1.2 | mA |
| Output Drive Supply Voltage | DRVDD |  |  | 3 DVDD | V |
| Output Drive Supply Current | I(DRVDD) | 10pF loads on D0–D15 and DAV |  | 0.1 0.6 | mA |
| Power Dissipation | PDSS |  |  | 273 377 | mW |
| Warm-Up Time |  |  |  | 0.1 | sec |
| Power-Supply Rejection Ratio | PSRR | Offset |  | 55 | dB |
|  |  | Gain |  | 55 |  |

3

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

TIMING CHARACTERISTICS  (Figures 7, 8, 9)

(AVDD = +5V ±5%,  DVDD = DRVDD = +3.3V, fCLK = 2.048MHz,  TA = TMIN to TMAX, unless otherwise  noted.  Typical values are at

TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Conversion Time | tCONV |  | 4 / fSAMPLE | ns |
| Clock Period | tCLK |  | 488 | ns |
| Clock HIGH Time | tCH |  | 187 244 301 | ns |
| Clock LOW Time | tCL |  | 187 244 301 | ns |
| Acquisition Time | tACQ |  | tCLK / 2 | ns |
| Output Delay | tOD |  | 70 150 | ns |
| DAV Pulse Width | tDAV |  | 1 / fCLK | ns |
| CLK-to-DAV Rising Edge | tS |  | 65 145 | ns |
| Data Access Time | tAC | CL = 20pF | 16 75 | ns |
| Bus Relinquish Time | tREL |  | 16 75 | ns |
| Calibration Time | tCAL | ST_CAL = DVDD | 17,400 | fCLK Cycles |

DIGITAL INPUT AND OUTPUT CHARACTERISTICS

(AVDD = +5V ±5%, DVDD = DRVDD  = +3.3V, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Input LOW Voltage | VIL |  | 0.8 | V |
| Input HIGH Voltage | VIH |  | DVDD - 0.8 | V |
| Input Capacitance |  |  | 4 | pF |
| CLK Input LOW Voltage | VCLK |  | 0.8 | V |
| CLK Input HIGH Voltage | VCLK |  | AVDD - 0.8 | V |
| CLK Input Current | ICLK | VIN = 0 or VDD | ±1 ±10 | µA |
| CLK Input Capacitance | CCLK |  | 9 | pF |
| Digital Input Current | IIN | VIN = 0 or DVDD | ±0.1 ±10 | µA |
| Output Low Voltage | VOL | ISINK = 1.6mA | 70 400 | mV |
| Output High Voltage | VOH | ISOURCE = 200µA | DVDD DVDD - 0.4 - 0.03 | V |
| Three-State Leakage Current | ILEAKAGE |  | ±0.1 ±10 | µA |
| Three-State Output Capacitance | COUT |  | 3.5 | pF |

Note 1: Reference inputs driven by operational amplifiers for Kelvin-sensed  operation.

Note 2: For unipolar mode, the analog input voltage, VINP, must be within 0 and VREF, VINN = VCM / 2; where VREF = VRFPS - VRFNS.

For differential mode, the analog input voltages VINP and VINN must be within 0 and VREF; where VREF = VRFPS - VRFNS. The common-mode voltage of the inputs INP and INN is VCM = (VRFPS + VRFNS) / 2.

Note 3: RI varies inversely with sample rate.

Note 4: Minimum and maximum parameters are not tested. Guaranteed by design.

Note 5: Calibration remains valid for temperature  changes  within ±20°C and power-supply variations ±5%. Guaranteed by design.

Note 6: All AC specifications are shown for the differential mode.

4

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

Typical Operating Characteristics

(AVDD = +5V ±5%, DVDD = DRVDD = +3.3V, VRFPS = +4.096V, VRFNS = AGND; VCM = +2.048V, differential input, fCLK = 2.048MHz, calibrated,  TA = +25°C, unless otherwise noted.)

MAX1200 toc01

INTEGRAL NONLINEARITY vs. TWO’S COMPLEMENT  OUTPUT CODE

5

4

3

2

INL (LSB)

1

0

-1

-2

-3

-4

-5

1.0

0.75

0.25

0

-0.25

DNL (LSB)

-0.75

-1.0

MAX1200toc02

DIFFERENTIAL NONLINEARITY vs. TWO’S COMPLEMENT  OUTPUT CODE

SINGLE-TONE SPURIOUS-FREE DYNAMIC  RANGE

MAX1200toc03

vs. INPUT AMPLITUDE (fIN = 99.5kHz)

120

SFDR (dB)

110

90

(dBc)

60

50

40

30

-32768     -16384         0              16384          32768

-32,768    -16,384         0             16,384         32,768

-80

-70   -60

-50   -40

vsMAX1200 toc06

-30   -20   -10    0

TWO’S COMPLEMENT OUTPUT CODE

TWO’S COMPLEMENT OUTPUT CODE

INPUT AMPLITUDE (dBFS)

MAX1200 toc04

SIGNAL-TO-NOISE RATIO PLUS DISTORTION vs. INPUT FREQUENCY

86

84

82

SINAD (dB)

80

78

AIN = -6dBFS

74

72

70                                   AIN = -20dBFS

68

1                       10                      100                    1000

INPUT FREQUENCY (kHz)

-74

-76

-78

THD (dB)

-80

-82

-84

-86

-88

-90

TOTAL HARMONIC DISTORTION

vs. INPUT FREQUENCY

MAX1200 toc05

AIN = -20dBFS

AIN = -6dBFS

AIN = -0.5dBFS

1                        10                     100                    1000

INPUT FREQUENCY (kHz)

SIGNAL-TO-NOISE RATIO

vs. INPUT FREQUENCY

90

AIN = -0.5dBFS

85

AIN = -6dBFS

SNR (dB)

80

75

70

AIN = -20dBFS

65

1                        10                      100                     1000

INPUT FREQUENCY (kHz)

SIGNAL-TO-NOISE RATIO PLUS DISTORTION

MAX1200 toc07

vs. SAMPLING RATE (fIN = 99.5kHz)

85

84

SINAD (dB)

83

82

81

80

0.1                                     1                                     10

SAMPLE  RATE (Msps)

0

-15

-30

AMPLITUDE (dBFS)

-45

-60

-75

-90

-105

-120

-135

-150

MAX1200 toc08

TYPICAL FFT, fIN = 99.5kHz,

8192  VALUE RECORD

0                       200                     400                     600

FREQUENCY (kHz)

0

-15

-30

AMPLITUDE (dBFS)

-45

-60

-75

-90

-105

-120

-135

-150

MAX1200 toc09

TYPICAL FFT, fIN = 504.5MHz,

8192  VALUE RECORD

0                       200                     400                     600

FREQUENCY (kHz)

5

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

Typical Operating Characteristics (AVDD = +5V ±5%,  DVDD = DRVDD = +3.3V, VRFPS  = +3.5V, VRFNS  = +1.5V; VCM = +2.5V, differential  input,  fCLK = 2.048MHz, calibrated,  TA = +25°C, unless otherwise noted.)

2.5

2.0

1.5

1.0

INL (LSB)

0.5

0

-0.5

-1.0

-1.5

-2.0

-2.5

MAX1200 toc11

INTEGRAL NONLINEARITY vs.

1.0

0.8

0.6

0.4

DNL (LSB)

0.2

0

-0.2

-0.4

-0.6

-0.8

-1.0

MAX1200 toc12

DIFFERENTIAL NONLINEARITY vs.

SINGLE-TONE SPURIOUS-FREE DYNAMIC  RANGE

110

100

90

80

SFDR (dB)

70

60

30

20

10

0

-32,768    -16,384         0             16,384         32,768

-32,768    -16,384         0             16,384         32,768

| -90 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dBFS) |  |  |  |  |  |  |
|  |  |  |  | (dB) |  |  |  |  |

-80 -70  -60 -50

-40  -30  -20

MAX1200 toc16

MAX1200 toc13

-10    0

TWO’S COMPLEMENT OUTPUT CODE

TWO’S COMPLEMENT OUTPUT CODE

INPUT AMPLITUDE (dBFS)

MAX1200 toc14

SIGNAL-TO-NOISE RATIO PLUS DISTORTION

vs. INPUT FREQUENCY

84

82             AIN = -0.5dBFS

80

78

-77

-79

-81

TOTAL HARMONIC DISTORTION

vs. INPUT FREQUENCY

MAX1200 TOC15

AIN = -20dBFS

SIGNAL-TO-NOISE RATIO

vs. INPUT FREQUENCY

85

AIN = -0.5dBFS

80

SINAD (dB)

THD (dB)

76                 AIN = -6dBFS

74

72

70

68

66

64

62                          AIN = -20dBFS

1                        10                      100                     1000

INPUT FREQUENCY (kHz)

-83

-85

-87

-89

-91

-93

AIN = -6dBFS

AIN = -0.5dBFS

1                        10                     100                    1000

INPUT FREQUENCY (kHz)

AIN = -6dBFS

SNR (dB)

75

70

65

AIN = -20dBFS

60

1                        10                      100                     1000

INPUT FREQUENCY (kHz)

SIGNAL-TO-NOISE RATIO PLUS DISTORTION

MAX1200 toc17

vs. SAMPLING RATE (fIN = 99.5kHz)

84

83

82

SINAD (dB)

81

79

78

0.1                                     1                                     10

SAMPLE  RATE (Msps)

0

-15

-30

AMPLITUDE (dBFS)

-45

-60

-75

-90

-105

-120

-135

-150

MAX1200 toc18

TYPICAL FFT, fIN = 99.5kHz,

8192  VALUE RECORD

0                       200                     400                     600

FREQUENCY (kHz)

0

-15

-30

AMPLITUDE (dBFS)

-45

-60

-75

-90

-105

-120

-135

-150

MAX1200 toc19

TYPICAL FFT, fIN = 504.5MHz,

8192  VALUE RECORD

0                       200                     400                     600

FREQUENCY (kHz)

6

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | ST_CAL | Digital Input to Start Calibration. ST_CAL = 0: Normal conversion mode. ST_CAL = 1: Start self-calibration. |
| 2, 4, 5 | AGND | Analog Ground |
| 3, 6 | AVDD | Analog Power Supply, +5V ±5% |
| 7 | DOR | Data Out-of-Range Bit |
| 8 | D15 | Bit 15 (MSB) |
| 9 | D14 | Bit 14 |
| 10 | D13 | Bit 13 |
| 11 | D12 | Bit 12 |
| 12 | D11 | Bit 11 |
| 13 | D10 | Bit 10 |
| 14 | D9 | Bit 9 |
| 15 | D8 | Bit 8 |
| 16 | DRVDD | Digital Power Supply for the Output Drivers. +3V to +5.25V, DRVDD ≤ DVDD |
| 17, 28, 29 | DGND | Digital Ground |
| 18 | D7 | Bit 7 |
| 19 | D6 | Bit 6 |
| 20 | D5 | Bit 5 |
| 21 | D4 | Bit 4 |
| 22 | D3 | Bit 3 |
| 23 | D2 | Bit 2 |
| 24 | D1 | Bit 1 |
| 25 | D0 | Bit 0 (LSB) |
| 26 | TEST1 | Test Pin 1. Do not connect. |
| 27, 30 | DVDD | Digital Power Supply, +3V to +5.25V |
| 31 | CLK | Input Clock. Receives power from AVDD to reduce jitter. |
| 32 | DAV | Data Valid Clock. This clock can be used to transfer the data to a memory or any other data acquisition system. |
| 33 | OE | Output Enable. OE = 0: D0–D15 and DOR are high impedance. OE = 1: All bits are active. |
| 34 | TEST0 | Test Pin 0. Do not connect. |
| 35 | CM | Common-Mode Voltage. Analog Input. Drive midway between positive and negative reference voltages. |
| 36 | RFPF | Positive Reference Voltage, Force Input |
| 37 | RFPS | Positive Reference Voltage, Sense Input |
| 38 | RFNF | Negative Reference Voltage, Force Input |
| 39 | RFNS | Negative Reference Voltage, Sense Input |
| 40 | INP | Positive Input Voltage |
| 41, 42 | N.C. | Not Connected. No internal connection. |
| 43 | INN | Negative Input Voltage |
| 44 | END_CAL | Digital Output for End of Calibration. END_CAL = 0: Calibration in progress. END_CAL = 1: Normal conversion mode. |

7

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

Detailed Description

Converter Operation The MAX1200 is a 16-bit, monolithic analog-to-digital converter (ADC) capable  of conversion rates up to

1Msps. It uses a multistage, fully differential, pipelined architecture with digital error correction and self-cali- bration to provide typically 91dB spurious-free dynamic range at a 1Msps sampling rate. It also provides excel- lent SNR and THD performance up to the Nyquist fre- quency. This makes the device suitable for applications such as data acquisition,  high-resolution imaging, scanners, digital communication, and instrumentation.

Figure 1 shows the simplified, internal structure of the ADC. A switched-capacitor,  pipelined  architecture is used to digitize the signal at a high throughput rate. The first four stages of the pipeline use a low-resolution quantizer to approximate the input signal. The multiply- ing digital-to-analog converter (MDAC) stage is used to subtract the quantized analog signal from the input. The residue is then amplified with a fixed gain and passed on to the next stage. The accuracy of the con- verter is improved by a digital calibration algorithm which corrects for mismatches between the capacitors in the switched-capacitor  MDAC. Note that the pipeline

introduces latency of four sampling periods between the input being sampled and the output appearing  at D15–D0.

While the device can handle both single-ended  or dif- ferential inputs (see the Requirements for Reference and Analog Signal Inputs section), the latter mode of operation will guarantee best THD and SFDR perfor- mance. The differential input provides  the following advantages compared to a single-ended operation:

   Twice as much signal input span

   Common-mode noise immunity

   Virtual elimination of the even-order harmonics

   Less stringent requirements on the input signal pro- cessing amplifiers

Requirements for Reference and Analog Signal Inputs

Fully differential switched-capacitor circuits (SC) are used for both the reference and analog inputs (Figure 2). This allows either single-ended  or differential signals to be used in the reference and/or analog signal paths. The signal voltage on these pins (INP, INN, RFP_, RFN_) should never exceed the analog supply rail, AVDD, nor fall below ground.

RFP_

RFN_

CM                 AVDD

AGND

INP INN

S/H

STAGE1

STAGE2           STAGE3           STAGE4

8X

ADC

ADC               MDAC

CLK DVDD

CLOCK GENERATOR

CORRECTION AND

DAV

ST_CAL

CALIBRATION LOGIC                                                                                           END_CAL

DGND

MAX1200

OE                                                                                                                                                       OUTPUT DRIVERS DRVDD

DOR                                                                       D15–D0

Figure 1. Internal Functional Diagram

8

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

RFPF INP

CM                                    RFPS RFPF

RFNF

INN RFPF

RFNF RFNS

CM

Figure 2. Simplified MDAC Architecture

Choice of Reference It is important to choose a low-noise reference such as the MAX6341, which can provide both excellent load

MAX1200

Figure 3. Equivalent Input at the Reference Pins. The sense pins should not draw any DC current.

regulation and low temperature drift. The equivalent input circuit for the reference pins is shown in Figure 3. Note that the reference pins drive approximately 1k of resistance on-chip. They also drive a switched capaci- tor of 21pF. To meet the dynamic performance, the ref- erence voltage is required to settle to 0.0015% within one clock cycle. Carefully choose an appropriate driv- ing circuit (Figure 4). The capacitors  at the reference pins (RFPF, RFNF) provide  the dynamic  charge required during each clock cycle, while the op amps ensure accuracy  of the reference signals. These capacitors must have low dielectric-absorption  charac- teristics, such as polystyrene or teflon capacitors.

The reference pins can be connected to either single- ended or differential voltages within the specified maxi- mum levels. Typically the positive reference pin (RFPF) would be driven to +4.096V, and the negative refer- ence pin (RFNF) connected to analog ground for best SNR performance. If THD performance is more impor- tant to the application than signal-to-noise ratio, choose a lower level, differential voltage such as VRFPS =

+3.5V and VRFNS = +1.5V.

There are sense pins, RFPS and RFNS, which can be

VRFP = +4.096V

5k

5k

VRFN = 0

MAX410

MAX410

MAX410

CHIP BOUNDARY

RFPF

RFPS

RFNF

RFNS

CM

used with external amplifiers to compensate for any resistive drop on these lines, internal or external to the chip. Ensure a correct reference voltage by using prop- er Kelvin connections at the sense pins.

Common-Mode Voltage The switched-capacitor  input circuit at the analog input allows signals between AGND and the analog power

supply. Since the common-mode voltage has a strong

Figure 4. Drive Circuit for Reference Pins and Common-Mode

Pin

influence on the performance  of the ADC, the best results are obtained  by choosing VCM = (VRFPS + VRFNS) / 2. This can be achieved by using a resistive divider between the two reference potentials. Figure 4 shows a typical driving circuit for good dynamic perfor- mance.

9

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

Analog Signal Conditioning For single-ended  inputs, the negative analog input pin (INN) is connected  to the common-mode voltage pin

(CM) and the positive analog input pin (INP) is connect-

ed to the input.

To take full advantage of the ADC’s superior AC perform- ance up to the Nyquist frequency, drive the chip with differential signals. In communication systems the sig- nals may inherently be available in differential mode; however medical and/or other applications  may only provide single-ended inputs. In this case, convert the single-ended signals into differential ones by using the circuit recommended in Figure 5. Use low-noise, wide- band amplifiers, such as the MAX4108, to maintain the signal purity over the full-power bandwidth  of the MAX1200 input.

Lowpass or bandpass  signals may be required  to improve the signal-to-noise and distortion of the incom- ing signal. For low-frequency signals (<100kHz), active filters may be used. For higher frequencies, passive fil- ters are more convenient.

Single-Ended to Differential

Conversion Using Transformers

An alternative single-ended  to differential-ended  con- version method is a balun transformer such as the CTX03-13675 from Coiltronics. An important benefit of these transformers is their ability to level-shift single- ended signals referred to ground on the primary side to optimum common-mode voltages on the secondary side. At frequencies below 20kHz the transformer core begins to saturate, causing odd-order harmonics.

Clock Source Requirements Pipelined ADCs typically need a 50% duty cycle clock. To avoid this constraint, the MAX1200 provides  a

divide-by-two  circuit  to relax this requirement. The

clock generator should be chosen commensurate with the frequency range, amplitude, and slew rate of the signal source. If the slew rate of the input signal is small, the jitter requirement on the clock  is relaxed. However, if the slew rate is high, the clock jitter needs to be kept at a minimum. For a full-scale amplitude input sine wave, the maximum possible signal-to-noise ratio (SNR) due completely to clock jitter is given by:

SNR                    1

V+                                                                                                                                           MAX     2

fIN     JITTER

IN

MAX4108

CM

V-

INP

For example, if fIN is 500kHz and JITTER is 10ps RMS, then the SNR limit due to jitter is about 90dB. Generating such a clock source requires a low-noise comparator and a low-phase-noise signal generator. The clock circuit shown in Figure 6 is a possible solution.

V+

V+                                                                                                                                                                                            0.1   F

MAX4108

INN

CLK_IN

V+

0.1   F

1k

5k       MAX961

CLK

V-

CM                                                                                                                               0.1  F                     1k

Figure 5. A simple circuit generates differential signals from a single-ended input referred to analog ground. The common- mode voltage at INP and INN is the same as CM.

Figure 6. Clock Generation Circuit Using Low-Noise Comparator

10

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

Calibration Procedure Since the MAX1200 is based on a pipelined  architec- ture, low-resolution quantizers (“coarse  ADCs”) are used to approximate the input signal. MDACs of the same resolution are then used to reconstruct the input signal, which is subtracted  from the input and the residue amplified by the SC gain stage. This residue is then passed on to the next stage.

The accuracy  of the MAX1200 is limited by the preci- sion of the MDAC, which is strongly dependent on the matching  of  the  capacitors  used.  The  mismatch

CLK ST_CAL

END_CAL

MIN 4 tCLK

MAX1200

~,400 CLK CYCLES

between the capacitors is determined and stored in an on-chip memory, which is later used during the conver- sion of the input signal.

During the calibration procedure,  the clock must be running continuously. ST_CAL (start of calibration) is initiated by a positive pulse with a minimum width of four clock  cycles, but not longer than about 17,400 clock cycles (Figure 8).

The ST_CAL input may be asynchronous with the clock, since it is retimed internally. With ST_CAL activated, END_CAL goes low one or two clock cycles later and remains low until the calibration is complete. During this period, the reference voltages must be stable to less than 0.01%; otherwise the calibration will be invalid. During calibration, the analog inputs INP and INN are not used; however, better performance is achieved if these inputs are static. Once END_CAL goes high (indi- cating that the calibration procedure is complete), the ADC is ready for conversion.

Once calibrated,  the MAX1200 is insensitive to small changes (±5%) in power-supply voltage or tempera- ture. Following calibration, if the temperature changes more than ±20°C, the device should be recalibrated to maintain optimum performance.

Figure 8. Timing for Start and End of Calibration

OE

tAC                                                                             tREL

D0–D15

DOR             Z                                                                                               Z Z = HIGH IMPEDANCE (THREE-STATED)

Figure 9. Timing for Bus Access and Bus Relinquish— Controlled by Output Enable (OE)

Two’s Complement Output

The MAX1200 outputs data in two’s complement for- mat. Table 1 shows how to convert the various full- scale inputs into their two’s complement output codes.

Applications Information

Signal-to-Noise Ratio (SNR) For a waveform perfectly  reconstructed  from digital samples, the theoretical maximum SNR is the ratio of

full-scale analog input (RMS value) to the RMS quanti-

N

AIN

CLK SAMPLE

CLOCK

tS

DAV

D0–D15

tCH

N+1

tOD

N+2 tCL

N+3

N+4

N+5

zation error (residual error). The ideal, theoretical mini- mum analog-to-digital noise is caused by quantization error only and results directly from the ADC’s resolution (N bits):

SNR(MAX) = (6.02 · N + 1.76)dB

In reality, there are other noise sources besides quanti- zation noise including  thermal noise, reference noise, clock jitter, etc. Therefore, SNR is computed  by taking the ratio of the RMS signal to the RMS noise which

N-3        N-2        N-1           N           N+1

includes all spectral components minus the fundamen-

tal, the first nine harmonics, and the DC offset.

Figure 7. Main Timing Diagram

11

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

Table 1. Two’s Complement Output Codes

| SCALE | OFFSET BINARY | ONE’S COMPLEMENT | TWO’S COMPLEMENT |
| --- | --- | --- | --- |
| +FSR - 1LSB | 1111 .... 1111 | 0111 .... 1111 | 0111 .... 1111 |
| +3/4FSR | 1110 .... 0000 | 0110 .... 0000 | 0110 .... 0000 |
| +1/2FSR | 1100 .... 0000 | 0100 .... 0000 | 0100 .... 0000 |
| +1/4FSR | 1010 .... 0000 | 0010 .... 0000 | 0010 .... 0000 |
| +0 | 1000 .... 0000 | 0000 .... 0000 | 0000 .... 0000 |
| -0 | —— .... —— | —— .... —— | 1111 .... 1111 |
| -1/4FSR | 0110 .... 0000 | 1110 .... 0000 | 1101 .... 1111 |
| -1/2FSR | 0100 .... 0000 | 1100 .... 0000 | 1011 .... 1111 |
| -3/4FSR | 0010 .... 0000 | 1010 .... 0000 | 1001 .... 1111 |
| -FSR +1LSB | 0000 .... 0001 | 1000 .... 0001 | 1000 .... 0000 |
| -FSR | 0000 .... 0000 | 1000 .... 0000 | —— .... —— |

Signal-to-Noise

Plus Distortion (SINAD)

SINAD is the ratio of the fundamental input frequency’s

RMS amplitude to all other ADC output signals:

SINAD (dB) = 20log [SignalRMS / (Noise + Distortion)RMS]

Effective Number of Bits (ENOB) ENOB indicates the global accuracy  of an ADC at a specific  input frequency and sampling rate. An ideal

ADC’s error consists of quantization noise only. With an

input range equal to the full-scale range of the ADC, the effective number of bits can be calculated as follows:

ENOB = (SINAD - 1.76) / 6.02

Total Harmonic Distortion (THD) THD is the ratio of the RMS sum of the first nine har- monics of the input signal to the fundamental itself. This is expressed as:

Spurious-Free

Dynamic Range (SFDR)

SFDR is the ratio of RMS amplitude of the fundamental (maximum signal component) to the RMS value of the next largest spurious component, excluding DC offset.

Grounding and Power-Supply Decoupling Grounding and power-supply decoupling strongly influ- ence the performance of the MAX1200. At 16-bit reso-

lution, unwanted digital crosstalk may couple through

the input, reference, power supply, and ground con- nections; this adversely affects the SNR or SFDR. In addition, electromagnetic interference (EMI) can either couple into or be generated by the MAX1200. Therefore, grounding  and power-supply  decoupling guidelines should be closely followed.

First, a multilayer printed circuit board (PCB) with sepa- rate ground and power-supply planes is recommend- ed. Run high-speed  signal traces directly above the ground plane. Since the MAX1200 has separate analog

THD      20log

2        2        2                2

V2     V3     V4              V9

V1

and digital ground buses (AGND and DGND respec- tively), the PCB should also have separate analog and digital ground sections connected  at only one point (star ground). Digital signals should run above the digi- tal ground plane and analog signals should run above the analog ground plane. Digital signals should be kept

where V1 is the fundamental amplitude, and V2 through V9 are the amplitudes of the 2nd through 9th-order har- monics.

far away from the sensitive analog inputs, reference input senses, common-mode input, and clock input.

12

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

The MAX1200 has three power-supply inputs: analog V DD  (AV DD ), digital  V DD  (DV DD ), and drive V DD (DRVDD). Each AVDD input should be decoupled  with parallel ceramic chip capacitors  of values 0.1µF and

0.001µF, with these capacitors  as close to the pin as possible and with the shortest possible connection to the ground plane. The DVDD pins should also have separate 0.1µF capacitors  again adjacent  to their respective pins, as should the DRVDD pin. Minimize the digital  load capacitance.  However, if the total load capacitance  on each digital output exceeds 20pF, the DRVDD decoupling  capacitor  should be increased or, preferably, digital buffers should be added.

The power-supply voltages should also be decoupled with large tantalum or electrolytic capacitors  at the point they enter the PCB. Ferrite beads with additional decoupling  capacitors  forming a pi network may improve performance.

The analog  power-supply  input  (AVDD )  for  the MAX1200 is typically +5V while the digital supplies can vary from +3V to +5V. Usually, DVDD and DRVDD pins

MAX1200

are connected to the same power supply. Note that the DVDD supply voltage must be greater than or equal to the DRVDD voltage. For example, a digital +3.3V sup- ply could be connected to DRVDD while a cleaner +5V supply  is connected  to DVDD, resulting in slightly improved performance. Alternatively, the +3.3V supply could  be  connected  to both  DRVDD and DV DD . However, the +3.3V supply must not be connected to DVDD while the +5V supply  is connected  to DRVDD (Table 2).

Table 2. Power-Supply Voltage

| Combinations AVDD (V) | DVDD (V) | DRVDD (V) | ALLOWED/ NOT ALLOWED |
| --- | --- | --- | --- |
| +5 | +5 | +5 | Allowed |
| +5 | +5 | +3.3 | Allowed |
| +5 | +3.3 | +3.3 | Allowed |
| +5 | +3.3 | +5 | Not Allowed |

Chip Information

TRANSISTOR COUNT: 56,577

SUBSTRATE CONNECTED TO AGND

13

+5V  Single-Supply, 1Msps, 16-Bit

Self-Calibrating ADC

MAX1200

MQFP44.EPS

Package Information

14

+5V  Single-Supply, 1Msps, 16-Bit

MAX1200

Self-Calibrating ADC

NOTES

15

+5V  Single-Supply, 1Msps, 16-Bit

MAX1200

Self Calibrating ADC

NOTES

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

16                                         Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  1998 Maxim Integrated  Products               Printed USA                 is a registered  trademark of Maxim Integrated  Products.