19-2852; Rev 2; 7/04

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

General Description

The MAX1226/MAX1228/MAX1230 are serial 12-bit ana- log-to-digital converters (ADCs) with an internal reference and an internal temperature sensor. These devices fea- ture on-chip FIFO, scan mode, internal clock mode, inter- nal averaging,  and AutoShutdown™. The maximum sampling rate is 300ksps using an external clock. The MAX1230 has 16 input channels, the MAX1228 has 12 input channels, and the MAX1226 has 8 input channels. All input channels are configurable  for single-ended  or differential inputs in unipolar or bipolar mode. All three devices operate from a +5V supply and contain a 10MHz SPI™/QSPI™/MICROWIRE™-compatible serial port.

The MAX1230 is available in 28-pin 5mm x 5mm thin QFN with exposed pad  and 24-pin QSOP packages. The MAX1226/MAX1228 are only available in QSOP packages.  All three devices  are specified  over the extended -40°C to +85°C temperature range.

Applications

System Supervision

Data-Acquisition Systems Industrial Control Systems Patient Monitoring

Data Logging

Instrumentation

AutoShutdown  is a trademark of Maxim Integrated  Products, Inc. SPI/QSPI are trademarks  of Motorola, Inc.

MICROWIRE is a trademark of National Semiconductor Corp.

MAX1226/MAX1228/MAX1230

Features

♦ Internal Temperature Sensor (±1°C Accuracy)

♦ 16-Entry First-In/First-Out (FIFO)

♦ Analog Multiplexer with True Differential

Track/Hold

16-, 12-, 8-Channel Single Ended

8-, 6-, 4-Channel True Differential

(Unipolar or Bipolar)

♦ Accuracy: ±1 LSB INL, ±1 LSB DNL, No Missing

Codes Over Temperature

♦ Scan Mode, Internal Averaging, and Internal Clock

♦ Low-Power Single +5V Operation

1.9mA at 300ksps

♦ Internal 4.096V Reference or External Differential

Reference

♦ 10MHz 3-Wire SPI/QSPI/MICROWIRE-Compatible

Interface

♦ Space-Saving 28-Pin 5mm x 5mm Thin QFN Package

Ordering Information

| PART | TEMP RANGE | PIN-PACKAGE |
| --- | --- | --- |
| MAX1226ACEE-T* | 0°C to +70°C | 16 QSOP |
| MAX1226AEEE-T* | -40°C to +85°C | 16 QSOP |

*Future product—contact factory for availability.

Ordering Information continued at end of data sheet.

Pin  Configurations

TOP VIEW

AIN0    1

AIN1    2

AIN2    3

AIN3    4

AIN4    5

AIN5    6

REF-/AIN6   7

CNVST/AIN7   8

MAX1226

16  EOC

15  DOUT

14  DIN

13  CS

12  SCLK

11  VDD

10  GND

9   REF+

AIN0    1

AIN1    2

AIN2    3

AIN3    4

AIN4    5

AIN5    6

AIN6    7

AIN7    8

AIN8    9

MAX1228

20  EOC

19  DOUT

18  DIN

17  CS

16  SCLK

15  VDD

14  GND

13  REF+

12  CNVST/AIN11

QSOP

Pin Configurations continued at end of data sheet.

AIN9   10

QSOP

11  REF-/AIN10

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

ABSOLUTE MAXIMUM RATINGS

VDD to GND ..............................................................-0.3V to +6V CS, SCLK, DIN, EOC, DOUT to GND.........-0.3V  to (VDD + 0.3V) AIN0–AIN13, REF-/AIN_, CNVST/AIN_,

REF+ to GND.........................................-0.3V to (VDD + 0.3V) Maximum Current into Any Pin............................................50mA Continuous Power Dissipation  (TA = +70°C)

16-Pin QSOP (derate 8.3mW/°C above +70°C)...........667mW

20-Pin QSOP (derate 9.1mW/°C above +70°C)...........727mW

24-Pin QSOP (derate 9.5mW/°C above +70°C)...........762mW

28-Pin Thin QFN 5mm x 5mm

(derate 20.8mW/°C above +70°C) ........................1667mW

Operating  Temperature  Ranges

MAX12    C   .......................................................0°C to +70°C MAX12    E    ....................................................-40°C to +85°C Storage Temperature  Range .............................-60°C to +150°C Junction Temperature ......................................................+150°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VDD = +5V ±5%, fSAMPLE = 300kHz, fSCLK = 4.8MHz (50% duty cycle),  VREF = 4.096V, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| DC ACCURACY (Note 1) |  |  |  |  |
| Resolution | RES |  | 12 | Bits |
| Integral Nonlinearity | INL |  | ±1.0 | LSB |
| Differential Nonlinearity | DNL | No missing codes over temperature | ±1.0 | LSB |
| Offset Error |  |  | ±0.5 ±4.0 | LSB |
| Gain Error |  | (Note 2) | ±0.5 ±4.0 | LSB |
| Offset Error Temperature Coefficient |  |  | ±2 | ppm/°C FSR |
| Gain Temperature Coefficient |  |  | ±0.8 | ppm/°C |
| Channel-to-Channel Offset Matching |  |  | ±0.1 | LSB |
| DYNAMIC SPECIFICATIONS (10kHz sine wave input, 4.096VP-P, 300ksps, fSCLK = 4.8MHz) |  |  |  |  |
| Signal-to-Noise Plus Distortion | SINAD |  | 70 | dB |
| Total Harmonic Distortion | THD | Up to the 5th harmonic | -82 | dBc |
| Spurious-Free Dynamic Range | SFDR |  | 80 | dBc |
| Intermodulation Distortion | IMD | fin1 = 9.9kHz, fin2 = 10.2kHz | 76 | dBc |
| Full-Power Bandwidth |  | -3dB point | 1 | MHz |
| Full-Linear Bandwidth |  | S / (N + D) > 68dB | 25 | kHz |

2

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

ELECTRICAL CHARACTERISTICS (continued)

(VDD = +5V ±5%, fSAMPLE = 300kHz, fSCLK = 4.8MHz (50% duty cycle),  VREF = 4.096V, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| CONVERSION RATE |  |  |  |  |
| Power-Up Time | tPU | External reference | 0.8 | µs |
|  |  | Internal reference (Note 3) | 65 |  |
| Acquisition Time | tACQ |  | 0.6 | µs |
| Conversion Time | tCONV | Internally clocked | 3.5 | µs |
|  |  | Externally clocked (Note 4) | 2.7 |  |
| External Clock Frequency | fSCLK | Externally clocked conversion | 0.1 4.8 | MHz |
|  |  | Data I/O | 10 |  |
| SCLK Duty Cycle |  |  | 40 60 | % |
| Aperture Delay |  |  | 30 | ns |
| Aperture Jitter |  |  | <50 | ps |
| ANALOG INPUT |  |  |  |  |
| Input Voltage Range |  | Unipolar | 0 VREF | V |
|  |  | Bipolar (Note 5) | -VREF / 2 VREF / 2 |  |
| Input Leakage Current |  | VIN = VDD | ±0.01 ±1 | µA |
| Input Capacitance |  | During acquisition time (Note 6) | 24 | pF |
| INTERNAL TEMPERATURE SENSOR |  |  |  |  |
| Measurement Error (Note 7) |  | Grade A, TA = +25C | ±0.3 | °C |
|  |  | Grade A =, TA = -20°C to +85°C | ±0.5 ±1 |  |
|  |  | Grade A, TA = TMIN to TMAX | ±0.75 ±1.5 |  |
|  |  | Grade B, TA = +25C | ±0.7 |  |
|  |  | Grade B, TA = TMIN to TMAX | ±1.2 ±3.0 |  |
| Temperature Measurement Noise |  |  | 0.1 | °CRMS |
| Temperature Resolution |  |  | 1/8 | °C |
| Power-Supply Rejection |  |  | 0.3 | °C/V |
| INTERNAL REFERENCE |  |  |  |  |
| REF Output Voltage |  |  | 4.024 4.096 4.168 | V |
| REF Temperature Coefficient | TCREF | Grade A | ±8 | ppm/°C |
|  |  | Grade B | ±30 |  |
| Output Resistance |  |  | 6.5 | kΩ |
| REF Output Noise |  |  | 200 | µVRMS |
| REF Power-Supply Rejection | PSRR |  | -70 | dB |
| EXTERNAL REFERENCE INPUT |  |  |  |  |
| REF- Input Voltage Range | VREF- |  | 0 500 | mV |
| REF+ Input Voltage Range | VREF+ |  | 1.0 VDD + 50mV | V |
| REF+ Input Current | IREF+ | VREF+ = 4.096V, fSAMPLE = 300ksps | 40 100 | µA |
|  |  | VREF+ = 4.096V, fSAMPLE = 0 | ±0.1 ±5 |  |

3

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

ELECTRICAL CHARACTERISTICS (continued)

(VDD = +5V ±5%, fSAMPLE = 300kHz, fSCLK = 4.8MHz (50% duty cycle),  VREF = 4.096V, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| DIGITAL INPUTS (SCLK, DIN, CS, CNVST) |  |  |  |  |  |
| Input Voltage Low | VIL |  |  | 0.8 | V |
| Input Voltage High | VIH |  |  | 2.0 | V |
| Input Hysteresis | VHYST |  |  | 200 | mV |
| Input Leakage Current | IIN | VIN = 0 or VDD |  | ±0.01 ±1.0 | µA |
| Input Capacitance | CIN |  |  | 15 | pF |
| DIGITAL OUTPUTS (DOUT, EOC) |  |  |  |  |  |
| Output Voltage Low | VOL | ISINK = 2mA |  | 0.4 | V |
|  |  | ISINK = 4mA |  | 0.8 |  |
| Output Voltage High | VOH | ISOURCE = 1.5mA |  | VDD - 0.5 | V |
| Tri-State Leakage Current | IL | CS = VDD |  | ±0.05 ±1 | µA |
| Tri-State Output Capacitance | COUT | CS = VDD |  | 15 | pF |
| POWER REQUIREMENTS |  |  |  |  |  |
| Supply Voltage | VDD |  |  | 4.75 5.25 | V |
| Supply Current (Note 8) | IDD | Internal reference | During temp sense | 2400 3100 | µA |
|  |  |  | fSAMPLE = 300ksps | 1950 2300 |  |
|  |  |  | fSAMPLE = 0, REF on | 1000 1350 |  |
|  |  |  | Shutdown | 0.2 5 |  |
|  |  | External reference | During temp sense | 1650 2300 |  |
|  |  |  | fSAMPLE = 300ksps | 1250 1500 |  |
|  |  |  | Shutdown | 0.2 5 |  |
| Power-Supply Rejection | PSR | VDD = 4.75V to 5.25V; full-scale input |  | ±0.2 ±1.2 | mV |

Note 1: Tested at VDD = +5V, unipolar input mode.

Note 2: Offset nulled.

Note 3: Time for reference to power up and settle to within 1 LSB.

Note 4: Conversion time is defined as the number of clock cycles multiplied  by the clock period; clock has 50% duty cycle.

Note 5: The operational  input voltage range for each individual  input of a differentially  configured pair is from GND to VDD. The operational input voltage difference is from -VREF / 2 to +VREF / 2.

Note 6: See Figure 3 (Input Equivalent Circuit) and the Sampling Error vs. Source Impedance curve in the Typical Operating

Characterisitcs section.

Note 7: Fast automated test, excludes self-heating effects.

Note 8: Supply current is specified depending on whether an internal or external reference is used for voltage conversions.

Temperature measurements always use the internal reference.

4

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

TIMING CHARACTERISTICS (Figure 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| SCLK Clock Period | tCP | Externally clocked conversion | 208 | ns |
|  |  | Data I/O | 100 |  |
| SCLK Duty Cycle | tCH |  | 40 60 | % |
| SCLK Fall to DOUT Transition | tDOT | CLOAD = 30pF | 40 | ns |
| CS Rise to DOUT Disable | tDOD | CLOAD = 30pF | 40 | ns |
| CS Fall to DOUT Enable | tDOE | CLOAD = 30pF | 40 | ns |
| DIN to SCLK Rise Setup | tDS |  | 40 | ns |
| SCLK Rise to DIN Hold | tDH |  | 0 | ns |
| CS to SCLK Rise Setup | tCSS |  | 40 | ns |
| SCLK Rise to CS Hold | tCSH |  | 0 4 | µs |
| CNVST Pulse Width | tCSW | CKSEL = 00, CKSEL = 01 (temp sense) | 40 | ns |
|  |  | CKSEL = 01 (voltage conversion) | 1.4 | µs |
| CS or CNVST Rise to EOC Low (Note 9) | t T S | Temp sense | 55 | µs |
|  |  | Voltage conversion | 7 |  |
|  | t R P | Reference power-up | 65 |  |

MAX1226/MAX1228/MAX1230

Note 9: This time is defined as the number of clock cycles needed for conversion  multiplied  by the clock period. If the internal refer- ence needs to be powered  up, the total time is additive.  The internal reference is always used for temperature  measurements.

Typical Operating Characteristics

(VDD = +5V, VREF = +4.096V, fSCLK = 4.8MHz, CLOAD = 30pF, TA = +25°C, unless otherwise noted.)

1.0

0.8

0.6

0.4

0.2

0

-0.2

-0.4

INTEGRAL NONLINEARITY (LSB)

-0.6

-0.8

-1.0

MAX1226/28/30 toc01

INTEGRAL NONLINEARITY

vs. OUTPUT  CODE

1.0

DIFFERENTIAL NONLINEARITY (LSB)

0.8

0.6

0.4

0.2

0

-0.2

-0.4

-0.6

-0.8

-1.0

MAX1226/28/30 toc02

DIFFERENTIAL NONLINEARITY

vs. OUTPUT  CODE

SINAD AMPLITUDE (dB)

100

90

80

70

60

50

40

30

20

10

0

MAX1226/28/30 toc03

SINAD vs. FREQUENCY

0            1024         2048         3072         4096

OUTPUT CODE

0            1024         2048         3072         4096

OUTPUT CODE

| 0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1              10

FREQUENCY (kHz)

100

1000

5

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

SFDR AMPLITUDE (dB)

Typical Operating Characteristics (continued)

(VDD = +5V, VREF = +4.096V, fSCLK = 4.8MHz, CLOAD = 30pF, TA = +25°C, unless otherwise noted.)

120

SFDR vs. FREQUENCY

MAX1226/28/30 toc04

1200

MAX1226/28/30 toc05

SUPPLY CURRENT vs. SAMPLING RATE

1200

MAX1226/28/30 toc06

| SUPPLY CURRENT vs. SUPPLY VOLTAGE |  |  |  |  |
| --- | --- | --- | --- | --- |

100

80

60

40

20

1000

800

SUPPLY CURRENT (A)

600

400

1150

SUPPLY CURRENT (A)

1100

0

0.1             1

10            100

| 1000 |  |  |  |  |
| --- | --- | --- | --- | --- |

200

1

10                 100

1000

1000

4.75

4.85

4.95

5.05

5.15

5.25

FREQUENCY (kHz)

SAMPLING  RATE (ksps)

MAX1226/28/30 toc08

SUPPLY VOLTAGE (V)

0.6

0.5

MAX1226/28/30 toc07

SHUTDOWN SUPPLY CURRENT

vs. SUPPLY VOLTAGE

1300

1250

SUPPLY CURRENT vs. TEMPERATURE

fS = 300ksps

0.4

0.3

1200

1150

0.2

SHUTDOWN SUPPLY CURRENT (A)

0.1

SUPPLY CURRENT (A)

1100

1050

0

4.75

4.85

4.95

5.05      5.15      5.25

1000

-40

-15         10

35         60         85

SUPPLY VOLTAGE (V)

TEMPERATURE (C)

0.6

MAX1226/28/30 toc09

SHUTDOWN SUPPLY CURRENT

vs. TEMPERATURE

4.0500

MAX1226/28/30 toc10

INTERNAL REFERENCE VOLTAGE

vs. SUPPLY VOLTAGE

0.5

4.0499

0.4

0.3

4.0498

4.0497

0.2

SHUTDOWN SUPPLY CURRENT (A)

0.1

4.0496

INTERNAL  REFERENCE VOLTAGE (V)

4.0495

0

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |

-15         10

35         60         85

| 4.0494 |  |  |  |  |
| --- | --- | --- | --- | --- |

4.75

4.85

4.95

5.05      5.15      5.25

TEMPERATURE (C)

SUPPLY VOLTAGE (V)

6

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Typical Operating Characteristics (continued)

(VDD = +5V, VREF = +4.096V, fSCLK = 4.8MHz, CLOAD = 30pF, TA = +25°C, unless otherwise noted.)

4.051

4.050

4.049

INTERNAL  REFERENCE VOLTAGE (V)

4.048

MAX1226/28/30 toc11

INTERNAL REFERENCE VOLTAGE

vs. TEMPERATURE

0.3

0.2

0.1

0

OFFSET ERROR (LSB)

-0.1

-0.2

OFFSET ERROR

vs. SUPPLY VOLTAGE

MAX1226/28/30 toc12

0.3

0.2

0.1

0

-0.1

-0.2

OFFSET ERROR

MAX1226/28/30 toc13

vs. TEMPERATURE

4.047

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |

-15         10

35         60         85

-0.3

| 4.75 |  |  |  |  |
| --- | --- | --- | --- | --- |

4.85

4.95

5.05      5.15      5.25

-0.3

-40

-15         10

OFFSET ERROR (LSB)

| 35 60 85 |  |  |  |  |
| --- | --- | --- | --- | --- |

TEMPERATURE (C)

SUPPLY VOLTAGE (V)

TEMPERATURE (C)

| 0.5 |  |  |  |  |
| --- | --- | --- | --- | --- |

GAIN ERROR vs. SUPPLY VOLTAGE

| 0.5 |  |  |  |  |
| --- | --- | --- | --- | --- |

MAX1226/28/30 toc15

GAIN ERROR vs. TEMPERATURE

MAX1226/28/30 toc14

0                                                                                                                  0

GAIN ERROR (LSB)

-0.5

GAIN ERROR (LSB)

-0.5

-1.0

-1.0

-1.5

4.75      4.85

4.95

5.05

5.15

MAX1226/28/30 toc16

5.25

-1.5

-40       -15         10

35         60         85

SUPPLY VOLTAGE (V)

TEMPERATURE (C)

1.00

TEMPERATURE  SENSOR ERROR (C)

0.75

0.50

0.25

0

TEMPERATURE SENSOR ERROR

vs. TEMPERATURE

GRADE A

SAMPLING  ERROR

MAX1226/28/30 toc17

vs. SOURCE IMPEDANCE

2

0

-2

-4

-0.25

-0.50

-0.75

SAMPLING ERROR (LSB)

-6

GRADE B

-8

-1.00

-40       -15         10

35         60         85

-10

| 0 2 |  |  |  |  |
| --- | --- | --- | --- | --- |

4           6           8          10

TEMPERATURE (C)

SOURCE IMPEDANCE (k)

7

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Pin  Description

| MAX1230 TQFN | MAX1230 QSOP | MAX1228 | MAX1226 | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- |
| 1, 17, 19, 25 | — | — | — | N.C. | No Connection. Not internally connected. |
| 2–12, 26, 27, 28 | 1–14 | — | — | AIN0–13 | Analog Inputs |
| — | — | 1–10 | — | AIN0–9 | Analog Inputs |
| — | — | — | 1–6 | AIN0–5 | Analog Inputs |
| 13 | 15 | — | — | REF-/AIN14 | Negative Input for External Differential Reference/Analog Input 14. See Table 3 for details on programming the setup register. |
| — | — | 11 | — | REF-/AIN10 | Negative Input for External Differential Reference/Analog Input 10. See Table 3 for details on programming the setup register. |
| — | — | — | 7 | REF-/AIN6 | Negative Input for External Differential Reference/Analog Input 6. See Table 3 for details on programming the setup register. |
| 14 | 16 | — | — | CNVST/ AIN15 | Active-Low Conversion Start Input/Analog Input 15. See Table 3 for details on programming the setup register. |
| — | — | 12 | — | CNVST/ AIN11 | Active-Low Conversion Start Input/Analog Input 11. See Table 3 for details on programming the setup register. |
| — | — | — | 8 | CNVST/ AIN7 | Active-Low Conversion Start Input/Analog Input 7. See Table 3 for details on programming the setup register. |
| 15 | 17 | 13 | 9 | REF+ | Positive Reference Input. Bypass to GND with a 0.1µF capacitor. |
| 16 | 18 | 14 | 10 | GND | Ground |
| 18 | 19 | 15 | 11 | VDD | Power Input. Bypass to GND with a 0.1µF capacitor. |
| 20 | 20 | 16 | 12 | SCLK | Serial Clock Input. Clocks data in and out of the serial interface. (Duty cycle must be 40% to 60%.) See Table 3 for details on programming the clock mode. |
| 21 | 21 | 17 | 13 | CS | Active-Low Chip-Select Input. When CS is low, the serial interface is enabled. When CS is high, DOUT is high impedance. |
| 22 | 22 | 18 | 14 | DIN | Serial Data Input. DIN data is latched into the serial interface on the rising edge of SCLK. |
| 23 | 23 | 19 | 15 | DOUT | Serial Data Output. Data is clocked out on the falling edge of SCLK. High impedance when CS is connected to VDD. |
| 24 | 24 | 20 | 16 | EOC | End of Conversion Output. Data is valid after EOC pulls low. |

8

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

CS

tCSS

tCH

MAX1226/MAX1228/MAX1230

tCP                                                                                            tCSH                                          tCSH

tCSS

SCLK

tDS

tDH

DIN

DOUT

tDOE

tDOT

tDOD

Figure 1. Detailed Serial-Interface Timing Diagram

CS DIN SCLK

SERIAL INTERFACE

DOUT

CNVST

OSCILLATOR

CONTROL

EOC

AIN1

AIN2

AIN15

TEMP SENSE

MUX

12-BIT SAR ADC

FIFO AND ACCUMULATOR

REF- REF+

INTERNAL REFERENCE

MAX1226

MAX1228

MAX1230

Figure 2. Functional Diagram

Detailed Description

The MAX1226/MAX1228/MAX1230 are low-power, seri- al-output, multichannel ADCs with temperature-sensing capability for temperature-control, process-control, and monitoring applications.  These 12-bit ADCs have inter- nal track and hold (T/H) circuitry  that supports single- ended  and fully differential inputs. Data is converted from an internal temperature sensor or analog voltage sources in a variety of channel and data-acquisition

configurations.  Microprocessor  (µP) control is made easy through a 3-wire SPI/QSPI/MICROWIRE-compati- ble serial interface.

Figure 2 shows a simplified functional diagram  of the MAX1226/MAX1228/MAX1230 internal architecture. The MAX1226 has eight single-ended  analog input channels or four differential channels. The MAX1228 has 12 single-ended analog input channels or six differ- ential channels. The MAX1230 has 16 single-ended analog input channels or eight differential channels.

9

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Converter Operation The MAX1226/MAX1228/MAX1230 ADCs use a fully dif- ferential, successive-approximation register (SAR) con- version technique and an on-chip T/H block to convert temperature and voltage signals into a 12-bit digital result. Both single-ended and differential configurations are supported,  with a unipolar signal range for single- ended mode and bipolar or unipolar ranges for differ- ential mode.

Input Bandwidth The ADC’s input-tracking  circuitry  has a 1MHz small- signal bandwidth,  so it is possible  to digitize  high- speed  transient events and measure periodic  signals with bandwidths exceeding the ADC’s sampling rate by using undersampling  techniques.  Anti-alias prefiltering of the input signals is necessary to avoid high-frequen- cy signals aliasing into the frequency band of interest.

Analog Input Protection Internal ESD protection  diodes clamp  all pins to VDD and GND, allowing the inputs to swing from (GND -

0.3V) to (VDD + 0.3V) without damage.  However, for accurate  conversions near full scale, the inputs must not exceed VDD by more than 50mV or be lower than GND by 50mV. If an off-channel analog input voltage exceeds the supplies, limit the input current to 2mA.

3-Wire Serial Interface The MAX1226/MAX1228/MAX1230 feature a serial interface compatible  with SPI/QSPI and MICROWIRE devices. For SPI/QSPI, ensure the CPU serial interface runs in master mode so it generates the serial clock signal. Select the SCLK frequency  of 10MHz or less, and set clock polarity (CPOL) and phase (CPHA) in the

µP control registers to the same value. The MAX1226/ MAX1228/MAX1230 operate with SCLK idling high or low, and thus operate with CPOL = CPHA = 0 or CPOL

= CPHA = 1. Set CS low to latch input data at DIN on the rising edge  of SCLK. Output  data at DOUT is updated  on the falling edge of SCLK. Bipolar true dif- ferential results and temperature  sensor results are available in two’s complement  format, while all others are in binary.

Serial communication always begins with an 8-bit input data byte (MSB first) loaded from DIN. Use a second byte, immediately following the setup byte, to write to the unipolar mode or bipolar  mode registers (see Tables 1, 3, 4, and 5). A high-to-low transition on CS ini- tiates the data input operation. The input data byte and the subsequent  data bytes are clocked  from DIN into the serial interface on the rising edge of SCLK.

Tables 1–7 detail the register descriptions. Bits 5 and 4, CKSEL1 and CKSEL0, respectively,  control the clock modes in the setup register (see Table 3). Choose between four different clock modes for various ways to start a conversion and determine whether the acquisi- tions are internally or externally timed. Select clock mode 00 to configure CNVST/AIN_ to act as a conver- sion start and use it to request the programmed  inter- nally timed conversions without tying up the serial bus. In clock mode 01, use CNVST to request conversions one channel at a time, controlling the sampling speed without tying up the serial bus. Request and start inter- nally timed conversions through the serial interface by writing to the conversion register in the default clock mode, 10. Use clock mode 11 with SCLK up to 4.8MHz for externally timed acquisitions  to achieve sampling rates up to 300ksps. Clock mode 11 disables scanning and averaging.  See Figures 4–7 for timing specifica- tions and how to begin a conversion.

These devices feature an active-low, end-of-conversion output. EOC goes low when the ADC completes  the last-requested operation and is waiting for the next input data byte (for clock modes 00 and 10). In clock mode

01, EOC goes low after the ADC completes  each requested  operation. EOC goes high when CS or CNVST goes low. EOC is always high in clock mode 11.

Single-Ended/Differential Input The MAX1226/MAX1228/MAX1230 use a fully differen- tial ADC for all conversions. The analog inputs can be configured  for either differential or single-ended  con- versions by writing to the setup register (see Table 3). Single-ended conversions are internally referenced  to GND (Figure 3).

In differential mode, the T/H samples the difference between two analog inputs, eliminating common-mode DC offsets and noise. IN+ and IN- are selected  from the following pairs: AIN0/AIN1, AIN2/AIN3, AIN4/AIN5, AIN6/AIN7, AIN8/AIN9, AIN10/AIN11, AIN12/AIN13, and AIN14/AIN15. AIN0–AIN7 are available on the MAX1226, MAX1228, and MAX1230. AIN8–AIN11 are only  available  on  the  MAX1228 and  MAX1230. AIN12–AIN15 are only available on the MAX1230. See Tables 2–5 for more details on configuring  the inputs. For the inputs that can be configured  as CNVST or an analog input, only one can be used at a time. For the inputs that can be configured  as REF- or an analog input, the REF- configuration excludes the analog input.

Unipolar/Bipolar Address the unipolar and bipolar registers through the setup register (bits 1 and 0). Program a pair of analog channels for differential operation by writing a 1 to the

10

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

AIN0-AIN15 (SINGLE ENDED); AIN0, AIN2, AIN4…AIN14 (DIFFERENTIAL)

GND (SINGLE ENDED); AIN1, AIN3, AIN5…AIN15

HOLD

REF

GND               DAC

CIN+

CIN-

COMPARATOR

+

-

MAX1226/MAX1228/MAX1230

where RIN = 1.5k, RS is the source impedance of the input signal, and tPWR = 1µs, the power-up time of the device. The varying power-up times are detailed in the explanation of the clock mode conversions.

tACQ  is never less than 1.4µs, and any source imped- ance below 300 does not significantly  affect the ADC’s AC performance. A high-impedance  source can be accommodated  either by lengthening  tACQ  or by placing a 1µF capacitor between the positive and neg- ative analog inputs.

Internal FIFO

(DIFFERENTIAL)                                  HOLD

VDD/2

Figure 3. Equivalent Input Circuit

HOLD

The MAX1226/MAX1228/MAX1230 contain a FIFO

buffer that can hold up to 16 ADC results plus one tem- perature result. This allows the ADC to handle multiple internally clocked conversions and a temperature mea- surement, without tying up the serial bus.

If the FIFO is filled and further conversions are request- ed without reading  from the FIFO, the oldest ADC

appropriate  bit of the bipolar  or unipolar register.

Unipolar mode sets the differential input range from 0 to VREF. A negative differential analog input in unipolar mode  causes the digital  output  code  to be zero. Selecting bipolar mode sets the differential input range to ±VREF / 2. The digital output code is binary in unipo- lar mode and two’s complement in bipolar mode. (See the transfer function graphs, Figures 8 and 9.)

In single-ended  mode,  the MAX1226/MAX1228/ MAX1230 always operate in unipolar mode. The analog inputs are internally referenced to GND with a full-scale input range from 0 to VREF.

True Differential Analog Input T/H The equivalent circuit of Figure 3 shows the MAX1226/MAX1228/MAX1230s’ input architecture.  In track mode, a positive input capacitor  is connected  to AIN0–AIN15 in single-ended  mode (and AIN0, AIN2, AIN4…AIN14 in differential mode). A negative input capacitor  is connected  to GND in single-ended  mode (or AIN1, AIN3, AIN5…AIN15 in differential mode). For external track-and-hold  timing, use clock  mode 01. After the T/H enters hold mode, the difference between the sampled  positive and negative input voltages is converted. The time required for the T/H to acquire an

input signal is determined  by how quickly  its input capacitance  is charged.  If the input signal’s source impedance is high, the required acquisition time length- ens. The acquisition  time, tACQ, is the maximum time needed for a signal to be acquired,  plus the power-up time. It is calculated by the following equation:

results are overwritten by the new ADC results. Each result contains 2 bytes, with the MSB preceded  by 4 leading zeros. After each falling edge of CS, the oldest available byte of data is available at DOUT, MSB first. When the FIFO is empty, DOUT is zero.

The first 2 bytes of data read out after a temperature mea- surement always contain the temperature result preceded by 4 leading zeros, MSB first. If another temperature mea- surement is performed before the first temperature result is read out, the old measurement is overwritten by the new result. Temperature results are in degrees Celsius (two’s complement) at a resolution of 1/8 of degree. See the Temperature Measurements section for details on converting the digital code to a temperature.

Internal Clock The MAX1226/MAX1228/MAX1230 operate from an inter- nal oscillator, which is accurate within 10% of the 4.4MHz nominal clock rate. The internal oscillator is active in clock modes 00, 01, and 10. Read out the data at clock speeds up to 10MHz. See Figures 4–7 for details on timing speci- fications and starting a conversion.

Applications Information

Register Descriptions The MAX1226/MAX1228/MAX1230 communicate between the internal registers and the external circuitry through the SPI-/QSPI-compatible serial interface. Table 1 details the registers and the bit names. Tables

2–7 show the various functions within the conversion register, setup register, averaging register, reset regis- ter, unipolar register, and bipolar register.

tAQC

  9 x RS

  RIN 

x 24pF  tPWR

11

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Conversion Time Calculations The conversion time for each scan is based on a num- ber of different factors: conversion time per sample, samples per result, results per scan, if a temperature measurement is requested,  and if the external refer- ence is in use.

Use the following formula to calculate the total conver- sion time for an internally timed conversion in clock modes 00 and 10 (see the Electrical Characteristics section as applicable):

total conversion time = tcnv ✕ navg x nresult + tTS + tRP

where:

tcnv = tacq(max) + tconv(max)

navg = samples per result (amount of averaging)

nresult = number of FIFO results requested; determined by number of channels being  scanned  by NSCAN1, NSCAN0

tTS = time required  for temperature measurement; set to zero if temp measurement is not requested

tRP = internal reference wake-up; set to zero if internal reference is already powered up or external reference is being used

In clock mode 01, the total conversion time depends on how long CNVST is held low or high, including any time required  to turn on the internal reference. Conversion time in externally clocked mode (CKSEL1, CKSEL0 = 11) depends  on the SCLK period and how long CS is held high between each set of eight SCLK cycles.

Conversion Register Select active analog input channels, scan modes, and a single temperature measurement per scan by writing to the conversion  register. Table 2 details channel selection, the four scan modes, and how to request a temperature measurement. Request a scan by writing to the conversion register when in clock mode 10 or 11, or by applying  a low pulse to the CNVST pin when in clock mode 00 or 01.

A conversion is not performed  if it is requested  on a channel that has been configured  as CNVST or REF-. Do not request conversions on channels 8–15 on the MAX1226 and channels 12–15 on the MAX1228. Set CHSEL3:CHSEL0 to the lower channel’s binary value. If the last two channels are configured  as a differential pair and one of them has been reconfigured as CNVST or REF-, the pair is ignored.

Table 1. Input Data Byte (MSB First)

| REGISTER NAME | BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conversion | 1 | CHSEL3 | CHSEL2 | CHSEL1 | CHSEL0 | SCAN1 | SCAN0 | TEMP |
| Setup | 0 | 1 | CKSEL1 | CKSEL0 | REFSEL1 | REFSEL0 | DIFFSEL1 | DIFFSEL0 |
| Averaging | 0 | 0 | 1 | AVGON | NAVG1 | NAVG0 | NSCAN1 | NSCAN0 |
| Reset | 0 | 0 | 0 | 1 | RESET | X | X | X |
| Unipolar mode (setup) | UCH0/1 | UCH2/3 | UCH4/5 | UCH6/7 | UCH8/9* | UCH10/11* | UCH12/13** | UCH14/15** |
| Bipolar mode (setup) | BCH0/1 | BCH1/2 | BCH4/5 | BCH6/7 | BCH8/9* | BCH10/11* | BCH12/13** | BCH14/15** |

*Unipolar/bipolar channels 8–15 are only valid on the MAX1228 and MAX1230.

**Unipolar/bipolar channels 12–15 are only valid on the MAX1230. X = Don’t care.

12

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

| Table 2. Conversion Register* BIT NAME | BIT | FUNCTION |
| --- | --- | --- |
| — | 7 (MSB) | Set to 1 to select conversion register. |
| CHSEL3 | 6 | Analog input channel select. |
| CHSEL2 | 5 | Analog input channel select. |
| CHSEL1 | 4 | Analog input channel select. |
| CHSEL0 | 3 | Analog input channel select. |
| SCAN1 | 2 | Scan mode select. |
| SCAN0 | 1 | Scan mode select. |
| TEMP | 0 (LSB) | Set to 1 to take a single temperature measurement. The first conversion result of a scan contains temperature information. |

| *See below for bit details. CHSEL3 | CHSEL2 | CHSEL1 | CHSEL0 | SELECTED CHANNEL (N) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | AIN0 |
| 0 | 0 | 0 | 1 | AIN1 |
| 0 | 0 | 1 | 0 | AIN2 |
| 0 | 0 | 1 | 1 | AIN3 |
| 0 | 1 | 0 | 0 | AIN4 |
| 0 | 1 | 0 | 1 | AIN5 |
| 0 | 1 | 1 | 0 | AIN6 |
| 0 | 1 | 1 | 1 | AIN7 |
| 1 | 0 | 0 | 0 | AIN8 |
| 1 | 0 | 0 | 1 | AIN9 |
| 1 | 0 | 1 | 0 | AIN10 |
| 1 | 0 | 1 | 1 | AIN11 |
| 1 | 1 | 0 | 0 | AIN12 |
| 1 | 1 | 0 | 1 | AIN13 |
| 1 | 1 | 1 | 0 | AIN14 |
| 1 | 1 | 1 | 1 | AIN15 |

MAX1226/MAX1228/MAX1230

Select scan mode 00 or 01 to return one result per sin- gle-ended  channel and one result per differential pair within the requested range, plus one temperature result if selected. Select scan mode 10 to scan a single input channel numerous times, depending  on NSCAN1 and NSCAN0 in the averaging register (Table 6). Select scan mode 11 to return only one result from a single channel.

Setup Register Write a byte to the setup register to configure the clock, reference, and power-down modes. Table 3 details the bits in the setup register. Bits 5 and 4 (CKSEL1 and CKSEL0) control the clock mode, acquisition and sam- pling, and the conversion start. Bits 3 and 2 (REFSEL1 and REFSEL0) control internal or external reference use. Bits 1 and 0 (DIFFSEL1 and DIFFSEL0) address  the unipolar mode and bipolar mode registers and configure the analog input channels for differential operation.

Unipolar/Bipolar Registers The final 2 bits (LSBs) of the setup register control the unipolar/bipolar mode address registers. Set bits 1 and

0 (DIFFSEL1 and DIFFSEL0) to 10 to write to the unipo- lar mode register. Set bits 1 and 0 to 11 to write to the bipolar  mode register. In both cases, the setup byte must be followed immediately by 1 byte of data written to the unipolar register or bipolar register. Hold CS low and run 16 SCLK cycles before pulling CS high. If the last 2 bits of the setup register are 00 or 01, neither the unipolar mode register nor the bipolar mode register is written. Any subsequent  byte is recognized  as a new input data byte. See Tables 4 and 5 to program  the unipolar and bipolar mode registers.

If a channel is configured as both unipolar and bipolar, the unipolar setting takes precedence.  In unipolar mode, AIN+ can exceed AIN- by up to VREF. The out- put format in unipolar mode is binary. In bipolar mode, either input can exceed the other by up to VREF / 2. The output format in bipolar mode is two's complement.

| Averaging Register Write to the averaging register to configure the ADC to average up to 32 samples for each requested result, and to independently control the number of results requested for single-channel scans. SCAN1 | SCAN0 | SCAN MODE (CHANNEL N IS SELECTED BY BITS CHSEL3–CHSEL0) |
| --- | --- | --- |
| 0 | 0 | Scans channels 0 through N. |
| 0 | 1 | Scans channels N through the highest numbered channel. |
| 1 | 0 | Scans channel N repeatedly. The averaging register sets the number of results. |
| 1 | 1 | No scan. Converts channel N once only. |

Table 2 details the four scan modes available in the con- version register. All four scan modes allow averaging as long as the AVGON bit, bit 4 in the averaging register, is set to 1. Select scan mode 10 to scan the same channel multiple times. Clock mode 11 disables averaging.

13

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Table 3. Setup Register*

| BIT NAME | BIT | FUNCTION |
| --- | --- | --- |
| — | 7 (MSB) | Set to zero to select setup register. |
| — | 6 | Set to 1 to select setup register. |
| CKSEL1 | 5 | Clock mode and CNVST configuration. Resets to 1 at power-up. |
| CKSEL0 | 4 | Clock mode and CNVST configuration. |
| REFSEL1 | 3 | Reference mode configuration. |
| REFSEL0 | 2 | Reference mode configuration. |
| DIFFSEL1 | 1 | Unipolar/bipolar mode register configuration for differential mode. |
| DIFFSEL0 | 0 (LSB) | Unipolar/bipolar mode register configuration for differential mode. |

*See below for bit details.

| CKSEL1 | CKSEL0 | CONVERSION CLOCK | ACQUISITION/SAMPLING | CNVST CONFIGURATION |
| --- | --- | --- | --- | --- |
| 0 | 0 | Internal | Internally timed | CNVST |
| 0 | 1 | Internal | Externally timed through CNVST | CNVST |
| 1 | 0 | Internal | Internally timed | AIN15/11/7 |
| 1 | 1 | External (4.8MHz max) | Externally timed through SCLK | AIN15/11/7 |

| REFSEL1 | REFSEL0 | VOLTAGE REFERENCE | AutoShutdown | REF- CONFIGURATION |
| --- | --- | --- | --- | --- |
| 0 | 0 | Internal | Reference off after scan; need wake-up delay. | AIN14/10/6 |
| 0 | 1 | External single ended | Reference off; no wake-up delay. | AIN14/10/6 |
| 1 | 0 | Internal | Reference always on; no wake-up delay. | AIN14/10/6 |
| 1 | 1 | External differential | Reference off; no wake-up delay. | REF- |

| DIFFSEL1 | DIFFSEL0 | FUNCTION |
| --- | --- | --- |
| 0 | 0 | No data follows the setup byte. Unipolar mode and bipolar mode registers remain unchanged. |
| 0 | 1 | No data follows the setup byte. Unipolar mode and bipolar mode registers remain unchanged. |
| 1 | 0 | One byte of data follows the setup byte and is written to the unipolar mode register. |
| 1 | 1 | One byte of data follows the setup byte and is written to the bipolar mode register. |

14

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

Reset Register Write to the reset register (as shown in Table 7) to clear the FIFO or to reset all registers to their default states. Set the RESET bit to 1 to reset the FIFO. Set the reset bit to zero to return the MAX1226/MAX1228/MAX1230 to its default power-up state.

Power-Up Default State The MAX1226/MAX1228/MAX1230 power up with all blocks in shutdown, including the reference. All registers power up in state 00000000, except for the setup regis- ter, which powers up in clock mode 10 (CKSEL1 = 1).

Temperature Measurements The MAX1226/MAX1228/MAX1230 perform tempera- ture measurements with an internal diode-connected transistor. The diode bias current changes  from 68µA to 4µA to produce  a temperature-dependent  bias volt- age difference. The second conversion result at 4µA is

MAX1226/MAX1228/MAX1230

subtracted  from the first at 68µA to calculate a digital value that is proportional to absolute temperature. The output data appearing  at DOUT is the above digital code minus an offset to adjust from Kelvin to Celsius.

The reference voltage used for the temperature mea- surements is derived from the internal reference source to ensure a resolution of 1/8 of a degree.

Output Data Format Figures 4–7 illustrate the conversion  timing for the MAX1226/MAX1228/MAX1230. The 12-bit conversion result is output in MSB-first format with 4 leading zeros. DIN data is latched into the serial interface on the rising edge of SCLK. Data on DOUT transitions on the falling edge of SCLK. Conversions in clock modes 00 and 01 are initiated by CNVST. Conversions in clock modes 10 and 11 are initiated by writing an input data byte to the conversion register. Data is binary for unipolar mode and two’s complement for bipolar mode.

Table 4. Unipolar Mode Register (Addressed Through Setup Register)

| BIT NAME | BIT | FUNCTION |
| --- | --- | --- |
| UCH0/1 | 7 (MSB) | Set to 1 to configure AIN0 and AIN1 for unipolar differential conversion. |
| UCH2/3 | 6 | Set to 1 to configure AIN2 and AIN3 for unipolar differential conversion. |
| UCH4/5 | 5 | Set to 1 to configure AIN4 and AIN5 for unipolar differential conversion. |
| UCH6/7 | 4 | Set to 1 to configure AIN6 and AIN7 for unipolar differential conversion. |
| UCH8/9 | 3 | Set to 1 to configure AIN8 and AIN9 for unipolar differential conversion (MAX1228/MAX1230 only). |
| UCH10/11 | 2 | Set to 1 to configure AIN10 and AIN11 for unipolar differential conversion (MAX1228/MAX1230 only). |
| UCH12/13 | 1 | Set to 1 to configure AIN12 and AIN13 for unipolar differential conversion (MAX1230 only). |
| UCH14/15 | 0 (LSB) | Set to 1 to configure AIN14 and AIN15 for unipolar differential conversion (MAX1230 only). |

Table 5. Bipolar Mode Register (Addressed Through Setup Register)

| BIT NAME | BIT | FUNCTION |
| --- | --- | --- |
| BCH0/1 | 7 (MSB) | Set to 1 to configure AIN0 and AIN1 for bipolar differential conversion. |
| BCH2/3 | 6 | Set to 1 to configure AIN2 and AIN3 for bipolar differential conversion. |
| BCH4/5 | 5 | Set to 1 to configure AIN4 and AIN5 for bipolar differential conversion. |
| BCH6/7 | 4 | Set to 1 to configure AIN6 and AIN7 for bipolar differential conversion. |
| BCH8/9 | 3 | Set to 1 to configure AIN8 and AIN9 for bipolar differential conversion (MAX1228/MAX1230 only). |
| BCH10/11 | 2 | Set to 1 to configure AIN10 and AIN11 for bipolar differential conversion (MAX1228/MAX1230 only). |
| BCH12/13 | 1 | Set to 1 to configure AIN12 and AIN13 for bipolar differential conversion (MAX1230 only). |
| BCH14/15 | 0 (LSB) | Set to 1 to configure AIN14 and AIN15 for bipolar differential conversion (MAX1230 only). |

15

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Table 6. Averaging Register*

| BIT NAME | BIT | FUNCTION |
| --- | --- | --- |
| — | 7 (MSB) | Set to zero to select averaging register. |
| — | 6 | Set to zero to select averaging register. |
| — | 5 | Set to 1 to select averaging register. |
| AVGON | 4 | Set to 1 to turn averaging on. Set to zero to turn averaging off. |
| NAVG1 | 3 | Configures the number of conversions for single-channel scans. |
| NAVG0 | 2 | Configures the number of conversions for single-channel scans. |
| NSCAN1 | 1 | Single-channel scan count. (Scan mode 10 only.) |
| NSCAN0 | 0 (LSB) | Single-channel scan count. (Scan mode 10 only.) |

*See below for bit details.

| AVGON | NAVG1 | NAVG0 | FUNCTION |
| --- | --- | --- | --- |
| 0 | x | x | Performs 1 conversion for each requested result. |
| 1 | 0 | 0 | Performs 4 conversions and returns the average for each requested result. |
| 1 | 0 | 1 | Performs 8 conversions and returns the average for each requested result. |
| 1 | 1 | 0 | Performs 16 conversions and returns the average for each requested result. |
| 1 | 1 | 1 | Performs 32 conversions and returns the average for each requested result. |

| NSCAN1 | NSCAN0 | FUNCTION (APPLIES ONLY IF SCAN MODE 10 IS SELECTED) |
| --- | --- | --- |
| 0 | 0 | Scans channel N and returns 4 results. |
| 0 | 1 | Scans channel N and returns 8 results. |
| 1 | 0 | Scans channel N and returns 12 results. |
| 1 | 1 | Scans channel N and returns 16 results. |

Table 7. Reset Register

| BIT NAME | BIT | FUNCTION |
| --- | --- | --- |
| — | 7 (MSB) | Set to zero to select reset register. |
| — | 6 | Set to zero to select reset register. |
| — | 5 | Set to zero to select reset register. |
| — | 4 | Set to 1 to select reset register. |
| RESET | 3 | Set to zero to reset all registers. Set to 1 to clear the FIFO only. |
| x | 2 | Reserved. Don’t care. |
| x | 1 | Reserved. Don’t care. |
| x | 0 (LSB) | Reserved. Don’t care. |

16

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

Internally Timed Acquisitions and

Conversions Using CNVST

Performing Conversions in Clock Mode 00

In clock mode 00, the wake up, acquisition, conversion, and shutdown sequences are initiated through CNVST and performed automatically using the internal oscilla- tor. Results are added  to the internal FIFO to be read out later. See Figure 4 for clock mode 00 timing.

Initiate a scan by setting CNVST low for at least 40ns before pulling it high again. The MAX1226/MAX1228/ MAX1230 then wake up, scan all requested channels, store the results in the FIFO, and shut down. After the scan is complete, EOC is pulled low and the results are available in the FIFO. Wait until EOC goes low before pulling CS low to communicate with the serial interface. EOC stays low until CS or CNVST is pulled low again. A temperature  measurement result, if requested,  pre- cedes all other FIFO results.

Do not initiate a second CNVST before EOC goes low;

otherwise the FIFO can become corrupted.

Externally Timed Acquisitions and

Internally Timed Conversions with CNVST

Performing Conversions in Clock Mode 01

In clock mode 01, conversions are requested one at a time using CNVST and performed automatically using the internal oscillator. See Figure 5 for clock mode 01 timing.

Setting CNVST low begins an acquisition, wakes up the ADC, and places it in track mode. Hold CNVST low for at least 1.4µs to complete the acquisition. If internal ref-

MAX1226/MAX1228/MAX1230

erence  needs  to wake up,  an additional  65µs is required for the internal reference to power up. If a tem- perature measurement is being  requested,  reference power-up and temperature measurement are internally timed. In this case, hold CNVST low for at least 40ns.

Set CNVST high to begin a conversion. After the con- version is complete,  the ADC shuts down and pulls EOC low. EOC stays low until CS or CNVST is pulled low again. Wait until EOC goes low before pulling CS or CNVST low.

If averaging is turned on, multiple CNVST pulses need to be performed before a result is written to the FIFO. Once the proper number of conversions has been per- formed to generate an averaged FIFO result, as speci- fied by the averaging register, the scan logic automatically switches the analog input multiplexer to the next-requested channel. If a temperature measure- ment is programmed, it is performed after the first rising edge of CNVST following the input data byte written to the conversion register. The result is available on DOUT once EOC has been pulled low.

Internally Timed Acquisitions and

Conversions Using the Serial Interface

Performing Conversions in Clock Mode 10

In clock mode 10, the wake-up, acquisition, conversion, and shutdown sequences  are initiated by writing an input data byte to the conversion register, and are per- formed automatically using the internal oscillator. This is the default clock mode upon power-up. See Figure 6 for clock mode 10 timing.

CNVST

(UP TO 514 INTERNALLY CLOCKED ACQUISITIONS AND CONVERSIONS)

CS

SCLK

DOUT

MSB1                                                                                             LSB1                                  MSB2

EOC

SET CNVST LOW FOR AT LEAST 40ns TO BEGIN A CONVERSION.

Figure 4. Clock Mode 00

17

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

CNVST

(ACQUISITION1)                  (ACQUISITION2) CS

(CONVERSION2)

SCLK

(CONVERSION1)

DOUT

MSB1                                                                                              LSB1                                 MSB2

EOC

REQUEST MULTIPLE  CONVERSIONS BY SETTING CNVST LOW FOR EACH CONVERSION.

Figure 5. Clock Mode 01

DIN

(CONVERSION BYTE)

(UP TO 514 INTERNALLY CLOCKED ACQUISITIONS AND CONVERSIONS)

CS

MAX1226/MAX1228/MAX1230

SCLK

DOUT

MSB1                                                                                     LSB1                                 MSB2

EOC

THE CONVERSION BYTE BEGINS THE ACQUISITION. CNVST IS NOT REQUIRED.

Figure 6. Clock Mode 10

Initiate a scan by writing a byte to the conversion regis- ter. The MAX1226/MAX1228/MAX1230 then power up, scan all requested  channels, store the results in the FIFO, and shut down. After the scan is complete, EOC is pulled low and the results are available in the FIFO. If a temperature measurement is requested, the tempera- ture result precedes  all other FIFO results. EOC stays low until CS is pulled low again.

Externally Clocked Acquisitions and

Conversions Using the Serial Interface

Performing Conversions in Clock Mode 11

In clock mode 11, acquisitions and conversions are ini- tiated by writing to the conversion register and are per- formed one at a time using the SCLK as the conversion clock.  Scanning and averaging  are disabled,  and the conversion result is available at DOUT during the con- version. See Figure 7 for clock mode 11 timing.

18

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

| DIN |  |  |  |  |  |  |  | (CONVERSION BYTE) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(ACQUISITION1)

(CONVERSION1)

(ACQUISITION2)

CS

SCLK

DOUT

MSB1                                                                                LSB1                                               MSB2

EOC

EXTERNALLY TIMED ACQUISITION, SAMPLING  AND CONVERSION WITHOUT CNVST.

Figure 7. Clock Mode 11

Initiate a conversion by writing a byte to the conversion register followed by 16 SCLK cycles. If CS is pulsed high between the eighth and ninth cycles,  the pulse width must be less than 100µs. To continuously con- vert at 16 cycles  per conversion, alternate 1 byte of zeros between each conversion byte.

If reference mode 00 is requested, or if an external ref- erence is selected but a temperature measurement is being requested,  wait 65µs with CS high after writing the conversion byte to extend the acquisition and allow the internal reference to power up. To perform a tem- perature measurement, write 24 bytes (192 cycles) of zeros after the conversion byte. The temperature result appears  on DOUT during  the last 2 bytes  of the

192 cycles.

Partial Reads and Partial Writes If the first byte of an entry in the FIFO is partially read (CS is pulled high after fewer than eight SCLK cycles), the second byte of data that is read out contains the next 8 bits (not b7–b0). The remaining bits are lost for that entry. If the first byte of an entry in the FIFO is read out fully, but the second byte is read out partially, the rest of the entry is lost. The remaining data in the FIFO is uncorrupted and can be read out normally after tak- ing CS low again, as long as the 4 leading bits (nor- mally zeros) are ignored.  Internal registers that are written partially through the SPI contain new values, starting at the MSB up to the point that the partial write is stopped.  The part of the register that is not written contains previously written values. If CS is pulled  low before EOC goes low, a conversion cannot be com- pleted and the FIFO is corrupted.

MAX1226/MAX1228/MAX1230

Transfer Function Figure 8 shows the unipolar transfer function for single- ended or differential inputs. Figure 9 shows the bipolar transfer function for differential inputs. Code transitions occur halfway between successive-integer LSB values. Output coding is binary, with 1 LSB = VREF / 4096V for unipolar and bipolar operation, and 1 LSB = 0.125°C for temperature measurements.

Layout, Grounding, and Bypassing For best performance, use PC boards. Do not use wire- wrap boards. Board layout should ensure that digital and analog signal lines are separated from each other. Do not run analog and digital (especially clock) signals parallel to one another or run digital lines underneath the MAX1226/MAX1228/MAX1230 package.  High-frequen- cy noise in the VDD power supply can affect perfor- mance. Bypass the VDD supply with a 0.1µF capacitor to GND, close to the VDD pin. Minimize capacitor  lead lengths for best supply-noise rejection. If the power sup- ply is very noisy, connect a 10 resistor in series with the supply to improve power-supply  filtering. For the QFN package, connect its exposed pad to ground.

Definitions

Integral Nonlinearity Integral nonlinearity (INL) is the deviation of the values on an actual transfer function from a straight line. This straight line can be either a best-straight-line  fit or a line drawn between the end points of the transfer func- tion, once offset and gain errors have been nullified. INL for the MAX1226/MAX1228/MAX1230 is measured using the end-point method.

19

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

OUTPUT CODE

OUTPUT CODE

11 . . . 111

11 . . . 110

11 . . . 101

FULL-SCALE TRANSITION

FS = VREF + VCOM

011 . . . 111

011 . . . 110

000 . . . 010

000 . . . 001

000 . . . 000

FS =  VREF   + VCOM

2

ZS = COM

-FS =  -VREF + VCOM

2

1 LSB =   VREF

00 . . . 011

00 . . . 010

00 . . . 001

00 . . . 000

0

1     2     3

ZS = VCOM

1 LSB =  VREF

4096

FS

111 . . . 110

111 . . . 101

100 . . . 001

100 . . . 000

- FS                                       COM*

+FS - 1 LSB

(COM)

INPUT VOLTAGE (LSB)

FS - 3/2 LSB

*VCOM  VREF / 2

INPUT VOLTAGE (LSB)

Figure 8. Unipolar Transfer Function, Full Scale (FS) = VREF                   Figure 9. Bipolar Transfer Function, Full Scale (±FS) = ±VREF / 2

MAX1226/MAX1228/MAX1230

Differential Nonlinearity Differential nonlinearity (DNL) is the difference between an actual step width and the ideal value of 1 LSB. A DNL error specification  of less than 1 LSB guarantees no missing codes and a monotonic transfer function.

Aperture Jitter Aperture jitter (tAJ) is the sample-to-sample variation in the time between the samples.

Aperture Delay Aperture delay (tAD) is the time between the rising edge  of the sampling clock  and the instant when an actual sample is taken.

Signal-to-Noise Ratio For a waveform perfectly  reconstructed  from digital samples, signal-to-noise ratio (SNR) is the ratio of the full-scale analog input (RMS value) to the RMS quanti- zation error (residual error). The ideal, theoretical mini- mum analog-to-digital  noise is caused by quantization error only and results directly from the ADC’s resolution (N bits):

In reality, there are other noise sources besides quanti- zation noise, including  thermal noise, reference noise, clock jitter, etc. Therefore, SNR is calculated by taking the ratio of the RMS signal to the RMS noise, which includes all spectral components minus the fundamen- tal, the first five harmonics, and the DC offset.

Signal-to-Noise Plus Distortion Signal-to-noise plus distortion (SINAD) is the ratio of the fundamental input frequency’s  RMS amplitude  to the RMS equivalent of all other ADC output signals:

SINAD (dB) = 20 x log (SignalRMS / NoiseRMS)

Effective Number of Bits Effective number of bits (ENOB) indicates the global accuracy  of an ADC at a specific  input frequency and sampling rate. An ideal ADC error consists of quantiza- tion noise only. With an input range equal to the full- scale range of the ADC, calculate the effective number of bits as follows:

ENOB = (SINAD - 1.76) / 6.02

SNR = (6.02 x N + 1.76)dB

20

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

Total Harmonic Distortion Total harmonic distortion (THD) is the ratio of the RMS sum of the first five harmonics of the input signal to the fundamental itself. This is expressed as:

MAX1226/MAX1228/MAX1230

Ordering Information (continued) PART TEMP RANGE PIN-PACKAGE

MAX1226BCEE-T 0°C to +70°C 16 QSOP

MAX1226BEEE-T -40°C to +85°C 16 QSOP

MAX1228ACEP-T* 0°C to +70°C 20 QSOP

MAX1228AEEP-T* -40°C to +85°C 20 QSOP

MAX1228BCEP-T 0°C to +70°C 20 QSOP

MAX1228BEEP-T -40°C to +85°C 20 QSOP

MAX1230ACEG-T 0°C to +70°C 24 QSOP

MAX1230AEEG-T* -40°C to +85°C 24 QSOP

MAX1230BCEG-T 0°C to +70°C 24 QSOP

MAX1230BEEG-T -40°C to +85°C 24 QSOP

MAX1230BCTI-T 0°C to +70°C 28 TQFN

MAX1230BETI-T -40°C to +85°C 28 TQFN

THD 

20 x log  





V 2       V 2





2            3

V 2       V 2





4            5

/ V  

1

where V1 is the fundamental amplitude, and V2–V5 are the amplitudes of the first five harmonics.

Spurious-Free Dynamic Range Spurious-free dynamic range (SFDR) is the ratio of the RMS amplitude  of the fundamental (maximum signal component) to the RMS value of the next-largest distor- tion component.

*Future product—contact factory for availability.

Chip Information

TRANSISTOR COUNT: 30,889

28    AIN2

27    AIN1

26

25

24

23

22

PROCESS: BiCMOS

Pin  Configurations (continued)

TOP VIEW

AIN0    1

24    EOC

AIN1    2

AIN2    3

AIN3    4

AIN4    5

AIN5    6

AIN6    7

AIN0

N.C. EOC DOUT DIN

AIN7    8

AIN8    9

AIN9  10

AIN10  11

AIN11  12

MAX1230

QSOP

23    DOUT

22   DIN

21   CS

20   SCLK

19    VDD

18    GND

17    REF+

16   CNVST/AIN15

15   REF-/AIN14

14   AIN13

13   AIN12

N.C.       1

AIN3       2

AIN4       3

AIN5       4

AIN6       5

AIN7       6

8

9

AIN11

AIN12

10

11

AIN8       7

MAX1230

AIN9

AIN10

AIN13

REF-/AIN14

CNVST/AIN15

TQFN

21      CS

12

13

14

20      SCLK

19      N.C.

18      VDD

17      N.C.

16      GND

15      REF+

21

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

QSOP.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) |  |  |
| --- | --- | --- |

|  | PACKAGE OUTLINE, QSOP .150", .025" LEAD PITCH |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 21-0055 | E | 1 1 |

22

12-Bit 300ksps ADCs with FIFO, Temp Sensor, Internal Reference

MAX1226/MAX1228/MAX1230

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

PIN # 1

I.D.

D

D/2

0.15  C  A

0.15  C  B

D2

CL

D2/2 k

b         0.10 M   C   A   B

QFN THIN.EPS

PIN # 1 I.D.

0.35x45∞

E/2

E

(NE-1) X  e

E2/2

CL      E2

k

L

DETAIL A                               e

(ND-1) X  e

DETAIL B

e

L1     L

CL                                                                                CL

L                                                                                                                     L

e                                                               e

0.10   C

A

0.08   C

C                                                                     A1    A3

PACKAGE OUTLINE

16, 20, 28, 32, 40L, THIN QFN, 5x5x0.8mm

21-0140

| E 1 2 EXPOSED PAD VARIATIONS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PKG. CODES | D2 |  |  | E2 |  |  | DOWN BONDS ALLOWED |
|  | MIN. | NOM. | MAX. | MIN. | NOM. | MAX. |  |
| T1655-1 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | NO |
| T1655-2 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | YES |
| T2055-2 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | NO |
| T2055-3 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | YES |
| T2055-4 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | NO |
| T2855-1 | 3.15 | 3.25 | 3.35 | 3.15 | 3.25 | 3.35 | NO |
| T2855-2 | 2.60 | 2.70 | 2.80 | 2.60 | 2.70 | 2.80 | NO |
| T2855-3 | 3.15 | 3.25 | 3.35 | 3.15 | 3.25 | 3.35 | YES |
| T2855-4 | 2.60 | 2.70 | 2.80 | 2.60 | 2.70 | 2.80 | YES |
| T2855-5 | 2.60 | 2.70 | 2.80 | 2.60 | 2.70 | 2.80 | NO |
| T2855-6 | 3.15 | 3.25 | 3.35 | 3.15 | 3.25 | 3.35 | NO |
| T2855-7 | 2.60 | 2.70 | 2.80 | 2.60 | 2.70 | 2.80 | YES |
| T3255-2 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | NO |
| T3255-3 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | YES |
| T3255-4 | 3.00 | 3.10 | 3.20 | 3.00 | 3.10 | 3.20 | NO |
| T4055-1 | 3.20 | 3.30 | 3.40 | 3.20 | 3.30 | 3.40 | YES |

| COMMON DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PKG. | 16L 5x5 |  |  | 20L 5x5 |  |  | 28L 5x5 |  |  | 32L 5x5 |  |  | 40L 5x5 |  |  |
| SYMBOL | MIN. | NOM. | MAX. | MIN. | NOM. | MAX. | MIN. | NOM. | MAX. | MIN. | NOM. | MAX. | MIN. | NOM. | MAX. |
| A | 0.70 | 0.75 | 0.80 | 0.70 | 0.75 | 0.80 | 0.70 | 0.75 | 0.80 | 0.70 | 0.75 | 0.80 | 0.70 | 0.75 | 0.80 |
| A1 | 0 | 0.02 | 0.05 | 0 | 0.02 | 0.05 | 0 | 0.02 | 0.05 | 0 | 0.02 | 0.05 | 0 | - | 0.05 |
| A3 | 0.20 REF. |  |  | 0.20 REF. |  |  | 0.20 REF. |  |  | 0.20 REF. |  |  | 0.20 REF. |  |  |
| b | 0.25 | 0.30 | 0.35 | 0.25 | 0.30 | 0.35 | 0.20 | 0.25 | 0.30 | 0.20 | 0.25 | 0.30 | 0.15 | 0.20 | 0.25 |
| D | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 |
| E | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 | 4.90 | 5.00 | 5.10 |
| e | 0.80 BSC. |  |  | 0.65 BSC. |  |  | 0.50 BSC. |  |  | 0.50 BSC. |  |  | 0.40 BSC. |  |  |
| k | 0.25 | - | - | 0.25 | - | - | 0.25 | - | - | 0.25 | - | - | 0.25 | 0.35 | 0.45 |
| L | 0.30 | 0.40 | 0.50 | 0.45 | 0.55 | 0.65 | 0.45 | 0.55 | 0.65 | 0.30 | 0.40 | 0.50 | 0.40 | 0.50 | 0.60 |
| L1 | - | - | - | - | - | - | - | - | - | - | - | - | 0.30 | 0.40 | 0.50 |
| N | 16 |  |  | 20 |  |  | 28 |  |  | 32 |  |  | 40 |  |  |
| ND | 4 |  |  | 5 |  |  | 7 |  |  | 8 |  |  | 10 |  |  |
| NE | 4 |  |  | 5 |  |  | 7 |  |  | 8 |  |  | 10 |  |  |
| JEDEC | WHHB |  |  | WHHC |  |  | WHHD-1 |  |  | WHHD-2 |  |  | - |  |  |

NOTES:

1.   DIMENSIONING & TOLERANCING CONFORM TO ASME Y14.5M-1994.

2.   ALL DIMENSIONS ARE IN MILLIMETERS. ANGLES ARE IN DEGREES.

3.   N IS THE TOTAL NUMBER OF TERMINALS.

4.   THE TERMINAL #1 IDENTIFIER AND TERMINAL NUMBERING CONVENTION SHALL CONFORM TO JESD 95-1

SPP-012.  DETAILS OF TERMINAL #1 IDENTIFIER ARE OPTIONAL, BUT MUST BE LOCATED WITHIN THE ZONE INDICATED. THE TERMINAL #1 IDENTIFIER MAY BE EITHER A MOLD OR MARKED FEATURE.

5.   DIMENSION b APPLIES TO METALLIZED TERMINAL AND IS MEASURED BETWEEN 0.25 mm AND 0.30 mm

FROM TERMINAL TIP.

6.   ND AND NE REFER TO THE NUMBER OF TERMINALS ON EACH D AND E SIDE RESPECTIVELY.

7.   DEPOPULATION IS POSSIBLE IN A SYMMETRICAL FASHION.

8.   COPLANARITY APPLIES TO THE EXPOSED HEAT SINK SLUG AS WELL AS THE TERMINALS.

9.   DRAWING CONFORMS TO JEDEC MO220, EXCEPT EXPOSED PAD DIMENSION FOR T2855-1, T2855-3 AND T2855-6.

10.   WARPAGE SHALL NOT EXCEED 0.10 mm.

PACKAGE OUTLINE

16, 20, 28, 32, 40L, THIN QFN, 5x5x0.8mm

2

21-0140                E      2

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600                                23

©  2004 Maxim Integrated Products             Printed USA                                is a registered trademark of Maxim Integrated Products.