19-1387; Rev 2; 10/05

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs in 8-Pin µMAX

General Description

The MAX144/MAX145 low-power,  12-bit analog-to- digital converters (ADCs) are available in 8-pin µMAX® and DIP packages. Both devices operate with a single

+2.7V to +5.25V supply and feature a 7.4µs succes- sive-approximation  ADC, automatic power-down,  fast wake-up (2.5µs), an on-chip clock, and a high-speed,

3-wire serial interface.

Power consumption is only 3.2mW (VDD = +3.6V) at the maximum sampling rate of 108ksps. At slower through- put rates, the automatic  shutdown  (0.2µA) further reduces power consumption.

The MAX144 provides 2-channel, single-ended  opera- tion and accepts  input signals from 0 to VREF. The MAX145 accepts  pseudo-differential  inputs ranging from 0 to VREF. An external clock  accesses  data- through the 3-wire serial interface, which is SPI™, QSPI™, and MICROWIRE™-compatible.

Excellent dynamic  performance  and low power, com- bined with ease of use and small package  size, make these converters ideal for battery-powered  and data- acquisition  applications,  or for other circuits  with demanding  power-consumption  and space  require- ments. For pin-compatible  10-bit  ADCs, see the MAX157 and MAX159 data sheets.

Applications

MAX144/MAX145

Features

♦ Single-Supply Operation (+2.7V to +5.25V)

♦ Two Single-Ended Channels (MAX144) One

Pseudo-Differential Channel (MAX145)

♦ Low Power

0.9mA (108ksps, +3V Supply)

100µA (10ksps, +3V Supply)

10µA (1ksps, +3V Supply)

0.2µA (Power-Down Mode)

♦ Internal Track/Hold

♦ 108ksps Sampling Rate

♦ SPI/QSPI/MICROWIRE-Compatible 3-Wire Serial

Interface

♦ Space-Saving 8-Pin µMAX Package

♦ Pin-Compatible 10-Bit Versions Available

Ordering Information PART TEMP PIN- INL PKG RANGE PACKAGE (LSB) CODE

MAX144ACUA 0°C to 8 µMAX ±0.5 U8-1 +70°C

MAX144BCUA 0°C to 8 µMAX ±1 U8-1 +70°C MAX144ACPA 0°C to 8 Plastic DIP ±0.5 P8-1 +70°C

MAX144BCPA 0°C to 8 Plastic DIP ±1 P8-1 +70°C MAX144BC/D 0°C to Dice* ±1 — +70°C

MAX144AEUA -40°C to 8 µMAX ±0.5 U8-1 +85°C

MAX144BEUA -40°C to 8 µMAX ±1 U8-1 +85°C

MAX144AEPA -40°C to 8 Plastic DIP ±0.5 P8-1 +85°C

MAX144BEPA -40°C to 8 Plastic DIP ±1 P8-1 +85°C

MAX144AMJA -55°C to 8 CERDIP** ±0.5 J8-2 +125°C

MAX144BMJA -55°C to 8 CERDIP** ±1 J8-2 +125°C

Battery-Powered Systems Portable Data Logging Isolated Data Acquisition Process-Control Monitoring

Instrumentation Test Equipment Medical Instruments System Supervision

Pin  Configuration

TOP VIEW

| VDD | MAX144 MAX145 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

CH0 (CH+) CH1 (CH-)

GND

SCLK DOUT CS/SHDN

REF

( ) ARE FOR MAX145 ONLY

µMAX/DIP

*Dice are specified at TA = +25°C, DC parameters only.

**Contact factory for availability.

µMAX is a registered trademark of Maxim Integrated Products, Inc. SPI and QSPI are trademarks of Motorola, Inc.

MICROWIRE is a trademark of National Semiconductor Corp.

Ordering Information continued at end of data sheet.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

ABSOLUTE MAXIMUM RATINGS

VDD to GND ..............................................................-0.3V to +6V CH0, CH1 (CH+, CH-) to GND . ................ -0.3V to (VDD + 0.3V) REF to GND .............................................. -0.3V to (VDD + 0.3V) Digital Inputs to GND. ............................................. -0.3V to +6V DOUT to GND............................................ -0.3V to (VDD + 0.3V) DOUT Sink Current  ........................................................... 25mA Continuous Power Dissipation (TA = +70°C)

µMAX (derate 4.1mW/°C above +70°C)  .................... 330mW

Plastic DIP (derate 9.09mW/°C above +70°C) ............727mW CERDIP (derate 8.00mW/°C above +70°C) . .............. 640mW

Operating Temperature Ranges (TA)

MAX144/MAX145_C_A .......................................0°C to +70°C

MAX144/MAX145_E_A. ...................................-40°C to +85°C MAX144/MAX145_M_A ................................  -55°C to +125°C Storage Temperature Range .............................-65°C  to +150°C Lead Temperature (soldering, 10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated in the operational sections of the specifications is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VDD =  +2.7V  to +5.25V,  VREF =  2.5V, 0.1µF capacitor  at REF, fSCLK =  2.17MHz,  16 clocks/conversion cycle  (108ksps), CH- = GND for MAX145, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| DC ACCURACY (Note 1) |  |  |  |  |
| Resolution | RES |  | 12 | Bits |
| Relative Accuracy (Note 2) | INL | MAX14_A | ±0.5 | LSB |
|  |  | MAX14_B | ±1 |  |
| Differential Nonlinearity | DNL | No missing codes over temperature | ±0.75 | LSB |
| Offset Error |  |  | ±3 | LSB |
| Gain Error |  | (Note 3) | ±3 | LSB |
| Gain Temperature Coefficient |  |  | ±0.8 | ppm/°C |
| Channel-to-Channel Offset Matching |  |  | ±0.05 | LSB |
| Channel-to-Channel Gain Matching |  |  | ±0.05 | LSB |
| DYNAMIC SPECIFICATIONS (fI | (sine-wave) | = 10kHz, VIN = 2.5Vp-p, 108ksps, fSCLK = 2 | .17MHz, CH- = GND for MAX145) |  |
| Signal-to-Noise Plus Distortion Ratio | SINAD |  | 70 | dB |
| Total Harmonic Distortion (including 5th-order harmonic) | THD |  | -80 | dB |
| Spurious-Free Dynamic Range | SFDR |  | 80 | dB |
| Channel-to-Channel Crosstalk |  | fIN = 65kHz, VIN = 2.5Vp-p (Note 4) | -85 | dB |
| Small-Signal Bandwidth |  | -3dB rolloff | 2.25 | MHz |
| Full-Power Bandwidth |  |  | 1.0 | MHz |
| CONVERSION RATE |  |  |  |  |
| Conversion Time (Note 5) | tCONV | External clock, fSCLK = 2.17MHz, 16 clocks/conversion cycle | 7.4 | µs |
|  |  | Internal clock | 5 7 |  |
| T/H Acquisition Time | tACQ |  | 2.5 | µs |
| Aperture Delay |  |  | 25 | ns |
| Aperture Jitter |  |  | <50 | ps |
| Serial Clock Frequency | fSCLK | External clock mode | 0.1 2.17 | MHz |
|  |  | Internal clock mode, for data transfer only | 0 5 |  |

2

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

ELECTRICAL CHARACTERISTICS (continued)

(VDD =  +2.7V  to +5.25V,  VREF =  2.5V, 0.1µF capacitor  at REF, fSCLK =  2.17MHz,  16 clocks/conversion cycle  (108ksps), CH- = GND for MAX145, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| ANALOG INPUTS |  |  |  |  |
| Analog Input Voltage Range | VIN | (Note 6) | 0 VREF | V |
| Multiplexer Leakage Current |  | On/off leakage current, VIN = 0 to VDD | ±0.01 ±1 | µA |
| Input Capacitance | CIN |  | 16 | pF |
| EXTERNAL REFERENCE |  |  |  |  |
| Input Voltage Range | VREF | (Note 7) | 0 VDD + 50mV | V |
| Input Current |  | VREF = 2.5V | 100 140 | µA |
| Input Resistance |  |  | 18 25 | kΩ |
| Shutdown REF Input Current |  |  | 0.01 10 | µA |
| DIGITAL INPUTS (CS/SHDN) A | D OUTPUT | (DOUT) |  |  |
| Input High Voltage | VIH | VDD ≤ 3.6V | 2.0 | V |
|  |  | VDD > 3.6V | 3.0 |  |
| Input Low Voltage | VIL |  | 0.8 | V |
| Input Hysteresis | VHYS |  | 0.2 | V |
| Input Leakage Current | IIN | VIN = 0 or VDD | ±1 | µA |
| Input Capacitance | CIN | (Note 8) | 15 | pF |
| Output Low Voltage | VOL | ISINK = 5mA | 0.4 | V |
|  |  | ISINK = 16mA | 0.5 |  |
| Output High Voltage | VOH | ISOURCE = 0.5mA | VDD - 0.5 | V |
| Three-State Output Leakage Current |  | CS/SHDN = VDD | ±10 | µA |
| Three-State Output Capacitance | COUT | CS/SHDN = VDD (Note 8) | 15 | pF |
| POWER REQUIREMENTS |  |  |  |  |
| Positive Supply Voltage | VDD |  | 2.7 5.25 | V |
| Positive Supply Current | IDD | Operating mode | 0.9 2.0 | mA |
|  |  | Shutdown, CS/SHDN = GND | 0.2 5 | µA |
| Power-Supply Rejection | PSR | VDD = 2.7V to 5.25V, VREF = 2.5V, full-scale input (Note 9) | ±0.15 | mV |

3

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs in 8-Pin µMAX

MAX144/MAX145

TIMING CHARACTERISTICS (Figure 7)

(VDD = +2.7V to +5.25V, VREF = 2.5V, 0.1µF capacitor at REF, fSCLK = 2.17MHz, 16 clocks/conversion cycle (108ksps),  CH-= GND

for MAX145, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Wake-Up Time |  |  | 2.5 | µs |
| CS/SHDN Fall to Output Enable | tDV | CL = 100pF | 120 | ns |
| CS /SHDN Rise to Output Disable | tTR | CL = 100pF, Figure 1 | 120 | ns |
| SCLK Fall to Output Data Valid | tDO | CL = 100pF, Figure 1 | 20 120 | ns |
| SCLK Clock Frequency | fSCLK | External clock | 0.1 2.17 | MHz |
|  |  | Internal clock, SCLK for data transfer only | 0 5 |  |
| SCLK Pulse Width High | tCH | External clock | 215 | ns |
|  |  | Internal clock, SCLK for data transfer only (Note 8) | 50 |  |
| SCLK Pulse Width Low | tCL | External clock | 215 | ns |
|  |  | Internal clock, SCLK for data transfer only (Note 8) | 50 |  |
| SCLK to CS /SHDN Setup | tSCLKS |  | 60 | ns |
| CS /SHDN Pulse Width | tCS |  | 60 | ns |

Note 1: Tested at VDD = +2.7V.

Note 2: Relative accuracy is the deviation of the analog value at any code from its theoretical  value after full-scale range has been calibrated.

Note 3: Offset nulled.

Note 4:  "On" channel is grounded; sine wave applied  to "off" channel (MAX144 only).

Note 5:  Conversion time is defined as the number of clock cycles times the clock period; clock has 50% duty cycle.

Note 6:  The common-mode range for the analog inputs is from GND to VDD (MAX145 only).

Note 7:  ADC performance is limited by the converter’s  noise floor, typically  300µVp-p.

Note 8:  Guaranteed  by design. Not subject to production testing.

Note 9:  Measured  as VFS(2.7V) -VFS(5.25V).

4

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

Typical Operating Characteristics (VDD = +3.0V, VREF = 2.5V, 0.1µF at REF, fSCLK = 2.17MHz, 16 clocks/conversion cycle (108ksps), CH- = GND for MAX145, TA = +25°C, unless otherwise noted.)

1500

1300

1100

SUPPLY CURRENT ( A)

900

SUPPLY CURRENT

vs. SUPPLY VOLTAGE

MAX144/5-01

1500

SUPPLY CURRENT ( A)

1250

1000

SUPPLY CURRENT

MAX144/5-02

vs. TEMPERATURE

10,000

1000

100

SUPPLY CURRENT ( A)

10

SUPPLY CURRENT

vs. SAMPLING RATE

VDD = VREF

CL = 20pF

MAX144/5-03

CODE = 101010100000

700

750

1

500

| 2.5 3.0 3.5 4.0 4.5 VR RL CL | EF = = = 5 | VDD 0pF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO | DE | = 10 | 101 | 0100 | 000 |  |  |  |  |  |  |

5.0

5.5

500

MAX144/5-04

-60 -40 -20  0     20    40

| 60 80 100 120 140 VRE RL CL | F = V = = 50p | DD F |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO | DE = 1 | 010 | 1010 | 0000 |  |  |  |  |  |

0.1

0.1

1          10

100        1k         10k

100k

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

SAMPLING  RATE (sps)

1000

SHUTDOWN CURRENT

vs. SUPPLY VOLTAGE

| 1000 VRE | F = VD | D |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SHUTDOWN CURRENT

vs. TEMPERATURE

MAX144/5-05

1.0

OFFSET ERROR

MAX144/5-06

vs. SUPPLY VOLTAGE

SHUTDOWN CURRENT (nA)

800

SHUTDOWN CURRENT (nA)

800

0.8

600

600

0.6

400

400

OFFSET ERROR (LSB)

0.4

200

200

0.2

0

2.5        3.0        3.5        4.0        4.5

5.0

| 5.5 VR | EF = | VDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

0

-60 -40 -20  0

20    40

60    80   100  120  140

0

2.5        3.0        3.5        4.0         4.5         5.0        5.5

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

| SUPPLY VOLTAGE (V) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 1.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

0.9

0.8

OFFSET ERROR (LSB)

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

OFFSET ERROR

MAX144/5-07

vs. TEMPERATURE

| 0.5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

0.4

0.3

0.2

0.1

0

-0.1

GAIN ERROR (LSB)

-0.2

-0.3

-0.4

-0.5

GAIN ERROR

MAX144/5-08

vs. SUPPLY VOLTAGE

| 0.5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

0.4

0.3

GAIN ERROR (LSB)

0.2

0.1

0

-0.1

-0.2

-0.3

-0.4

-0.5

GAIN ERROR

MAX144/5-09

vs. TEMPERATURE

-60

-35

-10   15

40      65      90     115    140

2.5

3.0

3.5

4.0        4.5        5.0        5.5

-60

-35

-10   15

40      65      90     115    140

TEMPERATURE (°C)

VDD (V)

TEMPERATURE (°C)

5

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

INL (LSB)

Typical Operating Characteristics (continued) (VDD = +3.0V, VREF = 2.5V, 0.1µF at REF, fSCLK = 2.17MHz, 16 clocks/conversion cycle (108ksps), CH- = GND for MAX145, TA = +25°C, unless otherwise noted.)

0.20

MAX144/5-10

INTEGRAL NONLINEARITY

vs. OUTPUT  CODE

0.5

MAX144/5-11

INTEGRAL NONLINEARITY

vs. SUPPLY VOLTAGE

0.5

MAX144/5-12

INTEGRAL NONLINEARITY

vs. TEMPERATURE

0.15

0.10

0.05

0

-0.05

0.4

INL (LSB)

0.3

0.2

0.4

INL (LSB)

0.3

0.2

-0.10

-0.15

0.1

0.1

-0.20

0              1024           2048          3072           4096

0

| 2.5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

3.0

3.5

4.0        4.5        5.0        5.5

0

-60

| -35 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-10   15

40      65      90     115    140

OUTPUT CODE

VDD (V)

TEMPERATURE (°C)

0

-20

FFT PLOT

VDD = +2.7V

fIN = 10kHz

MAX144/5-13

fSAMPLE = 108ksps

11.8

MAX144/5-14

EFFECTIVE NUMBER OF BITS

vs. FREQUENCY

VDD = +2.7V

-40

-60

AMPLITUDE (dB)

-80

11.6

11.4

-100

-120

EFFECTIVE NUMBER OF BITS

11.2

-140

0                                     27                                     54

FREQUENCY (kHz)

11.0

1                                      10                                    100

FREQUENCY (kHz)

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | VDD | Positive Supply Voltage, +2.7V to +5.25V |
| 2 | CH0 (CH+) | Analog Input: MAX144 = single-ended (CH0); MAX145 = differential (CH+) |
| 3 | CH1 (CH-) | Analog Input: MAX144 = single-ended (CH1); MAX145 = differential (CH-) |
| 4 | GND | Analog and Digital Ground |
| 5 | REF | External Reference Voltage Input. Sets the analog voltage range. Bypass with a 100nF capacitor close to the device. |
| 6 | CS/SHDN | Active-Low Chip-Select Input/Active-High Shutdown Input. Pulling CS/SHDN high puts the device into shutdown with a maximum current of 5µA. |
| 7 | DOUT | Serial Data Output. Data changes state at SCLK’s falling edge. High impedance when CS/SHDN is high. |
| 8 | SCLK | Serial Clock Input. DOUT changes on the falling edge of SCLK. |

6

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

VDD

DOUT

6k                                 CL

GND

DOUT

6k

CL

GND

a) HIGH-Z TO V0H, V0L TO V0H, AND VOH TO HIGH-Z

b) HIGH-Z TO V0L, V0H TO V0L, AND VOL TO HIGH-Z

Figure 1. Load Circuits for Enable and Disable Time

Detailed Description

The MAX144/MAX145 analog-to-digital converters (ADCs) use a successive-approximation  conversion (SAR) technique and on-chip track-and-hold (T/H) structure to convert an analog signal to a serial 12-bit digital output data stream.

This flexible serial interface provides easy interface to microprocessors (µPs). Figure 2 shows a simplified functional diagram of the internal architecture for both the MAX144 (2 channels, single-ended) and the MAX145 (1 channel, pseudo-differential).

CS/SHDN SCLK

CH0 (CH+)

CH1 (CH-)

ANALOG INPUT MUX

(2 CHANNEL)

CONTROL LOGIC

T/H

INTERNAL CLOCK

SCLK

12-BIT

IN      SAR  OUT ADC

OUTPUT REGISTER

MAX144

MAX145

MAX144/MAX145

DOUT

Analog Inputs: Single-Ended (MAX144)

and Pseudo-Differential (MAX145) The sampling architecture of the ADC’s analog com- parator is illustrated in the equivalent input circuit of Figure 3. In single-ended mode (MAX144), both chan- nels CH0 and CH1 are referred to GND and can be connected to two different signal sources. Following the

REF

Figure 2. Simplified Functional Diagram

12-BIT CAPACITIVE DAC REF

( ) ARE FOR MAX145

MAX144

MAX145

power-on reset, the ADC is set to convert CH0. After CH0 has been converted, CH1 will be converted and the conversions will continue to alternate between channels. Channel switching is performed by toggling

CH0 (CH+)

CH1 (CH-)

INPUT MUX

CHOLD

16pF

ZERO

COMPARATOR

TO SAR

the CS/SHDN pin. Conversions can be performed on the same channel by toggling CS/SHDN twice between conversions. If only one channel is required, CH0 and CH1 may be connected together; however, the output data will still contain the channel identification bit

GND

CSWITCH

RIN

9k

TRACK                   HOLD T/H

(before the MSB).

For the MAX145, the input channels form a single differ-

SINGLE-ENDED MODE: CH0, CH1 = IN+; GND = IN- DIFFERENTIAL-ENDED MODE: CH+ = IN+; CH- = IN-

CONTROL LOGIC

( ) ARE FOR MAX145

ential channel pair (CH+, CH-). This configuration is pseudo-differential to the effect that only the signal at IN+ is sampled. The return side IN- must remain stable within ±0.5LSB (±0.1LSB for optimum results) with respect to GND during a conversion. To accomplish this, connect a 0.1µF capacitor from IN- to GND.

During the acquisition interval, the channel selected as the positive input (IN+) charges capacitor CHOLD. The acquisition interval spans from when CS/SHDN falls to the falling edge of the second clock cycle (external

Figure 3. Analog Input Channel Structure

clock mode) or from when CS/SHDN falls to the first falling edge of SCLK (internal clock mode). At the end of the acquisition interval, the T/H switch opens, retain- ing charge on CHOLD as a sample of the signal at IN+.

The conversion interval begins with the input multiplex- er switching CHOLD from the positive input (IN+) to the negative input (IN-). This unbalances node ZERO at the comparator’s positive input.

7

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

The capacitive  digital-to-analog  converter (DAC) adjusts during the remainder of the conversion cycle to restore node ZERO to 0V within the limits of 12-bit resolution. This action is equivalent to transferring a

16pF · [(VIN+) - (VIN-)] charge from CHOLD to the bina- ry-weighted capacitive DAC, which in turn forms a digi- tal representation of the analog input signal.

Track/Hold (T/H) The ADC’s T/H stage enters its tracking mode on the falling edge of CS/SHDN. For the MAX144 (single- ended inputs), IN- is connected to GND and the con- verter samples the positive (“+”) input. For the MAX145 (pseudo-differential inputs), IN- connects to the nega- tive input (“-”) and the difference of [(VIN+) - (VIN-)] is sampled. At the end of the conversion, the positive input connects back to IN+ and CHOLD charges to the input signal.

The time required for the T/H stage to acquire an input signal is a function of how fast its input capacitance is charged. If the input signal’s source impedance is high, the acquisition time lengthens, and more time must be allowed between conversions. The acquisition time, tACQ, is the maximum time the device takes to acquire the signal, and is also the minimum time required for the signal to be acquired. Calculate this with the follow- ing equation:

tACQ = 9(RS + RIN)CIN

where RS is the source impedance of the input signal, RIN (9k) is the input resistance, and CIN (16pF) is the input capacitance  of the ADC. Source impedances below 1k have no significant impact on the AC perfor- mance of the MAX144/MAX145.

Higher source impedances can be used if a 0.01µF capacitor is connected to the individual analog inputs. Together with the input impedance,  this capacitor forms an RC filter, limiting the ADC’s signal bandwidth.

Input Bandwidth The MAX144/MAX145 T/H stage offers a 2.25MHz small-signal and a 1MHz full-power bandwidth, which make it possible to use the parts for digitizing high- speed transients and measuring periodic  signals with bandwidths  exceeding the ADCs sampling rate by using undersampling techniques. To avoid high-fre- quency signals being aliased into the frequency band of interest, anti-alias filtering is recommended. Most aliasing problems can be fixed easily with an external resistor and a capacitor.  However, if DC precision is required, it is usually best to choose a continuous or switched-capacitor  filter, such as the MAX7410/ MAX7414 (Figure 4). Their Butterworth characteristic generally provides the best compromise (with regard to rolloff and attenuation) in filter configurations, is easy to design, and provides a maximally flat passband response.

Analog Input Protection Internal protection diodes, which clamp the analog input to VDD and GND, allow each input channel to swing within GND - 300mV to VDD + 300mV without damage. However, for accurate conversions, both inputs must not exceed VDD + 50mV or be less than GND - 50mV.

If an off-channel analog input voltage exceeds the supplies, limit the input current to 4mA.

VDD

2   IN

fC = 15kHz

4

VDD

MAX7410

MAX7414

7

SHDN

OUT  5

CLK  8

470   **

0.1   F

0.01  F**

1

2                             VDD

CH0

MAX144

3   CH1

5

REF

DOUT  7

EXTERNAL REFERENCE

COM

1

0.01   F

OS          GND

6             3

1.5MHz

OSCILLATOR

8

SCLK

GND

4

6

CS/SHDN

P/  C

**USED  TO ATTENUATE SWITCHED-CAPACITOR  FILTER CLOCK NOISE

Figure 4. Analog Input with Anti-Aliasing Filter Structure

8

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

Selecting Clock Mode To start the conversion process on the MAX144/ MAX145, pull CS/SHDN low. At CS/SHDN’s falling edge, the part wakes up and the internal T/H enters

track  mode.  In  addition,  the  state  of  SCLK at CS/SHDN’s falling edge selects internal (SCLK = high) or external (SCLK = low) clock mode.

Internal Clock (fSCLK < 100kHz or fSCLK > 2.17MHz) In internal clock mode, the MAX144/MAX145 run from an internal, laser-trimmed oscillator to within 20% of the

2MHz specified clock rate. This releases the system microprocessor from running the SAR conversion clock and allows the conversion results to be read back at the processor’s convenience, at any clock rate from 0 to 5MHz. Operating the MAX144/MAX145 in internal clock mode is necessary for serial interfaces operating with clock frequencies lower than 100kHz or greater than 2.17MHz. Select internal clock mode (Figure 5), by holding SCLK high during a high/low transition of CS/SHDN. The first SCLK falling edge samples the data and initiates a conversion using the integrated on-chip oscillator. After the conversion, the oscillator shuts off and DOUT goes high, signaling the end of conversion (EOC). Data can then be read out with SCLK.

MAX144/MAX145

External Clock (fSCLK = 100kHz to 2.17MHz) The external clock mode (Figure 6) is selected by tran- sitioning CS/SHDN from high to low while SCLK is low. The external clock signal not only shifts data out, but also drives the analog-to-digital conversion. The input is sampled and conversion begins on the falling edge of the second clock pulse. Conversion must be com- pleted within 140µs to prevent degradation in the con- version results caused by droop on the T/H capacitors. External clock mode provides the best throughput for clock frequencies between 100kHz and 2.17MHz.

Output Data Format Table 1 illustrates the 16-bit, serial data stream output format for both the MAX144 and MAX145. The first three bits are always logic high (including the EOC bit for internal clock mode), followed by the channel identi- fication (CHID = 0 for CH0, CHID = 1 for CH1, CHID = 0 for the MAX145), and then 12 bits of data in MSB-first format. After the last bit has been read out, additional SCLK pulses will clock out trailing zeros. DOUT transi- tions on the falling edge of SCLK. The output remains high-impedance when CS/SHDN is high.

ACTIVE

POWER DOWN

ACTIVE

CS/SHDN SCLK

tCS

tWAKE

(tACQ)

tCONV

1       2       3       4       5

6       7       8        9      10      11

12     13     14     15     16

DOUT

HIGH-Z

SAMPLING INSTANT

EOC      1

1   CHID  MSB  D10    D9

D8     D7    D6     D5     D4    D3     D2     D1     D0

HIGH-Z

Figure 5. Internal Clock Mode Timing

ACTIVE

CS/SHDN

POWER DOWN

tCS

ACTIVE

tWAKE

(tACQ)

SAMPLING INSTANT

ACTIVE

POWER DOWN

SCLK

1       2       3       4       5

6       7       8        9      10      11

12     13     14     15     16

DOUT

HIGH-Z

CHID  MSB  D10    D9

D8     D7    D6     D5     D4    D3     D2     D1     D0

HIGH-Z

Figure 6. External Clock Mode Timing

9

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

Table 1.  Serial Output Data Stream for Internal and External Clock Mode

| SCLK CYCLE | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DOUT (Internal Clock) | EOC | 1 | 1 | CHID | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| DOUT (External Clock) | 1 | 1 | 1 | CHID | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |

External Reference An external reference is required for both the MAX144 and the MAX145. At REF, the DC input resistance is a minimum of 18k. During a conversion, a reference must be able to deliver 250µA of DC load current and have an output impedance of 10 or less. Use a 0.1µF bypass capacitor for best performance. The reference input structure allows a voltage range of 0 to VDD +

50mV, although noise levels will decrease effective res- olution at lower reference voltages.

Automatic Power-Down Mode Whenever the MAX144/MAX145 are not selected (CS/SHDN = VDD), the parts enter their shutdown mode. In shutdown all internal circuitry turns off, reduc- ing supply current to typically less than 0.2µA. With an external reference stable to within 1LSB, the wake-up time is 2.5µs. If the external reference is not stable with- in 1LSB, the wake-up time must be increased to allow the reference to stabilize.

__________Applications Information

Signal-to-Noise Ratio (SNR) For a waveform perfectly reconstructed  from digital samples, the theoretical maximum SNR is the ratio of full-scale analog input (RMS value) to the RMS quanti- zation error (residual error). The ideal, theoretical mini- mum analog-to-digital noise is caused by quantization error only and results directly from the ADC’s resolution (N bits):

SNR(MAX) = (6.02 x N + 1.76)dB

In reality, there are other noise sources besides quanti- zation noise: thermal noise, reference noise, clock jitter, etc. Therefore, SNR is computed by taking the ratio of the RMS signal to the RMS noise which includes all spectral components minus the fundamental, the first five harmonics, and the DC offset.

Signal-to-Noise Plus Distortion (SINAD) SINAD is the ratio of the fundamental input frequency’s RMS amplitude to RMS equivalent of all other ADC out- put signals:

SINAD(dB) =  20 x log           SIGNALRMS

(Noise + Distortion)RMS

Effective Number of Bits (ENOB) ENOB indicates the global accuracy of an ADC at a specific  input frequency and sampling rate. An ideal ADC’s error consists only of quantization noise. With an input range equal to the full-scale range of the ADC, the effective number of bits can be calculated as follows:

ENOB = (SINAD - 1.76) / 6.02

Total Harmonic Distortion (THD) THD is the ratio of the RMS sum of the first five harmon- ics of the input signal to the fundamental itself. This is expressed as:

V 22  + V 32  + V 42  + V 52

THD =  20 x log

V1

where V1 is the fundamental amplitude, and V2 through V5 are the amplitudes of the 2nd- through 5th-order harmonics.

Spurious-Free Dynamic Range (SFDR) SFDR is the ratio of RMS amplitude of the fundamental (maximum signal component) to the RMS value of the next largest spurious component, excluding DC offset.

Connection to Standard Interfaces The MAX144/MAX145 interface is fully compatible with SPI, QSPI, and MICROWIRE standard serial interfaces.

If a serial interface is available, establish the CPU’s seri- al interface as master so that the CPU generates the serial clock for the MAX144/MAX145. Select a clock fre- quency from 100kHz to 2.17MHz (external clock mode).

1) Use a general-purpose I/O line on the CPU to pull

CS/SHDN low while SCLK is low.

2) Wait for the minimum wake-up time (tWAKE) speci- fied before activating SCLK.

3) Activate SCLK for a minimum of 16 clock cycles.

The serial data stream of three leading ones, the channel identification, and the MSB of the digitized input signal begin at the first falling clock edge. DOUT transitions on SCLK’s falling edge and is available in MSB-first format. Observe the SCLK to

10

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

DOUT valid timing characteristic.  Data should be clocked into the µP on SCLK’s rising edge.

4) Pull CS/SHDN high at or after the 16th falling clock edge. If CS/SHDN remains low, trailing zeros will be clocked out after the LSB.

5) With CS/SHDN high, wait at least 60ns (tCS) before starting a new conversion by pulling CS/SHDN low. A conversion can be aborted by pulling CS/SHDN high before the conversion ends; wait at least 60ns before starting a new conversion.

Data can be output in two 8-bit sequences or continu- ously. The bytes will contain the result of the conversion

MAX144/MAX145

padded with three leading ones and the channel identi- fication before the MSB. If the serial clock hasn’t been idled after the last LSB and CS/SHDN is kept low, DOUT sends trailing zeros.

SPI and MICROWIRE Interface When using SPI (Figure 8a) or MICROWIRE (Figure 8b) interfaces, set CPOL = 0 and CPHA = 0. Conversion begins with a falling edge on CS/SHDN (Figure 8c). Two consecutive 8-bit readings are necessary to obtain the entire 12-bit result from the ADC. DOUT data transi- tions on the serial clock’s falling edge and is clocked into the µP on SCLK’s rising edge. The first 8-bit data stream contains three leading ones, the channel identi-

CS/SHDN

tSCLKS

tCL

tCH

tCS

SCLK

tDV

tDO                                                                      tTR

DOUT

HIGH-Z                                                                                                                                                                                                                                                            HIGH-Z

Figure 7. Detailed Serial-Interface Timing Sequence

SPI

I/O SCK MISO

SS

VDD

CS/SHDN SCLK DOUT

MAX144

MAX145

MICROWIRE

I/O SK SI

CS/SHDN SCLK DOUT

MAX144

MAX145

Figure 8a. SPI Connections                                                                 8b.  MICROWIRE Connections

SCLK CS/SHDN

1ST BYTE READ                                                                                                                         2ND BYTE READ

1          2          3          4          5          6         7         8                                               9         10        11        12        13        14       15       16

DOUT*

CHID    D11     D10      D9       D8

D7                      D6       D5        D4       D3

D2       D1        D0

HIGH-Z

SAMPLING INSTANT

*WHEN CS/SHDN IS HIGH, DOUT = HIGH-Z

MSB                                                                                                                                                                         LSB

Figure 8c.  SPI/MICROWIRE Interface Timing Sequence (CPOL = CPHA = 0)

11

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

fication, and the first four data bits starting with the MSB. The second 8-bit data stream contains the remaining bits, D7 through D0.

QSPI Interface Using the high-speed QSPI interface with CPOL = 0 and CPHA = 0, the MAX144/MAX145 support a maxi- mum fSCLK of 2.17MHz. The QSPI circuit in Figure 9a can be programmed to perform a conversion on each of the two channels for the MAX144. Figure 9b shows the QSPI interface timing.

PIC16 with SSP Module and PIC17 Interface The MAX144/MAX145 are compatible with a PIC16/ PIC17 controller (µC), using the synchronous serial-port (SSP) module.

To establish SPI communication, connect the controller as shown in Figure 10a and configure the PIC16/PIC17 as system master by initializing its synchronous serial- port control register (SSPCON) and synchronous serial- port status register (SSPSTAT) to the bit patterns shown in Tables 2 and 3.

In SPI mode, the PIC16/PIC17 µCs allow 8 bits of data to be synchronously transmitted and received simulta-

QSPI

CS SCK MISO

SS

VDD

CS/SHDN SCLK DOUT

MAX144

MAX145

neously. Two consecutive 8-bit readings (Figure 10b) are necessary to obtain the entire 12-bit result from the ADC. DOUT data transitions on the serial clock’s falling edge and is clocked into the µC on SCLK’s rising edge. The first 8-bit data stream contains three leading ones, the channel identification, and the first four data bits starting with the MSB. The second 8-bit data stream contains the remaining bits, D7 through D0.

Figure 9a. QSPI Connections

SCLK CS/SHDN

1          2          3          4          5          6         7         8          9         10        11        12        13        14       15       16

DOUT

CHID    D11     D10      D9       D8

D7       D6       D5        D4       D3

D2       D1        D0

HIGH-Z

SAMPLING INSTANT

*WHEN CS/SHDN IS HIGH, DOUT = HIGH-Z

MSB                                                                                                                                  LSB

Figure 9b.  QSPI Interface Timing Sequence (CPOL = CPHA = 0)

Table 2.  Detailed SSPCON Register Contents

| CONTROL BIT |  | MAX144/MAX145 SETTINGS | SYNCHRONOUS SERIAL-PORT CONTROL REGISTER (SSPCON) |
| --- | --- | --- | --- |
| WCOL | BIT7 | X | Write Collision Detection Bit |
| SSPOV | BIT6 | X | Receive Overflow Detect Bit |
| SSPEN | BIT5 | 1 | Synchronous Serial-Port Enable Bit. 0: Disables serial port and configures these pins as I/O port pins. 1: Enables serial port and configures SCK, SDO and SCI pins as serial port pins. |
| CKP | BIT4 | 0 | Clock Polarity Select Bit. CKP = 0 for SPI master mode selection. |
| SSPM3 | BIT3 | 0 | Synchronous Serial-Port Mode Select Bit. Sets SPI master mode and selects fCLK = fOSC / 16. |
| SSPM2 | BIT2 | 0 |  |
| SSPM1 | BIT1 | 0 |  |
| SSPM0 | BIT0 | 1 |  |

X = Don’t care

12

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

Table 3.  Detailed SSPSTAT Register Contents

| CONTROL BIT |  | MAX144/MAX145 SETTINGS | SYNCHRONOUS SERIAL-PORT STATUS REGISTER (SSPSTAT) |
| --- | --- | --- | --- |
| SMP | BIT7 | 0 | SPI Data Input Sample Phase. Input data is sampled at the middle of the data output time. |
| CKE | BIT6 | 1 | SPI Clock Edge Select Bit. Data will be transmitted on the rising edge of the serial clock. |
| D/A | BIT5 | X | Data Address Bit |
| P | BIT4 | X | Stop Bit |
| S | BIT3 | X | Start Bit |
| R/W | BIT2 | X | Read/Write Bit Information |
| UA | BIT1 | X | Update Address |
| BF | BIT0 | X | Buffer Full Status Bit |

X = Don’t care

Layout, Grounding, and Bypassing For best performance, use printed circuit boards (PCBs). Wire-wrap configurations are not recommend- ed, since the layout should ensure proper separation of analog and digital traces. Run analog and digital lines anti-parallel to each other, and don’t lay out digital sig- nal paths underneath the ADC package. Use separate analog and digital PCB ground sections with only one star-point (Figure 11) connecting the two ground systems

VDD                                                                                                 VDD

(analog and digital). For lowest-noise operation, ensure the ground return to the star ground’s power supply is low impedance and as short as possible. Route digital signals far away from sensitive analog and reference inputs.

High-frequency noise in the power supply VDD could influence the proper operation of the ADC’s fast com- parator. Bypass VDD to the star ground with a network of two parallel capacitors  (0.1µF and 1µF) located as close as possible to the power supply pin of MAX144/ MAX145. Minimize capacitor lead length for best sup- ply-noise rejection and add an attenuation resistor (10) if the power supply is extremely noisy.

SCLK

DOUT CS/SHDN

SCK

SDI I/O

MAX144

MAX145

PIC16/17

GND                                                                             GND

Figure 10a. SPI Interface Connection for a PIC16/PIC17

Controller

SCLK CS/SHDN

1ST BYTE READ                                                                                                                         2ND BYTE READ

1          2          3          4          5          6         7         8                                               9         10        11        12        13        14       15       16

DOUT*

CHID    D11     D10      D9       D8

D7                      D6       D5        D4       D3

D2       D1        D0

HIGH-Z

SAMPLING INSTANT

*WHEN CS/SHDN IS HIGH, DOUT = HIGH-Z

MSB                                                                                                                                                                         LSB

Figure 10b. SPI Interface Timing with PIC16/PIC17 in Master Mode (CKE = 1, CKP = 0, SMP = 0, SSPM3–SSPM0 = 0001)

13

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs in 8-Pin µMAX

MAX144/MAX145

+3V

POWER SUPPLIES

+3V

GND

Ordering Information (continued) PART TEMP PIN- INL PKG RANGE PACKAGE (LSB) CODE

MAX145ACUA 0°C to 8 µMAX ±0.5 U8-1 +70°C

MAX145BCUA 0°C to 8 µMAX ±1 U8-1 +70°C

MAX145ACPA 0°C to 8 Plastic DIP ±0.5 P8-1 +70°C 0°C to MAX145BCPA +70°C 8 Plastic DIP ±1 P8-1

MAX145BC/D 0°C to Dice* ±1 — +70°C MAX145AEUA -40°C to 8 µMAX ±0.5 U8-1 +85°C

MAX145BEUA -40°C to 8 µMAX ±1 U8-1 +85°C

MAX145AEPA -40°C to 8 Plastic DIP ±0.5 P8-1 +85°C

MAX145BEPA -40°C to 8 Plastic DIP ±1 P8-1 +85°C

MAX145AMJA 55°C to 8 CERDIP** ±0.5 J8-2 +125°C

MAX145BMJA 55°C to 8 CERDIP** ±1 J8-2 +125°C

R* = 10 W

1mF

V DD

0.1 mF

GND

+3V

DGND

MAX144

MAX145

DIGITAL CIRCUITRY

* OPTIONAL FILTER RESISTOR

Figure 11. Power-Supply Bypassing  and Grounding

Chip Information

TRANSISTOR COUNT: 2,058

SUBSTRATE CONNECTED TO GND

*Dice are specified at TA = +25°C, DC parameters  only.

**Contact factory for availability.

14

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs  in 8-Pin µMAX

MAX144/MAX145

8LUMAXD.EPS

Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications.  For the latest package outline information, go to www.maxim-ic.com/packages.)

15

+2.7V, Low-Power, 2-Channel, 108ksps, Serial 12-Bit ADCs in 8-Pin µMAX

MAX144/MAX145

PDIPN.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |
| --- | --- | --- |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

16                               Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2005 Maxim Integrated  Products               Printed USA                           is a registered  trademark  of Maxim Integrated  Products, Inc.