19-0532; Rev 0; 5/06

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

General Description

The MAX16803 current regulator operates from a 6.5V

to 40V input-voltage range and delivers up to a total of

350mA to one or more strings of high-brightness  LEDs (HB LEDs). The output current of the MAX16803 is adjusted by using an external current-sense resistor in series with the LEDs. A dimming  input allows wide- range “pulsed”  PWM operation. Wave-shaping circuitry reduces  EMI. The differential  current-sense input increases noise immunity. The MAX16803 is well suited for applications requiring high-voltage input and is able to withstand automotive load-dump  events up to 40V. An on-board pass element minimizes external compo- nents while providing  ±3.5%  output-current accuracy. Additional  features include a 5V regulated output and short-circuit and thermal protection.

The MAX16803 is available in a thermally enhanced,

5mm x 5mm, 16-pin TQFN package  and is specified over the automotive -40°C to +125°C temperature range.

Applications

Automotive Interior: Map, Courtesy, and Cluster

Lighting

Automotive Exterior: Rear Combination Lights (RCLs) Emergency Vehicle Warning Lights

MAX16803

Features

♦ +6.5V to +40V Operating Range

♦ Adjustable LED Current (35mA to 350mA)

♦ ±3.5% LED Current Accuracy

♦ High-Voltage DIM Pin for Dimming Interface

♦ Integrated Pass Element with Low-Dropout

Voltage (0.5V typ)

♦ +5V Regulated Output with 4mA Source

Capability

♦ Parallel Operation for LED Current > 350mA

♦ Differential LED Current Sense

♦ Low Shutdown Supply Current (12µA typ)

♦ Low 204mV Current-Sense Reference Reduces

Power Losses

♦ Wave-Shaped Edges Reduce Radiated EMI During

PWM Dimming

♦ Thermal Shutdown

♦ Output Short-Circuit Protection

♦ Available in Small, Thermally Enhanced,

5mm x 5mm, 16-Pin TQFN Package

♦ -40°C to +125°C Operating Temperature Range

Ordering Information

Navigation and Marine Indicators

PART               TEMP RANGE

PIN-

PACKAGE

PKG

CODE

GND

General Lighting

Signage, Gasoline Canopies, Beacons

CS+

Pin  Configuration

MAX16803ATE+       -40°C to +125°C    16 TQFN-EP*   T1655-3

+Denotes lead-free package.

*EP = Exposed pad.

Simplified Diagram

V5

CS-

TOP VIEW

12         11         10          9

+6.5V TO +40V

0.1µF

IN                                          OUT

+5V REG

DIM

N.C. EN

OUT

13

14

15

16           +

MAX16803

8     N.C.

7     N.C.

6     N.C.

5     N.C.

EN

PWM DIMMING

MAX16803

V5                                           LEDs

0.1µF

CS+

RSENSE

OUT

IN

IN

N.C.

1           2           3           4

DIM

GND

CS-

TQFN

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

ABSOLUTE MAXIMUM RATINGS

IN, OUT, DIM, and EN to GND ...............................-0.3V to +45V CS+, CS-, V5 to GND ...............................................-0.3V to +6V OUT Short Circuited  to GND Duration

(at VIN = +16V) ........................................................60 minutes Maximum Current Into Any Pin (except IN and OUT) .........±20mA Continuous Power Dissipation  (TA = +70°C)

16-Pin TQFN (derate 33.3mW/°C above +70°C) ......2666.7mW

Operating  Junction Temperature  Range ...........-40°C  to +125°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN = VEN = +12V, CV5 = 0.1µF to GND, IV5 = 0, VCS- = 0V, VDIM = +4V, connect  RSENSE = 0.58Ω between  CS+ and CS- (Note 1). TJ = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Voltage Range | VIN | (Note 2) | 6.5 40.0 | V |
| Ground Current | IG | ILOAD = 350mA | 1.4 3 | mA |
| Shutdown Supply Current | ISHDN | EN ≤ 0.6V | 12 40 | µA |
| Guaranteed Output Current | IOUT | RSENSE = 0.55Ω | 350 | mA |
| Output Current Accuracy |  | 35mA < IOUT < 350mA | ±3.5 | % |
| Dropout Voltage (Note 3) | ∆VDO | IOUT = 350mA (current pulsed), 12V < VIN < 40V | 0.5 1.2 | V |
|  |  | IOUT = 350mA (current pulsed), 6.5V < VIN < 12V | 0.5 1.5 |  |
| Output Current Slew Rate |  | Current rising, DIM rising to 4V | 17 | mA/µs |
|  |  | Current falling, DIM falling to 0.6V | 17 |  |
| Short-Circuit Current |  | VOUT = 0V, VIN = 12V | 400 600 800 | mA |
| LOGIC INPUT |  |  |  |  |
| EN Input Current | IEN |  | 100 | nA |
| EN Input Voltage High | VIH |  | 2.8 | V |
| EN Input Voltage Low | VIL |  | 0.6 | V |
| Turn–On Time | tON | EN rising edge to 90% of OUT | 80 | µs |
| CURRENT SENSE (Note 4) |  |  |  |  |
| Regulated RSENSE Voltage | VRSNS | VSENSE = VCS+ - VCS- | 196 203 210 | mV |
| CS- Voltage Range |  |  | -0.3 +4.1 | V |
| Input Current (CS+) |  | VCS+ = 220mV | +14 | µA |
| Input Current (CS-) |  | VCS+ = 220mV | -84 | µA |
| DIM Input Current |  |  | 1 | µA |
| DIM Input Voltage High | VIH |  | 4 | V |
| DIM Input Voltage Low | VIL |  | 0.6 | V |
| Turn-On Time | tON | After DIM rising to 4V (Note 5) | 52 | µs |
| Turn-Off Time | tOFF | After DIM falling to 0.6V (Note 5) | 38 | µs |

2

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

ELECTRICAL CHARACTERISTICS (continued)

(VIN = VEN = +12V, CV5 = 0.1µF to GND, IV5 = 0, VCS- = 0V, VDIM = +4V, connect  RSENSE = 0.58Ω between  CS+ and CS- (Note 1). TJ = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| THERMAL OVERLOAD |  |  |  |  |
| Thermal-Shutdown Temperature |  |  | +155 | °C |
| Thermal-Shutdown Hysteresis |  |  | 23 | °C |
| +5V REGULATOR |  |  |  |  |
| Output Voltage Regulation | V5 | (Note 6) | 5.0 5.27 5.5 | V |
| V5 Short-Circuit Current |  | V5 = 0V (Note 7) | 16 | mA |

Note 1:  All devices  100% production tested at TA = +25°C. Limits over the operating  temperature  range are guaranteed by design.

Note 2:  Resistors were added from OUT to CS+ to aid with the power dissipation during testing.

Note 3:  Dropout is measured  as follows:

Connect RO = 27Ω from OUT to CS+. Connect RSENSE = 0.58Ω from CS+ to CS-. Set VIN = +12V (record VOUT as VOUT1). Reduce VIN until VOUT = 0.97 x VOUT1 (record as VIN2 and VOUT2). ∆VDO = VIN2 - VOUT2.

Note 4:  IV5 = 0mA.

Note 5:  tON time includes  the delay and the rise time needed  for IOUT to reach 90% of its final value. tOFF time is the time needed for IOUT to drop below 10%. See the Typical Operating  Characteristics. tON and tOFF are tested with 13Ω from OUT to CST.

Note 6:  Current regulation varies with V5 load (see the Typical Operating  Characteristics).

Note 7:  Thermal shutdown does not function if the output of the 5V reference is shorted to ground.

Typical Operating Characteristics

(VIN = VEN = +12V, TA = +25°C, unless otherwise noted.)

450

400

OUTPUT CURRENT (mA)

350

300

250

200

150

100

50

0

OUTPUT CURRENT

vs. TEMPERATURE

ILOAD = 350mA

ILOAD = 200mA

ILOAD = 100mA ILOAD = 35mA

MAX16803 toc01

| 0.210 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

0.209

0.208

(VCS+ - VCS-) (V)

OUTPUT CURRENT (mA)

0.207

0.206

0.205

0.203

0.202

0.201

0.200

MAX16803 toc02

(VCS+ - VCS-) vs. OUTPUT CURRENT

400

350

300

200

100

50

0

OUTPUT CURRENT

MAX16803 toc03

vs. INPUT VOLTAGE

-40 -25 -10  5   20  35  50

65  80

95 110 125

20  50

80 110 140 170 200 230 260 290 320 350

| 0 5 10 15 20 25 30 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

35    40

TEMPERATURE (°C)

OUTPUT CURRENT (mA)

INPUT VOLTAGE (V)

3

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

DROPOUT VOLTAGE (V)

Typical Operating Characteristics (continued)

(VIN = VEN = +12V, TA = +25°C, unless otherwise noted.)

1.0

0.9

0.8

0.7

DROPOUT VOLTAGE

vs. TEMPERATURE

SHUTDOWN CURRENT

vs. TEMPERATURE

MAX16803 toc04

35

30                          VIN = 40V

SHUTDOWN CURRENT (µA)

25

MAX16803 toc05

5.5

5V REGULATOR OUTPUT (V)

5.4

MAX16803 toc06

+5V REGULATOR OUTPUT

vs. TEMPERATURE

ILOAD = 1mA

0.6

0.5

20

VIN = 20V

5.3

NO LOAD

0.4

0.3

0.2

15

VIN

10

= 12V

5.2

5.1

0.1

0

5                               VIN = 6.5V

0

5.0

ILOAD = 5mA

| -40 -25 -10 5 20 35 50 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ILOAD = 350mA |  |  |  |  |  |  |  |  |

65  80

95 110 125

-40 -25 -10  5

20  35

50  65

80  95 110 125

-40 -25 -10  5

20  35  50

65  80

95 110 125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

0.210

0.209

0.208

0.207

(VCS+ - VCS-) (V)

0.206

0.205

0.204

0.203

0.202

0.201

0.200

0.199

0.198

(VCS+ - VCS-) vs. IV5

MAX16803 toc07

5.30

5.25

5.20

5V REGULATOR OUTPUT (V)

5.15

5.10

+5V REGULATOR OUTPUT vs. VIN

MAX16803 toc08

NO LOAD

LOAD = 1mA LOAD = 5mA

200Hz DIMMED OPERATION

MAX16803 toc09

ILOAD = 350mA VIN = VEN = 12V DIM PULSED AT

200Hz (1% DUTY CYCLE)

VDIM

2V/div

0V

ILOAD

200mA/div

0A

0      1      2      3

| 4 5 6 7 8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ILOAD = 410mA |  |  |  |  |  |

5      10     15

20     25

30     35     40

20µs/div

IV5 (mA)

VIN (V)

200Hz DIMMED  OPERATION (EXPANDED)

MAX16803 toc10

200Hz DIMMED  OPERATION (EXPANDED)

MAX16803 toc11

ILOAD = 350mA VIN = VEN = 12V

ILOAD = 350mA

VIN = VEN = 12V

DIM PULSED AT

200Hz (50% DUTY CYCLE)

VDIM

2V/div

0V

DIM PULSED AT

200Hz (50% DUTY CYCLE)

VDIM

2V/div

0V

ILOAD

200mA/div

0A

ILOAD

200mA/div

0A

20µs/div

20µs/div

4

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

Typical Operating Characteristics (continued)

(VIN = VEN = +12V, TA = +25°C, unless otherwise noted.)

205

MAX16803 toc12

VSNSE vs. VCS-

| 300 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

MAX16803 toc13

IDIM vs. VDIM

204

250

VSNSE (mV)

203

202

201

200

VIN = 12V

ILOAD = 350mA

200

IDIM (µA)

150

100

50

0

2.0

2.5

3.0

3.5

VCS- (V)

4.0

4.5

5.0

0      2      4

6      8     10    12

VDIM (V)

14    16

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 16 | OUT | Current-Regulated Output. Connect pin 1 to pin 16. |
| 2, 3 | IN | Positive Input Supply. Bypass IN with a 0.1µF (min) capacitor to GND. Connect pin 2 to pin 3. |
| 4–8, 14 | N.C. | Not Internally Connected |
| 9 | CS+ | Positive Input of the Internal Differential Amplifier. Connect the current-sense resistor between CS+ and CS- to program the output current level. |
| 10 | CS- | Negative Input of the Internal Differential Amplifier. Connect the current-sense resistor between CS- and CS+ to program the output current level. |
| 11 | V5 | +5V Regulated Output. Connect a 0.1µF capacitor from V5 to GND. |
| 12 | GND | Ground |
| 13 | DIM | Pulsed Dimming Input. Drive DIM low to disable the output. Drive DIM high to enable the output. Must not be left unconnected. |
| 15 | EN | Enable Input. Drive EN high to enable the output. |
| — | EP | Exposed Pad. Connect to the ground plane for effective power dissipation. Do not use as the only ground connection. |

5

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

Functional Diagram

IN                          V5

DIM

REGULATOR

IN

EN                        ENABLE

REFERENCE GENERATOR

I_REG

OUT

MAX16803

DIFFERENTIAL SENSE AMPLIFIER

CS+ CS-

THERMAL SHUTDOWN

Detailed Description

The MAX16803 is a high-current  regulator capable  of providing  up to a total of 350mA of current to one or more strings of HB LEDs. A wide operating input volt- age range of +6.5V to +40V makes the MAX16803 ideal for automotive applications.  A +5V regulated out- put provides up to 4mA of current to power external cir- cuitry. In addition, the MAX16803 features thermal and output short-circuit protection. The wide operating volt- age range helps protect the MAX16803 against large transients such as those found in load-dump situations up to 40V.

The MAX16803 uses a feedback loop to control the out- put current. The differential voltage across the sense resistor is compared  to a fixed reference voltage, and the error is amplified to serve as the drive to the internal power series pass device (see the Functional Diagram). The regulation point is factory-set at (VCS+ - VCS-) = 203mV ±3.5%. The regulated current is user defined by the value of RSENSE.

The MAX16803 is a current controller internally opti- mized for driving the impedance  range expected  from one or more HB LEDs.

+5V Regulator The MAX16803 includes  a fixed +5V output regulator that delivers up to 4mA of load current throughout the

+6.5V to +40V input voltage range. Connect a 0.1µF compensation  capacitor  from V5 to ground.  Shorting V5 to ground disables the thermal shutdown. When EN is low, V5 is off. V5 stays on during PWM dimming.

Thermal Protection The MAX16803 enters a thermal-shutdown mode in the event of overheating. This typically occurs in overload or output short-circuit  conditions. If the junction tempera- ture exceeds TJ = +155°C (typ), the internal thermal-pro- tection circuitry  turns off the series pass device. The MAX16803 recovers from thermal-shutdown mode once the junction temperature drops by 23°C (typ). The part therefore protects itself by thermally cycling in the event of a short-circuit or overload condition.

Applications Information

Programming the LED  Current The MAX16803 uses a sense resistor across CS+ and CS- to set the LED current. The differential sense ampli- fier connected  across RSENSE provides  ground-loop immunity and low-frequency noise rejection. The LED current is given by

ILED = VSENSE / RSENSE

6

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

Input-Voltage Considerations For proper operation, the minimum input voltage must always be:

VIN(MIN) ≥ VRSENSE(MAX) + VFT(MAX) + ∆VDO(MAX)

where VFT(MAX) is the total forward voltage of all series connected LEDs. The minimum operating voltage of the device is +6.5V. The device operates below +6.5V; how- ever, output current may not meet the full regulation specification (see the Typical Operating Characteristics).

Low-Frequency PWM  Dimming at the Output

The MAX16803 provides  pulsed  or chopped  current

MAX16803

Paralleling Multiple MAX16803s to Drive

One  High-Power LED For applications  that require more than 350mA of LED current, two or more MAX16803s can be paralleled (see Figure 3). VCS- should not exceed 4.1V.

R1

IN                                OUT

dimming input (DIM). The other method is to connect DIM to V5 and EN to IN and pulse EN. Both methods generate a regulated-amplitude  PWM current (variable duty cycle)  that can provide  control over the LED brightness (see Figures 1 and 2).

EN

C1

V5

DIM

MAX16803

V5

C2

CS+

VDIM

GND

CS-

VIN

1      OUT

2      IN

16         15

OUT        EN

14

N.C.

13

DIM

GND  12

V5   11

ILED

D1

C1

3      IN

MAX16803

CS-   10

C2

RSENSE                             V5

R2

IN                                OUT

4      N.C.

CS+    9

N.C.      N.C.

5           6

N.C.

7

N.C.

8

EN                                 V5

C3                                                      C4

Figure 1. Dimming with EN Connected to VIN at a Constant

Voltage and DIM Pulsed

V5

DIM

MAX16803

CS+

LEDs

VIN

1      OUT

2      IN

16         15

OUT        EN

14

N.C.

13

DIM

GND  12

V5   11

ILED

D1

GND

CS-

C1

3      IN

MAX16803

CS-   10

C2

RSENSE

Figure 3. Paralleling MAX16803s

4      N.C.

CS+    9

N.C.      N.C.

5           6

N.C.

7

N.C.

8

Figure 2. Dimming with DIM Connected to V5, EN Connected to VIN

7

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

Two Brightness Levels for

TAIL/STOP Lights Figure 4 shows PWM dimming  operation  for the MAX16803 with an ICM7555 timer. The ICM7555 pro- vides adjustable duty cycle using two external resistors and a capacitor.  In TAIL operation, the output of the ICM7555 feeds into DIM and lights up the LEDs. The LED’s brightness  depends  on the duty cycle  of the ICM7555. When VSTOP is present, DIM is pulled up to VSTOP. The PWM dimming operation is disabled  and the LEDs light up to full brightness.  See the ICM7555 data sheet for formulas to calculate the dimming fre- quency and the duty cycle.

LED  Current Thermal Foldback With a minimum number of external components,  the MAX16803 provides  LED current thermal foldback using a negative temperature coefficient (NTC) thermis- tor. Figure 5 shows a thermistor connected  to V5 and the CS+ of the MAX16803. As the temperature increas- es, the voltage drop across R2 increases causing  the LED current to decrease.

ILED = [VSENSE - [R2 / (R2 + RT)] x V5] / R1

D1

STOP

TAIL

D2

IN                                                OUT

R2

DIS R3

IN

ICM7555

R1

C1

D3

D4

OUT

EN

DIM

MAX16803

V5

CS+

+5V REG

C2

LEDs

RSENSE

TH TRG

C3

GND

GND

CS-

Figure 4. PWM Dimming Operation with ICM7555 Timer

VIN

IN                                                OUT

PWM DIMMING C1

EN

DIM

MAX16803

V5

CS+

LEDs

C2

RT

R2

R1

GND

CS-

Figure 5. LED Current Thermal Foldback with an NTC Thermistor

8

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

Other Applications The application  circuit  in Figure 6 implements a two- level brightness  current for TAIL/STOP lights. In TAIL operation, Q1 is off and the R1 sets the output current. In STOP operation, Q1 turns on and the output current is set by a parallel combination of R1 and R2.

MAX16803

Figure 7 shows an application circuit with the MAX16803 using a single BJT to provide  high output current. For proper operation:

VIN(MIN) > VCESAT(MAX) + VFT(MAX) + VRSENSE

where VCESAT(MAX) is the maximum saturation voltage of the external BJT.

STOP

TAIL

IN                                                OUT

EN                                                V5

V5                            MAX16803                                    C2

LEDs

C1

DIM

CS+

R2

R1

Q1

GND

CS-

Figure 6. Two Brightness  Level with Current Level Switch for Tail/Stop Lights

LEDs

VIN

IN                                                OUT                                                    Q1

PWM DIMMING C1

EN

DIM

MAX16803

V5

C2

CS+

RSENSE

GND

CS-

Figure 7. Increased LED Current (Amper Range) with a Single BJT

9

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

Multichannel HB  LED  Driver Figure 8 shows an array of MAX16803s with independent DIM control. The MAX5094C, a current-mode PWM con- troller, provides the input power to each LED driver pre- regulated voltage to multiple MAX16803 drivers.

VIN

L1                     D1

CREF

CCOMP

C2

RCOMP

REF

COMP

FB

MAX5094C

VCC

OUT

GND

C3

Q1                    C4

IN R3

EN C6

DIM

U2

MAX16803

OUT

V5

CS+

C7

LEDs

C10

IN

EN

DIM

U4

MAX16803

OUT

V5

CS+

C11

LEDs

C8

IN

EN

DIM

U3

MAX16803

OUT

V5

CS+

C9

LEDs

R5                                                                                                                     R7                                                                                                                      R6

CT

CT/RT                                          CS

R4                                       GND R2

CS-

GND

CS-

GND

CS-

Figure 8. Multichannel HB LED Driver for LCD Backlight

Typical Operating Circuit

Chip Information

PROCESS: BiCMOS

VDIM

16       15

OUT         EN

14

N.C.

13

DIM

VIN

C1

1     OUT

2     IN

3     IN

4     N.C.

MAX16803

GND   12

V5   11

CS- 10

CS+    9

D1

C2

RSENSE

N.C.       N.C.

5         6

N.C.

7

N.C.

8

10

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

QFN THIN.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

11

High-Voltage, 350mA, High-Brightness LED Driver with PWM  Dimming and 5V  Regulator

MAX16803

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

12                                     Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2006 Maxim Integrated  Products                                                               is a registered  trademark  of Maxim Integrated  Products, Inc.