19-4235; Rev 1; 2/09

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

General Description

The MAX16834 is a current-mode high-brightness  LED (HB LED) driver for boost, buck-boost, SEPIC, and high- side buck topologies. In addition to driving an n-channel power MOSFET switch controlled by the switching con- troller, it also drives an n-channel PWM dimming switch to achieve LED PWM dimming. The MAX16834 integrates all the building blocks necessary to implement a fixed-fre- quency HB LED driver with wide-range dimming control. The MAX16834 features constant-frequency  peak cur- rent-mode control with programmable  slope compensa- tion to control the duty cycle of the PWM controller.

A dimming driver designed to drive an external n-chan- nel MOSFET in series with the LED string provides wide-range dimming control up to 20kHz. In addition to PWM dimming,  the MAX16834 provides analog dim- ming using a DC input at REFI. The programmable switching  frequency  (100kHz to 1MHz) allows design optimization for efficiency and board space reduction. A single resistor from RT/SYNC to ground  sets the switching  frequency  from 100kHz to 1MHz while an external clock  signal at RT/SYNC disables  the internal oscillator and allows the MAX16834 to synchronize to an external clock.  The MAX16834’s integrated  high- side current-sense amplifier eliminates the need for a separate high-side  LED current-sense amplifier in buck-boost applications.

The MAX16834 operates over a wide supply range of

4.75V to 28V and includes a 3A sink/source gate driver for driving a power MOSFET in high-power LED driver applications.  It can also operate at input voltages greater than 28V in boost configuration with an external voltage clamp. The MAX16834 is also suitable for DC- DC converter applications  such as boost or buck- boost. Additional features include external enable/disable  input, an on-chip oscillator, fault indica- tor output (FLT) for LED open/short or overtemperature

MAX16834

Features

♦ Wide Input Operating Voltage Range (4.75V to

28V)

♦ Works for Input Voltage >28V with External

Voltage Clamp on VIN for Boost Converter

♦ 3000:1 PWM Dimming/Analog Dimming

♦ Integrated PWM Dimming MOSFET Driver

♦ Integrated High-Side Current-Sense Amplifier for

LED Current Sense in Buck-Boost Converter

♦ 100kHz to 1MHz Programmable High-Frequency

Operation

♦ External Clock Synchronization Input

♦ Programmable UVLO

♦ Internal 7V Low-Dropout Regulator

♦ Fault Output (FLT) for Overvoltage, Overcurrent, and Thermal Warning Faults

♦ Programmable True Differential Overvoltage

Protection

♦ 20-Pin TQFN-EP and TSSOP Packages

Ordering Information

PART TEMP RANGE PIN-PACKAGE

MAX16834ATP+ -40°C to +125°C 20 TQFN-EP*

MAX16834AUP+ -40°C to +125°C 20 TSSOP-EP*

MAX16834AUP/V+ -40°C to +125°C 20 TSSOP-EP*

+Denotes a lead(Pb)-free/RoHS-compliant package.

*EP = Exposed pad.

/V denotes an automotive qualified part.

Simplified Application Circuit

VIN                                                            BOOST LED DRIVER

LED+

conditions,  and  an  overvoltage  protection  sense  input

(OVP+) for true overvoltage protection.

The MAX16834 is available in a thermally enhanced                                 IN

4mm x 4mm, 20-pin TQFN-EP package and in a thermal-

NDRV

LEDs

ly enhanced 20-pin TSSOP-EP package and is specified over the automotive -40°C to +125°C temperature range.

ON

OFF

MAX16834

PWMDIM                 CS

LED-

Applications

Single-String LED LCD Backlighting Automotive Rear and Front Lighting Projection System RGB LED Light Sources

Architectural and Decorative Lighting (MR16, M111)

Spot and Ambient Lights

ANALOG DIM

REFI PGND

DIMOUT SENSE+

DC-DC Boost/Buck-Boost Converters

Pin Configurations appear at end of data sheet.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim Direct at 1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

ABSOLUTE MAXIMUM RATINGS

IN, HV, LV to SGND ................................................-0.3V to +30V OVP+, SENSE+, DIMOUT, CLV to SGND ..............-0.3V to +30V SENSE+ to LV........................................................-0.3V to +0.3V HV, IN to LV ............................................................-0.3V to +30V OVP+, CLV, DIMOUT to LV ......................................-0.3V to +6V PGND to SGND .....................................................-0.3V to +0.3V VCC to SGND ..........................................................-0.3V to +12V NDRV to PGND...........................................-0.3V to (VCC + 0.3V) All Other Pins to SGND.............................................-0.3V to +6V NDRV Continuous Current ................................................±50mA DIMOUT Continuous Current..............................................±2mA VCC Short-Circuit  Current to SGND Duration ...........................1s Continuous Power Dissipation  (TA = +70°C)

20-Pin TQFN (4mm x 4mm)

(derate 25.6mW/°C* above +70°C) ............................2051mW

20-Pin TSSOP-EP (derate 26.5mW/°C above +70°C) ....2122mW Junction-to-Ambient Thermal Resistance (JA) (Note 1)

20-Pin TQFN 4mm x 4mm .................................................39°C/W

20-Pin TSSOP..................................................................37.7°C/W Junction-to-Case Thermal Resistance (JC) (Note 1)

20-Pin TQFN 4mm x 4mm ...............................................6°C/W

20-Pin TSSOP..................................................................2°C/W Operating  Temperature  Range .........................-40°C to +125°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

*As per JEDEC51 standard (multilayer board).

Note 1: Package thermal resistances  were obtained  using the method described in JEDEC specification JESD51-7, using a four- layer board. For detailed  information on package thermal considerations, refer to  www.maxim-ic.com/thermal-tutorial.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN =  VHV =  12V, VUVEN =  5V, VLV =  VPWMDIM =  SGND,  CVCC  =  4.7µF, CLCV =  100nF,  CREF =  100nF,  RSENSE+ =  0.1Ω, RRT = 10kΩ, TA = TJ = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Input Voltage Range | VIN |  | 4.75 28 | V |
| Quiescent Supply Current | IQ | Excluding ILED | 6 10 | mA |
| Shutdown Supply Current | ISHDN | VUVEN = 0 | 30 60 | µA |
| INTERNAL LINEAR REGULATOR (VCC) |  |  |  |  |
| Output Voltage | VCC | 0 ≤ ICC ≤ 50mA, 9.5V ≤ VIN ≤ 28V | 6.3 7 7.7 | V |
| Dropout Voltage | VDO | ICC = 35mA (Note 2) | 0.65 1 | V |
| Short-Circuit Current |  | VCC = 0, VIN = 12V | 80 300 | mA |
| LINEAR REGULATOR (CLV) |  |  |  |  |
| Output Voltage | (VCLV - VLV) | 0 ≤ ICLV ≤ 2mA, 6V ≤ VHV ≤ 28V, 6V ≤ V(HV-LV) ≤ 22V | 4.7 5 5.3 | V |
| Dropout Voltage | VDO | ICLV = 2mA, 0 ≤ VLV ≤ 23.3V (Note 3) | 0.5 | V |
| Short-Circuit Current |  | VCLV = 12V, VIN = 12V, VHV = 24V | 2.2 10 | mA |
| REFERENCE VOLTAGE (REF) |  |  |  |  |
| Output Voltage | VREF | 0 ≤ IREF ≤ 1mA, 4.75V ≤ VIN ≤ 28V | 3.625 3.70 3.775 | V |
| REF Short-Circuit Current |  | VREF = 0 | 30 | mA |
| UNDERVOLTAGE LOCKOUT/ENABLE INPUT (UVEN) |  |  |  |  |
| UVEN On Threshold Voltage | VUVEN_THUP |  | 1.395 1.435 1.475 | V |
| UVEN Threshold Voltage Hysteresis |  |  | 200 | mV |
| Input Leakage Current | ILEAK | VUVEN = 0 | I1I | µA |
| PWMDIM |  |  |  |  |
| PWMDIM On Threshold Voltage | VPWMDIM |  | 1.395 1.435 1.475 | V |
| PWMDIM Threshold Voltage Hysteresis |  |  | 200 | mV |

2

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

ELECTRICAL CHARACTERISTICS (continued)

(VIN =  VHV =  12V, VUVEN =  5V, VLV =  VPWMDIM =  SGND,  CVCC  =  4.7µF, CLCV =  100nF,  CREF =  100nF,  RSENSE+ =  0.1Ω, RRT = 10kΩ, TA = TJ = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Input Leakage Current |  | VPWMDIM = 0 | I1I | µA |
| OSCILLATOR |  |  |  |  |
| Oscillator Frequency | fOSC | RRT/SYNC = 5kΩ | 0.9 1 1.1 | MHz |
|  |  | RRT/SYNC = 25kΩ | 180 200 220 | kHz |
| Oscillator Frequency Range |  | (Note 4) | 100 1000 | kHz |
| External Sync Input Clock High Threshold |  | (Note 4) | 2 | V |
| External Sync Input Clock Low Threshold |  | (Note 4) | 0.4 | V |
| External Sync Input High Pulse Width |  | (Note 4) | 200 | ns |
| Maximum External Sync Period |  |  | 50 | µs |
| SLOPE COMPENSATION (SC) |  |  |  |  |
| SC Pullup Current | ISCPU | VSC = 100mV | 80 100 120 | µA |
| SC Discharge Resistance | RSCD | VSC = 100mV | 8 | Ω |
| REFI |  |  |  |  |
| REFI Input Bias Current |  | VREFI = 1V | I1I | µA |
| REFI Input Common-Mode Range |  | (Note 4) | 0 2 | V |
| SENSE+ |  |  |  |  |
| SENSE+ Input Bias Current |  | (VSENSE+ - VLV) = 100mV | 250 | µA |
| HIGH-SIDE LED CURRENT-SENSE AMPLIFIER (VSENSE+ - VLV) |  |  |  |  |
| Input Offset Voltage |  | VLV > 5V, (VSENSE+ - VLV) = 5mV | -2.4 0 +2.4 | mV |
| Voltage Gain | AV | VLV > 5V, (VSENSE+ - VLV) = 0.2V | 9.7 9.9 10.1 | V/V |
| 3dB Bandwidth |  | (VSENSE+ - VLV) = 0.1V, no load | 1.8 | MHz |
|  |  | (VSENSE+ - VLV) = 0.02V, no load | 600 | kHz |
| LOW-SIDE LED CURRENT-SENSE AMPLIFIER |  |  |  |  |
| Input Offset Voltage |  | VLV < 1V, (VSENSE+ - VLV) = 0V | -2 0 +2 | mV |
| Voltage Gain | AV | VLV < 1V, (VSENSE+ - VLV) = 0.2V | 9.7 9.9 10.1 | V/V |
| 3dB Bandwidth |  |  | 600 | kHz |
| CURRENT ERROR AMPLIFIER (TRANSCONDUCTANCE AMPLIFIER) |  |  |  |  |
| Transconductance | gm | VCOMP = 2V, VPWMDIM = 5V | 400 500 600 | µS |
| Open-Loop DC Gain | AV |  | 60 | dB |
| Input Offset Voltage |  |  | -10 0 +10 | mV |
| COMP Voltage Range | VCOMP | (Note 4) | 0.4 2.5 | V |
| PWM COMPARATOR |  |  |  |  |
| Input Offset Voltage |  |  | 0.6 0.65 0.70 | V |
| Propagation Delay | tPD | 50mV overdrive | 40 | ns |

3

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

ELECTRICAL CHARACTERISTICS (continued)

(VIN =  VHV =  12V, VUVEN =  5V, VLV =  VPWMDIM =  SGND,  CVCC  =  4.7µF, CLCV =  100nF,  CREF =  100nF,  RSENSE+ =  0.1Ω, RRT = 10kΩ, TA = TJ = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Minimum On-Time | tON(MIN) | On-time includes blanking time | 100 | ns |
| Duty Cycle |  | (Note 4) | 90 99.5 | % |
| CURRENT PEAK LIMIT COMPARATOR |  |  |  |  |
| Trip Threshold Voltage |  |  | 0.25 0.3 0.35 | V |
| Propagation Delay |  | 50mV overdrive with respect to NDRV | 40 | ns |
| OVERVOLTAGE PROTECTION INPUT (OVP+) |  |  |  |  |
| OVP+ On Threshold Voltage | VOVP_ON |  | 1.375 1.435 1.495 | V |
| OVP+ Hysteresis |  |  | 200 | mV |
| OVP+ Input Leakage Current |  | (VOVP - VLV) = 1.235V | -1 +1 | µA |
| HIGH-SIDE LED SHORT COMPARATOR |  |  |  |  |
| Off Threshold |  | VCLV - VLV | 4.0 4.3 4.6 | V |
| On Threshold |  | VCLV - VLV | 4.1 4.4 4.7 | V |
| Error Reject Blankout |  | fOSC = 500kHz | 256 | µs |
| LOW-SIDE LED SHORT COMPARATOR |  |  |  |  |
| Off Threshold |  |  | 0.27 0.30 0.33 | V |
| Error Reject Blankout |  |  | 5 | µs |
| HICCUP TIMER |  |  |  |  |
| Hiccup Time |  | fOSC = 500kHz | 8.2 | ms |
| GATE-DRIVER OUTPUT (NDRV) |  |  |  |  |
| NDRV Peak Pullup Current |  | VCC = 7V | 3 | A |
| NDRV Peak Pulldown Current |  | VCC = 7V | 3 | A |
| p-Channel MOSFET RDSON |  | (VCC - VNDRV) = 0.1V | 1.2 1.9 | Ω |
| n-Channel MOSFET RDSON |  | VNDRV = 0.1V | 0.9 1.7 | Ω |
| DIMOUT |  |  |  |  |
| DIMOUT Peak Pullup Current |  | (VCLV - VLV) = 5V | 25 50 | mA |
| DIMOUT Peak Pulldown Current |  | (VCLV - VLV) = 5V | 25 50 | mA |
| p-Channel MOSFET RDSON |  | (VCLV - VDIMOUT) = 0.1V | 31 | Ω |
| n-Channel MOSFET RDSON |  | (VDIMOUT - VLV) = 0.1V | 25 | Ω |
| PWMDIM to DIMOUT Propagation Delay |  |  | 200 | ns |
| FAULT FLAG (FLT) |  |  |  |  |
| FLT Pulldown Current |  | VFLT = 0.2V | 2 5 10 | mA |
| FLT Leakage Current |  | VFLT = 1.0V | I1I | µA |
| Thermal Warning On Threshold |  |  | +140 | °C |
| Thermal Warning Threshold Hysteresis |  |  | 20 | °C |

Note 2: Dropout voltage is defined as VIN - VCC, when VCC is 100mV below the value of VCC for VIN = 9.5V.

Note 3: Dropout is defined as VHV - VCLV, when VCLV is 100mV below the value of VCLV for VHV = 8V.

Note 4: Not production tested. Guaranteed by design.

4

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

Typical Operating Characteristics (VIN  =  VHV =  12V, VUVEN =  5V, VLV =  VPWMDIM =  SGND,  CVCC  =  4.7µF, CLCV =  100nF,  CREF =  100nF,  RSENSE+ =  0.1Ω, RRT = 10kΩ, TA = +25°C, unless otherwise noted.)

3.74

3.72

3.70

3.68

VREF (V)

3.66

3.64

3.62

3.60

VREF vs. TEMPERATURE

VIN = 12V

MAX16834 toc01

3.80

3.75

3.70

VREF (V)

3.65

3.60

3.55

3.50

| VREF vs. SUPPLY VOLTAGE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

MAX16834 toc02

3.7020

3.7015

3.7010

VREF (V)

3.7005

3.7000

3.6995

3.6990

3.6985

3.6980

VREF vs. IREF

MAX16834 toc03

VIN = 12V

-40 -25 -10  5

20  35  50  65  80

95 110 125

4         8        12

16       20       24       28

0    1    2    3

4    5    6

7    8    9   10

TEMPERATURE (C)

SUPPLY VOLTAGE (V)

IREF (mA)

SUPPLY CURRENT

vs. SUPPLY VOLTAGE

| 20 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PWMDIM = 0 |  |  |

18

16

SUPPLY CURRENT (mA)

14

12

10

8

6

4

SUPPLY CURRENT

vs. TEMPERATURE

10

9

MAX16834 toc04

8

7

6

5

4

SUPPLY CURRENT (mA)

3

2

MAX16834 toc05

RT (k)

100

10

MAX16834 toc06

RT vs. SWITCHING FREQUENCY

PWMDIM = 0

0                                                                                          0

VIN = 12V

1

4         8        12

16       20

24       28

-40 -25 -10  5   20  35  50

65  80

95 110 125

100

1000

SUPPLY VOLTAGE (V)

TEMPERATURE (C)

SWITCHING FREQUENCY (kHz)

| 605 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | VIN = 12V |  |  |

604

603

SWITCHING FREQUENCY (kHz)

602

601

600

599

598

597

596

595

594

593

592

591

590

SWITCHING  FREQUENCY

vs. TEMPERATURE

7.16

7.14

7.12

MAX16834 toc07

7.10

7.08

7.06

7.04

VCC (V)

7.02

7.00

6.98

6.96

6.94

6.92

6.90

VCC vs. ICC

MAX16834 toc08

VIN = 12V

7.2

VCC (V)

7.1

7.0

6.9

6.8

TA = +125C

TA = +25C

VCC vs. ICC

TA = +100C

TA = -40C

MAX16834 toc09

VIN = 12V

-40 -25 -10  5   20  35  50

65  80

95 110 125

0   10

20  30  40  50

60  70  80

90 100

0   10

20  30

40  50  60

70  80

90 100

TEMPERATURE (C)

ICC (mA)

ICC (mA)

5

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

Typical Operating Characteristics (continued) (VIN  =  VHV =  12V, VUVEN =  5V, VLV =  VPWMDIM =  SGND,  CVCC  =  4.7µF, CLCV =  100nF,  CREF =  100nF,  RSENSE+ =  0.1Ω, RRT = 10kΩ, TA = +25°C, unless otherwise noted.)

7.20

7.18

7.16

7.14

VCC (V)

7.12

7.10

7.08

7.06

7.04

7.02

7.00

VCC vs. VIN

MAX16834 toc10

TA = +125C      TA = +25C        TA = -40C

MAX16834 toc11

NDRV RISE/FALL TIME

vs. CAPACITANCE

50

VIN = 12V

40

30

RISE TIME

NDRV RISE TIME (ns)

20                                                FALL TIME

10

0

6         10        14

MAX16834 toc12

18        22        26

0    1    2    3

4    5    6    7    8

MAX16834 toc13

9   10

VIN (V)

CAPACITANCE (nF)

5.50

5.00

4.50

4.00

VCLV (V)

3.50

3.00

2.50

2.00

1.50

1.00

0.50

0

VIN = 12V

VCLV vs. ICLV

5.10

5.09

5.08

5.07

VCLV (V)

5.06

5.05

5.04

5.03

5.02

5.01

5.00

VCLV vs. VHV

0   0.5 1.0 1.5 2.0 2.5 3.0 3.5 4.0 4.5 5.0

ICLV (mA)

| 6 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

14        18        22        26

VHV (V)

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| TQFN | TSSOP |  |  |
| 1 | 3 | OVP+ | LED-String Overvoltage Protection Input. Connect a resistive voltage-divider between the positive output, OVP+, and LV to set the overvoltage threshold. OVP+ has a 1.435V threshold voltage with a 200mV hysteresis. |
| 2 | 4 | SGND | Signal Ground |
| 3 | 5 | COMP | Error-Amplifier Output. Connect an RC network from COMP to SGND for stable operation. See the Feedback Compensation section. |
| 4 | 6 | REF | 3.7V Reference Output Voltage. Bypass REF to SGND with a 0.1µF to 0.22µF ceramic capacitor. |
| 5 | 7 | REFI | Current Reference Input. VREFI provides a reference voltage for the current-sense amplifier to set the LED current. |
| 6 | 8 | SC | Current-Mode Slope Compensation Setting. Connect to an appropriate external capacitor from SC to SGND to generate a ramp signal for stable operation. |

6

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

Pin  Description (continued)

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| TQFN | TSSOP |  |  |
| 7 | 9 | FLT | Active-Low, Open-Drain Fault Indicator Output. See the Fault Indicator (FLT) section. |
| 8 | 10 | RT/SYNC | Resistor-Programmable Switching Frequency Setting/Sync Control Input. Connect a resistor from RT/SYNC to SGND to set the switching frequency. Drive RT/SYNC to synchronize the switching frequency with an external clock. |
| 9 | 11 | UVEN | Undervoltage-Lockout (UVLO) Threshold/Enable Input. UVEN is a dual-function adjustable UVLO threshold input with an enable feature. Connect UVEN to VIN through a resistive voltage-divider to program the UVLO threshold. Observe the absolute maximum value for this pin. |
| 10 | 12 | PWMDIM | PWM Dimming Input. Connect to an external PWM signal for dimming operation. |
| 11 | 13 | CS | Current-Sense Amplifier Positive Input. Connect a resistor from CS to PGND to set the inductor peak current limit. |
| 12 | 14 | PGND | Power Ground |
| 13 | 15 | NDRV | External n-Channel Gate-Driver Output |
| 14 | 16 | VCC | 7V Low-Dropout Voltage Regulator. Bypass to PGND with at least a 1µF low-ESR ceramic capacitor. VCC provides power to the n-channel gate driver (NDRV). |
| 15 | 17 | IN | Positive Power-Supply Input. Bypass to PGND with at least a 0.1µF ceramic capacitor. |
| 16 | 18 | HV | High-Side Positive Supply Input Referred to LV. HV provides power to high-side linear regulator |
| 17 | 19 | CLV | 5V High-Side Regulator Output. CLV provides power to the dimming MOSFET driver. Connect a 0.1µF to 1µF ceramic capacitor from CLV to LV for stable operation. |
| 18 | 20 | DIMOUT | External Dimming MOSFET Gate Driver. DIMOUT is capable of sinking/sourcing 50mA. |
| 19 | 1 | LV | High-Side Reference Voltage Input. Connect to SGND for boost configuration. Connect to IN for buck-boost configuration. |
| 20 | 2 | SENSE+ | LED Current-Sense Positive Input |
| — | — | EP | Exposed Pad. Connect EP to a large-area contiguous copper ground plane for effective power dissipation. Do not use as the main IC ground connection. EP must be connected to SGND. |

Detailed Description

The MAX16834 is a current-mode, high-brightness LED (HB LED) driver designed to control a single-string LED current regulator with two external n-channel MOSFETs.

The MAX16834 integrates all the building  blocks  nec- essary to implement a fixed-frequency  HB LED driver with wide-range  dimming  control. The MAX16834 allows implementation of different converter topologies such as SEPIC, boost, buck-boost,  or high-side buck current regulator.

The MAX16834 features a constant-frequency, peak-cur- rent-mode control with programmable slope compensa- tion to control the duty cycle of the PWM controller. A dimming driver offers a wide-range dimming control for the external n-channel MOSFET in series with the LED string. In addition  to PWM dimming,  the MAX16834 allows for analog dimming of LED current.

The MAX16834 switching frequency (100kHz to 1MHz) is adjustable using a single resistor from RT/SYNC. The MAX16834 disables the internal oscillator and synchro- nizes if an external clock  is applied  to RT/SYNC. The switching MOSFET driver sinks and sources up to 3A, making it suitable for high-power  MOSFETs driving in HB LED applications,  and the dimming control allows for wide PWM dimming at frequencies up to 20kHz.

The MAX16834 is suitable for boost and buck-boost

LED drivers (Figures 2 and 3).

The MAX16834 alone operates over a wide 4.75V to

28V supply range. With a voltage clamp that limits the IN pin voltage to less than 28V, it can operate in boost configuration  for input voltages greater than 28V. Additional  features include  external enable/disable input, an on-chip oscillator, fault indicator output (FLT) for LED open/short or overtemperature conditions, and an overvoltage protection  circuit  for true differential overvoltage protection (Figure 1).

7

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

IN

REF

REFERENCE

TO INTERNAL CIRCUITRY

TEMPERATURE                        OT SENSE

SGND

MAX16834

UVEN

VBG

7V LDO

UVLO

VCC

S

RT/SYNC SC

CS

OSC

5k

BLANK

RAMP GENERATOR

CURRENT-LIMIT COMPARATOR

0.6V

Q R

PWM COMP

NDRVB

NDRV

PGND

REFI SENSE+

NDRVB

LPF

VLV

0.3V VREF

AV = 9.9

ERROR AMPLIFIER

gm

PWMDIM OT

AND

FLTB

FLTA

AND

FLT

COMP HV

LED CURRENT- SENSE AMPLIFIERS

CLV

DIMOUT

LV

PWMDIM

VBG

HIGH-SIDE

5V REGULATOR

VBG

LV REFERENCE SWITCH

REFHI

4.3V

VIN

VLV

128 TOSC ERROR REJECT DELAY

REFHI

4096 TOSC

FLTB

OVP+

VBG

VHV

FLTB

FLTA

AND

VLV

0.3V SENSE+

| VREF |  |  |
| --- | --- | --- |
|  | 5s ERROR REJECT DELAY |  |

HICCUP

TIMER

VLV

VBG

VLV

MAX16834

Figure 1. Internal Block Diagram

8

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

The MAX16834 is also suitable for DC-DC converter applications  such as boost or buck-boost  (Figures 6 and 7). Other applications  include  boost LED drivers with automotive load dump  protection  (Figure 4) and high-side buck LED drivers (Figure 5).

Undervoltage Lockout/Enable The MAX16834 features an adjustable UVLO using the enable input (UVEN). Connect UVEN to VIN through a resistive divider  to set the UVLO threshold.  The MAX16834 is enabled  when the VUVEN exceeds  the

1.435V (typ) threshold.  See the Setting the UVLO Threshold section for more information.

UVEN also functions as an enable/disable  input to the device. Drive UVEN low to disable the output and high to enable the output.

Reference Voltage (REF) The MAX16834 features a 3.7V reference output, REF. REF provides power to most of the internal circuit blocks except for the output drivers and is capable of sourcing

1mA to external circuits.  Connect a 0.1µF to 0.22µF ceramic capacitor  from REF to SGND. Connect REF to REFI through a resistive divider to set the LED current.

Reference Input (REFI) The output current is proportional  to the voltage at REFI. Applying an external DC voltage at REFI or using a potentiometer from REF to SGND allows analog dim- ming of the output current.

High-Side Reference Voltage Input (LV) LV is a reference input. Connect LV to SGND for boost and SEPIC topologies. Connect LV to IN for buck-boost and high-side buck topologies.

Dimming Driver Regulator

Input Voltage (HV) The voltage at HV provides the input voltage for the dimming driver regulator. For boost or SEPIC topology, connect HV either to IN or to VCC. For buck-boost, con- nect HV to a voltage higher than IN. The voltage at HV must not exceed  28V with respect  to PGND. For the high-side buck, connect HV to IN.

Dimming MOSFET Driver (DIMOUT) The MAX16834 requires an external n-channel MOSFET for PWM dimming. Connect the gate of the MOSFET to the output of the dimming driver, DIMOUT, for normal operation. The dimming driver is capable  of sinking or sourcing up to 50mA of current.

MAX16834

n-Channel MOSFET Switch Driver (NDRV) The MAX16834 drives an external n-channel switching MOSFET. NDRV swings between VCC and PGND. NDRV is capable of sinking/sourcing 3A of peak current, allowing the MAX16834 to switch MOSFETs in high- power applications.  The average current demanded from the supply to drive the external MOSFET depends on the total gate charge  (QG ) and the operating frequency of the converter, fSW. Use the following equa- tion to calculate  the driver  supply  current INDRV required for the switching MOSFET:

INDRV = QG x fSW

Pulse Dimming Inputs (PWMDIM) The MAX16834 offers a dimming input (PWMDIM) for pulse-width  modulating  the output current. PWM dim- ming can be achieved by driving PWMDIM with a pul- sating voltage source. When the voltage at PWMDIM is greater than 1.435V, the PWM dimming MOSFET turns on and when the voltage on PWMDIM is below 1.235V, the PWM dimming MOSFET turns off.

High-Side Linear Regulator (VCLV) The MAX16834’s 5V high-side regulator (CLV) powers up the dimming MOSFET driver. VCLV is measured with respect  to LV and sources up to 2mA of current. Bypass CLV to LV with a 0.1µF to 1µF low-ESR ceramic capacitor.  The maximum voltage on CLV with respect to PGND must not exceed 28V. This limits the input volt- age for buck-boost topology.

Low-Side Linear Regulator (VCC) The MAX16834’s 7V low-side linear regulator (VCC) pow- ers up the switching MOSFET driver with sourcing capa- bility of up to 50mA. Use at least a 1µF low-ESR ceramic capacitor from VCC to PGND for stable operation.

LED  Current-Sense  Input (SENSE+) The differential voltage from SENSE+ to LV is fed to an internal current-sense amplifier. This amplified signal is then connected  to the negative input of the transcon- ductance error amplifier. The voltage gain factor of this amplifier is 9.9 (typ).

Whenever VLV is greater than 5V, the input impedance of the LED current-sense amplifier seen at the SENSE+ pin is 1kΩ ±30%.  In that condition,  a bias current of

20µA (±30%) is pulled from SENSE+, in addition to the current due to the 1kΩ resistor. When VLV is less than

1V, the amplifier input (SENSE+ pin) is in high imped- ance and the bias current of 20µA (±30%)  is pushed out of that pin.

9

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

Internal Transconductance Error Amplifier The MAX16834 has a built-in transconductance amplifi- er used to amplify the error signal inside the feedback loop. The amplified current-sense signal is connected to the negative input of the gm amplifier with the current reference connected to REFI. The output of the op amp is controlled by the input at PWMDIM. When the signal at PWMDIM is high, the output of the op amp connects to COMP; when the signal at PWMDIM is low, the out- put of the op amp disconnects from COMP to preserve the charge on the compensation capacitor.  When the voltage at PWMDIM goes high, the voltage on the com- pensation capacitor  forces the converter into a steady state. COMP is connected  to the negative input of the PWM comparator  with CMOS inputs, which draw very little current from the compensation capacitor at COMP and thus prevent discharge  of the compensation capacitor when the PWMDIM input is low.

Internal Oscillator The internal oscillator of the MAX16834 is programma- ble from 100kHz to 1MHz using a single resistor at RT/SYNC. Use the following formula to calculate  the switching frequency:

f       (kHz)    5000k  (kHz)

OSC                   RT(k)

where RT is the resistor from RT/SYNC to SGND.

The MAX16834 synchronizes to an external clock signal at RT/SYNC. The application  of an external clock  dis- ables the internal oscillator allowing the MAX16834 to use the external clock  for switching  operation. The internal oscillator is enabled  if the external clock  is absent for more than 50µs. The synchronizing  pulse minimum width for proper synchronization is 200ns.

Switching MOSFET Current-Sense Input (CS)

CS is part of the current-mode control loop. The switch-

ing control uses the voltage on CS, set by RCS, to termi- nate the on pulse width of the switching  cycle,  thus achieving peak current-mode control. Internal leading- edge blanking is provided to prevent premature turn-off of the switching MOSFET in each switching cycle.

Slope Compensation (SC) The MAX16834 uses an internal-ramp  generator for slope compensation. The ramp signal also resets at the beginning  of each cycle  and slews at the rate pro-

grammed  by the external capacitor  connected  at SC. The current source charging the capacitor is 100µA.

Overvoltage Protection (OVP+) OVP+ sets the overvoltage threshold limit across the LEDs. Use a resistive divider  between output OVP+ and LV to set the overvoltage threshold limit. An internal overvoltage protection comparator senses the differen- tial voltage across OVP+ and LV. If the differential volt- age is greater than 1.435V, NDRV is disabled and FLT asserts. When the differential voltage drops by 200mV, NDRV is enabled  and FLT deasserts. The PWM dim- ming MOSFET is still controlled by the PWMDIM input.

Fault Indicator (FLT) The MAX16834 features an active-low, open-drain fault indicator  (FLT). FLT asserts when one of the following occurs:

1)  Overvoltage across the LED string

2)  Short-circuit condition across the LED string, or

3)  Overtemperature condition

When the output voltage drops below the overvoltage set point minus the hysteresis, FLT deasserts. Similarly during the short-circuit period, the fault signal deasserts when the dimming  MOSFET is on, which happens every hiccup cycle during short circuit. During overtemperature fault, the FLT signal is the inverse of the PWM input.

Applications Information

Setting the UVLO Threshold The UVLO threshold is set by resistors R1 and R2 (see Figure 2). The MAX16834 turns on when the voltage across R2 exceeds  1.435V, the UVLO threshold. Use the following equation to set the desired UVLO thresh- old:

VUVEN  1.435V(R1 R2) / R2

In a typical application,  use a 10kΩ resistor for R2 and then calculate R1 based on the desired UVLO threshold.

Setting the Overvoltage Threshold The overvoltage threshold is set by resistors R4 and R9 (see Figure 2). The overvoltage circuit in the MAX16834 is activated when the voltage on OVP+ with respect to LV exceeds  1.435V. Use the following equation to set the desired overvoltage threshold:

VOV  1.435V(R4  R9) / R9

10

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

VIN

C1

L1

R1

LV                                            FLT

D1                                                          LED+

IN UVEN

HV C2

SC

MAX16834

Q1

NDRV

CS

ON

C3

MAX16834

LEDs

R4

R3

RT/SYNC C5

R2                                                                                  VCC

C4

REF

R6

R5

REFI SGND

PWMDIM DIMOUT

SENSE+ OVP+ CLV COMP

PGND

OFF

Q2

R8                 R9

C7

C6

R7

R10

LED-

Figure 2. Boost LED Driver

Programming the LED  Current

Calculate maximum duty cycle using the below equation.

The LED current is programmed  using the voltage on

VLED

VD    V

REFI and the LED current-sense resistor R10 (see

D                             INMIN

Figure 2). The current is given by:

MAX

VLED  VD  VFET

I                 VREF  R5          A

where VLED is the forward voltage of the LED string in

LED

   

R10  (R6  R5)  9.9

volts, VD is the forward drop of the rectifier diode D1 in volts (approximately 0.6V), VINMIN is the minimum input

where VREF is 3.7V and the resistors R5, R6, and R10

are in ohms. The regulation voltage on the LED current-

sense resistor must not exceed 0.3V to prevent activa- tion of the LED short-circuit protection circuit.

Boost Configuration In the boost converter (Figure 2), the average inductor current varies with the line voltage. The maximum aver- age current occurs at the lowest line voltage. For the boost converter, the average inductor  current is equal to the input current.

supply voltage in volts, and VFET is the average drain to source voltage of the MOSFET Q1 in volts when it is on. Use an approximate  value of 0.2V initially to calculate DMAX. A more accurate  value of the maximum duty cycle  can be calculated  once the power MOSFET is selected based on the maximum inductor current.

Use the following equations to calculate the maximum average inductor current ILAVG, peak-to-peak inductor current ripple ∆IL, and the peak inductor current ILP in amperes:

11

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

IL AVG

     ILED

1 DMAX

Allowing the peak-to-peak  inductor  ripple  ∆IL  to be

±30% of the average inductor current:

IL  IL AVG  0.3  2

Allowing the peak-to-peak  inductor  ripple  (∆IL) to be

±30% of the average inductor current:

I IL P  IL AVG    2

L

IL  IL AVG  0.3  2

The inductance value (L) of the inductor L1 in henries is calculated as:

MAX16834

and

I

L  (VINMIN

 VFET ) 

DMAX

IL P  IL AVG     L

2

The inductance  value (L) of the inductor L1 in henries

(H) is calculated as:

L  (VINMIN  VFET )  DMAX

fSW  IL

where fSW is the switching frequency  in hertz, VINMIN

and VFET are in volts, and ∆IL is in amperes.

Choose an inductor  that has a minimum inductance

fSW  IL

where fSW is the switching frequency  in hertz, VINMIN and VFET are in volts, and ∆IL is in amperes. Choose an inductor  that has a minimum inductance  greater than the calculated value.

Peak Current-Sense Resistor (R8) The value of the switch current-sense resistor R8 for the boost and buck-boost  configurations  is calculated  as follows:

greater than the calculated value. The current rating of the inductor should be higher than ILP at the operating temperature.

R8 

0.25



(IL P  1.25)

Buck-Boost Configuration In the buck-boost  LED driver (Figure 3), the average inductor  current is equal to the input current plus the LED current.

Calculate maximum duty cycle using the below equation:

D                        VLED  VD                 MAX      VLED  VD  VINMIN  VFET

where VLED is the forward voltage of the LED string in volts, VD is the forward drop of the rectifier diode D1 (approximately  0.6V) in volts, VINMIN is the minimum input supply voltage in volts, and VFET is the average drain to source voltage of the MOSFET Q1 in volts when it is on. Use an approximate value of 0.2V initially to cal- culate DMAX. A more accurate value of maximum duty cycle  can be calculated  once the power MOSFET is selected based on the maximum inductor current.

Use the below equations to calculate  the maximum average inductor current ILAVG, peak-to-peak inductor current ripple ∆IL, and the peak inductor current ILP in amperes:

where 0.25V is the minimum peak current-sense thresh- old, ILP is the peak inductor  current in amperes, and the factor 1.25 provides a 25% margin to account  for tolerances. The worst cycle-by-cycle  current limiter trig- gers at 350mV (max). The ISAT of the inductor  should be higher than 0.35V/R8.

Output Capacitor The function of the output capacitor  is to reduce  the output ripple to acceptable  levels. The ESR, ESL, and the bulk capacitance  of the output capacitor contribute to the output ripple.  In most applications,  the output ESR and ESL effects can be dramatically  reduced  by using low-ESR ceramic capacitors.  To reduce the ESL and ESR effects, connect  multiple ceramic  capacitors in parallel to achieve the required bulk capacitance.  To minimize audible  noise generated  by the ceramic capacitors  during PWM dimming, it may be necessary to minimize the number of ceramic  capacitors  on the output. In these cases an additional electrolytic or tan- talum capacitor provides most of the bulk capacitance.

Boost and buck-boost configurations: The calcula- tion of the output capacitance  is the same for both boost and buck-boost configurations. The output ripple is caused by the ESR and the bulk capacitance  of the

IL AVG

     ILED

1 DMAX

output capacitor if the ESL effect is considered negligi- ble. For simplicity, assume that the contributions  from

12

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

VIN

C1

L1

R1                                                                                                                                                                                                      D1

LED+

LV                                            HV

Q1

IN                                         NDRV

UVEN C2

SC

CS

MAX16834                                 ON

C3                         R4

MAX16834

LEDs

R3

RT/SYNC C5

R2                                                                                  VCC

C4

REF

R6

R5

REFI

FLT SGND

PWMDIM DIMOUT

SENSE+

OVP+ CLV

COMP

PGND

OFF

R8

C7

C6

R7

Q2

R9                  R10

LED-

VIN

Figure 3. Buck-Boost LED Driver (VLED+ < 28V)

ESR and the bulk capacitance  are equal, allowing 50% of the ripple for the bulk capacitance.  The capacitance is given by:

Use the below equation to calculate the RMS current rating of the output capacitor:

2

ICOUT(RMS) 

(IL

AVG  (1 - DMAX ))

 DMAX

COUT

    ILED  2  DMAX

VOUTRIPPLE  fSW

(IL AVG  DMAX )2  (1- DMAX )

Input Capacitor

where ILED is in amperes, COUT is in farads, fSW is in

hertz, and ∆VOUTRIPPLE is in volts. The remaining 50%

of allowable ripple is for the ESR of the output capaci-

tor. Based on this, the ESR of the output capacitor  is given by:

The input filter capacitor  bypasses  the ripple  current drawn by the converter and reduces  the amplitude of high-frequency  current conducted  to the input supply. The ESR, ESL, and the bulk capacitance  of the input capacitor  contribute to the input ripple. Use a low-ESR input capacitor  that can handle the maximum input

ESR

COUT

VOUTRIPPLE ()



(IL P  2)

RMS ripple current from the converter.

For the boost configuration,  the input current is the

where ILP is the peak inductor current in amperes.

same  as  the  inductor  current.  For  buck-boost

13

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

configuration, the input current is the inductor  current minus the LED current. But for both configurations, the ripple current that the input filter capacitor  has to sup- ply is the same as the inductor  ripple current with the condition that the output filter capacitor should be con- nected  to ground  for buck-boost  configuration.  This reduces the size of the input capacitor, as the inductor current is continuous  with maximum ±30%  ripple.

source and discharged at the beginning of each switch- ing cycle to generate the slope compensation ramp.

The value of the slope compensation capacitor  C2 is calculated as shown below:

Boost configuration:

3  L  100  10 -6

Neglecting the effect of LED current ripple, the calcula- tion of the input capacitor  for boost as well as buck-

C2  

(VLED - VINMIN )  R8  2

boost configurations is the same.

Neglecting the effect of the ESL, the ESR, and the bulk capacitance at the input contributes to the input voltage ripple. For simplicity, assume that the contribution from the ESR and the bulk capacitance  is equal. This allows

50% of the ripple for the bulk capacitance.  The capaci- tance is given by:

where C2 is in farads, L is the inductance of the induc- tor L1 in henries, 100µA is the pullup current from SC, VLED and VINMIN are in volts, and R8 is the switch cur- rent-sense resistor in ohms.

Buck-boost configuration:

-6

C               IL

C2  

3  L  100  10

(VLED )  R8  2

IN     4  VIN  fSW

where ∆IL is in amperes, CIN is in farads, fSW is in hertz, and ∆VIN is in volts. The remaining 50% of allowable ripple is for the ESR of the output capacitor. Based on this, the ESR of the input capacitor is given by:

ESR         VIN CIN     I   2

L

where ∆IL is in amperes, ESRCIN is in ohms, and ∆VIN

is in volts.

Use the below equation to calculate the RMS current rating of the input capacitor:

IL

where C2 is in farads, L is the inductance of the induc- tor L1 in henries, 100µA is the pullup current from SC, VLED is in volts, and R8 is the switch current-sense resistor in ohms.

Selection of Power Semiconductors

Switching MOSFET The switching MOSFET (Q1) should have a voltage rat- ing sufficient to withstand the maximum output voltage together with the diode drop of the rectifier diode D1 and any possible  overshoot due to ringing caused by parasitic  inductances  and  capacitances.  Use a MOSFET with a drain-to-source  voltage rating higher than that calculated by the following equations:

Boost configuration:

ICIN(RMS)   

2  3

Slope Compensation Slope compensation  should be added  to converters with peak current-mode control operating in continuous conduction  mode with more than 50% duty cycle  to avoid current loop instability and subharmonic  oscilla- tions. The minimum amount of slope added to the peak inductor  current to stabilize the current control loop is half of the falling slope of the inductor.

In the MAX16834, the slope compensating  ramp is added to the current-sense signal before it is fed to the PWM comparator. Connect a capacitor (C2 in the appli- cation circuit) from SC to ground for slope compensa- tion. This capacitor  is charged  with a 100µA current

VDS  VLED  VD   1.2

where VDS is the drain-to-source  voltage in volts and VD is the forward drop of the rectifier diode D1. The fac- tor of 1.2 provides a 20% safety margin.

Buck-boost configuration:

VDS  VLED  VINMAX  VD   1.2

where VDS is the drain-to-source  voltage in volts and VD is the forward drop of the rectifier diode D1. The fac- tor of 1.2 provides a 20% safety margin.

The continuous drain current rating of the selected MOSFET, when the case temperature  is at +70°C, should be greater than the value calculated by the fol-

14

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

lowing equation. The MOSFET must be mounted on a board  as per manufacturer specifications  to dissipate the heat.

The RMS current rating of the switching MOSFET Q1 is calculated as follows for boost and buck-boost configu- rations:

⎛                 2                ⎞

MAX16834

Rectifier Diode Use a Schottky diode  as the rectifier (D1) for fast switching and to reduce power dissipation. The select- ed Schottky diode  must have a voltage rating 20% above the maximum converter output voltage. The max- imum converter output voltage is VLED in boost configu- ration and VLED + VINMAX in buck-boost configuration.

The current rating of the diode should be greater than

IDRMS 

IL AVG     DMAX ⎠⎟  1.3

ID in the following equation:

where IDRMS is the MOSFET Q1’s drain RMS current in amperes.

The MOSFET Q1 will dissipate  power  due to both switching losses as well as conduction losses. The con- duction losses in the MOSFET is calculated as follows:

PCOND  IL AVG 2  DMAX  RDSON

where RDSON is the on-resistance of Q1 in ohms with an assumed junction temperature of +100°C, PCOND is in watts, and ILAVG is in amperes.

Use the following equations to calculate the switching losses in the MOSFET:

Boost configuration:

2

ID  IL AVG  (1- DMAX )  1.5

Dimming MOSFET Select a dimming MOSFET (Q2) with continuous current rating at +70°C, higher than the LED current by 30%. The drain-to-source  voltage rating of the dimming MOSFET must be higher than VLED by 20%.

Feedback Compensation The LED current control loop comprising  of the switch- ing converter, the LED current amplifier, and the error amplifier should be compensated  for stable control of the LED current. The switching converter small-signal transfer function has a right half-plane (RHP) zero for both boost and buck-boost configurations as the induc- tor current is in continuous conduction mode. The RHP

PSW

⎛ IL AVG  VLED

 ⎜

⎝                         2

 CGD  fSW ⎞

⎟

⎠

zero adds  a 20dB/decade  gain together with a 90° phase lag, which is difficult to compensate. The easiest way to avoid this zero is to roll off the loop gain to 0dB

⎛    1





⎜⎝ IGON

1    ⎞

IGOFF ⎠⎟

at a frequency less than one-fifth of the RHP zero fre- quency with a -20dB/decade slope.

The worst-case RHP zero frequency (fZRHP) is calculat-

Buck-boost configuration:

2

ed as follows:

PSW

⎛ IL AVG  (VLED  VINMAX )

 ⎜

 CGD  fSW ⎞

⎟

Boost configuration:

⎝                                   2                               ⎠

V       (1- D       )2

⎛    1



⎜⎝ IGON

1    ⎞

IGOFF ⎠⎟



fZRHP 

LED             MAX

2  L  ILED

where IGON and IGOFF are the gate currents of the

MOSFET Q1 in amperes when it is turned  on and

Buck-boost configuration:

VLED  (1- DMAX )2

turned off, respectively,  VLED and VINMAX are in volts, ILAVG is in amperes, fSW is in hertz, and CGD is the

fZRHP 

2  L  ILED

 DMAX

gate-to-drain MOSFET capacitance in farads.

Choose a MOSFET that has a higher power rating than that calculated  by the following equation when the MOSFET case temperature is at +70°C:

PTOT (W)  PCOND(W)  PSW (W)

where fZRHP is in hertz, VLED is in volts, L is the induc- tance value of L1 in henries (H), and ILED is in amperes.

The switching  converter small-signal transfer function also has an output pole for both boost and buck-boost configurations.  The effective output impedance  that determines the output pole frequency together with the output filter capacitance is calculated as:

15

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

Boost configuration:

C7              1

ROUT 

(RLED  R10)  VLED

(RLED  R10)  ILED  VLED

2  R7  fP2

where C7 is in farads, fP2 is in hertz, and R7 is in ohms.

Buck-boost configuration:

(R

 R10)  V

To minimize switching  frequency  noise, an additional capacitor can be added in parallel with the series com-

ROUT 

LED                   LED

(RLED  R10)  ILED  DMAX  VLED

bination of R7 and C7. The pole from this capacitor and R7 must be a decade  higher than the loop crossover frequency.

where RLED is the dynamic impedance (rate of change

of voltage with current) of the LED string at the operat-

ing current, R10 is the LED current-sense resistor in ohms, VLED is in volts, and ILED is in amperes.

The output pole frequency  for both boost and buck- boost configurations is calculated as follows:

Short-Circuit Protection

Boost Configuration In the boost configuration (Figure 2), if the LED string is shorted then the excess current flowing in the LED cur- rent-sense resistor will cause NDRV to stop switching. The input voltage will appear on the output capacitor,

fP2 

1

2  COUT  ROUT

and this causes very high peak currents to flow in the LED current-sense resistor R10 because  the dimming MOSFET (Q2) is on. Once the voltage across the LED

where fP2 is in hertz, COUT  is the output filter capaci- tance in farads, ROUT is the effective output impedance in ohms calculated above.

Compensation components  R7 and C7 perform two functions. C7 introduces  a low-frequency  pole that introduces a -20dB/decade slope into the loop gain. R7 flattens the gain of the error amplifier for frequencies above the zero formed by R7 and C7. For compensa- tion, this zero is placed at the output pole frequency fP2 such that it provides a -20dB/decade slope for frequen- cies above fP2 for the complete loop gain.

The value of R7 needed to fix the total loop gain at fP2

such   that   the   total   loop   gain   crosses   0dB   at

-20dB/decade  at one-fifth of the RHP zero can be cal- culated as follows:

fZRHP  R8

current-sense resistor exceeds  300mV for more than

5µs, then the dimming MOSFET Q2 turns off and stays off for 4096 switching clock  cycles. At the same time, NDRV is also off. The MAX16834 goes into the hiccup mode and recovers from hiccup  once the short has been removed. The power dissipation  in the dimming MOSFET (Q2) is minimized during  a short across the LED string. During the same period, FLT only goes high when the dimming MOSFET is on.

Buck-Boost Configuration In the case of the buck-boost  configuration  (Figure 3), once an LED string short occurs then the behavior is different. A short across the LED string causes a high current spike due to the external capacitors at the out- put. The regulation  loop will cause  NDRV to stop switching. This causes the voltage on HV to drop if its voltage is derived from LED+. The voltage on CLV will

R7 

5  fP2

 (1 D

MAX )  R10  9.9  GMCOMP

drop, and this drop is detected  after 128 clock cycles. The dimming MOSFET and the switching MOSFET will

where R7 is the compensation resistor in ohms, fZRHP and fP2 are in hertz, R8 is the switch current-sense resistor in ohms, R10 is the LED current-sense resistor in ohms, factor 9.9 is the gain of the LED current ampli- fier, and GMCOMP is the transconductance  of the error amplifier in Siemens.

The value of C7 can be calculated as:

stop switching.  It stays off for 4096 clock  cycles, and the cycle repeats itself. The short across the LED string will cause the MAX16834 to go into a hiccup mode. At the same time the FLT signal asserts itself for 4096 clock cycles every hiccup cycle. In the case where the HV voltage is derived  from a source different than LED+, then the LED current will stay in regulation even during a short across the LED string. In this case, FLT does not assert itself during the short.

16

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

VIN

C1

L1

R1

D2

24V

Q3

C8                LV

IN UVEN

HV C2

SC

MAX16834

FLT

NDRV

CS

D1

MAX16834

LED+

C3

Q1

R4

ON

LEDs

R3

RT/SYNC C5

R2                                                                                  VCC

C4

REF

R6

R5

REFI SGND

PWMDIM DIMOUT

SENSE+ OVP+ CLV COMP

PGND

OFF

Q2

R8                 R9

C7

C6

R7

R10

LED-

Figure 4. Boost LED Driver with Automotive Load Dump Protection

17

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

VIN

LED+

C1

D1                          C3

R1

LV                                            HV

IN                                         NDRV

L1

Q1

VLV

LEDs

UVEN C2

SC

R3

CS

MAX16834                                ON

RT/SYNC C5

R2                                                                                  VCC

C4

REF

R6

R5

REFI

PWMDIM

DIMOUT SENSE+

OVP+ CLV

OFF

R4

LED- Q2

FLT SGND

COMP PGND

C7                           R8

C6

R7

VLV

R9                       R10

VLV

Figure 5. High-Side Buck LED Driver

18

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

VIN

C1

L1

R1

D1

LV                                            FLT

MAX16834

VOUT

IN                                         NDRV                                     Q1

UVEN C2                            HV

SC

R3

CS

MAX16834

VREF

C3               R4

RT/SYNC C5

R2                                                                  VCC

C4

REF

PWMDIM

DIMOUT SENSE+

OVP+

R6

R5

REFI SGND

CLV COMP

PGND

C6              R8

OPTIONAL

R9              R10

R7

Figure 6. Boost DC-DC Converter

19

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

VIN

C1

L1

R1

D1

LV                                            HV

VOUT

IN                                         NDRV                                     Q1

UVEN C2

SC

R3

RT/SYNC

CS

MAX16834

VREF

C3                R4              R11

C5

R2                                                                  VCC

C4

REF

R6

R5

REFI FLT

SGND

PWMDIM

DIMOUT SENSE+ OVP+

CLV COMP

PGND

N.C.

C6              R8

R9              R10

R7                                                                                                          VIN

Figure 7. Buck-Boost DC-DC Converter

20

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

Layout Recommendations Typically, there are two sources of noise emission in a switching power supply: high di/dt loops and high dv/dt surfaces. For example, traces that carry the drain cur- rent often form high di/dt loops. Similarly, the heatsink of the MOSFET connected to the device drain presents a dv/dt source; therefore, minimize the surface area of the heatsink as much as is compatible  with the MOS- FET power dissipation or shield it. Keep all PCB traces carrying switching currents as short as possible to mini- mize current loops. Use ground planes for best results.

Careful PCB layout is critical  to achieve low switching losses and clean, stable operation. Use a multilayer board whenever possible for better noise immunity and power dissipation.  Follow these guidelines  for good PCB layout:

1)  Use a large contiguous  copper  plane under the MAX16834 package.  Ensure that all heat-dissipat- ing components have adequate cooling.

2)  Isolate the power components  and high-current path from the sensitive analog circuitry.

3)  Keep the high-current paths short, especially at the ground terminals. This practice  is essential for sta- ble, jitter-free operation. Keep switching loops short such that:

a)  The anode of D1 must be connected very close to the drain of the MOSFET Q1.

MAX16834

b)  The cathode  of D1 must be connected  very close to COUT.

c)  COUT and the current-sense resistor R8 must be connected directly to the ground plane.

4)  Connect PGND and SGND to a star-point configura- tion.

5)  Keep the power traces and load connections short.

This practice  is essential for high efficiency.  Use thick copper  PCBs (2oz vs. 1oz) to enhance full- load efficiency.

6)  Route high-speed  switching nodes away from the sensitive analog areas. Use an internal PCB layer for the PGND and SGND plane as an EMI shield to keep radiated  noise away from the device,  feed- back dividers, and analog bypass capacitors.

7)  To prevent discharge  of the compensation capaci- tors during  the off-time of the dimming  cycle, ensure that the PCB area close to these compo- nents has extremely low leakage. Discharge  of these capacitors due to leakage results in reduced performance of the dimming circuitry.

21

NDRV

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

IN

VCC

PGND

Pin  Configurations

TOP VIEW

HV   16

15       14       13

12       11

10    PWMDIM

TOP VIEW

CS

+

LV   1

SENSE+  2

OVP+   3

20   DIMOUT

19   CLV

18   HV

CLV   17

DIMOUT   18

LV    19

SENSE+   20             +

MAX16834

*EP

9     UVEN

8     RT/SYNC

7     FLT

6     SC

SGND   4

COMP   5

REF   6

REFI   7

SC   8

FLT   9

MAX16834

17  IN

16   VCC

15   NDRV

14   PGND

13  CS

12  PWMDIM

OVP+

SGND

COMP

REF

REFI

1          2         3         4         5

RT/SYNC  10

11   UVEN

*EP = EXPOSED PAD.

TQFN

TSSOP

Chip Information

PROCESS: BiCMOS–DMOS

Package Information For the latest package outline information and land patterns, go to  www.maxim-ic.com/packages.

| PACKAGE TYPE | PACKAGE CODE | DOCUMENT NO. |
| --- | --- | --- |
| 20-TQFN-EP | T2044-3 | 21-0139 |
| 20-TSSOP-EP | U20E+1 | 21-0108 |

22

High-Power LED Driver with Integrated High-Side LED Current Sense and  PWM Dimming MOSFET Driver

MAX16834

Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- | --- |
| 0 | 8/08 | Initial release | — |
| 1 | 2/09 | Added TSSOP package and automotive version. Also updated Electrical Characteristics, Pin Description, Detailed Description, and LED Current- Sense Input (SENSE+) section, Pin Configuration and Package Information. | 1, 2, 6, 7, 8, 9, 22 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600                                23

©  2009 Maxim Integrated  Products                                                  Maxim is a registered  trademark  of Maxim Integrated  Products, Inc.