19-4029; Rev 2; 4/09

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

General Description

The MAX16836 current regulator operates from a 6.5V

to 40V input-voltage range and delivers up to a total of

350mA to one or more strings of high-brightness  LEDs (HB LEDs). The output current of the MAX16836 is adjusted by using an external current-sense resistor in series with the LEDs. A dimming  input allows wide- range “pulsed”  PWM operation. Wave-shaping circuitry reduces  EMI. The differential  current-sense input increases noise immunity. The MAX16836 is well suited for applications  requiring  a high-voltage  input and is able to withstand automotive load-dump  events up to

40V. An on-board  pass element minimizes external components  while providing  ±3.5%  output-current accuracy.  Additional  features include  a 5V regulated output and short-circuit and thermal protection.

The MAX16836 is available in a thermally enhanced,

5mm x 5mm, 16-pin TQFN package and in an 8-pin SO package with exposed pad. The MAX16836 is specified over the automotive -40°C to +125°C temperature range.

Applications

Automotive Interior: Map, Courtesy, and Cluster

Lighting

Automotive Exterior: Tail Lights and CHMSL Emergency Vehicle Warning Lights Navigation and Marine Indicators

General Lighting

Signage, Gasoline Canopies, Beacons

Typical Operating Circuit

MAX16836

Features

♦ +6.5V to +40V Operating Range

♦ Adjustable LED Current (35mA to 350mA)

♦ ±3.5% LED Current Accuracy

♦ High-Voltage DIM Pin for Dimming Interface

♦ Integrated Pass Element with Low-Dropout

Voltage (0.55V typ)

♦ +5V Regulated Output with 4mA Source

Capability

♦ Parallel Operation for LED Current > 350mA

♦ Differential LED Current Sense

♦ Low Shutdown Supply Current (35µA typ)

♦ Low 200mV Current-Sense Reference Reduces

Power Losses

♦ Wave-Shaped Edges Reduce Radiated EMI During

PWM Dimming

♦ Thermal Shutdown

♦ Output Short-Circuit Protection

♦ Available in Small, Thermally Enhanced,

5mm x 5mm, 16-Pin TQFN Package and in 8-Pin

SO with Exposed Pad Package

♦ -40°C to +125°C Operating Temperature Range

Ordering Information

| PART | TEMP RANGE | PIN-PACKAGE |
| --- | --- | --- |
| MAX16836ATE+ | -40°C to +125°C | 16 TQFN-EP* |
| MAX16836ASA+ | -40°C to +125°C | 8 SO-EP* |

+Denotes a lead(Pb)-free/RoHS-compliant package.

*EP = Exposed pad.

Simplified Diagram

VDIM

+6.5V TO +40V

0.1F

IN                                          OUT

+5V REG

VIN

1     OUT

2     IN

16       15

OUT         EN

14

N.C.

13

DIM

GND   12

D1

V5   11

EN          MAX16836

V5                                           LEDs

0.1F

CS+

C1

3     IN

MAX16836

CS- 10

C2

RSENSE

PWM DIMMING

RSENSE

4     N.C.

N.C.       N.C.

5         6

N.C.

7

N.C.

8

CS+    9

DIM

GND

CS-

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim Direct at 1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

ABSOLUTE MAXIMUM RATINGS

IN to GND ...............................................................-0.3V to +45V OUT, DIM, and EN to GND ..........................-0.3V to (VIN + 0.3V) CS+, CS-, V5 to GND ...............................................-0.3V to +6V OUT Short Circuited  to GND Duration

(at VIN = +16V) .........................................................60 minutes Maximum Current into Any Pin (except IN and OUT) .........±20mA Continuous Power Dissipation  (TA = +70°C)

16-Pin TQFN 5mm x 5mm

(derate 33.3mW/°C above +70°C) ............................2666.7mW

8-Pin SO (derate 23.3mW/°C above +70°C).............1860.5mW

Junction-to-Case Thermal Resistance (JC)(Note 1)

8-Pin SO ...........................................................................7°C/W

16-Pin TQFN.....................................................................2°C/W Junction-to-Ambient Thermal Resistance (JA)(Note 1)

8-Pin SO .........................................................................43°C/W

16-Pin TQFN...................................................................30°C/W Operating  Junction Temperature  Range ...........-40°C  to +125°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Note 1:  Package thermal resistances  obtained  using the method described in JEDEC specification JESD51-7, using a four-layer board. For detailed  information on package thermal considerations, see www.maxim-ic.com/thermal-tutorial.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN =  VEN =  +12V,  CV5 =  0.1µF to GND,  IV5 =  0, VCS- =  0V, VDIM =  +4V,  connect  RSENSE =  0.58Ω  between  CS+  and  CS-. TA = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Voltage Range | VIN | (Note 3) | 6.5 40.0 | V |
| Ground Current | IG | ILOAD = 350mA | 1.28 3 | mA |
| Shutdown Supply Current | ISHDN | VEN ≤ 0.6V | 35 70 | µA |
| Guaranteed Output Current | IOUT | RSENSE = 0.55Ω | 350 | mA |
| Output Current Accuracy |  | 35mA < IOUT < 350mA | ±3.5 | % |
| Dropout Voltage (Note 4) | ∆VDO | IOUT = 350mA (current pulsed), 12V < VIN < 40V | 0.55 1.2 | V |
|  |  | IOUT = 350mA (current pulsed), 6.5V < VIN < 12V | 0.55 1.5 |  |
| Output Current Slew Rate |  | Current rising, VDIM rising to 4V | 7.8 | mA/µs |
|  |  | Current falling, VDIM falling to 0.6V | 7.8 |  |
| Short-Circuit Current |  | VIN = 12V, VCS+ = 0V | 400 500 650 | mA |
| LOGIC INPUT |  |  |  |  |
| EN Input Current | IEN |  | -2.5 -1.0 -0.2 | µA |
| EN Input Voltage High | VIH |  | 2.8 | V |
| EN Input Voltage Low | VIL |  | 0.6 | V |
| Turn-On Time | tON | VEN rising edge to 90% of OUT | 110 350 | µs |
| CURRENT SENSE |  |  |  |  |
| Regulated RSENSE Voltage | VSENSE | VSENSE = VCS+ - VCS- | 193 200 207 | mV |
| CS- Voltage Range |  |  | -0.3 +4.1 | V |
| Input Current (CS+) |  | VCS+ = 220mV | 2.7 7.0 | µA |
| Input Current (CS-) |  | VCS+ = 220mV | -50 17.2 | µA |
| DIM Input Current |  |  | -2.5 -1.0 -0.2 | µA |
| DIM Input Voltage High | VIH |  | 4 | V |
| DIM Input Voltage Low | VIL |  | 0.6 | V |
| Turn-On Time | tON | After VDIM rising to 4V (Note 5) | 110 | µs |
| Turn-Off Time | tOFF | After VDIM falling to 0.6V (Note 5) | 70 | µs |

2

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

ELECTRICAL CHARACTERISTICS (continued)

(VIN =  VEN =  +12V,  CV5 =  0.1µF to GND,  IV5 =  0, VCS- =  0V, VDIM =  +4V,  connect  RSENSE =  0.58Ω  between  CS+  and  CS-. TA = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| THERMAL OVERLOAD |  |  |  |  |
| Thermal-Shutdown Temperature |  |  | +159 | °C |
| Thermal-Shutdown Hysteresis |  |  | 24 | °C |
| +5V REGULATOR |  |  |  |  |
| Output Voltage Regulation | VV5 | (Note 6) | 4.85 5.00 5.20 | V |
| V5 Short-Circuit Current |  | VV5 = 0V (Note 7) | 15 | mA |

Note 2:  All devices  are 100% production tested at TA = +25°C. Limits over the operating  temperature  range are guaranteed by design.

Note 3:  Resistors were added  from OUT to CS+ to aid with the power dissipation  during testing.

Note 4:  Dropout is measured  as follows:

Connect RO = 27Ω from OUT to CS+. Connect RSENSE = 0.58Ω from CS+ to CS-. Set VIN = +12V (record VOUT as VOUT1). Reduce VIN until VOUT = 0.97 x VOUT1 (record as VIN2 and VOUT2). ∆VDO = VIN2 - VOUT2.

Note 5:  tON time includes  the delay and the rise time needed  for IOUT to reach 90% of its final value. tOFF time is the time needed

for IOUT to drop below 10%. See the Typical Operating Characteristics. tON and tOFF are tested with 13Ω from OUT to CS+.

Note 6:  Current regulation varies with V5 load (see the Typical Operating Characteristics).

Note 7:  Thermal shutdown does not function if the output of the 5V reference is shorted to ground.

Typical Operating Characteristics

(VIN = VEN = +12V, TA = +25°C, unless otherwise noted.)

450

400

OUTPUT CURRENT (mA)

350

300

250

200

150

100

50

0

OUTPUT CURRENT

vs. TEMPERATURE

ILOAD = 350mA

ILOAD = 200mA

ILOAD = 100mA ILOAD = 35mA

MAX16836 toc01

0.208

(VCS+ - VCS-) (V)

0.206

0.204

0.200

0.198

(VCS+ - VCS-)

vs. OUTPUT CURRENT

MAX16836 toc02

400

350

300

200

OUTPUT CURRENT (mA)

100

50

0

OUTPUT CURRENT

MAX16836 toc03

vs. INPUT VOLTAGE

-40 -25 -10  5   20  35  50

65  80

95 110 125

| 20 50 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

80 110 140 170 200 230 260 290 320 350

0      5     10    15    20    25

| 30 35 40 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

TEMPERATURE (C)

OUTPUT CURRENT (mA)

INPUT VOLTAGE (V)

3

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

DROPOUT VOLTAGE (V)

Typical Operating Characteristics (continued)

(VIN = VEN = +12V, TA = +25°C, unless otherwise noted.)

1.0

0.9

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

DROPOUT VOLTAGE

vs. TEMPERATURE

MAX16836 toc04

100

90

80

70

60

50

40

30

20

10

0

MAX16836 toc05

SHUTDOWN CURRENT

vs. TEMPERATURE

VIN = 40V

VIN = 20V

VIN = 12V

VIN = 6.5V

5.2

+5V REGULATOR OUTPUT (V)

5.1

5.0

4.8

MAX16836 toc06

+5V REGULATOR OUTPUT

vs. TEMPERATURE

IV5 = 1mA

NO LOAD

IV5 = 5mA

-40 -25 -10  5   20  35  50

65  80

| 95 110 125 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOAD = 350mA |  |  |  |  |  |  |  |  |  |
| I |  |  |  |  |  |  |  |  |  |  |

-40 -25 -10  5

20  35  50

65  80

95 110 125

-40 -25 -10  5

20  35  50

65  80

95 110 125

TEMPERATURE (C)

SHUTDOWN CURRENT (A)

TEMPERATURE (C)

TEMPERATURE (C)

0.206

MAX16836 toc07

(VCS+ - VCS-) vs. IV5

5.2

MAX16836 toc08

+5V REGULATOR OUTPUT vs. VIN

0.204

0.202

(VCS+ - VCS-) (V)

0.200

ILOAD = 350mA

5.1

5.0

NO LOAD                IV5 = 5mA

0.198

+5V REGULATOR OUTPUT (V)

4.9

IV5 = 5mA

0.196

0

1         2         3

4         5         6

4.8

5

10     15

20     25

30     35     40

IV5 (mA)

VIN (V)

200Hz DIMMED OPERATION

MAX16836 toc09

200Hz DIMMED  OPERATION (EXPANDED)

MAX16836 toc10

ILOAD = 350mA VIN = VEN = 12V DIM PULSED AT

200Hz (5% DUTY CYCLE)

VDIM

2V/div

0V

ILOAD

200mA/div

0A

ILOAD = 350mA VIN = VEN = 12V DIM PULSED AT

200Hz (50% DUTY CYCLE)

VDIM

2V/div

0V

ILOAD

200mA/div

0A

100s/div

20s/div

4

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

Typical Operating Characteristics (continued)

(VIN = VEN = +12V, TA = +25°C, unless otherwise noted.)

200Hz DIMMED  OPERATION (EXPANDED)

MAX16836 toc11

ILOAD = 350mA VIN = VEN = 12V DIM PULSED AT

200Hz (50% DUTY CYCLE)

VDIM

2V/div

0V

204

VSENSE vs. VCS-

MAX16836 toc12

ILOAD = 350mA

VSENSE (mV)

ILOAD

200mA/div

202

0A                                                                                    200

20s/div

198

0

0.5 1.0 1.5 2.0 2.5 3.0 3.5 4.0 4.5 5.0

VCS- (V)

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| TQFN | SO |  |  |
| 1, 16 | 8 | OUT | Current-Regulated Output. Connect pin 1 to pin 16. |
| 2, 3 | 1 | IN | Positive Input Supply. Bypass IN with a 0.1µF (min) capacitor to GND. Connect pin 2 to pin |
| 4–8, 14 | — | N.C. | No Connection. Not internally connected. |
| 9 | 2 | CS+ | Positive Input of the Internal Differential Amplifier. Connect the current-sense resistor between CS+ and CS- to program the output current level. |
| 10 | 3 | CS- | Negative Input of the Internal Differential Amplifier. Connect the current-sense resistor between CS- and CS+ to program the output current level. |
| 11 | 4 | V5 | +5V Regulated Output. Connect a 0.1µF capacitor from V5 to GND. |
| 12 | 5 | GND | Ground |
| 13 | 6 | DIM | Pulsed Dimming Input. Drive DIM low to disable the output. Drive DIM high or leave unconnected to enable the output. |
| 15 | 7 | EN | Enable Input. Drive EN high to enable the output. |
| — | — | EP | Exposed Pad. Connect to the ground plane for effective power dissipation. Do not use as the only ground connection. |

5

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

Functional Diagram

IN                          V5

DIM

IN

REGULATOR

1A

IN

EN                        ENABLE

REFERENCE GENERATOR

I_REG

OUT

MAX16836

DIFFERENTIAL SENSE AMPLIFIER

CS+ CS-

THERMAL SHUTDOWN

Detailed Description

The MAX16836 is a high-current  regulator capable  of providing  up to a total of 350mA of current to one or more strings of HB LEDs. A wide operating input volt- age range of +6.5V to +40V makes the MAX16836 ideal for automotive applications.  A +5V regulated out- put provides up to 4mA of current to power external cir- cuitry. In addition, the MAX16836 features thermal and output short-circuit protection. The wide operating volt- age range helps protect the MAX16836 against large transients such as those found in load-dump situations up to 40V.

The MAX16836 uses a feedback loop to control the out- put current. The differential voltage across the sense resistor is compared  to a fixed reference voltage, and the error is amplified to serve as the drive to the internal power  series  pass  device  (see  the  Functional Diagram). The regulation point is factory-set at (VCS+ - VCS-) = 200mV ±3.5%. The regulated current is user defined by the value of RSENSE.

The MAX16836 is a current controller internally opti- mized for driving the impedance  range expected  from one or more HB LEDs.

+5V Regulator The MAX16836 includes  a fixed +5V output regulator that delivers up to 4mA of load current throughout the

+6.5V to +40V input voltage range. Connect a 0.1µF compensation  capacitor  from V5 to ground.  Shorting V5 to ground disables the thermal shutdown. When EN is low, V5 is off. V5 stays on during  PWM dimming, using the DIM input.

Thermal Protection The MAX16836 enters a thermal-shutdown mode in the event of overheating. This typically occurs in overload or output short-circuit  conditions. If the junction tempera- ture exceeds TJ = +159°C (typ), the internal thermal-pro- tection circuitry  turns off the series pass device. The MAX16836 recovers from thermal-shutdown mode once the junction temperature drops by 24°C (typ). The part therefore protects itself by thermally cycling in the event of a short-circuit  or overload condition. Shorting V5 to ground disables the thermal shutdown.

Applications Information

Programming the LED  Current The MAX16836 uses a sense resistor across CS+ and CS- to set the LED current. The differential sense ampli- fier connected  across RSENSE provides ground-loop immunity and low-frequency noise rejection. The LED current is given by:

ILED = VSENSE/RSENSE

6

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

Input-Voltage Considerations For proper operation, the minimum input voltage must always be:

VIN(MIN) ≥ VSENSE(MAX) + VFT(MAX) + ∆VDO(MAX)

where VFT(MAX) is the total forward voltage of all series connected LEDs. The minimum operating voltage of the device is +6.5V. The device operates below +6.5V; how- ever, output current may not meet the full regulation specification.

Low-Frequency PWM  Dimming at the Output

The MAX16836 provides  pulsed  or chopped  current

MAX16836

Paralleling Multiple MAX16836s to Drive One  High-Power LED

For applications  that require more than 350mA of LED

current, two or more MAX16836s can be paralleled

(see Figure 3). VCS- should not exceed 4.1V.

R1

IN                                 OUT

dimming input (DIM). The other method is to connect DIM to V5 and EN to IN and pulse EN. Both methods generate a regulated-amplitude  PWM current (variable duty cycle)  that can provide  control over the LED brightness (see Figures 1 and 2).

EN

C1

V5

DIM

MAX16836

V5

C2

CS+

VDIM

GND

CS-

VIN

1      OUT

2      IN

16         15

OUT        EN

14

N.C.

13

DIM

GND  12

V5   11

ILED

D1

C1

3      IN

MAX16836

CS-   10

C2

RSENSE

VIN

R2

IN                                OUT

4      N.C.

CS+    9

N.C.      N.C.

5           6

N.C.

7

N.C.

8

EN                                 V5

C3                                                      C4

Figure 1. Dimming with EN Connected to VIN at a Constant

Voltage and DIM Pulsed

V5

DIM

MAX16836

CS+

LEDs

VIN

1      OUT

2      IN

16         15

OUT        EN

14

N.C.

13

DIM

GND  12

V5   11

ILED

D1

GND

CS-

C1

3      IN

MAX16836

CS-   10

C2

RSENSE

Figure 3. Paralleling MAX16836s

4      N.C.

CS+    9

N.C.      N.C.

5           6

N.C.

7

N.C.

8

Figure 2. Dimming with DIM Connected to V5, EN Connected to VIN

7

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

Two Brightness Levels for

TAIL/STOP Lights Figure 4 shows PWM dimming  operation  for the MAX16836 with an ICM7555 timer. The ICM7555 pro- vides adjustable duty cycle using two external resistors and a capacitor.  In TAIL operation, the output of the ICM7555 feeds into DIM and lights up the LEDs. The LED’s brightness  depends  on the duty cycle  of the ICM7555. When VSTOP is present, DIM is pulled up to VSTOP. The PWM dimming operation is disabled  and

LED  Current Thermal Foldback With a minimum number of external components,  the MAX16836 provides  LED current thermal foldback using a negative temperature coefficient (NTC) thermis- tor. Figure 5 shows a thermistor connected  to V5 and the CS+ of the MAX16836. As the temperature increas- es, the voltage drop across R2 increases causing  the LED current to decrease.

⎛ RT  R  ⎞

[              [

2 /(  2

)]     5]   ⎜            2 ⎟

the LEDs light up to full brightness.  See the ICM7555

data sheet for formulas to calculate the dimming fre-

ILED 

VSENSE

R    R   RT   V



⎝   R1RT   ⎠

quency and the duty cycle.

(assuming IRT < < ILED).

D1

STOP

TAIL

D2

IN                                                OUT

R2

DIS R3

TH TRG

C3

IN

ICM7555

R1

D3

OUT

GND

D5              C1

D4

R4*

*R4  300k

EN DIM GND

MAX16836

V5

CS+

CS-

+5V REG

C2

LEDs

RSENSE

Figure 4. PWM Dimming Operation with ICM7555 Timer

VIN

IN                                                OUT

PWM DIMMING C1

EN

DIM

MAX16836

V5

CS+

LEDs

C2

RT

R2

R1

GND

CS-

Figure 5. LED Current Thermal Foldback with an NTC Thermistor

8

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

Other Applications The application  circuit  in Figure 6 implements a two- level brightness  current for TAIL/STOP lights. In TAIL operation, Q1 is off and the R1 sets the output current. In STOP operation, Q1 turns on and the output current is set by a parallel combination of R1 and R2.

MAX16836

Figure 7 shows an application circuit with the MAX16836 using a single BJT to provide  high output current. For proper operation:

VIN(MIN) > VCESAT(MAX) + VFT(MAX) + VSENSE

where VCESAT(MAX) is the maximum saturation voltage of the external BJT.

STOP

TAIL

IN                                                OUT

EN                                                V5

V5                            MAX16836                                    C2

LEDs

C1

DIM

CS+

R2

R1

Q1

GND

CS-

Figure 6. Two Brightness Level with Current Level Switch for Tail/Stop Lights

LEDs

VIN

IN                                                OUT                                                    Q1

PWM DIMMING C1

EN

DIM

MAX16836

V5

C2

CS+

RSENSE

GND

CS-

Figure 7. Increased LED Current (Ampere Range) with a Single BJT

9

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

Multichannel HB  LED  Driver Figure 8 shows an array of MAX16836s with independent DIM control. The MAX5094C, a current-mode  PWM

controller, provides the input power to each LED driver preregulated voltage to multiple MAX16836 drivers.

VIN

L1                     D1

CREF

CCOMP

C2

RCOMP

REF

COMP

FB

MAX5094C

VCC

OUT

GND

C3

Q1                    C4

IN R3

EN C6

DIM

U2

MAX16836

OUT

V5

CS+

C7

LEDs

C10

IN

EN

DIM

U4

MAX16836

OUT

V5

CS+

C11

LEDs

C8

IN

EN

DIM

U3

MAX16836

OUT

V5

CS+

C9

LEDs

R5                                                                                                                     R7                                                                                                                      R6

CT

CT/RT                                          CS

R4                                       GND R2

CS-

GND

CS-

GND

CS-

V5

CS-

CS+

Figure 8. Multichannel HB LED Driver for LCD Backlight

GND

Pin  Configurations

TOP VIEW

12         11         10          9

TOP VIEW

DIM  13

8     N.C.

| IN OUT | + MAX16836 EP* |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

N.C. EN OUT

14

15

16           +

MAX16836

7     N.C.

6     N.C.

5     N.C.

CS+ CS- V5

EN DIM GND

SO

OUT

IN

IN

N.C.

1           2           3           4

*EXPOSED PAD

TQFN

Chip Information

PROCESS: BiCMOS

Package Information For the latest package outline information and land patterns, go to www.maxim-ic.com/packages.

| PACKAGE TYPE | PACKAGE CODE | DOCUMENT NO. |
| --- | --- | --- |
| 8 SO-EP | S8E-12 | 21-0111 |
| 16 TQFN-EP | T1655-3 | 21-0140 |

10

High-Voltage, 350mA, High-Brightness LED Driver with PWM Dimming and  5V Regulator

MAX16836

Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- | --- |
| 0 | 2/08 | Initial release | — |
| 1 | 5/08 | Updated Ordering Information, Electrical Characteristics, and Package Information. | 1, 2, 11, 12 |
| 2 | 4/09 | Added the SO package to the data sheet. | 1, 2, 5, 10 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600                                11

©  2009 Maxim Integrated  Products                                                  Maxim is a registered  trademark  of Maxim Integrated  Products, Inc.