19-1975; Rev 2; 7/03

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

General Description

The MAX1776 high-efficiency step-down converter pro- vides an adjustable output voltage from 1.25V to VIN from supply voltages as high as 24V. An internal current-limit- ed 0.4 MOSFET delivers load currents up to 600mA. Operation to 100% duty cycle minimizes dropout volt- age (240mV at 600mA).

The MAX1776 has a low 15µA quiescent current to improve light-load efficiency and conserve battery life. The device draws only 3µA while in shutdown.

High switching frequencies (up to 200kHz) allow the use of tiny surface-mount inductors and output capaci- tors. The MAX1776 is available in an 8-pin µMAX pack- age, which uses half the space of an 8-pin SO. For increased output drive capability,  use the MAX1626/ MAX1627 step-down controllers, which drive an exter- nal P-channel MOSFET to deliver up to 20W.

MAX1776

Features

♦ Fixed 5V or Adjustable Output

♦ 4.5V to 24V Input Voltage Range

♦ Up to 600mA Output Current

♦ Internal 0.4 P-Channel MOSFET

♦ Efficiency Over 95%

♦ 15µA Quiescent Supply Current

♦ 3µA Shutdown Current

♦ 100% Maximum Duty Cycle for Low Dropout

♦ Current-Limited Architecture

♦ Thermal Shutdown

♦ Small 8-µMAX Package

Applications

Notebook Computers Distributed Power Systems Keep-Alive Supplies

Hand-Held Devices

| Ordering Information PART | TEMP RANGE | PIN-PACKAGE |
| --- | --- | --- |
| MAX1776EUA | -40°C to +85°C | 8 µMAX |

Typical Operating Circuit

Pin  Configuration

SHDN

IN               VIN

TOP VIEW

ILIM                          LX

MAX1776

VOUT

FB GND

ILIM

OUT SHDN

ILIM2

ILIM2

OUT

| LX IN | MAX1776EUA |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

FB                                     GND

MAX

MAX

Maxim Integrated Products    1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

ABSOLUTE MAXIMUM RATINGS

IN, SHDN, ILIM, ILIM2 to GND .................................-0.3V to 25V LX to GND.......................................................-2V to (VIN + 0.3V) OUT, FB to GND .........................................................-0.3V to 6V Peak Input Current .................................................................. 2A Maximum DC Input Current.............................................. 500mA

Continuous Power Dissipation (TA = +70°C)

8-Pin µMAX (derate 4.1mW/°C above +70°C) .............330mW Operating Temperature Range ...........................-40°C to +85°C Junction Temperature ......................................................+150°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(Circuit of Figure 1, VIN = +12V, SHDN = IN, TA = 0°C to +85°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Voltage Range | VIN |  |  | 4.5 24 | V |
| Input Supply Current | IIN | No load |  | 15 28 | µA |
| Input Supply Current in Dropout | IIN(DROP) | No load |  | 50 70 | µA |
| Input Shutdown Current |  | SHDN = GND |  | 3 7 | µA |
| Input Undervoltage Lockout Threshold | VUVLO | VIN rising |  | 3.6 4.0 4.4 | V |
|  |  | VIN falling |  | 3.5 3.9 4.3 |  |
| Output Voltage (Preset Mode) | VOUT | FB = GND |  | 4.80 5.00 5.20 | V |
| Feedback Set Voltage (Adjustable Mode) | VFB |  |  | 1.212 1.25 1.288 | V |
| OUT Bias Current |  | VOUT = 5.5V |  | 1.65 3.5 6.25 | µA |
| OUT Pin Maximum Voltage |  |  |  | 5.5 | V |
| FB Bias Current | IFB | VFB = 1.3V |  | -25 +25 | nA |
| FB Dual Mode™ Threshold Low |  |  |  | 50 100 150 | mV |
| LX Switch Minimum Off-Time | tOFF(MIN) |  |  | 0.22 0.42 0.62 | µs |
| LX Switch Maximum On-Time | tON(MAX) | VFB = 1.3V |  | 8 10 12 | µs |
| LX Switch On-Resistance | RLX | VIN = 6V | ILIM = ILIM2 = GND | 1.6 3.2 | Ω |
|  |  |  | ILIM = GND, ILIM2 = IN | 0.8 1.6 |  |
|  |  |  | ILIM = IN, ILIM2 = GND | 0.4 0.8 |  |
|  |  |  | ILIM = ILIM2 = IN | 0.4 0.8 |  |
|  |  | VIN = 4.5V | ILIM = ILIM2 = GND | 1.9 3.8 |  |
|  |  |  | ILIM = GND, ILIM2 = IN | 1.0 1.9 |  |
|  |  |  | ILIM = IN, ILIM2 = GND | 0.5 0.95 |  |
|  |  |  | ILIM = ILIM2 = IN | 0.5 0.95 |  |
| LX Current Limit | ILX(PEAK) | ILIM = ILIM2 = GND |  | 120 150 180 | mA |
|  |  | ILIM = GND, ILIM2 = IN |  | 240 300 360 |  |
|  |  | ILIM = IN, ILIM2 = GND |  | 480 600 720 |  |
|  |  | ILIM = ILIM2 = IN |  | 960 1200 1440 |  |
| LX Zero-Crossing Threshold |  |  |  | -75 +75 | mV |
| Zero-Crossing Timeout |  | LX does not rise above the threshold |  | 30 | µs |
| LX Switch Leakage Current |  | VIN = 24V, LX = GND | TA = +25°C | 1 | µA |
|  |  |  | TA = 0°C to +85°C | 10 |  |

Dual Mode is a trademark of Maxim Integrated  Products, Inc.

2

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

ELECTRICAL CHARACTERISTICS  (continued)

(Circuit of Figure 1, VIN = +12V, SHDN = IN, TA = 0°C to +85°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Dropout Voltage | VDROPOUT | IOUT = 525mA, ILIM = ILIM2 = IN |  | 0.2 | V |
| Line Regulation |  | VIN = 8V/24V, 200Ω load |  | 0.1 | %/V |
| Load Regulation |  | No load/full load |  | 0.9 | % |
| Digital Input Level |  | SHDN, ILIM2 | Low | 0.8 | V |
|  |  |  | High | 2.4 |  |
| Digital Input Leakage Current |  | V SHDN, VILIM, VILIM2 = 0 or 24V, VIN = 24V |  | -1 +1 | µA |
| ILIM Input Level |  | Low |  | 0.05 | V |
|  |  | High |  | 2.2 |  |
| Thermal Shutdown |  | 10°C hysteresis |  | 160 | °C |

ELECTRICAL CHARACTERISTICS

(Circuit of Figure 1, VIN = +12V, SHDN = IN, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Voltage Range | VIN |  |  | 4.5 24 | V |
| Input Supply Current | IIN | No load |  | 28 | µA |
| Input Supply Current in Dropout | IIN(DROP) | No load |  | 70 | µA |
| Input Shutdown Current |  | SHDN = GND |  | 7 | µA |
| Input Undervoltage Lockout Threshold | VUVLO | VIN rising |  | 3.6 4.4 | V |
|  |  | VIN falling |  | 3.5 4.3 |  |
| Output Voltage (Preset Mode) | VOUT | FB = GND |  | 4.75 5.25 | V |
| Feedback Set Voltage (Adjustable Mode) | VFB |  |  | 1.2 1.3 | V |
| OUT Bias Current |  | VOUT = 5.5V |  | 1.65 6.25 | µA |
| OUT Pin Maximum Voltage |  |  |  | 5.5 | V |
| FB Bias Current | IFB | VFB = 1.3V |  | -25 +25 | nA |
| FB Dual Mode Threshold Low |  |  |  | 45 155 | mV |
| LX Switch Minimum Off-Time | tOFF(MIN) |  |  | 0.22 0.64 | µs |
| LX Switch Maximum On-Time | tON(MAX) | VFB = 1.3V |  | 7.5 12.5 | µs |
| LX Switch On-Resistance | RLX | VIN = 6V | ILIM = ILIM2 = GND | 3.2 | Ω |
|  |  |  | ILIM = GND, ILIM2 = IN | 1.6 |  |
|  |  |  | ILIM = IN, ILIM2 = GND | 0.8 |  |
|  |  |  | ILIM = ILIM2 = IN | 0.8 |  |
|  |  | VIN = 4.5V | ILIM = ILIM2 = GND | 3.8 |  |
|  |  |  | ILIM = GND, ILIM2 = IN | 1.9 |  |
|  |  |  | ILIM = IN, ILIM2 = GND | 0.95 |  |
|  |  |  | ILIM = ILIM2 = IN | 0.95 |  |
| LX Current Limit | ILX(PEAK) | ILIM = ILIM2 = GND |  | 100 200 | mA |
|  |  | ILIM = GND, ILIM2 = IN |  | 200 400 |  |
|  |  | ILIM = IN, ILIM2 = GND |  | 400 800 |  |
|  |  | ILIM = ILIM2 = IN |  | 800 1600 |  |

3

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

ELECTRICAL CHARACTERISTICS  (continued)

(Circuit of Figure 1, VIN = +12V, SHDN = IN, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| LX Zero-Crossing Threshold |  |  |  | -75 75 | mV |
| LX Switch Leakage Current |  | VIN = 24V, LX = GND |  | 10 | µA |
| Digital Input Level |  | SHDN, ILIM2 | Low | 0.8 | V |
|  |  |  | High | 2.4 |  |
| Digital Input Leakage Current |  | V SHDN, VILIM, VILIM2 = 0 or 24V, VIN = 24V |  | -1 1 | µA |
| ILIM Input Level |  | Low |  | 0.05 | V |
|  |  | High |  | 2.2 |  |

Note 1: Specifications to -40°C are guaranteed  by design, not production  tested.

Typical Operating Characteristics

(Circuit of Figure 1, components  from Table 3, VIN = +12V, SHDN = IN, TA = +25°C.)

0.2

0

-0.2

-0.4

VOUTPUT (%)

-0.6

MAX1776 toc01

LOAD REGULATION, CIRCUIT 1, VOUTPUT = 5V

0.6

0.4

0.2

VOUTPUT (%)

0

-0.2

-0.4

MAX1776 toc02

LOAD REGULATION, CIRCUIT 1, VOUTPUT = 3.3V

VIN = 5V

VIN = 15V VIN = 12V

0.2

VOUTPUT (%)

0

-0.2

-0.4

-0.6

MAX1776 toc03

LOAD REGULATION, CIRCUIT 2

VIN = 24V

-0.8

-1.0

-0.6

-0.8

VIN

= 24V

-0.8

-1.0

VIN

= 12V

VIN = 15V

-1.2

| 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VIN = 12V |  |  |  |
|  |  |  |  |  | VIN = 24V |  |
|  |  |  |  | VIN = 15V |  |  |

100

200     300

400     500     600     700

-1.0

0

100

200

300       400       500       600

-1.2

0

50     100    150

200    250    300    350    400

0

-0.1

VOUTPUT (% FROM VOUT(NOM))

-0.2

-0.3

-0.4

-0.5

-0.6

-0.7

-0.8

-0.9

1.0

ILOAD (mA)

MAX1776 toc04

LOAD REGULATION, CIRCUIT 5

ILOAD (mA)

MAX1776 toc05

VOUTPUT vs. VIN, CIRCUIT 5, VOUTPUT = 5V

3

2

VOUTPUT (%)

1                            ILOAD = 1mA            ILOAD = 50mA

VOUTPUT (%)

0

ILOAD = 500mA

-1

-2

-3

2.0

1.5

1.0

0.5

0

-0.5

-1.0

ILOAD (mA)

MAX1776 toc06

VOUTPUT vs. VIN, CIRCUIT 5, VOUTPUT = 3.3V

0          0.1

0.2

| 0.3 0.4 0.5 0.6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VIN = 12V |  |  |  |
|  |  |  |  |  |  | VIN = 24V |  |
|  |  |  |  | VIN = 15V |  |  |  |

ILOAD (A)

5      7      9

11    13

15    17    19    21    23    25

VIN (V)

| 5 7 9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ILOAD = 1mA |  |  |  |  |  |
|  | ILOAD = 10mA |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ILOAD = 50mA |  |  |  |  |

11    13

15    17    19    21    23    25

VIN (V)

4

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

Typical Operating Characteristics (continued)

(Circuit of Figure 1, components  from Table 3, VIN = +12V, SHDN = IN, TA = +25°C.)

0.4

VOUTPUT vs. VIN, CIRCUIT 1, VOUTPUT = 5V

ILOAD = 10mA

0.6

VOUTPUT vs. VIN, CIRCUIT 1, VOUTPUT = 3.3V

100

95

EFFICIENCY vs. ILOAD, CIRCUIT 1, VOUT = 5V

VIN = 6V

0.2

0

-0.2

MAX1776 toc07

ILOAD = 1mA

0.4

0.2

VOUTPUT (%)

0

-0.2

ILOAD

= 10mA

MAX1776 toc08

ILOAD = 1mA

90

EFFICIENCY (%)

85

80

75

MAX1776 toc09

VIN = 12V

VIN = 15V

VIN = 24V

VOUTPUT (%)

-0.4

-0.6

-0.8

1.0

ILOAD = 50mA

ILOAD = 500mA

-0.4

-0.6

-0.8

-1.0

-1.2

ILOAD = 50mA

ILOAD = 500mA                                 70

65

60

55

50

5      7      9

11    13

15    17    19    21    23    25

VIN (V)

5      7      9

11    13

15    17    19    21    23    25

VIN (V)

0.10              1                  10               100              1000

ILOAD (mA)

100

95

90

EFFICIENCY (%)

85

80

MAX1776 toc10

EFFICIENCY vs. ILOAD, CIRCUIT 5, VOUTPUT = 3.3V

VIN = 6V VIN = 12V

100

95

90

85

80

EFFICIENCY vs. ILOAD, CIRCUIT 1, VOUTPUT = 3.3V

VIN = 6V

VIN = 12V

100

95

MAX1776 toc11

90

EFFICIENCY (%)

85

80

MAX1776 toc12

| EFFICIENCY vs. VIN, ILOAD = 500mA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CIRCUIT 5, 5V |  |  |  | CIRCUIT 1, 5V |  |  |  |  |
| CIRCUIT 1, 3.3V |  |  |  |  | CIRCUIT 5, 3.3V |  |  |  |

75                                  VIN = 15V

70

65

60

55

50

VIN = 24V

75                                   VIN = 24V                                                          75

EFFICIENCY (%)

70                                                                                                             70

65                                                                                                             65

60                                                                                                             60

55                                                                                                             55

50                                                                                                            50

0.10              1                  10               100              1000

0.10              1                  10               100              1000

7       8       9      10     11

12     13     14     15     16

ILOAD (mA)

ILOAD (mA)

VIN (V)

| 200 |  |  | VIN = 15V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | VIN = 24V |  |  |
|  |  |  | VIN = 12V |  |  |  |  |  |

180

160

FREQUENCY (kHz)

140

120

100

80

60

40

20

0

MAX1776 toc13

SWITCHING FREQUENCY vs. LOAD CURRENT, CIRCUIT 1

140

120

FREQUENCY (kHz)

100

80

60

40

20

0

SWITCHING FREQUENCY vs.

VIN, CIRCUIT 1

MAX1776 toc14

ILOAD = 500mA ILOAD = 375mA

ILOAD = 250mA

ILOAD = 5mA ILOAD = 50mA      ILOAD = 10mA

1.5

1.0

0.5

0

VOUT ACCURACY (%)

-0.5

-1.0

-1.5

MAX1776 toc15

| VOUTPUT ACCURACY vs. TEMPERATURE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0     100   200   300   400   500   600   700   800   900

5                 10                15                20                25

-40

-20     0

20       40

60       80      100

ILOAD (mA)

VIN (V)

TEMPERATURE (C)

5

| 24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MAX1776

Typical Operating Characteristics (continued)

(Circuit of Figure 1, components  from Table 3, VIN = +12V, SHDN = IN, TA = +25°C.)

18.0

17.5

17.0

16.5

16.0

QUIESCENT SUPPLY CURRENT (A)

15.5

15.0

MAX1776 toc16

QUIESCENT SUPPLY CURRENT

vs. TEMPERATURE

14.20

QUIESCENT SUPPLY CURRENT (A)

14.15

14.10

14.05

14.00

13.95

13.90

13.85

13.80

13.75

13.70

MAX1776 toc17

QUIESCENT SUPPLY CURRENT

vs. SUPPLY VOLTAGE

-40     -20      0          20         40       60         80

5      7      9     11    13

15    17    19    21

23    25

0.8

0.7

0.6

0.5

0.4

0.3

PEAK SWITCH CURRENT (A)

0.2

0.1

0

TEMPERATURE (C)

PEAK SWITCH CURRENT

vs. INPUT VOLTAGE, CIRCUIT 3, 0.3A

MAX1776 toc18

L = 10H L = 22H

L = 47H L = 100H

0              5             10            15            20            25

VIN (V)

IL

VLX

VOUT

ILOAD

SUPPLY VOLTAGE (V)

LOAD-TRANSIENT RESPONSE, CIRCUIT 5

MAX1776 toc19

10s/div

1A

0

10V

0

AC COUPLED

50mV/div

500mA

10mA

VOUT

LINE-TRANSIENT RESPONSE, CIRCUIT 5, ILOAD = 500mA

MAX1776 toc20

AC-COUPLED

200mv/div

VOUT

LINE-TRANSIENT RESPONSE, CIRCUIT 5, ILOAD = 50mA

MAX1776 toc21

AC-COUPLED

200mv/div

VIN

10V

5V

VIN

15V

10V

10V

VLX

200s/div

5V                             VLX

0

5V

0

200s/div

6

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

Typical Operating Characteristics (continued)

(Circuit of Figure 1, components  from Table 3, VIN = +12V, SHDN = IN, TA = +25°C.)

IL

VLX

VOUT

LX WAVEFORM, CIRCUIT 1

VIN = 15V, ILOAD = 500mA

MAX1776 toc22

1A

0

10V

0

50mV/div

VSHDN

IL

VOUT

STARTUP WAVEFORM, CIRCUIT 1, RLOAD = 100

MAX1776 toc23

5V

0

1A

0

6V

4V

2V

0

2s/div

2s/div

100

MAX1776 toc24

EFFICIENCY vs. ILOAD, CIRCUIT 3, VIN = 12V

100

MAX1776 toc25

EFFICIENCY vs. ILOAD, CIRCUIT 3, VIN = 12V

95

L = 22H

90

95

L = 22H, 0.6A

EFFICIENCY (%)

90

L = 47H, 0.3A

EFFICIENCY (%)

85                             L = 47H         L = 100H

80

85

L = 10H, 1.2A

80

75

0.10

1                 10               100

ILOAD (mA)

1000

75

0.10

1                 10               100

ILOAD (mA)

1000

7

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | FB | Dual-Mode Feedback Input. Connect to GND for the preset 5V output. Connect to a resistive divider between OUT and GND to adjust the output voltage between 1.25V and VIN. |
| 2 | GND | Ground |
| 3 | ILIM | Peak Current Control Input. Connect to IN or GND to set peak current limit. ILIM and ILIM2 together set the peak current limit. See Setting Current Limit. |
| 4 | LX | Inductor Connection. Connect LX to external inductor and diode as shown in Figure 1. |
| 5 | IN | Input Supply Voltage. Input voltage range is 4.5V to 24V. |
| 6 | ILIM2 | Peak Current Control Input 2. Connect to IN or GND. ILIM and ILIM2 together set the peak current limit. See Setting Current Limit. |
| 7 | SHDN | Shutdown Input. A logic low shuts down the MAX1776 and reduces the supply current to 3A. LX is high impedance in shutdown. Connect to IN for normal operation. |
| 8 | OUT | Regulated Output Voltage High-Impedance Sense Input. Internally connected to a resistive divider. Do not connect for output voltages higher than 5.5V. Connect to GND when not used. |

Detailed Description

The MAX1776 step-down converter is designed primar- ily for battery-powered devices and notebook comput- ers. The unique  current-limited  control  scheme provides  high efficiency  over a wide load range. Operation up to 100% duty cycle allows the lowest pos- sible dropout voltage, increasing the usable supply voltage range. Under no load, the MAX1776 draws only

15µA, and in shutdown mode, it draws only 3µA to fur- ther reduce power consumption and extend battery life. Additionally, an internal 24V switching MOSFET, inter- nal current sensing, and a high switching frequency minimize PC board space and component costs.

Current-Limited Control Architecture The MAX1776 uses a proprietary current-limited control scheme with operation to 100% duty cycle. This DC-DC converter pulses as needed to maintain regulation, resulting in a variable switching frequency that increas- es with the load. This eliminates the high supply cur- rents associated with conventional constant-frequency pulse-width-modulation (PWM) controllers that switch the MOSFET unnecessarily.

When the output voltage is too low, the error comparator sets a flip-flop, which turns on the internal P-channel MOSFET and begins a switching cycle (Figure 2). As shown in Figure 3, the inductor current ramps up linear- ly, storing energy in a magnetic field while charging the output capacitor  and servicing the load. The MOSFET turns off when the peak current limit is reached, or when

INPUT

4.5V TO 24V CIN

J1

J2              J3

J4

IN SHDN

ILIM ILIM2

GND

MAX1776

L1

LX

D1

OUT

FB

OUTPUT

5V COUT

the maximum on-time of 10µs is exceeded and the out- put voltage is in regulation. If the output is out of regula- tion and the peak current is never obtained,  the MOSFET remains on, allowing a duty cycle up to 100%. This feature ensures the lowest possible dropout volt- age. Once the MOSFET turns off, the flip-flop resets, the inductor current is pulled through D1, and the current through the inductor ramps back down, transferring the stored energy to the output capacitor  and load. The MOSFET remains off until the 0.42µs minimum off-time expires, and the output voltage drops out of regulation.

CIN: 10F, 25V CERAMIC

NOTE: HIGH-CURRENT PATHS SHOWN WITH BOLD LINES.

SEE TABLE 3 FOR OTHER COMPONENT VALUES

Figure 1. Typical Application Circuit

8

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

D CIN

MAX1776

LX           L1

D1

OUTPUT

MAX1776

COUT

OUT

SHDN                                                                                                Q      R

FB S

ILIM

ILIM2

ILIM SET

MAXIMUM ON-TIME DELAY

MINIIMUM OFF-TIME DELAY

100mV

VSET

1.25V

GND

Figure 2. Simplified Functional Diagram

IL

VLX

VOUT

LX WAVEFORM, CIRCUIT 1

VIN = 15V, ILOAD = 500mA

2s/div

1A

0

10V

0

50mV/div

Input-Output (Dropout) Voltage A step-down converter’s minimum input-to-output volt- age differential (dropout voltage) determines the lowest usable supply  voltage. In battery-powered  systems, this limits the useful end-of-life battery voltage. To maxi- mize battery life, the MAX1776 operates with duty cycles up to 100%, which minimizes the dropout volt- age and eliminates switching losses while in dropout. When the supply voltage approaches  the output volt- age, the P-channel MOSFET remains on continuously to supply the load.

Dropout voltage is defined as the difference between the input and output voltages when the input is low enough for the output to drop out of regulation. For a step-down converter with 100% duty cycle, dropout depends on the MOSFET drain-to-source on-resistance and inductor series resistance; therefore, it is propor- tional to the load current:

Figure 3. Discontinuous-Conduction Operation

VDROPOUT = IOUT ✕ (RDS(ON) + RINDUCTOR)

9

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

Shutdown (SHDN) A logic low level on SHDN shuts down the MAX1776 converter. When in shutdown, the supply current drops to 3µA to maximize battery life, and the internal P-chan- nel MOSFET turns off to isolate the output from the input. The output capacitance and load current determine the rate at which the output voltage decays. A logic level high on SHDN activates the MAX1776. Do not leave SHDN floating. If unused, connect SHDN to IN.

Thermal-Overload Protection Thermal-overload protection limits total power dissipa- tion in the MAX1776. When the junction temperature exceeds TJ = +160°C, a thermal sensor turns off the pass transistor, allowing the IC to cool. The thermal sen- sor turns the pass transistor on again after the IC’s junc- tion temperature cools by 10°C, resulting in a pulsed output during continuous thermal-overload conditions.

Design Information

Output Voltage Selection The feedback  input features dual-mode operation. Connect FB to GND for the 5.0V preset output voltage. Alternatively, adjust the output voltage by connecting a voltage-divider  from the output to GND (Figure 4). Select a value for R2 between 10k and 100k . Calculate R1 with the following equation:



Table 1. Current-Limit Configuration

| CURRENT LIMIT (mA) | ILIM CONNECTED TO | ILIM2 CONNECTED TO |
| --- | --- | --- |
| 150 | GND | GND |
| 300 | GND | IN |
| 600 | IN | GND |
| 1200 | IN | IN |

Choose a current limit that realistically reflects the maxi- mum load current. The maximum output current is half of the peak current limit. Although choosing a lower current limit allows using an inductor with a lower cur- rent rating, it requires a higher inductance  (see Inductor Selection) and does little to reduce inductor package size.

Inductor Selection When selecting the inductor, consider these four para- meters: inductance  value, saturation rating, series resistance, and size. The MAX1776 operates with a wide range of inductance  values. For most applica- tions, values between 10µH and 100µH work best with the controller’s high switching frequency. Larger induc- tor values will reduce the switching frequency and thereby improve efficiency and EMI. The trade-off for improved efficiency is a higher output ripple and slower

R1  R2  

 VOUTPUT  -1

transient response. On the other hand, low-value induc-

 

VFB     



tors respond faster to transients, improve output ripple, offer smaller physical size, and minimize cost. If the

where VFB = 1.25V, and VOUTPUT may range from

1.25V to VIN.

Setting Current Limit

The MAX1776 has an adjustable peak current limit. Configure this peak current limit by connecting  ILIM and ILIM2 as shown in Table 1.

inductor value is too small, the peak inductor current exceeds the current limit due to current-sense com- parator propagation  delay, potentially exceeding  the inductor’s current rating. Calculate the minimum induc- tance value as follows:

VIN(MAX) - VOUTPUT tON(MIN)

L(MIN) =

ILX (PEAK )

INPUT

4.5V TO 24V

IN

OUTPUT L1             1.25V TO VIN

LX

where tON(MIN) = 1µs.

The inductor’s saturation current rating must be greater

CIN                              SHDN                                                  D1

R1

COUT

than the peak switch current limit, plus the overshoot

due to the 250ns current-sense comparator propaga-

ILIM ILIM2

GND

MAX1776

FB

R2

OUT

tion delay. Saturation occurs when the inductor’s mag- netic flux density reaches the maximum level the core can support and the inductance  starts to fall. Choose an inductor with a saturation rating greater than IPEAK in the following equation:

IPEAK = ILX(PEAK) + (VIN - VOUTPUT) ✕ 250ns / L

Figure 4. Adjustable  Output Voltage

10

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

Inductor series resistance affects both efficiency and dropout voltage (see Input-Output (Dropout) Voltage). High series resistance limits the maximum current avail- able at lower input voltages, and increases the dropout voltage. For optimum performance, select an inductor with the lowest possible DC resistance that fits in the allotted dimensions. Some recommended  component manufacturers are listed in Table 2.

MAX1776

Input Capacitor The input filter capacitor reduces peak currents drawn from the power source and reduces noise and voltage ripple on the input caused by the circuit’s switching. The input capacitor  must meet the ripple-current requirement (IRMS) imposed by the switching current defined by the following equation:

Maximum Output Current The MAX1776 converter’s output current determines the regulator’s switching frequency. When the convert-

IRMS



ILOADVOUTPUT

VIN

 4   VIN

       VOUTPUT

 3



 1



er approaches  continuous mode, the output voltage falls out of regulation. For the typical application,  the maximum output current is approximately:

ILOAD(MAX) = 1/2 ILX (PEAK)(MIN)

For low-input voltages, the maximum on-time may be reached and the load current is limited by:

ILOAD = 1/2 (VIN - VOUT) ✕ 10µs / L

Output Capacitor Choose the output capacitor  to service the maximum load current with acceptable voltage ripple. The output ripple has two components: variations in the charge stored in the output capacitor with each LX pulse, and the voltage drop  across the capacitor’s  equivalent series resistance (ESR) caused by the current into and out of the capacitor:

VRIPPLE  VRIPPLE(ESR) + VRIPPLE(C)

The output voltage ripple as a consequence of the ESR

and output capacitance is:

VRIPPLE(ESR)  ESR  IPEAK

For most applications, nontantalum chemistries (ceram-

ic, aluminum, polymer, or OS-CON) are preferred due to their robustness to high inrush currents typical of sys- tems with low-impedance battery inputs. Alternatively, connect two (or more) smaller value low-ESR capacitors in parallel to reduce cost. Choose an input capacitor that exhibits less than +10°C temperature rise at the RMS input current for optimal circuit longevity.

| Table 2. Component Suppliers SUPPLIER | WEBSITE |
| --- | --- |
| DIODES |  |
| Central Semiconductor | www.centralsemi.com |
| Fairchild | www.fairchildsemi.com |
| General Semiconductor | www.gensemi.com |
| International Rectifier | www.irf.com |
| Nihon | www.niec.co.jp/engver2/ niec.co.jp_eg.htm |
| On Semi | www.onsemi.com |
| Vishay-Siliconix | www.vishay.com/brands/siliconix/ main.html |
| Zetex | www.zetex.com |
| CAPACITORS |  |
| AVX | www.avxcorp.com |
| Kemet | www.kemet.com |
| Nichicon | www.nichicon-us.com |
| Sanyo | www.sanyo.com |
| Taiyo Yuden | www.t-yuden.com |
| INDUCTORS |  |
| Coilcraft | www.coilcraft.com |
| Coiltronics | www.cooperet.com |
| Pulse Engineering | www.pulseeng.com |
| Sumida USA | www.sumida.com |
| Toko | www.tokoam.com |

L  I         - I

2           V           

VRIPPLE(C) 

PEAK   OUTPUT                   IN

                          

2COUT  VOUTPUT

 VIN - VOUTPUT 

where IPEAK is the peak inductor current (see Inductor Selection). The worst-case ripple occurs  at no-load. These equations are suitable for initial capacitor selec- tion, but final values should be set by testing a proto- type or evaluation circuit. As a general rule, a smaller amount of charge delivered in each pulse results in less output ripple. Since the amount of charge deliv- ered in each oscillator pulse is determined  by the inductor value and input voltage, the voltage ripple increases with larger inductance, and as the input volt- age decreases. See Table 3 for recommended capaci- tor values and Table 2 for recommended  component manufacturers.

11

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

MAX1776

Table 3. Recommended Components

| CIRCUIT | INPUT VOLTAGE (V) | MAXIMUM LOAD CURRENT (mA) | ILX(PEAK) CURRENT (A) | INDUCTOR | CAPACITOR |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 to 24 | 600 | 1.20 | 10µH, 1.56A, 70mΩ Toko D75F 646FY-100M, 10µH, 1.70A, 48mΩ Sumida CDRH6D28-100NC, or 10µH, 1.63A, 55mΩ Toko D75C 646CY-100M 0.055 | 100µF, 6.3V Sanyo POSCAP 6TPC100M |
| 2 | 10 to 24 | 300 | 0.60 | 22µH, 1.17A, 120mΩ Toko D75F 646FY-220M, 22µH, 1.09A, 115mΩ Toko D75C 646CY-220M, or 22µH, 1.20A, 95mΩ Sumida CDRH6D28-220NC | 47µF, 6.3V Sanyo POSCAP 6TPA47M |
| 3 | 10 to 24 | 150 | 0.30 | 47µH, 0.54A, 440mΩ Sumida CDRH5D18-470 | 22µF, 6.3V, 1210 case Taiyo Youden JMK325BJ226MM |
| 4 | 10 to 24 | 75 | 0.15 | 100µH, 0.29A, 766mΩ Sumida CDRH4D28-101 | 10µF, 6.3V, X7R, 1206 case Taiyo Youden JMK316BJ106ML |
| 5 | 5 to 15 | 600 | 1.20 | 5.4µH, 1.6A, 56mΩ Sumida CDRH5D18-5R4 | 100µF, 6.3V Sanyo POSCAP 6TPC100m |
| 6 | 5 to 15 | 300 | 0.60 | 10µH, 1.04A, 80mΩ Toko D73LC 817CY-100M | 47µF, 6.3V Sanyo POSCAP 6TPA47M |
| 7 | 5 to 15 | 150 | 0.30 | 22µH, 0.41A, 294mΩ Sumida CDRH4D18-220 | 22µF, 6.3V, 1210 case Taiyo Youden JMK325BJ226MM |
| 8 | 5 to 15 | 75 | 0.15 | 47µH, 0.33A, 230mΩ Coilcraft DS1608C-473 | 10µF, 6.3V, X7R, 1206 case Taiyo Youden JMK316BJ106ML |

Diode Selection The current in the external diode  (D1 in Figure 1) changes abruptly from zero to its peak value each time the LX switch turns off. To avoid excessive losses, the diode must have a fast turn-on time and a low forward voltage.

Make sure that the diode’s peak current rating exceeds the peak current limit set by the current limit, and that

its breakdown  voltage exceeds  VIN. Use Schottky diodes when possible.

MAX1776 Stability Instability is frequently caused by excessive noise on OUT, FB, or GND due to poor layout or improper com- ponent selection. Instability typically manifests itself as “motorboating,”  which is characterized  by grouped switching pulses with large gaps and excessive low- frequency output ripple during  no-load or light-load conditions.

PC Board Layout and Grounding High switching frequencies and large peak currents make PC board layout an important part of the design. Poor layout introduces switching noise into the feed- back path, resulting in jitter, instability, or degraded performance.  High-power traces, highlighted  in the

12

24V, 600mA Internal Switch, 100% Duty Cycle, Step-Down Converter

Typical Application  Circuit (Figure 1), should be as short and wide as possible. Additionally, the current loops formed by the power components (CIN, COUT, L1, and D1) should be as short as possible to avoid radiated  noise. Connect the ground  pins of these power components at a common node in a star-ground configuration. Separate the noisy traces, such as the LX node, from the feedback  network with grounded copper.  Furthermore, keep the extra copper  on the

MAX1776

board and integrate it into a pseudo-ground  plane. When using external feedback,  place the resistors as close to the feedback pin as possible to minimize noise coupling.

Chip Information

TRANSISTOR COUNT: 932

PROCESS: BiCMOS

8LUMAXD.EPS

| Package Information | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| b | 0.010 | 0.014 | 0.25 | 0.36 |
| c | 0.005 | 0.007 | 0.13 | 0.18 |
| D | 0.116 | 0.120 | 2.95 | 3.05 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| E | 0.116 | 0.120 | 2.95 | 3.05 |
| H | 0.188 | 0.198 | 4.78 | 5.03 |
| L | 0.016 | 0.026 | 0.41 | 0.66 |
|  | 0∞ | 6∞ | 0∞ | 6∞ |
| S | 0.0207 BSC |  | 0.5250 BSC |  |

| 4X S 8 8 E H ÿ 0.50±0.1 0.6±0.1 1 1 0.6±0.1 D BOTTOM VIEW TOP VIEW A2 A1 A c  e b L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, 8L uMAX/uSOP |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0036 | REV. J | 1 1 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600                                   13

©  2003 Maxim Integrated  Products               Printed USA                                  is a registered  trademark of Maxim Integrated  Products.