19-2357; Rev 0; 4/02

Current-Limited Switch for Single USB  Port

General Description

The MAX1931 is a current-limited, 60mΩ switch with built-in fault blanking. Its accurate preset current limit of

0.64A to 1.06A makes it ideally suited for USB applica- tions. The device’s low quiescent supply current (14µA) and shutdown current (1µA) conserve battery power in portable  applications.  The MAX1931 operates with inputs from 2.7V to 5.5V, making it ideal for both 3V and

5V systems.

A fault signal notifies the microprocessor that the inter- nal current limit has been reached. A 10ms fault-blank- ing feature allows momentary faults (such as those caused when hot-swapping into a capacitive load) to be ignored, thus preventing false alarms to the host system. This fault blanking  also prevents a fault signal from being issued when the device is powering up.

In the MAX1931, an output overcurrent condition causes the switch to current limit at 0.64A to 1.06A and FAULT to go low after the 10ms blanking period. When the overcurrent condition is removed, FAULT returns to its high-impedance state.

The MAX1931 has several safety features to ensure that the USB port is protected.  Built-in thermal overload protection limits power dissipation and junction tempera- tures. The device has an accurate internal current-limit-

MAX1931

Features

♦ 0.5A Guaranteed Output Current

♦ Guaranteed 0.75A Short-Circuit Protection

♦ 10ms Internal Fault-Blanking Timeout

♦ No Fault Signal During Power-Up

♦ Thermal Shutdown Protection

♦ 2.7V to 5.5V Supply Range

♦ 14µA Supply Current

♦ Small 10-Pin µMAX Package

Ordering Information

PART                   TEMP RANGE        PIN-PACKAGE

MAX1931EUB               -40°C to +85°C        10µMAX

Pin  Configuration

TOP VIEW

ing circuitry to protect the input supply against overload. The MAX1931 is available in a space-saving  10-pin

µMAX package.

Applications

USB Ports and Hubs Notebook Computers Portable Equipment Docking Stations

IN    1

OUT    2

IN    3

OUT    4

ON    5

MAX1931

µMAX

10    OUT

9     IN

8     OUT

7     FAULT

6     GND

Hot Plug-In Power Supplies

Battery-Charger Circuits

Typical Operating Circuit

INPUT

2.7V TO 5.5V

IN                                          OUT

OUTPUT

FAULT

MAX1931

OFF ON

ON

GND

Maxim Integrated Products     1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Current-Limited Switch for Single USB  Port

MAX1931

ABSOLUTE MAXIMUM RATINGS

IN, ON (ON), FAULT to GND ....................................-0.3V to +6V OUT to GND ................................................-0.3V to (VIN + 0.3V) Maximum Continuous Switch Current .....1.2A (internally limited) OUT Short Circuit to GND ..........................................Continuous Continuous Power Dissipation (TA = +70°C)

10-Pin µMAX (derate 5.6mW/°C above +70°C) ............444mW

Operating Temperature Range ............................-40°C to +85°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN = 5V, TA = 0°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Operating Voltage | VIN |  |  | 2.7 5.5 | V |
| Quiescent Current | IQ | V ON = GND, IOUT = 0 | Timer not running | 14 35 | µA |
|  |  |  | Timer running | 35 |  |
| Off Supply Current |  | V ON = VIN = VOUT = 5.5V |  | 0.001 1 | µA |
| Undervoltage Lockout | UVLO | Rising edge, 100mV hysteresis |  | 2.0 2.6 | V |
| Off Switch Leakage |  | V ON = VIN VIN = 5.5V, VOUT = GND | TA = +25°C | 0.01 4 | µA |
|  |  |  | TA = 0°C to +85°C | 20 |  |
| On-Resistance | RON | TA = +25°C | VIN = 4.4V to 5.5V | 60 110 | mΩ |
|  |  | TA = 0°C to +85°C | VIN = 4.4V to 5.5V | 150 |  |
|  |  |  | VIN = 3V | 72 180 |  |
| Current Limit | ILIMIT | VOUT = 4.5V |  | 640 850 1060 | mA |
| Continuous Short-Circuit Current Limit | ISC | OUT shorted to GND |  | 500 700 | mA |
| ON Input Logic Low Voltage | VIL | VIN = 2.7V to 5.5V |  | 0.8 | V |
| ON Input Logic High Voltage | VIH | VIN = 2.7V to 3.6V |  | 2 | V |
|  |  | VIN = 3.7V to 5.5V |  | 2.4 |  |
| ON Input Leakage |  | V ON = VIN or GND |  | ±1 | µA |
| FAULT Output Logic Low Voltage | VOL | ISINK = 1mA, VIN = 3V |  | 0.4 | V |
| FAULT Output High Leakage Current |  | VIN = V F AULT = 5.5V |  | 1 | µA |
| Fault-Blanking Timeout Period | tFB | From overcurrent condition to FAULT assertion |  | 6 10 13 | ms |
| Startup Time |  | VIN = 5V, COUT = 150µF, RL = 15Ω, from ON driven low to 50% full VOUT |  | 1 | ms |
| Switch Turn-On Time | tON | ILOAD = 400mA |  | 80 200 | µs |
| Switch Turn-Off Time | tOFF | ILOAD = 400mA |  | 3 6 20 | µs |
| Thermal Shutdown Threshold |  |  |  | 165 | °C |

2

Current-Limited Switch for Single USB  Port

MAX1931

ELECTRICAL CHARACTERISTICS

(VIN = 5V, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Operating Voltage | VIN |  | 3 5.5 | V |
| Quiescent Current | IQ | V ON = GND, IOUT = 0, timer not running | 35 | µA |
| Off Supply Current |  | V ON = VIN = VOUT = 5.5V | 2 | µA |
| Undervoltage Lockout | UVLO | Rising edge, 100mV hysteresis | 2.0 2.9 | V |
| Off Switch Leakage |  | V ON = VIN = 5.5V, VOUT = GND | 20 | µA |
| On-Resistance | RON | VIN = 4.4V to 5.5V | 150 | mΩ |
|  |  | VIN = 3V | 180 |  |
| Current Limit | ILIMIT | VOUT = 4.5V | 600 1100 | mA |
| Continuous Short-Circuit Current Limit |  | OUT shorted to GND | 750 | mA |
| ON Input Logic Low Voltage | VIL | VIN = 3V to 5.5V | 0.8 | V |
| ON Input Logic High Voltage | VIH | VIN = 3V to 3.6V | 2 | V |
|  |  | VIN = 3.7V to 5.5V | 2.4 |  |
| ON Input Leakage |  | V ON = VIN or GND | ±1 | µA |
| FAULT Output Logic Low Voltage | VOL | ISINK = 1mA, VIN = 3V | 0.4 | V |
| FAULT Output High Leakage Current |  | VIN = V F AULT = 5.5V | 1 | µA |
| Fault-Blanking Timeout Period | tFB | From overcurrent condition to FAULT assertion | 6 14 | ms |
| Switch Turn-On Time | tON | ILOAD = 400mA | 200 | µs |
| Switch Turn-Off Time | tOFF | ILOAD = 400mA | 1 20 | µs |

Note 1:  Specifications to -40°C are guaranteed  by design, not production  tested.

3

Current-Limited Switch for Single USB  Port

MAX1931

QUIESCENT CURRENT (µA)

Typical Operating Characteristics

(VIN = 5V, TA = +25°C, unless otherwise noted.)

QUIESCENT CURRENT vs. INPUT VOLTAGE

16

VON (ON) = GND (VIN)

14        IOUT = O

12

10

8

6

4

2

QUIESCENT CURRENT vs. TEMPERATURE

16

MAX1931 toc02

VIN = 5V

15

MAX1931 toc01

14

VIN = 3V

13

12

QUIESCENT CURRENT (µA)

11

10

100

10

1

OFF SUPPLY CURRENT (nA)

0.1

OFF SUPPLY CURRENT

vs. TEMPERATURE

MAX1931 toc03

VON (ON) = VIN (GND)

0

0           1           2

3           4           5           6

9

-40

-20

0        20

40       60       80      100

0.01

-40

-20       0

20       40       60       80      100

INPUT VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

1000

OFF SWITCH CURRENT (nA)

100

10

1

0.1

MAX1931 toc04

OFF SWITCH CURRENT vs. TEMPERATURE

V0N (ON) = VIN (GND)

1.4

1.2

NORMALIZED RON

1.0

0.8

0.6

0.4

0.2

MAX1931 toc05

| NORMALIZED ON-RESISTANCE IOUT = 425mA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

vs. TEMPERATURE

1.2

NORMALIZED OUTPUT CURRENT

1.0

0.8

0.6

0.4

0.2

NORMALIZED OUTPUT CURRENT

vs. OUTPUT VOLTAGE

MAX1931 toc06

| LIMIT |  |  |  |  | I |
| --- | --- | --- | --- | --- | --- |
|  | SHORT-CIRCUIT CURRENT |  |  |  |  |

0.01

-40

-20      0

20       40

60       80

100

0

-40

-20      0

20       40

60       80

100

0

0           1           2           3

4           5           6

TEMPERATURE (°C)

TEMPERATURE (°C)

OUTPUT VOLTAGE (V)

160

140

120

tON (µs)

100

80

60

40

20

MAX1931 toc07

TURN-ON TIME vs. TEMPERATURE

TURN-OFF TIME vs. TEMPERATURE

7

ILOAD = 425mA

MAX1931 toc08

6                                                   VIN = 5V

5

tOFF (µs)

4                                                VIN = 3V

3

2

1

| 0 ILOAD = 425mA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | VIN = 3V |  |
|  |  |  |  |  | VIN = 5V |  |

-40

-20     0

20       40       60       80      100

0

-40

-20

0        20

40       60       80      100

TEMPERATURE (°C)

TEMPERATURE (°C)

4

Current-Limited Switch for Single USB  Port

MAX1931

Typical Operating Characteristics (continued)

(VIN = 5V, TA = +25°C, unless otherwise noted.)

MAX1931 toc09

FAULT-BLANKING TIMEOUT

vs. TEMPERATURE

12

CURRENT-LIMIT  AND FAULT RESPONSE

MAX1931 toc11

CURRENT-LIMIT  RESPONSE

MAX1931 toc10

1                                                                                                            1

FAULT-BLANKING TIMEOUT (ms)

10

8

2                                                                                                            2

6

3                                                                                                            3

4

2

4                                                                                                            4

0

| -40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IOUT VIN = | = 850m 3V OR | A 5V |  |  |  |  |

-20

0        20

40       60       80      100

2ms/div

10µs/div

TEMPERATURE (°C)

CH1 = VIN, 200mV/div,  AC-COUPLED; CH2 = VOUT,

5V/div; CH3 = VFAULT, 5V/div; CH4 = IOUT, 500mA/div

CH1 = VIN, 200mV/div,  AC-COUPLED; CH2 = VOUT,

5V/div; CH3 = VFAULT, 5V/div; CH4 = IOUT, 1A/div

MAX1931 toc12

SWITCH TURN-ON TIME

MAX1931 toc13

SWITCH TURN-OFF TIME

STARTUP TIME (TYPICAL USB APPLICATION)

1

1                                                                                                           1

2                                                                                                           2                                                                                                             2

MAX1931 toc14

CL = 150µF RL = 15Ω

FULL CURRENT LIMIT CHARGES CAPACITOR

3

4

20µs/div

3

4

1µs/div

3                FOLDBACK CURRENT LIMIT

4

1ms/div

RESISTOR LOAD CURRENT

CH1 = VOUT, 5V/div;  CH2 = VON, 5V/div;

CH3 = VFAULT, 5V/div; CH4 = IOUT, 200mA/div

CH1 = VOUT, 5V/div;  CH2 = VON, 5V/div;

CH3 = VFAULT, 5V/div; CH4 = IOUT, 200mA/div

CH1 = VOUT, 5V/div;  CH2 = VON, 5V/div;

CH3 = IOUT, 500mA/div;  CH4 = VFAULT, 5V/div

5

Current-Limited Switch for Single USB  Port

MAX1931

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 3, 9 | IN | Input. P-channel MOSFET source. Connect all IN pins together and bypass with a 1µF ceramic capacitor to ground. |
| 2, 4, 8, 10 | OUT | Switch Output. P-channel MOSFET drain. Connect all OUT pins together and bypass with a 0.1µF capacitor to ground. |
| 5 | ON | Active-Low Switch On Input. A logic low turns the switch on. |
| 6 | GND | Ground |
| 7 | FAULT | Fault-Indicator Output. This open-drain output goes low when the device is in thermal shut- down, undervoltage lockout, or on a sustained (>10ms) current-limit condition. |

IN FAULT

N

P

P

REPLICA AMPLIFIER

ON

MAX1931

P

OUT

Continuous Short-Circuit Protection The MAX1931 is a foldback  short-circuit-protected switch. In the event of an output short-circuit or current- overload condition, the current through the switch is foldback-current-limited to 500mA continuous.

Thermal Shutdown The MAX1931 features thermal shutdown. The switch turns off and the FAULT output goes low immediately (no fault blanking) when the junction temperature exceeds +165°C. When the MAX1931 cools 20°C, the switch turns back on. If the fault short-circuit condition is not removed, the switch cycles on and off, resulting

ON        CURRENT-LIMIT AMPLIFIER

in a pulsed output.

FAULT Indicator

10ms

TIMER HOT

ON

CONTROL

1.24V

GND

The MAX1931 provides  a fault output (FAULT). A

100kΩ pullup  resistor from FAULT to IN provides  a logic control signal. This open-drain output goes low when any of the following conditions occur:

•  The input voltage is below the undervoltage lockout

(UVLO) threshold.

Figure 1. Functional Diagram

CIRCUITRY

•  The die temperature exceeds the thermal shutdown temperature limit of +165°C.

•  The device  is in current limit and the 10ms fault- blanking period is exceeded.

Detailed Description

The MAX1931 P-channel MOSFET power switch limits output current to 0.64A (min) to 1.06A (max). When the output current is increased beyond the current limit (ILIMIT), the current also increases through the replica switch (IOUT / 6500). The current-limit error amplifier compares the voltage to the internal 1.24V reference and regulates the current back to the ILIMIT.

This switch is not bidirectional; therefore, the input volt- age must be higher than the output voltage.

Fault Blanking The MAX1931 features 10ms fault blanking.  Fault blanking allows current-limit faults, including  momen- tary short-circuit faults that occur when hot-swapping a capacitive load, and also ensures that no fault is issued during  power-up. When a load transient causes the device to enter current limit, an internal counter starts. If the load fault persists beyond the 10ms fault-blanking timeout, the FAULT output asserts low. Ensure that the MAX1931’s input is adequately bypassed  to prevent input glitches from triggering  spurious FAULT outputs. Input voltage glitches less than 150mV do not cause a

6

Current-Limited Switch for Single USB  Port

spurious FAULT output. Load-transient faults less than

10ms (typ) do not cause a FAULT output assertion.

Only current-limit faults are blanked. Die overtempera- ture faults and input voltage droops below the UVLO threshold cause an immediate fault output.

Applications Information

Input Capacitor To limit the input voltage drop during momentary output short-circuit conditions, connect a capacitor  from IN to GND. A 1µF ceramic capacitor  is adequate for most applications;  however, higher capacitor  values further reduce the voltage drop at the input (see Figure 2).

Output Capacitor Connect a 0.1µF capacitor from OUT to GND. This cap- acitor helps prevent inductive parasitics from pulling OUT negative during turn-off.

Layout and Thermal Dissipation To optimize the switch-response time to output short- circuit conditions, it is very important to keep all traces as short as possible to reduce the effect of undesirable parasitic inductance. Place input and output capacitors as close to the device as possible (no more than 5mm).

MAX1931

All IN and all OUT pins must be connected with short traces to the power bus. Wide power bus planes pro- vide superior heat dissipation through the switch IN and OUT pins. Figure 3 shows suggested  pin connections for a single-layer board.

Under normal operating conditions, the package  can dissipate and channel heat away. Calculate the maxi- mum power dissipation as follows:

P = (ILIMIT)2 ✕ RON

where ILIMIT is the preset current limit (1.1A max) and

RON is the on-resistance of the switch (150mΩ max).

When the output is short-circuited, foldback-current-lim- iting activates and the voltage drop across the switch equals the input supply. The power dissipated across the switch increases, as does the die temperature. If the fault condition is not removed, the thermal-overload pro- tection circuitry activates (see the Thermal Shutdown section). Wide power-bus planes connected to IN and OUT and a ground plane in contact with the device help dissipate additional heat.

Chip Information

TRANSISTOR COUNT: 715

INPUT

2.7V TO 5.5V

IN

OUT

OUTPUT

MAX1931

1µF                                                                                                           0.1µF*

MAX1931

100kΩ

1                       IN

OUT                    10

OFF ON

FAULT ON

GND

2                        OUT

3                       IN

4                        OUT

5                        ON

IN                      9

OUT                     8

FAULT                     7

GND                      6

*USB  SPECIFICATIONS  REQUIRE A LARGER CAPACITOR

Figure 2. Typical Application Circuit

Figure 3. IN and OUT Cross Connections  for a Single-Layer

Board

7

Current-Limited Switch for Single USB  Port

MAX1931

10LUMAX.EPS

Package Information (The package  drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package  outline information, go to www.maxim-ic.com/packages.)

Note: MAX1931 does not have an exposed pad.

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600                                     8

©  2002 Maxim Integrated  Products               Printed USA                                  is a registered  trademark of Maxim Integrated  Products.