19-1138; Rev 3; 12/05

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

General Description

The MAX3080–MAX3089 high-speed  transceivers for RS-485/RS-422 communication contain one driver and one receiver. These devices  feature fail-safe circuitry, which guarantees a logic-high receiver output when the receiver inputs are open or shorted. This means that the receiver output will be a logic high if all transmitters on a terminated bus are disabled  (high impedance). The MAX3080/MAX3081/MAX3082 feature reduced slew-rate drivers that minimize EMI and reduce reflec- tions caused by improperly terminated cables, allowing error-free data transmission up  to 115kbps.  The MAX3083/MAX3084/MAX3085 offer higher driver out- put slew-rate limits, allowing transmit speeds  up to

500kbps.  The MAX3086/MAX3087/MAX3088’s driver slew rates are not limited, making transmit speeds up to 10Mbps  possible.  The MAX3089’s slew rate is selectable  between 115kbps,  500kbps,  and 10Mbps by driving a selector pin with a single three-state driver.

These transceivers  typically  draw 375µA of supply current when unloaded,  or when fully loaded with the drivers disabled.

MAX3080–MAX3089

Next Generation Device Features

♦ For Fault-Tolerant Applications:

MAX3430: ±80V Fault-Protected, Fail-Safe, 1/4- Unit Load, +3.3V RS-485 Transceiver

♦ For Low-Voltage Applications:

MAX3362: +3.3V, High-Speed, RS-485/RS-422

Transceiver in a SOT23 Package

♦ For Multiple Transceiver Applications: MAX3030E–MAX3033E: ±15kV ESD-Protected,

+3.3V, Quad, RS-422 Transmitters

MAX3040–MAX3045: ±10kV ESD-Protected, Quad,

+5V, RS-485/RS-422 Transmitters

Applications

RS-422/RS-485 Communications

Level Translators

Transceivers for EMI-Sensitive Applications

Industrial-Control Local Area Networks

All devices  have a 1/8-unit-load receiver input imped-                                            Ordering Information

ance that allows up to 256 transceivers on the bus. The

MAX3082/MAX3085/MAX3088 are intended  for half- duplex communications, while the MAX3080/MAX3081/ MAX3083/MAX3084/MAX3086/MAX3087 are intended for full-duplex communications. The MAX3089 is selec- table between half-duplex and full-duplex operation. It also features independently  programmable  receiver

and transmitter output phase via separate pins.

Ordering Information continued at end of data sheet. PART TEMP RANGE PIN-PACKAGE

MAX3080CSD 0°C to +70°C 14 SO

MAX3080CPD 0°C to +70°C 14 Plastic DIP

MAX3080ESD -40°C to +85°C 14 SO

MAX3080EPD -40°C to +85°C 14 Plastic DIP

Selection Table

| Part | Half/Full Duplex | Data Rate (Mbps) | Slew Rate Limited | Low- Power Shutdown | Receiver/ Driver Enable | Quiescent Current (µA) | Transceivers On Bus | Pin Count | Industry- Standard Pinout |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX3080 | Full | 0.115 | Yes | Yes | Yes | 375 | 256 | 14 | 75180 |
| MAX3081 | Full | 0.115 | Yes | No | No | 375 | 256 | 8 | 75179 |
| MAX3082 | Half | 0.115 | Yes | Yes | Yes | 375 | 256 | 8 | 75176 |
| MAX3083 | Full | 0.5 | Yes | Yes | Yes | 375 | 256 | 14 | 75180 |
| MAX3084 | Full | 0.5 | Yes | No | No | 375 | 256 | 8 | 75179 |
| MAX3085 | Half | 0.5 | Yes | Yes | Yes | 375 | 256 | 8 | 75176 |
| MAX3086 | Full | 10 | No | Yes | Yes | 375 | 256 | 14 | 75180 |
| MAX3087 | Full | 10 | No | No | No | 375 | 256 | 8 | 75179 |
| MAX3088 | Half | 10 | No | Yes | Yes | 375 | 256 | 8 | 75176 |
| MAX3089 | Selectable | Selectable | Selectable | Yes | Yes | 375 | 256 | 14 | 75180* |

*Pin-compatible with 75180, with additional features implemented using pins 1, 6, 8, and 13.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VCC) ............................................................+7V Control Input Voltage (RE, DE)...................-0.3V  to (VCC + 0.3V) Special Input Voltage

(H/F, SRL, TXP, RXP)..................................-0.3V to (VCC + 0.3V) Driver Input Voltage (DI).............................-0.3V to (VCC + 0.3V) Driver Output Voltage (A, B, Y, Z)........................................±13V Receiver Input Voltage (A, B) ..............................................±13V Receiver Input Voltage, Full Duplex (A, B) ..........................±25V Receiver Output Voltage (RO)....................-0.3V to (VCC + 0.3V)

Continuous Power Dissipation

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) ...727mW

8-Pin SO (derate 5.88mW/°C above +70°C)................471mW

14-Pin Plastic DIP (derate 10.0mW/°C above +70°C) ....800mW

14-Pin SO (derate 8.33mW/°C above +70°C)..............667mW Operating  Temperature  Ranges

MAX308_C_ _ .....................................................0°C to +70°C MAX308_E_ _...................................................-40°C to +85°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DC ELECTRICAL CHARACTERISTICS

(VCC = +5V ± 5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +5V and TA = +25°C.) (Note 1)

| IN4 PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| DRIVER |  |  |  |  |  |
| Differential Driver Output (no load) | VOD1 | Figure 5 |  | 5 | V |
| Differential Driver Output | VOD2 | Figure 5, R = 50Ω (RS-422) |  | 2.0 | V |
|  |  | Figure 5, R = 27Ω (RS-485) |  | 1.5 |  |
| Change in Magnitude of Differential Output Voltage (Note 2) | ∆VOD | Figure 5, R = 50Ω or R = 27Ω |  | 0.2 | V |
| Driver Common-Mode Output Voltage | VOC | Figure 5, R = 50Ω or R = 27Ω |  | 3 | V |
| Change In Magnitude of Common-Mode Voltage (Note 2) | ∆VOC | Figure 5, R = 50Ω or R = 27Ω |  | 0.2 | V |
| Input High Voltage | VIH1 | DE, DI, RE, H/F, TXP, RXP |  | 2.0 | V |
| Input Low Voltage | VIL1 | DE, DI, RE, H/F, TXP, RXP |  | 0.8 | V |
| DI Input Hysteresis | VHYS | MAX3080–MAX3085, and MAX3089 with SRL = VCC or unconnected |  | 100 | mV |
| SRL Input Current | IIN1 | DE, DI, RE |  | ±2 | µA |
|  | IIN2 | H/F, TXP, RXP, internal pulldown |  | 10 40 |  |
| Input High Voltage | VIH2 | SRL |  | VCC - 0.8 | V |
| Input Middle Voltage | VIM2 | SRL (Note 3) |  | 0.4VCC 0.6VCC | V |
| Input Low Voltage | VIL2 | SRL |  | 0.8 | V |
| SRL Input Current | IIN3 | SRL = VCC |  | 75 | µA |
|  |  | SRL = GND (Note 3) |  | -75 |  |
| Input Current (A and B) Full Duplex | I | DE = GND, VCC = GND or 5.25V | VIN = 12V | 125 | µA |
|  |  |  | VIN = -7V | -75 |  |
| Output Leakage (Y and Z) Full Duplex | IO | DE = GND, VCC = GND or 5.25V | VIN = 12V | 125 | µA |
|  |  |  | VIN = -7V | -100 |  |
| Driver Short-Circuit Output Current (Note 4) | VOD1 | -7V ≤ VOUT ≤ VCC |  | -250 | mA |
|  |  | 0V ≤ VOUT ≤ 12V |  | 250 |  |
|  |  | 0V ≤ VOUT ≤ VCC |  | ±25 |  |

2

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

DC ELECTRICAL CHARACTERISTICS (continued)

(VCC = +5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +5V and TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| RECEIVER |  |  |  |  |  |
| Receiver Differential Threshold Voltage | VTH | -7V ≤ VCM ≤ +12V |  | -200 -125 -50 | mV |
| Receiver Input Hysteresis | ∆VTH |  |  | 25 | mV |
| Receiver Output High Voltage | VOH | IO = -4mA, VID = -50mV |  | VCC -1.5 | V |
| Receiver Output Low Voltage | VOL | IO = 4mA, VID = -200mV |  | 0.4 | V |
| Three-State Output Current at Receiver | IOZR | 0.4V ≤ VO ≤ 2.4V |  | ±1 | µA |
| Receiver Input Resistance | RIN | -7V ≤ VCM ≤ +12V |  | 96 | kΩ |
| Receiver Output Short-Circuit Current | IOSR | 0V ≤ VRO ≤ VCC |  | ±7 ±95 | mA |
| SUPPLY CURRENT |  |  |  |  |  |
| Supply Current | ICC | No load, RE = DI = GND or VCC, SRL = VCC | DE = VCC | 430 900 | µA |
|  |  |  | DE = GND | 375 600 |  |
|  |  | No load, RE = DI = GND or VCC, SRL = GND | DE = VCC | 475 1000 | µA |
|  |  |  | DE = GND | 420 800 |  |
| Supply Current in Shutdown Mode | ISHDN | DE = GND, VRE = VCC |  | 0.001 10 | µA |

Note 1:  All currents into the device are positive; all currents out of the device are negative. All voltages are referred to device ground unless otherwise noted.

Note 2:  ∆VOD and ∆VOC are the changes  in VOD and VOC, respectively, when the DI input changes  state.

Note 3:  The SRL pin is internally biased to VCC / 2 by a 100kΩ/100kΩ  resistor divider. It is guaranteed to be VCC / 2 if left unconnected.

Note 4:  Maximum current level applies to peak current just prior to foldback-current limiting; minimum current level applies during current limiting.

3

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

SWITCHING CHARACTERISTICS—MAX3080–MAX3082, and MAX3089 with

SRL = Unconnected

(VCC = +5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +5V and TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Input to Output | tDPLH | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | 500 2030 2600 | ns |
|  | tDPHL |  | 500 2030 2600 |  |
| Driver Output Skew ¦ tDPLH - tDPHL ¦ | tDSKEW | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | -3 ±200 | ns |
| Driver Rise or Fall Time | tDR, tDF | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | 667 1320 2500 | ns |
| Maximum Data Rate | fMAX |  | 115 | kbps |
| Driver Enable to Output High | tDZH | Figures 8 and 10, CL = 100pF, S2 closed | 3500 | ns |
| Driver Enable to Output Low | tDZL | Figures 8 and 10, CL = 100pF, S1 closed | 3500 | ns |
| Driver Disable Time from Low | tDLZ | Figures 8 and 10, CL = 15pF, S1 closed | 100 | ns |
| Driver Disable Time from High | tDHZ | Figures 8 and 10, CL = 15pF, S2 closed | 100 | ns |
| Receiver Input to Output | tRPLH, tRPHL | Figures 11 and 13; ¦ VID ¦ ≥ 2.0V; rise and fall time of VID ≤ 15ns | 127 200 | ns |
| ¦ tRPLH - tRPHL ¦ Differential Receiver Skew | tRSKD | Figures 11 and 13; ¦ VID ¦ ≥ 2.0V; rise and fall time of VID ≤ 15ns | 3 ±30 | ns |
| Receiver Enable to Output Low | tRZL | Figures 6 and 12, CL = 100pF, S1 closed | 20 50 | ns |
| Receiver Enable to Output High | tRZH | Figures 6 and 12, CL = 100pF, S2 closed | 20 50 | ns |
| Receiver Disable Time from Low | tRLZ | Figures 6 and 12, CL = 100pF, S1 closed | 20 50 | ns |
| Receiver Disable Time from High | tRHZ | Figures 6 and 12, CL = 100pF, S2 closed | 20 50 | ns |
| Time to Shutdown | tSHDN | (Note 5) | 50 200 600 | ns |
| Driver Enable from Shutdown to Output High | tDZH(SHDN) | Figures 8 and 10, CL = 15pF, S2 closed | 6000 | ns |
| Driver Enable from Shutdown to Output Low | tDZL(SHDN) | Figures 8 and 10, CL = 15pF, S1 closed | 6000 | ns |
| Receiver Enable from Shutdown to Output High | tRZH(SHDN) | Figures 6 and 12, CL = 100pF, S2 closed | 3500 | ns |
| Receiver Enable from Shutdown to Output Low | tRZL(SHDN) | Figures 6 and 12, CL = 100pF, S1 closed | 3500 | ns |

4

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

SWITCHING CHARACTERISTICS—MAX3083–MAX3085, and MAX3089 with SRL = VCC

(VCC = +5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +5V and TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Input to Output | tDPLH | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | 250 720 1000 | ns |
|  | tDPHL |  | 250 720 1000 |  |
| Driver Output Skew ¦ tDPLH - tDPHL ¦ | tDSKEW | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | -3 ±100 | ns |
| Driver Rise or Fall Time | tDR, tDF | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | 200 530 750 | ns |
| Maximum Data Rate | fMAX |  | 500 | kbps |
| Driver Enable to Output High | tDZH | Figures 8 and 10, CL = 100pF, S2 closed | 2500 | ns |
| Driver Enable to Output Low | tDZL | Figures 8 and 10, CL = 100pF, S1 closed | 2500 | ns |
| Driver Disable Time from Low | tDLZ | Figures 8 and 10, CL = 15pF, S1 closed | 100 | ns |
| Driver Disable Time from High | tDHZ | Figures 8 and 10, CL = 15pF, S2 closed | 100 | ns |
| Receiver Input to Output | tRPLH, tRPHL | Figures 11 and 13; ¦ VID ¦ ≥ 2.0V; rise and fall time of VID ≤ 15ns | 127 200 | ns |
| ¦ tRPLH - tRPHL ¦ Differential Receiver Skew | tRSKD | Figures 11 and 13; ¦ VID ¦ ≥ 2.0V; rise and fall time of VID ≤ 15ns | 3 ±30 | ns |
| Receiver Enable to Output Low | tRZL | Figures 6 and 12, CL = 100pF, S1 closed | 20 50 | ns |
| Receiver Enable to Output High | tRZH | Figures 6 and 12, CL = 100pF, S2 closed | 20 50 | ns |
| Receiver Disable Time from Low | tRLZ | Figures 6 and 12, CL = 100pF, S1 closed | 20 50 | ns |
| Receiver Disable Time from High | tRHZ | Figures 6 and 12, CL = 100pF, S2 closed | 20 50 | ns |
| Time to Shutdown | tSHDN | (Note 5) | 50 200 600 | ns |
| Driver Enable from Shutdown to Output High | tDZH(SHDN) | Figures 8 and 10, CL = 15pF, S2 closed | 4500 | ns |
| Driver Enable from Shutdown to Output Low | tDZL(SHDN) | Figures 8 and 10, CL = 15pF, S1 closed | 4500 | ns |
| Receiver Enable from Shutdown to Output High | tRZH(SHDN) | Figures 6 and 12, CL = 100pF, S2 closed | 3500 | ns |
| Receiver Enable from Shutdown to Output Low | tRZL(SHDN) | Figures 6 and 12, CL = 100pF, S1 closed | 3500 | ns |

5

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

SWITCHING CHARACTERISTICS—MAX3086–MAX3088, and MAX3089 with SRL = GND

(VCC = +5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +5V and TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Input to Output | tDPLH | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | 34 60 | ns |
|  | tDPHL |  | 34 60 |  |
| Driver Output Skew ¦ tDPLH - tDPHL ¦ | tDSKEW | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | -2.5 ±10 | ns |
| Driver Rise or Fall Time | tDR, tDF | Figures 7 and 9, RDIFF = 54Ω, CL1 = CL2 = 100pF | 14 25 | ns |
| Maximum Data Rate | fMAX |  | 10 | Mbps |
| Driver Enable to Output High | tDZH | Figures 8 and 10, CL = 100pF, S2 closed | 150 | ns |
| Driver Enable to Output Low | tDZL | Figures 8 and 10, CL = 100pF, S1 closed | 150 | ns |
| Driver Disable Time from Low | tDLZ | Figures 8 and 10, CL = 15pF, S1 closed | 100 | ns |
| Driver Disable Time from High | tDHZ | Figures 8 and 10, CL = 15pF, S2 closed | 100 | ns |
| Receiver Input to Output | tRPLH, tRPHL | Figures 11 and 13; ¦ VID ¦ ≥ 2.0V; rise and fall time of VID ≤ 15ns | 106 150 | ns |
| ¦ tRPLH - tRPHL ¦ Differential Receiver Skew | tRSKD | Figures 11 and 13; ¦ VID ¦ ≥ 2.0V; rise and fall time of VID ≤ 15ns | 0 ±10 | ns |
| Receiver Enable to Output Low | tRZL | Figures 6 and 12, CL = 100pF, S1 closed | 20 50 | ns |
| Receiver Enable to Output High | tRZH | Figures 6 and 12, CL = 100pF, S2 closed | 20 50 | ns |
| Receiver Disable Time from Low | tRLZ | Figures 6 and 12, CL = 100pF, S1 closed | 20 50 | ns |
| Receiver Disable Time from High | tRHZ | Figures 6 and 12, CL = 100pF, S2 closed | 20 50 | ns |
| Time to Shutdown | tSHDN | (Note 5) | 50 200 600 | ns |
| Driver Enable from Shutdown to Output High | tDZH(SHDN) | Figures 8 and 10, CL = 15pF, S2 closed | 250 | ns |
| Driver Enable from Shutdown to Output Low | tDZL(SHDN) | Figures 8 and 10, CL = 15pF, S1 closed | 250 | ns |
| Receiver Enable from Shutdown to Output High | tRZH(SHDN) | Figures 6 and 12, CL = 100pF, S2 closed | 3500 | ns |
| Receiver Enable from Shutdown to Output Low | tRZL(SHDN) | Figures 6 and 12, CL = 100pF, S1 closed | 3500 | ns |

Note 5:  The device is put into shutdown by bringing RE high and DE low. If the enable inputs are in this state for less than 50ns, the device is guaranteed not to enter shutdown. If the enable inputs are in this state for at least 600ns, the device is guaranteed to have entered shutdown.

6

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

Typical Operating Characteristics

(VCC = +5V, TA = +25°C, unless otherwise noted.)

525

NO-LOAD SUPPLY CURRENT (µA)

500

475

450

425

NO-LOAD SUPPLY CURRENT

vs. TEMPERATURE

A: MAX3086–MAX3088, MAX3089 WITH

SRL = GND

MAX3080/3089 TOC-16

DE = VCC

A

OUTPUT CURRENT

MAX3080/3089 TOC-2

vs. RECEIVER OUTPUT LOW VOLTAGE

| 60 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

50

40

OUTPUT CURRENT

MAX3080/3089 TOC-3

| vs. RECEIVER OUTPUT HIGH VOLTAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

30

25

20

400          B

375          A

350          B

325

300

OUTPUT CURRENT (mA)

OUTPUT CURRENT (mA)

DE = GND                   30                                                                                          15

20                                                                                          10

B: MAX3080–MAX3085,

MAX3089 WITH                          10                                                                                           5

SRL = OPEN OR VCC

0                                                                                            0

-60

-40   -20     0

20    40    60

80   100

0           1           2           3            4           5

0           1          2           3           4           5

TEMPERATURE (°C)

OUTPUT LOW VOLTAGE (V)

OUTPUT HIGH VOLTAGE (V)

SHUTDOWN CURRENT

vs. TEMPERATURE

20

18

SHUTDOWN CURRENT (nA)

16

14

12

10

8

6

4

2

0

MAX3080/3089 TOC-1

0.50

0.45

0.40

0.35

0.30

0.25

OUTPUT LOW VOLTAGE (V)

0.20

0.15

0.10

MAX3080/3089 TOC-5

RECEIVER OUTPUT LOW VOLTAGE

vs. TEMPERATURE

IRO = 8mA

4.5

4.4

4.3

4.2

4.1

OUTPUT VOLTAGE (V)

4.0

3.9

3.8

MAX3080/3089 TOC-4

| RECEIVER OUTPUT HIGH VOLTAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IRO = 8mA |  |  |  |  |  |  |  |  |

vs. TEMPERATURE

| -60 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-40   -20     0

20    40    60

80   100

-60

-40   -20    0

20    40

60    80   100

-60

-40   -20    0

20    40

60    80   100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

140

PROPAGATION DELAY (ns)

135

130

125

120

115

RECEIVER PROPAGATION DELAY (500kbps MODE) vs. TEMPERATURE

CLOAD = 100pF

112

110

PROPAGATION DELAY (ns)

108

106

104

102

100

98

96

94

MAX3080/3089 TOC-7

MAX3080/3089 TOC-8

RECEIVER PROPAGATION DELAY (10Mbps  MODE) vs. TEMPERATURE

2.20

2.10

2.05

PROPAGATION DELAY (ms)

2.00

1.95

1.90

MAX3080/3089 TOC-9

DRIVER PROPAGATION DELAY (115kbps MODE) vs. TEMPERATURE

Rt = 54Ω

-60

-40   -20     0

20    40    60

80   100

| -60 CLOAD = 100pF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

-40   -20     0

20    40    60

80   100

-60

-40   -20    0

20    40    60

80   100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

7

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

PROPAGATION DELAY (ns)

Typical Operating Characteristics (continued)

(VCC = +5V, TA = +25°C, unless otherwise noted.)

920

880

840

800

760

720

680

640

560

520

DRIVER PROPAGATION DELAY (500kbps MODE) vs. TEMPERATURE

Rt = 54Ω

MAX3080/3089 TOC-11

DRIVER PROPAGATION DELAY (10Mbps  MODE) vs. TEMPERATURE

60

55

45

40

35

PROPAGATION DELAY (ns)

30

MAX3080/3089 TOC-10

25

20

1.90

1.89

1.88

1.87

OUTPUT VOLTAGE (V)

1.85

1.84

1.83

DRIVER DIFFERENTIAL  OUTPUT VOLTAGE

vs. TEMPERATURE

MAX3080/3089 TOC-13

Rt = 54Ω

-60

-40   -20     0

20    40    60

80   100

| -60 | Rt = 54Ω |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

-40   -20     0

20    40    60

80   100

-60

-40   -20     0

20    40    60

80   100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

100

10

1

OUTPUT CURRENT (mA)

0.1

0.01

DRIVER OUTPUT CURRENT

MAX3080 TOC-12

vs. DIFFERENTIAL  OUTPUT VOLTAGE

| 140 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

120

100

80

60

OUTPUT CURRENT (mA)

40

20

0

MAX3080-25

OUTPUT CURRENT vs. DRIVER OUTPUT LOW VOLTAGE

-100

-90

OUTPUT CURRENT (mA)

-80

-70

-60

-50

-40

-30

-20

-10

0

MAX3080/3089 TOC-27

| OUTPUT CURRENT vs. DRIVER OUTPUT HIGH VOLTAGE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0            1           2           3           4           5

0         2         4         6         8        10       12

-8      -6      -4       -2       0

2       4       6

DIFFERENTIAL OUTPUT VOLTAGE (V)

OUTPUT LOW VOLTAGE (V)

OUTPUT HIGH VOLTAGE (V)

8

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

Typical Operating Characteristics (continued)

(VCC = +5V, TA = +25°C, unless otherwise noted.)

RECEIVER PROPAGATION DELAY MAX3086–MAX3088, AND MAX3089

WITH SRL = GND

MAX3080/3089 TYP-18

RECEIVER PROPAGATION DELAY MAX3080–MAX3085, AND MAX3089

WITH SRL = OPEN OR VCC

MAX3080/3089 TYP-17

VA - VB

RO

2V/div

5V/div

VA - VB

RO

2V/div

5V/div

50ns/div

50ns/div

DRIVER PROPAGATION DELAY MAX3080/MAX3081/MAX3082, AND MAX3089

WITH SRL = OPEN

MAX3080/3089 TYP-20

DI

VY - VZ

5V/div

2.5V/div

2µs/div

DRIVER PROPAGATION DELAY MAX3083/MAX3084/MAX3085, AND MAX3089

WITH SRL = VCC

MAX3080/3089 TYP-21

DRIVER PROPAGATION DELAY MAX3086/MAX3087/MAX3088, AND MAX3089

WITH SRL = GND

MAX3080/3089 TYP-22

DI

VY - VZ

5V/div

2.5V/div

DI

VY - VZ

5V/div

2.5V/div

500ns/div

50ns/div

9

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

Pin  Description

|  |  | PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- |
| MAX3080 MAX3083 MAX3086 | MAX3081 MAX3084 MAX3087 | MAX3082 MAX3085 MAX3088 | MAX3089 |  |  |  |
| FULL-DUPLEX DEVICES |  | HALF- DUPLEX DEVICES | FULL- DUPLEX MODE | HALF- DUPLEX MODE |  |  |
| — | — | — | 1 | 1 | H/F | Half/Full-Duplex Selector Pin. Connect H/F to VCC for half-duplex mode; connect to GND or leave unconnected for full-duplex mode. |
| 2 | 2 | 1 | 2 | 2 | RO | Receiver Output. When RE is low and if A - B ≥ -50mV, RO will be high; if A - B ≤ -200mV, RO will be low. |
| 3 | — | 2 | 3 | 3 | RE | Receiver Output Enable. Drive RE low to enable RO; RO is high impedance when RE is high. Drive RE high and DE low to enter low-power shutdown mode. |
| 4 | — | 3 | 4 | 4 | DE | Driver Output Enable. Drive DE high to enable driver outputs. These outputs are high impedance when DE is low. Drive RE high and DE low to enter low-power shutdown mode. |
| 5 | 3 | 4 | 5 | 5 | DI | Driver Input. With DE high, a low on DI forces noninverting output low and inverting output high. Similarly, a high on DI forces noninverting output high and inverting output low. |
| — | — | — | 6 | 6 | SRL | Slew-Rate-Limit Selector Pin. Connect SRL to GND for 10Mbps communication rate; connect to VCC for 500kbps communication rate. Leave unconnected for 115kbps communication rate. |
| 6, 7 | 4 | 5 | 7 | 7 | GND | Ground |
| — | — | — | 8 | 8 | TXP | Transmitter Phase. Connect TXP to GND, or leave floating for normal transmitter phase/polarity. Connect to VCC to invert the transmitter phase/polarity. |
| 9 | 5 | — | 9 | — | Y | Noninverting Driver Output |
| — | — | — | — | 9 | Y | Noninverting Receiver Input and Noninverting Driver Output* |
| 10 | 6 | — | 10 | — | Z | Inverting Driver Output |
| — | — | — | — | 10 | Z | Inverting Receiver Input and Inverting Driver Output* |
| 11 | 7 | — | 11 | — | B | Inverting Receiver Input |
| — | — | — | — | 11 | B | Receiver Input Resistors* |
| — | — | 7 | — | — | B | Inverting Receiver Input and Inverting Driver Output |

10

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

Pin  Description (continued)

|  |  | PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- |
| MAX3080 MAX3083 MAX3086 | MAX3081 MAX3084 MAX3087 | MAX3082 MAX3085 MAX3088 | MAX3089 |  |  |  |
| FULL-DUPLEX DEVICES |  | HALF- DUPLEX DEVICES | FULL- DUPLEX MODE | HALF- DUPLEX MODE |  |  |
| 12 | 8 | — | 12 | — | A | Noninverting Receiver Input |
| — | — | — | — | 12 | A | Receiver Input Resistors* |
| — | — | 6 | — | — | A | Noninverting Receiver Input and Noninverting Driver Output |
| — | — | — | 13 | 13 | RXP | Receiver Phase. Connect RXP to GND, or leave unconnected for normal transmitter phase/polarity. Connect to VCC to invert the receiver phase/polarity. |
| 14 | 1 | 8 | 14 | 14 | VCC | Positive Supply; 4.75V ≤ VCC ≤ 5.25V |
| 1, 8, 13 | — | — | — | — | N.C. | Not Connected. Not internally connected. |

*(MAX3089 only.) In half-duplex mode, the driver outputs serve as receiver inputs. The full-duplex  receiver inputs (A and B) will still have a 1/8-unit load, but are not connected to the receiver.

Function Tables

MAX3080/MAX3083/MAX3086

| MAX3081/MAX3084/MAX3087 TRANSMITTING |  |  |
| --- | --- | --- |
| INPUT | OUTPUTS |  |
| DI | Z | Y |
| 1 | 0 | 1 |
| 0 | 1 | 0 |

| TRANSMITTING |  |  |  |  |
| --- | --- | --- | --- | --- |
| INPUTS |  |  | OUTPUTS |  |
| RE | DE | DI | Z | Y |
| X | 1 | 1 | 0 | 1 |
| X | 1 | 0 | 1 | 0 |
| 0 | 0 | X | High- Z | High - Z |
| 1 | 0 | X | Shutdown |  |

| RECEIVING |  |  |  |
| --- | --- | --- | --- |
| INPUTS |  |  | OUTPUT |
| RE | DE | A-B | RO |
| 0 | X | ≥ -0.05V | 1 |
| 0 | X | ≤ -0.2V | 0 |
| 0 | X | Open/shorted | 1 |
| 1 | 1 | X | High-Z |
| 1 | 0 | X | Shutdown |

X = Don’t care

| Shutdown mode, driver and receiver outputs high impedance RECEIVING |  |
| --- | --- |
| INPUTS | OUTPUT |
| A-B | RO |
| ≥ -0.05V | 1 |
| ≤ -0.2V | 0 |
| Open/shorted | 1 |

11

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

Function Tables (continued)

MAX3082/MAX3085/MAX3088

| MAX3089 TRANSMITTING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| INPUTS |  |  |  | OUTPUTS |  |
| TXP | RE | DE | DI | Z | Y |
| 0 | X | 1 | 1 | 0 | 1 |
| 0 | X | 1 | 0 | 1 | 0 |
| 1 | X | 1 | 1 | 1 | 0 |
| 1 | X | 1 | 0 | 0 | 1 |
| X | 0 | 0 | X | High- Z | High - Z |
| X | 1 | 0 | X | Shut | own |

| RECEIVING |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| INPUTS |  |  |  |  |  | OUTPUT |
| H/F | RXP | RE | DE | A-B | Y-Z | RO |
| 0 | 0 | 0 | X | ≥ -0.05V | X | 1 |
| 0 | 0 | 0 | X | ≤ -0.2V | X | 0 |
| 0 | 1 | 0 | X | ≥ -0.05V | X | 0 |
| 0 | 1 | 0 | X | ≤ -0.2V | X | 1 |
| 1 | 0 | 0 | 0 | X | ≥ -0.05V | 1 |
| 1 | 0 | 0 | 0 | X | ≤ -0.2V | 0 |
| 1 | 1 | 0 | 0 | X | ≥ -0.05V | 0 |
| 1 | 1 | 0 | 0 | X | ≤ -0.2V | 1 |
| 0 | 0 | 0 | X | Open/ shorted | X | 1 |
| 1 | 0 | 0 | 0 | X | Open/ shorted | 1 |
| 0 | 1 | 0 | X | Open/ shorted | X | 0 |
| 1 | 1 | 0 | 0 | X | Open/ shorted | 0 |
| X | X | 1 | 1 | X | X | High-Z |
| X | X | 1 | 0 | X | X | Shutdown |

| TRANSMITTING |  |  |  |  |
| --- | --- | --- | --- | --- |
| INPUTS |  |  | OUTPUTS |  |
| RE | DE | DI | B/Z | A/Y |
| X | 1 | 1 | 0 | 1 |
| X | 1 | 0 | 1 | 0 |
| 0 | 0 | X | High- Z | High - Z |
| 1 | 0 | X | Shutdown |  |

| RECEIVING |  |  |  |
| --- | --- | --- | --- |
| INPUTS |  |  | OUTPUT |
| RE | DE | A-B | RO |
| 0 | X | ≥ -0.05V | 1 |
| 0 | X | ≤ -0.2V | 0 |
| 0 | X | Open/shorted | 1 |
| 1 | 1 | X | High- Z |
| 1 | 0 | X | Shutdown |

12

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

TOP VIEW TOP VIEW

N.C.    1

RO    2                 R RE   3

DE    4

DI    5

D GND    6

14   VCC

13   N.C.

12   A

11   B

10    Z

9    Y

DI            5

RO            2

VCC

DE

4           14

D

R

0.1µF

9      Y

10

Z

12     A

Rt

11

B

MAX3080

MAX3083

MAX3086

VCC    RE

MAX3080–MAX3089

Rt                           R                                RO

D                 DI

GND    7

DIP/SO

8    N.C.

NC  1, 8, 13

| 3 6, 7 GND RE |  | GND | DE |
| --- | --- | --- | --- |

Figure 1.  MAX3080/MAX3083/MAX3086 Pin Configuration and Typical Full-Duplex Operating  Circuit

TOP VIEW TOP VIEW

VCC     1             R RO   2

3

8    A                           DI

7     B

VCC  1

D

0.1µF

5   Y

6   Z

MAX3081

MAX3084

MAX3087

Rt

VCC

R                              RO

DI    3

GND    4

6     Z

2

D                                 5     Y                             RO

DIP/SO

8   A

R           7        Rt

B

D                DI

4    GND

GND

Figure 2.  MAX3081/MAX3084/MAX3087 Pin Configuration and Typical Full-Duplex Operating  Circuit

TOTOPPVIVEIWEW

RO    1             R RE   2

DE    3

8     VCC

7     B

6     A

RO    1             R RE   2

DE    3

8     VCC

7  B

Rt

6

0.1µF

MAX3082

MAX3085                                     DE

MAX3088

DI D

B Rt

DI    4           D

5     GND

DI    4           D

A

5    GND

A                                          RO R

DIP/SO                                                                                                                                        RE

NOTE: PIN LABELS Y AND Z ON TIMING, TEST, AND WAVEFORM DIAGRAMS REFER TO PINS A AND B WHEN DE IS HIGH.

Figure 3. MAX3082/MAX3085/MAX3088 Pin Configuration and Typical Half-Duplex Operating Circuit

13

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

Detailed Description

The MAX3080–MAX3089 high-speed  transceivers for RS-485/RS-422 communication contain one driver and one receiver. These devices  feature fail-safe circuitry, which guarantees a logic-high receiver output when the receiver inputs are open or shorted, or when they are connected  to a terminated transmission line with all drivers disabled (see Fail-Safe section). The MAX3080/ MAX3081/MAX3082 feature reduced  slew-rate drivers that minimize EMI and reduce  reflections caused  by improperly  terminated cables, allowing error-free data transmission up to 115kbps  (see Reduced  EMI and Reflections section). The MAX3083/MAX3084/MAX3085 offer higher driver output slew-rate limits, allowing transmit  speeds  up  to 500kbps.  The MAX3086/ MAX3087/MAX3088’s driver slew rates are not limited, making transmit speeds  up to 10Mbps  possible. The MAX3089’s slew rate is selectable  between 115kbps,

500kbps, and 10Mbps by driving a selector pin with a three-state driver.

The MAX3082/MAX3085/MAX3088 are half-duplex transceivers, while the MAX3080/MAX3081/MAX3083/ MAX3084/MAX3086/MAX3087 are full-duplex  trans- ceivers. The MAX3089 is selectable between half- and full-duplex  communication  by driving  a selector pin high or low, respectively.

All of these parts operate from a single +5V supply. Drivers are output short-circuit current limited. Thermal shutdown circuitry  protects  drivers against excessive power dissipation.  When activated,  the thermal shut- down circuitry  places  the driver outputs into a high- impedance state.

Receiver Input Filtering The receivers  of the MAX3080–MAX3085, and the MAX3089 when operating  in 115kbps  or 500kbps mode, incorporate  input filtering in addition  to input hysteresis. This filtering enhances noise immunity with differential signals that have very slow rise and fall times. Receiver propagation  delay increases by 20% due to this filtering.

VCC            RE

MAX3089

A

RD

TOP VIEW

H/F    1

RO    2

RE   3

DE    4

DI    5

SRL    6

GND    7

MAX3089

DIP/SO

14   V CC

13   RXP

12    A

11    B

10    Z

9    Y

8    TXP

B

H/F

Z

TXP

Y

DI

NOTE: SWITCH  POSITIONS INDICATED FOR H/F = GND

GND       DE          SRL

Figure 4. MAX3089 Pin Configuration and Functional Diagram

14

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

Y

R                                                                                                               RECEIVER OUTPUT

TEST POINT

1kΩ

S1

VCC

VOD

Z

R           VOC

CRL

15pF

1kΩ

S2

Figure 5.  Driver DC Test Load

Fail-Safe

Figure 6.  Receiver Enable/Disable Timing Test Load

TXP and RXP low, connect  them to ground,  or leave

The MAX3080 family guarantees a logic-high  receiver output when the receiver inputs are shorted or open, or when they are connected  to a terminated transmission line with all drivers disabled. This is done by setting the receiver threshold between -50mV and -200mV. If the differential receiver input voltage (A-B) is greater than or equal to -50mV, RO is logic high. If A-B is less than or equal to -200mV, RO is logic low. In the case of a terminated  bus with all transmitters disabled,  the receiver’s differential input voltage is pulled  to 0V by the termination. With the receiver thresholds of the MAX3080 family, this results in a logic high with a 50mV minimum noise margin.  Unlike previous  fail-safe devices, the -50mV to -200mV threshold complies with the ±200mV EIA/TIA-485 standard.

MAX3089 Programming The MAX3089 has several programmable  operating modes. Transmitter rise and fall times are programma- ble between 2500ns, 750ns, and 25ns, resulting in maximum  data  rates  of  115kbps,  500kbps,  and

10Mbps, respectively.  To select the desired data rate, drive SRL to one of three possible  states by using a three-state driver, by connecting  it to VCC or GND, or by leaving it unconnected.  For 115kbps operation, set the three-state device  in high-impedance  mode or leave SRL unconnected.  For 500kbps operation, drive SRL high or connect it to VCC. For 10Mbps operation, drive SRL low or connect  it to GND. SRL can be changed  during  operation without interrupting  data communications.

Occasionally,  twisted-pair  lines are connected  back- ward from normal orientation. The MAX3089 has two pins that invert the phase of the driver and the receiver to correct for this problem. For normal operation, drive

them unconnected (internal pulldown). To invert the dri-

MAX3080–MAX3089

ver phase, drive TXP high or connect  it to VCC. To invert the receiver phase, drive RXP high or connect it to VCC. Note that the receiver threshold is positive when RXP is high.

The MAX3089 can operate in full- or half-duplex mode. Drive the H/F pin low, leave it unconnected  (internal pulldown),  or connect  it to GND for full-duplex  opera- tion, and drive it high for half-duplex operation. In full- duplex mode, the pin configuration  of the driver and receiver is the same as that of a MAX3080 (Figure 4). In half-duplex  mode, the receiver inputs are switched to the driver outputs, connecting outputs Y and Z to inputs A and B, respectively. In half-duplex mode, the internal full-duplex receiver input resistors are still connected to pins 11 and 12.

Applications Information

256 Transceivers on  the Bus The standard RS-485 receiver input impedance is 12kΩ (one-unit load), and the standard driver can drive up to

32 unit loads. The MAX3080 family of transceivers have a 1/8-unit-load receiver input impedance (96kΩ), allow- ing up to 256 transceivers to be connected  in parallel on one communication  line. Any combination  of these devices and/or other RS-485 transceivers with a total of

32 unit loads or less can be connected to the line.

Reduced EMI  and Reflections The MAX3080–MAX3085, and MAX3089 with SRL = VCC or unconnected,  are slew-rate limited, minimizing EMI and reducing  reflections caused  by improperly  termi- nated cables. Figure 14 shows the driver output wave- form and  its Fourier analysis of a 20kHz signal transmitted by a MAX3086/MAX3087/MAX3088, and MAX3089 with SRL = GND. High-frequency  harmonic

15

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

3V

DE                                           CL1

DI                   Y                            RDIFF

VID                                                                                                                                             OUTPUT

UNDER TEST

Z                                                                                                                                                                 CL

CL2

500Ω

VCC

S1

S2

Figure 7.  Driver Timing Test Circuit                                                          Figure 8.  Driver Enable/Disable Timing Test Load

MAX3080–MAX3089

3V

DI                 1.5V

0V

tPLH

tPHL

1.5V

1/2 VO

3V

DE                   1.5V                                                                                   1.5V

0V

tZL(SHDN), tZL                      tLZ

Z

Y

V          VO

VO

1/2 VO

VDIFF

= V (Y) - V (Z)

Y, Z

VOL

Y, Z

2.3V  OUTPUT NORMALLY LOW

OUTPUT NORMALLY HIGH

2.3V

VOL +0.5V

VOH -0.5V

DIFF   0V

-VO

10%

tR

90%                            90%

tF

tSKEW = | tPLH - tPHL |

10%                                 0V

tZH(SHDN), tZH                    tHZ

Figure 9.  Driver Propagation  Delays                                                        Figure 10.  Driver Enable and Disable Times (except

MAX3081/MAX3084/MAX3087)

VOH

RO

VOL

1.5V

OUTPUT

1.5V

3V RE

0V

VCC

1.5V                                                                                   1.5V

tZL(SHDN), tZL                      tLZ

1V       A

-1V    B

tPHL

INPUT

tPLH

RO                                             1.5V

OUTPUT NORMALLY LOW

OUTPUT NORMALLY HIGH

VOL

+ 0.5V

RO                                            1.5V

0V

tZH(SHDN), tZH                    tHZ

VOH - 0.5V

Figure 11. Receiver Propagation Delays                                              Figure 12. Receiver Enable and Disable Times (except

MAX3081/MAX3084/MAX3087)

16

MAX3080/3089 FIG-14

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

ATE

B

VID                                                 RR A

RECEIVER OUTPUT

20dB/div

0Hz

100kHz/div

1MHz

MAX3080/3089 FIG-15

Figure 13.  Receiver Propagation  Delay Test Circuit

MAX3080–MAX3089

MAX3080/3089 FIG-16

Figure 14.  Driver Output Waveform and FFT Plot of MAX3086/MAX3087/MAX3088, and MAX3089 with SRL = GND, Transmitting  a 20kHz Signal

20dB/div

20dB/div

0Hz

100kHz/div

1MHz

0Hz

100kHz/div

1MHz

Figure 15.  Driver Output Waveform and FFT Plot of

MAX3083/MAX3084/MAX3085, and MAX3089 with SRL = VCC, Transmitting  a 20kHz Signal

Figure 16.  Driver Output Waveform and FFT Plot of

MAX3080/MAX3081/MAX3082, and MAX3089

with SRL = Unconnected, Transmitting a 20kHz Signal

components  with large amplitudes  are evident. Figure

15 shows the same signal displayed  for a MAX3083/ MAX3084/MAX3085, and MAX3089 with SRL = VCC), transmitting under the same conditions.  Figure 15’s high-frequency  harmonic components  are much lower in amplitude, compared with Figure 14’s, and the poten- tial  for  EMI is  significantly  reduced.  Figure  16 shows the same signal displayed  for a MAX3080/ MAX3081/MAX3082, and MAX3089 with SRL = uncon- nected, transmitting under the same conditions. Figure

16’s high-frequency  harmonic components  are even lower.

In general, a transmitter’s rise time relates directly to the length of an unterminated stub, which can be driven with only minor waveform reflections. The following equation expresses this relationship conservatively:

Length = tRISE / (10 x 1.5ns/ft)

where tRISE is the transmitter’s rise time.

For example, the MAX3080’s rise time is typically

1320ns, which results in excellent waveforms with a stub length up to 90 feet. A system can work well with longer unterminated stubs, even with severe reflections, if the waveform settles out before the UART samples them.

17

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

RO                   R

RE

DE

MAX3080/MAX3081/MAX3083/ MAX3084/MAX3086/MAX3087/ MAX3089 (FULL DUPLEX)

A

B

120Ω                           DATA IN

Z

DI

VA - VB

MAX3080/3089 FIG-18

5V/div

1V/div

DI            D

120Ω

Y

DATA OUT

RO

5V/div

NOTE:  RE AND DE ON MAX3080/MAX3083/MAX3086/MAX3089 ONLY.

5µs/div

Figure 17.  Line Repeater for MAX3080/MAX3081/MAX3083/ MAX3084/MAX3086/MAX3087, and MAX3089

MAX3080–MAX3089

in Full-Duplex Mode

Figure 18.  MAX3080/MAX3081/MAX3082, and MAX3089

with SRL = Unconnected, System Differential Voltage at 50kHz

Driving 4000 feet of Cable

Low-Power Shutdown Mode

(Except MAX3082/MAX3085/MAX3088) Low-power shutdown mode is initiated by bringing both RE high and DE low. In shutdown, the devices typically draw only 1nA of supply current.

RE and DE may be driven simultaneously; the parts are guaranteed not to enter shutdown if RE is high and DE is low for less than 50ns. If the inputs are in this state for at least 600ns, the parts are guaranteed  to enter shutdown.

Enable times t ZH  and t ZL  in the Switching  Char- acteristics  tables assume the part was not in a low- power shutdown state. Enable times tZH(SHDN)  and tZL(SHDN)  assume the parts were shut down. It takes drivers and receivers longer to become  enabled  from low-power shutdown mode (tZH(SHDN), tZH(SHDN)) than from driver/receiver-disable  mode (tZH, tZL).

Driver Output Protection Two mechanisms prevent excessive output current and power dissipation  caused  by faults or by bus con- tention. The first, a foldback  current limit on the output stage, provides immediate protection against short cir- cuits over the whole common-mode voltage range (see Typical Operating Characteristics). The second, a ther- mal shutdown circuit,  forces the driver outputs into a high-impedance  state if the die temperature becomes excessive.

Line Length vs.  Data Rate

The RS-485/RS-422 standard covers line lengths up to

4000 feet. For line lengths greater than 4000 feet, use the repeater application shown in Figure 17.

Figures 18, 19, and 20 show the system differential volt- age for the parts driving 4000 feet of 26AWG twisted- pair wire at 110kHz into 120Ω loads.

Typical Applications The MAX3082/MAX3085/MAX3088/MAX3089 trans- ceivers are designed for bidirectional data communica- tions on multipoint bus transmission lines. Figures 21 and 22 show typical  network applications  circuits. These parts can also be used as line repeaters, with cable lengths longer than 4000 feet, as shown in Figure 17.

To minimize reflections, the line should be terminated at both ends in its characteristic  impedance,  and stub lengths off the main line should be kept as short as possible.  The slew-rate-limited MAX3082/MAX3085, and the two modes of the MAX3089, are more tolerant of imperfect termination.

18

Fail-Safe, High-Speed (10Mbps), Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080/3089  FIG-19

MAX3080/3089 FIG-20

DI                                                                                                             5V/div                                  DI

5V/div

VA - VB

1V/div

VA - VB

1V/div

RO                                                                                                                         5V/div                                 RO

5V/div

2µs/div

1µs/div

Figure 19.  MAX3083/MAX3084/MAX3085, and MAX3089 with SRL = VCC, System Differential Voltage at 50kHz Driving 4000 feet of Cable

MAX3080–MAX3089

Figure 20.  MAX3086/MAX3087/MAX3088, and MAX3089 with

SRL = GND, System Differential Voltage at 200kHz Driving

4000 feet of Cable

DI                                                          B/Z*

D

120Ω                                                                                                                  120Ω

DE B

D

DI

DE                                                               A/Y*                B                  A

B                  A                                   A

RO                                 R

RE

MAX3082

MAX3085

MAX3088

R                               RO RE

R                                                                    R D                                                           D

*MAX3089 (HALF-DUPLEX)

DI                 DE

RO  RE

DI                 DE

RO  RE

Figure 21. Typical Half-Duplex RS-485 Network

19

Fail-Safe, High-Speed (10Mbps),

Slew-Rate-Limited RS-485/RS-422 Transceivers

MAX3080–MAX3089

RO                         R

RE DE

DI                D

A                                                                                                                                                                                                 Y

120Ω                                                                                                                                                                        120Ω                           D              DI B                                                                                                                                                                                                      Z

DE Z                                                                                                                                                                                                 B

RE

120Ω                                                                                                                                                                 120Ω                 R                        RO

Y                                                                                                                                                                                                 A

Y              Z        B               A

Y               Z        B               A

R                                                                                        R

D                                                                            D                                                        MAX3080

MAX3081

MAX3083

DI            DE

RE RO

DI              DE  RE RO

MAX3084

MAX3086

MAX3087

NOTE:  RE AND DE ON MAX3080/MAX3083/MAX3086/MAX3089 ONLY.

MAX3089 (FULL-DUPLEX)

Figure 22.  Typical Full-Duplex RS-485 Network

Ordering Information (continued)

PART TEMP RANGE PIN-PACKAGE

MAX3086CSD 0°C to +70°C 14 SO

MAX3086CPD 0°C to +70°C 14 Plastic DIP

MAX3086ESD -40°C to +85°C 14 SO

MAX3086EPD -40°C to +85°C 14 Plastic DIP

MAX3087CSA 0°C to +70°C 8 SO

MAX3087CPA 0°C to +70°C 8 Plastic DIP

MAX3087ESA -40°C to +85°C 8 SO

MAX3087EPA -40°C to +85°C 8 Plastic DIP

MAX3088CSA 0°C to +70°C 8 SO

MAX3088CPA 0°C to +70°C 8 Plastic DIP

MAX3088ESA -40°C to +85°C 8 SO

MAX3088EPA -40°C to +85°C 8 Plastic DIP

MAX3089CSD 0°C to +70°C 14 SO

MAX3089CPD 0°C to +70°C 14 Plastic DIP

MAX3089ESD -40°C to +85°C 14 SO

MAX3089EPD -40°C to +85°C 14 Plastic DIP

Package Information

For the latest package outline information, go to

www.maxim-ic.com/packages. PART TEMP RANGE PIN-PACKAGE

MAX3081CSA 0°C to +70°C 8 SO

MAX3081CPA 0°C to +70°C 8 Plastic DIP

MAX3081ESA -40°C to +85°C 8 SO

MAX3081EPA -40°C to +85°C 8 Plastic DIP

MAX3082CSA 0°C to +70°C 8 SO

MAX3082CPA 0°C to +70°C 8 Plastic DIP

MAX3082ESA -40°C to +85°C 8 SO

MAX3082EPA -40°C to +85°C 8 Plastic DIP

MAX3083CSD 0°C to +70°C 14 SO

MAX3083CPD 0°C to +70°C 14 Plastic DIP

MAX3083ESD -40°C to +85°C 14 SO

MAX3083EPD -40°C to +85°C 14 Plastic DIP

MAX3084CSA 0°C to +70°C 8 SO

MAX3084CPA 0°C to +70°C 8 Plastic DIP

MAX3084ESA -40°C to +85°C 8 SO

MAX3084EPA -40°C to +85°C 8 Plastic DIP

MAX3085CSA 0°C to +70°C 8 SO

MAX3085CPA 0°C to +70°C 8 Plastic DIP

MAX3085ESA -40°C to +85°C 8 SO

MAX3085EPA -40°C to +85°C 8 Plastic DIP

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

20                                      Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA 94086 408-737-7600

©  2007 Maxim Integrated  Products                                                               is a registered  trademark  of Maxim Integrated  Products, Inc.