19-0273; Rev 7; 1/07

3.0V to 5.5V, Low-Power, up to 1Mbps, True RS-232

Transceivers Using Four 0.1µF External Capacitors

General Description

The MAX3222/MAX3232/MAX3237/MAX3241 trans- ceivers have a proprietary  low-dropout  transmitter out- put stage enabling true RS-232 performance  from a

3.0V to 5.5V supply  with a dual charge  pump.  The devices require only four small 0.1µF external charge- pump  capacitors.  The MAX3222, MAX3232, and MAX3241  are  guaranteed  to  run  at  data  rates  of

120kbps while maintaining RS-232 output levels. The MAX3237 is guaranteed to run at data rates of 250kbps in the normal operating  mode  and 1Mbps  in the MegaBaud™ operating mode, while maintaining RS-232 output levels.

The MAX3222/MAX3232 have 2 receivers  and  2 drivers. The MAX3222 features a 1µA shutdown mode that reduces  power consumption  and extends battery life in portable  systems. Its receivers remain active in shutdown mode, allowing external devices  such as modems to be monitored using only 1µA supply cur- rent. The MAX3222 and MAX3232 are pin, package, and functionally compatible  with the industry-standard MAX242 and MAX232, respectively.

The MAX3241 is a complete  serial port (3 drivers/

5 receivers) designed  for notebook and subnotebook computers. The MAX3237 (5 drivers/3 receivers) is ideal for fast modem applications.  Both these devices feature a shutdown mode in which all receivers can remain active while using only 1µA supply current. Receivers R1 (MAX3237/MAX3241) and R2 (MAX3241) have extra out- puts in addition to their standard outputs. These extra outputs are always active, allowing external devices such as a modem to be monitored without forward bias- ing the protection diodes in circuitry that may have VCC completely removed.

The MAX3222, MAX3237, and MAX3241 are available in space-saving TSSOP and SSOP packages.

Next Generation Device Features

♦ For Smaller Packaging: MAX3228E/MAX3229E: +2.5V to +5.5V RS-232

Transceivers in UCSP™

♦ For Integrated ESD Protection: MAX3222E/MAX3232E/MAX3237E/MAX3241E*/ MAX3246E: ±15kV ESD-Protected, Down to 10nA,

3.0V to 5.5V, Up to 1Mbps, True RS-232

Transceivers

♦ For Low-Voltage or Data Cable Applications: MAX3380E/MAX3381E: +2.35V to +5.5V, 1µA,

2 Tx/2 Rx RS-232 Transceivers with ±15kV ESD-Protected I/O and Logic Pins

Ordering Information

PKG PART TEMP RANGE PIN-PACKAGE CODE

MAX3222CUP+ 0°C to +70°C 20 TSSOP U20+2

MAX3222CAP+ 0°C to +70°C 20 SSOP A20+1

MAX3222CWN+ 0°C to +70°C 18 SO W18+1

MAX3222CPN+ 0°C to +70°C 18 Plastic Dip P18+5

+Denotes  lead-free package.

Ordering Information  continued at end of data sheet.

Pin  Configurations

TOP VIEW 	   	+

Applications Notebook, Subnotebook, and Palmtop Computers High-Speed Modems

Battery-Powered Equipment Hand-Held Equipment Peripherals

Printers

Typical Operating Circuits appear at end of data sheet.

EN   1

C1+    2

V+    3

C1-  4

C2+    5

C2-  6

V-   7

T2OUT   8

R2IN    9

MAX3222

MAX3222

DIP/SO

18   SHDN

17    VCC

16    GND

15    T1OUT

14   R1IN

13   R1OUT

12    T1IN

11   T2IN

10    R2OUT

MegaBaud and UCSP are trademarks of Maxim Integrated Products, Inc.

Pin Configurations continued at end of data sheet.

MAX3222/MAX3232/MAX3237/MAX3241*

*Covered by U.S. Patent numbers 4,636,930; 4,679,134; 4,777,577; 4,797,899; 4,809,152; 4,897,774; 4,999,761; and other patents pending.

Maxim Integrated Products   1

ABSOLUTE MAXIMUM RATINGS

VCC ...........................................................................-0.3V to +6V V+ (Note 1) ...............................................................-0.3V to +7V V- (Note 1) ................................................................+0.3V to -7V V+ + V- (Note 1)...................................................................+13V Input Voltages

T_IN, SHDN, EN ...................................................-0.3V to +6V MBAUD...................................................-0.3V to (VCC + 0.3V) R_IN .................................................................................±25V

Output Voltages

T_OUT ...........................................................................±13.2V R_OUT ....................................................-0.3V to (VCC + 0.3V)

Short-Circuit  Duration

T_OUT ....................................................................Continuous

Continuous Power Dissipation  (TA = +70°C)

16-Pin TSSOP (derate 6.7mW/°C above +70°C) .............533mW

16-Pin Narrow SO (derate 8.70mW/°C above +70°C) ....696mW

16-Pin Wide SO (derate 9.52mW/°C above +70°C) ........762mW

16-Pin Plastic DIP (derate 10.53mW/°C above +70°C)...842mW

18-Pin SO (derate 9.52mW/°C above +70°C)..............762mW

18-Pin Plastic DIP (derate 11.11mW/°C above +70°C) ..889mW

20-Pin SSOP (derate 7.00mW/°C above +70°C) .........559mW

20-Pin TSSOP (derate 8.0mW/°C above +70°C) .............640mW

28-Pin TSSOP (derate 8.7mW/°C above +70°C) .............696mW

28-Pin SSOP (derate 9.52mW/°C above +70°C) .........762mW

28-Pin SO (derate 12.50mW/°C above +70°C) .....................1W Operating  Temperature  Ranges

MAX32_ _C_ _.....................................................0°C  to +70°C

MAX32_ _E_ _ .................................................-40°C to +85°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX3222/MAX3232/MAX3237/MAX3241

Note 1: V+ and V- can have a maximum magnitude  of 7V, but their absolute difference  cannot exceed 13V.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +3.0V to +5.5V, C1–C4 = 0.1µF (Note 2), TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

TA = +25°C

VCC = 5.0V

| PARAMETER | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| DC CHARACTERISTICS |  |  |  |  |  |
| VCC Power-Supply Current | No load, VCC = 3.3V or 5.0V, TA = +25°C |  | MAX3222/MAX3232/ MAX3241 | 0.3 1.0 | mA |
|  |  |  | MAX3237 | 0.5 2.0 |  |
| Shutdown Supply Current | SHDN = GND, TA = +25°C |  |  | 1.0 10 | µA |
| LOGIC INPUTS AND RECEIV | R OUTPUTS |  |  |  |  |
| Input Logic Threshold Low (Note 3) | T_IN, EN, SHDN, MBAUD |  |  | 0.8 | V |
| Input Logic Threshold High (Note 3) | VCC = 3.3V |  |  | 2.0 | V |
|  | VCC = 5.0V |  |  | 2.4 |  |
| Input Leakage Current | T_IN, EN, SHDN, MBAUD |  |  | ±0.01 ±1.0 | µA |
| Output Leakage Current | Receivers disabled |  |  | ±0.05 ±10 | µA |
| Output Voltage Low | IOUT = 1.6mA |  |  | 0.4 | V |
| Output Voltage High | IOUT = -1.0mA |  |  | VCC - 0.6 VCC - 0.1 | V |
| RECEIVER INPUTS |  |  |  |  |  |
| Input Voltage Range |  |  |  | -25 25 | V |
| Input Threshold Low | TA = +25°C | VCC = 3.3V |  | 0.6 1.2 | V |
|  |  | VCC = 5.0V |  | 0.8 1.5 |  |
| Input Threshold High |  | VCC = 3.3V |  | 1.5 2.4 | V |
|  |  | VCC = 5.0V |  | 1.8 2.4 |  |

MAX3222/MAX3232/MAX3237/MAX3241

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +3.0V to +5.5V, C1–C4 = 0.1µF (Note 2), TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Input Hysteresis |  | 0.3 | V |
| Input Resistance | TA = +25°C | 3 5 7 | kΩ |
| TRANSMITTER OUTPUTS |  |  |  |
| Output Voltage Swing | All transmitter outputs loaded with 3kΩ to ground | ±5.0 ±5.4 | V |
| Output Resistance | VCC = V+ = V- = 0V, TOUT = ±2V | 300 10M | Ω |
| Output Short-Circuit Current |  | ±35 ±60 | mA |
| Output Leakage Current | VOUT = ±12V, VCC = 0V or 3V to 5.5V, transmitters disabled | ±25 | µA |
| MOUSE DRIVEABILITY (MAX | 3241) |  |  |
| Transmitter Output Voltage | T1IN = T2IN = GND, T3IN = VCC, T3OUT loaded with 3kΩ to GND, T1OUT and T2OUT loaded with 2.5mA each | ±5.0 | V |

TIMING CHARACTERISTICS—MAX3222/MAX3232/MAX3241

(VCC = +3.0V to +5.5V, C1–C4 = 0.1µF (Note 2), TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.)

| L PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Maximum Data Rate | RL = 3kΩ, CL = 1000pF, one transmitter switching |  | 120 235 | kbps |
| Receiver Propagation Delay | R_IN to R_OUT, CL = 150pF | tPHL | 0.3 | µs |
|  |  | tPLH | 0.3 |  |
| Receiver Output Enable Time | Normal operation |  | 200 | ns |
| Receiver Output Disable Time | Normal operation |  | 200 | ns |
| Transmitter Skew | ¦ tPHL - tPLH ¦ |  | 300 | ns |
| Receiver Skew | ¦ tPHL - tPLH ¦ |  | 300 | ns |
| Transition-Region Slew Rate | VCC = 3.3V, RL = 3kΩ to 7kΩ, +3V to -3V or -3V to +3V, TA = +25°C, one transmitter switching | CL = 150pF to 1000pF | 6 30 | V/µs |
|  |  | C = 150pF to 2500pF | 4 30 |  |

TIMING CHARACTERISTICS—MAX3237

(VCC = +3.0V to +5.5V, C1–C4 = 0.1µF (Note 2), TA = TMIN to TMAX, unless otherwise noted.  Typical values are at TA = +25°C.)

| PARAMETER | CONDITIONS |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Data Rate | RL = 3kΩ, CL = 1000pF, one transmitter switching, MBAUD = GND |  |  |  | 250 | kbps |
|  | VCC = 3.0V to 4.5V, RL = 3kΩ, CL = 250pF, one transmitter switching, MBAUD = VCC |  |  |  | 1000 |  |
|  | VCC = 4.5V to 5.5V, RL = 3kΩ, CL = 1000pF, one transmitter switching, MBAUD = VCC |  |  |  | 1000 |  |
| Receiver Propagation Delay | R_IN to R_OUT, CL = 150pF |  | tPHL |  | 0.15 | µs |
|  |  |  | tPLH |  | 0.15 |  |
| Receiver Output Enable Time | Normal operation |  |  |  | 200 | ns |
| Receiver Output Disable Time | Normal operation |  |  |  | 200 | ns |
| Transmitter Skew | ¦ tPHL - tPLH ¦, MBAUD = GND |  |  |  | 100 | ns |
|  | ¦ tPHL - tPLH ¦, MBAUD = VCC |  |  |  | 25 | ns |
| Receiver Skew | ¦ tPHL - tPLH ¦ |  |  |  | 50 | ns |
| Transition-Region Slew Rate | VCC = 3.3V, RL = 3Ω to 7kΩ, +3V to -3V or -3V to +3V, TA = +25°C | CL = 150pF to 1000pF |  | MBAUD = GND | 6 30 | V/µs |
|  |  |  |  | MBAUD = VCC | 24 150 |  |
|  |  | CL = 150pF to 2500pF, MBAUD = GND |  |  | 4 30 |  |

Note 2:  MAX3222/MAX3232/MAX3241: C1–C4 = 0.1µF tested at 3.3V ±10%; C1 = 0.047µF, C2–C4 = 0.33µF tested at 5.0V ±10%.

MAX3237: C1–C4 = 0.1µF tested at 3.3V ±5%; C1–C4 = 0.22µF tested at 3.3V ±10%; C1 = 0.047µF, C2–C4 = 0.33µF tested at 5.0V ±10%.

Note 3:  Transmitter input hysteresis is typically  250mV.

Typical Operating Characteristics

(VCC = +3.3V, 235kbps  data rate, 0.1µF capacitors, all transmitters loaded with 3kΩ, TA = +25°C, unless otherwise noted.)

MAX3222/MAX3232

MAX3222-01

TRANSMITTER  OUTPUT VOLTAGE

vs. LOAD CAPACITANCE

| 6 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | VOUT+ |  |  |  |
|  |  |  |  |  |  |  | VOUT- |  |  |  |

MAX3222/MAX3232

SLEW RATE

MAX3222-02

vs. LOAD CAPACITANCE

22

MAX3222/MAX3232

MAX3222-03

SUPPLY CURRENT vs. LOAD CAPACITANCE WHEN TRANSMITTING DATA

40

4

3

0

-1

-2

-3

-4

TRANSMITTER OUTPUT VOLTAGE (V)

-5

-6

0	1000

2000	3000

4000	5000

18

SLEW RATE (V/µs)

16

14

12

10

8

6

2

0

150

1000

+SLEW

2000	3000

4000	5000

30

25

20

15

SUPPLY CURRENT (mA)

10

5

0

0	1000

235kbps

120kbps

2000	3000

4000	5000

MAX3222/MAX3232/MAX3237/MAX3241

LOAD CAPACITANCE (pF)

LOAD CAPACITANCE (pF)

LOAD CAPACITANCE (pF)

Typical Operating Characteristics (continued)

(VCC = +3.3V, 235kbps  data rate, 0.1µF capacitors, all transmitters loaded with 3kΩ, TA = +25°C, unless otherwise noted.)

5.0

2.5

0

-2.5

TRANSMITTER OUTPUT VOLTAGE (V)

-5.0

-7.5

MAX3241

TRANSMITTER  OUTPUT VOLTAGE

vs. LOAD CAPACITANCE

MAX3222-04

VOUT+

1 TRANSMITTER AT 235kbps

2 TRANSMITTERS AT 30kbps

VOUT-

MAX3241

SLEW RATE

vs. LOAD CAPACITANCE

22

20

SLEW RATE (V/µs)

18

16

14

10

8

6

4

ALL OUTPUTS LOADED WITH 3kΩ +CL

0.1µF CHARGE-PUMP CAPACITORS

FOR ALL DATA RATES UP TO 235kbps

|  |  | -SLEW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | +SLEW |  |  |  |  |  |  |  |

MAX3241

SUPPLY CURRENT vs. LOAD CAPACITANCE WHEN TRANSMITTING DATA

MAX3222-06

235kbps

MAX3222-05

40

35

30	120kbps

25

20

SUPPLY CURRENT (mA)

15

10

5

0

0	1000

2000	3000

4000	5000

0	1000

2000	3000

4000	5000

0	1000

2000	3000

4000	5000

7.5

5.0

2.5

0

-2.5

TRANSMITTER OUTPUT VOLTAGE (V)

-5.0

-7.5

LOAD CAPACITANCE (pF)

MAX3237

TRANSMITTER  OUTPUT VOLTAGE

MAX3222-07

vs. LOAD CAPACITANCE  (MBAUD  = GND)

1 TRANSMITTER AT 240kbps

4 TRANSMITTERS AT 15kbps

3kΩ + CL LOADS VCC = 3.3V

7.5

5.0

2.5

0

-2.5

TRANSMITTER OUTPUT VOLTAGE (V)

-5.0

-7.5

LOAD CAPACITANCE (pF)

MAX3237

TRANSMITTER  OUTPUT VOLTAGE

vs. LOAD CAPACITANCE (MBAUD = VCC)

MAX3222-08

1Mbps

2Mbps	1.5Mbps

1 TRANSMITTER AT FULL DATA RATE

4 TRANSMITTERS AT 1/16 DATA RATE

3kΩ + CL LOAD, EACH OUTPUT VCC = 3.3V

1Mbps

LOAD CAPACITANCE (pF)

MAX3237

SLEW RATE vs. LOAD CAPACITANCE (MBAUD  = GND)

MAX3222-09

12

10	-SLEW

8

6

SLEW RATE (V/µs)

4

4 TRANSMITTERS AT 15kbps

2	3kΩ + CL LOADS

0

0	1000

2000	3000

4000	5000

0	500	1000	1500	2000

0	1000

2000	3000

4000	5000

LOAD CAPACITANCE (pF)

MAX3237

SLEW RATE vs. LOAD CAPACITANCE (MBAUD = VCC)

LOAD CAPACITANCE (pF)

MAX3237

SUPPLY CURRENT vs.

MAX3222-11

LOAD CAPACITANCE  (MBAUD  = GND)

LOAD CAPACITANCE (pF)

MAX3237

SKEW vs. LOAD CAPACITANCE (tPLH - tPHL)

MAX3222-10

60	50

-SLEW, 1Mbps

SLEW RATE (V/µs)

50	+SLEW, 1Mbps

SUPPLY CURRENT (mA)

-SLEW, 2Mbps	40

40	+SLEW, 2Mbps

30

20

240kbps

120kbps

20kbps

SKEW (ns)

1 TRANSMITTER AT 512kbps

MAX3222-12

60	4 TRANSMITTERS AT 32kbps

3kΩ + CL LOADS

VCC = 3.3V

50	MBAUD = VCC

40		MAX AVERAGE; 10 PARTS

30

20

1 TRANSMITTER AT FULL DATA RATE

3kΩ + CL LOAD EACH OUTPUT

0	VCC = 3.3V

1 TRANSMITTER AT FULL DATA RATE	20

4 TRANSMITTERS AT 1/16 DATA RATE

VCC = 3.3V

0	0

MIN

0	500	1000	1500	2000

0	1000

2000	3000

4000	5000

0	500

1000	1500

2000	2500

MAX3222/MAX3232/MAX3237/MAX3241

LOAD CAPACITANCE (pF)

LOAD CAPACITANCE (pF)

LOAD CAPACITANCE (pF)

MAX3222/MAX3232/MAX3237/MAX3241

Pin  Description

| PIN |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- |
| MAX3222 |  | MAX3232 | MAX3237 | MAX3241 |  |  |
| DIP/SO | SSOP |  |  |  |  |  |
| 1 | 1 | — | 13 | 23 | EN | Receiver Enable. Active low. |
| 2 | 2 | 1 | 28 | 28 | C1+ | Positive Terminal of Voltage-Doubler Charge-Pump Capacitor |
| 3 | 3 | 2 | 27 | 27 | V+ | +5.5V Generated by the Charge Pump |
| 4 | 4 | 3 | 25 | 24 | C1- | Negative Terminal of Voltage-Doubler Charge-Pump Capacitor |
| 5 | 5 | 4 | 1 | 1 | C2+ | Positive Terminal of Inverting Charge-Pump Capacitor |
| 6 | 6 | 5 | 3 | 2 | C2- | Negative Terminal of Inverting Charge-Pump Capacitor |
| 7 | 7 | 6 | 4 | 3 | V- | -5.5V Generated by the Charge Pump |
| 8, 15 | 8, 17 | 7, 14 | 5, 6, 7, 10, 12 | 9, 10, 11 | T_OUT | RS-232 Transmitter Outputs |
| 9, 14 | 9, 16 | 8, 13 | 8, 9, 11 | 4–8 | R_IN | RS-232 Receiver Inputs |
| 10, 13 | 10, 15 | 9, 12 | 18, 20, 21 | 15–19 | R_OUT | TTL/CMOS Receiver Outputs |
| 11, 12 | 12, 13 | 10, 11 | 17, 19, 22, 23, 24 | 12, 13, 14 | T_IN | TTL/CMOS Transmitter Inputs |
| 16 | 18 | 15 | 2 | 25 | GND | Ground |
| 17 | 19 | 16 | 26 | 26 | VCC | +3.0V to +5.5V Supply Voltage |
| 18 | 20 | — | 14 | 22 | SHDN | Shutdown Control. Active low. |
| — | 11, 14 | — | — | — | N.C. | No Connection |
| — | — | — | 15 | — | MBAUD | MegaBaud Control Input. Connect to GND for normal operation; connect to VCC for 1Mbps transmission rates. |
| — | — | — | 16 | 20, 21 | R_OUTB | Noninverting Complementary Receiver Outputs. Always active. |

Detailed Description

Dual Charge-Pump Voltage Converter The MAX3222/MAX3232/MAX3237/MAX3241’s internal power supply consists of a regulated dual charge pump that provides output voltages of +5.5V (doubling charge pump) and -5.5V (inverting charge pump), regardless of the input voltage (VCC) over the 3.0V to 5.5V range. The charge pumps operate in a discontinuous mode; if the output voltages are less than 5.5V, the charge pumps are enabled, and if the output voltages exceed 5.5V, the charge  pumps  are disabled.  Each charge  pump requires a flying capacitor  (C1, C2) and a reservoir capacitor (C3, C4) to generate the V+ and V- supplies.

RS-232 Transmitters The transmitters are inverting level translators that con- vert CMOS-logic levels to 5.0V EIA/TIA-232 levels.

The MAX3222/MAX3232/MAX3241 transmitters guaran- tee a 120kbps data rate with worst-case loads of 3kΩ in parallel with 1000pF, providing  compatibility  with PC-to- PC communication  software (such as LapLink™). Typically, these three devices can operate at data rates of 235kbps. Transmitters can be paralleled to drive multi- ple receivers or mice.

The MAX3222/MAX3237/MAX3241’s output stage is turned off (high impedance)  when the device is in shut- down mode. When the power is off, the MAX3222/ MAX3232/MAX3237/MAX3241 permit the outputs to be driven up to ±12V.

The transmitter inputs do not have pullup  resistors. Connect unused inputs to GND or VCC.

MAX3237 MegaBaud Operation In normal operating  mode  (MBAUD =  GND), the MAX3237 transmitters guarantee a 250kbps data rate with worst-case loads of 3kΩ in parallel with 1000pF. This provides compatibility  with PC-to-PC communica- tion software, such as Laplink.

For higher speed serial communications, the MAX3237 features MegaBaud operation. In MegaBaud operating mode (MBAUD = VCC), the MAX3237 transmitters guar- antee a 1Mbps data rate with worst-case loads of 3kΩ in parallel with 250pF for 3.0V < VCC < 4.5V. For 5V ±10% operation, the MAX3237 transmitters guarantee a 1Mbps data rate into worst-case loads of 3kΩ in parallel with

1000pF.

LapLink is a trademark of Traveling Software, Inc.

VCC

0.1µF

VCC

0.1µF

C1+ C1

C1-

C2+ C2

C2-

T_ IN

VCC

MAX3222

MAX3232

MAX3237

MAX3241

V+

C3

V-

C4

T_ OUT

C1+ C1

C1-

C2+ C2

C2-

T_ IN

VCC

MAX3222

MAX3232

MAX3237

MAX3241

V+

C3

V-

C4

T_ OUT

0V VCC

R_ OUT EN*

SHDN*

GND

R_ IN

5kΩ

3kΩ 	2500pF

0V VCC

R_ OUT EN*

SHDN*

GND

R_ IN

5kΩ

7kΩ 	150pF

MAX3222/MAX3232/MAX3237/MAX3241

* MAX3222/MAX3241  ONLY

MINIMUM SLEW-RATE TEST CIRCUIT	MAXIMUM SLEW-RATE TEST CIRCUIT

Figure 1. Slew-Rate Test Circuits

VCC

Rx

PROTECTION DIODE

UART

Tx

GND

VCC

SHDN = GND

PREVIOUS RS-232

5kΩ

RS-232 Receivers The receivers convert RS-232 signals to CMOS-logic out- put levels. The MAX3222/MAX3237/MAX3241 receivers have inverting three-state outputs. In shutdown, the receivers can be active or inactive (Table 1).

The complementary outputs on the MAX3237 (R1OUTB) and the MAX3241 (R1OUTB, R2OUTB) are always active, regardless of the state of EN or SHDN. This allows for Ring Indicator applications  without forward biasing other devices connected  to the receiver outputs. This is ideal for systems where VCC is set to 0V in shutdown to accommodate peripherals, such as UARTs (Figure 2).

MAX3222/MAX3237/MAX3241

Shutdown Mode Supply current falls to less than 1µA in shutdown mode (SHDN = low). When shut down, the device’s  charge pumps are turned off, V+ is pulled down to VCC, V- is pulled  to ground,  and the transmitter outputs are dis- abled (high impedance). The time required to exit shut-

a) OLDER RS-232:  POWERED-DOWN UART DRAWS CURRENT FROM

ACTIVE RECEIVER OUTPUT IN SHUTDOWN.

VCC

down is typically 100µs, as shown in Figure 3.  Connect SHDN to VCC if the shutdown mode is not used.  SHDN has no effect on R_OUT or R_OUTB.

MAX3222/MAX3237/MAX3241

Enable Control

VCC

TO	LOGIC

µP     TRANSITION

DETECTOR

Rx

R1OUTB

R1OUT

MAX3237

MAX3241

R1IN

The inverting receiver outputs (R_OUT) are put into a high-impedance  state when EN is high. The complemen- tary outputs R1OUTB and R2OUTB are always active, regardless of the state of EN and SHDN (Table 1). EN has no effect on T_OUT.

Applications Information

Capacitor Selection The capacitor  type used for C1–C4 is not critical  for proper operation; polarized or nonpolarized capacitors

PROTECTION DIODE

UART

Tx

GND

EN = VCC

T1IN SHDN = GND

5kΩ

T1OUT

can be used. The charge pump requires 0.1µF capaci- tors for 3.3V operation. For other supply voltages, refer to Table 2 for required capacitor values. Do not use values lower than those listed in Table 2. Increasing the capaci- tor values (e.g., by a factor of 2) reduces ripple on the transmitter outputs and slightly reduces power consump- tion. C2, C3, and C4 can be increased without changing C1’s value. However, do not increase C1 without also increasing the values of C2, C3, and C4, to maintain the

MAX3222/MAX3232/MAX3237/MAX3241

b) NEW MAX3237/MAX3241: EN SHUTS DOWN RECEIVER OUTPUTS  (EXCEPT FOR

B OUTPUTS), SO NO CURRENT FLOWS TO UART IN SHUTDOWN. B OUTPUTS INDICATE  RECEIVER ACTIVITY  DURING SHUTDOWN  WITH EN HIGH.

Figure 2. Detection of RS-232 Activity when the UART and Interface are Shut Down; Comparison of MAX3237/MAX3241 (b) with Previous Transceivers (a).

proper ratios (C1 to the other capacitors).

When using the minimum required  capacitor  values, make sure the capacitor value does not degrade exces- sively with temperature. If in doubt, use capacitors with a higher nominal value. The capacitor’s  equivalent series resistance (ESR), which usually rises at low tempera- tures, influences the amount of ripple on V+ and V-.

Table 1. MAX3222/MAX3237/MAX3241

Shutdown and Enable Control Truth Table

| SHDN | EN | T_OUT | R_OUT | R_OUTB (MAX3237/ MAX3241) |
| --- | --- | --- | --- | --- |
| 0 | 0 | High-Z | Active | Active |
| 0 | 1 | High-Z | High-Z | Active |
| 1 | 0 | Active | Active | Active |
| 1 | 1 | Active | High-Z | Active |

| Table 2. Required Minimum Capacitor Values VCC (V) | C1 (µF) | C2, C3, C4 (µF) |
| --- | --- | --- |
| MAX3222/MAX3232/MAX3241 |  |  |
| 3.0 to 3.6 | 0.1 | 0.1 |
| 4.5 to 5.5 | 0.047 | 0.33 |
| 3.0 to 5.5 | 0.1 | 0.47 |
| MAX3237 |  |  |
| 3.0 to 3.6 | 0.22 | 0.22 |
| 3.15 to 3.6 | 0.1 | 0.1 |
| 4.5 to 5.5 | 0.047 | 0.33 |
| 3.0 to 5.5 | 0.22 | 1.0 |

Power-Supply Decoupling In most circumstances,  a 0.1µF bypass  capacitor  is adequate.  In applications  that are sensitive to power- supply noise, decouple  VCC to ground with a capacitor of the same value as charge-pump capacitor C1. Connect bypass capacitors as close to the IC as possible.

Operation Down to 2.7V

Transmitter outputs will meet EIA/TIA-562 levels of

±3.7V with supply voltages as low as 2.7V.

Transmitter Outputs when

Exiting Shutdown Figure 3 shows two transmitter outputs when exiting shutdown mode. As they become active, the two trans- mitter outputs are shown going to opposite RS-232 lev- els (one transmitter input is high, the other is low). Each transmitter is loaded  with 3kΩ in parallel with

2500pF. The transmitter outputs display no ringing or undesirable  transients as they come out of shutdown. Note that the transmitters are enabled  only when the magnitude of V- exceeds approximately 3V.

Mouse Driveability The MAX3241 has been specifically  designed  to power serial mice while operating from low-voltage power sup- plies. It has been tested with leading mouse brands from manufacturers such as Microsoft and Logitech.  The MAX3241 successfully  drove all serial mice tested and met their respective current and voltage requirements. Figure 4a shows the transmitter output voltages under increasing load current at 3.0V. Figure 4b shows a typical mouse connection using the MAX3241.

5V/div	T2

2V/div

T1

VCC = 3.3V C1–C4 = 0.1µF

50µs/div

MAX3222/MAX3232/MAX3237/MAX3241

Figure 3. Transmitter Outputs when Exiting Shutdown or

Powering Up

6

TRANSMITTER OUTPUT VOLTAGE (V)

5

4

3	VCC = 3.0V

2

1	T1

0

-1	T2

-2

VOUT+

MAX3222-04

VOUT+

-3	VCC     T3

-4

-5

-6

VOUT-

VOUT-

0	1	2	3	4	5	6	7

LOAD CURRENT PER TRANSMITTER (mA)

Figure 4a. MAX3241 Transmitter Output Voltage vs. Load Current per Transmitter

VCC = 3V

to 5.5V

0.1µF

26

C1

C2

VCC

28    C1+

24     C1-

1    C2+

2    C2-

14     T1IN

13    T2IN

12    T3IN

21     R1OUTB

20    R2OUTB

19    R1OUT

VCC

MAX3241

27

V+

C3

3

V-	C4

T1OUT   9

T2OUT   10

T3OUT   11

R1IN    4

COMPUTER SERIAL PORT

+V

+V

-V GND

Tx

18     R2OUT

17     R3OUT

5kΩ

5kΩ

R2IN    5

R3IN    6

16     R4OUT

15     R5OUT

5kΩ

5kΩ

R4IN    7

R5IN    8

MOUSE

23      EN

GND

5kΩ

SHDN    22

MAX3222/MAX3232/MAX3237/MAX3241

25

Figure 4b. Mouse Driver Test Circuit

VCC

0.1µF

T1IN

5V/div

C1+ C1

C1-

C2+ C2

C2-

T_ IN

VCC

MAX3222

MAX3232

MAX3237

MAX3241

V+

C3

V-

C4

T_ OUT

T1OUT R1OUT

VCC = 3.3V

5µs/div

5V/div

5V/div

Figure 6. MAX3241 Loopback Test Result at 120kbps

0V VCC

R_ OUT EN*

SHDN*

GND

R_ IN

5kΩ

1000pF

T1IN

T1OUT

5V/div

5V/div

* MAX3222/MAX3241  ONLY

Figure 5. Loopback Test Circuit

R1OUT

VCC = 3.3V

2µs/div

5V/div

High Data Rates

The MAX3222/MAX3232/MAX3241 maintain the RS-232

±5.0V minimum transmitter output voltage even at high data rates. Figure 5 shows a transmitter loopback  test circuit.  Figure  6  shows  a  loopback  test  result  at

Figure 7. MAX3241 Loopback Test Result at 235kbps

+5V

120kbps, and Figure 7 shows the same test at 235kbps. For Figure 6, all transmitters were driven simultaneously at 120kbps into RS-232 loads in parallel with 1000pF. For Figure 7, a single transmitter was driven at 235kbps, and all transmitters were loaded with an RS-232 receiver in parallel with 1000pF.

The MAX3237 maintains the RS-232 ±5.0V minimum transmitter output voltage at data rates up to 1Mbps. Figure 8 shows a loopback  test result at 1Mbps with MBAUD = VCC. For Figure 8, all transmitters were loaded with an RS-232 receiver in parallel with 250pF.

0V

+5V

0V

-5V

+5V

0V

VCC

= 3.3V

200ns/div

T_IN

T_OUT = R_IN

5kΩ + 250pF

R_OUT

150pF

MAX3222/MAX3232/MAX3237/MAX3241

Figure 8. MAX3237 Loopback Test Result at 1000kbps

(MBAUD = VCC)

| Interconnection with 3V and 5V Logic The MAX3222/MAX3232/MAX3237/MAX3241 can directly interface with various 5V logic families, includ- ing ACT and HCT CMOS. See Table 3 for more informa- tion on possible combinations of interconnections. SYSTEM POWER- SUPPLY VOLTAGE (V) | MAX32_ _ VCC SUPPLY VOLTAGE (V) | COMPATIBILITY |
| --- | --- | --- |
| 3.3 | 3.3 | Compatible with all CMOS families. |
| 5 | 5 | Compatible with all TTL and CMOS-logic families. |
| 5 | 3.3 | Compatible with ACT and HCT CMOS, and with TTL. Incompatible with AC, HC, and CD4000 CMOS. |

Table 3. Logic-Family Compatibility with Various Supply Voltages

Typical Operating Circuits

+3.3V

0.1µF

C1

0.1µF

C2

0.1µF

2   C1+

4   C1-

5    C2+

6   C2-

12   T1IN

17

VCC

MAX3222

3

V+

7

V-

T1OUT  15

C3*

0.1µF

C4

0.1µF

+3.3V

0.1µF

0.1µF

0.1µF

1   C1+

3   C1-

4    C2+

5   C2-

11   T1IN

16

VCC

MAX3232

2

V+

6

V-

T1OUT  14

C3*

0.1µF

C4

0.1µF

TTL/CMOS INPUTS

11   T2IN

T2OUT   8

RS-232

OUTPUTS

TTL/CMOS INPUTS

10   T2IN

T2OUT   7

RS-232

OUTPUTS

TTL/CMOS OUTPUTS

13    R1OUT

10    R2OUT

R1IN   14

5kΩ

R2IN   9

RS-232

INPUTS

TTL/CMOS OUTPUTS

12    R1OUT

9    R2OUT

R1IN   13

5kΩ

R2IN   8

RS-232

INPUTS

1     EN

GND

16

5kΩ

18

SHDN

GND

15

5kΩ

PIN NUMBERS  REFER TO DIP/SO PACKAGES.

* C3 CAN BE RETURNED TO EITHER VCC OR GROUND.

* C3 CAN BE RETURNED TO EITHER VCC OR GROUND.

MAX3222/MAX3232/MAX3237/MAX3241

SEE TABLE 2 FOR CAPACITOR SELECTION

Typical Operating Circuits (continued)

0.1µF

0.1µF

28   C1+

25   C1-

26

VCC

0.1µF

27

V+

+3.3V

0.1µF

0.1µF

28

24

C1+ C1-

26

VCC

27

V+

C3*

0.1µF

0.1µF

LOGIC INPUTS

LOGIC OUTPUTS

1    C2+

3   C2-

24    T1IN

23    T2IN

22    T3IN

19    T4IN

17    T5IN

16    R1OUTB

21    R1OUT

20    R2OUT

18    R3OUT

13    EN

MAX3237

T1

T2

T3

T4

T5

R1

R2

R3

GND

2

4

V-

T1OUT   5

T2OUT   6

T3OUT   7

T4OUT   10

T5OUT   12

R1IN   8

5kΩ

R2IN    9

5kΩ

R3IN   11

5kΩ

MBAUD    15

14

SHDN

0.1µF

RS-232

OUTPUTS

RS-232

INPUTS

0.1µF

TTL/CMOS INPUTS

TTL/CMOS OUTPUTS

1    C2+

2   C2-

14    T1IN

13    T2IN

12    T3IN

21    R1OUTB

20    R2OUTB

19    R1OUT

18    R2OUT

17    R3OUT

16    R4OUT

15    R5OUT

23    EN

MAX3241

GND

25

3

V-

T1OUT   9

T2OUT   10

T3OUT   11

R1IN   4

5kΩ

R2IN    5

5kΩ

R3IN   6

5kΩ

R4IN   7

5kΩ

R5IN   8

5kΩ

SHDN    22

C4

0.1µF

RS-232

OUTPUTS

RS-232

INPUTS

MAX3222/MAX3232/MAX3237/MAX3241

* C3 CAN BE RETURNED TO EITHER VCC OR GROUND.

* C3 CAN BE RETURNED TO EITHER VCC OR GROUND.

Pin  Configurations (continued)

TOP VIEW

+

EN   1

C1+   2

V+   3

C1-  4

C2+    5

C2-  6

V-   7

T2OUT   8

R2IN   9

MAX3222

20    SHDN

19    VCC

18    GND

17    T1OUT

16    R1IN

15    R1OUT

14    N.C.

13    T1IN

12    T2IN

| + C1+ | 1 |  |  | 16 | VCC |
| --- | --- | --- | --- | --- | --- |
| V+ | 2 |  |  | 15 | GND |
| C1- | 3 |  |  | 14 | T1OUT |
| C2+ | 4 | MAX3232 |  | 13 | R1IN |
| C2- | 5 |  |  | 12 | R1OUT |
| V- | 6 |  |  | 11 | T1IN |
| T2OUT | 7 |  |  | 10 | T2IN |

R2IN   8

9    R2OUT

R2OUT 10

11

SSOP/TSSOP

N.C.

DIP/SO/TSSOP

+

C2+    1

GND    2

C2-  3

V-   4

T1OUT   5

28   C1+

27   V+

26   VCC

25   C1-

24    T1IN

+

C2+    1

C2-  2

V-   3

R1IN    4

R2IN    5

28   C1+

27   V+

26   VCC

25   GND

24   C1-

MAX3237

MAX3241

T2OUT   6

T3OUT   7

R1IN    8

R2IN    9

T4OUT  10

R3IN  11

T5OUT  12

EN  13

SHDN  14

23   T2IN

22   T3IN

21   R1OUT

20    R2OUT

19   T4IN

18   R3OUT

17   T5IN

16   R1OUTB

15   MBAUD

R3IN    6

R4IN    7

R5IN    8

T1OUT   9

T2OUT  10

T3OUT  11

T3IN  12

T2IN  13

T1IN   14

23    EN

22   SHDN

21   R1OUTB

20   R2OUTB

19    R1OUT

18   R2OUT

17   R3OUT

16   R4OUT

15   R5OUT

MAX3222/MAX3232/MAX3237/MAX3241

SSOP

SO/SSOP/TSSOP

MAX3222/MAX3232/MAX3237/MAX3241

PART TEMP RANGE PIN-PACKAGE PKG CODE

MAX3232EWE+ -40°C to +85°C 16 Wide SO W16+1

MAX3232EPE+ -40°C to +85°C 16 Plastic DIP P16+1

MAX3232CAI+ 0°C to +70°C 28 SSOP A28+2

MAX3237EAI+ -40°C to +85°C 28 SSOP A28+2

MAX3237CUI+ 0°C to +70°C 28 TSSOP U28+1

MAX3241CAI+ 0°C to +70°C 28 SSOP A28+1

MAX3241CWI+ 0°C to +70°C 28 SO W28+1

MAX3241EUI+ -40°C to +85°C 28 TSSOP U28+1

MAX3241EAI+ -40°C to +85°C 28 SSOP A28+2

MAX3241EWI+ -40°C to +85°C 28 SO W28+1

*Dice are tested at TA = +25°C, DC parameters only. PART TEMP RANGE PIN-PACKAGE PKG CODE

MAX3222EUP+ -40°C to +85°C 20 TSSOP U20+2

MAX3222EAP+ -40°C to +85°C 20 SSOP A20+1

MAX3222EWN+ -40°C to +85°C 18 SO W18+1

MAX3222EPN+ -40°C to +85°C 18 Plastic Dip P18+5

MAX3222C/D 0°C to +70°C Dice* —

MAX3232CUE+ 0°C to +70°C 16 TSSOP U16+1

MAX3232CSE+ 0°C to +70°C 16 Narrow SO S16+1

MAX3232CWE+ 0°C to +70°C 16 Wide SO N16+1

MAX3232CPE+ 0°C to +70°C 16 Plastic DIP P16+1

MAX3232EUE+ -40°C to +85°C 16 TSSOP U16+1

MAX3232ESE+ -40°C to +85°C 16 Narrow SO S16+1

+Denotes  lead-free package.

3V-Powered EIA/TIA-232 and EIA/TIA-562 Transceivers from Maxim

| PART | POWER- SUPPLY VOLTAGE (V) | No. OF TRANSMITTERS/ RECEIVERS | No. OF RECEIVERS ACTIVE IN SHUTDOWN | GUAR- ANTEED DATA RATE (kbps) | EIA/ TIA-232 OR 562 | FEATURES |
| --- | --- | --- | --- | --- | --- | --- |
| MAX212 | 3.0 to 3.6 | 3/5 | 5 | 120 | 232 | Drives mice |
| MAX218 | 1.8 to 4.25 | 2/2 | 2 | 120 | 232 | Operates directly from batteries without a voltage regulator |
| MAX562 | 2.7 to 5.25 | 3/5 | 5 | 230 | 562 | Wide supply range |
| MAX563 | 3.0 to 3.6 | 2/2 | 2 | 230 | 562 | 0.1µF capacitors |
| MAX3212 | 2.7 to 3.6 | 3/5 | 5 | 235 | 232 | AutoShutdown, complementary receiver, drives mice, transient detection |
| MAX3222 | 3.0 to 5.5 | 2/2 | 2 | 120 | 232 | 0.1µF capacitors |
| MAX3223 | 3.0 to 5.5 | 2/2 | 2 | 120 | 232 | 0.1µF capacitors, AutoShutdown |
| MAX3232 | 3.0 to 5.5 | 2/2 | N/A | 120 | 232 | 0.1µF capacitors |
| MAX3237 | 3.0 to 5.5 | 5/3 | 3 | 250/1000 | 232 | 0.1µF capacitors, 1 complemen- tary receiver, MegaBaud operation |
| MAX3241 | 3.0 to 5.5 | 3/5 | 5 | 120 | 232 | 0.1µF capacitors, 2 complemen- tary receivers, drives mice |
| MAX3243 | 3.0 to 5.5 | 3/5 | 1 | 120 | 232 | 0.1µF capacitors, AutoShutdown, complementary receiver, drives mice |

Chip Topography

MAX3222

SHDN

Chip Information

V+     C1+

EN	V CC

GND

0.127"

C1- C2+

| T1OUT PART | TRANSISTOR COUNT |
| --- | --- |
| MAX3222 | 339 |
| MAX3232 | 339 |
| MAX3237 | 1212 |
| MAX3241 | 894 |

(3.225mm)

C2-

V-

R1IN

R1OUT

T2OUT   R2IN   R2OUT

T2IN  T1IN

MAX3222/MAX3232/MAX3237/MAX3241

0.087"

(2.209mm)

TRANSISTOR COUNT: 339

SUBSTRATE CONNECTED TO GND

TSSOP4.40mm.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

PACKAGE  OUTLINE, TSSOP 4.40mm BODY

1

21-0066	I	1

Revision History

Pages changed at Rev 7: 1, 15, 16, 17

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

17   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  2007 Maxim Integrated Products	is a registered trademark of Maxim Integrated Products, Inc.