19-1847; Rev 2; 12/06

1µA, 2Mbps, Low-Voltage Level

Translators in SC70 and µDFN

General Description

The MAX3370/MAX3371 logic-level translators are ideal for applications interfacing low-voltage devices to other logic  levels. Externally applied  voltages set the logic levels of the MAX3370/MAX3371. The devices  accept VCC from +2.5V to +5.5V and VL from +1.6V to +5.5V, allowing data transfer between low-voltage ASICs and higher voltage devices. The MAX3371 features a shut- down mode that reduces supply current to < 1µA and puts the I/O pins in a high-impedance  state.

The MAX3370/MAX3371 are bidirectional  level shifters, allowing data transfer from the VCC side to the VL side and from the VL side to the VCC side. Both devices operate at speeds  up to 2Mbps with an active driver and up to 500kbps with an open-drain driver.

The MAX3370/MAX3371 are available in space-saving

µDFN (1mm x 1.5mm) and SC70 packages.

Applications

Cell Phone Cradles

Cell Phone Hands-Free Kits

Portable POS Systems

Portable Communication Devices

Smart Card Readers

SPI™, MICROWIRE™, and I2C Level Translation

Low-Voltage ASIC Level Translation

RS-232-Compatible Translation

SPI is a trademark  of Motorola, Inc.

MICROWIRE is a trademark  of National Semiconductor Corp.

Features

♦ Allow Bidirectional Level Translation

♦ Miniature µDFN (1mm x 1.5mm) and SC70

Packages

♦ Operational Down to 1.6V

♦ Low Quiescent Current (< 100µA)

♦ Ultra-Low (< 1µA) Shutdown Supply Current

(MAX3371)

♦ Three-State Outputs in Shutdown (MAX3371)

♦ 2Mbps (10pF Load) Push-Pull Driving

♦ 1Mbps (50pF Load) Push-Pull Driving

♦ 500kbps (30pF Load) Open-Drain Driving

Ordering Information

PART* PIN- SHDN TOP PKG PACKAGE MARK CODE

MAX3370ELT-T 6 µDFN-6 NO KX L611-1

MAX3370EXK-T 5 SC70-5 NO ABV X5-1

MAX3371ELT-T 6 µDFN-6 YES KY L611-1

MAX3371EXK-T 6 SC70-6 YES AAO X6S-1

*All devices  are specified over the -40°C to +85°C operating temperature  range.

Pin  Configurations

TOP VIEW

Typical Operating Circuit

VL     1 	5    VCC

MAX3370

VL     1 	6    VCC

MAX3370

GND   2

GND   2

5    N.C.

+1.8V	+3.3V

I/O VL     3	4

I/O VCC

I/O VL     3	4

I/O VCC

1.8V SYSTEM CONTROLLER

VL

SHDN*

MAX3370

MAX3371

VCC

3.3V SYSTEM

SC70-5

VL     1 	6     VCC

µDFN-6 (1mm x 1.5mm)

VL     1 	6     VCC

GND

GND

GND

GND   2

MAX3371

5     SHDN

GND   2

MAX3371

5     SHDN

*MAX3371 ONLY

I/O VL     3	4

I/O VCC

I/O VL     3	4

I/O VCC

MAX3370/MAX3371†

†MAX3370/MAX3371 covered by U.S. Patent number 5,894,240.

SC70-6

µDFN-6 (1mm x 1.5mm)

ABSOLUTE MAXIMUM RATINGS

VCC to GND ..............................................................-0.3V to +7V SHDN to GND ...............................................-0.3V to (VL + 0.3V) I/O VL to GND................................................-0.3V to (VL + 0.3V) VL, I/O VCC to GND ...................................-0.3V to (VCC +  0.3V) Short-Circuit  Duration: I/O VL, I/O VCC to GND..........Continuous

Continuous Power Dissipation  (TA = +70°C)

SC70 (derate 3.1mW/°C above +70°C)  .......................245mW

6-Pin µDFN (derate 2.1mW/°C above +70°C) ..............168mW Operating  Temperature  Range ...........................-40°C to +85°C

Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX3370/MAX3371

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +2.5V to +5.5V, VL = +1.6V to +5.5V (Note 1), GND = 0; I/O VL, I/O VCC unconnected; TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| POWER SUPPLIES |  |  |  |  |
| VL Supply Range | VL | (Note 1) | 1.6 5.5 | V |
| VCC Supply Range | VCC |  | 2.5 5.5 | V |
| Supply Current from VCC | IQVCC |  | 70 100 | µA |
| Supply Current from VL | IQVL |  | 5 100 | µA |
| VCC Shutdown Supply Current |  | SHDN = GND, TA = +25°C, MAX3371 | 0.03 1 | µA |
| VL Shutdown Supply Current |  | SHDN = GND, TA = +25°C, MAX3371 | 0.03 1 | µA |
| Three-State Output Leakage Current |  | I/O VL, I/O VCC; SHDN = GND, TA = +25°C, MAX3371 | 0.02 1 | µA |
| LOGIC LEVEL THRESHOLDS |  |  |  |  |
| I/O VL Input-Voltage High Threshold | VIHL |  | VL - 0.2 | V |
| I/O VL Input-Voltage Low Threshold | VILL |  | 0.15 | V |
| I/O VCC Input-Voltage High Threshold | VIHC |  | VCC - 0.4 | V |
| I/O VCC Input-Voltage Low Threshold | VILC |  | 0.2 | V |
| I/O VL Output-Voltage High | VOHL | I/O VL sink current = 20µA, I/O VCC ≥ VCC - 0.4V (Note 3) | 2/3 × VL | V |
| I/O VL Output-Voltage Low | VOLL | I/O VL sink current = 1mA, I/O VCC ≤ 0.2V (Note 3) | 0.4 | V |
| I/O VCC Output-Voltage High | VOHC | I/O VCC source current = 20µA, I/O VL ≥ VL - 0.2V (Note 3) | 2/3 × VCC | V |
| I/O VCC Output-Voltage Low | VOLC | I/O VCC sink current = 1mA, I/O VL ≤ 0.15V (Note 3) | 0.4 | V |
| SHDN Input-Voltage High | VIH-SHDN |  | 2/3 × VL | V |
| SHDN Input-Voltage Low | VIL-SHDN | VL ≥ +1.8V | 0.4 | V |
|  |  | VL ≥ +1.6V | 0.2 |  |

MAX3370/MAX3371

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +2.5V to +5.5V, VL = +1.6V to +5.5V (Note 1), GND = 0; I/O VL, I/O VCC unconnected; TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 2)

LOAD

| LOAD PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| TIMING CHARACTERISTICS (RLOAD = 1MΩ, VCC = +2.5V, VL = +1.6V, I/O test signal rail-to-rail, unless otherwise noted, Figure 1) (Note 3) |  |  |  |  |
| I/O VCC Rise Time (Note 4) | tRVCC | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 100 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 200 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 400 |  |
| I/O VCC Fall Time (Note 4) | tFVCC | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 50 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 200 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 400 |  |
| I/O VL Rise Time (Note 4) | tRVL | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 100 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 200 |  |
|  |  | C = 30pF, data rate = 500kbps, open-drain drive | 400 |  |
| I/O VL Fall Time (Note 4) | tFVL | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 50 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 200 |  |
|  |  | C = 30pF, data rate = 500kbps, open-drain drive | 400 |  |
| HIGH-to-LOW Transition Propagation Delay (Driving I/O VL) | tPD-VCC-HL | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 50 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 200 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 400 |  |
| LOW-to-HIGH Transition Propagation Delay (Driving I/O VL) | tPD-VCC-LH | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 200 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 400 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 800 |  |
| HIGH-to-LOW Transition Propagation Delay (Driving I/O VCC) | tPD-VL-HL | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 50 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 200 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 400 |  |
| LOW-to-HIGH Transition Propagation Delay (Driving I/O VCC) | tPD-VL-LH | CLOAD = 10pF, data rate = 2Mbps (Note 5) | 200 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 400 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 800 |  |
| HIGH-to-LOW Transition Propagation Delay Device-to-Device Skew (Driving I/O VL) |  | CLOAD = 10pF, data rate = 2Mbps (Note 3) | 2 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 4 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 5 |  |

MAX3370/MAX3371

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +2.5V to +5.5V, VL = +1.6V to +5.5V (Note 1), GND = 0; I/O VL, I/O VCC unconnected; TA = -40°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| LOW-to-HIGH Transition Propagation Delay Device-to-Device Skew (Driving I/O VL) |  | CLOAD = 10pF, data rate = 2Mbps (Note 3) | 5 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 8 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 80 |  |
| HIGH-to-LOW Transition Propagation Delay Device-to-Device Skew (Driving I/O VCC) |  | CLOAD = 10pF, data rate = 2Mbps (Note 3) | 2 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 4 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 5 |  |
| LOW-to-HIGH Transition Propagation Delay Device-to-Device Skew (Driving I/O VCC) |  | CLOAD = 10pF, data rate = 2Mbps (Note 3) | 7 | ns |
|  |  | CLOAD = 50pF, data rate = 1Mbps | 8 |  |
|  |  | CLOAD = 30pF, data rate = 500kbps, open-drain drive | 50 |  |
| Maximum Data Rate |  | CLOAD = 10pF (Note 5) | 2 3 | Mbps |
|  |  | CLOAD = 50pF | 1 2 |  |
|  |  | CLOAD = 30pF, open-drain drive | 0.5 1 |  |

Note 1: VL must always be less than or equal to VCC.

Note 2: All units are 100% production tested at TA = +25°C. Limits over the operating  temperature  range are guaranteed by design

and not production tested.

Note 3: Tested only at worst case: VCC = +2.5V, VL = +1.6V.

Note 4: 10% to 90%.

Note 5: Guaranteed  by correlation  to CLOAD = 50pF.

+1.6V

VL 	VCC

+2.5V

+1.6V

VL 	VCC

+2.5V

MAX3370

MAX3371

MAX3370

MAX3371

I/O VL

I/O VCC

GND

DATA

RLOAD

CLOAD

CLOAD

DATA  I/O VL

RLOAD

I/O VCC

GND

I/O VL

tRISE, tFALL ≤ 10ns

tPD-VCC-LH 	tPD-VCC-HL

I/O VCC

tRISE, tFALL ≤ 10ns

tPD-VL-LH	tPD-VL-HL

I/O VCC

I/O VL

tRVCC 	tFVCC

tRVL 	tFVL

Figure 1a. Rail-to-Rail Driving I/O VL	Figure 1b. Rail-to-Rail Driving I/O VCC

+1.6V

VL 	VCC

+2.5V

+1.6V

VL 	VCC

+2.5V

MAX3370

MAX3371

MAX3370

MAX3371

I/O VL

I/O VCC

GND

DATA

RLOAD

CLOAD

CLOAD

DATA  I/O VL

RLOAD

I/O VCC

GND

I/O VL

tRISE, tFALL ≤ 10ns

tPD-VCC-LH 	tPD-VCC-HL

I/O VCC

tRISE, tFALL ≤ 10ns

tPD-VL-LH	tPD-VL-HL

I/O VCC

tRVCC 	tFVCC

I/O VL

tRVL 	tFVL

MAX3370/MAX3371

Figure 1c. Open-Drain Driving I/O VL	Figure 1d. Open-Drain Driving I/O VCC

Typical Operating Characteristics

(Driving I/O VL rail-to-rail, RL = 1MΩ, TA = +25°C, unless otherwise noted.)

102

100

VL SUPPLY CURRENT

vs. POWER-SUPPLY VOLTAGE

RATE = 500kbps,  CLOAD = 30pF, OPEN-DRAIN DRIVE

| 600 |  |  |  | VL = 1.8V |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RATE = 1Mbps, CLOAD = 50pF |  |  |  |  |  |  |
|  |  |  |  |  |  | 10pF |
|  |  | RATE = 2Mbps, CLOAD = |  |  |  |  |
|  | RATE = 500kbps, CLOAD = 30pF, OPEN-DRAIN DRIVE |  |  |  |  |  |

VCC SUPPLY CURRENT

MAX3370-02

vs. POWER-SUPPLY VOLTAGE

MAX3370-01

VL = 1.8V

98	RATE = 2Mbps,  CLOAD = 10pF

IVL (µA)

96

94

500

400

IVCC (µA)

300

92

90

88

RATE = 1Mbps,  CLOAD = 50pF

86

200

100

0

2.5

3.0

3.5

4.0	4.5	5.0	5.5

VCC (V)

2.5

3.0

3.5	4.0

MAX3370-04

VCC (V)

4.5	5.0	5.5

VL SUPPLY CURRENT

vs. TEMPERATURE

99

450

VCC SUPPLY CURRENT

vs. TEMPERATURE

MAX3370-03

97	VCC = 5.0V

IVL (µA)

95

94    VL = 1.8V, RATE = 500kbps,

IVCC (µA)

93    CLOAD = 30pF,

OPEN-DRAIN   DRIVE

92

91

90

89

VCC = 3.3V

VCC = 5.0V

300

250

200

100

VL = 1.8V, RATE = 500kbps, CLOAD = 30pF,

OPEN-DRAIN  DRIVE

VCC = 3.3V

MAX3370-06

VCC = 2.5V

-40

-15	10

35	60	85

-40

-15	10

35	60	85

TEMPERATURE (°C)

TEMPERATURE (°C)

MAX3370/MAX3371

115

110

105

IVL (µA)

100

95

90

85

80

VL SUPPLY CURRENT

MAX3370-05

vs. LOAD CAPACITANCE

| VL = 1.8V VCC = 3.3V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | pF, |
|  |  | RATE = 500kbps, CLOAD = 30 OPEN-DRAIN DRIVE RATE = 2Mbps, CLOAD = 10pF RATE = 1Mbps, CLOAD = 50pF |  |  |  |

0	50	100	150	200

CLOAD (pF)

780

680

IVCC (µA)

580

480

380

280

180

80

VCC SUPPLY CURRENT

vs. LOAD CAPACITANCE

VL = 1.8V VCC = 3.3V

RATE = 2Mbps,  CLOAD = 10pF RATE = 1Mbps,  CLOAD = 50pF

RATE = 500kbps,  CLOAD = 30pF, OPEN-DRAIN DRIVE

0	50	100	150	200

CLOAD (pF)

Typical Operating Characteristics (continued)

(Driving I/O VL rail-to-rail, RL = 1MΩ, TA = +25°C, unless otherwise noted.)

PROPAGATION DELAY

MAX3370-07

vs. LOAD CAPACITANCE (DRIVING I/O VL)

50

VL = 1.8V

45    VCC = 3.3V

PROPAGATION DELAY (ns)

40

LOW-TO-HIGH TRANSITION

35

30

25

20

15	HIGH-TO-LOW TRANSITION

10

5

0

0	50	100	150	200

CLOAD (pF)

PROPAGATION DELAY

MAX3370-08

| vs. LOAD CAPACITANCE (DRIVING I/O VCC) VL = 1.8V VCC = 3.3V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| HIGH-TO-LOW TRANSITION |  |  |  |  |  |
|  |  | LOW-TO-HIGH TRANSITION |  |  |  |

MAX3370-09

35

30

25

20

15

PROPAGATION DELAY (ns)

10

5

0

0	50	100	150	200

CLOAD (pF)

0

1V/div

0

RAIL-TO-RAIL DRIVING (RATE = 2Mbps,  CLOAD = 10pF, VCC = 3.3V, VL = 1.8V)

100ns/div

I/O VL

I/O VCC

MAX3370-10

RAIL-TO-RAIL DRIVING (RATE = 1Mbps,  CLOAD = 50pF, VCC = 3.3V, VL = 1.8V)

OPEN-DRAIN DRIVING

MAX3370-11

(RATE = 500kbps, CLOAD = 30pF, VCC = 3.3V, VL = 1.8V)

I/O VL

I/O VL

0

1V/div

0

1V/div

MAX3370-12

I/O VCC

I/O VCC

0	0

200ns/div

400ns/div

OPEN-DRAIN DRIVING

(RATE = 500kbps, CLOAD = 30pF, VCC = 5V, VL = 2.5V)

MAX3371

EXITING SHUTDOWN

0

2V/div

0

I/O VL

I/O VCC

0

2V/div

0

0

I/O VCC

MAX3370-13

I/O VL

VSHDN

MAX3370/MAX3371

400ns/div

100ns/div

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX3370 |  | MAX3371 (SC70-6 AND µDFN-6) |  |  |
| (SC70-5) | (µDFN-6) |  |  |  |
| 1 | 1 | 1 | VL | Logic Supply Voltage |
| 2 | 2 | 2 | GND | Ground |
| 3 | 3 | 3 | I/O VL | Input/Output Referred to VL |
| 4 | 4 | 4 | I/O VCC | Input/Output Referred to VCC |
| 5 | 6 | 6 | VCC | Power-Supply Voltage |
| — | — | 5 | SHDN | Shutdown. A high turns on the device. A low shuts down the device. I/O VCC and I/O VL are high impedance in shutdown. |
| — | 5 | — | N.C. | No Connection |

Detailed Description

The MAX3370/MAX3371 provide  the necessary level translation to allow data transfer in a multivoltage sys-

Functional Diagram

tem. These devices  transmit data between an I/O pin referenced to VCC and an I/O pin referenced to VL. The VCC supply voltage range is from +2.5V to +5.5V, and the VL supply  voltage range is between +1.6V and

+5.5V. The MAX3371 features a shutdown mode in which I/O VCC and I/O VL are placed in a high-imped- ance state and supply current drops to 1µA.

The MAX3370/MAX3371 are bidirectional  level shifters allowing data transfer from the VCC side to the VL side, and from the VL side to the VCC side. Both devices operate at speeds  up to 2Mbps with an active driver and up to 500kbps with an open-drain driver.

Level Translation

VL

S1

R1

I/O VL

MNSW SPEEDUP

MAX3370

S2

GND

VCC

R2

I/O VCC

MAX3370/MAX3371

The MAX3370/MAX3371 provide  bidirectional  level translation between I/O pins referred to VCC and VL. I/O VCC and I/O VL are internally pulled  up to their respective  power-supply  rails through 10kΩ resistors. VCC must be between +2.5V and +5.5V, and VL must be between +1.6V and +5.5V. For proper operation, VL can not exceed VCC.

The MAX3370/MAX3371 can operate at data rates up to 2Mbps when driven by an active (push-pull)  driver with a 10pF load, 1Mbps when driven by an active dri- ver with a 50pF load, or 500kbps when driven by an open-drain driver with a 30pF load. The internal pullups allow these devices to be driven by open-drain drivers.

MAX3371 Shutdown Mode The MAX3371 enters a low-power  shutdown mode when SHDN is driven low. Connect SHDN to VL or drive high for normal operation. Activating  shutdown mode disconnects  the internal 10kΩ pullup  resistors on I/O V CC and  I/O V L . As a result, the supply  current decreases to < 1µA, and the I/O lines are high imped- ance. The high impedance  I/O lines in shutdown allow use in a multidrop network. When in shutdown, I/O VL can be driven to VL and I/O VCC can be driven to VCC.

MAX3370/MAX3371

Speed-Up The speed-up circuit is a one-shot generator that helps the rise time of the output waveform in the low-to-high transition. When triggered,  switches S1 and S2 turn on for 320ns to pull up I/O VL and I/O VCC. This greatly reduces  the rise time and propagation  delay for the low-to-high transition as well as improves the duty cycle (closer to 50% for an ideal square-wave input). See the scope plots in the Typical Operating Characteristics for the speed-up circuitry in operation.

Applications Information

Power-Supply Decoupling To reduce ripple and the chance of transmitting incor- rect data, decouple VCC and VL to ground with a 0.1µF capacitor as close to the device as possible.

I2C Level Translation The MAX3370/MAX3371 are ideal for level translation between a low-voltage ASIC and an I2C device.  The devices’ bidirectional natures allow their use in the data line of I2C communications.  A typical  application  is interfacing a low-voltage microprocessor  to a 3V or 5V D/A converter, such as the MAX517.

The I/O lines on the MAX3370/MAX3371 are bidirec- tional, can be level-shifted up to +5.5V, and contain internal 10kΩ pullup  resistors to allow open-drain  dri- ving (see the Typical Operating Circuit).

Push-Pull vs.  Open-Drain Driving The MAX3370/MAX3371 I/O pins can be driven by a push-pull or open-drain device. When using a push-pull driver, the MAX3370/MAX3371 operate up to 2Mbps with a 10pF load or 1Mbps with a 50pF load. The inter- nal pullup resistors on the I/O pins allow use with open- drain devices.  The MAX3370/MAX3371 operate up to

500kbps  with a 30pF load when driven by an open- drain device.

Data rates higher than those listed in the Electrical Characteristics  table can be achieved.  The maximum data rate is limited to 3Mbps by the speed-up circuitry.

Unidirectional vs. Bidirectional Level Translator The MAX3370/MAX3371 may also be used to translate unidirectional  signals without signal inversion. The devices provide the smallest solution (SC70 package) for unidirectional level translation without inversion.

Chip Information

TRANSISTOR COUNT: 75

PROCESS: BiCMOS

MAX3370/MAX3371†

SC70, 6L.EPS

SC70, 5L.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |  |  |
| --- | --- | --- | --- | --- |

|  | PACKAGE OUTLINE, 5L SC70 |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 21-0076 | E | 1 1 |

PACKAGE OUTLINE, 6L SC70

1

21-0077 	E      1

6L UDFN.EPS

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

TABLE 1       Translation Table for Calendar Year Code

Calendar Year

2005      2006       2007      2008       2009      2010       2011      2012       2013      2014

Legend: 	Marked with bar 	Blank space - no bar required

TABLE 2        Translation Table for Payweek Binary Coding

Payweek

06-11

12-17      18-23      24-29      30-35      36-41      42-47      48-51     52-05

Legend: 	Marked with bar 	Blank space - no bar required

TITLE:

PACKAGE OUTLINE, 6L uDFN, 1.5x1.0x0.8mm

-DRAWING NOT TO SCALE-

Revision History

Pages changed at Rev 2: 1–11

APPROVAL

DOCUMENT CONTROL NO.

21-0147

REV.           2

D      2

MAX3370/MAX3371†

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600   	11

©  2006 Maxim Integrated  Products 	is a registered  trademark  of Maxim Integrated  Products, Inc.