19-2328; Rev 2; 11/07

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

General Description

The MAX3372E–MAX3379E and MAX3390E–MAX3393E

±15kV ESD-protected level translators provide the level shifting necessary to allow data transfer in a multivoltage system. Externally applied voltages, VCC and VL, set the logic levels on either side of the device. A low-voltage logic signal present on the VL side of the device appears as a high-voltage  logic signal on the VCC side of the device,  and vice-versa. The MAX3374E/MAX3375E/ MAX3376E/MAX3379E and MAX3390E–MAX3393E unidi- rectional level translators level shift data in one direction (VL → VCC or VCC → VL) on any single data line. The MAX3372E/MAX3373E and MAX3377E/MAX3378E bidi- rectional level translators utilize a transmission-gate- based design (Figure 2) to allow data translation in either direction  (VL ↔ VCC) on any single data line. The MAX3372E–MAX3379E and MAX3390E–MAX3393E accept VL from +1.2V to +5.5V and VCC from +1.65V to

+5.5V, making them ideal for data transfer between low- voltage ASICs/PLDs and higher voltage systems.

All devices in the MAX3372E–MAX3379E, MAX3390E– MAX3393E family feature a three-state output mode that reduces supply current to less than 1µA, thermal short- circuit protection, and ±15kV ESD protection on the VCC

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Features

♦ Guaranteed Data Rate Options

230kbps

8Mbps (+1.2V ≤ VL ≤ VCC ≤ +5.5V)

10Mbps (+1.2V ≤ VL ≤ VCC ≤ +3.3V)

16Mbps (+1.8V ≤ VL ≤ VCC ≤ +2.5V and +2.5V

≤ VL ≤ VCC ≤ +3.3V)

♦ Bidirectional Level Translation

(MAX3372E/MAX3373E and

MAX3377E/MAX3378E)

♦ Operation Down to +1.2V on VL

♦ ±15kV ESD Protection on I/O VCC Lines

♦ Ultra-Low 1µA Supply Current in Three-State

Output Mode

♦ Low-Quiescent Current (130µA typ)

♦ UCSP, TDFN, SOT23, and TSSOP Packages

♦ Thermal Short-Circuit Protection

Ordering Information

side for greater protection in applications  that route sig- nals externally. The MAX3372E/MAX3377E operate at a

PART                      TEMP

RANGE

PIN-

PACKAGE

PKG

CODE

guaranteed  data rate of 230kbps.  Slew-rate limiting reduces  EMI emissions in all 230kbps devices.  The MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E operate at a guaranteed data rate of 8Mbps over the entire specified  operating voltage range. Within specific voltage domains, higher data rates are possible. (See the Timing Characteristics table.)

The MAX3372E–MAX3376E are dual level shifters available  in 3 x 3 UCSP™, 8-pin TDFN, and 8-pin SOT23-8 packages.  The MAX3377E/MAX3378E/ MAX3379E and MAX3390E–MAX3393E are quad level shifters available in 3 x 4 UCSP, 14-pin TDFN, and 14- pin TSSOP packages.

Applications

SPI™, MICROWIRE™, and I2C Level

MAX3372EEKA+T      -40°C to +85°C        8 SOT23          K8S-3

+Denotes  a lead-free package. T = Tape and reel.

Ordering Information continued at end of data sheet. Selector Guide appears at end of data sheet.

UCSP is a trademark of Maxim Integrated  Products, Inc. SPI is a trademark of Motorola, Inc.

MICROWIRE is a trademark of National Semiconductor Corp.

Pin  Configurations

TOP VIEW

Translation

Low-Voltage ASIC Level Translation

Smart Card Readers Cell-Phone Cradles Portable POS Systems

Portable Communication Devices

Low-Cost Serial Interfaces

Cell Phones

GPS

MAX3377E/ MAX3378E

I/O VL1     1

I/O VL2     2

THREE-STATE  3

N.C.     4

I/O VL3     5

I/O VL4     6

GND    7

14

13

12

11

10

9

8

TDFN-14 (3mm x 3mm)

VCC

I/0 VCC1

I/0 VCC2

N.C. VL

I/0 VCC3

I/0 VCC4

Telecommunications Equipment

Pin Configurations continued at end of data sheet.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim Direct at 1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

ABSOLUTE MAXIMUM RATINGS

(All voltages referenced to GND.)

VCC ...........................................................................-0.3V to +6V I/O VCC_......................................................-0.3V to (VCC + 0.3V) I/O VL_ ...........................................................-0.3V to (VL + 0.3V) THREE-STATE ...............................................-0.3V to (VL + 0.3V) Short-Circuit  Duration I/O VL, I/O VCC to GND...........Continuous Short-Circuit  Duration I/O VL or I/O VCC to GND

Driven from 40mA Source

(except MAX3372E and MAX3377E) .....................Continuous

Continuous Power Dissipation  (TA = +70°C)

8-Pin SOT23 (derate 8.9mW/°C above +70°C)...........714mW

8-Pin TDFN (derate 18.2mW/°C above +70°C) ........1455mW

3 x 3 UCSP (derate 4.7mW/°C above +70°C) ............379mW

3 x 4 UCSP (derate 6.5mW/°C above +70°C) ............579mW

14-Pin TSSOP (derate 9.1mW/°C above +70°C) ........727mW

14-Pin TDFN (derate 18.2mW/°C above +70°C) ......1454mW Operating  Temperature  Range ...........................-40°C to +85°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +1.65V to +5.5V, VL = +1.2V to (VCC + 0.3V), GND = 0, I/O VL_ and I/O VCC_ unconnected, TA = TMIN to TMAX, unless other- wise noted. Typical values are at VCC = +3.3V, VL = +1.8V, TA = +25°C.)  (Notes 1, 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| POWER SUPPLIES |  |  |  |  |
| VL Supply Range | VL |  | 1.2 5.5 | V |
| VCC Supply Range | VCC |  | 1.65 5.50 | V |
| Supply Current from VCC | IQVCC |  | 130 300 | µA |
| Supply Current from VL | IQVL |  | 16 100 | µA |
| VCC Three-State Output Mode Supply Current | ITHREE-STATE-VCC | TA = +25°C, THREE-STATE = GND | 0.03 1 | µA |
| VL Three-State Output Mode Supply Current | ITHREE-STATE-VL | TA = +25°C, THREE-STATE = GND | 0.03 1 | µA |
| Three-State Output Mode Leakage Current I/O VL_ and I/O VCC_ | ITHREE-STATE-LKG | TA = +25°C, THREE-STATE = GND | 0.02 1 | µA |
| THREE-STATE Pin Input Leakage |  | TA = +25°C | 0.02 1 | µA |
| ESD PROTECTION |  |  |  |  |
| I/O VCC (Note 3) |  | IEC 1000-4-2 Air-Gap Discharge | ±8 | kV |
|  |  | IEC 1000-4-2 Contact Discharge | ±8 |  |
|  |  | Human Body Model | ±15 |  |
| LOGIC-LEVEL THRESHOLDS (MAX3372E/MAX3377E) |  |  |  |  |
| I/O VL_ Input-Voltage High | VIHL |  | VL - 0.2 | V |
| I/O VL_ Input-Voltage Low | VILL |  | 0.15 | V |

2

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +1.65V to +5.5V, VL = +1.2V to (VCC + 0.3V), GND = 0, I/O VL_ and I/O VCC_ unconnected, TA = TMIN to TMAX, unless other- wise noted. Typical values are at VCC = +3.3V, VL = +1.8V, TA = +25°C.)  (Notes 1, 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| I/O VCC_ Input-Voltage High | VIHC |  | VCC - 0.4 | V |
| I/O VCC_ Input-Voltage Low | VILC |  | 0.15 | V |
| I/O VL_ Output-Voltage High | VOHL | I/O VL_ source current = 20µA, I/O VCC_ > VCC - 0.4V | 0.67 ✕ VL | V |
| I/O VL_ Output-Voltage Low | VOLL | I/O VL_ sink current = 20µA, I/O VCC_ < 0.15V | 0.4 | V |
| I/O VCC_ Output-Voltage High | VOHC | I/O VCC_ source current = 20µA, I/O VL _ > VL - 0.2V | 0.67 ✕ VCC | V |
| I/O VCC_ Output-Voltage Low | VOLC | I/O VCC_ sink current = 20µA, I/O VL_ < 0.15V | 0.4 | V |
| THREE-STATE Input-Voltage High | VIL-THREE-STATE |  | VL - 0.2 | V |
| THREE-STATE Input-Voltage Low | VIL-THREE-STATE |  | 0.15 | V |
| LOGIC-LEVEL THRESHOLDS (MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E) |  |  |  |  |
| I/O VL_ Input-Voltage High | VIHL |  | VL - 0.2 | V |
| I/O VL_ Input-Voltage Low | VILL |  | 0.15 | V |
| I/O VCC_ Input-Voltage High | VIHC |  | VCC - 0.4 | V |
| I/O VCC_ Input-Voltage Low | VILC |  | 0.15 | V |
| I/O VL_ Output-Voltage High | VOHL | I/O VL_ source current = 20µA, I/O VCC_ ≥ VCC - 0.4V | 0.67 ✕ VL | V |
| I/O VL_ Output-Voltage Low | VOLL | I/O VL_ sink current = 1mA, I/O VCC_ ≤ 0.15V | 0.4 | V |
| I/O VCC_ Output-Voltage High | VOHC | I/O VCC_ source current = 20µA, I/O VL_ ≥ VL - 0.2V | 0.67 ✕ VCC | V |
| I/O VCC_ Output-Voltage Low | VOLC | I/O VCC_ sink current = 1mA, I/O VL_ ≤ 0.15V | 0.4 | V |
| THREE-STATE Input-Voltage High | VIH-THREE-STATE |  | VL - 0.2 | V |
| THREE-STATE Input-Voltage Low | VIL-THREE-STATE |  | 0.15 | V |

3

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

TIMING CHARACTERISTICS

(VCC = +1.65V to +5.5V, VL = +1.2V to (VCC + 0.3V), GND = 0, RLOAD = 1MΩ, I/O test signal of Figure 1, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +3.3V, VL = +1.8V, TA = +25°C, unless otherwise noted.)  (Notes 1, 2)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| MAX3372E/MAX3377E (CLOAD = 50pF) |  |  |  |  |  |
| I/O VCC_ Rise Time (Note 4) | tRVCC |  |  | 1100 | ns |
| I/O VCC_ Fall Time (Note 5) | tFVCC |  |  | 1000 | ns |
| I/O VL _ Rise Time (Note 4) | tRVL |  |  | 600 | ns |
| I/O VL _ Fall Time (Note 5) | tFVL |  |  | 1100 | ns |
| Propagation Delay | I/OVL-VCC | Driving I/O VL _ |  | 1.6 | µs |
|  | I/OVCC-VL | Driving I/O VCC_ |  | 1.6 |  |
| Channel-to-Channel Skew | tSKEW | Each translator equally loaded |  | 500 | ns |
| Maximum Data Rate |  | CL = 25pF |  | 230 | kbps |
| MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E (CLOAD = 15pF, Driver Output Impedance ≤ 50Ω) |  |  |  |  |  |
| +1.2V ≤ VL ≤ VCC ≤ +5.5V |  |  |  |  |  |
| I/O VCC_ Rise Time (Note 4) | tRVCC |  |  | 7 25 | ns |
|  |  | Open-drain driving |  | 170 400 |  |
| I/O VCC_ Fall Time (Note 5) | tFVCC |  |  | 6 37 | ns |
|  |  | Open-drain driving |  | 20 50 |  |
| I/O VL _ Rise Time (Note 4) | tRVL |  |  | 8 30 | ns |
|  |  | Open-drain driving |  | 180 400 |  |
| I/O VL _ Fall Time (Note 5) | tLFV |  |  | 3 30 | ns |
|  |  | Open-drain driving |  | 30 60 |  |
| Propagation Delay | I/OVL-VCC | Driving I/O VL _ |  | 5 30 | ns |
|  |  |  | Open-drain driving | 210 1000 |  |
|  | I/OVCC-VL | Driving I/O VCC_ |  | 4 30 |  |
|  |  |  | Open-drain driving | 190 1000 |  |
| Channel-to-Channel Skew | tSKEW | Each translator equally loaded |  | 20 | ns |
|  |  |  | Open-drain driving | 50 |  |
| Maximum Data Rate |  |  |  | 8 | Mbps |
|  |  | Open-drain driving |  | 500 | kbps |

4

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

TIMING CHARACTERISTICS (continued)

(VCC = +1.65V to +5.5V, VL = +1.2V to (VCC + 0.3V), GND = 0, RLOAD = 1MΩ, I/O test signal of Figure 1, TA = TMIN to TMAX, unless otherwise noted. Typical values are at VCC = +3.3V, VL = +1.8V, TA = +25°C, unless otherwise noted.)  (Notes 1, 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| +1.2V ≤ VL ≤ VCC ≤ +3.3V |  |  |  |  |
| I/O VCC_ Rise Time (Note 4) | tRVCC |  | 25 | ns |
| I/O VCC_ Fall Time (Note 5) | tFVCC |  | 30 | ns |
| I/O VL _ Rise Time (Note 4) | tRVL |  | 30 | ns |
| I/O VL _ Fall Time (Note 5) | tFVL |  | 30 | ns |
| Propagation Delay | I/OVL-VCC | Driving I/O VL _ | 20 | ns |
|  | I/OVCC-VL | Driving I/O VCC_ | 20 |  |
| Channel-to-Channel Skew | tSKEW | Each translator equally loaded | 10 | ns |
| Maximum Data Rate |  |  | 10 | Mbps |
| +2.5V ≤ VL ≤ VCC ≤ +3.3V |  |  |  |  |
| I/O VCC_ Rise Time (Note 4) | tRVCC |  | 15 | ns |
| I/O VCC_ Fall Time (Note 5) | tFVCC |  | 15 | ns |
| I/O VL _ Rise Time (Note 4) | tRVL |  | 15 | ns |
| I/O VL _ Fall Time (Note 5) | tFVL |  | 15 | ns |
| Propagation Delay | I/OVL-VCC | Driving I/O VL _ | 15 | ns |
|  | I/OVCC-VL | Driving I/O VCC_ | 15 |  |
| Channel-to-Channel Skew | tSKEW | Each translator equally loaded | 10 | ns |
| Maximum Data Rate |  |  | 16 | Mbps |
| +1.8V ≤ VL ≤ VCC ≤ +2.5V |  |  |  |  |
| I/O VCC_ Rise Time (Note 4) | tRVCC |  | 15 | ns |
| I/O VCC_ Fall Time (Note 5) | tFVCC |  | 15 | ns |
| I/O VL _ Rise Time (Note 4) | tRVL |  | 15 | ns |
| I/O VL _ Fall Time (Note 5) | tFVL |  | 15 | ns |
| Propagation Delay | I/OVL-VCC | Driving I/O VL _ | 15 | ns |
|  | I/OVCC-VL | Driving I/O VCC_ | 15 |  |
| Channel-to-Channel Skew | tSKEW | Each translator equally loaded | 10 | ns |
| Maximum Data Rate |  |  | 16 | Mbps |

Note 1: All units are 100% production tested at TA  = +25°C. Limits over the operating temperature range are guaranteed by design and not production tested.

Note 2: For normal operation, ensure VL < (VCC + 0.3V). During power-up, VL > (VCC + 0.3V) will not damage the device. Note 3: To ensure maximum ESD protection, place a 1µF capacitor between VCC and GND. See Applications Circuits. Note 4: 10% to 90%

Note 5: 90% to 10%

5

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Typical Operating Characteristics

(RLOAD = 1MΩ, TA = +25°C, unless otherwise noted. All 230kbps  TOCs apply to MAX3372E/MAX3377E only. All 8Mbps and

500kbps  TOCs apply to MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E only.)

VL SUPPLY CURRENT vs. SUPPLY VOLTAGE (DRIVING I/O VL, VCC = +3.3V, VL = +1.8V)

600

500

8Mbps, CLOAD = 15pF

400

MAX3372E toc02

VCC SUPPLY CURRENT vs. SUPPLY VOLTAGE (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

MAX3372E toc01

3.5

3.0

500kbps, OPEN-DRAIN, CLOAD = 15pF

2.5

VL SUPPLY CURRENT vs. TEMPERATURE (DRIVING I/O VCC, VCC = +3.3V, VL = +1.8V)

MAX3372E toc03

400

350

300

8Mbps, CLOAD = 15pF

300

SUPPLY CURRENT (μA)

200

100

500kbps, OPEN-DRAIN, CLOAD = 15pF

230kbps, CLOAD = 50pF

2.0

1.5

SUPPLY CURRENT (mA)

1.0

0.5

8Mbps, CLOAD = 15pF

230kbps, CLOAD = 50pF

250

SUPPLY CURRENT (μA)

200

150

100

50

500kbps, OPEN-DRAIN, CLOAD = 15pF

230kbps, CLOAD = 50pF

0

1.65

2.20

2.75

3.30

3.85

4.40

4.95

5.50

0

1.65

2.20

2.75

3.30

3.85

4.40

4.95

MAX3372E toc05

5.50

0

-40

-15         10         35

MAX3372E toc06

60         85

VCC (V)

VCC (V)

TEMPERATURE (°C)

MAX3372E toc04

VCC SUPPLY CURRENT vs. TEMPERATURE (DRIVING I/O VCC, VCC = +3.3V, VL = +1.8V)

1600

1400

VL SUPPLY CURRENT vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

350

300

VCC SUPPLY CURRENT vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

2500

SUPPLY CURRENT (μA)

1200

1000

800

600

400

8Mbps, CLOAD = 15pF

500kbps, OPEN-DRAIN, CLOAD = 15pF

250

200

150

SUPPLY CURRENT (μA)

100

8Mbps

SUPPLY CURRENT (μA)

500kbps, OPEN-DRAIN

2000

1500

1000

500kbps, OPEN-DRAIN

8Mbps

230kbps

200

230kbps, CLOAD = 50pF

50

230kbps

500

0

-40

-15         10         35         60         85

0

10       25       40

55       70

85      100

0

10       25

40       55

70       85

100

2500

2000

TEMPERATURE (°C)

MAX3372E toc07

RISE/FALL TIME vs. CAPACITIVE  LOAD (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

CAPACITIVE LOAD (pF)

MAX3372E toc08

| RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +3.3V, VL = +1.8V) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tLH |  |  |  |  |  |  |  |  |
|  |  |  |  |  | tHL |  |  |  |
|  | DATA RATE = 8Mbps |  |  |  |  |  |  |  |

18

16

14

250

200

CAPACITIVE LOAD (pF)

RISE/FALL TIME vs. CAPACITIVE  LOAD (DRIVING I/O VL, VCC = +3.3V, VL = +1.8V)

tLH

1500

RISE/FALL TIME (ns)

1000

500

0

tLH

tHL

RISE/FALL TIME (ns)

12

10

8

6

DATA RATE = 230kbps

4

2

0

150

RISE/FALL TIME (ns)

100

50

0

MAX3372E toc09

DATA RATE = 500kbps, OPEN-DRAIN

tHL

20    30

40    50

60    70

80    90

100

10    15

20    25

30    35

40    45    50

10    15    20

25    30

35    40

45    50

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

6

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Typical Operating Characteristics (continued)

MAX3372E toc12

(RLOAD = 1MΩ, TA = +25°C, unless otherwise noted. All 230kbps  TOCs apply to MAX3372E/MAX3377E only. All 8Mbps and

500kbps  TOCs apply to MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E only.)

PROPAGATION DELAY vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

700

MAX3372E toc10

MAX3372E toc11

PROPAGATION DELAY vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

15

PROPAGATION DELAY vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +3.3V,  VL = +1.8V)

| 300 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DATA RATE = 500kbps, |  |  |  |  |
|  | tPHL |  |  |  |  |  |  |

600

500

400

300

PROPAGATION DELAY (ns)

200

tPHL

tPLH

tPHL

9

6

250

200

150

PROPAGATION DELAY (ns)

100

100

0

PROPAGATION DELAY (ns)

DATA RATE = 230kbps                                     3

0

50 tPLH

0

20    30

40    50

60    70

80    90

100

10    15

20    25

30    35    40

45    50

10    15    20

25    30    35

40    45    50

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

2500

2000

MAX3372E toc13

| RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +2.5V, VL = +1.8V) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | tLH |  |  |  |  |  |  |  |  |
|  |  |  |  | DATA RATE = 230kbps |  |  |  |  |  |
|  |  | tHL |  |  |  |  |  |  |  |

MAX3372E toc14

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VL, VCC = +2.5V, VL = +1.8V)

14

DATA RATE = 8Mbps

12

RISE/FALL TIME (ns)

10            tLH

MAX3372E toc15

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +2.5V,  VL = +1.8V)

300

250

RISE/FALL TIME (ns)

200

1500

RISE/FALL TIME (ns)

1000

500

8

6

tHL

4

2

150

100

| 50 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tLH |  |  |  |  |  |  |  |
|  |  |  | DATA RATE = 500kbps, OPEN-DRAIN |  |  |  |  |
|  | tHL |  |  |  |  |  |  |

0

20    30

40    50

60    70

80    90

100

0

10    15

20    25

30    35

40    45    50

0

10    15    20

25    30

35    40

MAX3372E toc18

45    50

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +3.3V,  VL = +1.8V)

2500

DATA RATE = 230kbps

MAX3372E toc16

MAX3372E toc17

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +3.3V,  VL = +1.8V)

12

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +3.3V,  VL = +1.8V)

| 300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATA RATE = 8Mbps |  |  |  |  |  |  |  |  |
| tLH |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | tHL |  |  |  |  |

2000

10                                                                                       250

tLH

1500

RISE/FALL TIME (ns)

1000

500

0

tHL

8

6

RISE/FALL TIME (ns)

4

2 tLH

0

200

150

RISE/FALL TIME (ns)

100

50

0

tHL

DATA RATE = 500kbps, OPEN-DRAIN

20    30

40    50

60    70

80    90

100

10    15

20    25

30    35

40    45    50

10    15    20

25    30

35    40

45    50

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

7

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Typical Operating Characteristics (continued)

(RLOAD = 1MΩ, TA = +25°C, unless otherwise noted. All 230kbps  TOCs apply to MAX3372E/MAX3377E only. All 8Mbps and

500kbps  TOCs apply to MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E only.)

MAX3372E toc19

PROPAGATION DELAY vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +3.3V, VL = +1.8V)

700

PROPAGATION DELAY vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +3.3V,  VL = +1.8V)

MAX3372E toc20

| 6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATA RATE = 8Mbps |  |  |  |  |  |  |  |  |
| tPHL |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | tPLH |  |  |  |  |

PROPAGATION DELAY vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +3.3V,  VL = +1.8V)

MAX3372E toc21

300

600

500

400

300

PROPAGATION DELAY (ns)

200

100

0

5                                                                                        250

4                                                                                        200

PROPAGATION DELAY (ns)

3                                                                                        150

PROPAGATION DELAY (ns)

2                                                                                        100

1                                                                                         50

0                                                                                           0

tPLH

tPHL

DATA RATE = 500kbps, OPEN-DRAIN

| 20 30 |  |  |  |  | s |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATA RATE = 230kbp |  |  |  |  |  |  |  |
|  |  | tPHL |  |  |  |  |  |  |
|  |  |  |  |  |  | tPHL |  |  |

40    50    60

70    80

MAX3372E toc22

90   100

10    15    20    25

30    35    40

MAX3372E toc23

45    50

10    15

20    25    30

35    40    45    50

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +2.5V,  VL = +1.8V)

2500

DATA RATE = 230kbps

2000

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +2.5V,  VL = +1.8V)

12

DATA RATE = 8Mbps

10

8

RISE/FALL TIME vs. CAPACITIVE LOAD (DRIVING I/O VCC, VCC = +2.5V,  VL = +1.8V)

MAX3373E toc24

350

250

1500

RISE/FALL TIME (ns)

1000

500

0

tHL

tLH

tLH

6

RISE/FALL TIME (ns)

4

2

0

tHL

200

150

RISE/FALl TIME (ns)

100

0

tLH

DATA RATE = 500kbps,

20    30

40    50

60    70

80    90

100

10             20

30             40             50

10             20             30

40             50

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

CAPACITIVE LOAD (pF)

RAIL-TO-RAIL  DRIVING

MAX3372E toc25

(DRIVING I/O VL, VCC = +3.3V, VL = +1.8V, CLOAD = 50pF,  DATA RATE = 230kbps)

RAIL-TO-RAIL  DRIVING

MAX3372E toc26

(DRIVING I/O VL, VCC = +3.3V, VL = +1.8V, CLOAD = 15pF,  DATA RATE = 8Mbps)

I/O VL_

I/O VCC_

1V/div

2V/div

I/O VL_

I/O VCC_

1V/div

2V/div

1μs/div

200ns/div

8

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Typical Operating Characteristics (continued)

(RLOAD = 1MΩ, TA = +25°C, unless otherwise noted. All 230kbps  TOCs apply to MAX3372E/MAX3377E only. All 8Mbps and

500kbps  TOCs apply to MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E only.)

OPEN-DRAIN DRIVING

MAX3372E toc27

(DRIVING I/O VL, VCC = +3.3V, VL = +1.8V, CLOAD = 15pF,  DATA RATE = 500kbps)

EXITING THREE-STATE OUTPUT MODE (VCC = +3.3V,  VL = +1.8V,  CLOAD = 50pF)

MAX3372E toc28

I/O VL_

I/O VCC_

1V/div

2V/div

I/O VCC_

I/O VL_

THREE-STATE

2V/div

1V/div

1V/div

200ns/div

2μs/div

Pin  Description

| PIN |  |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 x 4 UCSP | TSSOP | SOT23-8 | 3 x 3 UCSP | TDFN 8 | TDFN 14 |  |  |
| A1 | 2 | 5 | C2 | 6 | 1 | I/O VL1 | Input/Output 1. Referenced to VL. (Note 6) |
| A2 | 3 | 4 | C3 | 8 | 2 | I/O VL2 | Input/Output 2. Referenced to VL. (Note 6) |
| A3 | 4 | — | — | — | 5 | I/O VL3 | Input/Output 3. Referenced to VL. (Note 6) |
| A4 | 5 | — | — | — | 6 | I/O VL4 | Input/Output 4. Referenced to VL. (Note 6) |
| B1 | 14 | 7 | A1 | 4 | 14 | VCC | VCC Input Voltage +1.65V ≤ VCC ≤ +5.5V. |
| B2 | 1 | 3 | C1 | 7 | 10 | VL | Logic Input Voltage +1.2V ≤ VL ≤ (VCC + 0.3V) |
| B3 | 8 | 6 | B1 | 5 | 3 | THREE- STATE | Three-State Output Mode Enable. Pull THREE-STATE low to place device in three-state output mode. I/O VCC_ and I/O VL_ are high impedance in three-state output mode. Note: Logic referenced to VL (for logic thresholds see the Electrical Characteristics table). |
| B4 | 7 | 2 | B3 | 2 | 7 | GND | Ground |
| C1 | 13 | 8 | A2 | 3 | 13 | I/O VCC1 | Input/Output 1. Referenced to VCC. (Note 6) |
| C2 | 12 | 1 | A3 | 1 | 12 | I/O VCC2 | Input/Output 2. Referenced to VCC. (Note 6) |
| C3 | 11 | — | — | — | 9 | I/O VCC3 | Input/Output 3. Referenced to VCC. (Note 6) |
| C4 | 10 | — | — | — | 8 | I/O VCC4 | Input/Output 4. Referenced to VCC. (Note 6) |
| — | 6, 9 | — | B2 | — | 4, 11 | N.C. | No Connection. Not internally connected. |
| — | — | — | — | EP | EP | EP | Exposed Pad. Connect to ground. |

Note 6: For unidirectional devices (MAX3374E/MAX3375E/MAX3376E/MAX3379E and MAX3390E–MAX3393E) see the Pin

Configurations for input/output configurations.

9

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Detailed Description

The MAX3372E–MAX3379E and MAX3390E–MAX3393E ESD-protected level translators provide the level shifting necessary to allow data transfer in a multivoltage system. Externally applied voltages, VCC and VL, set the logic lev- els on either side of the device. A low-voltage logic signal present on the VL side of the device appears as a high- voltage logic signal on the VCC side of the device, and vice-versa. The MAX3374E/MAX3375E/MAX3376E/ MAX3379E and MAX3390E–MAX3393E unidirectional level translators level shift data in one direction  (VL → VCC  or  V CC  →  VL) on  any  single  data  line.  The MAX3372E/MAX3373E and MAX3377E/MAX3378E bidi- rectional level translators utilize a transmission-gate- based design (see Figure 2) to allow data translation in either direction (VL ↔ VCC) on any single data line. The MAX3372E–MAX3379E and MAX3390E–MAX3393E

accept VL from +1.2V to +5.5V and VCC from +1.65V to

+5.5V, making them ideal for data transfer between low-

voltage ASICs/PLDs and higher voltage systems.

All devices in the MAX3372E–MAX3379E, MAX3390E– MAX3393E family feature a three-state output mode that reduces supply current to less than 1µA, thermal short- circuit protection, and ±15kV ESD protection on the VCC side for greater protection in applications  that route sig- nals externally. The MAX3372E/MAX3377E operate at a guaranteed  data rate of 230kbps.  Slew-rate limiting reduces  EMI emissions in all 230kbps devices.  The MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E operate at a guaranteed data rate of 8Mbps over the entire specified  operating voltage range. Within specific voltage domains, higher data rates are possible. (See the Timing Characteristics table.)

VL                                                                                         VCC

VL                                                                                         VCC

VL         VCC

VL         VCC

MAX3372E–MAX3379E AND MAX3390E–MAX3393E

DATA

DATA

MAX3372E–MAX3379E AND MAX3390E–MAX3393E

I/O VL_

I/O VCC_

I/O VL_

I/O VCC_

GND

RLOAD                 CLOAD

CLOAD

RLOAD

GND

I/O VL_ (tRISE,

tFALL < 10ns)

tPD-VCC-LH                                                      tPD-VCC-HL

I/O VCC_ (tRISE,

tFALL < 10ns)

tPD-VL-LH                                                         tPD-VL-HL

I/O VCC_

tRVCC                                                                       tFVCC

I/O VL_

tRVL                                                                        tFVL

Figure 1a. Rail-to-Rail Driving I/O VL

Figure 1b. Rail-to-Rail Driving I/O VCC

10

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

Level Translation

For proper  operation  ensure that +1.65V ≤ VCC ≤

+5.5V, +1.2V ≤ VL ≤ +5.5V, and VL ≤ (VCC + 0.3V).

During power-up  sequencing,  VL ≥ (VCC + 0.3V) will

not damage the device. During power-supply sequenc-

ing, when VCC is floating and VL is powering up, a cur- rent may be sourced,  yet the device will not latch up. The speed-up circuitry limits the maximum data rate for devices  in the MAX3372E–MAX3379E, MAX3390E– MAX3393E family to 16Mbps. The maximum data rate also depends heavily on the load capacitance  (see the Typical Operating  Characteristics),  output impedance of the driver, and the operational voltage range (see the Timing Characteristics table).

Speed-Up Circuitry The MAX3373E–MAX3376E/MAX3378E/MAX3379E and MAX3390E–MAX3393E feature a one-shot generator that decreases the rise time of the output. When triggered, MOSFETs PU1 and PU2 turn on for a short time to pull up

MAX3372E–MAX3379E/MAX3390E–MAX3393E

I/O VL_ and I/O VCC_ to their respective supplies (see Figure 2b). This greatly reduces the rise time and propa- gation delay for the low-to-high transition. The scope photo of Rail-to-Rail Driving for 8Mbps Operation in the Typical Operating Characteristics shows the speed-up circuitry in operation.

Rise-Time Accelerators The MAX3373E–MAX3376E/MAX3378E/MAX3379E and the MAX3390E–MAX3393E have internal rise-time accelerators  allowing operation up to 16Mbps.  The rise-time accelerators are present on both sides of the device and act to speed  up the rise time of the input and output of the device, regardless of the direction of the data. The triggering mechanism for these accelera- tors is both level and edge sensitive. To prevent false triggering of the rise-time accelerators, signal fall times of less than 20ns/V are recommended  for both the inputs and outputs of the device. Under less noisy con- ditions, longer signal fall times may be acceptable.

VL                                                                                         VCC

VL                                                                                         VCC

VL         VCC

VL         VCC

MAX3372E–MAX3379E AND MAX3390E–MAX3393E

DATA

DATA

MAX3373E–MAX3376E, MAX3378E/MAX3379E

AND MAX3390E–MAX3393E

I/O VL_                           I/O VCC_

GND

RLOAD

CLOAD

CLOAD

RLOAD

I/O VL_

GND

I/O VCC_

I/O VL_

I/O VCC_

tPD-VCC-LH

tPD-VCC-HL

tPD-VL-LH

tPD-VL-HL

I/O VCC_

I/O VL_

tRVCC                                                                                      tFVCC

tRVL                                                                                         tFVL

Figure 1c. Open-Drain Driving I/O VCC

Figure 1d. Open-Drain Driving I/O VL

11

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Three-State Output Mode

Pull THREE-STATE  low to place  the MAX3372E– MAX3379E and MAX3390E–MAX3393E in three-state out- put mode. Connect THREE-STATE to VL (logic-high)  for normal operation. Activating the three-state output mode disconnects  the internal 10kΩ pullup resistors on the I/O VCC and I/O VL lines. This forces the I/O lines to a high- impedance  state, and decreases the supply current to less than 1µA. The high-impedance  I/O lines in three- state output mode allow for use in a multidrop network. When in three-state output mode, do not allow the voltage

VL

at I/O VL_ to exceed (VL + 0.3V), or the voltage at I/O VCC_ to exceed (VCC + 0.3V).

Thermal Short-Circuit Protection Thermal overload detection protects  the MAX3372E– MAX3379E and MAX3390E–MAX3393E from short-circuit fault conditions. In the event of a short-circuit fault, when the junction temperature (TJ) reaches +152°C, a thermal sensor signals the three-state output mode logic to force the device into three-state output mode. When TJ has cooled to +142°C, normal operation resumes.

VCC

P                                                                                                                                     P

GATE BIAS

I/O VL                                                                                                                                                                                          I/O VCC

N

Figure 2a. Functional Diagram, MAX3372E/MAX3377E (1 I/O line)

VL                                                                                                                                                                                            VCC

PU1

ONE-SHOT BLOCK

ONE-SHOT BLOCK

PU2

GATE BIAS

I/O VL_                                                                                                                                                                                                                            I/O VCC_

N

Figure 2b. Functional Diagram, MAX3373E/MAX3378E (1 I/O line)

12

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

±15kV ESD Protection As with all Maxim devices, ESD-protection structures are incorporated  on all pins to protect against electrostatic discharges encountered during handling and assembly. The I/O VCC lines have extra protection against static electricity. Maxim’s engineers have developed state-of- the-art structures to protect these pins against ESD of

±15kV without damage. The ESD structures withstand high ESD in all states: normal operation, three-state output mode, and powered down. After an ESD event, Maxim’s E versions keep working without latchup, whereas competing  products  can latch and must be powered down to remove latchup.

ESD protection can be tested in various ways. The I/O VCC lines of this product  family are characterized  for protection to the following limits:

1) ±15kV using the Human Body Model

2) ±8kV using the Contact Discharge method specified in IEC 1000-4-2

3) ±10kV using IEC 1000-4-2’s Air-Gap Discharge method

ESD Test Conditions ESD performance  depends  on a variety of conditions. Contact Maxim for a reliability report that documents test setup, test methodology, and test results.

Human Body Model Figure 3a shows the Human Body Model and Figure 3b shows the current waveform it generates when dis- charged into a low impedance.  This model consists of a 100pF capacitor charged to the ESD voltage of inter- est, which is then discharged  into the test device through a 1.5kΩ resistor.

MAX3372E–MAX3379E/MAX3390E–MAX3393E

IEC 1000-4-2

The IEC 1000-4-2 standard  covers ESD testing and performance of finished equipment; it does not specifi- cally refer to integrated  circuits.  The MAX3372E– MAX3379E and MAX3390E–MAX3393E help to design equipment that meets Level 3 of IEC 1000-4-2, without the need for additional ESD-protection components.

The major difference  between tests done using the Human Body Model and IEC 1000-4-2 is higher peak current in IEC 1000-4-2, because  series resistance is lower in the IEC 1000-4-2 model. Hence, the ESD with- stand voltage measured to IEC 1000-4-2 is generally lower than that measured using the Human Body Model. Figure 4a shows the IEC 1000-4-2 model, and Figure 4b shows the current waveform for the ±8kV, IEC 1000-4-2, Level 4, ESD contact-discharge  test.

The air-gap test involves approaching  the device with a charged  probe.  The contact-discharge  method con- nects  the probe  to the device  before  the probe is energized.

Machine Model

The Machine Model for ESD tests all pins using a

200pF storage capacitor  and zero discharge  resis- tance. Its objective is to emulate the stress caused by contact that occurs with handling and assembly during manufacturing. Of course, all pins require this protec- tion during manufacturing, not just inputs and outputs. Therefore, after PCB assembly, the Machine Model is less relevant to I/O ports.

RC 1MΩ               RD 1500Ω

IP 100%

Ir               PEAK-TO-PEAK RINGING

HIGH-

CHARGE-CURRENT- LIMIT RESISTOR

DISCHARGE RESISTANCE

DEVICE-

AMPERES

90%

(NOT DRAWN TO SCALE)

VOLTAGE DC SOURCE

Cs

100pF

STORAGE

CAPACITOR

UNDER- TEST

36.8%

10%

0

0

tRL                            TIME

tDL

CURRENT WAVEFORM

Figure 3a. Human Body ESD Test Model

Figure 3b. Human Body Current Waveform

13

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

I

RC 50MΩ to 100MΩ       RD 330Ω

100%

IPEAK

90%

CHARGE-CURRENT- LIMIT RESISTOR

DISCHARGE RESISTANCE

HIGH- VOLTAGE DC SOURCE

Cs

150pF

STORAGE CAPACITOR

DEVICE- UNDER- TEST

10%

tr = 0.7ns to 1ns

30ns

t

60ns

Figure 4a. IEC 1000-4-2 ESD Test Model

Applications Information

Power-Supply Decoupling To reduce ripple and the chance of transmitting incor- rect data, bypass VL and VCC to ground with a 0.1µF capacitor. See the Typical Operating Circuit. To ensure full ±15kV ESD protection, bypass VCC to ground with a

1µF capacitor.  Place all capacitors  as close to the power-supply inputs as possible.

I2C Level Translation The MAX3373E–MAX3376E, MAX3378E/MAX3379E and MAX3390E–MAX3393E level-shift the data present on the I/O lines between +1.2V and +5.5V, making them ideal for level translation between a low-voltage

Figure 4b. IEC 1000-4-2 ESD Generator Current Waveform

ASIC and an I2C device. A typical application  involves interfacing a low-voltage microprocessor  to a 3V or 5V D/A converter, such as the MAX517.

Push-Pull vs.  Open-Drain Driving All  devices   in  the  MAX3372E–MAX3379E and MAX3390E–MAX3393E family may be driven in a push- pull configuration. The MAX3373E–MAX3376E/ MAX3378E/MAX3379E and MAX3390E–MAX3393E include internal 10kΩ resistors that pull up I/O VL_ and I/O VCC_ to their respective  power supplies, allowing operation of the I/O lines with open-drain devices. See the Timing Characteristics table for maximum data rates when using open-drain drivers.

Typical Operating Circuit

MAX3372E–MAX3379E/MAX3390E–MAX3393E

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM

VL

THREE-STATE

VCC

+3.3V SYSTEM

CONTROLLER

MAX3378E–MAX3383E

DATA

I/O VL_

I/O VCC_

DATA

14

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Applications Circuits

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3372E/MAX3373E

DATA

I/O VL1

I/O VL2

I/O VCC1

I/O VCC2

DATA

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3374E

DATA

I VL1

I VL2

O VCC1

O VCC2

DATA

15

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Applications Circuits (continued)

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3375E

DATA

O VL1

I VL2

I VCC1

O VCC2

DATA

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3376E

DATA

O VL1

O VL2

I VCC1

I VCC2

DATA

16

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Applications Circuits (continued)

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3377E/MAX3378E

DATA

I/O VL1

I/O VL2

I/O VL3

I/O VL4

I/O VCC1

I/O VCC2

I/O VCC3

I/O VCC4

DATA

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3379E

DATA

I VL1

I VL2

I VL3

I VL4

O VCC1

O VCC2

O VCC3

O VCC4

DATA

17

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Applications Circuits (continued)

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3390E

DATA

O VL1

I VL2

I VL3

I VL4

I VL1

O VCC2

O VCC3

O VCC4

DATA

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3391E

DATA

O VL1

O VL2

I VL3

I VL4

I VCC1

I VCC2

O VCC3

O VCC4

DATA

18

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Applications Circuits (continued)

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3392E

DATA

O VL1

O VL2

O VL3

I VL4

I VCC1

I VCC2

I VCC3

O VCC4

DATA

+1.8V

0.1μF

0.1μF

1μF

+3.3V

+1.8V SYSTEM CONTROLLER

VL

THREE-STATE

VCC

+3.3V SYSTEM

MAX3393E

DATA

O VL1

O VL2

O VL3

I VL4

I VCC1

I VCC2

I VCC3

I VCC4

DATA

19

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

| Selector Guide PART | LEVEL TRANS- LATION | Tx/ Rx† | DATA RATE | TOP MARK |
| --- | --- | --- | --- | --- |
| MAX3372EEKA+T | Bi | 2/2 | 230kbps | AAKO |
| MAX3372EEBL+T | Bi | 2/2 |  | AAR |
| MAX3372EETA+T | Bi | 2/2 |  | AQG |
| MAX3373EEKA+T | Bi | 2/2 | 8Mbps* | AAKS |
| MAX3373EEBL+T | Bi | 2/2 |  | AAZ |
| MAX3373EETA+T | Bi | 2/2 |  | AQH |
| MAX3374EEKA+T | Uni | 2/0 |  | AALH |
| MAX3374EEBL+T | Uni | 2/0 |  | ABA |
| MAX3374EETA+T | Uni | 2/0 |  | AQI |
| MAX3375EEKA+T | Uni | 1/1 |  | AALI |
| MAX3375EEBL+T | Uni | 1/1 |  | ABB |
| MAX3375EETA+T | Uni | 1/1 |  | AQJ |
| MAX3376EEKA+T | Uni | 0/2 |  | AALG |
| MAX3376EEBL+T | Uni | 0/2 |  | AAV |
| MAX3376EETA+T | Uni | 0/2 |  | AQK |
| MAX3377EEUD | Bi | 4/4 | 230kbps | — |
| MAX3377EEBC+T | Bi | 4/4 |  | AAX |
| MAX3377EETD+T | Bi | 4/4 |  | AAG |
| MAX3378EEUD | Bi | 4/4 | 8Mbps* | — |
| MAX3378EEBC+T | Bi | 4/4 |  | AAY |
| MAX3378EETD+T | Bi | 4/4 |  | AAH |
| MAX3379EEUD | Uni | 4/0 |  | — |
| MAX3379EEBC+T | Uni | 4/0 |  | AAZ |
| MAX3379EETD+T | Uni | 4/0 |  | AAI |
| MAX3390EEUD | Uni | 3/1 |  | — |
| MAX3390EEBC+T | Uni | 3/1 |  | ABA |
| MAX3390EETD+T | Uni | 3/1 |  | AAJ |
| MAX3391EEUD | Uni | 2/2 |  | — |
| MAX3391EEBC+T | Uni | 2/2 |  | ABB |
| MAX3391EETD+T | Uni | 2/2 |  | AAK |
| MAX3392EEUD | Uni | 1/3 |  | — |
| MAX3392EEBC+T | Uni | 1/3 |  | ABC |
| MAX3392EETD+T | Uni | 1/3 |  | AAL |
| MAX3393EEUD | Uni | 0/4 |  | — |
| MAX3393EEBC+T | Uni | 0/4 |  | ABD |
| MAX3393EETD+T | Uni | 0/4 |  | AAM |

Ordering Information (continued)

TEMP PIN- PKG PART RANGE PACKAGE CODE

MAX3372EEBL+T -40°C to +85°C 9 UCSP B9-2 (1.5mm x 1.5mm)

MAX3372EETA+T -40°C to +85°C 8 TDFN-EP** T833-2 (3mm x 3mm)

MAX3373EEKA+T -40°C to +85°C 8 SOT23 K8S-3

MAX3373EEBL+T -40°C to +85°C 9 UCSP B9-2 (1.5mm x 1.5mm)

MAX3373EETA+T -40°C to +85°C 8 TDFN-EP** T833-2 (3mm x 3mm)

MAX3374EEKA+T -40°C to +85°C 8 SOT23 K8S-3

MAX3374EEBL+T -40°C to +85°C 9 UCSP B9-2 (1.5mm x 1.5mm)

8 TDFN-EP** MAX3374EETA+T -40°C to +85°C (3mm x 3mm) T833-2

MAX3375EEKA+T -40°C to +85°C 8 SOT23 K8S-3

9 UCSP MAX3375EEBL+T -40°C to +85°C (1.5mm x 1.5mm) B9-2

8 TDFN-EP** MAX3375EETA+T -40°C to +85°C (3mm x 3mm) T833-2

MAX3376EEKA+T -40°C to +85°C 8 SOT23 K8S-3

9 UCSP MAX3376EEBL+T -40°C to +85°C (1.5mm x 1.5mm) B9-2

8 TDFN-EP** MAX3376EETA+T -40°C to +85°C (3mm x 3mm) T833-2

MAX3377EEUD -40°C to +85°C 14 TSSOP U14-1

9 UCSP MAX3377EEBC+T -40°C to +85°C (1.5mm x 1.5mm) B12-1

14 TDFN-EP** MAX3377EETD+T -40°C to +85°C (3mm x 3mm) T1433-2

MAX3378EEUD -40°C to +85°C 14 TSSOP U14-1

MAX3378EEBC+T -40°C to +85°C 12 UCSP B12-1 (1.5mm x 2.0mm)

MAX3378EETD+T -40°C to +85°C 14 TDFN-EP** T1433-2 (3mm x 3mm)

+Denotes a lead-free package.

**EP = Exposed pad. T = Tape and reel.

†Tx = VL → VCC, Rx = VCC → VL

*Higher data rates are possible (see the Timing Characteristics table).

20

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Ordering Information (continued)

PART TEMP PIN- PKG RANGE PACKAGE CODE

† MAX3379EEUD -40°C to +85°C 14 TSSOP U14-1

MAX3379EEBC+T† -40°C to +85°C 12 UCSP B12-1 (1.5mm x 2.0mm)

MAX3379EETD+T† -40°C to +85°C 14 TDFN-EP** T1433-2 (3mm x 3mm)

† MAX3390EEUD -40°C to +85°C 14 TSSOP U14-1

MAX3390EEBC+T† -40°C to +85°C 12 UCSP B12-1 (1.5mm x 2.0mm)

MAX3390EETD+T -40°C to +85°C 14 TDFN-EP** T1433-2 (3mm x 3mm)

† MAX3391EEUD -40°C to +85°C 14 TSSOP U14-1

MAX3391EEBC+T† -40°C to +85°C 12 UCSP B12-1 (1.5mm x 2.0mm)

MAX3391EETD+T -40°C to +85°C 14 TDFN-EP** T1433-2 (3mm x 3mm)

† MAX3392EEUD -40°C to +85°C 14 TSSOP U14-1

MAX3392EEBC+T† -40°C to +85°C 12 UCSP B12-1 (1.5mm x 2.0mm)

MAX3392EETD+T -40°C to +85°C 14 TDFN-EP** T1433-2 (3mm x 3mm)

† MAX3393EEUD -40°C to +85°C 14 TSSOP U14-1

MAX3393EEBC+T† -40°C to +85°C 12 UCSP B12-1 (1.5mm x 2.0mm)

MAX3393EETD+T† -40°C to +85°C 14 TDFN-EP** T1433-2 (3mm x 3mm)

+Denotes  a lead-free package.

**EP = Exposed pad. T = Tape and reel.

†Future product—contact factory for availability.

21

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Pin  Configurations (continued)

A                  B                  C

1

VCC

2

I/O VCC1

3

I/O VCC2

THREE-STATE N.C.

GND

VL

I/O VL1

I/O VL2

MAX3372E MAX3373E

I/O VCC2    1

GND    2

VL     3

I/O VL2    4

8  I/O VCC1

7  VCC

6   THREE-STATE

5  I/O VL1

I/O VCC2    1

GND    2

I/O VCC1    3

VCC     4

8  I/O VL2

7  VL

6   I/O VL1

5  THREE-STATE

9 UCSP (1.5mm  x 1.5mm) BOTTOM VIEW

SOT23-8

TOP VIEW

TDFN-8 (3mm x 3mm)

TOP VIEW

A                  B                  C

1

VCC

2

O VCC1

3

O VCC2

THREE-STATE N.C.

GND

VL

I VL1

I VL2

MAX3374E

O VCC2    1

GND    2

VL     3

I VL2    4

8  O VCC1

7  VCC

6   THREE-STATE

5  I VL1

O VCC2    1

GND    2

O VCC1    3

VCC    4

8  I VL2

7  VL

6   I VL1

5  THREE-STATE

9 UCSP (1.5mm  x 1.5mm) BOTTOM VIEW

SOT23-8

TOP VIEW

TDFN-8 (3mm x 3mm)

TOP VIEW

A                  B                  C

1

VCC

2

O VCC1

3

I VCC2

THREE-STATE N.C.

GND

VL

I VL1

O VL2

MAX3375E

O VCC2    1

GND    2

VL     3

I VL2    4

8  I VCC1

7  VCC

6   THREE-STATE

5  O VL1

O VCC2    1

GND    2

I VCC1    3

VCC    4

8  I VL2

7  VL

6   O VL1

5  THREE-STATE

9 UCSP (1.5mm  x 1.5mm) BOTTOM VIEW

SOT23-8

TOP VIEW

TDFN-8 (3mm x 3mm)

TOP VIEW

A                  B                  C

1

VCC

2

I VCC1

3

I VCC2

THREE-STATE N.C.

GND

VL

O VL1

O VL2

MAX3376E

I VCC2    1

GND    2

VL     3

O VL2    4

8  I VCC1

7  VCC

6   THREE-STATE

5  O VL1

I VCC2    1

GND    2

I VCC1    3

VCC    4

8  O VL2

7  VL

6   O VL1

5  THREE-STATE

9 UCSP (1.5mm  x 1.5mm) BOTTOM VIEW

SOT23-8

TOP VIEW

TDFN-8 (3mm x 3mm)

TOP VIEW

22

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Pin  Configurations (continued)

A                  B                  C

1

I/O VL1

2

VCC            I/O VCC1

MAX3377E MAX3378E

VL     1

I/O VL1     2

I/O VL2     3

14     VCC

13    I/0 VCC1

12    I/0 VCC2

I/O VL1     1

I/O VL2     2

THREE-STATE  3

14     VCC

13    I/0 VCC1

12    I/0 VCC2

I/O VL2

3

VL              I/O VCC2

I/O VL3     4

I/O VL4     5

11    I/0 VCC3

10    I/0 VCC4

N.C.     4

I/O VL3     5

11     N.C.

10     VL

I/O VL3  THREE-STATE I/O VCC3

4

N.C.     6

GND    7

9      N.C.

8      THREE-STATE

I/O VL4     6

GND    7

9     I/0 VCC3

8     I/0 VCC4

I/O VL4

GND

I/O VCC4

TSSOP-14

TDFN-14 (3mm x 3mm)

12 UCSP (1.5mm  x 2.0mm) BOTTOM VIEW

TOP VIEW

TOP VIEW

A                  B                  C

1

I VL1

2

VCC            O VCC1

MAX3379E

VL      1

I VL1     2

I VL2     3

14    VCC

13    O VCC1

12    O VCC2

I VL1     1

I VL2     2

THREE-STATE  3

14     VCC

13     0 VCC1

12     0 VCC2

I VL2

3

I VL3

4

VL              O VCC2

THREE-STATE  O VCC3

I VL3     4

I VL4     5

N.C.     6

GND     7

11    O VCC3

10    O VCC4

9    N.C.

8     THREE-STATE

N.C.     4

I VL3     5

I VL4     6

GND    7

11     N.C.

10     VL

9      0 VCC3

8     0 VCC4

I VL4

GND

O VCC4

TSSOP-14

TDFN-14 (3mm x 3mm)

12 UCSP (1.5mm  x 2.0mm) BOTTOM VIEW

TOP VIEW

TOP VIEW

A                  B                  C

1

O VL1

2

VCC             I VCC1

MAX3390E

VL     1

O VL1     2

I VL2     3

14    VCC

13   I VCC1

12    O VCC2

O VL1     1

I VL2     2

THREE-STATE  3

14     VCC

13     I VCC1

12     0 VCC2

I VL2

3

I VL3

4

VL               O VCC2

THREE-STATE   O VCC3

I VL3    4

I VL4    5

N.C.    6

GND    7

11    O VCC3

10    O VCC4

9    N.C.

8     THREE-STATE

N.C.     4

I VL3     5

I VL4     6

GND    7

11     N.C.

10     VL

9      0 VCC3

8     0 VCC4

I VL4

GND

O VCC4

TSSOP-14

TDFN-14 (3mm x 3mm)

12 UCSP (1.5mm  x 2.0mm) BOTTOM VIEW

TOP VIEW

TOP VIEW

23

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Pin  Configurations (continued)

A                  B                  C

1

O VL1

2

VCC              I VCC1

MAX3391E

VL    1

O VL1    2

O VL2    3

14    VCC

13   I VCC1

12   I VCC2

O VL1     1

O VL2     2

THREE-STATE  3

14     VCC

13     I VCC1

12     I VCC2

O VL2

3

I VL3

4

VL               I VCC2

THREE-STATE   O VCC3

I VL3    4

I VL4    5

N.C.    6

GND   7

11    O VCC3

10    O VCC4

9    N.C.

8     THREE-STATE

N.C.     4

I VL3     5

I VL4     6

GND    7

11     N.C.

10     VL

9      0 VCC3

8     0 VCC4

I VL4

GND

O VCC4

TSSOP-14

TDFN-14 (3mm x 3mm)

12 UCSP (1.5mm  x 2.0mm) BOTTOM VIEW

TOP VIEW

TOP VIEW

A                  B                  C

1

O VL1

2

VCC             I VCC1

MAX3392E

VL    1

O VL1    2

O VL2    3

14   VCC

13   I VCC1

12   I VCC2

O VL1     1

O VL2     2

THREE-STATE  3

14     VCC

13     I VCC1

12     I VCC2

O VL2

3

O VL3

4

I VL4

VL

THREE-STATE GND

I VCC2

I VCC3

O VCC4

O VL3    4

I VL4    5

N.C.    6

GND   7

11   I VCC3

10    O VCC4

9    N.C.

8    THREE-STATE

N.C.     4

O VL3     5

I VL4     6

GND    7

11     N.C.

10     VL

9      I VCC3

8     0 VCC4

12 UCSP (1.5mm  x 2.0mm) BOTTOM VIEW

TSSOP-14

TOP VIEW

TDFN-14 (3mm x 3mm) TOP VIEW

A                  B                  C

1

O VL1

2

VCC              I VCC1

MAX3393E

VL    1

O VL1    2

O VL2    3

14    VCC

13   I VCC1

12   I VCC2

O VL1     1

O VL2     2

THREE-STATE  3

14     VCC

13     I VCC1

12     I VCC2

O VL2

3

O VL3

4

O VL4

VL

THREE-STATE GND

I VCC2

I VCC3

I VCC4

O VL3    4

O VL4    5

N.C.    6

GND   7

11   I VCC3

10   I VCC4

9    N.C.

8     THREE-STATE

N.C.     4

O VL3     5

O VL4     6

GND    7

11     N.C.

10     VL

9      I VCC3

8     I VCC4

12 UCSP (1.5mm  x 2.0mm) BOTTOM VIEW

TSSOP-14

TOP VIEW

TDFN-14 (3mm x 3mm) TOP VIEW

24

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

SOT23, 8L.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

MARKING

0

0

PACKAGE OUTLINE, SOT-23, 8L BODY

1

21-0078            G      1

Chip Information

TRANSISTOR COUNT: MAX3372E–MAX3376E: 189

MAX3377E–MAX3379E, MAX3390E–MAX3393E: 295

PROCESS: BiCMOS

25

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

6, 8, &10L, DFN THIN.EPS

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

26

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) PACKAGE VARIATIONS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PKG. CODE | N | D2 | E2 | e | JEDEC SPEC | b | [(N/2)-1] x e |
| T633-2 | 6 | 1.50±0.10 | 2.30±0.10 | 0.95 BSC | MO229 / WEEA | 0.40±0.05 | 1.90 REF |
| T833-2 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF |
| T833-3 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF |
| T1033-1 | 10 | 1.50±0.10 | 2.30±0.10 | 0.50 BSC | MO229 / WEED-3 | 0.25±0.05 | 2.00 REF |
| T1033-2 | 10 | 1.50±0.10 | 2.30±0.10 | 0.50 BSC | MO229 / WEED-3 | 0.25±0.05 | 2.00 REF |
| T1433-1 | 14 | 1.70±0.10 | 2.30±0.10 | 0.40 BSC | - - - - | 0.20±0.05 | 2.40 REF |
| T1433-2 | 14 | 1.70±0.10 | 2.30±0.10 | 0.40 BSC | - - - - | 0.20±0.05 | 2.40 REF |

| COMMON DIMENSIONS |  |  |
| --- | --- | --- |
| SYMBOL | MIN. | MAX. |
| A | 0.70 | 0.80 |
| D | 2.90 | 3.10 |
| E | 2.90 | 3.10 |
| A1 | 0.00 | 0.05 |
| L | 0.20 | 0.40 |
| k | 0.25 MIN. |  |
| A2 | 0.20 REF. |  |

27

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

TSSOP4.40mm.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

PACKAGE OUTLINE, TSSOP 4.40mm BODY

1

21-0066                  I       1

28

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

9LUCSP, 3x3.EPS

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 3x3 UCSP

1

21-0093               L      1

29

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

12L, UCSP 4x3.EPS

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 4x3 UCSP

1

21-0104               F      1

30

±15kV ESD-Protected, 1µA, 16Mbps, Dual/Quad

Low-Voltage Level Translators in UCSP

MAX3372E–MAX3379E/MAX3390E–MAX3393E

Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGES CHANGED |
| --- | --- | --- | --- |
| 0 | 1/02 | Initial Release | — |
| 1 | 12/06 | Addition of 12-bump ECSP packaging | – |
| 2 | 11/07 | Addition of lead-free options | 1, 20–31 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600                                31

©  2007 Maxim Integrated  Products                                                               is a registered  trademark  of Maxim Integrated  Products, Inc.