19-0344; Rev 6; 12/05

3-Pin Microprocessor Reset Circuits

General Description

The MAX803/MAX809/MAX810 are microprocessor (µP) supervisory circuits used to monitor the power supplies in µP and digital systems. They provide excellent circuit reliability and low cost by eliminating external compo- nents and adjustments  when used with +5V, +3.3V,

+3.0V, or +2.5V powered circuits.

These circuits perform a single function: they assert a reset signal whenever the VCC supply voltage declines below a preset threshold, keeping  it asserted for at least 140ms after VCC has risen above the reset thresh- old. Reset thresholds suitable for operation with a vari- ety of supply voltages are available.

The MAX803 has an open-drain output stage, while the MAX809/MAX810 have push-pull outputs. The MAX803’s open-drain RESET output requires a pull-up resistor that can be connected to a voltage higher than VCC. The MAX803/MAX809 have an active-low RESET output, while the MAX810 has an active-high  RESET output. The reset comparator is designed to ignore fast transients on VCC, and the outputs are guaranteed  to be in the correct logic state for VCC down to 1V.

Low supply  current makes the MAX803/MAX809/ MAX810 ideal for use in portable  equipment.  The MAX803 is available in a 3-pin SC70 package, and the MAX809/MAX810 are available in 3-pin SC70 or SOT23 packages.

Applications

Computers

Controllers

Intelligent Instruments

Critical µP and µC Power Monitoring Portable/Battery-Powered Equipment Automotive

Typical Operating Circuit

MAX803L/M/R/S/T/Z, MAX809J/L/M/R/S/T/Z, MAX810L/M/R/S/T/Z

Features

♦ Precision Monitoring of +2.5V, +3V, +3.3V, and

+5V Power-Supply Voltages

♦ Fully Specified Over Temperature

♦ Available in Three Output Configurations Open-Drain RESET Output (MAX803) Push-Pull RESET Output (MAX809) Push-Pull RESET Output (MAX810)

♦ 140ms min Power-On Reset Pulse Width

♦ 12µA Supply Current

♦ Guaranteed Reset Valid to VCC = +1V

♦ Power Supply Transient Immunity

♦ No External Components

♦ 3-Pin SC70 and SOT23 Packages

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX803_EXR-T -40°C to +125°C 3 SC70-3

MAX803_EXR-T10 -40°C to +125°C 3 SC70-3

MAX809_EXR-T -40°C to +125°C 3 SC70-3

MAX809_EXR-T10 -40°C to +125°C 3 SC70-3

MAX809_EUR-T -40°C to +105°C 3 SOT23-3

MAX809_EUR-T10 -40°C to +105°C 3 SOT23-3

MAX810_EXR-T -40°C to +125°C 3 SC70-3

MAX810_EXR-T10 -40°C to +125°C 3 SC70-3

MAX810_EUR-T -40°C to +105°C 3 SOT23-3

MAX810_EUR-T10 -40°C to +105°C 3 SOT23-3

Note:  These parts are offered in 2.5k or 10k reels, and must be ordered in 2.5k or 10k increments.  Order MAX803_EXR-T for

2.5k reels and MAX803_EXR-T10 for 10k reels. Insert the desired suffix letter from the Selector Guide into the blank to complete the part number. All versions of these products may

not be available at the time of announcement. Contact factory for availability.

Devices are available in both leaded and lead-free packaging.

Specify lead-free by replacing “-T” with “+T” when ordering.

Pin  Configuration

VCC

TOP VIEW

VCC

MAX803

MAX809

RPULL-UP*

VCC

P

GND

VCC

RESET RESET                                         INPUT

GND

GND

RESET (RESET)

( ) ARE FOR MAX810.

| SC70-3/SOT23-3 | MAX803 MAX809 MAX810 |  |
| --- | --- | --- |
| 1 |  |  |
|  |  | 3 |
| 2 |  |  |

*MAX803 ONLY

MAX803 AVAILABLE IN SC70-3 ONLY.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

3-Pin Microprocessor Reset Circuits

MAX803L/M/R/S/T/Z, MAX809J/L/M/R/S/T/Z, MAX810L/M/R/S/T/Z

ABSOLUTE MAXIMUM RATINGS

Terminal Voltage (with respect to GND)

VCC ....................................................................-0.3V to +6.0V RESET, RESET (push-pull) .....................-0.3V  to (VCC + 0.3V) RESET (open drain) ...........................................-0.3V to +6.0V Input Current, VCC ..............................................................20mA Output Current, RESET, RESET ..........................................20mA Rate of Rise, VCC ............................................................100V/µs

Continuous Power Dissipation  (TA = +70°C)

3-Pin SC70 (derate 2.17mW/°C above +70°C) ............174mW

3-Pin SOT23 (derate 4mW/°C above +70°C)...............320mW Operating  Temperature  Range

3-Pin SC70 .....................................................-40°C to +125°C

3-Pin SOT23...................................................-40°C to +105°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = full range, TA = -40°C to +105°C (SOT23) or TA = -40°C to +125°C (SC70), unless otherwise noted. Typical values are at TA =

+25°C, VCC = 5V for L/M/J versions, VCC = 3.3V for T/S versions, VCC = 3V for R version, and VCC = 2.5V for Z version.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| VCC Range |  | TA = 0°C to +70°C |  | 1.0 5.5 | V |
|  |  | TA = -40°C to +105°C (MAX8_ _ _EUR) |  | 1.2 5.5 |  |
|  |  | TA = -40°C to +125°C (MAX8_ _ _EXR) |  | 1.2 5.5 |  |
| Supply Current (SOT23) | ICC | TA = -40°C to +85°C | VCC < 5.5V, MAX8_ _L/M | 24 60 | µA |
|  |  |  | VCC < 3.6V, MAX8_ _R/S/T/Z | 17 50 |  |
|  |  | TA = +85°C to +105°C | VCC < 5.5V, MAX8_ _L/M | 100 |  |
|  |  |  | VCC < 3.6V, MAX8_ _R/S/T/Z | 100 |  |
| Supply Current (SC70) | ICC | TA = -40°C to +85C | VCC < 5.5V, MAX8_ _L/M | 24 35 |  |
|  |  |  | VCC < 3.6V, MAX8_ _R/S/T/Z | 17 30 |  |
|  |  | TA = +85°C to +125°C | VCC < 5.5V, MAX8_ _L/M | 60 |  |
|  |  |  | VCC < 3.6V, MAX8_ _R/S/T/Z | 60 |  |
| Reset Threshold (SOT only) | VTH | MAX8_ _L | TA = +25°C | 4.56 4.63 4.70 | V |
|  |  |  | TA = -40°C to +85°C | 4.50 4.75 |  |
|  |  |  | TA = -40°C to +125°C | 4.40 4.86 |  |
|  |  | MAX8_ _M | TA = +25°C | 4.31 4.38 4.45 |  |
|  |  |  | TA = -40°C to +85°C | 4.25 4.50 |  |
|  |  |  | TA = -40°C to +125°C | 4.16 4.56 |  |
|  |  | MAX809J (SOT only) | TA = +25°C | 3.93 4.00 4.06 |  |
|  |  |  | TA = -40°C to +85°C | 3.89 4.10 |  |
|  |  |  | TA = -40°C to +125°C | 3.80 4.20 |  |
|  |  | MAX8_ _T | TA = +25°C | 3.04 3.08 3.11 |  |
|  |  |  | TA = -40°C to +85°C | 3.00 3.15 |  |
|  |  |  | TA = -40°C to +125°C | 2.92 3.23 |  |
|  |  | MAX8_ _S | TA = +25°C | 2.89 2.93 2.96 |  |
|  |  |  | TA = -40°C to +85°C | 2.85 3.00 |  |
|  |  |  | TA = -40°C to +125°C | 2.78 3.08 |  |
|  |  | MAX8_ _R | TA = +25°C | 2.59 2.63 2.66 |  |
|  |  |  | TA = -40°C to +85°C | 2.55 2.70 |  |
|  |  |  | TA = -40°C to +125°C | 2.50 2.76 |  |

2

3-Pin Microprocessor Reset Circuits

MAX803L/M/R/S/T/Z, MAX809J/L/M/R/S/T/Z, MAX810L/M/R/S/T/Z

ELECTRICAL CHARACTERISTICS (continued)

(VCC = full range, TA = -40°C to +105°C (SOT23) or TA = -40°C to +125°C (SC70), unless otherwise noted. Typical values are at TA =

+25°C, VCC = 5V for L/M/J versions, VCC = 3.3V for T/S versions, VCC = 3V for R version, and VCC = 2.5V for Z version.) (Note 1)

CC       TH

| SINK PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Reset Threshold (SC70 only) | VTH | MAX8_ _L | TA = +25°C | 4.56 4.63 4.70 | V |
|  |  |  | TA = -40°C to +85°C | 4.50 4.75 |  |
|  |  |  | TA = -40°C to +125°C | 4.44 4.82 |  |
|  |  | MAX8_ _M | TA = +25°C | 4.31 4.38 4.45 |  |
|  |  |  | TA = -40°C to +85°C | 4.25 4.50 |  |
|  |  |  | TA = -40°C to +125°C | 4.20 4.56 |  |
|  |  | MAX8_ _T | TA = +25°C | 3.04 3.08 3.11 |  |
|  |  |  | TA = -40°C to +85°C | 3.00 3.15 |  |
|  |  |  | TA = -40°C to +125°C | 2.95 3.21 |  |
|  |  | MAX8_ _S | TA = +25°C | 2.89 2.93 2.96 |  |
|  |  |  | TA = -40°C to +85°C | 2.85 3.00 |  |
|  |  |  | TA = -40°C to +125°C | 2.81 3.05 |  |
|  |  | MAX8_ _R | TA = +25°C | 2.59 2.63 2.66 |  |
|  |  |  | TA = -40°C to +85°C | 2.55 2.70 |  |
|  |  |  | TA = -40°C to +125°C | 2.52 2.74 |  |
|  |  | MAX8_ _Z (SC70 only) | TA = +25°C | 2.28 2.32 2.35 |  |
|  |  |  | TA = -40°C to +85°C | 2.25 2.38 |  |
|  |  |  | TA = -40°C to +125°C | 2.22 2.42 |  |
| Reset Threshold Tempco |  |  |  | 30 | ppm/°C |
| VCC to Reset Delay (Note 2) |  | VCC = VTH to (VTH - 100 mV) |  | 20 | µs |
| Reset Active Timeout Period (SOT23) |  | TA = -40°C to +85°C |  | 140 240 560 | ms |
|  |  | TA = +85°C to +105°C |  | 100 840 |  |
| Reset Active Timeout Period (SC70) |  | TA = -40°C to +85°C |  | 140 240 460 | ms |
|  |  | TA = +85°C to +125°C |  | 100 840 |  |
| RESET Output Voltage Low (push- pull active low and open-drain active low, MAX803 and MAX809) | VOL | VCC = VTH min, ISINK = 1.2mA, MAX803R/S/T/Z, MAX809R/S/T/Z |  | 0.3 | V |
|  |  | VCC = VTH min, ISINK = 3.2mA, MAX803L/M, MAX809J/L/M |  | 0.4 |  |
|  |  | VCC > 1.0V, ISINK = 50µA |  | 0.3 |  |
| RESET Output Voltage High (push-pull active low MAX809) | VOH | VCC > VTH max, ISOURCE = 500µA, MAX803R/S/T/Z, MAX809R/S/T/Z |  | 0.8VCC | V |
|  |  | VCC > VTH max, ISOURCE = 800µA, MAX803L/M, MAX809J/L/M |  | VCC - 1.5 |  |
| RESET Output Voltage Low (push- pull active high, MAX810) | VOL | VCC = VTH max, ISINK = 1.2mA, MAX810R/S/T/Z |  | 0.3 | V |
|  |  | V = V max, I = 3.2mA, MAX810L/M |  | 0.4 |  |

3

3-Pin Microprocessor Reset Circuits

MAX810L/M/R/S/T/Z

ELECTRICAL CHARACTERISTICS (continued)

(VCC = full range, TA = -40°C to +105°C (SOT23) or TA = -40°C to +125°C (SC70), unless otherwise noted. Typical values are at TA =

+25°C, VCC = 5V for L/M/J versions, VCC = 3.3V for T/S versions, VCC = 3V for R version, and VCC = 2.5V for Z version.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| RESET Output Voltage High (push-pull active high, MAX810) | VOH | 1.8V < VCC < VTH min, ISOURCE = 150µA | 0.8VCC | V |
| RESET Open-Drain Output Leakage Current (MAX803) (Note 3) |  | VCC > VTH, RESET deasserted | 1 | µA |

Note 1: Production  testing done at TA = +25°C; limits over temperature  guaranteed by design only.

Note 2: RESET output for MAX803/MAX809; RESET output for MAX810.

Note 3: Guaranteed by design, not production tested.

MAX809J/L/M/R/S/T/Z,

Typical Operating Characteristics (VCC = full range,  TA = -40°C to +105°C,  unless otherwise  noted. Typical values are at TA = +25°C,  VCC = +5V for L/M/J versions, VCC = +3.3V for T/S versions, VCC = +3V for R version, and VCC = +2.5V for Z version.)

SUPPLY CURRENT vs. TEMPERATURE (SC70 PACKAGE, NO LOAD)

15

MAX803 toc01

POWER-DOWN  RESET DELAY (s)

100

POWER-DOWN RESET DELAY vs. TEMPERATURE (MAX8_ _R/S/T/Z)

MAX803-TOC3

VOD = VTH - VCC

140

POWER-DOWN RESET DELAY vs. TEMPERATURE (MAX8_ _J/L/M)

MAX803-TOC4

VOD = VTH - VCC

MAX8_ _L/M, VCC = 5V                                                   80

10                            MAX8_ _R/S/T, VCC = 3.3V

60

MAX8_ _Z, VCC = 2.5V

VOD = 10mV

POWER-DOWN  RESET DELAY (s)

VOD = 20mV                           80

VOD = 20mV

SUPPLY CURRENT (A)

5

40

MAX8_ _L/M/R/S/T/Z, VCC = 1V

0                                                                                       0

20

VOD = 200mV

0

VOD = 200mV

-40

-20       0

20      40

60         85

-40

-20        0

20       40        60        85

-40

-20        0

20       40

60       85

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

| 250 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MAX8_ _J/L/M |  |  |  |
|  |  |  | MAX8_ _ | R/S/T/Z |  |  |

MAX803-TOC5

POWER-UP RESET TIMEOUT

vs. TEMPERATURE

| 1.003 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

MAX803-TOC6

NORMALIZED  RESET THRESHOLD

vs. TEMPERATURE

245

1.002

240

235

POWER-UP  RESET TIMEOUT  (ms)

230

1.001

1.000

NORMALIZED THRESHOLD

0.999

0.998

225

-40

-20        0

20       40

60       85

0.997

-40

-20        0

20       40

60       85

TEMPERATURE (°C)

TEMPERATURE (°C)

MAX803L/M/R/S/T/Z,

4

3-Pin Microprocessor Reset Circuits

MAX803L/M/R/S/T/Z, MAX809J/L/M/R/S/T/Z, MAX810L/M/R/S/T/Z

Selector Guide

| PART/SUFFIX | RESET THRESHOLD (V) | OUTPUT TYPE | TOP MARK |  |
| --- | --- | --- | --- | --- |
|  |  |  | SOT | SC70 |
| MAX803L | 4.63 | OPEN-DRAIN RESET |  | AAZ |
| MAX803M | 4.38 | OPEN-DRAIN RESET |  | ABA |
| MAX803T | 3.08 | OPEN-DRAIN RESET |  | ABB |
| MAX803S | 2.93 | OPEN-DRAIN RESET |  | ABC |
| MAX803R | 2.63 | OPEN-DRAIN RESET |  | ABD |
| MAX803Z | 2.32 | OPEN-DRAIN RESET |  | ABE |
| MAX809L | 4.63 | PUSH-PULL RESET | AAAA | AAN |
| MAX809M | 4.38 | PUSH-PULL RESET | ABAA | AAO |
| MAX809J | 4.00 | PUSH-PULL RESET | CWAA |  |
| MAX809T | 3.08 | PUSH-PULL RESET | ACAA | AAP |
| MAX809S | 2.93 | PUSH-PULL RESET | ADAA | AAQ |
| MAX809R | 2.63 | PUSH-PULL RESET | AFAA | AAR |
| MAX809Z | 2.32 | PUSH-PULL RESET |  | AAS |
| MAX810L | 4.63 | PUSH-PULL RESET | AGAA | AAT |
| MAX810M | 4.38 | PUSH-PULL RESET | AHAA | AAU |
| MAX810T | 3.08 | PUSH-PULL RESET | AJAA | AAV |
| MAX810S | 2.93 | PUSH-PULL RESET | AKAA | AAX |
| MAX810R | 2.63 | PUSH-PULL RESET | ALAA | AAW |
| MAX810Z | 2.32 | PUSH-PULL RESET |  | AAY |

Detailed Description

A microprocessor’s  (µP’s) reset input starts the µP in a known state. The MAX803/MAX809/MAX810 assert reset to prevent code-execution  errors during  power- up, power-down, or brownout conditions. They assert a reset signal whenever the VCC supply voltage declines below a preset threshold, keeping  it asserted for at least 140ms after VCC has risen above the reset thresh- old. The MAX803 uses an open-drain  output, and the MAX809/MAX810 have a push-pull  output  stage. Connect a pull-up resistor on the MAX803’s RESET out- put to any supply between 0 and 6V.

| Pin Description PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | RESET (MAX803/ MAX809) | RESET Output remains low while VCC is below the reset threshold, and for at least 140ms after VCC rises above the reset threshold. |
|  | RESET (MAX810) | RESET Output remains high while VCC is below the reset threshold, and for at least 140ms after VCC rises above the reset threshold. |
| 3 | VCC | Supply Voltage (+5V, +3.3V, +3.0V, or +2.5V) |

Applications Information

Negative-Going VCC Transients In addition to issuing a reset to the µP during power-up, power-down,  and brownout conditions,  the MAX803/ MAX809/MAX810 are relatively immune to short-duration negative-going VCC transients (glitches).

Figure 1 shows typical transient duration vs. reset com- parator overdrive, for which the MAX803/MAX809/ MAX810 do not generate a reset pulse. The graph was generated using a negative-going pulse applied to VCC, starting 0.5V above the actual reset threshold and end- ing below it by the magnitude indicated (reset compara- tor overdrive). The graph indicates the maximum pulse width a negative-going  VCC transient can have without causing a reset pulse. As the magnitude of the transient increases (goes farther below the reset threshold), the maximum allowable pulse width decreases. Typically, for the MAX8__L and MAX8__M, a VCC transient that goes

100mV below the reset threshold and lasts 20µs or less will not cause a reset pulse. A 0.1µF bypass capacitor mounted as close as possible to the VCC pin provides additional transient immunity.

Ensuring a Valid Reset Output

Down to VCC  = 0

When VCC falls below 1V, the MAX809 RESET output

no longer sinks current—it becomes  an open circuit.

5

3-Pin Microprocessor Reset Circuits

MAX803L/M/R/S/T/Z, MAX809J/L/M/R/S/T/Z, MAX810L/M/R/S/T/Z

Therefore, high-impedance  CMOS logic  inputs con- nected to RESET can drift to undetermined  voltages. This presents no problem  in most applications  since most µP and other circuitry  is inoperative with VCC below 1V. However, in applications  where RESET must be valid down to 0V, adding  a pull-down  resistor to RESET causes any stray leakage currents to flow to ground, holding RESET low (Figure 2). R1’s value is not critical; 100k is large enough not to load RESET and small enough to pull RESET to ground.

A 100k pull-up resistor to VCC is also recommended for the MAX810 if RESET is required to remain valid for VCC < 1V.

Interfacing to µPs  with

Bidirectional Reset Pins Since the RESET output on the MAX803 is open drain, this device interfaces easily with µPs that have bidirec- tional reset pins,  such  as the Motorola 68HC11. Connecting  the µP supervisor’s RESET output directly to the microcontroller’s (µC’s) RESET pin with a single pull-up  resistor allows either device  to assert reset (Figure 3).

MAX803 Open-Drain RESET Output

Allows Use with Multiple Supplies Generally, the pull-up  connected  to the MAX803 will connect to the supply voltage that is being monitored at the IC’s VCC pin. However, some systems may use the open-drain output to level-shift from the monitored sup- ply to reset circuitry  powered  by some other supply (Figure 4). Note that as the MAX803’s VCC decreases

400

320

TA = +25°C

VCC

240

160

MAX809

RESET

MAX8_ _J/L/M

MAXIMUM TRANSIENT DURATION (s)

80

MAX8_ _R/S/T/Z

0

GND

R1

100k

1                   10

100               1000

RESET COMPARATOR  OVERDRIVE, VTH - VCC (mV)

Figure 1.  Maximum Transient Duration Without Causing a

Reset Pulse vs. Reset Comparator  Overdrive

Figure 2.  RESET Valid to VCC = Ground Circuit

+3.3V

+5.0V

VCC

VCC

MAX803

VCC

P

MOTOROLA

RESET    68HCXX

VCC

MAX803

RESET

RPULLUP

RESET INPUT

VCC

5V SYSTEM

RESET

INPUT

GND

GND

GND

GND

Figure 3. Interfacing to µPs with Bidirectional Reset I/O

Figure 4. MAX803 Open-Drain RESET Output Allows Use with

Multiple Supplies

6

3-Pin Microprocessor Reset Circuits

below 1V, so does the IC’s ability to sink current at RESET. Also, with any pull-up,  RESET will be pulled high as VCC decays toward 0. The voltage where this occurs depends  on the pull-up resistor value and the voltage to which it is connected.

Benefits of Highly Accurate

Reset Threshold Most µP supervisor ICs have reset threshold voltages between 5% and 10% below the value of nominal sup- ply voltages. This ensures a reset will not occur within

5% of the nominal supply, but will occur when the sup- ply is 10% below nominal.

MAX803L/M/R/S/T/Z, MAX809J/L/M/R/S/T/Z, MAX810L/M/R/S/T/Z

When using ICs rated at only the nominal supply ±5%, this leaves a zone of uncertainty where the supply is between 5% and 10% low, and where the reset may or may not be asserted.

The MAX8_ _L/T/Z use highly accurate  circuitry  to ensure that reset is asserted close to the 5% limit, and

long before the supply  has declined  to 10% below nominal.

Chip Information

TRANSISTOR COUNT: 275 (SOT23)

380 (SC70)

SC70, 3L.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 3L SC70

1

21-0075                C      1

7

3-Pin Microprocessor Reset Circuits

MAX809J/L/M/R/S/T/Z,

MAX810L/M/R/S/T/Z

SOTPO3L.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |  |  |
| --- | --- | --- | --- | --- |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

8                                       Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

MAX803L/M/R/S/T/Z,

©  2005 Maxim Integrated  Products               Printed USA                           is a registered  trademark  of Maxim Integrated  Products, Inc.