19-1842; Rev 2; 1/07

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

General Description

The MAX9021/MAX9022/MAX9024 single/dual/quad comparators  are optimized for low-power consumption while still providing  a fast output response. They are designed  for single-supply  applications  from 2.5V to

5.5V, but can also operate from dual supplies. These comparators  have a 3µs propagation  delay and con- sume 2.8µA of supply current per comparator over the

-40°C to +125°C  operating  temperature  range. The combination  of low-power,  single-supply  operation down to 2.5V, and ultra-small footprint makes these devices ideal for portable applications.

The MAX9021/MAX9022/MAX9024 have 4mV of built-in hysteresis to provide noise immunity and prevent oscil- lations even with a slow-moving input signal. The input common-mode range extends from the negative supply to within 1.1V of the positive supply. The design of the comparator-output  stage substantially reduces  switch- ing current during output transitions, eliminating power- supply glitches.

The MAX9021 single comparator  is available in tiny 5- pin SC70 and SOT23 packages.  The MAX9022 dual comparator  is available in 8-pin SOT23, µMAX®, and SO packages,  and the MAX9024 quad  comparator  is available in 14-pin TSSOP and SO packages.

Applications

MAX9021/MAX9022/MAX9024

Features

♦ Low-Cost Solution Available in Space-Saving

SC70 Packages (Half the Size of SOT23)

♦ Low 2.8µA Supply Current

♦ 3µs Propagation Delay

♦ Internal 4mV Comparator Hysteresis

♦ Comparator Output Swings Rail-to-Rail

♦ 2.5 to 5.5V Single-Supply Voltage Range

♦ No Phase Reversal for Overdriven Inputs

♦ Space-Saving Packages

5-Pin SC70 (MAX9021)

8-Pin SOT23 (MAX9022)

8-Pin µMAX (MAX9022)

14-Pin TSSOP (MAX9024)

Ordering Information PIN- PKG PART TEMP RANGE PACKAGE CODE

MAX9021AXK-T -40°C to +125°C 5 SC70-5 X5-1

MAX9021AUK-T -40°C to +125°C 5 SOT23-5 U5-1

MAX9022AKA-T -40°C to +125°C 8 SOT23-8 K8-5

MAX9022AUA -40°C to +125°C 8 µMAX U8-1

MAX9022ASA -40°C to +125°C 8 SO S8-2

MAX9024AUD -40°C to +125°C 14 TSSOP U14-1

MAX9024ASD -40°C to +125°C 14 SO S14-2

Battery-Powered

Portable Systems

Mobile Communications Sensor-Signal Detection Photodiode Preamps

Digital Line Receivers Keyless Entry Systems Threshold Detectors/

Discriminators

Typical Application  Circuit appears at end of data sheet.

Pin  Configurations

TOP VIEW

IN+

1                                     5     VDD

OUTA   1

8     VDD

OUTA   1

14    OUTD

IN-

MAX9021

3                                    4

SC70/SOT23

OUT

INA+

MAX9022

3                                         6

S0T23/µMAX/SO

INB-

INA+    3

VDD    4

INB+    5

INB-    6

OUTB   7

MAX9024

12    IND+

11   VSS

10    INC+

9    INC-

8     OUTC

TSSOP/SO

µMAX is a registered trademark of Maxim Integrated Products, Inc.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

MAX9021/MAX9022/MAX9024

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VDD to VSS) ....................................-0.3V to +6V Voltage Inputs (IN+, IN- to VSS). ................-0.3V  to (VDD + 0.3V) Differential Input Voltage (IN+ to IN-)....................................6.6V Current into Input Pins ......................................................±20mA Output Short-Circuit  Duration ..................2s  to Either VDD or VSS Current into Any Pin ............................................................20mA Continuous Power Dissipation  (TA = +70°C)

5-Pin SC70 (derate 3.1mW/°C above +70°C) ...............247mW

5-Pin SOT23 (derate 7.1mW/°C above +70°C).............571mW

8-Pin SOT23 (derate 9.1mW/°C above +70°C).............727mW

8-Pin µMAX (derate 4.5mW/°C above +70°C) ..............362mW

8-Pin SO (derate 5.88mW/°C above +70°C).................471mW

14-Pin TSSOP (derate 9.1mW/°C above +70°C) ..........727mW

14-Pin SO (derate 8.3mW/°C above +70.......................667mW Operating  Temperature  Range

Automotive Application...................................-40°C to +125°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VDD = 5V, VSS = 0, VCM = 0, TA = -40°C to +125°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Operating Voltage Range | VDD | Guaranteed by PSRR test |  | 2.5 5.5 | V |
| Supply Current Per Comparator | IDD |  |  | 2.8 5 | µA |
| Input Offset Voltage | VOS | (Note 2) |  | ±1 ±8 | mV |
| Input Offset Voltage Temperature Coefficient | TCVOS |  |  | ±1 | µV/°C |
| Hysteresis |  | (Note 3) |  | 4 | mV |
| Input Bias Current | IBIAS |  |  | 3 80 | nA |
| Input Offset Current | IOS |  |  | ±2 ±60 | nA |
| Common-Mode Voltage Range | VCM | Guaranteed by CMRR test |  | VSS VDD - 1.1 | V |
| Common-Mode Rejection Ratio | CMRR | VSS ≤ VCM ≤ (VDD - 1.1V), VDD = 5.5V |  | 70 100 | dB |
| Power-Supply Rejection Ratio | PSRR | VDD = 2.5V to 5.5V |  | 60 80 | dB |
| Output-Voltage Swing | VOL, VOH | VOH = VDD - VOUT, (VIN+ - VIN-) ≥ 20mV | ISOURCE = 10µA | 2 | mV |
|  |  |  | ISOURCE = 4mA | 160 400 |  |
|  |  | VOL = VOUT - VSS, (VIN- - VIN+) ≥ 20mV | ISINK = 10µA | 2 |  |
|  |  |  | ISINK = 4mA | 180 400 |  |
| Output Short-Circuit Current | ISC |  |  | 50 | mA |
| Propagation Delay | tpd+, tpd- | RL = 10kΩ, CL = 15pF (Note 4) | VOD = 10mV | 8 | µs |
|  |  |  | VOD = 100mV | 3 |  |
| Rise and Fall Time | tR, tF | RL = 10kΩ, CL = 15pF (Note 5) |  | 20 | ns |
| Power-On Time |  | RL = 10kΩ, CL = 15pF |  | 150 | ns |
| Maximum Capacitive Load | CL | No sustained oscillations |  | 150 | pF |

Note 1: All devices are production tested at 25°C. All temperature limits are guaranteed by design.

Note 2: Comparator Input Offset is defined as the center of the hysteresis zone.

Note 3: Hysteresis is defined as the difference of the trip points required to change comparator output states.

Note 4: VOD is the overdrive voltage beyond the offset and hysteresis-determined trip points.

Note 5: Rise and fall times are measured between 10% and 90% at OUT.

2

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

MAX9021/MAX9022/MAX9024

Typical Operating Characteristics

(VDD = 5V, VSS = 0, VCM = 0, RL = 10kΩ, CL = 15pF, VOD = 100mV, TA = +25°C, unless otherwise noted.)

3.0

SUPPLY CURRENT (µA)

2.9

2.8

2.7

SUPPLY CURRENT

vs. SUPPLY VOLTAGE

MAX9021/2/4 toc01

3.0

2.9

2.8

SUPPLY CURRENT (µA)

2.7

2.6

MAX9021/2/4 toc02

| SUPPLY CURRENT vs. TEMPERATURE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1000

100

SUPPLY CURRENT (µA)

10

SUPPLY CURRENT

MAX9021/2/4 toc03

vs. OUTPUT TRANSITION  FREQUENCY

2.6

| 2 |  |  |  |
| --- | --- | --- | --- |

3              4               5               6

2.5

-50

-25     0

25     50     75    100   125

1

0.01

0.1          1

10        100

1000

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

OUTPUT TRANSITION FREQUENCY (kHz)

1.4

INPUT OFFSET VOLTAGE

vs. TEMPERATURE

MAX9021/2/4 toc04

500

OUTPUT HIGH VOLTAGE

| vs. SOURCE CURRENT |  |  |  |  |
| --- | --- | --- | --- | --- |

500

MAX9021/2/4 toc06

| OUTPUT LOW VOLTAGE |  |  |  |  |
| --- | --- | --- | --- | --- |

vs. SINK CURRENT

INPUT OFFSET VOLTAGE (mV)

1.2

1.0

0.8

0.6

400

300

(VDD - VOUT) (mV)

200

OUTPUT LOW VOLTAGE (mV)

400

MAX9021/2/4 toc05

300

200

0.4

0.2

100

100

0

| -50 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-25

0      25

50     75    100   125

0

0           2           4

6           8          10

0

0           2           4

6           8          10

TEMPERATURE (°C)

SOURCE CURRENT (mA)

SINK CURRENT (mA)

OUTPUT SHORT-CIRCUIT CURRENT

vs. TEMPERATURE

65

MAX9021/2/4 toc08

PROPAGATION DELAY vs. CAPACITIVE LOAD (VDD = 2.7V)

MAX9021/2/4 toc07

| 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | tPD- |  |  |
|  |  | tPD+ |  |  |  |

PROPAGATION DELAY vs. CAPACITIVE LOAD (VDD = 5V)

5

PROPAGATION DELAY (µs)

60                                                                                           4

55                                                                                           3

SOURCE CURRENT

50                                                                                           2

PROPAGATION DELAY (µs)

4

MAX9021/2/4 toc09

tPD-

3

2                                            tPD+

SINK CURRENT

OUTPUT SHORT-CIRCUIT CURRENT (mA)

45                                                                                           1                                                                                           1

40

-50

-25

0      25     50     75    100   125

TEMPERATURE (°C)

0

0             500          1000        1500        2000

CAPACITIVE LOAD (pF)

0

0             500          1000        1500        2000

CAPACITIVE LOAD (pF)

3

MAX9021/2/4 toc10

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

MAX9021/MAX9022/MAX9024

Typical Operating Characteristics (continued)

(VDD = 5V, VSS = 0, VCM = 0, RL = 10kΩ, CL = 15pF, VOD = 100mV, TA = +25°C, unless otherwise noted.)

PROPAGATION DELAY

vs. TEMPERATURE

5

PROPAGATION DELAY (µs)

4

MAX9021/2/4 toc11

tPD-

3

2                                                tPD+

1

0

PROPAGATION DELAY

vs. INPUT OVERDRIVE VOLTAGE

| 9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | tPD- |  |  |  |
|  |  |  | tPD+ |  |  |  |

8

PROPAGATION DELAY (µs)

7

6

5

MAX9021/2/4 toc13

4

3

2

1

0

IN+

100mV/div

VOUT

2.5V/div

MAX9021/2/4 toc12

PROPAGATION DELAY (tPD+)

-50

-25     0

25     50     75    100   125

0      20

40     60

80    100   120   140

1µs/div

TEMPERATURE (°C)

INPUT OVERDRIVE VOLTAGE (mV)

PROPAGATION DELAY (tPD-)

MAX9021/2/4 toc14

OUTPUT SWITCHING CURRENT, RISING

MAX9021/2/4 toc15

OUTPUT SWITCHING CURRENT, FALLING

IN+

100mV/div

VOUT

2.5V/div

IN+ - IN-

200mV/div

VOUT

5V/div

SWITCHING CURRENT

400µA/div

IN+ - IN-

200mV/div

VOUT

5V/div

SWITCHING CURRENT

400µA/div

1µs/div

20µs/div

20µs/div

10kHz  RESPONSE (VOD = 10mV)

MAX9021/2/4 toc17

10kHz  RESPONSE (VOD = 100mV)

MAX9021/2/4 toc18

POWER-UP TIME

IN+ - IN-

10mV/div

OUT

2.5V/div

IN+ - IN-

MAX9021/2/4 toc16

100mV/div

OUT

2.5V/div

VDD

2.5V/div

VOUT

2.5V/div

10µs/div

10µs/div

2µs/div

4

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

MAX9021/MAX9022/MAX9024

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX9021 | MAX9022 | MAX9024 |  |  |
| 1 | — | — | IN+ | Comparator Noninverting Input |
| 2 | 4 | 11 | VSS | Negative Supply Voltage |
| 3 | — | — | IN- | Comparator Inverting Input |
| 4 | — | — | OUT | Comparator Output |
| 5 | 8 | 4 | VDD | Positive Supply Voltage. Bypass with a 0.1µF capacitor to GND. |
| — | 1 | 1 | OUTA | Comparator A Output |
| — | 2 | 2 | INA- | Comparator A Inverting Input |
| — | 3 | 3 | INA+ | Comparator A Noninverting Input |
| — | 5 | 5 | INB+ | Comparator B Noninverting Input |
| — | 6 | 6 | INB- | Comparator B Inverting Input |
| — | 7 | 7 | OUTB | Comparator B Output |
| — | — | 8 | OUTC | Comparator C Output |
| — | — | 9 | INC- | Comparator C Inverting Input |
| — | — | 10 | INC+ | Comparator C Noninverting Input |
| — | — | 12 | IND+ | Comparator D Noninverting Input |
| — | — | 13 | IND- | Comparator D Inverting Input |
| — | — | 14 | OUTD | Comparator D Output |

Detailed Description

The MAX9021/MAX9022/MAX9024 are single/dual/ quad, low-cost, low-power comparators  that consume only 2.8µA and provide a propagation  delay, tPD, typi- cally 3µs. They have an operating-supply  voltage from

2.5V to 5.5V when operating from a single supply and from ±1.25V to ±2.75V when operating from dual power supplies.  Their common-mode  input voltage range extends from the negative supply to within 1.1V of the positive supply. Internal hysteresis ensures clean out- put switching, even with slow-moving input signals.

Applications Information

Adding Hysteresis Hysteresis extends the comparator’s  noise margin by increasing  the upper  threshold and decreasing  the lower threshold. A voltage-divider  from the compara-

tor’s output sets the trip voltage. Therefore, the trip volt- age is related to the output voltage.

These comparators  have 4mV internal hysteresis. Additional hysteresis can be generated with two resis- tors, using positive feedback (Figure 1). Use the follow- ing procedure to calculate resistor values:

1) Find the trip points of the comparator using these for- mulas:

VTH = VREF + ((VDD - VREF)R2) / (R1 + R2) VTL = VREF(1 - (R2 / (R1 + R2))

where VTH is the threshold voltage at which the com- parator switches its output from high to low as VIN rises above the trip point. VTL is the threshold volt- age at which the comparator switches its output from low to high as VIN drops below the trip point.

5

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

VDD

R1

VDD

R2

VREF

VIN

IN+

IN-

VDD

VSS

OUT

MAX9021

VIN

10kΩ

0.1µF

IN+

IN-

VDD

VSS

OUT

MAX9021

Figure 1. Additional  Hysteresis

Figure 2. Time Averaging  of the Input Signal for Data Recovery

2) The hysteresis band will be:

VHYS = VTH - VTL = VDD(R2 / (R1 + R2))

3) In this example, let VDD = 5V and VREF = 2.5V.

VTH = 2.5V + 2.5V(R2 / (R1 + R2))

and

VTL = 2.5V[(1 - (R2 / (R1 + R2))]

4) Select R2. In this example, we will choose 1kΩ.

5) Select VHYS. In this example, we will choose 50mV.

MAX9021/MAX9022/MAX9024

6) Solve for R1.

VHYS = VDD(R2 / (R1 + R2))

0.050V = 5(1000Ω/(R1 + 1000Ω)) V

where R1 ≈ 100kΩ, VTH = 2.525V, and VTL = 2.475V. The above-described  design procedure  assumes rail-

to-rail output swing. If the output is significantly loaded, the results should be corrected.

Board Layout and Bypassing Use 100nF bypass as a starting point. Minimize signal trace lengths to reduce stray capacitance.  Minimize the

capacitive  coupling  between IN- and OUT. For slow-

moving input signals (rise time > 1ms), use a 1nF

capacitor between IN+ and IN-.

Biasing for Data Recovery Digital data is often embedded  into a bandwidth  and amplitude-limited analog path. Recovering the data can

be difficult.  Figure 2 compares  the input signal to a

time-averaged  version of itself. This self-biases the threshold to the average input voltage for optimal noise margin. Even severe phase distortion is eliminated from the digital output signal. Be sure to choose R1 and C1 so that:

fCAR >> 1 / (2πR1C1)

where fCAR is the fundamental carrier frequency of the digital data stream.

6

Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators

MAX9021/MAX9022/MAX9024

Typical Application Circuit                                 Chip Information

VIN

R1

VREF

IN+

IN-

VDD

VDD

0.1µF

OUT

RL

MAX9021 TRANSISTOR COUNT: 106

MAX9022 TRANSISTOR COUNT: 212

MAX9024 TRANSISTOR COUNT: 424

R2                                                    MAX9021

SC70, 5L.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 5L SC70

1

21-0076                D      1

7

SOT-23 5L .EPS

| Micropower, Ultra-Small, Single/Dual/Quad, Single-Supply Comparators |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

MAX9021/MAX9022/MAX9024

SOT23, 8L .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Revision History

Pages changed at Rev 2: 1, 2, 6, 7, 8

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

8                                       Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2007 Maxim Integrated  Products                                                               is a registered  trademark  of Maxim Integrated  Products, Inc.