19-2840; Rev 0; 4/03

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

General Description

The MAX9812/MAX9813 are single/dual-input,  20dB fixed-gain microphone amplifiers. They offer tiny pack- aging and a low-noise, integrated microphone bias, making them ideal for portable audio applications such as notebook computers, cell phones, and PDAs. These amplifiers feature a 500kHz bandwidth,  Rail-to-Rail® outputs, an industry-leading 100dB power-supply rejec- tion ratio, and a very low 0.015% THD+N. Power-saving features include very low 230µA supply current and a total shutdown mode that cuts the combined  supply and BIAS currents to only 100nA.

The MAX9812 is a single amplifier in a 6-pin SC70 package (2mm x 2.1mm) and the MAX9813 is a dual- input amplifier available in an 8-pin SOT23 (3mm x

3mm) package. The MAX9813 has two inputs allowing two microphones to be multiplexed to a single output.

The MAX9812/MAX9813 are offered in two grades. The MAX9812L/MAX9813L are optimized for 3.3V supply operation (2.7V to 3.6V). The MAX9812H/MAX9813H are PC2001 compliant and are optimized for 5V operation (4.5V to 5.5V). Both devices are specified over the -40°C to +85°C extended operating temperature range.

Applications

Notebook Computers

PDAs

Smart Phones Car Kit Adapters Digital Cameras

Video Tape Recorders

MAX9812/MAX9813

Features

♦ PC2001 Compliant

♦ 100dB at 217Hz Power-Supply Rejection Ratio

♦ Very Low 230µA Quiescent Current

♦ Low 0.015% THD+N

♦ Available in Two Versions MAX9812L/MAX9813L—2.7V to 3.6V MAX9812H/MAX9813H—4.5V to 5.5V

♦ Internal Low-Noise Microphone Bias Supply

2.3V for MAX9812L/MAX9813L

4.0V for MAX9812H/MAX9813H

♦ 100nA Low-Power Shutdown Mode

♦ Rail-to-Rail Outputs

♦ 20dB Fixed Gain

♦ Available in Tiny 6-Pin SC70 (2mm x 2.1mm) and

8-Pin SOT23 (3mm x 3mm) Packages

♦ Extended Temperature Range -40°C to +85°C

Ordering Information

PIN- PART TEMP RANGE PACKAGE VCC (V)

MAX9812LEXT-T -40°C to +85°C 6 SC70-6 2.7 to 3.6

MAX9812HEXT-T -40°C to +85°C 6 SC70-6 4.5 to 5.5

MAX9813LEKA-T -40°C to +85°C 8 SOT23-8 2.7 to 3.6

MAX9813HEKA-T -40°C to +85°C 8 SOT23-8 4.5 to 5.5

Pin Configurations,  Selector Guide, and Typical Operating

Circuits appear at end of data sheet.

Rail-to-Rail is a trademark of Nippon Motorola Ltd.

Simplified Block Diagrams

VCC

VCC

BIAS

REFERENCE

2.3V (L VERSION)

4.0V (H VERSION)

SHDN

BIAS

REFERENCE

2.3V (L VERSION)

4.0V (H VERSION)

SHDN

IN

20dB

OUT

IN1

IN2

20dB

OUT

MAX9812

IN1/IN2

MAX9813

GND

GND

Maxim Integrated Products    1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

ABSOLUTE MAXIMUM RATINGS

(All voltages referenced  to GND)

VCC ........................................................................-0.3V to +6.0V All Other Pins..............................................-0.3V to (VCC + 0.3V) Continuous Current (IN, SHDN, IN1, IN2, IN1/IN2)...........±20mA OUT, BIAS Short-Circuit Duration (to GND or VCC) ...Continuous Continuous Power Dissipation (TA = +70°C)

6-Pin SC70 (derate 3.1mW/°C above +70°C) ..............245mW

8-Pin SOT23 (derate 8.9mw/°C above +70°C) ............714mW

Operating Temperature Range ...........................-40°C to +85°C Storage Temperature Range .............................-65°C to +150°C Junction Temperature ......................................................+150°C Lead Temperature (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = 3.3V, IBIAS = 500µA (MAX9812L/MAX9813L),  VCC = 5V, IBIAS = 800µA (MAX9812H/MAX9813H), GND = 0V, RL = open,

SHDN = VCC, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |
| Supply Voltage Range | VCC | MAX9812L/MAX9813L | Inferred from PSRR test | 2.7 3.6 | V |
|  |  | MAX9812H/MAX9813H |  | 4.5 5.5 |  |
| Supply Current | ICC | IBIAS = 0 |  | 230 400 | µA |
| Shutdown Supply Current | ICC_SHDN | SHDN = GND |  | 0.1 1 | µA |
| Amplifier Output Bias Voltage | VOUT_DC | MAX9812L/MAX9813L |  | 1.35 1.5 1.65 | V |
|  |  | MAX9812H/MAX9813H |  | 2.25 2.5 2.75 |  |
| Input Resistance | RIN |  |  | 85 | kΩ |
| Voltage Gain | AV |  |  | 19 20 21 | dB |
| Power-Supply Rejection Ratio | PSRROUT | Input referred, TA = +25°C | DC | 90 100 | dB |
|  |  |  | f = 217Hz | 100 |  |
|  |  |  | f = 1kHz | 100 |  |
|  |  |  | f = 10kHz | 90 |  |
| Output Voltage Swing | VOH | RL = 10kΩ to VCC/2 |  | VCC - 0.1 | V |
|  |  | RL = 1kΩ to VCC/2 |  | VCC - VCC - 0.25V 0.1V |  |
|  | VOL | RL = 10kΩ to VCC/2 |  | 0.1 |  |
|  |  | RL = 1kΩ to VCC/2 |  | 0.1 0.25 |  |
| Output Short-Circuit Current | IOUT_SC | Sinking or sourcing |  | 3 12 24 | mA |
| Small-Signal -3dB Bandwidth | BW | VOUT = 10mVP-P |  | 400 | kHz |
| Output Capacitive-Load Stability | CL | No sustained oscillations |  | 50 | pF |
| Output Impedance | ZOUT | f = 1kHz |  | 0.5 | Ω |
| Output Slew Rate | SR | VOUT = 1V step |  | 1 | V/µs |

2

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

ELECTRICAL CHARACTERISTICS  (continued)

(VCC = 3.3V, IBIAS = 500µA (MAX9812L/MAX9813L),  VCC = 5V, IBIAS = 800µA (MAX9812H/MAX9813H), GND = 0V, RL = open,

SHDN = VCC, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Amplifier Input Voltage-Noise Density | en | Inputs at AC GND | f = 1kHz | 40 | nV/√Hz |
| Total Integrated Input Noise | Vn | 22Hz to 22kHz BW, inputs at AC GND |  | 5 | µVRMS |
| Off-Isolation |  | Input referred, MAX9813 only | 1kHz | 75 | dB |
|  |  |  | 10kHz | 60 |  |
| Total Harmonic Distortion Plus Noise | THD+N | f = 1kHz, RL = 10kΩ to VCC/2, BW = 22Hz to 22kHz | VOUT = 1VP-P (L version) | 0.04 | % |
|  |  |  | VOUT = 4VP-P (H version) | 0.015 |  |
| BIAS |  |  |  |  |  |
| Bias Output Voltage Range | VBIAS | MAX9812L/MAX9813L |  | 2.1 2.30 2.55 | V |
|  |  | MAX9812H/MAX9813H |  | 3.6 4.0 4.4 |  |
| Bias Output Resistance | RBIAS |  |  | 0.1 | Ω |
| Power-Supply Rejection Ratio (VCC to BIAS) | PSRRBIAS | DC, TA = +25°C |  | 70 80 | dB |
|  |  | f = 217Hz |  | 80 |  |
|  |  | f = 1kHz |  | 75 |  |
|  |  | f = 10kHz |  | 55 |  |
| BIAS Current Limit | IBIAS_SC | BIAS short to GND |  | 5 22 50 | mA |
| BIAS Capacitive-Load Stability | CBIAS | No sustained oscillations |  | 50 | pF |
| Total Integrated BIAS Noise | Vn | 22Hz to 22kHz BW |  | 29 | µVRMS |
| DIGITAL INPUTS (SHDN, IN1/IN2) |  |  |  |  |  |
| Logic-Low Threshold | VIL |  |  | 0.8 | V |
| Logic-High Threshold | VIH |  |  | 2.0 | V |
| Logic Input Current | IIN | SHDN = GND or VCC |  | ±1 | µA |
| Shutdown Enable Time | tSHDN_ON | 95% of settled value |  | 10 | ms |
| Shutdown Disable Time | tSHDN_OFF |  |  | 50 | µs |
| IN1/IN2 Select Time | tSEL |  |  | 10 | µs |

Note 1: All specifications are 100% tested at TA = +25°C. Temperature limits are guaranteed  by design.

3

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

OUTPUT VOLTAGE GAIN (dB)

Typical Operating Characteristics (VCC = 3.3V (MAX9812L/MAX9813L),  VCC = 5V (MAX9812H/MAX9813H), GND = 0V, RL = 10kΩ to VCC/2, SHDN = VCC, TA = +25°C, unless otherwise noted.)

40

30

20

10

0

-10

-20

GAIN vs. FREQUENCY

MAX9812 toc01

0

-10

-20

-30

-40

-50

PSRR (dB)

-60

-70

-80

-90

-100

-110

-120

-130

-140

-150

POWER-SUPPLY REJECTION RATIO

vs. FREQUENCY

MAX9812 toc02

VRIPPLE = 200mVP-P

INPUT REFERRED

0

-10

PSRR (VCC TO BIAS) (dB)

-20

-30

-40

-50

-60

-70

-80

-90

-100

POWER-SUPPLY REJECTION RATIO (VCC TO BIAS) vs. FREQUENCY

MAX9812 toc03

VRIPPLE = 200mVP-P

| 10 100 RL = OPEN VOUT = 10mVP-P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

1k        10k

100k       1M     10M

10               100               1k

10k

100k

10               100               1k

10k

100k

FREQUENCY (Hz)

FREQUENCY (Hz)

FREQUENCY (Hz)

250

245

240

235

230

225

220

SUPPLY CURRENT (µA)

215

210

205

200

SUPPLY CURRENT

vs. TEMPERATURE

MAX9812H

MAX9812L

MAX9812 toc04

300

280

260

240

220

200

180

SUPPLY CURRENT (µA)

160

140

120

100

SUPPLY CURRENT

vs. SUPPLY VOLTAGE

MAX9812H

MAX9812 toc05

0.15

SHUTDOWN SUPPLY CURRENT (µA)

0.14

0.13

0.12

0.11

0.10

0.09

0.08

0.07

| 0.06 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

0.05

MAX9812 toc06

SHUTDOWN SUPPLY CURRENT

vs. TEMPERATURE

-40

-25

-10    5      20

35     50     65     80

2.0   2.4   2.8   3.2   3.6   4.0   4.4   4.8   5.2   5.6   6.0

-40

-25

-10    5      20

35     50     65     80

TEMPERATURE (°C)

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

2.33

2.32

2.31

2.30

VBIAS (L VERSION) (V)

2.29

2.28

VBIAS vs. TEMPERATURE

MAX9812H

MAX9812L

4.03

4.02

4.00

VBIAS (H VERSION) (V)

3.98

3.97

2.3002

2.3001

2.3000

VBIAS (L VERSION) (V)

2.2999

VBIAS vs. IBIAS

MAX9812L

MAX9812H

4.0002

VBIAS (H VERSION) (V)

4.0001

4.0000

3.9999

2.27

-40

-25

-10

5      20     35     50     65     80

TEMPERATURE (°C)

3.96

2.2997

0

3.9998

0.2   0.4   0.6   0.8   1.0   1.2   1.4   1.6   1.8   2.0

IBIAS (mA)

4

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

Typical Operating Characteristics (continued) (VCC = 3.3V (MAX9812L/MAX9813L),  VCC = 5V (MAX9812H/MAX9813H), GND = 0V, RL = 10kΩ to VCC/2, SHDN = VCC, TA = +25°C, unless otherwise noted.)

TOTAL HARMONIC DISTORTION PLUS NOISE

vs. FREQUENCY

1

MAX9812 toc09

BW = 22Hz TO 22kHz

TOTAL HARMONIC DISTORTION PLUS NOISE

vs. OUTPUT AMPLITUDE

MAX9812 toc10

10

H VERSION

BW = 22Hz TO 22kHz

THD+N (%)

0.1

VOUT = 1VRMS

RL = 2kΩ

1

THD+N (%)

0.1

f = 10kHz

0.01

0.001

VOUT = 1VRMS

RL = 10kΩ

0.01

0.001

f = 100Hz

f = 1kHz

10               100

1k

FREQUENCY (Hz)

10k

100k

0    0.2   0.4   0.6   0.8   1.0   1.2   1.4   1.6   1.8   2.0

OUTPUT AMPLITUDE (VRMS)

TOTAL HARMONIC DISTORTION PLUS NOISE

vs. OUTPUT AMPLITUDE

INPUT-REFERRED NOISE vs. FREQUENCY

L VERSION

BW = 22Hz TO 22kHz

10

1

MAX9812 toc11

f = 1kHz f = 10kHz

MAX9812 toc12

INPUTS AT AC GND

70

50

THD+N (%)

0.1

0.01

INPUT-REFERRED NOISE (nV/√Hz)

30 f = 100Hz

0.001

0

0.5

1.0

1.5

10

10               100               1k

MAX9812 toc14

10k             100k

OUTPUT AMPLITUDE (VRMS)

FREQUENCY (Hz)

10,000

1000

BIAS NOISE (nV/√Hz)

100

10

MAX9812 toc13

BIAS NOISE vs. FREQUENCY

10

0

-10

CROSSTALK (dB)

-20

-30

-40

-50

-60

-70

-80

-90

-100

OFF-ISOLATION vs. FREQUENCY

MAX9813

VIN1 = 100mVRMS

IN1 TO OUT WITH IN2 SELECTED

10               100

1k

FREQUENCY (Hz)

10k

100k

10               100

1k

FREQUENCY (Hz)

10k

100k

5

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

Typical Operating Characteristics (continued) (VCC = 3.3V (MAX9812L/MAX9813L),  VCC = 5V (MAX9812H/MAX9813H), GND = 0V, RL = 10kΩ to VCC/2, SHDN = VCC, TA = +25°C, unless otherwise noted.)

SMALL-SIGNAL  PULSE RESPONSE (RL = OPEN, CL = 10pF)

MAX9812 toc15

LARGE-SIGNAL PULSE RESPONSE (RL = OPEN, CL = 10pF)

MAX9812 toc16

IN

5mV/div

IN

100mV/div

OUT

50mV/div

OUT

1V/div

0V

1µs/div

2µs/div

SHDN

2V/div

OUT-OF-SHUTDOWN WAVEFORM INPUT FLOATING

MAX9812 toc17

1V/div

MAX9813

SWITCHING BETWEEN TWO INPUTS

MAX9812 toc18

IN2 = 200mVP-P  AT 20kHz

IN1 = 100mVP-P  AT 10kHz

OUT

BIAS

2V/div

OUT

2V/div

2V/div

IN1/IN2

1ms/div

100µs/div

OUTPUT OVERDRIVEN

MAX9812 toc19

OUT

1V/div

IN

500mV/div

2µs/div

6

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX9812L/ MAX9812H | MAX9813L/ MAX9813H |  |  |
| 1 | 3 | SHDN | Active-Low Shutdown Input. Connect SHDN to VCC for normal operation. Connect SHDN to GND for shutdown. SHDN is a high-impedance input; do not leave floating. |
| 2 | 2 | GND | Ground |
| 3 | 1 | OUT | Amplifier Output |
| 4 | 8 | VCC | Positive Supply. Bypass VCC to GND with a 0.1µF capacitor. |
| 5 | 7 | BIAS | Low-Noise Microphone Bias Output. 2.3V output for MAX9812L/MAX9813L. 4V output for MAX9812H/MAX9813H. |
| 6 | — | IN | Amplifier Input (MAX9812) |
| — | 6 | IN1 | Amplifer Input 1 (MAX9813) |
| — | 5 | IN2 | Amplifier Input 2 (MAX9813) |
| — | 4 | IN1/IN2 | Input Selector. When IN1/IN2 is high, IN1 is selected. When IN1/IN2 low, IN2 is selected. |

Detailed Description

The MAX9812_/MAX9813_ are low-power fixed-gain microphone amplifiers available in a single- or dual- input configuration. The gain is set at 10V/V (20dB) with a 400kHz, -3dB bandwidth.  They also feature a low- noise, integrated microphone input bias voltage.

Single/Dual Input The MAX9812L/MAX9812H are single-input amplifiers and the MAX9813L/MAX9813H are dual-input ampli- fiers. All devices typically have an input impedance of

85kΩ. The inputs to the dual version are controlled through a fast 2:1 mux, selectable through the IN1/IN2 pin. Driving IN1/IN2 high selects IN1 and driving the IN1/IN2 low selects IN2. IN1/IN2 is designed to be dri- ven   by   a   logic   high   of   ≥ 2V   and   a   logic   low

≤0.8V. The IN1/IN2 has a 10µs switching time from one channel to the other.

PC2001 Low-Noise Microphone BIAS The MAX9812_/MAX9813_ provide a low-noise voltage BIAS designed  for biasing electret condenser micro- phone (ECM) cartridges. The BIAS output is regulated to typically 2.3V for the MAX9812L/MAX9813L and 4V for the MAX9812H/MAX9813H. In the single-input ver- sion (MAX9812_), the BIAS output can source up to

1mA. In the dual-input version (MAX9813_), the BIAS output  can  source  up  to 2mA. The MAX9812H/ MAX9813H provides a PC2001-compliant BIAS voltage.

Output Stage The MAX9812_/MAX9813_ rail-to-rail output (OUT) typical- ly swings to within 100mV of the rails when driving 10kΩ.

The output DC bias point is set to 1.5V for the MAX9812L/ MAX9813L and 2.5V for the MAX9812H/MAX9813H.

Shutdown Mode SHDN controls whether the MAX9812_/MAX9813_ is active or in shutdown mode. Driving SHDN low forces a low-power (100nA) shutdown mode. In this mode, the OUT pin is set to a high-impedance state and the BIAS pin is pulled down (70kΩ). Driving SHDN high enables the MAX9812_/MAX9813_. SHDN is a high-impedance input and cannot be left floating.

Driving Capacitive Loads The MAX9812_/MAX9813_ output can drive up to 50pF of capacitance without sustained oscillations.

Thermal Shutdown The thermal shutdown feature protects the MAX9812_/MAX9813_ from destruction due to overheat- ing caused by shorting the outputs. This protection fea- ture causes OUT and BIAS to shut down and go high impedance when the die temperature reaches +140°C. The device restarts after the die temperature falls below

+120°C.

Applications Information

Power-Up

The MAX9812_/MAX9813_ output typically settles to

95% within 10ms after power-up.

Typical Application Circuit Figure 1 shows the MAX9813H used as a preamplifier with the MAX9760 3W audio power amplifier.

7

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

VDD

0.1µF

VDD

220kΩ

MAX9812/MAX9813

22kΩ

0.1µF

0.1µF

IN1/IN2   VCC

IN1

MAX9813

OUT

0.1µF

CODEC

HPF

1µF

0.68µF

0.68µF

15kΩ

15kΩ

VDD

BIAS SVDD

INL1

INL2

PVDD

GAINLB GAINLA OUTL+

OUTL-

0.047µF 27.4kΩ

33.2kΩ

15kΩ

220µF

10kΩ

2.2kΩ

IN2

0.68µF

15kΩ

INR1

MAX9760

2.2kΩ

0.1µF

BIAS

GND

SHDN

1kΩ

HPF

0.68µF VDD

1kΩ

15kΩ

10kΩ

INR2

OUTR-

OUTR+ GAINRA

GAINRB

15kΩ

33.2kΩ

220µF

VDD

SCL

27.4kΩ

10kΩ                        680kΩ

MICROCONTROLLER

SDA ADD INT

SHDN

HPS

0.047µF

47kΩ

Figure 1. Typical Application Circuit

8

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

Typical Operating Circuits

VCC

VCC

VBIAS

5

BIAS

4

VCC

0.1µF

VBIAS

7

BIAS

8

VCC

0.1µF

2.2kΩ

MIC

0.1µF

6

IN

SHDN

MAX9812

OUT

1

0.1µF

3

2.2kΩ               2.2kΩ

MIC1

0.1µF

0.1µF

6

IN1

5

IN2

4

SHDN

MAX9813

OUT

3

0.1µF

1

GND

2

MIC2

IN1/IN2   GND

2

Pin  Configurations

TOP VIEW

SHDN

1                                     6     IN

| OUT | MAX9813 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

VCC

GND   2

MAX9812

5     BIAS

GND SHDN

BIAS IN1

OUT   3                                    4

VCC

IN1/IN2

IN2

SC70

SOT23

| Selector Guide PART | PIN-PACKAGE | VCC (V) | TOP MARK |
| --- | --- | --- | --- |
| MAX9812LEXT-T | 6 SC70-6 | 2.7 to 3.6 | ABJ |
| MAX9812HEXT-T | 6 SC70-6 | 4.5 to 5.5 | ABK |
| MAX9813LEKA-T | 8 SOT23-8 | 2.7 to 3.6 | AEEU |
| MAX9813HEKA-T | 8 SOT23-8 | 4.5 to 5.5 | AEEV |

Chip Information

MAX9812 TRANSISTOR COUNT: 264

MAX9813 TRANSISTOR COUNT: 269

PROCESS: BiCMOS

9

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

SC70, 6L.EPS

Package Information (The package  drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package  outline information go to  www.maxim-ic.com/packages.)

10

Tiny, Low-Cost, Single/Dual-Input, Fixed-Gain

Microphone Amplifiers with Integrated Bias

MAX9812/MAX9813

SOT23, 8L .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) SYMBOL | MIN | MAX |
| --- | --- | --- |
| A | 0.90 | 1.45 |
| A1 | 0.00 | 0.15 |
| A2 | 0.90 | 1.30 |
| b | 0.28 | 0.45 |
| C | 0.09 | 0.20 |
| D | 2.80 | 3.00 |
| E | 2.60 | 3.00 |
| E1 | 1.50 | 1.75 |
| L | 0.30 | 0.60 |
| L2 | 0.25 BSC. |  |
| e | 0.65 BSC. |  |
| e1 | 1.95 REF. |  |
| 0 | 0 | 8 |

| SEE DETAIL "A" b e CL CL E CL E1 PIN 1 I.D. DOT (SEE NOTE 6) e1 D C CL L2 A A2 A1 GAUGE PLANE SEATING PLANE C L 0 NOTE: 1. ALL DIMENSIONS ARE IN MILLIMETERS. 2. FOOT LENGTH MEASURED FROM LEAD TIP TO UPPER RADIUS OF DETAIL "A" HEEL OF THE LEAD PARALLEL TO SEATING PLANE C. 3. PACKAGE OUTLINE EXCLUSIVE OF MOLD FLASH & METAL BURR. 4. PACKAGE OUTLINE INCLUSIVE OF SOLDER PLATING. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5. COPLANARITY 4 MILS. MAX. 6. PIN 1 I.D. DOT IS 0.3 MM MIN. LOCATED ABOVE PIN 1. 7. SOLDER THICKNESS MEASURED AT FLAT SECTION OF LEAD BETWEEN 0.08mm AND 0.15mm FROM LEAD TIP. 8. MEETS JEDEC MO178. | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, SOT-23, 8L BODY |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0078 | REV. D | 1 1 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600                                   11

©  2003 Maxim Integrated  Products               Printed USA                                  is a registered  trademark of Maxim Integrated  Products.