19-3406; Rev 1; 10/05

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

General Description

The single MAX9910/MAX9911 and dual MAX9912/ MAX9913 operational amplifiers (op amps) feature a maximized ratio of gain bandwidth  (GBW) to supply current and are ideal for battery-powered  applications such as portable  instrumentation, portable  medical equipment,  and wireless handsets. These CMOS op amps feature an ultra-low input-bias current of 1pA, rail- to-rail inputs and outputs, low supply current of 4µA, and operate from a single 1.8V to 5.5V supply.  For additional power conservation, the MAX9911/MAX9913 feature a low-power shutdown mode that reduces sup- ply current to 1nA, and puts the amplifiers’ outputs in a high-impedance  state. These devices  are unity-gain stable with a 200kHz GBW product.

The MAX9910/MAX9911 are available in 5-pin and

6-pin SC70 packages,  respectively.  The MAX9912 is available  in  an  8-pin  SOT23 package,  and  the MAX9913 is available in a 10-pin µMAX® package.  All devices are specified  over the -40°C to +85°C extend- ed operating temperature range.

Applications

Portable Medical Devices Portable Test Equipment Laptops

Data-Acquisition Equipment

Typical Operating Circuit

MAX9910–MAX9913

Features

♦ 200kHz GBW

♦ Ultra-Low 4µA Supply Current

♦ Single 1.8V to 5.5V Supply Voltage Range

♦ Ultra-Low 1pA Input Bias Current

♦ Rail-to-Rail Input and Output Voltage Ranges

♦ Low ±200µV Input Offset Voltage

♦ Low 0.001µA Shutdown Current

♦ High-Impedance Output During Shutdown

(MAX9911/MAX9913)

♦ Unity-Gain Stable

♦ Available in Tiny SC70, SOT23, and µMAX Packages

Ordering Information PART TEMP RANGE PIN- TOP PACKAGE MARK

MAX9910EXK+T -40°C to +85°C 5 SC70-5 AGA

MAX9910EXK-T -40°C to +85°C 5 SC70-5 AGA

MAX9911EXT+T -40°C to +85°C 6 SC70-6 ACA

MAX9911EXT-T -40°C to +85°C 6 SC70-6 ACA

MAX9912EKA+T -40°C to +85°C 8 SOT23-8 AEJY

MAX9912EKA-T -40°C to +85°C 8 SOT23-8 AEJY

MAX9913EUB -40°C to +85°C 10 µMAX —

MAX9913EUB+ -40°C to +85°C 10 µMAX —

OUTA

1.8V TO 5.5V

VDD

VMID

INA-

INA+

OUTB

INB-

VOUT

25kΩ

+Denotes  lead-free package.

ON OFF

VSS

SHDNA

MAX9913

INB+

SHDNB

VMID + VBIAS

ON

OFF

| Selector Guide PART | AMPLIFIERS PER PACKAGE | SHUTDOWN MODE |
| --- | --- | --- |
| MAX9910EXK-T | 1 | No |
| MAX9911EXT-T | 1 | Yes |
| MAX9912EKA-T | 2 | No |
| MAX9913EUB | 2 | Yes |

1MΩ

RE

CE  GLUCOSE   WE SENSOR

THREE ELECTRODE GLUCOSE METER APPLICATION WITH SHUTDOWN

µMAX is a registered trademark of Maxim Integrated Products, Inc.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

ABSOLUTE MAXIMUM RATINGS

Power-Supply Voltage (VDD to VSS) ......................-0.3V to +6.0V IN_+, IN_-, OUT_, SHDN_ ............... (VSS - 0.3V) to (VDD + 0.3V) Current into IN_+, IN_- ......................................................±20mA Output Short-Circuit  Duration to VDD or VSS ..............Continuous Continuous Power Dissipation  (TA = +70°C)

5-Pin SC70 (derate 3.1mW/°C above +70°C) ............. 247mW

6-Pin SC70 (derate 3.1mW/°C above +70°C) ............. 245mW

8-Pin SOT23 (derate 9.1mW/°C above +70°C)........... 727mW

10-Pin µMAX (derate 5.6mW/°C above +70°C) ...........444mW Operating  Temperature  Range .......................... -40°C to +85°C Junction Temperature  .....................................................+150°C Storage Temperature  Range  ............................-65°C to +150°C Lead Temperature  (soldering,  10s)  ............................... +300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VDD = 1.8V to 5.5V, VSS = 0V, VCM = 0V, VOUT = VDD / 2, RL = ∞ connected to VDD / 2, SHDN_ = VDD, TA = +25°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage Range | VDD | Guaranteed by PSRR test |  | 1.8 5.5 | V |
| Supply Current | IDD | MAX9910/MAX9911 | VDD = 1.8V | 4 | µA |
|  |  |  | VDD = 5.5V | 4 5.0 |  |
|  |  | MAX9912/MAX9913 | VDD = 1.8V | 7 |  |
|  |  |  | VDD = 5.5V | 7 9 |  |
| Shutdown Supply Current | IDD(SHDN_) | SHDN_ = GND, MAX9911/MAX9913 |  | 0.001 0.5 | µA |
| Input Offset Voltage | VOS |  |  | ±0.2 ±1 | mV |
| Input-Offset-Voltage Matching |  | MAX9912/MAX9913 |  | ±250 | µV |
| Input Bias Current | IB | (Note 2) |  | ±1 ±10 | pA |
| Input Offset Current | IOS | (Note 2) |  | ±1 ±10 | pA |
| Input Resistance | RIN | Common mode |  | 1 | GΩ |
|  |  | Differential mode, -1mV < VIN < +1mV |  | 10 |  |
| Input Common-Mode Range | VCM | Guaranteed by CMRR test |  | VSS - VDD + 0.1 0.1 | V |
| Common-Mode Rejection Ratio | CMRR | -0.1V < VCM < VDD + 0.1V, VDD = 5.5V |  | 70 80 | dB |
| Power-Supply Rejection Ratio | PSRR | 1.8V < VDD < 5.5V |  | 65 95 | dB |
| Open-Loop Gain | AVOL | 25mV < VOUT < VDD - 25mV, RL = 100kΩ, VDD = 5.5V |  | 95 120 | dB |
|  |  | 100mV < VOUT < VDD - 100mV, RL = 5kΩ, VDD = 5.5V |  | 95 110 |  |
| Output-Voltage-Swing High | VOH | VDD - VOUT | RL = 100kΩ | 2.5 5 | mV |
|  |  |  | RL = 5kΩ | 50 70 |  |
|  |  |  | RL = 1kΩ | 250 |  |
| Output-Voltage-Swing Low | VOL | VOUT - VSS | RL = 100kΩ | 2.5 5 | mV |
|  |  |  | RL = 5kΩ | 50 70 |  |
|  |  |  | RL = 1kΩ | 250 |  |
| Channel-to-Channel Isolation | CHISO | Specified at DC, MAX9912/MAX9913 |  | 100 | dB |
| Output Short-Circuit Current | IOUT(SC) |  |  | ±15 | mA |

2

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

ELECTRICAL CHARACTERISTICS (continued)

(VDD = 1.8V to 5.5V, VSS = 0V, VCM = 0V, VOUT = VDD / 2, RL = ∞ connected to VDD / 2, SHDN_ = VDD, TA = +25°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| SHDN_ Logic Low | VIL | VDD = 1.8V to 3.6V, MAX9911/MAX9913 |  | 0.4 | V |
|  |  | VDD = 3.6V to 5.5V, MAX9911/MAX9913 |  | 0.8 |  |
| SHDN_ Logic High | VIH | VDD = 1.8V to 3.6V, MAX9911/MAX9913 |  | 1.4 | V |
|  |  | VDD = 3.6V to 5.5V, MAX9911/MAX9913 |  | 2 |  |
| SHDN_ Input Bias Current | IIL | SHDN_ = VSS, MAX9911/MAX9913 (Note 2) |  | 1 | nA |
|  | IIH | SHDN_ = VDD , MAX9911/MAX9913 |  | 500 |  |
| Output Leakage in Shutdown | IOUT(SHDN_) | SHDN_ = VSS, VOUT = 0V to VDD, MAX9911/MAX9913 |  | 1 500 | nA |
| Gain-Bandwidth Product |  |  |  | 200 | kHz |
| Slew Rate |  |  |  | 0.1 | V/µs |
| Capacitive-Load Stability (See the Driving Capacitive Loads Section) | CLOAD | No sustained oscillations | AV = 1V/V | 30 | pF |
|  |  |  | AV = 10V/V | 250 |  |
|  |  |  | RL = 5kΩ, AV = 1V/V | 200 |  |
|  |  |  | RISO = 1kΩ, AV = 1V/V | 100 |  |
| Input Voltage-Noise Density |  | f = 1kHz |  | 400 | nV/√Hz |
| Input Current-Noise Density |  | f = 1kHz |  | 0.001 | pA/√Hz |
| Settling Time |  | To 0.1%, VOUT = 2V step, AV = -1V/V |  | 18 | µs |
| Delay Time to Shutdown | tSH | IDD = 5% of normal operation, VDD = 5.5V, VSHDN_ = 5.5V to 0 step |  | 2 | µs |
| Delay Time to Enable | tEN | VOUT = 2.7V, VOUT settles to 0.1%, VDD = 5.5V, VSHDN_ = 0 to 5.5V step |  | 30 | µs |
| Power-Up Time |  | VDD = 0 to 5.5V step |  | 5 | µs |

ELECTRICAL CHARACTERISTICS

(VDD = 1.8V to 5.5V, VSS = 0V, VCM = 0V, VOUT = VDD / 2, RL = ∞ connected to VDD / 2, SHDN_ = VDD, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage Range | VDD | Guaranteed by PSRR test |  | 1.8 5.5 | V |
| Supply Current | IDD | MAX9910/MAX9911 | VDD = 5.5V | 5.5 | µA |
|  |  | MAX9912/MAX9913 |  | 11 |  |
| Shutdown Supply Current | IDD(SHDN_) | SHDN_ = GND, MAX9911/MAX9913 |  | 1 | µA |
| Input Offset Voltage | VOS |  |  | ±5 | mV |
| Input-Offset-Voltage Temperature Coefficient | TCVOS |  |  | ±5 | µV/°C |

3

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

ELECTRICAL CHARACTERISTICS (continued)

(VDD = 1.8V to 5.5V, VSS = 0V, VCM = 0V, VOUT = VDD / 2, RL = ∞ connected to VDD / 2, SHDN_ = VDD, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Bias Current | IB |  |  | ±30 | pA |
| Input Offset Current | IOS |  |  | ±20 | pA |
| Input Common-Mode Range | VCM | Guaranteed by CMRR test |  | VSS - VDD + 0.05 0.05 | V |
| Common-Mode Rejection Ratio | CMRR | -0.05V < VCM < VDD + 0.05V, VDD = 5.5V |  | 60 | dB |
| Power-Supply Rejection Ratio | PSRR | 1.8V < VDD < 5.5V |  | 59 | dB |
| Open-Loop Gain | AVOL | 25mV < VOUT < VDD - 25mV, RL = 100kΩ, VDD = 5.5V |  | 85 | dB |
|  |  | 150mV < VOUT < VDD - 150mV, RL = 5kΩ, VDD = 5.5V |  | 80 |  |
| Output-Voltage-Swing High | VOH | VDD - VOUT | RL = 100kΩ | 5 | mV |
|  |  |  | RL = 5kΩ | 90 |  |
| Output-Voltage-Swing Low | VOL | VOUT - VSS | RL = 100kΩ | 5 | mV |
|  |  |  | RL = 5kΩ | 90 |  |
| SHDN_ Logic Low | VIL | VDD = 1.8V to 3.6V |  | 0.4 | V |
|  |  | VDD = 3.6V to 5.5V |  | 0.8 |  |
| SHDN_ Logic High | VIH | VDD = 1.8V to 3.6V, MAX9911/MAX9913 |  | 1.4 | V |
|  |  | VDD = 3.6V to 5.5V, MAX9911/MAX9913 |  | 2 |  |
| SHDN_ Input-Bias Current | IIL | SHDN_ = VSS, MAX9911/MAX9913 |  | 5 | nA |
|  | IIH | SHDN_ = VDD, MAX9911/MAX9913 |  | 1000 | nA |
| Output Leakage in Shutdown | IOUT(SHDN_) | SHDN_ = VSS, VOUT = 0V to VDD, MAX9911/MAX9913 |  | 1000 | nA |

Note 1: Specifications are 100% tested at TA = +25°C (exceptions noted). All temperature  limits are guaranteed by design.

Note 2: Guaranteed  by design, not production tested.

4

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

Typical Operating Characteristics

(VDD = 3V, VSS = VCM = 0V, RL to VDD / 2, TA = +25°C, unless otherwise noted.)

9

8     DUAL

7

SUPPLY CURRENT

vs. SUPPLY VOLTAGE

MAX9910 toc01

TA = -40°C

MAX9910 toc02

SHUTDOWN SUPPLY CURRENT

vs. TEMPERATURE

12

10

8

1000

750

500

INPUT OFFSET VOLTAGE

MAX9910 toc03

| vs. INPUT COMMON-MODE VOLTAGE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TA = -40°C |  |  |
|  |  | TA = +25°C |  |  |  |  |
|  | TA = +85°C |  |  |  |  |  |

ICC (µA)

6                       TA = +85°C

5

TA = +25°C

SHUTDOWN SUPPLY CURRENT (nA)

TA = -40°C                          6

VOS (µV)

250

0

4     SINGLE

3

2

TA = +25°C

4

TA = +85°C                                      2

-250

-500

-750

1

1.8

2.6         3.4

4.2

5.0

5.5

0

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |

-15         10         35

60         85

-1000

0

0.5      1.0      1.5

2.0

2.5

3.0

VSUPPLY (V)

TEMPERATURE (°C)

VCM (V)

200

150

100

50

VOS (µV)

0

-150

-200

-250

-300

-350

MAX9910 toc04

INPUT OFFSET VOLTAGE

vs. TEMPERATURE

INPUT BIAS CURRENT

vs. TEMPERATURE

7

6

5

4

IB (pA)

3                    IB-

2

1

0                     IB+

-1

-2

-3

INPUT BIAS CURRENT

MAX9910 toc06

| vs. INPUT COMMON-MODE VOLTAGE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

MAX9910 toc05

5

4

3

2

IB (pA)

1

0

-1

-2

-3

-4

-5

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |

-15         10         35         60         85

-40

-15         10         35

60         85

-0.5    0

| 0.5 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1.5   2.0

2.5

3.0

3.5

TEMPERATURE (°C)

TEMPERATURE (°C)

VCM (V)

0

-20

-40

PSRR (dB)

-60

-80

-100

-120

MAX9910 toc07

POWER-SUPPLY REJECTION RATIO

vs. FREQUENCY

140

120

100

80

CMRR (dB)

60

40

20

0

MAX9910 toc08

COMMON-MODE REJECTION RATIO

vs. TEMPERATURE

0

-20

-30

CMRR (dB)

-40

-50

-60

-70

-90

-100

MAX9910 toc09

COMMON-MODE REJECTION RATIO

vs. FREQUENCY

0.1         1

10        100        1k

10k

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |

-15         10         35         60         85

1          10

100        1k

10k

100k

FREQUENCY (Hz)

TEMPERATURE (°C)

FREQUENCY (Hz)

5

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

SLEW RATE (V/µS)

Typical Operating Characteristics (continued)

(VDD = 3V, VSS = VCM = 0V, RL to VDD / 2, TA = +25°C, unless otherwise noted.)

0.6

0.5

0.4

0.3

0.2

0.1

0

-0.1

-0.2

-0.3

-0.4

SLEW RATE

MAX9910 toc10

vs. SUPPLY VOLTAGE

1000

100

OUTPUT VOLTAGE SWING (mV)

10

1

OUTPUT-SWING HIGH

vs. TEMPERATURE

MAX9910 toc11

RL = 1kΩ

RL = 5kΩ

RL = 100kΩ

RL TO VSS

VOH = VDD - VOUT

10000

1000

100

OUTPUT VOLTAGE SWING (mV)

10

1

OUTPUT-SWING LOW

vs. TEMPERATURE

VOL = VOUT - VSS

RL TO VDD

MAX9910 toc12

RL = 1kΩ

RL = 5kΩ

RL = 100kΩ

| 1.8 |  |  |  |  |
| --- | --- | --- | --- | --- |

2.6

3.4

4.2

5.0

5.5

-40

MAX9910 toc14

-15         10         35         60         85

-40

-15         10         35

60         85

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

| 140 |  |  | RL = 100kΩ |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RL = 5kΩ |  |  |
|  |  |  | RL = 1kΩ |  |  |

OPEN-LOOP GAIN

MAX9910 toc13

vs. TEMPERATURE (RL TO VSS)

140

OPEN-LOOP GAIN

vs. TEMPERATURE (RL TO VDD)

GAIN AND PHASE

vs. FREQUENCY (RL = ∞, CLOAD = 15pF)

80                                                         MAX9910 toc15

180

120

100

80

AVOL (dB)

60

40

20

120

100

80

AVOL (dB)

60

40

20

RL = 100kΩ

RL = 1kΩ

RL = 5kΩ

60

40

GAIN (dB)

20

0

-20

-40

-60

GAIN

PHASE

AV = 1000V/V

135

90

45

0

PHASE (DEGREES)

-45

-90

-135

0

-40

-15         10         35         60         85

0

-40

-15         10         35         60         85

-80

1

10      100      1k

10k

100k

-180

1M

TEMPERATURE (°C)

TEMPERATURE (°C)

FREQUENCY (Hz)

GAIN AND PHASE

vs. FREQUENCY (RL = 5kΩ, CLOAD = 100pF)

MAX9910 toc16

CROSSTALK

vs. FREQUENCY

MAX9910 toc18

TOTAL HARMONIC DISTORTION PLUS NOISE vs. FREQUENCY

100

80

60

40

GAIN (dB)

20

0

-20

-40

-60

-80

-100

GAIN

PHASE

AV = 1000V/V

225

180

135

PHASE (DEGREES)

90

45

0

-45

-90

-135

-180

-225

0

-20

CROSSTALK (dB)

-40

-60

-80

-100

-120

MAX9912/MAX9913

MAX9910 toc17

-5

-15

-25

THD+N (dB)

-35

-45

-55

-65

-85

-95

VOUT = 2VP-P

1        10      100      1k

10k

100k       1M

10            100            1k

10k

100k                     10

100                 1k

10k

FREQUENCY (Hz)

FREQUENCY (Hz)

FREQUENCY (Hz)

6

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

Typical Operating Characteristics (continued)

(VDD = 3V, VSS = VCM = 0V, RL to VDD / 2, TA = +25°C, unless otherwise noted.)

10,000

MAX9910 toc19

VOLTAGE-NOISE DENSITY

vs. FREQUENCY

8000

7000

6000

RISO (Ω)

5000

RESISTOR ISOLATION

vs. CAPACITIVE LOAD

AV = 1V/V

FOR AV = 10V/V

MAX9910 toc20

NO RISO NEEDED

VDD

1V/div

POWER-UP SETTLING TIME

VOLTAGE NOISE (nV/√Hz)

1000

4000

3000

2000

1000

OUTPUT

500mV/div

IN+ = VDD / 2

100

1

10        100        1k           10k

FREQUENCY (Hz)

100k

0

10                 100

CLOAD (pF)

1000

10,000

500ns/div

SHUTDOWN RESPONSE

MAX9910 toc22

SMALL-SIGNAL  PULSE RESPONSE (CLOAD = 15pF)

MAX9910 toc23

SMALL-SIGNAL  PULSE RESPONSE (CLOAD = 100pF)

MAX9910 toc24

SHDN

OUTPUT

IN+ = VDD / 2

AV = 1V/V

3V

0V

1.5V

0V

IN+

50mV/div

OUTPUT

50mV/div

AV = 1V/V

IN+

5mV/div

OUTPUT

50mV/div

AV = 10V/V

20µs/div

10µs/div

20µs/div

LARGE-SIGNAL PULSE RESPONSE (CLOAD = 15pF)

MAX9910 toc25

AV = 1V/V

LARGE-SIGNAL PULSE RESPONSE (CLOAD = 100pF)

MAX9910 toc26

AV = 10V/V

IN+

1V/div

IN+

100mV/div

OUTPUT

1V/div

OUTPUT

1V/div

10µs/div

100µs/div

7

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

Typical Operating Characteristics (continued)

(VDD = 3V, VSS = VCM = 0V, RL to VDD / 2, TA = +25°C, unless otherwise noted.)

8

6

4

GAIN (dB)

2

0

-2

-4

-6

-8

-10

SMALL-SIGNAL GAIN

vs. FREQUENCY

AV = +1V/V

MAX9910 toc27

VOUT = 100mVP-P

CLOAD = 15pF

LARGE-SIGNAL GAIN

vs. FREQUENCY

AV = +1V/V

3    VOUT = 2VP-P

MAX9910 toc28

CLOAD = 15pF

2

1

0

GAIN (dB)

-1

-2

-3

-4

1                   10

100

1000

1                               10

100

FREQUENCY (kHz)

FREQUENCY (kHz)

4.0

3.5

3.0

2.5

2.0

OVERSHOOT (%)

1.5

1.0

0.5

PERCENT OVERSHOOT

vs. CAPACITIVE  LOAD

AV = 1V/V

MAX9910 toc29

RL = 1MΩ

RL = 100kΩ

RL = 10kΩ

IN+

50mV/div

OUTPUT

50mV/div

OUTPUT WAVEFORM WITH RISO

(CLOAD = 100pF,  RISO = 6.2kΩ)

MAX9910 toc30

AV = 1V/V

0

0          20         40         60

80        100

20µs/div

CLOAD (pF)

OUTPUT WAVEFORM WITHOUT RISO

(CLOAD = 100pF)

MAX9910 toc31

AV = 1V/V

IN+

50mV/div

OUTPUT

50mV/div

20µs/div

8

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

Pin  Description

| PIN |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- |
| MAX9910 | MAX9911 | MAX9912 | MAX9913 |  |  |
| 1 | 1 | — | — | IN+ | Noninverting Amplifier Input |
| 2 | 2 | 4 | 4 | VSS | Negative Supply Voltage |
| 3 | 3 | — | — | IN- | Inverting Amplifier Input |
| 4 | 4 | — | — | OUT | Amplifier Output |
| 5 | 6 | 8 | 10 | VDD | Positive Supply Voltage |
| — | 5 | — | — | SHDN | Shutdown |
| — | — | 1 | 1 | OUTA | Amplifier Output Channel A |
| — | — | 2 | 2 | INA- | Inverting Amplifier Input Channel A |
| — | — | 3 | 3 | INA+ | Noninverting Amplifier Input Channel A |
| — | — | — | 5 | SHDNA | Shutdown Channel A |
| — | — | — | 6 | SHDNB | Shutdown Channel B |
| — | — | 5 | 7 | INB+ | Noninverting Amplifier Input Channel B |
| — | — | 6 | 8 | INB- | Inverting Amplifier Input Channel B |
| — | — | 7 | 9 | OUTB | Amplifier Output Channel B |

Detailed Description

Featuring a maximized ratio of GBW to supply current, low operating  supply voltage, low input bias current, and rail-to-rail inputs  and outputs, the MAX9910– MAX9913 are an excellent choice for precision or gen- eral-purpose, low-current, low-voltage, battery-powered applications.  These CMOS devices  consume an ultra- low 4µA (typ) supply current and a 200µV (typ) offset voltage.  For additional  power  conservation,  the MAX9911/MAX9913 feature a low-power  shutdown mode that reduces  supply  current to 1nA (typ), and puts the amplifiers’ output in a high-impedance  state. These devices  are unity-gain stable with a 200kHz GBW product, driving capacitive loads up to 30pF. The capacitive  load can be increased  to 250pF when the amplifier is configured for a 10V/V gain.

Rail-to-Rail Inputs and Outputs All of the MAX9910–MAX9913 amplifiers have a parallel- connected n- and p-channel differential input stage that allows an input common-mode  voltage range that extends 100mV beyond the positive and negative sup- ply rails, with excellent common-mode rejection.

The MAX9910–MAX9913 are capable of driving the out- put to within 5mV of both supply  rails with a 100kΩ load. These devices can drive a 5kΩ load with swings to within 60mV of the rails. Figure 1 shows the output voltage swing of the MAX9910–MAX9913 configured as a unity-gain buffer powered from a single 3V supply.

Low Input Bias Current The MAX9910–MAX9913 feature ultra-low 1pA (typ) input bias current. The variation in the input bias current is minimal with changes in the input voltage due to very high input impedance (in the order of 1GΩ).

Applications Information

Driving Capacitive Loads The MAX9910–MAX9913 amplifiers are unity-gain sta- ble for loads up to 30pF. However, the capacitive load can be increased to 250pF when the amplifier is config- ured for a minimum gain of 10V/V. Applications  that require greater capacitive-drive  capability  should use an isolation resistor between the output and the capaci- tive load (Figure 2). Also, in unity-gain applications with relatively small RL (approximately 5kΩ), the capacitive load can be increased up to 200pF.

9

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

Power-Supply Considerations The MAX9910–MAX9913 are optimized for single 1.8V to 5.5V supply operation. A high amplifier power-supply rejection ratio of 95dB (typ) allows the devices  to be powered directly from a battery, simplifying design and extending battery life.

Power-Up Settling Time The MAX9910–MAX9913 typically  require 5µs after power-up. Supply settling time depends  on the supply voltage, the value of the bypass capacitor,  the output impedance of the incoming supply, and any lead resis- tance or inductance  between components.  Op-amp settling time depends  primarily on the output voltage and is slew-rate limited. Figure 3 shows the MAX991_ in a noninverting voltage follower configuration  with the input held at midsupply.  The output settles in approxi- mately 18µs for VDD = 3V (see the Typical Operating Characteristics for power-up settling time).

Shutdown Mode The MAX9911/MAX9913 feature active-low shutdown inputs. The MAX9911/MAX9913 enter shutdown in 2µs (typ) and exit in 30µs (typ). The amplifiers’ outputs are in a high-impedance  state in shutdown mode. Drive SHDN low to enter shutdown.  Drive SHDN high to enable the amplifier. The MAX9913 dual-amplifier fea- tures separate shutdown inputs. Shut down both ampli- fiers for the lowest quiescent current.

Power-Supply Bypassing and Layout

RAIL-TO-RAIL OUTPUT VOLTAGE RANGE

3V

0V

3V

0V

200µs/div

Figure 1. Rail-to-Rail Output Voltage Range

RISO

MAX9910– MAX9913

RL

RL

IN_

1V/div

OUT_

1V/div

CL

To minimize noise, bypass VDD with a 0.1µF capacitor to ground, as close to the pin as possible.

Good  layout  techniques  optimize  performance by decreasing  the amount of stray capacitance  and inductance  to the op  amps’  inputs  and  outputs. Minimize stray capacitance  and inductance by placing external components close to the IC.

AV =  RL + RISO  ≈ 1V/V

Figure 2. Using a Resistor to Isolate a Capacitive  Load from the Op Amp

5.5V

0V

100kΩ

IN-

VDD

IN+

MAX991_

VSS

OUT

100kΩ

Figure 3. Power-Up Test Configuration

10

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

Pin  Configurations

TOP VIEW

MAX9910

MAX9911

IN+

1                              5   VDD

IN+     1

SHDN

6   VDD

VSS    2

VSS     2

5   SHDN

IN-   3                              4

OUT

IN-   3                              4

OUT

SC70

SC70

MAX9912

MAX9913

OUTA    1

8   VDD

OUTA     1

10  VDD

INA-  2

7   OUTB

INA-   2

9   OUTB

INA+    3

6   INB-

INA+    3

VSS     4

SHDNA

8   INB-

7   INB+

VSS    4

5   INB+

SHDNA    5

SHDNB

6   SHDNB

SOT23

µMAX

Chip Information

MAX9910 TRANSISTOR COUNT: 180

MAX9911 TRANSISTOR COUNT: 180

MAX9912 TRANSISTOR COUNT: 292

MAX9913 TRANSISTOR COUNT: 292

PROCESS: BiCMOS

11

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

SC70, 5L.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

PACKAGE  OUTLINE, 5L SC70

1

21-0076                C      1

12

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

SC70, 6L.EPS

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE,  6L SC70

1

21-0077                C      1

13

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

SOT23, 8L .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) SYMBOL | MIN | MAX |
| --- | --- | --- |
| A | 0.90 | 1.45 |
| A1 | 0.00 | 0.15 |
| A2 | 0.90 | 1.30 |
| b | 0.28 | 0.45 |
| C | 0.09 | 0.20 |
| D | 2.80 | 3.00 |
| E | 2.60 | 3.00 |
| E1 | 1.50 | 1.75 |
| L | 0.30 | 0.60 |
| L2 | 0.25 BSC. |  |
| e | 0.65 BSC. |  |
| e1 | 1.95 REF. |  |
| 0 | 0∞ | 8∞ |

| SEE DETAIL "A" b e CL CL E CL E1 PIN 1 I.D. DOT (SEE NOTE 6) e1 D C CL L2 A A2 A1 GAUGE PLANE SEATING PLANE C L 0 NOTE: 1. ALL DIMENSIONS ARE IN MILLIMETERS. 2. FOOT LENGTH MEASURED FROM LEAD TIP TO UPPER RADIUS OF DETAIL "A" HEEL OF THE LEAD PARALLEL TO SEATING PLANE C. 3. PACKAGE OUTLINE EXCLUSIVE OF MOLD FLASH & METAL BURR. 4. PACKAGE OUTLINE INCLUSIVE OF SOLDER PLATING. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5. COPLANARITY 4 MILS. MAX. 6. PIN 1 I.D. DOT IS 0.3 MM ÿ MIN. LOCATED ABOVE PIN 1. 7. SOLDER THICKNESS MEASURED AT FLAT SECTION OF LEAD BETWEEN 0.08mm AND 0.15mm FROM LEAD TIP. 8. MEETS JEDEC MO178. | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, SOT-23, 8L BODY |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0078 | REV. D | 1 1 |

14

200kHz, 4µA, Rail-to-Rail

I/O Op Amps with Shutdown

MAX9910–MAX9913

10LUMAX.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| D1 | 0.116 | 0.120 | 2.95 | 3.05 |
| D2 | 0.114 | 0.118 | 2.89 | 3.00 |
| E1 | 0.116 | 0.120 | 2.95 | 3.05 |
| E2 | 0.114 | 0.118 | 2.89 | 3.00 |
| H | 0.187 | 0.199 | 4.75 | 5.05 |
| L | 0.0157 | 0.0275 | 0.40 | 0.70 |
| L1 | 0.037 REF |  | 0.940 REF |  |
| b | 0.007 | 0.0106 | 0.177 | 0.270 |
| e | 0.0197 BSC |  | 0.500 BSC |  |
| c | 0.0035 | 0.0078 | 0.090 | 0.200 |
| S | 0.0196 REF |  | 0.498 REF |  |
| α | 0° | 6° | 0° | 6° |

| e 4X S 10 10 H Ø0.50±0.1 0.6±0.1 1 1 0.6±0.1 TOP VIEW BOTTOM VIEW D2 E2 GAGE PLANE A2 A c b α E1 L A1 D1 L1 FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, 10L uMAX/uSOP |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0061 | REV. I | 1 1 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600                                15

©  2005 Maxim Integrated  Products               Printed USA                           is a registered  trademark  of Maxim Integrated  Products, Inc.