ADC121S101/ADC101S101/ADC081S101 1MSPS, 12-/10-/8-Bit A/D Converters in SOT-23 & LLP

January 2005

ADC121S101/ADC101S101/ADC081S101

1MSPS, 12-/10-/8-Bit A/D Converters in SOT-23 & LLP

General Description

The ADC121S101, ADC101S101, and ADC081S101 are low power, monolithic CMOS 12-, 10- and 8-bit analog-to-digital converters that operate at 1 MSPS. Each device is based on a successive  approximation  register architecture  with inter- nal track-and-hold.  The  serial  interface  is compatible  with several standards, such as SPI™, QSPI™, MICROWIRE™, and many common DSP serial interfaces.

The ADC121S101/101S101/081S101 uses the supply volt- age as a reference. This enables the devices to operate with a full-scale input range of 0 to VDD. The conversion  rate is determined from the serial clock (SCLK) speed. These con- verters offer a shutdown mode, which can be used to trade throughput   for   power   consumption.   The   ADC121S101/

101S101/081S101   are  operated  with  a single  supply  that can range from +2.7V to +5.25V. Normal power consumption during continuous conversion, using a +3V or +5V supply, is

2 mW  or  10  mW,  respectively.  The  power  down  feature, which  is  enabled  by  a chip  select  (CS)  pin,  reduces  the power consumption  to under 5 µW using a +5V supply. All three converters are available in a 6-lead SOT-23 package, which provides an extremely small footprint for applications where space is a critical consideration. The ADC081S101 is also available in a 6-lead LLP package. These products are designed for operation over the industrial temperature range of  −40˚C  to  +85˚C,  with  some  parameters   specified  to

+125˚C for the ADC121S101.

Features

n  Variable power management

n  Packaged in 6-lead SOT-23 (ADC081S101  also available in a 6-Lead LLP package)

n  Power supply used as reference

n  Single +2.7V to +5.25V supply operation

n  SPI™/QSPI™/MICROWIRE™/DSP compatible

Key Specifications

n  Resolution with no Missing Codes                   12/10/8 bits n  Conversion Rate                                                      1 MSPS n  DNL (ADC121S101)                             +0.5, -0.3 LSB (typ) n  INL (ADC121S101)                                      ± 0.4 LSB (typ) n  Power Consumption

— 3V Supply                                                       2 mW (typ)

— 5V Supply                                                     10 mW (typ)

Applications

n  Automotive Navigation

n  FA/ATM Equipment

n  Portable Systems

n  Medical Instruments

n  Mobile Communications

n  Instrumentation  and Control Systems

Connection Diagram

20110201

MICROWIRE™ is a trademark  of National  Semiconductor Corporation. TRI-STATE®  is a trademark  of National  Semiconductor Corporation. QSPI™  and SPI™  are trademarks of Motorola,  Inc.

© 2005 National Semiconductor Corporation       DS201102                                                                                                                     www.national.com

| Ordering Information Pin Descriptions |  |  |
| --- | --- | --- |
| Pin No. | Symbol | Description |
| ANALOG I/O |  |  |
| 3 | VIN | Analog input. This signal can range from 0V to VDD. |
| DIGITAL I/O |  |  |
| 4 | SCLK | Digital clock input. The range of frequencies for this input is 10 kHz to 20 MHz, with guaranteed performance at 20 MHz. This clock directly controls the conversion and readout processes. |
| 5 | SDATA | Digital data output. The output words are clocked out of this pin by the SCLK pin. |
| 6 | CS | Chip select. A conversion process begins on the falling edge of CS. |
| POWER SUPPLY |  |  |
| 1 | VDD | Positive supply pin. These pins should be connected to a quiet +2.7V to +5.25V source and bypassed to GND with 0.1 µF and 1 µF monolithic capacitors located within 1 cm of the power pin. The ADC121S101/101S101/081S101 uses this power supply as a reference, so it should be thoroughly bypassed. |
| 2 | GND | The ground return for the supply. |
| Block Diagram 20110218 |  |  |

ADC121S101/ADC101S101/ADC081S101

| www.national.com 2 Order Code | Temperature Range | Description | Top Mark |
| --- | --- | --- | --- |
| ADC121S101CIMF | −40˚C to +125˚C | 6-Lead SOT-23 Package | X01C |
| ADC101S101CIMF | −40˚C to +85˚C | 6-Lead SOT-23 Package | X02C |
| ADC081S101CIMF | −40˚C to +85˚C | 6-Lead SOT-23 Package | X03C |
| ADC121S101CIMFX | −40˚C to +125˚C | 6-Lead SOT-23 Package, Tape & Reel | X01C |
| ADC101S101CIMFX | −40˚C to +85˚C | 6-Lead SOT-23 Package, Tape & Reel | X02C |
| ADC081S101CIMFX | −40˚C to +85˚C | 6-Lead SOT-23 Package, Tape & Reel | X03C |
| ADC081S101CISDX | −40˚C to +85˚C | 6-Lead LLP Package, Tape & Reel | X3C |
| ADC081S101CISD | −40˚C to +85˚C | 6-Lead LLP Package, Tape & Partial Reel | X3C |
| ADC121S101EVAL |  | SOT-23 Evaluation Board |  |
| ADC101S101EVAL |  | SOT-23 Evaluation Board |  |
| ADC081S101EVAL |  | SOT-23 Evaluation Board |  |

| Absolute Maximum Ratings Operating Ratings (Note 2) (Notes 1, 2) Operating Temperature Range If Military/Aerospace specified devices are required, ADC121S101 −40˚C ≤ TA ≤ +125˚C please contact the National Semiconductor Sales Office/ ADC101S101 & ADC081S101 −40˚C ≤ TA ≤ +85˚C Distributors for availability and specifications. V Supply Voltage +2.7V to +5.25V DD Supply Voltage VDD −0.3V to +6.5V Digital Input Pins Voltage Range Voltage on Any Analog Pin to GND −0.3V to VDD +0.3V (Note 6) +2.7V to +5.25V Voltage on Any Digital Pin to GND -0.3V to 6.5V Input Current at Any Pin (Note 5) ±10 mA Package Thermal Resistance ESD Susceptibility Human Body Model 3500V Machine Model 200V Soldering Temperature, Infrared, 10 seconds 215˚C Junction Temperature +150˚C Storage Temperature −65˚C to +150˚C Soldering process must comply with National Semiconductor’s Reflow Temperature Profile specifications. Refer to www.national.com/packaging. (Note 4) ADC121S101 Converter Electrical Characteristics The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, fSAMPLE = 1 MSPS unless otherwise noted. Bold- face limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typical | Limits | Units |
| STATIC CONVERTER CHARACTERISTICS (VDD = 2.7V to 3.6V) |  |  |  |  |  |
|  | Resolution with No Missing Codes | −40˚C ≤ TA ≤ 125˚C |  | 12 | Bits |
| INL | Integral Non-Linearity | −40˚C ≤ TA ≤ 85˚C | ±0.4 | ±1 | LSB (max) |
|  |  | TA = 125˚C |  | +1 -1.1 | LSB (min) LSB (max) |
| DNL | Differential Non-Linearity | −40˚C ≤ TA ≤ 85˚C | +0.5 -0.3 | +1 -0.9 | LSB (max) LSB (min) |
|  |  | TA = 125˚C |  | ±1 | LSB (max) |
| VOFF | Offset Error | −40˚C ≤ TA ≤ 125˚C | ±0.1 | ±1.2 | LSB (max) |
| GE | Gain Error | −40˚C ≤ TA ≤ 125˚C | ±0.2 | ±1.2 | LSB (max) |
| DYNAMIC CONVERTER CHARACTERISTICS (fIN = 100 kHz, -0.02 dBFS sine wave unless otherwise noted) |  |  |  |  |  |
| SINAD | Signal-to-Noise Plus Distortion Ratio | −40˚C ≤ TA ≤ 125˚C | 72 | 70 | dB (min) |
| SNR | Signal-to-Noise Ratio | −40˚C ≤ TA ≤ 85˚C | 72.5 | 70.8 | dB (min) |
|  |  | TA = 125˚C |  | 70.6 | dB (min) |
| THD | Total Harmonic Distortion |  | -80 |  | dB |
| SFDR | Spurious-Free Dynamic Range |  | 82 |  | dB |
| IMD | Intermodulation Distortion, Second Order Terms | fa = 103.5 kHz, fb = 113.5 kHz | -78 |  | dB |
|  | Intermodulation Distortion, Third Order Terms | fa = 103.5 kHz, fb = 113.5 kHz | -78 |  | dB |
| FPBW | -3 dB Full Power Bandwidth | +5V Supply | 11 |  | MHz |
|  |  | +3V Supply | 8 |  | MHz |
| POWER SUPPLY CHARACTERISTICS |  |  |  |  |  |
| VDD | Supply Voltage | −40˚C ≤ TA ≤ 125˚C |  | 2.7 5.25 | V (min) V (max) |

ADC121S101/ADC101S101/ADC081S101

| 3 www.national.com Package | θJA |
| --- | --- |
| 6-Lead SOT-23 | 265˚C / W |
| 6-Lead LLP | 94˚C / W |

| ADC121S101 Converter Electrical Characteristics (Continued) The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, fSAMPLE = 1 MSPS unless otherwise noted. Bold- face limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typical | Limits | Units |
| POWER SUPPLY CHARACTERISTICS |  |  |  |  |  |
| IDD | Normal Mode (Static) | VDD = +4.75V to +5.25V, SCLK On or Off | 2 |  | mA |
|  |  | VDD = +2.7V to +3.6V, SCLK On or Off | 1 |  | mA |
|  | Normal Mode (Operational) | VDD = +4.75V to +5.25V, fSAMPLE = 1 MSPS | 2.0 | 3.2 | mA (max) |
|  |  | VDD = +2.7V to +3.6V, fSAMPLE = 1 MSPS | 0.6 | 1.5 | mA (max) |
|  | Shutdown Mode | VDD = +5V, SCLK Off | 0.5 |  | µA |
|  |  | VDD = +5V, SCLK On | 60 |  | µA |
| PD | Power Consumption, Normal Mode (Operational) | VDD = +5V, fSAMPLE = 1 MSPS | 10 | 16 | mW (max) |
|  |  | VDD = +3V, fSAMPLE = 1 MSPS | 2 | 4.5 | mW (max) |
|  | Power Consumption, Shutdown Mode | VDD = +5V, SCLK Off | 2.5 |  | µW |
|  |  | VDD = +3V, SCLK Off | 1.5 |  | µW |
| ANALOG INPUT CHARACTERISTICS |  |  |  |  |  |
| VIN | Input Range |  | 0 to VDD |  | V |
| IDCL | DC Leakage Current |  |  | ±1 | µA (max) |
| CINA | Input Capacitance (Note 3) |  | 30 |  | pF |
| DIGITAL INPUT CHARACTERISTICS |  |  |  |  |  |
| VIH | Input High Voltage |  |  | 2.4 | V (min) |
| VIL | Input Low Voltage | VDD = +5V |  | 0.8 | V (max) |
|  |  | VDD = +3V |  | 0.4 | V (max) |
| IIN | Input Current | VIN = 0V or VDD | ±10 nA | ±1 | µA (max) |
| CIND | Input Capacitance (Note 3) |  | 2 | 4 | pF (max) |
| DIGITAL OUTPUT CHARACTERISTICS |  |  |  |  |  |
| VOH | Output High Voltage | ISOURCE = 200 µA, VDD = +2.7V to +5.25V |  | VDD −0.2 | V (min) |
| VOL | Output Low Voltage | ISINK = 200 µA |  | 0.4 | V (max) |
| IOL | TRI-STATE Leakage Current |  |  | ±10 | µA (max) |
| COUT | TRI-STATE Output Capacitance |  | 2 | 4 | pF (max) |
|  | Output Coding |  | Straight (Natural) Binary |  |  |
| AC ELECTRICAL CHARACTERISTICS |  |  |  |  |  |
| fSCLK | Clock Frequency |  |  | 20 | MHz (max) |
| DC | SCLK Duty Cycle |  |  | 40 60 | % (min) % (max) |
| tTH | Track/Hold Acquisition Time |  |  | 400 | ns (max) |
| fRATE | Throughput Rate | See Serial Interface Section |  | 1 | MSPS (max) |
| tAD | Aperture Delay |  | 3 |  | ns |
| tAJ | Aperture Jitter |  | 30 |  | ps |

ADC121S101/ADC101S101/ADC081S101

www.national.com                                                                                                   4

| ADC101S101 Converter Electrical Characteristics The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, fSAMPLE = 1 MSPS unless otherwise noted. Bold- face limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typical | Limits | Units |
| STATIC CONVERTER CHARACTERISTICS |  |  |  |  |  |
|  | Resolution with No Missing Codes |  |  | 10 | Bits |
| INL | Integral Non-Linearity |  | ±0.2 | ±0.7 | LSB (max) |
| DNL | Differential Non-Linearity |  | +0.3 -0.2 | ±0.7 | LSB (max) LSB (max) |
| VOFF | Offset Error |  | ±0.1 | ±0.7 | LSB (max) |
| GE | Gain Error |  | ±0.2 | ±1 | LSB (max) |
| DYNAMIC CONVERTER CHARACTERISTICS |  |  |  |  |  |
| SINAD | Signal-to-Noise Plus Distortion Ratio | fIN = 100 kHz | 61.7 | 61 | dBFS (min) |
| SNR | Signal-to-Noise Ratio | fIN = 100 kHz | 62 |  | dB |
| THD | Total Harmonic Distortion | fIN = 100 kHz | -77 | -73 | dB (max) |
| SFDR | Spurious-Free Dynamic Range | fIN = 100 kHz | 78 | 74 | dB (min) |
| IMD | Intermodulation Distortion, Second Order Terms | fa = 103.5 kHz, fb = 113.5 kHz | -78 |  | dB |
|  | Intermodulation Distortion, Third Order Terms | fa = 103.5 kHz, fb = 113.5 kHz | -78 |  | dB |
| FPBW | -3 dB Full Power Bandwidth | +5V Supply | 11 |  | MHz |
|  |  | +3V Supply | 8 |  | MHz |
| POWER SUPPLY CHARACTERISTICS |  |  |  |  |  |
| VDD | Supply Voltage |  |  | 2.7 5.25 | V (min) V (max) |
| IDD | Normal Mode (Static) | VDD = +4.75V to +5.25V, SCLK On or Off | 2 |  | mA |
|  |  | VDD = +2.7V to +3.6V, SCLK On or Off | 1 |  | mA |
|  | Normal Mode (Operational) | VDD = +4.75V to +5.25V, fSAMPLE = 1 MSPS | 2.0 | 3.2 | mA (max) |
|  |  | VDD = +2.7V to +3.6V, fSAMPLE = 1 MSPS | 0.6 | 1.5 | mA (max) |
|  | Shutdown Mode | VDD = +5V, SCLK Off | 0.5 |  | µA (max) |
|  |  | VDD = +5V, SCLK On | 60 |  | µA (max) |
| PD | Power Consumption, Normal Mode (Operational) | VDD = +5V, fSAMPLE = 1 MSPS | 10 | 16 | mW (max) |
|  |  | VDD = +3V, fSAMPLE = 1 MSPS | 2 | 4.5 | mW (max) |
|  | Power Consumption, Shutdown Mode | VDD = +5V, SCLK Off | 2.5 |  | µW (max) |
|  |  | VDD = +3V, SCLK Off | 1.5 |  | µW (max) |
| ANALOG INPUT CHARACTERISTICS |  |  |  |  |  |
| VIN | Input Range |  | 0 to VDD |  | V |
| IDCL | DC Leakage Current |  |  | ±1 | µA (max) |
| CINA | Input Capacitance (Note 3) |  | 30 |  | pF |
| DIGITAL INPUT CHARACTERISTICS |  |  |  |  |  |
| VIH | Input High Voltage |  |  | 2.4 | V (min) |
| VIL | Input Low Voltage | VDD = +5V |  | 0.8 | V (max) |
|  |  | VDD = +3V |  | 0.4 | V (max) |
| IIN | Input Current | VIN = 0V or VDD | ±10 nA | ±1 | µA (max) |
| CIND | Input Capacitance (Note 3) |  | 2 | 4 | pF (max) |
| DIGITAL OUTPUT CHARACTERISTICS |  |  |  |  |  |
| VOH | Output High Voltage | ISOURCE = 200 µA, VDD = +2.7V to +5.25V |  | VDD −0.2 | V (min) |

ADC121S101/ADC101S101/ADC081S101

5                                                                                                  www.national.com

| ADC101S101 Converter Electrical Characteristics (Continued) The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, fSAMPLE = 1 MSPS unless otherwise noted. Bold- face limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typical | Limits | Units |
| DIGITAL OUTPUT CHARACTERISTICS |  |  |  |  |  |
| VOL | Output Low Voltage | ISINK = 200 µA |  | 0.4 | V (max) |
| IOL | TRI-STATE Leakage Current |  |  | ±10 | µA (max) |
| COUT | TRI-STATE Output Capacitance (Note 3) |  | 2 | 4 | pF (max) |
|  | Output Coding |  | Straight (Natural) Binary |  |  |
| AC ELECTRICAL CHARACTERISTICS |  |  |  |  |  |
| fSCLK | Clock Frequency |  |  | 20 | MHz (max) |
| DC | SCLK Duty Cycle |  |  | 40 60 | % (min) % (max) |
| tTH | Track/Hold Acquisition Time |  |  | 400 | ns (max) |
| fRATE | Throughput Rate | See Serial Interface Section |  | 1 | MSPS (max) |
| tAD | Aperture Delay |  | 3 |  | ns |
| tAJ | Aperture Jitter |  | 30 |  | ps |

ADC121S101/ADC101S101/ADC081S101

www.national.com                                                                                                   6

| ADC081S101 Converter Electrical Characteristics The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, fSAMPLE = 1 MSPS unless otherwise noted. Bold- face limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions |  | Typical | Limits | Units |
| STATIC CONVERTER CHARACTERISTICS |  |  |  |  |  |  |
|  | Resolution with No Missing Codes |  |  |  | 8 | Bits |
| INL | Integral Non-Linearity |  |  | ±0.05 | ±0.3 | LSB (max) |
| DNL | Differential Non-Linearity |  |  | ±0.07 | ±0.3 | LSB (max) |
| VOFF | Offset Error |  |  | ±0.03 | ±0.3 | LSB (max) |
| GE | Gain Error |  |  | ±0.08 | ±0.4 | LSB (max) |
|  | Total Unadjusted Error |  |  | ±0.07 | ±0.3 | LSB (max) |
| DYNAMIC CONVERTER CHARACTERISTICS |  |  |  |  |  |  |
| SINAD | Signal-to-Noise Plus Distortion Ratio | fIN = 100 kHz |  | 49.7 | 49 | dB (min) |
| SNR | Signal-to-Noise Ratio | fIN = 100 kHz |  | 49.7 |  | dB |
| THD | Total Harmonic Distortion | fIN = 100 kHz |  | -77 | -65 | dB (max) |
| SFDR | Spurious-Free Dynamic Range | fIN = 100 kHz |  | 69 | 65 | dB (min) |
| IMD | Intermodulation Distortion, Second Order Terms | fa = 103.5 kHz, fb = 113.5 kHz |  | -68 |  | dB |
|  | Intermodulation Distortion, Third Order Terms | fa = 103.5 kHz, fb = 113.5 kHz |  | -68 |  | dB |
| FPBW | -3 dB Full Power Bandwidth | +5V Supply |  | 11 |  | MHz |
|  |  | +3V Supply |  | 8 |  | MHz |
| POWER SUPPLY CHARACTERISTICS |  |  |  |  |  |  |
| VDD | Supply Voltage |  |  |  | 2.7 5.25 | V (min) V (max) |
| IDD | Normal Mode (Static) | VDD = +4.75V to +5.25V, SCLK On or Off |  | 2 |  | mA |
|  |  | VDD = +2.7V to +3.6V, SCLK On or Off |  | 1 |  | mA |
|  | Normal Mode (Operational) | VDD = +4.75V to +5.25V, fSAMPLE = 1 MSPS | SOT-23 | 2.0 | 3.2 | mA (max) |
|  |  |  | LLP |  | 2.6 |  |
|  |  | VDD = +2.7V to +3.6V, fSAMPLE = 1 MSPS | SOT-23 | 0.6 | 1.5 | mA (max) |
|  |  |  | LLP |  | 1.1 |  |
|  | Shutdown Mode | VDD = +5V, SCLK Off |  | 0.5 |  | µA (max) |
|  |  | VDD = +5V, SCLK On |  | 60 |  | µA (max) |
| PD | Power Consumption, Normal Mode (Operational) | VDD = +5V, fSAMPLE = 1 MSPS | SOT-23 | 10 | 16 | mW (max) |
|  |  |  | LLP |  | 13 |  |
|  |  | VDD = +3V, fSAMPLE = 1 MSPS |  | 2 | 4.5 | mW (max) |
|  | Power Consumption, Shutdown Mode | VDD = +5V, SCLK Off |  | 2.5 |  | µW (max) |
|  |  | VDD = +3V, SCLK Off |  | 1.5 |  | µW (max) |
| ANALOG INPUT CHARACTERISTICS |  |  |  |  |  |  |
| VIN | Input Range |  |  | 0 to VDD |  | V |
| IDCL | DC Leakage Current |  |  |  | ±1 | µA (max) |
| CINA | Input Capacitance |  |  | 30 |  | pF |
| DIGITAL INPUT CHARACTERISTICS |  |  |  |  |  |  |
| VIH | Input High Voltage |  |  |  | 2.4 | V (min) |
| VIL | Input Low Voltage | VDD = +5V |  |  | 0.8 | V (max) |
|  |  | VDD = +3V |  |  | 0.4 | V (max) |
| IIN | Digital Input Current | VIN = 0V or VDD |  | ±10 nA | ±1 | µA (max) |
| CIND | Input Capacitance(Note 3) |  |  | 2 | 4 | pF (max) |

ADC121S101/ADC101S101/ADC081S101

7                                                                                                  www.national.com

| ADC081S101 Converter Electrical Characteristics (Continued) The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, fSAMPLE = 1 MSPS unless otherwise noted. Bold- face limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typical | Limits | Units |
| DIGITAL OUTPUT CHARACTERISTICS |  |  |  |  |  |
| VOH | Output High Voltage | ISOURCE = 200 µA, VDD = +2.7V to +5.25V |  | VDD − 0.2 | V (min) |
| VOL | Output Low Voltage | ISINK = 200 µA |  | 0.4 | V (max) |
| IOL | TRI-STATE Leakage Current |  |  | ±10 | µA (max) |
| COUT | TRI-STATE Output Capacitance (Note 3) |  | 2 | 4 | pF (max) |
|  | Output Coding |  | Straight (Natural) Binary |  |  |
| AC ELECTRICAL CHARACTERISTICS |  |  |  |  |  |
| fSCLK | Clock Frequency |  |  | 20 | MHz (max) |
| DC | SCLK Duty Cycle |  |  | 40 60 | % (min) % (max) |
| tTH | Track/Hold Acquisition Time |  |  | 400 | ns (max) |
| fRATE | Throughput Rate | See Applications Section |  | 1 | MSPS (min) |
| tAD | Aperture Delay |  | 3 |  | ns |
| tAJ | Aperture Jitter |  | 30 |  | ps |

ADC121S101/ADC101S101/ADC081S101

www.national.com                                                                                                   8

| ADC121S101/ADC101S101/ADC081S101 Timing Specifications The following specifications apply for VDD = +2.7V to 5.25V, fSCLK = 20 MHz, Boldface limits apply for TA = −40˚C to +85˚C: all other limits TA = 25˚C, unless otherwise noted. (Note 11) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typical | Limits | Units |
| tCONVERT |  |  | 16 x tSCLK |  |  |
| tQUIET | (Note 7) |  |  | 50 | ns (min) |
| t1 | Minimum CS Pulse Width |  |  | 10 | ns (min) |
| t2 | CS to SCLK Setup Time |  |  | 10 | ns (min) |
| t3 | Delay from CS Until SDATA TRI-STATE® Disabled (Note 8) |  |  | 20 | ns (max) |
| t4 | Data Access Time after SCLK Falling Edge(Note 9) | VDD = +2.7 to +3.6 |  | 40 | ns (max) |
|  |  | VDD = +4.75 to +5.25 |  | 20 | ns (max) |
| t5 | SCLK Low Pulse Width |  |  | 0.4 x tSCLK | ns (min) |
| t6 | SCLK High Pulse Width |  |  | 0.4 x tSCLK | ns (min) |
| t7 | SCLK to Data Valid Hold Time | VDD = +2.7 to +3.6 |  | 7 | ns (min) |
|  |  | VDD = +4.75 to +5.25 |  | 5 | ns (min) |
| t8 | SCLK Falling Edge to SDATA High Impedance (Note 10) | VDD = +2.7 to +3.6 |  | 25 6 | ns (max) ns (min) |
|  |  | VDD = +4.75 to +5.25 |  | 25 5 | ns (max) ns (min) |
| tPOWER-UP | Power-Up Time from Full Power-Down |  | 1 |  | µs |
| Note 1: Absolute maximum ratings are limiting values, to be applied individually, and beyond which the serviceability of the circuit may be impaired. Functional operability under any of these conditions is not implied. Exposure to maximum ratings for extended periods may affect device reliability. Note 2: All voltages are measured with respect to GND = 0V, unless otherwise specified Note 3: Specification limit guaranteed by design. Note 4: See the section titled "Surface Mount" found in a current National Semiconductor Linear Databook for other methods of soldering suface mount devices. Note 5: Except power supply pins. Note 6: Independent of supply voltage. Note 7: Minimum Quiet Time Required Between Bus Relinquish and Start of Next Conversion Note 8: Measured with the load circuit shown above, and defined as the time taken by the output to cross 1.0V. Note 9: Measured with the load circuit shown above, and defined as the time taken by the output to cross 1.0V or 2.0V. Note 10: t8 is derived from the time taken by the outputs to change by 0.5V with the loading circuit shown above. The measured number is then adjusted to remove the effects of charging or discharging the 25pF capacitor. This means t8 is the true bus relinquish time, independent of the bus loading. Note 11: All input signals are specified as tr = tf = 5 ns (10% to 90% VDD) and timed from 1.6V. |  |  |  |  |  |

ADC121S101/ADC101S101/ADC081S101

9                                                                                                  www.national.com

Specification Definitions

APERTURE  DELAY is the time after the falling edge of CS

to when the input signal is acquired or held for conversion.

APERTURE  JITTER  (APERTURE  UNCERTAINTY)  is the variation in aperture delay from sample to sample. Aperture jitter manifests itself as noise in the output.

DIFFERENTIAL  NON-LINEARITY  (DNL) is the measure of the maximum deviation from the ideal step size of 1 LSB.

DUTY CYCLE is the ratio of the time that a repetitive digital waveform is high to the total time of one period. The speci- fication here refers to the SCLK.

EFFECTIVE  NUMBER  OF  BITS  (ENOB,  or  EFFECTIVE BITS) is another method of specifying  Signal-to-Noise  and Distortion  or SINAD. ENOB is defined as (SINAD - 1.76) /

6.02 and says that the converter  is equivalent  to a perfect

ADC of this (ENOB) number of bits.

ADC121S101/ADC101S101/ADC081S101

FULL POWER BANDWIDTH is a measure of the frequency at which the reconstructed  output fundamental  drops 3 dB below its low frequency value for a full scale input.

GAIN  ERROR  is the  deviation  of the  last  code  transition (111...110)  to (111...111)  from the ideal (VREF - 1.5 LSB for ADC121S101 and ADC101S101, VREF - 1 LSB for ADC081S101),  after adjusting for offset error.

INTEGRAL NON-LINEARITY (INL) is a measure of the deviation  of each  individual  code  from  a line  drawn  from negative  full scale (1⁄2   LSB below the first code transition) through positive full scale (1⁄2  LSB above the last code transition). The deviation of any given code from this straight line is measured from the center of that code value.

INTERMODULATION DISTORTION (IMD) is the creation of additional spectral components as a result of two sinusoidal frequencies being applied to the ADC input at the same time. It is defined as the ratio of the power in the either the two second order or all four third order intermodulation  products to the sum of the power in both of the original frequencies. IMD is usually expressed in dBFS.

MISSING CODES are those output codes that will never appear  at  the  ADC  outputs.  The  ADC121S101/101S101/

081S101 is guaranteed not to have any missing codes.

OFFSET ERROR is the deviation of the first code transition (000...000) to (000...001) from the ideal (i.e. GND + 0.5 LSB for the ADC121S101  and ADC101S101,  and GND + 1 LSB for the ADC081S101).

SIGNAL TO NOISE RATIO (SNR) is the ratio, expressed in dB, of the rms value of the input signal to the rms value of the sum of all other spectral components below one-half the sampling frequency, not including harmonics or dc.

SIGNAL TO NOISE PLUS DISTORTION (S/N+D or SINAD) Is the ratio, expressed  in dB, of the rms value of the input signal to the rms value of all of the other spectral  compo- nents  below  half the clock frequency,  including  harmonics but excluding dc.

SPURIOUS  FREE DYNAMIC RANGE (SFDR) is the differ- ence, expressed in dB, between the rms values of the input signal and the peak spurious signal, where a spurious signal is  any  signal  present  in  the  output  spectrum  that  is  not present at the input.

TOTAL  HARMONIC  DISTORTION  (THD)  is the ratio,  ex- pressed  in dBc, of the rms total of the first five harmonic levels  at the output  to the level of the fundamental  at the output. THD is calculated as

where Af1  is the  RMS  power  of the  fundamental  (output)

frequency and Af2 through Af6 are the RMS power in the first

5 harmonic frequencies.

TOTAL UNADJUSTED  ERROR is the worst deviation found from the ideal transfer function. As such, it is a comprehen- sive  specification  which  includes  full  scale  error,  linearity error, and offset error.

www.national.com                                                                                                  10

Timing Diagrams

ADC121S101/ADC101S101/ADC081S101

20110208

FIGURE 1. Timing Test Circuit

20110202

FIGURE 2. ADC121S101  Serial Interface Timing Diagram

20110203

FIGURE 3. ADC101S101  Serial Interface Timing Diagram

11                                                                                                 www.national.com

ADC121S101/ADC101S101/ADC081S101

Timing Diagrams  (Continued)

20110204

FIGURE 4. ADC081S101  Serial Interface Timing Diagram

www.national.com                                                                                                  12

ADC121S101/ADC101S101/ADC081S101

Typical Performance Characteristics  TA = +25˚C, VDD = 3V, fSAMPLE  = 1 MSPS, fSCLK  = 20 MHz, fIN = 100 kHz unless otherwise stated.

ADC121S101

ADC121S101  DNL                                                                               ADC121S101  INL

20110206                                                                                                                                                                                   20110205

ADC121S101  Spectral Response  @ 100 kHz Input                             ADC121S101  THD vs. Source Impedance

20110207                                                                                                                                                                                   20110250

ADC121S101  THD vs. Input Frequency,  600 kSPS                      ADC121S101  THD vs. Input Frequency,  1 MSPS

20110251                                                                                                                                                                                   20110252

13                                                                                                 www.national.com

ADC121S101/ADC101S101/ADC081S101

Typical Performance Characteristics TA = +25˚C, VDD = 3V, fSAMPLE  = 1 MSPS, fSCLK  = 20 MHz, fIN

= 100 kHz unless otherwise stated.  (Continued)

ADC121S101  SINAD vs. Input Frequency,  600 kSPS                  ADC121S101  SINAD vs. Input Frequency,  1 MSPS

20110253                                                                                                                                                                                   20110254

ADC121S101  SNR vs. fSCLK                                                                                              ADC121S101  SINAD vs. fSCLK

20110256                                                                                                                                                                                   20110257

www.national.com                                                                                                  14

ADC121S101/ADC101S101/ADC081S101

Typical Performance Characteristics TA = +25˚C, VDD = 3V, fSAMPLE  = 1 MSPS, fSCLK  = 20 MHz, fIN

= 100 kHz unless otherwise stated.  (Continued)

ADC101S101  DNL                                                                               ADC101S101  INL

20110270                                                                                                                                                                                   20110271

ADC101S101  Spectral Response  @ 100 kHz Input                                         ADC101S101  SNR vs. fSCLK

ADC101S101  SINAD vs. fSCLK

20110272                                                                                                                                                                                   20110273

20110274

15                                                                                                 www.national.com

ADC121S101/ADC101S101/ADC081S101

Typical Performance Characteristics TA = +25˚C, VDD = 3V, fSAMPLE  = 1 MSPS, fSCLK  = 20 MHz, fIN

= 100 kHz unless otherwise stated.  (Continued)

ADC081S101  DNL                                                                               ADC081S101  INL

20110260                                                                                                                                                                                   20110261

ADC081S101  Spectral Response  @ 100 kHz Input                                         ADC081S101  SNR vs. fSCLK

20110262                                                                                                                                                                                   20110263

ADC081S101  SINAD vs. fSCLK                                                                                 Power Consumption  vs. Throughput

20110264                                                                                                                                                                                   20110255

www.national.com                                                                                                  16

Applications Information

1.0 ADC121S101/101S101/081S101 OPERATION

The ADC121S101/101S101/081S101 are successive- approximation  analog-to-digital  converters designed around a charge-redistribution digital-to-analog  converter. Simplified schematics  of the ADC121S101/101S101/081S101 in both track and hold operation are shown in Figures 4 and 5, respectively. In Figure 4, the device is in track mode: switch SW1 connects the sampling capacitor to the input, and SW2 balances the comparator  inputs. The device is in this state until CS is brought low, at which point the device moves to hold mode.

ADC121S101/ADC101S101/ADC081S101

Figure 5 shows the device in hold mode: switch SW1 con- nects the sampling capacitor to ground, maintaining the sampled voltage, and switch SW2 unbalances the compara- tor. The control logic then instructs the charge-redistribution DAC to add or subtract  fixed amounts  of charge  from the sampling capacitor until the comparator  is balanced.  When the comparator is balanced, the digital word supplied to the DAC is the digital representation of the analog input voltage. The device moves from hold mode to track mode on the 13th rising edge of SCLK.

20110209

FIGURE 5. ADC121S101/101S101/081S101 in Track Mode

20110210

FIGURE 6. ADC121S101/101S101/081S101 in Hold Mode

2.0 USING THE ADC121S101/101S101/081S101

Serial   interface   timing   diagrams   for   the  ADC121S101/

101S101/081S101  are shown in Figures 1, 2, and 3. CS is chip select, which initiates conversions on the ADC121S101/

101S101/081S101   and  frames  the  serial  data  transfers. SCLK  (serial  clock)  controls  both  the  conversion  process and the timing of serial data. SDATA is the serial data out pin, where a conversion result is found as a serial data stream.

Basic operation of the ADC121S101/101S101/081S101 be- gins with CS going low, which initiates a conversion process and  data  transfer.  Subsequent  rising  and  falling  edges  of SCLK will be labelled  with reference  to the falling edge of CS; for example, "the third falling edge of SCLK" shall refer to the third falling edge of SCLK after CS goes low.

At the fall of CS, the SDATA pin comes out of TRI-STATE, and the converter moves from track mode to hold mode. The input signal is sampled and held for conversion on the falling edge of CS. The converter moves from hold mode to track

mode on the 13th rising edge of SCLK (see Figure 1, 2, or 3). The SDATA pin will be placed back into TRI-STATE after the

16th falling edge of SCLK, or at the rising edge of CS, whichever occurs first. After a conversion  is completed,  the quiet time tQUIET  must be satisfied  before bringing  CS low again to begin another conversion.

Sixteen  SCLK  cycles  are  required   to  read  a  complete sample from the ADC121S101/101S101/081S101. The sample bits (including any leading or trailing zeroes) are clocked out on falling edges of SCLK, and are intended to be clocked  in  by  a  receiver  on  subsequent  falling  edges  of SCLK. The ADC121S101/101S101/081S101 will produce three leading zero bits on SDATA, followed by twelve, ten, or eight data bits, most significant first. After the data bits, the ADC101S101 will clock out two trailing zeros, and the ADC081S101 will clock out four trailing zeros. The ADC121S101  will not clock out any trailing zeros; the least significant  data bit will be valid on the 16th falling edge of SCLK.

17                                                                                                 www.national.com

ADC121S101/ADC101S101/ADC081S101

Applications Information  (Continued)

If CS goes low before the rising edge of SCLK, an additional (fourth) zero bit may be captured by the next falling edge of SCLK.

3.0 ADC121S101/101S101/081S101 TRANSFER FUNCTION

The output format of the ADC121S101/101S101/081S101 is straight binary. Code transitions occur midway between suc-

cessive integer LSB values. The LSB widths for the ADC121S101  is VDD / 4096; for the ADC101S101  the LSB width is VDD / 1024; for the ADC081S101,  the LSB width is VDD / 256. The ideal transfer characteristic for the ADC121S101  and ADC101S101  is shown in Figure 6, while the  ideal  transfer  characteristic   for  the  ADC081S101   is shown in Figure 7.

20110211

FIGURE 7. ADC121S101/101S101 Ideal Transfer Characteristic

20110212

FIGURE 8. ADC081S101  Ideal Transfer Characteristic

www.national.com                                                                                                  18

Applications Information  (Continued)

4.0 SAMPLE CIRCUIT

20110214

FIGURE 10. Equivalent Input Circuit

FIGURE 9. Sample Circuit

20110213

6.0 DIGITAL INPUTS AND OUTPUTS

ADC121S101/ADC101S101/ADC081S101

The ADC121S101/101S101/081S101 digital  inputs  (SCLK and  CS)  are  not  limited  by  the  same  absolute  maximum ratings  as the analog  inputs. The digital  input pins are in- stead limited  to +6.5V  with respect  to GND, regardless  of VDD,  the  supply  voltage.  This  allows  the  ADC121S101/

101S101/081S101  to be interfaced with a wide range of logic

levels, independent  of the supply voltage.

A typical application of the ADC121S101/101S101/081S101 is  shown  in  Figure  8.  The  combined  analog  and  digital supplies   are  provided   in  this  example   by  the  National LP2950 low-dropout  voltage regulator, available in a variety of fixed and adjustable  output  voltages.  The supply  is by- passed with a capacitor network located close to the device. The three-wire interface is also shown connected to a micro- processor or DSP.

5.0 ANALOG INPUTS

An   equivalent    circuit    for   the   ADC121S101/101S101/

081S101 input channel is shown in Figure 9. The diodes D1 and D2 provide ESD protection for the analog inputs. At no time should an analog input exceed VDD + 300 mV or GND

- 300 mV, as these ESD diodes will begin conducting current

into the substrate and affect ADC operation.

The capacitor  C1 in Figure 9 typically has a value of 4 pF, and is mainly due to pin capacitance. The resistor R1 repre- sents the on resistance  of the multiplexer  and track / hold switch, and is typically  100 ohms. The capacitor  C2 is the ADC121S101/101S101/081S101 sampling capacitor, and is typically 26 pF.

The sampling nature of the analog input causes input current pulses that result in voltage spikes at the input. The ADC121S101/101S101/081S101 will deliver best perfor- mance when driven by a low-impedance  source to eliminate distortion  caused  by the charging  of the sampling  capaci- tance. In applications where dynamic performance is critical, the  input  might  need  to  be  driven  with  a  low  output- impedance amplifier. In addition, when using the ADC121S101/101S101/081S101 to  sample  AC  signals,  a band-pass or low-pass filter will reduce harmonics and noise and thus improve THD and SNR.

Note that, even though the digital inputs are tolerant of up to

+6.5V above  GND, the digital outputs  are only capable  of driving  VDD  out.  In addition,  the digital  input  pins  are not prone  to latch-up;  SCLK  and CS may be asserted  before VDD without any risk.

7.0 MODES OF OPERATION

The ADC121S101/101S101/081S101 has two possible modes of operation: normal mode, and shutdown mode. The ADC121S101/101S101/081S101 enters normal mode (and a conversion process is begun) when CS is pulled low. The device will enter shutdown mode if CS is pulled high before the tenth falling edge of SCLK after CS is pulled low, or will stay in normal mode if CS remains low. Once in shutdown mode,  the  device  will  stay  there  until  CS  is  brought  low again. By varying the ratio of time spent in the normal and shutdown  modes,  a  system  may  trade-off  throughput  for power consumption.

8.0 NORMAL MODE

The best possible throughput is obtained by leaving the ADC121S101/101S101/081S101  in  normal   mode   at  all times, so there are no power-up delays. To keep the device in normal mode continuously, CS must be kept low until after the 10th falling edge of SCLK after the start of a conversion (remember that a conversion is initiated by bringing CS low).

If CS is brought high after the 10th falling edge, but before the 16th falling edge, the device will remain in normal mode, but the current conversion  will be aborted,  and SDATA will return to TRI-STATE (truncating the output word).

Sixteen SCLK cycles are required to read all of a conversion word  from  the  device.  After  sixteen  SCLK  cycles  have elapsed,  CS may be idled either high or low until the next conversion. If CS is idled low, it must be brought high again before the start of the next conversion,  which begins when CS is again brought low.

After  sixteen  SCLK  cycles,  SDATA  returns  to TRI-STATE. Another   conversion   may   be   started,   after   tQUIET    has elapsed, by bringing CS low again.

19                                                                                                 www.national.com

ADC121S101/ADC101S101/ADC081S101

Applications Information  (Continued)

9.0 SHUTDOWN  MODE

Shutdown mode is appropriate for applications that either do not sample  continuously,  or are willing to trade throughput for power  consumption.  When the ADC121S101/101S101/

081S101 is in shutdown mode, all of the analog circuitry is turned off.

To enter shutdown mode, a conversion must be interrupted by bringing CS back high anytime between the second and

tenth falling edges of SCLK, as shown in Figure 10. Once CS has been brought high in this manner, the device will enter shutdown mode; the current conversion will be aborted and SDATA will enter TRI-STATE. If CS is brought high before the second  falling  edge  of SCLK,  the  device  will  not  change mode; this is to avoid accidentally changing mode as a result of noise on the CS line.

20110216

FIGURE 11. Entering Shutdown Mode

10.0 EXITING SHUTDOWN  MODE

20110217

FIGURE 12. Entering Normal Mode

To exit shutdown  mode, bring CS back low. Upon bringing CS low, the ADC121S101/101S101/081S101 will begin pow- ering up. Power up typically takes 1 µs. This microsecond of power-up  delay  results  in the first conversion  result  being unusable. The second conversion performed after power-up, however, is valid, as shown in Figure 11.

If CS is brought  back high before  the 10th falling edge of SCLK, the device will return to shutdown mode. This is done to avoid  accidentally  entering  normal  mode  as a result  of noise on the CS line. To exit shutdown mode and remain in normal mode, CS must be kept low until after the 10th falling edge of SCLK. The ADC121S101/101S101/081S101 will be fully powered-up  after 16 SCLK cycles.

11.0 POWER-UP TIMING

The ADC121S101/101S101/081S101 typically requires 1 µs to power up, either after first applying VDD, or after returning to normal mode from shutdown  mode. This corresponds  to one "dummy" conversion for any SCLK frequency within the specifications  in this document. After this first dummy con- version, the ADC121S101/101S101/081S101 will perform conversions  properly. Note that the tQUIET time must still be included between the first dummy conversion  and the sec- ond valid conversion.

12.0 STARTUP MODE

When  the  VDD  supply  is  first  applied,  the  ADC121S101/

101S101/081S101  may power up in either of the two modes:

normal   or  shutdown.   As  such,  one  dummy   conversion should be performed  after start-up, exactly as described  in Section 11.0. The part may then be placed into either normal mode or the shutdown  mode, as described  in Sections 8.0 and 9.0.

13.0 POWER MANAGEMENT

When the ADC121S101/101S101/081S101 is operated con- tinuously in normal mode, throughput up to 1 MSPS can be achieved.  The  user  may  trade  throughput  for power  con- sumption  by simply  performing  fewer conversions  per unit time, and putting  the ADC121S101/101S101/081S101 into shutdown  mode  between  conversions.  This method  is not advantageous  beyond 350 kSPS throughput.

A plot of maximum power consumption versus throughput is shown in Figure 12 below. To calculate the power consump- tion for a given throughput, remember that each time the part exits shutdown mode and enters normal mode, one dummy conversion  is required. Generally,  the user will put the part into normal mode, execute one dummy conversion followed by one  valid  conversion,  and  then  put  the  part  back  into shutdown  mode.  When  this  is  done,  the  fraction  of  time

www.national.com                                                                                                  20

Applications Information  (Continued)

spent in normal mode may be calculated by multiplying the throughput (in samples per second) by 2 µs, the time taken to perform one dummy and one valid conversion. The power consumption can then be found by multiplying the fraction of time spent in normal mode by the normal mode power consumption figure. The power dissipated while the part is in shutdown mode is negligible.

ADC121S101/ADC101S101/ADC081S101

For  example,  to calculate  the  power  consumption  at 300 kSPS with VDD = 5V, begin by calculating the fraction of time spent in normal mode: 300,000 samples/second·2  µs= 0.6, or 60%. The power consumption at 300 kSPS is then 60% of

17.5 mW (the maximum power consumption at VDD = 5V) or

10.5 mW.

21                                                                                                 www.national.com

ADC121S101/ADC101S101/ADC081S101

Physical Dimensions  inches (millimeters)  unless otherwise  noted

6-Lead SOT-23

Order Number ADC121S101CIMF, ADC121S101CIMFX, ADC101S101CIMF, ADC101S101CIMFX, ADC081S101CIMF or

ADC081S101CIMFX

NS Package Number MF06A

6-Lead LLP

Order Number ADC081S101CISD or ADC081S101CISDX NS Package Number SDB06A

www.national.com                                                                                                  22

ADC121S101/ADC101S101/ADC081S101 1MSPS, 12-/10-/8-Bit A/D Converters in SOT-23 & LLP

Notes

National does not assume any responsibility for use of any circuitry described, no circuit patent licenses are implied and National reserves the right at any time without notice to change said circuitry and specifications. For the most current product information visit us at www.national.com.

LIFE SUPPORT POLICY NATIONAL’S PRODUCTS ARE NOT AUTHORIZED FOR USE AS CRITICAL COMPONENTS IN LIFE SUPPORT DEVICES OR SYSTEMS WITHOUT THE EXPRESS WRITTEN APPROVAL OF THE PRESIDENT AND GENERAL COUNSEL OF NATIONAL SEMICONDUCTOR CORPORATION. As used herein: 1. Life support devices or systems are devices or systems 2. A critical component is any component of a life support which, (a) are intended for surgical implant into the body, or device or system whose failure to perform can be reasonably (b) support or sustain life, and whose failure to perform when expected to cause the failure of the life support device or properly used in accordance with instructions for use system, or to affect its safety or effectiveness. provided in the labeling, can be reasonably expected to result in a significant injury to the user.

BANNED SUBSTANCE COMPLIANCE National Semiconductor certifies that the products and packing materials meet the provisions of the Customer Products Stewardship Specification (CSP-9-111C2) and the Banned Substances and Materials of Interest Specification (CSP-9-111S2) and contain no ‘‘Banned Substances’’ as defined in CSP-9-111S2.

National Semiconductor National Semiconductor National Semiconductor National Semiconductor Americas Customer Europe Customer Support Center Asia Pacific Customer Japan Customer Support Center Support Center Fax: +49 (0) 180-530 85 86 Support Center Fax: 81-3-5639-7507 Email: new.feedback@nsc.com Email: europe.support@nsc.com Email: ap.support@nsc.com Email: jpn.feedback@nsc.com Tel: 1-800-272-9959 Deutsch Tel: +49 (0) 69 9508 6208 Tel: 81-3-5639-7560 English Tel: +44 (0) 870 24 0 2171 www.national.com Français Tel: +33 (0) 1 41 91 8790