LP2985

LP2985 Micropower 150 mA Low-Noise Ultra Low-Dropout Regulator in SOT-23 and micro SMD Packages

June 15, 2009

Micropower 150 mA Low-Noise Ultra Low-Dropout

Regulator in SOT-23 and micro SMD Packages

Designed for Use with Very Low ESR Output Capacitors

General Description

The LP2985 is a 150 mA, fixed-output  voltage regulator de- signed to provide ultra low-dropout  and low noise in battery powered applications.

Using an optimized VIP® (Vertically Integrated PNP) process, the LP2985 delivers unequalled performance  in all specifica- tions critical to battery-powered designs:

Dropout  Voltage:  Typically  300 mV @ 150 mA load, and

7 mV @ 1 mA load.

Ground Pin Current: Typically 850 µA @ 150 mA load, and

75 µA @ 1 mA load.

Enhanced  Stability:  The LP2985  is stable with output ca- pacitor ESR as low as 5 mΩ, which allows the use of ceramic capacitors on the output.

Sleep Mode: Less than 1 µA quiescent current when ON/OFF

pin is pulled low.

Smallest Possible Size: SOT-23 and micro SMD packages use absolute minimum board space.

Precision Output: 1% tolerance output voltages available (A

grade).

Low Noise: By adding a 10 nF bypass capacitor, output noise can be reduced to 30 µV (typical).

Multiple voltage options, from 2.5V to 5.0V, are available as standard products. Consult factory for custom voltages.

Features

■  Ultra low dropout voltage

■  Guaranteed 150 mA output current

■  Smallest possible size (SOT-23, micro SMD package)

■  Requires minimum external components

■  Stable with low-ESR output capacitor

■  <1 µA quiescent current when shut down

■  Low ground pin current at all loads

■  Output voltage accuracy 1% (A Grade)

■  High peak current capability

■  Wide supply voltage range (16V max)

■  Low ZOUT: 0.3Ω typical (10 Hz to 1 MHz)

■  Overtemperature/overcurrent protection

■  −40°C to +125°C junction temperature  range

■  Custom voltages available

Applications

■  Cellular Phone

■  Palmtop/Laptop Computer

■  Personal Digital Assistant (PDA)

■  Camcorder, Personal Stereo, Camera

Block Diagram

10014001

VIP®  is a registered trademark of National  Semiconductor Corporation.

© 2009 National Semiconductor Corporation        100140                                                                                                                         www.national.com

LP2985

Basic Application Circuit

*ON/OFF input must be actively terminated. Tie to VIN if this function is not to be used.

10014002

**Minimum capacitance is shown to ensure stability (may be increased without limit). Ceramic capacitor required for output (see Application Hints).

***Reduces output noise (may be omitted if application is not noise critical). Use ceramic or film type with very low leakage current (see Application Hints).

Connection Diagrams

SOT-23, 5-Lead Small Outline Package (M5)

micro SMD, 5 Bump Package

Top View

10014003

See NS Package Number MF05A

10014046

Note: The actual physical placement of the package marking will vary from part to part. Package marking contains date code and lot traceability information, and will vary considerably. Package marking does not correlate to device type.

Top View

See NS Package Numbers BPA05, TPA05, and TLA05

Pin Descriptions

Name                               Pin Number                                                                        Function

| VIN | SOT-23 1 | micro SMD C3 | Input Voltage |
| --- | --- | --- | --- |
| GND | 2 | A1 | Common Ground (device substrate) |
| ON/OFF | 3 | A3 | Logic high enable input |
| BYPASS | 4 | B2 | Bypass capacitor for low noise operation |
| VOUT | 5 | C1 | Regulated output voltage |

www.national.com                                                                                                   2

Ordering Information

LP2985

TABLE 1. Package Marking and Ordering Information

Output Voltage

(V)

Grade                     Order Information              Package Marking                           Supplied as:

For output voltages ≤ 2.0V, refer to LP2985LV datasheet. SOT-23, 5-Lead Small Outline Package (M5)

LP2985AIM-2.5                         LAUA                        1000 Units on Tape and Reel

2.5

2.6

A

STD A

LP2985AIM5X-2.5                       LAUA                        3000 Units on Tape and Reel LP2985IM5-2.5                         LAUB                        1000 Units on Tape and Reel LP2985IM5X-2.5                        LAUB                        3000 Units on Tape and Reel LP2985AIM5-2.6                        LCEA                        1000 Units on Tape and Reel LP2985AIM5X-2.6                       LCEA                        3000 Units on Tape and Reel

STD                        LP2985AIM5–2.6                        LCEB                         1000 Units on Tape and Ree

LP2985AIM5-2.7                        LALA                         1000 Units on Tape and Reel

2.7

2.8

2.9

3.0

3.1

3.2

3.3

3.5

3.6

A

STD A STD A STD A STD A STD A STD A STD A STD A STD

LP2985AIM5X-2.7                       LALA                         3000 Units on Tape and Reel LP2985IM5-2.7                          LALB                         1000 Units on Tape and Reel LP2985IM5X-2.7                        LALB                         3000 Units on Tape and Reel LP2985AIM5-2.8                        L0KA                         1000 Units on Tape and Reel LP2985AIM5X-2.8                       L0KA                         3000 Units on Tape and Reel LP2985IM5-2.8                          L0KB                         1000 Units on Tape and Reel LP2985IM5X-2.8                        L0KB                         3000 Units on Tape and Reel LP2985AIM5-2.9                        LAXA                        1000 Units on Tape and Reel LP2985AIM5X-2.9                       LAXA                        3000 Units on Tape and Reel LP2985IM5-2.9                         LAXB                        1000 Units on Tape and Reel LP2985IM5X-2.9                        LAXB                        3000 Units on Tape and Reel LP2985AIM5-3.0                        L0OA                        1000 Units on Tape and Reel LP2985AIM5X-3.0                       L0OA                        3000 Units on Tape and Reel LP2985IM5-3.0                         L0OB                        1000 Units on Tape and Reel LP2985IM5X-3.0                        L0OB                        3000 Units on Tape and Reel LP2985AIM5-3.1                        L0PA                         1000 Units on Tape and Reel LP2985AIM5X-3.1                       L0PA                         3000 Units on Tape and Reel LP2985IM5-3.1                          L0PB                         1000 Units on Tape and Reel LP2985IM5X-3.1                        L0PB                         3000 Units on Tape and Reel LP2985AIM5-3.2                        L0QA                        1000 Units on Tape and Reel LP2985AIM5X-3.2                       L0QA                        3000 Units on Tape and Reel LP2985AIM5-3.2                        L0QB                        1000 Units on Tape and Reel LP2985IM5X-3.2                       L0QB                      3000 Units on Tape and Reel LP2985AIM5-3.3                        L0RA                      1000 Units on Tape and Reel LP2985AIM5X-3.3                      L0RA                      3000 Units on Tape and Reel LP2985IM5-3.3                         L0RB                      1000 Units on Tape and Reel LP2985IM5X-3.3                        L0RB                      3000 Units on Tape and Reel LP2985AIM5-3.5                        LAIA                       1000 Units on Tape and Reel LP2985AIM5X-3.5                       LAIA                       3000 Units on Tape and Reel LP2985IM5-3.5                         LAIB                       1000 Units on Tape and Reel LP2985IM5X-3.5                        LAIB                       3000 Units on Tape and Reel LP2985AIM5-3.6                        L0SA                       1000 Units on Tape and Reel LP2985AIM5X-3.6                      L0SA                       3000 Units on Tape and Reel LP2985IM5-3.6                         L0SB                       1000 Units on Tape and Reel LP2985IM5X-3.6                        L0SB                       3000 Units on Tape and Reel

3                                                                                                  www.national.com

LP2985

Output Voltage

(V)

3.8

Grade                     Order Information              Package Marking                           Supplied as:

LP2985AIM5-3.8                        L0YA                         1000 Units on Tape and Reel

A

LP2985AIM5X-3.8                       L0YA                         3000 Units on Tape and Reel

LP2985IM5-3.8                          L0YB                         1000 Units on Tape and Reel

STD

LP2985IM5X-3.8                        L0YB                         3000 Units on Tape and Reel

LP2985AIM5-4.0                         L0TA                         1000 Units on Tape and Reel

4.0

4.5

5.0

5.7

6.1

A

STD A STD A STD A STD A STD

LP2985AIM5X-4.0                       L0TA                         3000 Units on Tape and Reel LP2985IM5-4.0                          L0TB                         1000 Units on Tape and Reel LP2985IM5X-4.0                         L0TB                         3000 Units on Tape and Reel LP2985AIM5-4.5                        LA7A                         1000 Units on Tape and Reel LP2985AIM5X-4.5                       LA7A                         3000 Units on Tape and Reel LP2985IM5X-4.5                        LA7B                         1000 Units on Tape and Reel LP2985IM5-4.5                          LA7B                         3000 Units on Tape and Reel LP2985AIM5-5.0                        L0UA                         1000 Units on Tape and Reel LP2985AIM5X-5.0                       L0UA                         3000 Units on Tape and Reel LP2985IM5-5.0                          L0UB                         1000 Units on Tape and Reel LP2985IM5X-5.0                        L0UB                         3000 Units on Tape and Reel LP2985AIM5-5.7                        LKTA                         1000 Units on Tape and Reel LP2985AIM5X-5.7                       LKTA                         3000 Units on Tape and Reel LP2985IM5-5.7                          LKTB                         1000 Units on Tape and Reel LP2985IM5X-5.7                        LKTB                         3000 Units on Tape and Reel LP2985AIM5-6.1                         LF6A                         1000 Units on Tape and Reel LP2985AIM5X-6.1                       LF6A                         3000 Units on Tape and Reel LP2985IM5-6.1                          LF6B                         1000 Units on Tape and Reel LP2985IM5X-6.1                         LF6B                         3000 Units on Tape and Reel

micro SMD, 5 Bump Package (BPA05 - 170 µm ball)

2.5                           A                          LP2985AIBP-2.5                                                          250 Units on Tape and Reel

LP2985AIBP-3.0                                                          250 Units on Tape and Reel

3.0

A

LP2985AIBPX-3.0                                                        3000 Units on Tape and Reel

STD                         LP2985IBP-3.0                                                           250 Units on Tape and Reel

LP2985AIBP-5.0                                                          250 Units on Tape and Reel

5.0

A

LP2985AIBPX-5.0                                                        3000 Units on Tape and Reel

STD                        LP2985IBPX-5.0                                                         3000 Units on Tape and Reel

micro SMD, 5 Bump Package (TPA05 - 170 µm ball)

LP2985AITP-2.5                                                          250 Units on Tape and Reel

2.5

A

LP2985AITPX-2.5                                                        3000 Units on Tape and Reel

STD                        LP2985ITP-2.5                                                          250 Units on Tape and Reel

LP2985AITP-2.6                                                         250 Units on Tape and Reel

2.6

2.7

2.8

A

STD A STD A STD

LP2985AITPX-2.6                                                       3000 Units on Tape and Reel LP2985ITP-2.6                                                          250 Units on Tape and Reel LP2985ITPX-2.6                                                        3000 Units on Tape and Reel LP2985AITP-2.7                                                         250 Units on Tape and Reel LP2985AITPX-2.7                                                       3000 Units on Tape and Reel LP2985ITP-2.7                                                          250 Units on Tape and Reel LP2985ITPX-2.7                                                        3000 Units on Tape and Reel LP2985AITP-2.8                                                         250 Units on Tape and Reel LP2985AITPX-2.8                                                       3000 Units on Tape and Reel LP2985ITP-2.8                                                          250 Units on Tape and Reel LP2985ITPX-2.8                                                        3000 Units on Tape and Reel

www.national.com                                                                                                   4

Output Voltage

(V)

2.9

Grade                     Order Information              Package Marking                           Supplied as:

LP2985

LP2985AITP-2.9                                                          250 Units on Tape and Reel

A

LP2985AITPX-2.9                                                        3000 Units on Tape and Reel

LP2985ITP-2.9                                                           250 Units on Tape and Reel

STD

LP2985ITPX-2.9                                                         3000 Units on Tape and Reel

LP2985AITP-3.0                                                          250 Units on Tape and Reel

3.0

3.3

5.0

A

STD A STD A STD

LP2985AITPX-3.0                                                        3000 Units on Tape and Reel LP2985ITP-3.0                                                           250 Units on Tape and Reel LP2985ITPX-3.0                                                         3000 Units on Tape and Reel LP2985AITP-3.3                                                          250 Units on Tape and Reel LP2985AITPX-3.3                                                        3000 Units on Tape and Reel LP2985ITP-3.3                                                           250 Units on Tape and Reel LP2985ITPX-3.3                                                         3000 Units on Tape and Reel LP2985AITP-5.0                                                          250 Units on Tape and Reel LP2985AITPX-5.0                                                        3000 Units on Tape and Reel LP2985ITP-5.0                                                           250 Units on Tape and Reel LP2985ITPX-5.0                                                         3000 Units on Tape and Reel

micro SMD, 5 Bump Package (TLA05 - 300 µm ball)

LP2985AITL-3.3                                                          250 Units on Tape and Reel

3.3

A

STD

LP2985AITLX-3.3                                                        3000 Units on Tape and Reel LP2985ITL-3.3                                                                250 Units on Tape and Reel LP2985ITLX-3.3                                                         3000 Units on Tape and Reel

micro SMD, 5 Bump Package (BLA05 - 300 µm ball)

3.3                           A                         LP2985AIBLX-3.3                                                        3000 Units on Tape and Reel

5                                                                                                  www.national.com

LP2985

Absolute Maximum Ratings (Note 1) If Military/Aerospace specified devices are required, please contact the National Semiconductor Sales Office/

Distributors  for availability and specifications.

Storage Temperature  Range                       −65°C to +150°C Operating Junction Temperature

Range                                                            −40°C to +125°C

Lead Temp. (Soldering, 5 sec.)                                     260°C ESD Rating (Note 2)                                                         2 kV

Electrical Characteristics  (Note 10)

Power Dissipation (Note 3)                          Internally Limited Input Supply Voltage (Survival)                        −0.3V to +16V Input Supply Voltage (Operating)                       2.5V to +16V Shutdown Input Voltage (Survival)                   −0.3V to +16V Output Voltage (Survival, (Note 4))                   −0.3V to +9V IOUT (Survival)                                                       Short Circuit

Protected

Input-Output  Voltage (Survival,                        −0.3V to +16V

(Note 5))

Limits in standard typeface are for TJ = 25°C. and limits in boldface type apply over the full operating temperature range. Unless otherwise specified: VIN = VO(NOM) + 1V, IL = 1 mA, CIN = 1 µF, COUT = 4.7 µF, VON/OFF = 2V.

LP2985AI-X.X            LP2985I-X.X

Symbol                   Parameter                             Conditions                      Typ

(Note 6)                    (Note 6)

Min          Max          Min          Max

Units

ΔVO                          Output Voltage

IL = 1 mA                                                        −1.0           1.0          −1.5           1.5

Tolerance

1 mA ≤ IL

≤ 50 mA                                        −1.5           1.5          −2.5           2.5

−2.5           2.5          −3.5           3.5

%VNOM

1 mA ≤ IL ≤ 150 mA                                      −2.5           2.5          −3.0           3.0

−3.5           3.5          −4.0           4.0

Output Voltage                  VO(NOM)+1V  ≤ VIN ≤ 16V          0.007                        0.014                         0.014

Line Regulation                                                                                                          0.032                         0.032

%/V

VIN–VO                    Dropout Voltage

(Note 7)

IL = 0                                                 1                               3                                3

5                                5

IL = 1 mA                                           7                              10                              10

15                              15

IL = 10 mA                                       40                             60                              60            mV

90                              90

IL = 50 mA                                      120                           150                            150

225                            225

IL = 150 mA                                    280                           350                            350

575                            575

IGND                          Ground Pin Current           IL = 0                                                65                             95                              95

125                            125

IL = 1 mA                                          75                            110                            110

170                            170

IL = 10 mA                                      120                           220                            220

400                          400

µA IL = 50 mA                                    350                         600                          600

1000                        1000

IL = 150 mA                                  850                        1500                        1500

2500                        2500

VON/OFF < 0.3V                             0.01                         0.8                           0.8

VON/OFF < 0.15V                           0.05                          2                              2

VON/OFF                   ON/OFF Input Voltage

(Note 8)

High = O/P ON                             1.4          1.6                           1.6

V Low = O/P OFF                           0.55                        0.15                         0.15

ION/OFF                     ON/OFF Input Current     VON/OFF = 0                                  0.01                         −2                           −2

µA

VON/OFF = 5V                                  5                            15                            15

www.national.com                                                                                                   6

|  |  |  |  | LP2985AI-X.X | LP2985I-X.X |  |
| --- | --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Typ | (Note 6) | (Note 6) | Units |

en                               Output Noise                     BW = 300 Hz to 50 kHz,

LP2985

Min          Max          Min          Max

Voltage (RMS)                  COUT = 10 µF                                   30                                                                                µV CBYPASS = 10 nF

| Ripple Rejection f = 1 kHz, CBYPASS = 10 nF IO(SC) | Short Circuit Current | RL = 0 (Steady State) |
| --- | --- | --- |
| IO(PK) | Peak Output Current | (Note 9) VOUT ≥ Vo(NOM) −5% |

COUT

= 10 µF

45                                                                                dB

400                                                                               mA

350                                                                               mA

Note 1: “Absolute Maximum Ratings” indicate limits beyond which damage to the component may occur. Electrical specifications do not apply when operating the device outside of its rated operating conditions.

Note 2: The ESD rating of pins 3 and 4 for the SOT-23 package, or pins 5 and 2 for the micro SMD package, is 1 kV.

Note 3: The maximum allowable power dissipation is a function of the maximum junction temperature, TJ(MAX), the junction-to-ambient thermal resistance, θJ- A, and the ambient temperature, TA. The maximum allowable power dissipation at any ambient temperture is calculated using:

Where the value of θJ-A for the SOT-23 package is 220°C/W in a typical PC board mounting and the micro SMD package is 225°C/W. Exceeding the maximum allowable dissipation will cause excessive die temperature, and the regulator will go into thermal shutdown.

Note 4: For 12V option, output voltage survival: -0.3 to +16V. If used in a dual-supply system where the regulator load is returned to a negative supply, the LP2985 output must be diode-clamped  to ground.

Note 5: The output PNP structure contains a diode between the VIN to VOUT  terminals that is normally reverse-biased.  Reversing the polarity from VIN to VOUT

will turn on this diode.

Note 6: Limits are 100% production tested at 25°C. Limits over the operating temperature range are guaranteed through correlation using Statistical Quality

Control (SQC) methods. The limits are used to calculate National's Average Outgoing Quality Level (AOQL).

Note 7: Dropout voltage is defined as the input to output differential at which the output voltage drops 100 mV below the value measured with a 1V differential.

Note 8: The ON/OFF input must be properly driven to prevent possible misoperation. For details, refer to Application Hints.

Note 9: The LP2985 has foldback current limiting which allows a high peak current when VOUT  > 0.5V, and then reduces the maximum output current as VOUT  is forced to ground (see Typical Performance Characteristics  curves).

Note 10: Exposing the micro SMD device to direct sunlight will cause misoperation. See Application Hints for additional information.

7                                                                                                  www.national.com

Typical Performance Characteristics  Unless otherwise specified: CIN = 1µF, COUT = 4.7µF, VIN = VOUT

LP2985

(NOM) +1, TA = 25°C, ON/OFF pin is tied to VIN.

VOUT vs Temperature

10014008

Short-Circuit  Current

10014009

Short-Circuit  Current

10014010

Short Circuit Current vs Output Voltage

10014011

Ripple Rejection

10014013

Ripple Rejection

10014014

www.national.com                                                                                                   8

Ripple Rejection

10014015

LP2985

Ripple Rejection

10014016

Ripple Rejection

10014017

Ripple Rejection

10014018

Ripple Rejection

10014019

Ripple Rejection

10014023

9                                                                                                  www.national.com

LP2985

Ripple Rejection

10014022

Output Impedance vs Frequency

10014020

Output Impedance vs Frequency

10014021

Output Noise Density

10014024

Output Noise Density

10014025

Ground Pin vs Load Current

10014026

www.national.com                                                                                                  10

Dropout Voltage vs Temperature

10014027

LP2985

Input Current vs Pin

10014029

GND Pin Current vs Temperature

10014030

Instantaneous Short Circuit Current

10014031

Load Transient Response

10014032

Load Transient Response

10014033

11                                                                                                 www.national.com

LP2985

Load Transient Response

10014034

Line Transient Response

10014035

Line Transient Response

10014036

Line Transient Response

10014037

Line Transient Response

10014038

Turn-On Time

10014039

www.national.com                                                                                                  12

Turn-On Time

10014040

LP2985

Turn-On Time

10014041

Turn-On Time

10014042

13                                                                                                 www.national.com

Application Hints

LP2985

EXTERNAL  CAPACITORS

Like any low-dropout regulator, the LP2985 requires external capacitors  for regulator  stability.  These capacitors  must be correctly selected for good performance.

Input Capacitor

An input capacitor  whose capacitance  is ≥ 1 µF is required between the LP2985 input and ground (the amount of capac-

itance may be increased without limit).

This capacitor must be located a distance of not more than 1 cm from the input pin and returned to a clean analog ground. Any good quality ceramic, tantalum, or film capacitor may be used at the input.

Important:  Tantalum capacitors  can suffer catastrophic  fail- ure due to surge current when connected to a low-impedance source of power (like a battery or very large capacitor).  If a Tantalum capacitor is used at the input, it must be guaranteed by the manufacturer  to have a surge current rating sufficient for the application.

There are no requirements for ESR on the input capacitor, but tolerance and temperature coefficient must be considered when selecting  the capacitor  to ensure the capacitance  will

be ≥ 1 µF over the entire operating temperature  range.

Output Capacitor

The LP2985 is designed specifically to work with ceramic out- put capacitors, utilizing circuitry which allows the regulator to be stable across the entire range of output current  with an output capacitor whose ESR is as low as 5 mΩ. It may also be possible to use Tantalum or film capacitors at the output, but these are not as attractive  for reasons of size and cost (see next section Capacitor Characteristics).

The output capacitor must meet the requirement for minimum amount of capacitance and also have an ESR (equivalent se- ries resistance) value which is within the stable range. Curves are provided which show the stable ESR range as a function of load current (see ESR graph below).

10014007

Important: The output capacitor must maintain its ESR within the stable region over the full operating temperature range of the application to assure stability.

The LP2985 requires a minimum of 2.2 µF on the output (out- put capacitor size can be increased without limit).

It is important to remember that capacitor tolerance and vari- ation with temperature must be taken into consideration when selecting an output capacitor so that the minimum required amount of output capacitance is provided over the full oper- ating temperature range. It should be noted that ceramic

capacitors can exhibit large changes in capacitance with tem- perature (see next section, Capacitor Characteristics).

The output capacitor must be located not more than 1 cm from the output pin and returned to a clean analog ground.

Noise Bypass Capacitor

Connecting  a 10 nF capacitor to the Bypass pin significantly reduces noise on the regulator output. It should be noted that the capacitor is connected directly to a high-impedance circuit in the bandgap reference.

Because this circuit has only a few microamperes  flowing in it, any significant loading on this node will cause a change in the regulated  output  voltage.  For this reason,  DC leakage current  through  the noise bypass capacitor  must never ex- ceed 100 nA, and should be kept as low as possible for best output voltage accuracy.

The types of capacitors best suited for the noise bypass ca- pacitor are ceramic and film. High-quality ceramic capacitors with either  NPO  or COG  dielectric  typically  have very low leakage. 10 nF polypropolene and polycarbonate  film capac- itors are available in small surface-mount  packages and typ- ically have extremely low leakage current.

CAPACITOR  CHARACTERISTICS

The LP2985 was designed to work with ceramic capacitors on the output to take advantage of the benefits they offer: for capacitance  values in the 2.2 µF to 4.7 µF range, ceramics are the least expensive and also have the lowest ESR values (which   makes   them   best  for  eliminating   high-frequency noise). The ESR of a typical 2.2 µF ceramic capacitor is in the range of 10 mΩ to 20 mΩ, which easily meets the ESR limits required for stability by the LP2985.

One disadvantage  of ceramic capacitors is that their capaci- tance can vary with temperature.  Most large value ceramic

capacitors (≥ 2.2 µF) are manufactured  with the Z5U or Y5V

temperature  characteristic,  which results in the capacitance

dropping by more than 50% as the temperature goes from 25° C to 85°C.

This could cause problems if a 2.2 µF capacitor were used on the output since it will drop down to approximately 1 µF at high ambient temperatures  (which could cause the LM2985 to os- cillate). If Z5U or Y5V capacitors  are used on the output, a minimum capacitance  value of 4.7 µF must be observed.

A better choice for temperature  coefficient in ceramic capac- itors is X7R, which holds the capacitance  within ±15%. Un- fortunately,  the larger values of capacitance  are not offered by all manufacturers in the X7R dielectric.

Tantalum

Tantalum capacitors are less desirable than ceramics for use as output capacitors because they are more expensive when comparing equivalent capacitance and voltage ratings in the

1 µF to 4.7 µF range.

Another important consideration is that Tantalum capacitors have higher ESR values than equivalent size ceramics. This means that while it may be possible to find a Tantalum ca- pacitor with an ESR value within the stable range, it would have to be larger in capacitance (which means bigger and more costly) than a ceramic capacitor with the same ESR value.

It should also be noted that the ESR of a typical Tantalum will increase about 2:1 as the temperature goes from 25°C down to −40°C, so some guard band must be allowed.

www.national.com                                                                                                  14

On/Off Input Operation

The LP2985 is shut off by driving the ON/OFF input low, and turned on by pulling it high. If this feature is not to be used, the ON/OFF input should be tied to VIN to keep the regulator output on at all times.

To assure proper operation,  the signal source used to drive the ON/OFF input must be able to swing above and below the specified turn-on/turn-off voltage thresholds listed in the Elec- trical Characteristics  section under VON/OFF. To prevent mis- operation, the turn-on (and turn-off) voltage signals applied to the   ON/OFF   input   must   have   a   slew   rate   which   is

≥ 40 mV/µs.

Caution: the regulator output voltage can not be guaranteed if a slow-moving  AC (or DC) signal is applied that is in the range between the specified turn-on and turn-off voltages list- ed under the electrical  specification  VON/OFF  (see Electrical

Characteristics).

REVERSE INPUT-OUTPUT VOLTAGE

The PNP power transistor  used as the pass element  in the LP2985 has an inherent diode connected between the regu- lator output  and input. During normal operation  (where the input voltage is higher than the output) this diode is reverse- biased.

LP2985

VIN to VOUT (cathode on VIN, anode on VOUT), to limit the re- verse voltage across the LP2985 to 0.3V (see Absolute Max- imum Ratings).

10014055

FIGURE 2. Reverse Current Protection

MICRO SMD MOUNTING

The micro SMD package requires specific mounting tech- niques which are detailed in National  Semiconductor Appli- cation Note # 1112. Referring to the section Surface Mount Technology (SMT) Assembly Considerations, it should be noted that the pad style which must be used with the 5-pin package is the NSMD (non-solder mask defined) type.

For best results during assembly,  alignment  ordinals on the PC board may be used to facilitate  placement  of the micro SMD device.

FIGURE 1. Reverse Current Path

10014054

MICRO SMD LIGHT SENSITIVITY

Exposing the micro SMD device to direct sunlight will cause misoperation  of the device. Light sources such as Halogen lamps can also affect electrical performance  if brought near to the device.

The wavelenghts  which have the most detrimental effect are reds and infra-reds, which means that the fluorescent lighting

However, if the output is pulled above the input, this diode will

turn ON and current will flow into the regulator output. In such cases, a parasitic SCR can latch which will allow a high cur- rent  to flow  into  VIN  (and  out  the  ground  pin),  which  can damage the part.

In any application where the output may be pulled above the input,  an external  Schottky  diode must be connected  from

used inside  most  buildings  has very little effect  on perfor- mance. A micro SMD test board was brought to within 1 cm of a fluorescent  desk lamp and the effect on the regulated output  voltage  was negligible,  showing  a deviation  of less than 0.1% from nominal.

15                                                                                                 www.national.com

LP2985

Physical Dimensions inches (millimeters)  unless otherwise noted

SOT-23, 5-Lead Small Outline Package (M5) NS Package Number MF05A.

www.national.com                                                                                                  16

LP2985

NOTES: UNLESS OTHERWISE SPECIFIED

1. EPOXY COATING

2. 63Sn/37Pb EUTECTIC BUMP

3. RECOMMEND  NON-SOLDER  MASK DEFINED LANDING PAD.

4. PIN 1 IS ESTABLISHED BY LOWER LEFT CORNER WITH RESPECT TO TEXT ORIENTATION. REMAINING PINS ARE NUMBERED COUNTER CLOCK- WISE.

5. XXX IN DRAWING NUMBER REPRESENTS PACKAGE SIZE VARIATION WHERE X1 IS PACKAGE WIDTH, X2 IS PACKAGE LENGTH AND X3 IS PACK- AGE HEIGHT.

6.NO JEDEC REGISTRATION  AS OF AUG.1999.

micro SMD, 5 Bump, Package (BPA05) NS Package Number BPA05A

The dimensions  for X1, X2 and X3 are as given: X1 = 0.930 +/− 0.030mm

X2 = 1.107 +/− 0.030mm

X3 = 0.850 +/− 0.050mm

17                                                                                                 www.national.com

LP2985

NOTES: UNLESS OTHERWISE SPECIFIED

1. EPOXY COATING

2. 63Sn/37Pb EUTECTIC BUMP

3. RECOMMEND  NON-SOLDER  MASK DEFINED LANDING PAD.

4. PIN 1 IS ESTABLISHED BY LOWER LEFT CORNER WITH RESPECT TO TEXT ORIENTATION. REMAINING PINS ARE NUMBERED COUNTER CLOCK- WISE.

5. XXX IN DRAWING NUMBER REPRESENTS PACKAGE SIZE VARIATION WHERE X1 IS PACKAGE WIDTH, X2 IS PACKAGE LENGTH AND X3 IS PACK- AGE HEIGHT.

6.NO JEDEC REGISTRATION  AS OF AUG.1999.

micro SMD, 5 Bump, Package (TPA05 - 170 µm ball) NS Package Number TPA05

The dimensions  for X1, X2 and X3 are as given: X1 = 0.930 +/− 0.030mm

X2 = 1.107 +/− 0.030mm

X3 = 0.500 +/− 0.075mm

www.national.com                                                                                                  18

LP2985

NOTES: UNLESS OTHERWISE SPECIFIED

1. EPOXY COATING

2. 63Sn/37Pb EUTECTIC BUMP

3. RECOMMEND  NON-SOLDER  MASK DEFINED LANDING PAD.

4. PIN 1 IS ESTABLISHED BY LOWER LEFT CORNER WITH RESPECT TO TEXT ORIENTATION. REMAINING PINS ARE NUMBERED COUNTER CLOCK- WISE.

5. XXX IN DRAWING NUMBER REPRESENTS PACKAGE SIZE VARIATION WHERE X1 IS PACKAGE WIDTH, X2 IS PACKAGE LENGTH AND X3 IS PACK- AGE HEIGHT.

6. NO JEDEC REGISTRATION  AS OF AUG.1999

micro SMD, 5 Bump, Package (BLA05 - 300 µm ball) NS Package Number BLA05A

The dimensions  for X1, X2 and X3 are as given: X1 = 1.057 +/− 0.030mm

X2 = 1.412 +/− 0.030mm

X3 = 0.945 +/− 0.100mm

19                                                                                                 www.national.com

LP2985

NOTES: UNLESS OTHERWISE SPECIFIED

1. EPOXY COATING

2. 63Sn/37Pb EUTECTIC BUMP

3. RECOMMEND  NON-SOLDER  MASK DEFINED LANDING PAD.

4. PIN 1 IS ESTABLISHED BY LOWER LEFT CORNER WITH RESPECT TO TEXT ORIENTATION. REMAINING PINS ARE NUMBERED COUNTER CLOCK- WISE.

5. XXX IN DRAWING NUMBER REPRESENTS PACKAGE SIZE VARIATION WHERE X1 IS PACKAGE WIDTH, X2 IS PACKAGE LENGTH AND X3 IS PACK- AGE HEIGHT.

6.NO JEDEC REGISTRATION  AS OF AUG.1999.

micro SMD, 5 Bump, Package (TLA05 - 300 µm ball) NS Package Number TLA05

The dimensions  for X1, X2 and X3 are as given: X1 = 1.031 +/− 0.030mm

X2 = 1.412 +/− 0.030mm

X3 = 0.600 +/− 0.075mm

www.national.com                                                                                                  20

LP2985

Notes

21                                                                                                 www.national.com

Notes

For more National Semiconductor product information and proven design tools, visit the following Web sites at:

|  | Products |  |  | Design Support |
| --- | --- | --- | --- | --- |
| Amplifiers | www.national.com/amplifiers |  | WEBENCH® Tools | www.national.com/webench |
| Audio | www.national.com/audio |  | App Notes | www.national.com/appnotes |
| Clock and Timing | www.national.com/timing |  | Reference Designs | www.national.com/refdesigns |
| Data Converters | www.national.com/adc |  | Samples | www.national.com/samples |
| Interface | www.national.com/interface |  | Eval Boards | www.national.com/evalboards |
| LVDS | www.national.com/lvds |  | Packaging | www.national.com/packaging |
| Power Management | www.national.com/power |  | Green Compliance | www.national.com/quality/green |
| Switching Regulators |  | www.national.com/switchers | Distributors | www.national.com/contacts |
| LDOs |  | www.national.com/ldo | Quality and Reliability | www.national.com/quality |
| LED Lighting |  | www.national.com/led | Feedback/Support | www.national.com/feedback |
| Voltage Reference |  | www.national.com/vref | Design Made Easy | www.national.com/easy |

PowerWise®  Solutions          www.national.com/powerwise     Solutions                                  www.national.com/solutions

Serial Digital Interface (SDI)  www.national.com/sdi                  Mil/Aero                                   www.national.com/milaero

Temperature  Sensors            www.national.com/tempsensors  SolarMagic™                           www.national.com/solarmagic

Wireless (PLL/VCO)               www.national.com/wireless         PowerWise®  Design

University

www.national.com/training

THE CONTENTS OF THIS DOCUMENT ARE PROVIDED IN CONNECTION WITH NATIONAL SEMICONDUCTOR CORPORATION (“NATIONAL”) PRODUCTS. NATIONAL MAKES NO REPRESENTATIONS OR WARRANTIES WITH RESPECT TO THE ACCURACY OR COMPLETENESS  OF THE  CONTENTS  OF THIS  PUBLICATION  AND  RESERVES  THE  RIGHT  TO MAKE  CHANGES  TO SPECIFICATIONS  AND  PRODUCT  DESCRIPTIONS  AT  ANY  TIME  WITHOUT  NOTICE.  NO  LICENSE,  WHETHER  EXPRESS, IMPLIED, ARISING BY ESTOPPEL OR OTHERWISE, TO ANY INTELLECTUAL PROPERTY RIGHTS IS GRANTED BY THIS DOCUMENT.

TESTING  AND OTHER  QUALITY  CONTROLS  ARE USED  TO THE EXTENT  NATIONAL  DEEMS  NECESSARY  TO SUPPORT NATIONAL’S  PRODUCT  WARRANTY.  EXCEPT  WHERE  MANDATED  BY GOVERNMENT  REQUIREMENTS,  TESTING  OF ALL PARAMETERS   OF  EACH  PRODUCT   IS  NOT  NECESSARILY   PERFORMED.   NATIONAL   ASSUMES   NO  LIABILITY   FOR APPLICATIONS  ASSISTANCE  OR BUYER  PRODUCT  DESIGN.  BUYERS  ARE  RESPONSIBLE  FOR  THEIR  PRODUCTS  AND APPLICATIONS  USING  NATIONAL  COMPONENTS.  PRIOR  TO USING  OR DISTRIBUTING  ANY PRODUCTS  THAT  INCLUDE NATIONAL COMPONENTS, BUYERS SHOULD PROVIDE ADEQUATE DESIGN, TESTING AND OPERATING SAFEGUARDS.

EXCEPT AS PROVIDED IN NATIONAL’S TERMS AND CONDITIONS OF SALE FOR SUCH PRODUCTS, NATIONAL ASSUMES NO LIABILITY  WHATSOEVER,  AND NATIONAL  DISCLAIMS  ANY EXPRESS  OR IMPLIED  WARRANTY  RELATING  TO THE SALE AND/OR USE OF NATIONAL PRODUCTS INCLUDING LIABILITY OR WARRANTIES RELATING TO FITNESS FOR A PARTICULAR PURPOSE, MERCHANTABILITY,  OR INFRINGEMENT  OF ANY PATENT, COPYRIGHT OR OTHER INTELLECTUAL  PROPERTY RIGHT.

LIFE SUPPORT POLICY

NATIONAL’S  PRODUCTS ARE NOT AUTHORIZED  FOR USE AS CRITICAL COMPONENTS  IN LIFE SUPPORT DEVICES OR SYSTEMS WITHOUT THE EXPRESS PRIOR WRITTEN APPROVAL OF THE CHIEF EXECUTIVE OFFICER AND GENERAL COUNSEL OF NATIONAL SEMICONDUCTOR CORPORATION. As used herein:

Life support devices or systems are devices which (a) are intended for surgical implant into the body, or (b) support or sustain life and whose failure to perform when properly used in accordance with instructions for use provided in the labeling can be reasonably expected to result in a significant injury to the user. A critical component is any component in a life support device or system whose failure to perform can be reasonably expected to cause the failure of the life support device or system or to affect its safety or effectiveness.

National Semiconductor and the National Semiconductor logo are registered trademarks of National Semiconductor Corporation. All other brand or product names may be trademarks or registered trademarks of their respective holders.

Copyright© 2009 National Semiconductor Corporation

LP2985 Micropower 150 mA Low-Noise Ultra Low-Dropout Regulator in SOT-23 and micro SMD Packages

For the most current product information visit us at www.national.com

National Semiconductor Americas Technical Support Center

Email: support@nsc.com

Tel: 1-800-272-9959

National Semiconductor Europe

Technical Support Center

Email: europe.support@nsc.com

National Semiconductor Asia Pacific Technical Support Center Email: ap.support@nsc.com

National Semiconductor Japan Technical Support Center Email: jpn.feedback@nsc.com

www.national.com