LP2995

DDR Termination Regulator

General Description

The LP2995 linear regulator is designed to meet the JEDEC SSTL-2 and SSTL-3 specifications  for termination  of DDR- SDRAM. The device contains a high-speed operational am- plifier to provide excellent response to load transients.  The output  stage  prevents  shoot  through  while delivering  1.5A continuous current and transient peaks up to 3A in the application  as required  for DDR-SDRAM  termination.  The

LP2995 DDR Termination Regulator

June 2006

Features

n  Low output voltage offset

n  Works with +5v, +3.3v and 2.5v rails

n  Source and sink current

n  Low external component count

n  No external resistors required

n  Linear topology

Available in SO-8, PSOP-8 or LLP-16 packages

LP2995 also incorporates  a VSENSE pin to provide superior load  regulation  and  a VREF  output  as a reference  for the chipset and DDR DIMMS.

Patents Pending

n

n  Low cost and easy to use

Applications

n  DDR Termination  Voltage

n  SSTL-2

n  SSTL-3

Typical Application Circuit

20039302

© 2006 National Semiconductor Corporation       DS200393                                                                                                                    www.national.com

| Connection Diagrams SO-8 (M08A) Package LQA- 16 Package 20039320 Top View 20039304 Top View PSOP-8 (MRA08A) Package 20039350 Top View Pin Descriptions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SO-8 Pin or PSOP-8 Pin | LLP Pin |  | Name |  | Function |  |
| 1 | 1,3,4,6,9, 13,16 |  | NC |  | No internal connection. Can be used for vias. |  |
| 2 | 2 |  | GND |  | Ground. |  |
| 3 | 5 |  | VSENSE |  | Feedback pin for regulating VTT. |  |
| 4 | 7 |  | VREF |  | Buffered internal reference voltage of VDDQ/2. |  |
| 5 | 8 |  | VDDQ |  | Input for internal reference equal to VDDQ/2. |  |
| 6 | 10 |  | AVIN |  | Analog input pin. |  |
| 7 | 11, 12 |  | PVIN |  | Power input pin. |  |
| 8 | 14, 15 |  | VTT |  | Output voltage for connection to termination resistors. |  |
|  | EP |  | EP |  | Exposed pad thermal connection. Connect to soft Ground. |  |
| Ordering Information |  |  |  |  |  |  |
| Order Number |  | Package Type |  | NSC Package Drawing |  | Supplied As |
| LP2995M |  | SO-8 |  | M08A |  | 95 Units per Rail |
| LP2995MX |  | SO-8 |  | M08A |  | 2500 Units Tape and Reel |
| LP2995MR |  | PSOP-8 |  | MRA08A |  | 95 Units per Rail |
| LP2995MRX |  | PSOP-8 |  | MRA08A |  | 2500 Units Tape and Reel |
| LP2995LQ |  | LLP-16 |  | LQA16A |  | 1000 Units Tape and Reel |
| LP2995LQX |  | LLP-16 |  | LQA16A |  | 4500 Units Tape and Reel |

LP2995

www.national.com                                                                                                   2

| Absolute Maximum Ratings (Note 1) LLP-16 Thermal Resistance (θJA) 51˚C/W If Military/Aerospace specified devices are required, Lead Temperature (Soldering, 10 sec) 260˚C please contact the National Semiconductor Sales Office/ ESD Rating (Note 7) 1kV Distributors for availability and specifications. PVIN, AVIN, VDDQ to GND −0.3V to +6V Operating Range Storage Temp. Range −65˚C to +150˚C Junction Temperature 150˚C Junction Temp. Range (Note 5) 0˚C to +125˚C PSOP-8 Thermal Resistance (θJA) 43˚C/W AVIN to GND 2.2V to 5.5V SO-8 Thermal Resistance (θJA) 151˚C/W PVIN to GND 2.2V to AVIN Electrical Characteristics Specifications with standard typeface are for TJ = 25˚C and limits in boldface type apply over the full Operating Temperature Range (TJ = 0˚C to +125˚C). Unless otherwise specified, AVIN = PVIN = 2.5V, VDDQ = 2.5V (Note 6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Symbol | Parameter | Conditions | Min | Typ | Max | Units |
| VREF | VREF Voltage | IREF_OUT = 0mA | 1.21 | 1.235 | 1.26 | V |
| VOSVTT | VTT Output Voltage Offset | IOUT = 0A (Note 2) | −15 −20 | 0 | 15 20 | mV |
| ∆VTT/VTT | Load Regulation (Note 3) | IOUT = 0 to 1.5A |  | 0.5 |  | % |
|  |  | IOUT = 0 to −1.5A |  | −0.5 |  |  |
| ZVREF | VREF Output Impedance | IREF = −5µA to +5µA |  | 5 |  | kΩ |
| ZVDDQ | VDDQ Input Impedance |  |  | 100 |  | kΩ |
| Iq | Quiescent Current | IOUT = 0A (Note 4) |  | 250 | 400 | µA |
| Note 1: Absolute maximum ratings indicate limits beyond which damage to the device may occur. Operating range indicates conditions for which the device is intended to be functional, but does not guarantee specific performance limits. For guaranteed specifications and test conditions see Electrical Characteristics. The guaranteed specifications apply only for the test conditions listed. Some performance characteristics may degrade when the device is not operated under the listed test conditions. Note 2: VTT offset is the voltage measurement defined as VTT subtracted from VREF. Note 3: Load regulation is tested by using a 10ms current pulse and measuring VTT. Note 4: Quiescent current defined as the current flow into AVIN. Note 5: At elevated temperatures, devices must be derated based on thermal resistance. The device in the SO-8 package must be derated at θJA = 151˚ C/W junction to ambient with no heat sink. The device in the LLP-16 must be derated at θJA = 51˚ C/W junction to ambient. Note 6: Limits are 100% production tested at 25˚C. Limits over the operating temperature range are guaranteed through correlation using Statistical Quality Control (SQC) methods. The limits are used to calculate National’s Average Outgoing Quality Level (AOQL). Note 7: The human body model is a 100pF capacitor discharged through a 1.5kΩ resistor into each pin. |  |  |  |  |  |  |

LP2995

3                                                                                                  www.national.com

LP2995

Typical Performance Characteristics

Iq vs VIN (25˚C)                                                                    Iq vs Temperature  ( VIN = 2.5V)

20039309                                                                                                                                                                                   20039310

Iq vs VIN (0, 25, 85, and 125˚C)                                                                        VREF vs IREF

20039311                                                                                                                                                                                 20039312

VREF vs Temperature  (No Load)                                                    VTT vs IOUT (0, 25, 85, and 125˚C)

20039313                                                                                                                                                                                 20039314

www.national.com                                                                                                   4

Typical Performance Characteristics  (Continued)

VTT vs IOUT

20039315

Maximum Output Current (Sinking) vs VIN

(VDDQ = 2.5)

20039317

LP2995

Maximum Output Current (Sourcing) vs VIN

(VDDQ = 2.5)

20039316

5                                                                                                  www.national.com

LP2995

Block Diagram

20039301

Description

The LP2995 is a linear bus termination regulator designed to meet the JEDEC requirements  of SSTL-2 and SSTL-3. The LP2995  is capable  of sinking  and sourcing  current  at the output  VTT, regulating  the  voltage  to  equal  VDDQ  / 2. A buffered reference voltage that also tracks VDDQ / 2 is generated on the VREF pin for providing a global reference to the DDR-SDRAM  and Northbridge Chipset. VTT is designed to  track  the  VREF  voltage  with  a tight  tolerance  over  the entire current range while preventing  shoot through on the output stage.

Series  Stub Termination  Logic  (SSTL)  was created  to im- prove  signal  integrity  of the  data  transmission  across  the memory bus. This termination scheme is essential to prevent data error from signal reflections  while transmitting  at high frequencies encountered with DDR RAM. The most common form of termination is Class II single parallel termination. This involves using one Rs series resistor from the chipset to the memory  and one Rt termination  resistor. This implementa- tion can be seen below in Figure 1.

20039308

FIGURE 1.

Typical values for RS  and RT  are 25 Ohms although these can be changed to scale the current requirements  from the LP2995.  For determination  of the current  requirements  of DDR-SDRAM  termination please refer to the accompanying application notes.

www.national.com                                                                                                   6

Pin Descriptions

AVIN AND PVIN

AVIN and PVIN are the input supply  pins for the LP2995. AVIN is used to supply all the internal control circuitry for the two op-amps  and the output  stage of VREF. PVIN is used exclusively to provide the rail voltage for the output stage on the  power  operational  amplifier  used  to  create  VTT.  For SSTL-2  applications  AVIN  and  PVIN  pins  should  be con- nected  directly  and tied to the 2.5V rail for optimal  perfor- mance. This eliminates the need for bypassing the two sup- ply pins separately.

VDDQ

VDDQ is the input that is used to create the internal refer- ence voltage  for regulating  VTT  and VREF. This voltage  is generated  by two internal  50kΩ  resistors.  This guarantees that VTT and VREF will track VDDQ / 2 precisely. The optimal implementation  of VDDQ is as a remote sense for the refer- ence  input.  This  can  be  achieved  by  connecting  VDDQ directly to the 2.5V rail at the DIMM. This ensures that the reference voltage tracks the DDR memory rails precisely without  a  large  voltage  drop  from  the  power  lines.  For SSTL-2 applications  VDDQ will be a 2.5V signal, which will create a 1.25V reference voltage on VREF and a 1.25V termination  voltage at VTT. For SSTL-3 applications  it may be desirable to have a different scaling factor for creating the internal reference voltage besides 0.5. For instance a typical value that is commonly used is to have the reference voltage equal VDDQ*0.45. This can be achieved by placing a resis- tor in series  with  the VDDQ  pin to effectively  change  the resistor divider.

VSENSE

The purpose of the sense pin is to provide improved remote load regulation. In most motherboard applications the termi- nation  resistors  will connect  to VTT  in a long plane.  If the output  voltage  was  regulated  only  at  the  output  of  the LP2995, then the long trace will cause a significant IR drop, resulting in a termination voltage lower at one end of the bus than the other. The VSENSE pin can be used to improve this performance,  by connecting it to the middle of the bus. This will provide a better distribution across the entire termination bus.

Note: If remote load regulation is not used, then the VSENSE  pin must still be connected to VTT.

VREF

VREF provides the buffered output of the internal reference voltage VDDQ / 2. This output should be used to provide the reference  voltage for the Northbridge  chipset and memory. Since  these  inputs  are typically  an extremely  high imped- ance,  there  should  be little  current  drawn  from  VREF.  For improved  performance,  an output bypass capacitor  can be used, located close to the pin, to help with noise. A ceramic capacitor in the range of 0.1 µF to 0.01 µF is recommended.

VTT

VTT is the regulated output that is used to terminate the bus resistors. It is capable of sinking and sourcing current while regulating the output precisely to VDDQ / 2. The LP2995 is designed to handle peak transient currents of up to ± 3A with a fast transient response. The maximum continuous current is a function of VIN and can be viewed in the TYPICAL PERFORMANCE   CHARACTERISTICS section.  If  a  tran- sient  is  expected  to  last  above  the  maximum  continuous

LP2995

current rating for a significant amount of time then the output capacitor should be sized large enough to prevent an exces- sive voltage  drop. Despite  the fact that the LP2995  is de- signed  to  handle  large  transient  output  currents  it  is  not capable of handling these for long durations, under all con- ditions. The reason for this is the standard packages are not able to thermally dissipate the heat as a result of the internal power  loss. If large  currents  are required  for longer  dura- tions, then care should be taken to ensure that the maximum junction temperature is not exceeded. Proper thermal derat- ing should always be used (please refer to the Thermal Dissipation section).

Component Selection

INPUT CAPACITOR

The LP2995 does not require a capacitor for input stability, but  it  is  recommended   for  improved  performance  during large load transients to prevent the input rail from dropping. The input capacitor should be located as close as possible to the PVIN pin. Several recommendations  exist dependent on the application required. A typical value recommended for AL electrolytic capacitors is 50 µF. Ceramic capacitors can also be used, a value in the range of 10 µF with X5R or better would be an ideal choice. The input capacitance can be reduced  if the LP2995  is placed  close to the bulk capaci- tance from the output of the 2.5V DC-DC converter.

OUTPUT CAPACITOR

The LP2995 has been designed to be insensitive  of output capacitor size or ESR (Equivalent  Series Resistance).  This allows the flexibility to use any capacitor desired. The choice for output capacitor will be determined solely on the applica- tion and the requirements for load transient response of VTT. As a general  recommendation  the output capacitor  should be sized above 100 µF with a low ESR for SSTL applications with DDR-SDRAM. The value of ESR should be determined by the maximum current spikes expected and the extent at which the output voltage is allowed to droop. Several capaci- tor options are available on the market and a few of these are highlighted below:

AL - It should be noted that many aluminum electrolytics only specify impedance at a frequency of 120 Hz, which indicates they have poor high frequency performance. Only aluminum electrolytics  that have an impedance  specified  at a higher frequency (between 20 kHz and 100 kHz) should be used for the LP2995. To improve the ESR several AL electrolytics can be combined in parallel for an overall reduction. An important note  to  be  aware  of  is  the  extent  at  which  the  ESR  will change over temperature.  Aluminum  electrolytic  capacitors can have their ESR rapidly increase at cold temperatures.

Ceramic  - Ceramic  capacitors  typically  have a low capaci- tance,  in the range  of 10 to 100 µF range,  but they have excellent AC performance  for bypassing  noise because  of very low ESR (typically  less than 10 mΩ). However,  some dielectric  types do not have good capacitance  characteris- tics as a function of voltage and temperature. Because of the typically low value of capacitance it is recommended  to use ceramic capacitors in parallel with another capacitor such as an  aluminum  electrolytic.  A dielectric  of  X5R  or  better  is recommended  for all ceramic capacitors.

Hybrid - Several hybrid capacitors such as OS-CON and SP are  available  from  several  manufacturers.   These  offer  a large capacitance  while maintaining  a low ESR. These are

7                                                                                                  www.national.com

Component Selection  (Continued)

LP2995

the best solution when size and performance are critical, although their cost is typically higher than any other capaci- tor.

Capacitor recommendations  for different application circuits can be seen in the accompanying application notes with supporting evaluation boards.

Thermal Dissipation

Since the LP2995 is a linear regulator any current flow from VTT will result in internal power dissipation generating heat. To prevent damaging the part from exceeding the maximum allowable  junction  temperature,  care  should  be  taken  to derate the part dependent on the maximum expected ambi- ent temperature and power dissipation. The maximum allow- able  internal  temperature  rise  (TRmax) can  be  calculated given  the  maximum  ambient  temperature  (TAmax) of  the application and the maximum allowable junction temperature (TJmax).

the DAP the θJA can be lowered significantly. Figure 3 shows the  LLP  thermal  data  when  placed  on  a  4-layer  JEDEC board with copper thickness of 0.5/1/1/0.5 oz. The number of vias,  with a pitch  of 1.27 mm, has been  increased  to the maximum  of 4 where a θJA of 50.41˚C/W  can be obtained. Via wall thickness  for this calculation  is 0.036 mm for 1oz. Copper.

TRmax = TJmax − TAmax

From this equation, the maximum power dissipation (PDmax)

of the part can be calculated:

PDmax = TRmax / θJA

The θJA  of the LP2995  will be dependent  on several  vari- ables: the package used; the thickness of copper; the num- ber of vias and the airflow. For instance, the θJA of the SO-8 is 163˚C/W  with  the package  mounted  to a standard  8x4

2-layer board with 1oz. copper, no airflow, and 0.5W dissi- pation at room temperature.  This value can be reduced  to

151.2˚C/W by changing to a 3x4 board with 2 oz. copper that is the JEDEC standard. Figure 2 shows how the θJA  varies with airflow for the two boards mentioned.

20039321

FIGURE 2. θJA vs Airflow (SO-8)

Layout  is  also  extremely  critical  to  maximize  the  output current with the LLP package. By simply placing vias under

20039322

FIGURE 3. LLP-16 θJA vs # of Vias (4 Layer JEDEC Board))

Additional improvements in lowering the θJA can also be achieved with a constant airflow across the package. Main- taining the same conditions  as above and utilizing the 2x2 via array, Figure 4 shows how the θJA varies with airflow.

20039323

FIGURE 4. θJA vs Airflow Speed (JEDEC Board with 4

Vias)

www.national.com                                                                                                   8

Typical Application Circuits

The typical  application  circuit used for SSTL-2  termination schemes with DDR-SDRAM  can be seen in Figure 5.

20039306

FIGURE 5. SSTL-2 Implementation

For SSTL-3  and other  applications  it may be desirable  to change internal reference voltage scaling from VDDQ * 0.5. An external resistor in series with the VDDQ pin can be used to lower the reference voltage. Internally two 50 kΩ resistors

LP2995

set the output VTT to be equal to VDDQ * 0.5. The addition of a 11.1 kΩ external resistor will change the internal reference voltage causing  the two outputs to track VDDQ * 0.45. An implementation  of this circuit can be seen in Figure 6.

20039307

FIGURE 6. SSTL-3 Implementation

Another application that is sometimes required is to increase the VTT output voltage from the scaling factor of VDDQ * 0.5. This can be accomplished independently of VREF by using a

resistor divider network between VTT, VSENSE  and Ground. An example of this circuit can be seen in Figure 7.

20039303

FIGURE 7.

9                                                                                                  www.national.com

LP2995

PCB Layout Considerations

1.   AVIN and PVIN should be tied together for optimal per- formance. A local bypass capacitor should be placed as close as possible to the PVIN pin.

2.   GND should be connected  to a ground plane with mul- tiple vias for improved thermal performance.

3.   VSENSE should be connected to the VTT termination bus at the point  where  regulation  is required.  For mother- board  applications  an  ideal  location  would  be  at  the

center of the termination bus.

4.   VDDQ  can  be  connected  remotely  to  the  VDDQ  rail input at either the DIMM or the Chipset. This provides the most accurate point for creating the reference  volt- age.

5.   VREF  should  be  bypassed  with  a  0.01  µF  or  0.1  µF ceramic  capacitor  for improved  performance.  This  ca- pacitor  should  be located  as close  as possible  to the VREF  pin.

www.national.com                                                                                                  10

Physical Dimensions  inches (millimeters)  unless otherwise  noted

LP2995

8-Lead Small Outline Package (M8) NS Package Number M08A

16-Lead LLP Package (LD) NS Package Number LQA16A

11                                                                                                 www.national.com

LP2995 DDR Termination Regulator

Physical Dimensions inches (millimeters) unless otherwise noted (Continued) 8-Lead PSOP Package (PSOP-8) NS Package Number MRA08A

National does not assume any responsibility for use of any circuitry described, no circuit patent licenses are implied and National reserves the right at any time without notice to change said circuitry and specifications. For the most current product information visit us at www.national.com.

LIFE SUPPORT POLICY NATIONAL’S PRODUCTS ARE NOT AUTHORIZED FOR USE AS CRITICAL COMPONENTS IN LIFE SUPPORT DEVICES OR SYSTEMS WITHOUT THE EXPRESS WRITTEN APPROVAL OF THE PRESIDENT AND GENERAL COUNSEL OF NATIONAL SEMICONDUCTOR CORPORATION. As used herein: 1. Life support devices or systems are devices or systems 2. A critical component is any component of a life support which, (a) are intended for surgical implant into the body, or device or system whose failure to perform can be reasonably (b) support or sustain life, and whose failure to perform when expected to cause the failure of the life support device or properly used in accordance with instructions for use system, or to affect its safety or effectiveness. provided in the labeling, can be reasonably expected to result in a significant injury to the user.

BANNED SUBSTANCE COMPLIANCE National Semiconductor follows the provisions of the Product Stewardship Guide for Customers (CSP-9-111C2) and Banned Substances and Materials of Interest Specification (CSP-9-111S2) for regulatory environmental compliance. Details may be found at: www.national.com/quality/green. Lead free products are RoHS compliant.

National Semiconductor National Semiconductor National Semiconductor National Semiconductor Americas Customer Europe Customer Support Center Asia Pacific Customer Japan Customer Support Center Support Center Fax: +49 (0) 180-530 85 86 Support Center Fax: 81-3-5639-7507 Email: new.feedback@nsc.com Email: europe.support@nsc.com Email: ap.support@nsc.com Email: jpn.feedback@nsc.com Tel: 1-800-272-9959 Deutsch Tel: +49 (0) 69 9508 6208 Tel: 81-3-5639-7560 English Tel: +44 (0) 870 24 0 2171 www.national.com Français Tel: +33 (0) 1 41 91 8790