INTEGRATED CIRCUITS

For a complete data sheet, please also download:

The IC06 74HC/HCT/HCU/HCMOS Logic Family Specifications The IC06 74HC/HCT/HCU/HCMOS Logic Package Information The IC06 74HC/HCT/HCU/HCMOS Logic Package Outlines

74HC/HCT273

Octal D-type flip-flop with reset;

positive-edge trigger

Product specification

File under Integrated Circuits, IC06

September 1993

Philips Semiconductors                                                                                                              Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                              74HC/HCT273

FEATURES

Ideal buffer for MOS microprocessor or memory

Common clock and master reset

Eight positive edge-triggered D-type flip-flops

See “377” for clock enable version

See “373” for transparent latch version

See “374” for 3-state version Output capability; standard ICC category: MSI

GENERAL DESCRIPTION

The 74HC/HCT273 are high-speed Si-gate CMOS devices and are pin compatible with low power Schottky TTL (LSTTL). They are specified in compliance with JEDEC standard no. 7A.

The 74HC/HCT273 have eight edge-triggered, D-type flip-flops with individual D inputs and Q outputs. The common clock (CP) and master reset (MR) inputs load and reset (clear) all flip-flops simultaneously.

The state of each D input, one set-up time before the LOW-to-HIGH clock transition, is transferred to the corresponding output (Qn) of the flip-flop.

All outputs will be forced LOW independently of clock or data inputs by a LOW voltage level on the MR input.

The device is useful for applications where the true output only is required and the clock and master reset are common to all storage elements.

QUICK REFERENCE DATA

GND = 0 V; Tamb = 25  C; tr = tf = 6 ns

| SYMBOL | PARAMETER | CONDITIONS | TYPICAL |  | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | HC | HCT |  |
| tPHL/ tPLH | propagation delay CP to Qn MR to Qn | CL = 15 pF; VCC = 5 V | 15 15 | 15 20 | ns ns |
| fmax | maximum clock frequency |  | 66 | 36 | MHz |
| CI | input capacitance |  | 3.5 | 3.5 | pF |
| CPD | power dissipation capacitance per flip-flop | notes 1 and 2 | 20 | 23 | pF |

Notes

1.   CPD is used to determine the dynamic power dissipation (PD in   W):

2     f        (C

V   2      f ) where:

PD = CPD     VCC        i

L        CC         o

fi = input frequency in MHz

fo = output frequency in MHz

(CL     VCC2     fo) = sum of outputs CL = output load capacitance in pF VCC = supply voltage in V

2.   For HC  the condition is VI = GND to VCC

For HCT the condition is VI = GND to VCC     1.5 V

ORDERING INFORMATION

See “74HC/HCT/HCU/HCMOS Logic Package Information”.

September 1993                                                                  2

Philips Semiconductors                                                                                                                     Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                                 74HC/HCT273

PIN DESCRIPTION

| PIN NO. | SYMBOL | NAME AND FUNCTION |
| --- | --- | --- |
| 1 2, 5, 6, 9, 12, 15, 16, 19 3, 4, 7, 8, 13, 14, 17, 18 10 11 20 | MR Q0 to Q7 D0 to D7 GND CP VCC | master reset input (active LOW) flip-flop outputs data inputs ground (0 V) clock input (LOW-to-HIGH, edge-triggered) positive supply voltage |

Fig.1 Pin configuration.                             Fig.2 Logic symbol.                             Fig.3 IEC logic symbol.

September 1993                                                                  3

Philips Semiconductors                                                                                                                     Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                                 74HC/HCT273

FUNCTION TABLE

| OPERATING MODES | INPUTS |  |  | OUTPUTS |
| --- | --- | --- | --- | --- |
|  | MR | CP | Dn | Qn |
| reset (clear) | L | X | X | L |
| load “1” | H |  | h | H |
| load “0” | H |  | I | L |

Note

1.   H = HIGH voltage level

h = HIGH voltage level one set-up time prior to the

LOW-to-HIGH CP transition

L = LOW voltage level

I = LOW voltage level one set-up time prior to the

LOW-to-HIGH CP transition

= LOW-to-HIGH transition

X = don’t care

Fig.4 Functional diagram.

Fig.5 Logic diagram.

September 1993                                                                  4

Philips Semiconductors                                                                                                                     Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                                 74HC/HCT273

DC CHARACTERISTICS FOR 74HC

For the DC characteristics see “74HC/HCT/HCU/HCMOS Logic Family Specifications”. Output capability: standard

ICC category: MSI

AC CHARACTERISTICS FOR 74HC

GND = 0 V; tr = tf = 6 ns; CL = 50 pF

| SYMBOL | PARAMETER | Tamb ( C) |  |  |  |  |  |  | UNIT | TEST CONDITIONS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 74HC |  |  |  |  |  |  |  | VCC (V) | WAVEFORMS |
|  |  | 25 |  |  | 40 to 85 |  | 40 to 125 |  |  |  |  |
|  |  | min. | typ. | max. | min. | max. | min. | max. |  |  |  |
| tPHL/ tPLH | propagation delay CP to Qn |  | 41 15 13 | 150 30 26 |  | 185 37 31 |  | 225 45 38 | ns | 2.0 4.5 6.0 | Fig.6 |
| tPHL | propagation delay MR to Qn |  | 44 16 14 | 150 30 26 |  | 185 37 31 |  | 225 45 38 | ns | 2.0 4.5 6.0 | Fig.7 |
| tTHL/ tTLH | output transition time |  | 19 7 6 | 75 15 13 |  | 95 19 15 |  | 110 22 19 | ns | 2.0 4.5 6.0 | Fig.6 |
| tW | clock pulse width HIGH or LOW | 80 16 14 | 14 5 4 |  | 100 20 17 |  | 120 24 20 |  | ns | 2.0 4.5 6.0 | Fig.6 |
| tW | master reset pulse width LOW | 60 12 10 | 17 6 5 |  | 75 15 13 |  | 90 18 15 |  | ns | 2.0 4.5 6.0 | Fig.7 |
| trem | removal time MR to CP | 50 10 9 | 6 2 2 |  | 65 13 11 |  | 75 15 13 |  | ns | 2.0 4.5 6.0 | Fig.7 |
| tsu | set-up time Dn to CP | 60 12 10 | 11 4 3 |  | 75 15 13 |  | 90 18 15 |  | ns | 2.0 4.5 6.0 | Fig.8 |
| th | hold time Dn to CP | 3 3 3 | 6 2 2 |  | 3 3 3 |  | 3 3 3 |  | ns | 2.0 4.5 6.0 | Fig.8 |
| fmax | maximum clock pulse frequency | 6.0 30 35 | 20.6 103 122 |  | 4.8 24 28 |  | 4.0 20 24 |  | MHz | 2.0 4.5 6.0 | Fig.6 |

September 1993                                                                  5

Philips Semiconductors                                                                                                                     Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                                 74HC/HCT273

DC CHARACTERISTICS FOR 74HCT

For the DC characteristics see “74HC/HCT/HCU/HCMOS Logic Family Specifications”. Output capability: standard

ICC category: MSI

Note to HCT types

The value of additional quiescent supply current (  ICC) for a unit load of 1 is given in the family specifications. To determine   ICC per input, multiply this value by the unit load coefficient shown in the table below.

| INPUT | UNIT LOAD COEFFICIENT |
| --- | --- |
| MR CP Dn | 1.00 1.75 0.15 |

AC CHARACTERISTICS FOR 74HCT

GND = 0 V; tr = tf = 6 ns; CL = 50 pF

| SYMBOL | PARAMETER | Tamb ( C) |  |  |  |  |  |  | UNIT | TEST CONDITIONS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 74HCT |  |  |  |  |  |  |  | VCC (V) | WAVEFORMS |
|  |  | 25 |  |  | 40 to 85 |  | 40 to 125 |  |  |  |  |
|  |  | min. | typ. | max. | min. | max. | min. | max. |  |  |  |
| tPHL/ tPLH | propagation delay CP to Qn |  | 16 | 30 |  | 38 |  | 45 | ns | 4.5 | Fig.6 |
| tPHL | propagation delay MR to Qn |  | 23 | 34 |  | 43 |  | 51 | ns | 4.5 | Fig.7 |
| tTHL/ tTLH | output transition time |  | 7 | 15 |  | 19 |  | 22 | ns | 4.5 | Fig.6 |
| tW | clock pulse width HIGH or LOW | 16 | 9 |  | 20 |  | 24 |  | ns | 4.5 | Fig.6 |
| tW | master reset pulse width LOW | 16 | 8 |  | 20 |  | 24 |  | ns | 4.5 | Fig.7 |
| trem | removal time MR to CP | 10 | 2 |  | 13 |  | 15 |  | ns | 4.5 | Fig.7 |
| tsu | set-up time Dn to CP | 12 | 5 |  | 15 |  | 18 |  | ns | 4.5 | Fig.8 |
| th | hold time Dn to CP | 3 | 4 |  | 3 |  | 3 |  | ns | 4.5 | Fig.8 |
| fmax | maximum clock pulse frequency | 30 | 56 |  | 24 |  | 20 |  | MHz | 4.5 | Fig.6 |

September 1993                                                                  6

Philips Semiconductors                                                                                                                     Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                                 74HC/HCT273

AC WAVEFORMS

(1)  HC  : VM = 50%; VI = GND to VCC.

HCT: VM = 1.3 V; VI = GND to 3 V.

Fig.6    Waveforms showing the clock (CP) to output (Qn) propagation delays, the clock pulse width output transition times and the maximum clock pulse frequency.

(1)  HC  : VM = 50%; VI = GND to VCC.

HCT: VM = 1.3 V; VI = GND to 3 V.

Fig.7    Waveforms showing the master reset (MR) pulse width, the master reset to output (Qn) propagation delays and the master reset to clock (CP) removal time.

The shaded areas indicate when the input is permitted to change for predictable output performance.

(1)  HC  : VM = 50%; VI = GND to VCC.

HCT: VM = 1.3 V; VI = GND to 3 V.

Fig.8 Waveforms showing the data set-up and hold times for the data input (Dn).

September 1993                                                                  7

Philips Semiconductors                                                                                                                     Product specification

Octal D-type flip-flop with reset;

positive-edge trigger                                                                 74HC/HCT273

PACKAGE OUTLINES

See “74HC/HCT/HCU/HCMOS Logic Package Outlines”.

September 1993                                                                  8