HCF4094B

8 STAGE SHIFT AND STORE BUS REGISTER WITH 3-STATE OUTPUTS

■       3- STATE PARALLEL  OUTPUTS FOR CONNECTION TO COMMON BUS

■       SEPARATE SERIAL OUTPUTS SYNCHRONOUS  TO BOTH POSITIVE AND NEGATIVE CLOCK EDGES FOR CASCADING

■       MEDIUM SPEED OPERATION 5MHz at 10V

■       QUIESCENT CURRENT SPECIFIED UP TO

20V

■       STANDARDIZED SYMMETRICAL OUTPUT CHARACTERISTICS

■       5V, 10V AND 15V PARAMETRIC RATINGS

■       INPUT LEAKAGE CURRENT

II = 100nA (MAX) AT VDD = 18V TA = 25°C

■       100% TESTED FOR QUIESCENT CURRENT

■       MEETS ALL REQUIREMENTS OF JEDEC JESD13B " STANDARD SPECIFICATIONS FOR DESCRIPTION OF B SERIES CMOS DEVICES"

DESCRIPTION

The HCF4094B  is a monolithic  integrated  circuit fabricated 	in 	Metal 	Oxide 	Semiconductor technology available in DIP and SOP packages. The HCF4094B is an 8 stages serial shift register having a storage latch associated with each stage for strobing  data from the serial input to parallel buffered 3-state outputs. The parallel outputs may be connected directly to common bus lines. Data

DIP 	SOP

ORDER CODES

| PACKAGE | TUBE | T & R |
| --- | --- | --- |
| DIP | HCF4094BEY |  |
| SOP | HCF4094BM1 | HCF4094M013TR |

is shifted on positive clock transition. The data in each  shift  register  stage  is  transferred  to  the storage register when the STROBE input is high. Data in the storage register appears at the outputs whenever  the OUTPUT-ENABLE  signal  is high. Two serial outputs are available for cascading  a number of HCF4094B  devices. Data is available at the QS serial  output terminal on positive clock

edges   to   allow   for   high   speed   operation   in

cascaded  system in which the clock rise time is fast. The same serial information, available at the Q’S  terminal  on  the  next  negative  clock  edge,

provides   a   means   for   cascading   HCF4094B

devices when the clock rise time is slow.

PIN CONNECTION

HCF4094B

IINPUT EQUIVALENT CIRCUIT 	PIN DESCRIPTION

| PIN No | SYMBOL | NAME AND FUNCTION |
| --- | --- | --- |
| 2 | DATA | Data Input |
| 1 | STROBE | Strobe Input |
| 3 | CLOCK | Clock Input |
| 9, 10 | QS, Q’S | Serial Outputs |
| 4, 5, 6, 7, 14, 13, 12, 11 | Q1 to Q8 | Parallel Outputs |
| 15 | OUTPUT ENABLE | Output Enable Input |
| 8 | VSS | Negative Supply Voltage |
| 16 | VDD | Positive Supply Voltage |

FUNCTIONAL DIAGRAM

TRUTH TABLE

| 1 n S S CLOCK | OUTPUTS ENABLE | STROBE | DATA | PARALLEL OUTPUTS |  | SERIAL OUTPUTS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Q | Q | Q* | Q’ |
|  | L | X | X | OC | OC | Q7 | No Change |
|  | L | X | X | OC | OC | No Change | Q7 |
|  | H | L | X | No Change | No Change | Q7 | No Change |
|  | H | H | L | L | Qn - 1 | Q7 | No Change |
|  | H | H | H | H | Qn - 1 | Q7 | No Change |
|  | H | H | H | No Change | No Change | No Change | Q7 |

X : Don’t Care

OC : Open Circuit

* At the positive clock edge information on the 7th shift register stage is transferred to the 8th register stage and the QS output.

HCF4094B

LOGIC DIAGRAM

TIMING CHART

HCF4094B

ABSOLUTE MAXIMUM RATINGS

| Symbol | Parameter | Value | Unit |
| --- | --- | --- | --- |
| VDD | Supply Voltage | -0.5 to +22 | V |
| VI | DC Input Voltage | -0.5 to VDD + 0.5 | V |
| II | DC Input Current | 10 | mA |
| PD | Power Dissipation per Package | 500 (*) | mW |
|  | Power Dissipation per Output Transistor | 100 | mW |
| Top | Operating Temperature | -55 to +125 | °C |
| Tstg | Storage Temperature | -65 to +150 | °C |

Absolute Maximum Ratings are those values beyond which damage to the device may occur. Functional operation under these conditions is not implied.

All voltage values are referred to VSS pin voltage.

(*) 500mW at 65  C; derate to 300mW by 10mW/ C from 65 C to 85 C

RECOMMENDED OPERATING CONDITIONS

| Symbol | Parameter | Value | Unit |
| --- | --- | --- | --- |
| VDD | Supply Voltage | 3 to 20 | V |
| VI | Input Voltage | 0 to VDD | V |
| Top | Operating Temperature | -55 to 125 | °C |

4/12

HCF4094B

DC SPECIFICATIONS

| Symbol | Parameter | Test Condition |  |  |  | Value |  |  |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VI (V) | VO (V) | ¦IO¦ ( A) | VDD (V) | TA = 25°C |  |  |  | -40 to 85°C |  | -55 to 125°C |  |  |
|  |  |  |  |  |  | Min. | Typ. | Max. | Min. |  | Max. | Min. | Max. |  |
| IL | Quiescent Current | 0/5 |  |  | 5 |  | 0.04 | 5 |  |  | 150 |  | 150 | A |
|  |  | 0/10 |  |  | 10 |  | 0.04 | 10 |  |  | 300 |  | 300 |  |
|  |  | 0/15 |  |  | 15 |  | 0.04 | 20 |  |  | 600 |  | 600 |  |
|  |  | 0/20 |  |  | 20 |  | 0.08 | 100 |  |  | 3000 |  | 3000 |  |
| VOH | High Level Output Voltage | 0/5 |  | <1 | 5 | 4.95 |  |  | 4.95 |  |  | 4.95 |  | V |
|  |  | 0/10 |  | <1 | 10 | 9.95 |  |  | 9.95 |  |  | 9.95 |  |  |
|  |  | 0/15 |  | <1 | 15 | 14.95 |  |  | 14.95 |  |  | 14.95 |  |  |
| VOL | Low Level Output Voltage | 5/0 |  | <1 | 5 |  | 0.05 |  |  |  | 0.05 |  | 0.05 | V |
|  |  | 10/0 |  | <1 | 10 |  | 0.05 |  |  |  | 0.05 |  | 0.05 |  |
|  |  | 15/0 |  | <1 | 15 |  | 0.05 |  |  |  | 0.05 |  | 0.05 |  |
| VIH | High Level Input Voltage |  | 0.5/4.5 | <1 | 5 | 3.5 |  |  | 3.5 |  |  | 3.5 |  | V |
|  |  |  | 1/9 | <1 | 10 | 7 |  |  | 7 |  |  | 7 |  |  |
|  |  |  | 1.5/13.5 | <1 | 15 | 11 |  |  | 11 |  |  | 11 |  |  |
| VIL | Low Level Input Voltage |  | 4.5/0.5 | <1 | 5 |  |  | 1.5 |  |  | 1.5 |  | 1.5 | V |
|  |  |  | 9/1 | <1 | 10 |  |  | 3 |  |  | 3 |  | 3 |  |
|  |  |  | 13.5/1.5 | <1 | 15 |  |  | 4 |  |  | 4 |  | 4 |  |
| IOH | Output Drive Current | 0/5 | 2.5 | <1 | 5 | -1.36 | -3.2 |  | -1.1 |  |  | -1.1 |  | mA |
|  |  | 0/5 | 4.6 | <1 | 5 | -0.44 | -1 |  | -0.36 |  |  | -0.36 |  |  |
|  |  | 0/10 | 9.5 | <1 | 10 | -1.1 | -2.6 |  | -0.9 |  |  | -0.9 |  |  |
|  |  | 0/15 | 13.5 | <1 | 15 | -3.0 | -6.8 |  | -2.4 |  |  | -2.4 |  |  |
| IOL | Output Sink Current | 0/5 | 0.4 | <1 | 5 | 0.44 | 1 |  | 0.36 |  |  | 0.36 |  | mA |
|  |  | 0/10 | 0.5 | <1 | 10 | 1.1 | 2.6 |  | 0.9 |  |  | 0.9 |  |  |
|  |  | 0/15 | 1.5 | <1 | 15 | 3.0 | 6.8 |  | 2.4 |  |  | 2.4 |  |  |
| II | Input Leakage Current | 0/18 | Any Input |  | 18 |  | 10-5 | 0.1 |  |  | 1 |  | 1 | A |
| IOH, IOL | 3-State Output Leakage Current | 0/18 | 0/18 |  | 18 |  | 10-4 | 0.4 |  |  | 12 |  | 12 | A |
| CI | Input Capacitance |  | Any Input |  |  |  | 5 | 7.5 |  |  |  |  |  | pF |

The Noise Margin for both "1" and "0" level is: 1V min. with VDD=5V, 2V min. with VDD=10V, 2.5V min. with VDD=15V

5/12

HCF4094B

DYNAMIC ELECTRICAL CHARACTERISTICS (Tamb = 25°C,  CL = 50pF, RL = 200K   ,  tr = tf = 20 ns)

| Symbol | Parameter | Test Condition |  | Value (*) |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VDD (V) |  | Min. | Typ. | Max. |  |
| tPLH tPHL | Propagation Delay Time (Clock to serial Output QS) | 5 |  |  | 300 | 600 | ns |
|  |  | 10 |  |  | 125 | 250 |  |
|  |  | 15 |  |  | 95 | 190 |  |
| tPLH tPHL | Propagation Delay Time (Clock to serial Output Q’S) | 5 |  |  | 230 | 460 | ns |
|  |  | 10 |  |  | 110 | 220 |  |
|  |  | 15 |  |  | 75 | 150 |  |
| tPLH tPHL | Propagation Delay Time (Clock to Parallel Output) | 5 |  |  | 420 | 840 | ns |
|  |  | 10 |  |  | 195 | 390 |  |
|  |  | 15 |  |  | 135 | 270 |  |
| tPLH tPHL | Propagation Delay Time (Strobe to Parallel Output) | 5 |  |  | 290 | 580 | ns |
|  |  | 10 |  |  | 145 | 290 |  |
|  |  | 15 |  |  | 100 | 200 |  |
| tPZL, tPZH | Propagation Delay Time Output Enable to Parallel Out : Output High to High Impedance | 5 |  |  | 140 | 280 | ns |
|  |  | 10 |  |  | 75 | 150 |  |
|  |  | 15 |  |  | 55 | 110 |  |
| tPHZ tPLZ | Propagation Delay Time Output Enable to Parallel Out : Output Low to High Impedance | 5 |  |  | 225 | 450 | ns |
|  |  | 10 |  |  | 95 | 190 |  |
|  |  | 15 |  |  | 70 | 140 |  |
| tW | Strobe Pulse Width | 5 |  | 200 | 100 |  | ns |
|  |  | 10 |  | 80 | 40 |  |  |
|  |  | 15 |  | 70 | 35 |  |  |
| tW | Clock Pulse Width | 5 |  | 200 | 100 |  | ns |
|  |  | 10 |  | 100 | 50 |  |  |
|  |  | 15 |  | 83 | 40 |  |  |
| tsetup | Data Setup Time | 5 |  | 125 | 60 |  | ns |
|  |  | 10 |  | 55 | 30 |  |  |
|  |  | 15 |  | 35 | 20 |  |  |
| thold | Minimum Hold Time | 5 |  | 0 | 0 | 0 | ns |
|  |  | 10 |  | 0 | 0 | 0 |  |
|  |  | 15 |  | 0 | 0 | 0 |  |
| tTLH tTHL | Transition Time | 5 |  |  | 100 | 200 | ns |
|  |  | 10 |  |  | 50 | 100 |  |
|  |  | 15 |  |  | 40 | 80 |  |
| tr, tf | Clock input Rise or Fall Time | 5 |  | 15 |  |  | s |
|  |  | 10 |  | 5 |  |  |  |
|  |  | 15 |  | 5 |  |  |  |
| fmax | Maximum Clock Input Frequency | 5 |  | 1.25 | 2.5 |  | MHz |
|  |  | 10 |  | 2.5 | 5 |  |  |
|  |  | 15 |  | 3 | 6 |  |  |

(*) Typical temperature coefficient for all VDD value is 0.3 %/°C.

6/12

HCF4094B

TYPICAL APPLICATION (REMOTE CONTROL HOLDING REGISTER)

TEST CIRCUIT

| TEST | SWITCH |
| --- | --- |
| tPLH, tPHL | Open |
| tPZL, tPLZ | VCC |
| tPZH, tPHZ | GND |

CL = 50pF or equivalent (includes jig and probe capacitance) RL = 200K

RT = ZOUT of pulse generator (typically 50  )

7/12

HCF4094B

WAVEFORM 1 : PROPAGATION DELAY TIMES, PULSE WIDTH (CLOCK), SETUP AND HOLD TIME (DATA IN TO CLOCK) (f=1MHz; 50% duty cycle)

WAVEFORM 2 : PROPAGATION DELAY TIME, PULSE WIDTH (STROBE), SETUP AND HOLD TIME (STROBE TO CLOCK) (f=1MHz; 50% duty cycle)

8/12

HCF4094B

WAVEFORM 3 : OUTPUT ENABLE AND DISABLE TIME  (f=1MHz; 50% duty cycle)

9/12

HCF4094B

| Plastic DIP-16 (0.25) MECHANICAL DATA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM. | mm. |  |  | inch |  |  |
|  | MIN. | TYP | MAX. | MIN. | TYP. | MAX. |
| a1 | 0.51 |  |  | 0.020 |  |  |
| B | 0.77 |  | 1.65 | 0.030 |  | 0.065 |
| b |  | 0.5 |  |  | 0.020 |  |
| b1 |  | 0.25 |  |  | 0.010 |  |
| D |  |  | 20 |  |  | 0.787 |
| E |  | 8.5 |  |  | 0.335 |  |
| e |  | 2.54 |  |  | 0.100 |  |
| e3 |  | 17.78 |  |  | 0.700 |  |
| F |  |  | 7.1 |  |  | 0.280 |
| I |  |  | 5.1 |  |  | 0.201 |
| L |  | 3.3 |  |  | 0.130 |  |
| Z |  |  | 1.27 |  |  | 0.050 |

P001C

10/12

HCF4094B

| SO-16 MECHANICAL DATA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM. | mm. |  |  | inch |  |  |
|  | MIN. | TYP | MAX. | MIN. | TYP. | MAX. |
| A |  |  | 1.75 |  |  | 0.068 |
| a1 | 0.1 |  | 0.2 | 0.003 |  | 0.007 |
| a2 |  |  | 1.65 |  |  | 0.064 |
| b | 0.35 |  | 0.46 | 0.013 |  | 0.018 |
| b1 | 0.19 |  | 0.25 | 0.007 |  | 0.010 |
| C |  | 0.5 |  |  | 0.019 |  |
| c1 | 45˚ (typ.) |  |  |  |  |  |
| D | 9.8 |  | 10 | 0.385 |  | 0.393 |
| E | 5.8 |  | 6.2 | 0.228 |  | 0.244 |
| e |  | 1.27 |  |  | 0.050 |  |
| e3 |  | 8.89 |  |  | 0.350 |  |
| F | 3.8 |  | 4.0 | 0.149 |  | 0.157 |
| G | 4.6 |  | 5.3 | 0.181 |  | 0.208 |
| L | 0.5 |  | 1.27 | 0.019 |  | 0.050 |
| M |  |  | 0.62 |  |  | 0.024 |
| S | 8˚ (max.) |  |  |  |  |  |

PO13H

11/12

HCF4094B

Information furnished is believed to be accurate and reliable. However, STMicroelectronics assumes no responsibility for the consequences of use of such information nor for any infringement of patents or other rights of third parties which may result from its use. No license is granted by implication or otherwise under any patent or patent rights of STMicroelectronics. Specifications mentioned in this publication are subject to change without notice. This publication supersedes and replaces all information previously supplied. STMicroelectronics products are not authorized for use as critical components in life support devices or systems without express written approval of STMicroelectronics.

© The ST logo is a registered trademark of STMicroelectronics

© 2002 STMicroelectronics - Printed in Italy - All Rights Reserved

STMicroelectronics GROUP OF COMPANIES

Australia - Brazil - Canada - China - Finland - France - Germany - Hong Kong - India - Israel - Italy - Japan - Malaysia - Malta - Morocco

Singapore - Spain - Sweden - Switzerland - United Kingdom - United States.

© http://www.st.com

12/12