DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

16-Bit, Single-Channel, Low-Noise, Voltage-Output

DIGITAL-TO-ANALOG CONVERTER

FEATURES                                                           DESCRIPTION

Relative Accuracy: ±0.5 LSB                                         The     DAC8881     is     a     16-bit,     single-channel,

16-Bit Monotonic Over Temperature Range

Low-Noise: 24nV/√Hz

voltage-output digital-to-analog converter (DAC) that offers low-power operation and a flexible SPI serial interface.   It   also   features   16-bit   monotonicity,

Fast Settling: 5μs                                                           excellent linearity, and fast settling time. The on-chip

On-Chip Output Buffer Amplifier with                          precision  output  amplifier  allows  rail-to-rail  output

Rail-to-Rail Operation

Wide, Single Power Supply: +2.7V to +5.5V DAC Loading Control

Selectable Power-On Reset to Zero-Scale or

Midscale

swing to be achieved over the full supply range of

2.7V to 5.5V.

The device supports a standard SPI serial interface capable of operating with input data clock frequencies up to 50MHz. The DAC8881 requires an external reference voltage to set the output range of the DAC

Power-Down Mode                                                         channel.  A  programmable  power-on  reset  circuit  is

Unipolar Straight Binary or

2's Complement Input Mode

Fast SPI™ Interface with Schmitt-Triggered

Inputs:

Up To 50MHz, 1.8V/3V/5V Logic

Small Package: QFN-24, 4x4mm

APPLICATIONS

Industrial Process Control

Data Acquisition Systems

also incorporated into the device to ensure that the DAC output powers up at zero-scale or midscale, and remains there until a valid write command.

Additionally, the device has the capability to function in either unipolar straight binary or 2's complement mode. The DAC8881 provides a power-down feature, accessed over the PDN pin, that reduces the current consumption to 25μA at 5V. Power consumption is

6mW at 5V, reducing to 125μW in power-down mode.

The DAC8881 is available in a 4x4mm QFN-24 package with a specified operating temperature range

Automatic Test Equipment                                            of –40°C to +105°C.

Communications

Optical Networking

RST RSTSEL USB/BTC GAIN

SPI Interface

Shift Register

SDI

DVDD            DGND       IOVDD            AGND        AVDD                     VREFH-S   VREFH-F

Power-On

Reset

Control

Logic

Resistor

Network

DAC8881

PDN

R

CS SCLK

SDOSEL SDO

Serial Out

Control

Input

Register

DAC Latch

DAC

(1) FB

NOTE: (1) RFB = 5kW for gain = 1, RFB = 10kW for gain = 2.

VOUT

RFB

LDAC           VREFL-S

VREFL-F

Please be aware that an important notice concerning availability, standard warranty, and use in critical applications of

Copyright © 2007, Texas Instruments Incorporated

Texas Instruments semiconductor products and disclaimers thereto appears at the end of this data sheet.

DSP is a trademark of Texas Instruments. SPI, QSPI are trademarks of Motorola, Inc.

MICROWIRE is a trademark of National Semiconductor.

All other trademarks are the property of their respective owners.

PRODUCTION DATA information is current as of publication date. Products conform to specifications per the terms of the Texas Instruments standard warranty. Production processing does not necessarily include testing of all parameters.

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

This integrated circuit can be damaged by ESD. Texas Instruments recommends that all integrated circuits be handled with appropriate precautions. Failure to observe proper handling and installation procedures can cause damage.

ESD damage can range from subtle performance degradation to complete device failure. Precision integrated circuits may be more susceptible to damage because very small parametric changes could cause the device not to meet its published specifications.

ORDERING INFORMATION (1)

| PRODUCT | PACKAGE- LEAD | PACKAGE DESIGNATOR | SPECIFIED TEMPERATURE RANGE | PACKAGE MARKING |
| --- | --- | --- | --- | --- |
| DAC8881 | QFN-24 | RGE | –40 C to +105 C | DAC8881 |

(1)   For the most current package and ordering information, see the Package Option Addendum at the end of this data sheet, or see the TI

website at www.ti.com.

ABSOLUTE MAXIMUM RATINGS (1)

Over operating free-air temperature range (unless otherwise noted).

|  |  | DAC8881 | UNIT |
| --- | --- | --- | --- |
| AVDD to AGND |  | –0.3 to 6 | V |
| DVDD to DGND |  | –0.3 to 6 | V |
| IOVDD to DGND |  | –0.3 to 6 | V |
| Digital input voltage to DGND |  | –0.3 to IOVDD + 0.3 | V |
| VOUT to AGND |  | –0.3 to AVDD + 0.3 | V |
| Operating temperature range |  | –40 to +105 | C |
| Storage temperature range |  | –65 to +150 | C |
| Maximum junction temperature (TJ max) |  | +150 | C |
| ESD ratings | Human body model (HBM) | 3000 | V |
|  | Charged device model (CDM) | 1000 | V |

(1)   Stresses above those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to absolute maximum conditions for extended periods may affect device reliability.

2              Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

ELECTRICAL CHARACTERISTICS

All specifications at TA = TMIN to TMAX, AVDD = DVDD = +2.7V to +5.5V, IOVDD = +1.8V to +5.5V, gain = 1X mode, unless otherwise noted.

| PARAMETER | CONDITIONS |  | DAC8881 | UNIT |
| --- | --- | --- | --- | --- |
|  |  |  | MIN TYP MAX |  |
| ACCURACY |  |  |  |  |
| Linearity error | Measured by line passing through codes 0200h and FE00h |  | ±0.5 ±1 | LSB |
| Differential linearity error | Measured by line passing through codes 0200h and FE00h |  | ±0.25 ±1 | LSB |
| Monotonicity |  |  | 16 | Bits |
| Zero-scale error | TA = +25 C, code = 0200h |  | ±4 | LSB |
|  | TMIN to TMAX, code = 0200h |  | ±8 | LSB |
| Zero-scale drift | Code = 0200h |  | ±0.5 ±1 | ppm/ C of FSR |
| Gain error | TA = +25 C, Measured by line passing through codes 0200h and FE00h |  | ±4 ±8 | LSB |
| Gain temperature drift | Measured by line passing through codes 0200h and FE00h |  | ±0.5 ±1 | ppm/ C |
| PSRR | VOUT = full-scale, AVDD = +5V ±10% |  | 2 | LSB/V |
| ANALOG OUTPUT (1) |  |  |  |  |
| Voltage output (2) |  |  | 0 AVDD | V |
| Output voltage drift vs time | Device operating for 500 hours |  | 5 | ppm of FSR |
|  | Device operating for 1000 hours |  | 8 | ppm of FSR |
| Output current |  |  | 2.5 | mA |
| Maximum load capacitance |  |  | 200 | pF |
| Short-circuit current |  |  | +31, –50 | mA |
| REFERENCE INPUT (1) |  |  |  |  |
| VREFH input voltage range | AVDD = +5.5V |  | 1.25 5.0 AVDD | V |
|  | AVDD = +3V |  | 1.25 2.5 AVDD | V |
| VREFH input capacitance |  |  | 5 | pF |
| VREFH input impedance |  |  | 4.5 | kΩ |
| VREFL input voltage range |  |  | –0.2 0 +0.2 | V |
| VREFL input capacitance |  |  | 4.5 | pF |
| VREFL input impedance |  |  | 5 | kΩ |
| DYNAMIC PERFORMANCE (1) |  |  |  |  |
| Settling time | To ±0.003% FS, RL = 10kΩ, CL = 50pF, code 1000h to F000h |  | 5 | μs |
| Slew rate | From 10% to 90% of 0V to +5V |  | 2.5 | V/μs |
| Code change glitch | Code = 7FFFh to 8000h to 7FFFh | VREFH = 5V, gain = 1X mode | 37 | nV-s |
|  |  | VREFH = 2.5V, gain = 1X mode | 18 | nV-s |
|  |  | VREFH = 1.25V, gain = 1X mode | 9 | nV-s |
|  |  | VREFH = 2.5V, gain = 2X mode | 21 | nV-s |
|  |  | VREFH = 1.25V, gain = 2X mode | 10 | nV-s |
| Digital feedthrough |  |  | 1 | nV-s |
| Output noise voltage density | f = 1kHz to 100kHz, full-scale output | Gain = 1 | 24 30 | nV/√Hz |
|  |  | Gain = 2 | 40 48 | nV/√Hz |
| Output noise voltage | f = 0.1Hz to 10Hz, full-scale output |  | 2 | μVPP |

(1)   Ensured by design. Not production tested.

(2)   The output from the VOUT pin = [(VREFH – VREFL)/65536]    CODE    Buffer GAIN + VREFL. The maximum range of VOUT is 0V to AVDD.

The full-scale of the output must be less than AVDD; otherwise, output saturation occurs.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback              3

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

ELECTRICAL CHARACTERISTICS (continued)

All specifications at TA = TMIN to TMAX, AVDD = DVDD = +2.7V to +5.5V, IOVDD = +1.8V to +5.5V, gain = 1X mode, unless otherwise noted.

| PARAMETER | CONDITIONS | DAC8881 | UNIT |
| --- | --- | --- | --- |
|  |  | MIN TYP MAX |  |
| DIGITAL INPUTS (3) |  |  |  |
| High-level input voltage, VIH | IOVDD = 4.5V to 5.5V | 3.8 IOVDD + 0.3 | V |
|  | IOVDD = 2.7V to 3.3V | 2.1 IOVDD + 0.3 | V |
|  | IOVDD = 1.7V to 2.0V | 1.5 IOVDD + 0.3 | V |
| Low-level input voltage, VIL | IOVDD = 4.5V to 5.5V | –0.3 0.8 | V |
|  | IOVDD = 2.7V to 3.3V | –0.3 0.6 | V |
|  | IOVDD = 1.7V to 2.0V | –0.3 0.3 | V |
| Digital input current (IIN) |  | ±1 ±10 | μA |
| Digital input capacitance |  | 5 | pF |
| DIGITAL OUTPUT (3) |  |  |  |
| High-level output voltage, VOH | IOVDD = 2.7V to 5.5V, IOH = –1mA | IOVDD – 0.2 | V |
|  | IOVDD = 1.7V to 2.0V, IOH = –500μA | IOVDD – 0.2 | V |
| Low-level output voltage, VOL | IOVDD = 2.7V to 5.5V, IOL = 1mA | 0.2 | V |
|  | IOVDD = 1.7 to 2.0V, IOL = 500μA | 0.2 | V |
| POWER SUPPLY |  |  |  |
| AVDD |  | +2.7 +5.5 | V |
| DVDD |  | +2.7 +5.5 | V |
| IOVDD |  | +1.7 DVDD | V |
| AIDD | VIH = IOVDD, VIL = DGND | 1.5 | mA |
| DIDD | VIH = IOVDD, VIL = DGND | 1 10 | μA |
| IOIDD | VIH = IOVDD, VIL = DGND | 1 10 | μA |
| AIDD power-down | PDN = IOVDD | 25 50 | μA |
| Power dissipation | AVDD = DVDD = 5.0V | 6 7.5 | mW |
| TEMPERATURE RANGE |  |  |  |
| Specified performance |  | –40 +105 | C |

(3)   Ensured by design. Not production tested.

4              Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

24

23

21

19

PIN CONFIGURATION

RGE PACKAGE(1)

IOVDD

SDO

22

DGND

DVDD

SDOSEL

20

CS

QFN-24 (TOP VIEW)

SCLK      1

SDI     2

18     PDN

17     RST

LDAC      3

AGND      4

AVDD        5

7

VREFL-S      6

DAC8881

VREFH-S VOUT RFB VREFL-F VREFH-F

NC

8

9

10

11

12

(Thermal Pad)

16     USB/BTC

15     GAIN

14     RSTSEL

13     NC

(1)   The thermal pad is internally connected to the substrate. This pad can be connected to the analog ground or left floating. Keep the thermal pad separate from the digital ground, if possible.

TERMINAL FUNCTIONS

| TERMINAL | I/O | DESCRIPTION |
| --- | --- | --- |
| NO. NAME |  |  |
| 1 SCLK | I | SPI bus serial clock input |
| 2 SDI | I | SPI bus serial data input |
| 3 LDAC | I | Load DAC latch control input (active low). When LDAC is low, the DAC latch is transparent, and the contents of the input register are transferred to the DAC latch. The DAC output changes to the corresponding level simultaneously when the DAC latch is updated. |
| 4 AGND | I | Analog ground |
| 5 AVDD | I | Analog power supply |
| 6 VREFL-S | I | Reference low input sense |
| 7 VREFH-S | I | Reference high input sense |
| 8 VOUT | O | Output of output buffer |
| 9 RFB | I | Feedback resistor connected to the inverting input of the output buffer. |
| 10 VREFL-F | I | Reference low input force |
| 11 VREFH-F | I | Reference high input force |
| 12 NC | — | Do not connect. |
| 13 NC | — | Do not connect. |
| 14 RSTSEL | I | Selects the value of the output from the VOUT pin after power-on or hardware reset. If RSTSEL = IOVDD, then register data = 8000h. If RSTSEL = DGND, then register data = 0000h. |
| 15 GAIN | I | Buffer gain setting. Gain = 1 when the pin is connected to DGND; Gain = 2 when the pin is connected to IOVDD. |
| 16 USB/BTC | I | Input data format selection. Input data are straight binary format when the pin is connected to IOVDD, and in two’s complement format when the pin is connected to DGND. |
| 17 RST | I | Reset input (active low). Logic low on this pin causes the device to perform a reset. |
| 18 PDN | I | Power-down input (active high). Logic high on this pin forces the device into power-down status. In power-down, the VOUT pin connects to AGND through 10kΩ resistor. |
| 19 CS | I | SPI bus chip select input (active low). Data bits are not clocked into the serial shift register unless CS is low. When CS is high, SDO is in high-impedance status. |
| 20 SDOSEL | I | SPI serial data output selection. When SDOSEL is tied to IOVDD, the contents of the existing input register are shifted out from the SDO pin; this is Stand-Alone mode. When SDOSEL is tied to DGND, the contents in the SPI input shift register are shifted out from the SDO pin; this is Daisy-Chain mode for daisy chaining communication. |
| 21 DVDD | I | Digital power supply (connect to AVDD, pin 5) |
| 22 DGND | I | Digital ground |
| 23 SDO | O | SPI bus serial data output. Refer to the Timing Diagrams for further detail. |
| 24 IOVDD | I | Interface power. Connect to +1.8V for 1.8V logic, +3V for 3V logic, and to +5V for 5V logic. |

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback              5

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

TIMING DIAGRAMS

Case 1: Standalone operation without SDO, LDAC tied low.

t1                                t2

CS

t3

t4                  t5                            t6

Input Register and DAC Latch Updated

t7

SCLK

t8                  t9

SDI

LDAC

Low

Bit 15 (N)               Bit 14 (N)                                               Bit 1 (N)                 Bit 0 (N)

Case 2: Standalone operation without SDO, LDAC active.

t1                               t2

CS

t3

t4                  t5                            t6

Input Register

Updated

t7

SCLK

t8                  t9

SDI

Bit 15 (N)               Bit 14 (N)                                               Bit 1 (N)                 Bit 0 (N)

LDAC

High

= Dont Care        Bit 15 = MSB Bit 0 = LSB

t14                  t15

DAC Latch

Updated

Figure 1. Timing Diagram of Standalone Operation without SDO

6              Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

TIMING CHARACTERISTICS for Figure 1(1) (2) (3)

At –40 C to +105 C, unless otherwise noted.

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

| 3 time PARAMETER | CONDITIONS | MIN MAX | UNIT |
| --- | --- | --- | --- |
| fSCLK Maximum clock frequency | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 40 | MHz |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 50 | MHz |
| t1 Minumum CS high time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 50 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 30 | ns |
| t2 CS falling edge to SCLK rising edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 8 | ns |
| t SCLK falling edge to CS falling edge setup | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t4 SCLK low time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t5 SCLK high time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 15 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t6 SCLK cycle time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 25 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 20 | ns |
| t7 SCLK rising edge to CS rising edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t8 Input data setup time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 8 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t9 Input data hold time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t14 CS rising edge to LDAC falling edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t15 LDAC pulse width | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 15 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |

(1)   All input signals are specified with tR = tF = 2ns (10% to 90% of IOVDD) and timed from a voltage level of IOVDD/2. (2)   Ensured by design. Not production tested.

(3)   Sample tested during the initial release and after any redesign or process changes that may affect these parameters.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback              7

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

Case 1: Standalone operation with output from SDO, LDAC tied low.

t1                                t2

CS

t3

t4                  t5                            t6

Input Register and DAC Latch Updated

t7

SCLK

t8                  t9

SDI

Bit 15 (N)               Bit 14 (N)                                               Bit 1 (N)                 Bit 0 (N)

t11

t12

t13

SDO

LDAC

Low

High-Z

Bit 15 (N - 1)

from Input Reg. t10

Bit 14 (N - 1)

from Input Reg.

Bit 1 (N - 1)

from Input Reg.

Bit 0 (N - 1)

from Input Reg.

High-Z

Case 2: Standalone operation with output from SDO, LDAC active.

t1                               t2

CS

t3

t4                  t5                            t6

Input Register

Updated

t7

SCLK

t8                  t9

SDI

Bit 15 (N)               Bit 14 (N)                                               Bit 1 (N)                 Bit 0 (N)

SDO

High-Z

Bit 15 (N - 1)

from Input Reg.

t10

Bit 14 (N - 1)

from Input Reg.

t11

t12

Bit 1 (N - 1)

from Input Reg.

t13

Bit 0 (N - 1)

from Input Reg.

High-Z

LDAC

High

= Dont Care        Bit 15 = MSB Bit 0 = LSB

t14                  t15

DAC Latch

Updated

Figure 2. Timing Diagram of Standalone Operation with SDO

8              Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

Case 1: Daisy Chain, LDAC tied low.

t1                                t2

CS

t3

t4                  t5                            t6

Input Register and DAC Latch Updated

t7

SCLK

t8                  t9

SDI

Bit 15 (N)               Bit 14 (N)

Bit 0 (N)

Bit 15 (N + 1)              Bit 0 (N + 1)

t11

t12

t13

SDO

LDAC

Low

High-Z

t10

Bit 15 (N)                Bit 0 (N)

High-Z

Case 2: Daisy Chain, LDAC active.

t1                                t2

CS

t3

t4                  t5                            t6

Input Register

Updated

t7

SCLK

t8                  t9

SDI

Bit 15 (N)               Bit 14 (N)

Bit 0 (N)

Bit 15 (N + 1)              Bit 0 (N + 1)

t11

t12

t13

SDO

LDAC

High

High-Z

t10

Bit 15 (N)                Bit 0 (N)

High-Z

t14                  t15

= Dont Care        Bit 15 = MSB Bit 0 = LSB

DAC Latch

Updated

Figure 3. Timing Diagram of Daisy Chain Mode, Two Cascaded Devices

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback              9

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TIMING CHARACTERISTICS for Figure 2 and Figure 3(1) (2) (3)

At –40 C to +105 C, unless otherwise noted.

www.ti.com

| 3 time PARAMETER | CONDITIONS | MIN MAX | UNIT |
| --- | --- | --- | --- |
| fSCLK Maximum clock frequency | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 20 | MHz |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 25 | MHz |
| t1 Minumum CS high time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 50 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 30 | ns |
| t2 CS falling edge to SCLK rising edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 8 | ns |
| t SCLK falling edge to CS falling edge setup | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t4 SCLK low time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 25 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 20 | ns |
| t5 SCLK high time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 25 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 20 | ns |
| t6 SCLK cycle time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 50 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 40 | ns |
| t7 SCLK rising edge to CS rising edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t8 Input data setup time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t9 Input data hold time | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t10 SDO active from CS falling edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 15 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
| t11 SDO data valid from SCLK falling edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 20 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 15 | ns |
| t12 SDO data hold from SCLK rising edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 25 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 20 | ns |
| t13 SDO High-Z from CS rising edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 8 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t14 CS rising edge to LDAC falling edge | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 5 | ns |
| t15 LDAC pulse width | 2.7 ≤ DVDD < 3.6V, 2.7 ≤ IOVDD ≤ DVDD | 15 | ns |
|  | 3.6 ≤ DVDD ≤ 5.5V, 2.7 ≤ IOVDD ≤ DVDD | 10 | ns |

(1)   All input signals are specified with tR = tF = 2ns (10% to 90% of IOVDD) and timed from a voltage level of IOVDD/2. (2)   Ensured by design. Not production tested.

(3)   Sample tested during the initial release and after any redesign or process changes that may affect these parameters.

10            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs DIGITAL INPUT CODE                                                                       vs DIGITAL INPUT CODE

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +25ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +25ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 4.                                                                                                   Figure 5.

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs DIGITAL INPUT CODE                                                                       vs DIGITAL INPUT CODE

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = 40ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = 40ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 6.                                                                                                   Figure 7.

LINEARITY ERROR                                                                     DIFFERENTIAL LINEARY ERROR

vs DIGITAL INPUT CODE                                                                       vs DIGITAL INPUT CODE

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +105ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +105ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 8.                                                                                                   Figure 9.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            11

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V (continued)

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

www.ti.com

| 1.0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | INL Max |  |  |  |  |  |
|  |  |  |  |  | INL Min |  |  |  |  |  |

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs TEMPERATURE                                                                                 vs TEMPERATURE

1.0

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

| 1.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DNL Max |  |  |  |  |
|  |  |  |  |  | DNL Min |  |  |  |  |

40     20        0        20       40       60       80      100     120

Temperature (ﾰC)

40     20        0        20       40       60       80      100     120

Temperature (ﾰC)

Figure 10.                                                                                                 Figure 11.

1.0

0.8

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs TEMPERATURE (GAIN = 2X MODE)                                                vs TEMPERATURE (GAIN = 2X MODE)

1.0

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

| REFL |  |  |  |  |  |  |  | VREFH = 2.5V V = 0V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | INL Max |  |  |  |  |  |
|  |  |  |  | INL Min |  |  |  |  |  |

40     20        0        20       40       60       80      100     120

Temperature (ﾰC)

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

| REFL |  |  |  |  |  |  |  | VREFH = 2.5V V = 0V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DNL Max |  |  |  |  |
|  |  |  |  |  | DNL Min |  |  |  |  |

40     20        0        20       40       60       80      100     120

Temperature (ﾰC)

Figure 12.                                                                                                 Figure 13.

1.0

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs SUPPLY VOLTAGE                                                                            vs SUPPLY VOLTAGE

1.0

0.8

0.6

VREFH = 2.5V

VREFL = 0V

0.8

0.6

VREFH = 2.5V VREFL = 0V

0.2

0

INL Max

DNL Error (LSB)

0.2

0

DNL Max

INL Error (LSB)

-0.4

-0.6

-0.8

-1.0

INL Min

2.5        3.0        3.5        4.0        4.5        5.0        5.5        6.0

Supply Voltage (V)

-0.4

-0.6

-0.8

-1.0

DNL Min

2.5        3.0        3.5        4.0        4.5        5.0        5.5        6.0

Supply Voltage (V)

Figure 14.                                                                                                 Figure 15.

12            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V (continued)

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

1.0

0.8

0.6

INL Error (LSB)

0.4

0.2

0

-0.2

-0.4

-0.6

-0.8

-1.0

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs REFERENCE VOLTAGE                                                                    vs REFERENCE VOLTAGE

1.0

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

-0.2

-0.4

-0.6

-0.8

-1.0

| 0 1 2 3 4 5 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | INL Max |  |  |  |  |
|  |  |  | INL Min |  |  |  |  |

Reference Voltage (V)

| 0 1 2 3 4 5 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DNL Max |  |  |  |  |
|  |  |  | DNL Min |  |  |  |  |

Reference Voltage (V)

Figure 16.                                                                                                 Figure 17.

| 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Plus Full-Scale Error |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Minus Full-Scale Error |  |  |  |  |  |  |

0.8

0.6

0.4

0.2

0

0.2

Endpoint Error (mV)

0.4

0.6

0.8

1.0

ENDPOINT ERROR                                                                                 ENDPOINT ERROR

vs TEMPERATURE                                                                 vs TEMPERATURE (GAIN = 2X MODE)

| 1.0 VREFH = 2.5V VREFL = 0V |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Plus Full-Scale Error |  |  |  |  |
|  |  |  |  |  |  |  | Minus Full-Scale Error |  |  |  |  |  |

0.8

0.6

0.4

0.2

0

0.2

Endpoint Error (mV)

0.4

0.6

0.8

1.0

55    35    15

5       25      45      65      85     105

Temperature (ﾰC)

125

55    35    15

5       25      45      65      85     105

Temperature (ﾰC)

125

Figure 18.                                                                                                 Figure 19.

1100

AVDD SUPPLY CURRENT                                                                       AVDD SUPPLY CURRENT

vs DIGITAL INPUT CODE                                                      vs DIGITAL INPUT CODE (GAIN = 2X MODE)

| 1000 AVDD = 5.0V VREFH = 2.5V VREFL = 0V |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AVDD = 2.7V VREFH = 1.25V VREFL = 0V |  |  |  |  |  |

AVDD = 5.0V

1000

900

800

700

600

500

AVDD Supply Current (mA)

400

300

200

AVDD = 5.0V VREFH = 2.5V VREFL = 0V

AVDD = 2.7V VREFH = 2.5V VREFL = 0V

VREFH = 5.0V VREFL = 0V

AVDD = 2.7V VREFH = 2.7V VREFL = 0V

900

800

700

600

500

400

300

AVDD Supply Current (mA)

200

100

0

0      8192   16384 24576 32768 40960 49152 57344 65536

Digital Input Code

0      8192   16384 24576 32768 40960 49152 57344 65536

Digital Input Code

Figure 20.                                                                                                 Figure 21.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            13

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V (continued)

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

www.ti.com

1200

AVDD Supply Current (A)

1000

AVDD SUPPLY CURRENT                                                                 AVDD POWER-DOWN CURRENT

vs TEMPERATURE                                                                                 vs TEMPERATURE

| 50 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AVDD = 5.0V |  |  |  |  |  |  |
|  |  |  |  | AVDD = 2.7V |  |  |  |  |  |  |

40

800

600

400

200

0

VREFH = 5.0V VREFL = 0V

Gain = 1X Mode

DAC Code Set to FC00h

30

VREFH = 2.5V

VREFL = 0V

AVDD Supply Current (A)

Gain = 2X Mode                                                  20

10

0

55    35    15

5       25      45      65      85     105

Temperature (ﾰC)

125

55    35    15

5       25      45      65      85     105

Temperature (ﾰC)

125

Figure 22.                                                                                                 Figure 23.

1.5

REFERENCE CURRENT                                                                         REFERENCE CURRENT

vs DIGITAL INPUT CODE                                                      vs DIGITAL INPUT CODE (GAIN = 2X MODE)

1.5

1.0

0.5

0

-0.5

Reference Current (mA)

-1.0

VREFL Current

VREFH Current

1.0

0.5

0

-0.5

Reference Current (mA)

-1.0

VREFH Current

VREFL Current

VREFH = 2.5V

VREFL = 0V

-1.5

0

8192

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

-1.5

0

8192

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 24.                                                                                                 Figure 25.

5.0

OUTPUT VOLTAGE OUTPUT VOLTAGE                                                                    vs DRIVE CURRENT CAPABILITY

vs DRIVE CURRENT CAPABILITY                                                              (Operation Near AVDD Rail)

| 5.00 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DAC Loaded with FFFFh |  |  |  |  |
|  |  | DAC Loaded with 0000h |  |  |  |  |

4.5

4.0

3.5

VOUT (V)

3.0

2.5

2.0

1.5

1.0

0.5

0

0                 3                 6                 9                12               15

I(SOURCE/SINK) (mA)

4.95

VOUT (V)

4.90

4.85

4.80

4.75

DAC Loaded with FFFFh

DAC Loaded with FE00h

DAC Loaded with FC00h

DAC Loaded with F800h

0                 1                 2                 3                 4                 5

ISOURCE (mA)

Figure 26.                                                                                                 Figure 27.

14            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V (continued)

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

0.25

0.20

OUTPUT VOLTAGE

vs DRIVE CURRENT CAPABILITY                                                               IOVDD SUPPLY CURRENT (Operation Near AGND Rail)                                                                  vs LOGIC INPUT VOLTAGE

200

180

IOVDD Supply Current (mA)

160

VOUT (V)

0.15

0.10

0.05

0

DAC Loaded with 0800h

DAC Loaded with 0400h

DAC Loaded with 0200h

DAC Loaded with 0000h

0                 1                 2                 3                 4                 5

ISINK (mA)

140

120

100

80

60

40

20

0

| 0 0.5 1.0 1.5 2.0 2.5 3.0 3.5 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | IOVDD = 5V |  |  |  |  |  |  |  |
|  |  |  | IOVDD = 2.7V |  |  |  |  |  |  |  |  |  |

Logic Input Voltage (V)

4.0

4.5

5.0

Figure 28.                                                                                                 Figure 29.

LARGE SIGNAL                                                                                      LARGE SIGNAL SETTLING TIME                                                                                      SETTLING TIME

Large-Signal Output

2V/div

Small-Signal Error

2V/div                                                                                                  1mV/div

Large-Signal Output

Small-Signal Error

1mV/div

5V/div

LDAC

Signal                              Code Change: 0000h to FFFFh

Output Loaded with 10kW and

50pF to AGND

5V/div

LDAC

Signal                              Code Change: FFFFh to 0000h

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 30.                                                                                                 Figure 31.

LARGE SIGNAL                                                                                      LARGE SIGNAL SETTLING TIME                                                                                      SETTLING TIME

Large-Signal Output

2V/div

2V/div                                                                                                  1mV/div

Small-Signal Error

Large-Signal Output

Small-Signal Error

1mV/div

5V/div

LDAC

Signal                              Code Change: 1000h to F000h

Output Loaded with 10kW and

50pF to AGND

5V/div

LDAC

Signal                              Code Change: F000h to 1000h

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 32.                                                                                                 Figure 33.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            15

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V (continued)

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

www.ti.com

LARGE SIGNAL                                                                                      LARGE SIGNAL SETTLING TIME (GAIN = 2X MODE)                                                     SETTLING TIME (GAIN = 2X MODE)

VREFH = 2.5V

Large-Signal Output

2V/div

VREFH = 2.5V

Code Change: FFFFh to 0000h

Output Loaded with 10kW and

50pF to AGND

Small-Signal Error

2V/div                                                                                                  1mV/div

Large-Signal Output

Small-Signal Error

1mV/div

5V/div

LDAC

Signal                              Code Change: 0000h to FFFFh

Output Loaded with 10kW and

50pF to AGND

5V/div

LDAC Signal

Time (2ms/div)

Time (2ms/div)

Figure 34.                                                                                                 Figure 35.

LARGE SIGNAL                                                                                      LARGE SIGNAL SETTLING TIME (GAIN = 2X MODE)                                                     SETTLING TIME (GAIN = 2X MODE)

VREFH = 2.5V

Large-Signal Output

2V/div

VREFH = 2.5V

Small-Signal Error

2V/div                                                                                                  1mV/div

Large-Signal Output

Small-Signal Error

1mV/div

5V/div

LDAC

Signal                              Code Change: 1000h to F000h

Output Loaded with 10kW and

50pF to AGND

5V/div

LDAC

Signal                              Code Change: F000h to 1000h

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 36.                                                                                                 Figure 37.

MAJOR CARRY GLITCH                                                                        MAJOR CARRY GLITCH

Gain = 1X Mode

VREFH = +5V

Code Change: 7FFFh to 8000h

Output Loaded with 10kW and

50pF to AGND

Gain = 1X Mode

VREFH = +5V

100mV/div

VOUT Signal

100mV/div

Integrated Glitch Energy (34nV-s) VOUT Signal

Integrated Glitch Energy (37nV-s)

5V/div

LDAC Signal

5V/div

LDAC Signal

Code Change: 8000h to 7FFFh

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 38.                                                                                                 Figure 39.

16            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +5V (continued)

At TA = +25 C, VREFH = +5.0V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

MAJOR CARRY GLITCH                                                                        MAJOR CARRY GLITCH

Gain = 1X Mode

VREFH = +2.5V

Code Change: 7FFFh to 8000h

Output Loaded with 10kW and

50pF to AGND

Gain = 1X Mode

VREFH = +2.5V

100mV/div

VOUT Signal

100mV/div

Integrated Glitch Energy (16nV-s) VOUT Signal

5V/div

Integrated Glitch Energy (18nV-s)

LDAC Signal

5V/div

LDAC Signal

Code Change: 8000h to 7FFFh

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 40.                                                                                                 Figure 41.

Output Voltage Noise Density (nV/ￖHz)

180

160

140

120

100

80

60

40

OUTPUT NOISE DENSITY                                                             LOW-FREQUENCY OUTPUT NOISE

vs FREQUENCY                                                                                       (0.1Hz to 10Hz)

2mV/div

DAC Code Set to 8000h

Output Unloaded

Gain = 2X Mode

20

Gain = 1X Mode

0

1                10              100              1k              10k            100k

Frequency (Hz)

Time (1s/div)

Figure 42.                                                                                                 Figure 43.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            17

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +2.7V

At TA = +25 C, VREFH = +2.5V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

www.ti.com

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs DIGITAL INPUT CODE                                                                       vs DIGITAL INPUT CODE

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +25ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +25ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 44.                                                                                                 Figure 45.

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs DIGITAL INPUT CODE                                                                       vs DIGITAL INPUT CODE

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = 40ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = 40ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 46.                                                                                                 Figure 47.

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs DIGITAL INPUT CODE                                                                       vs DIGITAL INPUT CODE

0.8

0.6

INL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +105ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

0.8

0.6

DNL Error (LSB)

0.4

0.2

0

0.2

0.4

0.6

0.8

1.0

0

8192

TA = +105ﾰC

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 48.                                                                                                 Figure 49.

18            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +2.7V (continued)

At TA = +25 C, VREFH = +2.5V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

1.0

0.8

0.6

INL Error (LSB)

0.4

0.2

0

-0.2

-0.4

-0.6

-0.8

-1.0

LINEARITY ERROR                                                                   DIFFERENTIAL LINEARITY ERROR

vs REFERENCE VOLTAGE                                                                    vs REFERENCE VOLTAGE

1.0

0.8

| 0.6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DNL Max |  |  |  |
|  |  |  |  | DNL Min |  |  |  |

DNL Error (LSB)

0.4

0.2

0

-0.2

-0.4

-0.6

-0.8

-1.0

| 0 0.5 1.0 1.5 2.0 2.5 3.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | INL Max |  |  |  |  |
|  |  |  | INL Min |  |  |  |  |

VREFH Reference Voltage (V)

0            0.5          1.0          1.5          2.0          2.5          3.0

VREFH Reference Voltage (V)

Figure 50.                                                                                                 Figure 51.

1000

900

AVDD SUPPLY CURRENT                                                                        REFERENCE CURRENT

vs TEMPERATURE                                                                            vs DIGITAL INPUT CODE

1.00

VREFH = 2.5V

AVDD Supply Current (A)

800

700

600

500

400

300

200

100

0

VREF = 2.5V, Gain = 1X Mode

VREF = 1.25V, Gain = 2X Mode

DAC Code Set to FFFFh

0.75

0.50

0.25

0

-0.25

Reference Current (mA)

-0.50

-0.75

-1.00

VREFH Current

VREFL Current

VREFL = 0V

55    35    15

5       25      45      65      85     105

Temperature (ﾰC)

125

0      8192

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

Figure 52.                                                                                                 Figure 53.

1.00

REFERENCE CURRENT                                                                             OUTPUT VOLTAGE

vs DIGITAL INPUT CODE (GAIN = 2X MODE)                                               vs DRIVE CURRENT CAPABILITY

3.0

0.75

0.50

0.25

V

VREFH Current

VREFH = 1.25V VREFL = 0V

2.5

2.0

DAC Loaded with FFFFh, VREFH = 2.7V

DAC Loaded with FFFFh, VREFH = 2.5V

0

-0.25

Reference Current (mA)

-0.50

-0.75

-1.00

0

8192

VREFL Current

16384  24576  32768 40960  49152 57344 65536

Digital Input Code

(V)

1.5

1.0

0.5

0

DAC Loaded with 0000h

OUT

0                 3                 6                 9                12               15

I(SOURCE/SINK) (mA)

Figure 54.                                                                                                 Figure 55.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            19

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +2.7V (continued)

At TA = +25 C, VREFH = +2.5V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

www.ti.com

2.70

2.65

OUTPUT VOLTAGE                                                                                OUTPUT VOLTAGE

vs DRIVE CURRENT CAPABILITY                                                        vs DRIVE CURRENT CAPABILITY

(Operation Near AVDD Rail)                                                                   (Operation Near AGND Rail)

0.25

DAC Loaded with FFFFh

0.20

2.60

VOUT (V)

2.55

2.50

DAC Loaded with FE00h

DAC Loaded with FC00h

DAC Loaded with F800h

VOUT (V)

0.15

0.10

DAC Loaded with 0400h

DAC Loaded with 0200h

DAC Loaded with 0800h

2.45

2.40

DAC Loaded with FFFFh, VREFH = 2.5V

VREFH = 2.7V, unless otherwise noted.

0                 1                 2                 3                 4                 5

ISOURCE (mA)

0.05

0

DAC Loaded with 0000h

0                 1                 2                 3                 4                 5

ISINK (mA)

Figure 56.                                                                                                 Figure 57.

LARGE SIGNAL                                                                                      LARGE SIGNAL SETTLING TIME                                                                                      SETTLING TIME

Large-Signal Output

1V/div

Code Change: FFFFh to 0000h

Output Loaded with 10kW and

50pF to AGND

Small-Signal Error

1V/div                                                                                                  1mV/div

Large-Signal Output

Small-Signal Error

1mV/div

5V/div

LDAC Signal

Code Change: 0000h to FFFFh

Output Loaded with 10kW and

50pF to AGND

5V/div

LDAC Signal

Time (2ms/div)

Time (2ms/div)

Figure 58.                                                                                                 Figure 59.

LARGE SIGNAL                                                                                      LARGE SIGNAL SETTLING TIME                                                                                      SETTLING TIME

Large-Signal Output

1V/div

Large-Signal Output

Small-Signal Error

1mV/div

Small-Signal Error

1mV/div

5V/div

1V/div

LDAC Signal

Code Change: 1000h to F000h

Output Loaded with 10kW and

50pF to AGND

5V/div

LDAC Signal

Code Change: F000h to 1000h

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 60.                                                                                                 Figure 61.

20            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

TYPICAL CHARACTERISTICS: VDD = +2.7V (continued)

At TA = +25 C, VREFH = +2.5V, VREFL = 0V, and Gain = 1X Mode, unless otherwise noted.

MAJOR CARRY GLITCH                                                                        MAJOR CARRY GLITCH

Gain = 1X Mode

VREFH = +2.5V

Code Change: 7FFFh to 8000h

Output Loaded with 10kW and

50pF to AGND

Gain = 1X Mode

VREFH = +2.5V

100mV/div

VOUT Signal

100mV/div

Integrated Glitch Energy (17.5nV-s) VOUT Signal

5V/div

Integrated Glitch Energy (16.5nV-s)

LDAC Signal

5V/div

LDAC Signal

Code Change: 8000h to 7FFFh

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 62.                                                                                                 Figure 63.

MAJOR CARRY GLITCH                                                                        MAJOR CARRY GLITCH

Gain = 1X Mode

VREFH = +1.25V

Code Change: 7FFFh to 8000h

Output Loaded with 10kW and

50pF to AGND

Gain = 1X Mode

VREFH = +1.25V

100mV/div

VOUT Signal

Integrated Glitch Energy (7.8nV-s)

100mV/div

Integrated Glitch Energy (9.4nV-s) VOUT Signal

5V/div

LDAC Signal

5V/div

LDAC Signal

Code Change: 8000h to 7FFFh

Output Loaded with 10kW and

50pF to AGND

Time (2ms/div)

Time (2ms/div)

Figure 64.                                                                                                 Figure 65.

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            21

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

THEORY OF OPERATION

GENERAL DESCRIPTION

The DAC8881 is a single-channel, 16-bit, serial-input, voltage-output digital-to-analog converter (DAC). The architecture is an R-2R ladder configuration with the four MSBs segmented, followed by an operational amplifier that serves as a buffer, as shown in Figure 66. The on-chip output buffer allows rail-to-rail output swings while providing a low output impedance to drive loads. The DAC8881 operates from a single analog power supply that ranges from 2.7V to 5.5V, and typically consumes 850μA when operating with a 3V supply. Data are written to the device in a 16-bit word format, via an SPI serial interface. To enable compatibility with 1.8V, 3V, or 5V logic families, an IOVDD  supply pin is provided. This pin allows the DAC8881 input and output logic to be powered from the same logic supply used to interface signals to and from the device. Internal voltage translators are included in the DAC8881 to interface digital signals to the device core. Separate AVDD and DVDD supply pins are provided, but should be connected together. See Figure 67 for the basic configuration of the DAC8881.

To ensure a known power-up state, the DAC8881 is designed with a power-on reset function. Upon power-up, the DAC8881 is reset to either zero-scale or midscale depending on the state of the RSTSEL pin. The device can also be hardware reset by using the RST and RSTSEL pins.

(1) FB

R

RFB

R

2R                2R

2R               2R               2R               2R               2R               2R               2R

VOUT

5kW

5kW

VREFH

NOTE: (1) RFB = 5kW for gain = 1

RFB = 10kW for gain = 2.

VREFH-F

VREFH-S

VREFL-F

Figure 66. DAC8881 Architecture

VREFL-S

22            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SDOSEL

Chip-Select

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

+5V

+

1.8V to 5V

Serial Data Out

0.1mF        1mF

0.1mF

+

1mF

Clock

Serial Data In

Load DAC Registers

SCLK

IOVDD

SDO

24

23

DGND

1

SDI

2

LDAC

3

AGND

4

AVDD

5

VREFL-S

VREFH-S

VOUT RFB VREFL-F VREFH-F

NC

7

6

22

21

CS

DAC8881

8

9

10

11

12

(Thermal Pad)

20

19

18    PDN

DVDD

SDOSEL

17    RST

16    USB/BTC

15    GAIN

14    RSTSEL

13    NC

Reset DAC Registers

0V to +5.0V

External Reference

+5.0000V

Figure 67. Basic Configuration

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            23

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

ANALOG OUTPUT

21

The DAC8881 offers a force and sense output configuration for the high open-loop gain output amplifier. This feature allows the loop around the output amplifier to be closed at the load (as shown in Figure 68), thus ensuring an accurate output voltage. The output buffer VOUT and RFB pins are provided so that the output op amp buffer feedback can be connected at the load. Without a driven load, the DAC8881 output typically swings to within 15mV of the AGND and AVDD supply rails. Because of the high accuracy of these DACs, system design problems  such  as  grounding  and  contact  resistance  become  very  important.  A  16-bit  converter  with  a  5V full-scale range has a 1LSB value of 76μV. With a load current of 1mA, a series wiring and connector resistance of only 80mΩ (RW2) causes a voltage drop of 80μV. In terms of a system layout, the resistivity of a typical

24

1-ounce copper-clad printed circuit board is 0.5mΩ per square. For a 1mA load, a 0.25mm wide printed circuit conductor 25mm long results in a voltage drop of 50μV.

+5V

0.1mF

+

1.8V to 5V

Serial Data Out

20

Chip-Select

1mF

0.1mF

+

1mF

Clock

Serial Data In

Load DAC Registers

SCLK

IOVDD

SDO

DGND

1

23

SDI

2

LDAC

DVDD

SDOSEL

3

AGND

4

AVDD

5

VREFL-S

VREFH-S

7

6

22

DAC8881

VOUT

RFB

8

9

VREFL-F

VREFH-F NC

10

11

12

(Thermal Pad)

SDOSEL

19

18    PDN

17    RST

16    USB/BTC

CS

15    GAIN

14    RSTSEL

13    NC

Reset DAC Registers

RW2

RW1

VOUT

External Reference

+5.0000V

Figure 68. Analog Output Closed-Loop Configuration

(RW1 and RW2 represent wiring resistance)

24            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

REFERENCE INPUTS

The reference high input, VREFH, can be set to any voltage in the range of 1.25V to AVDD. The reference low input, VREFL, can be set to any voltage in the range of –0.2V to +0.2V (to provide a small offset to the output of the DAC8881, if desired). The current into VREFH and out of VREFL depends on the DAC code, and can vary from approximately 0.5mA to 1mA in the gain = 1X mode of operation. The reference high and low inputs appear as varying loads to the external reference circuit. If the external references can source or sink the required current, and if low impedance connections are made to the VREFH  and VREFL  pins, external reference buffers are not required. Figure 67 shows a simple configuration of the DAC8881 using external references without force/sense reference buffers.

Kelvin sense connections for the reference high and low are included on the DAC8881. When properly used with external reference buffer op amps, these reference Kelvin sense pins ensure that the driven reference high and low voltages remain stable versus varying reference load currents. Figure 69 shows an example of a reference force/sense configuration of the DAC8881 operating from a single analog supply voltage. Both the VREFL  and VREFH reference voltages are set to levels of 100mV from the DAC8881 supply rails, and are derived from a +5V external reference. Figure 71 and Figure 70 illustrate the effect of not using the reference force/sense buffers to drive the DAC8881 VREFL and VREFH pins. A slight degradation in INL and DNL performance of approximately 0.1

LSB may be seen without the use of the force/sense buffer configuration.

External Reference

+5.0000V

2kW

+4.900V

+5V

OPA2350

2200pF

50W

1000pF

SCLK

1

SDI

2

LDAC

3

AGND

4

AVDD

5

VREFL-S

VREFH-S VOUT RFB VREFL-F VREFH-F

NC

7

6

8

9

10

11

12

DAC8881

96kW

+0.100V

50W

2kW

2200pF

1000pF

Figure 69. Buffered References (VREFH = +4.900V and VREFL = 100mV)

| 1.0 |  |  |  |  |  |  | +25ﾰC |
| --- | --- | --- | --- | --- | --- | --- | --- |

0.8

LE (LSB)

0.6

0.4

0.2

| 1.0 |  |  |  |  |  |  | +25ﾰC |
| --- | --- | --- | --- | --- | --- | --- | --- |

0.8

LE (LSB)

0.6

0.4

0.2

   0                                                                                                                                0

0.2

0.4

0.6

0.8

1.0

| 1.0 |  |  |  |  |  |  | +25ﾰC |
| --- | --- | --- | --- | --- | --- | --- | --- |

0.8

DLE (LSB)

0.6

0.4

0.2

0.2

0.4

0.6

0.8

1.0

1.0

0.8

0.6

0.4

0.2

DLE (LSB)

   0                                                                                                                                0

0.2

0.4

0.6

0.8

1.0

0       8192   16384 24576 32768

40960 49152 57344 65536

0.2

0.4

0.6

0.8

1.0

| 0 8192 16384 24576 32768 |  |  |  |  |  |  | +25ﾰC |
| --- | --- | --- | --- | --- | --- | --- | --- |

40960 49152 57344 65536

Digital Input Code

Digital Input Code

Figure 70. Linearity and Differential Linearity Error                           Figure 71. Linearity and DifferentialLinearity Error for Figure 67 without Reference Buffers                                                for Figure 69 with Reference Buffers

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            25

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

OUTPUT RANGE

The maximum output range of the DAC8881 is VREFL to VREFH      G, where G is the output buffer gain set by the GAIN pin. When the GAIN pin is connected to DGND, the output buffer gain = 1. When the GAIN pin is connected to IOVDD, the output buffer gain = 2. The output range must not be greater than AVDD; otherwise, output saturation occurs. The DAC8881 output transfer function is given in Equation 1:

V       VREFH  VREFL  ﾴ CODE ﾴ Buffer Gain + V

OUT =

Where:

65536

REFL

(1)

CODE = 0 to 65535. This is the digital code loaded to the DAC. Buffer Gain = 1 or 2 (set by the GAIN pin).

VREFH = reference high voltage applied to the device. VREFL = reference low voltage applied to the device.

INPUT DATA FORMAT

The USB/BTC pin defines the input data format. When this pin is connected to IOVDD, the input data format is straight binary, as shown in Table 1. When this pin is connected to DGND, the input data format is two's complement, as shown in Table 2.

Table 1. Output vs Straight Binary Code

| USB CODE | 5V RANGE | DESCRIPTION |
| --- | --- | --- |
| FFFFh | +4.99992 | +Full-Scale – 1LSB |
| C000h | +3.75000 | 3/4-Scale |
| 8000h | +2.50000 | Midscale |
| 4000h | +1.25000 | 1/4-Scale |
| 0000h | 0.00000 | Zero-Scale |

Table 2. Output vs Two's Complement Code

| BTC CODE | 5V RANGE | DESCRIPTION |
| --- | --- | --- |
| 7FFFh | +4.99992 | +Full-Scale – 1LSB |
| 4000h | +3.75000 | 3/4-Scale |
| 0000h | +2.50000 | Midscale |
| FFFFh | +2.49992 | Midscale – 1LSB |
| C000h | +1.25000 | 1/4-Scale |
| 8000h | 0.00000 | Zero-Scale |

POWER DOWN

The DAC8881 has a hardware power-down function. When the PDN pin is high, the device is in power-down mode. The VOUT  pin is connected to ground through an internal 10kΩ resistor, but the contents of the input register and the DAC latch do not change. In power-down mode, SPI communication is still active.

HARDWARE RESET

When the RST pin is low, the device is in hardware reset mode, and the input register and DAC latch are set to the value defined by the RSTSEL pin. After RST goes high, the device is in normal operating mode.

26            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

POWER-ON RESET

The DAC8881 has a power-on reset function. After power-on, the value of the input register, the DAC latch, and the output from the VOUT pin are set to the value defined by the RSTSEL pin.

PROGRAM RESET VALUE

After a power-on reset or a hardware reset, the output voltage from the VOUT  pin and the values of the input register and DAC latch are determined by the status of the RSTSEL pin and the input data format, as shown in Table 3.

Table 3. Reset Value

| RSTSEL PIN | USB/BTC PIN | INPUT FORMAT | VOUT | VALUE OF INPUT REGISTER AND DAC LATCH |
| --- | --- | --- | --- | --- |
| DGND | IOVDD | Straight Binary | 0 | 0000h |
| IOVDD | IOVDD | Straight Binary | Midscale | 8000h |
| DGND | DGND | Two's Complement | Midscale | 0000h |
| IOVDD | DGND | Two's Complement | 0 | 8000h |

SERIAL INTERFACE

The DAC8881 is controlled by a versatile 3-wire serial interface that operates at clock rates of up to 50MHz and is compatible with SPI, QSPI™, MICROWIRE™, and DSP™ interface standards.

Input Shift Register

Data are loaded into the device as a 16-bit word under the control of the serial clock input, SCLK. The timing diagrams for this operation are shown in the Timing Diagram section.

The CS input is a level-triggered input that acts as a frame synchronization signal and chip enable. Data can be transferred into the device only while CS is low. To start the serial data transfer, CS should be taken low, observing the minimum CS falling edge to SCLK rising edge setup time, t2. After CS goes low, serial data are clocked into the device input shift register on the rising edges of SCLK for 16 or more clock pulses. If a frame contains less than 16 bits of data, the frame is invalid. Invalid data are not written into the input register and DAC, although the input register and DAC will continue to hold data from the preceding valid data cycle. If more than 16 bits of data are transmitted in one frame, the last 16 bits are written into the shift register and DAC. CS may be taken high after the rising edge of the 16th SCLK pulse, observing the minimum SCLK rising edge to CS rising edge time, t7. The contents of the shift register are transferred into the input register on the rising edge of CS. When data have been transferred into the input register of the DAC, the corresponding DAC register and DAC output can be updated by taking the LDAC pin low.

Stand-Alone Mode

When the SDOSEL pin is tied to IOVDD, the interface is in Stand-Alone mode. This mode provides serial readback for diagnostic purposes. The new input data (16 bits) are clocked into the device shift register and the existing data in the input register (16 bits) are shifted out from the SDO pin. If more than 16 SCLKs are clocked when CS is low, the contents of the input register are shifted out from the SDO pin, followed by zeroes; the last

16 bits of input data remain in the shift register. If less than 16 SCLKs are clocked while CS is low, the data from the SDO pin are part of the data in the input register and must be ignored. Refer to Figure 2 for further detail.

Daisy-Chain Mode

When the SDOSEL pin is tied to GND, the interface is in Daisy-Chain mode. For systems that contain several

DACs, the SDO pin may be used to daisy-chain several devices together.

In Daisy-Chain mode, SCLK is continuously applied to the input shift register while CS is low. If more than 16 clock pulses are applied, the data ripples out of the shift register and appears on the SDO line. These data are clocked out on the falling edge of SCLK and are valid on the rising edge. By connecting this line to the DIN input

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            27

Product Folder Link(s): DAC8881

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

www.ti.com

on the next DAC in the chain, a multi-DAC interface is constructed. 16 clock pulses are required for each DAC in the system. Therefore, the total number of clock cycles must be equal to (16 x N), where N is the total number of devices in the chain. When the serial transfer to all devices is complete, CS should be taken high. This action prevents any further data from being clocked into the input shift register. The contents in the shift registers are transferred into the relevant input registers on the rising edge of the CS signal.

A  continuous  SCLK  source  may  be  used  if  CS  can  be  held  low  for  the  correct  number  of  clock  cycles. Alternatively, a burst clock containing the exact number of clock cycles can be used and CS can be taken high some time later. When the transfer to all input registers is complete, a common LDAC signal updates all DAC registers, and all analog outputs update simultaneously.

DOUBLE-BUFFERED INTERFACE

The DAC8881 has a double-buffered interface consisting of two register banks: the input register and the DAC latch. The input register is connected directly to the input shift register and the digital code is transferred to the input register upon completion of a valid write sequence. The DAC latch contains the digital code used by the resistor R-2R ladder. The contents of the DAC latch defines the output from the DAC.

Access to the DAC register is controlled by the LDAC pin. When LDAC is high, the DAC register is latched and the input register can change state without affecting the contents of the DAC latch. When LDAC is low, however, the DAC latch becomes transparent and the contents of the input register is transferred to the DAC register.

Load DAC Pin (LDAC)

LDAC transfers data from the input register to the DAC register (and, therefore, updates the DAC output). The contents of the DAC latch (and the output from DAC) can be changed in two ways, depending on the status of LDAC.

Synchronous Mode

When LDAC is tied low, the DAC register updates as soon as new data are transferred into the input register after the rising edge of CS.

Asynchronous Mode

When LDAC is high, the DAC latch is latched. The DAC latch (and DAC output) is not updated at the same time that the input register is written to. When LDAC goes low, the DAC register updates with the contents of the input register.

1.8V TO 5.5V LOGIC INTERFACE

All digital input and output pins are compatible with any logic supply voltage between 1.8V and 5.5V. Connect the interface logic supply voltage to the IOVDD  pin. Although timing is specified down to 2.7V (see the Timing Characteristics), IOVDD can operate as low as 1.8V, but with degraded timing and temperature performance. For the lowest power consumption, logic VIH  levels should be as close as possible to IOVDD, and logic VIL  levels should be as close as possible to GND. Note that the DAC8881 core internal digital logic operates from the same voltage as the 2.7V to 5.5V AVDD supply, so the DVDD pin must also be connected to the AVDD supply voltage.

28            Submit Documentation Feedback                                                                                      Copyright © 2007, Texas Instruments Incorporated

Product Folder Link(s): DAC8881

www.ti.com

DAC8881

SBAS422A – JULY 2007 – REVISED SEPTEMBER 2007

APPLICATION INFORMATION

BIPOLAR OPERATION USING THE DAC8881

The DAC8881 is designed for single-supply operation; however, a bipolar output range is also possible using the circuit shown in Figure 72. This circuit gives a bipolar output voltage range of VBIP, where VBIP is represented by Equation 2. Note that for this circuit to work, the DAC8881 must operate in the gain = 1X mode configuration with GAIN = DGND. The output voltage for any input code can be calculated with Figure 72:

VBIP(CODE) =  1 +

Where:

R3  + R3

R2        R1

ﾴ C    ODE

65536

  R3   ﾴ

R

REF

V

1

(2)

VBIP(CODE) = bipolar output voltage versus CODE from the OPA211. CODE = 0 to 65535. This is the digital code loaded to the DAC.

VREF = reference high voltage applied to the DAC8881.

By  first  choosing  a  value  for  resistor  R3,  R1  and  R2   can  be  determined  by  Equation  3  and  Equation  4, respectively:

VREF

R1 =   V

ﾴ R3

BIP                                                                                                                                                                                                                                                                                (3)

VREF ﾴ R3

R2 =

VBIP

 VREF                                                                                                                                                                                                                                                              (4)

Where:

VBIP= peak desired output voltage for bipolar output.

VREF = reference high voltage applied to the DAC8881. NOTE: VBIP ≥ VREF. R3 = OPA211 feedback resistor chosen by user.

Note that R2 is not required in the circuit of Figure 72 for bipolar output voltage ranges equal to ±VREF.

Using the previous equations, and with VREF = 5V and R3 set to 10kΩ, a ±8V output span can be achieved with

R1 calculated to be 6.25kΩ and R2 to be 16.67kΩ.

Similarly, a near ±15V rail-to-rail output can be achieved with R1 calculated to be 3.33kΩ and R2 calculated to be

5kΩ.

VREFL

DAC8881 VOUT

VREFH

+15V

R2                       OPA211

VBIP

VREF

-15V R1                                 R3

NOTE: Some pins omitted for clarity.

Figure 72. Bipolar Operation Using the DAC8881

Copyright © 2007, Texas Instruments Incorporated                                                                                                  Submit Documentation Feedback            29

Product Folder Link(s): DAC8881

PACKAGE OPTION ADDENDUM

www.ti.com                                                                                                                                                                                                                27-Feb-2008

PACKAGING INFORMATION

Orderable Device Status (1) Package Package Pins Package Eco Plan (2) Lead/Ball Finish MSL Peak Temp (3) Type Drawing Qty

DAC8881SRGER ACTIVE VQFN RGE 24 3000 Green (RoHS & CU NIPDAU Level-2-260C-1 YEAR no Sb/Br)

DAC8881SRGERG4 ACTIVE VQFN RGE 24 3000 Green (RoHS & CU NIPDAU Level-2-260C-1 YEAR no Sb/Br)

DAC8881SRGET ACTIVE VQFN RGE 24 250 Green (RoHS & CU NIPDAU Level-2-260C-1 YEAR no Sb/Br)

DAC8881SRGETG4 ACTIVE VQFN RGE 24 250 Green (RoHS & CU NIPDAU Level-2-260C-1 YEAR no Sb/Br)

(1) The marketing status values are defined as follows:

ACTIVE: Product device recommended for new designs.

LIFEBUY: TI has announced that the device will be discontinued, and a lifetime-buy period is in effect.

NRND: Not recommended for new designs. Device is in production to support existing customers, but TI does not recommend using this part in a new design.

PREVIEW: Device has been announced but is not in production. Samples may or may not be available.

OBSOLETE: TI has discontinued the production of the device.

(2) Eco Plan - The planned eco-friendly classification: Pb-Free (RoHS), Pb-Free (RoHS Exempt), or Green (RoHS & no Sb/Br) - please check http://www.ti.com/productcontent for the latest availability information and additional product content details.

TBD: The Pb-Free/Green conversion plan has not been defined.

Pb-Free (RoHS): TI's terms "Lead-Free" or "Pb-Free" mean semiconductor products that are compatible with the current RoHS requirements for all 6 substances, including the requirement that lead not exceed 0.1% by weight in homogeneous materials. Where designed to be soldered at high temperatures, TI Pb-Free products are suitable for use in specified lead-free processes.

Pb-Free (RoHS Exempt): This component has a RoHS exemption for either 1) lead-based flip-chip solder bumps used between the die and package, or 2) lead-based die adhesive used between the die and leadframe. The component is otherwise considered Pb-Free (RoHS compatible) as defined above.

Green (RoHS & no Sb/Br): TI defines "Green" to mean Pb-Free (RoHS compatible), and free of Bromine (Br) and Antimony (Sb) based flame retardants (Br or Sb do not exceed 0.1% by weight in homogeneous material)

(3)  MSL, Peak Temp. -- The Moisture Sensitivity Level rating according to the JEDEC industry standard classifications, and peak solder temperature.

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI

to Customer on an annual basis.

Addendum-Page 1

PACKAGE MATERIALS INFORMATION

www.ti.com                                                                                                                                                                                                                19-Mar-2008

TAPE AND REEL INFORMATION

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Reel Diameter (mm) | Reel Width W1 (mm) | A0 (mm) | B0 (mm) | K0 (mm) | P1 (mm) | W (mm) | Pin1 Quadrant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAC8881SRGER | VQFN | RGE | 24 | 3000 | 330.0 | 12.4 | 4.3 | 4.3 | 1.5 | 8.0 | 12.0 | Q2 |
| DAC8881SRGET | VQFN | RGE | 24 | 250 | 330.0 | 12.4 | 4.3 | 4.3 | 1.5 | 8.0 | 12.0 | Q2 |

Pack Materials-Page 1

PACKAGE MATERIALS INFORMATION

www.ti.com                                                                                                                                                                                                                19-Mar-2008

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Length (mm) | Width (mm) | Height (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAC8881SRGER | VQFN | RGE | 24 | 3000 | 340.5 | 333.0 | 20.6 |
| DAC8881SRGET | VQFN | RGE | 24 | 250 | 340.5 | 333.0 | 20.6 |

Pack Materials-Page 2

IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete. All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment.

TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third-party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

Reproduction of TI information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations, and notices. Reproduction of this information with alteration is an unfair and deceptive business practice. TI is not responsible or liable for such altered documentation. Information of third parties may be subject to additional restrictions.

Resale of TI products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service and is an unfair and deceptive business practice. TI is not responsible or liable for any such statements.

TI products are not authorized for use in safety-critical applications (such as life support) where a failure of the TI product would reasonably be expected to cause severe personal injury or death, unless officers of the parties have executed an agreement specifically governing

such use. Buyers represent that they have all necessary expertise in the safety and regulatory ramifications of their applications, and acknowledge and agree that they are solely responsible for all legal, regulatory and safety-related requirements concerning their products and any use of TI products in such safety-critical applications, notwithstanding any applications-related information or support that may be provided by TI. Further, Buyers must fully indemnify TI and its representatives against any damages arising out of the use of TI products in such safety-critical applications.

TI products are neither designed nor intended for use in military/aerospace applications or environments unless the TI products are specifically designated by TI as military-grade or "enhanced plastic." Only products designated by TI as military-grade meet military specifications. Buyers acknowledge and agree that any such use of TI products which TI has not designated as military-grade is solely at the Buyer's risk, and that they are solely responsible for compliance with all legal and regulatory requirements in connection with such use.

TI products are neither designed nor intended for use in automotive applications or environments unless the specific TI products are designated by TI as compliant with ISO/TS 16949 requirements. Buyers acknowledge and agree that, if they use any non-designated products in automotive applications, TI will not be responsible for any failure to meet such requirements.

Following are URLs where you can obtain information on other Texas Instruments products and application solutions:

| Products |  | Applications |  |
| --- | --- | --- | --- |
| Amplifiers | amplifier.ti.com | Audio | www.ti.com/audio |
| Data Converters | dataconverter.ti.com | Automotive | www.ti.com/automotive |
| DLP® Products | www.dlp.com | Broadband | www.ti.com/broadband |
| DSP | dsp.ti.com | Digital Control | www.ti.com/digitalcontrol |
| Clocks and Timers | www.ti.com/clocks | Medical | www.ti.com/medical |
| Interface | interface.ti.com | Military | www.ti.com/military |
| Logic | logic.ti.com | Optical Networking | www.ti.com/opticalnetwork |
| Power Mgmt | power.ti.com | Security | www.ti.com/security |
| Microcontrollers | microcontroller.ti.com | Telephony | www.ti.com/telephony |
| RFID | www.ti-rfid.com | Video & Imaging | www.ti.com/video |
| RF/IF and ZigBee® Solutions | www.ti.com/lprf | Wireless | www.ti.com/wireless |

Mailing Address: Texas Instruments, Post Office Box 655303, Dallas, Texas 75265

Copyright © 2009, Texas Instruments Incorporated