® 	INA138

INA168

For most current data sheet and other product information, visit www.burr-brown.com

High-Side Measurement

CURRENT SHUNT MONITOR

FEATURES

●  COMPLETE UNIPOLAR HIGH-SIDE CURRENT MEASUREMENT CIRCUIT

●  WIDE SUPPLY AND COMMON-MODE RANGE

●  INA138: 2.7V to 36V

●  INA168: 2.7V to 60V

●  INDEPENDENT SUPPLY AND INPUT COMMON-MODE VOLTAGES

●  SINGLE RESISTOR GAIN SET

●  LOW QUIESCENT CURRENT (25  A typ)

●  SOT23-5 Package

APPLICATIONS

●  CURRENT SHUNT MEASUREMENT Automotive, Telephone, Computers

●  PORTABLE & BATTERY BACKUP SYSTEMS

●  BATTERY CHARGERS

●  POWER MANAGEMENT

●  CELL PHONES

●  PRECISION CURRENT SOURCE

DESCRIPTION

The INA138 and INA168 are high-side, unipolar, current shunt monitors. Wide input common-mode voltage range, low quiescent current, and tiny SOT-23 packaging enable use in a variety of applications.

Input common-mode and power supply voltages are independent and can range from 2.7V to 36V for the INA138 and 2.7V to 60V for the INA168. Both models draw only 25  A quiescent current, which also permits connecting the power supply to either side of the current measurement shunt with minimal error.

The device converts a differential input voltage to a current output. This current is converted back to a voltage with an external load resistor that sets any gain from 1 to over 100. Although designed for current shunt measurement, the circuit invites creative applications in measurement and level shifting.

Both the INA138 and INA168 are available in SOT23-5 and are specified for the –40 C to +85 C industrial temperature range.

VIN+

RS	IS

3	4

VIN+	VIN–

Load

5k	5k

V+

5

GND

2

OUT

1

VO = ISRSRL/5k

RL

International Airport Industrial Park  •  Mailing Address: PO Box 11400, Tucson, AZ 85734  •  Street Address: 6730 S. Tucson Blvd., Tucson, AZ  85706  •  Tel: (520) 746-1111

Twx: 910-952-1111  •  Internet: http://www.burr-brown.com/  •  Cable: BBRCORP  •  Telex: 066-6491  •  FAX: (520) 889-1510  •  Immediate Product Info: (800) 548-6132

©1999 Burr-Brown Corporation	PDS-1576A	Printed in U.S.A. December, 1999

SPECIFICATIONS

At TA = –40 C to +85 C, VS = 5V, VIN+ = 12V, ROUT = 125k  , unless otherwise noted.

VSENSE = VIN – VIN

| VIN , VIN PARAMETER | CONDITION | INA138 |  |  | INA168 |  |  |  |  | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN | TYP | MAX | MIN |  | TYP |  | MAX |  |
| INPUT Full-Scale Sense Voltage Common-Mode Input Range Common-Mode Rejection Offset Voltage(1) vs Temperature vs Power Supply Input Bias Current | + – VIN+ = 2.7V to 40V, VSENSE = 50mV VIN+ = 2.7V to 60V, VSENSE = 50mV TMIN to TMAX V– = 2.7V to 40V, VSENSE = 50mV V– = 2.7V to 60V, VSENSE = 50mV + – | 2.7 100 | 100 120 0.2 1 0.1 2 | 500 36 1 10 |  |  | ✽ |  | ✽ 60 ✽ 10 | mV V dB dB mV V/ C V/V V/V uA |
|  |  |  |  |  |  | ✽ |  |  |  |  |
|  |  |  |  |  | 100 |  | 120 ✽ ✽ 0.1 ✽ |  |  |  |
| OUTPUT Transconductance vs Temperature Nonlinearity Error Total Output Error Output Impedance Voltage Output Swing to Power Supply, V+ Swing to Common Mode, VCM | VSENSE = 10mV – 150mV VSENSE = 100mV VSENSE = 10mV to 150mV VSENSE = 100mV | 198 | 200 10 0.01 0.5 1 ¦¦ 5 (V+) – 0.8 VCM – 0.5 | 202 0.1 2 (V+) – 1.0 VCM – 0.8 | ✽ |  | ✽ ✽ ✽ ✽ ✽ ✽ ✽ |  | ✽ ✽ ✽ ✽ ✽ | A/V nA/ C % % G ¦¦ pF V V |
| FREQUENCY RESPONSE Bandwidth Settling Time (0.1%) | ROUT = 5k ROUT = 125k 5V Step, ROUT = 5k 5V Step, ROUT = 125k |  | 800 32 1.8 30 |  |  |  | ✽ ✽ ✽ ✽ |  |  | kHz kHz s s |
| NOISE Output-Current Noise Density Total Output-Current Noise | BW = 100kHz |  | 9 3 |  |  |  | ✽ ✽ |  |  | pA/ Hz nA RMS |
| POWER SUPPLY Operating Range, V+ Quiescent Current | VSENSE = 0, IO = 0 | 2.7 | 25 | 36 45 | ✽ |  | ✽ |  | 60 ✽ | V A |
| TEMPERATURE RANGE Specification, TMIN to TMAX Operating Storage Thermal Resistance JA |  | –40 –55 –65 | 200 | 85 125 150 | ✽ ✽ ✽ |  | ✽ |  | ✽ ✽ ✽ | C C C C/W |

NOTES: (1) Defined as the amount of input voltage, VSENSE, to drive the output to zero.

PIN CONFIGURATION

TOP VIEW	SOT

ELECTROSTATIC DISCHARGE SENSITIVITY

This integrated circuit can be damaged by ESD. Burr-Brown

OUT

GND

V+	recommends  that  all  integrated  circuits  be  handled  with appropriate precautions. Failure to observe proper handling

+	–	and installation procedures can cause damage.

VIN

ABSOLUTE MAXIMUM RATINGS(1)

VIN

ESD damage can range from subtle performance degradation to complete device failure. Precision integrated circuits may be more susceptible to damage because very small parametric changes could cause the device not to meet its published specifications.

| Supply Voltage, V+ |  |  |
| --- | --- | --- |
| 1 |  | 5 |
| 2 |  |  |
| 3 |  | 4 |

INA138 ............................................................................... –0.3V to 60V INA168 ............................................................................... –0.3V to 60V

+	–

Analog Inputs, VIN , VIN

INA138

Common Mode ............................................................... –0.3V to 40V

+	–

Differential (VIN ) – (VIN) ..................................................... –40V to 2V

INA168

Common Mode ............................................................... –0.3V to 60V

+	–

Differential (VIN ) – (VIN) ..................................................... –40V to 2V

Analog Output, Out .............................................................. –0.3V to 40V

Operating Temperature .................................................. –55 C to +125 C Storage Temperature ..................................................... –55 C to +125 C Junction Temperature .................................................................... +150 C Lead Temperature (soldering, 10s) ............................................... +300 C

NOTE: (1) Stresses above these ratings may cause permanent damage. Exposure to absolute maximum conditions for extended periods may degrade device reliability. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those specified is not implied.

PACKAGE /ORDERING INFORMATION

| PRODUCT | PACKAGE | PACKAGE DRAWING NUMBER | SPECIFIED TEMPERATURE RANGE | PACKAGE MARKING | ORDERING NUMBER(1) | TRANSPORT MEDIA |
| --- | --- | --- | --- | --- | --- | --- |
| INA138NA " | SOT-23-5 Surface Mount " | 331 " | –40 C to +85 C " | " | INA138NA/250 INA138NA/3K | Tape and Reel Tape and Reel |
| INA168NA(2) " | SOT-23-5 Surface Mount " | 331 " | –40 C to +85 C " | " | INA168NA/250 INA168NA/3K | Tape and Reel Tape and Reel |

NOTE: (1) Models with a slash ( /) are available only in Tape and Reel in the quantities indicated (e.g., /3K indicates 3000 devices per reel). Ordering 3000 pieces of “INA138NA/3K” will get a single 3000-piece Tape and Reel. (2) INA168 available Q2'00.

The information provided herein is believed to be reliable; however, BURR-BROWN assumes no responsibility for inaccuracies or omissions. BURR-BROWN assumes no responsibility for the use of this information, and all use of such information shall be entirely at the user’s own risk. Prices and specifications are subject to change without notice. No patent rights or licenses to any of the circuits described herein are implied or granted to any third party. BURR-BROWN does not authorize or warrant any BURR-BROWN product for use in life support devices and/or systems.

TYPICAL PERFORMANCE CURVES

At TA = +25 C, V+ = 5V, V +  = 12V, R  = 125k  , unless otherwise noted.

IN	L

40

30

RL = 50k

20

Gain (dB)

10

RL = 5k

0

GAIN vs FREQUENCY

120

100

Common-Mode Rejection (dB)

80

60

40

COMMON-MODE REJECTION vs FREQUENCY

G = 100

G = 10

G = 1

–10

–20

CL = 10nF

CL = 1nF

20

CL = 100pF

0

100	1k	10k	100k	1M	10M

Frequency (Hz)

0.1	1	10	100	1k	10k

Frequency (Hz)

100k

140

POWER-SUPPLY REJECTION vs FREQUENCY

TOTAL OUTPUT ERROR vs VIN

5

+	–

Power-Supply Rejection (dB)

120

100

80

60

40

G = 1

G = 100

G = 10

Total Output Error (%)

0

–5

–10

–55C

+25C

VIN = (VIN  – VIN)

20

1	10	100	1k	10k	100k

Frequency (Hz)

–15

0	25	50	75	100	125

VIN (mV)

150	200

TOTAL OUTPUT ERROR

vs POWER-SUPPLY VOLTAGE

2

Output error is essentially independent of both

Total Output Error (%)

1	V+ supply voltage and

input common-mode voltage.

0	G = 1

QUIESCENT CURRENT

vs POWER-SUPPLY VOLTAGE

Quiescent Current (A)

50

40

–1	G = 25

G = 10

10

–2

0	10	20	30	40	50

Power-Supply Voltage (V)

60	70

0

| 0 10 20 30 40 50 |  |  |  |  |  | +150 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | +125 |  |
|  |  |  |  |  |  | +25 |  |
|  |  |  |  |  | Use ( | INA168 V+) > 36 | with V |

Power-Supply Voltage (V)

60	70

TYPICAL PERFORMANCE CURVES (Cont.)

At TA = +25 C, V+ = 5V, V +  = 12V, R  = 125k  , unless otherwise noted.

IN	L

STEP RESPONSE

STEP RESPONSE

200mV

G = 1

100mV

G = 25

0V

1V/div

50mV/div

100mV

G = 1

0mV

G = 10

0V

500mV/div

10  s/div

10  s/div

OPERATION

Figure 1 shows the basic circuit diagram for both the INA138 and INA168. Load current, IS, is drawn from supply, VS, through shunt resistor, Rs. The voltage drop in the shunt resistor, VS, is forced across Rg1  by the internal op-amp, causing current to flow into the collector of Q1. External resistor, RL, converts the output current to a volt- age, VOUT, at the Out pin.

The transfer function for the INA138 is:

BASIC CONNECTION

Figure 1 shows the basic connection of the INA138. The input pins, V +  and V – , should be connected as closely as

IN 	IN

possible to the shunt resistor to minimize any resistance in

series with the shunt resistance. The output resistor, RL, is shown connected between pin 1 and ground. Best accuracy is achieved with the output voltage measured directly across RL. This is especially important in high-current systems where load current could flow in the ground connections, affecting the measurement accuracy.

+ 	– 	No power supply bypass capacitors are required for stability

IO = gm  (VIN – VIN )	(1)

where gm  = 200  A/V	(2)

of the INA138. However, applications with noisy or high impedance power supplies may require de-coupling capaci- tors to reject power supply noise. Connect bypass capacitors close to the device pins.

+ 	–

In the circuit of Figure 1, the input voltage, (VIN – VIN), is

equal to IS  • RS and the output voltage, VOUT, is equal to

IO   •  RL.  The  transconductance,  gm,  of  the  INA138  is

200  A/V. The complete transfer function for the current

measurement amplifier in this application is:

VOUT = (IS) (RS) (200  A/V) (RL)	 (3) The maximum differential input voltage for accurate mea-

surements is 0.5V, which produces a 100  A output current. A differential input voltage of up to 2V will not cause damage. Differential measurements (pins 3 and 4)  must be unipolar with a more-positive voltage applied to pin 3. If a more-negative voltage is applied to pin 3, the output current, IO, will be zero, but it will not cause damage.

POWER SUPPLIES

The input circuitry of the INA138 can accurately measure beyond its power supply voltage, V+. For example, the V+ power supply can be 5V while the load power supply is voltage is up to +36V (or +60V with INA168). However, the output voltage range of the Out terminal is limited by the lesser of the two voltages (see “Output Voltage Range”).

SELECTING RS AND RL

The value chosen for the shunt resistor, RS, depends on the application and is a compromise between small-signal accu- racy and maximum permissible voltage loss in the measure- ment line. High values of RS  provide better accuracy at

VP

Load Power Supply

+2.7 to 36V(1)

Shunt

RS

V	V

I

S

V+ power can be common or

indepedent of	V+

+		– IN	 IN

3	4

Load

load supply.

2.7  (V+)  36V(1)

RG1

5k

RG2

5k

5

Q1

INA138

2

OUT

1

I0

+ RL	VO

–

| NOTE: (1) Maximum VP and V+ voltage is 60V with INA168. VOLTAGE GAIN | EXACT RL ( ) | NEAREST 1% RL ( ) |
| --- | --- | --- |
| 1 2 5 10 20 50 100 | 5k 10k 25k 50k 100k 250k 500k | 4.99k 10k 24.9k 49.9k 100k 249k 499k |

FIGURE 1. Basic Circuit Connections.

lower currents by minimizing the effects of offset, while low values of RS minimize voltage loss in the supply line. For most applications, best performance is attained with an RS value that provides a full-scale shunt voltage of 50mV to

100mV. Maximum input voltage for accurate measurements is 500mV.

output swing. The maximum output voltage compliance is limited by the lower of the two equations below:

+ 	–

Vout max = (V+) – 0.7V – (VIN  – VIN)	 (4)

or

–

RL is chosen to provide the desired full-scale output voltage.

The output impedance of the INA138 Out terminal is very

high which permits using values of RL  up to 500k	with excellent accuracy. The input impedance of any additional circuitry at the output should be much higher than the value

BANDWIDTH

Vout max = VIN – 0.5V	(5)

(whichever is lower)

of RL to avoid degrading accuracy.

Some A/D converters have input impedances that will sig- nificantly affect measurement gain. The input impedance of the A/D converter can be included as part of the effective RL if its input can be modeled as a resistor to ground. Alterna- tively, an op-amp can be used to buffer the A/D converter input. See Figure 1 for recommended values of RL.

OUTPUT VOLTAGE RANGE

The output of the INA138 is a current, which is converted to a voltage by the load resistor, RL. The output current remains accurate within the compliance voltage range of the output circuitry.  The shunt voltage and the input common-mode and  power  supply  voltages  limit  the  maximum  possible

IS

Measurement bandwidth is affected by the value of the load

resistor, RL. High gain produced by high values of RL will yield a narrower measurement bandwidth (see Typical Per- formance Curves). For widest possible bandwidth, keep the capacitive load on the output to a minimum. Reduction in bandwidth due to capacitive load is shown in the Typical Performance Curves.

If bandwidth limiting (filtering) is desired, a capacitor can be added to the output, as shown in Figure 3. This will not cause instability.

APPLICATIONS

The INA138 is designed for current shunt measurement circuits as shown in Figure 1, but its basic function is useful in a wide range of circuitry. A creative engineer will find many unforeseen uses in measurement and level shifting circuits. A few ideas are shown.

3	4

INA138

OPA340

ZIN

3	4

f–3dB

1

INA138	f–3dB =

2R C

L   L

VO

RL

Buffer of amp drives A/D converter

without affecting gain.

RL	CL

FIGURE 2. Buffering Output to Drive A/D Converter.

FIGURE 3. Output Filter.

3	4	V+

3	4	V+

INA138	R1

V0

1

R2

INA138

1

REF200

100A V0

RL

Gain Set by R1//R2

Output Offset = (V+)R2

R1+R2

a). Using resistor divider.

Gain Set by RL

Output Offset = (100A)(RL)

(independent of V+)

b). Using current source.

FIGURE 4. Offsetting the Output Voltage.