Low rDS(on) . . . 7  Typ

SLIS093C − MARCH 2000 − REVISED APRIL 2005

D, N, OR PW PACKAGE

Eight Power DMOS Transistor Outputs of

100-mA Continuous Current

250-mA Current Limit Capability

ESD Protection . . . 2500 V

Output Clamp Voltage . . . 33 V

Enhanced Cascading for Multiple Stages

All Registers Cleared With Single Input

VCC     1

SER IN    2

DRAIN0    3

DRAIN1    4

DRAIN2    5

DRAIN3    6

CLR    7

G    8

16    GND

15    SRCK

14    DRAIN7

13    DRAIN6

12    DRAIN5

11    DRAIN4

10    RCK

9    SER OUT

description

logic symbol†

The TPIC6C596 is a monolithic, medium-voltage, low-current power 8-bit shift register designed for use in systems that require relatively moderate load power such as LEDs. The device contains a built-in voltage clamp on the outputs for inductive transient protection. Power driver applications include relays, solenoids, and other low-current or medium-voltage loads.

This device contains an 8-bit serial-in, parallel-out shift register that feeds an 8-bit D-type storage register. Data transfers through both the shift and storage registers on the rising edge of the shift

G    8

RCK    10

7

CLR

15

SRCK

SER IN    2

EN3

C2

SRG8

R

C1

1D                  2

3

DRAIN0

4

DRAIN1

5

DRAIN2

6

DRAIN3

11

DRAIN4

12

DRAIN5

register clock (SRCK) and the register clock (RCK), respectively. The storage register trans- fers data to the output buffer when shift register clear (CLR) is high. When CLR is low, all registers in the device are cleared. When output enable (G) is held high, all data in the output buffers is held low and all drain outputs are off. When G is held

13

DRAIN6

14

2                    DRAIN7

9

SER OUT

† This symbol is in accordance with ANSI/IEEE Std 91-1984 and IEC Publication 617-12.

low, data from the storage register is transparent to the output buffers. When data in the output buffers is low, the DMOS transistor outputs are off. When data is high, the DMOS transistor outputs have sink-current capability. The serial output (SER OUT) is clocked out of the device on the falling edge of SRCK to provide additional hold time for cascaded applications. This will provide improved performance for applications where clock signals may be skewed, devices are not located near one another, or the system must tolerate electromagnetic interference.

This device contains circuits to protect its inputs and outputs against damage due to high static voltages or electrostatic fields. These circuits have been qualified to protect this device against electrostatic discharges (ESD) of up to 2 kV according to MIL-STD-883C, Method 3015; however, it is advised that precautions be taken to avoid application of any voltage higher than maximum-rated voltages to these high-impedance circuits. During storage or handling, the device leads should be shorted together or the device should be placed in conductive foam. In a circuit, unused inputs should always be connected to an appropriated logic voltage level, preferably either VCC or ground. Specific guidelines for handling devices of this type are contained in the publication Guidelines for

Handling Electrostatic-Discharge-Sensitive (ESDS) Devices and Assemblies available from Texas Instruments.

Please be aware that an important notice concerning availability, standard warranty, and use in critical applications of

Texas Instruments semiconductor products and disclaimers thereto appears at the end of this data sheet.

!  "#  $      !    %#&' "       (  $)

(#" ! "          !%$"    "     ! %$   *$  $ !    $+ !   !  # $  !

!  ( ( ,      -)    (#"    %   "$!!   . ( $!      $"$!!  '-   "'#($

$!  .    '' %     $ $ !)

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

Copyright  2000–2005, Texas Instruments Incorporated

1

SLIS093C − MARCH 2000 − REVISED APRIL 2005

description (continued)

Outputs are low-side, open-drain DMOS transistors with output ratings of 33 V and 100 mA continuous sink-current capability. Each output provides a 250-mA maximum current limit at TC = 25C. The current limit decreases as the junction temperature increases for additional device protection. The device also provides up to 2500 V of ESD protection when tested using the human-body model and the 200-V machine model.

The TPIC6C596 is characterized for operation over the operating case temperature range of − 40C to 125C.

logic diagram (positive logic)

G   8

RCK  10

CLR   7

D                                        D

3      DRAIN0

SRCK  15

C1

CLR

C2

CLR                                                                    4

SER IN    2

D

C1

CLR

D

C2

CLR

DRAIN1

5      DRAIN2

D

C1

CLR

D

C2

CLR

6       DRAIN3

D

C1

CLR

D

C1

CLR

D

C2

CLR

D

C2

CLR

11    DRAIN4

12     DRAIN5

D

C1

CLR

D

C1

CLR

D

C2

CLR

D

C2

CLR

13    DRAIN6

14     DRAIN7

D

C1

CLR

D

C2

CLR

16        GND

D

C1

CLR

9     SER OUT

2                                                                           POST OFFICE BOX 655303  DALLAS, TEXAS 75265

SLIS093C − MARCH 2000 − REVISED APRIL 2005

schematic of inputs and outputs

EQUIVALENT OF EACH INPUT                                                        TYPICAL OF ALL DRAIN OUTPUTS

VCC

DRAIN

33 V

Input

25 V

12 V

20 V

GND

GND

absolute  maximum  ratings  over  recommended  operating  case  temperature  range  (unless otherwise noted)†

Logic supply voltage, VCC (see Note 1)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   7 V

Logic input voltage range, VI

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

− 0.3 V to 7 V

Power DMOS drain-to-source voltage, VDS (see Note 2)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33 V

Continuous source-to-drain diode anode current

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

250 mA

Pulsed source-to-drain diode anode current (see Note 3)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

500 mA

Pulsed drain current, each output, all outputs on, ID, TC = 25C (see Note 3)

. . . . . . . . . . . . . . . . . . .

250 mA

Continuous drain current, each output, all outputs on, ID, TC = 25C

. . . . . . . . . . . . . . . . . . . . . . . . . . .

100 mA

Peak drain current single output, IDM,TC = 25C (see Note 3)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

250 mA

Single-pulse avalanche energy, EAS (see Figure 4)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 30 mJ

Avalanche current, IAS (see Note 4)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

200 mA

Continuous total dissipation

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

See Dissipation Rating Table

Operating virtual junction temperature range, TJ

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . −40C to 150C

Operating case temperature range, TC

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . −40C to 125C

Storage temperature range, Tstg

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . −65C to 150C

Lead temperature 1,6 mm (1/16 inch) from case for 10 seconds

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

260C

† Stresses beyond those listed under “absolute maximum ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under “recommended operating conditions” is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability.

NOTES:   1.  All voltage values are with respect to GND.

2.  Each power DMOS source is internally connected to GND.

3.  Pulse duration  100 s and duty cycle  2%.

4.  DRAIN supply voltage = 15 V, starting junction temperature (TJS) = 25C, L = 1.5 H, IAS = 200 mA (see Figure 4).

DISSIPATION RATING TABLE

PACKAGE                 TC  25C POWER RATING

DERATING FACTOR ABOVE TC = 25C

TC = 125C POWER RATING

D                          1087 mW                        8.7 mW/C                        217 mW N                          1470 mW                      11.7 mW/C                        294 mW

PW                        1372 mW                  10.976 mW/C                        274 mW

POST OFFICE BOX 655303  DALLAS, TEXAS 75265                                                                                                        3

SLIS093C − MARCH 2000 − REVISED APRIL 2005

recommended operating conditions

|  | MIN MAX | UNIT |
| --- | --- | --- |
| Logic supply voltage, VCC | 4.5 5.5 | V |
| High-level input voltage, VIH | 0.85 VCC | V |
| Low-level input voltage, VIL | 0.15 VCC | V |
| Pulsed drain output current, TC = 25C, VCC = 5 V, all outputs on (see Notes 3 and 5 and Figure 11) | 250 | mA |
| Setup time, SER IN high before SRCK↑, tsu (see Figure 2) | 15 | ns |
| Hold time, SER IN high after SRCK↑, th (see Figure 2) | 15 | ns |
| Pulse duration, tw (see Figure 2) | 40 | ns |
| Operating case temperature, TC | −40 125 | C |

NOTES:   3.  Pulse duration  100 s and duty cycle  2%.

5.  Technique should limit TJ − TC to 10C maximum.

electrical characteristics, VCC = 5 V, TC = 25C (unless otherwise noted)

| PARAMETER | TEST CONDITIONS |  | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- |
| V(BR)DSX Drain-to-source breakdown voltage | ID = 1 mA |  | 33 37 | V |
| VSD Source-to-drain diode forward voltage | IF = 100 mA |  | 0.85 1.2 | V |
| VOH High-level output voltage, SER OUT | IOH = − 20 A, VCC = 4.5 V |  | 4.4 4.49 | V |
|  | IOH = − 4 mA, VCC = 4.5 V |  | 4 4.2 |  |
| VOL Low-level output voltage, SER OUT | IOL = 20 A, VCC = 4.5 V |  | 0.005 0.1 | V |
|  | IOL = 4 mA, VCC = 4.5 V |  | 0.3 0.5 |  |
| IIH High-level input current | VCC = 5.5 V, VI = VCC |  | 1 | A |
| IIL Low-level input current | VCC = 5.5 V, VI = 0 |  | −1 | A |
| ICC Logic supply current | VCC = 5.5 V | All outputs off | 20 200 | A |
|  |  | All outputs on | 150 500 |  |
| ICC(FRQ) Logic supply current at frequency | fSRCK = 5 MHz, CL = 30 pF, All outputs off, See Figures 2 and 6 |  | 1.2 5 | mA |
| IN Nominal current | VDS(on) = 0.5 V, IN = ID, TC = 85C, See Notes 5, 6 and 7 |  | 90 | mA |
| IDSX Off-state drain current | VDS = 30 V, VCC = 5.5 V |  | 0.1 0.2 | A |
|  | VDS = 30 V, VCC = 5.5 V, TC = 125C |  | 0.15 0.3 |  |
| rDS(on) Static drain-source on-state resistance | ID = 50 mA, VCC = 4.5 V | See Notes 5 and 6 and Figures 7 and 8 | 6.5 9 | Ω |
|  | ID = 50 mA, TC = 125C, VCC = 4.5 V |  | 9.9 12 |  |
|  | ID = 100 mA, VCC = 4.5 V |  | 6.8 10 |  |

NOTES:   5.  Technique should limit TJ − TC to 10C maximum.

6.  These parameters are measured with voltage-sensing contacts separate from the current-carrying contacts.

7.  Nominal current is defined for a consistent comparison between devices from different sources. It is the current that produces a voltage drop of 0.5 V at TC = 85C.

4                                                                           POST OFFICE BOX 655303  DALLAS, TEXAS 75265

switching characteristics, VCC = 5 V, TC = 25C

SLIS093C − MARCH 2000 − REVISED APRIL 2005

| PARAMETER | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- |
| tPLH Propagation delay time, low-to-high-level output from G | CL = 30 pF, ID = 75 mA, See Figures 1, 2, and 9 | 80 | ns |
| tPHL Propagation delay time, high-to-low-level output from G |  | 50 | ns |
| tr Rise time, drain output |  | 100 | ns |
| tf Fall time, drain output |  | 80 | ns |
| tpd Propagation delay time, SRCK↓ to SEROUT | CL = 30 pF, ID = 75 mA, See Figure 2 | 15 | ns |
| f(SRCK) Serial clock frequency | CL = 30 pF, ID = 75 mA, See Note 8 | 10 | MHz |
| ta Reverse-recovery-current rise time | IF = 100 mA, di/dt = 10 A/s, See Notes 5 and 6 and Figure 3 | 100 | ns |
| trr Reverse-recovery time |  | 120 |  |

NOTES:   5.  Technique should limit TJ − TC to 10C maximum.

6.  These parameters are measured with voltage-sensing contacts separate from the current-carrying contacts.

8.  This is the maximum serial clock frequency assuming cascaded operation where serial data is passed from one stage to a second stage. The clock period allows for SRCK  SEROUT propagation delay and setup time plus some timing margin.

thermal resistance

| PARAMETER |  | TEST CONDITIONS | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| RJA Thermal resistance, junction-to-ambient | D package | All 8 outputs with equal power | 115 | C/W |
|  | N package |  | 85 |  |
|  | PW package |  | 108 |  |

PARAMETER MEASUREMENT INFORMATION

Generator

(see Note A)

7

CLR

15

SRCK

SER IN

10  RCK

8   G

5 V

1

VCC DUT

GND

DRAIN

ID

3− 6,

15 V

Output

CL = 30 pF (see Note B)

SRCK

RCK CLR

7    6    5    4    3    2    1    0

5 V

0 V

5 V

0 V

5 V

0 V

5 V

0 V

5 V

0 V

16

TEST CIRCUIT

DRAIN1

VOLTAGE WAVEFORMS

15 V

0.5 V

NOTES:  A.  The word generator has the following characteristics: tr  10 ns, tf  10 ns, tw = 300 ns, pulsed repetition rate (PRR) = 5 kHz, ZO = 50 .

B.  CL includes probe and jig capacitance.

Figure 1. Resistive-Load Test Circuit and Voltage Waveforms

POST OFFICE BOX 655303  DALLAS, TEXAS 75265                                                                                                        5

SLIS093C − MARCH 2000 − REVISED APRIL 2005

PARAMETER MEASUREMENT INFORMATION

5 V                         15 V

1

G

Output

50%

tPLH

90%

90%

50%

tPHL

5 V

0 V

24 V

Word Generator (see Note A)

7

CLR

15

SRCK

2

SER IN

10

RCK

8

G

VCC

DUT

GND

DRAIN

ID

3− 6,

11 −14

RL = 200 

Output

CL = 30 pF (see Note B)

SRCK

10%

tsu

tr

SWITCHING TIMES

50%

th

10%

tf

0.5 V

5 V

0 V

16

TEST CIRCUIT

SER IN

5 V

50%                   50%

0 V

tw

INPUT SETUP AND HOLD WAVEFORMS

SRCK

50%

50%

SER OUT

tpd                                                 tpd

50%                                               50%

SER OUT PROPAGATION DELAY WAVEFORM

NOTES:  A.  The word generator has the following characteristics: tr  10 ns, tf  10 ns, tw = 300 ns, pulsed repetition rate (PRR) = 5 kHz, ZO = 50 .

B.  CL includes probe and jig capacitance.

Figure 2. Test Circuit, Switching Times, and Voltage Waveforms

6                                                                           POST OFFICE BOX 655303  DALLAS, TEXAS 75265

SLIS093C − MARCH 2000 − REVISED APRIL 2005

PARAMETER MEASUREMENT INFORMATION

Circuit Under Test

DRAIN

TP K

2500 F

250 V

+

| 0.1 A IF di/dt = 10 A/s |  |  |  |
| --- | --- | --- | --- |
|  |  |  | 25% of IRM |
|  | trr |  |  |

IF (see Note A)

L = 0.85 mH

15 V

−

0

TP A

t2

t1                                 t3

RG

Driver

IRM

(see Note B)

TEST CIRCUIT                                                                                         CURRENT WAVEFORM

NOTES:  A.  The DRAIN terminal under test is connected to the TP K test point. All other terminals are connected together and connected to the

TP A test point.

B.  The VGG amplitude and RG are adjusted for di/dt = 10 A/s. A VGG double-pulse train is used to set IF = 0.1 A, where t1 = 10 s, t2 = 7 s, and t3 = 3 s.

Figure 3. Reverse-Recovery-Current Test Circuit and Waveforms of Source-to-Drain Diode

7

CLR

15  SRCK

5 V

1

VCC

15 V

ID

30 

Input

tw

av

5 V

t

See Note B                                  0 V

Word Generator (see Note A)

2

SER IN

10

RCK

8

G

DUT

GND

DRAIN

3− 6,

11 −14

1.5 H VDS

ID

VDS

IAS = 200 mA

V(BR)DSX = 33 V MIN

16

SINGLE-PULSE AVALANCHE ENERGY TEST CIRCUIT

VOLTAGE AND CURRENT WAVEFORMS

NOTES:  A.  The word generator has the following characteristics: tr  10 ns, tf  10 ns, ZO = 50 .

B.  Input pulse duration, tw, is increased until peak current IAS = 200 mA.

Energy test level is defined as EAS = IAS  V(BR)DSX  tav/2 = 30 mJ

Figure 4. Single-Pulse Avalanche Energy Test Circuit and Waveforms

POST OFFICE BOX 655303  DALLAS, TEXAS 75265                                                                                                        7

SLIS093C − MARCH 2000 − REVISED APRIL 2005

TYPICAL CHARACTERISTICS

PEAK AVALANCHE CURRENT

vs

TIME DURATION OF AVALANCHE

1

TC = 25C

6

VCC = 5 V

SUPPLY CURRENT

vs

FREQUENCY

TC = −40C to 125C

5

4

IAS − Peak Avalanche Current − A

ICC − Supply Current − mA

0.1                                                                                                                    3

2

1

0.01

0.1                                       1                                       10

tav − Time Duration of Avalanche − ms

Figure 5

0

0.1                         1                         10                       100

f − Frequency − MHz

Figure 6

DRAIN-TO-SOURCE ON-STATE RESISTANCE

vs

DRAIN CURRENT

rDS(on) − Drain-to-Source On-State Resistance − 

30

VCC = 5 V

STATIC

DRAIN-TO-SOURCE ON-STATE RESISTANCE

vs

LOGIC SUPPLY VOLTAGE

12

ID = 50 mA

See Note A

25

20

TC = 125C

TC = 125C

10

8

See Note A

15                                                                                                                    6

TC = 25C

10                                                    TC = 25C

4

TC = − 40C

rDS(on) − Static Drain-to-Source On-State Resistance − 

5                                                                                                                    2

TC = − 40C

0

50      70      90     110    130    150    170    190

ID − Drain Current − mA

Figure 7

2510

0

4.0         4.5         5.0         5.5         6.0         6.5         7.0

VCC − Logic Supply Voltage − V

Figure 8

NOTE A:  Technique should limit TJ − TC to 10C maximum.

8                                                                           POST OFFICE BOX 655303  DALLAS, TEXAS 75265

SLIS093C − MARCH 2000 − REVISED APRIL 2005

TYPICAL CHARACTERISTICS

140

SWITCHING TIME

vs

CASE TEMPERATURE

ID = 75 mA

120

See Note A                              tr

Switching Time − ns

100                                                       tf

80                                                 tPLH

60

tPHL

40

20

0

−50      −25        0         25        50        75       100      125

TC − Case Temperature − C

Figure 9

NOTE A:  Technique should limit TJ − TC to 10C maximum.

POST OFFICE BOX 655303  DALLAS, TEXAS 75265                                                                                                        9

SLIS093C − MARCH 2000 − REVISED APRIL 2005

THERMAL INFORMATION

ID − Maximum Continuous Drain Current of Each Output − A

0.25

0.20

0.15

0.10

MAXIMUM CONTINUOUS DRAIN CURRENT OF EACH OUTPUT vs

NUMBER OF OUTPUTS CONDUCTING SIMULTANEOUSLY

VCC = 5 V

TC = 25C TC = 100C

ID − Maximum Peak Drain Current of Each Output − A

0.30

0.25

0.20

0.15

0.10

MAXIMUM PEAK DRAIN CURRENT OF EACH OUTPUT

vs

NUMBER OF OUTPUTS CONDUCTING SIMULTANEOUSLY

d = 10%

d = 20%

d = 50%

d = 80%

0.05

0.00

TC = 125C

1          2          3          4          5          6          7          8

N − Number of Outputs Conducting Simultaneously

0.05

0.00

VCC = 5 V TC = 25C

d = tw/tperiod

= 1 ms/tperiod

1          2          3          4          5          6          7          8

N − Number of Outputs Conducting Simultaneously

Figure 10

Figure 11

10                                                                         POST OFFICE BOX 655303  DALLAS, TEXAS 75265

SLIS093C − MARCH 2000 − REVISED APRIL 2005

THERMAL INFORMATION

D PACKAGE†

NORMALIZED JUNCTION - TO -AMBIENT THERMAL RESISTANCE

vs

PULSE DURATION

10

DC Conditions

1

d = 0.5

d = 0.2

0.1

d = 0.1

d = 0.05

d = 0.02

0.01

d = 0.01

Single Pulse

RJA − Normalized Junction-to-Ambient Thermal Resistance − C/W

0.001

| t w | tc |  |  |  |
| --- | --- | --- | --- | --- |

ID

0

0.0001

0.0001

0.001                             0.01                               0.1                                 1                                  10 tw − Pulse Duration − s

† Device mounted on FR4 printed-circuit board with no heat sink

NOTES:   ZA(t) = r(t) RJA tw = pulse duration tc = cycle time

d = duty cycle = tw/tc

Figure 12

POST OFFICE BOX 655303  DALLAS, TEXAS 75265                                                                                                      11

PACKAGE OPTION ADDENDUM

www.ti.com                                                                                                                                                                                                                  7-May-2008

PACKAGING INFORMATION

Orderable Device Status (1) Package Package Pins Package Eco Plan (2) Lead/Ball Finish MSL Peak Temp (3) Type Drawing Qty

TPIC6C596D ACTIVE SOIC D 16 40 TBD CU NIPDAU Level-1-220C-UNLIM

TPIC6C596DG4 ACTIVE SOIC D 16 40 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPIC6C596DR ACTIVE SOIC D 16 2500 TBD CU NIPDAU Level-1-220C-UNLIM

TPIC6C596DRG4 ACTIVE SOIC D 16 2500 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPIC6C596N ACTIVE PDIP N 16 25 Pb-Free CU NIPDAU N / A for Pkg Type (RoHS)

TPIC6C596PW ACTIVE TSSOP PW 16 90 TBD CU NIPDAU Level-1-220C-UNLIM

TPIC6C596PWG4 ACTIVE TSSOP PW 16 90 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPIC6C596PWR ACTIVE TSSOP PW 16 2000 TBD CU NIPDAU Level-1-220C-UNLIM

TPIC6C596PWRG4 ACTIVE TSSOP PW 16 2000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

(1) The marketing status values are defined as follows:

ACTIVE: Product device recommended for new designs.

LIFEBUY: TI has announced that the device will be discontinued, and a lifetime-buy period is in effect.

NRND: Not recommended for new designs. Device is in production to support existing customers, but TI does not recommend using this part in a new design.

PREVIEW: Device has been announced but is not in production. Samples may or may not be available.

OBSOLETE: TI has discontinued the production of the device.

(2) Eco Plan - The planned eco-friendly classification: Pb-Free (RoHS), Pb-Free (RoHS Exempt), or Green (RoHS & no Sb/Br) - please check http://www.ti.com/productcontent for the latest availability information and additional product content details.

TBD: The Pb-Free/Green conversion plan has not been defined.

Pb-Free (RoHS): TI's terms "Lead-Free" or "Pb-Free" mean semiconductor products that are compatible with the current RoHS requirements for all 6 substances, including the requirement that lead not exceed 0.1% by weight in homogeneous materials. Where designed to be soldered at high temperatures, TI Pb-Free products are suitable for use in specified lead-free processes.

Pb-Free (RoHS Exempt): This component has a RoHS exemption for either 1) lead-based flip-chip solder bumps used between the die and package, or 2) lead-based die adhesive used between the die and leadframe. The component is otherwise considered Pb-Free (RoHS compatible) as defined above.

Green (RoHS & no Sb/Br): TI defines "Green" to mean Pb-Free (RoHS compatible), and free of Bromine (Br) and Antimony (Sb) based flame retardants (Br or Sb do not exceed 0.1% by weight in homogeneous material)

(3)  MSL, Peak Temp. -- The Moisture Sensitivity Level rating according to the JEDEC industry standard classifications, and peak solder temperature.

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI

to Customer on an annual basis.

Addendum-Page 1

MECHANICAL DATA

MTSS001C – JANUARY 1995 – REVISED FEBRUARY 1999

PW (R-PDSO-G**)                                                                                      PLASTIC SMALL-OUTLINE PACKAGE

14 PINS SHOWN

0,65

14

0,30

0,19

8

0,10  M

4,50

4,30

6,60

6,20

0,15 NOM

Gage Plane

1                            7

0– 8

A

0,25

0,75

0,50

1,20 MAX

0,15

0,05

Seating Plane

0,10

| PINS ** DIM | 8 | 14 | 16 | 20 | 24 | 28 |
| --- | --- | --- | --- | --- | --- | --- |
| A MAX | 3,10 | 5,10 | 5,10 | 6,60 | 7,90 | 9,80 |
| A MIN | 2,90 | 4,90 | 4,90 | 6,40 | 7,70 | 9,60 |

4040064/F 01/97

NOTES:  A.  All linear dimensions are in millimeters.

B.  This drawing is subject to change without notice.

C.  Body dimensions do not include mold flash or protrusion not to exceed 0,15. D.  Falls within JEDEC MO-153

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete. All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment.

TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third-party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

Reproduction of TI information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations, and notices. Reproduction of this information with alteration is an unfair and deceptive business practice. TI is not responsible or liable for such altered documentation. Information of third parties may be subject to additional restrictions.

Resale of TI products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service and is an unfair and deceptive business practice. TI is not responsible or liable for any such statements.

TI products are not authorized for use in safety-critical applications (such as life support) where a failure of the TI product would reasonably be expected to cause severe personal injury or death, unless officers of the parties have executed an agreement specifically governing

such use. Buyers represent that they have all necessary expertise in the safety and regulatory ramifications of their applications, and acknowledge and agree that they are solely responsible for all legal, regulatory and safety-related requirements concerning their products and any use of TI products in such safety-critical applications, notwithstanding any applications-related information or support that may be provided by TI. Further, Buyers must fully indemnify TI and its representatives against any damages arising out of the use of TI products in such safety-critical applications.

TI products are neither designed nor intended for use in military/aerospace applications or environments unless the TI products are specifically designated by TI as military-grade or "enhanced plastic." Only products designated by TI as military-grade meet military specifications. Buyers acknowledge and agree that any such use of TI products which TI has not designated as military-grade is solely at the Buyer's risk, and that they are solely responsible for compliance with all legal and regulatory requirements in connection with such use.

TI products are neither designed nor intended for use in automotive applications or environments unless the specific TI products are designated by TI as compliant with ISO/TS 16949 requirements. Buyers acknowledge and agree that, if they use any non-designated products in automotive applications, TI will not be responsible for any failure to meet such requirements.

Following are URLs where you can obtain information on other Texas Instruments products and application solutions:

| Products |  | Applications |  |
| --- | --- | --- | --- |
| Amplifiers | amplifier.ti.com | Audio | www.ti.com/audio |
| Data Converters | dataconverter.ti.com | Automotive | www.ti.com/automotive |
| DLP® Products | www.dlp.com | Broadband | www.ti.com/broadband |
| DSP | dsp.ti.com | Digital Control | www.ti.com/digitalcontrol |
| Clocks and Timers | www.ti.com/clocks | Medical | www.ti.com/medical |
| Interface | interface.ti.com | Military | www.ti.com/military |
| Logic | logic.ti.com | Optical Networking | www.ti.com/opticalnetwork |
| Power Mgmt | power.ti.com | Security | www.ti.com/security |
| Microcontrollers | microcontroller.ti.com | Telephony | www.ti.com/telephony |
| RFID | www.ti-rfid.com | Video & Imaging | www.ti.com/video |
| RF/IF and ZigBee® Solutions | www.ti.com/lprf | Wireless | www.ti.com/wireless |

Mailing Address: Texas Instruments, Post Office Box 655303, Dallas, Texas 75265

Copyright © 2009, Texas Instruments Incorporated